3GPP TSG SA WG3 (Security) Ad-hoc Meeting on FS_NSA	S3-161308

27-29. Sep 2016, SanDiego, USA

Source:	ZTE

Title:	Identity ( and Ownership ) Management

Document for:	 		  Approval

Agenda Item:	4.1  Security Architecture

Abstract

This contribution raises the issue of identity management in the security framework of NextGen,, it proposes to take the issue into the study area of security  architecture in the  work program (33.899)

------------------------------------------------------------------------------------------------------------------

Introduction:

Two major goals of 5G [ NGMN White Paper, 22.891]  are

  - supporting the vertical business and

  - flexible deployment (or tear-down) of diverse services

The concept introduced by NextGen to reach these goals is the network slicing.  A slice consists of a group of network functions so that a complete telecom (traditional or advanced) service can be provided.  The  implication of this capability for the industry is, however,  revolutionary.  While the telecom network today is often owned by a single operator, the future network infrastructure, thanks to the network slicing and other technological advancement,  will be capable of supporting multiple ownership in a layered manner.  This is a new paradigm of business model .

The enabler of this business model is the possibility of ownership  of  layers of a network infrastructure.   Figure 1 shows 4 layers (from bottom to top) : Hardware(HW),  Software(SW),  Network(NW or NS for slice),  Service Provision (SP)  that is capable of  supporting vertical industry and flexible deployment.

Hardware(HW)

Software(SW)

Network(NS)

Service(SP)

Hardware(HW)

Software(SW)

Network(NS)

Service(SP)

Figure 1:  Network components in monolithic (left) and multi-business (right) network infrastructures: Different color refers to  potentially different ownerships

Once they are, as components, implemented and operated independently, they can be owned by different administrations ,e.g.  business owners.  In total, 15 possibilities of ownerships are shown in Table 1, where “yes” means the owned part and blank means not owned by the same owner.

Table 1: Constellation of ownership in 5G and beyond: (yes) for owning and (blank) for not owning the respective component.

| Case# | HW | SW | NW | SP | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | y | y | y | y | Today’s Monolithic |
| 2 | y | y | y |  |  |
| 3 | y | y |  | y |  |
| 4 | y | y |  |  |  |
| 5 | y |  | y | y |  |
| 6 | y |  | y |  |  |
| 7 | y |  |  | y |  |
| 8 | y |  |  |  |  |
| 9 |  | y | y | y | NFV |
| 10 |  | y | y |  | NFV |
| 11 |  | y |  | y | NFV |
| 12 |  | y |  |  | NFV |
| 13 |  |  | y | y | NFV |
| 14 |  |  | y |  | NFV |
| 15 |  |  |  | y | NMVO etc. |
| 16 |  |  |  |  | All Rental |

Without loss of generality, it can be assumed that each lower layer entity can/will support multiple upper layer entities. The owner of a layer owns everything hosted by that layer: network elements, functions and customers.  The ownership can be manifested by deeds, purchase contract, rental contract or SLA. Thus, a owner holds the identity and related private information about the subscriber, together with the software instance and equipments on the corresponding layer.   Moreover, the relation of ownership is recursive from bottom to top.

The terms “ownership” and “identity” are two sides of the same coin: The owner owns the identities of a subscriber and software instances/hardware and is also obliged to keep the private information in this regard.   In the following the “identity holder” and “owner” are used alternately. In specifics,   need of identity management enters the NextGen within the context of “network slicing”, where a “network” supports multiple “slices” , each is capable of providing at least complete network service or vertical network services.  An identity management will enable network slicing and help to manage the proliferated identities in the NextGen.

************************************Start of changes: Add*******************************************************

### 5.1.3.y. KI#1.y: Identity Management in the Security Architecture

While identity management in LTE/EPC deals with customer credentials in terms of  privacy and secure communication, the identity issue in NextGen  has a broader scope.   It deals with subscriber identities of different trust domains within the same network . Any approach should try to minimize the impact on service experience, while reusing of the established procedures and components.  For instance, an identity can belong to a network slice that is hosted by another operator who hosts more than one slices or none.  As identity we refer primarily to the subscriber identity, but  including also identity  of  the network function instance, network slice instance,  user devices or even services.  A feasible authentication mechanism relies on a feasible identity definition and management architecture. The latter can have at least one, or a combination, of the followings:

- Arch 1: Flat distribution as in Figure 3(left):  Each identity holder acts in coordination with other equal identity holders during authentication and authorization. Information is exchanged  between them in a secure manner.  The actual information to be exchanged, the protocol to be used and the procedure involved will be determined such that the secrecy is maintained for each identity holder during operation.

Figure 2: Possible architecture for 3 identity holders:  Flat(left), Hierarchy(middle) and Agent (right). Red arrows indicate interaction with HSS(Anchor) and UE. Orange button refers to distinguished identity holder.

- Arch 2: Hierarchical as in Figure 3(midle):  One identity holder possesses more information of,  and works on behalf of,  other identity holders.  The information division between the hierarchies may be specified depending on ownership constellation and  the identity holder of the higher hierarchy could be implemented as a proxy of the identity holders of the lower hierarchy.

This may apply where CN and RN are not on the same slice, or when a primary MNO owns the entire infrastructure and it rents some slices to a tenant, i.e. another MNO.

The demarcation between Architecture 1 and Architecture 2 can be obscured when function is split between different identity holders to facilitate operation in certain scenarios.

- Arch 3: Identity Agent as in Figure 3(right):  A third party works on behalf of all identity holders to handle the identities related communication and operation.  It needs secure association with each identity holder, obfuscation of data transfer and it takes part in the authentication and authorization of subscribers as well as network instances.

An agent acts like an identity provider, but the confidential information about the subscribers are still under the jurisdiction of individual MNOs. It is deemed generic, as it may well include Arch1 and Arch 2 through collocation, thus reducing interfaces.

Caveat:  Both centralized or distributed identity management (IM) have pros and cons:

- Centralized IM:

  - Pro:  More feasible of being placed in physical secure location

  - Con: Possibly not very scalable. But this is an issue only for large number of identity holders

- Distributed IM:

  - Pro:  More capable of  adapting to changed identity ownership relation and physical network change

  - Con: Cost and potential security risk due to number of interfaces.

Conceptually,  Arch 3 can serve as the generic architecture, while the other two the implementation variants.

Threats(Rational):   LTE/EPC is a monolithic system, where the infrastructure and major services are operated and administrated by a single authority.   This condition is no more valid in the future network, when trust boundaries will present within the same (physical) infrastructure.  The security architecture needs to adapt itself to this new environment. Otherwise the evolution path of NextGen will be impaired.

What used to be single HSS can now be shared by multiple identity holders, possibly of different trust domains,   within the same infrastructure, i.e. (HW,SW,NW,SP).       Different designs may have different security risks, such as user information loss,  impersonation, DoS  and  negative impact on performance etc.

Migration: Given that the current business model assumes that a single operator owns everything, the migration into a multi-ownership network future may start with Arch2, where a single operator, i.e. the infrastructure owner,  maintains identity management for him/herself as well as  on behalf of his/her tenants.

Example:  In case of network slicing ,  a network slice (NS) and the network infrastructure hosting the slice may have a different subscriber identity holders, as they are owned by different business owners.   The feasibility and scalability of an architecture may well depends on whether CN, RN or CN&RN are completely sliced or partially or not at all, in addition to the ownership constellation of different layers..

Requirement:

- Definition of identity holder function for secure handling  of user , device and network information.

- Interfaces from the identity management to other network entities in an appropriate architecture.

- Potential reuse of existing identity management techniques (GBA, OpenID, etc.) or components

- Evaluation of  architecture design against feasibility, security and scalability.

************************************End of changes*******************************************************