# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171771

# 07-11 August 2017 Dali (China)	revision of S3-17xabc

# Source:	CATT

# Title:	Questions and interim agreements for KI #8.2-Security Differentiation

# Document for:	Discussion and Approval

Agenda Item:	8.3.8 Network slicing security

1	Decision/action requested

This contribution proposes an interim agreement for the Key Issue #8.2: Security mechanism differentiation for network slices.

2	References

N/A

3	Rationale

This contribution proposes an answer for an interim agreement question related to the key issue #8.2: Security mechanism differentiation for network slices, for TR33.899.

4	Detailed proposal

It is proposed to introduce the following changes to the TR 33.899.

* * * Start of changes * * * *

E.8.2.1 	Security mechanism differentiation for network slices

E.8.2.1.1 	Description of Question

If different types of slices have different requirements of keys (e.g. key length or policies of key expiration and so on) because of different service requirements, should there be slice-specific keys for different slices when one UE simultaneously access to multiple network slices which are different slice types  ?

E.8.2.1.2 	Interim Agreement

The 5G system will not support security mechanism differentiation (e.g. cryptographic algorithm, key length) for network slices in 5G phase 1.

The agreement should not preclude introducing security mechanism differentiation for network slices in 5G phase 2.

* * * End of Changes * * *