# 3GPP TSG SA WG3 (Security) Meeting #89	S3-173215

# 27 November – 1 December, 2017, Reno, USA	revision of S3-17xabc

# Source:	China Unicom

# Title:	Key issue on network slice instance commissioning

# Document for:	Approval

Agenda Item:	8.4 FS_ NETSLICE-MGT_Sec (Rel-15)

1	Introduction

This contribution proposes to add a key issue on Network Slice Instance commissioning. During the commissioning phase, the NSMF determines to reuse an existing NSI or create a new NSI. If an existing NSI can be reused, the operator may reconfigure the existing NSI. This reconfiguration needs to consider with the potential attack by resources exhaustion, e.g. the malicious user requires the same network slice instance continually. The related security threats and potential security requirements are also proposed.

We kindly propose SA3 to agree the following pCR.

2	Proposal

It is proposed to approve below pCR.

**********************Begin of changes********************************

5.x	Key issue #x: Network slice instance commissioning

5.x.1	Key issue details

The management aspects of a Network Slice Instance are described in TS 28.530 [2] by the four phases:

- Preparation

- Commissioning

- Operation

- Decommissioning

Network slice provisioning in the commissioning phase involves the creation of an NSI. During NSI creation all resources to the NSI have been created and configured to satisfy the network slice requirements. The creation of an NSI can include the creation or modification of NSI constituents [2].

During the commissioning phase, the NSMF determines to reuse an existing NSI or create a new NSI. If an existing NSI can be reused, the operator may reconfigure the existing NSI. This reconfiguration needs to consider with the potential attack by resources exhaustion, e.g. the malicious user requires the same network slice instance continually.

5.x.2	Security threats

The resources exhaustion attack may be launched by the malicious uer through require the same network slice instance continually.

5.x.3	Potential security requirements

It shall not be feasible for a user to require the same network slice instance frequently.

The detection mechanism in NSMF shall be supported to avoid the resources exhaustion attack by requires the same network slice instance continually.

*********************************End of changes*******************************