3GPP TSG-SA WG3 Meeting #91Bis 	S3-181910

# La Jolla (USA), 21- 25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deletion of Editor Notes in 6.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-05-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Clause 6.4.2 there is an EN “It is FFS if an explicit indication is required to signal whether to activate a newly created partial NAS security context simultaneously on all the NAS connections or only the current one“. It doesn’t look feasible to activate a newly created partial NAS security context simultaneously on all the NAS connections. Clarify that a newly created partial NAS security context is activated only on the NAS connection reporting the mobility, NAS context is enabled one by one for multiple connections. In clause 6.4.2.2 a paragraph is repeated, this needs deletion. There is also an EN “ “Editor's Note: Above requirements need to be revisited after studying concurrency, mobility and interworking use cases”. This can be deleted. Since concurrency rules have been agreed. During mobility and interworking each NAS link have to be managed individually, combined simultaneous procedures for multiple connections doesn’t look feasible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the Editor’s Note in 6.4.2 and 6.4.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ********************* Start of 1st change *************************

### 6.4.2	Security for multiple NAS connections

#### 6.4.2.1	Multiple active NAS connections with different PLMNs

TS;23.501 [2] has a scenario when the UE is registered to a VPLMN's serving network via 3GPP access and to another VPLMN's or HPLMN's serving network via non-3GPP access at the same time. When the UE is registered in one PLMN's serving network over a certain type of access (e.g. 3GPP) and is registered to another PLMN's serving network over another type of access (e.g. non-3GPP), then the UE has two active NAS connections with different AMF's in different PLMNs. As described in clause 6.3.2.1, the UE shall independently maintain and use two different 5G security contexts, one per PLMN serving network. Each security context shall be established separately via a successful primary authentication procedure with the Home PLMN. All the NAS and AS security mechanisms defined for single registration mode are applicable independently on each access using the corresponding 5G security context.

NOTE: 	The subscription belongs to a single HPLMN.

#### 6.4.2.2	Multiple active NAS connections in the same PLMN's serving network

When the UE is registered in a serving network over two types of access (e.g. 3GPP and non-3GPP), then the UE has two active NAS connections with the same AMF. A common 5G NAS security context is created during the registration procedure over the first access type. In order to realize cryptographic separation and replay protection, the common NAS security-context shall have parameters specific to each NAS connection. The connection specific parameters include a pair of NAS COUNTs for uplink and downlink and unique NAS connection identifier. The value of the unique NAS connection identifier shall be set to "0" for 3GPP access and set to “1" for non-3GPP access.

In mobility and interworking scenarios, a newly created partial NAS security context is activated only on the NAS connection reporting the mobility, NAS context is enabled one by one for multiple connections.

## ********************* End of 1st change *************************