3GPP TSG-SA WG3 Meeting #92 ad-hoc	 		S3-182860

Harbin (China) 24-28 September 2018

Title:	DRAFT LS on ResumeMAC-I Calculation for RRC Resume Request

Response to:

Release:	Rel-15

Work Item:	5GS_Ph1-SEC

Source:	Huawei to be SA3

To:	RAN2, RAN3

Cc:

Contact Person:

#### Name:	Ahmad Muhanna

Tel. Number:	+1-972-814-8203

E-mail Address:	ahmad dot muhanna at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-182852, S3-182853

1. Overall Description:

SA3 has discussed the ResumeMAC-I input parameters in general and in specific for RNAU without context relocation. SA3 concluded that using source (C-RNTI+PCI) as an input to ResumeMAC-I calculation in the case of RNAU without context transfer creates a mismatch between the UE and the old gNB. SA3 concluded that it is best for ResumeMAC-I input to use the I-RNTI instead of the Source (C-RNTI+PCI).

In addition, “Resume cause” which can be assigned a value of “emergency”, “highPriorityAccess”, “delayToleranceAccess”, etc. is also used to indicate “RNA Update”. Therefore, SA3 concluded that “Resume cause” field shall be protected by the authentication token when calculating ResumeMAC-I for RRC Resume Request message to prevent a denial of service to the UE, e.g., UE requesting RRC Resume with resumecause is set to “emergency” but an attacker changes the resumecause value to “RNAU” which will cause the UE to be sent back to INACTIVE again rather than receiving emergency service.

SA3 would like to kindly request RAN2 and RAN3 to use the following when calculating the ResumeMAC-I:

- The current VarResumeMAC-Input parameters (Source C-RNTI, Source PCI, Resume Constant, and Target Cell ID) as in TS38.331 shall be replaced by the following parameters: (I-RNTI, Resumecause, Target Cell ID). Changes can be found in TS33.501 and the attached contribution (S3-182852).

- In the case of RNA Update without context relocation, the old gNB (anchor gNB) shall assign a new I-RNTI to the UE after each RNA Update without context relocation. Changes can be found in TS33.501 and the attached contribution (S3-182853).

2. Actions:

To RAN2 and RAN3 groups.

ACTION: SA3 kindly requests RAN2 and RAN3 to take the above information into account when calculating ResumeMAC-I and addressing RNAU w/o context transfer procedure.

3. Date of Next TSG-SA WG3 Meetings:

SA3#93	                12-16 November 2018			Spokane, USA

SA3#94	                28 Jan. – 1st Feb. 2019		Kochi, India