# 3GPP TSG SA WG3 (Security) Meeting #96	S3-192916

# 26 – 30 Aug, 2019, Wroclaw (PL)

# Source:	NIST, ATT, Sprint, CableLabs, Cisco

# Title:	New KI: Botnet threats caused from improper CIOT device usage

# Document for:	Approval

# Agenda Item:	7.8

# 1	Decision/action requested

Approve this contribution for a new key issue in TR 33.861.

# 2	References

[1]  Enhancing the Resilience of the Internet and Communications Ecosystem Against Botnets and Other Automated, Distributed Threats. May 22, 2018. Available at https://csrc.nist.gov/CSRC/media/Publications/white-paper/2018/05/30/enhancing-resilience-against-botnets--report-to-the-president/final/documents/eo_13800_botnet_report_-_finalv2.pdf.

[2] United States Computer Emergency Readiness Team, Alert (TA16-288A): Heightened DDoS Threat Posed by Mirai and Other Botnets, https://www.us-cert.gov/ncas/alerts/TA16-288A (last revised Oct. 17, 2017).

[3] Brian Krebs, Fear the Reaper, or Reaper Madness? Krebs on Security (Oct. 27, 2017, 4:39 PM), https://krebsonsecurity.com/2017/10/fear-the-reaper-or-reaper-madness/.

[4] Lili Hay Newman, GitHub Survived the Biggest DDoS Attack Ever Recorded, Wired (Mar. 1, 2018, 11:01 AM), https://www.wired.com/story/github-ddos-memcached/.

# 3	Rationale

Automated and distributed attacks, such as botnet-based distributed denial of service (DDoS) attacks, have been a concern since the early days of the Internet. Traditional DDoS mitigation techniques relied on excess network capacity to absorb the effects of botnets. With new botnets that capitalize on the sheer number of “Internet of Things” (IoT) devices, DDoS attacks have outstripped excess capacity, resulting in significant outages. [1]

IoT devices may be designed with limited security functionality to accommodate constraints in power and computation and minimize development cost. For example, many of the devices compromised by the Mirai botnet were shipped with hardcoded administrative passwords. [2] While there have been advances in IoT security, new techniques that allow edge networks to effectively protect locally connected IoT devices are urgently needed. By communicating the communications requirements required by a locally connected device to support intended operations, networks can detect or even block attempts to compromise devices or limit compromised devices’ participation in a botnet attack.

# 4	Detailed proposal

************************************* Start of Changes *****************************************

## Key issue #X: Botnet Attacks from Improper CIOT Device Usage

## 5.X.1	Description

A vulnerability posed to CIOT devices is the ability for them to be exploited by botnets and other automated distributed threats.  Once compromised, a CIOT device can be used to attack any other system on the internet, often times there is nothing limiting the functionality of a compromised CIOT devices to be used in ways the manufacture did not intend. The ability for the network to be aware what type of access and network functionality a CIOT device requires to operate properly enables the network to block any behaviour outside of that scope. Constraining the communication abilities of exploited CIOT devices reduces the potential for the devices to be used in these types of attacks.

### 5.X.2 	Potential security threat

CIOT devices support limited security functionality (relative to modern laptops and cell phones) and are highly vulnerable to compromise.[2] [3] Once compromised, devices may be directed to perform operations the manufacturer did not intend. While the capabilities of individual CIOT devices are often limited, the capabilities of CIOT-based botnets in aggregate may be sufficient to cause major network and enterprise outages.[4] Misuse of compromised IoT devices has been a critical component in recent automated botnet attacks, specifically distributed denial of service attacks.

### 5.X.3 	Potential security requirements

It should be possible for usage description information for the device type to be available to the serving network. It should be possible to verify the source of usage description information, and to verify the usage description applies to the specific CIOT device.

************************************* End of Changes *****************************************