3GPP TSG-SA3 Meeting #98Bis-e 	S3-200614

# e-meeting, 14-17 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft-CR | rev | --- | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Token-based authorization in Scenario D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes a way forward for token-based authorization in Scenario D. It is based on the proposal for authentication between NF and NRF in indirect communication scenarios in the companion contribution S3-200613. The idea is that for Option 1 (where NF and NRF can mutually authenticate, see S3-200613), Solution 1 described in S3-200284 is used. This solution uses authorization codes signed by the NRF. For Option 2 (where there are NFs that cannot mutually authenticate with an NRF), this contribution leaves the solution for further study. This draft-CR re-uses text from the earlier draft-CR S3-200287. Changes are by user "Ericsson" on top of changes by user "S3-200287". This is for convenience, so that it is easier to understand what is re-used from the earlier proposal in S3-200287 and what is new. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** BEGIN CHANGE 1 ***

##### 13.4.1.X.3		Authorization for indirect communication with delegated discovery procedure

##### 13.4.1.X.3.0	General

The choice of deployment option for NRF-NF authentication as specified in clause 13.3.1.1 influences which procedure to use for token-based authorization.

Editor's Note: When the procedures are sufficiently stable, add a brief summary of the differences and similarities of the procedures.

NOTE: 	In both procedures, the access tokens are requested by an SCP representing the NF consumer on behalf of the NF consumer, but the tokens are issued for the NF consumer itself (i.e. the subject claim of the access token is the NF instance ID of the NF consumer). As the SCP requests the token, however, the NRF cannot distinguish between access token requests by different NF consumers represented by the same SCP. Hence, access token granularity finer than on SCP level cannot be based on information verified by the NRF itself.

A mechanism that depends on the chosen deployment option ensures that the NRF has verified information about which SCP is authorized to request access tokens on behalf of which NF consumer.

##### 13.4.1.X.3.1	Option 1

In deployment option 1, the NF service consumer and NRF are able to authenticate each other directly.

Before sending a service request to the SECOP, the NF Service Consumer requests an authorization code from the NRF. The authorization code is signed by the NRF. Authorization codes shall be JWT according to RFC 7523 [x]. The request of the authorization code shall be performed over a mutually authenticated TLS connection between NF Service Consumer and NRF.

Request of authorization code:

NRF

NF consumer

1. Nnrf_AuthorizationCode_Get Request

2. Nnrf_AuthorizationCode_Get Response (authorization code)

Figure 13.4.1.X.3.1-1: Request of authorization code by the NF Service Consumer

1. 	Nnrf_AuthorizationCode_Get Request: The NF consumer requests a new authorization code from the NRF.

2.	Nnrf_AuthorizationCode_Get Response: The NRF retrieves the NF instance ID of the consumer from the client certificate used to establish the TLS connection. The NRF issues an authorization code according to RFC 7523 [x]. The issuer is the NRF, the subject is the NF consumer, the audience is the NRF. The NRF also sets an expiry time for the authorization code. The NRF signs the authorization code with its private key and sends the authorization code to the NF consumer.

Service Request, discovery and access token request: These steps shall be performed over mutually authenticated TLS connections between NF Service Consumer and SECOP, SECOP and NRF, and SECOP and NF Service Producer.

NF consumer

NRF

SECOP

NF producer

1. Service request (authorization code)

2. Discovery

3. Nnrf_AccessToken_Get Request (authorization code)

4. Nnrf_AccessToken_Get Response (access token)

5. Service Request (access token)

6. Service Response

7. Service Response

Figure 13.4.1.X.3.1-2: Service Request, delegated discovery and access token request

1. 	The NF Service Consumer sends a service request to the SECOP, it includes a valid authorization code received during the authorization code request procedure with the NRF.

2.	If necessary, the SECOP performs service discovery with the NRF.

3. 	Nnrf_AccessToken_Get Request: The SECOP sends an access token request to the NRF according to clause 13.4.1.1, with the following differences: The SECOP takes the role of the NF Service Consumer. The SECOP includes the authorization code received from the NF Service Consumer in Step 1.

4.	Nnrf_AccessToken_Get Response: The NRF validates the authorization code received in the access token request and retrieves the NF instance ID of the consumer from the authorization code. If the NRF determines that the authorization code is valid and the NF consumer is authorized, the NRF issues an access token with the NF instance ID of the NF Service Consumer retrived from the authorization code as subject. The NRF then sends the access token to the SECOP. The NRF should not issue access tokens with a lifetime that exceeds the validity time of the authorization code significantly.

NOTE 2: An authorization code used in this context is generally a short-lived representation of the authorization grant. In practice, this requirement will usually mean that refresh tokens are not issued in response to this grant request, and access token will be issued with a reasonably short lifetime. The SECOP can refresh an expired access token by requesting a new one using the same authorization code, if it is still valid.

5.	The SECOP sends the service request to the producer, including the access token received in Step 4.

6.	The producer validates the access token. If the token is valid, the producer sends the service response to the SECOP.

7.	The SECOP forwards the service response to the NF consumer.

##### 13.4.1.X.3.2	Option 2

Editor's Note: to be added.

## *** END CHANGE 1 ***