3GPP TSG-SA3 Meeting #98 	S3-200046

# Sophia Antipolis,France 2-6 March 2020

# Source:	3GPP SA2

Title:	Questions on onboarding requirements

# Document for:	Information, Discussion

# Agenda Item:  6.1

# SA WG2 Meeting #136AH	S2-2001729

# 13-17 January 2020, Incheon, Republic of Korea	(e-mail revision 8 of S2-2001669)

Title:		Questions on onboarding requirements

Reply to:	-

Release:	Release 17

Work Item:	FS_eNPN

Source:	SA2

To:		SA1

Cc:		SA, CT1, SA3, CT6

Contact person:	Haris Zisimopoulos

harisz@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 has started working on the study on enhanced support of non-public networks. Based on service requirements documented in clause 5.1 of TS 22.263, SA2 developed key issue #4: UE Onboarding and remote provisioning in TR 23.700-07.

SA2 started working on solutions for this key issue and is looking for clarification on the interpretation of the term: “non-3GPP identities and credentials” documented in the following service requirement in TS 22.263:

The 5G system shall support a secure mechanism for a network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.

Q1) SA2 would like to verify with SA1 whether the above-quoted requirement includes the provisioning of the following for Stand-alone non-public networks (SNPNs):

- IMSI accompanied by AKA credentials, both used for SNPN authentication

- IMSI accompanied by AKA credentials, the IMSI being used to derive a Network Specific Identifier that will be used for SNPN authentication with the AKA credentials

SA2 would like to emphasise that use of (a) above is explicitly supported and (b) is not precluded by SA2 specifications for SNPNs.

Q2) SA2 would like to verify with SA1 whether the above-quoted requirement applies to PNI-NPN, which is the NPN “hosted by a PLMN” as described in TS 22.261 clause 6.25.1, or not, and what would be the corresponding use cases.

Q3) If SA1 confirm the above-quoted requirement applies to PNI-NPN in Q2, SA2 have further Q3 as below.

For PNI-NPN, a UE may perform secondary PDU session authentication using 3rd party credentials, if the NPN is integrated in PLMN by means of dedicated DNNs, and/or a UE may perform Network specific slice authentication and authorisation (NSSAA) using 3rd party credentials if the NPN is integrated in PLMN by means of network slice. Given the authentication procedures already specified in TS 23.501, TS 24.501 and TS 33.501, SA2 would also like to ask whether provisioning for identities and credentials used for Network specific slice authentication and authorisation (NSSAA) and secondary PDU session authentication should be considered to be covered as part of NPN service requirement for onboarding and remote provisioning solution.

# 2	Actions

To SA1

ACTION: 	SA2 would like SA1 to provide clarification of the service requirements in TS 22.263 and answer the questions above.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 137		24-28 February	2020		New Delhi, India

TSG SA WG2 Meeting 138		20-24 April 2020		Dubrovnik, Croatia