3GPP TSG-SA3 Meeting #99e 	S3-201477

# e-meeting, 11 -15 May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.220 | CR | draftCR | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living document of Service Based Interfaces for GBA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | GBA_5G |  |  |  |  |  | Date: |  |  | 2020-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | GBA related services are still specificied as reference point interfaces. The integration of GBA to 5GC requires these interfaces to be specified in SBA form. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GBA service interfaces are specified as an Annex to the 33.220. Currently the following interactions are specified with reference point interfaces: - BSF requests the authentication vectors from the HSS for the bootstrapping procedure. Therefore similar interfaces to the HSS need to be specified. - NAF performs an authentication request to the BSF in order to authenticate the UE for the purpose of GBA and get the derived application key, lifetime etc. - The network functions offering these new services (HSSand BSF) need to register to the NRF so that they are discoverable by any relevant network function. The contribution includes a skeleton for the Annex in TS 33.220 for the introduction of SBA interfaces of GBA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no SBA interfaces for GBA in 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

# Annex X (normative): Support of SBA in GBA

## X.1	General

Editor’s Note: More details to be specified.

### X.1.0	 Overview

### X.1.1 Architectural Support

### X.1.2	 Reference point to support SBA in GBA

### X.1.3	 Service based interface to support SBA in GBA

## X.2 	GAA/GBA SBA Services

Editor’s Note: More details to be specified.

Editor’s Note: For 5G, we only have the AUSF, UDM. Who will play the role of HSS is FFS.

Editor’s Note: Whether GBA AKA is still application in 5G-GBA is FFS.

### X.2.1	HSS Services

#### X.2.1.1 General

### X.2.2 BSF Services

## X.2.2.1 General

## X.X	SBI Capable NF Discovery and Selection

Editor's Note: The NF and detailed contents to be specified.

**** END OF CHANGES ****