3GPP TSG-SA WG3-LI Meeting #35 	DocNumber SA3LI09_096r1

- Scottsdale, USA, 03-05 November 2009

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum |  rev | - |  Current version: | 8.8.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction of misalignments for values of “initiator” parameter for EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3-LI |  |  |  |  |  |  |  |  |  |
| Work item code:  | LI-8 |  |  |  |  |  |  | Date:  |  | 03/11/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | In the current version of the specification, parameter “initiator” provided in some IRI records can only assume values “network-initiated” or “intercept-subject-initiated”. However, when the value is not available, in line with the ASN.1 coding, a specific value needs to be provided. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The value “not-available” is added in the tables in which the value was omitted. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Misalignment between different sections of the specification. It would be unclear how to include in IRIs the information that the parameter initiator is not available. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 10.5.1.1 (Table 10.5.1.1.3), 10.5.1.2 (Table 10.5.1.2.1), 10.5.1.3 (Table 10.5.1.3.1), 10.5.1.4 (Table 10.5.1.4.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

Table 10.5.1.1.3: Bearer Activation (unsuccessful) REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| event type | C | Provide EPS Bearer Activation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the attempt to connect was made; this information may be provided by the UE; the parameter is applicable only for default bearer activation. |
| RAT type | C | Provide the Radio Access Type used by the target subscriber. |
| initiator | C | Provide to indicate whether the EPS bearer activation is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| failed bearer activation reason | C | Provide information about the reason for failed bearer activation attempts of the target subscriber. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. The parameter carries the requested EPS bearer QOS. |
| Bearer activation type | C | Provides information on default or dedicated bearer failed activation |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. The parameter carries the subscribed APN-AMBR. |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE |
| Procedure transaction identifier | C | Used to associate the EPS bearer activation attempt to other messages triggering the procedure. |
| Linked EPS bearer identity | C | Provides, in case of failed dedicated bearer activation attempt, the EPS bearer id of the associated default bearer; not applicable in case of default bearer activation attempt. |
| Traffic Flow Template TFT | C | The TFT associated to the dedicated bearer activation attempt; not applicable in case of default bearer activation attemot |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover |

*** NEXT MODIFICATION ***

Table 10.5.1.2.1: Bearer Activation (successful) and Start of Interception with active bearer BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide, depending on the reported event, Bearer activation or Start of interception with active bearer event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | Provide to identify the packet data network to which the connection is made (valid only for default bearer activation). |
| PDN address allocation | C | Provides the PDN type and PDN address(es) used by the network. |
| initiator | C | Provide to indicate whether the EPS bearer activation is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. It carries the EPS bearer QOS associated to the established bearer. |
| Bearer activation type | C | Provides information on default or dedicated bearer activation. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. The parameter carries the APN-AMBR used for the established bearer |
| Protocol configuration options | C | Provide information about the protocol configuration options requested by the UE. |
| Procedure transaction identifier | C | Used to associate the EPS bearer activation to other messages triggering the procedure. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Linked EPS bearer identity | C | Provides, in case of dedicated bearer activation, the EPS bearer id of the associated default bearer; not applicable in case of default bearer activation. |
| Traffic Flow Template(s) TFT | C | The TFT associated to the dedicated bearer activation; not applicable in case of default bearer activation. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| RAT type | C | The Radio Access Type used by the target subscriber (only applicable to default bearer activation). |
| Correlation number | C | Provide to uniquely identify the EPS bearer delivered to the LEMF and to correlate IRI records with CC. |

*** NEXT MODIFICATION ***

Table 10.5.1.3.1: Bearer Modification CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Bearer modification event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| initiator | C | Provide to indicate whether the EPS bearer modification is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's UE. |
| EPS bearer QOS | C | Provide to identify the QOS parameters. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate for the APN. |
| Procedure transaction identifier | C | Used to associate the EPS bearer modification to other messages triggering the procedure. |
| EPS bearer id | C | Provides the EPS bearer id allocated by the network. |
| Traffic Flow Template(s) TFT | C | The TFT associated to the EPS bearer modification; |
| RAT type | C | The Radio Access Type used by the target subscriber. |
| APN-AMBR | C | The Aggregate Maximum Bit Rate foreseen for the APN. |
| Handover indication | C | Provide information that the procedure is triggered as part of a handover. |
| Correlation number | C | Provide to uniquely identify the EPS bearer delivered to the LEMF and to correlate IRI records with CC. |

*** NEXT MODIFICATION ***

Table 10.5.1.4.1: Bearer Deactivation END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN |  |  |
| observed IMSI | C | Provide at least one and others when available. |
| observed ME Id |  |  |
| event type | C | Provide Bearer Deactivation event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| initiator | C | Provide to indicate whether the EPS deactivation is network-initiated, intercept-subject-initiated, or not available. |
| network identifier | M | Shall be provided. |
| correlation number | C | Provide to uniquely identify the PDP context delivered to the LEM and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject's MS. |
| Bearer deactivation type | C | Provides information on default or dedicated bearer deactivation. |
| Bearer deactivation cause | C | Provide to indicate reason for deactivation. |
| EPS bearer id | O | Provides the identity of the deactivated bearer. |
| Procedure Transaction Identifier | C | Used to associate the EPS bearer deactivation to other messages triggering the procedure. |

*** END OF MODIFICATIONS ***