3GPP TSG-SA WG3-LI Metting #41

Philadelphia, USA, 10-12 May 2011

CONTRIBUTION

TITLE:	Discussion Paper: Comparing IMS Conference Calls with Advanced Conferencing

SOURCE*:	Nokia Siemens Networks

ISSUE NUMBER:

_______________________________

ABSTRACT

This contribution for discussion purposes and to evaluate in determining whether any action has to be taken within the SA3-LI in references to the 3GPP IMS Conferencing with the noted differences from the PTSC LAES Advanced Conferencing.

_______________________________

# INTRODUCTION

The LAES for Advanced Conferencing was going through a revision in handling the comments received during the initial ballot review process. Within the 3GPP SA3 LI, a similar work effort is in progress under the name IMS Conferencing. While PTSC LAES is attempting to complete the document so as to send it for a default letter ballot, this contribution compares the two work efforts focusing the differences between the two.

The contribution is for discussion and proposes the 3GPP SA3 LI to determine what action has to be taken for the differences noted.

# DISCUSSION

For this comparison, the 3GPP Release 10 work effort is considered:

| # | Topic | Advanced Conferencing | IMS Conferencing |
| --- | --- | --- | --- |
| 1 | Working Group | PTSC-LAES | 3GPP SA3 LI |
| 2 | Type of Specification | Technical Report (informative) | Standard (normative) |
| 3 | Conference Model | IETF RFC 4353 | 3GPP TS 24.147 |
| 4 | Functional Entities | Conference Manager, Conference Focus, Conference Mixer | MRFC, Application Server, MRFP |
| 5 | Stage 1 | Clause 5 of the document | 3GPP TS 33.107 Clause 11 |
| 6 | Stage 2 | Clause 6, Clause 7 and Annex A of the document | 3GPP TS 33.107 Clause 11 |
| 7 | Stage 3 | Clause 8 and Annex A of the document | 3GPP TS 33.108 |
| 8 | Media for conference | Voice | Not specific |
| 9 | Conference Id as target | Footnote | Normative |
| 10 | Subject joining associate’s conference | Not in scope | FFS |
| 11 | Floor Control Events | Not in scope | FFS |
| 12 | Conference creation via web interface | Not specifically mentioned | Not in the scope |
| 13 | Start of Conference | When the first party joins the conference | When the first party joins the conference When the host joins the conference (defined at one place in 33.108 ) |
| 14 | Subject on conference call | Subject need not be on the conference for the interception to begin | Subject need not be on the conference for the interception to begin |
| 15 | Conference creation | Target sends a request for conference creation | Target creates the conference Target provisions the conference |
| Events |  |  |  |
| 16 | Conference Create | Optional Conference Reservation Created | Same as other IRI (i.e., not optional) Stage 2: Creation of Conference Stage 3: Conference Service Creation |
| 17 | Conference Update | Optional Conference Reservation Updated | Same as other IRI (i.e., not optional) Stage 2: Update of Conference Stage 3: Conference Service Update |
| 18 | Conference Delete | Optional Conference Reservation Deleted | Same as other IRI (i.e., not optional) Stage 2: Update of Conference Stage 3: Conference Service Update |
| 19 | Conference Already Create | Optional Conference Reservation Already Created | Not defined |
| 20 | Conference Start (successful) | Conference Started | Stage 2: Start of Conference Stage 3: Conference Service Start (successful) |
| 21 | Conference Start (unsuccessful) | Conference Party Invited Failed | Stage 2: Start of Conference Stage 3: Conference Service Start (unsuccessful) |
| 22 | Conference Already Start | Conference Already Started | Stage 2: Start of Interception on Active Conference Stage 3: Start of Intercept with Conference Active |
| 23 | Conference Stop (successful) | Conference Stopped | Stage 2: Conference End Stage 3: Conference Service End (successful) |
| 24 | Conference Stop (unsuccessful) | Not defined | Stage 2: Conference End Stage 3: Conference Service End (unsuccessful) |
| 25 | Party Join (successful) | Conference Party Joined | Stage 2: Party Join Stage 3: Conference Service Party Join (successful) |
| 26 | Party Join (unsuccessful) | Conference Party Invited Failed | Stage 2: Party Join Stage 3: Conference Service Party Join (unsuccessful) |
| 27 | Party Leave (successful) | Conference Party Dropped | Stage 2: Party Leave Stage 3: Conference Service Party Leave (successful) |
| 28 | Party Leave (unsuccessful) | Not defined | Stage 2: Party Leave Stage 3: Conference Service Party Leave (unsuccessful) |
| 29 | Party Invited | Conference Party Invited | Not defined |
| 30 | Bearer Modification (successful) | Not defined | Stage 2: Conference Bearer Modification Stage 3: Conference Service Bearer Modify (successful) |
| 31 | Bearer Modification (unsuccessful) | Not defined | Stage 2: Conference Bearer Modification Stage 3: Conference Service Bearer Modify (unsuccessful) |
| 32 | CC | Mixed Form | Combined Separated |

In the above comparison, the following points will have to be evaluated:

- Clarification on whether to send the optional CII messages when the conference reservation related functions are done via web access.   3GPP clearly states that such methods if used are outside the scope of 3GPP LI specs.

- Should there be an event that differentiates the two cases: conference begins when the first party joins the conference or conference begins when the host joins the conference? The 3GPP 33.108 has such an event indicated at one place (3GPP LI specs may be modified).

- In 3GPP, there are two events that could lead to conference reservation created: target creating the conference and target provisioning the conference. Better to clarify if the two are in fact separate events since the ACS spec talks about subject requesting the conference.

In the 3GPP LI specification, there are quite a few differences for the information carried in the IRI as compared to the information carried in the messages defined in the ACS TR. The following are few examples:

| Message | Advanced Conferencing | IMS Conferencing |
| --- | --- | --- |
| Conference Started | Case Identity (M) | Lawful Intercept Identifier (M) |
|  | IAP System Identity (M) | Network Identifier (M) |
|  | Time Stamp (M) | Event Date, Event Time (M) |
|  | Conference Id (M) | Correlation Number (M) |
|  |  | Conference URI and/or Temporary Conference URI (M) |
|  | Conference Parties (M) | List of Conferees and/or List of Waiting Conferees (M) |
|  |  | List of Potential Conferees (C) |
|  | CC Address (C) | -- |
|  | -- | Supported Bearer (M) |
|  | -- | Target Identity (M) - Observed IMPU and/or - Observed IMPI |
| Conference Party Join | Case Identity (M) | Lawful Intercept Identifier (M) |
|  | IAP System Identity (M) | Network Identifier (M) |
|  | Time Stamp (M) | Event Date, Event Time (M) |
|  | Conference Id (M) | Correlation Number (M) |
|  |  | Conference URI and/or Temporary Conference URI (M) |
|  | Conference Parties (M) | Join Party Id (M) |
|  | -- | Initiator of Party Join Request (C) --- if difference from the party joined |
|  | CC Address (C) | -- |
|  | -- | Join Party Supported Bearer (M) |
|  | -- | Target Identity (M) - Observed IMPU and/or - Observed IMPI |
| Conference End | Case Identity (M) | Lawful Intercept Identifier (M) |
|  | IAP System Identity (M) | Network Identifier (M) |
|  | Time Stamp (M) | Event Date, Event Time (M) |
|  | Conference Id (M) | Correlation Number (M) |
|  |  | Conference URI and/or Temporary Conference URI (M) |
|  | -- | Initiator of Conference End Request (M) |
|  | -- | Conference End Reason (M) |
|  | -- | Target Identity (M) - Observed IMPU and/or - Observed IMPI |
| Conference Create | Case Identity (M) | Lawful Intercept Identifier (M) |
|  | IAP System Identity (M) | Network Identifier (M) |
|  | Time Stamp (M) | Event Date, Event Time (M) |
|  | Conference Id (M) | Correlation Number (M) |
|  |  | Conference URI and/or Temporary Conference URI (M) |
|  | Conference Requesting Party (M) | Identity(ies) of the Conference Controller (C) |
|  | List of Potential Conference parties (C) | List of potential conferees (C) |
|  | Conference Start Time (C) | Potential Conference Start Time (C) |
|  | Conference End Time (C) | Potential Conference StopTime (C) |
|  | Recurrence Information (C) | Recurrence Information (C) |
|  | -- | Target Identity (M) - Observed IMPU and/or - Observed IMPI |

Also while reporting the PTSC LAES ACS progress to the WTSC LI as a part of liaison report; RCMP indicated that in Canada, the interception of a conference can begin only if the intercept subject joins the conference call. If the Technical Report is meant to address North American region, perhaps, it is worth having a clarification. The WTSC LI discussion did not lost long as members felt such a discussion should not occur during the liaison reporting session.

# PROPOSAL

Discuss and determine whether any action has to be taken.