3GPP TSG-SA3 Meeting #66	s3i170235

# Ljubljana, Slovenia, 25th Jul 2017 - 28th Jul 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | 0257 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Push to Talk over Cellular Service addition of PTC feature, part B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2017-07-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of feature for Push to Talk over Cellular type services |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of informational elements and events to add further clarity for PTC type services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will not be able to meet regulatory requirements since LI requirments for PTC is not currently defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause addition Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Table X: Information Elements for PTC Event Records

A provisioned target identity can be a SIP URI , TEL URI , MCPTT ID or an IMEI.

A PTC Client MAY support multiple PTC Addresses and be involved in one or more PTC Sessions at the same time using the same or different PTC Addresses.

The following message structure are common parameters used for the PTC messages within this clause unless otherwise noted:

AbandonCause  - Identifies the reason for the abandoned PTC Session.

Access_Policy_Type – Identifies the type of access policy list being managed or queried by the PTC Intercept target.

Access_PolicyFailure - Reports the error code or reason for failure when Access Policy Request is unsuccessful.

Alert indicator - Indicates an emergency alert sent, received or cancelled.

AssociatePresenceStatus - Provides the Associate Presence Status, List of:

PresenceID: Identity of PTC Client(s) or PTC group, when known.

PresenceType: Identifies type of ID [PTC Client(s) or PTC group].

PresenceStatus: Presence state of each ID.

Broadcast indicator - Indicates that the group call request is for a broadcast group call.

Contact_Identity - Identity of the contact in the list, one contact per Contact List or Group List.

Implicit floor request - When originating client requests the floor.

Observed SIP URI  - The provisioned target identity can be a SIP URI

Observed TEL URI  - The provisioned target identity can be a TEL URI

Observed IMEI - The provisioned International Mobile Equipment Identity target identity.

MCPTT ID –  Mission Critical Push-To-Talk IDentity.

Event Type - Description of which type of event is delivered: PTC Session Initiation/Abandon, PTC session end, PTC Party Join/Drop/Hold/Retrieve, MCPPT emergency/imminent group/peril etc.

Event Time - Time of the event generation in the PTC Server or Client

Event Date - Date of the event generation in the PTC Server or Client.

Emergency indicator - Indicates the request is an MCPTT emergency call.

InitationCause - The network receives an invitation from the PoC Intercept Target to initiate a PTC Session.

Failure_Code – The reason or code for the failure or closing of the session.

FloorSpeakerIdentity - Identification of the PoC Client that has the Talk Burst.

IPAPartyIdentity - Instant Personal Alert a request for one PoC subscriber to initiate a PoC Session.

Invited_PoC_Client - A PoC Client that is invited to a PoC Session

Inviting_PoC_user – The PoC User who has been invited to a PoC Session.

Join_PoC_user – The PTC User who has joined the session, i.e., associate identity or targets.

Group_Ad_Sender – The group administrator who was the originator of the group call.

Group_Ad_Reveiver – The group administrator who was the receiver of the group call.

GroupAuthorizationRules - Identifies the action requested by the target to the PoC Group authorization rules.

Group_Characteristics –

Group_Identity - Identifies the PTC Group Identity, Nick Name, and characteristics.

Hold_retrieve_Indication - The PTC Session is put on hold (deactivate Media Bursts) or a new Primary PTC Session is activated or another PoC Session is locked for talking/listening.

Hold_Retrieve_user – The PTC Session is removed from hold by the PTC User and this indicates the PTC user identity.

Imminent peril indicator - Indicates that the PTC call is an imminent peril call.

Implicit floor request - When originating client requests the floor.

ListManagementType – Different PoC Group lists

ContactListManagementAttempt

GroupListManagementAttempt

ContactListManagementResult

GroupListManagementResult

Request unsuccessful

ListManagmentAction –Identifies the action requested by the PoC Intercept Subject to the Contact Lists or Group Lists.

List_ManagementFailure - Reports the error code or reason for failure.

Network Element Identifier - Unique identifier for the network element reporting the event.

Max_TB_Time - Include the maximum duration value for the talkburst before the permission is revoked, provide when known.

Party_Drop - Member of a PTC Group Session and leaves the PTC Session.

PTCCorrelationId - Uniquely identifies the SIP Session, correlates CII messages, and correlates CII and CC messages.

PTCSessionInfo - Provides PTC Session information such as PTC Session URI, PTC Session type, and Nickname.

SubjectID - The PTC identifier for the PTC Intercept Subject.

PTC group ID - The PTC group ID of the group on which the call is initiated.

PTCHost - Indentifies the PTC participant who has authority to initiate and administrate an active PTC Group Session. Provide when known.

PTC_ID_List - The list of PTC IDs of the PTC group members.

PTCOriginatingId - Identifies the originating party.

PTCParticipants - Identifies the invited PTC participants.

PTCUserAccessPolicy – Identifies the action requested by the PoC Intercept Subject to the PoC user access policy.

PreEstablishedSessionID –

PreEstablishedStatus –

Queued_FloorControl - Indicates if queuing is supported by the PoC Server and the intercept subject’s PoC Client.

Queued_Position_Status - If queued floor control is supported, indicates the queue position.

RTP_Setting –

Location - Report when a PTC Session is initiated by the intercept subject.This parameter is not reported when the PTC Intercept Subject  receives an invitation to join a PTC Session;  rather this information is reported by the PTC Session Start message (see PTC Session Start message for usage). Include when reporting of the PTC Intercept Subject’s location information is authorized and known.

MediaStream_Availability –

MCPTT ID - The Mission Critical Push To Talk identity.

MCPTT group ID - The Mission Critical Push To Talk group Identity.

MCPTT indicator – Indicates direction of the received request as either from the client or from the group to the client.

MCPTT Organization name – Name of the organization that the Mission Critical device belongs to.

TalkburstControl_Setting –

TargetIdentity - The PoC identifier for the PoC Intercept Subject.

TargetPresenceStatus - PoC-related presence information of the PoC intercept subject.

Talk_burst_priority - If more than one level of priority is supported, indicates the talk burst priority level of the PoC Client.

Talk_burst_reason_code - The reason code for the denial or revoke of a talk burst.

TBCP_Request - Used by the PoC Client to request permission from the PoC Server to send a Talk Burst.

TBCP_Granted - Used by the PoC Server to notify the PoC Client that it has been granted permission to send a Talk Burst.

TBCP_Deny - Used by the PoC Server to notify a PoC Client that it has been denied permission to send a Talk Burst.

TBCP_Idle - Used by the PoC Server to notify all PoC Clients that no one has the permission to send a Talk Burst at the moment and that it may accept the TBCP Talk Burst Request message.

TBCP_Taken - Used by the PoC Server to notify all PoC Clients, except the PoC Client that has been given permission to send a Talk Burst, that another PoC Client has been given permission to send a Talk Burst.

TBCP_Revoke - Used by the PoC Server to revoke the media resource from a PoC Client and can be used for preemption functionality, but is also used by the system to prevent overly long use of the media resource.

TBCP_Queued - Indicates the request to talk is queued, if queued floor control is supported.  Include identification of the PoC Client that has the queued Talk Burst, if known.

TBCP_Release - Indicates the request to talk has completed.

RegistrationRequest - Identifies the type of registration request (e.g., register, re-register, de-register).

RegistrationOutcome - Identifies success or failure of registration and the failure reason.

SDP – Answer, offer

PoCHost - Indentifies the PTC participant who has authority to initiate and administrate an active PTC Group Session.

PoCOriginatingId - Identifies the originating party. Provide when known.

PoCParticipants - Identifies the invited PTC participants, when known,  if other than the PTC Intercept Subject.

AssociatePresence - Provides the Associate Presence Status, List of:

PresenceID: Identity of PTC Client(s) or PTC group, when known.

PresenceType: Identifies type of ID [PTC Client(s) or PTC group].

PresenceStatus: Presence state of each ID.

Location - Report when a PTC Session is initiated by the intercept subject.This parameter is not reported when the PTC Intercept Subject  receives an invitation to join a PTC Session;  rather this information is reported by the PTC Session Start message (see PTC Session Start message for usage). Include when reporting of the PTC Intercept Subject’s location information is authorized and known,

InitiationCause - Identifies the reason for the generation of the PTC Session Initiated message (e.g., SIP INVITE).

BearerCapability - Identifies the bearer characteristics (e.g., vocoder type).

Z.4	 PTC Surveillance Events

Z.4.1        PTC Service Registration event

The PTC Service Registration event occurs when the target registers, re-registers, or deregisters for the PTC service, regardless of whether it is successful or unsuccessful.

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event Type

Event Time

Event Date

Network Element Identifier

Any other IMPU or IMPI of the target (if available)

RegistrationRequest

RegistrationOutcome

Z.4.2 	PTC Serving System event

A PTC Serving System event is generated when there is a change to the SP serving the PTC target access network (i.e., for mobility). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

Serving System Address (AVP name such as Visited-PLMN-Id)

Any other IMPU or IMPI of the target (if available)

Z.4.3 	PTC Session Initiation

## When a PTC Session Initiation event occurs is when the PTC target initiates any type of PTC Session or the target receives an invitation to join a PTC Session regardless of the success of the initiation or the final disposition of the invitation. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

SubjectID

PTCHost

PTCOriginatingID

PTCParticipants

AssociatePresenceStatus

Location

InitationCause

BearerCapability

Z.4.4 	PTC Session Abandon

The PTC Session Abandon event is triggered when the PTC Session is not established and the request is abandoned before the PTC Session established end to end connectivity, if known. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationID

PoCSessionInfo

SubjectID

PoCOriginatingId

PoCParticipants

AssociatePresenceStatus:

Location

AbandonCause

Z.4.5 	PTC Session Start event

This event occurs when a PTC Session (e.g., One-to-One, One-to-Many, or One-to-Many-to-One) is answered, and voice communication begins.  These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationID

PoCSessionInfo

SubjectID

PoCOriginatingId

PoCParticipants

AssociatePresenceStatus:

Location

BearerCapability

Z.4.6 	PTC Session End event

The PTC Session End event occurs when the PTC Session is released for any reason (i.e., normal or abnormal release) and voice communications ends, if known. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationID

PoCSessionInfo

SubjectID

PoCParticipants

Location

Cause

Z.4.7 	PTC Start of Interception

This event occurs when the PTC Intercept target’s ongoing PTC Session(s) that is already in progress (e.g., the PTC target is a member of a Pre-Arranged PTC Group or Chat PTC Group), regardless of whether the PTC target is actively transmitting or receiving talk bursts. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationID

PoCSessionInfo

SubjectID

PoCOriginatingID

PoCHost

PoCParticipants

BearerCapability

Correlation information

Z.4.8 	PTC Pre-Established Session

This event occurs when a SIP Session is established, modified, or released between the PTC targets, PTC Client and the PTC Server that performs the Participating PTC Function.  These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

SubjectIdentity

PoC_Server_URI

RTP_Setting

PoC_Media_Capability

PreEstablishSessionId

PreEstablishedStatus

TalkburstControl_Setting

MediaStream_Availability

Location

Failure_Code

Z.4.9 	PTC Instant Personal Alert

This event occurs when an Instant Personal Alert (i.e., a request for one PTC subscriber to initiate a PTC Session at a later time) is initiated or sent to the PTC target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

IPAPartyIdentity

Z.4.10.	10 PTC Party Join event

This event occurs when a PTC target joins (or re-joins) a PTC Group Session (e.g., Pre-arranged PTC Group, Ad-hoc PTC Chat Group) already in progress. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationId

PoCSessionInfo

TargetIdentity

Inviting_PoC_user

Join_PoC_user

AssociatePresenceStatus

BearerCapability

Z.4.11 	PTC Party Drop

This occurs when the target is a participating member of a PTC Group Session and leaves the PTC Session in which the PTC target is also participating. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationId

PoCSessionInfo

TargetIdentity

Party_Drop

AssociatePresenceStatus:

Z.4.12 	PTC Party Hold or Retrieve

If the PTC Client supports ‘on-hold’ procedures for PTC Sessions, a PTC Party Hold_Retrieve event occurs when the target places an ongoing PTC Session on hold or retrieves an ongoing PTC Session. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationId

PoCSessionInfo

TargetIdentity

Hold_ Retrieve_Indication

Hold_Retrieve_user

Z.4.13 	PTC Media Modification

During the PTC Session, a PTC Client may modify the voice frame packetization or voice codec mode by Out-of-band signalling using SDP payload within SIP messages. The Media_Modification event is generated when a re-negotiation of the media parameters occurs during a PTC Session involving the target UE. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PocCorrelationId

PoCSessionInfo

SubjectIdentity

BearerCapability

Z.4.14 	PTC Group Advertisement

This event is generated when a PTC Intercept Subject’s MS sends Group Advertisement information to a single PTC user, a list of PTC users or to all members of the Group using the Group Identity. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Group_Ad_Sender

Group_Ad_Receiver

PoCHost

Group_Characteristics

Z.4.15 	PTC Floor Control

Floor Control arbitrates requests from the PTC Clients for the right to send media (i.e., the right to speak).  Note, the term “Floor Control” is used to mean the same as term “Talk Burst Control”. When the PTC target is participating in a PTC Session, a Floor Control event is generated when the target requests to speak (e.g., presses the PTT mechanism) or the target is given permission to speak in response to a request (e.g., the network responds positively to the PTC Subscriber’s request) or is refused the request to speak and when the target is finished speaking (e.g., the PTC Intercept target releases the PTT mechanism). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

PocCorrelationId

PoCSessionInfo

FloorActivity Choice of: - TBCP_Request - TBCP_Granted - TBCP_Deny - TBCP_Idle - TBCP_Taken - TBCP_Revoke - TBCP_Queued - TBCP_Release

FloorSpeakerIdentity

Queued_FloorControl

Queued_Position_Status

Max_TB_Time

Talk_burst_priority

Talk_burst_reason_code

Z.4.16 	PTC Target Presence

If the Presence functionality is supported by the PTC Server and the PTC Server assumes the role of a Presence Source, this event is generated when the PTC Server publishes network presence information to the Presence server on behalf of PTC target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

TargetPresenceStatus

Z.4.17 	PTC Associate Presence

This event is generated when the PTC Server receives presence status notifications from the Presence Servers after having subscribed to the PTC presence status of other PTC Clients (i.e., Associates of the PTC Intercept targett). The default value of the Associate Presence event is ‘activate’. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

AssociatePresenceStatus:

Z.4.18	PTC List Management Events

This event is generated when the target PTC Client attempts to change their own contact list or their own PTC Group list(s).  This event is also generated when the network notifies the Intercept target’s PTC Client of changes made to their PTC-specific documents stored in the network (i.e., contact lists or PTC Group lists). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

SIP message header offer

SIP message header answer

SDP offer

SDP answer

TargetIdentity

ListManagementType Choice of: - ContactListManagementAttempt - GroupListManagementAttempt - ContactListManagementResult - GroupListManagementResult - Request unsuccessful

ListManagmentAction Choice of: - Create - Modify - Retrieve - Delete - Notify

Contact_Identity

Group_Identity

PoCHost

List_ManagementFailure

Z.4.19 	PTC Access Policy event

This event is generated when the PTC Intercept target attempts to change the access control lists (e.g. PTC user access policy and PTC Group authorization rules).  In addition this event is generated when the network responds to a modification or query by the PTC Intercept target to the access control lists (e.g., PTC user access policy and PTC Group authorization rules). These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Access_Policy_Type Choice of: - PoCUserAccessPolicyAttempt - GroupAuthorizationRulesAttempt - PoCUserAccessPolicyQuery - GroupAuthorizationRulesQuery - PoCUserAccessPolicyResult - GroupAuthorizationRulesResult - Request unsuccessful

PoCUserAccessPolicy Choice of: - Allow_Incoming_PoC_Session_request - Block_Incoming_PoC_Session_request - Allow_Auto_Answer_Mode - Allow_Override_Manual_Answer_Mode

GroupAuthorizationRules Choice of: - Allow_Initiating_Conference - Block_Initiating_Conference - Allow_Joining_Conference - Block_Joining_Conference - Allow_Add_Participants - Block_Add_Participants - Allow_Subscription_Conference_State - Block_Subscription_Conference_State - Allow_Anonymity - Forbid_Anonymity

Contact_Identity

Group_Identity

PoCHost

Access_PolicyFailure

Z.5	PTC Group Calls

These events occur when a PTC Session has a One-to-Many, or One-to-Many-to-One is requested, answered, joined, and voice communication begins, ends, or forced disconnected.

Z.5.1 	Group Call Request (Client to Server)

This event is generated when a PTC Group Call to join, rejoin, request or the release of the group call is generated from the targets PTC client to the PTC server is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

SDP Answer

Implicit floor request

Broadcast indicator

Z.5.2 	Group Call Request Event (Server to Server)

A Group Call Request event received at the PTC Server serving the target from a separate PTC server (outside the SP architecture) and the PTC Server serving the subject would generate a Group Call Request server to server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Broadcast indicator

Z.5.3 	Group Call Request (Server to Client)

This event is generated when a PTC Group Call request from the PTC server serving the target to the targets PTC Client is received. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Broadcast indicator

Z.5.4 	Group Call Response (Server to Client)

A Group Call Response event received at the PTC Server serving the target from a PTC server (outside the SP architecture) and the PTC Server serving the subject would generate a Group Call Response event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.5 	Group call response (Server to Server)

A Group Call Response event generated at the PTC Server serving the target to a separate PTC server (outside the SP architecture) would generate a Group Call Response server to server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.6 	Group call response (Client to Server)

This event is generated when a PTC Group Call response is generated at the targets PTC server to the targets PTC Client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.7 	Group Call Notify request (Server to Client)

This event is generated when a PTC Group Call Notify request is generated from the PTC server to the PTC client is generated. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.5.8 	Group call release request (Server – Client)

This event is generated when a PTC Group Call release request is generated at the PTC Server to the Client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.5.9 	Group call release request (Server – Server)

This event is generated when a PTC Group Call release request is received at the PTC Server serving the target from a separate PTC server (outside the SP architecture) would generate a Group Call Release Request server event. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.10 		Group call release response (Client – Server)

This event is generated when a PTC Group Call release response is generated at the targets PTC client to the targets PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.11 		Group call release response (Server – Server)

This event is generated when a PTC Group Call release response is generated at the targets PTC server to the any other PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.12 		Group Call Rejoin Request (Client – Server)

This event is generated when a PTC Group Call rejoin request is generated at the targets PTC client to the targets PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP offer

Z.5.13 	Group Call Rejoin Response (Server – Client)

This event is generated when a PTC Group Call rejoin response is generated at the targets PTC Server to the targets PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.14 	Group Join Request (Client – Server)

This event is generated when a PTC Group Call rejoin response is generated at the targets PTC Server to the targets PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

SDP offer

Z.5.15 	Group Join Response (Server – Client)
This event is generated when a PTC Group join response is generated from the PTC server to the PTC target Client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP answer

Z.5.15 	Group Call Leave Request (Server – Client)

This event is generated when the group call leave request from the PTC server to the PTC client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.16 	Group Call Leave Response (Client – Server)

This event is generated when the group call leave response is received at the PTC server from the PTC client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.17 Group Interrogate Request (Server – Server)

This event is generated when the group interrogate request is received at the PTC Server serving the Target. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

Z.5.18 Group Interrogate Response (Server – Server)

This event is generated when a PTC Group Interrogate response is generated at the targets PTC server to the any other PTC server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

Z.6 	MCPTT Priority Calls and Alerts Messages

MCPTT emergency group calls are defined by TS 23.379 [3] and TS 22.179 [2]. The user profile/group configuration determines which MCPTT Group ID is used, if any.  Group calls are enabled in both on-network and off-network. This subclause contains events for group call across a single and multiple MCPTT servers, and associated functions such as emergency call, broadcast call and others.

There are a number of states that are key in managing aspects of MCPTT emergency group calls, which include:

- MCPTT emergency state: indicates that the MCPTT user is in a life-threatening situation.

- In-progress emergency group state: indicates whether or not there is an MCPTT emergency group call ongoing on the specified group.

- MCPTT emergency group (MEG) state: this is an internal state managed by the MCPTT client which tracks the in-progress emergency state of the group and managed by the controlling MCPTT function.

- MCPTT emergency group call (MEGC) state: this is an internal state managed by the MCPTT client which in conjunction with the MCPTT emergency alert state aids in managing the MCPTT emergency state and related actions.

- MCPTT emergency alert (MEA) state: this is also an internal state of the MCPTT client which in conjunction with the MCPTT emergency group call state aids in managing the MCPTT emergency state and related actions.

Z.6.1 	MCPTT Emergency Group Call Request

This event occurs when a MCPTT Emergency Group Call Request is received at the MCPTT server from the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

Z.6.2 	MCPTT Emergency Group Call Response

This event is generated when received by a MCPTT Emergency Group Call Response from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

Z.6.3 	MCPTT Emergency Group Call Cancel Request

This event is generated when a MCPTT Emergency group Call Cancel event is generated when the cancel request from the MCPTT client to the MCPTT server is recieved. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT indicator

Z.6.4 	MCPTT Emergency Group Call Cancel Response

This event is generated when a MCPTT Emergency group Call Cancel response event is generated when the cancel response from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT indicator

Z.6.5 	MCPTT Emergency Alert Request

This event is generated when a MCPTT emergency alert request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Alert indicator

Organization name

Location

Z.6.6 	MCPTT Emergency Alert Response

This event is generated when a MCPTT emergency alert response is generated from the MCPTT server to the MCPTT client is generated. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Alert indicator

Organization name

Location

Z.6.7 	MCPTT Emergency State Cancel Request

This event is generated when the MCPTT Emergency State Cancel Request event is generated from the MCPTT client to the MCPTT server is generated. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel inform (NOTE)

Group emergency condition cancel request (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.8 	MCPTT Emergency State Cancel Response

This event is generated when the MCPTT Emergency State Cancel Response event is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel inform (NOTE)

Group emergency condition cancel request (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.9 	MCPTT Emergency State Cancel Response

This event is generated when the MCPTT Emergency State Cancel Response event is generated from the MCPTT Serve to the MCPTT Client. These elements will be delivered to the DF2 if available:

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Client emergency state cancel response (NOTE)

Group emergency state cancel response (NOTE)

NOTE: At least one of these information elements shall be present

Z.6.10 	MCPTT Imminent Peril Group Call Request

This event is generated when a MCPTT Imminent Peril Group Call Request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.11 	MCPTT Imminent Peril Group Call Response

This event is generated when a MCPTT Imminent Peril Group Call response event is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.11 	MCPTT Imminent Peril Group Call Cancel Request

This event is generated when a MCPTT Imminent Peril Group request is generated from the MCPTT client to the MCPTT server. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.6.12 	MCPTT Imminent Peril Group Call Cancel Response

This event is generated when a MCPTT Imminent Peril Group response is generated from the MCPTT server to the MCPTT client. These elements will be delivered to the DF2 if available:

Observed SIP URI

Observed TEL URI

Observed IMEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Imminent peril indicator

MCPTT indicator

Z.7	Communication Content (CC) Delivery

Z.7.1 	Communication Content (CC)

When communication content (CC) delivery is authorized, it will be delivered through the DF3 interface to provide packetized CC for the PTC Service under surveillance.  The PoC_CC-Application Protocol Data Unit (APDU) event is used to encapsulate communications content packets for transfer over the CC delivery interface, in accordance with this standard.  These elements will be delivered to the DF3 if available and authorized:

Observed SIP URI

Observed TEL URI

Observed IMEI

Event type

Event Time

Event Date

Network Element Identifier

PoCSessionInfo

PocCorrelationId

Payload

*** End of FIRST MODIFICATION ***

*** End of MODIFICATIONS ***