| 3GPP TSG-T WG2 / SWG1 (MEXE) meeting #6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | T2-99810 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kyongju, Korean, 04-06 Oct 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 23.057 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 1.5.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  | x | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | x | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Alcatel Business System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 05/10/99 |  |  |  |  |  |
| Subject: |  |  | MExE Profile of PKCS#15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | MExE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | MExE select PKCS#15 standard to store certificates in SIM. MExE profile of PKCS#15 shall be defined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 2.1 : addition of new references Creation of annexe A normative Creation of annexe B informative |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  | x |  List of CRs: |  |  |  |  |  |  | CR on USIM specification 31.102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  | x |  List of CRs: |  |  |  |  |  |  | CR on SIM specification 11.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | This CR need addition of domain attribute in PKCS#15 specification provided by RSA, but a fall back solution is available. This CR should be sent to WAP forum (WIM and PKI) for feedback to avoid incompatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

## 2.1	Normative references

[1]	GSM 01.04: “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java 1.1.1, Sun Microsystems http://java.sun.com/products/personaljava/spec-1-1-1/index.html.

[4]	JavaPhone API version 0.9, http://java.sun.com/products/javaphone/.

[5]	JTAPI 1.2, Sun Microsystems http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.1 http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, http://www.java.sun.com

[11]	UMTS TR 22.170: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.121: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.101: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

[15]	CC/PP Exchange Protocol based on HTTP Extension Framework; W3C http://www.w3.org/TR/NOTE-CCPPexchange

[16]	Composite Capability/Preference Profiles (CC/PP):A user side framework for content negotiation;  Available at W3C web pages.

[17]	UAProf  Specification http://www.wapforum.org

[18]	JDK 1.1 security http://www.javasoft.com/products/jdk/1.1/docs/guide/security/index.html

[19]	Java 2 security http://www.javasoft.com/products/jdk/1.2/docs/guide/security/index.html

[20]	Java security tutorial http://java.sun.com/docs/books/tutorial/security1.2/overview/index.html

[21]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium http://www.opencard.org

[22]	RFC 1738 Uniform Resource Locators (URL) http://www.w3.org/pub/WWW/Addressing/rfc1738.txt

[23]	The MD5 Message Digest Algorithm", Rivest, R., RFC 1321, April 1992.  URL: ftp://ftp.isi.edu/in-notes/rfc1321.txt

[24]	ISO/IEC 10118-3 1996, Information technology - Security techniques - Hash-functions - Part 3: Dedicated hash-functions

[25]	IETF RFC 2368: The mailto URL scheme

[26]	ITU-T X.509: Information technology – Open Systems Interconnection – The Directory: Authentication framework

[27]	GSM 11.11: "Digital cellular telecommunications system (Phase 2+); Specification of the Subscriber Identity Module – Mobile Equipment (SIM-ME) interface.

[28]	PKCS #15 “Cryptographic Token Information Standard” version 1.0, RSA Laboratories, April 1999
URL: ftp://ftp.rsa.com/pub/pkcs/pkcs-15/pkcs15v1.doc

[29]	RFC 2510 Internet X.509 Public Key Infrastructure January 1999

# Annexe A		MExE profile of PKCS#15 (Normative)

## A.1	PKCS#15 certificate object attributes presentation

### A.1.1	Object common attributes :

Label			human readable label to describe the certificate

Flags			indicates whether object is  private (CHVi authentication request), whether object is read only.

Authentication object identifier		a cross-reference back to the authentication object, which describes the properties of a CHVi, used to protect this object

### A.1.2	Certificate common attributes :

identifier  		identifier is used for correlation between public key containing in certificate and associated private key

Authority		indicates whether the certificate is for an authority (i.e. CA or AA) or not.

Request identifier 	Used to search a certificate : Issuer and serial number SHA-1 hash, or issuer public key SHA-1 hash, or public key subject SHA-1 hash

Thumbprint 		used as secure way to validate a root certificate : hash on to be signed certificate (internet)

### A.1.2	Certificate attributes

Type of certificate	indicates the type of certificate : WTLS, X509, SPKI, PGP, X9.68

Value			direct value or indirect file path or URL.

### A.1.3	Specific X.509 Certificate attributes

For information See PKCS#15 (reference 28)

## A.2	MExE profile of PKCS#15

PKCS15CommonObjectAttributes.label must be present. The value content is unspecified.

PKCS15CommonObjectAttributes.Flag must be present. The value shall be private, not modifiable by ME

PKCS15CommonObjectAttributes.Authentication must be present. The value shall be “CHV1”

PKCS15CommonCertificateAttributes.Id must be present. The value content is unspecified

PKCS15CommonCertificateAttributes.Authority must be present if and only if certificate is a CA certificate. The value is true.

PKCS15CommonCertificateAttributes.RequestId must be present if and only if certificate is a operator or third party root certificate. The value shall be the same as these use in issuer/authority key identifier field of  the certificates provided by this issuer (as in  RFC2459 document [29]).

PKCS15CommonCertificateAttributes.Thumbprint must be present if and only if certificate is a third party root certificate. The value shall be the same as these use in CCM.

PKCS15(type)CertificateAttributes.value must be present  Value is a indirect file path (path, index, offset). Index and offset default value is 0.

Specific X509 attributes are not supported :

PKCS15X509CertificateAttributes.subject must not be present.

PKCS15X509CertificateAttributes.issuer must not be present.

PKCS15X509CertificateAttributes.serialNumber must not be present.

## A.3	Coding and storage in SIM

See detail of file hierarchy and file properties in SIM document [27].

See abstract syntax definition and coding detail in PKCS#15 document [28].

Address of certificate descriptor Elementary File is fixed.

According to PKCS#15 [28 : sub-clause 7.6 The PKCS15Certificates type] , the contents of an certificate descriptor Elementary File  must be the value of the DER encoding of a SEQUENCE OF PKCS15Certificate (i.e. excluding the outermost tag and length bytes).

Address of certificate data Elementary File is free.

According to PKCS#15 [28 : sub-clause 7.6.1 to 7.6.6], the certificate data  value is coded according to the related certificate type (e.g. DER for X5.09, base64 for SPKI and PGP, WTLS network format for WTLS, DER or PER for X9.68).

# Annexe B		PKCS#15 certificate objects ASN1 expanded syntax extract (Informative)

{  -- sequence of certificate

x509Certificate,[0] x509AttributeCertificate,[1] spkiCertificate, [2] pgpCertificate, [3] wtlsCertificate,[4] x9-68Certificate : {

commonObjectAttributes {

label  "" UTF8 string OPTIONAL,

flags  {private (0), modificable (1)} bit string OPTIONAL, --

authId  octet string OPTIONAL, --

},

CommonCertificateAttributes {

iD      octet string,

authority boolean default not an authority,

requestId {

idtype	integer

IdValue	octet string

pkcs15IssuerAndSerialNumber PKCS15KEY-IDENTIFIER::=

{SYNTAX PKCS15-OPAQUE.&Type IDENTIFIED BY 1}

-- As defined in RFC [CMS]

pkcs15SubjectKeyIdentifier PKCS15KEY-IDENTIFIER ::=

{SYNTAX OCTET STRING IDENTIFIED BY 2}

-- From x509v3 certificate extension

pkcs15IssuerAndSerialNumberHash PKCS15KEY-IDENTIFIER ::=

{SYNTAX OCTET STRING IDENTIFIED BY 3}

-- Assumes SHA-1 hash of DER encoding of IssuerAndSerialNumber

pkcs15SubjectKeyHash PKCS15KEY-IDENTIFIER ::=

{SYNTAX OCTET STRING IDENTIFIED BY 4}

-- Hash method defined in Section 7.

pkcs15IssuerKeyHash PKCS15KEY-IDENTIFIER ::=

{SYNTAX OCTET STRING IDENTIFIED BY 5}

-- Hash method defined in Section 7.

} OPTIONAL,

thumbprint 	[0] OOBCertHash OPTIONAL, -- hash on to be signed certificate, used for secure certificate identification as CCM using

},

[1] typeAttributes {

value indirect : path : {

path  		octet string, -- '4331'H Reference by file identifier

index		integer OPTIONAL, -- ‘XXXX’H offset in file

[0] length 	integer OPTIONAL, -- ‘XXXX’H length in file

}

-- other optional attributes are defined for X509 certificate

}

},

}