3GPP Conference Call on 3GPP Spec Modernization #4	6GSM- 250307

Electronic, November 10th, 2025

Source:	Interdigital, Inc.

Title:	pCR on Potential New Formats for 3GPP Specification

Agenda item:	4

Document for:	Approval

Spec:	TR21.802

Version:	v0.1.2

Work Item:	FS_6GSpecs

# Introduction

In June SA #108 and RAN #108, new study on modernization of specification format and procedures for 6G was approved. The ISD Objective 2 focuses on investigation of potential new formats for 3GPP specifications. The following is a copy of objective 2 of the SID.

Objective 2)

Taking into account the outcome of objective 1, consider, with detailed feasibility analysis, potential new format(s) for 3GPP specifications, including, but not necessarily limited to, plain-text format(s) such as Markdown or LaTeX, or other format(s) such as OpenDocument, including:

- Identify the kinds of content and styles that need to be represented in the specifications, taking into account all TSGs and WGs (e.g. text, headings, equations, tables, figures of various kinds, etc).

- Study and identify how the identified content may be represented in the potential new format(s).

- Study and identify how the potential new format(s) may be automatically and accurately converted to and from other formats (e.g. pdf, html, docx).

- Study and identify how new navigational features may be achieved in the potential new formats, including, for example, cross-linking between specifications and collapsing of multi-level procedure text.

- Study suitable tools for reading, editing and reviewing specifications in the potential new formats.

- Identify the pros and cons of the potential new format(s) relative to each other and relative to the existing format (docx).

In this contribution, we provide pCR on LaTeX as a potential format for 3GPP specification.

# Proposed Changes

* * * Start of Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 21.801: "Specification Drafting rules".

[3]	Report on User Requirements for a Next Generation Specification Development Tool, NWM Project Oversight Committee, 08.12.22. https://www.3gpp.org/ftp/Email_Discussions/3GPP/221208-Report-Requirements-for-NG-Spec-Tool.zip (accessed 07.07.2025)

[4]	LaTeX project. https://www.latex-project.org/

* * * Unchanged Text Omitted * * * *

## 5.X	Proposal #X: LaTeX

### 5.X.1	Description

#### 5.X.1.1	Introduction of LaTeX

LaTeX is high-quality typesetting system, intended for production of technical and scientific documentation [4]. LaTeX has a stable specification and uses widely in academic publications for high-quality literature prints. As the file format and typeset-ting system is intended for technical and scientific documentation, it supports all formatting that Microsoft Word can produce and even more. While LaTeX was not designed to be a WYSIWYG typesetting system, there are tools available that allow users to get “what you see is what you mean” (WYSIWYM) what is often called “near WYSWYG experience”. A short list of such tools is LyX, TeXmacs, Overleaf, Scientific Word, and TeXstudio. As LaTex is a mature file format, there are numerous editors that aid document processing. Comparison of various TeX editors are available in https://en.wikipedia.org/wiki/Comparison_of_TeX_editors.

As LaTeX is targeted for technical documentation, it might be difficult to fully list of formatting and features that it can support. The Latest LaTeX specification is LaTeX2e, which was created in 1994. There was some effort to create a new LaTeX system called LaTeX3. However, this was determined non-feasible to due backwards compatibility issues. Features intended for LaTeX3 has slowly been updated to LaTeX2e. As LaTeX2e will continue to support full backwards compatibility, any new feature additions to the typesetting should not impact future 3GPP specifications if LaTeX is leveraged today.

What may be more challenging for adoption of LaTeX is actually the number of library packages that provide additional functionality to LaTeX. As LaTeX is extremely mature, the number of support libraries are also numerous. Many libraries provide similar functionality. In fact, they are too numerous to a point where MCC may want to limit the usages of libraries for 3GPP specification to make sure a consistent experience for all users working with 3GPP specification.

#### 5.X.1.2	Benefits of LaTeX

The following are some formatting features that are distinct from Microsoft Word which may be of interest for 3GPP specifications.

-	Separation of typesetting configuration & actual literature content

-	LaTeX is consists of two aspects. Typesetting configuration and literature content. It is possible to separate the two. The default formatting rules for a document could be programmed to follow specific rules needed and can be stored in a separate file with cls extension. These default formatting rules can be determined by MCC, and all users (editors, delegates, etc.) will not need to deal with the default formatting rules.

-	Separation of large document into separate files & insertion of multiple files into publication document

-	LaTeX allows insertion of technical content that is stored in separate files, but all belong to the same publication. This allows different Clauses that may have large content to be separated into different files and handled separately.

-	Re-use of same sub-content file in different specifications

-	Since it is possible to store a small part of the specification and insert the files into a publication, it also allows same file to be re-used in different specifications without any duplication. If different 3GPP specification must leverage the same technical content (say a table), it may be possible to use the same source file for multiple specifications and complete avoid any mis-synchronization issues between different specifications.

-	Changing page size or orientation of select pages of the specification

-	Similar to Microsoft Word, it is possible to change the page size or even change the orientation (either horizontal or vertical orientation) of the text. This supports to fit large tables in a single TS/TR document.

-	Integration of math symbols and text

-	LaTeX excels in rendering beautifully typed mathematical equations as part of the text.

-	Complex tables

-	LaTeX allows support for complex tables that have merged cells, vertical texts, etc. However, it should be noted that source code representation for tables is not user friendly. The rendered output representation may look clean and pretty, but the actual source code may look complex.

#### 5.X.1.3	Examples of LaTeX drafting

In this section, some examples of how LaTeX is drafted and rendered is provided. Additionally, comparison with another popular markup language, Markdown, is provided to provide context of LaTeX as a markup language and to show case how LaTeX handles different use cases of 3GPP specification drafting. The example shows the rendering output based on provided example the source. Four 4 different uses cases have been selected as examples.

-	Case 1: handling general text (including bullet lists)

-	Case 2: handling tables & mathematical equations

-	Case 3: handling of ASN.1 code snippets

-	Case 4: handling of figures

For LaTeX, some front matter in the beginning of the document is needed to correctly render the text, lists, tables, figures, and code. Once completed, most users should not need to change the front matter and could be potentially handled by MCC.

It should be noted that when converting Markdown directly into PDF using pandoc, pandoc internally converts the Markdown into LaTeX and uses PDFlatex to generate the PDF.

Overall, LaTeX provides excellent control of text, tables, and figures at the expense of some of extra code.

Table 5.X.1.3-1: Use case 1 - handling general text (including bullet list)

| Markdown | LaTeX |
| --- | --- |
| No Setup Needed: Source File: ## 4.4.4.2 Point A Point A serves as a common reference point for resource block grids and is obtained from: - for a PCell downlink where *offsetToPointA* represents the frequency offset between point A and the lowest subcarrier of the lowest resource block, which overlaps with the SS/PBCH block, or the SS/PBCH block after puncturing if applicable, used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 and FR2-NTN; - for operation without shared spectrum channel access in FR1, FR2-1 and FR2-NTN, the lowest resource block has the subcarrier spacing provided by the higher layer parameter *subCarrierSpacingCommon*; - for operation with shared spectrum channel access in FR1 or FR2, and for operation without shared spectrum channel access in FR2-2, the lowest resource block has the subcarrier spacing same as the SS/PBCH block used by the UE for initial cell selection; - *absoluteFrequencyPointA* for all other cases where *absoluteFrequencyPointA* represents the frequency-location of point A expressed as in ARFCN. | Front Matter for LaTeX document (Setup): \documentclass{article} % Packages for formatting \usepackage{amsmath, amssymb} \usepackage{geometry} \geometry{margin=1in} \renewcommand{\labelitemi}{--} % global dash style \begin{document} Source File: \subsubsection*{4.4.4.2 Point A} Point A serves as a common reference point for resource block grids and is obtained from: \begin{itemize} \item for a PCell downlink where \emph{offsetToPointA} represents the frequency offset between point A and the lowest subcarrier of the lowest resource block, which overlaps with the SS/PBCH block, or the SS/PBCH block after puncturing if applicable, used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 and FR2-NTN; \item for operation without shared spectrum channel access in FR1, FR2-1 and FR2-NTN, the lowest resource block has the subcarrier spacing provided by the higher layer parameter \emph{subCarrierSpacingCommon}; \item for operation with shared spectrum channel access in FR1 or FR2, and for operation without shared spectrum channel access in FR2-2, the lowest resource block has the subcarrier spacing same as the SS/PBCH block used by the UE for initial cell selection; \item \emph{absoluteFrequencyPointA} for all other cases where \emph{absoluteFrequencyPointA} represents the frequency-location of point A expressed as in ARFCN. \end{itemize} End Matter for LaTeX document: \end{document} |
| Rendered output (Pandoc MD to PDF conversion): | Rendered output: |

Table 5.X.1.3-2: Use case 2 - handling tables & mathematic equations

| Markdown | LaTeX |
| --- | --- |
| No Setup Needed: Source File: ## 4 Frame structure and physical resources ### 4.1 General Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units $T_{c} = 1/\left(\Delta f_{\text{max}} \right)$ where $\Delta f_{\text{max}} = 480 \cdot 10^{3}$ Hz and $N_{f} = 4096$. The constant $\kappa = T_{s}/T_{c} = 64$ where $T_{s} = 1/\left( \Delta f_{\text{ref}} \cdot N_{\text{f,ref}} \right)$, $\Delta f_{\text{ref}} = 15 \cdot 10^{3}\text{Hz}$ and $N_{\text{f,ref}} = 2048$. Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4, 5, 6, or 7 are equally applicable to the IAB-MT part of an IAB-node and the NCR-MT part of an NCR node. ### 4.2 Numerologies Multiple OFDM numerologies are supported as given by Table 4.2-1 where $\mu$ and the cyclic prefix for a downlink or uplink bandwidth part are obtained from the higher-layer parameters *subcarrierSpacing* and *cyclicPrefix*, respectively. <div align="center"> Table 4.2-1: Supported transmission numerologies. ¦ $\mu$ ¦ $\Delta f =2^{\mu}\cdot 15$ [kHz] ¦ Cyclic prefix ¦ ¦ :---: ¦ :---: ¦ :---: ¦ ¦ 0 ¦ 15 ¦ Normal ¦ ¦ 1 ¦ 30 ¦ Normal ¦ ¦ 2 ¦ 60 ¦ Normal, Extended ¦ ¦ 3 ¦ 120 ¦ Normal ¦ ¦ 4 ¦ 240 ¦ Normal ¦ ¦ 5 ¦ 480 ¦ Normal ¦ ¦ 6 ¦ 960 ¦ Normal ¦ </div> | Front Matter for LaTeX document (Setup): \documentclass{article} % Packages for formatting \usepackage{amsmath, amssymb} \usepackage{caption} \usepackage{geometry} \geometry{margin=1in} \setlength{\parindent}{0pt} \begin{document} Source File: \section*{4 Frame structure and physical resources} \subsection*{4.1 General} Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units $T_{c} = 1/\left(\Delta f_{\text{max}} \right)$ where $\Delta f_{\text{max}} = 480 \cdot 10^{3}$ Hz and $N_{f} = 4096$. The constant $\kappa = T_{s}/T_{c} = 64$ where $T_{s} = 1/\left( \Delta f_{\text{ref}} \cdot N_{\text{f,ref}} \right)$, $\Delta f_{\text{ref}} = 15 \cdot 10^{3}\text{Hz}$ and $N_{\text{f,ref}} = 2048$. Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4, 5, 6, or 7 are equally applicable to the IAB-MT part of an IAB-node and the NCR-MT part of an NCR node. \subsection*{4.2 Numerologies} Multiple OFDM numerologies are supported as given by Table 4.2-1 where $\mu$ and the cyclic prefix for a downlink or uplink bandwidth part are obtained from the higher-layer parameters \emph{subcarrierSpacing} and \emph{cyclicPrefix}, respectively. \begin{table}[h] \centering \caption*{Table 4.2-1: Supported transmission numerologies.} \begin{tabular}{¦c¦c¦c¦} \hline $\mu$ & $\Delta f =2^{\mu}\cdot 15$ [kHz] & Cyclic prefix \\ \hline 0 & 15 & Normal \\ 1 & 30 & Normal \\ 2 & 60 & Normal, Extended \\ 3 & 120 & Normal \\ 4 & 240 & Normal \\ 5 & 480 & Normal \\ 6 & 960 & Normal \\ \hline \end{tabular} \end{table} End Matter for LaTeX document: \end{document} |
| Rendered output (Pandoc MD to PDF conversion): | Rendered output: |
| Notes of drafting: Markdown does not natively support tables and math equations. The source code is pandoc markdown variant method for tables and equations. Markdown does not natively support centered text. Therefore, html block insertion is needed to center text using <div align=“center”> and </div> commands. It should be noted that pandoc does not seem to parse all the html blocks correctly as the Table caption does not center align properly even if html code. |  |

Table 5.X.1.3-3: Use case 3 - handling ASN.1 code

| Markdown | LaTeX |
| --- | --- |
| No Setup Needed: Source File: ### ***– FrequencyInfoDL-SIB*** The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon. <div align="center"> **FrequencyInfoDL-SIB information element** </div> ```asn1 -- ASN1START -- TAG-FREQUENCYINFODL-SIB-START FrequencyInfoDL-SIB ::= SEQUENCE { frequencyBandList MultiFrequencyBandListNR-SIB, offsetToPointA INTEGER (0..2199), scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier } ``` | Front Matter for LaTeX document (Setup): \documentclass{article} % Packages for formatting \usepackage{xcolor} \usepackage{listings} \usepackage{geometry} \geometry{margin=1in} % Color palette \definecolor{asnKeyword}{HTML}{8B0000} % maroon \definecolor{asnComment}{HTML}{6A737D} % gray \definecolor{asnString}{HTML}{032F62} % dark blue \definecolor{asnBg}{HTML}{F6F8FA} % light gray background % Style for code blocks \lstdefinelanguage{ASN1}{ morekeywords={SEQUENCE,CHOICE,INTEGER,SIZE,OF}, sensitive=true, morecomment=[l]{--}, % ASN.1 line comments morestring=[b]" } \lstdefinestyle{asnStyle}{ language=ASN1, basicstyle=\ttfamily\small, keywordstyle=\color{asnKeyword}\bfseries, commentstyle=\color{asnComment}\itshape, stringstyle=\color{asnString}, backgroundcolor=\color{asnBg}, numbers=left, numberstyle=\scriptsize\color{asnComment}, stepnumber=1, numbersep=10pt, showstringspaces=false, tabsize=2, frame=single, rulecolor=\color{asnComment}, breaklines=true, breakatwhitespace=true, keepspaces=true } \begin{document} Source File: {\large \textbf{\emph{– FrequencyInfoDL-SIB}}} The IE FrequencyInfoDL-SIB provides basic parameters of a downlink carrier and transmission thereon. \begin{center} \textbf{FrequencyInfoDL-SIB information element} \end{center} \begin{lstlisting}[style=asnStyle, language=ASN1] -- ASN1START -- TAG-FREQUENCYINFODL-SIB-START FrequencyInfoDL-SIB ::= SEQUENCE { frequencyBandList MultiFrequencyBandListNR-SIB, offsetToPointA INTEGER (0..2199), scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier } \end{lstlisting} End Matter for LaTeX document: \end{document} |
| Rendered output (Pandoc MD to PDF conversion): | Rendered output: |
| Notes of drafting: Markdown does not natively support ASN.1 code highlighting. The ASN.1 code highlights for Pandoc based Markdown rendering are enabled by Haskell library called “skylighting” (https://github.com/jgm/skylighting) and any changes to this library will result in different render output for Pandoc. For different Markdown variants, the code highlighting is enabled by different libraries and could result in different output. |  |

Table 5.X.1.3-4: Use case 4 - handling of figures

| Markdown | LaTeX |
| --- | --- |
| No Setup Needed: Source File: ## 4.3.1 Frames and subframes Downlink, uplink, and sidelink transmissions are organized into frames with $T_{f} = \left( \Delta f_{\text{max}} N_f / 100 \right) \cdot T_c$ ms duration, each consisting of ten subframes of $T_{sf} = \left( \Delta f_{\text{max}} N_f / 10000 \right) \cdot T_c = 1$ ms duration. The number of consecutive OFDM symbols per subframe is $N_{\text{symb}}^{\text{subframe},\mu} = N_{\text{symb}}^{\text{slot}} N_{\text{slot}}^{\text{subframe},\mu}$. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 - 4 and half-frame 1 consisting of subframes 5 - 9. <div align="center"> ![Uplink-downlink timing relation](media/figure4-3-1-1.png) **Figure 4.3.1-1 Uplink-downlink timing relation** </div> | Front Matter for LaTeX document (Setup): \documentclass{article} % Packages for formatting \usepackage{amsmath, amssymb} \usepackage{caption} \usepackage{graphicx} \usepackage{geometry} \geometry{margin=1in} \begin{document} Source File: \subsubsection*{4.3.1 Frames and subframes} Downlink, uplink, and sidelink transmissions are organized into frames with $T_{f} = \left( \Delta f_{\text{max}} N_f / 100 \right) \cdot T_c$ ms duration, each consisting of ten subframes of $T_{sf} = \left( \Delta f_{\text{max}} N_f / 10000 \right) \cdot T_c = 1$ ms duration. The number of consecutive OFDM symbols per subframe is $N_{\text{symb}}^{\text{subframe},\mu} = N_{\text{symb}}^{\text{slot}} N_{\text{slot}}^{\text{subframe},\mu}$. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 -- 4 and half-frame 1 consisting of subframes 5 -- 9. \begin{figure}[h] \centering \includegraphics[width=0.5\textwidth]{media/figure4-3-1-1.png} \caption*{Uplink-downlink timing relation} \end{figure} End Matter for LaTeX document: \end{document} |
| Rendered output (Pandoc MD to PDF conversion): | Rendered output: |
| Notes of drafting: Markdown does not natively support scaling of the image being inserted. For advanced image scaling, html block codes need to be leveraged. Image file formats supported by Markdown depends on Markdown variant. In case of Pandoc Markdown, PNG, JPEG/JPG, GIF, and SVG are supported. | Notes of drafting: LaTeX does not support any image format. The supported image formats are EPS, PDF, PNG, JPEG/JPG, and JBIG2. |

### 5.X.2	Evaluation against requirements of section 4.3

Table 5.X.2-1: Requirements of Clause 4.3

| # | Requirement | Description | Evaluation of LaTeX |
| --- | --- | --- | --- |
| 1 | Global availability | There shall be no geographic limitations on availability and usability of tools and formats. | Yes |
| 2 | Cost and licensing | Tools shall be available at reasonable cost. Ideally, the use of open source and free tools (under reasonable licensing terms) should be prioritized, if possible. | LaTeX is free and open sourced |
| 3 | Security and Availability | Tools shall be secure, i.e. robust to manipulation of source code and shall be systematically available (i.e. usable) to avoid 3GPP coming to a halt. | Yes |
| 4 | Cross platform | It shall be possible to view and edit the specifications, including figures, on all major computer platforms (not necessarily including mobile platforms). Ideally, online variants of these tools should be available (a.k.a. web-GUIs). | Yes |
| 5 | Simplicity and learning curve | Tools shall be reasonably easy to use and should not cause significant change of the existing working procedures and/or drafting rules. | Drafting of the front matter to set up thetypesettingg requires some domain knowledge on LaTeX, which may be handled by MCC. Actual content drafting is relatively simple. |
| 6 | Limited number of tools and formats. | Specifications shall only require a limited number of approved formats and recommended tools. | Possible. As LaTeX is mature there are a number of tools available. It should be possible to down-select reference tools for drafting and rendering of the specification. |
| 7 | One reference | It should be possible to produce representations of the specifications in various formats suitable for different situations. However, at the same time there shall be only one reference. The representation (i.e. reference output) should have consistent appearance to specifications of past generations. | Yes |
| 8 | Fast opening, navigating and editing | Specifications shall be fast (no longer than 30 seconds on a reasonably modern computer even for large and complex specifications) to open. Furthermore, there shall be no noticeable lag when searching, navigating or editing the specifications. The tool shall be stable (no crashes) and shall not require splitting large documents into multiple files. | Yes |
| 9 | CR implementation | It shall be possible to quickly and easily (ideally automatically) implement a CR. This applies to both MCC and delegates (e.g. CR/TR rapporteurs). A tool for automation of CR implementation should allow indicating the exact conflicts and allow specification rapporteurs to review and address such conflicts manually. Furthermore, it should be possible to automatically detect CR conflicts (e.g. detect if multiple CRs propose changes to text in the same clauses, or the same text, or if a definition is changed for a term that is used in other CRs, etc). In all cases, human verification shall be possible before final approval of the merges of conflict resolutions. | N/A Depends on the tool paired for CR implementation handling. LaTeX as a type-setting language is purely text-based source code, which can be used by automation tools to process the source and manipulate it as necessary. |
| 10 | CR Traceability | Specifications format and tools shall support an easy way for implementors to track introduction of requirements, new features and corrections to the existing ones. The specification creation process shall allow recording all changes to the specification, including errors corrected during CR implementation. Tools used for handling CRs (submission, revisions, agreement, rejection, etc) should allow that all decisions be traceable along with the reasons for decision. | N/A LaTeX is a type-setting language and not a tool for creating change marks. For change mark representation, different sets of tools that can use LaTeX and represent change marks would be needed. It should be possible to create CRs based on LaTeX using various open-sourced tools. |
| 11 | Feature Traceability | The tools shall allow to document requirements, features and capabilities in a format that can be automatically parsed. It should be possible (if feasible) to associate a requirement or a feature with the relevant text in all specifications which are related to the feature. | Yes |
| 12 | Cross-referencing and hyperlinking | It shall be possible to cross reference multiple specifications including (sub-)clauses. Such cross referencing shall be easy to navigate e.g. using hyperlinks. A specification shall be able to cross-reference another specification so that the referenced specification text is unambiguous. (NOTE 1) Tools shall support hyperlinks, including: - Internal (within the document) - External (to other documents) - Ideally, to specific clauses and sections within a document - Ideally, using relative path (so that the link works anywhere provided the folder structure is maintained) | Yes |
| 13 | Tracking of Editor’s notes | It shall be possible to automatically track Editor’s Notes (EN) addition/removal/resolution (including the origin of the comment such as work item or contributing company), if ENs are used as an issue raising/resolving mechanism. | Partially. LaTeX as a type-setting language is purely text-based source code, which can be used by automation tools to process the source and manipulate it as necessary. |
| 14 | References to discussion papers | It shall be possible to reference discussion papers (for background information) in CR cover page (or its equivalent in the new tool/format). | Yes |
| 15 | Automation | Specification format shall enable automation, e.g. to: - Ensure consistent use of styles - Extract ASN.1 (if in the new format ASN.1 isn’t stored separately) - Extract table content for verification, usage and presentation. - Extract text for verification, usage, and presentation. - Generate content from other structured data e.g. schemas or JSON data. - Automated alignment of bands/band configurations between RAN4 specs(e.g. TS 38.101-1/-2/-3) and RAN5 documents (e.g., Excel in PRD21) - Automated alignment of the completion status of test cases between RAN5 test cases specs (e.g. TS 38.521-1/-2/-3/-4/-5, TS 38.533) and applicability specs (e.g. TS 38.522) | Yes |
| 16 | Specification browsing | It shall be possible to search and navigate within and across specs via both online and offline versions. It shall be possible to select the exact release and version of a TR or TS to access. | Yes |
| 17 | CR drafting during and in between meetings | It shall be possible to handle the process of drafting a CR involving a large number of contributors who provide a large number of comments and propose a large number of changes. It shall be possible to trace changes and comments to the individual contributors. CR handling needs to work for normal and electronic meetings, where either only one author can provide revisions or multiple authors from different companies can provide revisions and add comments during the meeting. For the latter scenario, tools shall also support the handling of concurrent revisions by different authors of the same document in order to prevent inconsistent revisions. (NOTE 7) Tools need to also work before and after meetings for collaborative work between source companies preparing input documents. (NOTE 2) | N/A. LaTeX is a type-setting language and not a tool to trace changes and add comments. Other tools (editors) may need to leverage to aid with CR drafting during and in between meetings. |
| 18 | Collaboration on CR drafting between companies | Sharing of draft CRs within and between the source companies for preparing input documents shall be possible. | N/A. LaTeX is a type-setting language and not a tool for creating collaborative settings. For collaboration, different set of tools that can use LaTeX and represent change marks would be needed. One example is Overleaf. |
| 19 | Consistent access to CRs and specifications | The tools should allow 3GPP delegates to access CRs and specifications in a similar way as today during pre-meeting, post-meeting and online meeting. | Yes. |
| 20 | Visual representation of changes | Tools shall allow visual representation of the changes in a CR and between the original spec and the revision. The author, i.e., at least the source company, of proposed changes should be visible. The effects or changes imposed by a CR should be easily readable and navigable such that all the changes can be viewed in the context of the clause affected. | Partially. LaTeX is a type-setting language and not a tool for creating change marks. For change mark representation, different set of tools that can use LaTeX and represent change marks would be needed. |
| 21 | Supported styles | Tools shall support the following styles: - Headings - Heading 1, Heading 2, up to Heading 6 - Hyperlinks - Internal and external - Lists (bulleted) - Lists (hierarchical) - Lists (numbered) (NOTE 3) - Tables (simple cell contents; allows different justification {left, center, right}) - Table (column header contents) - Table of contents - Text bolding (or equivalent highlighting) - Text italicization (or equivalent highlighting) - Text subscript - Text superscript - code (in the case code is not stored separately in its native format) - symbols (non-alpha-numeric characters), non-breaking spaces and hyphens - Notes - Editor's Notes - Table footers - Table headers - References - Equations Ideally, there should be a way to preclude usage of styles not listed above. In particular, complex tables shall be avoided. Excerpted text from other languages than English (e.g. in references or examples) should also be supported where necessary. | Yes. |
| 22 | Supported objects | Tools shall support: - Figures (e.g. block diagrams, call flow diagrams, images) - Equations (inline and block). | Yes. |
| 23 | Seeing output while editing | The tool shall support the ability to view the content of (part of a specification) in the form in which it will be published (as a specification), including tables and diagrams while and after editing. | With some limitations. LaTeX generally provides “what you see is what you mean” (WYSIWYM), which is often called “near WYSWYG experience”. There are some editors such as LyX that provide near WYSWYG experience with LaTeX. |
| 24 | Offline work | The tool shall support the ability to read and edit specifications and CRs and to verify CRs offline, in particular to allow private editing before submission. | Yes. |
| 25 | Meeting decisions and reports (meeting minutes) | Tools shall support tracking decisions of the following: CR, draft CR, pCR, TS, TR with Tdoc numbers in meeting reports. | N/A |
| 26 | Public access to specifications and CRs | Tools shall allow easy access to CRs and Specifications to the larger community, including those who do not actively participate in 3GPP, via both online and offline versions. The specifications shall be convertible to PDF for publishing. |  |
| 27 | Fault tolerance | Tools and working methods should have reasonable fault tolerance against minor human errors. Corner cases should be manageable. The specification creation process should aim to minimize the possibility for human errors. | Yes. |
| 28 | Consistent appearance | New specifications (for 6G) need to have a consistent appearance and quality, similar to previous generations. | Yes. |
| 29 | Supported style functions | A delegate, MCC rapporteur shall be able to highlight text, to draw attention to it, even though highlighting is not allowed in the drafting rules and cannot be in an approved CR. A delegate, MCC rapporteur shall be able to remove all formatting from text. A delegate, MCC rapporteur shall be able to view non-printing characters (non-breaking spaces, tabs, new lines, page breaks, etc.) if these are added to the content of the CR. | Yes. |
| 30 | Comments | It shall be possible to associate a comment with specific provisions (e.g. a paragraph, single word, figure, etc.) of any CR. This comment includes comment text, the identity of the commenter and the time in which the comment was provided. It shall be possible to remove or 'disassociate' comments with text in a CR. | N/A. LaTeX is a type-setting language and not a tool to trace changes and add comments. Other tools (editors) may need to leverage to aid CR drafting during and in between meetings. |
| 31 | CR format | CRs using the new specification format for 6G should support visual emphasis of modified sections, be compatible with file formats acceptable for submission to 3GPP meetings, and allow for seamless transfer of the proposed changes into discussion papers as Text Proposals | N/A. LaTeX is a type-setting language and not a editing tool. |
| 32 | CR cover pages | The tool shall support CR cover page (or its equivalent in the new tool) checking (e.g. no dates in the future, CRs to specifications that don't exist, the other specs affected tick boxes are neither yes nor no, and so on). The tool shall also support automatic generation of CR cover pages (where possible). The tool shall also support reviewing, editing and commenting on CR cover pages during CR review process. | N/A. LaTeX is a type-setting language and not an editing tool. |
| 33 | Code | The tool and formats shall support code (e.g. ASN.1). If code is in the same document as the CR/TS, there should be a means to extract the code portions from the technical specification so that the code can be used as input to machine processing (e.g. an interpreter, compiler, etc.) Code that is part of a CR/TS should be able to be checked for syntax errors, and compile time errors. Code that is part of a CR should be able to be checked in combination with other code provided in the specification, to determine if there are redundancies, mismatches, etc. | Yes. |
| 34 | Support review of figures | Every change to a figure shall result in a 'change marking' visible, indicating what changed and who made the change. (NOTE 4) | N/A. LaTeX can support rendering of output that includes figures. The rendering output highlights with change marks should be performed by the text editor. LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. |
| 35 | Support consistent common configuration | It shall be possible to enforce simple and consistent configuration for users, so that errors arising while creating, editing and cut-n-pasting due to inconsistencies are rare or impossible. (NOTE 5) | Yes. |
| 36 | CRs comprising multiple files | The tools shall support (if needed) CRs comprising multiple files in simple and consistent order (i.e. as separate documents stored in a mandatory way for the associated CR or specification). | Yes. |
| 37 | CR database | Changes to CR documents (including headers and content) shall be automatically captured in CR databases to ensure the correct correspondence between an approved and implemented CR and the CR database. | N/A. LaTeX is a type-setting language and not a CR database tool. |
| 38 | Support for gathering input on a CR | The tool shall support gathering input on a CR or its content for highly active topics involving many participants, without divergence in the process. Use of tools to collect comments and feedback shall enable orderly capture of responses, so that the order, source and input from each reviewer is captured without ambiguity or loss. This facility shall scale up to 100s of active reviewers posting 1000s of comments a day. (NOTE 6) | N/A LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. |
| 39 | Simplicity to follow drafting rules | The tool shall enable editors not to be concerned with styles, but rather with the technical content and its hierarchy in the document. | Yes. LaTeX allows styles and typesetting to be configured separately, and most users should not need to understand the type-setting rules. |
| 40 | Support for visualizing different parts of a document concurrently on the same screen | The tool shall enable visualizing different (incl. disjoint) parts of the same document concurrently on the same screen similar to Split View in MS Word. | N/A. LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. Many LaTeX editors support multi-part text visualization. |
| 41 | Support for visualizing different documents concurrently on the same screen | The tool should enable visualizing different documents concurrently on the same screen. | N/A. LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. Many LaTeX editors support multi-part text visualization. |
| 42 | Support for personal scripts | The tool should enable delegate, consumer, MCC/Rapporteur to develop and use their own scripts to manipulate a document locally or together with other documents (similar to MS Office Macros). | N/A. LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. Many LaTeX editors support scripting. |
| 43 | Language proofing | The format shall allow checking a document for errors in spelling, grammar, and punctuation, and ensuring it adheres to the rules of the English language. | N/A. LaTeX is a type-setting language and not an editing tool. There are several editors that work with LaTeX. Many LaTeX editors support proofing. |
| 44 | Folder synchronization | The tools shall support an easy, fast and efficient way to synchronize folders | N/A. LaTeX is a type-setting language and not a file synchronization tool. |

* * * End of Change * * * *

# Reference

- SP-250802, “New SID on Modernization of Specification Format and Procedures for 6G”