# 8 XnAP procedures

## 8.2 Basic mobility procedures

### 8.2.1 Handover Preparation

#### 8.2.1.1 General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2 Successful Operation

Figure 8.2.1.2-1 Handover Preparation, successful operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Target NG-RAN node UE XnAP ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node shall remove the existing prepared conditional HO identified by the Target NG-RAN node UE XnAP ID IE and the Target Cell Global ID IE. It is up to the implementation of the target NG-RAN node when to remove the HO information.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. The source NG-RAN node is then defined to have a Prepared Handover for that Xn UE-associated signalling.

For each E-RAB ID IE included in the QoS Flows To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the content of the IE in the UE context and use it for subsequent inter-system handover.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

At reception of the HANDOVER REQUEST message the target NG-RAN node shall prepare the configuration of the AS security relation between the UE and the target NG-RAN node by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE, as specified in TS 33.501 [28].

Upon reception of the PDU Session Resources To Be Setup List IE, contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as specified in TS 38.413 [5] for the PDU Session Resource Setup procedure. The target NG-RAN node shall report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU session resources. When the target NG-RAN node reports the unsuccessful establishment of a PDU session resource, the cause value should be precise enough to enable the source NG-RAN node to know the reason for the unsuccessful establishment.

For each PDU session if the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store the received PDU Session Aggregate Maximum Bit Rate in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [7].

For each QoS flow for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node shall include the DL Forwarding IE set to "DL forwarding proposed" for all the QoS flows mapped to a DRB, if it requests a DAPS handover for that DRB.

For each PDU session for which the target NG-RAN node decides to admit the data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session level DL data forwarding UP TNL Information IE within the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resource Admitted Info IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

For each QoS flow for which the source NG-RAN node has not yet received the SDAP end marker packet if QoS flow re-mapping happened before handover, the source NG-RAN node shall include the UL Forwarding Proposal IE within the Data Forwarding and Offloading Info from source NG-RAN node IE in the HANDOVER REQUEST message, and if the target NG-RAN node decides to admit uplink data forwarding for at least one QoS flow, the target NG-RAN node may include the PDU Session level UL data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted Item IE contained in the PDU Session Resources Admitted List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the uplink data forwarding.

For each PDU session resource successfully setup at the target NG-RAN, the target NG-RAN node may allocate resources for additional Xn-U PDU session resource GTP-U tunnels, indicated in the Secondary Data Forwarding Info from target NG-RAN node List IE.

If the direct data forwarding path is available between the target NG-RAN node and the source S-NG-RAN node for a PDU session, the target M-NG-RAN node shall, if supported, include the Direct Forwarding Path Availability IE set to "direct path available" in the Data Forwarding Info from target NG-RAN node IE or in the Secondary Data Forwarding Info from target NG-RAN node IE corresponding to the target node of that PDU session in the HANDOVER REQUEST ACKNOWLEDGE message.

For each PDU session in the HANDOVER REQUEST message, if the Alternative QoS Parameters Set List IE is included in the GBR QoS Flow Information IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node may accept the setup of the involved QoS flow when notification control has been enabled if the requested QoS parameters set or at least one of the alternative QoS parameters sets can be fulfilled at the time of handover as specified in TS 23.501 [7]. In case the target NG-RAN node accepts the handover fulfilling one of the alternative QoS parameters it shall indicate the alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE within the PDU Session Resources Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message while setting the QoS parameters towards the UE according to the requested QoS parameters set as specified in TS 23.501 [7] , and behave the same as the NG-RAN node in the PDU Session Resource Setup procedure specified in TS 38.413 [5].

For each DRB for which the source NG-RAN node proposes to perform forwarding of downlink data, the source NG-RAN node shall include the DRB ID IE and the mapped QoS Flows List IE within the Source DRB to QoS Flow Mapping List IE contained in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message. The source NG-RAN node may include the QoS Flow Mapping Indication IE in the Source DRB to QoS Flow Mapping List IE to indicate that only the uplink or downlink QoS flow is mapped to the DRB. If the target NG-RAN node decides to use the same DRB configuration and to map the same QoS flows as the source NG-RAN node, the target NG-RAN node includes the DL Forwarding GTP Tunnel Endpoint IE within the Data Forwarding Response DRB List IE in the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this DRB.

If the HANDOVER REQUEST ACKNOWLEDGE message contains the UL Forwarding UP TNL Information IE for a given DRB in the Data Forwarding Response DRB List IE within Data Forwarding Info from target NG-RAN node IE in the PDU Session Resources Admitted List IE and the source NG-RAN node accepts the data forwarding proposed by the target NG-RAN node, the source NG-RAN node shall perform forwarding of uplink data for the DRB.

If the HANDOVER REQUEST includes PDU session resources for PDU sessions associated to S-NSSAIs not supported by target NG-RAN, the target NG-RAN node shall reject such PDU session resources. In this case, and if at least one PDU Session Resources To Be Setup Item IE is admitted, the target NG-RAN node shall send the HANDOVER REQUEST ACKNOWLEDGE message including the PDU Session Resources Not Admitted List IE listing corresponding PDU sessions rejected at the target NG-RAN.

If the Mobility Restriction List IE is

- contained in the HANDOVER REQUEST message, the target NG-RAN node shall

- store the information received in the Mobility Restriction List IE in the UE context;

- use this information to determine a target for the UE during subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE, except when one of the PDU sessions has a particular ARP value (TS 23.501 [7]) in which case the information shall not apply;

- use this information to select a proper SCG during dual connectivity operation.

- use this information to select proper RNA(s) for the UE when moving the UE to RRC_INACTIVE.

- not contained in the HANDOVER REQUEST message, the target NG-RAN node shall

- consider that no roaming and no access restriction apply to the UE except for the PNI-NPN mobility as described in TS 23.501 [7].

The target NG-RAN node shall consider that roaming or access to CAG cells is only allowed if the Allowed PNI-NPN ID List IE is contained in the HANDOVER REQUEST message, as described in TS 23.501 [7].

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Index to RAT/Frequency Selection Priority IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the UE Context Reference at the S-NG-RAN node IE is contained in the HANDOVER REQUEST message the target NG-RAN node may use it as specified in TS 37.340 [8]. In this case, the source NG-RAN node may expect the target NG-RAN node to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [8].

For each PDU session, if the Network Instance IE is included in the PDU Session Resources To Be Setup List IE and the Common Network Instance IE is not present, the target NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

- For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resources To Be Setup List IE, the target NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for the redundant transmission for the concerned PDU session.

- For each PDU session, if the Additional Redundant UL NG-U UP TNL Information at UPF List IE is included in the PDU Session Resources To Be Setup List IE, the target NG-RAN node shall, if supported, use them as the uplink termination points for the user plane data for the redundant transmission for the concerned PDU session.

- For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resources To Be Setup List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

- For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context and set up the redundant user plane for the concerned PDU session, as specified in TS 23.501 [7]. If the PDU Session Pair ID IE is included in the Redundant PDU Session Information IE, the target NG-RAN node may store and use it to identify the paired PDU sessions.

If the TSC Traffic Characteristics IE is included in the QoS Flows To Be Setup List IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resources To Be Setup List IE or in the Additional UL NG-U UP TNL Information at UPF List IE, or in the Additional Redundant UL NG-U UP TNL Information at UPF List IE, the target NG-RAN node shall, if supported, use it when selecting transport network resource for the concerned NG-U transport bearer as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resources To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", the target NG-RAN node shall perform user plane integrity protection or ciphering, respectively. If the NG-RAN node is not able to perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall behave as specified in TS 33.501 [28].

For each PDU session for which the Security Indication IE is included in the PDU Session Resources To Be Setup List IE and the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", the target NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively and shall notify the SMF whether it succeeded the user plane integrity protection or ciphering or not for the concerned security policy.

For each PDU session for which the Maximum Integrity Protected Data Rate IE is included in the Security Indication IE in the PDU Session Resources To Be Setup List IE, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic corresponding to the received Maximum Integrity Protected Data Rate IE, for the concerned PDU session and concerned UE, as specified in TS 23.501 [7].

For each PDU session for which the Security Indication IE is included in the PDU Session Resources To Be Setup List IE and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", the target NG-RAN node shall not perform user plane integrity protection or ciphering, respectively, for the concerned PDU session.

For each PDU session, if the Additional UL NG-U UP TNL Information at UPF List IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node may forward the UP transport layer information to the target S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message, then the target NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

If the Trace Activation IE is included in the HANDOVER REQUEST message which includes

- the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

- the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

- the MDT Location Information IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

- the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

NOTE: In case of intra-PLMN handover, if the Signalling based MDT PLMN List within the MDT Configuration-EUTRA is not available in the source NG-RAN node it includes the serving PLMN in the Signalling based MDT PLMN List IE within the MDT Configuration-EUTRA IE.

- the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

- the WLAN Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

- the Sensor Measurement Configuration IE, within the MDT Configuration IE, the target NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

- the MDT Configuration IE and if the target NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the target NG-RAN node shall store it as part of the UE context, and use it as described in TS 37.320 [43].

- the MN only MDT collection IE, within the MDT Configuration IE, set to "MN Only", the NG-RAN node considers that the MDT Configuration-NR IE or the MDT Configuration-EUTRA IE is only applicable for the Master Node if the UE is configured with MR-DC.

If the Area Scope IE is not present in the MDT Configuration IE, the target NG-RAN node shall consider that the MDT Configuration is applied to all PLMNs indicated in the MDT PLMN List, as described in TS 32.422 [23].

If the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the HANDOVER REQUEST message includes the Management Based MDT PLMN List IE, the target NG-RAN node shall, if supported, store it in the UE context, and take it into account if it includes information regarding the PLMN serving the UE in the target NG-RAN node.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information. The target NG-RAN shall, if supported, store the C-RNTI assigned at the source cell as received in the HANDOVER REQUEST message.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

For each QoS flow which has been successfully established in the target NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting. For each QoS Flow, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the target NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

If the HANDOVER REQUEST message includes the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE set to "true", the target NG-RAN node shall, if supported, report in the HANDOVER REQUEST ACKNOWLEDGE message the PDU Set based Handling Indicator IE.

For each QoS flow which has been successfully established in the target NG-RAN node, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it accordingly for the specific QoS flow.

For a QoS flow established with PDU Set QoS parameters, if the PDU Set based Handling Indicator IE set to "supported" is included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the PDU Set Information Container in the data to be forwarded.

If the 5GC Mobility Restriction List Container IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

V2X:

- If the NR V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the LTE V2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

- If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

A2X:

- If the NR A2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the LTE A2X Services Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the NR A2X UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR A2X services.

- If the LTE A2X UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE A2X services.

- If the A2X PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.256 [56].

5G ProSe:

- If the 5G ProSe Authorized IE is included in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- If the 5G ProSe PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

Ranging and SL Positioning Services:

- If the Ranging and Sidelink Positioning Authorized IE, within the Ranging and Sidelink Positioning Services Information IE, is included in the HANDOVER REQUEST message and set to "authorized", the target NG-RAN node shall, if supported, consider that the UE is authorized for the Ranging and Sidelink Positioning services.

If the PC5 QoS Parameters IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

If the Aerial UE Subscription Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 38.300 [9].

If the Candidate Relay UE Info List IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure the path switch to indirect path as specified in TS 38.300 [9].

If the DAPS Request Information IE is included for a given DRB in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a DAPS handover for that DRB, as described in TS 38.300 [9]. Accordingly, the target NG-RAN node shall include the DAPS Response Information IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Maximum Number of CHO Preparations IE is included in the Conditional Handover Information Acknowledge IE contained in the HANDOVER REQUEST ACKNOWLEDGE message, then the source NG-RAN node should not prepare more candidate target cells for a CHO for the same UE towards the target NG-RAN node than the number indicated in the IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use this information to allocate necessary resources for the incoming CHO.

If the Maximum Number of Conditional Reconfigurations to Prepare IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to prepare for CHO with candidate PSCell(s) configuration and may include the CHO-CPAC Information IE within the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message. The target NG-RAN node may also provide a CHO-only configuration, which is counted as one toward the maximum number indicated by the Maximum Number of Conditional Reconfigurations to Prepare IE.

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node. In addition:

- If the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Setup List IE, and shall not take any action with respect to PDU session setup. Subsequently, the source NG-RAN node shall, if supported, ignore the PDU Session Resources Admitted To Be Added List IE in the HANDOVER REQUEST ACKNOWLEDGE message.

- If the IAB Authorization Status IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received IAB authorization status information in the UE context and use it as specified in TS 38.401 [2].

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If for a given QoS Flow the Source DL Forwarding IP Address IE is included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resources To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the MBS Session Information List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, establish an NG-RAN MBS session resources context as specified in TS 23.247 [46] and TS 38.300 [9], if applicable.

If the HANDOVER REQUEST message includes the MBS Area Session ID IE, the target NG-RAN, if supported, shall use this information as an indication from which MBS Area Session ID the UE is handed over. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item List IE, the target NG-RAN shall, if supported, use this information to setup respective MBS session resources. The target NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the HANDOVER REQUEST ACKNOWLEDGE message contains in the MBS Session Information Response List IE the MBS Data Forwarding Response Info from target NG-RAN node IE that the source NG-RAN node shall use the information for forwarding MBS traffic to the target NG-RAN node.

If the MBS Session Associated Information List IE is included in the PDU Session Resources To Be Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use the information contained in the Associated QoS Flow Information List IE as specified in TS 23.247 [46].

For each MRB indicated in the MBS Mapping and Data Forwarding Request Info from source NG-RAN node IE, the target NG-RAN node shall use the MRB ID IE and, if included, the MRB Progress Information IE which includes the highest PDCP SN of the packet which has already been delivered to the UE for the MRB, to decide whether to apply data forwarding for that MRB and to establish respective resources.

The source NG-RAN shall, for each MRB in the MBS Data Forwarding Response Info from target NG-RAN node IE in the HANDOVER REQUEST ACKNOWLEDGE message, start data forwarding to the indicated DL Forwarding UP TNL Information. If the MRB Progress Information IE is included the source NG-RAN node may use the information to determine when to stop data forwarding.

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the SN-related QMC Information at MN IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

If the Source SN to Target SN QMC Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

If the Assistance Information for QoE Measurement IE is included in the UE Application Layer Measurement Configuration Information IE within the QMC Configuration Information IE in the HANDOVER REQUEST message, the target NG-RAN node may take it into account for controlling the UE’s application layer measurement reporting when the NG-RAN node is overloaded, as described in TS 38.300 [9].

If the UE Slice Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Cell Based UE Trajectory Prediction IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider the content of this list as a prediction by the source NG-RAN node of the cells that the UE will be connected to, and may use it for e.g. mobility decisions.

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN. Upon reception of the PNI-NPN Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection of PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the RRC Config Indication IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the target NG-RAN node applied a full configuration or delta configuration, e.g., as part of conditional handover procedure involving dual connectivity operation.

If the Mobile IAB Authorization Status IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received Mobile IAB authorization status information in the UE context and consider that the handover is for a mobile IAB-node. If the Mobile IAB Authorization Status IE is set to "not authorized" for a mobile IAB-MT, the target NG-RAN node shall, if supported, store it and use it as defined in TS 38.401[2].

If the DL LBT Failure Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the DL LBT failure information of the UE in the target cell for MRO analysis, as specified in TS 38.300 [9].

For each GBR QoS flow which has been successfully established in the target NG-RAN node, if the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource To Be Setup List IE contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

For each QoS flow, if the MMSID IE is included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [7] and TS 38.300 [9].

For each QoS flow, if the Indication of Bitrate Adaptation IE set to "uplink" is included in the QoS Flow Level QoS Parameters IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received indication and use it for uplink rate control, as defined in TS 38.300 [9].

If the LTM Handover Information Request IE included in the HANDOVER REQUEST message and the LTM Indicator IE is set to "true", the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration. If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI-RS Resource Configuration for Early CSI Acquisition IE is included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, act as specified in TS 38.300 [9].

If the CSI Resource Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM Candidate Configuration IE for the requested candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Proposed LTM L2 Reset Configuration List IE is included in the LTM Handover Information Request IE in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM L2 Reset Configuration IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, consider that the source NG-RAN node requests LTM preparation for an already established UE association.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Continuous MDT IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account to configure the UE with Continuous Management Based MDT.

If the S-NSSAI dedicated to WAB-MT’s backhaul PDU session(s) is included in the UE Context Information IE in the HANDOVER REQUEST message, the target NG-RAN node shall act as specified in TS 38.401 [2].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover via the Data Collection Reporting procedure and according to clause 8.4.13.2, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node,.

#### 8.2.1.3 Unsuccessful Operation

Figure 8.2.1.3-1 Handover Preparation, unsuccessful operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message and the target NG-RAN node rejects the handover or a failure occurs during the Handover Preparation, the target NG-RAN node shall include the Requested Target Cell ID IE in the HANDOVER PREPARATION FAILURE message.

Interactions with Handover Cancel procedure:

If there is no response from the target NG-RAN node to the HANDOVER REQUEST message before timer TXnRELOCprep expires in the source NG-RAN node, the source NG-RAN node should cancel the Handover Preparation procedure towards the target NG-RAN node by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source NG-RAN node shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned Xn UE-associated signalling.

#### 8.2.1.4 Abnormal Conditions

If the supported algorithms for encryption defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EEA0 and NEA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the supported algorithms for integrity defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EIA0 and NIA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the CHO trigger IE is set to "CHO-replace" in the HANDOVER REQUEST message, but there is no CHO prepared for the included Target NG-RAN node UE XnAP ID, or the candidate cell in the Target Cell ID IE was not prepared using the same UE-associated signaling connection, the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the HANDOVER REQUEST message includes information for a PLMN not serving the UE in the target NG-RAN node in the Management Based MDT PLMN List IE, the target NG-RAN node shall ignore information for that PLMN within the Management Based MDT PLMN List.

If both the PNI-NPN Area Scope of MDT IE and the Area Scope of MDT-NR IE are included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the Area Scope of MDT-NR IE is set to "PNI-NPN based", the target NG-RAN node shall, if supported, use the Area Scope of MDT-NR IE to derive the MDT area scope for MDT measurement collections in PNI-NPN areas, and ignore the PNI-NPN Area Scope of MDT IE.

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the Area Scope of MDT-NR IE is not included, the target NG-RAN node shall ignore the PNI-NPN Area Scope of MDT IE, and consider that the MDT Configuration for NR is applied to all PLMNs indicated in the MDT PLMN List described in TS 32.422 [23].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the MDT Activation IE is set to “Logged MDT only”, the NG-RAN node shall ignore the Network Slice Area Scope of MDT IE.

### 8.2.13 Partial UE Context Transfer

#### 8.2.13.1 General

The purpose of the Partial UE Context Transfer procedure is to partially transfer the UE context from the old NG-RAN node to the new NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.13.2 Successful Operation

Figure 8.2.13.2-1: Partial UE Context Transfer, successful operation

Figure 8.2.13.2-1: Partial UE Context Transfer, successful operation

The old NG-RAN node initiates the procedure by sending the PARTIAL UE CONTEXT TRANSFER message to the new NG-RAN node.

If the new NG-RAN node is able to accept the SDT session without anchor relocation, it shall, if supported, respond to the old NG-RAN node with the PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message.

If the Partial UE Context Information for SDT IE is included in the PARTIAL UE CONTEXT TRANSFER message, the new NG-RAN node may include data forwarding related information in the SDT Data Forwarding DRB List IE in the PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message.

If the Partial UE Context Information for Positioning IE is contained in the PARTIAL UE CONTEXT TRANSFER message, the new NG-RAN node may take it into account to allocate proper SRS resources and include the SRS Configuration IE in the PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message.

#### 8.2.13.3 Unsuccessful Operation

Figure 8.2.13.3-1: Partial UE Context Transfer, unsuccessful operation

Figure 8.2.13.3-1: Partial UE Context Transfer, unsuccessful operation

If the new NG-RAN is not able to accept the SDT session without anchor relocation, it shall respond to the old NG-RAN node with the PARTIAL UE CONTEXT TRANSFER FAILURE message.

#### 8.2.13.4 Abnormal Condition

Void.

### 8.2.4 Retrieve UE Context

#### 8.2.4.1 General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission. The procedure can also be used to transfer the authorization status information of the mobile IAB-node.

The procedure uses UE-associated signalling.

#### 8.2.4.2 Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, initiate the requested trace function as specified in TS 32.422 [23].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

If the Trace Activation IE is included in the RETRIEVE UE CONTEXT RESPONSE message which includes

- the MDT Activation IE set to "Immediate MDT and Trace", then the new NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23].

- the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the new NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

- the MDT Location Information IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

- the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the new NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [43].

NOTE: If the Signalling based MDT PLMN List within the MDT Configuration-EUTRA is not available in the old NG-RAN node, it includes the current serving PLMN in the Signalling based MDT PLMN List IE within the MDT Configuration-EUTRA IE.

- the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

- the WLAN Measurement Configuration IE, within the MDT Configuration IE, the new NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

- the Sensor Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [43].

- the MDT Configuration and if the new NG-RAN node is a gNB receiving a MDT Configuration-EUTRA IE, or the target NG-RAN node is a ng-eNB receiving a MDT Configuration-NR IE, the new NG-RAN node shall store it as part of the UE context, and use it as described in TS 37.320 [43].

- the MN only MDT collection IE, within the MDT Configuration IE, set to "MN Only", the NG-RAN node consider that the MDT Configuration-NR IE or the MDT Configuration-EUTRA IE is only applicable for Master the Node if the UE is configured with MR-DC.

If the Area Scope IE is not present in the MDT Configuration IE, the new NG-RAN node shall consider that the MDT Configuration is applied to all PLMNs indicated in the MDT PLMN List, as described in TS 32.422 [23].

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and shall, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall store this information, and shall, if supported, use it for RAN part delay reporting. For each QoS Flow, if the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall, if supported, use it as specified in TS 23.501 [7].

For each QoS flow which has been successfully established in the new NG-RAN node, if the ECN Marking or Congestion Information Reporting Request IE is included in the PDU Session Resources To Be Setup List IE contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it accordingly for the specific QoS flow.

V2X:

- If the NR V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the LTE V2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

- If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE Context Information Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

A2X:

- If the NR A2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the LTE A2X Services Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the NR A2X UE PC5 Aggregate Maximum Bit Rate IE is included in the UE Context Information – Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR A2X services.

- If the LTE A2X UE PC5 Aggregate Maximum Bit Rate IE is included in the UE Context Information – Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE A2X services.

- If the A2X PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.256 [56].

5G ProSe:

- If the 5G ProSe Authorized IE is included in the RETRIEVE UE CONTEXT RESPONSE message and it contains one or more IEs set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the relevant service(s).

- If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE Context Information – Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

- If the 5G ProSe PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.304 [48].

If the PC5 QoS Parameters IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it as defined in TS 23.287 [38].

Ranging and SL Positioning Services:

- If the Ranging and Sidelink Positioning Authorized IE, within the Ranging and Sidelink Positioning Services Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message and set to "authorized", the new NG-RAN node shall, if supported, consider that the UE is authorized for the Ranging and Sidelink Positioning services.

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the Aerial UE Subscription Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported, store this information in the UE context and use it as defined in TS 38.300 [9].

If the Management Based MDT PLMN List IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store it in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the RETRIEVE UE CONTEXT RESPONSE message includes the MBS Area Session ID IE, the new NG-RAN node shall, if supported, use this information as an indication in which MBS Area Session ID the UE has been suspended. For each MBS session for which the Active MBS Session Information IE is included in the MBS Session Information Item IE, the new NG-RAN node shall, if supported, use this information to setup respective MBS session resources. The new NG-RAN node shall, if supported, consider that the MBS sessions for which the Active MBS Session Information IE is not included are inactive.

If the IAB Node Indication IE set to "true" is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider that the procedure is performed for an IAB-node. In addition:

- If the No PDU Session Indication IE set to "true" is contained in the UE Context Information – Retrieve UE Context Response IE of the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resources To Be Setup List IE in the UE Context Information – Retrieve UE Context Response IE, and shall not take any action with respect to PDU session setup.

If the Time Synchronisation Assistance Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

If the QMC Configuration Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.300 [9].

If the Assistance Information for QoE Measurement IE is included in the UE Application Layer Measurement Configuration Information IE within the QMC Configuration Information IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node may store this information and consider it for controlling the UE reporting when the new NG-RAN node is overloaded, as described in TS 38.300 [9].

If the SDT Support Request IE is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN node shall, if supported, consider that the UE has requested for SDT as defined in TS 38.300 [9].

If the SRS Positioning Configuration Or Activation Request IE set to "true" is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN may include the NRPPa Positioning Information IE within the UE Context Information – Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message.

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN. Upon reception of the PNI-NPN Area Scope of MDT IE, the new NG-RAN node shall consider that the area scope for MDT measurement collections of PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the UE is a mobile IAB-node, the old NG-RAN node shall include the Mobile IAB Authorization Status IE in the RETRIEVE UE CONTEXT RESPONSE message. If the Mobile IAB Authorization Status IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider that the UE is a mobile IAB-node, then store it and use it accordingly as defined in TS 38.401 [2].

For each GBR QoS flow in the new NG-RAN node, if the Monitoring Request on Available Bitrate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource To Be Setup List IE contained in RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the MMSID IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [7] and TS 38.300 [9].

For each QoS flow in the RETRIEVE UE CONTEXT RESPONSE message, if the Indication of Bitrate Adaptation IE set to "uplink" is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resources To Be Setup List IE, the new NG-RAN node shall, if supported, store the received indication and use it for uplink rate control, as defined in TS 38.300 [9].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the new NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the UE context information – Retrieve UE Context Response IE in the RETRIEVE UE CONTEXT RESPONSE message includes:

- Mobility Restriction List IE, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

- Index to RAT/Frequency Selection Priority IE, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

- 5GC Mobility Restriction List Container IE, the new NG-RAN node shall, if supported, store this information in the UE context and use it as specified in TS 38.300 [9].

- UE Radio Capability ID IE, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

- MBS Session Information List IE, the new NG-RAN node shall, if supported, use this information to establish an NG-RAN MBS session resources context, if applicable.

- UE Slice Maximum Bit Rate List IE, the new NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

- Positioning Information IE, the new NG-RAN node shall, if supported, take it into account to allocate proper SRS resources and make corresponding response to LMF when positioning a UE.

If the Continuous MDT IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, take it into account to configure the UE with Continuous Management Based MDT.

Interaction with the Retrieve UE Context Confirm procedure

If the UE Context Reference at the S-NG-RAN node IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node may use it to establish dual connectivity with the S-NG-RAN node and shall trigger the Retrieve UE Context Confirm procedure to the old NG-RAN node when the UE successfully resumes on the new NG-RAN node.

#### 8.2.4.3 Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU or in case of RACH based SDT, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

If the Semi-persistent Positioning Information IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node shall, if supported, take it into account to activate or deactivate the semi-persistent SRS transmission for area-specific SRS configuration.

Interaction with Partial UE Context Transfer procedure

In case of RACH based SDT, if the old NG-RAN node decides to not transfer/relocate the UE Context to the new NG-RAN node, it may trigger the Partial UE Context Transfer procedure as specified in TS 38.300 [9]. After the old NG-RAN node has decided to end the SDT session, it shall terminate the Retrieve UE Context procedure by sending the RETRIEVE UE CONTEXT FAILURE message.

#### 8.2.4.4 Abnormal Conditions

If both the PNI-NPN Area Scope of MDT IE and the Area Scope of MDT-NR IE are included in the MDT Configuration-NR IE in the RETRIEVE UE CONTEXT RESPONSE message, and the Area Scope of MDT-NR IE is set to "PNI-NPN based", the new NG-RAN node shall, if supported, use the Area Scope of MDT-NR IE to derive the MDT area scope for MDT measurement collection in PNI-NPN areas, and ignore the PNI-NPN Area Scope of MDT IE.

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE in the RETRIEVE UE CONTEXT RESPONSE message, and the Area Scope of MDT-NR IE is not included, the target NG-RAN node shall ignore the PNI-NPN Area Scope of MDT IE, and consider that the MDT Configuration for NR is applied to all PLMNs indicated in the MDT PLMN List described in TS 32.422 [23].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the MDT Activation IE is set to “Logged MDT only”, the NG-RAN node shall ignore the Network Slice Area Scope of MDT IE.

-- 9.3.3    Elementary Procedure Definitions
-- ASN1START
-- **************************************************************
--
-- Elementary Procedure definitions
--
-- **************************************************************

XnAP-PDU-Descriptions {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************
--
-- IE parameter types from other modules.
--
-- **************************************************************

IMPORTS
    Criticality,
    ProcedureCode

FROM XnAP-CommonDataTypes

    HandoverRequest,
    HandoverRequestAcknowledge,
    HandoverPreparationFailure,
    SNStatusTransfer,
    UEContextRelease,
    HandoverCancel,
    NotificationControlIndication,
    RANPaging,
    RetrieveUEContextRequest,
    RetrieveUEContextResponse,
    RetrieveUEContextConfirm,
    RetrieveUEContextFailure,
    XnUAddressIndication,
    SecondaryRATDataUsageReport,
    SNodeAdditionRequest,
    SNodeAdditionRequestAcknowledge,
    SNodeAdditionRequestReject,
    SNodeReconfigurationComplete,
    SNodeModificationRequest,
    SNodeModificationRequestAcknowledge,
    SNodeModificationRequestReject,
    SNodeModificationRequired,
    SNodeModificationConfirm,
    SNodeModificationRefuse,
    SNodeReleaseRequest,
    SNodeReleaseRequestAcknowledge,
    SNodeReleaseReject,
    SNodeReleaseRequired,
    SNodeReleaseConfirm,
    SNodeCounterCheckRequest,
    SNodeChangeRequired,
    SNodeChangeConfirm,
    SNodeChangeRefuse,
    RRCTransfer,
    XnRemovalRequest,
    XnRemovalResponse,
    XnRemovalFailure,
    XnSetupRequest,
    XnSetupResponse,
    XnSetupFailure,
    NGRANNodeConfigurationUpdate,
    NGRANNodeConfigurationUpdateAcknowledge,
    NGRANNodeConfigurationUpdateFailure,
    E-UTRA-NR-CellResourceCoordinationRequest,
    E-UTRA-NR-CellResourceCoordinationResponse,
    ActivityNotification,
    CellActivationRequest,
    CellActivationResponse,
    CellActivationFailure,
    ResetRequest,
    ResetResponse,
    ErrorIndication,
    PrivateMessage,
    DeactivateTrace,
    TraceStart,
    HandoverSuccess,
    ConditionalHandoverCancel,
    EarlyStatusTransfer,
    FailureIndication,
    HandoverReport,
    SCGFailureIndication,
    ResourceStatusRequest,
    ResourceStatusResponse,
    ResourceStatusFailure,
    ResourceStatusUpdate,
    MobilityChangeRequest,
    MobilityChangeAcknowledge,
    MobilityChangeFailure,
    AccessAndMobilityIndication,
    CellTrafficTrace,
    RANMulticastGroupPaging,
    ScgFailureInformationReport,
    ScgFailureTransfer,
    F1CTrafficTransfer,
    IABTransportMigrationManagementRequest,
    IABTransportMigrationManagementResponse,
    IABTransportMigrationManagementReject,
    IABTransportMigrationModificationRequest,
    IABTransportMigrationModificationResponse,
    IABResourceCoordinationRequest,
    IABResourceCoordinationResponse,
    CPCCancel,
    PartialUEContextTransfer,
    PartialUEContextTransferAcknowledge,
    PartialUEContextTransferFailure,
    RachIndication,
    DataCollectionRequest,
    DataCollectionResponse,
    DataCollectionFailure,
    DataCollectionUpdate,
    ODSIB1ConfigurationProvisionRequest,
    ODSIB1ConfigurationProvisionResponse,
    ODSIB1ConfigurationProvisionFailure,
    ODSIB1ConfigurationProvisionStatusUpdate,
    LTMConfigurationUpdate,
    LTMConfigurationUpdateAcknowledge,
    LTMConfigurationUpdateFailure,
    CSIRSCoordinationRequest,
    CSIRSCoordinationResponse,
    CellSwitchNotification,
    TAInformationTransfer,
    LTMCancel,
    CLI-Indication





FROM XnAP-PDU-Contents

    id-handoverPreparation,
    id-sNStatusTransfer,
    id-handoverCancel,
    id-notificationControl,
    id-retrieveUEContext,
    id-rANPaging,
    id-xnUAddressIndication,
    id-uEContextRelease,
    id-secondaryRATDataUsageReport,
    id-sNGRANnodeAdditionPreparation,
    id-sNGRANnodeReconfigurationCompletion,
    id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,
    id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,
    id-mNGRANnodeinitiatedSNGRANnodeRelease,
    id-sNGRANnodeinitiatedSNGRANnodeRelease,
    id-sNGRANnodeCounterCheck,
    id-sNGRANnodeChange,
    id-activityNotification,
    id-rRCTransfer,
    id-xnRemoval,
    id-xnSetup,
    id-nGRANnodeConfigurationUpdate,
    id-e-UTRA-NR-CellResourceCoordination,
    id-cellActivation,
    id-reset,
    id-errorIndication,
    id-privateMessage,
    id-deactivateTrace,
    id-traceStart,
    id-handoverSuccess,
    id-conditionalHandoverCancel,
    id-earlyStatusTransfer,
    id-failureIndication,
    id-handoverReport,
    id-scgFailureIndication,
    id-resourceStatusReportingInitiation,
    id-resourceStatusReporting,
    id-mobilitySettingsChange,
    id-accessAndMobilityIndication,
    id-cellTrafficTrace,
    id-RANMulticastGroupPaging,
    id-scgFailureInformationReport,
    id-scgFailureTransfer,
    id-f1CTrafficTransfer,
    id-iABTransportMigrationManagement,
    id-iABTransportMigrationModification,
    id-iABResourceCoordination,
    id-retrieveUEContextConfirm,
    id-cPCCancel,
    id-partialUEContextTransfer,
    id-rachIndication,
    id-dataCollectionReportingInitiation,
    id-dataCollectionReporting,
    id-ODSIB1ConfigurationProvision,
    id-ODSIB1ConfigurationProvisionStatusUpdate,
    id-lTMConfigurationUpdate,
    id-cSIRSCoordination,
    id-cellSwitchNotification,
    id-tAInformationTransfer,
    id-lTMCancel,
    id-cLI-Indication





FROM XnAP-Constants;

-- **************************************************************
--
-- Interface Elementary Procedure Class
--
-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {
    &InitiatingMessage              ,
    &SuccessfulOutcome              OPTIONAL,
    &UnsuccessfulOutcome                OPTIONAL,
    &procedureCode          ProcedureCode   UNIQUE,
    &criticality            Criticality     DEFAULT ignore
}
WITH SYNTAX {
    INITIATING MESSAGE      &InitiatingMessage
    [SUCCESSFUL OUTCOME     &SuccessfulOutcome]
    [UNSUCCESSFUL OUTCOME       &UnsuccessfulOutcome]
    PROCEDURE CODE          &procedureCode
    [CRITICALITY            &criticality]
}

-- **************************************************************
--
-- Interface PDU Definition
--
-- **************************************************************

XnAP-PDU ::= CHOICE {
    initiatingMessage   InitiatingMessage,
    successfulOutcome   SuccessfulOutcome,
    unsuccessfulOutcome UnsuccessfulOutcome,
    ...
}

InitiatingMessage ::= SEQUENCE {
    procedureCode   XNAP-ELEMENTARY-PROCEDURE.&procedureCode        ({XNAP-ELEMENTARY-PROCEDURES}),
    criticality     XNAP-ELEMENTARY-PROCEDURE.&criticality          ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),
    value           XNAP-ELEMENTARY-PROCEDURE.&InitiatingMessage    ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})
}

SuccessfulOutcome ::= SEQUENCE {
    procedureCode   XNAP-ELEMENTARY-PROCEDURE.&procedureCode        ({XNAP-ELEMENTARY-PROCEDURES}),
    criticality     XNAP-ELEMENTARY-PROCEDURE.&criticality          ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),
    value           XNAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome    ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})
}

UnsuccessfulOutcome ::= SEQUENCE {
    procedureCode   XNAP-ELEMENTARY-PROCEDURE.&procedureCode        ({XNAP-ELEMENTARY-PROCEDURES}),
    criticality     XNAP-ELEMENTARY-PROCEDURE.&criticality          ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode}),
    value           XNAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome  ({XNAP-ELEMENTARY-PROCEDURES}{@procedureCode})
}

-- **************************************************************
--
-- Interface Elementary Procedure List
--
-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {
    XNAP-ELEMENTARY-PROCEDURES-CLASS-1          |
    XNAP-ELEMENTARY-PROCEDURES-CLASS-2          ,
    ...
}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {
    handoverPreparation                                     |
    retrieveUEContext                                       |
    sNGRANnodeAdditionPreparation                           |
    mNGRANnodeinitiatedSNGRANnodeModificationPreparation    |
    sNGRANnodeinitiatedSNGRANnodeModificationPreparation    |
    mNGRANnodeinitiatedSNGRANnodeRelease                    |
    sNGRANnodeinitiatedSNGRANnodeRelease                    |
    sNGRANnodeChange                                        |
    xnRemoval                                               |
    xnSetup                                                 |
    nGRANnodeConfigurationUpdate                            |
    e-UTRA-NR-CellResourceCoordination                      |
    cellActivation                                          |
    reset                                                   |
    resourceStatusReportingInitiation                       |
    mobilitySettingsChange                                  |
    iABTransportMigrationManagement                         |
    iABTransportMigrationModification                       |
    iABResourceCoordination                                 |
    partialUEContextTransfer                                |
    dataCollectionReportingInitiation                       |
    oDSIB1ConfigurationProvision                            |
    lTMConfigurationUpdate                                  |
    cSIRSCoordination                                       |
    cLI-Indication,
    ...
}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {
    sNStatusTransfer                        |
    handoverCancel                          |
    rANPaging                               |
    xnUAddressIndication                    |
    uEContextRelease                        |
    sNGRANnodeReconfigurationCompletion     |
    sNGRANnodeCounterCheck                  |
    rRCTransfer                             |
    errorIndication                         |
    privateMessage                          |
    notificationControl                     |
    activityNotification                    |
    secondaryRATDataUsageReport             |
    deactivateTrace                         |
    traceStart                              |
    handoverSuccess                         |
    conditionalHandoverCancel               |
    earlyStatusTransfer                     |
    failureIndication                       |
    handoverReport                          |
    scgFailureIndication                    |
    resourceStatusReporting                 |
    accessAndMobilityIndication             |
    cellTrafficTrace                        |
    rANMulticastGroupPaging                 |
    scgFailureInformationReport             |
    scgFailureTransfer                      |
    f1CTrafficTransfer                      |
    retrieveUEContextConfirm                |
    cPCCancel                               |
    rachIndication                          |
    dataCollectionReporting                 |
    oDSIB1ConfigurationProvisionStatusUpdate|
    cellSwitchNotification                  |
    tAInformationTransfer                   |
    lTMCancel,
    ...

}

-- **************************************************************
--
-- Interface Elementary Procedures
--
-- **************************************************************

handoverPreparation XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      HandoverRequest
    SUCCESSFUL OUTCOME      HandoverRequestAcknowledge
    UNSUCCESSFUL OUTCOME    HandoverPreparationFailure
    PROCEDURE CODE          id-handoverPreparation
    CRITICALITY             reject
}


sNStatusTransfer    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNStatusTransfer
    PROCEDURE CODE          id-sNStatusTransfer
    CRITICALITY             ignore
}


handoverCancel  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      HandoverCancel
    PROCEDURE CODE          id-handoverCancel
    CRITICALITY             ignore
}


retrieveUEContext   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      RetrieveUEContextRequest
    SUCCESSFUL OUTCOME      RetrieveUEContextResponse
    UNSUCCESSFUL OUTCOME    RetrieveUEContextFailure
    PROCEDURE CODE          id-retrieveUEContext
    CRITICALITY             reject
}


rANPaging   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      RANPaging
    PROCEDURE CODE          id-rANPaging
    CRITICALITY             reject
}


xnUAddressIndication    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      XnUAddressIndication
    PROCEDURE CODE          id-xnUAddressIndication
    CRITICALITY             reject
}


uEContextRelease    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      UEContextRelease
    PROCEDURE CODE          id-uEContextRelease
    CRITICALITY             reject
}


sNGRANnodeAdditionPreparation   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeAdditionRequest
    SUCCESSFUL OUTCOME      SNodeAdditionRequestAcknowledge
    UNSUCCESSFUL OUTCOME    SNodeAdditionRequestReject
    PROCEDURE CODE          id-sNGRANnodeAdditionPreparation
    CRITICALITY             reject
}


sNGRANnodeReconfigurationCompletion XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeReconfigurationComplete
    PROCEDURE CODE          id-sNGRANnodeReconfigurationCompletion
    CRITICALITY             reject
}


mNGRANnodeinitiatedSNGRANnodeModificationPreparation    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeModificationRequest
    SUCCESSFUL OUTCOME      SNodeModificationRequestAcknowledge
    UNSUCCESSFUL OUTCOME    SNodeModificationRequestReject
    PROCEDURE CODE          id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation
    CRITICALITY             reject
}


sNGRANnodeinitiatedSNGRANnodeModificationPreparation    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeModificationRequired
    SUCCESSFUL OUTCOME      SNodeModificationConfirm
    UNSUCCESSFUL OUTCOME    SNodeModificationRefuse
    PROCEDURE CODE          id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation
    CRITICALITY             reject
}


mNGRANnodeinitiatedSNGRANnodeRelease    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeReleaseRequest
    SUCCESSFUL OUTCOME      SNodeReleaseRequestAcknowledge
    UNSUCCESSFUL OUTCOME    SNodeReleaseReject
    PROCEDURE CODE          id-mNGRANnodeinitiatedSNGRANnodeRelease
    CRITICALITY             reject
}


sNGRANnodeinitiatedSNGRANnodeRelease    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeReleaseRequired
    SUCCESSFUL OUTCOME      SNodeReleaseConfirm
    PROCEDURE CODE          id-sNGRANnodeinitiatedSNGRANnodeRelease
    CRITICALITY             reject
}


sNGRANnodeCounterCheck  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeCounterCheckRequest
    PROCEDURE CODE          id-sNGRANnodeCounterCheck
    CRITICALITY             reject
}


sNGRANnodeChange        XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SNodeChangeRequired
    SUCCESSFUL OUTCOME      SNodeChangeConfirm
    UNSUCCESSFUL OUTCOME    SNodeChangeRefuse
    PROCEDURE CODE          id-sNGRANnodeChange
    CRITICALITY             reject
}


rRCTransfer XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      RRCTransfer
    PROCEDURE CODE          id-rRCTransfer
    CRITICALITY             reject
}


xnRemoval   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      XnRemovalRequest
    SUCCESSFUL OUTCOME      XnRemovalResponse
    UNSUCCESSFUL OUTCOME        XnRemovalFailure
    PROCEDURE CODE          id-xnRemoval
    CRITICALITY             reject
}


xnSetup XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      XnSetupRequest
    SUCCESSFUL OUTCOME      XnSetupResponse
    UNSUCCESSFUL OUTCOME        XnSetupFailure
    PROCEDURE CODE          id-xnSetup
    CRITICALITY             reject
}


nGRANnodeConfigurationUpdate    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      NGRANNodeConfigurationUpdate
    SUCCESSFUL OUTCOME      NGRANNodeConfigurationUpdateAcknowledge
    UNSUCCESSFUL OUTCOME    NGRANNodeConfigurationUpdateFailure
    PROCEDURE CODE          id-nGRANnodeConfigurationUpdate
    CRITICALITY             reject
}

partialUEContextTransfer    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      PartialUEContextTransfer
    SUCCESSFUL OUTCOME      PartialUEContextTransferAcknowledge
    UNSUCCESSFUL OUTCOME    PartialUEContextTransferFailure
    PROCEDURE CODE          id-partialUEContextTransfer
    CRITICALITY             reject
}

e-UTRA-NR-CellResourceCoordination  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      E-UTRA-NR-CellResourceCoordinationRequest
    SUCCESSFUL OUTCOME      E-UTRA-NR-CellResourceCoordinationResponse
    PROCEDURE CODE          id-e-UTRA-NR-CellResourceCoordination
    CRITICALITY             reject
}


cellActivation  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      CellActivationRequest
    SUCCESSFUL OUTCOME      CellActivationResponse
    UNSUCCESSFUL OUTCOME    CellActivationFailure
    PROCEDURE CODE          id-cellActivation
    CRITICALITY             reject
}


reset   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ResetRequest
    SUCCESSFUL OUTCOME      ResetResponse
    PROCEDURE CODE          id-reset
    CRITICALITY             reject
}


errorIndication XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ErrorIndication
    PROCEDURE CODE          id-errorIndication
    CRITICALITY             ignore
}


notificationControl         XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      NotificationControlIndication
    PROCEDURE CODE          id-notificationControl
    CRITICALITY             ignore
}


activityNotification        XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ActivityNotification
    PROCEDURE CODE          id-activityNotification
    CRITICALITY             ignore
}


privateMessage          XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      PrivateMessage
    PROCEDURE CODE          id-privateMessage
    CRITICALITY             ignore
}

secondaryRATDataUsageReport XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SecondaryRATDataUsageReport
    PROCEDURE CODE          id-secondaryRATDataUsageReport
    CRITICALITY             reject
}

deactivateTrace XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      DeactivateTrace
    PROCEDURE CODE          id-deactivateTrace
    CRITICALITY             ignore
}

traceStart XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      TraceStart
    PROCEDURE CODE          id-traceStart
    CRITICALITY             ignore
}

handoverSuccess         XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      HandoverSuccess
    PROCEDURE CODE          id-handoverSuccess
    CRITICALITY             ignore
}

conditionalHandoverCancel   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ConditionalHandoverCancel
    PROCEDURE CODE          id-conditionalHandoverCancel
    CRITICALITY             ignore
}

earlyStatusTransfer     XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      EarlyStatusTransfer
    PROCEDURE CODE          id-earlyStatusTransfer
    CRITICALITY             ignore
}

failureIndication XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      FailureIndication
    PROCEDURE CODE          id-failureIndication
    CRITICALITY             ignore
}

handoverReport XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      HandoverReport
    PROCEDURE CODE          id-handoverReport
    CRITICALITY             ignore
}

scgFailureIndication XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      SCGFailureIndication
    PROCEDURE CODE          id-scgFailureIndication
    CRITICALITY             ignore
}

resourceStatusReportingInitiation   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              ResourceStatusRequest
    SUCCESSFUL OUTCOME              ResourceStatusResponse
    UNSUCCESSFUL OUTCOME            ResourceStatusFailure
    PROCEDURE CODE                  id-resourceStatusReportingInitiation
    CRITICALITY                     reject
}

resourceStatusReporting XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ResourceStatusUpdate
    PROCEDURE CODE          id-resourceStatusReporting
    CRITICALITY             ignore
}

mobilitySettingsChange  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              MobilityChangeRequest
    SUCCESSFUL OUTCOME              MobilityChangeAcknowledge
    UNSUCCESSFUL OUTCOME            MobilityChangeFailure
    PROCEDURE CODE                  id-mobilitySettingsChange
    CRITICALITY                     reject
}


accessAndMobilityIndication XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      AccessAndMobilityIndication
    PROCEDURE CODE          id-accessAndMobilityIndication
    CRITICALITY             ignore
}

cellTrafficTrace XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      CellTrafficTrace
    PROCEDURE CODE          id-cellTrafficTrace
    CRITICALITY             ignore
}

rANMulticastGroupPaging     XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      RANMulticastGroupPaging
    PROCEDURE CODE          id-RANMulticastGroupPaging
    CRITICALITY             reject
}

scgFailureInformationReport XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      ScgFailureInformationReport
    PROCEDURE CODE          id-scgFailureInformationReport
    CRITICALITY             ignore
}

scgFailureTransfer XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      ScgFailureTransfer
    PROCEDURE CODE          id-scgFailureTransfer
    CRITICALITY             ignore
}

f1CTrafficTransfer          XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      F1CTrafficTransfer
    PROCEDURE CODE          id-f1CTrafficTransfer
    CRITICALITY             reject
}

iABTransportMigrationManagement XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      IABTransportMigrationManagementRequest
    SUCCESSFUL OUTCOME      IABTransportMigrationManagementResponse
    UNSUCCESSFUL OUTCOME    IABTransportMigrationManagementReject
    PROCEDURE CODE          id-iABTransportMigrationManagement
    CRITICALITY             reject
}

iABTransportMigrationModification XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      IABTransportMigrationModificationRequest
    SUCCESSFUL OUTCOME      IABTransportMigrationModificationResponse
    PROCEDURE CODE          id-iABTransportMigrationModification
    CRITICALITY             reject
}

iABResourceCoordination XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      IABResourceCoordinationRequest
    SUCCESSFUL OUTCOME      IABResourceCoordinationResponse
    PROCEDURE CODE          id-iABResourceCoordination
    CRITICALITY             reject
}

retrieveUEContextConfirm    XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      RetrieveUEContextConfirm
    PROCEDURE CODE          id-retrieveUEContextConfirm
    CRITICALITY             reject
}

cPCCancel XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      CPCCancel
    PROCEDURE CODE          id-cPCCancel
    CRITICALITY             ignore
}

rachIndication XNAP-ELEMENTARY-PROCEDURE ::={
    INITIATING MESSAGE      RachIndication
    PROCEDURE CODE          id-rachIndication
    CRITICALITY             ignore
}

dataCollectionReportingInitiation   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              DataCollectionRequest
    SUCCESSFUL OUTCOME              DataCollectionResponse
    UNSUCCESSFUL OUTCOME            DataCollectionFailure
    PROCEDURE CODE                  id-dataCollectionReportingInitiation
    CRITICALITY                     reject
}

dataCollectionReporting XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      DataCollectionUpdate
    PROCEDURE CODE          id-dataCollectionReporting
    CRITICALITY             ignore
}

oDSIB1ConfigurationProvision        XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              ODSIB1ConfigurationProvisionRequest
    SUCCESSFUL OUTCOME              ODSIB1ConfigurationProvisionResponse
    UNSUCCESSFUL OUTCOME            ODSIB1ConfigurationProvisionFailure
    PROCEDURE CODE                  id-ODSIB1ConfigurationProvision
    CRITICALITY                     reject
}

oDSIB1ConfigurationProvisionStatusUpdate    XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      ODSIB1ConfigurationProvisionStatusUpdate
    PROCEDURE CODE          id-ODSIB1ConfigurationProvisionStatusUpdate
    CRITICALITY             ignore
}


lTMConfigurationUpdate      XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              LTMConfigurationUpdate
    SUCCESSFUL OUTCOME              LTMConfigurationUpdateAcknowledge
    UNSUCCESSFUL OUTCOME            LTMConfigurationUpdateFailure
    PROCEDURE CODE                  id-lTMConfigurationUpdate
    CRITICALITY                     reject
}


cSIRSCoordination           XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              CSIRSCoordinationRequest
    SUCCESSFUL OUTCOME              CSIRSCoordinationResponse
    PROCEDURE CODE                  id-cSIRSCoordination
    CRITICALITY                     reject
}


cellSwitchNotification      XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              CellSwitchNotification
    PROCEDURE CODE                  id-cellSwitchNotification
    CRITICALITY                     ignore
}

tAInformationTransfer       XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              TAInformationTransfer
    PROCEDURE CODE                  id-tAInformationTransfer
    CRITICALITY                     ignore
}

lTMCancel                   XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE              LTMCancel
    PROCEDURE CODE                  id-lTMCancel
    CRITICALITY                     ignore
}

cLI-Indication  XNAP-ELEMENTARY-PROCEDURE ::= {
    INITIATING MESSAGE      CLI-Indication
    PROCEDURE CODE          id-cLI-Indication
    CRITICALITY             ignore
}


END
-- ASN1STOP

-- 9.3.4    PDU Definitions
-- ASN1START
-- **************************************************************
--
-- PDU definitions for XnAP.
--
-- **************************************************************

XnAP-PDU-Contents {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN   

-- **************************************************************
--
-- IE parameter types from other modules.
--
-- **************************************************************

IMPORTS

    ActivationIDforCellActivation,
    AMF-Region-Information,
    AMF-UE-NGAP-ID,
    AS-SecurityInformation,
    AssistanceDataForRANPaging,
    AerialUESubscriptionInformation,
    A2XPC5QoSParameters,
    BitRate,
    Cause,
    CellAndCapacityAssistanceInfo-EUTRA,
    CellAndCapacityAssistanceInfo-NR,
    CellAssistanceInfo-EUTRA,
    CellAssistanceInfo-NR,
    CHOinformation-Req,
    CHOinformation-Ack,
    CHOinformation-AddReq,
    CHOinformation-AddReqAck,
    CHOinformation-ModReq,
    CHO-MRDC-EarlyDataForwarding,
    CHO-MRDC-Indicator,
    CPTransportLayerInformation,
    TNLA-To-Add-List,
    TNLA-To-Update-List,
    TNLA-To-Remove-List,
    TNLA-Setup-List,
    TNLA-Failed-To-Setup-List,
    CriticalityDiagnostics,
    XnUAddressInfoperPDUSession-List,
    DAPSResponseInfo-List,
    DataTrafficResourceIndication,
    DeliveryStatus,
    DesiredActNotificationLevel,
    DRB-ID,
    DRB-List,
    DRB-Number,
    DRBsSubjectToDLDiscarding-List,
    DRBsSubjectToEarlyStatusTransfer-List,
    DRBsSubjectToStatusTransfer-List,
    DRBToQoSFlowMapping-List,
    E-UTRA-CGI,
    ExpectedUEActivityBehaviour,
    ExpectedUEBehaviour,
    ExtendedUEIdentityIndexValue,
    FiveGCMobilityRestrictionListContainer,
    GlobalCell-ID,
    GlobalNG-RANNode-ID,
    GlobalNG-RANCell-ID,
    GUAMI,
    InterfaceInstanceIndication,
    I-RNTI,
    Local-NG-RAN-Node-Identifier,
    LocationInformationSNReporting,
    LocationReportingInformation,
    LowerLayerPresenceStatusChange,
    LTEA2XServicesAuthorized,
    LTEUESidelinkAggregateMaximumBitRate,
    LTEV2XServicesAuthorized,
    MR-DC-ResourceCoordinationInfo,
    ServedCells-E-UTRA,
    ServedCells-NR,
    ServedCellsToUpdate-E-UTRA,
    ServedCellsToUpdate-NR,
    MAC-I,
    MaskedIMEISV,
    MDT-Configuration,
    MDTPLMNList,
    MobilityRestrictionList,
    Neighbour-NG-RAN-Node-List,
    NG-RAN-Cell-Identity,
    NG-RANnodeUEXnAPID,
    NR-CGI,
    NE-DC-TDM-Pattern,
    NRA2XServicesAuthorized,
    NRUESidelinkAggregateMaximumBitRate,
    NRV2XServicesAuthorized,
    PagingDRX,
    EUTRAPagingeDRXInformation,
    PagingPriority,
    PartialListIndicator,
    PLMN-Identity,
    PDCPChangeIndication,
    PDUSessionAggregateMaximumBitRate,
    PDUSession-ID,
    PDUSession-List,
    PDUSession-List-withCause,
    PDUSession-List-withDataForwardingFromTarget,
    PDUSession-List-withDataForwardingRequest,
    PDUSessionResourcesAdmitted-List,
    PDUSessionResourcesNotAdmitted-List,
    PDUSessionResourcesToBeSetup-List,
    PDUSessionResourceChangeRequiredInfo-SNterminated,
    PDUSessionResourceChangeRequiredInfo-MNterminated,
    PDUSessionResourceChangeConfirmInfo-SNterminated,
    PDUSessionResourceChangeConfirmInfo-MNterminated,
    PDUSessionResourceSecondaryRATUsageList,
    PDUSessionResourceSetupInfo-SNterminated,
    PDUSessionResourceSetupInfo-MNterminated,
    PDUSessionResourceSetupResponseInfo-SNterminated,
    PDUSessionResourceSetupResponseInfo-MNterminated,
    PDUSessionResourceModificationInfo-SNterminated,
    PDUSessionResourceModificationInfo-MNterminated,
    PDUSessionResourceModificationResponseInfo-SNterminated,
    PDUSessionResourceModificationResponseInfo-MNterminated,
    PDUSessionResourceModConfirmInfo-SNterminated,
    PDUSessionResourceModConfirmInfo-MNterminated,
    PDUSessionResourceModRqdInfo-SNterminated,
    PDUSessionResourceModRqdInfo-MNterminated,
    PDUSessionType,
    PC5QoSParameters,
    QoSFlowIdentifier,
    QoSFlowNotificationControlIndicationInfo,
    QoSFlows-List,
    RANPagingArea,
    ResetRequestTypeInfo,
    ResetResponseTypeInfo,
    RFSP-Index,
    RRCConfigIndication,
    RRCResumeCause,
    SCGConfigurationQuery,
    SCGreconfigNotification,
    SecurityIndication,
    S-NG-RANnode-SecurityKey,
    SpectrumSharingGroupID,
    SplitSRBsTypes,
    S-NG-RANnode-Addition-Trigger-Ind,
    S-NSSAI,
    TargetCellList,
    TAISupport-List,
    Target-CGI,
    TimeToWait,
    TraceActivation,
    UEAggregateMaximumBitRate,
    UEContextID,
    UEContextInfoRetrUECtxtResp,
    UEContextKeptIndicator,
    UEHistoryInformation,
    UEIdentityIndexValue,
    UERadioCapabilityForPaging,
    UERadioCapabilityID,
    UERANPagingIdentity,
    UESecurityCapabilities,
    UPTransportLayerInformation,
    UserPlaneTrafficActivityReport,
    XnBenefitValue,
    RANPagingFailure,
    TNLConfigurationInfo,
    MaximumCellListSize,
    MessageOversizeNotification,
    NG-RANTraceID,
    MobilityInformation,
    InitiatingCondition-FailureIndication,
    HandoverReportType,
    TargetCellinEUTRAN,
    C-RNTI,
    UERLFReportContainer,
    Measurement-ID,
    RegistrationRequest,
    ReportCharacteristics,
    CellToReport,
    ReportingPeriodicity,
    CellMeasurementResult,
    UEHistoryInformationFromTheUE,
    MobilityParametersInformation,
    MobilityParametersModificationRange,
    RAReport,
    IABNodeIndication,
    SNTriggered,
    SCGIndicator,
    UESpecificDRX,
    DirectForwardingPathAvailability,
    TransportLayerAddress,
    PrivacyIndicator,
    URIaddress,
    MBS-Session-ID,
    UEIdentityIndexList-MBSGroupPaging,
    MBS-SessionInformation-List,
    MBS-SessionInformationResponse-List,
    SuccessfulHOReportInformation,
    PSCellHistoryInformationRetrieve,
    SSBOffsets-List,
    NG-RANnode2SSBOffsetsModificationRange,
    Coverage-Modification-List,
    SCGFailureReportContainer,
    SNMobilityInformation,
    PSCellChangeHistory,
    CHOConfiguration,
    SCGUEHistoryInformation,
    F1CTrafficContainer,
    NoPDUSessionIndication,
    IAB-TNL-Address-Request,
    IAB-TNL-Address-Response,
    TrafficIndex,
    TrafficProfile,
    TrafficToBeReleaseInformation,
    F1-TerminatingTopologyBHInformation,
    Non-F1-TerminatingTopologyBHInformation,
    BHInfoList,
    IABTNLAddress,
    IABCellInformation,
    IABTNLAddressException,
    TimeSynchronizationAssistanceInformation,
    SCGActivationRequest,
    SCGActivationStatus,
    CPAInformationRequest,
    CPAInformationAck,
    CPCInformationRequired,
    CPCInformationConfirm,
    CPAInformationModReq,
    CPAInformationModReqAck,
    CPC-DataForwarding-Indicator,
    CPCInformationUpdate,
    CPACInformationModRequired,
    QMCConfigInfo,
    FiveGProSeAuthorized,
    FiveGProSePC5QoSParameters,
    ServedCellSpecificInfoReq-NR,
    NRPagingeDRXInformation,
    NRPagingeDRXInformationforRRCINACTIVE,
    SDTSupportRequest,
    SDT-Termination-Request,
    SDTPartialUEContextInfo,
    SDTDataForwardingDRBList,
    PEIPSassistanceInformation,
    UESliceMaximumBitRateList,
    PagingCause,
    MDTPLMNModificationList,
    F1-terminatingIAB-donorIndicator,
    SRB-ID,
    AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,
    HashedUEIdentityIndexValue,
    MBS-DataForwarding-Indicator,
    IABAuthorizationStatus,
    NID,
    MT-SDT-Information,
    PosPartialUEContextInfo,
    SRSConfiguration,
    RaReportIndicationList,
    SuccessfulPSCellChangeReportInformation,
    CPACConfiguration,
    TimeSinceFailure,
    SPRAvailability,
    DLLBTFailureInformationRequest,
    DLLBTFailureInformationList,
    CellBasedUETrajectoryPrediction,
    DataCollectionID,
    RequestedPredictionTime,
    NodeMeasurementInitiationResult-List,
    CellMeasurementInitiationResult-List,
    UEAssociatedInfoResult-List,
    UETrajectoryCollectionConfiguration,
    UEPerformanceCollectionConfiguration,
    CellMeasurementResultForDataCollection-List,
    CellToReportForDataCollection-List,
    CandidateRelayUEInfoList,
    NRPagingLongeDRXInformationforRRCINACTIVE,
    QMCCoordinationRequest,
    QMCCoordinationResponse,
    DirectForwardingPathAvailabilityWithSourceMN,
    Conditional-Reconfig-List,
    PDUSetbasedHandlingIndicator,
    MobileIAB-AuthorizationStatus,
    BAPAddress,
    S-CPAC-Request,
    SK-COUNTER,
    RegistrationRequestForDataCollection,
    ReportCharacteristicsForDataCollection,
    ReportingPeriodicityForDataCollection,
    NodeAssociatedInfoResult,
    SLPositioning-Ranging-Services-Info,
    PDUSessionsListToBeReleased-UPError,
    UserPlaneFailureIndication,
    SRSPositioningConfigOrActivationRequest,
    NRPPaPositioningInformation,
    LTMHandoverInformationRequest,
    EarlySyncInformationRequest,
    LTMHandoverInformationRequestAcknowledge,
    EarlySyncInformation,
    LTMCellSwitchInformation,
    LTMUEAssociationInformation-List,
    CellSwitchTAInformation-List,
    TAInformation-List,
    LTMUpdatesToCandidateNodeInformation,
    LTMUpdatesToCandidateCellInformation-List,
    LTMUESecurityInformation,
    LTMUpdatesFromCandidateCellInformation-List,
    LTMCandidateCellsToBeCancelled-List,
    CSI-RSCoordinationRequestList,
    CSI-RSCoordinationResultList,
    CLI-MeasurementResult-List,
    SRS-Resource-Indication,
    WAB-MT-ID,
    LPWUSPSassistanceInformation,
    FurtherExtended-UEIdentityIndexValue,
    Future-Coverage-Modification-List,
    TimeSCG-Failure,
    FailureType,
    LTMInformationSCG-AddReq,
    LTMInformationSCG-AddReqAck,
    LTMInformationSCG-ModReq,
    LTMInterSNExecutionNotification,
    LTMPSCellInformation-AddReq,
    LTMPSCellInformation-AddReqAck,
    LTMPSCellInformation-UpdateReq,
    LTMPSCellInformation-UpdateReqAck,
    LTMPSCellInformation-UpdateRequired,
    LTMPSCellInformation-UpdateConfirm,
    LTMPSCellInformation-ChangeRequired,
    LTMPSCellInformation-ChangeConfirm,
    LTM-DC-DataForwarding-Indicator,
    SemipersistentPositioningInformation,
    LP-WUS-Disable-Indication






FROM XnAP-IEs

    PrivateIE-Container{},
    ProtocolExtensionContainer{},
    ProtocolIE-Container{},
    ProtocolIE-ContainerList{},
    ProtocolIE-ContainerPair{},
    ProtocolIE-ContainerPairList{},
    ProtocolIE-Single-Container{},
    XNAP-PRIVATE-IES,
    XNAP-PROTOCOL-EXTENSION,
    XNAP-PROTOCOL-IES,
    XNAP-PROTOCOL-IES-PAIR
FROM XnAP-Containers


    id-A2XPC5QoSParameters,
    id-ActivatedServedCells,
    id-ActivationIDforCellActivation,
    id-AdditionalDRBIDs,
    id-AerialUESubscriptionInformation,
    id-AMF-Region-Information,
    id-AMF-Region-Information-To-Add,
    id-AMF-Region-Information-To-Delete,
    id-AssistanceDataForRANPaging,
    id-AvailableDRBIDs,
    id-Cause,
    id-cellAssistanceInfo-EUTRA,
    id-cellAssistanceInfo-NR,
    id-CellAndCapacityAssistanceInfo-EUTRA,
    id-CellAndCapacityAssistanceInfo-NR,
    id-ConfigurationUpdateInitiatingNodeChoice,
    id-UEContextID,
    id-CriticalityDiagnostics,
    id-XnUAddressInfoperPDUSession-List,
    id-DesiredActNotificationLevel,
    id-DRBsSubjectToStatusTransfer-List,
    id-ExpectedUEBehaviour,
    id-ExtendedUEIdentityIndexValue,
    id-FiveGCMobilityRestrictionListContainer,
    id-GlobalNG-RAN-node-ID,
    id-GUAMI,
    id-indexToRatFrequSelectionPriority,
    id-List-of-served-cells-E-UTRA,
    id-List-of-served-cells-NR,
    id-LocationInformationSN,
    id-LocationInformationSNReporting,
    id-LocationReportingInformation,
    id-LTEA2XServicesAuthorized,
    id-LTEA2XUEPC5AggregateMaximumBitRate,
    id-LTEUESidelinkAggregateMaximumBitRate,
    id-LTEV2XServicesAuthorized,
    id-MAC-I,
    id-MaskedIMEISV,
    id-MDT-Configuration,
    id-MDTPLMNList,
    id-MN-to-SN-Container,
    id-MobilityRestrictionList,
    id-M-NG-RANnodeUEXnAPID,
    id-new-NG-RAN-Cell-Identity,
    id-newNG-RANnodeUEXnAPID,
    id-NRA2XServicesAuthorized,
    id-NRA2XUEPC5AggregateMaximumBitRate,
    id-NRUESidelinkAggregateMaximumBitRate,
    id-NRV2XServicesAuthorized,
    id-oldNG-RANnodeUEXnAPID,
    id-OldtoNewNG-RANnodeResumeContainer,
    id-PagingCause,
    id-PagingDRX,
    id-EUTRAPagingeDRXInformation,
    id-PagingPriority,
    id-PartialListIndicator-EUTRA,
    id-PartialListIndicator-NR,
    id-PCellID,
    id-PDUSessionResourceSecondaryRATUsageList,
    id-PDUSessionResourcesActivityNotifyList,
    id-PDUSessionResourcesAdmitted-List,
    id-PDUSessionResourcesNotAdmitted-List,
    id-PDUSessionResourcesNotifyList,
    id-PDUSessionToBeAddedAddReq,
    id-PDUSessionToBeReleased-RelReqAck,
    id-procedureStage,
    id-RANPagingArea,
    id-requestedSplitSRB,
    id-RequiredNumberOfDRBIDs,
    id-ResetRequestTypeInfo,
    id-ResetResponseTypeInfo,
    id-RespondingNodeTypeConfigUpdateAck,
    id-RRCResumeCause,
    id-SCGreconfigNotification,
    id-selectedPLMN,
    id-ServedCellsToActivate,
    id-servedCellsToUpdate-E-UTRA,
    id-ServedCellsToUpdateInitiatingNodeChoice,
    id-servedCellsToUpdate-NR,
    id-sourceNG-RANnodeUEXnAPID,
    id-SpareDRBIDs,
    id-S-NG-RANnodeMaxIPDataRate-UL,
    id-S-NG-RANnodeMaxIPDataRate-DL,
    id-S-NG-RANnodeUEXnAPID,
    id-TAISupport-list,
    id-Target2SourceNG-RANnodeTranspContainer,
    id-targetCellGlobalID,
    id-targetNG-RANnodeUEXnAPID,
    id-TimeToWait,
    id-TNLA-To-Add-List,
    id-TNLA-To-Update-List,
    id-TNLA-To-Remove-List,
    id-TNLA-Setup-List,
    id-TNLA-Failed-To-Setup-List,
    id-TraceActivation,
    id-UEContextInfoHORequest,
    id-UEContextInfoRetrUECtxtResp,
    id-UEContextKeptIndicator,
    id-UEContextRefAtSN-HORequest,
    id-UEHistoryInformation,
    id-UEIdentityIndexValue,
    id-UERANPagingIdentity,
    id-UESecurityCapabilities,
    id-UserPlaneTrafficActivityReport,
    id-XnRemovalThreshold,
    id-PDUSessionAdmittedAddedAddReqAck,
    id-PDUSessionNotAdmittedAddReqAck,
    id-SN-to-MN-Container,
    id-RRCConfigIndication,
    id-SplitSRB-RRCTransfer,
    id-UEReportRRCTransfer,
    id-PDUSessionReleasedList-RelConf,
    id-BearersSubjectToCounterCheck,
    id-PDUSessionToBeReleasedList-RelRqd,
    id-ResponseInfo-ReconfCompl,
    id-initiatingNodeType-ResourceCoordRequest,
    id-respondingNodeType-ResourceCoordResponse,
    id-PDUSessionToBeReleased-RelReq,
    id-PDUSession-SNChangeRequired-List,
    id-PDUSession-SNChangeConfirm-List,
    id-PDCPChangeIndication,
    id-PC5QoSParameters,
    id-SCGConfigurationQuery,
    id-UEContextInfo-SNModRequest,
    id-requestedSplitSRBrelease,
    id-PDUSessionAdmitted-SNModResponse,
    id-PDUSessionNotAdmitted-SNModResponse,
    id-admittedSplitSRB,
    id-admittedSplitSRBrelease,
    id-PDUSessionAdmittedModSNModConfirm,
    id-PDUSessionReleasedSNModConfirm,
    id-s-ng-RANnode-SecurityKey,
    id-PDUSessionToBeModifiedSNModRequired,
    id-S-NG-RANnodeUE-AMBR,
    id-PDUSessionToBeReleasedSNModRequired,
    id-target-S-NG-RANnodeID,
    id-S-NSSAI,
    id-MR-DC-ResourceCoordinationInfo,
    id-RANPagingFailure,
    id-UERadioCapabilityForPaging,
    id-PDUSessionDataForwarding-SNModResponse,
    id-Secondary-MN-Xn-U-TNLInfoatM,
    id-NE-DC-TDM-Pattern,
    id-InterfaceInstanceIndication,
    id-S-NG-RANnode-Addition-Trigger-Ind,
    id-SNTriggered,
    id-DRBs-transferred-to-MN,
    id-TNLConfigurationInfo,
    id-MessageOversizeNotification,
    id-NG-RANTraceID,
    id-FastMCGRecoveryRRCTransfer-SN-to-MN,
    id-FastMCGRecoveryRRCTransfer-MN-to-SN,
    id-RequestedFastMCGRecoveryViaSRB3,
    id-AvailableFastMCGRecoveryViaSRB3,
    id-RequestedFastMCGRecoveryViaSRB3Release,
    id-ReleaseFastMCGRecoveryViaSRB3,
    id-CHOinformation-Req,
    id-CHOinformation-Ack,
    id-targetCellsToCancel,
    id-requestedTargetCellGlobalID,
    id-DAPSResponseInfo-List,
    id-CHO-MRDC-EarlyDataForwarding,
    id-CHO-MRDC-Indicator,
    id-MobilityInformation,
    id-InitiatingCondition-FailureIndication,
    id-UEHistoryInformationFromTheUE,
    id-HandoverReportType,
    id-HandoverCause,
    id-SourceCellCGI,
    id-TargetCellCGI,
    id-ReEstablishmentCellCGI,
    id-TargetCellinEUTRAN,
    id-SourceCellCRNTI,
    id-UERLFReportContainer,
    id-NGRAN-Node1-Measurement-ID,
    id-NGRAN-Node2-Measurement-ID,
    id-RegistrationRequest,
    id-ReportCharacteristics,
    id-CellToReport,
    id-ReportingPeriodicity,
    id-CellMeasurementResult,
    id-NG-RANnode1CellID,
    id-NG-RANnode2CellID,
    id-NG-RANnode1MobilityParameters,
    id-NG-RANnode2ProposedMobilityParameters,
    id-MobilityParametersModificationRange,
    id-RAReport,
    id-IABNodeIndication,
    id-UERadioCapabilityID,
    id-SCGIndicator,
    id-UESpecificDRX,
    id-PDUSessionExpectedUEActivityBehaviour,
    id-DirectForwardingPathAvailability,
    id-SourceNG-RAN-node-ID,
    id-TargetNodeID,
    id-ManagementBasedMDTPLMNList,
    id-PrivacyIndicator,
    id-TraceCollectionEntityIPAddress,
    id-TraceCollectionEntityURI,
    id-MBS-Session-ID,
    id-UEIdentityIndexList-MBSGroupPaging,
    id-MulticastRANPagingArea,
    id-MBS-SessionInformation-List,
    id-MBS-SessionInformationResponse-List,
    id-SuccessfulHOReportInformation,
    id-PSCellHistoryInformationRetrieve,
    id-SSBOffsets-List,
    id-NG-RANnode2SSBOffsetsModificationRange,
    id-Coverage-Modification-List,
    id-SourcePSCellCGI,
    id-FailedPSCellCGI,
    id-SCGFailureReportContainer,
    id-SNMobilityInformation,
    id-SourcePSCellID,
    id-SuitablePSCellCGI,
    id-PSCellChangeHistory,
    id-CHOConfiguration,
    id-SCGUEHistoryInformation,
    id-F1CTrafficContainer,
    id-NoPDUSessionIndication,
    id-F1-Terminating-IAB-DonorUEXnAPID,
    id-nonF1-Terminating-IAB-DonorUEXnAPID,
    id-IAB-TNL-Address-Request,
    id-IAB-TNL-Address-Response,
    id-TrafficToBeAddedList,
    id-TrafficToBeModifiedList,
    id-TrafficToBeReleaseInformation,
    id-TrafficAddedList,
    id-TrafficModifiedList,
    id-TrafficNotAddedList,
    id-TrafficNotModifiedList,
    id-TrafficRequiredToBeModifiedList,
    id-TrafficRequiredModifiedList,
    id-TrafficReleasedList,
    id-IABTNLAddressToBeAdded,
    id-IABTNLAddressToBeReleasedList,
    id-BoundaryNodeCellsList,
    id-ParentNodeCellsList,
    id-IABTNLAddressException,
    id-CHOinformation-AddReq,
    id-CHOinformation-AddReqAck,
    id-CHOinformation-ModReq,
    id-TimeSynchronizationAssistanceInformation,
    id-SCGActivationRequest,
    id-SCGActivationStatus,
    id-CPAInformationRequest,
    id-CPAInformationAck,
    id-CPCInformationRequired,
    id-CPCInformationConfirm,
    id-CPAInformationModReq,
    id-CPAInformationModReqAck,
    id-CPC-DataForwarding-Indicator,
    id-CPCInformationUpdate,
    id-CPACInformationModRequired,
    id-QMCConfigInfo,
    id-Local-NG-RAN-Node-Identifier,
    id-Neighbour-NG-RAN-Node-List,
    id-Local-NG-RAN-Node-Identifier-Removal,
    id-FiveGProSeAuthorized,
    id-FiveGProSePC5QoSParameters,
    id-FiveGProSeUEPC5AggregateMaximumBitRate,
    id-ServedCellSpecificInfoReq-NR,
    id-NRPagingeDRXInformation,
    id-NRPagingeDRXInformationforRRCINACTIVE,
    id-SDTSupportRequest,
    id-SDT-SRB-between-NewNode-OldNode,
    id-SDT-Termination-Request,
    id-SDTPartialUEContextInfo,
    id-SDTDataForwardingDRBList,
    id-PEIPSassistanceInformation,
    id-UESliceMaximumBitRateList,
    id-S-NG-RANnodeUE-Slice-MBR,
    id-ManagementBasedMDTPLMNModificationList,
    id-F1-terminatingIAB-donorIndicator,
    id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,
    id-HashedUEIdentityIndexValue,
    id-MBS-DataForwarding-Indicator,
    id-IABAuthorizationStatus,
    id-SelectedNID,
    id-MT-SDT-Information,
    id-PosPartialUEContextInfo,
    id-SRSConfiguration,
    id-RaReportIndicationList,
    id-SuccessfulPSCellChangeReportInformation,
    id-CPACConfiguration,
    id-TargetCellCRNTI,
    id-TimeSinceFailure,
    id-SPRAvailability,
    id-DLLBTFailureInformationRequest,
    id-DLLBTFailureInformationList,
    id-CellBasedUETrajectoryPrediction,
    id-DataCollectionID,
    id-RequestedPredictionTime,
    id-NodeMeasurementInitiationResult-List,
    id-CellMeasurementInitiationResult-List,
    id-UEAssociatedInfoResult-List,
    id-UETrajectoryCollectionConfiguration,
    id-UEPerformanceCollectionConfiguration,
    id-CellMeasurementResultForDataCollection-List,
    id-CellToReportForDataCollection-List,
    id-CandidateRelayUEInfoList,
    id-NRCellsAndSSBsList,
    id-ActivatedNRCellsAndSSBsList,
    id-NRPagingLongeDRXInformationforRRCINACTIVE,
    id-QMCCoordinationRequest,
    id-QMCCoordinationResponse,
    id-QoE-Measurement-Results,
    id-Src-SN-to-Tgt-SNQMCInfoInquiry,
    id-DirectForwardingPathAvailabilityWithSourceMN,
    id-accessed-PSCellID,
    id-conditional-Reconfig-ToCancel-List,
    id-PDUSetbasedHandlingIndicator,
    id-MobileIAB-AuthorizationStatus,
    id-MIAB-MT-BAP-Address,
    id-S-CPAC-Request,
    id-sk-Counter,
    id-Source-M-NG-RANnodeID,
    id-SourceSN-to-TargetSN-QMCInfo,
    id-RegistrationRequestForDataCollection,
    id-ReportCharacteristicsForDataCollection,
    id-ReportingPeriodicityForDataCollection,
    id-NodeAssociatedInfoResult,
    id-SLPositioning-Ranging-Services-Info,
    id-PDUSessionsListToBeReleased-UPError,
    id-UserPlaneFailureIndication,
    id-SRSPositioningConfigOrActivationRequest,
    id-NRPPaPositioningInformation,
    id-RequestType,
    id-NES-Cell-ID,
    id-Cell-A-ID,
    id-ProvisionStatus,
    id-LTMHandoverInformationRequest,
    id-EarlySyncInformationRequest,
    id-LTMHandoverInformationRequestAcknowledge,
    id-EarlySyncInformationResponse,
    id-LTMCellSwitchInformation,
    id-LTMUEAssociationInformation-List,
    id-CellSwitchTAInformation-List,
    id-TAInformation-List,
    id-LTMUpdatesToCandidateNodeInformation,
    id-LTMUpdatesToCandidateCellInformation-List,
    id-LTMUESecurityInformation,
    id-NGRAN-Node1-ID,
    id-NGRAN-Node2-ID,
    id-LTMUpdatesFromCandidateCellInformation-List,
    id-LTMCandidateCellsToBeCancelled-List,
    id-CSI-RSCoordinationRequest,
    id-CSI-RSCoordinationResponse,
    id-CLI-MeasurementResult-List,
    id-SRS-Resource-Indication,
    id-WAB-MT-ID,
    id-LPWUSPSassistanceInformation,
    id-FurtherExtended-UEIdentityIndexValue,
    id-Future-Coverage-Modification-List,
    id-GeographicalArea,
    id-TimeSCG-Failure,
    id-FailureType,
    id-LTMInformationSCG-AddReq,
    id-LTMInformationSCG-AddReqAck,
    id-LTMInformationSCG-ModReq,
    id-LTMInterSNExecutionNotification,
    id-LTMPSCellInformation-AddReq,
    id-LTMPSCellInformation-AddReqAck,
    id-LTMPSCellInformation-UpdateReq,
    id-LTMPSCellInformation-UpdateReqAck,
    id-LTMPSCellInformation-UpdateRequired,
    id-LTMPSCellInformation-UpdateConfirm,
    id-LTMPSCellInformation-ChangeRequired,
    id-LTMPSCellInformation-ChangeConfirm,
    id-LTM-DC-DataForwarding-Indicator,
    id-SemipersistentPositioningInformation,
    id-LP-WUS-Disable-Indication,
    id-ContinuousMDT,





    maxnoofCellsinNG-RANnode,
    maxnoofDRBs,
    maxnoofPDUSessions,
    maxnoofQoSFlows,
    maxnoofServedCellsIAB,
    maxnoofTrafficIndexEntries,
    maxnoofTLAsIAB,
    maxnoofBAPControlPDURLCCHs,
    maxnoofServingCells,
    maxnoofSSBAreas

FROM XnAP-Constants;

-- **************************************************************
--
-- HANDOVER REQUEST
--
-- **************************************************************

HandoverRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{HandoverRequest-IEs}},
    ...
}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-Cause                                       CRITICALITY reject  TYPE Cause                                                  PRESENCE mandatory}|
    { ID id-targetCellGlobalID                          CRITICALITY reject  TYPE Target-CGI                                             PRESENCE mandatory}|
    { ID id-GUAMI                                       CRITICALITY reject  TYPE GUAMI                                                  PRESENCE mandatory}|
    { ID id-UEContextInfoHORequest                      CRITICALITY reject  TYPE UEContextInfoHORequest                             PRESENCE mandatory}|
    { ID id-TraceActivation                             CRITICALITY ignore  TYPE TraceActivation                                        PRESENCE optional }|
    { ID id-MaskedIMEISV                                CRITICALITY ignore  TYPE MaskedIMEISV                                           PRESENCE optional }|
    { ID id-UEHistoryInformation                        CRITICALITY ignore  TYPE UEHistoryInformation                               PRESENCE mandatory}|
    { ID id-UEContextRefAtSN-HORequest                  CRITICALITY ignore  TYPE UEContextRefAtSN-HORequest                     PRESENCE optional }|
    { ID id-CHOinformation-Req                          CRITICALITY reject  TYPE CHOinformation-Req                                 PRESENCE optional }|
    { ID id-NRV2XServicesAuthorized                     CRITICALITY ignore  TYPE NRV2XServicesAuthorized                            PRESENCE optional }|
    { ID id-LTEV2XServicesAuthorized                    CRITICALITY ignore  TYPE LTEV2XServicesAuthorized                           PRESENCE optional }|
    { ID id-PC5QoSParameters                            CRITICALITY ignore  TYPE PC5QoSParameters                                       PRESENCE optional }|
    { ID id-MobilityInformation                         CRITICALITY ignore  TYPE MobilityInformation                                    PRESENCE optional }|
    { ID id-UEHistoryInformationFromTheUE               CRITICALITY ignore  TYPE UEHistoryInformationFromTheUE                  PRESENCE optional }|
    { ID id-IABNodeIndication                           CRITICALITY reject  TYPE IABNodeIndication                                  PRESENCE optional }|
    { ID id-NoPDUSessionIndication                      CRITICALITY ignore  TYPE NoPDUSessionIndication                             PRESENCE optional }|
    { ID id-TimeSynchronizationAssistanceInformation    CRITICALITY ignore  TYPE TimeSynchronizationAssistanceInformation       PRESENCE optional }|
    { ID id-QMCConfigInfo                               CRITICALITY ignore  TYPE QMCConfigInfo                                          PRESENCE optional }|
    { ID id-FiveGProSeAuthorized                        CRITICALITY ignore  TYPE FiveGProSeAuthorized                               PRESENCE optional }|
    { ID id-FiveGProSePC5QoSParameters                  CRITICALITY ignore  TYPE FiveGProSePC5QoSParameters                     PRESENCE optional }|
    { ID id-IABAuthorizationStatus                      CRITICALITY ignore  TYPE IABAuthorizationStatus                             PRESENCE optional }|
    { ID id-DLLBTFailureInformationRequest              CRITICALITY ignore  TYPE DLLBTFailureInformationRequest                 PRESENCE optional }|
    { ID id-AerialUESubscriptionInformation             CRITICALITY ignore  TYPE AerialUESubscriptionInformation                    PRESENCE optional }|
    { ID id-NRA2XServicesAuthorized                     CRITICALITY ignore  TYPE NRA2XServicesAuthorized                            PRESENCE optional }|
    { ID id-LTEA2XServicesAuthorized                    CRITICALITY ignore  TYPE LTEA2XServicesAuthorized                           PRESENCE optional }|
    { ID id-A2XPC5QoSParameters                         CRITICALITY ignore  TYPE A2XPC5QoSParameters                                    PRESENCE optional }|
    { ID id-CellBasedUETrajectoryPrediction             CRITICALITY ignore  TYPE CellBasedUETrajectoryPrediction                    PRESENCE optional }|
    { ID id-DataCollectionID                            CRITICALITY ignore  TYPE DataCollectionID                                       PRESENCE optional }|
    { ID id-CandidateRelayUEInfoList                    CRITICALITY reject  TYPE CandidateRelayUEInfoList                           PRESENCE optional }|
    { ID id-SourceSN-to-TargetSN-QMCInfo                CRITICALITY ignore  TYPE QMCConfigInfo                                          PRESENCE optional }|
    { ID id-MobileIAB-AuthorizationStatus               CRITICALITY reject  TYPE MobileIAB-AuthorizationStatus                  PRESENCE optional }|
    { ID id-SLPositioning-Ranging-Services-Info         CRITICALITY ignore  TYPE SLPositioning-Ranging-Services-Info            PRESENCE optional }|
    { ID id-LTMHandoverInformationRequest               CRITICALITY reject  TYPE LTMHandoverInformationRequest                  PRESENCE optional}|
    { ID id-EarlySyncInformationRequest                 CRITICALITY ignore  TYPE EarlySyncInformationRequest                        PRESENCE optional}|
    { ID id-ContinuousMDT                               CRITICALITY ignore  TYPE NG-RANTraceID                                          PRESENCE optional },
    ...
}

UEContextInfoHORequest ::= SEQUENCE {
    ng-c-UE-reference                       AMF-UE-NGAP-ID,
    cp-TNL-info-source                      CPTransportLayerInformation,
    ueSecurityCapabilities                  UESecurityCapabilities,
    securityInformation                     AS-SecurityInformation,
    indexToRatFrequencySelectionPriority    RFSP-Index                                                          OPTIONAL,
    ue-AMBR                                 UEAggregateMaximumBitRate,
    pduSessionResourcesToBeSetup-List       PDUSessionResourcesToBeSetup-List,
    rrc-Context                             OCTET STRING,
    locationReportingInformation            LocationReportingInformation                                        OPTIONAL,
    mrl                                     MobilityRestrictionList                                             OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { {UEContextInfoHORequest-ExtIEs} }  OPTIONAL,
    ...
}

UEContextInfoHORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-FiveGCMobilityRestrictionListContainer  CRITICALITY ignore  EXTENSION FiveGCMobilityRestrictionListContainer        PRESENCE optional }|
    { ID id-NRUESidelinkAggregateMaximumBitRate     CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-LTEUESidelinkAggregateMaximumBitRate    CRITICALITY ignore  EXTENSION LTEUESidelinkAggregateMaximumBitRate          PRESENCE optional }|
    { ID id-MDTPLMNList                             CRITICALITY ignore  EXTENSION MDTPLMNList                                           PRESENCE optional }|
    { ID id-UERadioCapabilityID                     CRITICALITY reject  EXTENSION UERadioCapabilityID                               PRESENCE optional }|
    { ID id-MBS-SessionInformation-List             CRITICALITY ignore  EXTENSION MBS-SessionInformation-List                       PRESENCE optional }|
    { ID id-FiveGProSeUEPC5AggregateMaximumBitRate  CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-UESliceMaximumBitRateList               CRITICALITY ignore  EXTENSION UESliceMaximumBitRateList                     PRESENCE optional }|
    { ID id-NRA2XUEPC5AggregateMaximumBitRate       CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-LTEA2XUEPC5AggregateMaximumBitRate      CRITICALITY ignore  EXTENSION LTEUESidelinkAggregateMaximumBitRate          PRESENCE optional },
    ...
}

UEContextRefAtSN-HORequest ::= SEQUENCE {
    globalNG-RANNode-ID             GlobalNG-RANNode-ID,
    sN-NG-RANnodeUEXnAPID           NG-RANnodeUEXnAPID,
    iE-Extensions                   ProtocolExtensionContainer { {UEContextRefAtSN-HORequest-ExtIEs} }  OPTIONAL,
    ...
}

UEContextRefAtSN-HORequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

-- **************************************************************
--
-- HANDOVER REQUEST ACKNOWLEDGE
--
-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{HandoverRequestAcknowledge-IEs}},
    ...
}

HandoverRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY ignore  TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY ignore  TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-PDUSessionResourcesAdmitted-List            CRITICALITY ignore  TYPE PDUSessionResourcesAdmitted-List               PRESENCE mandatory}|
    { ID id-PDUSessionResourcesNotAdmitted-List         CRITICALITY ignore  TYPE PDUSessionResourcesNotAdmitted-List            PRESENCE optional }|
    { ID id-Target2SourceNG-RANnodeTranspContainer      CRITICALITY ignore  TYPE OCTET STRING                                           PRESENCE mandatory}|
    { ID id-UEContextKeptIndicator                      CRITICALITY ignore  TYPE UEContextKeptIndicator                             PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore  TYPE CriticalityDiagnostics                             PRESENCE optional }|
    { ID id-DRBs-transferred-to-MN                      CRITICALITY ignore  TYPE DRB-List                                               PRESENCE optional }|
    { ID id-DAPSResponseInfo-List                       CRITICALITY reject  TYPE DAPSResponseInfo-List                              PRESENCE optional }|
    { ID id-CHOinformation-Ack                          CRITICALITY reject  TYPE CHOinformation-Ack                                 PRESENCE optional }|
    { ID id-MBS-SessionInformationResponse-List         CRITICALITY ignore  TYPE MBS-SessionInformationResponse-List            PRESENCE optional }|
    { ID id-RRCConfigIndication                         CRITICALITY ignore  TYPE RRCConfigIndication                                    PRESENCE optional }|
    { ID id-PDUSetbasedHandlingIndicator                CRITICALITY ignore  TYPE PDUSetbasedHandlingIndicator                       PRESENCE optional }|
    { ID id-LTMHandoverInformationRequestAcknowledge    CRITICALITY ignore  TYPE LTMHandoverInformationRequestAcknowledge       PRESENCE optional }|
    { ID id-EarlySyncInformationResponse                CRITICALITY ignore  TYPE EarlySyncInformation                               PRESENCE optional },
    ...
}

-- **************************************************************
--
-- HANDOVER PREPARATION FAILURE
--
-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{HandoverPreparationFailure-IEs}},
    ...
}

HandoverPreparationFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                CRITICALITY ignore  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore  TYPE Cause                                      PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore  TYPE CriticalityDiagnostics                 PRESENCE optional }|
    { ID id-requestedTargetCellGlobalID             CRITICALITY reject  TYPE Target-CGI                                 PRESENCE optional },
    ...
}

-- **************************************************************
--
-- SN STATUS TRANSFER
--
-- **************************************************************

SNStatusTransfer ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{SNStatusTransfer-IEs}},
    ...
}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-DRBsSubjectToStatusTransfer-List        CRITICALITY ignore      TYPE DRBsSubjectToStatusTransfer-List       PRESENCE mandatory}|
    { ID id-CHOConfiguration                            CRITICALITY ignore      TYPE CHOConfiguration                               PRESENCE optional }|
    { ID id-MobilityInformation                     CRITICALITY ignore      TYPE MobilityInformation                            PRESENCE optional },
    ...
}

-- **************************************************************
--
-- UE CONTEXT RELEASE
--
-- **************************************************************

UEContextRelease ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{UEContextRelease-IEs}},
    ...
}

UEContextRelease-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- HANDOVER CANCEL
--
-- **************************************************************

HandoverCancel ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{HandoverCancel-IEs}},
    ...
}

HandoverCancel-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE optional }|
    { ID id-Cause                                       CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-targetCellsToCancel                         CRITICALITY reject      TYPE TargetCellList                             PRESENCE optional },
    ...
}

-- **************************************************************
--
-- HANDOVER SUCCESS
--
-- **************************************************************

HandoverSuccess ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{HandoverSuccess-IEs}},
    ...
}

HandoverSuccess-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-requestedTargetCellGlobalID                 CRITICALITY reject      TYPE Target-CGI                                     PRESENCE mandatory}|
    { ID id-accessed-PSCellID                           CRITICALITY ignore      TYPE NR-CGI                                         PRESENCE optional },
    ...
}

-- **************************************************************
--
-- CONDITIONAL HANDOVER CANCEL
--
-- **************************************************************

ConditionalHandoverCancel ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ ConditionalHandoverCancel-IEs}},
    ...
}

ConditionalHandoverCancel-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                       CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-targetCellsToCancel                         CRITICALITY reject      TYPE TargetCellList                             PRESENCE optional }|
    { ID id-conditional-Reconfig-ToCancel-List          CRITICALITY reject      TYPE Conditional-Reconfig-List              PRESENCE optional },
    ...
}

-- **************************************************************
--
-- EARLY STATUS TRANSFER
--
-- **************************************************************

EarlyStatusTransfer ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ EarlyStatusTransfer-IEs}},
    ...
}

EarlyStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-procedureStage                              CRITICALITY reject      TYPE ProcedureStageChoice                       PRESENCE mandatory},
    ...
}

ProcedureStageChoice ::= CHOICE {
    first-dl-count                      FirstDLCount,
    dl-discarding                       DLDiscarding,
    choice-extension                    ProtocolIE-Single-Container { {ProcedureStageChoice-ExtIEs} }
}

ProcedureStageChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

FirstDLCount ::= SEQUENCE {
    dRBsSubjectToEarlyStatusTransfer            DRBsSubjectToEarlyStatusTransfer-List,
    iE-Extension                                ProtocolExtensionContainer { {FirstDLCount-ExtIEs} }    OPTIONAL,
    ...
}

FirstDLCount-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DLDiscarding ::= SEQUENCE {
    dRBsSubjectToDLDiscarding                   DRBsSubjectToDLDiscarding-List,
    iE-Extension                                ProtocolExtensionContainer { {DLDiscarding-ExtIEs} }    OPTIONAL,
    ...
}

DLDiscarding-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- RAN PAGING
--
-- **************************************************************

RANPaging ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{RANPaging-IEs}},
    ...
}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-UEIdentityIndexValue                        CRITICALITY reject  TYPE UEIdentityIndexValue                               PRESENCE mandatory}|
    { ID id-UERANPagingIdentity                         CRITICALITY ignore  TYPE UERANPagingIdentity                                    PRESENCE mandatory}|
    { ID id-PagingDRX                                   CRITICALITY ignore  TYPE PagingDRX                                              PRESENCE mandatory}|
    { ID id-RANPagingArea                               CRITICALITY reject  TYPE RANPagingArea                                          PRESENCE mandatory}|
    { ID id-PagingPriority                              CRITICALITY ignore  TYPE PagingPriority                                         PRESENCE optional }|
    { ID id-AssistanceDataForRANPaging                  CRITICALITY ignore  TYPE AssistanceDataForRANPaging                     PRESENCE optional }|
    { ID id-UERadioCapabilityForPaging                  CRITICALITY ignore  TYPE UERadioCapabilityForPaging                     PRESENCE optional }|
    { ID id-ExtendedUEIdentityIndexValue                CRITICALITY ignore  TYPE ExtendedUEIdentityIndexValue                       PRESENCE optional }|
    { ID id-EUTRAPagingeDRXInformation                  CRITICALITY ignore  TYPE EUTRAPagingeDRXInformation                     PRESENCE optional }|
    { ID id-UESpecificDRX                               CRITICALITY ignore  TYPE UESpecificDRX                                          PRESENCE optional }|
    { ID id-NRPagingeDRXInformation                     CRITICALITY ignore  TYPE NRPagingeDRXInformation                            PRESENCE optional }|
    { ID id-NRPagingeDRXInformationforRRCINACTIVE       CRITICALITY ignore  TYPE NRPagingeDRXInformationforRRCINACTIVE          PRESENCE optional }|
    { ID id-PagingCause                                 CRITICALITY ignore  TYPE PagingCause                                            PRESENCE optional }|
    { ID id-PEIPSassistanceInformation                  CRITICALITY ignore  TYPE PEIPSassistanceInformation                     PRESENCE optional }|
    { ID id-HashedUEIdentityIndexValue                  CRITICALITY ignore  TYPE HashedUEIdentityIndexValue                     PRESENCE optional }|
    { ID id-MT-SDT-Information                          CRITICALITY ignore  TYPE MT-SDT-Information                                 PRESENCE optional }|
    { ID id-NRPagingLongeDRXInformationforRRCINACTIVE   CRITICALITY ignore  TYPE NRPagingLongeDRXInformationforRRCINACTIVE  PRESENCE optional }|
    { ID id-LPWUSPSassistanceInformation                CRITICALITY ignore  TYPE LPWUSPSassistanceInformation                       PRESENCE optional }|
    { ID id-FurtherExtended-UEIdentityIndexValue        CRITICALITY ignore  TYPE FurtherExtended-UEIdentityIndexValue           PRESENCE optional }|
    { ID id-LP-WUS-Disable-Indication                   CRITICALITY ignore  TYPE LP-WUS-Disable-Indication                          PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RETRIEVE UE CONTEXT REQUEST
--
-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{RetrieveUEContextRequest-IEs}},
    ...
}

RetrieveUEContextRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-UEContextID                             CRITICALITY reject      TYPE UEContextID                                    PRESENCE mandatory}|
    { ID id-MAC-I                                   CRITICALITY reject      TYPE MAC-I                                          PRESENCE mandatory}|
    { ID id-new-NG-RAN-Cell-Identity                CRITICALITY reject      TYPE NG-RAN-Cell-Identity                       PRESENCE mandatory}|
    { ID id-RRCResumeCause                          CRITICALITY ignore      TYPE RRCResumeCause                                 PRESENCE optional }|
    { ID id-SDTSupportRequest                       CRITICALITY ignore      TYPE SDTSupportRequest                          PRESENCE optional }|
    { ID id-SRSPositioningConfigOrActivationRequest CRITICALITY ignore  TYPE SRSPositioningConfigOrActivationRequest    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RETRIEVE UE CONTEXT RESPONSE
--
-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ RetrieveUEContextResponse-IEs}},
    ...
}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-newNG-RANnodeUEXnAPID                       CRITICALITY ignore  TYPE NG-RANnodeUEXnAPID                                     PRESENCE mandatory}|
    { ID id-oldNG-RANnodeUEXnAPID                       CRITICALITY ignore  TYPE NG-RANnodeUEXnAPID                                     PRESENCE mandatory}|
    { ID id-GUAMI                                       CRITICALITY reject  TYPE GUAMI                                                      PRESENCE mandatory}|
    { ID id-UEContextInfoRetrUECtxtResp                 CRITICALITY reject  TYPE UEContextInfoRetrUECtxtResp                            PRESENCE mandatory}|
    { ID id-TraceActivation                             CRITICALITY ignore  TYPE TraceActivation                                            PRESENCE optional }|
    { ID id-MaskedIMEISV                                CRITICALITY ignore  TYPE MaskedIMEISV                                               PRESENCE optional }|
    { ID id-LocationReportingInformation                CRITICALITY ignore  TYPE LocationReportingInformation                           PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore  TYPE CriticalityDiagnostics                                 PRESENCE optional }|
    { ID id-NRV2XServicesAuthorized                     CRITICALITY ignore  TYPE NRV2XServicesAuthorized                                PRESENCE optional }|
    { ID id-LTEV2XServicesAuthorized                    CRITICALITY ignore  TYPE LTEV2XServicesAuthorized                               PRESENCE optional }|
    { ID id-PC5QoSParameters                            CRITICALITY ignore  TYPE PC5QoSParameters                                           PRESENCE optional }|
    { ID id-UEHistoryInformation                        CRITICALITY ignore  TYPE UEHistoryInformation                                   PRESENCE optional }|
    { ID id-UEHistoryInformationFromTheUE               CRITICALITY ignore  TYPE UEHistoryInformationFromTheUE                      PRESENCE optional }|
    { ID id-MDTPLMNList                                 CRITICALITY ignore  TYPE MDTPLMNList                                                PRESENCE optional }|
    { ID id-IABNodeIndication                           CRITICALITY reject  TYPE IABNodeIndication                                      PRESENCE optional }|
    { ID id-UEContextRefAtSN-HORequest                  CRITICALITY ignore  TYPE UEContextRefAtSN-HORequest                         PRESENCE optional }|
    { ID id-TimeSynchronizationAssistanceInformation    CRITICALITY ignore  TYPE TimeSynchronizationAssistanceInformation           PRESENCE optional }|
    { ID id-QMCConfigInfo                               CRITICALITY ignore  TYPE QMCConfigInfo                                              PRESENCE optional }|
    { ID id-FiveGProSeAuthorized                        CRITICALITY ignore  TYPE FiveGProSeAuthorized                                   PRESENCE optional }|
    { ID id-FiveGProSePC5QoSParameters                  CRITICALITY ignore  TYPE FiveGProSePC5QoSParameters                         PRESENCE optional }|
    { ID id-AerialUESubscriptionInformation             CRITICALITY ignore  TYPE AerialUESubscriptionInformation                        PRESENCE optional }|
    { ID id-NRA2XServicesAuthorized                     CRITICALITY ignore  TYPE NRA2XServicesAuthorized                                PRESENCE optional }|
    { ID id-LTEA2XServicesAuthorized                    CRITICALITY ignore  TYPE LTEA2XServicesAuthorized                               PRESENCE optional }|
    { ID id-A2XPC5QoSParameters                         CRITICALITY ignore  TYPE A2XPC5QoSParameters                                        PRESENCE optional }|
    { ID id-MobileIAB-AuthorizationStatus               CRITICALITY reject  TYPE MobileIAB-AuthorizationStatus                      PRESENCE optional }|
    { ID id-SLPositioning-Ranging-Services-Info         CRITICALITY ignore  TYPE SLPositioning-Ranging-Services-Info                PRESENCE optional }|
    { ID id-ContinuousMDT                               CRITICALITY ignore  TYPE NG-RANTraceID                                              PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RETRIEVE UE CONTEXT CONFIRM
--
-- **************************************************************

RetrieveUEContextConfirm ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{RetrieveUEContextConfirm-IEs}},
    ...
}
RetrieveUEContextConfirm-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-UEContextKeptIndicator                  CRITICALITY ignore      TYPE UEContextKeptIndicator                     PRESENCE optional }|
    { ID id-SDT-Termination-Request                 CRITICALITY ignore      TYPE SDT-Termination-Request                    PRESENCE optional },
    ...
}



-- **************************************************************
--
-- RETRIEVE UE CONTEXT FAILURE
--
-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ RetrieveUEContextFailure-IEs}},
    ...
}

RetrieveUEContextFailure-IEs XNAP-PROTOCOL-IES ::= {    
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-OldtoNewNG-RANnodeResumeContainer       CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-SemipersistentPositioningInformation    CRITICALITY ignore      TYPE SemipersistentPositioningInformation   PRESENCE optional },
    ...
}

-- **************************************************************
--
-- XN-U ADDRESS INDICATION
--
-- **************************************************************

XnUAddressIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnUAddressIndication-IEs}},
    ...
}

XnUAddressIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-XnUAddressInfoperPDUSession-List        CRITICALITY reject      TYPE XnUAddressInfoperPDUSession-List           PRESENCE mandatory}|
    { ID id-CHO-MRDC-Indicator                      CRITICALITY reject      TYPE CHO-MRDC-Indicator                             PRESENCE optional }|
    { ID id-CHO-MRDC-EarlyDataForwarding            CRITICALITY ignore      TYPE CHO-MRDC-EarlyDataForwarding                   PRESENCE optional }|
    { ID id-CPC-DataForwarding-Indicator            CRITICALITY reject      TYPE CPC-DataForwarding-Indicator                   PRESENCE optional }|
    { ID id-MBS-DataForwarding-Indicator            CRITICALITY ignore      TYPE MBS-DataForwarding-Indicator                   PRESENCE optional }|
    { ID id-MBS-SessionInformationResponse-List     CRITICALITY ignore      TYPE MBS-SessionInformationResponse-List        PRESENCE optional }|
    { ID id-PDUSetbasedHandlingIndicator            CRITICALITY ignore      TYPE PDUSetbasedHandlingIndicator                   PRESENCE optional }|
    { ID id-LTM-DC-DataForwarding-Indicator         CRITICALITY reject      TYPE LTM-DC-DataForwarding-Indicator                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE ADDITION REQUEST
--
-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeAdditionRequest-IEs}},
    ...
}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-UESecurityCapabilities              CRITICALITY reject      TYPE UESecurityCapabilities                     PRESENCE mandatory}|
    { ID id-s-ng-RANnode-SecurityKey            CRITICALITY reject      TYPE S-NG-RANnode-SecurityKey                   PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUE-AMBR                 CRITICALITY reject      TYPE UEAggregateMaximumBitRate                  PRESENCE mandatory}|
    { ID id-selectedPLMN                        CRITICALITY ignore      TYPE PLMN-Identity                                  PRESENCE optional }|
    { ID id-MobilityRestrictionList             CRITICALITY ignore      TYPE MobilityRestrictionList                        PRESENCE optional }|
    { ID id-indexToRatFrequSelectionPriority    CRITICALITY reject      TYPE RFSP-Index                                     PRESENCE optional }|
    { ID id-PDUSessionToBeAddedAddReq           CRITICALITY reject      TYPE PDUSessionToBeAddedAddReq                  PRESENCE mandatory}|
    { ID id-MN-to-SN-Container                  CRITICALITY reject      TYPE OCTET STRING                                   PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE optional }|
    { ID id-ExpectedUEBehaviour                 CRITICALITY ignore      TYPE ExpectedUEBehaviour                            PRESENCE optional }|
    { ID id-requestedSplitSRB                   CRITICALITY reject      TYPE SplitSRBsTypes                                 PRESENCE optional }|
    { ID id-PCellID                             CRITICALITY reject      TYPE GlobalNG-RANCell-ID                            PRESENCE optional }|
    { ID id-DesiredActNotificationLevel         CRITICALITY ignore      TYPE DesiredActNotificationLevel                PRESENCE optional }|
    { ID id-AvailableDRBIDs                     CRITICALITY reject      TYPE DRB-List                                       PRESENCE conditional}
-- This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. --|
    { ID id-S-NG-RANnodeMaxIPDataRate-UL        CRITICALITY reject      TYPE BitRate                                        PRESENCE optional }|
    { ID id-S-NG-RANnodeMaxIPDataRate-DL        CRITICALITY reject      TYPE BitRate                                        PRESENCE optional }|
    { ID id-LocationInformationSNReporting      CRITICALITY ignore      TYPE LocationInformationSNReporting         PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo      CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo         PRESENCE optional }|
    { ID id-MaskedIMEISV                        CRITICALITY ignore      TYPE MaskedIMEISV                                   PRESENCE optional }|
    { ID id-NE-DC-TDM-Pattern                   CRITICALITY ignore      TYPE NE-DC-TDM-Pattern                              PRESENCE optional }|
    { ID id-S-NG-RANnode-Addition-Trigger-Ind   CRITICALITY reject      TYPE S-NG-RANnode-Addition-Trigger-Ind      PRESENCE optional }|
    { ID id-TraceActivation                     CRITICALITY ignore      TYPE TraceActivation                                PRESENCE optional }|
    { ID id-RequestedFastMCGRecoveryViaSRB3     CRITICALITY ignore      TYPE RequestedFastMCGRecoveryViaSRB3            PRESENCE optional }|
    { ID id-UERadioCapabilityID                 CRITICALITY reject      TYPE UERadioCapabilityID                            PRESENCE optional }|
    { ID id-SourceNG-RAN-node-ID                CRITICALITY ignore      TYPE GlobalNG-RANNode-ID                            PRESENCE optional }|
    { ID id-ManagementBasedMDTPLMNList          CRITICALITY ignore      TYPE MDTPLMNList                                    PRESENCE optional }|
    { ID id-UEHistoryInformation                CRITICALITY ignore      TYPE UEHistoryInformation                           PRESENCE optional }|
    { ID id-UEHistoryInformationFromTheUE       CRITICALITY ignore      TYPE UEHistoryInformationFromTheUE              PRESENCE optional }|
    { ID id-PSCellChangeHistory                 CRITICALITY ignore      TYPE PSCellChangeHistory                            PRESENCE optional }|
    { ID id-IABNodeIndication                   CRITICALITY reject      TYPE IABNodeIndication                              PRESENCE optional }|
    { ID id-NoPDUSessionIndication              CRITICALITY ignore      TYPE NoPDUSessionIndication                     PRESENCE optional }|
    { ID id-CHOinformation-AddReq               CRITICALITY reject      TYPE CHOinformation-AddReq                      PRESENCE optional }|
    { ID id-SCGActivationRequest                CRITICALITY ignore      TYPE SCGActivationRequest                           PRESENCE optional }|
    { ID id-CPAInformationRequest               CRITICALITY reject      TYPE CPAInformationRequest                      PRESENCE optional }|
    { ID id-S-NG-RANnodeUE-Slice-MBR            CRITICALITY reject      TYPE UESliceMaximumBitRateList                  PRESENCE optional }|
    { ID id-F1-terminatingIAB-donorIndicator    CRITICALITY reject      TYPE F1-terminatingIAB-donorIndicator           PRESENCE optional }|
    { ID id-SelectedNID                         CRITICALITY ignore      TYPE NID                                            PRESENCE optional }|
    { ID id-QMCCoordinationRequest              CRITICALITY ignore      TYPE QMCCoordinationRequest                     PRESENCE optional }|
    { ID id-SourceSN-to-TargetSN-QMCInfo        CRITICALITY ignore      TYPE QMCConfigInfo                                  PRESENCE optional }|
    { ID id-IABAuthorizationStatus              CRITICALITY ignore      TYPE IABAuthorizationStatus                     PRESENCE optional }|
    { ID id-Source-M-NG-RANnodeID               CRITICALITY ignore      TYPE GlobalNG-RANNode-ID                            PRESENCE optional }|
    { ID id-DataCollectionID                    CRITICALITY ignore      TYPE DataCollectionID                               PRESENCE optional }|
    { ID id-LTMPSCellInformation-AddReq         CRITICALITY reject      TYPE LTMPSCellInformation-AddReq                PRESENCE optional }|
    { ID id-LTMInformationSCG-AddReq            CRITICALITY reject      TYPE LTMInformationSCG-AddReq                   PRESENCE optional },
    ...
}

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    s-NSSAI                 S-NSSAI,
    sN-PDUSessionAMBR       PDUSessionAggregateMaximumBitRate           OPTIONAL,
    sn-terminated           PDUSessionResourceSetupInfo-SNterminated    OPTIONAL,
    mn-terminated           PDUSessionResourceSetupInfo-MNterminated    OPTIONAL,
-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE
-- nor the PDU Session Resource Setup Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.1.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************
--
-- S-NODE ADDITION REQUEST ACKNOWLEDGE
--
-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeAdditionRequestAcknowledge-IEs}},
    ...
}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSessionAdmittedAddedAddReqAck    CRITICALITY ignore      TYPE PDUSessionAdmittedAddedAddReqAck       PRESENCE mandatory}|
    { ID id-PDUSessionNotAdmittedAddReqAck      CRITICALITY ignore      TYPE PDUSessionNotAdmittedAddReqAck     PRESENCE optional }|
    { ID id-SN-to-MN-Container                  CRITICALITY reject      TYPE OCTET STRING                               PRESENCE mandatory}|
    { ID id-admittedSplitSRB                    CRITICALITY reject      TYPE SplitSRBsTypes                             PRESENCE optional }|
    { ID id-RRCConfigIndication                 CRITICALITY reject      TYPE RRCConfigIndication                        PRESENCE optional }|
    { ID id-CriticalityDiagnostics              CRITICALITY ignore      TYPE CriticalityDiagnostics                 PRESENCE optional }|
    { ID id-LocationInformationSN               CRITICALITY ignore      TYPE Target-CGI                                 PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo      CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo     PRESENCE optional }|
    { ID id-AvailableFastMCGRecoveryViaSRB3     CRITICALITY ignore      TYPE AvailableFastMCGRecoveryViaSRB3        PRESENCE optional }|
    { ID id-DirectForwardingPathAvailability    CRITICALITY ignore      TYPE DirectForwardingPathAvailability       PRESENCE optional }|
    { ID id-SCGActivationStatus                 CRITICALITY ignore      TYPE SCGActivationStatus                        PRESENCE optional }|
    { ID id-CPAInformationAck                   CRITICALITY ignore      TYPE CPAInformationAck                          PRESENCE optional }|
    { ID id-SNMobilityInformation               CRITICALITY ignore      TYPE SNMobilityInformation                  PRESENCE optional }|
    { ID id-QMCCoordinationResponse             CRITICALITY ignore      TYPE QMCCoordinationResponse                    PRESENCE optional }|
    { ID id-CHOinformation-AddReqAck            CRITICALITY reject      TYPE CHOinformation-AddReqAck               PRESENCE optional   }|
    { ID id-DirectForwardingPathAvailabilityWithSourceMN CRITICALITY ignore TYPE DirectForwardingPathAvailabilityWithSourceMN   PRESENCE optional }|
    { ID id-PDUSetbasedHandlingIndicator        CRITICALITY ignore      TYPE PDUSetbasedHandlingIndicator           PRESENCE optional }|
    { ID id-LTMPSCellInformation-AddReqAck      CRITICALITY reject      TYPE LTMPSCellInformation-AddReqAck     PRESENCE optional   }|
    { ID id-LTMInformationSCG-AddReqAck         CRITICALITY ignore      TYPE LTMInformationSCG-AddReqAck            PRESENCE optional   },
    ...
}

PDUSessionAdmittedAddedAddReqAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedAddedAddReqAck-Item

PDUSessionAdmittedAddedAddReqAck-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    sn-terminated           PDUSessionResourceSetupResponseInfo-SNterminated    OPTIONAL,
    mn-terminated           PDUSessionResourceSetupResponseInfo-MNterminated    OPTIONAL,
-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE
-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.1.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionNotAdmittedAddReqAck ::= SEQUENCE {
    pduSessionResourcesNotAdmitted-SNterminated     PDUSessionResourcesNotAdmitted-List OPTIONAL,
    pduSessionResourcesNotAdmitted-MNterminated     PDUSessionResourcesNotAdmitted-List OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionNotAdmittedAddReqAck-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionNotAdmittedAddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AvailableFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************
--
-- S-NODE ADDITION REQUEST REJECT
--
-- **************************************************************

SNodeAdditionRequestReject ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeAdditionRequestReject-IEs}},
    ...
}

SNodeAdditionRequestReject-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-PCellID                                 CRITICALITY reject      TYPE GlobalNG-RANCell-ID                            PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE RECONFIGURATION COMPLETE
--
-- **************************************************************

SNodeReconfigurationComplete ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReconfigurationComplete-IEs}},
    ...
}

SNodeReconfigurationComplete-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-ResponseInfo-ReconfCompl                CRITICALITY ignore      TYPE ResponseInfo-ReconfCompl                   PRESENCE mandatory},
    ...
}

ResponseInfo-ReconfCompl ::= SEQUENCE {
    responseType-ReconfComplete     ResponseType-ReconfComplete,
    iE-Extensions                       ProtocolExtensionContainer { {ResponseInfo-ReconfCompl-ExtIEs} } OPTIONAL,
    ...
}

ResponseInfo-ReconfCompl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ResponseType-ReconfComplete ::= CHOICE {
    configuration-successfully-applied          Configuration-successfully-applied,
    configuration-rejected-by-M-NG-RANNode      Configuration-rejected-by-M-NG-RANNode,
    choice-extension                    ProtocolIE-Single-Container { {ResponseType-ReconfComplete-ExtIEs} }
}

ResponseType-ReconfComplete-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

Configuration-successfully-applied ::= SEQUENCE {
    m-NG-RANNode-to-S-NG-RANNode-Container      OCTET STRING        OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {Configuration-successfully-applied-ExtIEs} } OPTIONAL,
    ...
}

Configuration-successfully-applied-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-sk-Counter  CRITICALITY ignore  EXTENSION SK-COUNTER        PRESENCE optional},
    ...
}

Configuration-rejected-by-M-NG-RANNode ::= SEQUENCE {
    cause                                           Cause,
    m-NG-RANNode-to-S-NG-RANNode-Container      OCTET STRING        OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {Configuration-rejected-by-M-NG-RANNode-ExtIEs} } OPTIONAL,
    ...
}

Configuration-rejected-by-M-NG-RANNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE MODIFICATION REQUEST
--
-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationRequest-IEs}},
    ...
}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                              PRESENCE mandatory}|
    { ID id-PDCPChangeIndication                    CRITICALITY ignore      TYPE PDCPChangeIndication                           PRESENCE optional }|
    { ID id-selectedPLMN                            CRITICALITY ignore      TYPE PLMN-Identity                                      PRESENCE optional }|
    { ID id-MobilityRestrictionList                 CRITICALITY ignore      TYPE MobilityRestrictionList                        PRESENCE optional }|
    { ID id-SCGConfigurationQuery                   CRITICALITY ignore      TYPE SCGConfigurationQuery                          PRESENCE optional }|
    { ID id-UEContextInfo-SNModRequest              CRITICALITY reject      TYPE UEContextInfo-SNModRequest                 PRESENCE optional }|
    { ID id-MN-to-SN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                       PRESENCE optional }|
    { ID id-requestedSplitSRB                       CRITICALITY ignore      TYPE SplitSRBsTypes                                     PRESENCE optional }|
    { ID id-requestedSplitSRBrelease                CRITICALITY ignore      TYPE SplitSRBsTypes                                     PRESENCE optional }|
    { ID id-DesiredActNotificationLevel             CRITICALITY ignore      TYPE DesiredActNotificationLevel                    PRESENCE optional }|
    { ID id-AdditionalDRBIDs                        CRITICALITY reject      TYPE DRB-List                                           PRESENCE optional }|
    { ID id-S-NG-RANnodeMaxIPDataRate-UL            CRITICALITY reject      TYPE BitRate                                            PRESENCE optional }|
    { ID id-S-NG-RANnodeMaxIPDataRate-DL            CRITICALITY reject      TYPE BitRate                                            PRESENCE optional }|
    { ID id-LocationInformationSNReporting          CRITICALITY ignore      TYPE LocationInformationSNReporting             PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo          CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo             PRESENCE optional }|
    { ID id-PCellID                                 CRITICALITY reject      TYPE GlobalNG-RANCell-ID                                PRESENCE optional }|
    { ID id-NE-DC-TDM-Pattern                       CRITICALITY ignore      TYPE NE-DC-TDM-Pattern                              PRESENCE optional }|
    { ID id-RequestedFastMCGRecoveryViaSRB3         CRITICALITY ignore      TYPE RequestedFastMCGRecoveryViaSRB3                PRESENCE optional }|
    { ID id-RequestedFastMCGRecoveryViaSRB3Release  CRITICALITY ignore      TYPE RequestedFastMCGRecoveryViaSRB3Release PRESENCE optional }|
    { ID id-SNTriggered                             CRITICALITY ignore      TYPE SNTriggered                                        PRESENCE optional }|
    { ID id-TargetNodeID                            CRITICALITY ignore      TYPE GlobalNG-RANNode-ID                                PRESENCE optional }|
    { ID id-PSCellHistoryInformationRetrieve        CRITICALITY ignore      TYPE PSCellHistoryInformationRetrieve           PRESENCE optional }|
    { ID id-UEHistoryInformationFromTheUE           CRITICALITY ignore      TYPE UEHistoryInformationFromTheUE              PRESENCE optional }|
    { ID id-CHOinformation-ModReq                   CRITICALITY ignore      TYPE CHOinformation-ModReq                          PRESENCE optional }|
    { ID id-SCGActivationRequest                    CRITICALITY ignore      TYPE SCGActivationRequest                           PRESENCE optional }|
    { ID id-CPAInformationModReq                    CRITICALITY ignore      TYPE CPAInformationModReq                           PRESENCE optional }|
    { ID id-CPCInformationUpdate                    CRITICALITY ignore      TYPE CPCInformationUpdate                           PRESENCE optional }|
    { ID id-S-NG-RANnodeUE-Slice-MBR                CRITICALITY ignore      TYPE UESliceMaximumBitRateList                      PRESENCE optional }|
    { ID id-ManagementBasedMDTPLMNModificationList  CRITICALITY ignore      TYPE MDTPLMNModificationList                        PRESENCE optional }|
    { ID id-SelectedNID                             CRITICALITY ignore      TYPE NID                                                PRESENCE optional }|
    { ID id-QMCCoordinationRequest                  CRITICALITY ignore      TYPE QMCCoordinationRequest                         PRESENCE optional }|
    { ID id-Src-SN-to-Tgt-SNQMCInfoInquiry          CRITICALITY ignore      TYPE Src-SN-to-Tgt-SNQMCInfoInquiry             PRESENCE optional }|
    { ID id-IABAuthorizationStatus                  CRITICALITY ignore  TYPE IABAuthorizationStatus                         PRESENCE optional }|
    { ID id-LTMPSCellInformation-UpdateReq          CRITICALITY ignore  TYPE LTMPSCellInformation-UpdateReq             PRESENCE optional }|
    { ID id-LTMInformationSCG-ModReq                CRITICALITY ignore      TYPE LTMInformationSCG-ModReq                       PRESENCE optional }|
    { ID id-LTMInterSNExecutionNotification             CRITICALITY reject  TYPE LTMInterSNExecutionNotification                PRESENCE optional },
    ...
}

UEContextInfo-SNModRequest ::= SEQUENCE {
    ueSecurityCapabilities                          UESecurityCapabilities                              OPTIONAL,
    s-ng-RANnode-SecurityKey                        S-NG-RANnode-SecurityKey                            OPTIONAL,
    s-ng-RANnodeUE-AMBR                             UEAggregateMaximumBitRate                           OPTIONAL,
    indexToRatFrequencySelectionPriority            RFSP-Index                                          OPTIONAL,
    lowerLayerPresenceStatusChange                  LowerLayerPresenceStatusChange                      OPTIONAL,
    pduSessionResourceToBeAdded                     PDUSessionsToBeAdded-SNModRequest-List              OPTIONAL,
    pduSessionResourceToBeModified                  PDUSessionsToBeModified-SNModRequest-List           OPTIONAL,
    pduSessionResourceToBeReleased                  PDUSessionsToBeReleased-SNModRequest-List           OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {UEContextInfo-SNModRequest-ExtIEs} }          OPTIONAL,
    ...
}

UEContextInfo-SNModRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionsToBeAdded-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeAdded-SNModRequest-Item

PDUSessionsToBeAdded-SNModRequest-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    s-NSSAI                 S-NSSAI,
    sN-PDUSessionAMBR       PDUSessionAggregateMaximumBitRate           OPTIONAL,
    sn-terminated           PDUSessionResourceSetupInfo-SNterminated    OPTIONAL,
    mn-terminated           PDUSessionResourceSetupInfo-MNterminated    OPTIONAL,
-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE
-- nor the PDU Session Resource Setup Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.3.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionsToBeAdded-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-PDUSessionExpectedUEActivityBehaviour        CRITICALITY ignore  EXTENSION ExpectedUEActivityBehaviour       PRESENCE optional},
    ...
}

PDUSessionsToBeModified-SNModRequest-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionsToBeModified-SNModRequest-Item

PDUSessionsToBeModified-SNModRequest-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    sN-PDUSessionAMBR       PDUSessionAggregateMaximumBitRate               OPTIONAL,
    sn-terminated           PDUSessionResourceModificationInfo-SNterminated OPTIONAL,
    mn-terminated           PDUSessionResourceModificationInfo-MNterminated OPTIONAL,
-- NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE
-- nor the PDU Session Resource Modification Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.3.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionsToBeModified-SNModRequest-Item-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionsToBeModified-SNModRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-S-NSSAI                                     CRITICALITY reject  EXTENSION S-NSSAI                                   PRESENCE optional }|
    { ID id-PDUSessionExpectedUEActivityBehaviour       CRITICALITY ignore  EXTENSION ExpectedUEActivityBehaviour       PRESENCE optional }|
    { ID id-UserPlaneFailureIndication                  CRITICALITY ignore  EXTENSION UserPlaneFailureIndication            PRESENCE optional },
    ...
}

PDUSessionsToBeReleased-SNModRequest-List ::= SEQUENCE {
    pdu-session-list        PDUSession-List-withCause               OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionsToBeReleased-SNModRequest-List-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionsToBeReleased-SNModRequest-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RequestedFastMCGRecoveryViaSRB3Release ::= ENUMERATED {true, ...}

Src-SN-to-Tgt-SNQMCInfoInquiry  ::= ENUMERATED {true, ...}

-- **************************************************************
--
-- S-NODE MODIFICATION REQUEST ACKNOWLEDGE
--
-- **************************************************************

SNodeModificationRequestAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationRequestAcknowledge-IEs}},
    ...
}

SNodeModificationRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-PDUSessionAdmitted-SNModResponse        CRITICALITY ignore      TYPE PDUSessionAdmitted-SNModResponse               PRESENCE optional }|
    { ID id-PDUSessionNotAdmitted-SNModResponse     CRITICALITY ignore      TYPE PDUSessionNotAdmitted-SNModResponse            PRESENCE optional }|
    { ID id-SN-to-MN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                           PRESENCE optional }|
    { ID id-admittedSplitSRB                        CRITICALITY ignore      TYPE SplitSRBsTypes                                         PRESENCE optional }|
    { ID id-admittedSplitSRBrelease                 CRITICALITY ignore      TYPE SplitSRBsTypes                                         PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                             PRESENCE optional }|
    { ID id-LocationInformationSN                   CRITICALITY ignore      TYPE Target-CGI                                             PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo          CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo                 PRESENCE optional }|
    { ID id-PDUSessionDataForwarding-SNModResponse  CRITICALITY ignore      TYPE PDUSessionDataForwarding-SNModResponse     PRESENCE optional }|
    { ID id-RRCConfigIndication                     CRITICALITY reject      TYPE RRCConfigIndication                                    PRESENCE optional }|
    { ID id-AvailableFastMCGRecoveryViaSRB3         CRITICALITY ignore      TYPE AvailableFastMCGRecoveryViaSRB3                    PRESENCE optional }|
    { ID id-ReleaseFastMCGRecoveryViaSRB3           CRITICALITY ignore      TYPE ReleaseFastMCGRecoveryViaSRB3                  PRESENCE optional }|
    { ID id-DirectForwardingPathAvailability        CRITICALITY ignore      TYPE DirectForwardingPathAvailability               PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                            PRESENCE optional }|
    { ID id-SCGActivationStatus                     CRITICALITY ignore      TYPE SCGActivationStatus                                    PRESENCE optional }|
    { ID id-CPAInformationModReqAck                 CRITICALITY ignore      TYPE CPAInformationModReqAck                            PRESENCE optional }|
    { ID id-QMCCoordinationResponse                 CRITICALITY ignore      TYPE QMCCoordinationResponse                            PRESENCE optional }|
    { ID id-SourceSN-to-TargetSN-QMCInfo            CRITICALITY ignore      TYPE QMCConfigInfo                                          PRESENCE optional }|
    { ID id-PDUSetbasedHandlingIndicator            CRITICALITY ignore      TYPE PDUSetbasedHandlingIndicator                       PRESENCE optional }|
    { ID id-LTMPSCellInformation-UpdateReqAck       CRITICALITY ignore      TYPE LTMPSCellInformation-UpdateReqAck              PRESENCE optional },
    ...
}
PDUSessionAdmitted-SNModResponse ::= SEQUENCE {
    pduSessionResourcesAdmittedToBeAdded            PDUSessionAdmittedToBeAddedSNModResponse            OPTIONAL,
    pduSessionResourcesAdmittedToBeModified         PDUSessionAdmittedToBeModifiedSNModResponse     OPTIONAL,
    pduSessionResourcesAdmittedToBeReleased         PDUSessionAdmittedToBeReleasedSNModResponse         OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmitted-SNModResponse-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionAdmittedToBeAddedSNModResponse ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeAddedSNModResponse-Item
PDUSessionAdmittedToBeAddedSNModResponse-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    sn-terminated           PDUSessionResourceSetupResponseInfo-SNterminated    OPTIONAL,
    mn-terminated           PDUSessionResourceSetupResponseInfo-MNterminated    OPTIONAL,
-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE
-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.3.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionAdmittedToBeAddedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionAdmittedToBeModifiedSNModResponse::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedToBeModifiedSNModResponse-Item
PDUSessionAdmittedToBeModifiedSNModResponse-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    sn-terminated           PDUSessionResourceModificationResponseInfo-SNterminated OPTIONAL,
    mn-terminated           PDUSessionResourceModificationResponseInfo-MNterminated OPTIONAL,
-- NOTE: If neither the PDU Session Resource Modification Response Info – SN terminated IE
-- nor the PDU Session Resource Modification Response Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.3.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionAdmittedToBeModifiedSNModResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionAdmittedToBeReleasedSNModResponse ::= SEQUENCE {
    sn-terminated           PDUSession-List-withDataForwardingRequest       OPTIONAL,
    mn-terminated           PDUSession-List-withCause                       OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionAdmittedToBeReleasedSNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionNotAdmitted-SNModResponse ::= SEQUENCE {
    pdu-Session-List        PDUSession-List OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionNotAdmitted-SNModResponse-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionNotAdmitted-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-PDUSessionResourcesNotAdmitted-List     CRITICALITY ignore      EXTENSION PDUSessionResourcesNotAdmitted-List   PRESENCE optional },
    ...
}


PDUSessionDataForwarding-SNModResponse ::= SEQUENCE {
    sn-terminated       PDUSession-List-withDataForwardingRequest,
    iE-Extensions       ProtocolExtensionContainer { {PDUSessionDataForwarding-SNModResponse-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionDataForwarding-SNModResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ReleaseFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}



-- **************************************************************
--
-- S-NODE MODIFICATION REQUEST REJECT
--
-- **************************************************************

SNodeModificationRequestReject ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationRequestReject-IEs}},
    ...
}

SNodeModificationRequestReject-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE MODIFICATION REQUIRED
--
-- **************************************************************

SNodeModificationRequired ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationRequired-IEs}},
    ...
}

SNodeModificationRequired-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-PDCPChangeIndication                    CRITICALITY ignore      TYPE PDCPChangeIndication                       PRESENCE optional }|
    { ID id-PDUSessionToBeModifiedSNModRequired     CRITICALITY ignore      TYPE PDUSessionToBeModifiedSNModRequired    PRESENCE optional }|
    { ID id-PDUSessionToBeReleasedSNModRequired     CRITICALITY ignore      TYPE PDUSessionToBeReleasedSNModRequired    PRESENCE optional }|
    { ID id-SN-to-MN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-SpareDRBIDs                             CRITICALITY ignore      TYPE DRB-List                                       PRESENCE optional }|
    { ID id-RequiredNumberOfDRBIDs                  CRITICALITY ignore      TYPE DRB-Number                                     PRESENCE optional }|
    { ID id-LocationInformationSN                   CRITICALITY ignore      TYPE Target-CGI                                     PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo          CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo         PRESENCE optional }|
    { ID id-RRCConfigIndication                     CRITICALITY reject      TYPE RRCConfigIndication                            PRESENCE optional }|
    { ID id-AvailableFastMCGRecoveryViaSRB3         CRITICALITY ignore      TYPE AvailableFastMCGRecoveryViaSRB3            PRESENCE optional }|
    { ID id-ReleaseFastMCGRecoveryViaSRB3           CRITICALITY ignore      TYPE ReleaseFastMCGRecoveryViaSRB3          PRESENCE optional }|
    { ID id-SCGIndicator                            CRITICALITY ignore      TYPE SCGIndicator                                   PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                    PRESENCE optional }|
    { ID id-SCGActivationRequest                    CRITICALITY ignore      TYPE SCGActivationRequest                       PRESENCE optional }|
    { ID id-CPACInformationModRequired              CRITICALITY ignore      TYPE CPACInformationModRequired             PRESENCE optional }|
    { ID id-SCGreconfigNotification                 CRITICALITY ignore      TYPE SCGreconfigNotification                    PRESENCE optional }|
    { ID id-SPRAvailability                         CRITICALITY ignore      TYPE SPRAvailability                                PRESENCE optional }|
    { ID id-QMCCoordinationRequest                  CRITICALITY ignore      TYPE QMCCoordinationRequest                     PRESENCE optional }|
    { ID id-S-CPAC-Request                          CRITICALITY reject      TYPE S-CPAC-Request                                 PRESENCE    optional }|
    { ID id-PDUSessionsListToBeReleased-UPError     CRITICALITY ignore      TYPE PDUSessionsListToBeReleased-UPError    PRESENCE optional }|
    { ID id-LTMPSCellInformation-UpdateRequired     CRITICALITY ignore      TYPE LTMPSCellInformation-UpdateRequired    PRESENCE optional },
    ...
}
PDUSessionToBeModifiedSNModRequired::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF  PDUSessionToBeModifiedSNModRequired-Item

PDUSessionToBeModifiedSNModRequired-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    sn-terminated           PDUSessionResourceModRqdInfo-SNterminated   OPTIONAL,
    mn-terminated           PDUSessionResourceModRqdInfo-MNterminated   OPTIONAL,
-- NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE
-- nor the PDU Session Resource Modification Required Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.4.4 apply.
    iE-Extension        ProtocolExtensionContainer { {PDUSessionToBeModifiedSNModRequired-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionToBeModifiedSNModRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionToBeReleasedSNModRequired ::= SEQUENCE {
    sn-terminated           PDUSession-List-withDataForwardingRequest       OPTIONAL,
    mn-terminated           PDUSession-List-withCause                       OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionToBeReleasedSNModRequired-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionToBeReleasedSNModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- S-NODE MODIFICATION CONFIRM
--
-- **************************************************************

SNodeModificationConfirm ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationConfirm-IEs}},
    ...
}

SNodeModificationConfirm-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSessionAdmittedModSNModConfirm       CRITICALITY ignore      TYPE PDUSessionAdmittedModSNModConfirm      PRESENCE optional }|
    { ID id-PDUSessionReleasedSNModConfirm          CRITICALITY ignore      TYPE PDUSessionReleasedSNModConfirm         PRESENCE optional }|
    { ID id-MN-to-SN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-AdditionalDRBIDs                        CRITICALITY reject      TYPE DRB-List                                       PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-MR-DC-ResourceCoordinationInfo          CRITICALITY ignore      TYPE MR-DC-ResourceCoordinationInfo         PRESENCE optional }|
    { ID id-QMCCoordinationResponse                 CRITICALITY ignore      TYPE QMCCoordinationResponse                    PRESENCE optional }|
    { ID id-LTMPSCellInformation-UpdateConfirm      CRITICALITY ignore      TYPE LTMPSCellInformation-UpdateConfirm     PRESENCE optional },
    ...
}

PDUSessionAdmittedModSNModConfirm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedModSNModConfirm-Item

PDUSessionAdmittedModSNModConfirm-Item ::= SEQUENCE {
    pduSessionId                PDUSession-ID,
    sn-terminated           PDUSessionResourceModConfirmInfo-SNterminated   OPTIONAL,
    mn-terminated           PDUSessionResourceModConfirmInfo-MNterminated   OPTIONAL,
-- NOTE: If neither the PDU Session Resource Modification Confirm Info – SN terminated IE
-- nor the PDU Session Resource Modification Confirm Info – MN terminated IE is present,
-- abnormal conditions as specified in clause 8.3.4.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedModSNModConfirm-Item-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionAdmittedModSNModConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PDUSessionReleasedSNModConfirm ::= SEQUENCE {
    sn-terminated           PDUSession-List-withDataForwardingFromTarget                        OPTIONAL,
    mn-terminated           PDUSession-List                                                     OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionAdmittedToBeReleasedSNModConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE MODIFICATION REFUSE
--
-- **************************************************************

SNodeModificationRefuse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeModificationRefuse-IEs}},
    ...
}

SNodeModificationRefuse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-MN-to-SN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE RELEASE REQUEST
--
-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReleaseRequest-IEs}},
    ...
}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE optional }|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-PDUSessionToBeReleased-RelReq           CRITICALITY ignore      TYPE PDUSession-List-withCause                  PRESENCE optional }|
    { ID id-UEContextKeptIndicator                  CRITICALITY ignore      TYPE UEContextKeptIndicator                     PRESENCE optional }|
    { ID id-MN-to-SN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-DRBs-transferred-to-MN                  CRITICALITY ignore      TYPE DRB-List                                       PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE RELEASE REQUEST ACKNOWLEDGE
--
-- **************************************************************

SNodeReleaseRequestAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReleaseRequestAcknowledge-IEs}},
    ...
}

SNodeReleaseRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                                 PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                                 PRESENCE optional }|
    { ID id-PDUSessionToBeReleased-RelReqAck            CRITICALITY ignore      TYPE PDUSessionToBeReleasedList-RelReqAck           PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore      TYPE CriticalityDiagnostics                         PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                     CRITICALITY ignore      TYPE SCGUEHistoryInformation                            PRESENCE optional }|
    { ID id-SNMobilityInformation                       CRITICALITY ignore      TYPE SNMobilityInformation                              PRESENCE optional },
    ...
}

PDUSessionToBeReleasedList-RelReqAck ::= SEQUENCE {
    pduSessionsToBeReleasedList-SNterminated        PDUSession-List-withDataForwardingRequest                                               OPTIONAL,
    iE-Extensions                                   ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelReqAck-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionToBeReleasedList-RelReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- S-NODE RELEASE REJECT
--
-- **************************************************************

SNodeReleaseReject ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReleaseReject-IEs}},
    ...
}

SNodeReleaseReject-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE optional }|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}

-- **************************************************************
--
-- S-NODE RELEASE REQUIRED
--
-- **************************************************************

SNodeReleaseRequired ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReleaseRequired-IEs}},
    ...
}

SNodeReleaseRequired-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSessionToBeReleasedList-RelRqd       CRITICALITY ignore      TYPE PDUSessionToBeReleasedList-RelRqd      PRESENCE optional }|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-SN-to-MN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                    PRESENCE optional }|
    { ID id-PDUSessionsListToBeReleased-UPError     CRITICALITY ignore      TYPE PDUSessionsListToBeReleased-UPError    PRESENCE optional },
    ...
}

PDUSessionToBeReleasedList-RelRqd ::= SEQUENCE {
    pduSessionsToBeReleasedList-SNterminated        PDUSession-List-withDataForwardingRequest                       OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionToBeReleasedList-RelRqd-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionToBeReleasedList-RelRqd-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE RELEASE CONFIRM
--
-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeReleaseConfirm-IEs}},
    ...
}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSessionReleasedList-RelConf          CRITICALITY ignore      TYPE PDUSessionReleasedList-RelConf         PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                    PRESENCE optional },
    ...
}

PDUSessionReleasedList-RelConf ::= SEQUENCE {
    pduSessionsReleasedList-SNterminated        PDUSession-List-withDataForwardingFromTarget                    OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE COUNTER CHECK REQUEST
--
-- **************************************************************

SNodeCounterCheckRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeCounterCheckRequest-IEs}},
    ...
}

SNodeCounterCheckRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-BearersSubjectToCounterCheck            CRITICALITY ignore      TYPE BearersSubjectToCounterCheck-List      PRESENCE mandatory},
    ...
}

BearersSubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF BearersSubjectToCounterCheck-Item

BearersSubjectToCounterCheck-Item ::= SEQUENCE {
    drb-ID                          DRB-ID,
    ul-count                        INTEGER (0.. 4294967295),
    dl-count                        INTEGER (0.. 4294967295),
    iE-Extensions                   ProtocolExtensionContainer { {BearersSubjectToCounterCheck-Item-ExtIEs} }   OPTIONAL,
    ...
}

BearersSubjectToCounterCheck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE CHANGE REQUIRED
--
-- **************************************************************

SNodeChangeRequired ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeChangeRequired-IEs}},
    ...
}

SNodeChangeRequired-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-target-S-NG-RANnodeID                   CRITICALITY reject      TYPE GlobalNG-RANNode-ID                            PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-PDUSession-SNChangeRequired-List        CRITICALITY ignore      TYPE PDUSession-SNChangeRequired-List       PRESENCE optional }|
    { ID id-SN-to-MN-Container                      CRITICALITY reject      TYPE OCTET STRING                                   PRESENCE mandatory}|
    { ID id-SCGUEHistoryInformation                 CRITICALITY ignore      TYPE SCGUEHistoryInformation                    PRESENCE optional }|
    { ID id-SNMobilityInformation                   CRITICALITY ignore      TYPE SNMobilityInformation                      PRESENCE optional }|
    { ID id-SourcePSCellID                          CRITICALITY ignore      TYPE GlobalNG-RANCell-ID                            PRESENCE optional }|
    { ID id-CPCInformationRequired                  CRITICALITY ignore      TYPE CPCInformationRequired                     PRESENCE optional }|
    { ID id-SourceSN-to-TargetSN-QMCInfo            CRITICALITY ignore      TYPE QMCConfigInfo                                  PRESENCE optional }|
    { ID id-LTMPSCellInformation-ChangeRequired     CRITICALITY reject      TYPE LTMPSCellInformation-ChangeRequired    PRESENCE optional },
    ...
}

PDUSession-SNChangeRequired-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeRequired-Item

PDUSession-SNChangeRequired-Item ::= SEQUENCE {
    pduSessionId                PDUSession-ID,
    sn-terminated           PDUSessionResourceChangeRequiredInfo-SNterminated   OPTIONAL,
    mn-terminated           PDUSessionResourceChangeRequiredInfo-MNterminated   OPTIONAL,
-- NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present,
-- abnormal conditions as specified in clause 8.3.5.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSession-SNChangeRequired-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSession-SNChangeRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- S-NODE CHANGE CONFIRM
--
-- **************************************************************

SNodeChangeConfirm ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeChangeConfirm-IEs}},
    ...
}

SNodeChangeConfirm-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSession-SNChangeConfirm-List         CRITICALITY ignore      TYPE PDUSession-SNChangeConfirm-List            PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-CPCInformationConfirm                   CRITICALITY ignore      TYPE CPCInformationConfirm                      PRESENCE optional }|
    { ID id-MN-to-SN-Container                      CRITICALITY ignore      TYPE OCTET STRING                                   PRESENCE optional }|
    { ID id-LTMPSCellInformation-ChangeConfirm      CRITICALITY ignore      TYPE LTMPSCellInformation-ChangeConfirm     PRESENCE optional },
    ...
}
PDUSession-SNChangeConfirm-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSession-SNChangeConfirm-Item

PDUSession-SNChangeConfirm-Item ::= SEQUENCE {
    pduSessionId            PDUSession-ID,
    sn-terminated           PDUSessionResourceChangeConfirmInfo-SNterminated    OPTIONAL,
    mn-terminated           PDUSessionResourceChangeConfirmInfo-MNterminated    OPTIONAL,
-- NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present,
-- abnormal conditions as specified in clause 8.3.5.4 apply.
    iE-Extension            ProtocolExtensionContainer { {PDUSession-SNChangeConfirm-Item-ExtIEs} } OPTIONAL,
    ...
}

PDUSession-SNChangeConfirm-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated        CRITICALITY ignore  EXTENSION           AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated                PRESENCE optional},
    ...
}


-- **************************************************************
--
-- S-NODE CHANGE REFUSE
--
-- **************************************************************

SNodeChangeRefuse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SNodeChangeRefuse-IEs}},
    ...
}

SNodeChangeRefuse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RRC TRANSFER
--
-- **************************************************************

RRCTransfer ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ RRCTransfer-IEs}},
    ...
}

RRCTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-SplitSRB-RRCTransfer                    CRITICALITY reject      TYPE SplitSRB-RRCTransfer                       PRESENCE optional }|
    { ID id-UEReportRRCTransfer                     CRITICALITY reject      TYPE UEReportRRCTransfer                            PRESENCE optional }|
    { ID id-FastMCGRecoveryRRCTransfer-SN-to-MN     CRITICALITY ignore      TYPE FastMCGRecoveryRRCTransfer             PRESENCE optional }|
    { ID id-FastMCGRecoveryRRCTransfer-MN-to-SN     CRITICALITY ignore      TYPE FastMCGRecoveryRRCTransfer             PRESENCE optional }|
    { ID id-SDT-SRB-between-NewNode-OldNode         CRITICALITY ignore      TYPE SDT-SRB-between-NewNode-OldNode            PRESENCE optional }|
    { ID id-QoE-Measurement-Results                 CRITICALITY ignore      TYPE QoE-Measurement-Results                    PRESENCE optional },
    ...
}

SplitSRB-RRCTransfer ::= SEQUENCE {
    rrcContainer                    OCTET STRING                        OPTIONAL,
    srbType                         ENUMERATED {srb1, srb2, ...},
    deliveryStatus                  DeliveryStatus                      OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {SplitSRB-RRCTransfer-ExtIEs} } OPTIONAL,
    ...
}

SplitSRB-RRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UEReportRRCTransfer::= SEQUENCE {
    rrcContainer                    OCTET STRING,
    iE-Extensions                   ProtocolExtensionContainer { {UEReportRRCTransfer-ExtIEs} } OPTIONAL,
    ...
}

UEReportRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FastMCGRecoveryRRCTransfer::= SEQUENCE {
    rrcContainer                    OCTET STRING,
    iE-Extensions                   ProtocolExtensionContainer { { FastMCGRecoveryRRCTransfer-ExtIEs} } OPTIONAL,
    ...
}

FastMCGRecoveryRRCTransfer-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SDT-SRB-between-NewNode-OldNode::= SEQUENCE {
    rrcContainer                    OCTET STRING,
    srb-ID                          SRB-ID,
    iE-Extensions                   ProtocolExtensionContainer { { SDT-SRB-between-NewNode-OldNode-ExtIEs} } OPTIONAL,
    ...
}

SDT-SRB-between-NewNode-OldNode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoE-Measurement-Results ::= SEQUENCE {
    qOEReference                                OCTET STRING (SIZE(6)),
    rrcContainerForRVQoEReport                  OCTET STRING                                    OPTIONAL,
    rrcContainerForQoEReport                    OCTET STRING                                    OPTIONAL,
    appLayerSessionStatus                       ENUMERATED {started, stopped, ...}              OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {QoE-Measurement-Results-ExtIEs} } OPTIONAL,
    ...
}

QoE-Measurement-Results-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- NOTIFICATION CONTROL INDICATION
--
-- **************************************************************

NotificationControlIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{NotificationControlIndication-IEs}},
    ...
}

NotificationControlIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-PDUSessionResourcesNotifyList       CRITICALITY reject      TYPE PDUSessionResourcesNotifyList          PRESENCE optional },
    ...
}

PDUSessionResourcesNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesNotify-Item

PDUSessionResourcesNotify-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    qosFlowsNotificationContrIndInfo    QoSFlowNotificationControlIndicationInfo,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionResourcesNotify-Item-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionResourcesNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- ACTIVITY NOTIFICATION
--
-- **************************************************************

ActivityNotification ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ActivityNotification-IEs}},
    ...
}

ActivityNotification-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-UserPlaneTrafficActivityReport          CRITICALITY ignore      TYPE UserPlaneTrafficActivityReport         PRESENCE optional }|
    { ID id-PDUSessionResourcesActivityNotifyList   CRITICALITY ignore      TYPE PDUSessionResourcesActivityNotifyList  PRESENCE optional }|
    { ID id-RANPagingFailure                        CRITICALITY ignore      TYPE RANPagingFailure                               PRESENCE optional },
    ...
}

PDUSessionResourcesActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesActivityNotify-Item

PDUSessionResourcesActivityNotify-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    pduSessionLevelUPactivityreport     UserPlaneTrafficActivityReport                                          OPTIONAL,
    qosFlowsActivityNotifyList          QoSFlowsActivityNotifyList                                              OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionResourcesActivityNotify-Item-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionResourcesActivityNotify-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoSFlowsActivityNotifyList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsActivityNotifyItem

QoSFlowsActivityNotifyItem ::= SEQUENCE {
    qosFlowIdentifier                   QoSFlowIdentifier,
    pduSessionLevelUPactivityreport     UserPlaneTrafficActivityReport,
    iE-Extensions                       ProtocolExtensionContainer { {QoSFlowsActivityNotifyItem-ExtIEs} } OPTIONAL,
    ...
}

QoSFlowsActivityNotifyItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- XN SETUP REQUEST
--
-- **************************************************************

XnSetupRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnSetupRequest-IEs}},
    ...
}

XnSetupRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-GlobalNG-RAN-node-ID                CRITICALITY reject  TYPE GlobalNG-RANNode-ID                        PRESENCE mandatory}|
    { ID id-TAISupport-list                     CRITICALITY reject  TYPE TAISupport-List                            PRESENCE mandatory}|
    { ID id-AMF-Region-Information              CRITICALITY reject  TYPE AMF-Region-Information                     PRESENCE mandatory}|
    { ID id-List-of-served-cells-NR             CRITICALITY reject  TYPE ServedCells-NR                             PRESENCE optional }|
    { ID id-List-of-served-cells-E-UTRA         CRITICALITY reject  TYPE ServedCells-E-UTRA                         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication         CRITICALITY reject  TYPE InterfaceInstanceIndication                PRESENCE optional }|
    { ID id-TNLConfigurationInfo                CRITICALITY ignore  TYPE TNLConfigurationInfo                       PRESENCE optional }|
    { ID id-PartialListIndicator-NR             CRITICALITY ignore  TYPE PartialListIndicator                       PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-NR    CRITICALITY ignore  TYPE CellAndCapacityAssistanceInfo-NR       PRESENCE optional }|
    { ID id-PartialListIndicator-EUTRA          CRITICALITY ignore  TYPE PartialListIndicator                       PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-EUTRA CRITICALITY ignore  TYPE CellAndCapacityAssistanceInfo-EUTRA    PRESENCE optional }|
    { ID id-Local-NG-RAN-Node-Identifier        CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier           PRESENCE optional }|
    { ID id-Neighbour-NG-RAN-Node-List          CRITICALITY ignore  TYPE Neighbour-NG-RAN-Node-List             PRESENCE optional }|
    { ID id-WAB-MT-ID                           CRITICALITY ignore  TYPE WAB-MT-ID                                  PRESENCE optional },
    ...
}

-- **************************************************************
--
-- XN SETUP RESPONSE
--
-- **************************************************************

XnSetupResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnSetupResponse-IEs}},
    ...
}

XnSetupResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-GlobalNG-RAN-node-ID                CRITICALITY reject  TYPE GlobalNG-RANNode-ID                        PRESENCE mandatory}|
    { ID id-TAISupport-list                     CRITICALITY reject  TYPE TAISupport-List                            PRESENCE mandatory}|
    { ID id-List-of-served-cells-NR             CRITICALITY reject  TYPE ServedCells-NR                             PRESENCE optional }|
    { ID id-List-of-served-cells-E-UTRA         CRITICALITY reject  TYPE ServedCells-E-UTRA                         PRESENCE optional }|
    { ID id-CriticalityDiagnostics              CRITICALITY ignore  TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-AMF-Region-Information              CRITICALITY reject  TYPE AMF-Region-Information                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication         CRITICALITY reject  TYPE InterfaceInstanceIndication                PRESENCE optional }|
    { ID id-TNLConfigurationInfo                CRITICALITY ignore  TYPE TNLConfigurationInfo                       PRESENCE optional }|
    { ID id-PartialListIndicator-NR             CRITICALITY ignore  TYPE PartialListIndicator                       PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-NR    CRITICALITY ignore  TYPE CellAndCapacityAssistanceInfo-NR       PRESENCE optional }|
    { ID id-PartialListIndicator-EUTRA          CRITICALITY ignore  TYPE PartialListIndicator                       PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-EUTRA CRITICALITY ignore  TYPE CellAndCapacityAssistanceInfo-EUTRA    PRESENCE optional }|
    { ID id-Local-NG-RAN-Node-Identifier        CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier           PRESENCE optional }|
    { ID id-Neighbour-NG-RAN-Node-List          CRITICALITY ignore  TYPE Neighbour-NG-RAN-Node-List             PRESENCE optional }|
    { ID id-WAB-MT-ID                           CRITICALITY ignore  TYPE WAB-MT-ID                                  PRESENCE optional },
    ...
}

-- **************************************************************
--
-- XN SETUP FAILURE
--
-- **************************************************************

XnSetupFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnSetupFailure-IEs}},
    ...
}

XnSetupFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-Cause                       CRITICALITY ignore  TYPE Cause                          PRESENCE mandatory}|
    { ID id-TimeToWait                  CRITICALITY ignore  TYPE TimeToWait                     PRESENCE optional }|
    { ID id-CriticalityDiagnostics      CRITICALITY ignore  TYPE CriticalityDiagnostics         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication CRITICALITY reject  TYPE InterfaceInstanceIndication    PRESENCE optional }|
    { ID id-MessageOversizeNotification CRITICALITY ignore  TYPE MessageOversizeNotification    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- NG-RAN NODE CONFIGURATION UPDATE
--
-- **************************************************************

NGRANNodeConfigurationUpdate ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ NGRANNodeConfigurationUpdate-IEs}},
    ...
}

NGRANNodeConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-TAISupport-list                         CRITICALITY reject  TYPE TAISupport-List                                    PRESENCE optional }|
    { ID id-ConfigurationUpdateInitiatingNodeChoice CRITICALITY ignore  TYPE ConfigurationUpdateInitiatingNodeChoice    PRESENCE mandatory}|
    { ID id-TNLA-To-Add-List                        CRITICALITY ignore  TYPE TNLA-To-Add-List                                   PRESENCE optional }|
    { ID id-TNLA-To-Remove-List                     CRITICALITY ignore  TYPE TNLA-To-Remove-List                                PRESENCE optional }|
    { ID id-TNLA-To-Update-List                     CRITICALITY ignore  TYPE TNLA-To-Update-List                                PRESENCE optional }|
    { ID id-GlobalNG-RAN-node-ID                    CRITICALITY reject  TYPE GlobalNG-RANNode-ID                                PRESENCE optional }|
    { ID id-AMF-Region-Information-To-Add           CRITICALITY reject  TYPE AMF-Region-Information                         PRESENCE optional }|
    { ID id-AMF-Region-Information-To-Delete        CRITICALITY reject  TYPE AMF-Region-Information                         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication                    PRESENCE optional }|
    { ID id-TNLConfigurationInfo                    CRITICALITY ignore  TYPE TNLConfigurationInfo                               PRESENCE optional }|
    { ID id-Coverage-Modification-List              CRITICALITY reject  TYPE Coverage-Modification-List                     PRESENCE optional }|
    { ID id-Local-NG-RAN-Node-Identifier            CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier                   PRESENCE optional }|
    { ID id-Neighbour-NG-RAN-Node-List              CRITICALITY ignore  TYPE Neighbour-NG-RAN-Node-List                     PRESENCE optional }|
    { ID id-Local-NG-RAN-Node-Identifier-Removal    CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier                   PRESENCE optional }|
    { ID id-WAB-MT-ID                               CRITICALITY ignore  TYPE WAB-MT-ID                                          PRESENCE optional }|
    { ID id-Future-Coverage-Modification-List       CRITICALITY ignore  TYPE Future-Coverage-Modification-List          PRESENCE optional },
    ...
}

ConfigurationUpdateInitiatingNodeChoice ::= CHOICE {
    gNB                                 ProtocolIE-Container    { {ConfigurationUpdate-gNB} },
    ng-eNB                              ProtocolIE-Container    { {ConfigurationUpdate-ng-eNB} },
    choice-extension                    ProtocolIE-Single-Container { {ServedCellsToUpdateInitiatingNodeChoice-ExtIEs} }
}

ServedCellsToUpdateInitiatingNodeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ConfigurationUpdate-gNB XNAP-PROTOCOL-IES ::= {
    { ID id-servedCellsToUpdate-NR          CRITICALITY ignore TYPE ServedCellsToUpdate-NR                          PRESENCE optional }|
    { ID id-cellAssistanceInfo-NR           CRITICALITY ignore TYPE CellAssistanceInfo-NR                           PRESENCE optional }|
    { ID id-cellAssistanceInfo-EUTRA        CRITICALITY ignore TYPE CellAssistanceInfo-EUTRA                        PRESENCE optional }|
    { ID id-ServedCellSpecificInfoReq-NR    CRITICALITY ignore TYPE ServedCellSpecificInfoReq-NR                    PRESENCE optional },
    ...
}


ConfigurationUpdate-ng-eNB XNAP-PROTOCOL-IES ::= {
    { ID id-servedCellsToUpdate-E-UTRA          CRITICALITY ignore TYPE ServedCellsToUpdate-E-UTRA              PRESENCE optional }|
    { ID id-cellAssistanceInfo-NR               CRITICALITY ignore TYPE CellAssistanceInfo-NR                       PRESENCE optional }|
    { ID id-cellAssistanceInfo-EUTRA            CRITICALITY ignore TYPE CellAssistanceInfo-EUTRA                    PRESENCE optional },
    ...
}



-- **************************************************************
--
-- NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE
--
-- **************************************************************

NGRANNodeConfigurationUpdateAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ NGRANNodeConfigurationUpdateAcknowledge-IEs}},
    ...
}

NGRANNodeConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-RespondingNodeTypeConfigUpdateAck           CRITICALITY ignore  TYPE RespondingNodeTypeConfigUpdateAck      PRESENCE mandatory}|
    { ID id-TNLA-Setup-List                             CRITICALITY ignore  TYPE TNLA-Setup-List                                PRESENCE optional }|
    { ID id-TNLA-Failed-To-Setup-List                   CRITICALITY ignore  TYPE TNLA-Failed-To-Setup-List                  PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore  TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication                 CRITICALITY reject  TYPE InterfaceInstanceIndication                PRESENCE optional }|
    { ID id-TNLConfigurationInfo                        CRITICALITY ignore  TYPE TNLConfigurationInfo                       PRESENCE optional   }|
    { ID id-Local-NG-RAN-Node-Identifier                CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier               PRESENCE optional   }|
    { ID id-Neighbour-NG-RAN-Node-List                  CRITICALITY ignore  TYPE Neighbour-NG-RAN-Node-List             PRESENCE optional   }|
    { ID id-Local-NG-RAN-Node-Identifier-Removal        CRITICALITY ignore  TYPE Local-NG-RAN-Node-Identifier               PRESENCE optional   }|
    { ID id-WAB-MT-ID                                   CRITICALITY ignore  TYPE WAB-MT-ID                                      PRESENCE optional },
    ...
}
RespondingNodeTypeConfigUpdateAck ::= CHOICE {
    ng-eNB                  RespondingNodeTypeConfigUpdateAck-ng-eNB,
    gNB                     RespondingNodeTypeConfigUpdateAck-gNB,
    choice-extension        ProtocolIE-Single-Container { {RespondingNodeTypeConfigUpdateAck-ExtIEs} }
}

RespondingNodeTypeConfigUpdateAck-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

RespondingNodeTypeConfigUpdateAck-ng-eNB ::= SEQUENCE {
    iE-Extension        ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs} }    OPTIONAL,
    ...
}

RespondingNodeTypeConfigUpdateAck-ng-eNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-List-of-served-cells-E-UTRA             CRITICALITY ignore  EXTENSION ServedCells-E-UTRA                            PRESENCE optional }|
    { ID id-PartialListIndicator-EUTRA              CRITICALITY ignore  EXTENSION PartialListIndicator                      PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-EUTRA     CRITICALITY ignore  EXTENSION CellAndCapacityAssistanceInfo-EUTRA   PRESENCE optional },
    ...
}


RespondingNodeTypeConfigUpdateAck-gNB ::= SEQUENCE {
    served-NR-Cells     ServedCells-NR                                                              OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs} }   OPTIONAL,
    ...
}

RespondingNodeTypeConfigUpdateAck-gNB-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-PartialListIndicator-NR             CRITICALITY ignore  EXTENSION PartialListIndicator                      PRESENCE optional }|
    { ID id-CellAndCapacityAssistanceInfo-NR    CRITICALITY ignore  EXTENSION CellAndCapacityAssistanceInfo-NR      PRESENCE optional },
    ...
}


-- **************************************************************
--
-- NG-RAN NODE CONFIGURATION UPDATE FAILURE
--
-- **************************************************************

NGRANNodeConfigurationUpdateFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{NGRANNodeConfigurationUpdateFailure-IEs}},
    ...
}

NGRANNodeConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-Cause                       CRITICALITY ignore  TYPE Cause                          PRESENCE mandatory}|
    { ID id-TimeToWait                  CRITICALITY ignore  TYPE TimeToWait                     PRESENCE optional }|
    { ID id-CriticalityDiagnostics      CRITICALITY ignore  TYPE CriticalityDiagnostics         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication CRITICALITY reject  TYPE InterfaceInstanceIndication    PRESENCE optional },
    ...
}


-- **************************************************************
--
-- E-UTRA - NR CELL RESOURCE COORDINATION REQUEST
--
-- **************************************************************

E-UTRA-NR-CellResourceCoordinationRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{E-UTRA-NR-CellResourceCoordinationRequest-IEs}},
    ...
}

E-UTRA-NR-CellResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-initiatingNodeType-ResourceCoordRequest CRITICALITY reject  TYPE InitiatingNodeType-ResourceCoordRequest        PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication                        PRESENCE optional },
    ...
}

InitiatingNodeType-ResourceCoordRequest ::= CHOICE {
    ng-eNB                              ResourceCoordRequest-ng-eNB-initiated,
    gNB                                 ResourceCoordRequest-gNB-initiated,
    choice-extension                    ProtocolIE-Single-Container { {InitiatingNodeType-ResourceCoordRequest-ExtIEs} }
}

InitiatingNodeType-ResourceCoordRequest-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ResourceCoordRequest-ng-eNB-initiated ::= SEQUENCE {
    dataTrafficResourceIndication       DataTrafficResourceIndication,
    spectrumSharingGroupID              SpectrumSharingGroupID,
    listofE-UTRACells                   SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI                         OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {ResourceCoordRequest-ng-eNB-initiated-ExtIEs} }   OPTIONAL,
    ...
}

ResourceCoordRequest-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ResourceCoordRequest-gNB-initiated ::= SEQUENCE {
    dataTrafficResourceIndication       DataTrafficResourceIndication,
    listofE-UTRACells                   SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI                     OPTIONAL,
    spectrumSharingGroupID              SpectrumSharingGroupID,
    listofNRCells                       SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI                             OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {ResourceCoordRequest-gNB-initiated-ExtIEs} }  OPTIONAL,
    ...
}

ResourceCoordRequest-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE
--
-- **************************************************************

E-UTRA-NR-CellResourceCoordinationResponse::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{E-UTRA-NR-CellResourceCoordinationResponse-IEs}},
    ...
}

E-UTRA-NR-CellResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-respondingNodeType-ResourceCoordResponse    CRITICALITY reject  TYPE RespondingNodeType-ResourceCoordResponse   PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication                 CRITICALITY reject  TYPE InterfaceInstanceIndication                        PRESENCE optional },
    ...
}

RespondingNodeType-ResourceCoordResponse ::= CHOICE {
    ng-eNB                              ResourceCoordResponse-ng-eNB-initiated,
    gNB                                 ResourceCoordResponse-gNB-initiated,
    choice-extension                    ProtocolIE-Single-Container { {RespondingNodeType-ResourceCoordResponse-ExtIEs} }
}

RespondingNodeType-ResourceCoordResponse-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ResourceCoordResponse-ng-eNB-initiated ::= SEQUENCE {
    dataTrafficResourceIndication       DataTrafficResourceIndication,
    spectrumSharingGroupID              SpectrumSharingGroupID,
    listofE-UTRACells                   SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI                             OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {ResourceCoordResponse-ng-eNB-initiated-ExtIEs} }      OPTIONAL,
    ...
}

ResourceCoordResponse-ng-eNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ResourceCoordResponse-gNB-initiated ::= SEQUENCE {
    dataTrafficResourceIndication       DataTrafficResourceIndication,
    spectrumSharingGroupID              SpectrumSharingGroupID,
    listofNRCells                       SEQUENCE (SIZE(1.. maxnoofCellsinNG-RANnode)) OF NR-CGI                                 OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {ResourceCoordResponse-gNB-initiated-ExtIEs} }     OPTIONAL,
    ...
}

ResourceCoordResponse-gNB-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- SECONDARY RAT DATA USAGE REPORT
--
-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container        {{SecondaryRATDataUsageReport-IEs}},
    ...
}

SecondaryRATDataUsageReport-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-PDUSessionResourceSecondaryRATUsageList CRITICALITY reject      TYPE PDUSessionResourceSecondaryRATUsageList    PRESENCE mandatory},
    ...
}


-- **************************************************************
--
-- XN REMOVAL REQUEST
--
-- **************************************************************

XnRemovalRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnRemovalRequest-IEs}},
    ...
}

XnRemovalRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-GlobalNG-RAN-node-ID        CRITICALITY reject  TYPE GlobalNG-RANNode-ID            PRESENCE mandatory}|
    { ID id-XnRemovalThreshold          CRITICALITY reject  TYPE XnBenefitValue                 PRESENCE optional }|
    { ID id-InterfaceInstanceIndication CRITICALITY reject  TYPE InterfaceInstanceIndication    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- XN REMOVAL RESPONSE
--
-- **************************************************************

XnRemovalResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnRemovalResponse-IEs}},
    ...
}

XnRemovalResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-GlobalNG-RAN-node-ID        CRITICALITY reject  TYPE GlobalNG-RANNode-ID            PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics      CRITICALITY ignore  TYPE CriticalityDiagnostics         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication CRITICALITY reject  TYPE InterfaceInstanceIndication    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- XN REMOVAL FAILURE
--
-- **************************************************************

XnRemovalFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ XnRemovalFailure-IEs}},
    ...
}

XnRemovalFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-Cause                       CRITICALITY ignore  TYPE Cause                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics      CRITICALITY ignore  TYPE CriticalityDiagnostics         PRESENCE optional }|
    { ID id-InterfaceInstanceIndication CRITICALITY reject  TYPE InterfaceInstanceIndication    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- CELL ACTIVATION REQUEST
--
-- **************************************************************

CellActivationRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ CellActivationRequest-IEs}},
    ...
}

CellActivationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-ServedCellsToActivate                   CRITICALITY reject      TYPE ServedCellsToActivate                      PRESENCE mandatory}|
    { ID id-ActivationIDforCellActivation           CRITICALITY reject      TYPE ActivationIDforCellActivation          PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

ServedCellsToActivate ::= CHOICE {
    nr-cells                            SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,
    e-utra-cells                        SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,
    choice-extension                    ProtocolIE-Single-Container { {ServedCellsToActivate-ExtIEs} }
}

ServedCellsToActivate-ExtIEs XNAP-PROTOCOL-IES ::= {
    { ID id-NRCellsAndSSBsList          CRITICALITY ignore      TYPE ToBeActivatedNRCellsAndSSBsList                PRESENCE mandatory},
    ...
}

ToBeActivatedNRCellsAndSSBsList ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF ToBeActivatedNRCellsAndSSBs-Item

ToBeActivatedNRCellsAndSSBs-Item ::= SEQUENCE {
    nrCGI                       NR-CGI,
    sSBstobeActivatedList       SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSBsToBeActivated-Item  OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { ToBeActivatedNRCellsAndSSBs-Item-ExtIEs} }   OPTIONAL,
    ...
}

ToBeActivatedNRCellsAndSSBs-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SSBsToBeActivated-Item ::= SEQUENCE {
    ssbIndex            INTEGER(0..63),
    iE-Extensions       ProtocolExtensionContainer { { SSBsToBeActivated-Item-ExtIEs} } OPTIONAL,
    ...
}

SSBsToBeActivated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- CELL ACTIVATION RESPONSE
--
-- **************************************************************

CellActivationResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{CellActivationResponse-IEs}},
    ...
}

CellActivationResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-ActivatedServedCells                    CRITICALITY reject      TYPE ActivatedServedCells                       PRESENCE mandatory}|
    { ID id-ActivationIDforCellActivation           CRITICALITY reject      TYPE ActivationIDforCellActivation          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

ActivatedServedCells ::= CHOICE {
    nr-cells                            SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,
    e-utra-cells                        SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,
    choice-extension                    ProtocolIE-Single-Container { {ActivatedServedCells-ExtIEs} }
}

ActivatedServedCells-ExtIEs XNAP-PROTOCOL-IES ::= {
    { ID id-ActivatedNRCellsAndSSBsList             CRITICALITY ignore      TYPE ActivatedNRCellsAndSSBsList            PRESENCE mandatory},
    ...
}

ActivatedNRCellsAndSSBsList ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF ActivatedNRCellsAndSSBs-Item

ActivatedNRCellsAndSSBs-Item ::= SEQUENCE {
    nrCGI                   NR-CGI,
    sSBsActivatedList   SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBsActivated-Item       OPTIONAL,
    iE-Extensions   ProtocolExtensionContainer { {ActivatedNRCellsAndSSBs-Item-ExtIEs} }    OPTIONAL,
    ...
}

ActivatedNRCellsAndSSBs-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SSBsActivated-Item ::= SEQUENCE {
    ssbIndex        INTEGER(0..63),
    iE-Extensions   ProtocolExtensionContainer { {SSBsActivated-Item-ExtIEs} }  OPTIONAL,
    ...
}

SSBsActivated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- CELL ACTIVATION FAILURE
--
-- **************************************************************

CellActivationFailure ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{CellActivationFailure-IEs}},
    ...
}

CellActivationFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-ActivationIDforCellActivation           CRITICALITY reject      TYPE ActivationIDforCellActivation          PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RESET REQUEST
--
-- **************************************************************

ResetRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ResetRequest-IEs}},
    ...
}

ResetRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-ResetRequestTypeInfo                    CRITICALITY reject      TYPE ResetRequestTypeInfo                       PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RESET RESPONSE
--
-- **************************************************************

ResetResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ResetResponse-IEs}},
    ...
}

ResetResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-ResetResponseTypeInfo                   CRITICALITY reject      TYPE ResetResponseTypeInfo                      PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- ERROR INDICATION
--
-- **************************************************************

ErrorIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ErrorIndication-IEs}},
    ...
}

ErrorIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE optional }|
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE optional }|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject      TYPE InterfaceInstanceIndication                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- PRIVATE MESSAGE
--
-- **************************************************************

PrivateMessage ::= SEQUENCE {
    privateIEs      PrivateIE-Container {{PrivateMessage-IEs}},
    ...
}

PrivateMessage-IEs XNAP-PRIVATE-IES ::= {
    ...
}


-- **************************************************************
--
-- TRACE START
--
-- **************************************************************

TraceStart ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container        { {TraceStartIEs} },
    ...
}

TraceStartIEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-TraceActivation                         CRITICALITY ignore  TYPE TraceActivation                            PRESENCE optional },
    ...
}

-- **************************************************************
--
-- DEACTIVATE TRACE
--
-- **************************************************************

DeactivateTrace ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container        { {DeactivateTraceIEs} },
    ...
}

DeactivateTraceIEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-NG-RANTraceID                           CRITICALITY ignore  TYPE NG-RANTraceID                              PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- FAILURE INDICATION
--
-- **************************************************************

FailureIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{FailureIndication-IEs}},
    ...
}

FailureIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-InitiatingCondition-FailureIndication               CRITICALITY reject      TYPE InitiatingCondition-FailureIndication                          PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- HANDOVER REPORT
--
-- **************************************************************

HandoverReport ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ HandoverReport-IEs}},
    ...
}

HandoverReport-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-HandoverReportType          CRITICALITY ignore      TYPE HandoverReportType             PRESENCE mandatory}|
    { ID id-HandoverCause               CRITICALITY ignore      TYPE Cause                          PRESENCE mandatory}|
    { ID id-SourceCellCGI               CRITICALITY ignore      TYPE GlobalNG-RANCell-ID            PRESENCE mandatory }|
    { ID id-TargetCellCGI               CRITICALITY ignore      TYPE GlobalNG-RANCell-ID            PRESENCE mandatory }|
    { ID id-ReEstablishmentCellCGI      CRITICALITY ignore      TYPE GlobalCell-ID                  PRESENCE conditional }|
-- This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell"
    { ID id-TargetCellinEUTRAN          CRITICALITY ignore      TYPE TargetCellinEUTRAN             PRESENCE conditional }|
-- This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong"
    { ID id-SourceCellCRNTI             CRITICALITY ignore      TYPE C-RNTI                         PRESENCE optional }|
    { ID id-MobilityInformation         CRITICALITY ignore      TYPE MobilityInformation            PRESENCE optional }|
    { ID id-UERLFReportContainer        CRITICALITY ignore      TYPE UERLFReportContainer           PRESENCE optional }|
    { ID id-CHOConfiguration            CRITICALITY ignore      TYPE CHOConfiguration               PRESENCE optional }|
    { ID id-TargetCellCRNTI             CRITICALITY ignore      TYPE C-RNTI                         PRESENCE optional }|
    { ID id-TimeSinceFailure            CRITICALITY ignore      TYPE TimeSinceFailure               PRESENCE optional },
    ...
}

-- **************************************************************
--
-- SCG FAILURE INDICATION
--
-- **************************************************************

SCGFailureIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ SCGFailureIndication-IEs}},
    ...
}

SCGFailureIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-targetNG-RANnodeUEXnAPID    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID             PRESENCE mandatory}|
    { ID id-SCGFailureReportContainer   CRITICALITY ignore      TYPE SCGFailureReportContainer      PRESENCE mandatory}|
    { ID id-FailureType                 CRITICALITY ignore      TYPE FailureType                    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RESOURCE STATUS REQUEST
--
-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ResourceStatusRequest-IEs}},
    ...
}

ResourceStatusRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID          CRITICALITY reject  TYPE Measurement-ID                 PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID          CRITICALITY ignore  TYPE Measurement-ID                 PRESENCE conditional}|
-- This IE shall be present if the Registration Request IE is set to the value "stop" or "add".
    { ID id-RegistrationRequest                 CRITICALITY reject  TYPE RegistrationRequest            PRESENCE mandatory}|
    { ID id-ReportCharacteristics               CRITICALITY reject  TYPE ReportCharacteristics          PRESENCE conditional}|
-- This IE shall be present if the Registration Request IE is set to the value "start".
    { ID id-CellToReport                        CRITICALITY ignore  TYPE CellToReport                   PRESENCE optional }|
    { ID id-ReportingPeriodicity                CRITICALITY ignore  TYPE ReportingPeriodicity           PRESENCE optional },
    ...
}


-- **************************************************************
--
-- RESOURCE STATUS RESPONSE
--
-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ResourceStatusResponse-IEs}},
    ...
}

ResourceStatusResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID          CRITICALITY reject  TYPE Measurement-ID                         PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID          CRITICALITY reject  TYPE Measurement-ID                         PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics              CRITICALITY ignore  TYPE CriticalityDiagnostics                 PRESENCE optional },
    ...
}


-- **************************************************************
--
-- RESOURCE STATUS FAILURE
--
-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ResourceStatusFailure-IEs}},
    ...
}

ResourceStatusFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID              CRITICALITY reject  TYPE Measurement-ID                                 PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID              CRITICALITY reject  TYPE Measurement-ID                                 PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore  TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore  TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}


-- **************************************************************
--
-- RESOURCE STATUS UPDATE
--
-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ResourceStatusUpdate-IEs}},
    ...
}

ResourceStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID      CRITICALITY reject  TYPE Measurement-ID                 PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID      CRITICALITY reject  TYPE Measurement-ID                 PRESENCE mandatory}|
    { ID id-CellMeasurementResult           CRITICALITY ignore  TYPE CellMeasurementResult          PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- MOBILITY CHANGE REQUEST
--
-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{MobilityChangeRequest-IEs}},
    ...
}

MobilityChangeRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NG-RANnode1CellID                           CRITICALITY reject  TYPE GlobalNG-RANCell-ID                            PRESENCE mandatory}|
    { ID id-NG-RANnode2CellID                           CRITICALITY reject  TYPE GlobalNG-RANCell-ID                            PRESENCE mandatory}|
    { ID id-NG-RANnode1MobilityParameters               CRITICALITY ignore  TYPE MobilityParametersInformation          PRESENCE optional }|
    { ID id-NG-RANnode2ProposedMobilityParameters       CRITICALITY reject  TYPE MobilityParametersInformation          PRESENCE mandatory}|
    { ID id-Cause                                       CRITICALITY ignore  TYPE Cause                                          PRESENCE mandatory}|
    { ID id-SSBOffsets-List                             CRITICALITY ignore  TYPE SSBOffsets-List                                PRESENCE optional },
    ...
}


-- **************************************************************
--
-- MOBILITY CHANGE ACKNOWLEDGE
--
-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{MobilityChangeAcknowledge-IEs}},
    ...
}

MobilityChangeAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NG-RANnode1CellID                   CRITICALITY reject  TYPE GlobalNG-RANCell-ID                            PRESENCE mandatory}|
    { ID id-NG-RANnode2CellID                   CRITICALITY reject  TYPE GlobalNG-RANCell-ID                            PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics              CRITICALITY ignore  TYPE CriticalityDiagnostics                         PRESENCE optional },
    ...
}


-- **************************************************************
--
-- MOBILITY CHANGE FAILURE
--
-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{MobilityChangeFailure-IEs}},
    ...
}

MobilityChangeFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NG-RANnode1CellID                       CRITICALITY reject  TYPE GlobalNG-RANCell-ID                                    PRESENCE mandatory}|
    { ID id-NG-RANnode2CellID                       CRITICALITY reject  TYPE GlobalNG-RANCell-ID                                    PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore  TYPE Cause                                                  PRESENCE mandatory}|
    { ID id-MobilityParametersModificationRange     CRITICALITY ignore  TYPE MobilityParametersModificationRange                PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore  TYPE CriticalityDiagnostics                             PRESENCE optional }|
    { ID id-NG-RANnode2SSBOffsetsModificationRange  CRITICALITY ignore  TYPE NG-RANnode2SSBOffsetsModificationRange         PRESENCE optional },
    ...
}


-- **************************************************************
--
-- ACCESS AND MOBILITY INDICATION
--
-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ AccessAndMobilityIndication-IEs}},
    ...
}
AccessAndMobilityIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-RAReport                                CRITICALITY ignore  TYPE RAReport                                               PRESENCE optional}|
    { ID id-SuccessfulHOReportInformation               CRITICALITY ignore  TYPE SuccessfulHOReportInformation                  PRESENCE optional}|
    { ID id-SuccessfulPSCellChangeReportInformation     CRITICALITY ignore  TYPE SuccessfulPSCellChangeReportInformation        PRESENCE optional}|
    { ID id-DLLBTFailureInformationList                 CRITICALITY ignore  TYPE DLLBTFailureInformationList                        PRESENCE optional},
    ...
}

-- **************************************************************
--
-- CELL TRAFFIC TRACE
--
-- **************************************************************

CellTrafficTrace ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container        { {CellTrafficTraceIEs} },
    ...
}

CellTrafficTraceIEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID             PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID             PRESENCE mandatory}|
    { ID id-NG-RANTraceID                   CRITICALITY ignore  TYPE NG-RANTraceID                  PRESENCE mandatory}|
    { ID id-TraceCollectionEntityIPAddress  CRITICALITY ignore  TYPE TransportLayerAddress          PRESENCE mandatory}|
    { ID id-PrivacyIndicator                CRITICALITY ignore  TYPE PrivacyIndicator               PRESENCE optional }|
    { ID id-TraceCollectionEntityURI        CRITICALITY ignore  TYPE URIaddress                     PRESENCE optional },
    ...
}

-- **************************************************************
--
-- RAN MULTICAST GROUP PAGING
--
-- **************************************************************

RANMulticastGroupPaging ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{RANMulticastGroupPaging-IEs}},
    ...
}

RANMulticastGroupPaging-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-MBS-Session-ID                          CRITICALITY reject  TYPE MBS-Session-ID                                 PRESENCE mandatory}|
    { ID id-UEIdentityIndexList-MBSGroupPaging      CRITICALITY reject  TYPE UEIdentityIndexList-MBSGroupPaging     PRESENCE mandatory}|
    { ID id-MulticastRANPagingArea                  CRITICALITY reject  TYPE RANPagingArea                                  PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- SCG FAILURE INFORMATION REPORT
--
-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ ScgFailureInformationReport-IEs}},
    ...
}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-SourcePSCellCGI                         CRITICALITY ignore      TYPE GlobalNG-RANCell-ID                            PRESENCE optional }|
    { ID id-FailedPSCellCGI                         CRITICALITY ignore      TYPE GlobalNG-RANCell-ID                            PRESENCE optional }|
    { ID id-SCGFailureReportContainer               CRITICALITY ignore      TYPE SCGFailureReportContainer                  PRESENCE mandatory}|
    { ID id-SNMobilityInformation                   CRITICALITY ignore      TYPE SNMobilityInformation                      PRESENCE optional }|
    { ID id-CPACConfiguration                       CRITICALITY ignore      TYPE CPACConfiguration                          PRESENCE optional }|
    { ID id-TimeSCG-Failure                         CRITICALITY ignore      TYPE TimeSCG-Failure                                PRESENCE optional },
    ...
}

-- **************************************************************
--
-- SCG FAILURE TRANSFER
--
-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ ScgFailureTransfer-IEs}},
    ...
}
ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                    CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory},
    ...
}


-- **************************************************************
--
-- F1-C TRAFFIC TRANSFER

--
-- **************************************************************

F1CTrafficTransfer ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container        {{ F1CTrafficTransfer-IEs}},
    ...
}

F1CTrafficTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                        CRITICALITY reject  TYPE NG-RANnodeUEXnAPID         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                        CRITICALITY reject  TYPE NG-RANnodeUEXnAPID         PRESENCE mandatory}|
    { ID id-F1CTrafficContainer                         CRITICALITY reject  TYPE F1CTrafficContainer            PRESENCE mandatory},
    ...
}


-- **************************************************************
--
-- IAB TRANSPORT MIGRATION MANAGEMENT REQUEST
--
-- **************************************************************

IABTransportMigrationManagementRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABTransportMigrationManagementRequest-IEs}},
    ...
}

IABTransportMigrationManagementRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID     CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-TrafficToBeAddedList                    CRITICALITY reject      TYPE TrafficToBeAddedList                   PRESENCE optional }|
    { ID id-TrafficToBeModifiedList                 CRITICALITY reject      TYPE TrafficToBeModifiedList                PRESENCE optional }|
    { ID id-TrafficToBeReleaseInformation           CRITICALITY reject      TYPE TrafficToBeReleaseInformation      PRESENCE optional }|
    { ID id-IAB-TNL-Address-Request                 CRITICALITY reject      TYPE IAB-TNL-Address-Request                PRESENCE optional }|
    { ID id-IABTNLAddressException                  CRITICALITY reject      TYPE IABTNLAddressException                 PRESENCE optional }|
    { ID id-MIAB-MT-BAP-Address                     CRITICALITY reject      TYPE BAPAddress                                 PRESENCE optional },
    ...
}

TrafficToBeAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeAdded-Item

TrafficToBeAdded-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    trafficProfile          TrafficProfile,
    f1-TerminatingTopologyBHInformation     F1-TerminatingTopologyBHInformation     OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {TrafficToBeAdded-Item-ExtIEs} }   OPTIONAL,
    ...
}

TrafficToBeAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TrafficToBeModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeModified-Item

TrafficToBeModified-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    trafficProfile          TrafficProfile      OPTIONAL,
    f1-TerminatingTopologyBHInformation     F1-TerminatingTopologyBHInformation     OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {TrafficToBeModified-Item-ExtIEs} }    OPTIONAL,
    ...
}

TrafficToBeModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- **************************************************************
--
-- IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE
--
-- **************************************************************

IABTransportMigrationManagementResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABTransportMigrationManagementResponse-IEs}},
    ...
}

IABTransportMigrationManagementResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID     CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-TrafficAddedList                        CRITICALITY reject      TYPE TrafficAddedList                           PRESENCE optional }|
    { ID id-TrafficModifiedList                     CRITICALITY reject      TYPE TrafficModifiedList                        PRESENCE optional }|
    { ID id-TrafficNotAddedList                     CRITICALITY reject      TYPE TrafficNotAddedList                        PRESENCE optional }|
    { ID id-TrafficNotModifiedList                  CRITICALITY reject      TYPE TrafficNotModifiedList                 PRESENCE optional }|
    { ID id-IAB-TNL-Address-Response                CRITICALITY reject      TYPE IAB-TNL-Address-Response               PRESENCE optional }|
    { ID id-TrafficReleasedList                     CRITICALITY reject      TYPE TrafficReleasedList                        PRESENCE optional },
    ...
}

TrafficAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficAdded-Item

TrafficAdded-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    non-F1-TerminatingTopologyBHInformation     Non-F1-TerminatingTopologyBHInformation,
    iE-Extensions           ProtocolExtensionContainer { {TrafficAdded-Item-ExtIEs} }   OPTIONAL,
    ...
}

TrafficAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TrafficModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficModified-Item

TrafficModified-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    non-F1-TerminatingTopologyBHInformation     Non-F1-TerminatingTopologyBHInformation,
    iE-Extensions           ProtocolExtensionContainer { {TrafficModified-Item-ExtIEs} }    OPTIONAL,
    ...
}

TrafficModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TrafficNotAddedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotAdded-Item

TrafficNotAdded-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    casue                   Cause           OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {TrafficNotAdded-Item-ExtIEs} }    OPTIONAL,
    ...
}

TrafficNotAdded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TrafficNotModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficNotModified-Item

TrafficNotModified-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    cause                   Cause       OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {TrafficNotModified-Item-ExtIEs} } OPTIONAL,
    ...
}

TrafficNotModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TrafficReleasedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficReleased-Item

TrafficReleased-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    bHInfoList              BHInfoList      OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { TrafficReleased-Item-ExtIEs} }   OPTIONAL,
    ...
}

TrafficReleased-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- IAB TRANSPORT MIGRATION MANAGEMENT REJECT
--
-- **************************************************************

IABTransportMigrationManagementReject ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABTransportMigrationManagementReject-IEs}},
    ...
}

IABTransportMigrationManagementReject-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID     CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                      PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                 PRESENCE optional },
    ...
}


-- **************************************************************
--
-- IAB TRANSPORT MIGRATION MODIFICATION REQUEST
--
-- **************************************************************

IABTransportMigrationModificationRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABTransportMigrationModificationRequest-IEs}},
    ...
}

IABTransportMigrationModificationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID         CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-TrafficRequiredToBeModifiedList             CRITICALITY reject  TYPE TrafficRequiredToBeModifiedList            PRESENCE optional }|
    { ID id-TrafficToBeReleaseInformation               CRITICALITY reject  TYPE TrafficToBeReleaseInformation          PRESENCE optional }|
    { ID id-IABTNLAddressToBeAdded                      CRITICALITY reject  TYPE IAB-TNL-Address-Response                   PRESENCE optional }|
    { ID id-IABTNLAddressToBeReleasedList               CRITICALITY reject  TYPE IABTNLAddressToBeReleasedList          PRESENCE optional }|
    { ID id-IABAuthorizationStatus                      CRITICALITY ignore  TYPE IABAuthorizationStatus                     PRESENCE optional }|
    { ID id-MobileIAB-AuthorizationStatus               CRITICALITY ignore  TYPE MobileIAB-AuthorizationStatus          PRESENCE optional },
    ...
}

TrafficRequiredToBeModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficRequiredToBeModified-Item

TrafficRequiredToBeModified-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    non-f1-TerminatingTopologyBHInformation     Non-F1-TerminatingTopologyBHInformation,
    iE-Extensions           ProtocolExtensionContainer{ { TrafficRequiredToBeModified-Item-ExtIEs} }        OPTIONAL,
    ...
}

TrafficRequiredToBeModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABTNLAddressToBeReleasedList ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB)) OF IABTNLAddressToBeReleased-Item

IABTNLAddressToBeReleased-Item ::= SEQUENCE {
    iabTNLAddress           IABTNLAddress,
    iE-Extensions           ProtocolExtensionContainer{ { IABTNLAddressToBeReleased-Item-ExtIEs} }  OPTIONAL,
    ...
}

IABTNLAddressToBeReleased-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



-- **************************************************************
--
-- IAB TRANSPORT MIGRATION MODIFICATION RESPONSE
--
-- **************************************************************

IABTransportMigrationModificationResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABTransportMigrationModificationResponse-IEs}},
    ...
}

IABTransportMigrationModificationResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID         CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-TrafficRequiredModifiedList                 CRITICALITY reject  TYPE TrafficRequiredModifiedList            PRESENCE optional }|
    { ID id-TrafficReleasedList                         CRITICALITY reject  TYPE TrafficReleasedList                        PRESENCE optional },
    ...
}



TrafficRequiredModifiedList ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficRequiredModified-Item

TrafficRequiredModified-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    iE-Extensions           ProtocolExtensionContainer { { TrafficRequiredModified-Item-ExtIEs} }   OPTIONAL,
    ...
}

TrafficRequiredModified-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- IAB RESOURCE COORDINATION REQUEST
--
-- **************************************************************

IABResourceCoordinationRequest ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABResourceCoordinationRequest-IEs}},
    ...
}

IABResourceCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID    CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                 PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                 PRESENCE mandatory}|
    { ID id-BoundaryNodeCellsList               CRITICALITY reject      TYPE BoundaryNodeCellsList          PRESENCE optional }|
    { ID id-ParentNodeCellsList                 CRITICALITY reject      TYPE ParentNodeCellsList                PRESENCE optional },
    ...
}


BoundaryNodeCellsList ::= SEQUENCE (SIZE(1..maxnoofServedCellsIAB)) OF BoundaryNodeCellsList-Item

BoundaryNodeCellsList-Item ::= SEQUENCE {
    boundaryNodeCellInformation         IABCellInformation,
    iE-Extensions                       ProtocolExtensionContainer { {BoundaryNodeCellsList-Item-ExtIEs} }      OPTIONAL,
    ...
}

BoundaryNodeCellsList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ParentNodeCellsList ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF ParentNodeCellsList-Item

ParentNodeCellsList-Item ::= SEQUENCE {
    parentNodeCellInformation           IABCellInformation,
    iE-Extensions                       ProtocolExtensionContainer { {ParentNodeCellsList-Item-ExtIEs} }    OPTIONAL,
    ...
}

ParentNodeCellsList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- IAB RESOURCE COORDINATION RESPONSE
--
-- **************************************************************

IABResourceCoordinationResponse ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ IABResourceCoordinationResponse-IEs}},
    ...
}

IABResourceCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-F1-Terminating-IAB-DonorUEXnAPID        CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                 PRESENCE mandatory}|
    { ID id-nonF1-Terminating-IAB-DonorUEXnAPID     CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                 PRESENCE mandatory}|
    { ID id-BoundaryNodeCellsList                   CRITICALITY reject      TYPE BoundaryNodeCellsList              PRESENCE optional }|
    { ID id-ParentNodeCellsList                     CRITICALITY reject      TYPE ParentNodeCellsList                    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- CONDITIONAL PSCELL CHANGE CANCEL
--
-- **************************************************************

CPCCancel ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ CPCCancel-IEs}},
    ...
}
CPCCancel-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-M-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-S-NG-RANnodeUEXnAPID                CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                               CRITICALITY ignore      TYPE Cause                                      PRESENCE optional }|
    { ID id-target-S-NG-RANnodeID               CRITICALITY reject      TYPE GlobalNG-RANNode-ID                        PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- PARTIAL UE CONTEXT TRANSFER
--
-- **************************************************************
PartialUEContextTransfer ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ PartialUEContextTransfer-IEs}},
    ...
}

PartialUEContextTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY reject      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-SDTPartialUEContextInfo                 CRITICALITY ignore      TYPE SDTPartialUEContextInfo                    PRESENCE mandatory}|
    { ID id-PosPartialUEContextInfo                 CRITICALITY ignore      TYPE PosPartialUEContextInfo                    PRESENCE optional },
    ...
}

-- **************************************************************
--
-- PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE
--
-- **************************************************************
PartialUEContextTransferAcknowledge ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ PartialUEContextTransferAcknowledge-IEs}},
    ...
}

PartialUEContextTransferAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-SDTDataForwardingDRBList                CRITICALITY ignore      TYPE SDTDataForwardingDRBList                   PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional }|
    { ID id-SRSConfiguration                        CRITICALITY ignore      TYPE SRSConfiguration                               PRESENCE optional },
    ...
}

-- **************************************************************
--
-- PARTIAL UE CONTEXT TRANSFER FAILURE
--
-- **************************************************************

PartialUEContextTransferFailure::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ PartialUEContextTransferFailure-IEs}},
    ...
}

PartialUEContextTransferFailure-IEs XNAP-PROTOCOL-IES ::= { 
    { ID id-newNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-oldNG-RANnodeUEXnAPID                   CRITICALITY ignore      TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore      TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore      TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}


-- **************************************************************
--
-- RACH INDICATION
--
-- **************************************************************

RachIndication ::= SEQUENCE {
    protocolIEs         ProtocolIE-Container    {{ RachIndication-IEs}},
    ...
}

RachIndication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-RaReportIndicationList                  CRITICALITY reject      TYPE RaReportIndicationList                     PRESENCE mandatory },
    ...
}

-- **************************************************************
--
-- DATA COLLECTION REQUEST
--
-- **************************************************************

DataCollectionRequest ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{DataCollectionRequest-IEs}},
    ...
}

DataCollectionRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID                  CRITICALITY reject  TYPE Measurement-ID                                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID                  CRITICALITY ignore  TYPE Measurement-ID                                     PRESENCE conditional}|
-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".
    { ID id-RegistrationRequestForDataCollection        CRITICALITY reject  TYPE RegistrationRequestForDataCollection       PRESENCE mandatory}|
    { ID id-ReportCharacteristicsForDataCollection      CRITICALITY reject  TYPE ReportCharacteristicsForDataCollection PRESENCE conditional}|
-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "start".
    { ID id-CellToReportForDataCollection-List          CRITICALITY ignore  TYPE CellToReportForDataCollection-List         PRESENCE optional }|
    { ID id-ReportingPeriodicityForDataCollection       CRITICALITY ignore  TYPE ReportingPeriodicityForDataCollection      PRESENCE optional }|
    { ID id-RequestedPredictionTime                     CRITICALITY ignore  TYPE RequestedPredictionTime                        PRESENCE optional }|
    { ID id-UETrajectoryCollectionConfiguration         CRITICALITY ignore  TYPE UETrajectoryCollectionConfiguration        PRESENCE optional }|
    { ID id-UEPerformanceCollectionConfiguration        CRITICALITY ignore  TYPE UEPerformanceCollectionConfiguration       PRESENCE optional },
    ...
}


-- **************************************************************
--
-- DATA COLLECTION RESPONSE
--
-- **************************************************************

DataCollectionResponse ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{DataCollectionResponse-IEs}},
    ...
}

DataCollectionResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID                  CRITICALITY reject  TYPE Measurement-ID                                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID                  CRITICALITY reject  TYPE Measurement-ID                                     PRESENCE mandatory}|
    { ID id-NodeMeasurementInitiationResult-List        CRITICALITY reject  TYPE NodeMeasurementInitiationResult-List       PRESENCE optional }|
    { ID id-CellMeasurementInitiationResult-List        CRITICALITY reject  TYPE CellMeasurementInitiationResult-List       PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore  TYPE CriticalityDiagnostics                         PRESENCE optional },
    ...
}


-- **************************************************************
--
-- DATA COLLECTION FAILURE
--
-- **************************************************************

DataCollectionFailure ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{DataCollectionFailure-IEs}},
    ...
}

DataCollectionFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID              CRITICALITY reject  TYPE Measurement-ID                                 PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID              CRITICALITY reject  TYPE Measurement-ID                                 PRESENCE mandatory}|
    { ID id-Cause                                   CRITICALITY ignore  TYPE Cause                                          PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore  TYPE CriticalityDiagnostics                     PRESENCE optional },
    ...
}


-- **************************************************************
--
-- DATA COLLECTION UPDATE
--
-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{DataCollectionUpdate-IEs}},
    ...
}

DataCollectionUpdate-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-Measurement-ID                  CRITICALITY reject  TYPE Measurement-ID                                             PRESENCE mandatory}|
    { ID id-NGRAN-Node2-Measurement-ID                  CRITICALITY reject  TYPE Measurement-ID                                             PRESENCE mandatory}|
    { ID id-CellMeasurementResultForDataCollection-List CRITICALITY ignore  TYPE CellMeasurementResultForDataCollection-List        PRESENCE optional }|
    { ID id-UEAssociatedInfoResult-List                 CRITICALITY ignore  TYPE UEAssociatedInfoResult-List                            PRESENCE optional }|
    { ID id-NodeAssociatedInfoResult                    CRITICALITY ignore  TYPE NodeAssociatedInfoResult                               PRESENCE optional },
    ...
}


-- **************************************************************
--
-- OD-SIB1 CONFIGURATION PROVISION REQUEST
--
-- **************************************************************

ODSIB1ConfigurationProvisionRequest ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ODSIB1ConfigurationProvisionRequest-IEs}},
    ...
}

ODSIB1ConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-RequestType                             CRITICALITY reject  TYPE RequestTypeChoice                          PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication            PRESENCE optional },
    ...
}

RequestTypeChoice ::= CHOICE {
    start                           ODSIB1ConfigurationInformation,
    stop                            NR-CGI,
    choice-extension                ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }
}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ODSIB1ConfigurationInformation ::= SEQUENCE {
    oDSIB1Configuration                 OCTET STRING,
    nES-Cell-ID                         NR-CGI,
    cEll-A-ID                           NR-CGI                      OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {ODSIB1ConfigurationInformation-ExtIEs} }  OPTIONAL,
    ...
}

ODSIB1ConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- OD-SIB1 CONFIGURATION PROVISION RESPONSE
--
-- **************************************************************

ODSIB1ConfigurationProvisionResponse ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ODSIB1ConfigurationProvisionResponse-IEs}},
    ...
}

ODSIB1ConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NES-Cell-ID                             CRITICALITY reject  TYPE NR-CGI                                     PRESENCE mandatory}|
    { ID id-Cell-A-ID                               CRITICALITY ignore  TYPE NR-CGI                                     PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication            PRESENCE optional },
    ...
}



-- **************************************************************
--
-- OD-SIB1 CONFIGURATION PROVISION FAILURE
--
-- **************************************************************

ODSIB1ConfigurationProvisionFailure ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ ODSIB1ConfigurationProvisionFailure-IEs}},
    ...
}

ODSIB1ConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= { 
    { ID id-Cause                                   CRITICALITY ignore  TYPE Cause                                      PRESENCE mandatory}|
    { ID id-NES-Cell-ID                             CRITICALITY reject  TYPE NR-CGI                                     PRESENCE mandatory}|
    { ID id-Cell-A-ID                               CRITICALITY ignore  TYPE NR-CGI                                     PRESENCE optional }|
    { ID id-CriticalityDiagnostics                  CRITICALITY ignore  TYPE CriticalityDiagnostics                 PRESENCE optional }|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication            PRESENCE optional },
    ...
}


-- **************************************************************
--
-- OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE
--
-- **************************************************************

ODSIB1ConfigurationProvisionStatusUpdate ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{ODSIB1ConfigurationProvisionStatusUpdate-IEs}},
    ...
}

ODSIB1ConfigurationProvisionStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NES-Cell-ID                             CRITICALITY reject  TYPE NR-CGI                                     PRESENCE mandatory}|
    { ID id-Cell-A-ID                               CRITICALITY ignore  TYPE NR-CGI                             PRESENCE optional }|
    { ID id-ProvisionStatus                         CRITICALITY reject  TYPE ProvisionStatus                            PRESENCE mandatory}|
    { ID id-InterfaceInstanceIndication             CRITICALITY reject  TYPE InterfaceInstanceIndication            PRESENCE optional },
    ...
}

ProvisionStatus ::= ENUMERATED {stopped, ...}



-- **************************************************************
--
-- CELL SWITCH NOTIFICATION
--
-- **************************************************************

CellSwitchNotification ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{CellSwitchNotification-IEs}},
    ...
}

CellSwitchNotification-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID                    CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                         PRESENCE mandatory}|
    { ID id-targetCellGlobalID                          CRITICALITY reject  TYPE Target-CGI                                     PRESENCE mandatory}|
    { ID id-LTMCellSwitchInformation                    CRITICALITY ignore  TYPE LTMCellSwitchInformation                   PRESENCE optional }|
    { ID id-LTMUEAssociationInformation-List            CRITICALITY ignore  TYPE LTMUEAssociationInformation-List       PRESENCE optional }|
    { ID id-CellSwitchTAInformation-List                CRITICALITY ignore  TYPE CellSwitchTAInformation-List               PRESENCE optional },
    ...
}



-- **************************************************************
--
-- TA INFORMATION TRANSFER
--
-- **************************************************************

TAInformationTransfer ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{TAInformationTransfer-IEs}},
    ...
}

TAInformationTransfer-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-TAInformation-List  CRITICALITY reject  TYPE TAInformation-List     PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- LTM CONFIGURATION UPDATE
--
-- **************************************************************

LTMConfigurationUpdate ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{LTMConfigurationUpdate-IEs}},
    ...
}

LTMConfigurationUpdate-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-ID                              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-ID                                  CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                                     PRESENCE optional }|
    { ID id-LTMUpdatesToCandidateNodeInformation        CRITICALITY ignore  TYPE LTMUpdatesToCandidateNodeInformation               PRESENCE optional }|
    { ID id-LTMUpdatesToCandidateCellInformation-List   CRITICALITY ignore  TYPE LTMUpdatesToCandidateCellInformation-List      PRESENCE optional }|    { ID id-LTMUESecurityInformation                    CRITICALITY ignore  TYPE LTMUESecurityInformation                               PRESENCE optional },
    ...
}


-- **************************************************************
--
-- LTM CONFIGURATION UPDATE ACKNOWLEDGE
--
-- **************************************************************

LTMConfigurationUpdateAcknowledge ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{LTMConfigurationUpdateAcknowledge-IEs}},
    ...
}

LTMConfigurationUpdateAcknowledge-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-ID                              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-ID                              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                                     PRESENCE mandatory}|
    { ID id-LTMUpdatesFromCandidateCellInformation-List CRITICALITY ignore  TYPE LTMUpdatesFromCandidateCellInformation-List        PRESENCE optional }|
    { ID id-CriticalityDiagnostics                      CRITICALITY ignore  TYPE CriticalityDiagnostics                                 PRESENCE optional },
    ...
}


-- **************************************************************
--
-- LTM CONFIGURATION UPDATE FAILURE
--
-- **************************************************************

LTMConfigurationUpdateFailure ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{LTMConfigurationUpdateFailure-IEs}},
    ...
}

LTMConfigurationUpdateFailure-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-ID                  CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                 PRESENCE mandatory}|
    { ID id-NGRAN-Node2-ID                  CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                 PRESENCE optional }|
    { ID id-Cause                           CRITICALITY ignore  TYPE Cause                              PRESENCE mandatory}|
    { ID id-CriticalityDiagnostics          CRITICALITY ignore  TYPE CriticalityDiagnostics             PRESENCE optional },
    ...
}




-- **************************************************************
--
-- LTM CANCEL
--
-- **************************************************************

LTMCancel ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{LTMCancel-IEs}},
    ...
}

LTMCancel-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-sourceNG-RANnodeUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-targetNG-RANnodeUEXnAPID            CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                             PRESENCE mandatory}|
    { ID id-Cause                               CRITICALITY ignore  TYPE Cause                                      PRESENCE optional }|
    { ID id-LTMCandidateCellsToBeCancelled-List CRITICALITY ignore  TYPE LTMCandidateCellsToBeCancelled-List    PRESENCE optional },
    ...
}



-- **************************************************************
--
-- CSI-RS COORDINATION REQUEST
--
-- **************************************************************

CSIRSCoordinationRequest ::= SEQUENCE {

    protocolIEs     ProtocolIE-Container    {{CSIRSCoordinationRequest-IEs}},
    ...
}

CSIRSCoordinationRequest-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-ID              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-ID              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-CSI-RSCoordinationRequest   CRITICALITY reject  TYPE CSI-RSCoordinationRequestList          PRESENCE mandatory},
    ...
}





-- **************************************************************
--
-- CSI-RS COORDINATION RESPONSE
--
-- **************************************************************

CSIRSCoordinationResponse ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{CSIRSCoordinationResponse-IEs}},
    ...
}

CSIRSCoordinationResponse-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-NGRAN-Node1-ID              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-NGRAN-Node2-ID              CRITICALITY reject  TYPE NG-RANnodeUEXnAPID                     PRESENCE mandatory}|
    { ID id-CSI-RSCoordinationResponse  CRITICALITY reject  TYPE CSI-RSCoordinationResultList           PRESENCE mandatory},
    ...
}

-- **************************************************************
--
-- CLI INDICATION
--
-- **************************************************************

CLI-Indication ::= SEQUENCE {
    protocolIEs     ProtocolIE-Container    {{CLI-Indication-IEs}},
    ...
}

CLI-Indication-IEs XNAP-PROTOCOL-IES ::= {
    { ID id-CLI-MeasurementResult-List                  CRITICALITY ignore  TYPE CLI-MeasurementResult-List                         PRESENCE optional }|
    { ID id-SRS-Resource-Indication                     CRITICALITY ignore  TYPE SRS-Resource-Indication                                PRESENCE optional },
    ...
}


END
-- ASN1STOP

-- 9.3.5    Information Element definitions
-- ASN1START
-- **************************************************************
--
-- Information Element Definitions
--
-- **************************************************************

XnAP-IEs {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS


    id-CNTypeRestrictionsForEquivalent,
    id-CNTypeRestrictionsForServing,
    id-Additional-UL-NG-U-TNLatUPF-List,
    id-ConfiguredTACIndication,
    id-AlternativeQoSParaSetList,
    id-CurrentQoSParaSetIndex,
    id-DefaultDRB-Allowed,
    id-DLCarrierList,
    id-EndpointIPAddressAndPort,
    id-ExtendedReportIntervalMDT,
    id-ExtendedTAISliceSupportList,
    id-FiveGCMobilityRestrictionListContainer,
    id-SecondarydataForwardingInfoFromTarget-List,
    id-LastE-UTRANPLMNIdentity,
    id-LTEA2XUEPC5AggregateMaximumBitRate,
    id-IntendedTDD-DL-ULConfiguration-NR,
    id-MaxIPrate-DL,
    id-SecurityResult,
    id-OldQoSFlowMap-ULendmarkerexpected,
    id-PDUSessionCommonNetworkInstance,
    id-PDUSession-PairID,
    id-BPLMN-ID-Info-EUTRA,
    id-BPLMN-ID-Info-NR,
    id-DRBsNotAdmittedSetupModifyList,
    id-Secondary-MN-Xn-U-TNLInfoatM,
    id-ULForwardingProposal,
    id-DRB-IDs-takenintouse,
    id-SplitSessionIndicator,
    id-NonGBRResources-Offered,
    id-MDT-Configuration,
    id-TraceCollectionEntityURI,
    id-NPN-Broadcast-Information,
    id-NPNPagingAssistanceInformation,
    id-NPNMobilityInformation,
    id-NPN-Support,
    id-LTEUESidelinkAggregateMaximumBitRate,
    id-NRA2XUEPC5AggregateMaximumBitRate,
    id-NRUESidelinkAggregateMaximumBitRate,
    id-ExtendedRATRestrictionInformation,
    id-QoSMonitoringRequest,
    id-QoSMonitoringDisabled,
    id-QosMonitoringReportingFrequency,
    id-DAPSRequestInfo,
    id-OffsetOfNbiotChannelNumberToDL-EARFCN,
    id-OffsetOfNbiotChannelNumberToUL-EARFCN,
    id-NBIoT-UL-DL-AlignmentOffset,
    id-TDDULDLConfigurationCommonNR,
    id-CarrierList,
    id-ULCarrierList,
    id-FrequencyShift7p5khz,
    id-SSB-PositionsInBurst,
    id-NRCellPRACHConfig,
    id-Redundant-UL-NG-U-TNLatUPF,
    id-Redundant-DL-NG-U-TNLatNG-RAN,
    id-CNPacketDelayBudgetDownlink,
    id-CNPacketDelayBudgetUplink,
    id-ExtendedPacketDelayBudget,
    id-Additional-Redundant-UL-NG-U-TNLatUPF-List,
    id-RedundantCommonNetworkInstance,
    id-TSCTrafficCharacteristics,
    id-RedundantQoSFlowIndicator,
    id-Additional-PDCP-Duplication-TNL-List,
    id-RedundantPDUSessionInformation,
    id-UsedRSNInformation,
    id-RLCDuplicationInformation,
    id-CSI-RSTransmissionIndication,
    id-UERadioCapabilityID,
    id-secondary-SN-UL-PDCP-UP-TNLInfo,
    id-pdcpDuplicationConfiguration,
    id-duplicationActivation,
    id-NPRACHConfiguration,
    id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,
    id-DL-scheduling-PDCCH-CCE-usage,
    id-UL-scheduling-PDCCH-CCE-usage,
    id-SFN-Offset,
    id-QoS-Mapping-Information,
    id-AdditionLocationInformation,
    id-dataForwardingInfoFromTargetE-UTRANnode,
    id-Cause,
    id-SecurityIndication,
    id-RRCConnReestab-Indicator,
    id-SourceDLForwardingIPAddress,
    id-SourceNodeDLForwardingIPAddress,
    id-M4ReportAmount,
    id-M5ReportAmount,
    id-M6ReportAmount,
    id-M7ReportAmount,
    id-BeamMeasurementIndicationM1,
    id-Supported-MBS-FSA-ID-List,
    id-MBS-AssistanceInformation,
    id-MBS-SessionAssociatedInformation,
    id-MBS-SessionInformation-List,
    id-SliceRadioResourceStatus-List,
    id-CompositeAvailableCapacitySupplementaryUplink,
    id-SSBOffsets-List,
    id-NG-RANnode2SSBOffsetsModificationRange,
    id-NR-U-Channel-List,
    id-NR-U-ChannelInfo-List,
    id-MIMOPRBusageInformation,
    id-UEAssistantIdentifier,
    id-IAB-MT-Cell-List,
    id-NoPDUSessionIndication,
    id-permutation,
    id-UL-GNB-DU-Cell-Resource-Configuration,
    id-DL-GNB-DU-Cell-Resource-Configuration,
    id-tdd-GNB-DU-Cell-Resource-Configuration,
    id-Additional-Measurement-Timing-Configuration-List,
    id-SurvivalTime,
    id-Local-NG-RAN-Node-Identifier,
    id-Neighbour-NG-RAN-Node-List,
    id-FiveGProSeUEPC5AggregateMaximumBitRate,
    id-Redcap-Bcast-Information,
    id-UESliceMaximumBitRateList,
    id-PositioningInformation,
    id-ServedCellSpecificInfoReq-NR,
    id-TAINSAGSupportList,
    id-earlyMeasurement,
    id-BeamMeasurementsReportConfiguration,
    id-CoverageModificationCause,
    id-UERLFReportContainerLTEExtension,
    id-ExcessPacketDelayThresholdConfiguration,
    id-Full-and-Short-I-RNTI-Profile-List,
    id-QosFlowMappingIndication,
    id-EquivalentSNPNs,
    id-CHOTimeBasedInformation,
    id-ChannelOccupancyTimePercentageUL,
    id-EnergyDetectionThresholdUL,
    id-PSCellListContainer,
    id-RadioResourceStatusNR-U,
    id-FiveGProSeLayer2Multipath,
    id-FiveGProSeLayer2UEtoUERelay,
    id-FiveGProSeLayer2UEtoUERemote,
    id-ClockQualityReportingControlInfo,
    id-CapabilityForBATAdaptation,
    id-PNI-NPNBasedMDT,
    id-PNI-NPN-AreaScopeofMDT,
    id-SNPN-CellBasedMDT,
    id-SNPN-TAIBasedMDT,
    id-SNPN-BasedMDT,
    id-S-CPAC-Request,
    id-S-CPAC-Request-Info,
    id-S-CPAC-ReferenceConfigRequest,
    id-S-CPAC-InterSN-ExecutionNotify,
    id-S-CPAC-dataforwardinginfofromSource,
    id-CPACcandidatePSCells-wotherInfo-list,
    id-eRedcap-Bcast-Information,
    id-NRPagingLongeDRXInformationforRRCINACTIVE,
    id-MBSCommServiceType,
    id-AssistanceInformationQoE-Meas,
    id-QoERVQoEReportingPaths,
    id-DirectForwardingPathAvailability,
    id-CHO-CPAC-Info,
    id-CHO-Maxnoof-CondReconfig,
    id-PDUSetQoSParameters,
    id-N6JitterInformation,
    id-ECNMarkingorCongestionInformationReportingRequest,
    id-TAISliceUnavailableCellList,
    id-MobileIABCell,
    id-XR-Bcast-Information,
    id-MaximumDataBurstVolume,
    id-CPAC-Preparation-Type,
    id-MN-only-MDT-collection,
    id-BarringExemptionforEmerCallInfo,
    id-Transmission-Bandwidth-asymmetric,
    id-NRPPaPositioningInformation,
    id-AerialUEFlightInformationReportingControl,
    id-PDCPSNGapTransfer-UL,
    id-ECNMarkingorCongestionInformationReportingStatus,
    id-AdditionalDRBSetupInfoList,
    id-PSIbasedSDUdiscardUL,
    id-PSIbasedSDUdiscardDL,
    id-DLPDUSetInformationMarkingSupportIndication,
    id-PduSetDelayBudgetDownlink, 
    id-PduSetDelayBudgetUplink,
    id-PduSetErrorRateDownlink,
    id-PduSetErrorRateUplink,
    id-MonitoringRequestonAvailableBitrate,
    id-MMSID,
    id-Indication-of-bitrate-adaptation,
    id-GeographyBasedMDT,
    id-NetworkSliceAreaScopeofMDT,
    id-SBFD-Frequency-Configuration,
    id-NZP-CSI-RS-Resources-Config,
    id-SRS-Resource-Configuration,
    id-SliceToReportForDataCollection-List,
    id-PredictedSliceAvailableCapacity,
    id-SliceMeasurementInitiationResult,
    id-SliceUEPerformance,
    id-FiveGProSeLayer3MHUEtoNetworkRelay,
    id-FiveGProSeLayer2MHUEtoNetworkRelay,
    id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,
    id-FiveGProSeLayer2MHRemote,
    id-UEAveragePacketLossUL,
    id-SemipersistentPositioningInformation,
    maxEARFCN,
    maxnoofAllowedAreas,
    maxnoofAMFRegions,
    maxnoofAoIs,
    maxnoofBPLMNs,
    maxnoofCAGs,
    maxnoofCAGsperPLMN,
    maxnoofCellsinAoI,
    maxnoofCellsinNG-RANnode,
    maxnoofCellsinRNA,
    maxnoofCellsinUEHistoryInfo,
    maxnoofCellsUEMovingTrajectory,
    maxnoofDRBs,
    maxnoofEPLMNs,
    maxnoofEPLMNsplus1,
    maxnoofEUTRABands,
    maxnoofEUTRABPLMNs,
    maxnoofForbiddenTACs,
    maxnoofMBSFNEUTRA,
    maxnoofMultiConnectivityMinusOne,
    maxnoofNeighbours,
    maxnoofNIDs,
    maxnoofNRCellBands,
    maxnoofPDUSessions,
    maxnoofPLMNs,
    maxnoofProtectedResourcePatterns,
    maxnoofQoSFlows,
    maxnoofQoSParaSets,
    maxnoofRANAreaCodes,
    maxnoofRANAreasinRNA,
    maxnoofSCellGroups,
    maxnoofSCellGroupsplus1,
    maxnoofSliceItems,
    maxnoofExtSliceItems,
    maxnoofSNPNIDs,
    maxnoofsupportedTACs,
    maxnoofsupportedPLMNs,
    maxnoofTAI,
    maxnoofTAIsinAoI,
    maxnoofTNLAssociations,
    maxnoofUEContexts,
    maxNRARFCN,
    maxNrOfErrors,
    maxnoofRANNodesinAoI,
    maxnooftimeperiods,
    maxnoofslots,
    maxnoofExtTLAs,
    maxnoofGTPTLAs,
    maxnoofCHOcells,
    maxnoofPC5QoSFlows,
    maxnoofSSBAreas,
    maxnoofNRSCSs,
    maxnoofPhysicalResourceBlocks,
    maxnoofRAReports,
    maxnoofAdditionalPDCPDuplicationTNL,
    maxnoofRLCDuplicationstate,
    maxnoofBluetoothName,
    maxnoofCellIDforMDT,
    maxnoofMDTPLMNs,
    maxnoofTAforMDT,
    maxnoofWLANName,
    maxnoofSensorName,
    maxnoofNeighPCIforMDT,
    maxnoofFreqforMDT,
    maxnoofNonAnchorCarrierFreqConfig,
    maxnoofDataForwardingTunneltoE-UTRAN,
    maxnoofUEIDIndicesforMBSPaging,
    maxnoofMBSFSAs,
    maxnoofMBSQoSFlows,
    maxnoofMRBs,
    maxnoofCellsforMBS,
    maxnoofMBSServiceAreaInformation,
    maxnoofTAIforMBS,
    maxnoofAssociatedMBSSessions,
    maxnoofMBSSessions,
    maxnoofSuccessfulHOReports,
    maxnoofPSCellsPerSN,
    maxnoofNR-UChannelIDs,
    maxnoofCellsinCHO,
    maxnoofCHOexecutioncond,
    maxnoofServingCells,
    maxnoofBHInfo,
    maxnoofTLAsIAB,
    maxnoofTrafficIndexEntries,
    maxnoofBAPControlPDURLCCHs,
    maxnoofServedCellsIAB,
    maxnoofDUFSlots,
    maxnoofSymbols,
    maxnoofHSNASlots,
    maxnoofRBsetsPerCell,
    maxnoofChildIABNodes,
    maxnoofIABSTCInfo,
    maxnoofPSCellCandidates,
    maxnoofTargetSNs,
    maxnoofUEAppLayerMeas,
    maxnoofSNSSAIforQMC,
    maxnoofCellIDforQMC,
    maxnoofPLMNforQMC,
    maxnoofTAforQMC,
    maxnoofMTCItems,
    maxnoofCSIRSconfigurations,
    maxnoofCSIRSneighbourCells,
    maxnoofCSIRSneighbourCellsInMTC,
    maxnoofNeighbour-NG-RAN-Nodes,
    maxnoofSRBs,
    maxnoofSMBR,
    maxnoofNSAGs,
    maxnoofRBsetsPerCell1,
    maxnoofTargetSNsMinusOne,
    maxnoofThresholdsForExcessPacketDelay,
    maxnoofESNPNs,
    maxnoofSuccessfulPSCellChangeReports,
    maxnoofUEsforRAReportIndications,
    maxnoofPSCellsinCPAC,
    maxnoofCPACexecutioncond,
    maxnoofLBTFailureInformation,
    maxnoofCellsTrajectoryPredict,
    maxnoofCellsTrajectory,
    maxFailedCellMeasObjects,
    maxFailedMeasPerNode,
    maxnoofUEReports,
    maxnoofCandidateRelayUEs,
    maxnoofCAGforMDT,
    maxnoofMDTSNPNs,
    maxnoofSecurityConfigurations,
    maxnoofRSPPQoSFlows,
    maxnoofThresholds,
    maxnoofEarlyRACHResourcesID,
    maxnoofLTMCells,
    maxNoSSBs,
    maxnoofTAList,
    maxnoofPreambleIndexes,
    maxnoofCSIResourceConfigurations,
    maxnoofNZP-CSI-RS-ResourcesPerSet,
    maxnoofSRS-Resource,
    maxnoofFailedSliceMeasObjects,
    maxnoofSliceItemsforMDT,
    maxnoofAreaNTNforMDT,
    maxnoofLTMCellsPlusOne,
    maxnoofSCGSecurityConfigurations,
    maxnoofLTM-CSI-ResourcesPerSet




FROM XnAP-Constants

    Criticality,
    ProcedureCode,
    ProtocolIE-ID,
    TriggeringMessage
FROM XnAP-CommonDataTypes

    ProtocolExtensionContainer{},
    ProtocolIE-Single-Container{},
    
    XNAP-PROTOCOL-EXTENSION,
    XNAP-PROTOCOL-IES
FROM XnAP-Containers;


-- A

AerialUEFlightInformationReportingControl ::= SEQUENCE {
    higherAltitudeThreshold                 Altitude,
    lowerAltitudeThreshold                  Altitude,
    aerialUEReportingPeriodicity            AerialUEReportingPeriodicity                                    OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { AerialUEFlightInformationReportingControl-ExtIEs} }      OPTIONAL,
    ...
}

AerialUEFlightInformationReportingControl-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AerialUEReportingPeriodicity ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, ...}

Altitude ::= INTEGER (-420..10000, ...)


A2XPC5QoSParameters ::= SEQUENCE {
    a2XPC5QoSFlowList                   A2XPC5QoSFlowList,
    aA2XPC5LinkAggregateBitRates            BitRate                                         OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { A2XPC5QoSParameters-ExtIEs} }        OPTIONAL,
    ...
}

A2XPC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

A2XPC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF A2XPC5QoSFlowItem

A2XPC5QoSFlowItem ::= SEQUENCE {
    a2XpQI              FiveQI,
    a2Xpc5FlowBitRates  A2XPC5FlowBitRates                                              OPTIONAL,
    a2Xrange            Range                                                           OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { A2XPC5QoSFlowItem-ExtIEs} }      OPTIONAL,
    ...
}

A2XPC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

A2XPC5FlowBitRates ::= SEQUENCE {
    a2XguaranteedFlowBitRate        BitRate,
    a2XmaximumFlowBitRate           BitRate,
    iE-Extensions       ProtocolExtensionContainer { { A2XPC5FlowBitRates-ExtIEs} }     OPTIONAL,
    ...
}

A2XPC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AvailableBitrateReportThresholdList ::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF AvailableBitrateReportThresholdItem

AvailableBitrateReportThresholdItem ::= SEQUENCE {
    reportingThreshold              ReportingThreshold,
    iE-Extensions                   ProtocolExtensionContainer { { AvailableBitrateReportThresholdItem-ExtIEs} }    OPTIONAL,
    ...
}

AvailableBitrateReportThresholdItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE (SIZE(1..maxnoofTargetSNsMinusOne)) OF AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item ::= SEQUENCE {
    pDUSessionResourceChangeConfirmInfo-SNterminated                PDUSessionResourceChangeConfirmInfo-SNterminated,
    iE-Extensions       ProtocolExtensionContainer { { AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item-ExtIEs} } OPTIONAL,
    ...
}

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AveragePacketDelay ::= SEQUENCE {
    uL-AveragePacketDelay       AveragePacketDelayValue,
    dL-AveragePacketDelay       AveragePacketDelayValue,
    iE-Extensions       ProtocolExtensionContainer { {AveragePacketDelay-ExtIEs} }  OPTIONAL,
    ...
}

AveragePacketDelay-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AveragePacketDelayValue ::= INTEGER (0..10000)


AdditionLocationInformation ::= ENUMERATED {
    includePSCell,
    ...
}

Additional-PDCP-Duplication-TNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF Additional-PDCP-Duplication-TNL-Item
Additional-PDCP-Duplication-TNL-Item ::= SEQUENCE {
    additional-PDCP-Duplication-UP-TNL-Information  UPTransportLayerInformation,
    iE-Extensions       ProtocolExtensionContainer { { Additional-PDCP-Duplication-TNL-ExtIEs} }    OPTIONAL,
    ...
}
Additional-PDCP-Duplication-TNL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Additional-UL-NG-U-TNLatUPF-Item ::= SEQUENCE {
    additional-UL-NG-U-TNLatUPF             UPTransportLayerInformation,
    iE-Extensions       ProtocolExtensionContainer { { Additional-UL-NG-U-TNLatUPF-Item-ExtIEs} }   OPTIONAL,
    ...
}

Additional-UL-NG-U-TNLatUPF-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-PDUSessionCommonNetworkInstance     CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance       PRESENCE optional},
    ...
}

Additional-UL-NG-U-TNLatUPF-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF Additional-UL-NG-U-TNLatUPF-Item

Additional-Measurement-Timing-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofMTCItems)) OF Additional-Measurement-Timing-Configuration-Item

Additional-Measurement-Timing-Configuration-Item ::= SEQUENCE {
    additionalMeasurementTimingConfigurationIndex       INTEGER (0..16),
    csi-RS-MTC-Configuration-List                       CSI-RS-MTC-Configuration-List,
    iE-Extensions                       ProtocolExtensionContainer { { Additional-Measurement-Timing-Configuration-Item-ExtIEs} }   OPTIONAL,
    ...
}

Additional-Measurement-Timing-Configuration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ActivationIDforCellActivation   ::= INTEGER (0..255)

Active-MBS-SessionInformation ::= SEQUENCE {
    mBS-QoSFlowsToAdd-List                              MBS-QoSFlowsToAdd-List,
    mBS-ServiceArea                                     MBS-ServiceArea                                             OPTIONAL,
    mBS-MappingandDataForwardingRequestInfofromSource   MBS-MappingandDataForwardingRequestInfofromSource       OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { Active-MBS-SessionInformation-ExtIEs} }      OPTIONAL,
    ...
}

Active-MBS-SessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DataCollectionID ::= SEQUENCE {
    nGRAN-Node1-Measurement-ID                  Measurement-ID,
    nGRAN-Node2-Measurement-ID                  Measurement-ID,
    iE-Extensions                           ProtocolExtensionContainer { { DataCollectionID-ExtIEs} } OPTIONAL,
    ...
}

DataCollectionID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AdditionalDRBSetupInfoList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF AdditionalDRBsToBeSetupInfo-Item

AdditionalDRBsToBeSetupInfo-Item ::= SEQUENCE {
    drb-ID                                                  DRB-ID,
    additionalQoSFlowMappedtoDRBSetupInfoList               AdditionalQoSFlowMappedtoDRBSetupInfoList,
    eCNMarkingorCongestionInformationReportingStatus        ECNMarkingorCongestionInformationReportingStatus,
    iE-Extensions                   ProtocolExtensionContainer { { AdditionalDRBsToBeSetupInfo-Item-ExtIEs} }   OPTIONAL,
    ...
}

AdditionalDRBsToBeSetupInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
        ...
}

AdditionalQoSFlowMappedtoDRBSetupInfoList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF AdditionalQoSFlowMappedtoDRBSetupInfoList-Item

AdditionalQoSFlowMappedtoDRBSetupInfoList-Item ::= SEQUENCE {
    qoSFlowIdentifier               QoSFlowIdentifier,
    iE-Extensions                   ProtocolExtensionContainer { { AdditionalQoSFlowMappedtoDRBSetupInfoList-Item-ExtIEs} }     OPTIONAL,
    ...
}

AdditionalQoSFlowMappedtoDRBSetupInfoList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
        ...
}


AerialControllerUE ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

AerialUE ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

AerialUESubscriptionInformation ::= ENUMERATED {
    allowed,
    not-allowed,
    ...
}



AllocationandRetentionPriority ::= SEQUENCE {
    priorityLevel                   INTEGER (0..15,...),
    pre-emption-capability          ENUMERATED {shall-not-trigger-preemption, may-trigger-preemption, ...},
    pre-emption-vulnerability       ENUMERATED {not-preemptable, preemptable, ...},
    iE-Extensions                   ProtocolExtensionContainer { {AllocationandRetentionPriority-ExtIEs} } OPTIONAL,
    ...
}

AllocationandRetentionPriority-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ActivationSFN ::= INTEGER (0..1023)

AllowedCAG-ID-List-perPLMN ::= SEQUENCE (SIZE(1..maxnoofCAGsperPLMN)) OF CAG-Identifier

AllowedPNI-NPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofEPLMNsplus1)) OF AllowedPNI-NPN-ID-Item

AllowedPNI-NPN-ID-Item ::= SEQUENCE {
    plmn-id                             PLMN-Identity,
    pni-npn-restricted-information      PNI-NPN-Restricted-Information,
    allowed-CAG-id-list-per-plmn        AllowedCAG-ID-List-perPLMN,
    iE-Extensions                       ProtocolExtensionContainer { {AllowedPNI-NPN-ID-Item-ExtIEs} } OPTIONAL,
    ...
}

AllowedPNI-NPN-ID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AllTrafficIndication ::= ENUMERATED {true,...}


AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {
    alternativeQoSParaSetIndex          QoSParaSetIndex,
    guaranteedFlowBitRateDL             BitRate                 OPTIONAL,
    guaranteedFlowBitRateUL             BitRate                 OPTIONAL,
    packetDelayBudget                   PacketDelayBudget       OPTIONAL,
    packetErrorRate                     PacketErrorRate         OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }   OPTIONAL,
    ...
}

AlternativeQoSParaSetItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
        { ID id-MaximumDataBurstVolume  CRITICALITY ignore  EXTENSION MaximumDataBurstVolume                PRESENCE optional }|
        { ID id-PduSetDelayBudgetDownlink   CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget     PRESENCE optional }|
        { ID id-PduSetDelayBudgetUplink     CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget     PRESENCE optional }|
        { ID id-PduSetErrorRateDownlink     CRITICALITY ignore  EXTENSION PacketErrorRate                   PRESENCE optional }|
        { ID id-PduSetErrorRateUplink       CRITICALITY ignore  EXTENSION PacketErrorRate                   PRESENCE optional },
...
}


AMF-Region-Information ::= SEQUENCE (SIZE (1..maxnoofAMFRegions)) OF GlobalAMF-Region-Information

GlobalAMF-Region-Information ::= SEQUENCE {
    plmn-ID             PLMN-Identity,
    amf-region-id       BIT STRING (SIZE (8)),
    iE-Extensions                   ProtocolExtensionContainer { {GlobalAMF-Region-Information-ExtIEs} } OPTIONAL,
    ...
}

GlobalAMF-Region-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)


AreaOfInterestInformation ::= SEQUENCE (SIZE(1..maxnoofAoIs)) OF AreaOfInterest-Item

AreaOfInterest-Item ::= SEQUENCE {
    listOfTAIsinAoI                 ListOfTAIsinAoI                                             OPTIONAL,
    listOfCellsinAoI                ListOfCells                                                 OPTIONAL,
    listOfRANNodesinAoI             ListOfRANNodesinAoI                                         OPTIONAL,
    requestReferenceID  RequestReferenceID,
    iE-Extensions                   ProtocolExtensionContainer { {AreaOfInterest-Item-ExtIEs} } OPTIONAL,
    ...
}

AreaOfInterest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


AreaScopeOfMDT-NR ::= CHOICE {  
    cellBased                   CellBasedMDT-NR,
    tABased                     TABasedMDT,
    tAIBased                    TAIBasedMDT,
    ...,
    choice-extension        ProtocolIE-Single-Container { {AreaScopeOfMDT-NR-ExtIEs} }
}

AreaScopeOfMDT-NR-ExtIEs XNAP-PROTOCOL-IES ::= {
    { ID id-PNI-NPNBasedMDT         CRITICALITY ignore  TYPE PNI-NPNBasedMDT            PRESENCE mandatory}|
    { ID id-SNPN-CellBasedMDT       CRITICALITY ignore  TYPE SNPN-CellBasedMDT          PRESENCE mandatory}|
    { ID id-SNPN-TAIBasedMDT        CRITICALITY ignore  TYPE SNPN-TAIBasedMDT           PRESENCE mandatory}|
    { ID id-SNPN-BasedMDT           CRITICALITY ignore  TYPE SNPN-BasedMDT              PRESENCE mandatory}|
    { ID id-GeographyBasedMDT       CRITICALITY ignore  TYPE GeographyBasedMDT          PRESENCE mandatory},
    ...
}
AreaScopeOfMDT-EUTRA ::= CHOICE {   
    cellBased                   CellBasedMDT-EUTRA,
    tABased                     TABasedMDT,
    tAIBased                    TAIBasedMDT,
    ...,
    choice-extension        ProtocolIE-Single-Container { {AreaScopeOfMDT-EUTRA-ExtIEs} }

}

AreaScopeOfMDT-EUTRA-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}



AreaScopeOfNeighCellsList ::= SEQUENCE (SIZE(1..maxnoofFreqforMDT)) OF AreaScopeOfNeighCellsItem
AreaScopeOfNeighCellsItem ::= SEQUENCE {
    nrFrequencyInfo             NRFrequencyInfo,
    pciListForMDT               PCIListForMDT       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { AreaScopeOfNeighCellsItem-ExtIEs} }  OPTIONAL,
    ...
}

AreaScopeOfNeighCellsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AreaScopeOfQMC ::= CHOICE { 
    cellBased                   CellBasedQMC,
    tABased                     TABasedQMC,
    tAIBased                    TAIBasedQMC,
    pLMNAreaBased               PLMNAreaBasedQMC,
    choice-extension            ProtocolIE-Single-Container { {AreaScopeOfQMC-ExtIEs} }
}

AreaScopeOfQMC-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

AS-SecurityInformation ::= SEQUENCE {
    key-NG-RAN-Star                 BIT STRING (SIZE(256)),
    ncc                             INTEGER (0..7),
    iE-Extensions                   ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,
    ...
}

AS-SecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


AssistanceDataForRANPaging ::= SEQUENCE {
    ran-paging-attempt-info         RANPagingAttemptInfo    OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {AssistanceDataForRANPaging-ExtIEs} } OPTIONAL,
    ...
}

AssistanceDataForRANPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-NPNPagingAssistanceInformation  CRITICALITY ignore  EXTENSION NPNPagingAssistanceInformation    PRESENCE optional },
    ...
}

AssistanceInformationQoE-Meas ::= INTEGER (1..16, ...)

Associated-QoSFlowInfo-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF Associated-QoSFlowInfo-Item

Associated-QoSFlowInfo-Item ::= SEQUENCE {
    mBS-QoSFlowIdentifier               QoSFlowIdentifier,
    associatedUnicastQoSFlowIdentifier  QoSFlowIdentifier,
    iE-Extensions                       ProtocolExtensionContainer { { Associated-QoSFlowInfo-Item-ExtIEs} }    OPTIONAL,
    ...
}

Associated-QoSFlowInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


AvailableCapacity ::= INTEGER (1.. 100,...)


AvailableRRCConnectionCapacityValue ::= INTEGER (0..100)


AvailableRVQoEMetrics ::= SEQUENCE {
    applicationLayerBufferLevelList                     ENUMERATED {true, ...}  OPTIONAL,
    playoutDelayForMediaStartup     ENUMERATED {true, ...}      OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {AvailableRVQoEMetrics-ExtIEs} } OPTIONAL,
    ...
}

AvailableRVQoEMetrics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


AveragingWindow ::= INTEGER (0..4095, ...)


-- B

BAPAddress ::= BIT STRING (SIZE(10))

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= SEQUENCE {
    bAPAddress      BAPAddress,
    bAPPathID       BAPPathID,
    iE-Extensions   ProtocolExtensionContainer { {BAPRoutingID-ExtIEs} }    OPTIONAL,
    ...
}

BAPRoutingID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BeamMeasurementIndicationM1 ::= ENUMERATED {true, ...}

BeamMeasurementsReportConfiguration ::= SEQUENCE {
    beamMeasurementsReportQuantity          BeamMeasurementsReportQuantity          OPTIONAL,
    maxNrofRS-IndexesToReport               MaxNrofRS-IndexesToReport       OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,
    ...
}

BeamMeasurementsReportConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BeamMeasurementsReportQuantity ::= SEQUENCE {
    rSRP                        ENUMERATED {true, ...},
    rSRQ                        ENUMERATED {true, ...},
    sINR                        ENUMERATED {true, ...},
    iE-Extensions           ProtocolExtensionContainer { { BeamMeasurementsReportQuantity-ExtIEs} } OPTIONAL,
    ...
}

BeamMeasurementsReportQuantity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BHInfoIndex ::= INTEGER (1.. maxnoofBHInfo)

BHInfoList ::= SEQUENCE (SIZE(1.. maxnoofBHInfo)) OF BHInfo-Item

BHInfo-Item ::= SEQUENCE {
    bHInfoIndex         BHInfoIndex,
    iE-Extensions           ProtocolExtensionContainer { { BHInfo-Item-ExtIEs} }    OPTIONAL,
    ...
}

BHInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BHRLCChannelID ::= BIT STRING (SIZE(16))

BAPControlPDURLCCH-List ::= SEQUENCE (SIZE(1.. maxnoofBAPControlPDURLCCHs)) OF BAPControlPDURLCCH-Item


BAPControlPDURLCCH-Item ::= SEQUENCE {
    bHRLCCHID           BHRLCChannelID,
    nexthopBAPAddress   BAPAddress,
    iE-Extensions           ProtocolExtensionContainer { { BAPControlPDURLCCH-Item-ExtIEs} }    OPTIONAL,
    ...
}

BAPControlPDURLCCH-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
BarringExemptionforEmerCallInfo::= ENUMERATED {true,...}


BluetoothMeasurementConfiguration ::= SEQUENCE {
    bluetoothMeasConfig             BluetoothMeasConfig,
    bluetoothMeasConfigNameList     BluetoothMeasConfigNameList     OPTIONAL,
    bt-rssi                         ENUMERATED {true, ...}          OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } OPTIONAL,
    ...
}

BluetoothMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))


BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {
    broadcastPLMNs                  BroadcastEUTRAPLMNs,
    tac                             TAC,
    e-utraCI                        E-UTRA-Cell-Identity,
    ranac                           RANAC OPTIONAL,
    iE-Extension                    ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,
    ...
}

BPLMN-ID-Info-EUTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {
    broadcastPLMNs                  BroadcastPLMNs,
    tac                             TAC,
    nr-CI                           NR-Cell-Identity,
    ranac                           RANAC OPTIONAL,
    iE-Extension                    ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,
    ...
}

BPLMN-ID-Info-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-ConfiguredTACIndication     CRITICALITY ignore  EXTENSION ConfiguredTACIndication       PRESENCE optional }|
    { ID id-NPN-Broadcast-Information   CRITICALITY reject  EXTENSION NPN-Broadcast-Information     PRESENCE optional },
    ...
}

BitRate ::= INTEGER (0..4000000000000,...)



BroadcastCAG-Identifier-List ::= SEQUENCE (SIZE(1..maxnoofCAGs)) OF BroadcastCAG-Identifier-Item

BroadcastCAG-Identifier-Item ::= SEQUENCE {
    cag-Identifier                  CAG-Identifier,
    iE-Extension                    ProtocolExtensionContainer { {BroadcastCAG-Identifier-Item-ExtIEs} } OPTIONAL,
    ...
}

BroadcastCAG-Identifier-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BroadcastNID-List ::= SEQUENCE (SIZE(1..maxnoofNIDs)) OF BroadcastNID-Item

BroadcastNID-Item ::= SEQUENCE {
    nid                         NID,
    iE-Extension                ProtocolExtensionContainer { {BroadcastNID-Item-ExtIEs} } OPTIONAL,
    ...
}

BroadcastNID-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastEUTRAPLMNs ::= SEQUENCE (SIZE(1..maxnoofEUTRABPLMNs)) OF PLMN-Identity


BroadcastPLMNinTAISupport-Item ::= SEQUENCE {
    plmn-id                         PLMN-Identity,
    tAISliceSupport-List            SliceSupport-List,
    iE-Extension                    ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,
    ...
}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-NPN-Support                     CRITICALITY reject  EXTENSION NPN-Support                       PRESENCE optional}|
    { ID id-ExtendedTAISliceSupportList     CRITICALITY reject  EXTENSION ExtendedSliceSupportList      PRESENCE optional}|
    { ID id-TAINSAGSupportList              CRITICALITY ignore  EXTENSION TAINSAGSupportList                PRESENCE optional}|
    { ID id-TAISliceUnavailableCellList     CRITICALITY ignore  EXTENSION TAISliceUnavailableCellList   PRESENCE optional},
    ...
}


BroadcastPNI-NPN-ID-Information ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPNI-NPN-ID-Information-Item

BroadcastPNI-NPN-ID-Information-Item ::= SEQUENCE {
    plmn-id                         PLMN-Identity,
    broadcastCAG-Identifier-List    BroadcastCAG-Identifier-List,
    iE-Extension                    ProtocolExtensionContainer { {BroadcastPNI-NPN-ID-Information-Item-ExtIEs} } OPTIONAL,
    ...
}


BroadcastPNI-NPN-ID-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


BroadcastSNPNID-List ::= SEQUENCE (SIZE(1..maxnoofSNPNIDs)) OF BroadcastSNPNID


BroadcastSNPNID ::= SEQUENCE {
    plmn-id                         PLMN-Identity,
    broadcastNID-List                               BroadcastNID-List,
    iE-Extension                    ProtocolExtensionContainer { {BroadcastSNPNID-ExtIEs} } OPTIONAL,
    ...
}

BroadcastSNPNID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- C


CAG-Identifier  ::= BIT STRING (SIZE (32))

CandidateRelayUEInfoList ::= SEQUENCE (SIZE(1..maxnoofCandidateRelayUEs)) OF CandidateRelayUEInfoItem

CandidateRelayUEInfoItem ::= SEQUENCE {
    candidateRelayUEID      BIT STRING(SIZE(24)),
    iE-Extensions           ProtocolExtensionContainer { { CandidateRelayUEInfoItem-ExtIEs } }      OPTIONAL,
    ...
}

CandidateRelayUEInfoItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CapacityValue ::= INTEGER (0..100)



CapacityValueInfo ::= SEQUENCE {
    capacityValue               CapacityValue,
    ssbAreaCapacityValueList    SSBAreaCapacityValue-List   OPTIONAL,
    iE-Extension                ProtocolExtensionContainer { {CapacityValueInfo-ExtIEs} } OPTIONAL,
    ...
}

CapacityValueInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


Cause ::= CHOICE {
    radioNetwork        CauseRadioNetworkLayer,
    transport           CauseTransportLayer,
    protocol            CauseProtocol,
    misc                CauseMisc,
    choice-extension    ProtocolIE-Single-Container { {Cause-ExtIEs} }
}

Cause-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

CauseRadioNetworkLayer ::= ENUMERATED {
    cell-not-available,
    handover-desirable-for-radio-reasons,
    handover-target-not-allowed,
    invalid-AMF-Set-ID,
    no-radio-resources-available-in-target-cell,
    partial-handover,
    reduce-load-in-serving-cell,
    resource-optimisation-handover,
    time-critical-handover,
    tXnRELOCoverall-expiry,
    tXnRELOCprep-expiry,
    unknown-GUAMI-ID,
    unknown-local-NG-RAN-node-UE-XnAP-ID,
    inconsistent-remote-NG-RAN-node-UE-XnAP-ID,
    encryption-and-or-integrity-protection-algorithms-not-supported,
    not-used-causes-value-1,
    multiple-PDU-session-ID-instances,
    unknown-PDU-session-ID,
    unknown-QoS-Flow-ID,
    multiple-QoS-Flow-ID-instances,
    switch-off-ongoing,
    not-supported-5QI-value,
    tXnDCoverall-expiry,
    tXnDCprep-expiry,
    action-desirable-for-radio-reasons,
    reduce-load,
    resource-optimisation,
    time-critical-action,
    target-not-allowed,
    no-radio-resources-available,
    invalid-QoS-combination,
    encryption-algorithms-not-supported,
    procedure-cancelled,
    rRM-purpose,
    improve-user-bit-rate,
    user-inactivity,
    radio-connection-with-UE-lost,
    failure-in-the-radio-interface-procedure,
    bearer-option-not-supported,
    up-integrity-protection-not-possible,
    up-confidentiality-protection-not-possible,
    resources-not-available-for-the-slice-s,
    ue-max-IP-data-rate-reason,
    cP-integrity-protection-failure,
    uP-integrity-protection-failure,
    slice-not-supported-by-NG-RAN,
    mN-Mobility,
    sN-Mobility,
    count-reaches-max-value,
    unknown-old-NG-RAN-node-UE-XnAP-ID,
    pDCP-Overload,
    drb-id-not-available,
    unspecified,
    ...,
    ue-context-id-not-known,
    non-relocation-of-context,
    cho-cpc-resources-tobechanged,
    rSN-not-available-for-the-UP,
    npn-access-denied,
    report-characteristics-empty,
    existing-measurement-ID,
    measurement-temporarily-not-available,
    measurement-not-supported-for-the-object,
    ue-power-saving,
    not-existing-NG-RAN-node2-Measurement-ID,
    insufficient-ue-capabilities,
    normal-release,
    value-out-of-allowed-range,
    scg-activation-deactivation-failure,
    scg-deactivation-failure-due-to-data-transmission,
    ssb-not-available,
    lTM-triggered,
    no-Backhaul-Resource,
    mIAB-node-not-authorized,
    iAB-not-authorized
}

CauseTransportLayer ::= ENUMERATED {
    transport-resource-unavailable,
    unspecified,
    ...
}

CauseProtocol ::= ENUMERATED {
    transfer-syntax-error,
    abstract-syntax-error-reject,
    abstract-syntax-error-ignore-and-notify,
    message-not-compatible-with-receiver-state,
    semantic-error,
    abstract-syntax-error-falsely-constructed-message,
    unspecified,
    ...
}

CauseMisc ::= ENUMERATED {
    control-processing-overload,
    hardware-failure,
    o-and-M-intervention,
    not-enough-user-plane-processing-resources,
    unspecified,
    ...
}

CellAssistanceInfo-NR   ::= CHOICE {
    limitedNR-List              SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF NR-CGI,
    full-List                   ENUMERATED {all-served-cells-NR, ...},
    choice-extension            ProtocolIE-Single-Container { {CellAssistanceInfo-NR-ExtIEs} }
}

CellAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

CellAndCapacityAssistanceInfo-NR    ::= SEQUENCE {
    maximumCellListSize                 MaximumCellListSize                             OPTIONAL,
    cellAssistanceInfo-NR       CellAssistanceInfo-NR               OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-NR-ExtIEs} }   OPTIONAL,
    ...
}


CellAndCapacityAssistanceInfo-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CellAndCapacityAssistanceInfo-EUTRA ::= SEQUENCE {
    maximumCellListSize                 MaximumCellListSize                             OPTIONAL,
    cellAssistanceInfo-EUTRA            CellAssistanceInfo-EUTRA                OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CellAndCapacityAssistanceInfo-EUTRA-ExtIEs} }    OPTIONAL,
    ...
}


CellAndCapacityAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CellAssistanceInfo-EUTRA    ::= CHOICE {
    limitedEUTRA-List           SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI,
    full-List                   ENUMERATED {all-served-cells-E-UTRA, ...},
    choice-extension            ProtocolIE-Single-Container { {CellAssistanceInfo-EUTRA-ExtIEs} }
}

CellAssistanceInfo-EUTRA-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

CellBasedMDT-NR::= SEQUENCE {
    cellIdListforMDT-NR CellIdListforMDT-NR,
    iE-Extensions       ProtocolExtensionContainer { {CellBasedMDT-NR-ExtIEs} } OPTIONAL,
    ...
}

CellBasedMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CellIdListforMDT-NR ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF NR-CGI

CellBasedQMC::= SEQUENCE {
    cellIdListforQMC        CellIdListforQMC,
    iE-Extensions       ProtocolExtensionContainer { {CellBasedQMC-ExtIEs} } OPTIONAL,
    ...
}

CellBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CellIdListforQMC ::= SEQUENCE (SIZE(1..maxnoofCellIDforQMC)) OF GlobalNG-RANCell-ID


CellBasedMDT-EUTRA::= SEQUENCE {
    cellIdListforMDT-EUTRA  CellIdListforMDT-EUTRA,
    iE-Extensions       ProtocolExtensionContainer { {CellBasedMDT-EUTRA-ExtIEs} } OPTIONAL,
    ...
}

CellBasedMDT-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
CellIdListforMDT-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF E-UTRA-CGI


CellCapacityClassValue ::= INTEGER (1..100,...)

CellDeploymentStatusIndicator ::= ENUMERATED {pre-change-notification, ...}

CellGroupID ::= INTEGER (0..maxnoofSCellGroups)


CellMeasurementResult ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementResult-Item

CellMeasurementResult-Item  ::= SEQUENCE {
    cell-ID                             GlobalNG-RANCell-ID,
    radioResourceStatus                 RadioResourceStatus             OPTIONAL,
    tNLCapacityIndicator                TNLCapacityIndicator            OPTIONAL,
    compositeAvailableCapacityGroup     CompositeAvailableCapacityGroup OPTIONAL,
    sliceAvailableCapacity              SliceAvailableCapacity          OPTIONAL,
    numberofActiveUEs                   NumberofActiveUEs               OPTIONAL,
    rRCConnections                      RRCConnections                  OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CellMeasurementResult-Item-ExtIEs} } OPTIONAL,
    ...
}


CellMeasurementResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-NR-U-Channel-List   CRITICALITY ignore  EXTENSION NR-U-Channel-List PRESENCE optional },
    ...
}


CellReplacingInfo ::= SEQUENCE {
    replacingCells                  ReplacingCells,
    iE-Extensions                   ProtocolExtensionContainer { {CellReplacingInfo-ExtIEs}}    OPTIONAL,
    ...
}

CellReplacingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CellToReport ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReport-Item

CellToReport-Item   ::= SEQUENCE {
    cell-ID                                 GlobalNG-RANCell-ID,
    sSBToReport-List                        SSBToReport-List            OPTIONAL,
    sliceToReport-List                      SliceToReport-List          OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CellToReport-Item-ExtIEs} }  OPTIONAL,
    ...
}


CellToReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CellToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReportForDataCollection-Item

CellToReportForDataCollection-Item  ::= SEQUENCE {
    cell-ID                             GlobalNG-RANCell-ID,
    iE-Extensions                       ProtocolExtensionContainer { { CellToReportForDataCollection-Item-ExtIEs} } OPTIONAL,
    ...
}

CellToReportForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SliceToReportForDataCollection-List     CRITICALITY ignore  EXTENSION SliceToReportForDataCollection-List   PRESENCE optional },
    ...
}
CellBasedUETrajectoryPrediction ::= SEQUENCE (SIZE(1..maxnoofCellsTrajectoryPredict)) OF PredictedUETrajectory-Item



CellMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementInitiationResult-Item

CellMeasurementInitiationResult-Item ::= SEQUENCE {
    cellID                                          GlobalNG-RANCell-ID,
    cellMeasurementFailureCause-List                CellMeasurementFailureCause-List OPTIONAL,
    iE-Extensions                                   ProtocolExtensionContainer { { CellMeasurementInitiationResult-Item-ExtIEs} }   OPTIONAL,
    ...
}
CellMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SliceMeasurementInitiationResult        CRITICALITY reject  EXTENSION SliceMeasurementInitiationResult  PRESENCE optional},
    ...
}

CLI-MeasurementResult-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CLI-MeasurementResult-Item

CLI-MeasurementResult-Item ::= SEQUENCE {
    cellID                                          GlobalNG-RANCell-ID,
    ssbIndex                                        INTEGER(0..63,...)                  OPTIONAL,
    nZP-CSI-RS-ResourceIndication                   INTEGER(1..64,...)                  OPTIONAL,
    cLI-MitigationIndication                        CLI-MitigationIndication            OPTIONAL,
    iE-Extensions                                   ProtocolExtensionContainer { {CLI-MeasurementResult-Item-ExtIEs} }  OPTIONAL,
    ...
}

CLI-MeasurementResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CLI-MitigationIndication ::= ENUMERATED {true,...}

CellMeasurementResultForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellInfoResultForDataCollection-Item

CellInfoResultForDataCollection-Item ::= SEQUENCE {
    cellID                                          GlobalNG-RANCell-ID,
    predictedRadioResourceStatus                    RadioResourceStatus                 OPTIONAL,
    predictedNumberofActiveUEs                      NumberofActiveUEs                   OPTIONAL,
    predictedRRCConnections                         RRCConnections                      OPTIONAL,
    iE-Extensions                                   ProtocolExtensionContainer { {CellInfoResultForDataCollection-Item-ExtIEs} }    OPTIONAL,
    ...
}
CellInfoResultForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-PredictedSliceAvailableCapacity     CRITICALITY ignore  EXTENSION PredictedSliceAvailableCapacityGroup  PRESENCE optional},
    ...
}


Cell-Type-Choice ::= CHOICE {
    ng-ran-e-utra           E-UTRA-Cell-Identity,
    ng-ran-nr               NR-Cell-Identity,
    e-utran                 E-UTRA-Cell-Identity,
    choice-extension        ProtocolIE-Single-Container { { Cell-Type-Choice-ExtIEs} }
}

Cell-Type-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

CellMeasurementFailureCause-List ::= SEQUENCE (SIZE(1..maxFailedCellMeasObjects)) OF CellMeasurementFailureCause-Item

CellMeasurementFailureCause-Item ::= SEQUENCE {
    cellmeasurementFailedReportCharacteristics      BIT STRING(SIZE(32)),
    cause                                       Cause,
    iE-Extensions                               ProtocolExtensionContainer { { CellMeasurementFailureCause-Item-ExtIEs} } OPTIONAL,
    ...
}

CellMeasurementFailureCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CHOConfiguration ::= SEQUENCE {
    choCandidateCell-List               CHOCandidateCell-List,
    iE-Extensions                       ProtocolExtensionContainer { { CHOConfiguration-ExtIEs} }   OPTIONAL,
    ...
}

CHOConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CHOCandidateCell-List ::= SEQUENCE (SIZE(1..maxnoofCellsinCHO)) OF CHOCandidateCell-Item

CHOCandidateCell-Item ::= SEQUENCE {
    choCandidateCellID                  GlobalNG-RANCell-ID,
    choExecutionCondition-List          CHOExecutionCondition-List,
    iE-Extensions                       ProtocolExtensionContainer { { CHOCandidateCell-Item-ExtIEs} }  OPTIONAL,
    ...
}

CHOCandidateCell-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CHOExecutionCondition-List ::= SEQUENCE (SIZE(1..maxnoofCHOexecutioncond)) OF CHOExecutionCondition-Item

CHOExecutionCondition-Item ::= SEQUENCE {   measObjectContainer                 MeasObjectContainer,
    reportConfigContainer               ReportConfigContainer,
    iE-Extensions                       ProtocolExtensionContainer { { CHOExecutionCondition-Item-ExtIEs} } OPTIONAL,
    ...
}

CHOExecutionCondition-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ClockQualityAcceptanceCriteria ::= SEQUENCE {
    synchronisationState            BIT STRING (SIZE(8, ...))                   OPTIONAL,
    traceabletoUTC                  ENUMERATED {true, ...}                      OPTIONAL,
    traceabletoGNSS                 ENUMERATED {true, ...}                      OPTIONAL,
    clockFrequencyStability         BIT STRING (SIZE(16))                       OPTIONAL,
    clockAccuracy                   INTEGER (1..40000000, ...)                  OPTIONAL,
    parentTimeSource                BIT STRING (SIZE(16, ...))                  OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { ClockQualityAcceptanceCriteria-ExtIEs} } OPTIONAL,
    ...
}

ClockQualityAcceptanceCriteria-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ClockQualityReportingControlInfo ::= SEQUENCE {
    clockQualityDetailLevel     ClockQualityDetailLevel,
    iE-Extensions               ProtocolExtensionContainer { {ClockQualityReportingControlInfo-ExtIEs} } OPTIONAL,
    ...
}

ClockQualityReportingControlInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ClockQualityDetailLevel ::= CHOICE {
    clockQualityMetrics         NULL,
    acceptanceIndication        ClockQualityAcceptanceCriteria,
    choice-extension            ProtocolIE-Single-Container { {ClockQualityDetailLevel-ExtIEs} }
}

ClockQualityDetailLevel-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


CapabilityForBATAdaptation ::= ENUMERATED {true, ...}

CompositeAvailableCapacityGroup ::= SEQUENCE {
    compositeAvailableCapacityDownlink      CompositeAvailableCapacity,
    compositeAvailableCapacityUplink        CompositeAvailableCapacity,
    iE-Extensions               ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} }    OPTIONAL,
    ...
}

CompositeAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CompositeAvailableCapacitySupplementaryUplink   CRITICALITY ignore  EXTENSION CompositeAvailableCapacity    PRESENCE optional   },
    ...
}

CompositeAvailableCapacity ::= SEQUENCE {
    cellCapacityClassValue      CellCapacityClassValue              OPTIONAL,
    capacityValueInfo           CapacityValueInfo, -- this IE represents the IE "CapacityValue" in 9.2.2.52, it’s used to distinguish the "CapacityValue" in 9.2.2.54
    iE-Extensions               ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} }OPTIONAL,
    ...
}

CompositeAvailableCapacity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ControlPlaneTrafficType ::= INTEGER (1..3, ...)

CHO-MRDC-EarlyDataForwarding ::= ENUMERATED {stop, ...}

CHO-MRDC-Indicator ::= ENUMERATED {true, ..., coordination-only }



CHOtrigger ::= ENUMERATED {
    cho-initiation,
    cho-replace,
    ...
}

CHOinformation-Req ::= SEQUENCE {
    cho-trigger                     CHOtrigger,
    targetNG-RANnodeUEXnAPID        NG-RANnodeUEXnAPID                                      OPTIONAL
        -- This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace" --,
    cHO-EstimatedArrivalProbability CHO-Probability                                         OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CHOinformation-Req-ExtIEs} } OPTIONAL,
    ...
}

CHOinformation-Req-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-CHOTimeBasedInformation     CRITICALITY reject  EXTENSION CHOTimeBasedInformation       PRESENCE optional }|
    { ID id-CHO-Maxnoof-CondReconfig    CRITICALITY reject  EXTENSION CHO-Maxnoof-CondReconfig      PRESENCE optional },
    ...
}

CHOTimeBasedInformation ::= SEQUENCE {
    cHO-HOWindowStart           CHO-HandoverWindowStart,
    cHO-HOWindowDuration        CHO-HandoverWindowDuration,
    iE-Extensions               ProtocolExtensionContainer { {CHOTimeBasedInformation-ExtIEs} } OPTIONAL,
    ...
}

CHOTimeBasedInformation-ExtIEs  XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CHOinformation-Ack ::= SEQUENCE {
    requestedTargetCellGlobalID     Target-CGI,
    maxCHOoperations                MaxCHOpreparations                                          OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CHOinformation-Ack-ExtIEs} } OPTIONAL,
    ...
}

CHOinformation-Ack-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-CHO-CPAC-Info               CRITICALITY reject      EXTENSION CHO-CPAC-Information  PRESENCE optional },
    ...
}


CHOinformation-AddReq ::= SEQUENCE {
    source-M-NGRAN-node-ID              GlobalNG-RANNode-ID,
    source-M-NGRAN-node-UE-XnAP-ID      NG-RANnodeUEXnAPID,
    cHO-EstimatedArrivalProbability     CHO-Probability                                                 OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CHOinformation-AddReq-ExtIEs} }  OPTIONAL,
    ...
}

CHOinformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CHOinformation-AddReqAck ::= SEQUENCE {
    pCell-ID                        GlobalNG-RANCell-ID         OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CHOinformation-AddReqAck-ExtIEs} }   OPTIONAL,
    ...
}

CHOinformation-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CHOinformation-ModReq ::= SEQUENCE {
    conditionalReconfig                 ENUMERATED {intra-mn-cho, ...},
    cHO-EstimatedArrivalProbability     CHO-Probability                                                 OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CHOinformation-ModReq-ExtIEs} }  OPTIONAL,
    ...
}

CHOinformation-ModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CHO-Maxnoof-CondReconfig ::= INTEGER (1..8,...)

CHO-CPAC-Information ::= SEQUENCE {
    cHO-CPAC-config-indicator       CHO-CPAC-Config-Indicator       OPTIONAL,
    cHO-target-SN-node-list         CHO-target-SN-node-list,
    iE-Extensions                   ProtocolExtensionContainer { {CHO-CPAC-Information-ExtIEs}} OPTIONAL,
    ...
}

CHO-CPAC-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CHO-CPAC-Config-Indicator ::= ENUMERATED {
    cho-only-not-prepared,
    ...
}


CHO-Probability ::= INTEGER (1..100)

CHO-HandoverWindowStart ::= INTEGER (0.. 549755813887)

CHO-HandoverWindowDuration ::= INTEGER (1..6000)

CHO-target-SN-node-list ::= SEQUENCE (SIZE(1.. maxnoofTargetSNs)) OF CHO-target-SN-node-Item

CHO-target-SN-node-Item ::= SEQUENCE {
    target-S-NG-RANnodeID               GlobalNG-RANNode-ID,
    pduSessionResourcesAdmittedList     PDUSessionResourcesAdmitted-List,
    cho-Candidate-PSCells-list          CHO-Candidate-PSCells-list,
    iE-Extensions                   ProtocolExtensionContainer { {CHO-target-SN-node-Item-ExtIEs}}  OPTIONAL,
    ...
}

CHO-target-SN-node-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}


CHO-Candidate-PSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CHO-Candidate-PSCells-Item

CHO-Candidate-PSCells-Item ::= SEQUENCE {
    pscell-id                               NR-CGI,
    target2source-NG-RANNode-Container      OCTET STRING,
    iE-Extensions                   ProtocolExtensionContainer { {CHO-Candidate-PSCells-Item-ExtIEs}}   OPTIONAL,
    ...
}

CHO-Candidate-PSCells-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

Circle ::= SEQUENCE {
    referenceLocation           OCTET STRING,
    distanceRadius              INTEGER(1..65535, ...),
    iE-Extensions               ProtocolExtensionContainer { { Circle-ExtIEs} } OPTIONAL,
    ...
}
Circle-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CNsubgroupID ::= INTEGER (0..7, ...)

CompleteCandidateConfig-Indicator ::= ENUMERATED {complete-candidate-config, ...}

Conditional-Reconfig-List ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF Conditional-Reconfig-Item

Conditional-Reconfig-Item ::= SEQUENCE {
    pCell-ID            Target-CGI,
    pSCell-ID           NR-CGI              OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {Conditional-Reconfig-Item-ExtIEs} }   OPTIONAL,
    ...

}

Conditional-Reconfig-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ConfiguredTACIndication ::= ENUMERATED {
    true,
    ...
}


Connectivity-Support        ::= SEQUENCE {
    eNDC-Support            ENUMERATED {supported, not-supported, ...},
    iE-Extensions           ProtocolExtensionContainer { {Connectivity-Support-ExtIEs} }    OPTIONAL,
    ...
}

Connectivity-Support-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ContainerAppLayerMeasConfig ::= OCTET STRING (SIZE (1..8000))

COUNT-PDCP-SN12 ::= SEQUENCE {
    pdcp-SN12                       INTEGER (0..4095),
    hfn-PDCP-SN12                   INTEGER (0..1048575),
    iE-Extensions                   ProtocolExtensionContainer { {COUNT-PDCP-SN12-ExtIEs} } OPTIONAL,
    ...
}

COUNT-PDCP-SN12-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


COUNT-PDCP-SN18 ::= SEQUENCE {
    pdcp-SN18                       INTEGER (0..262143),
    hfn-PDCP-SN18                   INTEGER (0..16383),
    iE-Extensions                   ProtocolExtensionContainer { {COUNT-PDCP-SN18-ExtIEs} } OPTIONAL,
    ...
}

COUNT-PDCP-SN18-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CoverageModificationCause ::=   ENUMERATED {
    coverage,
    cell-edge-capacity,
    ...,
    network-energy-saving}

Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofCellsinNG-RANnode)) OF Coverage-Modification-List-Item

Coverage-Modification-List-Item ::= SEQUENCE {
    globalNG-RANCell-ID             GlobalCell-ID,
    cellCoverageState               INTEGER (0..63, ...),
    cellDeploymentStatusIndicator   CellDeploymentStatusIndicator           OPTIONAL,
    cellReplacingInfo               CellReplacingInfo                       OPTIONAL,
-- This IE shall be present if the Cell Deployment Status Indicator IE is present.
    sSB-Coverage-Modification-List  SSB-Coverage-Modification-List,
    iE-Extension            ProtocolExtensionContainer { { Coverage-Modification-List-Item-ExtIEs} } OPTIONAL,
    ...
}
Coverage-Modification-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-CoverageModificationCause       CRITICALITY ignore  EXTENSION CoverageModificationCause         PRESENCE optional},
    ...
}


CPTransportLayerInformation ::= CHOICE {
    endpointIPAddress           TransportLayerAddress,
    choice-extension            ProtocolIE-Single-Container { {CPTransportLayerInformation-ExtIEs} }
}

CPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {
    { ID id-EndpointIPAddressAndPort        CRITICALITY reject  TYPE EndpointIPAddressAndPort       PRESENCE mandatory},
    ...
}

CPACcandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCells-item

CPACcandidatePSCells-item ::= SEQUENCE {
    pscell-id                       NR-CGI,
    iE-Extensions                   ProtocolExtensionContainer { {CPACcandidatePSCells-item-ExtIEs}}    OPTIONAL,
    ...
}

CPACcandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CPACcandidatePSCells-wotherInfo-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCells-wotherInfo-item

CPACcandidatePSCells-wotherInfo-item ::= SEQUENCE {
    pscell-id                           NR-CGI,
    s-CPAC-CompleteCandidateConfig-Indicator        CompleteCandidateConfig-Indicator       OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {CPACcandidatePSCells-wotherInfo-item-ExtIEs}} OPTIONAL,
    ...
}

CPACcandidatePSCells-wotherInfo-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CPACConfiguration ::= SEQUENCE {
    cpacCandidateCell-List              CPACCandidateCell-List,
    iE-Extensions                       ProtocolExtensionContainer { { CPACConfiguration-ExtIEs} }  OPTIONAL,
    ...
}

CPACConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CPACCandidateCell-List ::= SEQUENCE (SIZE(1..maxnoofPSCellsinCPAC)) OF CPACCandidateCell-Item

CPACCandidateCell-Item ::= SEQUENCE {
    cpacCandidateCellID                 GlobalNG-RANCell-ID,
    cpacExecutionCondition-List         CPACExecutionCondition-List,
    iE-Extensions                       ProtocolExtensionContainer { { CPACCandidateCell-Item-ExtIEs} } OPTIONAL,
    ...
}

CPACCandidateCell-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CPACExecutionCondition-List ::= SEQUENCE (SIZE(1..maxnoofCPACexecutioncond)) OF CPACExecutionCondition-Item

CPACExecutionCondition-Item ::= SEQUENCE {  
    measObjectContainer                 MeasObjectContainer,
    reportConfigContainer               ReportConfigContainer,
    iE-Extensions                       ProtocolExtensionContainer { { CPACExecutionCondition-Item-ExtIEs} }    OPTIONAL,
    ...
}

CPACExecutionCondition-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CPCindicator ::= ENUMERATED {cpc-initiation, cpc-modification, cpc-cancellation, ...}

CPAInformationRequest ::= SEQUENCE {
    max-no-of-pscells                   INTEGER (1..maxnoofPSCellCandidates, ...),
    cpac-EstimatedArrivalProbability    CHO-Probability     OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CPAInformationRequest-ExtIEs} } OPTIONAL,
    ...
}

CPAInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-S-CPAC-Request-Info             CRITICALITY reject      EXTENSION   S-CPAC-Request-Info                     PRESENCE        optional}|
    { ID id-S-CPAC-ReferenceConfigRequest   CRITICALITY ignore      EXTENSION   S-CPAC-ReferenceConfig-Request      PRESENCE        optional},
    ...
}

CPAInformationAck ::= SEQUENCE {
    candidate-pscells                   CPACcandidatePSCells-list,
    iE-Extensions       ProtocolExtensionContainer { { CPAInformationAck-ExtIEs} } OPTIONAL,
    ...
}

CPAInformationAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CPACcandidatePSCells-wotherInfo-list    CRITICALITY reject      EXTENSION CPACcandidatePSCells-wotherInfo-list      PRESENCE optional},
    ...
}

CPCInformationRequired::= SEQUENCE {
    cpc-target-sn-required-list     CPC-target-SN-required-list,
    iE-Extensions                   ProtocolExtensionContainer { {CPCInformationRequired-ExtIEs} } OPTIONAL,
    ...
}

CPCInformationRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-S-CPAC-Request              CRITICALITY reject      EXTENSION   S-CPAC-Request                  PRESENCE        optional},
    ...
}

CPC-target-SN-required-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-required-list-Item

CPC-target-SN-required-list-Item    ::= SEQUENCE {
    target-S-NG-RANnodeID               GlobalNG-RANNode-ID,
    cpc-indicator                       CPCindicator,
    max-no-of-pscells                   INTEGER (1..maxnoofPSCellCandidates, ...),
    cpac-EstimatedArrivalProbability    CHO-Probability                                                                     OPTIONAL,
    sN-to-MN-Container                  OCTET STRING,
    iE-Extensions                       ProtocolExtensionContainer { { CPC-target-SN-required-list-Item-ExtIEs} }   OPTIONAL,
    ...
}

CPC-target-SN-required-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



CPCInformationConfirm ::= SEQUENCE {
    cpc-target-sn-confirm-list CPC-target-SN-confirm-list,
    iE-Extensions       ProtocolExtensionContainer { { CPCInformationConfirm-ExtIEs} } OPTIONAL,
    ...
}

CPCInformationConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CPC-target-SN-confirm-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-confirm-list-Item

CPC-target-SN-confirm-list-Item ::= SEQUENCE {
    target-S-NG-RANnodeID           GlobalNG-RANNode-ID,
    candidate-pscells               CPACcandidatePSCells-list,
    iE-Extensions                   ProtocolExtensionContainer { { CPC-target-SN-confirm-list-Item-ExtIEs} } OPTIONAL,
    ...
}

CPC-target-SN-confirm-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CPAC-Preparation-Type       CRITICALITY ignore      EXTENSION CPAC-Preparation-Type     PRESENCE optional},
    ...
}

CPAInformationModReq ::= SEQUENCE {
    max-no-of-pscells                   INTEGER (1..8, ...) OPTIONAL,
    cpac-EstimatedArrivalProbability    CHO-Probability     OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CPAInformationModReq-ExtIEs} } OPTIONAL,
    ...
}

CPAInformationModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-S-CPAC-Request-Info                 CRITICALITY reject      EXTENSION   S-CPAC-Request-Info                 PRESENCE        optional}|
    { ID id-S-CPAC-ReferenceConfigRequest       CRITICALITY ignore      EXTENSION   S-CPAC-ReferenceConfig-Request  PRESENCE        optional}|
    { ID id-S-CPAC-InterSN-ExecutionNotify      CRITICALITY reject      EXTENSION   S-CPAC-InterSN-ExecutionNotify  PRESENCE        optional},
    ...
}

CPAInformationModReqAck ::= SEQUENCE {
    candidate-pscells                   CPACcandidatePSCells-list,
    iE-Extensions                       ProtocolExtensionContainer { { CPAInformationModReqAck-ExtIEs} } OPTIONAL,
    ...
}

CPAInformationModReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CPACcandidatePSCells-wotherInfo-list    CRITICALITY reject      EXTENSION CPACcandidatePSCells-wotherInfo-list  PRESENCE optional},
    ...
}

CPC-DataForwarding-Indicator ::= ENUMERATED {triggered, early-data-transmission-stop, ..., coordination-only}

CPAC-Preparation-Type ::= ENUMERATED {s-cpac, ...}

CPACInformationModRequired ::= SEQUENCE {
    candidate-pscells   CPACcandidatePSCells-list,
    iE-Extensions       ProtocolExtensionContainer { { CPACInformationModRequired-ExtIEs} } OPTIONAL,
    ...
}

CPACInformationModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CPACcandidatePSCells-wotherInfo-list    CRITICALITY reject      EXTENSION CPACcandidatePSCells-wotherInfo-list      PRESENCE optional},
    ...
}

CPCInformationUpdate::= SEQUENCE {
    cpc-target-sn-list                  CPC-target-SN-mod-list,
    iE-Extensions       ProtocolExtensionContainer { { CPCInformationUpdate-ExtIEs} } OPTIONAL,
    ...
}

CPCInformationUpdate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CPC-target-SN-mod-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-mod-item

CPC-target-SN-mod-item ::= SEQUENCE {
    target-S-NG-RANnodeID               GlobalNG-RANNode-ID,
    candidate-pscells                   CPCInformationUpdatePSCells-list,
    iE-Extensions       ProtocolExtensionContainer { {CPC-target-SN-mod-item-ExtIEs} } OPTIONAL,
    ...
}

CPC-target-SN-mod-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CPCInformationUpdatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPCInformationUpdatePSCells-item

CPCInformationUpdatePSCells-item ::= SEQUENCE {
    pscell-id                       NR-CGI,
    iE-Extensions                   ProtocolExtensionContainer { {CPCInformationUpdatePSCells-item-ExtIEs}} OPTIONAL,
    ...
}

CPCInformationUpdatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CriticalityDiagnostics ::= SEQUENCE {
    procedureCode                   ProcedureCode                   OPTIONAL,
    triggeringMessage               TriggeringMessage               OPTIONAL,
    procedureCriticality            Criticality                     OPTIONAL,
    iEsCriticalityDiagnostics       CriticalityDiagnostics-IE-List  OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }  OPTIONAL,
    ...
}

CriticalityDiagnostics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF
    SEQUENCE {
        iECriticality           Criticality,
        iE-ID                   ProtocolIE-ID,
        typeOfError             TypeOfError,
        iE-Extensions           ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,
        ...
}

CriticalityDiagnostics-IE-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


C-RNTI ::= BIT STRING (SIZE(16))

CSI-RS-MTC-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSconfigurations)) OF CSI-RS-MTC-Configuration-Item

CSI-RS-MTC-Configuration-Item ::= SEQUENCE {
    csi-RS-Index            INTEGER(0..95),
    csi-RS-Status           ENUMERATED {activated, deactivated, ...},
    csi-RS-Neighbour-List   CSI-RS-Neighbour-List OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { CSI-RS-MTC-Configuration-Item-ExtIEs} }  OPTIONAL,
    ...
}

CSI-RS-MTC-Configuration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CSI-RS-Neighbour-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSneighbourCells)) OF CSI-RS-Neighbour-Item

CSI-RS-Neighbour-Item ::= SEQUENCE {
    nr-cgi                          NR-CGI,
    csi-RS-MTC-Neighbour-List   CSI-RS-MTC-Neighbour-List OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { CSI-RS-Neighbour-Item-ExtIEs} }  OPTIONAL,
    ...
}

CSI-RS-Neighbour-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CSI-RS-MTC-Neighbour-List ::= SEQUENCE (SIZE(1.. maxnoofCSIRSneighbourCellsInMTC)) OF CSI-RS-MTC-Neighbour-Item

CSI-RS-MTC-Neighbour-Item ::= SEQUENCE {
    csi-RS-Index        INTEGER(0..95),
    iE-Extensions       ProtocolExtensionContainer { { CSI-RS-MTC-Neighbour-Item-ExtIEs} }  OPTIONAL,
    ...
}


CSI-RS-MTC-Neighbour-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



CyclicPrefix-E-UTRA-DL ::= ENUMERATED {
    normal,
    extended,
    ...
}


CyclicPrefix-E-UTRA-UL ::= ENUMERATED {
    normal,
    extended,
    ...
}

CSI-RSTransmissionIndication ::= ENUMERATED {
    activated,
    deactivated,
    ...
}

CAGListforMDT ::= SEQUENCE (SIZE(1.. maxnoofCAGforMDT))OF CAGListforMDTItem 

CAGListforMDTItem ::= SEQUENCE {
    plmnID              PLMN-Identity,
    cAGID               CAG-Identifier,
    iE-Extensions       ProtocolExtensionContainer { {CAGListforMDTItem-ExtIEs} }   OPTIONAL,
    ...
}

CAGListforMDTItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CSIResourceConfiguration ::= SEQUENCE {
    cSIResourceConfigurationToAddModList            OCTET STRING OPTIONAL,
    cSIResourceConfigurationToReleaseList           OCTET STRING OPTIONAL,
    iE-Extensions                                   ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} } OPTIONAL,
    ...
}


CSIResourceConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

CompleteCandidateConfigurationIndicator ::= ENUMERATED {complete, ...}

CSI-RSResourceConfiguration ::= SEQUENCE {
    periodicNZP-CSI-RS-ResourceConfiguration            NZP-CSI-RS-ResourceConfiguration    OPTIONAL,
    semiPersistentNZP-CSI-RS-ResourceConfiguration      NZP-CSI-RS-ResourceConfiguration    OPTIONAL,
    periodicNZP-CSI-RS-ResourceSetConfiguration         NZP-CSI-RS-ResourceSetConfiguration OPTIONAL,
    semiPersistentNZP-CSI-RS-ResourceSetConfiguration   NZP-CSI-RS-ResourceSetConfiguration OPTIONAL,
    periodicCSI-IM-ResourceConfiguration                CSI-IM-ResourceConfiguration        OPTIONAL,
    semiPersistentCSI-IM-ResourceConfiguration          CSI-IM-ResourceConfiguration        OPTIONAL,
    iE-Extensions                                       ProtocolExtensionContainer { { CSI-RSResourceConfiguration-ExtIEs} } OPTIONAL,
    ...
}


CSI-RSResourceConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NZP-CSI-RS-ResourceConfiguration ::= SEQUENCE { cSI-RSResourceToAddModList          OCTET STRING OPTIONAL,
    cSI-RSResourceToReleaseList         OCTET STRING OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { NZP-CSI-RS-ResourceConfiguration-ExtIEs} } OPTIONAL,
    ...
}


NZP-CSI-RS-ResourceConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NZP-CSI-RS-ResourceSetConfiguration ::= SEQUENCE {
    cSI-RSResourceSetToAddModList               OCTET STRING OPTIONAL,
    cSI-RSResourceSetToReleaseList              OCTET STRING OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { NZP-CSI-RS-ResourceSetConfiguration-ExtIEs} } OPTIONAL,
...
}


NZP-CSI-RS-ResourceSetConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
...
}


CSI-IM-ResourceConfiguration ::= SEQUENCE {
    cSI-IMResourceToAddModList                  OCTET STRING OPTIONAL,
    cSI-IMResourceToReleaseList                 OCTET STRING OPTIONAL,
    cSI-IMResourceSetToAddModList               OCTET STRING OPTIONAL,
    cSI-IMResourceSetToReleaseList              OCTET STRING OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CSI-IM-ResourceConfiguration-ExtIEs} } OPTIONAL,
...
}


CSI-IM-ResourceConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


CellSwitchTAInformation-List ::= SEQUENCE (SIZE (1.. maxnoofTAList)) OF CellSwitchTAInformation-Item


CellSwitchTAInformation-Item ::= SEQUENCE {
    candidateCellID                 NR-CGI,
    tAValue                         TAValue,
    tagIDPointer                    TagIDPointer OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { CellSwitchTAInformation-List-ExtIEs } }  OPTIONAL,
    ...
}

CellSwitchTAInformation-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



CSI-RSCoordinationRequestList ::= SEQUENCE (SIZE(1.. maxnoofCSIResourceConfigurations)) OF CSI-RSCoordinationRequest-Item

CSI-RSCoordinationRequest-Item  ::= SEQUENCE {
    transmissionRequest                 ENUMERATED{activate, deactivate},
    cSIResourceConfigurationID          INTEGER (0..111),
    tci-State-InformationList           Tci-State-InformationList   OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { CSI-RSCoordinationRequest-Item-ExtIEs} } OPTIONAL,
    ...
}



CSI-RSCoordinationRequest-Item-ExtIEs   XNAP-PROTOCOL-EXTENSION ::= {
    ...
}




CSI-RSCoordinationResultList ::= SEQUENCE (SIZE(1.. maxnoofCSIResourceConfigurations)) OF CSI-RSCoordinationResult-Item 


CSI-RSCoordinationResult-Item   ::= SEQUENCE {
    transmissionStatus                  ENUMERATED{activated, deactivated},
    cSIResourceConfigurationID          INTEGER (0..111),
    iE-Extensions                       ProtocolExtensionContainer { { CSI-RSCoordinationResult-Item-ExtIEs} }  OPTIONAL,
    ...
}

CSI-RSCoordinationResult-Item-ExtIEs    XNAP-PROTOCOL-EXTENSION ::= {
    ...
}




CSIResourceConfigurationID ::= INTEGER (0..111, ...)


-- D


XnUAddressInfoperPDUSession-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF XnUAddressInfoperPDUSession-Item

XnUAddressInfoperPDUSession-Item ::= SEQUENCE {
    pduSession-ID           PDUSession-ID,
    dataForwardingInfoFromTargetNGRANnode       DataForwardingInfoFromTargetNGRANnode                                       OPTIONAL,
    pduSessionResourceSetupCompleteInfo-SNterm          PDUSessionResourceBearerSetupCompleteInfo-SNterminated      OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { { XnUAddressInfoperPDUSession-Item-ExtIEs} }                   OPTIONAL,
    ...
}

XnUAddressInfoperPDUSession-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-SecondarydataForwardingInfoFromTarget-List  CRITICALITY ignore  EXTENSION SecondarydataForwardingInfoFromTarget-List    PRESENCE optional}|
{ ID id-DRB-IDs-takenintouse                        CRITICALITY reject  EXTENSION DRB-List                                              PRESENCE optional}|
{ ID id-dataForwardingInfoFromTargetE-UTRANnode     CRITICALITY ignore  EXTENSION DataForwardingInfoFromTargetE-UTRANnode       PRESENCE optional},
    ...
}

DataForwardingInfoFromTargetE-UTRANnode ::= SEQUENCE {
    dataForwardingInfoFromTargetE-UTRANnode-List            DataForwardingInfoFromTargetE-UTRANnode-List,
    iE-Extension        ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-ExtIEs} }    OPTIONAL,
    ...
}

DataForwardingInfoFromTargetE-UTRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DataForwardingInfoFromTargetE-UTRANnode-List ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingInfoFromTargetE-UTRANnode-Item

DataForwardingInfoFromTargetE-UTRANnode-Item ::= SEQUENCE {
    dlForwardingUPTNLInformation    UPTransportLayerInformation,
    qosFlowsToBeForwarded-List  QoSFlowsToBeForwarded-List,
    iE-Extension        ProtocolExtensionContainer { { DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs} }   OPTIONAL,
    ...
}

DataForwardingInfoFromTargetE-UTRANnode-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoSFlowsToBeForwarded-List ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeForwarded-Item
QoSFlowsToBeForwarded-Item ::= SEQUENCE {
    qosFlowIdentifier           QoSFlowIdentifier,
    iE-Extension        ProtocolExtensionContainer { { QoSFlowsToBeForwarded-Item-ExtIEs} } OPTIONAL,
    ...
}

QoSFlowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DataForwardingInfoFromTargetNGRANnode ::= SEQUENCE {
    qosFlowsAcceptedForDataForwarding-List          QoSFLowsAcceptedToBeForwarded-List,
    pduSessionLevelDLDataForwardingInfo             UPTransportLayerInformation                             OPTIONAL,
    pduSessionLevelULDataForwardingInfo             UPTransportLayerInformation                             OPTIONAL,
    dataForwardingResponseDRBItemList               DataForwardingResponseDRBItemList                       OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DataForwardingInfoFromTargetNGRANnode-ExtIEs} }   OPTIONAL,
    ...
}

DataForwardingInfoFromTargetNGRANnode-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DirectForwardingPathAvailability    CRITICALITY ignore      EXTENSION DirectForwardingPathAvailability      PRESENCE optional },
    ...
}


QoSFLowsAcceptedToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsAcceptedToBeForwarded-Item

QoSFLowsAcceptedToBeForwarded-Item ::= SEQUENCE {
    qosFlowIdentifier           QoSFlowIdentifier,
    iE-Extension                ProtocolExtensionContainer { {QoSFLowsAcceptedToBeForwarded-Item-ExtIEs} }  OPTIONAL,
    ...
}

QoSFLowsAcceptedToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DataforwardingandOffloadingInfofromSource ::= SEQUENCE {
    qosFlowsToBeForwarded           QoSFLowsToBeForwarded-List,
    sourceDRBtoQoSFlowMapping       DRBToQoSFlowMapping-List                                                    OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DataforwardingandOffloadingInfofromSource-ExtIEs} }   OPTIONAL,
    ...
}

DataforwardingandOffloadingInfofromSource-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoSFLowsToBeForwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoSFLowsToBeForwarded-Item

QoSFLowsToBeForwarded-Item ::= SEQUENCE {
    qosFlowIdentifier           QoSFlowIdentifier,
    dl-dataforwarding           DLForwarding,
    ul-dataforwarding           ULForwarding,
    iE-Extension        ProtocolExtensionContainer { {QoSFLowsToBeForwarded-Item-ExtIEs} }  OPTIONAL,
    ...
}

QoSFLowsToBeForwarded-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-ULForwardingProposal                    CRITICALITY ignore  EXTENSION ULForwardingProposal      PRESENCE optional }|
{ ID id-SourceDLForwardingIPAddress             CRITICALITY ignore  EXTENSION TransportLayerAddress     PRESENCE optional }|
{ ID id-SourceNodeDLForwardingIPAddress         CRITICALITY ignore  EXTENSION TransportLayerAddress     PRESENCE optional },
    ...
}



DataForwardingResponseDRBItemList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {
    drb-ID              DRB-ID,
    dlForwardingUPTNL   UPTransportLayerInformation                                             OPTIONAL,
    ulForwardingUPTNL   UPTransportLayerInformation                                             OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DataForwardingResponseDRBItem-ExtIEs} }   OPTIONAL,
    ...
}

DataForwardingResponseDRBItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DataTrafficResources ::= BIT STRING (SIZE(6..17600))


DataTrafficResourceIndication ::= SEQUENCE {
    activationSFN                   ActivationSFN,
    sharedResourceType              SharedResourceType,
    reservedSubframePattern         ReservedSubframePattern                                     OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} }   OPTIONAL,
    ...
}

DataTrafficResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DAPSRequestInfo ::= SEQUENCE {
    dapsIndicator               ENUMERATED {daps-HO-required, ...},
    iE-Extensions               ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,
    ...
}

DAPSRequestInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DAPSResponseInfo-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DAPSResponseInfo-Item

DAPSResponseInfo-Item ::= SEQUENCE {
    drbID                       DRB-ID,
    dapsResponseIndicator       ENUMERATED {daps-HO-accepted, daps-HO-not-accepted, ...},
    iE-Extensions               ProtocolExtensionContainer { {DAPSResponseInfo-Item-ExtIEs} } OPTIONAL,
    ...
}

DAPSResponseInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DeliveryStatus  ::= INTEGER (0..4095, ...)


DesiredActNotificationLevel ::= ENUMERATED {none, qos-flow, pdu-session, ue-level, ...}

DefaultDRB-Allowed ::= ENUMERATED {true, false, ...}

DirectForwardingPathAvailability ::= ENUMERATED {direct-path-available, ...}

DirectForwardingPathAvailabilityWithSourceMN ::= ENUMERATED {direct-path-available, ...}

DLCountChoice ::= CHOICE {
    count12bits             COUNT-PDCP-SN12,
    count18bits             COUNT-PDCP-SN18,
    choice-extension        ProtocolIE-Single-Container { {DLCountChoice-ExtIEs} }
}

DLCountChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


DLForwarding    ::= ENUMERATED {dl-forwarding-proposed, ...}


DL-GBR-PRB-usage::= INTEGER (0..100)

DL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

DLF1Terminating-BHInfo ::= SEQUENCE {
    egressBAPRoutingID      BAPRoutingID,
    egressBHRLCCHID         BHRLCChannelID,
    iE-Extensions           ProtocolExtensionContainer { { DLF1Terminating-BHInfo-ExtIEs} } OPTIONAL,
    ...
}

DLF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DLLBTFailureInformationRequest ::= ENUMERATED {inquiry, ...}
DLLBTFailureInformationList ::= SEQUENCE (SIZE(1.. maxnoofLBTFailureInformation)) OF DLLBTFailureInformationList-Item

DLLBTFailureInformationList-Item::= SEQUENCE {
    uEAssistantIdentifier       NG-RANnodeUEXnAPID,
    numberOfDLLBTFailures       INTEGER (1..1000,...)                           OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { { DLLBTFailureInformationList-Item-ExtIEs} }   OPTIONAL,
    ...
}

DLLBTFailureInformationList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DLNonF1Terminating-BHInfo ::= SEQUENCE {
    ingressBAPRoutingID         BAPRoutingID,
    ingressBHRLCCHID            BHRLCChannelID,
    priorhopBAPAddress          BAPAddress,
    iabqosMappingInformation    IAB-QoS-Mapping-Information,
    iE-Extensions       ProtocolExtensionContainer { { DLNonF1Terminating-BHInfo-ExtIEs} } OPTIONAL,
    ...
}

DLNonF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DL-Total-PRB-usage::= INTEGER (0..100)

DL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)

DRB-ID  ::= INTEGER (1..32, ...)


DRB-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-ID


DRB-List-withCause ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRB-List-withCause-Item

DRB-List-withCause-Item ::= SEQUENCE {
    drb-id      DRB-ID,
    cause       Cause,
    rLC-Mode    RLCMode                             OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DRB-List-withCause-Item-ExtIEs} } OPTIONAL,
    ...
}

DRB-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRB-Number ::= INTEGER (1..32, ...)


DRBsSubjectToDLDiscarding-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToDLDiscarding-Item

DRBsSubjectToDLDiscarding-Item ::= SEQUENCE {
    drbID               DRB-ID,
    dlCount             DLCountChoice,
    iE-Extension        ProtocolExtensionContainer { { DRBsSubjectToDLDiscarding-Item-ExtIEs} } OPTIONAL,
    ...
}

DRBsSubjectToDLDiscarding-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRBsSubjectToEarlyStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToEarlyStatusTransfer-Item

DRBsSubjectToEarlyStatusTransfer-Item ::= SEQUENCE {
    drbID               DRB-ID,
    dlCount             DLCountChoice,
    iE-Extension        ProtocolExtensionContainer { { DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs} }  OPTIONAL,
    ...
}

DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRBsSubjectToStatusTransfer-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToStatusTransfer-Item

DRBsSubjectToStatusTransfer-Item ::= SEQUENCE {
    drbID               DRB-ID,
    pdcpStatusTransfer-UL   DRBBStatusTransferChoice,
    pdcpStatusTransfer-DL   DRBBStatusTransferChoice,
    iE-Extension        ProtocolExtensionContainer { {DRBsSubjectToStatusTransfer-Item-ExtIEs} }    OPTIONAL,
    ...
}

DRBsSubjectToStatusTransfer-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-OldQoSFlowMap-ULendmarkerexpected   CRITICALITY reject      EXTENSION QoSFlows-List                 PRESENCE optional }|
    { ID id-PDCPSNGapTransfer-UL                CRITICALITY ignore      EXTENSION PDCPSNGapTransfer-UL      PRESENCE optional },
    ...
}


DRBBStatusTransferChoice ::= CHOICE {
    pdcp-sn-12bits      DRBBStatusTransfer12bitsSN,
    pdcp-sn-18bits      DRBBStatusTransfer18bitsSN,
    choice-extension        ProtocolIE-Single-Container { {DRBBStatusTransferChoice-ExtIEs} }
}

DRBBStatusTransferChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


DRBBStatusTransfer12bitsSN ::= SEQUENCE {
    receiveStatusofPDCPSDU  BIT STRING (SIZE(1..2048))                                          OPTIONAL,
    cOUNTValue              COUNT-PDCP-SN12,
    iE-Extension            ProtocolExtensionContainer { {DRBBStatusTransfer12bitsSN-ExtIEs} }  OPTIONAL,
    ...
}

DRBBStatusTransfer12bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRBBStatusTransfer18bitsSN ::= SEQUENCE {
    receiveStatusofPDCPSDU  BIT STRING (SIZE(1..131072))                                        OPTIONAL,
    cOUNTValue              COUNT-PDCP-SN18,
    iE-Extension            ProtocolExtensionContainer { {DRBBStatusTransfer18bitsSN-ExtIEs} }  OPTIONAL,
    ...
}

DRBBStatusTransfer18bitsSN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDCPSNGapTransfer-UL ::= CHOICE {
    pdcp-sn-12bits          DRBStatusTransfer12bitsSN-ULGap,
    pdcp-sn-18bits          DRBStatusTransfer18bitsSN-ULGap,
    choice-extension        ProtocolIE-Single-Container { { PDCPSNGapTransfer-UL-ExtIEs} }
}

PDCPSNGapTransfer-UL-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

DRBStatusTransfer12bitsSN-ULGap ::= SEQUENCE {
    discardBitmap           BIT STRING (SIZE(1..2048)),
    iE-Extension            ProtocolExtensionContainer { { DRBStatusTransfer12bitsSN-ULGap-ExtIEs} }    OPTIONAL,
    ...
}

DRBStatusTransfer12bitsSN-ULGap-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRBStatusTransfer18bitsSN-ULGap ::= SEQUENCE {
    discardBitmap           BIT STRING (SIZE(1..131072)),
    iE-Extension            ProtocolExtensionContainer { {DRBStatusTransfer18bitsSN-ULGap-ExtIEs} } OPTIONAL,
    ...
}

DRBStatusTransfer18bitsSN-ULGap-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}





DRBToQoSFlowMapping-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBToQoSFlowMapping-Item

DRBToQoSFlowMapping-Item ::= SEQUENCE {
    drb-ID                          DRB-ID,
    qosFlows-List                   QoSFlows-List,
    rLC-Mode                        RLCMode                             OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {DRBToQoSFlowMapping-Item-ExtIEs} }    OPTIONAL,
    ...
}

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DAPSRequestInfo     CRITICALITY ignore      EXTENSION DAPSRequestInfo       PRESENCE optional },
    ...
}


DUF-Slot-Config-List    ::= SEQUENCE (SIZE(1..maxnoofDUFSlots)) OF DUF-Slot-Config-Item


DUF-Slot-Config-Item    ::= CHOICE {
    explicitFormat              ExplicitFormat,
    implicitFormat              ImplicitFormat,
    choice-extension                ProtocolIE-Single-Container { { DUF-Slot-Config-Item-ExtIEs} }
}

DUF-Slot-Config-Item-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


DUFSlotformatIndex ::= INTEGER(0..254)

DUFTransmissionPeriodicity ::= ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ...}


DU-RX-MT-RX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-TX-MT-TX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-RX-MT-TX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}

DU-TX-MT-RX ::= ENUMERATED {supported, not-supported, supported-FDM-required, ...}



DuplicationActivation ::= ENUMERATED {active, inactive, ...}


Dynamic5QIDescriptor ::= SEQUENCE {
    priorityLevelQoS            PriorityLevelQoS,
    packetDelayBudget           PacketDelayBudget,
    packetErrorRate             PacketErrorRate,
    fiveQI                      FiveQI                                                  OPTIONAL,
    delayCritical               ENUMERATED {delay-critical, non-delay-critical, ...}    OPTIONAL,
-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.
    averagingWindow             AveragingWindow                                         OPTIONAL,
-- This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.
    maximumDataBurstVolume      MaximumDataBurstVolume                                  OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs } }   OPTIONAL,
    ...
}

Dynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-ExtendedPacketDelayBudget       CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget     PRESENCE optional}|
    { ID id-CNPacketDelayBudgetDownlink     CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget     PRESENCE optional}|
    { ID id-CNPacketDelayBudgetUplink       CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget     PRESENCE optional},
    ...
}

DataForwardingInfoForLTM-List ::=  SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF DataForwardingInfoForLTM-Item

DataForwardingInfoForLTM-Item ::= SEQUENCE {
    pDUSessionID                            PDUSession-ID,
    dataForwardingInfoFromTargetNGRANnode   DataForwardingInfoFromTargetNGRANnode,
    iE-Extension                            ProtocolExtensionContainer { { DataForwardingInfoForLTM-Item-ExtIEs} }  OPTIONAL,
    ...
}

DataForwardingInfoForLTM-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DLPDUSetInformationMarkingSupportIndication ::= ENUMERATED {true, ...}

-- E

EarlyMeasurement ::= ENUMERATED {true, ...}

ECNMarkingorCongestionInformationReportingRequest ::= CHOICE {
    eCNMarkingAtRANRequest              ECNMarkingAtRANRequest,
    eCNMarkingAtUPFRequest              ECNMarkingAtUPFRequest,
    congestionInformationRequest        CongestionInformationRequest,
    choice-Extensions               ProtocolIE-Single-Container { {ECNMarkingorCongestionInformationReportingRequest-ExtIEs} }
}

ECNMarkingorCongestionInformationReportingRequest-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...

}

ECNMarkingAtRANRequest ::= ENUMERATED {ul, dl, both, stop,...}

ECNMarkingAtUPFRequest ::= ENUMERATED {ul, dl, both, stop,...}


CongestionInformationRequest    ::= ENUMERATED {ul, dl, both, stop, ...}

ECNMarkingorCongestionInformationReportingStatus ::= ENUMERATED {active, not-active, ...}

EnergyCost ::= INTEGER (0..10000, ...)


EquivalentSNPNs ::= SEQUENCE (SIZE(1..maxnoofESNPNs)) OF SNPNIdentity


E-RAB-ID        ::= INTEGER (0..15, ...)


E-UTRAARFCN ::= INTEGER (0..maxEARFCN)


E-UTRA-Cell-Identity            ::= BIT STRING (SIZE(28))

ERedcap-Bcast-Information ::= BIT STRING(SIZE(8))

E-UTRA-CGI ::= SEQUENCE {
    plmn-id             PLMN-Identity,
    e-utra-CI           E-UTRA-Cell-Identity,
    iE-Extension        ProtocolExtensionContainer { {E-UTRA-CGI-ExtIEs} }  OPTIONAL,
    ...
}

E-UTRA-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


E-UTRAFrequencyBandIndicator ::= INTEGER (1..256, ...)


E-UTRAMultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofEUTRABands)) OF E-UTRAFrequencyBandIndicator


EUTRAPagingeDRXInformation ::= SEQUENCE {
    eutrapaging-eDRX-Cycle      EUTRAPaging-eDRX-Cycle,
    eutrapaging-Time-Window     EUTRAPaging-Time-Window                                         OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {EUTRAPagingeDRXInformation-ExtIEs} }  OPTIONAL,
    ...
}

EUTRAPagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

EUTRAPaging-eDRX-Cycle ::= ENUMERATED {
    hfhalf, hf1, hf2, hf4, hf6,
    hf8, hf10, hf12, hf14, hf16,
    hf32, hf64, hf128, hf256,
    ...
}


EUTRAPaging-Time-Window ::= ENUMERATED {
    s1, s2, s3, s4, s5,
    s6, s7, s8, s9, s10,
    s11, s12, s13, s14, s15, s16,
    ...
}

E-UTRAPCI ::= INTEGER (0..503, ...)


E-UTRAPRACHConfiguration ::= SEQUENCE {
    rootSequenceIndex                       INTEGER (0..837),
    zeroCorrelationIndex                    INTEGER (0..15),
    highSpeedFlag                           ENUMERATED {true, false, ...},
    prach-FreqOffset                        INTEGER (0..94),
    prach-ConfigIndex                       INTEGER (0..63)                             OPTIONAL,
-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Served Cell Information E-UTRA IE is set to the value "TDD" --
    iE-Extensions                           ProtocolExtensionContainer { {E-UTRAPRACHConfiguration-ExtIEs} }    OPTIONAL,
    ...
}

E-UTRAPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


E-UTRATransmissionBandwidth ::= ENUMERATED {bw6, bw15, bw25, bw50, bw75, bw100, ..., bw1}

EndpointIPAddressAndPort ::=SEQUENCE {
    endpointIPAddress               TransportLayerAddress,
    portNumber                      PortNumber,
    iE-Extensions                   ProtocolExtensionContainer { { EndpointIPAddressAndPort-ExtIEs} } OPTIONAL
}

EndpointIPAddressAndPort-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


EventTriggered ::= SEQUENCE {
    loggedEventTriggeredConfig          LoggedEventTriggeredConfig,
    iE-Extensions       ProtocolExtensionContainer { { EventTriggered-ExtIEs} } OPTIONAL,
    ...
}

EventTriggered-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

EventType ::= ENUMERATED {
    report-upon-change-of-serving-cell,
    report-UE-moving-presence-into-or-out-of-the-Area-of-Interest,
    ...,
    report-upon-change-of-serving-cell-and-Area-of-Interest,
    report-aerial-UE-flight-information
}


EventTypeTrigger ::= CHOICE {
    outOfCoverage                   ENUMERATED {true, ...},
    eventL1     EventL1,
    choice-Extensions       ProtocolIE-Single-Container { {EventTypeTrigger-ExtIEs} }
}

EventTypeTrigger-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

EventL1 ::= SEQUENCE {
    l1Threshold                 MeasurementThresholdL1LoggedMDT,
    hysteresis                  Hysteresis,
    timeToTrigger               TimeToTrigger,
    iE-Extensions       ProtocolExtensionContainer { { EventL1-ExtIEs} } OPTIONAL,
    ...
}

EventL1-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MeasurementThresholdL1LoggedMDT ::= CHOICE {
    threshold-RSRP              Threshold-RSRP,
    threshold-RSRQ              Threshold-RSRQ,
    ...,
    choice-extension        ProtocolIE-Single-Container { {MeasurementThresholdL1LoggedMDT-ExtIEs} }
}

MeasurementThresholdL1LoggedMDT-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ExcessPacketDelayThresholdConfiguration ::= SEQUENCE (SIZE(1..maxnoofThresholdsForExcessPacketDelay)) OF ExcessPacketDelayThresholdItem

ExcessPacketDelayThresholdItem ::= SEQUENCE {
    fiveQI                                  FiveQI,
    excessPacketDelayThresholdValue         ExcessPacketDelayThresholdValue,
    iE-Extensions       ProtocolExtensionContainer { { ExcessPacketDelayThresholdItem-ExtIEs } }    OPTIONAL,
    ...
}

ExcessPacketDelayThresholdItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ExcessPacketDelayThresholdValue ::= ENUMERATED {
    ms0dot25, ms0dot5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500,
    ...
}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {
    sec15, sec30, sec60, sec90, sec120, sec180, long-time,
    ...
}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {
    expectedActivityPeriod                      ExpectedActivityPeriod                      OPTIONAL,
    expectedIdlePeriod                          ExpectedIdlePeriod                          OPTIONAL,
    sourceOfUEActivityBehaviourInformation      SourceOfUEActivityBehaviourInformation      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,
    ...
}

ExpectedUEActivityBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ExpectedUEBehaviour ::= SEQUENCE {
    expectedUEActivityBehaviour     ExpectedUEActivityBehaviour                         OPTIONAL,
    expectedHOInterval              ExpectedHOInterval                              OPTIONAL,
    expectedUEMobility              ExpectedUEMobility                              OPTIONAL,
    expectedUEMovingTrajectory      ExpectedUEMovingTrajectory                      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} } OPTIONAL,
    ...
}

ExpectedUEBehaviour-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ExpectedUEMobility ::= ENUMERATED {
    stationary,
    mobile,
    ...
}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {
    nGRAN-CGI               GlobalNG-RANCell-ID,
    timeStayedInCell        INTEGER (0..4095)                                                   OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }  OPTIONAL,
    ...
}

ExpectedUEMovingTrajectoryItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {
    subscription-information,
    statistics,
    ...
}

ExplicitFormat ::=  SEQUENCE {
    permutation         Permutation,
    noofDownlinkSymbols INTEGER(0..14)      OPTIONAL,
    noofUplinkSymbols   INTEGER(0..14)      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { ExplicitFormat-ExtIEs} } OPTIONAL,
    ...
}

ExplicitFormat-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ExtendedRATRestrictionInformation ::= SEQUENCE {
    primaryRATRestriction       BIT STRING (SIZE(8, ..., 16)),
    secondaryRATRestriction     BIT STRING (SIZE(8, ...)),
    iE-Extensions       ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }   OPTIONAL,
    ...
}

ExtendedRATRestrictionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ExtendedPacketDelayBudget ::= INTEGER (0..65535, ..., 65536..109999)

ExtendedSliceSupportList    ::= SEQUENCE (SIZE(1..maxnoofExtSliceItems)) OF S-NSSAI

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

ExtTLAs ::= SEQUENCE (SIZE(1..maxnoofExtTLAs)) OF ExtTLA-Item

ExtTLA-Item ::= SEQUENCE {
    iPsecTLA                            TransportLayerAddress               OPTIONAL,
    gTPTransportLayerAddresses          GTPTLAs                             OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ExtTLA-Item-ExtIEs} } OPTIONAL,
    ...
}

ExtTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


GTPTLAs ::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF  GTPTLA-Item


GTPTLA-Item ::= SEQUENCE {
    gTPTransportLayerAddresses              TransportLayerAddress,
    iE-Extensions   ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }       OPTIONAL,
    ...
}

GTPTLA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

EarlySyncInformationRequest ::= SEQUENCE {
    earlySyncConfigurationRequest           ENUMERATED {true, ...},
    earlyRACHResourcesProviders-List        EarlyRACHResourcesProviders-List OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { EarlySyncInformationRequest-ExtIEs} }    OPTIONAL,
    ...
}

EarlySyncInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


EarlySyncInformation ::= SEQUENCE {
    earlyULSyncConfig                       EarlyULSyncConfig OPTIONAL,
    earlyULSyncConfigSUL                    EarlyULSyncConfig OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { EarlySyncInformation-ExtIEs} }   OPTIONAL,
    ...
}

EarlySyncInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



EarlyRACHResourcesProviders-List ::= SEQUENCE (SIZE(1..maxnoofEarlyRACHResourcesID)) OF EarlyRACHResourcesProviders-Item

EarlyRACHResourcesProviders-Item ::= SEQUENCE {
    globalgNBID                                 GlobalgNB-ID,
    earlyRACHResourcesRequesterID               EarlyRACHResourcesRequesterID,
    iE-Extensions                             ProtocolExtensionContainer { { EarlyRACHResourcesProviders-Item-ExtIEs} } OPTIONAL,
    ...
}

EarlyRACHResourcesProviders-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



EarlyULSyncConfig ::= SEQUENCE {
    rACHConfiguration                       OCTET STRING,
    earlyRACHResources-List                 EarlyRACHResources-List OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { EarlyULSyncConfig-ExtIEs} } OPTIONAL,
    ...
}

EarlyULSyncConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

EarlyRACHResources-List ::= SEQUENCE (SIZE(1..maxnoofEarlyRACHResourcesID)) OF EarlyRACHResources-Item

EarlyRACHResources-Item ::= SEQUENCE {
    globalgNBID                                 GlobalgNB-ID    ,
    earlyRACHResourcesRequesterID               EarlyRACHResourcesRequesterID,
    preambleIndexList                           PreambleIndexList           OPTIONAL,
    iE-Extensions                               ProtocolExtensionContainer { { EarlyRACHResources-Item-ExtIEs} }    OPTIONAL,
    ...
}

EarlyRACHResources-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

EarlyRACHResourcesRequesterID ::= INTEGER (0..68719476735)



-- F
FailureType ::= ENUMERATED {
    toolateCPCExecution,
    cPCorCPA-ExecutiontowrongPSCell,
    ...
}

F1CTrafficContainer ::= OCTET STRING

F1-terminatingIAB-donorIndicator ::= ENUMERATED {true, ...}

F1-TerminatingTopologyBHInformation ::= SEQUENCE {
    f1TerminatingBHInformation-List             F1TerminatingBHInformation-List,
    iE-Extensions                           ProtocolExtensionContainer { {F1-TerminatingTopologyBHInformation-ExtIEs} } OPTIONAL,
    ...
}

F1-TerminatingTopologyBHInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

F1TerminatingBHInformation-List ::= SEQUENCE (SIZE(1..maxnoofBHInfo)) OF F1TerminatingBHInformation-Item

F1TerminatingBHInformation-Item ::= SEQUENCE {
    bHInfoIndex                 BHInfoIndex,
    dLTNLAddress                IABTNLAddress,
    dlF1TerminatingBHInfo   DLF1Terminating-BHInfo      OPTIONAL,
    ulF1TerminatingBHInfo   ULF1Terminating-BHInfo      OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { { F1TerminatingBHInformation-Item-ExtIEs} }    OPTIONAL,
    ...
}

F1TerminatingBHInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


FiveGCMobilityRestrictionListContainer ::= OCTET STRING
-- This octets of the OCTET STRING contain the Mobility Restriction List IE as specified in TS 38.413 [5]. --

FiveGProSeAuthorized ::= SEQUENCE {
    fiveGproSeDirectDiscovery                   FiveGProSeDirectDiscovery                                   OPTIONAL,
    fiveGproSeDirectCommunication               FiveGProSeDirectCommunication                               OPTIONAL,
    fiveGnrProSeLayer2UEtoNetworkRelay      FiveGProSeLayer2UEtoNetworkRelay                                OPTIONAL,
    fiveGnrProSeLayer3UEtoNetworkRelay          FiveGProSeLayer3UEtoNetworkRelay                            OPTIONAL,
    fiveGnrProSeLayer2RemoteUE                  FiveGProSeLayer2RemoteUE                                    OPTIONAL,
    iE-Extensions                               ProtocolExtensionContainer { {FiveGProSeAuthorized-ExtIEs} }    OPTIONAL,
    ...
}

FiveGProSeAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-FiveGProSeLayer2Multipath                       CRITICALITY ignore  EXTENSION FiveGProSeLayer2Multipath                 PRESENCE optional }|
    { ID id-FiveGProSeLayer2UEtoUERelay                     CRITICALITY ignore  EXTENSION FiveGProSeLayer2UEtoUERelay               PRESENCE optional }|
    { ID id-FiveGProSeLayer2UEtoUERemote                    CRITICALITY ignore  EXTENSION FiveGProSeLayer2UEtoUERemote              PRESENCE optional }|
    { ID id-FiveGProSeLayer3MHUEtoNetworkRelay              CRITICALITY ignore  EXTENSION FiveGProSeLayer3MHUEtoNetworkRelay        PRESENCE optional }|
    { ID id-FiveGProSeLayer2MHUEtoNetworkRelay              CRITICALITY ignore  EXTENSION FiveGProSeLayer2MHUEtoNetworkRelay        PRESENCE optional }|
    { ID id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay  CRITICALITY ignore  EXTENSION FiveGProSeLayer2MHIntermediateUEtoNetworkRelay        PRESENCE optional }|
    { ID id-FiveGProSeLayer2MHRemote                        CRITICALITY ignore  EXTENSION FiveGProSeLayer2MHRemote                  PRESENCE optional },
    ...
}

FiveGProSeDirectDiscovery ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeDirectCommunication ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2UEtoNetworkRelay ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer3UEtoNetworkRelay ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2RemoteUE ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2Multipath ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2UEtoUERelay ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2UEtoUERemote ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer3MHUEtoNetworkRelay  ::= ENUMERATED { 
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2MHUEtoNetworkRelay  ::= ENUMERATED { 
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2MHIntermediateUEtoNetworkRelay  ::= ENUMERATED { 
    authorized,
    not-authorized,
    ...
}

FiveGProSeLayer2MHRemote    ::= ENUMERATED { 
    authorized,
    not-authorized,
    ...
}


FiveGProSePC5QoSParameters ::= SEQUENCE {
    fiveGProSepc5QoSFlowList                        FiveGProSePC5QoSFlowList,
    fiveGproSepc5LinkAggregateBitRates              BitRate             OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { FiveGProSePC5QoSParameters-ExtIEs} } OPTIONAL,
    ...
}

FiveGProSePC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FiveGProSePC5QoSFlowList::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF FiveGProSePC5QoSFlowItem

FiveGProSePC5QoSFlowItem::= SEQUENCE {
    fiveGproSepQI               FiveQI,
    fiveGproSepc5FlowBitRates   FiveGProSePC5FlowBitRates               OPTIONAL,
    fiveGproSerange             Range                       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { FiveGProSePC5QoSFlowItem-ExtIEs} }   OPTIONAL,
    ...
}

FiveGProSePC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FiveGProSePC5FlowBitRates ::= SEQUENCE {
    fiveGproSeguaranteedFlowBitRate     BitRate,
    fiveGproSemaximumFlowBitRate        BitRate,
    iE-Extensions       ProtocolExtensionContainer { { FiveGProSePC5FlowBitRates-ExtIEs} }  OPTIONAL,
    ...
}

FiveGProSePC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FiveQI ::= INTEGER (0..255, ...)

Flows-Mapped-To-DRB-List    ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item    ::= SEQUENCE {
    qoSFlowIdentifier                       QoSFlowIdentifier,
    qoSFlowLevelQoSParameters               QoSFlowLevelQoSParameters,
    qoSFlowMappingIndication                QoSFlowMappingIndication                                                    OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { Flows-Mapped-To-DRB-Item-ExtIEs} }   OPTIONAL
}

Flows-Mapped-To-DRB-Item-ExtIEs     XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FreqDomainHSNAconfiguration-List ::= SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF FreqDomainHSNAconfiguration-List-Item

FreqDomainHSNAconfiguration-List-Item ::= SEQUENCE {
    rBsetIndex                                  INTEGER(0.. maxnoofRBsetsPerCell1, ...),
    freqDomainSlotHSNAconfiguration-List        FreqDomainSlotHSNAconfiguration-List,   
    iE-Extensions       ProtocolExtensionContainer { { FreqDomainHSNAconfiguration-List-Item-ExtIEs} }  OPTIONAL,
    ...
}

FreqDomainHSNAconfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


FreqDomainSlotHSNAconfiguration-List ::= SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF FreqDomainSlotHSNAconfiguration-List-Item

FreqDomainSlotHSNAconfiguration-List-Item ::=   SEQUENCE {
    slotIndex       INTEGER(1..maxnoofHSNASlots),   
    hSNADownlink    HSNADownlink        OPTIONAL,
    hSNAUplink      HSNAUplink          OPTIONAL,
    hSNAFlexible    HSNAFlexible        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { FreqDomainSlotHSNAconfiguration-List-Item-ExtIEs} }  OPTIONAL,
    ...
}

FreqDomainSlotHSNAconfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

FurtherExtended-UEIdentityIndexValue ::= BIT STRING (SIZE(20))

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {
    nR-CGI                      NR-CGI,
    future-cellCoverageState                INTEGER (0..63, ...),
    future-SSB-Coverage-Modification-List   Future-SSB-Coverage-Modification-List       OPTIONAL,
    predicted-Coverage-Modification-Cause   Predicted-CoverageModificationCause         OPTIONAL,
    timeForFutureCoverageState              INTEGER (1..60, ...)                        OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }  OPTIONAL,
    ...
}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF Future-SSB-Coverage-Modification-Item

Future-SSB-Coverage-Modification-Item ::= SEQUENCE {
    sSBIndex                        INTEGER(0..63),
    future-SSBCoverageState         INTEGER (0..15, ...),
    iE-Extension                    ProtocolExtensionContainer { { Future-SSB-Coverage-Modification-Item-ExtIEs} }  OPTIONAL,
    
    ...
}

Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- G


GBRQoSFlowInfo ::= SEQUENCE {
    maxFlowBitRateDL            BitRate,
    maxFlowBitRateUL            BitRate,
    guaranteedFlowBitRateDL     BitRate,
    guaranteedFlowBitRateUL     BitRate,
    notificationControl         ENUMERATED {notification-requested, ...}                OPTIONAL,
    maxPacketLossRateDL         PacketLossRate                                          OPTIONAL,
    maxPacketLossRateUL         PacketLossRate                                          OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }  OPTIONAL,
    ...
}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-AlternativeQoSParaSetList               CRITICALITY ignore  EXTENSION AlternativeQoSParaSetList                     PRESENCE optional }|
{ ID id-MonitoringRequestonAvailableBitrate     CRITICALITY ignore  EXTENSION MonitoringRequestonAvailableBitrate       PRESENCE optional },
    ...
}

GeographyBasedMDT::= SEQUENCE {
    geographicalArea        GeographicalArea,
    iE-Extensions       ProtocolExtensionContainer { {GeographyBasedMDT-ExtIEs} } OPTIONAL,
    ...
}

GeographyBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

GeographicalArea ::= SEQUENCE {
    nTNGeographicalArea     NTNGeographicalArea,
    nTNPLMNList             MDTPLMNList                                         OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {GeographicalArea-ExtIEs} }    OPTIONAL,
    ...
}

GeographicalArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

GlobalgNB-ID    ::= SEQUENCE {
    plmn-id         PLMN-Identity,
    gnb-id          GNB-ID-Choice,
    iE-Extensions       ProtocolExtensionContainer { {GlobalgNB-ID-ExtIEs} } OPTIONAL,
    ...
}

GlobalgNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

GNB-DU-Cell-Resource-Configuration  ::= SEQUENCE {
    subcarrierSpacing               SubcarrierSpacing,
    dUFTransmissionPeriodicity      DUFTransmissionPeriodicity      OPTIONAL,
    dUF-Slot-Config-List            DUF-Slot-Config-List            OPTIONAL,
    hSNATransmissionPeriodicity     HSNATransmissionPeriodicity,
    hNSASlotConfigList              HSNASlotConfigList              OPTIONAL,
    rBsetConfiguration                      RBsetConfiguration      OPTIONAL,
    freqDomainHSNAconfiguration-List        FreqDomainHSNAconfiguration-List    OPTIONAL,
    nACellResourceConfigurationList         NACellResourceConfigurationList         OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL,
    ...
}

GNB-DU-Cell-Resource-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



GNB-ID-Choice ::= CHOICE {
    gnb-ID                  BIT STRING (SIZE(22..32)),
    choice-extension        ProtocolIE-Single-Container { {GNB-ID-Choice-ExtIEs} }
}

GNB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


GNB-RadioResourceStatus ::= SEQUENCE {
    ssbAreaRadioResourceStatus-List             SSBAreaRadioResourceStatus-List,
    iE-Extensions                           ProtocolExtensionContainer { { GNB-RadioResourceStatus-ExtIEs} }    OPTIONAL,
    ...
}

GNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SliceRadioResourceStatus-List   CRITICALITY ignore  EXTENSION SliceRadioResourceStatus-List PRESENCE optional   }|
    { ID id-MIMOPRBusageInformation         CRITICALITY ignore  EXTENSION MIMOPRBusageInformation           PRESENCE optional   },
    ...
}


GlobalCell-ID   ::= SEQUENCE {
    plmn-id             PLMN-Identity,
    cell-type           Cell-Type-Choice,
    iE-Extensions       ProtocolExtensionContainer { { GlobalCell-ID-ExtIEs} } OPTIONAL,
    ...
}

GlobalCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


GlobalngeNB-ID  ::= SEQUENCE {
    plmn-id         PLMN-Identity,
    enb-id          ENB-ID-Choice,
    iE-Extensions       ProtocolExtensionContainer { {GlobaleNB-ID-ExtIEs} } OPTIONAL,
    ...
}

GlobaleNB-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ENB-ID-Choice ::= CHOICE {
    enb-ID-macro            BIT STRING (SIZE(20)),
    enb-ID-shortmacro       BIT STRING (SIZE(18)),
    enb-ID-longmacro        BIT STRING (SIZE(21)),
    choice-extension        ProtocolIE-Single-Container { {ENB-ID-Choice-ExtIEs} }
}

ENB-ID-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


GlobalNG-RANCell-ID ::= SEQUENCE {
    plmn-id                 PLMN-Identity,
    ng-RAN-Cell-id          NG-RAN-Cell-Identity,
    iE-Extensions       ProtocolExtensionContainer { {GlobalNG-RANCell-ID-ExtIEs} } OPTIONAL,
    ...
}

GlobalNG-RANCell-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


GlobalNG-RANNode-ID ::= CHOICE {
    gNB                     GlobalgNB-ID,
    ng-eNB                  GlobalngeNB-ID,
    choice-extension        ProtocolIE-Single-Container { {GlobalNG-RANNode-ID-ExtIEs} }
}

GlobalNG-RANNode-ID-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


GTP-TEID    ::= OCTET STRING (SIZE(4))


GTPtunnelTransportLayerInformation ::= SEQUENCE {
    tnl-address         TransportLayerAddress,
    gtp-teid            GTP-TEID,
    iE-Extensions       ProtocolExtensionContainer { {GTPtunnelTransportLayerInformation-ExtIEs} } OPTIONAL,
    ...
}

GTPtunnelTransportLayerInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-QoS-Mapping-Information  CRITICALITY reject  EXTENSION QoS-Mapping-Information   PRESENCE optional },
    ...
}


GUAMI ::= SEQUENCE {
    plmn-ID             PLMN-Identity,
    amf-region-id       BIT STRING (SIZE (8)),
    amf-set-id          BIT STRING (SIZE (10)),
    amf-pointer         BIT STRING (SIZE (6)),
    iE-Extensions       ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,
    ...
}

GUAMI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- H


HandoverReportType ::= ENUMERATED {
    hoTooEarly,
    hoToWrongCell,
    intersystempingpong,
    ...,
    tooLateCHOwithcandidateSCG
}

HashedUEIdentityIndexValue ::= BIT STRING (SIZE(13, ...))

HSNASlotConfigList ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF HSNASlotConfigItem

HSNASlotConfigItem  ::= SEQUENCE {
    hSNADownlink            HSNADownlink        OPTIONAL,
    hSNAUplink              HSNAUplink          OPTIONAL,
    hSNAFlexible            HSNAFlexible        OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { HSNASlotConfigItem-ExtIEs } } OPTIONAL,
    ...
}

HSNASlotConfigItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

HSNADownlink ::= ENUMERATED { hard, soft, notavailable }

HSNAFlexible ::= ENUMERATED { hard, soft, notavailable }

HSNAUplink ::= ENUMERATED { hard, soft, notavailable }

HSNATransmissionPeriodicity ::= ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, ...}


Hysteresis ::=      INTEGER (0..30)


-- I

IABCellInformation::=   SEQUENCE{
    nRCGI                               NR-CGI,
    iAB-DU-Cell-Resource-Configuration-Mode-Info    IAB-DU-Cell-Resource-Configuration-Mode-Info    OPTIONAL,
    iAB-STC-Info                        IAB-STC-Info                OPTIONAL,
    rACH-Config-Common                  RACH-Config-Common          OPTIONAL,
    rACH-Config-Common-IAB              RACH-Config-Common-IAB      OPTIONAL,
    cSI-RS-Configuration                OCTET STRING    OPTIONAL,
    sR-Configuration                    OCTET STRING    OPTIONAL,
    pDCCH-ConfigSIB1                    OCTET STRING    OPTIONAL,
    sCS-Common                          OCTET STRING    OPTIONAL,
    multiplexingInfo                    MultiplexingInfo    OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { IABCellInformation-ExtIEs} } OPTIONAL,
    ...
}


IABCellInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



IAB-DU-Cell-Resource-Configuration-Mode-Info    ::= CHOICE {
    tDD     IAB-DU-Cell-Resource-Configuration-TDD-Info,
    fDD     IAB-DU-Cell-Resource-Configuration-FDD-Info,
    choice-extension            ProtocolIE-Single-Container { { IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs} }
}

IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

IAB-DU-Cell-Resource-Configuration-FDD-Info ::= SEQUENCE {
    gNB-DU-Cell-Resource-Configuration-FDD-UL               GNB-DU-Cell-Resource-Configuration,
    gNB-DU-Cell-Resource-Configuration-FDD-DL               GNB-DU-Cell-Resource-Configuration,
    uLFrequencyInfo                     NRFrequencyInfo         OPTIONAL,
    dLFrequencyInfo                     NRFrequencyInfo         OPTIONAL,
    uLTransmissionBandwidth             NRTransmissionBandwidth OPTIONAL,
    dlTransmissionBandwidth             NRTransmissionBandwidth OPTIONAL,
    uLCarrierList                   NRCarrierList           OPTIONAL,
    dlCarrierList                   NRCarrierList           OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,
    ...
}

IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IAB-DU-Cell-Resource-Configuration-TDD-Info ::= SEQUENCE {
    gNB-DU-Cell-Resource-Configuration-TDD              GNB-DU-Cell-Resource-Configuration,
    frequencyInfo                   NRFrequencyInfo         OPTIONAL,
    transmissionBandwidth           NRTransmissionBandwidth OPTIONAL,
    carrierList                     NRCarrierList           OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,
    ...
}

IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IAB-MT-Cell-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF IAB-MT-Cell-List-Item

IAB-MT-Cell-List-Item ::=   SEQUENCE {
    nRCellIdentity              NR-Cell-Identity,
    dU-RX-MT-RX                 DU-RX-MT-RX,
    dU-TX-MT-TX                 DU-TX-MT-TX,
    dU-RX-MT-TX                 DU-RX-MT-TX,
    dU-TX-MT-RX                 DU-TX-MT-RX,
    iE-Extensions               ProtocolExtensionContainer { { IAB-MT-Cell-List-Item-ExtIEs } } OPTIONAL,
    ...
}

IAB-MT-Cell-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABNodeIndication ::= ENUMERATED {true,...}

IAB-QoS-Mapping-Information ::= SEQUENCE {
    dscp                            BIT STRING (SIZE(6))            OPTIONAL,
    flow-label                      BIT STRING (SIZE(20))       OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {IAB-QoS-Mapping-Information-ExtIEs} } OPTIONAL,
    ...
}

IAB-QoS-Mapping-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IAB-STC-Info    ::= SEQUENCE{
    iAB-STC-Info-List   IAB-STC-Info-List,
    iE-Extensions       ProtocolExtensionContainer { { IAB-STC-Info-ExtIEs } } OPTIONAL,
    ...
}

IAB-STC-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IAB-STC-Info-List ::=   SEQUENCE (SIZE(1..maxnoofIABSTCInfo)) OF IAB-STC-Info-Item

IAB-STC-Info-Item::=    SEQUENCE {
    sSB-freqInfo                        SSB-freqInfo,
    sSB-subcarrierSpacing               SSB-subcarrierSpacing,
    sSB-transmissionPeriodicity         SSB-transmissionPeriodicity,
    sSB-transmissionTimingOffset        SSB-transmissionTimingOffset,
    sSB-transmissionBitmap              SSB-transmissionBitmap,
    iE-Extensions       ProtocolExtensionContainer { { IAB-STC-Info-Item-ExtIEs } } OPTIONAL,
    ...
}

IAB-STC-Info-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IAB-TNL-Address-Request ::= SEQUENCE {
    iABIPv4AddressesRequested               IABTNLAddressesRequested,
    iABIPv6RequestType                      IABIPv6RequestType,
    iABTNLAddressToRemove-List              IABTNLAddressToRemove-List,
    iE-Extensions                           ProtocolExtensionContainer { {IAB-TNL-Address-Request-ExtIEs} } OPTIONAL,
    ...
}

IAB-TNL-Address-Request-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABIPv6RequestType  ::= CHOICE {
    iPv6Address                     IABTNLAddressesRequested,
    iPv6Prefix                      IABTNLAddressesRequested,
    choice-extension                ProtocolIE-Single-Container { {IABIPv6RequestType-ExtIEs} }
}

IABIPv6RequestType-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


IAB-TNL-Address-Response ::= SEQUENCE {
    iABAllocatedTNLAddress-List     IABAllocatedTNLAddress-List,
    iE-Extensions                   ProtocolExtensionContainer { {IAB-TNL-Address-Response-ExtIEs} }    OPTIONAL,
    ...
}

IAB-TNL-Address-Response-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABAllocatedTNLAddress-List ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))  OF IABAllocatedTNLAddress-Item

IABAllocatedTNLAddress-Item ::= SEQUENCE {
    iABTNLAddress               IABTNLAddress,
    iABTNLAddressUsage          IABTNLAddressUsage      OPTIONAL,
    associatedDonorDUAddress    BAPAddress              OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {IABAllocatedTNLAddress-Item-ExtIEs} } OPTIONAL,
    ...
}

IABAllocatedTNLAddress-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABTNLAddress ::= CHOICE {
    iPv4Address                     BIT STRING (SIZE(32)),
    iPv6Address                     BIT STRING (SIZE(128)),
    iPv6Prefix                      BIT STRING (SIZE(64)),
    choice-extension                ProtocolIE-Single-Container { {IABTNLAddress-ExtIEs} }
}

IABTNLAddress-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

IABTNLAddressesRequested ::= SEQUENCE {
    tNLAddressesOrPrefixesRequestedAllTraffic   INTEGER (1..256)    OPTIONAL,
    tNLAddressesOrPrefixesRequestedF1-C         INTEGER (1..256)    OPTIONAL,
    tNLAddressesOrPrefixesRequestedF1-U         INTEGER (1..256)    OPTIONAL,
    tNLAddressesOrPrefixesRequestedNoNF1        INTEGER (1..256)    OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {IABTNLAddressesRequested-ExtIEs} } OPTIONAL
}

IABTNLAddressesRequested-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


IABTNLAddressToRemove-List  ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))  OF IABTNLAddressToRemove-Item

IABTNLAddressToRemove-Item ::=  SEQUENCE {
    iABTNLAddress           IABTNLAddress,
    iE-Extension            ProtocolExtensionContainer { {IABTNLAddressToRemove-Item-ExtIEs} }  OPTIONAL,
    ...
}

IABTNLAddressToRemove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

IABTNLAddressUsage ::= ENUMERATED {
    f1-c,
    f1-u,
    non-f1,
    ...,
    all
}

IABTNLAddressException ::= SEQUENCE (SIZE(1..maxnoofTLAsIAB)) OF IABTNLAddress-Item


IABTNLAddress-Item ::= SEQUENCE {
    iABTNLAddress                   IABTNLAddress,
    iE-Extensions                   ProtocolExtensionContainer { { IABTNLAddress-ItemExtIEs } } OPTIONAL,
    ...}

IABTNLAddress-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ImmediateMDT-NR ::= SEQUENCE {
    measurementsToActivate      MeasurementsToActivate,
    m1Configuration             M1Configuration             OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the first bit set to "1".--
    m4Configuration             M4Configuration             OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1".--
    m5Configuration             M5Configuration             OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1".--
    mDT-Location-Info           MDT-Location-Info           OPTIONAL,
    m6Configuration             M6Configuration             OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1".--
    m7Configuration             M7Configuration             OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1".--
    bluetoothMeasurementConfiguration           BluetoothMeasurementConfiguration           OPTIONAL,
    wLANMeasurementConfiguration                WLANMeasurementConfiguration                OPTIONAL,
    sensorMeasurementConfiguration              SensorMeasurementConfiguration              OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { { ImmediateMDT-NR-ExtIEs} }    OPTIONAL,
    ...
}

ImmediateMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ImplicitFormat  ::= SEQUENCE    {
    dUFSlotformatIndex          DUFSlotformatIndex,
    iE-Extensions       ProtocolExtensionContainer { { ImplicitFormat-ExtIEs } } OPTIONAL,
    ...
}

ImplicitFormat-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Indication-of-bitrate-adaptation ::= ENUMERATED {uplink, ...}

InitiatingCondition-FailureIndication ::= CHOICE {
    rRCReestab                  RRCReestab-initiated,
    rRCSetup                    RRCSetup-initiated,
    choice-extension                ProtocolIE-Single-Container { {InitiatingCondition-FailureIndication-ExtIEs} }
}

InitiatingCondition-FailureIndication-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

IntendedTDD-DL-ULConfiguration-NR ::= SEQUENCE {
    nrscs                           NRSCS,
    nrCyclicPrefix                  NRCyclicPrefix,
    nrDL-ULTransmissionPeriodicity  NRDL-ULTransmissionPeriodicity,
    slotConfiguration-List          SlotConfiguration-List,
    iE-Extensions                   ProtocolExtensionContainer { {IntendedTDD-DL-ULConfiguration-NR-ExtIEs} }   OPTIONAL,
    ...
}

IntendedTDD-DL-ULConfiguration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

InterfaceInstanceIndication ::= INTEGER (0..255, ...)


I-RNTI ::= CHOICE {
    i-RNTI-full         BIT STRING (SIZE(40)),
    i-RNTI-short        BIT STRING (SIZE(24)),
    choice-extension    ProtocolIE-Single-Container { {I-RNTI-ExtIEs} }
}

I-RNTI-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

IABAuthorizationStatus ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}


-- J

JointorDLTCIStateID  ::= OCTET STRING



-- K


-- L


Local-NG-RAN-Node-Identifier ::= CHOICE {
    full-I-RNTI-Profile-List                    Full-I-RNTI-Profile-List,
    short-I-RNTI-Profile-List                   Short-I-RNTI-Profile-List,
    choice-extension                            ProtocolIE-Single-Container { { Local-NG-RAN-Node-Identifier-ExtIEs} }
}

Local-NG-RAN-Node-Identifier-ExtIEs XNAP-PROTOCOL-IES ::= {
    { ID id-Full-and-Short-I-RNTI-Profile-List      CRITICALITY ignore  TYPE Full-and-Short-I-RNTI-Profile-List PRESENCE mandatory},
    ...
}

Full-and-Short-I-RNTI-Profile-List::= SEQUENCE {

    full-I-RNTI-Profile-List                    Full-I-RNTI-Profile-List,
    short-I-RNTI-Profile-List                   Short-I-RNTI-Profile-List,
    iE-Extensions                               ProtocolExtensionContainer { { Full-and-Short-I-RNTI-Profile-List-ExtIEs} } OPTIONAL,
    ...
}

Full-and-Short-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


Full-I-RNTI-Profile-List ::= CHOICE {
    full-I-RNTI-Profile-0   BIT STRING (SIZE (21)),
    full-I-RNTI-Profile-1   BIT STRING (SIZE (18)),
    full-I-RNTI-Profile-2   BIT STRING (SIZE (15)),
    full-I-RNTI-Profile-3   BIT STRING (SIZE (12)),
    choice-extension        ProtocolIE-Single-Container { { Full-I-RNTI-Profile-List-ExtIEs} }
}

Full-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

Short-I-RNTI-Profile-List ::= CHOICE {
    short-I-RNTI-Profile-0  BIT STRING (SIZE (8)),
    short-I-RNTI-Profile-1  BIT STRING (SIZE (6)),
    choice-extension        ProtocolIE-Single-Container { { Short-I-RNTI-Profile-List-ExtIEs} }
}

Short-I-RNTI-Profile-List-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


LastVisitedCell-Item ::= CHOICE {
    nG-RAN-Cell                     LastVisitedNGRANCellInformation,
    e-UTRAN-Cell                    LastVisitedEUTRANCellInformation,
    uTRAN-Cell                      LastVisitedUTRANCellInformation,
    gERAN-Cell                      LastVisitedGERANCellInformation,
    choice-extension                ProtocolIE-Single-Container { { LastVisitedCell-Item-ExtIEs} }
}

LastVisitedCell-Item-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation ::= OCTET STRING

LastVisitedUTRANCellInformation ::= OCTET STRING

LastVisitedPSCellInformation    ::= OCTET STRING

LastVisitedPSCellList   ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerSN)) OF LastVisitedPSCellList-Item

LastVisitedPSCellList-Item  ::= SEQUENCE {
    lastVisitedPSCellInformation        LastVisitedPSCellInformation,
    iE-Extensions       ProtocolExtensionContainer { { LastVisitedPSCellList-Item-ExtIEs} } OPTIONAL,
    ...
}

LastVisitedPSCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SCGUEHistoryInformation ::= SEQUENCE {
    lastVisitedPSCellList           LastVisitedPSCellList       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { SCGUEHistoryInformation-ExtIEs} } OPTIONAL,
    ...
}

SCGUEHistoryInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LCID ::= INTEGER (1..32, ...)

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}


ListOfCells ::= SEQUENCE (SIZE(1..maxnoofCellsinAoI)) OF CellsinAoI-Item

CellsinAoI-Item ::= SEQUENCE {
    pLMN-Identity           PLMN-Identity,
    ng-ran-cell-id          NG-RAN-Cell-Identity,
    iE-Extensions           ProtocolExtensionContainer { {CellsinAoI-Item-ExtIEs} } OPTIONAL,
    ...
}

CellsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ListOfRANNodesinAoI ::= SEQUENCE (SIZE(1.. maxnoofRANNodesinAoI)) OF GlobalNG-RANNodesinAoI-Item

GlobalNG-RANNodesinAoI-Item ::= SEQUENCE {
    global-NG-RAN-Node-ID       GlobalNG-RANNode-ID,
    iE-Extensions       ProtocolExtensionContainer { {GlobalNG-RANNodesinAoI-Item-ExtIEs} } OPTIONAL,
    ...
}

GlobalNG-RANNodesinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ListOfTAIsinAoI ::= SEQUENCE (SIZE(1..maxnoofTAIsinAoI)) OF TAIsinAoI-Item

TAIsinAoI-Item ::= SEQUENCE {
    pLMN-Identity       PLMN-Identity,
    tAC                 TAC,
    iE-Extensions       ProtocolExtensionContainer { {TAIsinAoI-Item-ExtIEs} } OPTIONAL,
    ...
}

TAIsinAoI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LocationInformationSNReporting ::= ENUMERATED {
    pSCell,
    ...
}

LocationReportingInformation ::= SEQUENCE {
    eventType           EventType,
    reportArea          ReportArea,
    areaOfInterest      AreaOfInterestInformation           OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,
    ...
}

LocationReportingInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-AdditionLocationInformation                 CRITICALITY ignore  EXTENSION AdditionLocationInformation                   PRESENCE optional }|
    { ID id-AerialUEFlightInformationReportingControl   CRITICALITY ignore  EXTENSION AerialUEFlightInformationReportingControl PRESENCE optional },
    ...
}

LoggedEventTriggeredConfig ::= SEQUENCE {
    eventTypeTrigger                    EventTypeTrigger,
    iE-Extensions       ProtocolExtensionContainer { { LoggedEventTriggeredConfig-ExtIEs} } OPTIONAL,
    ...
}

LoggedEventTriggeredConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LoggedMDT-NR ::= SEQUENCE {
    loggingInterval                     LoggingInterval,
    loggingDuration                     LoggingDuration,
    reportType                          ReportType,
    bluetoothMeasurementConfiguration   BluetoothMeasurementConfiguration           OPTIONAL,
    wLANMeasurementConfiguration            WLANMeasurementConfiguration            OPTIONAL,
    sensorMeasurementConfiguration      SensorMeasurementConfiguration              OPTIONAL,
    areaScopeOfNeighCellsList           AreaScopeOfNeighCellsList                   OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {LoggedMDT-NR-ExtIEs} } OPTIONAL,
    ...
}

LoggedMDT-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-earlyMeasurement     CRITICALITY ignore  EXTENSION EarlyMeasurement      PRESENCE optional       },
...
}

LoggingInterval ::= ENUMERATED { ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440, infinity,...}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120}

LowerLayerPresenceStatusChange ::= ENUMERATED {
    release-lower-layers,
    re-establish-lower-layers,
    ...,
    suspend-lower-layers,
    resume-lower-layers
}

LPWUSPSassistanceInformation ::= SEQUENCE {
    lPWUScNsubgroupID           LPWUSCNsubgroupID,
    iE-Extensions               ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,
    ...
}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

LP-WUS-Disable-Indication ::= ENUMERATED {true, ...}


LTEA2XServicesAuthorized ::= SEQUENCE {
    aerialUE                AerialUE                                                    OPTIONAL,
    aerialControllerUE      AerialControllerUE                                          OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {LTEA2XServicesAuthorized-ExtIEs} }    OPTIONAL,
    ...
}

LTEA2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTEV2XServicesAuthorized ::= SEQUENCE {
    vehicleUE           VehicleUE                                           OPTIONAL,
    pedestrianUE        PedestrianUE                                        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }    OPTIONAL,
    ...
}

LTEV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTEUESidelinkAggregateMaximumBitRate ::= SEQUENCE {
    uESidelinkAggregateMaximumBitRate       BitRate,
    iE-Extensions                   ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,
    ...
}

LTEUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTMHandoverInformationRequest ::= SEQUENCE {
    lTMIndicator                            LTMIndicator,
    proposedLTM-NoSecurityChangeID-List     LTM-NoSecurityChangeID-List     OPTIONAL,               -- This IE may need to be refined.
    targetNG-RANnodeUEXnAPID                NG-RANnodeUEXnAPID              OPTIONAL,
    referenceConfiguration                  ReferenceConfiguration          OPTIONAL,
    lTMConfigurationIDMappingList           LTMConfigurationIDMappingList   OPTIONAL,
    cSIResourceConfiguration                CSIResourceConfiguration        OPTIONAL,
    requestForCSI-RSResourceConfigForLayer1Measurements     RequestForCSI-RSResourceConfigForLayer1Measurements     OPTIONAL,
    proposedLTML2ResetConfig-List           LTML2ResetConfig-List           OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL,
    ...
}

LTMHandoverInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...     
}


LTMConfigurationIDMappingList ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{
    lTMCellID           NR-CGI,
    lTMConfigurationID  LTMConfigurationID,
    iE-Extensions       ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}       OPTIONAL
}

LTMConfigurationIDMapping-Item-ExtIEs   XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTMConfigurationID ::=  INTEGER (1..8)


LTM-NoSecurityChangeID-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTM-NoSecurityChangeID

LTML2ResetConfig-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTML2ResetConfig

LTM-NoSecurityChangeID ::= INTEGER (1..9,...)

LTML2ResetConfig    ::= INTEGER (1..9,...)

LTMHandoverInformationRequestAcknowledge ::= SEQUENCE {
    sSBInformation                              SSBInformation,
    tCIStateConfigurationList                   OCTET STRING,
    lTMCandidateConfiguration                   OCTET STRING,
    lTM-NoSecurityChangeID                      LTM-NoSecurityChangeID,
    completeCandidateConfigurationIndicator     CompleteCandidateConfigurationIndicator     OPTIONAL,
    cSI-RSResourceConfigurationForLayer1Measurements        CSI-RSResourceConfiguration     OPTIONAL,
    cSI-RSResourceConfigurationForEarlyCSIAcquisition       CSI-RSResourceConfiguration     OPTIONAL,
    lTMCFRAResourceInformation                              LTMCFRAResourceInformation      OPTIONAL,
    ltML2ResetConfig                            LTML2ResetConfig                            OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMHandoverInformationAcknowledge-ExtIEs} } OPTIONAL,
    ...
}

LTMHandoverInformationAcknowledge-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMCellSwitchInformation    ::= SEQUENCE {
    jointorDLTCIStateID             JointorDLTCIStateID,
    uLTCIStateID                    ULTCIStateID                OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }  OPTIONAL,
    ...
}

LTMCellSwitchInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTMUEAssociationInformation-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTMUEAssociationInformation-Item

LTMUEAssociationInformation-Item ::= SEQUENCE {
    candidateCellID                                 NR-CGI,    -- This IE needs to be refined
    lastTarget-NG-RANnodeUEXnAPID                   NG-RANnodeUEXnAPID,
    iE-Extensions       ProtocolExtensionContainer { { LTMUEAssociationInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

LTMUEAssociationInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMUpdatesToCandidateNodeInformation ::= SEQUENCE {
    referenceConfiguration                  ReferenceConfiguration          OPTIONAL,
    lTMConfigurationIDMappingList           LTMConfigurationIDMappingList   OPTIONAL,
    cSIResourceConfiguration                CSIResourceConfiguration        OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMUpdatesToCandidateNodeInformation-ExtIEs} } OPTIONAL,
    ...
}

LTMUpdatesToCandidateNodeInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



LTMUpdatesToCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesToCandidateCellInformation-Item

LTMUpdatesToCandidateCellInformation-Item ::= SEQUENCE {
    candidateCellID                         NR-CGI,
    earlySyncInformation                    EarlySyncInformation                OPTIONAL,
    lTMCFRAResourceInformation              LTMCFRAResourceInformation          OPTIONAL,
    ueBasedTAMeasurementConfiguration       OCTET STRING                        OPTIONAL,
    aSSecurityInformation                   AS-SecurityInformation              OPTIONAL,
    lTM-NoSecurityChangeID                  LTM-NoSecurityChangeID              OPTIONAL,
    lTMUEAssociation                        LTMUEAssociationInformation-List    OPTIONAL,
    dataForwardingInfoForLTM-List           DataForwardingInfoForLTM-List       OPTIONAL,
    lastTarget-NG-RANnodeUEXnAPID           NG-RANnodeUEXnAPID                  OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMUpdatesToCandidateCellInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

LTMUpdatesToCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMCFRAResourceInformation ::= SEQUENCE {
    lTMCFRAResourceConfiguration            OCTET STRING                        OPTIONAL,
    lTMCFRAResourceConfigurationSUL         OCTET STRING                        OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMCFRAResourceInformation-ExtIEs} } OPTIONAL,
    ...
}

LTMCFRAResourceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMUpdatesFromCandidateCellInformation-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMUpdatesFromCandidateCellInformation-Item


LTMUpdatesFromCandidateCellInformation-Item ::= SEQUENCE {
    candidateCellID                                     NR-CGI,
    lTMCandidateConfiguration                           OCTET STRING,
    completeCandidateConfigurationIndicator             CompleteCandidateConfigurationIndicator     OPTIONAL,
    cSI-RSReportConfigurationForEarlyCSIAcquisition     OCTET STRING                                OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMUpdatesFromCandidateCellInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

LTMUpdatesFromCandidateCellInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}




LTMUESecurityInformation ::= SEQUENCE {
    lTMUESecurityCapabilities                       LTMUESecurityCapabilities,
    lTMUserPlaneSecurityInformation-List            LTMUserPlaneSecurityInformation-List,
    iE-Extension        ProtocolExtensionContainer { { LTMUESecurityInformation-ExtIEs} }   OPTIONAL,
    ...
}

LTMUESecurityInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMUESecurityCapabilities ::= SEQUENCE {
    nr-EncyptionAlgorithms                  BIT STRING {nea1-128(1),
                                                        nea2-128(2),
                                                        nea3-128(3)} (SIZE(16, ...)),
    nr-IntegrityProtectionAlgorithms            BIT STRING {nia1-128(1),
                                                        nia2-128(2),
                                                        nia3-128(3)} (SIZE(16, ...)),
    iE-Extension            ProtocolExtensionContainer { {LTMUESecurityCapabilities-ExtIEs} } OPTIONAL,
    ...
}

LTMUESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMUserPlaneSecurityInformation-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF LTMUserPlaneSecurityInformation-Item


LTMUserPlaneSecurityInformation-Item ::= SEQUENCE {
    pDUSessionID                PDUSession-ID,
    securityIndication          SecurityIndication      OPTIONAL,
    iE-Extension                ProtocolExtensionContainer { { LTMUserPlaneSecurityInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

LTMUserPlaneSecurityInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


LTMCandidateCellsToBeCancelled-List ::= SEQUENCE (SIZE(0..maxnoofLTMCells)) OF LTMCandidateCellsToBeCancelled-Item

LTMCandidateCellsToBeCancelled-Item ::= SEQUENCE {
    targetCellID                NR-CGI,
    iE-Extensions               ProtocolExtensionContainer { { LTMCandidateCellsToBeCancelled-Item-ExtIEs } }   OPTIONAL,
    ...
}

LTMCandidateCellsToBeCancelled-Item-ExtIEs  XNAP-PROTOCOL-EXTENSION ::= {
    ...
}






LTMInformationSCG-AddReq ::= SEQUENCE {
    source-M-NGRAN-node-ID                  GlobalNG-RANNode-ID,
    source-M-NGRAN-node-UE-XnAP-ID          NG-RANnodeUEXnAPID,
    iE-Extensions                           ProtocolExtensionContainer { { LTMInformationSCG-AddReq-ExtIEs} } OPTIONAL,
    ...
}

LTMInformationSCG-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMInformationSCG-AddReqAck ::= SEQUENCE {
    pCell-ID                    GlobalNG-RANCell-ID,
    iE-Extensions               ProtocolExtensionContainer { { LTMInformationSCG-AddReqAck-ExtIEs} } OPTIONAL,
    ...
}

LTMInformationSCG-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMInformationSCG-ModReq ::= SEQUENCE {
    lTMReconfig                 ENUMERATED {intra-mn-ltm, ...},
    iE-Extensions               ProtocolExtensionContainer { { LTMInformationSCG-ModReq-ExtIEs } } OPTIONAL,
    ...
}

LTMInformationSCG-ModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMInterSNExecutionNotification ::= ENUMERATED {executed, ...}

LTM-DC-DataForwarding-Indicator ::= ENUMERATED {triggered, ...}

LTMPSCellInformation-AddReq ::= SEQUENCE {
    lTM-RequestIndication                   ENUMERATED {request, ...},
    maxNrofPSCellsToPrepare                 MaxNrofPSCellsToPrepare,
    suggestedLTMCandidatePSCellList         LTM-PSCell-Request-List,
    proposedLTM-NoSecurityChangeID-List     LTM-NoSecurityChangeID-List,
    cSI-ResourceConfiguration               CSIResourceConfiguration                                                    OPTIONAL,
    sCG-ReferenceConfigRequest              ENUMERATED {request, ...}                                                   OPTIONAL,   
    lTM-ConfigurationIDMappingList          LTMConfigurationIDMappingList                                               OPTIONAL,
    proposedLTML2ResetConfig-List           LTML2ResetConfig-List                                                       OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-AddReq-ExtIEs} }            OPTIONAL,
    ...
}

LTMPSCellInformation-AddReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMPSCellInformation-AddReqAck ::= SEQUENCE {
    lTM-CandidatePSCellPreparedList     LTM-PSCell-Prepared-List                                                OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { LTMPSCellInformation-AddReqAck-ExtIEs} }         OPTIONAL,
    ...
}

LTMPSCellInformation-AddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMPSCellInformation-UpdateReq ::= SEQUENCE {
    multipleSN-List                         MultipleCandidateSN-List,
    cSI-ResourceConfiguration               CSIResourceConfiguration                                            OPTIONAL,
    lTM-ConfigurationIDMappingList          LTMConfigurationIDMappingList                                       OPTIONAL,
    proposedLTM-NoSecurityChangeID-List     LTM-NoSecurityChangeID-List                                         OPTIONAL,
    lTM-SCG-SecurityConfiguration           LTM-SCG-SecurityConfiguration-List                                  OPTIONAL,
    lTM-CandidatePSCellToBeCancelled        LTMCandidateCellsToBeCancelled-List                                 OPTIONAL,
    proposedLTML2ResetConfig-List           LTML2ResetConfig-List                                               OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReq-ExtIEs} }             OPTIONAL,
    ...
}

LTMPSCellInformation-UpdateReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMPSCellInformation-UpdateReqAck ::= SEQUENCE {
    lTM-CandidatePSCellPreparedList         LTM-PSCell-Prepared-List                                            OPTIONAL,
    cSI-ResourceConfiguration               CSIResourceConfiguration                                            OPTIONAL,
    lTM-ConfigurationIDMappingList          LTMConfigurationIDMappingList                                       OPTIONAL,
    cSI-RSReportConfigurationForEarlyCSIAcquisition     OCTET STRING                                            OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-UpdateReqAck-ExtIEs} }          OPTIONAL,
    ...
}

LTMPSCellInformation-UpdateReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}



LTMPSCellInformation-UpdateRequired ::= SEQUENCE {  lTM-CandidatePSCellToBeCancelled        LTMCandidateCellsToBeCancelled-List                                 OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-UpdateRequired-ExtIEs} }        OPTIONAL,
    ...
}

LTMPSCellInformation-UpdateRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMPSCellInformation-UpdateConfirm ::= SEQUENCE {
    lTM-CandidatePSCellPreparedList         LTM-PSCell-Prepared-List                                            OPTIONAL,
    cSI-ResourceConfiguration               CSIResourceConfiguration                                            OPTIONAL,
    lTM-ConfigurationIDMappingList          LTMConfigurationIDMappingList                                       OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-UpdateConfirm-ExtIEs} }     OPTIONAL,
    ...
}

LTMPSCellInformation-UpdateConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}



LTMPSCellInformation-ChangeRequired ::= SEQUENCE {
    lTM-RequestIndication                   LTM-RequestIndication,
    multipleSNChangeRequired-List           MultipleCandidateSNChangeRequired-List,
    cSI-ResourceConfiguration               CSIResourceConfiguration                                            OPTIONAL,
    lTM-ConfigurationIDMappingList          LTMConfigurationIDMappingList                                       OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-ChangeRequired-ExtIEs } }       OPTIONAL,
    ...
}

LTMPSCellInformation-ChangeRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

LTMPSCellInformation-ChangeConfirm ::= SEQUENCE {
    lTM-SCG-SecurityConfiguration           LTM-SCG-SecurityConfiguration-List                                  OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { LTMPSCellInformation-ChangeConfirm-ExtIEs} }     OPTIONAL,
    ...
}

LTMPSCellInformation-ChangeConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}



LTM-PSCell-Request-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Request-Item

LTM-PSCell-Request-Item ::= SEQUENCE {
    pscell-id                                               NR-CGI,
    earlySyncInformationRequest                             EarlySyncInformationRequest                                     OPTIONAL,
    requestForCSI-RSResourceConfigForLayer1Measurements     RequestForCSI-RSResourceConfigForLayer1Measurements     OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { LTM-PSCell-Request-Item-ExtIEs} }                                    OPTIONAL,
    ...
}

LTM-PSCell-Request-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTM-PSCell-Prepared-List ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTM-PSCell-Prepared-Item

LTM-PSCell-Prepared-Item ::= SEQUENCE {
    pscell-id                                           NR-CGI,
    lTM-NoSecurityChangeID                              LTM-NoSecurityChangeID,
    tCI-StatesConfigurationsList                        OCTET STRING,
    sSBInformation                                      SSBInformation,
    earlySyncInformation                                EarlySyncInformation                                    OPTIONAL,
    lTMCFRAResourceInformation                          LTMCFRAResourceInformation                              OPTIONAL,
    cSI-RSResourceConfigurationForLayer1Measurements    CSI-RSResourceConfiguration                             OPTIONAL,
    cSI-RSResourceConfigurationForEarlyCSIAcquisition   CSI-RSResourceConfiguration                             OPTIONAL,
    complete-CandidateConfigurationIndicator            ENUMERATED {complete, ...}                              OPTIONAL,
    lTML2ResetConfig                                    LTML2ResetConfig                                        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { LTM-PSCell-Prepared-Item-ExtIEs} }                       OPTIONAL,
    ...
}

LTM-PSCell-Prepared-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTM-RequestIndication ::= ENUMERATED {request, ...}

LTM-SCG-SecurityConfiguration-List ::= SEQUENCE (SIZE(1..maxnoofLTMCellsPlusOne)) OF LTM-SCG-SecurityConfiguration-Item

LTM-SCG-SecurityConfiguration-Item ::= SEQUENCE {
    lTM-NoSecurityChangeID              LTM-NoSecurityChangeID,
    lTM-SCG-SecurityConfig-Info         LTM-SCG-SecurityConfig-Info-List,
    iE-Extensions                       ProtocolExtensionContainer { { LTM-SCG-SecurityConfiguration-Item-ExtIEs} } OPTIONAL,
    ...
}

LTM-SCG-SecurityConfiguration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTM-SCG-SecurityConfig-Info-List ::= SEQUENCE (SIZE(1..maxnoofSCGSecurityConfigurations)) OF LTM-SCG-SecurityConfig-Info-Item

LTM-SCG-SecurityConfig-Info-Item ::= SEQUENCE {
    s-ng-RANnode-SecurityKey            S-NG-RANnode-SecurityKey,
    sk-counter                          SK-COUNTER,
    iE-Extensions       ProtocolExtensionContainer { { LTM-SCG-SecurityConfig-Info-Item-ExtIEs} } OPTIONAL,
    ...
}

LTM-SCG-SecurityConfig-Info-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

LTMIndicator ::= ENUMERATED {true, ...}


-- M

MaxNrofRS-IndexesToReport::= INTEGER (1..64, ...)

MaxNrofPSCellsToPrepare::= INTEGER (1..8, ...)

MBSCommServiceType ::= ENUMERATED {multicast, broadcast, ...}

MDTAlignmentInfo ::= CHOICE {   
    s-BasedMDT                      S-BasedMDT,
    choice-extension                ProtocolIE-Single-Container { {MDTAlignmentInfo-ExtIEs} }
}


MDTAlignmentInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

MeasCollectionEntityIPAddress ::= TransportLayerAddress


M1Configuration ::= SEQUENCE {
    m1reportingTrigger          M1ReportingTrigger,
    m1thresholdeventA2          M1ThresholdEventA2              OPTIONAL,
-- This IE shall be present if the Measurements to Activate IE has the first bit set to "1" and the M1 Reporting Trigger IE is set to "A2event-triggered" or to "A2event-triggered periodic".
    m1periodicReporting         M1PeriodicReporting             OPTIONAL,
-- This IE shall be present if the M1 Reporting Trigger IE is set to "periodic", or to "A2event-triggered periodic".
    iE-Extensions               ProtocolExtensionContainer { { M1Configuration-ExtIEs} } OPTIONAL,
    ...
}

M1Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-BeamMeasurementIndicationM1              CRITICALITY ignore  EXTENSION BeamMeasurementIndicationM1                   PRESENCE optional       }|
    {ID id-BeamMeasurementsReportConfiguration      CRITICALITY ignore  EXTENSION BeamMeasurementsReportConfiguration       PRESENCE conditional    },
-- This IE shall be present if the Include Beam Measurements Indication IE is set to "true".
    ...
}


M1PeriodicReporting ::= SEQUENCE {
    reportInterval              ReportIntervalMDT,
    reportAmount                ReportAmountMDT,
    iE-Extensions               ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,
    ...
}

M1PeriodicReporting-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-ExtendedReportIntervalMDT        CRITICALITY ignore  EXTENSION ExtendedReportIntervalMDT     PRESENCE optional},
    ...
}

M1ReportingTrigger ::= ENUMERATED{
    periodic,
    a2eventtriggered,
    a2eventtriggered-periodic,
    ...
    
}

M1ThresholdEventA2 ::= SEQUENCE {
    measurementThreshold    MeasurementThresholdA2,
    iE-Extensions           ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,
    ...
}

M1ThresholdEventA2-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



M4Configuration ::= SEQUENCE {
    m4period            M4period,
    m4-links-to-log     Links-to-log,
    iE-Extensions       ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,
    ...
}

M4Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-M4ReportAmount      CRITICALITY ignore  EXTENSION M4ReportAmountMDT         PRESENCE optional       },
    ...
}

M4ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {
    m5period            M5period,
    m5-links-to-log     Links-to-log,
    iE-Extensions       ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,
    ...
}

M5Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-M5ReportAmount      CRITICALITY ignore  EXTENSION M5ReportAmountMDT         PRESENCE optional       },
    ...
}

M5ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {
    m6report-Interval   M6report-Interval,
    m6-links-to-log     Links-to-log,
    iE-Extensions       ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,
    ...
}

M6Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-M6ReportAmount                          CRITICALITY ignore  EXTENSION M6ReportAmountMDT                                     PRESENCE optional}|
    { ID id-ExcessPacketDelayThresholdConfiguration CRITICALITY ignore  EXTENSION ExcessPacketDelayThresholdConfiguration       PRESENCE optional},
    ...
}

M6ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30,... }


M7Configuration ::= SEQUENCE {
    m7period            M7period,
    m7-links-to-log     Links-to-log,
    iE-Extensions       ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,
    ...
}

M7Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-M7ReportAmount      CRITICALITY ignore  EXTENSION M7ReportAmountMDT     PRESENCE optional       },
    ...
}

M7ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M7period ::= INTEGER(1..60, ...)


MAC-I ::= BIT STRING (SIZE(16))


MaskedIMEISV    ::= BIT STRING (SIZE(64))


MaxCHOpreparations ::= INTEGER (1..8, ...)


MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)


MaximumIPdatarate ::= SEQUENCE {
    maxIPrate-UL            MaxIPrate,
    iE-Extensions       ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }   OPTIONAL,
    ...
}

MaximumIPdatarate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-MaxIPrate-DL    CRITICALITY ignore  EXTENSION MaxIPrate PRESENCE optional},
    ...
}

MaxIPrate ::= ENUMERATED {
    bitrate64kbs,
    max-UErate,
    ...
}


MBSFNControlRegionLength ::= INTEGER (0..3)


MBSFNSubframeAllocation-E-UTRA ::= CHOICE {
    oneframe                BIT STRING (SIZE(6)),
    fourframes              BIT STRING (SIZE(24)),
    choice-extension        ProtocolIE-Single-Container { {MBSFNSubframeAllocation-E-UTRA-ExtIEs} }
}

MBSFNSubframeAllocation-E-UTRA-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


MBSFNSubframeInfo-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofMBSFNEUTRA)) OF MBSFNSubframeInfo-E-UTRA-Item


MBSFNSubframeInfo-E-UTRA-Item ::= SEQUENCE {
    radioframeAllocationPeriod      ENUMERATED{n1,n2,n4,n8,n16,n32,...},
    radioframeAllocationOffset      INTEGER (0..7, ...),
    subframeAllocation              MBSFNSubframeAllocation-E-UTRA,
    iE-Extensions                   ProtocolExtensionContainer { {MBSFNSubframeInfo-E-UTRA-Item-ExtIEs} } OPTIONAL,
    ...
}

MBSFNSubframeInfo-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

MBS-FrequencySelectionArea-Identity ::= OCTET STRING (SIZE(3))

MBS-Area-Session-ID ::= INTEGER (0..65535, ...)

MBS-MappingandDataForwardingRequestInfofromSource ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-MappingandDataForwardingRequestInfofromSource-Item

MBS-MappingandDataForwardingRequestInfofromSource-Item ::= SEQUENCE {
    mRB-ID                      MRB-ID,
    mBS-QoSFlow-List            MBS-QoSFlow-List,
    mRB-ProgressInformation     MRB-ProgressInformation             OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { MBS-MappingandDataForwardingRequestInfofromSource-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-MappingandDataForwardingRequestInfofromSource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-DataForwarding-Indicator ::= ENUMERATED{mbs-only, ...}

MBS-DataForwardingResponseInfofromTarget ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-DataForwardingResponseInfofromTarget-Item

MBS-DataForwardingResponseInfofromTarget-Item ::= SEQUENCE {
    mRB-ID                  MRB-ID,
    dlForwardingUPTNL       UPTransportLayerInformation,
    mRB-ProgressInformation MRB-ProgressInformation                 OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { MBS-DataForwardingResponseInfofromTarget-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-DataForwardingResponseInfofromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-QoSFlow-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF QoSFlowIdentifier

MBS-QoSFlowsToAdd-List ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF MBS-QoSFlowsToAdd-Item

MBS-QoSFlowsToAdd-Item ::= SEQUENCE {
    mBS-QosFlowIdentifier               QoSFlowIdentifier,
    mBS-QosFlowLevelQosParameters       QoSFlowLevelQoSParameters,
    iE-Extensions                   ProtocolExtensionContainer { { MBS-QoSFlowsToAdd-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-QoSFlowsToAdd-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-ServiceArea ::= CHOICE {
    locationindependent     MBS-ServiceAreaInformation,
    locationdependent       MBS-ServiceAreaInformationList,
    choice-extension        ProtocolIE-Single-Container { {MBS-ServiceArea-ExtIEs} }
}

MBS-ServiceArea-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

MBS-ServiceAreaCell-List ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaInformation ::= SEQUENCE {
    mBS-ServiceAreaCell-List        MBS-ServiceAreaCell-List                                                    OPTIONAL,
    mBS-ServiceAreaTAI-List         MBS-ServiceAreaTAI-List                                                     OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }      OPTIONAL,
    ...
}

MBS-ServiceAreaInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformation-Item

MBS-ServiceAreaInformation-Item ::= SEQUENCE {  mBS-Area-Session-ID         MBS-Area-Session-ID,
    mBS-ServiceAreaInformation  MBS-ServiceAreaInformation,
    iE-Extensions           ProtocolExtensionContainer { { MBS-ServiceAreaInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-ServiceAreaInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-ServiceAreaTAI-List ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF MBS-ServiceAreaTAI-Item

MBS-ServiceAreaTAI-Item ::= SEQUENCE {
    plmn-ID                 PLMN-Identity,
    tAC                     TAC,
    iE-Extensions           ProtocolExtensionContainer { {MBS-ServiceAreaTAI-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-ServiceAreaTAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MBS-Session-ID ::= SEQUENCE {
    tMGI                            TMGI,
    nID                             NID                                                             OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {MBS-Session-ID-ExtIEs} }          OPTIONAL,
    ...
}

MBS-Session-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MBS-AssistanceInformation ::= ENUMERATED {true, ...}

MBS-SessionAssociatedInformation ::= SEQUENCE (SIZE(1..maxnoofAssociatedMBSSessions)) OF MBS-SessionAssociatedInformation-Item

MBS-SessionAssociatedInformation-Item ::= SEQUENCE {
    mBS-Session-ID              MBS-Session-ID,
    associated-QoSFlowInfo-List Associated-QoSFlowInfo-List,
    iE-Extensions               ProtocolExtensionContainer { { MBS-SessionAssociatedInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

MBS-SessionAssociatedInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {
    mBS-Session-ID          MBS-Session-ID,
    mBS-Area-Session-ID     MBS-Area-Session-ID                                                     OPTIONAL,
    active-MBS-SessioInformation            Active-MBS-SessionInformation                           OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }    OPTIONAL,
    ...
}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-MBS-AssistanceInformation           CRITICALITY ignore          EXTENSION MBS-AssistanceInformation     PRESENCE    optional    },
    ...
}


MBS-SessionInformationResponse-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformationResponse-Item

MBS-SessionInformationResponse-Item ::= SEQUENCE {
    mBS-Session-ID                      MBS-Session-ID,
    mBS-DataForwardingResponseInfofromTarget        MBS-DataForwardingResponseInfofromTarget                    OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { MBS-SessionInformationResponse-Item-ExtIEs} }    OPTIONAL,
    ...
}

MBS-SessionInformationResponse-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MRB-ID ::= INTEGER (1..512, ...)

MRB-ProgressInformation ::= CHOICE {
    pdcp-SN12           INTEGER (0..4095),
    pdcp-SN18           INTEGER (0..262143),
    choice-extension        ProtocolIE-Single-Container { { MRB-ProgressInformation-ExtIEs} }
}

MRB-ProgressInformation-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

MDT-Activation  ::= ENUMERATED {
    immediate-MDT-only,
    immediate-MDT-and-Trace,
    logged-MDT-only,
    ...
}

MDT-Configuration ::= SEQUENCE {
    mDT-Configuration-NR        MDT-Configuration-NR        OPTIONAL,
    mDT-Configuration-EUTRA     MDT-Configuration-EUTRA     OPTIONAL,
iE-Extensions       ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,
    ...
}
MDT-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-MN-only-MDT-collection  CRITICALITY ignore      EXTENSION MN-only-MDT-collection            PRESENCE optional },
    ...
}

MN-only-MDT-collection ::= ENUMERATED {
    mN-Only,
    ...
}

MDT-Configuration-NR ::= SEQUENCE {
    mdt-Activation              MDT-Activation,
    areaScopeOfMDT-NR           AreaScopeOfMDT-NR   OPTIONAL,
    mDTMode-NR                  MDTMode-NR,
    signallingBasedMDTPLMNList  MDTPLMNList         OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } OPTIONAL,
    ...
}
MDT-Configuration-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-PNI-NPN-AreaScopeofMDT       CRITICALITY ignore  EXTENSION PNI-NPN-AreaScopeofMDT        PRESENCE optional }|
    {ID id-NetworkSliceAreaScopeofMDT   CRITICALITY ignore  EXTENSION NetworkSliceAreaScopeofMDT    PRESENCE optional },
    ...
}

MDT-Configuration-EUTRA ::= SEQUENCE {
    mdt-Activation              MDT-Activation,
    areaScopeOfMDT-EUTRA        AreaScopeOfMDT-EUTRA    OPTIONAL,
    mDTMode-EUTRA               MDTMode-EUTRA,
    signallingBasedMDTPLMNList  MDTPLMNList,
    iE-Extensions       ProtocolExtensionContainer { { MDT-Configuration-EUTRA-ExtIEs} } OPTIONAL,
    ...
}
MDT-Configuration-EUTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MDT-Location-Info ::= BIT STRING (SIZE (8))


MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTMode-NR ::= CHOICE {
    immediateMDT                ImmediateMDT-NR,
    loggedMDT                   LoggedMDT-NR,
    ...,
    mDTMode-NR-Extension            MDTMode-NR-Extension
}

MDTMode-NR-Extension ::= ProtocolIE-Single-Container {{ MDTMode-NR-ExtensionIE }}

MDTMode-NR-ExtensionIE XNAP-PROTOCOL-IES ::= {
    ...
}

MDTMode-EUTRA ::= OCTET STRING


MeasObjectContainer ::= OCTET STRING

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {
    threshold-RSRP              Threshold-RSRP,
    threshold-RSRQ              Threshold-RSRQ,
    threshold-SINR              Threshold-SINR,
    choice-extension    ProtocolIE-Single-Container { { MeasurementThresholdA2-ExtIEs} }
}

MeasurementThresholdA2-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


Measurement-ID  ::= INTEGER (1..4095,...)


MIMOPRBusageInformation ::= SEQUENCE {
    dl-GBR-PRB-usage-for-MIMO                   DL-GBR-PRB-usage-for-MIMO,
    ul-GBR-PRB-usage-for-MIMO                   UL-GBR-PRB-usage-for-MIMO,
    dl-non-GBR-PRB-usage-for-MIMO               DL-non-GBR-PRB-usage-for-MIMO,
    ul-non-GBR-PRB-usage-for-MIMO               UL-non-GBR-PRB-usage-for-MIMO,
    dl-Total-PRB-usage-for-MIMO                 DL-Total-PRB-usage-for-MIMO,
    ul-Total-PRB-usage-for-MIMO                 UL-Total-PRB-usage-for-MIMO,
    iE-Extensions                           ProtocolExtensionContainer { { MIMOPRBusageInformation-ExtIEs} }    OPTIONAL,
    ...
}

MIMOPRBusageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MobileIAB-AuthorizationStatus ::= ENUMERATED {authorized, not-authorized,...}

MobileIABCell ::= ENUMERATED {
    true,
    ...
}

MobilityInformation ::= BIT STRING (SIZE(32))

MobilityParametersModificationRange ::= SEQUENCE {
    handoverTriggerChangeLowerLimit     INTEGER (-20..20),
    handoverTriggerChangeUpperLimit     INTEGER (-20..20),
    ...
}

MobilityParametersInformation ::= SEQUENCE {
    handoverTriggerChange           INTEGER (-20..20),
    ...
}


MobilityRestrictionList ::= SEQUENCE {
    serving-PLMN                        PLMN-Identity,
    equivalent-PLMNs                    SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity          OPTIONAL,
    rat-Restrictions                    RAT-RestrictionsList                                        OPTIONAL,
    forbiddenAreaInformation            ForbiddenAreaList                                           OPTIONAL,
    serviceAreaInformation              ServiceAreaList                                             OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }             OPTIONAL,
    ...
}

MobilityRestrictionList-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
{ ID id-LastE-UTRANPLMNIdentity                 CRITICALITY ignore  EXTENSION PLMN-Identity                                 PRESENCE optional       }|
{ ID id-CNTypeRestrictionsForServing            CRITICALITY ignore  EXTENSION CNTypeRestrictionsForServing              PRESENCE optional       }|
{ ID id-CNTypeRestrictionsForEquivalent         CRITICALITY ignore  EXTENSION CNTypeRestrictionsForEquivalent           PRESENCE optional       }|
{ ID id-NPNMobilityInformation                  CRITICALITY reject  EXTENSION NPNMobilityInformation                        PRESENCE optional       },
    ...
}

MonitoringRequestonAvailableBitrate ::= SEQUENCE {
    monitoringRequest                       MonitoringRequest,
    dlAvailableBitrateReportThresholds      AvailableBitrateReportThresholdList         OPTIONAL,
-- The above IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”
    ulAvailableBitrateReportThresholds      AvailableBitrateReportThresholdList         OPTIONAL,
-- The above IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”
    iE-Extensions                           ProtocolExtensionContainer { { MonitoringRequestonAvailableBitrate-ExtIEs} } OPTIONAL,
    ...
}

MonitoringRequestonAvailableBitrate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MonitoringRequest ::= ENUMERATED {ul, dl, both, stop,...}


MMSID ::= OCTET STRING (SIZE (1))

CNTypeRestrictionsForEquivalent ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF CNTypeRestrictionsForEquivalentItem

CNTypeRestrictionsForEquivalentItem ::= SEQUENCE {
    plmn-Identity                       PLMN-Identity,
    cn-Type                             ENUMERATED {epc-forbidden, fiveGC-forbidden, ...},
    iE-Extensions                       ProtocolExtensionContainer { {CNTypeRestrictionsForEquivalentItem-ExtIEs} }             OPTIONAL,
    ...
}

CNTypeRestrictionsForEquivalentItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

CNTypeRestrictionsForServing ::= ENUMERATED {
    epc-forbidden,
    ...
}

RAT-RestrictionsList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF RAT-RestrictionsItem


RAT-RestrictionsItem ::= SEQUENCE {
    plmn-Identity                   PLMN-Identity,
    rat-RestrictionInformation      RAT-RestrictionInformation,
    iE-Extensions       ProtocolExtensionContainer { {RAT-RestrictionsItem-ExtIEs} } OPTIONAL,
    ...
}

RAT-RestrictionsItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-ExtendedRATRestrictionInformation   CRITICALITY ignore  EXTENSION ExtendedRATRestrictionInformation     PRESENCE optional},
    ...
}


RAT-RestrictionInformation ::= BIT STRING {e-UTRA (0),nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} (SIZE(8, ...))


ForbiddenAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ForbiddenAreaItem


ForbiddenAreaItem ::= SEQUENCE {
    plmn-Identity       PLMN-Identity,
    forbidden-TACs      SEQUENCE (SIZE(1..maxnoofForbiddenTACs)) OF TAC,
    iE-Extensions       ProtocolExtensionContainer { {ForbiddenAreaItem-ExtIEs} } OPTIONAL,
    ...
}

ForbiddenAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}


ServiceAreaList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF ServiceAreaItem


ServiceAreaItem ::= SEQUENCE {
    plmn-Identity                       PLMN-Identity,
    allowed-TACs-ServiceArea            SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC      OPTIONAL,
    not-allowed-TACs-ServiceArea        SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ServiceAreaItem-ExtIEs} }             OPTIONAL,
    ...
}

ServiceAreaItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

MR-DC-ResourceCoordinationInfo ::= SEQUENCE {
        ng-RAN-Node-ResourceCoordinationInfo            NG-RAN-Node-ResourceCoordinationInfo,
        iE-Extension                                            ProtocolExtensionContainer { {MR-DC-ResourceCoordinationInfo-ExtIEs}}   OPTIONAL,
        ...
}

MR-DC-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
...
}

NG-RAN-Node-ResourceCoordinationInfo ::= CHOICE {
        eutra-resource-coordination-info                    E-UTRA-ResourceCoordinationInfo,
        nr-resource-coordination-info                       NR-ResourceCoordinationInfo
}

E-UTRA-ResourceCoordinationInfo ::= SEQUENCE {
        e-utra-cell                                             E-UTRA-CGI,
        ul-coordination-info                                    BIT STRING (SIZE (6..4400)),
        dl-coordination-info                                    BIT STRING (SIZE (6..4400)) OPTIONAL,
        nr-cell                                                 NR-CGI  OPTIONAL,
        e-utra-coordination-assistance-info             E-UTRA-CoordinationAssistanceInfo   OPTIONAL,
        iE-Extension            ProtocolExtensionContainer { {E-UTRA-ResourceCoordinationInfo-ExtIEs} } OPTIONAL,
    ...
}

E-UTRA-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

E-UTRA-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

NR-ResourceCoordinationInfo ::= SEQUENCE {
        nr-cell                                                 NR-CGI,
        ul-coordination-info                                    BIT STRING (SIZE (6..4400)),
        dl-coordination-info                                    BIT STRING (SIZE (6..4400)) OPTIONAL,
        e-utra-cell                                             E-UTRA-CGI  OPTIONAL,
        nr-coordination-assistance-info                 NR-CoordinationAssistanceInfo       OPTIONAL,
        iE-Extension            ProtocolExtensionContainer { {NR-ResourceCoordinationInfo-ExtIEs} }     OPTIONAL,
    ...
}

NR-ResourceCoordinationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NR-CoordinationAssistanceInfo ::= ENUMERATED {coordination-not-required, ...}

MessageOversizeNotification ::= SEQUENCE {
    maximumCellListSize                             MaximumCellListSize,
    iE-Extension                ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}  OPTIONAL,
    ...
}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {
    mT-SDT-Indicator            MT-SDT-Indicator,
    mT-SDT-DataSize         MT-SDT-DataSize,
    iE-Extensions               ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,
    ...
}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MT-SDT-DataSize ::= INTEGER (1..96000, ...)

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MultiplexingInfo    ::= SEQUENCE{
    iAB-MT-Cell-List    IAB-MT-Cell-List,
    iE-Extensions       ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,
    ...
}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MeasuredUETrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsTrajectory)) OF MeasuredUETrajectory-Item

MeasuredUETrajectory-Item ::= SEQUENCE{
    measuredtrajectoryCellInfo      MeasuredTrajectoryCellInfo,
    iE-Extensions                   ProtocolExtensionContainer { { MeasuredUETrajectory-Item-ExtIEs} }  OPTIONAL,
    ...
}

MeasuredUETrajectory-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MeasuredTrajectoryCellInfo ::= CHOICE {
    nG-RAN-Cell                     MeasuredTrajectoryNGRANCellInfo,
    choice-extension                ProtocolIE-Single-Container { { MeasuredTrajectoryCellInfo-ExtIEs} }
}

MeasuredTrajectoryCellInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

MeasuredTrajectoryNGRANCellInfo ::= SEQUENCE {
    globalNG-RANCell-ID             GlobalNG-RANCell-ID,
    timeUEStaysInCell               INTEGER (0..4095),
    iE-Extensions                   ProtocolExtensionContainer { { MeasuredTrajectoryNGRANCellInfo-ExtIEs} }    OPTIONAL,
    ...
}

MeasuredTrajectoryNGRANCellInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MultipleCandidateSN-List ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF MultipleCandidateSN-Item

MultipleCandidateSN-Item ::= SEQUENCE {
    s-NG-RANnodeID                      GlobalNG-RANNode-ID,
    lTM-CandidatePSCellPreparedList     LTM-PSCell-Prepared-List                                    OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { MultipleCandidateSN-Item-ExtIEs} }           OPTIONAL,
    ...
}

MultipleCandidateSN-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

MultipleCandidateSNChangeRequired-List ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF MultipleCandidateSNChangeRequired-Item

MultipleCandidateSNChangeRequired-Item ::= SEQUENCE {
    s-NG-RANnodeID                              GlobalNG-RANNode-ID,
    maxNrofPSCellsToPrepare                     MaxNrofPSCellsToPrepare,
    sN-to-MN-Container                          OCTET STRING,
    suggestedLTMCandidatePSCellList             LTM-PSCell-Request-List                                         OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { MultipleCandidateSNChangeRequired-Item-ExtIEs} }     OPTIONAL,
    ...
}

MultipleCandidateSNChangeRequired-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- N

N6JitterInformation ::= SEQUENCE {
    n6JitterLowerBound      INTEGER (-127..127),
    n6JitterUpperBound      INTEGER (-127..127),
    iE-Extensions           ProtocolExtensionContainer { { N6JitterInformationExtIEs } }    OPTIONAL,
    ...
}
N6JitterInformationExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NACellResourceConfigurationList ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF NACellResourceConfiguration-Item

NACellResourceConfiguration-Item ::= SEQUENCE {
    nAdownlink          ENUMERATED {true, false, ...}   OPTIONAL,
    nAuplink            ENUMERATED {true, false, ...}   OPTIONAL,
    nAflexible          ENUMERATED {true, false, ...}   OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { NACellResourceConfiguration-Item-ExtIEs} } OPTIONAL,
    ...
}

NACellResourceConfiguration-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NBIoT-UL-DL-AlignmentOffset ::= ENUMERATED {
    khz-7dot5,
    khz0,
    khz7dot5,
    ...
}
NE-DC-TDM-Pattern ::= SEQUENCE {
        subframeAssignment          ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6},
        harqOffset                  INTEGER (0..9),
        iE-Extension                ProtocolExtensionContainer { {NE-DC-TDM-Pattern-ExtIEs}}    OPTIONAL,
        ...
}

NE-DC-TDM-Pattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
...
}

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

NeighbourInformation-E-UTRA-Item ::= SEQUENCE {
    e-utra-PCI          E-UTRAPCI,
    e-utra-cgi          E-UTRA-CGI,
    earfcn              E-UTRAARFCN,
    tac                 TAC,
    ranac               RANAC                                                                       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {NeighbourInformation-E-UTRA-Item-ExtIEs} }    OPTIONAL,
    ...
}

NeighbourInformation-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}


NeighbourInformation-NR ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-NR-Item

NeighbourInformation-NR-Item ::= SEQUENCE {
    nr-PCI                              NRPCI,
    nr-cgi                              NR-CGI,
    tac                                 TAC,
    ranac                               RANAC                                                       OPTIONAL,
    nr-mode-info                        NeighbourInformation-NR-ModeInfo,
    connectivitySupport                 Connectivity-Support,
    measurementTimingConfiguration      OCTET STRING,
    iE-Extensions           ProtocolExtensionContainer { {NeighbourInformation-NR-Item-ExtIEs} }    OPTIONAL,
    ...
}

NeighbourInformation-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    { ID id-MobileIABCell                       CRITICALITY ignore  EXTENSION MobileIABCell PRESENCE optional},
    ...
}


NeighbourInformation-NR-ModeInfo ::= CHOICE {
    fdd-info                NeighbourInformation-NR-ModeFDDInfo,
    tdd-info                NeighbourInformation-NR-ModeTDDInfo,
    choice-extension        ProtocolIE-Single-Container { {NeighbourInformation-NR-ModeInfo-ExtIEs} }
}

NeighbourInformation-NR-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


NeighbourInformation-NR-ModeFDDInfo ::= SEQUENCE {
    ul-NR-FreqInfo      NRFrequencyInfo,
    dl-NR-FequInfo      NRFrequencyInfo,
    ie-Extensions       ProtocolExtensionContainer { {NeighbourInformation-NR-ModeFDDInfo-ExtIEs} } OPTIONAL,
    ...
}

NeighbourInformation-NR-ModeFDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NeighbourInformation-NR-ModeTDDInfo ::= SEQUENCE {
    nr-FreqInfo         NRFrequencyInfo,
    ie-Extensions       ProtocolExtensionContainer { {NeighbourInformation-NR-ModeTDDInfo-ExtIEs} } OPTIONAL,
    ...
}

NeighbourInformation-NR-ModeTDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


Neighbour-NG-RAN-Node-List ::= SEQUENCE (SIZE(0..maxnoofNeighbour-NG-RAN-Nodes)) OF Neighbour-NG-RAN-Node-Item

Neighbour-NG-RAN-Node-Item ::= SEQUENCE {
    globalNG-RANNodeID              GlobalNG-RANNode-ID,
    local-NG-RAN-Node-Identifier    Local-NG-RAN-Node-Identifier,
    ie-Extensions       ProtocolExtensionContainer { {Neighbour-NG-RAN-Node-Item-ExtIEs} } OPTIONAL,
    ...
}

Neighbour-NG-RAN-Node-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NetworkSliceListforMDT ::= SEQUENCE (SIZE(1.. maxnoofMDTPLMNs)) OF NetworkSliceItemforMDT

NetworkSliceItemforMDT ::= SEQUENCE {
    plmnID              PLMN-Identity,
    sliceMDTList        SliceMDTList,
    iE-Extensions       ProtocolExtensionContainer { {NetworkSliceListforMDT-ExtIEs} }  OPTIONAL,
    ...
}

NetworkSliceListforMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

NetworkSliceAreaScopeofMDT ::= SEQUENCE {
    networkSliceListforMDT      NetworkSliceListforMDT,
    iE-Extensions               ProtocolExtensionContainer { {NetworkSliceAreaScopeofMDT-ExtIEs} } OPTIONAL,
    ...
}

NetworkSliceAreaScopeofMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NID ::= BIT STRING (SIZE(44))


NRCarrierList ::= SEQUENCE (SIZE(1..maxnoofNRSCSs)) OF NRCarrierItem

NRCarrierItem ::= SEQUENCE {
    carrierSCS                  NRSCS,
    offsetToCarrier             INTEGER (0..2199, ...),
    carrierBandwidth            INTEGER (0..maxnoofPhysicalResourceBlocks, ...),
    iE-Extension        ProtocolExtensionContainer { {NRCarrierItem-ExtIEs} }       OPTIONAL,
    ...
}

NRCarrierItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NRCellPRACHConfig ::= OCTET STRING


NG-RAN-Cell-Identity ::= CHOICE {
    nr                      NR-Cell-Identity,
    e-utra                  E-UTRA-Cell-Identity,
    choice-extension        ProtocolIE-Single-Container { {NG-RAN-Cell-Identity-ExtIEs} }
}

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


NG-RAN-CellPCI ::= CHOICE {
    nr                  NRPCI,
    e-utra              E-UTRAPCI,
    choice-extension    ProtocolIE-Single-Container { {NG-RAN-CellPCI-ExtIEs} }
}

NG-RAN-CellPCI-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


NG-RANnode2SSBOffsetsModificationRange ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBOffsetModificationRange

NG-RANnodeUEXnAPID ::= INTEGER (0.. 4294967295)


NumberofActiveUEs::= INTEGER(0..16777215, ...)


NodeAssociatedInfoResult ::= SEQUENCE {
    energyCost              EnergyCost          OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { NodeAssociatedInfoResult-ExtIEs} } OPTIONAL,
    ...
}

NodeAssociatedInfoResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NodeMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxFailedMeasPerNode)) OF NodeMeasurementInitiationResult-Item


NodeMeasurementInitiationResult-Item ::= SEQUENCE {
    nodemeasurementFailedReportCharacteristics      BIT STRING(SIZE(32)),
    cause                                           Cause,
    iE-Extensions                                   ProtocolExtensionContainer { { NodeMeasurementInitiationResult-Item-ExtIEs} } OPTIONAL,
    ...
}

NodeMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NoofRRCConnections ::= INTEGER (1..65536,...)


NonDynamic5QIDescriptor ::= SEQUENCE {
    fiveQI                      FiveQI,
    priorityLevelQoS            PriorityLevelQoS                                                    OPTIONAL,
    averagingWindow             AveragingWindow                                                     OPTIONAL,
    maximumDataBurstVolume      MaximumDataBurstVolume                                              OPTIONAL,
    iE-Extension                ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs } }    OPTIONAL,
    ...
}

NonDynamic5QIDescriptor-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CNPacketDelayBudgetDownlink     CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget PRESENCE optional}|
    { ID id-CNPacketDelayBudgetUplink       CRITICALITY ignore  EXTENSION ExtendedPacketDelayBudget PRESENCE optional},
    ...
}


NRARFCN ::= INTEGER (0.. maxNRARFCN)


NG-eNB-RadioResourceStatus  ::= SEQUENCE {
    dL-GBR-PRB-usage                            DL-GBR-PRB-usage,
    uL-GBR-PRB-usage                            UL-GBR-PRB-usage,
    dL-non-GBR-PRB-usage                        DL-non-GBR-PRB-usage,
    uL-non-GBR-PRB-usage                        UL-non-GBR-PRB-usage,
    dL-Total-PRB-usage                          DL-Total-PRB-usage,
    uL-Total-PRB-usage                          UL-Total-PRB-usage,
    iE-Extensions                           ProtocolExtensionContainer { { NG-eNB-RadioResourceStatus-ExtIEs} } OPTIONAL,
    ...
}

NG-eNB-RadioResourceStatus-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DL-scheduling-PDCCH-CCE-usage       CRITICALITY ignore  EXTENSION DL-scheduling-PDCCH-CCE-usage PRESENCE optional}|
    { ID id-UL-scheduling-PDCCH-CCE-usage       CRITICALITY ignore  EXTENSION UL-scheduling-PDCCH-CCE-usage PRESENCE optional},
    ...
}

DL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)
UL-scheduling-PDCCH-CCE-usage ::= INTEGER (0.. 100)


TNLCapacityIndicator ::= SEQUENCE {
    dLTNLOfferedCapacity                    OfferedCapacity,
    dLTNLAvailableCapacity                  AvailableCapacity,
    uLTNLOfferedCapacity                    OfferedCapacity,
    uLTNLAvailableCapacity                  AvailableCapacity,
    iE-Extensions                           ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL,
    ...
}

TNLCapacityIndicator-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Non-F1-TerminatingTopologyBHInformation ::= SEQUENCE {
    nonF1TerminatingBHInformation-List      NonF1TerminatingBHInformation-List,
    bAPControlPDURLCCH-List                 BAPControlPDURLCCH-List         OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { {Non-F1-TerminatingTopologyBHInformation-ExtIEs} } OPTIONAL,
    ...
}

Non-F1-TerminatingTopologyBHInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NonF1TerminatingBHInformation-List ::= SEQUENCE (SIZE(1..maxnoofBHInfo)) OF NonF1TerminatingBHInformation-Item

NonF1TerminatingBHInformation-Item ::= SEQUENCE {
    bHInfoIndex                     BHInfoIndex,
    dlNon-F1TerminatingBHInfo       DLNonF1Terminating-BHInfo       OPTIONAL,
    ulNon-F1TerminatingBHInfo       ULNonF1Terminating-BHInfo       OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { { NonF1TerminatingBHInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

NonF1TerminatingBHInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NonUPTraffic ::= CHOICE {
    nonUPTrafficType                NonUPTrafficType,
    controlPlaneTrafficType         ControlPlaneTrafficType,
    choice-extension                ProtocolIE-Single-Container { { NonUPTraffic-ExtIEs} }
}

NonUPTraffic-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NonUPTrafficType ::= ENUMERATED {ueassociatedf1ap, nonueassociatedf1ap, nonf1, ...}

NoPDUSessionIndication  ::= ENUMERATED {true, ...}

NPN-Broadcast-Information ::= CHOICE {
    snpn-Information                    NPN-Broadcast-Information-SNPN,
    pni-npn-Information                 NPN-Broadcast-Information-PNI-NPN,
    choice-extension                    ProtocolIE-Single-Container { {NPN-Broadcast-Information-ExtIEs} }
}

NPN-Broadcast-Information-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NPN-Broadcast-Information-SNPN ::= SEQUENCE {
    broadcastSNPNID-List        BroadcastSNPNID-List,
    iE-Extension                ProtocolExtensionContainer { {NPN-Broadcast-Information-SNPN-ExtIEs} }  OPTIONAL,
    ...
}

NPN-Broadcast-Information-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
NPN-Broadcast-Information-PNI-NPN ::= SEQUENCE {
    broadcastPNI-NPN-ID-Information     BroadcastPNI-NPN-ID-Information,
    iE-Extension                        ProtocolExtensionContainer { {NPN-Broadcast-Information-PNI-NPN-ExtIEs} }   OPTIONAL,
    ...
}

NPN-Broadcast-Information-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NPNMobilityInformation::= CHOICE {
    snpn-mobility-information           NPNMobilityInformation-SNPN,
    pni-npn-mobility-information        NPNMobilityInformation-PNI-NPN,
    choice-extension                    ProtocolIE-Single-Container { {NPNMobilityInformation-ExtIEs} }
}

NPNMobilityInformation-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NPNMobilityInformation-SNPN ::= SEQUENCE {
    serving-NID                 NID,
    iE-Extension                ProtocolExtensionContainer { {NPNMobilityInformation-SNPN-ExtIEs} } OPTIONAL,
    ...
}

NPNMobilityInformation-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-EquivalentSNPNs     CRITICALITY reject  EXTENSION EquivalentSNPNs   PRESENCE optional},
    ...
}

NPNMobilityInformation-PNI-NPN ::= SEQUENCE {
    allowedPNI-NPN-ID-List      AllowedPNI-NPN-ID-List,
    iE-Extension                ProtocolExtensionContainer { {NPNMobilityInformation-PNI-NPN-ExtIEs} }  OPTIONAL,
    ...
}

NPNMobilityInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NPNPagingAssistanceInformation ::= CHOICE {
    pni-npn-Information                 NPNPagingAssistanceInformation-PNI-NPN,
    choice-extension                    ProtocolIE-Single-Container { {NPNPagingAssistanceInformation-ExtIEs} }
}

NPNPagingAssistanceInformation-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NPNPagingAssistanceInformation-PNI-NPN ::= SEQUENCE {
    allowedPNI-NPN-ID-List          AllowedPNI-NPN-ID-List,
    iE-Extension                    ProtocolExtensionContainer { {NPNPagingAssistanceInformation-PNI-NPN-ExtIEs} }  OPTIONAL,
    ...
}

NPNPagingAssistanceInformation-PNI-NPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NPN-Support ::= CHOICE {
    sNPN                    NPN-Support-SNPN,
    choice-Extensions       ProtocolIE-Single-Container { {NPN-Support-ExtIEs} }
}

NPN-Support-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NPN-Support-SNPN ::= SEQUENCE {
    nid                 NID,
    ie-Extension        ProtocolExtensionContainer { {NPN-Support-SNPN-ExtIEs} }    OPTIONAL,
    ...
}

NPN-Support-SNPN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NPRACHConfiguration::= SEQUENCE {
    fdd-or-tdd              CHOICE {
        fdd         NPRACHConfiguration-FDD,
        tdd         NPRACHConfiguration-TDD,
        choice-extension        ProtocolIE-Single-Container { { FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs} }
    },  
    iE-Extensions                       ProtocolExtensionContainer { { NPRACHConfiguration-ExtIEs} }    OPTIONAL,
    ...
}

NPRACHConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

FDD-or-TDD-in-NPRACHConfiguration-Choice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NPRACHConfiguration-FDD::= SEQUENCE {
    nprach-CP-length                        NPRACH-CP-Length,
    anchorCarrier-NPRACHConfig                      OCTET STRING,
    anchorCarrier-EDT-NPRACHConfig                  OCTET STRING                        OPTIONAL,
    anchorCarrier-Format2-NPRACHConfig              OCTET STRING                        OPTIONAL,
    anchorCarrier-Format2-EDT-NPRACHConfig          OCTET STRING                        OPTIONAL,
    non-anchorCarrier-NPRACHConfig                  OCTET STRING                        OPTIONAL,
    non-anchorCarrier-Format2-NPRACHConfig          OCTET STRING                        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { NPRACHConfiguration-FDD-ExtIEs} }    OPTIONAL,
    ...
}

NPRACHConfiguration-FDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NPRACHConfiguration-TDD::= SEQUENCE {
    nprach-preambleFormat                           NPRACH-preambleFormat,
    anchorCarrier-NPRACHConfigTDD                   OCTET STRING,
    non-anchorCarrierFequencyConfiglist             Non-AnchorCarrierFrequencylist      OPTIONAL,
    non-anchorCarrier-NPRACHConfigTDD               OCTET STRING                        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { NPRACHConfiguration-TDD-ExtIEs} }    OPTIONAL,

...
}

NPRACHConfiguration-TDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NPRACH-CP-Length::=     ENUMERATED {
    us66dot7,
    us266dot7,
    ...
}

NPRACH-preambleFormat::=    ENUMERATED {fmt0,fmt1,fmt2,fmt0a,fmt1a,...}

Non-AnchorCarrierFrequencylist ::= SEQUENCE (SIZE(1..maxnoofNonAnchorCarrierFreqConfig)) OF
    SEQUENCE {
        non-anchorCarrierFrquency       OCTET STRING,
        iE-Extensions                   ProtocolExtensionContainer { { Non-AnchorCarrierFrequencylist-ExtIEs} } OPTIONAL,
        ...
    }

Non-AnchorCarrierFrequencylist-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NR-Cell-Identity        ::= BIT STRING (SIZE (36))


NG-RAN-Cell-Identity-ListinRANPagingArea ::= SEQUENCE (SIZE (1..maxnoofCellsinRNA)) OF NG-RAN-Cell-Identity


NR-CGI ::= SEQUENCE {
    plmn-id             PLMN-Identity,
    nr-CI               NR-Cell-Identity,
    iE-Extension        ProtocolExtensionContainer { {NR-CGI-ExtIEs} }  OPTIONAL,
    ...
}

NR-CGI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {
    nR-U-ChannelID                      NR-U-ChannelID,
    channelOccupancyTimePercentageDL    ChannelOccupancyTimePercentage,
    energyDetectionThresholdDL          EnergyDetectionThreshold,
    iE-Extension        ProtocolExtensionContainer { {NR-U-Channel-Item-ExtIEs} }   OPTIONAL,
    ...
}

NR-U-Channel-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-ChannelOccupancyTimePercentageUL    CRITICALITY ignore  EXTENSION ChannelOccupancyTimePercentage    PRESENCE optional}|
    { ID id-EnergyDetectionThresholdUL          CRITICALITY ignore  EXTENSION EnergyDetectionThreshold          PRESENCE optional}|
    { ID id-RadioResourceStatusNR-U             CRITICALITY ignore  EXTENSION RadioResourceStatusNR-U           PRESENCE optional},
    ...
}

NR-U-ChannelID ::= INTEGER (1..maxnoofNR-UChannelIDs, ...)

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)

EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)


NR-U-ChannelInfo-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-ChannelInfo-Item

NR-U-ChannelInfo-Item ::= SEQUENCE {
    nR-U-ChannelID  NR-U-ChannelID,
    nRARFCN         NRARFCN,
    bandwidth       Bandwidth,
    iE-Extension        ProtocolExtensionContainer { {NR-U-ChannelInfo-Item-ExtIEs} }   OPTIONAL,
    ...
}

NR-U-ChannelInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


Bandwidth ::= ENUMERATED{mhz10, mhz20, mhz40, mhz60, mhz80, ...,mhz100}

NRA2XServicesAuthorized ::= SEQUENCE {
    aerialUE                AerialUE                                                    OPTIONAL,
    aerialControllerUE  AerialControllerUE                                      OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {NRA2XServicesAuthorized-ExtIEs} } OPTIONAL,
    ...
}

NRA2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NRCyclicPrefix ::= ENUMERATED {normal, extended, ...}

NRDL-ULTransmissionPeriodicity ::= ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ...}

NRFrequencyBand ::= INTEGER (1..1024, ...)


NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBandItem

NRFrequencyBandItem ::= SEQUENCE {
    nr-frequency-band           NRFrequencyBand,
    supported-SUL-Band-List     SupportedSULBandList                                            OPTIONAL,
    iE-Extension                ProtocolExtensionContainer { {NRFrequencyBandItem-ExtIEs} }     OPTIONAL,
    ...
}

NRFrequencyBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



NRFrequencyInfo ::= SEQUENCE {
    nrARFCN             NRARFCN,
    sul-information     SUL-Information                                             OPTIONAL,
    frequencyBand-List  NRFrequencyBand-List,
    iE-Extension        ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} }     OPTIONAL,
    ...
}

NRFrequencyInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-FrequencyShift7p5khz    CRITICALITY ignore  EXTENSION FrequencyShift7p5khz  PRESENCE optional },...
}

NRMobilityHistoryReport ::= OCTET STRING


NRModeInfo ::= CHOICE {
    fdd                         NRModeInfoFDD,
    tdd                         NRModeInfoTDD,
    choice-extension            ProtocolIE-Single-Container { {NRModeInfo-ExtIEs} }
}

NRModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

NRModeInfoFDD ::= SEQUENCE {
    ulNRFrequencyInfo           NRFrequencyInfo,
    dlNRFrequencyInfo           NRFrequencyInfo,
    ulNRTransmissonBandwidth    NRTransmissionBandwidth,
    dlNRTransmissonBandwidth    NRTransmissionBandwidth,
    iE-Extension        ProtocolExtensionContainer { {NRModeInfoFDD-ExtIEs} }   OPTIONAL,
    ...
}

NRModeInfoFDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-ULCarrierList                           CRITICALITY ignore  EXTENSION NRCarrierList                                     PRESENCE optional }|
    { ID id-DLCarrierList                           CRITICALITY ignore  EXTENSION NRCarrierList                                     PRESENCE optional }|
    { ID id-UL-GNB-DU-Cell-Resource-Configuration   CRITICALITY ignore  EXTENSION GNB-DU-Cell-Resource-Configuration        PRESENCE optional }|
    { ID id-DL-GNB-DU-Cell-Resource-Configuration   CRITICALITY ignore  EXTENSION GNB-DU-Cell-Resource-Configuration        PRESENCE optional },
...
}


NRModeInfoTDD ::= SEQUENCE {
    nrFrequencyInfo         NRFrequencyInfo,
    nrTransmissonBandwidth  NRTransmissionBandwidth,
    iE-Extension            ProtocolExtensionContainer { {NRModeInfoTDD-ExtIEs} }   OPTIONAL,
    ...
}

NRModeInfoTDD-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-IntendedTDD-DL-ULConfiguration-NR       CRITICALITY ignore  EXTENSION IntendedTDD-DL-ULConfiguration-NR     PRESENCE optional }|
    { ID id-TDDULDLConfigurationCommonNR            CRITICALITY ignore  EXTENSION TDDULDLConfigurationCommonNR          PRESENCE optional }|
    { ID id-CarrierList                             CRITICALITY ignore  EXTENSION NRCarrierList                                 PRESENCE optional }|
    { ID id-tdd-GNB-DU-Cell-Resource-Configuration  CRITICALITY ignore  EXTENSION GNB-DU-Cell-Resource-Configuration    PRESENCE optional }|
    { ID id-Transmission-Bandwidth-asymmetric       CRITICALITY ignore  EXTENSION Transmission-Bandwidth-asymmetric     PRESENCE optional }|
    { ID id-SBFD-Frequency-Configuration            CRITICALITY ignore  EXTENSION SBFD-Frequency-Configuration          PRESENCE optional },
    ...
}


NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242, nrb15, nrb35}


NRPagingeDRXInformation ::= SEQUENCE {
    nRPaging-eDRX-Cycle     NRPaging-eDRX-Cycle,
    nRPaging-Time-Window    NRPaging-Time-Window                    OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} } OPTIONAL,
    ...
}

NRPagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NRPaging-eDRX-Cycle ::= ENUMERATED {
    hfquarter, hfhalf, hf1, hf2, hf4,
    hf8, hf16,
    hf32, hf64, hf128, hf256,
    hf512, hf1024,
    ...
}


NRPaging-Time-Window ::= ENUMERATED {
    s1, s2, s3, s4, s5,
    s6, s7, s8, s9, s10,
    s11, s12, s13, s14, s15, s16,
    ...,s17, s18, s19, s20, s21, s22,
    s23, s24, s25, s26, s27, s28, s29,
    s30, s31, s32
}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {
    nRPaging-eDRX-Cycle-Inactive        NRPaging-eDRX-Cycle-Inactive,
    iE-Extensions                       ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }  OPTIONAL,
    ...
}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {
    hfquarter, hfhalf, hf1,
    ...
}

NRPagingLongeDRXInformationforRRCINACTIVE ::= SEQUENCE {
    nRPaging-long-eDRX-Cycle-Inactive   NRPaging-long-eDRX-Cycle-Inactive,
    nRPaging-Time-Window-Inactive       NRPaging-Time-Window-Inactive,
    iE-Extensions                       ProtocolExtensionContainer { {NRPagingLongeDRXInformationforRRCINACTIVE-ExtIEs} }   OPTIONAL,
    ...
}

NRPagingLongeDRXInformationforRRCINACTIVE-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NRPaging-long-eDRX-Cycle-Inactive ::= ENUMERATED {
    hf2, hf4, hf8, hf16,
    hf32, hf64, hf128, hf256,
    hf512, hf1024,
    ...
}


NRPaging-Time-Window-Inactive ::= ENUMERATED {
    s1, s2, s3, s4, s5,
    s6, s7, s8, s9, s10,
    s11, s12, s13, s14, s15, s16,
    s17, s18, s19, s20, s21, s22,
    s23, s24, s25, s26, s27, s28, s29,
    s30, s31, s32, ...
}

NRPCI ::= INTEGER (0..1007, ...)

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ..., scs480, scs960}


NRTransmissionBandwidth ::= SEQUENCE {
    nRSCS   NRSCS,
    nRNRB   NRNRB,
    iE-Extensions               ProtocolExtensionContainer { {NRTransmissionBandwidth-ExtIEs} } OPTIONAL,
    ...
}

NRPPaPositioningInformation ::= SEQUENCE {
    routingID                                           RoutingID,
    nRPPaTransactionID                                  INTEGER (0..32767),
    iE-Extension                ProtocolExtensionContainer { { NRPPaPositioningInformation-ExtIEs} } OPTIONAL,
    ...
}

NRPPaPositioningInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NRTransmissionBandwidth-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Transmission-Bandwidth-asymmetric ::= SEQUENCE {
    ul-Transmission-Bandwidth   NRTransmissionBandwidth,
    dl-Transmission-Bandwidth   NRTransmissionBandwidth,
    iE-Extensions               ProtocolExtensionContainer { { Transmission-Bandwidth-asymmetric-ExtIEs} } OPTIONAL,
    ...
}

Transmission-Bandwidth-asymmetric-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NumberOfAntennaPorts-E-UTRA ::= ENUMERATED {an1, an2, an4, ...}

NG-RANTraceID               ::=OCTET STRING (SIZE (8))

NonGBRResources-Offered ::= ENUMERATED {true, ...}

NRV2XServicesAuthorized ::= SEQUENCE {
    vehicleUE           VehicleUE                           OPTIONAL,
    pedestrianUE        PedestrianUE                        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} } OPTIONAL,
    ...
}

NRV2XServicesAuthorized-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


NRUESidelinkAggregateMaximumBitRate ::= SEQUENCE {
    uESidelinkAggregateMaximumBitRate       BitRate,
    iE-Extensions                   ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitRate-ExtIEs} } OPTIONAL,
    ...
}

NRUESidelinkAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NSAG-ID ::= INTEGER (0..255, ...)

NZP-CSI-RS-Resources-Config ::= SEQUENCE {
    nZP-CSI-RS-ResourceSet                  OCTET STRING,
    nZP-CSI-RS-Resource-List                NZP-CSI-RS-Resource-List,
    iE-Extensions                           ProtocolExtensionContainer { {NZP-CSI-RS-Resources-Config-ExtIEs} } OPTIONAL,
    ...
}

NZP-CSI-RS-Resources-Config-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NZP-CSI-RS-Resource-List ::= SEQUENCE (SIZE(1..maxnoofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-Resource-Item

NZP-CSI-RS-Resource-Item ::= SEQUENCE {
    nZP-CSI-RS-Resource                     OCTET STRING,
    iE-Extensions                           ProtocolExtensionContainer { {NZP-CSI-RS-Resource-Item-ExtIEs} } OPTIONAL,
    ...
}

NZP-CSI-RS-Resource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

NTNGeographicalArea ::= SEQUENCE (SIZE(1.. maxnoofAreaNTNforMDT)) OF NTNGeographicalAreaItem

NTNGeographicalAreaItem ::= CHOICE {
    circle                      Circle,
    polygon                     Polygon,
    cHOICE-Extensions           ProtocolIE-Single-Container { {NTNGeographicalAreaItem-ExtIEs} }
}

NTNGeographicalAreaItem-ExtIEs XNAP-PROTOCOL-IES ::={
    ...
}



-- O


OfferedCapacity ::= INTEGER (1.. 16777216,...)

OffsetOfNbiotChannelNumberToEARFCN ::= ENUMERATED {
        minusTen,
        minusNine,
        minusEightDotFive,
        minusEight,
        minusSeven,
        minusSix,
        minusFive,
        minusFourDotFive,
        minusFour,
        minusThree,
        minusTwo,
        minusOne,
        minusZeroDotFive,
        zero,
        one,
        two,
        three,
        threeDotFive,
        four,
        five,
        six,
        seven,
        sevenDotFive,
        eight,
        nine,
        ...
}

-- P
Polygon ::= OCTET STRING

PositioningInformation ::= SEQUENCE {
    requestedSRSTransmissionCharacteristics             RequestedSRSTransmissionCharacteristics,
    routingID                                           RoutingID,
    nRPPaTransactionID                                  INTEGER (0..32767),
    iE-Extension                ProtocolExtensionContainer { { PositioningInformation-ExtIEs} } OPTIONAL,
    ...
}

PositioningInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PacketDelayBudget ::= INTEGER (0..1023, ...)


PacketErrorRate ::= SEQUENCE {
    pER-Scalar          PER-Scalar,
    pER-Exponent        PER-Exponent,
    iE-Extensions       ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} } OPTIONAL,
    ...
}

PacketErrorRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PagingCause ::= ENUMERATED {
    voice,
    ...
}

PedestrianUE ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PEIPSassistanceInformation ::= SEQUENCE {
    cNsubgroupID            CNsubgroupID,
    iE-Extensions       ProtocolExtensionContainer { {PEIPSassistanceInformation-ExtIEs} } OPTIONAL,
    ...
}

PEIPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PacketLossRate ::= INTEGER (0..1000, ...)


PagingDRX   ::= ENUMERATED {
    v32,
    v64,
    v128,
    v256,
    ... ,
    v512,
    v1024
    }


PagingPriority ::= ENUMERATED {
    priolevel1,
    priolevel2,
    priolevel3,
    priolevel4,
    priolevel5,
    priolevel6,
    priolevel7,
    priolevel8,
    ...
}

PartialListIndicator ::= ENUMERATED {partial, ...}

PC5QoSParameters ::= SEQUENCE {
    pc5QoSFlowList              PC5QoSFlowList,
    pc5LinkAggregateBitRates    BitRate             OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs} }   OPTIONAL,
    ...
}


PC5QoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF PC5QoSFlowItem
-- The size of the PC5 QoS Flow List shall not exceed 2048 items.

PC5QoSFlowItem::= SEQUENCE {
    pQI                         FiveQI,
    pc5FlowBitRates             PC5FlowBitRates             OPTIONAL,
    range                       Range                       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { PC5QoSFlowItem-ExtIEs} } OPTIONAL,
    ...
}

PC5QoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PC5FlowBitRates ::= SEQUENCE {
    guaranteedFlowBitRate       BitRate,
    maximumFlowBitRate          BitRate,
    iE-Extensions       ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs} }    OPTIONAL,
    ...
}

PC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDCPChangeIndication ::= CHOICE {
    from-S-NG-RAN-node              ENUMERATED {s-ng-ran-node-key-update-required, pdcp-data-recovery-required, ...},
    from-M-NG-RAN-node              ENUMERATED {pdcp-data-recovery-required, ...},
    choice-extension                ProtocolIE-Single-Container { {PDCPChangeIndication-ExtIEs} }
}

PDCPChangeIndication-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


PDCPDuplicationConfiguration ::= ENUMERATED {
    configured,
    de-configured,
    ...
}


PDCPSNLength ::= SEQUENCE {
    ulPDCPSNLength          ENUMERATED {v12bits, v18bits, ...},
    dlPDCPSNLength          ENUMERATED {v12bits, v18bits, ...},
    iE-Extension            ProtocolExtensionContainer { {PDCPSNLength-ExtIEs} }        OPTIONAL,
    ...
}

PDCPSNLength-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSetQoSParameters ::= SEQUENCE {
    ulPDUSetQoSInformation                  PDUSetQoSInformation    OPTIONAL,
    dlPDUSetQoSInformation                  PDUSetQoSInformation    OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { PDUSetQoSParameters-ExtIEs } }   OPTIONAL
}

PDUSetQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
    }


PDUSetQoSInformation    ::= SEQUENCE {
    pduSetDelayBudget                       ExtendedPacketDelayBudget       OPTIONAL,
    pduSetErrorRate                         PacketErrorRate                 OPTIONAL,
    pduSetIntegratedHandlingInformation     ENUMERATED {true, false, ...}   OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }  OPTIONAL
}

PDUSetQoSInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSetbasedHandlingIndicator ::= ENUMERATED {
    supported,
    ...
}



PDUSessionAggregateMaximumBitRate ::= SEQUENCE {
    downlink-session-AMBR               BitRate,
    uplink-session-AMBR                 BitRate,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PDUSession-List ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-ID


PDUSession-List-withCause ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSession-List-withCause-Item

PDUSession-List-withCause-Item ::= SEQUENCE {
    pduSessionId        PDUSession-ID,
    cause               Cause               OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {PDUSession-List-withCause-Item-ExtIEs} }  OPTIONAL,
    ...
}

PDUSession-List-withCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PDUSession-List-withDataForwardingFromTarget ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF
                                                            PDUSession-List-withDataForwardingFromTarget-Item

PDUSession-List-withDataForwardingFromTarget-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    dataforwardinginfoTarget            DataForwardingInfoFromTargetNGRANnode,
    iE-Extension        ProtocolExtensionContainer { {PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs} }   OPTIONAL,
    ...
}

PDUSession-List-withDataForwardingFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DRB-IDs-takenintouse        CRITICALITY reject  EXTENSION DRB-List  PRESENCE optional},
    ...
}


PDUSession-List-withDataForwardingRequest ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF
                                                            PDUSession-List-withDataForwardingRequest-Item

PDUSession-List-withDataForwardingRequest-Item ::= SEQUENCE {
    pduSessionId                            PDUSession-ID,
    dataforwardingInfofromSource            DataforwardingandOffloadingInfofromSource                   OPTIONAL,
    dRBtoBeReleasedList                     DRBToQoSFlowMapping-List                                    OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {PDUSession-List-withDataForwardingRequest-Item-ExtIEs} }  OPTIONAL,
    ...
}

PDUSession-List-withDataForwardingRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-Cause                CRITICALITY ignore EXTENSION Cause              PRESENCE optional},
...
}


PDUSessionsListToBeReleased-UPError ::= SEQUENCE (SIZE (1.. maxnoofPDUSessions)) OF PDUSessionsListToBeReleased-UPError-Item

PDUSessionsListToBeReleased-UPError-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    userPlaneErrorIndicator             UserPlaneErrorIndicator,
    iE-Extension        ProtocolExtensionContainer { {PDUSessionsListToBeReleased-UPError-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionsListToBeReleased-UPError-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session related message level IEs BEGIN
--
-- **************************************************************


-- **************************************************************
--
-- PDU Session Resources Admitted List
--
-- **************************************************************


PDUSessionResourcesAdmitted-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesAdmitted-Item

PDUSessionResourcesAdmitted-Item ::= SEQUENCE {
    pduSessionId                        PDUSession-ID,
    pduSessionResourceAdmittedInfo      PDUSessionResourceAdmittedInfo,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionResourcesAdmitted-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourcesAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PDUSessionResourceAdmittedInfo ::= SEQUENCE {
    dL-NG-U-TNL-Information-Unchanged       ENUMERATED {true, ...}                                                      OPTIONAL,
    qosFlowsAdmitted-List                   QoSFlowsAdmitted-List,
    qosFlowsNotAdmitted-List                QoSFlows-List-withCause                                                     OPTIONAL,
    dataForwardingInfoFromTarget            DataForwardingInfoFromTargetNGRANnode                                       OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionResourceAdmittedInfo-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionResourceAdmittedInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-SecondarydataForwardingInfoFromTarget-List  CRITICALITY ignore  EXTENSION SecondarydataForwardingInfoFromTarget-List    PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resources Not Admitted List
--
-- **************************************************************


PDUSessionResourcesNotAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofPDUSessions)) OF PDUSessionResourcesNotAdmitted-Item

PDUSessionResourcesNotAdmitted-Item ::= SEQUENCE {
    pduSessionId                PDUSession-ID,
    cause                       Cause               OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourcesNotAdmitted-Item-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resources To Be Setup List
--
-- **************************************************************


PDUSessionResourcesToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourcesToBeSetup-Item

PDUSessionResourcesToBeSetup-Item ::= SEQUENCE {
    pduSessionId                    PDUSession-ID,
    s-NSSAI                         S-NSSAI,
    pduSessionAMBR                  PDUSessionAggregateMaximumBitRate                                                   OPTIONAL,
    uL-NG-U-TNLatUPF                UPTransportLayerInformation,
    source-DL-NG-U-TNL-Information  UPTransportLayerInformation                                                         OPTIONAL,
    securityIndication              SecurityIndication                                                                  OPTIONAL,
    pduSessionType                  PDUSessionType,
    pduSessionNetworkInstance       PDUSessionNetworkInstance                                                           OPTIONAL,
    qosFlowsToBeSetup-List          QoSFlowsToBeSetup-List,
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource                                           OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourcesToBeSetup-Item-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourcesToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-UL-NG-U-TNLatUPF-List            CRITICALITY ignore  EXTENSION Additional-UL-NG-U-TNLatUPF-List          PRESENCE optional}|
    { ID id-PDUSessionCommonNetworkInstance             CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance               PRESENCE optional}|
    { ID id-Redundant-UL-NG-U-TNLatUPF                  CRITICALITY ignore  EXTENSION UPTransportLayerInformation                   PRESENCE optional}|
    { ID id-Additional-Redundant-UL-NG-U-TNLatUPF-List  CRITICALITY ignore  EXTENSION Additional-UL-NG-U-TNLatUPF-List              PRESENCE optional}|
    { ID id-RedundantCommonNetworkInstance              CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance               PRESENCE optional}|
    { ID id-RedundantPDUSessionInformation              CRITICALITY ignore  EXTENSION RedundantPDUSessionInformation                PRESENCE optional}|
    { ID id-MBS-SessionAssociatedInformation            CRITICALITY ignore  EXTENSION MBS-SessionAssociatedInformation              PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Setup Info - SN terminated
--
-- **************************************************************


PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {
    uL-NG-U-TNLatUPF                UPTransportLayerInformation,
    pduSessionType                  PDUSessionType,
    pduSessionNetworkInstance       PDUSessionNetworkInstance                                                                   OPTIONAL,
    qosFlowsToBeSetup-List          QoSFlowsToBeSetup-List-Setup-SNterminated,
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource                                                   OPTIONAL,
    securityIndication              SecurityIndication                                                                          OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SecurityResult                      CRITICALITY reject  EXTENSION SecurityResult                            PRESENCE optional}|
    { ID id-PDUSessionCommonNetworkInstance     CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance       PRESENCE optional}|
    { ID id-DefaultDRB-Allowed                  CRITICALITY ignore  EXTENSION DefaultDRB-Allowed                        PRESENCE optional}|
    { ID id-SplitSessionIndicator               CRITICALITY reject  EXTENSION SplitSessionIndicator                 PRESENCE optional}|
    { ID id-NonGBRResources-Offered             CRITICALITY ignore  EXTENSION NonGBRResources-Offered               PRESENCE optional}|
    { ID id-Redundant-UL-NG-U-TNLatUPF          CRITICALITY ignore  EXTENSION UPTransportLayerInformation           PRESENCE optional}|
    { ID id-RedundantCommonNetworkInstance      CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance       PRESENCE optional}|
    { ID id-RedundantPDUSessionInformation      CRITICALITY ignore  EXTENSION RedundantPDUSessionInformation        PRESENCE optional},
    ...
}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {
    qfi                             QoSFlowIdentifier,
    qosFlowLevelQoSParameters       QoSFlowLevelQoSParameters,
    offeredGBRQoSFlowInfo           GBRQoSFlowInfo                                                                                      OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }  OPTIONAL,
    ...
}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-TSCTrafficCharacteristics       CRITICALITY ignore  EXTENSION TSCTrafficCharacteristics     PRESENCE optional}|
    { ID id-RedundantQoSFlowIndicator       CRITICALITY ignore  EXTENSION RedundantQoSFlowIndicator PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingRequest       CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingRequest PRESENCE optional},
    ...
}

-- **************************************************************
--
-- PDU Session Resource Setup Response Info - SN terminated
--
-- **************************************************************


PDUSessionResourceSetupResponseInfo-SNterminated ::= SEQUENCE {
    dL-NG-U-TNLatNG-RAN             UPTransportLayerInformation,
    dRBsToBeSetup                   DRBsToBeSetupList-SetupResponse-SNterminated    OPTIONAL,
    dataforwardinginfoTarget        DataForwardingInfoFromTargetNGRANnode           OPTIONAL,
    qosFlowsNotAdmittedList         QoSFlows-List-withCause                         OPTIONAL,
    securityResult                  SecurityResult                                  OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceSetupResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DRB-IDs-takenintouse                    CRITICALITY reject  EXTENSION DRB-List                                              PRESENCE optional}|
    { ID id-Redundant-DL-NG-U-TNLatNG-RAN           CRITICALITY ignore  EXTENSION UPTransportLayerInformation                       PRESENCE optional}|
    { ID id-UsedRSNInformation                      CRITICALITY ignore  EXTENSION RedundantPDUSessionInformation                    PRESENCE optional}|
    { ID id-S-CPAC-dataforwardinginfofromSource     CRITICALITY ignore  EXTENSION DataforwardingandOffloadingInfofromSource PRESENCE optional}|
    { ID id-AdditionalDRBSetupInfoList              CRITICALITY ignore  EXTENSION AdditionalDRBSetupInfoList                        PRESENCE optional},
    ...
}

DRBsToBeSetupList-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-SetupResponse-SNterminated-Item

DRBsToBeSetupList-SetupResponse-SNterminated-Item ::= SEQUENCE {
    drb-ID                                                  DRB-ID,
    sN-UL-PDCP-UP-TNLInfo                                   UPTransportParameters,
    dRB-QoS                                                 QoSFlowLevelQoSParameters,
    pDCP-SNLength                                           PDCPSNLength                        OPTIONAL,
    rLC-Mode                                                RLCMode,
    uL-Configuration                                        ULConfiguration                     OPTIONAL,
    secondary-SN-UL-PDCP-UP-TNLInfo                         UPTransportParameters               OPTIONAL,
    duplicationActivation                                   DuplicationActivation               OPTIONAL,
    qoSFlowsMappedtoDRB-SetupResponse-SNterminated          QoSFlowsMappedtoDRB-SetupResponse-SNterminated,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs} }   OPTIONAL,
    ...
}

DRBsToBeSetupList-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus        CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingStatus  PRESENCE optional}| 
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF
                                                                        QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item ::= SEQUENCE {
    qoSFlowIdentifier               QoSFlowIdentifier,
    mCGRequestedGBRQoSFlowInfo      GBRQoSFlowInfo                                                      OPTIONAL,
    qosFlowMappingIndication        QoSFlowMappingIndication                                            OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

QoSFlowsMappedtoDRB-SetupResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CurrentQoSParaSetIndex              CRITICALITY ignore  EXTENSION QoSParaSetIndex               PRESENCE optional }|
    { ID id-SourceDLForwardingIPAddress         CRITICALITY ignore  EXTENSION TransportLayerAddress     PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Setup Info - MN terminated
--
-- **************************************************************


PDUSessionResourceSetupInfo-MNterminated ::= SEQUENCE {
    pduSessionType                  PDUSessionType,
    dRBsToBeSetup                   DRBsToBeSetupList-Setup-MNterminated,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-MNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceSetupInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DRBsToBeSetupList-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-Setup-MNterminated-Item

DRBsToBeSetupList-Setup-MNterminated-Item ::= SEQUENCE {
    drb-ID                                                  DRB-ID,
    mN-UL-PDCP-UP-TNLInfo                                   UPTransportParameters,
    rLC-Mode                                                RLCMode,
    uL-Configuration                                        ULConfiguration                     OPTIONAL,
    dRB-QoS                                                 QoSFlowLevelQoSParameters,
    pDCP-SNLength                                           PDCPSNLength                        OPTIONAL,
    secondary-MN-UL-PDCP-UP-TNLInfo                         UPTransportParameters               OPTIONAL,
    duplicationActivation                                   DuplicationActivation               OPTIONAL,
    qoSFlowsMappedtoDRB-Setup-MNterminated      QoSFlowsMappedtoDRB-Setup-MNterminated,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs} }   OPTIONAL,
    ...
}

DRBsToBeSetupList-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingRequest   CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingRequest PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}

QoSFlowsMappedtoDRB-Setup-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-Setup-MNterminated-Item

QoSFlowsMappedtoDRB-Setup-MNterminated-Item ::= SEQUENCE {
    qoSFlowIdentifier               QoSFlowIdentifier,
    qoSFlowLevelQoSParameters       QoSFlowLevelQoSParameters,
    qosFlowMappingIndication        QoSFlowMappingIndication        OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

QoSFlowsMappedtoDRB-Setup-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-TSCTrafficCharacteristics       CRITICALITY ignore  EXTENSION TSCTrafficCharacteristics     PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Setup Response Info - MN terminated
--
-- **************************************************************


PDUSessionResourceSetupResponseInfo-MNterminated ::= SEQUENCE {
    dRBsAdmittedList                DRBsAdmittedList-SetupResponse-MNterminated,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceSetupResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-DRBsNotAdmittedSetupModifyList   CRITICALITY ignore  EXTENSION DRB-List-withCause        PRESENCE optional},
    ...
}

DRBsAdmittedList-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-SetupResponse-MNterminated-Item

DRBsAdmittedList-SetupResponse-MNterminated-Item ::= SEQUENCE {
    drb-ID                                  DRB-ID,
    sN-DL-SCG-UP-TNLInfo                    UPTransportParameters,
    secondary-SN-DL-SCG-UP-TNLInfo          UPTransportParameters               OPTIONAL,
    lCID                                    LCID                                OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs} }    OPTIONAL,
    ...
}

DRBsAdmittedList-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated  CRITICALITY ignore  EXTENSION   QoSFlowsMappedtoDRB-SetupResponse-MNterminated  PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus    CRITICALITY ignore  EXTENSION   ECNMarkingorCongestionInformationReportingStatus    PRESENCE optional},
    ...
}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item ::= SEQUENCE {
    qoSFlowIdentifier               QoSFlowIdentifier,
    currentQoSParaSetIndex          QoSParaSetIndex,
    iE-Extensions                   ProtocolExtensionContainer { {QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

QoSFlowsMappedtoDRB-SetupResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Info - SN terminated
--
-- **************************************************************


PDUSessionResourceModificationInfo-SNterminated ::= SEQUENCE {
    uL-NG-U-TNLatUPF                UPTransportLayerInformation                     OPTIONAL,
    pduSessionNetworkInstance       PDUSessionNetworkInstance                       OPTIONAL,
    qosFlowsToBeSetup-List          QoSFlowsToBeSetup-List-Setup-SNterminated       OPTIONAL,
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource       OPTIONAL,
    qosFlowsToBeModified-List       QoSFlowsToBeSetup-List-Modified-SNterminated    OPTIONAL,
    qoSFlowsToBeReleased-List       QoSFlows-List-withCause                         OPTIONAL,
    drbsToBeModifiedList            DRBsToBeModified-List-Modified-SNterminated     OPTIONAL,
    dRBsToBeReleased                DRB-List-withCause                              OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-SNterminated-ExtIEs} }     OPTIONAL,
    ...
}

PDUSessionResourceModificationInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-PDUSessionCommonNetworkInstance     CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance       PRESENCE optional}|
    {ID id-DefaultDRB-Allowed                   CRITICALITY ignore  EXTENSION DefaultDRB-Allowed                        PRESENCE optional}|
    {ID id-NonGBRResources-Offered              CRITICALITY ignore  EXTENSION NonGBRResources-Offered               PRESENCE optional}|
    {ID id-Redundant-UL-NG-U-TNLatUPF           CRITICALITY ignore  EXTENSION UPTransportLayerInformation           PRESENCE optional}|
    {ID id-RedundantCommonNetworkInstance       CRITICALITY ignore  EXTENSION PDUSessionCommonNetworkInstance       PRESENCE optional}|
    {ID id-SecurityIndication                   CRITICALITY ignore  EXTENSION SecurityIndication                        PRESENCE optional},
    ...
}

QoSFlowsToBeSetup-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Modified-SNterminated-Item

QoSFlowsToBeSetup-List-Modified-SNterminated-Item ::= SEQUENCE {
    qfi                             QoSFlowIdentifier,
    qosFlowLevelQoSParameters       QoSFlowLevelQoSParameters                           OPTIONAL,
    offeredGBRQoSFlowInfo           GBRQoSFlowInfo                                      OPTIONAL,
    qosFlowMappingIndication        QoSFlowMappingIndication                            OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs} }   OPTIONAL,
    ...
}

QoSFlowsToBeSetup-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-TSCTrafficCharacteristics       CRITICALITY ignore  EXTENSION TSCTrafficCharacteristics     PRESENCE optional}|
    { ID id-RedundantQoSFlowIndicator       CRITICALITY ignore  EXTENSION RedundantQoSFlowIndicator PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingRequest       CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingRequest PRESENCE optional},
    ...
}

DRBsToBeModified-List-Modified-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-Modified-SNterminated-Item

DRBsToBeModified-List-Modified-SNterminated-Item ::= SEQUENCE {
    drb-ID                                  DRB-ID,
    mN-DL-SCG-UP-TNLInfo                    UPTransportParameters       OPTIONAL,
    secondary-MN-DL-SCG-UP-TNLInfo          UPTransportParameters       OPTIONAL,
    lCID                                    LCID                        OPTIONAL,
    rlc-status                              RLC-Status                  OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs} }    OPTIONAL,
    ...
}

DRBsToBeModified-List-Modified-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional},
    ...
}

-- **************************************************************
--
-- PDU Session Resource Modification Response Info - SN terminated
--
-- **************************************************************


PDUSessionResourceModificationResponseInfo-SNterminated ::= SEQUENCE {
    dL-NG-U-TNLatNG-RAN             UPTransportLayerInformation                             OPTIONAL,
    dRBsToBeSetup                   DRBsToBeSetupList-SetupResponse-SNterminated            OPTIONAL,
    dataforwardinginfoTarget        DataForwardingInfoFromTargetNGRANnode                   OPTIONAL,
    dRBsToBeModified                DRBsToBeModifiedList-ModificationResponse-SNterminated  OPTIONAL,
    dRBsToBeReleased                DRB-List-withCause                                      OPTIONAL,
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource               OPTIONAL,
    qosFlowsNotAdmittedTBAdded      QoSFlows-List-withCause                                 OPTIONAL,
    qosFlowsReleased                QoSFlows-List-withCause                                 OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs} }     OPTIONAL,
    ...
}

PDUSessionResourceModificationResponseInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DRB-IDs-takenintouse            CRITICALITY reject  EXTENSION DRB-List                              PRESENCE optional}|
    { ID id-Redundant-DL-NG-U-TNLatNG-RAN   CRITICALITY ignore  EXTENSION UPTransportLayerInformation       PRESENCE optional}|
    { ID id-SecurityResult                  CRITICALITY ignore  EXTENSION SecurityResult                        PRESENCE optional}|
    { ID id-AdditionalDRBSetupInfoList      CRITICALITY ignore  EXTENSION AdditionalDRBSetupInfoList            PRESENCE optional},
    ...
}

DRBsToBeModifiedList-ModificationResponse-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF
                                                                                DRBsToBeModifiedList-ModificationResponse-SNterminated-Item

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item ::= SEQUENCE {
    drb-ID                                                  DRB-ID,
    sN-UL-PDCP-UP-TNLInfo                                   UPTransportParameters                                   OPTIONAL,
    dRB-QoS                                                 QoSFlowLevelQoSParameters                               OPTIONAL,
    qoSFlowsMappedtoDRB-SetupResponse-SNterminated          QoSFlowsMappedtoDRB-SetupResponse-SNterminated      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

DRBsToBeModifiedList-ModificationResponse-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-secondary-SN-UL-PDCP-UP-TNLInfo             CRITICALITY ignore  EXTENSION UPTransportParameters                     PRESENCE optional}|
    { ID id-pdcpDuplicationConfiguration                CRITICALITY ignore  EXTENSION PDCPDuplicationConfiguration              PRESENCE optional}|
    { ID id-duplicationActivation                       CRITICALITY ignore  EXTENSION DuplicationActivation                     PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus        CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingStatus  PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Info - MN terminated
--
-- **************************************************************


PDUSessionResourceModificationInfo-MNterminated ::= SEQUENCE {
    pduSessionType                  PDUSessionType,
    dRBsToBeSetup                   DRBsToBeSetupList-Setup-MNterminated                        OPTIONAL,
    dRBsToBeModified                DRBsToBeModifiedList-Modification-MNterminated              OPTIONAL,
    dRBsToBeReleased                DRB-List-withCause                                          OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModificationInfo-MNterminated-ExtIEs} }     OPTIONAL,
    ...
}

PDUSessionResourceModificationInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DRBsToBeModifiedList-Modification-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF
                                                                                    DRBsToBeModifiedList-Modification-MNterminated-Item

DRBsToBeModifiedList-Modification-MNterminated-Item ::= SEQUENCE {
    drb-ID                                                  DRB-ID,
    mN-UL-PDCP-UP-TNLInfo                                   UPTransportParameters                       OPTIONAL,
    dRB-QoS                                                 QoSFlowLevelQoSParameters                   OPTIONAL,
    secondary-MN-UL-PDCP-UP-TNLInfo                         UPTransportParameters                       OPTIONAL,
    uL-Configuration                                        ULConfiguration                             OPTIONAL,
    pdcpDuplicationConfiguration                            PDCPDuplicationConfiguration                OPTIONAL,
    duplicationActivation                                   DuplicationActivation                       OPTIONAL,
    qoSFlowsMappedtoDRB-Setup-MNterminated                  QoSFlowsMappedtoDRB-Setup-MNterminated  OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

DRBsToBeModifiedList-Modification-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingRequest   CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingRequest PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Response Info - MN terminated
--
-- **************************************************************


PDUSessionResourceModificationResponseInfo-MNterminated ::= SEQUENCE {
    dRBsAdmittedList                        DRBsAdmittedList-ModificationResponse-MNterminated,
    dRBsReleasedList                        DRB-List                                                                        OPTIONAL,
    dRBsNotAdmittedSetupModifyList          DRB-List-withCause                                                              OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs} }     OPTIONAL,
    ...
}

PDUSessionResourceModificationResponseInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DRBsAdmittedList-ModificationResponse-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsAdmittedList-ModificationResponse-MNterminated-Item

DRBsAdmittedList-ModificationResponse-MNterminated-Item ::= SEQUENCE {
    drb-ID                                  DRB-ID,
    sN-DL-SCG-UP-TNLInfo                    UPTransportParameters               OPTIONAL,
    secondary-SN-DL-SCG-UP-TNLInfo          UPTransportParameters               OPTIONAL,
    lCID                                    LCID                                OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

DRBsAdmittedList-ModificationResponse-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated  CRITICALITY ignore  EXTENSION   QoSFlowsMappedtoDRB-SetupResponse-MNterminated  PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus    CRITICALITY ignore  EXTENSION   ECNMarkingorCongestionInformationReportingStatus    PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Change Required Info - SN terminated
--
-- **************************************************************


PDUSessionResourceChangeRequiredInfo-SNterminated ::= SEQUENCE {
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource                                           OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceChangeRequiredInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resource Change Confirm Info - SN terminated
--
-- **************************************************************


PDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE {
    dataforwardinginfoTarget        DataForwardingInfoFromTargetNGRANnode               OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceChangeConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DRB-IDs-takenintouse    CRITICALITY reject  EXTENSION DRB-List  PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Change Required Info - MN terminated
--
-- **************************************************************


PDUSessionResourceChangeRequiredInfo-MNterminated ::= SEQUENCE {
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceChangeRequiredInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resource Change Confirm Info - MN terminated
--
-- **************************************************************


PDUSessionResourceChangeConfirmInfo-MNterminated ::= SEQUENCE {
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs} }    OPTIONAL,
    ...
}

PDUSessionResourceChangeConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Required Info - SN terminated
--
-- **************************************************************


PDUSessionResourceModRqdInfo-SNterminated ::= SEQUENCE {
    dL-NG-U-TNLatNG-RAN             UPTransportLayerInformation                     OPTIONAL,
    qoSFlowsToBeReleased-List       QoSFlows-List-withCause                         OPTIONAL,
    dataforwardinginfofromSource    DataforwardingandOffloadingInfofromSource       OPTIONAL,
    drbsToBeSetupList               DRBsToBeSetup-List-ModRqd-SNterminated          OPTIONAL,
    drbsToBeModifiedList            DRBsToBeModified-List-ModRqd-SNterminated       OPTIONAL,
    dRBsToBeReleased                DRB-List-withCause                              OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-SNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceModRqdInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-AdditionalDRBSetupInfoList      CRITICALITY ignore  EXTENSION AdditionalDRBSetupInfoList            PRESENCE optional},
    ...
}

DRBsToBeSetup-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetup-List-ModRqd-SNterminated-Item

DRBsToBeSetup-List-ModRqd-SNterminated-Item ::= SEQUENCE {
    drb-ID                                          DRB-ID,
    pDCP-SNLength                                   PDCPSNLength                                        OPTIONAL,
    sn-UL-PDCP-UPTNLinfo                            UPTransportParameters,
    dRB-QoS                                         QoSFlowLevelQoSParameters,
    secondary-SN-UL-PDCP-UP-TNLInfo                 UPTransportParameters                               OPTIONAL,
    duplicationActivation                           DuplicationActivation                               OPTIONAL,
    uL-Configuration                                ULConfiguration                                     OPTIONAL,
    qoSFlowsMappedtoDRB-ModRqd-SNterminated         QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated,
    rLC-Mode                                        RLCMode,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

DRBsToBeSetup-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus        CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingStatus  PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF
                                                                        QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {
    qoSFlowIdentifier               QoSFlowIdentifier,
    mCGRequestedGBRQoSFlowInfo      GBRQoSFlowInfo                                                      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} }   OPTIONAL,
    ...
}

QoSFlowsSetupMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-QosFlowMappingIndication CRITICALITY ignore  EXTENSION QoSFlowMappingIndication      PRESENCE optional},
    ...
}

DRBsToBeModified-List-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-SNterminated-Item

DRBsToBeModified-List-ModRqd-SNterminated-Item ::= SEQUENCE {
    drb-ID                                          DRB-ID,
    sN-UL-PDCP-UP-TNLInfo                           UPTransportParameters                                   OPTIONAL,
    dRB-QoS                                         QoSFlowLevelQoSParameters                               OPTIONAL,
    secondary-SN-UL-PDCP-UP-TNLInfo                 UPTransportParameters                                   OPTIONAL,
    uL-Configuration                                ULConfiguration                                         OPTIONAL,
    pdcpDuplicationConfiguration                    PDCPDuplicationConfiguration                            OPTIONAL,
    duplicationActivation                           DuplicationActivation                                   OPTIONAL,
    qoSFlowsMappedtoDRB-ModRqd-SNterminated     QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated             OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs} }  OPTIONAL,
    ...
}

DRBsToBeModified-List-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-RLCDuplicationInformation                   CRITICALITY ignore  EXTENSION RLCDuplicationInformation                 PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus        CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingStatus  PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardUL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardUL                          PRESENCE optional}|
    { ID id-PSIbasedSDUdiscardDL                        CRITICALITY ignore  EXTENSION PSIbasedSDUdiscardDL                          PRESENCE optional},
    ...
}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF
                                                                        QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item ::= SEQUENCE {
    qoSFlowIdentifier                   QoSFlowIdentifier,
    mCGRequestedGBRQoSFlowInfo          GBRQoSFlowInfo                                                      OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs} }    OPTIONAL,
    ...
}

QoSFlowsModifiedMappedtoDRB-ModRqd-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-QosFlowMappingIndication CRITICALITY ignore  EXTENSION QoSFlowMappingIndication      PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Confirm Info - SN terminated
--
-- **************************************************************


PDUSessionResourceModConfirmInfo-SNterminated ::= SEQUENCE {
    uL-NG-U-TNLatUPF                        UPTransportLayerInformation                             OPTIONAL,
    dRBsAdmittedList                        DRBsAdmittedList-ModConfirm-SNterminated,
    dRBsNotAdmittedSetupModifyList          DRB-List-withCause                                      OPTIONAL,
    dataforwardinginfoTarget                DataForwardingInfoFromTargetNGRANnode                   OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceModConfirmInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DRB-IDs-takenintouse        CRITICALITY reject  EXTENSION DRB-List  PRESENCE optional},
    ...
}

DRBsAdmittedList-ModConfirm-SNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF
                                                                                DRBsAdmittedList-ModConfirm-SNterminated-Item

DRBsAdmittedList-ModConfirm-SNterminated-Item ::= SEQUENCE {
    drb-ID                                          DRB-ID,
    mN-DL-CG-UP-TNLInfo                             UPTransportParameters                           OPTIONAL,
    secondary-MN-DL-CG-UP-TNLInfo                   UPTransportParameters                           OPTIONAL,
    lCID                                            LCID                                            OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs} }   OPTIONAL,
    ...
}

DRBsAdmittedList-ModConfirm-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Required Info - MN terminated
--
-- **************************************************************


PDUSessionResourceModRqdInfo-MNterminated ::= SEQUENCE {
    dRBsToBeModified                DRBsToBeModified-List-ModRqd-MNterminated               OPTIONAL,
    dRBsToBeReleased                DRB-List-withCause                                      OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModRqdInfo-MNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceModRqdInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


DRBsToBeModified-List-ModRqd-MNterminated ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeModified-List-ModRqd-MNterminated-Item

DRBsToBeModified-List-ModRqd-MNterminated-Item ::= SEQUENCE {
    drb-ID                              DRB-ID,
    sN-DL-SCG-UP-TNLInfo                UPTransportLayerInformation,
    secondary-SN-DL-SCG-UP-TNLInfo      UPTransportLayerInformation     OPTIONAL,
    lCID                                LCID                            OPTIONAL,
    rlc-status                          RLC-Status                      OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs} } OPTIONAL,
    ...
}

DRBsToBeModified-List-ModRqd-MNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-Additional-PDCP-Duplication-TNL-List        CRITICALITY ignore  EXTENSION Additional-PDCP-Duplication-TNL-List  PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingStatus    CRITICALITY ignore  EXTENSION   ECNMarkingorCongestionInformationReportingStatus    PRESENCE optional},
    ...
}


-- **************************************************************
--
-- PDU Session Resource Modification Confirm Info - MN terminated
--
-- **************************************************************


PDUSessionResourceModConfirmInfo-MNterminated ::= SEQUENCE {
    iE-Extensions                   ProtocolExtensionContainer { {PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs} }   OPTIONAL,
    ...
}

PDUSessionResourceModConfirmInfo-MNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- **************************************************************
--
-- PDU Session Resource Setup Complete Info - SN terminated
--
-- **************************************************************

PDUSessionResourceBearerSetupCompleteInfo-SNterminated ::= SEQUENCE {
    dRBsToBeSetupList           SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item,
    iE-Extensions               ProtocolExtensionContainer { {PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs} }  OPTIONAL,
    ...
}

PDUSessionResourceBearerSetupCompleteInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item ::= SEQUENCE {
    dRB-ID                      DRB-ID,
    mN-Xn-U-TNLInfoatM          UPTransportLayerInformation,
    iE-Extensions               ProtocolExtensionContainer { {DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs} }     OPTIONAL,
    ...
}

DRBsToBeSetupList-BearerSetupComplete-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-Secondary-MN-Xn-U-TNLInfoatM CRITICALITY ignore  EXTENSION UPTransportLayerInformation       PRESENCE optional},
    ...
}

-- **************************************************************
--
-- PDU Session related message level IEs END
--
-- **************************************************************

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {
    pDUSessionID                                PDUSession-ID,
    secondaryRATUsageInformation                        SecondaryRATUsageInformation,
    iE-Extensions       ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} } OPTIONAL,
    ...
}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionUsageReport ::= SEQUENCE {
    rATType                             ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},
    pDUSessionTimedReportList           VolumeTimedReportList,
    iE-Extensions                       ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,
...
}

PDUSessionUsageReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PDUSessionType ::= ENUMERATED {ipv4, ipv6, ipv4v6, ethernet, unstructured, ...}

PDUSession-ID   ::= INTEGER (0..255)

PDUSessionNetworkInstance   ::= INTEGER (1..256, ...)

PDUSessionCommonNetworkInstance ::= OCTET STRING

PDUSession-PairID   ::= INTEGER (0..255, ...)



Periodical ::= SEQUENCE {
    iE-Extensions       ProtocolExtensionContainer { { Periodical-ExtIEs} } OPTIONAL,
    ...
}



Periodical-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Permutation ::= ENUMERATED {dfu, ufd, ...}


PLMN-Identity ::= OCTET STRING (SIZE(3))

PLMNAreaBasedQMC ::= SEQUENCE {
    plmnListforQMC      PLMNListforQMC,
    iE-Extensions       ProtocolExtensionContainer { {PLMNAreaBasedQMC-ExtIEs} } OPTIONAL,
    ...
}

PLMNAreaBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PLMNListforQMC ::= SEQUENCE (SIZE(1..maxnoofPLMNforQMC)) OF PLMN-Identity

PCIListForMDT ::= SEQUENCE (SIZE(1.. maxnoofNeighPCIforMDT)) OF NRPCI


PNI-NPN-Restricted-Information ::= ENUMERATED { restriced, not-restricted, ...}

PortNumber ::= BIT STRING (SIZE (16))

PosPartialUEContextInfo ::= SEQUENCE {
    requestedSRSTransmissionCharacteristics     RequestedSRSTransmissionCharacteristics OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { PosPartialUEContextInfo-ExtIEs} }    OPTIONAL,
    ...
}

PosPartialUEContextInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PredictedUETrajectory-Item ::= SEQUENCE{
    predictedtrajectoryCellInfo     PredictedTrajectoryCellInfo,
    iE-Extensions                   ProtocolExtensionContainer { { PredictedUETrajectory-Item-ExtIEs} } OPTIONAL,
    ...
}

PredictedUETrajectory-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


PredictedTrajectoryCellInfo::= CHOICE {
    nG-RAN-Cell-Predicted           PredictedTrajectoryNGRANCellInfo,
    choice-extension                ProtocolIE-Single-Container { { PredictedTrajectoryCellInfo-ExtIEs} }
}

PredictedTrajectoryCellInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

PredictedTrajectoryNGRANCellInfo ::= SEQUENCE {
    globalNG-RANCell-ID             GlobalNG-RANCell-ID,
    predictedTimeUEStaysInCell      INTEGER (0..4095)   OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { PredictedTrajectoryNGRANCellInfo-ExtIEs} }   OPTIONAL,
    ...
}

PredictedTrajectoryNGRANCellInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



PriorityLevelQoS ::= INTEGER (1..127, ...)


ProtectedE-UTRAResourceIndication ::= SEQUENCE {
    activationSFN                   ActivationSFN,
    protectedResourceList           ProtectedE-UTRAResourceList,
    mbsfnControlRegionLength        MBSFNControlRegionLength                    OPTIONAL,
    pDCCHRegionLength               INTEGER (1..3),
    iE-Extensions                   ProtocolExtensionContainer { {ProtectedE-UTRAResourceIndication-ExtIEs} }   OPTIONAL,
    ...
}

ProtectedE-UTRAResourceIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ProtectedE-UTRAResourceList ::= SEQUENCE (SIZE (1.. maxnoofProtectedResourcePatterns)) OF ProtectedE-UTRAResource-Item

ProtectedE-UTRAResource-Item ::= SEQUENCE {
    resourceType                            ENUMERATED {downlinknonCRS, cRS, uplink, ...},
    intra-PRBProtectedResourceFootprint     BIT STRING (SIZE(84, ...)),
    protectedFootprintFrequencyPattern      BIT STRING (SIZE(6..110, ...)),
    protectedFootprintTimePattern           ProtectedE-UTRAFootprintTimePattern,
    iE-Extensions                   ProtocolExtensionContainer { {ProtectedE-UTRAResource-Item-ExtIEs} }    OPTIONAL,
    ...
}

ProtectedE-UTRAResource-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ProtectedE-UTRAFootprintTimePattern ::= SEQUENCE {
    protectedFootprintTimeperiodicity           INTEGER (1..320, ...),
    protectedFootrpintStartTime                 INTEGER (1..20, ...),
    iE-Extensions                   ProtocolExtensionContainer { {ProtectedE-UTRAFootprintTimePattern-ExtIEs} }     OPTIONAL,
    ...
}

ProtectedE-UTRAFootprintTimePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PrivacyIndicator ::= ENUMERATED {
    immediate-MDT,
    logged-MDT,
    ...
}


PSCellChangeHistory ::= ENUMERATED {reporting-full-history, ...}

PSCellHistoryInformationRetrieve ::= ENUMERATED {query, ...}

PSCellListContainer ::= OCTET STRING

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

PSIbasedSDUdiscardDL ::= ENUMERATED {configured, not-configured, ...}

PNI-NPN-AreaScopeofMDT ::= SEQUENCE {
    cAGListforMDT       CAGListforMDT,
    iE-Extensions       ProtocolExtensionContainer { {PNI-NPN-AreaScopeofMDT-ExtIEs} } OPTIONAL,
    ...
}

PNI-NPN-AreaScopeofMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PNI-NPNBasedMDT::= SEQUENCE {
    cAGListforMDT       CAGListforMDT,
    iE-Extensions       ProtocolExtensionContainer { {PNI-NPNBasedMDT-ExtIEs} } OPTIONAL,
    ...
}

PNI-NPNBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

PreambleIndexList ::= SEQUENCE (SIZE (1.. maxnoofPreambleIndexes)) OF PreambleIndex

PreambleIndex ::= INTEGER (0..63)

Predicted-CoverageModificationCause ::= ENUMERATED {
    coverage,
    cell-edge-capacity,
    cancel,
    ...
}


PredictedSliceAvailableCapacityGroup    ::= SEQUENCE {
    predictedSliceAvailableCapacity SliceAvailableCapacity,
    cellCapacityClassValueDownlink      CellCapacityClassValue          OPTIONAL,
    cellCapacityClassValueUplink        CellCapacityClassValue          OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { PredictedSliceAvailableCapacityGroup-ExtIEs} }   OPTIONAL,
    ...
}

PredictedSliceAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- Q


QMCConfigInfo ::= SEQUENCE {
    uEAppLayerMeasInfoList          UEAppLayerMeasInfoList,
    iE-Extensions                   ProtocolExtensionContainer { {QMCConfigInfo-ExtIEs} }   OPTIONAL,
    ...
}

QMCConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UEAppLayerMeasInfoList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfo-Item

UEAppLayerMeasInfo-Item ::= SEQUENCE {
    uEAppLayerMeasConfigInfo    UEAppLayerMeasConfigInfo,
    iE-Extensions               ProtocolExtensionContainer { { UEAppLayerMeasInfo-Item-ExtIEs} }    OPTIONAL,
    ...
}

UEAppLayerMeasInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QMCCoordinationRequest ::= SEQUENCE {
    mN-to-SN-QMCCoordRequestList    MN-to-SN-QMCCoordRequestList    OPTIONAL,
    sN-to-MN-QMCCoordRequestList    SN-to-MN-QMCCoordRequestList    OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {QMCCoordinationRequest-ExtIEs} }  OPTIONAL,
    ...
}

QMCCoordinationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MN-to-SN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordRequestList-Item


MN-to-SN-QMCCoordRequestList-Item ::= SEQUENCE {
    qOEReference                    QOEReference,
    qOEMeasConfigAppLayerID         QOEMeasConfAppLayerID                       OPTIONAL,
    measCollectionEntityIPAddress   MeasCollectionEntityIPAddress               OPTIONAL,
    qoEReportingPathRequest         ENUMERATED{srb4, srb5, ...}                 OPTIONAL,
    rVQoEReportingPathRequest       ENUMERATED{srb4, srb5, ...}                 OPTIONAL,
    furtherRVQoEInterestInquiry     ENUMERATED{true, ...}                       OPTIONAL,
    furtherRVQoEReportingPathInquiry        ENUMERATED{true, ...}               OPTIONAL,
    currentRVQoEConfig              RVQoEConfig                                 OPTIONAL,
    availableRVQoEMetrics           AvailableRVQoEMetrics                       OPTIONAL,
    configReleaseIndication         ENUMERATED{rvqoe,qoe-and-rvqoe, ...}        OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { MN-to-SN-QMCCoordRequestList-Item-ExtIEs} }  OPTIONAL,
    ...
}

MN-to-SN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SN-to-MN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordRequestList-Item


SN-to-MN-QMCCoordRequestList-Item ::= SEQUENCE {
    qOEReference                    QOEReference,
    measCollectionEntityIPAddress   MeasCollectionEntityIPAddress               OPTIONAL,
    qoEReportingPathRequest         ENUMERATED{srb4, srb5, ...}                 OPTIONAL,
    rVQoEReportingPathRequest       ENUMERATED{srb4, srb5, ...}                 OPTIONAL,
    furtherRVQoEInterestInquiry     ENUMERATED{true, ...}                       OPTIONAL,
    furtherRVQoEReportingPathInquiry        ENUMERATED{true, ...}               OPTIONAL,
    currentRVQoEConfig              RVQoEConfig                                 OPTIONAL,
    availableRVQoEMetrics       AvailableRVQoEMetrics                           OPTIONAL,
    configReleaseIndication         ENUMERATED{rvqoe,qoe-and-rvqoe, ...}        OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { SN-to-MN-QMCCoordRequestList-Item-ExtIEs} }  OPTIONAL,
    ...
}

SN-to-MN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


QMCCoordinationResponse ::= SEQUENCE {
    mN-to-SN-QMCCoordResponseList   MN-to-SN-QMCCoordResponseList   OPTIONAL,
    sN-to-MN-QMCCoordResponseList   SN-to-MN-QMCCoordResponseList   OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {QMCCoordinationResponse-ExtIEs} } OPTIONAL,
    ...
}

QMCCoordinationResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


MN-to-SN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordResponseList-Item


MN-to-SN-QMCCoordResponseList-Item ::= SEQUENCE {
    qOEReference                    QOEReference,
    qOEMeasConfigAppLayerID         QOEMeasConfAppLayerID                       OPTIONAL,
    qoEConfigSendingPath            ENUMERATED{mn, sn, ...}                     OPTIONAL,
    qoEReportingPathResponse        ENUMERATED{accepted, rejected, ...}         OPTIONAL,
    rVQoEReportingPathResponse      ENUMERATED{accepted, rejected, ...}             OPTIONAL,
    furtherRVQoEInterestResponse    ENUMERATED{interested, not-interested, ...} OPTIONAL,
    furtherRVQoEReportingPathResponse       ENUMERATED{srb4, srb5, ...}         OPTIONAL,
    preferredRVQoEConfig            RVQoEConfig                                 OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { MN-to-SN-QMCCoordResponseList-Item-ExtIEs} } OPTIONAL,
    ...
}

MN-to-SN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SN-to-MN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordResponseList-Item


SN-to-MN-QMCCoordResponseList-Item ::= SEQUENCE {
    qOEReference                    QOEReference,
    qoEReportingPathResponse        ENUMERATED{accepted, rejected, ...}                                     OPTIONAL,
    rVQoEReportingPathResponse      ENUMERATED{accepted, rejected, ...}                                         OPTIONAL,
    furtherRVQoEInterestResponse    ENUMERATED{interested, not-interested, ...}                             OPTIONAL,
    furtherRVQoEReportingPathResponse       ENUMERATED{srb4, srb5, ...}                                     OPTIONAL,
    preferredRVQoEConfig            RVQoEConfig                                                             OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { SN-to-MN-QMCCoordResponseList-Item-ExtIEs} } OPTIONAL,
    ...
}

SN-to-MN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


QoERVQoEReportingPaths ::= SEQUENCE {
    qoEReportingPath                    ENUMERATED{srb4, srb5, ...} OPTIONAL,
    rVQoEReportingPath                  ENUMERATED{srb4, srb5, ...} OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {QoERVQoEReportingPaths-ExtIEs} } OPTIONAL,
    ...
}

QoERVQoEReportingPaths-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RVQoEConfig ::= SEQUENCE {
    availableRANVisibleQoEMetrics       AvailableRVQoEMetrics           OPTIONAL,
    reportingPeriodicity                RVQoEReportingPeriodicity       OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {RVQoEConfig-ExtIEs} } OPTIONAL,
    ...
}

RVQoEConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RVQoEReportingPeriodicity ::= ENUMERATED {
    ms120,
    ms240,
    ms480,
    ms640,
    ms1024,
    ...
}

QOEMeasConfAppLayerID ::= INTEGER (0..15, ...)

QOEMeasStatus ::= ENUMERATED {ongoing, ...}

QOEReference ::= OCTET STRING (SIZE (6))

QoSCharacteristics ::= CHOICE {
    non-dynamic                     NonDynamic5QIDescriptor,
    dynamic                         Dynamic5QIDescriptor,
    choice-extension                ProtocolIE-Single-Container { {QoSCharacteristics-ExtIEs} }
}

QoSCharacteristics-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


QoSFlowIdentifier   ::= INTEGER (0..63, ...)


QoSFlowLevelQoSParameters ::= SEQUENCE {
    qos-characteristics         QoSCharacteristics,
    allocationAndRetentionPrio  AllocationandRetentionPriority,
    gBRQoSFlowInfo              GBRQoSFlowInfo                                                      OPTIONAL,
    reflectiveQoS               ReflectiveQoSAttribute                                              OPTIONAL,
    additionalQoSflowInfo       ENUMERATED {more-likely, ...}                                       OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {QoSFlowLevelQoSParameters-ExtIEs} } OPTIONAL,
    ...
}

QoSFlowLevelQoSParameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-QoSMonitoringRequest                 CRITICALITY ignore  EXTENSION QosMonitoringRequest                      PRESENCE optional}|
    {ID id-QosMonitoringReportingFrequency      CRITICALITY ignore  EXTENSION QosMonitoringReportingFrequency           PRESENCE optional}|
    {ID id-QoSMonitoringDisabled                CRITICALITY ignore  EXTENSION QoSMonitoringDisabled                     PRESENCE optional}|
    {ID id-PDUSetQoSParameters                  CRITICALITY ignore  EXTENSION PDUSetQoSParameters                           PRESENCE optional}|
    {ID id-DLPDUSetInformationMarkingSupportIndication      CRITICALITY ignore  EXTENSION DLPDUSetInformationMarkingSupportIndication           PRESENCE optional}|
    {ID id-MMSID                                CRITICALITY ignore  EXTENSION   MMSID                                       PRESENCE optional}|
    {ID id-Indication-of-bitrate-adaptation     CRITICALITY ignore  EXTENSION   Indication-of-bitrate-adaptation        PRESENCE optional},
    ...
}


QoSFlowMappingIndication ::= ENUMERATED {
    ul,
    dl,
    ...
}


QoSFlowNotificationControlIndicationInfo ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowNotify-Item

QoSFlowNotify-Item ::= SEQUENCE {
    qosFlowIdentifier           QoSFlowIdentifier,
    notificationInformation     ENUMERATED {fulfilled, not-fulfilled, ... , not-fulfilled-dl, not-fulfilled-ul },
    iE-Extensions               ProtocolExtensionContainer { {QoSFlowNotificationControlIndicationInfo-ExtIEs} } OPTIONAL,
    ...
}

QoSFlowNotificationControlIndicationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{   ID id-CurrentQoSParaSetIndex    CRITICALITY ignore  EXTENSION QoSParaSetNotifyIndex     PRESENCE optional },
    ...
}


QoSFlows-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlow-Item

QoSFlow-Item ::= SEQUENCE {
    qfi                             QoSFlowIdentifier,
    qosFlowMappingIndication        QoSFlowMappingIndication                    OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {QoSFlow-Item-ExtIEs} }    OPTIONAL,
    ...
}

QoSFlow-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


QoSFlows-List-withCause ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowwithCause-Item

QoSFlowwithCause-Item ::= SEQUENCE {
    qfi                 QoSFlowIdentifier,
    cause               Cause                   OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {QoSFlowwithCause-Item-ExtIEs} }   OPTIONAL,
    ...
}

QoSFlowwithCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoS-Mapping-Information ::= SEQUENCE {
    dscp                            BIT STRING (SIZE(6))            OPTIONAL,
    flow-label                      BIT STRING (SIZE(20))           OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {QoS-Mapping-Information-ExtIEs} } OPTIONAL,
    ...
}

QoS-Mapping-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QoSParaSetIndex ::= INTEGER (1..8,...)
QoSParaSetNotifyIndex ::= INTEGER (0..8,...)


QoSFlowsAdmitted-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsAdmitted-Item

QoSFlowsAdmitted-Item ::= SEQUENCE {
    qfi                             QoSFlowIdentifier,
    iE-Extension        ProtocolExtensionContainer { {QoSFlowsAdmitted-Item-ExtIEs} }   OPTIONAL,
    ...
}

QoSFlowsAdmitted-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
{ ID id-CurrentQoSParaSetIndex  CRITICALITY ignore  EXTENSION QoSParaSetIndex   PRESENCE optional   },
    ...
}


QoSFlowsToBeSetup-List ::= SEQUENCE (SIZE (1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-Item

QoSFlowsToBeSetup-Item ::= SEQUENCE {
    qfi                             QoSFlowIdentifier,
    qosFlowLevelQoSParameters       QoSFlowLevelQoSParameters,
    e-RAB-ID                        E-RAB-ID                                            OPTIONAL,
    iE-Extension        ProtocolExtensionContainer { {QoSFlowsToBeSetup-Item-ExtIEs} }  OPTIONAL,
    ...
}

QoSFlowsToBeSetup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-TSCTrafficCharacteristics                               CRITICALITY ignore  EXTENSION TSCTrafficCharacteristics     PRESENCE optional}|
    { ID id-RedundantQoSFlowIndicator                               CRITICALITY ignore  EXTENSION RedundantQoSFlowIndicator PRESENCE optional}|
    { ID id-ECNMarkingorCongestionInformationReportingRequest       CRITICALITY ignore  EXTENSION ECNMarkingorCongestionInformationReportingRequest PRESENCE optional},
    ...
}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {
    qosFlowIdentifier                   QoSFlowIdentifier,
    rATType                             ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},
    qoSFlowsTimedReportList             VolumeTimedReportList,
    iE-Extensions                       ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,
...
}

QoSFlowsUsageReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}
QoSMonitoringDisabled ::= ENUMERATED {true, ...}
QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

-- R

RACH-Config-Common  ::= OCTET STRING

RACH-Config-Common-IAB  ::= OCTET STRING

RAReport    ::= SEQUENCE (SIZE(1.. maxnoofRAReports)) OF RAReportList-Item
RAReportList-Item   ::= SEQUENCE {
    rAReport                RAReportContainer,
    iE-Extensions                       ProtocolExtensionContainer { { RAReportList-Item-ExtIEs} }  OPTIONAL,
    ...
}

RAReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-UEAssistantIdentifier   CRITICALITY ignore  EXTENSION NG-RANnodeUEXnAPID    PRESENCE optional}|
    { ID id-PSCellListContainer     CRITICALITY ignore  EXTENSION PSCellListContainer   PRESENCE optional},
    ...
}

RAReportContainer   ::= OCTET STRING


RadioResourceStatus ::= CHOICE {
    ng-eNB-RadioResourceStatus  NG-eNB-RadioResourceStatus,
    gNB-RadioResourceStatus     GNB-RadioResourceStatus,
    choice-extension            ProtocolIE-Single-Container { { RadioResourceStatus-ExtIEs} }

}

RadioResourceStatus-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


RANAC ::= INTEGER (0..255)


RANAreaID ::= SEQUENCE {
    tAC                 TAC,
    rANAC               RANAC                                               OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {RANAreaID-ExtIEs} }   OPTIONAL,
    ...
}

RANAreaID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


RANAreaID-List ::= SEQUENCE (SIZE(1..maxnoofRANAreasinRNA)) OF RANAreaID

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RANPagingArea ::= SEQUENCE {
    pLMN-Identity           PLMN-Identity,
    rANPagingAreaChoice     RANPagingAreaChoice,
    iE-Extensions           ProtocolExtensionContainer { {RANPagingArea-ExtIEs} } OPTIONAL,
    ...
}

RANPagingArea-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RANPagingAreaChoice ::= CHOICE {
    cell-List           NG-RAN-Cell-Identity-ListinRANPagingArea,
    rANAreaID-List      RANAreaID-List,
    choice-extension    ProtocolIE-Single-Container { {RANPagingAreaChoice-ExtIEs} }
}

RANPagingAreaChoice-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


RANPagingAttemptInfo ::= SEQUENCE {
    pagingAttemptCount                  INTEGER (1..16, ...),
    intendedNumberOfPagingAttempts      INTEGER (1..16, ...),
    nextPagingAreaScope                 ENUMERATED {same, changed, ...} OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {RANPagingAttemptInfo-ExtIEs} } OPTIONAL,
    ...
}

RANPagingAttemptInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RANPagingFailure        ::=     ENUMERATED {
    true,
    ...
}

RBsetConfiguration ::= SEQUENCE {
    subcarrierSpacing       SubcarrierSpacing,
    rBsetSize               ENUMERATED {rb2, rb4, rb8, rb16, rb32, rb64},
    numberofRBSets          INTEGER(1.. maxnoofRBsetsPerCell),
    iE-Extensions               ProtocolExtensionContainer { { RBsetConfiguration-ExtIEs} } OPTIONAL,
    ...
}

RBsetConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


Redcap-Bcast-Information ::= BIT STRING(SIZE(8))

RedundantQoSFlowIndicator ::= ENUMERATED {true, false}

RedundantPDUSessionInformation ::= SEQUENCE {
    rSN                 RSN,
    iE-Extensions       ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }  OPTIONAL,
    ...
}

RedundantPDUSessionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-PDUSession-PairID   CRITICALITY ignore  EXTENSION PDUSession-PairID PRESENCE optional   },
    ...
}

RSN ::= ENUMERATED {v1, v2, ...}


ReflectiveQoSAttribute ::= ENUMERATED {subject-to-reflective-QoS, ...}

RequestedSRSTransmissionCharacteristics ::= OCTET STRING


RoutingID ::= OCTET STRING

ReplacingCells ::= SEQUENCE (SIZE(0.. maxnoofCellsinNG-RANnode)) OF ReplacingCells-Item

ReplacingCells-Item ::= SEQUENCE {
    globalNG-RANCell-ID             GlobalCell-ID,
    iE-Extensions       ProtocolExtensionContainer { {ReplacingCells-Item-ExtIEs} } OPTIONAL,
    ...
}

ReplacingCells-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, infinity, ...}


ReportArea ::= ENUMERATED {
    cell,
    ...
}

ReportConfigContainer ::= OCTET STRING

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60, ...}

ReportType ::= CHOICE {
    periodical                  Periodical,
    eventTriggered              EventTriggered,
    ...,
    choice-extension        ProtocolIE-Single-Container { {ReportType-ExtIEs} }
}

ReportType-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


ExtendedReportIntervalMDT ::= ENUMERATED {
    ms20480,
    ms40960,
    ...
}


ReportCharacteristics ::= BIT STRING(SIZE(32))

ReportCharacteristicsForDataCollection ::= BIT STRING(SIZE(32))



ReportingPeriodicity ::= ENUMERATED {
    half-thousand-ms,
    one-thousand-ms,
    two-thousand-ms,
    five-thousand-ms,
    ten-thousand-ms,
    ...
}

ReportingPeriodicityForDataCollection ::= ENUMERATED {
    half-thousand-ms,
    one-thousand-ms,
    two-thousand-ms,
    five-thousand-ms,
    ten-thousand-ms,
    ...
}

RequestedPredictionTime ::= INTEGER (1..60, ...)


RegistrationRequest ::= ENUMERATED {start, stop, add, ... }

RegistrationRequestForDataCollection ::= ENUMERATED {start, stop, ... }


RequestReferenceID ::= INTEGER (1..64, ...)


ReservedSubframePattern ::= SEQUENCE {
    subframeType                    ENUMERATED {mbsfn, non-mbsfn, ...},
    reservedSubframePattern         BIT STRING (SIZE(10..160)),
    mbsfnControlRegionLength        MBSFNControlRegionLength                    OPTIONAL,
    iE-Extension                    ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} } OPTIONAL,
    ...
}

ReservedSubframePattern-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



ResetRequestTypeInfo ::= CHOICE {
    fullReset           ResetRequestTypeInfo-Full,
    partialReset        ResetRequestTypeInfo-Partial,
    choice-extension    ProtocolIE-Single-Container { {ResetRequestTypeInfo-ExtIEs} }
}

ResetRequestTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ResetRequestTypeInfo-Full ::= SEQUENCE {
    iE-Extension                    ProtocolExtensionContainer { {ResetRequestTypeInfo-Full-ExtIEs} } OPTIONAL,
    ...
}

ResetRequestTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ResetRequestTypeInfo-Partial ::= SEQUENCE {
    ue-contexts-ToBeReleasedList    ResetRequestPartialReleaseList,
    iE-Extension                    ProtocolExtensionContainer { {ResetRequestTypeInfo-Partial-ExtIEs} } OPTIONAL,
    ...
}

ResetRequestTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ResetRequestPartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetRequestPartialReleaseItem

ResetRequestPartialReleaseItem ::= SEQUENCE {
    ng-ran-node1UEXnAPID                        NG-RANnodeUEXnAPID          OPTIONAL,
    ng-ran-node2UEXnAPID                        NG-RANnodeUEXnAPID          OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { {ResetRequestPartialReleaseItem-ExtIEs} } OPTIONAL,
    ...
}

ResetRequestPartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ResetResponseTypeInfo ::= CHOICE {
    fullReset           ResetResponseTypeInfo-Full,
    partialReset        ResetResponseTypeInfo-Partial,
    choice-extension    ProtocolIE-Single-Container { {ResetResponseTypeInfo-ExtIEs} }
}

ResetResponseTypeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

ResetResponseTypeInfo-Full ::= SEQUENCE {
    iE-Extension                    ProtocolExtensionContainer { {ResetResponseTypeInfo-Full-ExtIEs} } OPTIONAL,
    ...
}

ResetResponseTypeInfo-Full-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ResetResponseTypeInfo-Partial ::= SEQUENCE {
    ue-contexts-AdmittedToBeReleasedList    ResetResponsePartialReleaseList,
    iE-Extension                    ProtocolExtensionContainer { {ResetResponseTypeInfo-Partial-ExtIEs} } OPTIONAL,
    ...
}

ResetResponseTypeInfo-Partial-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ResetResponsePartialReleaseList ::= SEQUENCE (SIZE(1..maxnoofUEContexts)) OF ResetResponsePartialReleaseItem

ResetResponsePartialReleaseItem ::= SEQUENCE {
    ng-ran-node1UEXnAPID                        NG-RANnodeUEXnAPID      OPTIONAL,
    ng-ran-node2UEXnAPID                        NG-RANnodeUEXnAPID      OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { {ResetResponsePartialReleaseItem-ExtIEs} } OPTIONAL,
    ...
}

ResetResponsePartialReleaseItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


RLCMode ::= ENUMERATED {
    rlc-am,
    rlc-um-bidirectional,
    rlc-um-unidirectional-ul,
    rlc-um-unidirectional-dl,
    ...
    }


RLC-Status ::= SEQUENCE {
    reestablishment-Indication  Reestablishment-Indication,
    iE-Extensions                   ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,
    ...
}

RLC-Status-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RLCDuplicationInformation ::=       SEQUENCE {
    rLCDuplicationStateList         RLCDuplicationStateList,
    rLC-PrimaryIndicator        ENUMERATED {true, false}        OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { {RLCDuplicationInformation-ItemExtIEs} }   OPTIONAL
}

RLCDuplicationInformation-ItemExtIEs    XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RLCDuplicationStateList ::=     SEQUENCE (SIZE(1..maxnoofRLCDuplicationstate)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=    SEQUENCE {
    duplicationState            ENUMERATED {active,inactive, ...},
    iE-Extensions   ProtocolExtensionContainer { {RLCDuplicationState-ItemExtIEs } }    OPTIONAL,
    ...
}

RLCDuplicationState-ItemExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Reestablishment-Indication ::= ENUMERATED {
    reestablished,
    ...
}


RFSP-Index ::= INTEGER (1..256)


RRCConfigIndication ::= ENUMERATED {
    full-config,
    delta-config,
    ...
}

RRCConnections::= SEQUENCE {
    noofRRCConnections                          NoofRRCConnections,
    availableRRCConnectionCapacityValue         AvailableRRCConnectionCapacityValue,
    iE-Extensions               ProtocolExtensionContainer { { RRCConnections-ExtIEs} } OPTIONAL,
    ...
}

RRCConnections-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


RRCConnReestab-Indicator ::= ENUMERATED { reconfigurationFailure, handoverFailure, otherFailure, ...}

RRCReestab-initiated ::= SEQUENCE {
    rRRCReestab-initiated-reporting RRCReestab-Initiated-Reporting,
    iE-Extensions           ProtocolExtensionContainer { { RRCReestab-initiated-ExtIEs} } OPTIONAL,
    ...
}

RRCReestab-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RRCReestab-Initiated-Reporting ::= CHOICE {
    rRCReestab-reporting-wo-UERLFReport                 RRCReestab-Initiated-Reporting-wo-UERLFReport,
    rRCReestab-reporting-with-UERLFReport               RRCReestab-Initiated-Reporting-with-UERLFReport,
    choice-extension                ProtocolIE-Single-Container { {RRCReestab-Initiated-Reporting-ExtIEs} }
}

RRCReestab-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

RRCReestab-Initiated-Reporting-wo-UERLFReport ::= SEQUENCE {
    failureCellPCI      NG-RAN-CellPCI,
    reestabCellCGI      GlobalNG-RANCell-ID,
    c-RNTI              C-RNTI,
    shortMAC-I          MAC-I,
    iE-Extensions       ProtocolExtensionContainer { { RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs} } OPTIONAL,
    ...
}

RRCReestab-Initiated-Reporting-wo-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-RRCConnReestab-Indicator    CRITICALITY ignore  EXTENSION RRCConnReestab-Indicator      PRESENCE optional },
    ...
}

RRCReestab-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {
    uERLFReportContainer    UERLFReportContainer,
    iE-Extensions           ProtocolExtensionContainer { {RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,
    ...
}

RRCReestab-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RRCSetup-initiated ::= SEQUENCE {
    rRRCSetup-Initiated-Reporting   RRCSetup-Initiated-Reporting,
    uERLFReportContainer            UERLFReportContainer            OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { { RRCSetup-initiated-ExtIEs} } OPTIONAL,
    ...
}

RRCSetup-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

RRCSetup-Initiated-Reporting ::= CHOICE {
    rRCSetup-reporting-with-UERLFReport             RRCSetup-Initiated-Reporting-with-UERLFReport,
    choice-extension                ProtocolIE-Single-Container { {RRCSetup-Initiated-Reporting-ExtIEs} }
}

RRCSetup-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

RRCSetup-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {
    uERLFReportContainer    UERLFReportContainer,
    iE-Extensions           ProtocolExtensionContainer { {RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,
    ...
}

RRCSetup-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


RRCResumeCause ::= ENUMERATED {
    rna-Update,
    ...
}


RaReportIndicationList ::= SEQUENCE (SIZE(1..maxnoofUEsforRAReportIndications)) OF RaReportIndicationList-Item

RaReportIndicationList-Item ::= SEQUENCE {
    m-NG-RAN-node-UE-XnAP-ID            NG-RANnodeUEXnAPID,
    iE-Extensions                       ProtocolExtensionContainer { { RaReportIndicationList-Item-ExtIEs} }    OPTIONAL,
    ...
}

RaReportIndicationList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



RadioResourceStatusNR-U ::= SEQUENCE {
    dL-Total-PRB-usage          INTEGER (0..100),
    uL-Total-PRB-usage          INTEGER (0..100),
    iE-Extensions       ProtocolExtensionContainer {{ RadioResourceStatusNR-U-ExtIEs}} OPTIONAL,
    ...
}

RadioResourceStatusNR-U-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ReportingThreshold  ::= INTEGER (0.. 4000000000, ...)

ReferenceConfiguration ::= OCTET STRING

RequestForCSI-RSResourceConfigForLayer1Measurements  ::= ENUMERATED {true, ...}



RA-RNTI ::= INTEGER (0..65535, ...)



-- S

SBFD-Frequency-Configuration ::= OCTET STRING

SCGreconfigNotification ::= ENUMERATED {executed, ... , executed-deleted, deleted }

S-NSSAIListQoE ::= SEQUENCE (SIZE(1..maxnoofSNSSAIforQMC)) OF S-NSSAI

S-BasedMDT ::= SEQUENCE {
    ng-ran-TraceID              NG-RANTraceID,
    iE-Extension                ProtocolExtensionContainer { {S-BasedMDT-ExtIEs} }  OPTIONAL,
    ...
}

S-BasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-CPAC-Request ::= ENUMERATED {initiation, ...}

S-CPAC-Request-Info ::= SEQUENCE {
    s-CPAC-Security-Config-List         S-CPAC-SecurityConfig-List,
    s-CPAC-MultiTargetSN-List           S-CPAC-MultiTargetSN-List           OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { {S-CPAC-Request-Info-ExtIEs} } OPTIONAL,
    ...
}

S-CPAC-Request-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-CPAC-ReferenceConfig-Request ::= ENUMERATED {request, ...}

S-CPAC-SecurityConfig-List ::= SEQUENCE (SIZE(1..maxnoofSecurityConfigurations)) OF S-CPAC-SecurityConfig-Item

S-CPAC-SecurityConfig-Item ::= SEQUENCE {
    s-ng-RANnode-SecurityKey            S-NG-RANnode-SecurityKey,
    sk-counter                          SK-COUNTER,
    iE-Extensions       ProtocolExtensionContainer { {S-CPAC-SecurityConfig-Item-ExtIEs} } OPTIONAL,
    ...
}

S-CPAC-SecurityConfig-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-CPAC-MultiTargetSN-List ::= SEQUENCE (SIZE(1..maxnoofTargetSNsMinusOne)) OF S-CPAC-MultiTargetSN-Item

S-CPAC-MultiTargetSN-Item ::= SEQUENCE {
    target-S-NG-RANnodeID               GlobalNG-RANNode-ID,
    recommendedCandidatePSCells         OCTET STRING,
    iE-Extensions       ProtocolExtensionContainer { {S-CPAC-MultiTargetSN-Item-ExtIEs} } OPTIONAL,
    ...
}

S-CPAC-MultiTargetSN-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-CPAC-InterSN-ExecutionNotify ::= ENUMERATED {executed, ...}

ServiceType ::= ENUMERATED{
    qMC-for-streaming-service,
    qMC-for-MTSI-service,
    qMC-for-VR-service,
    ...
}

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {
    secondarydataForwardingInfoFromTarget               DataForwardingInfoFromTargetNGRANnode,
    iE-Extensions       ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} } OPTIONAL,
    ...
}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SecondarydataForwardingInfoFromTarget-List ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF SecondarydataForwardingInfoFromTarget-Item

SCGActivationRequest ::= ENUMERATED {activate-scg, deactivate-scg, ...}

SCGActivationStatus ::= ENUMERATED {scg-activated, scg-deactivated, ...}

SCGConfigurationQuery   ::= ENUMERATED {true, ...}

SCGIndicator    ::= ENUMERATED{released, ...}

SCGFailureReportContainer   ::= OCTET STRING

SDTSupportRequest ::= SEQUENCE {
    sdtindicator                SDTIndicator,
    sdtAssistantInfo            SDTAssistantInfo        OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { { SDTSupportRequest-ExtIEs} } OPTIONAL,
    ...
}

SDTSupportRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SDTIndicator ::= ENUMERATED {true, ...}

SDTAssistantInfo ::= ENUMERATED {single-packet, multiple-packets, ...}

SDT-Termination-Request ::= ENUMERATED {radio-link-problem, normal, ..., large-sdt-volume-from-BSR}

SDTPartialUEContextInfo ::= SEQUENCE {
    dRBsToBeSetup                   SDT-DRBsToBeSetupList       OPTIONAL,
    sRBsToBeSetup                   SDT-SRBsToBeSetupList,
    iE-Extensions                   ProtocolExtensionContainer { { SDTPartialUEContextInfo-ExtIEs} }    OPTIONAL,
    ...
}
SDTPartialUEContextInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SDT-DRBsToBeSetupList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF SDT-DRBsToBeSetupList-Item

SDT-DRBsToBeSetupList-Item  ::= SEQUENCE {
    drb-ID                          DRB-ID,
    uL-TNLInfo                      UPTransportLayerInformation,
    dRB-RLC-Bearer-Configuration    OCTET STRING,
    dRB-QoS                         QoSFlowLevelQoSParameters,
    rLC-Mode                        RLCMode,
    s-nssai                         S-NSSAI,
    pDCP-SNLength                   PDCPSNLength,
    flows-Mapped-To-DRB-List        Flows-Mapped-To-DRB-List,
    iE-Extensions       ProtocolExtensionContainer { { SDT-DRBsToBeSetupList-Item-ExtIEs} } OPTIONAL,
    ...
}

SDT-DRBsToBeSetupList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SDT-SRBsToBeSetupList ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SDT-SRBsToBeSetupList-Item

SDT-SRBsToBeSetupList-Item  ::= SEQUENCE {
    srb-ID                          SRB-ID,
    sRB-RLC-Bearer-Configuration    OCTET STRING,
    iE-Extensions                   ProtocolExtensionContainer { { SDT-SRBsToBeSetupList-Item-ExtIEs} } OPTIONAL,
    ...
}

SDT-SRBsToBeSetupList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SRB-ID ::= INTEGER (0..4, ...)

SDTDataForwardingDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF SDTDataForwardingDRBList-Item

SDTDataForwardingDRBList-Item ::= SEQUENCE {
    drb-ID                          DRB-ID,
    dL-TNLInfo                      UPTransportLayerInformation     OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { SDTDataForwardingDRBList-Item-ExtIEs} } OPTIONAL,
    ...
}

SDTDataForwardingDRBList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SecondaryRATUsageInformation ::= SEQUENCE {
    pDUSessionUsageReport       PDUSessionUsageReport               OPTIONAL,
    qosFlowsUsageReportList     QoSFlowsUsageReportList             OPTIONAL,
    iE-Extension                ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }    OPTIONAL,
    ...
}

SecondaryRATUsageInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SecurityIndication ::= SEQUENCE {
    integrityProtectionIndication           ENUMERATED {required, preferred, not-needed, ...},
    confidentialityProtectionIndication     ENUMERATED {required, preferred, not-needed, ...},
    maximumIPdatarate                       MaximumIPdatarate                                               OPTIONAL,
-- This IE shall be present if the Integrity Protection IE within the Security Indication IE is present and set to "required" or "preferred". --
    iE-Extensions                           ProtocolExtensionContainer { {SecurityIndication-ExtIEs} } OPTIONAL,
    ...
}

SecurityIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SecurityResult ::= SEQUENCE {
    integrityProtectionResult               ENUMERATED {performed, not-performed, ...},
    confidentialityProtectionResult         ENUMERATED {performed, not-performed, ...},
    iE-Extensions                           ProtocolExtensionContainer { {SecurityResult-ExtIEs} } OPTIONAL,
    ...
}

SecurityResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SensorMeasurementConfiguration ::= SEQUENCE {
    sensorMeasConfig                SensorMeasConfig,
    sensorMeasConfigNameList        SensorMeasConfigNameList            OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { SensorMeasurementConfiguration-ExtIEs } } OPTIONAL,
    ...
}

SensorMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SensorMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorName

SensorMeasConfig::= ENUMERATED {setup,...}

SensorName ::= SEQUENCE {
    uncompensatedBarometricConfig   ENUMERATED {true, ...}      OPTIONAL,
    ueSpeedConfig                   ENUMERATED {true, ...}      OPTIONAL,
    ueOrientationConfig             ENUMERATED {true, ...}      OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {SensorNameConfig-ExtIEs} } OPTIONAL,
...
}

SensorNameConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- Served Cells E-UTRA IEs


ServedCellInformation-E-UTRA ::= SEQUENCE {
    e-utra-pci                              E-UTRAPCI,
    e-utra-cgi                              E-UTRA-CGI,
    tac                                     TAC,
    ranac                                   RANAC                                                                                   OPTIONAL,
    broadcastPLMNs                          SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,
    e-utra-mode-info                        ServedCellInformation-E-UTRA-ModeInfo,
    numberofAntennaPorts                    NumberOfAntennaPorts-E-UTRA                                                             OPTIONAL,
    prach-configuration                     E-UTRAPRACHConfiguration                                                                OPTIONAL,
    mBSFNsubframeInfo                       MBSFNSubframeInfo-E-UTRA                                                                OPTIONAL,
    multibandInfo                           E-UTRAMultibandInfoList                                                                 OPTIONAL,
    freqBandIndicatorPriority               ENUMERATED {not-broadcast, broadcast, ...}                                              OPTIONAL,
    bandwidthReducedSI                      ENUMERATED {scheduled, ...}                                                             OPTIONAL,
    protectedE-UTRAResourceIndication       ProtectedE-UTRAResourceIndication                                                       OPTIONAL,
    iE-Extensions               ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }                    OPTIONAL,
    ...
}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-BPLMN-ID-Info-EUTRA     CRITICALITY ignore  EXTENSION BPLMN-ID-Info-EUTRA       PRESENCE optional }|
    { ID id-NPRACHConfiguration     CRITICALITY ignore  EXTENSION   NPRACHConfiguration     PRESENCE optional},
    ...
}


ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {
    plmn-id                 PLMN-Identity,
    iE-Extensions           ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,
    ...
}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ServedCellInformation-E-UTRA-ModeInfo ::= CHOICE {
    fdd                 ServedCellInformation-E-UTRA-FDDInfo,
    tdd                 ServedCellInformation-E-UTRA-TDDInfo,
    choice-extension    ProtocolIE-Single-Container{ {ServedCellInformation-E-UTRA-ModeInfo-ExtIEs} }
}

ServedCellInformation-E-UTRA-ModeInfo-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


ServedCellInformation-E-UTRA-FDDInfo ::= SEQUENCE {
    ul-earfcn           E-UTRAARFCN,
    dl-earfcn           E-UTRAARFCN,
    ul-e-utraTxBW       E-UTRATransmissionBandwidth,
    dl-e-utraTxBW       E-UTRATransmissionBandwidth,
    iE-Extensions       ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-FDDInfo-ExtIEs} } OPTIONAL,
    ...
}

ServedCellInformation-E-UTRA-FDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-OffsetOfNbiotChannelNumberToDL-EARFCN   CRITICALITY reject  EXTENSION OffsetOfNbiotChannelNumberToEARFCN        PRESENCE optional}|
    { ID id-OffsetOfNbiotChannelNumberToUL-EARFCN   CRITICALITY reject  EXTENSION OffsetOfNbiotChannelNumberToEARFCN        PRESENCE optional},
    ...
}


ServedCellInformation-E-UTRA-TDDInfo ::= SEQUENCE {
    earfcn                  E-UTRAARFCN,
    e-utraTxBW              E-UTRATransmissionBandwidth,
    subframeAssignmnet      ENUMERATED {sa0,sa1,sa2,sa3,sa4,sa5,sa6,...},
    specialSubframeInfo     SpecialSubframeInfo-E-UTRA,
    iE-Extensions           ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-TDDInfo-ExtIEs} } OPTIONAL,
    ...
}

ServedCellInformation-E-UTRA-TDDInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-OffsetOfNbiotChannelNumberToDL-EARFCN   CRITICALITY reject  EXTENSION OffsetOfNbiotChannelNumberToEARFCN        PRESENCE optional}|
    { ID id-NBIoT-UL-DL-AlignmentOffset             CRITICALITY reject  EXTENSION NBIoT-UL-DL-AlignmentOffset                   PRESENCE optional},
    ...
}


ServedCells-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-E-UTRA-Item

ServedCells-E-UTRA-Item ::= SEQUENCE {
    served-cell-info-E-UTRA     ServedCellInformation-E-UTRA,
    neighbour-info-NR           NeighbourInformation-NR                                 OPTIONAL,
    neighbour-info-E-UTRA       NeighbourInformation-E-UTRA                             OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,
    ...
}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SFN-Offset              CRITICALITY ignore EXTENSION SFN-Offset                 PRESENCE optional },
...
}


ServedCellsToUpdate-E-UTRA ::= SEQUENCE {
    served-Cells-ToAdd-E-UTRA       ServedCells-E-UTRA                                                              OPTIONAL,
    served-Cells-ToModify-E-UTRA    ServedCells-ToModify-E-UTRA                                                     OPTIONAL,
    served-Cells-ToDelete-E-UTRA    SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF E-UTRA-CGI                         OPTIONAL,
iE-Extensions                   ProtocolExtensionContainer { {ServedCellsToUpdate-E-UTRA-ExtIEs} }  OPTIONAL,
    ...
}

ServedCellsToUpdate-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ServedCells-ToModify-E-UTRA ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-E-UTRA-Item

ServedCells-ToModify-E-UTRA-Item ::= SEQUENCE {
    old-ECGI                    E-UTRA-CGI,
    served-cell-info-E-UTRA     ServedCellInformation-E-UTRA,
    neighbour-info-NR           NeighbourInformation-NR                                         OPTIONAL,
    neighbour-info-E-UTRA       NeighbourInformation-E-UTRA                                     OPTIONAL,
    deactivation-indication     ENUMERATED {deactivated, ...}                                   OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {Served-cells-ToModify-E-UTRA-Item-ExtIEs} }   OPTIONAL,
    ...
}

Served-cells-ToModify-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SFN-Offset              CRITICALITY ignore EXTENSION SFN-Offset                 PRESENCE optional },
    ...
}


-- Served Cells NR IEs


ServedCellInformation-NR ::= SEQUENCE {
    nrPCI                               NRPCI,
    cellID                              NR-CGI,
    tac                                 TAC,
    ranac                               RANAC                       OPTIONAL,
    broadcastPLMN                       BroadcastPLMNs,
    nrModeInfo                          NRModeInfo,
    measurementTimingConfiguration      OCTET STRING,
    connectivitySupport                 Connectivity-Support,   
    iE-Extensions                       ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,
    ...
}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-BPLMN-ID-Info-NR                CRITICALITY ignore  EXTENSION BPLMN-ID-Info-NR                      PRESENCE optional }|
    { ID id-ConfiguredTACIndication         CRITICALITY ignore  EXTENSION ConfiguredTACIndication               PRESENCE optional }|
    { ID id-SSB-PositionsInBurst            CRITICALITY ignore  EXTENSION SSB-PositionsInBurst                  PRESENCE optional }|
    { ID id-NRCellPRACHConfig               CRITICALITY ignore  EXTENSION NRCellPRACHConfig                     PRESENCE optional }|
    { ID id-NPN-Broadcast-Information       CRITICALITY reject  EXTENSION NPN-Broadcast-Information         PRESENCE optional }|
    { ID id-CSI-RSTransmissionIndication    CRITICALITY ignore  EXTENSION CSI-RSTransmissionIndication      PRESENCE optional }|
    { ID id-SFN-Offset                      CRITICALITY ignore  EXTENSION SFN-Offset                            PRESENCE optional }|
    { ID id-Supported-MBS-FSA-ID-List       CRITICALITY ignore  EXTENSION Supported-MBS-FSA-ID-List         PRESENCE optional }|
    { ID id-NR-U-ChannelInfo-List           CRITICALITY ignore  EXTENSION NR-U-ChannelInfo-List                 PRESENCE optional }|
    { ID id-Additional-Measurement-Timing-Configuration-List        CRITICALITY ignore  EXTENSION Additional-Measurement-Timing-Configuration-List      PRESENCE optional }|
    { ID id-Redcap-Bcast-Information        CRITICALITY ignore  EXTENSION Redcap-Bcast-Information          PRESENCE optional }|
    { ID id-eRedcap-Bcast-Information       CRITICALITY ignore  EXTENSION ERedcap-Bcast-Information         PRESENCE optional }|
    { ID id-MobileIABCell                   CRITICALITY ignore  EXTENSION MobileIABCell                         PRESENCE optional }|
    { ID id-XR-Bcast-Information            CRITICALITY ignore  EXTENSION XR-Bcast-Information                  PRESENCE optional }|
    { ID id-BarringExemptionforEmerCallInfo CRITICALITY ignore  EXTENSION BarringExemptionforEmerCallInfo   PRESENCE optional }|
    { ID id-NZP-CSI-RS-Resources-Config     CRITICALITY ignore  EXTENSION NZP-CSI-RS-Resources-Config       PRESENCE optional }|
    { ID id-SRS-Resource-Configuration      CRITICALITY ignore  EXTENSION SRS-Resource-Configuration            PRESENCE optional },
    ...
}

SFN-Offset ::= SEQUENCE {
    sFN-Time-Offset                 BIT STRING (SIZE(24)),
    
    iE-Extensions       ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,
    ...
}
SFN-Offset-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
        ...
}


ServedCells-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-NR-Item

ServedCells-NR-Item ::= SEQUENCE {
    served-cell-info-NR         ServedCellInformation-NR,
    neighbour-info-NR           NeighbourInformation-NR             OPTIONAL,
    neighbour-info-E-UTRA       NeighbourInformation-E-UTRA         OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ServedCells-NR-Item-ExtIEs} } OPTIONAL,
    ...
}

ServedCells-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-ServedCellSpecificInfoReq-NR    CRITICALITY ignore EXTENSION    ServedCellSpecificInfoReq-NR    PRESENCE optional },
    ...
}


ServedCells-ToModify-NR ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF ServedCells-ToModify-NR-Item

ServedCells-ToModify-NR-Item ::= SEQUENCE {
    old-NR-CGI                  NR-CGI,
    served-cell-info-NR         ServedCellInformation-NR,
    neighbour-info-NR           NeighbourInformation-NR                                         OPTIONAL,
    neighbour-info-E-UTRA       NeighbourInformation-E-UTRA                                     OPTIONAL,
    deactivation-indication     ENUMERATED {deactivated, ...}                                   OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {Served-cells-ToModify-NR-Item-ExtIEs} }   OPTIONAL,
    ...
}

Served-cells-ToModify-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ServedCellSpecificInfoReq-NR    ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF ServedCellSpecificInfoReq-NR-Item

ServedCellSpecificInfoReq-NR-Item   ::= SEQUENCE {
    nRCGI                                   NR-CGI,
    additionalMTCListRequestIndicator       ENUMERATED {additionalMTCListRequested, ...}            OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { ServedCellSpecificInfoReq-NR-Item-ExtIEs} }  OPTIONAL,
    ...
}

ServedCellSpecificInfoReq-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {  
    ...
}

ServedCellsToUpdate-NR ::= SEQUENCE {
    served-Cells-ToAdd-NR       ServedCells-NR                                                          OPTIONAL,
    served-Cells-ToModify-NR    ServedCells-ToModify-NR                                                 OPTIONAL,
    served-Cells-ToDelete-NR    SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI                     OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {ServedCellsToUpdate-NR-ExtIEs} } OPTIONAL,
    ...
}

ServedCellsToUpdate-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}




SharedResourceType ::= CHOICE {
    ul-onlySharing              SharedResourceType-UL-OnlySharing,
    ul-and-dl-Sharing           SharedResourceType-ULDL-Sharing,
    choice-extension            ProtocolIE-Single-Container { {SharedResourceType-ExtIEs} }
}

SharedResourceType-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SharedResourceType-UL-OnlySharing ::= SEQUENCE {
    ul-resourceBitmap           DataTrafficResources,
    iE-Extensions           ProtocolExtensionContainer { {SharedResourceType-UL-OnlySharing-ExtIEs} } OPTIONAL,
    ...
}

SharedResourceType-UL-OnlySharing-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SharedResourceType-ULDL-Sharing ::= CHOICE {
    ul-resources                SharedResourceType-ULDL-Sharing-UL-Resources,
    dl-resources                SharedResourceType-ULDL-Sharing-DL-Resources,
    choice-extension            ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-ExtIEs} }
}

SharedResourceType-ULDL-Sharing-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SharedResourceType-ULDL-Sharing-UL-Resources ::= CHOICE {
    unchanged                   NULL,
    changed                     SharedResourceType-ULDL-Sharing-UL-ResourcesChanged,
    choice-extension            ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs} }
}

SharedResourceType-ULDL-Sharing-UL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged ::= SEQUENCE {
    ul-resourceBitmap           DataTrafficResources,
    iE-Extensions           ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs} } OPTIONAL,
    ...
}

SharedResourceType-ULDL-Sharing-UL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SharedResourceType-ULDL-Sharing-DL-Resources ::= CHOICE {
    unchanged                   NULL,
    changed                     SharedResourceType-ULDL-Sharing-DL-ResourcesChanged,
    choice-extension            ProtocolIE-Single-Container { {SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs} }
}

SharedResourceType-ULDL-Sharing-DL-Resources-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged ::= SEQUENCE {
    dl-resourceBitmap           DataTrafficResources,
    iE-Extensions           ProtocolExtensionContainer { {SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs} } OPTIONAL,
    ...
}

SharedResourceType-ULDL-Sharing-DL-ResourcesChanged-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SK-COUNTER ::= INTEGER (0..65535)

SliceAvailableCapacity ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceAvailableCapacity-Item

SliceAvailableCapacity-Item ::= SEQUENCE {
    pLMNIdentity                        PLMN-Identity,
    sNSSAIAvailableCapacity-List        SNSSAIAvailableCapacity-List,
    iE-Extensions                       ProtocolExtensionContainer { { SliceAvailableCapacity-Item-ExtIEs} }    OPTIONAL,
    ...
}


SliceAvailableCapacity-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SliceMDTList ::= SEQUENCE (SIZE(1.. maxnoofSliceItemsforMDT)) OF SliceMDTItem


SliceMDTItem ::= SEQUENCE {
    sNSSAI              S-NSSAI,
    iE-Extensions       ProtocolExtensionContainer { { SliceMDTItem-ExtIEs} }   OPTIONAL,
    ...
}

SliceMDTItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::={
    ...
}

SNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIAvailableCapacity-Item

SNSSAIAvailableCapacity-Item ::= SEQUENCE {
    sNSSAI      S-NSSAI,
    sliceAvailableCapacityValueDownlink INTEGER (0..100),
    sliceAvailableCapacityValueUplink   INTEGER (0..100),
    iE-Extensions               ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }  OPTIONAL
}

SNSSAIAvailableCapacity-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SliceRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceRadioResourceStatus-Item

SliceRadioResourceStatus-Item   ::= SEQUENCE {
    plmn-Identity                       PLMN-Identity,
    sNSSAIRadioResourceStatus-List      SNSSAIRadioResourceStatus-List,
    iE-Extensions                       ProtocolExtensionContainer { { SliceRadioResourceStatus-Item-ExtIEs} }  OPTIONAL,
    ...
}

SliceRadioResourceStatus-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
SLPositioning-Ranging-Services-Info::= SEQUENCE{
    sLPositioning-Ranging-Authorized    SLPositioning-Ranging-Authorized,
    rSPP-transport-QoS-parameters       RSPP-transport-QoS-parameters       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { SLPositioning-Ranging-Services-Info-ExtIEs} }        OPTIONAL
}

SLPositioning-Ranging-Services-Info-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SLPositioning-Ranging-Authorized ::= ENUMERATED { 
    authorized,
    not-authorized,
    ...
}

RSPP-transport-QoS-parameters ::= SEQUENCE {
    rSPPQoSFlowList             RSPPQoSFlowList,
    rSPPLinkAggregateBitRates   BitRate                                             OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { RSPP-transport-QoS-parameters-ExtIEs} }  OPTIONAL,
    ...
}

RSPP-transport-QoS-parameters-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
...
}

RSPPQoSFlowList ::= SEQUENCE (SIZE(1..maxnoofRSPPQoSFlows)) OF RSPPQoSFlowItem

RSPPQoSFlowItem ::= SEQUENCE {
    pQI                 FiveQI,
    rSPPFlowBitRates    RSPPFlowBitRates                                        OPTIONAL,
    range               Range                                                   OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { RSPPQoSFlowItem-ExtIEs} }    OPTIONAL,
    ...
}

RSPPQoSFlowItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
...
}

RSPPFlowBitRates ::= SEQUENCE {
    guaranteedFlowBitRate       BitRate,
    maximumFlowBitRate          BitRate,
    iE-Extensions       ProtocolExtensionContainer { { RSPPFlowBitRates-ExtIEs} }   OPTIONAL,
    ...
}

RSPPFlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNSSAIRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SNSSAIRadioResourceStatus-Item

SNSSAIRadioResourceStatus-Item  ::= SEQUENCE {
    sNSSAI                              S-NSSAI,
    slice-DL-GBR-PRB-Usage              Slice-DL-GBR-PRB-Usage,
    slice-UL-GBR-PRB-Usage              Slice-UL-GBR-PRB-Usage,
    slice-DL-non-GBR-PRB-Usage          Slice-DL-non-GBR-PRB-Usage,
    slice-UL-non-GBR-PRB-Usage          Slice-UL-non-GBR-PRB-Usage,
    slice-DL-Total-PRB-Allocation       Slice-DL-Total-PRB-Allocation,
    slice-UL-Total-PRB-Allocation       Slice-UL-Total-PRB-Allocation,
    iE-Extensions                       ProtocolExtensionContainer { { SNSSAIRadioResourceStatus-Item-ExtIEs} } OPTIONAL,
    ...
}

SNSSAIRadioResourceStatus-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Slice-DL-GBR-PRB-Usage          ::= INTEGER (0..100)

Slice-UL-GBR-PRB-Usage          ::= INTEGER (0..100)

Slice-DL-non-GBR-PRB-Usage      ::= INTEGER (0..100)

Slice-UL-non-GBR-PRB-Usage      ::= INTEGER (0..100)

Slice-DL-Total-PRB-Allocation   ::= INTEGER (0..100)

Slice-UL-Total-PRB-Allocation   ::= INTEGER (0..100)


SliceSupport-List   ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAI

SliceToReport-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReport-List-Item

SliceToReport-List-Item ::= SEQUENCE {
    pLMNIdentity                PLMN-Identity,
    sNSSAIlist                  SNSSAI-list,
    iE-Extensions                       ProtocolExtensionContainer { { SliceToReport-List-Item-ExtIEs} }    OPTIONAL,
    ...
}


SliceToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {
    sNSSAI      S-NSSAI,
    iE-Extensions               ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }   OPTIONAL
}

SNSSAI-Item-ExtIEs  XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SlotConfiguration-List ::= SEQUENCE (SIZE (1..maxnoofslots)) OF SlotConfiguration-List-Item

SlotConfiguration-List-Item ::= SEQUENCE {
    slotIndex                       INTEGER (0..5119),
    symbolAllocation-in-Slot        SymbolAllocation-in-Slot,
    iE-Extensions               ProtocolExtensionContainer { {SlotConfiguration-List-Item-ExtIEs} } OPTIONAL,
    ...
}

SlotConfiguration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-NG-RANnode-SecurityKey ::= BIT STRING (SIZE(256))

S-NG-RANnode-Addition-Trigger-Ind ::= ENUMERATED {
    sn-change,
    inter-MN-HO,
    intra-MN-HO,
    ...
}

S-NSSAI ::= SEQUENCE {
    sst                     OCTET STRING (SIZE(1)),
    sd                      OCTET STRING (SIZE(3))                          OPTIONAL,
    iE-Extensions           ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,
    ...
}

S-NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNMobilityInformation ::= BIT STRING (SIZE(32))


SNPNIdentity ::= SEQUENCE {
    plmnID              PLMN-Identity,
    nid                 NID,
    iE-Extensions       ProtocolExtensionContainer { {SNPNIdentity-ExtIEs} }    OPTIONAL,
    ...
}

SNPNIdentity-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SNTriggered ::=ENUMERATED{
    true,
    ...
}

SpecialSubframeInfo-E-UTRA ::= SEQUENCE {
    specialSubframePattern  SpecialSubframePatterns-E-UTRA,
    cyclicPrefixDL          CyclicPrefix-E-UTRA-DL,
    cyclicPrefixUL          CyclicPrefix-E-UTRA-UL,
    iE-Extensions           ProtocolExtensionContainer { {SpecialSubframeInfo-E-UTRA-ExtIEs} } OPTIONAL,
    ...
}

SpecialSubframeInfo-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SpecialSubframePatterns-E-UTRA ::= ENUMERATED {
    ssp0,
    ssp1,
    ssp2,
    ssp3,
    ssp4,
    ssp5,
    ssp6,
    ssp7,
    ssp8,
    ssp9,
    ssp10,
    ...
}


SpectrumSharingGroupID ::= INTEGER (1..maxnoofCellsinNG-RANnode)

SplitSessionIndicator ::= ENUMERATED {
    split,
    ...
}

SplitSRBsTypes ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SPRAvailability ::= ENUMERATED {spr-available, ...}

SRSPositioningConfigOrActivationRequest::= ENUMERATED {true, ...}


SRSConfiguration ::= OCTET STRING

SRS-Resource-Indication ::= ENUMERATED {true, ...}

SRS-Resource-Configuration ::= SEQUENCE {
    sRS-Resource-Configuration-List     SRS-Resource-Configuration-List,
    iE-Extensions                       ProtocolExtensionContainer { {SRS-Resource-Configuration-ExtIEs} } OPTIONAL,
    ...
}

SRS-Resource-Configuration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SRS-Resource-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofSRS-Resource)) OF SRS-Resource-Configuration-List-Item

SRS-Resource-Configuration-List-Item ::= SEQUENCE {
    sRS-Resource                        OCTET STRING,
    iE-Extensions                       ProtocolExtensionContainer { {SRS-Resource-Configuration-List-Item-ExtIEs} } OPTIONAL,
    ...
}

SRS-Resource-Configuration-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SSBAreaCapacityValue-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaCapacityValue-List-Item

SSBAreaCapacityValue-List-Item  ::= SEQUENCE {
    sSBIndex                INTEGER(0..63),
    ssbAreaCapacityValue    INTEGER (0..100),
    iE-Extensions                       ProtocolExtensionContainer { { SSBAreaCapacityValue-List-Item-ExtIEs} } OPTIONAL,
    ...
}


SSBAreaCapacityValue-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SSBAreaRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-List-Item

SSBAreaRadioResourceStatus-List-Item    ::= SEQUENCE {
    sSBIndex                            INTEGER(0..63),
    ssb-Area-DL-GBR-PRB-usage           DL-GBR-PRB-usage,
    ssb-Area-UL-GBR-PRB-usage           UL-GBR-PRB-usage,
    ssb-Area-dL-non-GBR-PRB-usage       DL-non-GBR-PRB-usage,
    ssb-Area-uL-non-GBR-PRB-usage       UL-non-GBR-PRB-usage,
    ssb-Area-dL-Total-PRB-usage         DL-Total-PRB-usage,
    ssb-Area-uL-Total-PRB-usage         UL-Total-PRB-usage,
    iE-Extensions                       ProtocolExtensionContainer { { SSBAreaRadioResourceStatus-List-Item-ExtIEs} }   OPTIONAL,
    ...
}


SSBAreaRadioResourceStatus-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-DL-scheduling-PDCCH-CCE-usage       CRITICALITY ignore  EXTENSION DL-scheduling-PDCCH-CCE-usage PRESENCE optional}|
    { ID id-UL-scheduling-PDCCH-CCE-usage       CRITICALITY ignore  EXTENSION UL-scheduling-PDCCH-CCE-usage PRESENCE optional},
    ...
}

SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (0..maxnoofSSBAreas)) OF SSB-Coverage-Modification-List-Item

SSB-Coverage-Modification-List-Item ::= SEQUENCE {
    sSBIndex                        INTEGER(0..63),
    sSBCoverageState                INTEGER (0..15, ...),
    iE-Extension            ProtocolExtensionContainer { { SSB-Coverage-Modification-List-Item-ExtIEs} } OPTIONAL,
    
    ...
}


SSB-Coverage-Modification-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SSB-PositionsInBurst ::= CHOICE {
    shortBitmap                     BIT STRING (SIZE (4)),
    mediumBitmap                    BIT STRING (SIZE (8)),
    longBitmap                      BIT STRING (SIZE (64)),
    choice-extension                ProtocolIE-Single-Container { {SSB-PositionsInBurst-ExtIEs} }
}

SSB-PositionsInBurst-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SSB-freqInfo ::= INTEGER (0..maxNRARFCN)

SSB-subcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ...}

SSBOffsets-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBOffsets-Item

SSBOffsets-Item ::= SEQUENCE {
    nG-RANnode1SSBOffsets           SSBOffsetInformation                                        OPTIONAL,
    nG-RANnode2ProposedSSBOffsets   SSBOffsetInformation,
    iE-Extensions                   ProtocolExtensionContainer { { SSBOffsets-Item-ExtIEs} }    OPTIONAL,
    ...
}

SSBOffsets-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SSBOffsetInformation    ::= SEQUENCE {
    sSBIndex            INTEGER(0..63),
    sSBTriggeringOffset         MobilityParametersInformation,
    iE-Extensions               ProtocolExtensionContainer { { SSBOffsetInformation-ExtIEs} }   OPTIONAL,
    ...
}

SSBOffsetInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SSBOffsetModificationRange  ::= SEQUENCE {
    sSBIndex                INTEGER(0..63),
    sSBobilityParametersModificationRange       MobilityParametersModificationRange,
    iE-Extensions               ProtocolExtensionContainer { { SSBOffsetModificationRange-ExtIEs} } OPTIONAL,
    ...
}

SSBOffsetModificationRange-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SSBToReport-List ::= SEQUENCE (SIZE(1..maxnoofSSBAreas)) OF SSBToReport-List-Item

SSBToReport-List-Item   ::= SEQUENCE {
    sSBIndex                INTEGER(0..63),
    iE-Extensions                       ProtocolExtensionContainer { { SSBToReport-List-Item-ExtIEs} }  OPTIONAL,
    ...
}


SSBToReport-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SSB-transmissionPeriodicity ::= ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, ..., sf5}

SSB-transmissionTimingOffset ::= INTEGER (0..127, ...)

SSB-transmissionBitmap ::= CHOICE {
    shortBitmap         BIT STRING (SIZE (4)),
    mediumBitmap        BIT STRING (SIZE (8)),
    longBitmap          BIT STRING (SIZE (64)),
    choice-extension    ProtocolIE-Single-Container { { SSB-transmisisonBitmap-ExtIEs} }
}

SSB-transmisisonBitmap-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

SuccessfulHOReportInformation   ::= SEQUENCE (SIZE(1.. maxnoofSuccessfulHOReports)) OF SuccessfulHOReportList-Item
SuccessfulHOReportList-Item ::= SEQUENCE {
    successfulHOReport                  SuccessfulHOReportContainer,
    iE-Extensions                       ProtocolExtensionContainer { { SuccessfulHOReportList-Item-ExtIEs} }    OPTIONAL,
    ...
}

SuccessfulHOReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SuccessfulHOReportContainer ::= OCTET STRING

SuccessfulPSCellChangeReportInformation ::= SEQUENCE (SIZE(1.. maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReportList-Item
SuccessfulPSCellChangeReportList-Item   ::= SEQUENCE {
    successfulPSCellChangeReport        SuccessfulPSCellChangeReportContainer,
    sNMobilityInformation               SNMobilityInformation                                                           OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { SuccessfulPSCellChangeReportList-Item-ExtIEs} }  OPTIONAL,
    ...
}

SuccessfulPSCellChangeReportList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SuccessfulPSCellChangeReportContainer ::= OCTET STRING


SUL-FrequencyBand ::= INTEGER (1..1024)


SUL-Information ::= SEQUENCE {
    sulFrequencyInfo            NRARFCN,
    sulTransmissionBandwidth    NRTransmissionBandwidth,
    iE-Extensions               ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,
    ...
}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-CarrierList             CRITICALITY ignore  EXTENSION NRCarrierList         PRESENCE optional }|
    { ID id-FrequencyShift7p5khz    CRITICALITY ignore  EXTENSION FrequencyShift7p5khz  PRESENCE optional },

...
}

Supported-MBS-FSA-ID-List ::= SEQUENCE (SIZE(1..maxnoofMBSFSAs)) OF MBS-FrequencySelectionArea-Identity

SupportedSULBandList ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF SupportedSULBandItem

SupportedSULBandItem ::= SEQUENCE {
    sulBandItem                 SUL-FrequencyBand,
    iE-Extensions               ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,
    ...
}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SurvivalTime ::= INTEGER (0..1920000, ...)


SymbolAllocation-in-Slot ::= CHOICE {
    allDL               SymbolAllocation-in-Slot-AllDL,
    allUL               SymbolAllocation-in-Slot-AllUL,
    bothDLandUL         SymbolAllocation-in-Slot-BothDLandUL,
    choice-extension    ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }
}

SymbolAllocation-in-Slot-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


SymbolAllocation-in-Slot-AllDL ::= SEQUENCE {
    iE-Extension        ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllDL-ExtIEs} }  OPTIONAL,
    ...
}

SymbolAllocation-in-Slot-AllDL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SymbolAllocation-in-Slot-AllUL ::= SEQUENCE {
    iE-Extension        ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllUL-ExtIEs} }  OPTIONAL,
    ...
}

SymbolAllocation-in-Slot-AllUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {
    numberofDLSymbols   INTEGER (0..13),
    numberofULSymbols   INTEGER (0..13),
    iE-Extension        ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }    OPTIONAL,
    ...
}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-permutation     CRITICALITY ignore  EXTENSION Permutation   PRESENCE optional },
    ...
}


SNPN-CellBasedMDT::= SEQUENCE {
    sNPN-CellIdListforMDT       SNPN-CellIdListforMDT,
    iE-Extensions               ProtocolExtensionContainer { {SNPN-CellBasedMDT-ExtIEs} } OPTIONAL,
    ...
}

SNPN-CellBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNPN-CellIdListforMDT ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF SNPN-CellIdforMDT-Item

SNPN-CellIdforMDT-Item ::= SEQUENCE {
    nRCGI               NR-CGI,
    nID                 NID,
    iE-Extensions       ProtocolExtensionContainer { {SNPN-CellIdforMDT-Item-ExtIEs} }  OPTIONAL,
    ...
}

SNPN-CellIdforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SNPN-TAIBasedMDT ::= SEQUENCE {
    sNPN-TAIListforMDT      SNPN-TAIListforMDT,
    iE-Extensions           ProtocolExtensionContainer { {SNPN-TAIBasedMDT-ExtIEs} } OPTIONAL,
    ...
}

SNPN-TAIBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNPN-TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF SNPN-TAIforMDT-Item

SNPN-TAIforMDT-Item ::= SEQUENCE {
    plmn-ID             PLMN-Identity,
    tAC                 TAC,
    nID                 NID,
    iE-Extensions           ProtocolExtensionContainer { {SNPN-TAIforMDT-Item-ExtIEs} } OPTIONAL,
    ...
}

SNPN-TAIforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SNPN-BasedMDT::= SEQUENCE {
    sNPNListforMDT      SNPNListforMDT,
    iE-Extensions       ProtocolExtensionContainer { {SNPN-BasedMDT-ExtIEs} } OPTIONAL,
    ...
}

SNPN-BasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SNPNListforMDT ::= SEQUENCE (SIZE(1.. maxnoofMDTSNPNs)) OF SNPNforMDT-Item

SNPNforMDT-Item ::= SEQUENCE {
    plmn-ID             PLMN-Identity,
    nID                 NID,
    iE-Extensions       ProtocolExtensionContainer {{SNPNforMDT-Item-ExtIEs}}       OPTIONAL,
    ...
}

SNPNforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SubcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ..., kHz60}

SSBInformation ::= SEQUENCE (SIZE(1.. maxNoSSBs)) OF SSBInformation-Item

SSBInformation-Item ::= SEQUENCE {
    sSBConfig                               SSBConfig,
    pCI                                     NRPCI,
    iE-Extensions                           ProtocolExtensionContainer { { SSBInformation-Item-ExtIEs} } OPTIONAL,
    ...
}

SSBInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
 

SSBConfig ::= SEQUENCE {
    sSB-freqInfo                        SSB-freqInfo,
    sSB-subcarrierSpacing               SSB-subcarrierSpacing,
    sSB-Transmit-power                  INTEGER (-60..50),
    sSB-periodicity                     ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},
    sSB-half-frame-offset               INTEGER(0..1),
    sSB-SFN-offset                      INTEGER(0..15),
    sSB-position-in-burst               SSB-PositionsInBurst        OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { SSBConfig-ExtIEs} } OPTIONAL,
    ...
}

SSBConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SliceToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReportForDataCollection-List-Item

SliceToReportForDataCollection-List-Item    ::= SEQUENCE {
    pLMNIdentity                PLMN-Identity,
    sNSSAIlist                  SNSSAI-list,
    iE-Extensions               ProtocolExtensionContainer { { SliceToReportForDataCollection-List-Item-ExtIEs} }   OPTIONAL,
    ...
}

SliceToReportForDataCollection-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SliceMeasurementInitiationResult::= SEQUENCE {
    sliceMeasurementInitiationResult-List       SliceMeasurementInitiationResult-List,
    iE-Extensions                   ProtocolExtensionContainer { {SliceMeasurementInitiationResult-ExtIEs} } OPTIONAL,
    ...
}

SliceMeasurementInitiationResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SliceMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceMeasurementInitiationResult-Item

SliceMeasurementInitiationResult-Item ::= SEQUENCE {
    pLMNIdentity                                    PLMN-Identity,
    sNSSAIMeasurementInitiationResult-List          SNSSAIMeasurementInitiationResult-List,
    iE-Extensions                                   ProtocolExtensionContainer { { SliceMeasurementInitiationResult-Item-ExtIEs} }  OPTIONAL,
    ...
}

SliceMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SNSSAIMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SNSSAIMeasurementInitiationResult-Item

SNSSAIMeasurementInitiationResult-Item ::= SEQUENCE {
    sNSSAI                              S-NSSAI,
    sNSSAIMeasurementFailureCause-List      SNSSAIMeasurementFailureCause-List      OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { {SNSSAIMeasurementInitiationResult-Item-ExtIEs} }  OPTIONAL,
    ...
}

SNSSAIMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


SNSSAIMeasurementFailureCause-List ::=SEQUENCE (SIZE(1..maxnoofFailedSliceMeasObjects)) OF SNSSAIMeasurementFailureCause-Item

SNSSAIMeasurementFailureCause-Item ::= SEQUENCE {
    sNSSAIMeasurementFailedReportCharacteristics    BIT STRING(SIZE(32)),
    cause                                           Cause,
    iE-Extensions                                   ProtocolExtensionContainer { {SNSSAIMeasurementFailureCause-Item-ExtIEs} }      OPTIONAL,
    ...
}

SNSSAIMeasurementFailureCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SliceUEPerformance ::= SEQUENCE {
    pLMNIdentity                                PLMN-Identity,
    s-NSSAIUEPerformance-List                   S-NSSAIUEPerformance-List,
    iE-Extensions                               ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,
    ...
}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

S-NSSAIUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAIUEPerformance-Item

S-NSSAIUEPerformance-Item   ::= SEQUENCE {
    sNSSAI                              S-NSSAI,
    sliceBasedUEPerformance             SliceBasedUEPerformance,
    iE-Extensions                       ProtocolExtensionContainer { { S-NSSAIUEPerformance-Item-ExtIEs} }  OPTIONAL,
    ...
}


S-NSSAIUEPerformance-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SliceBasedUEPerformance ::= SEQUENCE {
    sliceAverageUEThroughputDL          BitRate     OPTIONAL,
    sliceAverageUEThroughputUL          BitRate     OPTIONAL,
    sliceAverageUEPacketDelay           AveragePacketDelay                  OPTIONAL,
    sliceAveragePacketDropDL            INTEGER(0..1000000, ...)            OPTIONAL,
    sliceAveragePacketLossUL            INTEGER(0..1000000, ...)            OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { SliceBasedUEPerformance-ExtIEs} } OPTIONAL,
    ...
}

SliceBasedUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}
SemipersistentPositioningInformation ::= SEQUENCE {
    sRSTransmissionType                 ENUMERATED {activate, deactivate, ...},
    sRSResourceSetID                    OCTET STRING,
    sRSSpatialRelation                  OCTET STRING        OPTIONAL,
    spatialRelationInforperSRSResource  OCTET STRING        OPTIONAL,
    iE-Extensions                       ProtocolExtensionContainer { { SemipersistentPositioningInformation-ExtIEs} } OPTIONAL,
    ...
}

SemipersistentPositioningInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


-- T

TABasedMDT ::= SEQUENCE {
    tAListforMDT        TAListforMDT,
    iE-Extensions       ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,
    ...
}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Tci-State-InformationList   ::=  SEQUENCE (SIZE(1.. maxnoofLTM-CSI-ResourcesPerSet)) OF Tci-State-Information-Item

Tci-State-Information-Item  ::= SEQUENCE {
    jointorDLTCIStateID         JointorDLTCIStateID,
    iE-Extensions               ProtocolExtensionContainer { { Tci-State-Information-Item-ExtIEs } }    OPTIONAL,
    ...
}

Tci-State-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



TAIBasedMDT ::= SEQUENCE {
    tAIListforMDT           TAIListforMDT,
    iE-Extensions           ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,
    ...
}

TAIBasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAIforMDT-Item

TAIforMDT-Item ::= SEQUENCE {
    plmn-ID             PLMN-Identity,
    tAC                     TAC,
    iE-Extensions           ProtocolExtensionContainer { {TAIforMDT-Item-ExtIEs} } OPTIONAL,
    ...
}

TAIforMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TAC ::= OCTET STRING (SIZE (3))

TAINSAGSupportList ::= SEQUENCE (SIZE(1..maxnoofNSAGs)) OF TAINSAGSupportItem

TAINSAGSupportItem ::= SEQUENCE {
    nSAG-ID                             NSAG-ID,
    nSAGSliceSupportList            ExtendedSliceSupportList,
    iE-Extensions       ProtocolExtensionContainer { {TAINSAGSupportItem-ExtIEs} } OPTIONAL,
    ...
}

TAINSAGSupportItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TAISliceUnavailableCellList ::= SEQUENCE (SIZE(1..maxnoofExtSliceItems)) OF TAISliceUnavailableCellItem

TAISliceUnavailableCellItem ::= SEQUENCE {
    sNSSAI                  S-NSSAI,
    sliceAvailabilityList   SliceAvailabilityList,
    iE-Extensions           ProtocolExtensionContainer { {TAISliceUnavailableCellItem-ExtIEs} } OPTIONAL,
    ...
}

TAISliceUnavailableCellItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

SliceAvailabilityList ::= CHOICE {  
    unavailableCellList         UnavailableCellList,
    availableCellList           AvailableCellList,
    choice-extension        ProtocolIE-Single-Container { {SliceAvailabilityList-ExtIEs} },
    ...
}

SliceAvailabilityList-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


AvailableCellList ::= SEQUENCE {
    availableNRCellList                 AvailableNRCellList,
    iE-Extensions           ProtocolExtensionContainer { {AvailableCellList-ExtIEs} }   OPTIONAL,
    ...
}

AvailableCellList-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

AvailableNRCellList ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI

UnavailableCellList ::= SEQUENCE {
    unavailableNRCellList                   UnavailableNRCellList,
    iE-Extensions           ProtocolExtensionContainer { {UnavailableCellList-ExtIEs} } OPTIONAL,
    ...
}

UnavailableCellList-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UnavailableNRCellList ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF NR-CGI


TAISupport-List ::= SEQUENCE (SIZE(1..maxnoofsupportedTACs)) OF TAISupport-Item

TAISupport-Item ::= SEQUENCE {
    tac                             TAC,
    broadcastPLMNs                  SEQUENCE (SIZE(1..maxnoofsupportedPLMNs)) OF BroadcastPLMNinTAISupport-Item,
    iE-Extensions                   ProtocolExtensionContainer { {TAISupport-Item-ExtIEs} } OPTIONAL,
    ...
}

TAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC


TABasedQMC ::= SEQUENCE {
    tAListforQMC        TAListforQMC,
    iE-Extensions       ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,
    ...
}

TABasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC


TAIBasedQMC ::= SEQUENCE {
    tAIListforQMC       TAIListforQMC,
    iE-Extensions       ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,
    ...
}

TAIBasedQMC-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item


TAI-Item ::= SEQUENCE {
    tAC                 TAC,
    pLMN-Identity       PLMN-Identity,
    iE-Extensions       ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,
    ...
}

TAI-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TargetCellinEUTRAN ::= OCTET STRING -- This IE is to be encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31]



Target-CGI ::= CHOICE {
    nr                          NR-CGI,
    e-utra                      E-UTRA-CGI,
    choice-extension            ProtocolIE-Single-Container { {TargetCGI-ExtIEs} }
}

TargetCGI-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


TDDULDLConfigurationCommonNR ::= OCTET STRING


TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {
    target-cell                             Target-CGI,
    iE-Extensions                           ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL
}

TargetCellList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

Threshold-RSRQ ::= INTEGER(0..127)
Threshold-RSRP ::= INTEGER(0..127)
Threshold-SINR ::= INTEGER(0..127)

TimeSCG-Failure ::= INTEGER (0..1023)

TimeSinceFailure ::= INTEGER (0..172800, ...)

TimeSynchronizationAssistanceInformation ::= SEQUENCE {
    timeDistributionIndication                  ENUMERATED {enabled, disabled, ...},
    uuTimeSynchronizationErrorBudget            INTEGER (0..1000000, ...)               OPTIONAL,
    -- This IE shall be present if the Time Distribution Indication IE is set to “enabled”.
    ie-Extension                                ProtocolExtensionContainer { { TimeSynchronizationAssistanceInformation-ExtIEs} } OPTIONAL,
    ... 
}

TimeSynchronizationAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

    { ID id-ClockQualityReportingControlInfo    CRITICALITY ignore  EXTENSION ClockQualityReportingControlInfo      PRESENCE optional},
    ...
}

TimeToTrigger ::= ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}


TimeToWait ::= ENUMERATED {
    v1s,
    v2s,
    v5s,
    v10s,
    v20s,
    v60s,
    ...
}


TMGI ::= OCTET STRING (SIZE(6))


TNLConfigurationInfo ::= SEQUENCE {
    extendedUPTransportLayerAddressesToAdd          ExtTLAs                                     OPTIONAL,
    extendedUPTransportLayerAddressesToRemove       ExtTLAs                                     OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {TNLConfigurationInfo-ExtIEs} }    OPTIONAL,
    ...
}

TNLConfigurationInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TNLA-To-Add-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Add-Item

TNLA-To-Add-Item ::= SEQUENCE {
    tNLAssociationTransportLayerAddress     CPTransportLayerInformation,
    tNLAssociationUsage                     TNLAssociationUsage,
    iE-Extensions                           ProtocolExtensionContainer { { TNLA-To-Add-Item-ExtIEs} } OPTIONAL
}

TNLA-To-Add-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TNLA-To-Update-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Update-Item

TNLA-To-Update-Item::= SEQUENCE {
    tNLAssociationTransportLayerAddress     CPTransportLayerInformation,
    tNLAssociationUsage                     TNLAssociationUsage     OPTIONAL,
    iE-Extensions                           ProtocolExtensionContainer { { TNLA-To-Update-Item-ExtIEs} } OPTIONAL
}

TNLA-To-Update-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TNLA-To-Remove-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-To-Remove-Item

TNLA-To-Remove-Item::= SEQUENCE {
    tNLAssociationTransportLayerAddress     CPTransportLayerInformation,
    iE-Extensions                           ProtocolExtensionContainer { { TNLA-To-Remove-Item-ExtIEs} } OPTIONAL
}

TNLA-To-Remove-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TNLA-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Setup-Item

TNLA-Setup-Item ::= SEQUENCE {
    tNLAssociationTransportLayerAddress     CPTransportLayerInformation,
    iE-Extensions                           ProtocolExtensionContainer { { TNLA-Setup-Item-ExtIEs} } OPTIONAL,
    ...
}

TNLA-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TNLA-Failed-To-Setup-List ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLA-Failed-To-Setup-Item

TNLA-Failed-To-Setup-Item ::= SEQUENCE {
    tNLAssociationTransportLayerAddress     CPTransportLayerInformation,
    cause                                   Cause,
    iE-Extensions                           ProtocolExtensionContainer { { TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL
}

TNLA-Failed-To-Setup-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


TNLAssociationUsage ::= ENUMERATED {
    ue,
    non-ue,
    both,
    ...
}


TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))


TraceActivation ::= SEQUENCE {
    ng-ran-TraceID          NG-RANTraceID,
    interfaces-to-trace     BIT STRING { ng-c (0), x-nc (1), uu (2), f1-c (3), e1 (4)} (SIZE(8)),
    trace-depth             Trace-Depth,
    trace-coll-address      TransportLayerAddress,
    ie-Extension            ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,
    ...
}

TraceActivation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
-- Extension to support MDT –
    { ID id-TraceCollectionEntityURI    CRITICALITY ignore  EXTENSION URIaddress                PRESENCE optional}|
    { ID id-MDT-Configuration           CRITICALITY ignore  EXTENSION MDT-Configuration         PRESENCE optional},
    ...
}


Trace-Depth ::= ENUMERATED {
    minimum,
    medium,
    maximum,
    minimumWithoutVendorSpecificExtension,
    mediumWithoutVendorSpecificExtension,
    maximumWithoutVendorSpecificExtension,
    ...,
    minimumOnlyVendorSpecificTraceRecord,
    mediumOnlyVendorSpecificTraceRecord,
    maximumOnlyVendorSpecificTraceRecord
}

TrafficIndex ::= INTEGER (1..1024, ...)

TrafficProfile ::= CHOICE {
    uPTraffic                   QoSFlowLevelQoSParameters,
    nonUPTraffic                NonUPTraffic,
    choice-extension            ProtocolIE-Single-Container { {TrafficProfile-ExtIEs} }
}

TrafficProfile-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

TrafficReleaseType ::= CHOICE {
    fullRelease             AllTrafficIndication,
    partialRelease          TrafficToBeRelease-List,
    choice-extension        ProtocolIE-Single-Container { {TrafficReleaseType-ExtIEs} }
}

TrafficReleaseType-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


TrafficToBeReleaseInformation ::= SEQUENCE {
    releaseType         TrafficReleaseType,
    ie-Extensions           ProtocolExtensionContainer { {TrafficToBeReleaseInformation-ExtIEs} } OPTIONAL,
    ...
}

TrafficToBeReleaseInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TrafficToBeRelease-List ::= SEQUENCE (SIZE(1..maxnoofTrafficIndexEntries)) OF TrafficToBeRelease-Item

TrafficToBeRelease-Item ::= SEQUENCE {
    trafficIndex            TrafficIndex,
    bHInfoList              BHInfoList      OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {TrafficToBeRelease-Item-ExtIEs} } OPTIONAL,
    ...
}

TrafficToBeRelease-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TSCTrafficCharacteristics ::= SEQUENCE {
    tSCAssistanceInformationDownlink    TSCAssistanceInformation OPTIONAL,
    tSCAssistanceInformationUplink      TSCAssistanceInformation OPTIONAL,
    ie-Extension            ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} } OPTIONAL,
    ...
}

TSCTrafficCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

TSCAssistanceInformation ::= SEQUENCE {
    periodicity         INTEGER (0.. 640000, ...),
    burstArrivalTime    OCTET STRING                OPTIONAL,
    ie-Extension            ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs} } OPTIONAL,
    ... 
}

TSCAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {   ID id-SurvivalTime                      CRITICALITY ignore  EXTENSION SurvivalTime                          PRESENCE optional}|
    {   ID id-CapabilityForBATAdaptation        CRITICALITY ignore  EXTENSION CapabilityForBATAdaptation        PRESENCE optional}|
    {   ID id-N6JitterInformation               CRITICALITY ignore  EXTENSION N6JitterInformation                   PRESENCE optional},
    ...
}


TypeOfError ::= ENUMERATED {
    not-understood,
    missing,
    ...
}


TAValue ::= INTEGER (0..4095)

TagIDPointer ::= OCTET STRING


TAInformation-List ::= SEQUENCE (SIZE(1..maxnoofTAList)) OF TAInformation-Item


TAInformation-Item ::= SEQUENCE {
    earlyRACHResourcesRequesterID   EarlyRACHResourcesRequesterID,
    candidateCellID                 NR-CGI,
    tAValue                         TAValue,
    preambleIndex                   PreambleIndex,
    rA-RNTI                         RA-RNTI,    tagIDPointer                    TagIDPointer    OPTIONAL,
    ie-Extension                    ProtocolExtensionContainer { { TAInformation-Item-ExtIEs} } OPTIONAL,
    ... 
}

TAInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}



-- U


UEAggregateMaximumBitRate ::= SEQUENCE {
    dl-UE-AMBR              BitRate,
    ul-UE-AMBR              BitRate,
    iE-Extension            ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,
    ...
}

UEAggregateMaximumBitRate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UEAppLayerMeasConfigInfo ::= SEQUENCE {
    qOEReference                    QOEReference,
    qOEMeasConfigAppLayerID         QOEMeasConfAppLayerID               OPTIONAL,
    serviceType                     ServiceType,
    qOEMeasStatus                   QOEMeasStatus                       OPTIONAL,
    containerAppLayerMeasConfig ContainerAppLayerMeasConfig             OPTIONAL,
    mDTAlignmentInfo                MDTAlignmentInfo                    OPTIONAL,
    measCollectionEntityIPAddress   MeasCollectionEntityIPAddress       OPTIONAL,
    areaScopeOfQMC                  AreaScopeOfQMC                      OPTIONAL,
    s-NSSAIListQoE                  S-NSSAIListQoE                      OPTIONAL,
    availableRVQoEMetrics           AvailableRVQoEMetrics               OPTIONAL,
    iE-Extension                    ProtocolExtensionContainer { {UEAppLayerMeasConfigInfo-ExtIEs} } OPTIONAL,
    ...
}

UEAppLayerMeasConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-MBSCommServiceType              CRITICALITY ignore  EXTENSION MBSCommServiceType                            PRESENCE optional }|
    { ID id-AssistanceInformationQoE-Meas   CRITICALITY ignore  EXTENSION AssistanceInformationQoE-Meas             PRESENCE optional }|
    { ID id-QoERVQoEReportingPaths          CRITICALITY ignore  EXTENSION QoERVQoEReportingPaths                        PRESENCE optional },
    ...
}


UEContextKeptIndicator ::= ENUMERATED {true, ...}


UEContextID ::= CHOICE {
    rRCResume               UEContextIDforRRCResume,
    rRRCReestablishment     UEContextIDforRRCReestablishment,
    choice-extension        ProtocolIE-Single-Container { {UEContextID-ExtIEs} }
}

UEContextID-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


UEContextIDforRRCResume ::= SEQUENCE {
    i-rnti                  I-RNTI,
    allocated-c-rnti            C-RNTI,
    accessPCI               NG-RAN-CellPCI,
    iE-Extension            ProtocolExtensionContainer { {UEContextIDforRRCResume-ExtIEs} } OPTIONAL,
    ...
}

UEContextIDforRRCResume-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UEContextIDforRRCReestablishment ::= SEQUENCE {
    c-rnti                  C-RNTI,
    failureCellPCI          NG-RAN-CellPCI,
    iE-Extension            ProtocolExtensionContainer { {UEContextIDforRRCReestablishment-ExtIEs} } OPTIONAL,
    ...
}

UEContextIDforRRCReestablishment-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UEContextInfoRetrUECtxtResp ::= SEQUENCE {
    ng-c-UE-signalling-ref                  AMF-UE-NGAP-ID,
    signalling-TNL-at-source                CPTransportLayerInformation,
    ueSecurityCapabilities                  UESecurityCapabilities,
    securityInformation                     AS-SecurityInformation,
    ue-AMBR                                 UEAggregateMaximumBitRate,
    pduSessionResourcesToBeSetup-List       PDUSessionResourcesToBeSetup-List,
    rrc-Context                             OCTET STRING,
    mobilityRestrictionList                 MobilityRestrictionList                                 OPTIONAL,
    indexToRatFrequencySelectionPriority    RFSP-Index                                              OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {UEContextInfoRetrUECtxtResp-ExtIEs} }     OPTIONAL,
    ...
}

UEContextInfoRetrUECtxtResp-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-FiveGCMobilityRestrictionListContainer  CRITICALITY ignore  EXTENSION FiveGCMobilityRestrictionListContainer        PRESENCE optional }|
    { ID id-NRUESidelinkAggregateMaximumBitRate     CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-LTEUESidelinkAggregateMaximumBitRate    CRITICALITY ignore  EXTENSION LTEUESidelinkAggregateMaximumBitRate          PRESENCE optional }|
    { ID id-UERadioCapabilityID                     CRITICALITY reject  EXTENSION UERadioCapabilityID                               PRESENCE optional }|
    { ID id-MBS-SessionInformation-List             CRITICALITY ignore  EXTENSION MBS-SessionInformation-List                       PRESENCE optional }|
    { ID id-NoPDUSessionIndication                  CRITICALITY ignore  EXTENSION NoPDUSessionIndication                            PRESENCE optional }|
    { ID id-FiveGProSeUEPC5AggregateMaximumBitRate  CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-UESliceMaximumBitRateList               CRITICALITY ignore  EXTENSION UESliceMaximumBitRateList                     PRESENCE optional }|
    { ID id-PositioningInformation                  CRITICALITY ignore  EXTENSION PositioningInformation                            PRESENCE optional }|
    { ID id-NRA2XUEPC5AggregateMaximumBitRate       CRITICALITY ignore  EXTENSION NRUESidelinkAggregateMaximumBitRate           PRESENCE optional }|
    { ID id-LTEA2XUEPC5AggregateMaximumBitRate      CRITICALITY ignore  EXTENSION LTEUESidelinkAggregateMaximumBitRate          PRESENCE optional }|
    { ID id-NRPPaPositioningInformation             CRITICALITY ignore  EXTENSION NRPPaPositioningInformation                       PRESENCE optional }|
    { ID id-SemipersistentPositioningInformation    CRITICALITY ignore  EXTENSION SemipersistentPositioningInformation          PRESENCE optional },
    ...
}


UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCell-Item


UEHistoryInformationFromTheUE ::= CHOICE {
    nR                      NRMobilityHistoryReport,
    choice-extension            ProtocolIE-Single-Container { {UEHistoryInformationFromTheUE-ExtIEs} }
}

UEHistoryInformationFromTheUE-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


UEIdentityIndexValue ::= CHOICE {
    indexLength10               BIT STRING (SIZE(10)),
    choice-extension            ProtocolIE-Single-Container { {UEIdentityIndexValue-ExtIEs} }
}

UEIdentityIndexValue-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}

UEIdentityIndexList-MBSGroupPaging ::= SEQUENCE (SIZE(1..maxnoofUEIDIndicesforMBSPaging)) OF UEIdentityIndexList-MBSGroupPaging-Item

UEIdentityIndexList-MBSGroupPaging-Item ::= SEQUENCE {
    ueIdentityIndexList-MBSGroupPagingValue     UEIdentityIndexList-MBSGroupPagingValue,
    pagingDRX                                   UESpecificDRX       OPTIONAL,
    iE-Extension            ProtocolExtensionContainer { {UEIdentityIndexList-MBSGroupPaging-Item-ExtIEs} }     OPTIONAL,
    ...
}

UEIdentityIndexList-MBSGroupPaging-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UEIdentityIndexList-MBSGroupPagingValue ::= CHOICE {
    uEIdentityIndexValueMBSGroupPaging      BIT STRING (SIZE(10)),
    choice-extension            ProtocolIE-Single-Container { {UEIdentityIndexValueMBSGroupPaging-ExtIEs} }
}

UEIdentityIndexValueMBSGroupPaging-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


UERadioCapabilityForPaging ::= SEQUENCE {
    uERadioCapabilityForPagingOfNR          UERadioCapabilityForPagingOfNR          OPTIONAL,
    uERadioCapabilityForPagingOfEUTRA       UERadioCapabilityForPagingOfEUTRA       OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }  OPTIONAL,
    ...
}

UERadioCapabilityForPaging-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERadioCapabilityID ::= OCTET STRING

UERANPagingIdentity ::= CHOICE {
    i-RNTI-full         BIT STRING ( SIZE (40)),
    choice-extension    ProtocolIE-Single-Container { {UERANPagingIdentity-ExtIEs} }
}

UERANPagingIdentity-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


UERLFReportContainer ::= CHOICE {
    nR-UERLFReportContainer         UERLFReportContainerNR,
    lTE-UERLFReportContainer        UERLFReportContainerLTE,
    choice-Extension        ProtocolIE-Single-Container { {UERLFReportContainer-ExtIEs} }
}
UERLFReportContainer-ExtIEs XNAP-PROTOCOL-IES ::= {
    {ID id-UERLFReportContainerLTEExtension CRITICALITY ignore  TYPE UERLFReportContainerLTEExtension       PRESENCE mandatory},
    ...
}

UERLFReportContainerLTE ::= OCTET STRING
-- This IE is a transparent container and includes the rlf-Report-r9 contained in the UEInformationResponse message as defined in TS 36.331 [14].


UERLFReportContainerLTEExtension ::= SEQUENCE {
    ueRLFReportContainerLTE                 UERLFReportContainerLTE,
    ueRLFReportContainerLTEExtendBand       UERLFReportContainerLTEExtendBand,
    iE-Extensions                   ProtocolExtensionContainer { { UERLFReportContainerLTEExtension-ExtIEs} } OPTIONAL,
    ...
}


UERLFReportContainerLTEExtendBand ::= OCTET STRING
-- This IE is a transparent container and includes the rlf-Report-v9e0 contained in the UEInformationResponse message as defined in TS 36.331 [14].


UERLFReportContainerLTEExtension-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


UERLFReportContainerNR ::= OCTET STRING
-- This IE is a transparent container and includes the nr-RLF-Report IE contained in the UEInformationResponse message as defined in TS 38.331 [10].


UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1.. maxnoofSMBR)) OF UESliceMaximumBitRate-Item

UESliceMaximumBitRate-Item ::= SEQUENCE {
    s-NSSAI                     S-NSSAI,
    dl-UE-Slice-MBR             BitRate,
    ul-UE-Slice-MBR             BitRate,
    iE-Extensions                       ProtocolExtensionContainer { { UESliceMaximumBitRate-Item-ExtIEs} } OPTIONAL,
    ...
}

UESliceMaximumBitRate-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UESecurityCapabilities ::= SEQUENCE {
    nr-EncyptionAlgorithms                  BIT STRING {nea1-128(1),
                                                        nea2-128(2),
                                                        nea3-128(3)} (SIZE(16, ...)),
    nr-IntegrityProtectionAlgorithms        BIT STRING {nia1-128(1),
                                                        nia2-128(2),
                                                        nia3-128(3)} (SIZE(16, ...)),
    e-utra-EncyptionAlgorithms              BIT STRING {eea1-128(1),
                                                        eea2-128(2),
                                                        eea3-128(3)} (SIZE(16, ...)),
    e-utra-IntegrityProtectionAlgorithms    BIT STRING {eia1-128(1),
                                                        eia2-128(2),
                                                        eia3-128(3)} (SIZE(16, ...)),
    iE-Extension            ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} } OPTIONAL,
    ...
}

UESecurityCapabilities-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UESpecificDRX ::= ENUMERATED {
    v32,
    v64,
    v128,
    v256,
    ...
}

ULConfiguration::= SEQUENCE {
    uL-PDCP                         UL-UE-Configuration,
    iE-Extensions                   ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,
    ...
}

ULConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UL-UE-Configuration::= ENUMERATED {no-data, shared, only, ...}

ULF1Terminating-BHInfo ::= SEQUENCE {
    ingressBAPRoutingID             BAPRoutingID,
    ingressBHRLCCHID                BHRLCChannelID,
    iE-Extensions       ProtocolExtensionContainer { { ULF1Terminating-BHInfo-ExtIEs} } OPTIONAL,
    ...
}

ULF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ULNonF1Terminating-BHInfo ::= SEQUENCE {
    egressBAPRoutingID          BAPRoutingID,
    egressBHRLCCHID             BHRLCChannelID,
    nexthopBAPAddress           BAPAddress,
    iE-Extensions       ProtocolExtensionContainer { { ULNonF1Terminating-BHInfo-ExtIEs} } OPTIONAL,
    ...
}

ULNonF1Terminating-BHInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}


ULForwarding    ::= ENUMERATED {ul-forwarding-proposed, ...}

ULForwardingProposal    ::= ENUMERATED {ul-forwarding-proposed, ...}


UL-GBR-PRB-usage::= INTEGER (0..100)


UL-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-non-GBR-PRB-usage::= INTEGER (0..100)


UL-non-GBR-PRB-usage-for-MIMO::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)


UL-Total-PRB-usage-for-MIMO::= INTEGER (0..100)


UPTransportLayerInformation ::= CHOICE {
    gtpTunnel                   GTPtunnelTransportLayerInformation,
    choice-extension            ProtocolIE-Single-Container { {UPTransportLayerInformation-ExtIEs} }
}

UPTransportLayerInformation-ExtIEs XNAP-PROTOCOL-IES ::= {
    ...
}


UPTransportParameters ::= SEQUENCE (SIZE(1..maxnoofSCellGroupsplus1)) OF UPTransportParametersItem

UPTransportParametersItem ::= SEQUENCE {
    upTNLInfo       UPTransportLayerInformation,
    cellGroupID     CellGroupID,
    iE-Extension    ProtocolExtensionContainer { {UPTransportParametersItem-ExtIEs} } OPTIONAL,
    ...
}

UPTransportParametersItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
        ...
}

UserPlaneErrorIndicator ::= ENUMERATED {gtpu-error-indication-received, ...}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

UserPlaneFailureIndication ::= SEQUENCE {
    userPlaneFailureType        UserPlaneFailureType,
    dL-NG-U-TNLatNG-RAN         UPTransportLayerInformation,
    uL-NG-U-TNLatNG-RAN         UPTransportLayerInformation,
    iE-Extensions       ProtocolExtensionContainer {{ UserPlaneFailureIndication-ExtIEs} }  OPTIONAL,
    ...
}

UserPlaneFailureIndication-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UserPlaneFailureType ::= ENUMERATED {
    gtp-u-error-indication-received,
    up-path-failure,
    ...
}

URIaddress ::= VisibleString

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {
    uEAssistantIdentifier                       NG-RANnodeUEXnAPID, 
    uEPerformance                               UEPerformance           OPTIONAL,
    measuredUETrajectory                        MeasuredUETrajectory    OPTIONAL,
    iE-Extensions                               ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,
    ...
}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    { ID id-SliceUEPerformance  CRITICALITY ignore  EXTENSION SliceUEPerformance PRESENCE optional },
    ...
}

UEPerformance ::= SEQUENCE {
    dL-UE-AverageThroughput                     BitRate                 OPTIONAL,
    uL-UE-AverageThroughput                     BitRate                 OPTIONAL,
    uE-AveragePacketDelay                       AveragePacketDelay      OPTIONAL,
    uE-AveragePacketDropDL                  INTEGER(0..1000000, ...)            OPTIONAL,
    iE-Extensions                               ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,
    ...
}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    {ID id-UEAveragePacketLossUL        CRITICALITY ignore  EXTENSION UEAveragePacketLossUL         PRESENCE optional },
    ...
}

UEAveragePacketLossUL ::= INTEGER (0..1000000,...)

UEPerformanceCollectionConfiguration ::= SEQUENCE {
    collectionTimeDurationForUEPerformance              INTEGER(1..5000, ...),
    iE-Extensions                               ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,
    ...
}

UEPerformanceCollectionConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

UETrajectoryCollectionConfiguration ::= SEQUENCE {
    collectionTimeDurationForUETrajectory                               INTEGER (1..4096, ...),
    numberOfVisitedCells                                INTEGER (1..16, ...)                            OPTIONAL,
    iE-Extensions                   ProtocolExtensionContainer { { UETrajectoryCollectionConfiguration-ExtIEs} }        OPTIONAL,
    ...
}

UETrajectoryCollectionConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

ULTCIStateID  ::= OCTET STRING

-- V

VehicleUE ::= ENUMERATED {
    authorized,
    not-authorized,
    ...
}

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {
    startTimeStamp                  OCTET STRING (SIZE(4)),
    endTimeStamp                    OCTET STRING (SIZE(4)),
    usageCountUL                    INTEGER (0..18446744073709551615),
    usageCountDL                    INTEGER (0..18446744073709551615),
    iE-Extensions               ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,
...
}

VolumeTimedReport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

-- W

WAB-MT-ID ::= SEQUENCE {
    nrCGI                       NR-CGI,
    cRNTI                       BIT STRING (SIZE(16)),
    iE-Extensions               ProtocolExtensionContainer { { WAB-MT-ID-ExtIEs } } OPTIONAL,
    ...
}

WAB-MT-ID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

WLANMeasurementConfiguration ::= SEQUENCE {
    wlanMeasConfig              WLANMeasConfig,
    wlanMeasConfigNameList      WLANMeasConfigNameList              OPTIONAL,
    wlan-rssi                   ENUMERATED {true, ...}              OPTIONAL,
    wlan-rtt                    ENUMERATED {true, ...}              OPTIONAL,
    iE-Extensions       ProtocolExtensionContainer { { WLANMeasurementConfiguration-ExtIEs } } OPTIONAL,
    ...
}

WLANMeasurementConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {
    ...
}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))


-- X

XnBenefitValue ::= INTEGER (1..8, ...)

XR-Bcast-Information ::= ENUMERATED {true,...}


-- Y


-- Z


END
-- ASN1STOP

-- 9.3.6    Common definitions
-- ASN1START
-- **************************************************************
--
-- Common definitions
--
-- **************************************************************

XnAP-CommonDataTypes {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-access (22) modules (3) xnap (2) version1 (1) xnap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************
--
-- Extension constants
--
-- **************************************************************

maxPrivateIEs                                   INTEGER ::= 65535
maxProtocolExtensions                           INTEGER ::= 65535
maxProtocolIEs                                  INTEGER ::= 65535

-- **************************************************************
--
-- Common Data Types
--
-- **************************************************************

Criticality     ::= ENUMERATED { reject, ignore, notify }

Presence        ::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID    ::= CHOICE {
    local               INTEGER (0.. maxPrivateIEs),
    global              OBJECT IDENTIFIER
}

ProcedureCode       ::= INTEGER (0..255)


ProtocolIE-ID       ::= INTEGER (0..maxProtocolIEs)


TriggeringMessage   ::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END
-- ASN1STOP

-- 9.3.7    Constant definitions
-- ASN1START
-- **************************************************************
--
-- Constant definitions
--
-- **************************************************************

XnAP-Constants {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS
    ProcedureCode,
    ProtocolIE-ID
FROM XnAP-CommonDataTypes;

-- **************************************************************
--
-- Elementary Procedures
--
-- **************************************************************

id-handoverPreparation                                          ProcedureCode ::= 0
id-sNStatusTransfer                                             ProcedureCode ::= 1
id-handoverCancel                                               ProcedureCode ::= 2
id-retrieveUEContext                                            ProcedureCode ::= 3
id-rANPaging                                                    ProcedureCode ::= 4
id-xnUAddressIndication                                         ProcedureCode ::= 5
id-uEContextRelease                                             ProcedureCode ::= 6
id-sNGRANnodeAdditionPreparation                                ProcedureCode ::= 7
id-sNGRANnodeReconfigurationCompletion                          ProcedureCode ::= 8
id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation         ProcedureCode ::= 9
id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation         ProcedureCode ::= 10
id-mNGRANnodeinitiatedSNGRANnodeRelease                         ProcedureCode ::= 11
id-sNGRANnodeinitiatedSNGRANnodeRelease                         ProcedureCode ::= 12
id-sNGRANnodeCounterCheck                                       ProcedureCode ::= 13
id-sNGRANnodeChange                                             ProcedureCode ::= 14
id-rRCTransfer                                                  ProcedureCode ::= 15
id-xnRemoval                                                    ProcedureCode ::= 16
id-xnSetup                                                      ProcedureCode ::= 17
id-nGRANnodeConfigurationUpdate                                 ProcedureCode ::= 18
id-cellActivation                                               ProcedureCode ::= 19
id-reset                                                        ProcedureCode ::= 20
id-errorIndication                                              ProcedureCode ::= 21
id-privateMessage                                               ProcedureCode ::= 22
id-notificationControl                                          ProcedureCode ::= 23
id-activityNotification                                         ProcedureCode ::= 24
id-e-UTRA-NR-CellResourceCoordination                           ProcedureCode ::= 25
id-secondaryRATDataUsageReport                                  ProcedureCode ::= 26
id-deactivateTrace                                              ProcedureCode ::= 27
id-traceStart                                                   ProcedureCode ::= 28
id-handoverSuccess                                              ProcedureCode ::= 29
id-conditionalHandoverCancel                                    ProcedureCode ::= 30
id-earlyStatusTransfer                                          ProcedureCode ::= 31
id-failureIndication                                            ProcedureCode ::= 32
id-handoverReport                                               ProcedureCode ::= 33
id-resourceStatusReportingInitiation                            ProcedureCode ::= 34
id-resourceStatusReporting                                      ProcedureCode ::= 35
id-mobilitySettingsChange                                       ProcedureCode ::= 36
id-accessAndMobilityIndication                                  ProcedureCode ::= 37
id-cellTrafficTrace                                             ProcedureCode ::= 38
id-RANMulticastGroupPaging                                      ProcedureCode ::= 39
id-scgFailureInformationReport                                  ProcedureCode ::= 40
id-ProcedureCode41-NotToBeUsed                                  ProcedureCode ::= 41
id-scgFailureTransfer                                           ProcedureCode ::= 42
id-f1CTrafficTransfer                                           ProcedureCode ::= 43
id-iABTransportMigrationManagement                              ProcedureCode ::= 44
id-iABTransportMigrationModification                            ProcedureCode ::= 45
id-iABResourceCoordination                                      ProcedureCode ::= 46
id-retrieveUEContextConfirm                                     ProcedureCode ::= 47
id-cPCCancel                                                    ProcedureCode ::= 48
id-partialUEContextTransfer                                     ProcedureCode ::= 49
id-rachIndication                                               ProcedureCode ::= 50
id-dataCollectionReportingInitiation                            ProcedureCode ::= 51
id-dataCollectionReporting                                      ProcedureCode ::= 52
id-ODSIB1ConfigurationProvision                                 ProcedureCode ::= 53
id-ODSIB1ConfigurationProvisionStatusUpdate                     ProcedureCode ::= 54
id-lTMConfigurationUpdate                                       ProcedureCode ::= 55
id-cSIRSCoordination                                            ProcedureCode ::= 56
id-cellSwitchNotification                                       ProcedureCode ::= 57
id-tAInformationTransfer                                        ProcedureCode ::= 58
id-lTMCancel                                                    ProcedureCode ::= 59
id-cLI-Indication                                               ProcedureCode ::= 60
id-scgFailureIndication                                         ProcedureCode ::= 61



-- **************************************************************
--
-- Lists
--
-- **************************************************************

maxEARFCN                                   INTEGER ::= 262143
maxnoofAllowedAreas                         INTEGER ::= 16
maxnoofAMFRegions                           INTEGER ::= 16
maxnoofAoIs                                 INTEGER ::= 64
maxnoofBluetoothName                        INTEGER ::= 4
maxnoofBPLMNs                               INTEGER ::= 12
maxnoofCAGs                                 INTEGER ::= 12
maxnoofCAGsperPLMN                          INTEGER ::= 256
maxnoofCellIDforMDT                         INTEGER ::= 32
maxnoofCellsinAoI                           INTEGER ::= 256
maxnoofCellsinUEHistoryInfo                 INTEGER ::= 16
maxnoofCellsinNG-RANnode                    INTEGER ::= 16384
maxnoofCellsinRNA                           INTEGER ::= 32
maxnoofCellsUEMovingTrajectory              INTEGER ::= 16
maxnoofDRBs                                 INTEGER ::= 32
maxnoofEUTRABands                           INTEGER ::= 16
maxnoofEUTRABPLMNs                          INTEGER ::= 6
maxnoofEPLMNs                               INTEGER ::= 15
maxnoofExtSliceItems                        INTEGER ::= 65535
maxnoofEPLMNsplus1                          INTEGER ::= 16
maxnoofForbiddenTACs                        INTEGER ::= 4096
maxnoofFreqforMDT                           INTEGER ::= 8
maxnoofMBSFNEUTRA                           INTEGER ::= 8
maxnoofMDTPLMNs                             INTEGER ::= 16
maxnoofMultiConnectivityMinusOne            INTEGER ::= 3
maxnoofNeighbours                           INTEGER ::= 1024
maxnoofNeighPCIforMDT                       INTEGER ::= 32
maxnoofNIDs                                 INTEGER ::= 12
maxnoofNRCellBands                          INTEGER ::= 32
maxnoofPLMNs                                INTEGER ::= 16
maxnoofPDUSessions                          INTEGER ::= 256
maxnoofProtectedResourcePatterns            INTEGER ::= 16
maxnoofQoSFlows                             INTEGER ::= 64
maxnoofQoSParaSets                          INTEGER ::= 8
maxnoofRANAreaCodes                         INTEGER ::= 32
maxnoofRANAreasinRNA                        INTEGER ::= 16
maxnoofRANNodesinAoI                        INTEGER ::= 64
maxnoofSCellGroups                          INTEGER ::= 3
maxnoofSCellGroupsplus1                     INTEGER ::= 4
maxnoofSensorName                           INTEGER ::= 3
maxnoofSliceItems                           INTEGER ::= 1024
maxnoofSNPNIDs                              INTEGER ::= 12
maxnoofsupportedPLMNs                       INTEGER ::= 12
maxnoofsupportedTACs                        INTEGER ::= 256
maxnoofTAforMDT                             INTEGER ::= 8
maxnoofTAI                                  INTEGER ::= 16
maxnoofTAIsinAoI                            INTEGER ::= 16
maxnooftimeperiods                          INTEGER ::= 2
maxnoofTNLAssociations                      INTEGER ::= 32
maxnoofUEContexts                           INTEGER ::= 8192
maxNRARFCN                                  INTEGER ::= 3279165
maxNrOfErrors                               INTEGER ::= 256
maxnoofslots                                INTEGER ::= 5120
maxnoofExtTLAs                              INTEGER ::= 16
maxnoofGTPTLAs                              INTEGER ::= 16
maxnoofCHOcells                             INTEGER ::= 8
maxnoofPC5QoSFlows                          INTEGER ::= 2064
maxnoofSSBAreas                             INTEGER ::= 64
maxnoofRAReports                            INTEGER ::= 64
maxnoofNRSCSs                               INTEGER ::= 5
maxnoofPhysicalResourceBlocks               INTEGER ::= 275
maxnoofAdditionalPDCPDuplicationTNL         INTEGER ::= 2
maxnoofRLCDuplicationstate                  INTEGER ::= 3
maxnoofWLANName                             INTEGER ::= 4
maxnoofNonAnchorCarrierFreqConfig           INTEGER ::= 15
maxnoofDataForwardingTunneltoE-UTRAN        INTEGER ::= 256
maxnoofMBSFSAs                              INTEGER ::= 256
maxnoofUEIDIndicesforMBSPaging              INTEGER ::= 4096
maxnoofMBSQoSFlows                          INTEGER ::= 64
maxnoofMRBs                                 INTEGER ::= 32
maxnoofCellsforMBS                          INTEGER ::= 8192
maxnoofMBSServiceAreaInformation            INTEGER ::= 256
maxnoofTAIforMBS                            INTEGER ::= 1024
maxnoofAssociatedMBSSessions                INTEGER ::= 32
maxnoofMBSSessions                          INTEGER ::= 256
maxnoofSuccessfulHOReports                  INTEGER ::= 64
maxnoofPSCellsPerSN                         INTEGER ::= 8
maxnoofNR-UChannelIDs                       INTEGER ::= 16
maxnoofCellsinCHO                           INTEGER ::= 8
maxnoofCHOexecutioncond                     INTEGER ::= 2
maxnoofServedCellsIAB                       INTEGER ::= 512
maxnoofServingCells                         INTEGER ::= 32
maxnoofBHInfo                               INTEGER ::= 1024
maxnoofTrafficIndexEntries                  INTEGER ::= 1024
maxnoofTLAsIAB                              INTEGER ::= 1024
maxnoofBAPControlPDURLCCHs                  INTEGER ::= 2
maxnoofIABSTCInfo                           INTEGER ::= 45
maxnoofSymbols                              INTEGER ::= 14
maxnoofDUFSlots                             INTEGER ::= 320
maxnoofHSNASlots                            INTEGER ::= 5120
maxnoofRBsetsPerCell                        INTEGER ::= 8
maxnoofRBsetsPerCell1                       INTEGER ::= 7
maxnoofChildIABNodes                        INTEGER ::= 1024
maxnoofPSCellCandidates                     INTEGER ::= 8
maxnoofTargetSNs                            INTEGER ::= 8
maxnoofUEAppLayerMeas                       INTEGER ::= 16
maxnoofSNSSAIforQMC                         INTEGER ::= 16
maxnoofCellIDforQMC                         INTEGER ::= 32
maxnoofPLMNforQMC                           INTEGER ::= 16
maxnoofTAforQMC                             INTEGER ::= 8
maxnoofMTCItems                             INTEGER ::= 16
maxnoofCSIRSconfigurations                  INTEGER ::= 96
maxnoofCSIRSneighbourCells                  INTEGER ::= 16
maxnoofCSIRSneighbourCellsInMTC             INTEGER ::= 16
maxnoofNeighbour-NG-RAN-Nodes               INTEGER ::= 256
maxnoofSRBs                                 INTEGER ::= 5
maxnoofSMBR                                 INTEGER ::= 8
maxnoofNSAGs                                INTEGER ::= 256
maxnoofTargetSNsMinusOne                    INTEGER ::= 7
maxnoofThresholdsForExcessPacketDelay       INTEGER ::= 255
maxnoofESNPNs                               INTEGER ::= 15
maxnoofSuccessfulPSCellChangeReports        INTEGER ::= 64
maxnoofUEsforRAReportIndications            INTEGER ::= 64
maxnoofPSCellsinCPAC                        INTEGER ::= 8
maxnoofCPACexecutioncond                    INTEGER ::= 2
maxnoofLBTFailureInformation                INTEGER ::= 64
maxnoofCellsTrajectoryPredict               INTEGER ::= 16
maxnoofCellsTrajectory                      INTEGER ::= 16
maxFailedCellMeasObjects                    INTEGER ::= 124
maxFailedMeasPerNode                        INTEGER ::= 124
maxnoofUEReports                            INTEGER ::= 16
maxnoofCandidateRelayUEs                    INTEGER ::= 32
maxnoofCAGforMDT                            INTEGER ::= 256
maxnoofMDTSNPNs                             INTEGER ::= 16
maxnoofSecurityConfigurations               INTEGER ::= 8
maxnoofRSPPQoSFlows                         INTEGER ::= 2048
maxnoofThresholds                           INTEGER ::= 8
maxnoofEarlyRACHResourcesID                 INTEGER ::= 8
maxnoofLTMCells                             INTEGER ::= 8
maxNoSSBs                                   INTEGER ::= 255
maxnoofTAList                               INTEGER ::= 8
maxnoofPreambleIndexes                      INTEGER ::= 64
maxnoofCSIResourceConfigurations            INTEGER ::= 112
maxnoofNZP-CSI-RS-ResourcesPerSet           INTEGER ::= 64
maxnoofSRS-Resource                         INTEGER ::= 64
maxnoofFailedSliceMeasObjects               INTEGER ::= 124
maxnoofSliceItemsforMDT                     INTEGER ::= 1024
maxnoofAreaNTNforMDT                        INTEGER ::= 32
maxnoofLTMCellsPlusOne                      INTEGER ::= 9
maxnoofSCGSecurityConfigurations            INTEGER ::= 8
maxnoofLTM-CSI-ResourcesPerSet              INTEGER ::= 512





-- **************************************************************
--
-- IEs
--
-- **************************************************************

id-ActivatedServedCells                                                                             ProtocolIE-ID ::= 0
id-ActivationIDforCellActivation                                                                    ProtocolIE-ID ::= 1
id-admittedSplitSRB                                                                                 ProtocolIE-ID ::= 2
id-admittedSplitSRBrelease                                                                          ProtocolIE-ID ::= 3
id-AMF-Region-Information                                                                           ProtocolIE-ID ::= 4
id-AssistanceDataForRANPaging                                                                       ProtocolIE-ID ::= 5
id-BearersSubjectToCounterCheck                                                                     ProtocolIE-ID ::= 6
id-Cause                                                                                            ProtocolIE-ID ::= 7
id-cellAssistanceInfo-NR                                                                            ProtocolIE-ID ::= 8
id-ConfigurationUpdateInitiatingNodeChoice                                                          ProtocolIE-ID ::= 9
id-CriticalityDiagnostics                                                                           ProtocolIE-ID ::= 10
id-XnUAddressInfoperPDUSession-List                                                                 ProtocolIE-ID ::= 11
id-DRBsSubjectToStatusTransfer-List                                                                 ProtocolIE-ID ::= 12
id-ExpectedUEBehaviour                                                                              ProtocolIE-ID ::= 13
id-GlobalNG-RAN-node-ID                                                                             ProtocolIE-ID ::= 14
id-GUAMI                                                                                            ProtocolIE-ID ::= 15
id-indexToRatFrequSelectionPriority                                                                 ProtocolIE-ID ::= 16
id-initiatingNodeType-ResourceCoordRequest                                                          ProtocolIE-ID ::= 17
id-List-of-served-cells-E-UTRA                                                                      ProtocolIE-ID ::= 18
id-List-of-served-cells-NR                                                                          ProtocolIE-ID ::= 19
id-LocationReportingInformation                                                                     ProtocolIE-ID ::= 20
id-MAC-I                                                                                            ProtocolIE-ID ::= 21
id-MaskedIMEISV                                                                                     ProtocolIE-ID ::= 22
id-M-NG-RANnodeUEXnAPID                                                                             ProtocolIE-ID ::= 23
id-MN-to-SN-Container                                                                               ProtocolIE-ID ::= 24
id-MobilityRestrictionList                                                                          ProtocolIE-ID ::= 25
id-new-NG-RAN-Cell-Identity                                                                         ProtocolIE-ID ::= 26
id-newNG-RANnodeUEXnAPID                                                                            ProtocolIE-ID ::= 27
id-UEReportRRCTransfer                                                                              ProtocolIE-ID ::= 28
id-oldNG-RANnodeUEXnAPID                                                                            ProtocolIE-ID ::= 29
id-OldtoNewNG-RANnodeResumeContainer                                                                ProtocolIE-ID ::= 30
id-PagingDRX                                                                                        ProtocolIE-ID ::= 31
id-PCellID                                                                                          ProtocolIE-ID ::= 32
id-PDCPChangeIndication                                                                             ProtocolIE-ID ::= 33
id-PDUSessionAdmittedAddedAddReqAck                                                                 ProtocolIE-ID ::= 34
id-PDUSessionAdmittedModSNModConfirm                                                                ProtocolIE-ID ::= 35
id-PDUSessionAdmitted-SNModResponse                                                                 ProtocolIE-ID ::= 36
id-PDUSessionNotAdmittedAddReqAck                                                                   ProtocolIE-ID ::= 37
id-PDUSessionNotAdmitted-SNModResponse                                                              ProtocolIE-ID ::= 38
id-PDUSessionReleasedList-RelConf                                                                   ProtocolIE-ID ::= 39
id-PDUSessionReleasedSNModConfirm                                                                   ProtocolIE-ID ::= 40
id-PDUSessionResourcesActivityNotifyList                                                            ProtocolIE-ID ::= 41
id-PDUSessionResourcesAdmitted-List                                                                 ProtocolIE-ID ::= 42
id-PDUSessionResourcesNotAdmitted-List                                                              ProtocolIE-ID ::= 43
id-PDUSessionResourcesNotifyList                                                                    ProtocolIE-ID ::= 44
id-PDUSession-SNChangeConfirm-List                                                                  ProtocolIE-ID ::= 45
id-PDUSession-SNChangeRequired-List                                                                 ProtocolIE-ID ::= 46
id-PDUSessionToBeAddedAddReq                                                                        ProtocolIE-ID ::= 47
id-PDUSessionToBeModifiedSNModRequired                                                              ProtocolIE-ID ::= 48
id-PDUSessionToBeReleasedList-RelRqd                                                                ProtocolIE-ID ::= 49
id-PDUSessionToBeReleased-RelReq                                                                    ProtocolIE-ID ::= 50
id-PDUSessionToBeReleasedSNModRequired                                                              ProtocolIE-ID ::= 51
id-RANPagingArea                                                                                    ProtocolIE-ID ::= 52
id-PagingPriority                                                                                   ProtocolIE-ID ::= 53
id-requestedSplitSRB                                                                                ProtocolIE-ID ::= 54
id-requestedSplitSRBrelease                                                                         ProtocolIE-ID ::= 55
id-ResetRequestTypeInfo                                                                             ProtocolIE-ID ::= 56
id-ResetResponseTypeInfo                                                                            ProtocolIE-ID ::= 57
id-RespondingNodeTypeConfigUpdateAck                                                                ProtocolIE-ID ::= 58
id-respondingNodeType-ResourceCoordResponse                                                         ProtocolIE-ID ::= 59
id-ResponseInfo-ReconfCompl                                                                         ProtocolIE-ID ::= 60
id-RRCConfigIndication                                                                              ProtocolIE-ID ::= 61
id-RRCResumeCause                                                                                   ProtocolIE-ID ::= 62
id-SCGConfigurationQuery                                                                            ProtocolIE-ID ::= 63
id-selectedPLMN                                                                                     ProtocolIE-ID ::= 64
id-ServedCellsToActivate                                                                            ProtocolIE-ID ::= 65
id-servedCellsToUpdate-E-UTRA                                                                       ProtocolIE-ID ::= 66
id-ServedCellsToUpdateInitiatingNodeChoice                                                          ProtocolIE-ID ::= 67
id-servedCellsToUpdate-NR                                                                           ProtocolIE-ID ::= 68
id-s-ng-RANnode-SecurityKey                                                                         ProtocolIE-ID ::= 69
id-S-NG-RANnodeUE-AMBR                                                                              ProtocolIE-ID ::= 70
id-S-NG-RANnodeUEXnAPID                                                                             ProtocolIE-ID ::= 71
id-SN-to-MN-Container                                                                               ProtocolIE-ID ::= 72
id-sourceNG-RANnodeUEXnAPID                                                                         ProtocolIE-ID ::= 73
id-SplitSRB-RRCTransfer                                                                             ProtocolIE-ID ::= 74
id-TAISupport-list                                                                                  ProtocolIE-ID ::= 75
id-TimeToWait                                                                                       ProtocolIE-ID ::= 76
id-Target2SourceNG-RANnodeTranspContainer                                                           ProtocolIE-ID ::= 77
id-targetCellGlobalID                                                                               ProtocolIE-ID ::= 78
id-targetNG-RANnodeUEXnAPID                                                                         ProtocolIE-ID ::= 79
id-target-S-NG-RANnodeID                                                                            ProtocolIE-ID ::= 80
id-TraceActivation                                                                                  ProtocolIE-ID ::= 81
id-UEContextID                                                                                      ProtocolIE-ID ::= 82
id-UEContextInfoHORequest                                                                           ProtocolIE-ID ::= 83
id-UEContextInfoRetrUECtxtResp                                                                      ProtocolIE-ID ::= 84
id-UEContextInfo-SNModRequest                                                                       ProtocolIE-ID ::= 85
id-UEContextKeptIndicator                                                                           ProtocolIE-ID ::= 86
id-UEContextRefAtSN-HORequest                                                                       ProtocolIE-ID ::= 87
id-UEHistoryInformation                                                                             ProtocolIE-ID ::= 88
id-UEIdentityIndexValue                                                                             ProtocolIE-ID ::= 89
id-UERANPagingIdentity                                                                              ProtocolIE-ID ::= 90
id-UESecurityCapabilities                                                                           ProtocolIE-ID ::= 91
id-UserPlaneTrafficActivityReport                                                                   ProtocolIE-ID ::= 92
id-XnRemovalThreshold                                                                               ProtocolIE-ID ::= 93
id-DesiredActNotificationLevel                                                                      ProtocolIE-ID ::= 94
id-AvailableDRBIDs                                                                                  ProtocolIE-ID ::= 95
id-AdditionalDRBIDs                                                                                 ProtocolIE-ID ::= 96
id-SpareDRBIDs                                                                                      ProtocolIE-ID ::= 97
id-RequiredNumberOfDRBIDs                                                                           ProtocolIE-ID ::= 98
id-TNLA-To-Add-List                                                                                 ProtocolIE-ID ::= 99
id-TNLA-To-Update-List                                                                              ProtocolIE-ID ::= 100
id-TNLA-To-Remove-List                                                                              ProtocolIE-ID ::= 101
id-TNLA-Setup-List                                                                                  ProtocolIE-ID ::= 102
id-TNLA-Failed-To-Setup-List                                                                        ProtocolIE-ID ::= 103
id-PDUSessionToBeReleased-RelReqAck                                                                 ProtocolIE-ID ::= 104
id-S-NG-RANnodeMaxIPDataRate-UL                                                                     ProtocolIE-ID ::= 105
id-PDUSessionResourceSecondaryRATUsageList                                                          ProtocolIE-ID ::= 107
id-Additional-UL-NG-U-TNLatUPF-List                                                                 ProtocolIE-ID ::= 108
id-SecondarydataForwardingInfoFromTarget-List                                                       ProtocolIE-ID ::= 109
id-LocationInformationSNReporting                                                                   ProtocolIE-ID ::= 110
id-LocationInformationSN                                                                            ProtocolIE-ID ::= 111
id-LastE-UTRANPLMNIdentity                                                                          ProtocolIE-ID ::= 112
id-S-NG-RANnodeMaxIPDataRate-DL                                                                     ProtocolIE-ID ::= 113
id-MaxIPrate-DL                                                                                     ProtocolIE-ID ::= 114
id-SecurityResult                                                                                   ProtocolIE-ID ::= 115
id-S-NSSAI                                                                                          ProtocolIE-ID ::= 116
id-MR-DC-ResourceCoordinationInfo                                                                   ProtocolIE-ID ::= 117
id-AMF-Region-Information-To-Add                                                                    ProtocolIE-ID ::= 118
id-AMF-Region-Information-To-Delete                                                                 ProtocolIE-ID ::= 119
id-OldQoSFlowMap-ULendmarkerexpected                                                                ProtocolIE-ID ::= 120
id-RANPagingFailure                                                                                 ProtocolIE-ID ::= 121
id-UERadioCapabilityForPaging                                                                       ProtocolIE-ID ::= 122
id-PDUSessionDataForwarding-SNModResponse                                                           ProtocolIE-ID ::= 123
id-DRBsNotAdmittedSetupModifyList                                                                   ProtocolIE-ID ::= 124
id-Secondary-MN-Xn-U-TNLInfoatM                                                                     ProtocolIE-ID ::= 125
id-NE-DC-TDM-Pattern                                                                                ProtocolIE-ID ::= 126
id-PDUSessionCommonNetworkInstance                                                                  ProtocolIE-ID ::= 127
id-BPLMN-ID-Info-EUTRA                                                                              ProtocolIE-ID ::= 128
id-BPLMN-ID-Info-NR                                                                                 ProtocolIE-ID ::= 129
id-InterfaceInstanceIndication                                                                      ProtocolIE-ID ::= 130
id-S-NG-RANnode-Addition-Trigger-Ind                                                                ProtocolIE-ID ::= 131
id-DefaultDRB-Allowed                                                                               ProtocolIE-ID ::= 132
id-DRB-IDs-takenintouse                                                                             ProtocolIE-ID ::= 133
id-SplitSessionIndicator                                                                            ProtocolIE-ID ::= 134
id-CNTypeRestrictionsForEquivalent                                                                  ProtocolIE-ID ::= 135
id-CNTypeRestrictionsForServing                                                                     ProtocolIE-ID ::= 136
id-DRBs-transferred-to-MN                                                                           ProtocolIE-ID ::= 137
id-ULForwardingProposal                                                                             ProtocolIE-ID ::= 138
id-EndpointIPAddressAndPort                                                                         ProtocolIE-ID ::= 139
id-IntendedTDD-DL-ULConfiguration-NR                                                                ProtocolIE-ID ::= 140
id-TNLConfigurationInfo                                                                             ProtocolIE-ID ::= 141
id-PartialListIndicator-NR                                                                          ProtocolIE-ID ::= 142
id-MessageOversizeNotification                                                                      ProtocolIE-ID ::= 143
id-CellAndCapacityAssistanceInfo-NR                                                                 ProtocolIE-ID ::= 144
id-NG-RANTraceID                                                                                    ProtocolIE-ID ::= 145
id-NonGBRResources-Offered                                                                          ProtocolIE-ID ::= 146
id-FastMCGRecoveryRRCTransfer-SN-to-MN                                                              ProtocolIE-ID ::= 147
id-RequestedFastMCGRecoveryViaSRB3                                                                  ProtocolIE-ID ::= 148
id-AvailableFastMCGRecoveryViaSRB3                                                                  ProtocolIE-ID ::= 149
id-RequestedFastMCGRecoveryViaSRB3Release                                                           ProtocolIE-ID ::= 150
id-ReleaseFastMCGRecoveryViaSRB3                                                                    ProtocolIE-ID ::= 151
id-FastMCGRecoveryRRCTransfer-MN-to-SN                                                              ProtocolIE-ID ::= 152
id-ExtendedRATRestrictionInformation                                                                ProtocolIE-ID ::= 153
id-QoSMonitoringRequest                                                                             ProtocolIE-ID ::= 154
id-FiveGCMobilityRestrictionListContainer                                                           ProtocolIE-ID ::= 155
id-PartialListIndicator-EUTRA                                                                       ProtocolIE-ID ::= 156
id-CellAndCapacityAssistanceInfo-EUTRA                                                              ProtocolIE-ID ::= 157
id-CHOinformation-Req                                                                               ProtocolIE-ID ::= 158
id-CHOinformation-Ack                                                                               ProtocolIE-ID ::= 159
id-targetCellsToCancel                                                                              ProtocolIE-ID ::= 160
id-requestedTargetCellGlobalID                                                                      ProtocolIE-ID ::= 161
id-procedureStage                                                                                   ProtocolIE-ID ::= 162
id-DAPSRequestInfo                                                                                  ProtocolIE-ID ::= 163
id-DAPSResponseInfo-List                                                                            ProtocolIE-ID ::= 164
id-CHO-MRDC-Indicator                                                                               ProtocolIE-ID ::= 165
id-OffsetOfNbiotChannelNumberToDL-EARFCN                                                            ProtocolIE-ID ::= 166
id-OffsetOfNbiotChannelNumberToUL-EARFCN                                                            ProtocolIE-ID ::= 167
id-NBIoT-UL-DL-AlignmentOffset                                                                      ProtocolIE-ID ::= 168
id-LTEV2XServicesAuthorized                                                                         ProtocolIE-ID ::= 169
id-NRV2XServicesAuthorized                                                                          ProtocolIE-ID ::= 170
id-LTEUESidelinkAggregateMaximumBitRate                                                             ProtocolIE-ID ::= 171
id-NRUESidelinkAggregateMaximumBitRate                                                              ProtocolIE-ID ::= 172
id-PC5QoSParameters                                                                                 ProtocolIE-ID ::= 173
id-AlternativeQoSParaSetList                                                                        ProtocolIE-ID ::= 174
id-CurrentQoSParaSetIndex                                                                           ProtocolIE-ID ::= 175
id-MobilityInformation                                                                              ProtocolIE-ID ::= 176
id-InitiatingCondition-FailureIndication                                                            ProtocolIE-ID ::= 177
id-UEHistoryInformationFromTheUE                                                                    ProtocolIE-ID ::= 178
id-HandoverReportType                                                                               ProtocolIE-ID ::= 179
id-HandoverCause                                                                                    ProtocolIE-ID ::= 180
id-SourceCellCGI                                                                                    ProtocolIE-ID ::= 181
id-TargetCellCGI                                                                                    ProtocolIE-ID ::= 182
id-ReEstablishmentCellCGI                                                                           ProtocolIE-ID ::= 183
id-TargetCellinEUTRAN                                                                               ProtocolIE-ID ::= 184
id-SourceCellCRNTI                                                                                  ProtocolIE-ID ::= 185
id-UERLFReportContainer                                                                             ProtocolIE-ID ::= 186
id-NGRAN-Node1-Measurement-ID                                                                       ProtocolIE-ID ::= 187
id-NGRAN-Node2-Measurement-ID                                                                       ProtocolIE-ID ::= 188
id-RegistrationRequest                                                                              ProtocolIE-ID ::= 189
id-ReportCharacteristics                                                                                ProtocolIE-ID ::= 190
id-CellToReport                                                                                             ProtocolIE-ID ::= 191
id-ReportingPeriodicity                                                                                 ProtocolIE-ID ::= 192
id-CellMeasurementResult                                                                                ProtocolIE-ID ::= 193
id-NG-RANnode1CellID                                                                                        ProtocolIE-ID ::= 194
id-NG-RANnode2CellID                                                                                ProtocolIE-ID ::= 195
id-NG-RANnode1MobilityParameters                                                                    ProtocolIE-ID ::= 196
id-NG-RANnode2ProposedMobilityParameters                                                            ProtocolIE-ID ::= 197
id-MobilityParametersModificationRange                                                              ProtocolIE-ID ::= 198
id-TDDULDLConfigurationCommonNR                                                                     ProtocolIE-ID ::= 199
id-CarrierList                                                                                      ProtocolIE-ID ::= 200
id-ULCarrierList                                                                                    ProtocolIE-ID ::= 201
id-FrequencyShift7p5khz                                                                             ProtocolIE-ID ::= 202
id-SSB-PositionsInBurst                                                                             ProtocolIE-ID ::= 203
id-NRCellPRACHConfig                                                                                ProtocolIE-ID ::= 204
id-RAReport                                                                                         ProtocolIE-ID ::= 205
id-IABNodeIndication                                                                                ProtocolIE-ID ::= 206
id-Redundant-UL-NG-U-TNLatUPF                                                                       ProtocolIE-ID ::= 207
id-CNPacketDelayBudgetDownlink                                                                      ProtocolIE-ID ::= 208
id-CNPacketDelayBudgetUplink                                                                        ProtocolIE-ID ::= 209
id-Additional-Redundant-UL-NG-U-TNLatUPF-List                                                       ProtocolIE-ID ::= 210
id-RedundantCommonNetworkInstance                                                                   ProtocolIE-ID ::= 211
id-TSCTrafficCharacteristics                                                                        ProtocolIE-ID ::= 212
id-RedundantQoSFlowIndicator                                                                        ProtocolIE-ID ::= 213
id-Redundant-DL-NG-U-TNLatNG-RAN                                                                    ProtocolIE-ID ::= 214
id-ExtendedPacketDelayBudget                                                                        ProtocolIE-ID ::= 215
id-Additional-PDCP-Duplication-TNL-List                                                             ProtocolIE-ID ::= 216
id-RedundantPDUSessionInformation                                                                   ProtocolIE-ID ::= 217
id-UsedRSNInformation                                                                               ProtocolIE-ID ::= 218
id-RLCDuplicationInformation                                                                        ProtocolIE-ID ::= 219
id-NPN-Broadcast-Information                                                                        ProtocolIE-ID ::= 220
id-NPNPagingAssistanceInformation                                                                   ProtocolIE-ID ::= 221
id-NPNMobilityInformation                                                                           ProtocolIE-ID ::= 222
id-NPN-Support                                                                                      ProtocolIE-ID ::= 223
id-MDT-Configuration                                                                                ProtocolIE-ID ::= 224
id-MDTPLMNList                                                                                      ProtocolIE-ID ::= 225
id-TraceCollectionEntityURI                                                                         ProtocolIE-ID ::= 226
id-UERadioCapabilityID                                                                              ProtocolIE-ID ::= 227
id-CSI-RSTransmissionIndication                                                                     ProtocolIE-ID ::= 228
id-SNTriggered                                                                                      ProtocolIE-ID ::= 229
id-DLCarrierList                                                                                    ProtocolIE-ID ::= 230
id-ExtendedTAISliceSupportList                                                                      ProtocolIE-ID ::= 231
id-cellAssistanceInfo-EUTRA                                                                         ProtocolIE-ID ::= 232
id-ConfiguredTACIndication                                                                          ProtocolIE-ID ::= 233
id-secondary-SN-UL-PDCP-UP-TNLInfo                                                                  ProtocolIE-ID ::= 234
id-pdcpDuplicationConfiguration                                                                     ProtocolIE-ID ::= 235
id-duplicationActivation                                                                            ProtocolIE-ID ::= 236
id-NPRACHConfiguration                                                                              ProtocolIE-ID ::= 237
id-QosMonitoringReportingFrequency                                                                  ProtocolIE-ID ::= 238
id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated                                                   ProtocolIE-ID ::= 239
id-DL-scheduling-PDCCH-CCE-usage                                                                    ProtocolIE-ID ::= 240
id-UL-scheduling-PDCCH-CCE-usage                                                                    ProtocolIE-ID ::= 241
id-SFN-Offset                                                                                       ProtocolIE-ID ::= 242
id-QoSMonitoringDisabled                                                                            ProtocolIE-ID ::= 243
id-ExtendedUEIdentityIndexValue                                                                     ProtocolIE-ID ::= 244
id-EUTRAPagingeDRXInformation                                                                       ProtocolIE-ID ::= 245
id-CHO-MRDC-EarlyDataForwarding                                                                     ProtocolIE-ID ::= 246
id-SCGIndicator                                                                                     ProtocolIE-ID ::= 247
id-UESpecificDRX                                                                                    ProtocolIE-ID ::= 248
id-PDUSessionExpectedUEActivityBehaviour                                                            ProtocolIE-ID ::= 249
id-QoS-Mapping-Information                                                                          ProtocolIE-ID ::= 250
id-AdditionLocationInformation                                                                      ProtocolIE-ID ::= 251
id-dataForwardingInfoFromTargetE-UTRANnode                                                          ProtocolIE-ID ::= 252
id-DirectForwardingPathAvailability                                                                 ProtocolIE-ID ::= 253
id-SourceNG-RAN-node-ID                                                                             ProtocolIE-ID ::= 254
id-SourceDLForwardingIPAddress                                                                      ProtocolIE-ID ::= 255
id-SourceNodeDLForwardingIPAddress                                                                  ProtocolIE-ID ::= 256
id-ExtendedReportIntervalMDT                                                                        ProtocolIE-ID ::= 257
id-SecurityIndication                                                                               ProtocolIE-ID ::= 258
id-RRCConnReestab-Indicator                                                                         ProtocolIE-ID ::= 259
id-TargetNodeID                                                                                     ProtocolIE-ID ::= 260
id-ManagementBasedMDTPLMNList                                                                       ProtocolIE-ID ::= 261
id-PrivacyIndicator                                                                                 ProtocolIE-ID ::= 262
id-TraceCollectionEntityIPAddress                                                                   ProtocolIE-ID ::= 263
id-M4ReportAmount                                                                                   ProtocolIE-ID ::= 264
id-M5ReportAmount                                                                                   ProtocolIE-ID ::= 265
id-M6ReportAmount                                                                                   ProtocolIE-ID ::= 266
id-M7ReportAmount                                                                                   ProtocolIE-ID ::= 267
id-BeamMeasurementIndicationM1                                                                      ProtocolIE-ID ::= 268
id-MBS-Session-ID                                                                                   ProtocolIE-ID ::= 269
id-UEIdentityIndexList-MBSGroupPaging                                                                   ProtocolIE-ID ::= 270
id-MulticastRANPagingArea                                                                           ProtocolIE-ID ::= 271
id-Supported-MBS-FSA-ID-List                                                                        ProtocolIE-ID ::= 272
id-MBS-SessionInformation-List                                                                      ProtocolIE-ID ::= 273
id-MBS-SessionInformationResponse-List                                                              ProtocolIE-ID ::= 274
id-MBS-SessionAssociatedInformation                                                                 ProtocolIE-ID ::= 275
id-SuccessfulHOReportInformation                                                                    ProtocolIE-ID ::= 276
id-SliceRadioResourceStatus-List                                                                    ProtocolIE-ID ::= 277
id-CompositeAvailableCapacitySupplementaryUplink                                                    ProtocolIE-ID ::= 278
id-SCGUEHistoryInformation                                                                          ProtocolIE-ID ::= 279
id-SSBOffsets-List                                                                                  ProtocolIE-ID ::= 280
id-NG-RANnode2SSBOffsetModificationRange                                                            ProtocolIE-ID ::= 281
id-Coverage-Modification-List                                                                       ProtocolIE-ID ::= 282
id-NR-U-Channel-List                                                                                ProtocolIE-ID ::= 283
id-SourcePSCellCGI                                                                                  ProtocolIE-ID ::= 284
id-FailedPSCellCGI                                                                                  ProtocolIE-ID ::= 285
id-SCGFailureReportContainer                                                                        ProtocolIE-ID ::= 286
id-SNMobilityInformation                                                                            ProtocolIE-ID ::= 287
id-SourcePSCellID                                                                                   ProtocolIE-ID ::= 288
id-SuitablePSCellCGI                                                                                ProtocolIE-ID ::= 289
id-PSCellChangeHistory                                                                              ProtocolIE-ID ::= 290
id-CHOConfiguration                                                                                 ProtocolIE-ID ::= 291
id-NR-U-ChannelInfo-List                                                                            ProtocolIE-ID ::= 292
id-PSCellHistoryInformationRetrieve                                                                 ProtocolIE-ID ::= 293
id-NG-RANnode2SSBOffsetsModificationRange                                                           ProtocolIE-ID ::= 294
id-MIMOPRBusageInformation                                                                          ProtocolIE-ID ::= 295
id-F1CTrafficContainer                                                                              ProtocolIE-ID ::= 296
id-IAB-MT-Cell-List                                                                                 ProtocolIE-ID ::= 297
id-NoPDUSessionIndication                                                                           ProtocolIE-ID ::= 298
id-IAB-TNL-Address-Request                                                                          ProtocolIE-ID ::= 299
id-IAB-TNL-Address-Response                                                                         ProtocolIE-ID ::= 300
id-TrafficToBeAddedList                                                                             ProtocolIE-ID ::= 301
id-TrafficToBeModifiedList                                                                          ProtocolIE-ID ::= 302
id-TrafficToBeReleaseInformation                                                                    ProtocolIE-ID ::= 303
id-TrafficAddedList                                                                                 ProtocolIE-ID ::= 304
id-TrafficModifiedList                                                                              ProtocolIE-ID ::= 305
id-TrafficNotAddedList                                                                              ProtocolIE-ID ::= 306
id-TrafficNotModifiedList                                                                           ProtocolIE-ID ::= 307
id-TrafficRequiredToBeModifiedList                                                                  ProtocolIE-ID ::= 308
id-TrafficRequiredModifiedList                                                                      ProtocolIE-ID ::= 309
id-TrafficReleasedList                                                                              ProtocolIE-ID ::= 310
id-IABTNLAddressToBeAdded                                                                           ProtocolIE-ID ::= 311
id-IABTNLAddressToBeReleasedList                                                                    ProtocolIE-ID ::= 312
id-nonF1-Terminating-IAB-DonorUEXnAPID                                                              ProtocolIE-ID ::= 313
id-F1-Terminating-IAB-DonorUEXnAPID                                                                 ProtocolIE-ID ::= 314
id-BoundaryNodeCellsList                                                                            ProtocolIE-ID ::= 315
id-ParentNodeCellsList                                                                              ProtocolIE-ID ::= 316
id-tdd-GNB-DU-Cell-Resource-Configuration                                                           ProtocolIE-ID ::= 317
id-UL-GNB-DU-Cell-Resource-Configuration                                                            ProtocolIE-ID ::= 318
id-DL-GNB-DU-Cell-Resource-Configuration                                                            ProtocolIE-ID ::= 319
id-permutation                                                                                      ProtocolIE-ID ::= 320
id-IABTNLAddressException                                                                           ProtocolIE-ID ::= 321
id-CHOinformation-AddReq                                                                            ProtocolIE-ID ::= 322
id-CHOinformation-ModReq                                                                            ProtocolIE-ID ::= 323
id-SurvivalTime                                                                                     ProtocolIE-ID ::= 324
id-TimeSynchronizationAssistanceInformation                                                         ProtocolIE-ID ::= 325
id-SCGActivationRequest                                                                             ProtocolIE-ID ::= 326
id-SCGActivationStatus                                                                              ProtocolIE-ID ::= 327
id-CPAInformationRequest                                                                            ProtocolIE-ID ::= 328
id-CPAInformationAck                                                                                ProtocolIE-ID ::= 329
id-CPCInformationRequired                                                                           ProtocolIE-ID ::= 330
id-CPCInformationConfirm                                                                            ProtocolIE-ID ::= 331
id-CPAInformationModReq                                                                             ProtocolIE-ID ::= 332
id-CPAInformationModReqAck                                                                          ProtocolIE-ID ::= 333
id-CPC-DataForwarding-Indicator                                                                     ProtocolIE-ID ::= 334
id-CPCInformationUpdate                                                                             ProtocolIE-ID ::= 335
id-CPACInformationModRequired                                                                       ProtocolIE-ID ::= 336
id-QMCConfigInfo                                                                                    ProtocolIE-ID ::= 337
id-ProtocolIE-ID338-NotToBeUsed                                                                     ProtocolIE-ID ::= 338
id-Additional-Measurement-Timing-Configuration-List                                                 ProtocolIE-ID ::= 339
id-PDUSession-PairID                                                                                ProtocolIE-ID ::= 340
id-Local-NG-RAN-Node-Identifier                                                                     ProtocolIE-ID ::= 341
id-Neighbour-NG-RAN-Node-List                                                                       ProtocolIE-ID ::= 342
id-Local-NG-RAN-Node-Identifier-Removal                                                             ProtocolIE-ID ::= 343
id-FiveGProSeAuthorized                                                                             ProtocolIE-ID ::= 344
id-FiveGProSePC5QoSParameters                                                                       ProtocolIE-ID ::= 345
id-FiveGProSeUEPC5AggregateMaximumBitRate                                                           ProtocolIE-ID ::= 346
id-ServedCellSpecificInfoReq-NR                                                                     ProtocolIE-ID ::= 347
id-NRPagingeDRXInformation                                                                          ProtocolIE-ID ::= 348
id-NRPagingeDRXInformationforRRCINACTIVE                                                            ProtocolIE-ID ::= 349
id-Redcap-Bcast-Information                                                                         ProtocolIE-ID ::= 350
id-SDTSupportRequest                                                                                ProtocolIE-ID ::= 351
id-SDT-SRB-between-NewNode-OldNode                                                                  ProtocolIE-ID ::= 352
id-SDT-Termination-Request                                                                          ProtocolIE-ID ::= 353
id-SDTPartialUEContextInfo                                                                          ProtocolIE-ID ::= 354
id-SDTDataForwardingDRBList                                                                         ProtocolIE-ID ::= 355
id-PagingCause                                                                                      ProtocolIE-ID ::= 356
id-PEIPSassistanceInformation                                                                       ProtocolIE-ID ::= 357
id-UESliceMaximumBitRateList                                                                        ProtocolIE-ID ::= 358
id-S-NG-RANnodeUE-Slice-MBR                                                                         ProtocolIE-ID ::= 359
id-PositioningInformation                                                                           ProtocolIE-ID ::= 360
id-UEAssistantIdentifier                                                                            ProtocolIE-ID ::= 361
id-ManagementBasedMDTPLMNModificationList                                                           ProtocolIE-ID ::= 362
id-F1-terminatingIAB-donorIndicator                                                                 ProtocolIE-ID ::= 363
id-TAINSAGSupportList                                                                               ProtocolIE-ID ::= 364
id-SCGreconfigNotification                                                                          ProtocolIE-ID ::= 365
id-earlyMeasurement                                                                                 ProtocolIE-ID ::= 366
id-BeamMeasurementsReportConfiguration                                                              ProtocolIE-ID ::= 367
id-CoverageModificationCause                                                                        ProtocolIE-ID ::= 368
id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated                                 ProtocolIE-ID ::= 369
id-UERLFReportContainerLTEExtension                                                             ProtocolIE-ID ::= 370
id-ExcessPacketDelayThresholdConfiguration                                                          ProtocolIE-ID ::= 371
id-HashedUEIdentityIndexValue                                                                       ProtocolIE-ID ::= 372
id-QosFlowMappingIndication                                                                         ProtocolIE-ID ::= 373
id-Full-and-Short-I-RNTI-Profile-List                                                               ProtocolIE-ID ::= 374
id-MBS-DataForwarding-Indicator                                                                     ProtocolIE-ID ::= 375
id-IABAuthorizationStatus                                                                           ProtocolIE-ID ::= 376
id-EquivalentSNPNs                                                                                  ProtocolIE-ID ::= 377
id-SelectedNID                                                                                      ProtocolIE-ID ::= 378
id-MT-SDT-Information                                                                               ProtocolIE-ID ::= 379
id-PosPartialUEContextInfo                                                                          ProtocolIE-ID ::= 380
id-SRSConfiguration                                                                                 ProtocolIE-ID ::= 381
id-CHOTimeBasedInformation                                                                          ProtocolIE-ID ::= 382
id-ChannelOccupancyTimePercentageUL                                                                 ProtocolIE-ID ::= 383
id-EnergyDetectionThresholdUL                                                                       ProtocolIE-ID ::= 384
id-SuccessfulPSCellChangeReportInformation                                                          ProtocolIE-ID ::= 385
id-PSCellListContainer                                                                              ProtocolIE-ID ::= 386
id-RadioResourceStatusNR-U                                                                          ProtocolIE-ID ::= 387
id-CPACConfiguration                                                                                ProtocolIE-ID ::= 388
id-RaReportIndicationList                                                                           ProtocolIE-ID ::= 389
id-SPRAvailability                                                                                  ProtocolIE-ID ::= 390
id-DLLBTFailureInformationRequest                                                                   ProtocolIE-ID ::= 391
id-DLLBTFailureInformationList                                                                      ProtocolIE-ID ::= 392
id-TargetCellCRNTI                                                                                  ProtocolIE-ID ::= 393
id-TimeSinceFailure                                                                                 ProtocolIE-ID ::= 394
id-AerialUESubscriptionInformation                                                                  ProtocolIE-ID ::= 395
id-LTEA2XServicesAuthorized                                                                         ProtocolIE-ID ::= 396
id-NRA2XServicesAuthorized                                                                          ProtocolIE-ID ::= 397
id-LTEA2XUEPC5AggregateMaximumBitRate                                                               ProtocolIE-ID ::= 398
id-NRA2XUEPC5AggregateMaximumBitRate                                                                ProtocolIE-ID ::= 399
id-A2XPC5QoSParameters                                                                              ProtocolIE-ID ::= 400
id-CellBasedUETrajectoryPrediction                                                                  ProtocolIE-ID ::= 401
id-DataCollectionID                                                                                 ProtocolIE-ID ::= 402
id-RequestedPredictionTime                                                                          ProtocolIE-ID ::= 403
id-NodeMeasurementInitiationResult-List                                                             ProtocolIE-ID ::= 404
id-CellMeasurementInitiationResult-List                                                             ProtocolIE-ID ::= 405
id-UEAssociatedInfoResult-List                                                                      ProtocolIE-ID ::= 406
id-ProtocolIE-ID-407-not-to-be-used                                                                 ProtocolIE-ID ::= 407
id-UETrajectoryCollectionConfiguration                                                              ProtocolIE-ID ::= 408
id-UEPerformanceCollectionConfiguration                                                             ProtocolIE-ID ::= 409
id-CellMeasurementResultForDataCollection-List                                                      ProtocolIE-ID ::= 410
id-CellToReportForDataCollection-List                                                               ProtocolIE-ID ::= 411
id-FiveGProSeLayer2Multipath                                                                        ProtocolIE-ID ::= 412
id-FiveGProSeLayer2UEtoUERelay                                                                      ProtocolIE-ID ::= 413
id-FiveGProSeLayer2UEtoUERemote                                                                     ProtocolIE-ID ::= 414
id-CandidateRelayUEInfoList                                                                         ProtocolIE-ID ::= 415
id-NRCellsAndSSBsList                                                                               ProtocolIE-ID ::= 416
id-ActivatedNRCellsAndSSBsList                                                                      ProtocolIE-ID ::= 417
id-ClockQualityReportingControlInfo                                                                 ProtocolIE-ID ::= 418
id-CapabilityForBATAdaptation                                                                       ProtocolIE-ID ::= 419
id-PNI-NPN-AreaScopeofMDT                                                                           ProtocolIE-ID ::= 420
id-PNI-NPNBasedMDT                                                                                  ProtocolIE-ID ::= 421
id-SNPN-CellBasedMDT                                                                                ProtocolIE-ID ::= 422
id-SNPN-TAIBasedMDT                                                                                 ProtocolIE-ID ::= 423
id-SNPN-BasedMDT                                                                                    ProtocolIE-ID ::= 424
id-S-CPAC-Request                                                                                   ProtocolIE-ID ::= 425
id-S-CPAC-Request-Info                                                                              ProtocolIE-ID ::= 426
id-S-CPAC-ReferenceConfigRequest                                                                    ProtocolIE-ID ::= 427
id-S-CPAC-InterSN-ExecutionNotify                                                                   ProtocolIE-ID ::= 428
id-S-CPAC-dataforwardinginfofromSource                                                              ProtocolIE-ID ::= 429
id-CPACcandidatePSCells-wotherInfo-list                                                             ProtocolIE-ID ::= 430
id-eRedcap-Bcast-Information                                                                        ProtocolIE-ID ::= 431
id-NRPagingLongeDRXInformationforRRCINACTIVE                                                        ProtocolIE-ID ::= 432
id-MBS-AssistanceInformation                                                                        ProtocolIE-ID ::= 433
id-QMCCoordinationRequest                                                                           ProtocolIE-ID ::= 434
id-QMCCoordinationResponse                                                                          ProtocolIE-ID ::= 435
id-QoE-Measurement-Results                                                                          ProtocolIE-ID ::= 436
id-MBSCommServiceType                                                                               ProtocolIE-ID ::= 437
id-AssistanceInformationQoE-Meas                                                                    ProtocolIE-ID ::= 438
id-ProtocolIE-ID-439-not-to-be-used                                                                 ProtocolIE-ID ::= 439
id-QoERVQoEReportingPaths                                                                           ProtocolIE-ID ::= 440
id-Src-SN-to-Tgt-SNQMCInfoInquiry                                                                   ProtocolIE-ID ::= 441
id-DirectForwardingPathAvailabilityWithSourceMN                                                     ProtocolIE-ID ::= 442
id-CHO-Maxnoof-CondReconfig                                                                         ProtocolIE-ID ::= 443
id-accessed-PSCellID                                                                                ProtocolIE-ID ::= 444
id-conditional-Reconfig-ToCancel-List                                                               ProtocolIE-ID ::= 445
id-CHOinformation-AddReqAck                                                                         ProtocolIE-ID ::= 446
id-CHO-CPAC-Info                                                                                    ProtocolIE-ID ::= 447
id-PDUSetQoSParameters                                                                              ProtocolIE-ID ::= 448
id-N6JitterInformation                                                                              ProtocolIE-ID ::= 449
id-ECNMarkingorCongestionInformationReportingRequest                                                ProtocolIE-ID ::= 450
id-PDUSetbasedHandlingIndicator                                                                     ProtocolIE-ID ::= 451
id-TAISliceUnavailableCellList                                                                      ProtocolIE-ID ::= 452
id-MobileIAB-AuthorizationStatus                                                                    ProtocolIE-ID ::= 453
id-MIAB-MT-BAP-Address                                                                              ProtocolIE-ID ::= 454
id-MobileIABCell                                                                                    ProtocolIE-ID ::= 455
id-sk-Counter                                                                                       ProtocolIE-ID ::= 456
id-Source-M-NG-RANnodeID                                                                            ProtocolIE-ID ::= 457
id-ProtocolIE-ID458-NotToBeUsed                                                                     ProtocolIE-ID ::= 458
id-SourceSN-to-TargetSN-QMCInfo                                                                     ProtocolIE-ID ::= 459
id-RegistrationRequestForDataCollection                                                             ProtocolIE-ID ::= 460
id-ReportCharacteristicsForDataCollection                                                           ProtocolIE-ID ::= 461
id-ReportingPeriodicityForDataCollection                                                            ProtocolIE-ID ::= 462
id-NodeAssociatedInfoResult                                                                         ProtocolIE-ID ::= 463
id-SLPositioning-Ranging-Services-Info                                                              ProtocolIE-ID ::= 464
id-XR-Bcast-Information                                                                             ProtocolIE-ID ::= 465
id-PDUSessionsListToBeReleased-UPError                                                              ProtocolIE-ID ::= 466
id-MaximumDataBurstVolume                                                                           ProtocolIE-ID ::= 467
id-CPAC-Preparation-Type                                                                            ProtocolIE-ID ::= 468
id-UserPlaneFailureIndication                                                                       ProtocolIE-ID ::= 469
id-MN-only-MDT-collection                                                                           ProtocolIE-ID ::= 470
id-BarringExemptionforEmerCallInfo                                                                  ProtocolIE-ID ::= 471
id-Transmission-Bandwidth-asymmetric                                                                ProtocolIE-ID ::= 472
id-SRSPositioningConfigOrActivationRequest                                                          ProtocolIE-ID ::= 473
id-NRPPaPositioningInformation                                                                      ProtocolIE-ID ::= 474
id-AerialUEFlightInformationReportingControl                                                        ProtocolIE-ID ::= 475
id-PDCPSNGapTransfer-UL                                                                             ProtocolIE-ID ::= 476
id-RequestType                                                                                      ProtocolIE-ID ::= 477
id-NES-Cell-ID                                                                                      ProtocolIE-ID ::= 478
id-Cell-A-ID                                                                                        ProtocolIE-ID ::= 479
id-ProvisionStatus                                                                                  ProtocolIE-ID ::= 480
id-ECNMarkingorCongestionInformationReportingStatus                                                 ProtocolIE-ID ::= 481
id-AdditionalDRBSetupInfoList                                                                       ProtocolIE-ID ::= 482
id-PSIbasedSDUdiscardUL                                                                             ProtocolIE-ID ::= 483
id-PSIbasedSDUdiscardDL                                                                             ProtocolIE-ID ::= 484
id-DLPDUSetInformationMarkingSupportIndication                                                      ProtocolIE-ID ::= 485
id-PduSetDelayBudgetDownlink                                                                        ProtocolIE-ID ::= 486
id-PduSetDelayBudgetUplink                                                                          ProtocolIE-ID ::= 487
id-PduSetErrorRateDownlink                                                                          ProtocolIE-ID ::= 488
id-PduSetErrorRateUplink                                                                            ProtocolIE-ID ::= 489
id-MonitoringRequestonAvailableBitrate                                                              ProtocolIE-ID ::= 490
id-MMSID                                                                                            ProtocolIE-ID ::= 491
id-Indication-of-bitrate-adaptation                                                                 ProtocolIE-ID ::= 492
id-LTMHandoverInformationRequest                                                                    ProtocolIE-ID ::= 493
id-EarlySyncInformationRequest                                                                      ProtocolIE-ID ::= 494
id-LTMHandoverInformationRequestAcknowledge                                                         ProtocolIE-ID ::= 495
id-EarlySyncInformationResponse                                                                     ProtocolIE-ID ::= 496
id-LTMCellSwitchInformation                                                                         ProtocolIE-ID ::= 497
id-LTMUEAssociationInformation-List                                                                 ProtocolIE-ID ::= 498
id-CellSwitchTAInformation-List                                                                     ProtocolIE-ID ::= 499
id-TAInformation-List                                                                               ProtocolIE-ID ::= 500
id-LTMUpdatesToCandidateNodeInformation                                                             ProtocolIE-ID ::= 501
id-LTMUpdatesToCandidateCellInformation-List                                                        ProtocolIE-ID ::= 502
id-LTMUESecurityInformation                                                                         ProtocolIE-ID ::= 503
id-NGRAN-Node1-ID                                                                                   ProtocolIE-ID ::= 504
id-NGRAN-Node2-ID                                                                                   ProtocolIE-ID ::= 505
id-LTMUpdatesFromCandidateCellInformation-List                                                      ProtocolIE-ID ::= 506
id-LTMCandidateCellsToBeCancelled-List                                                              ProtocolIE-ID ::= 507
id-CSI-RSCoordinationRequest                                                                        ProtocolIE-ID ::= 508
id-CSI-RSCoordinationResponse                                                                       ProtocolIE-ID ::= 509
id-CLI-MeasurementResult-List                                                                       ProtocolIE-ID ::= 510
id-SBFD-Frequency-Configuration                                                                     ProtocolIE-ID ::= 511
id-NZP-CSI-RS-Resources-Config                                                                      ProtocolIE-ID ::= 512
id-SRS-Resource-Configuration                                                                       ProtocolIE-ID ::= 513
id-SRS-Resource-Indication                                                                          ProtocolIE-ID ::= 514
id-WAB-MT-ID                                                                                        ProtocolIE-ID ::= 515
id-LPWUSPSassistanceInformation                                                                     ProtocolIE-ID ::= 516
id-FurtherExtended-UEIdentityIndexValue                                                             ProtocolIE-ID ::= 517
id-Future-Coverage-Modification-List                                                                ProtocolIE-ID ::= 518
id-SliceToReportForDataCollection-List                                                              ProtocolIE-ID ::= 519
id-PredictedSliceAvailableCapacity                                                                  ProtocolIE-ID ::= 520
id-SliceMeasurementInitiationResult                                                                 ProtocolIE-ID ::= 521
id-SliceUEPerformance                                                                               ProtocolIE-ID ::= 522
id-NetworkSliceAreaScopeofMDT                                                                       ProtocolIE-ID ::= 523
id-GeographicalArea                                                                             ProtocolIE-ID ::= 524
id-GeographyBasedMDT                                                                                    ProtocolIE-ID ::= 525
id-TimeSCG-Failure                                                                                  ProtocolIE-ID ::= 526
id-FailureType                                                                                      ProtocolIE-ID ::= 527
id-FiveGProSeLayer3MHUEtoNetworkRelay                                                               ProtocolIE-ID ::= 528
id-FiveGProSeLayer2MHUEtoNetworkRelay                                                               ProtocolIE-ID ::= 529
id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay                                                   ProtocolIE-ID ::= 530
id-FiveGProSeLayer2MHRemote                                                                         ProtocolIE-ID ::= 531
id-LTMInformationSCG-AddReq                                                                         ProtocolIE-ID ::= 532
id-LTMInformationSCG-AddReqAck                                                                      ProtocolIE-ID ::= 533
id-LTMInformationSCG-ModReq                                                                         ProtocolIE-ID ::= 534
id-LTMInterSNExecutionNotification                                                                  ProtocolIE-ID ::= 535
id-LTMPSCellInformation-AddReq                                                                      ProtocolIE-ID ::= 536
id-LTMPSCellInformation-AddReqAck                                                                   ProtocolIE-ID ::= 537
id-LTMPSCellInformation-UpdateReq                                                                   ProtocolIE-ID ::= 538
id-LTMPSCellInformation-UpdateReqAck                                                                ProtocolIE-ID ::= 539
id-LTMPSCellInformation-UpdateRequired                                                              ProtocolIE-ID ::= 540
id-LTMPSCellInformation-UpdateConfirm                                                               ProtocolIE-ID ::= 541
id-LTMPSCellInformation-ChangeRequired                                                              ProtocolIE-ID ::= 542
id-LTMPSCellInformation-ChangeConfirm                                                               ProtocolIE-ID ::= 543
id-LTM-DC-DataForwarding-Indicator                                                                  ProtocolIE-ID ::= 544
id-SemipersistentPositioningInformation                                                             ProtocolIE-ID ::= 545
id-UEAveragePacketLossUL                                                                            ProtocolIE-ID ::= 546
id-LP-WUS-Disable-Indication                                                                        ProtocolIE-ID ::= 547
id-ContinuousMDT                                                                                    ProtocolIE-ID ::= 548


END
-- ASN1STOP

-- 9.3.8    Container definitions
-- ASN1START
-- **************************************************************
--
-- Container definitions
--
-- **************************************************************

XnAP-Containers {
itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)
ngran-access (22) modules (3) xnap (2) version1 (1) xnap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************
--
-- IE parameter types from other modules.
--
-- **************************************************************

IMPORTS
    maxPrivateIEs,
    maxProtocolExtensions,
    maxProtocolIEs,
    Criticality,
    Presence,
    PrivateIE-ID,
    ProtocolIE-ID   
FROM XnAP-CommonDataTypes;

-- **************************************************************
--
-- Class Definition for Protocol IEs
--
-- **************************************************************

XNAP-PROTOCOL-IES ::= CLASS {
    &id             ProtocolIE-ID           UNIQUE,
    &criticality    Criticality,
    &Value,
    &presence       Presence
}
WITH SYNTAX {
    ID              &id
    CRITICALITY     &criticality
    TYPE            &Value
    PRESENCE        &presence
}

-- **************************************************************
--
-- Class Definition for Protocol IE pairs
--
-- **************************************************************

XNAP-PROTOCOL-IES-PAIR ::= CLASS {
    &id                     ProtocolIE-ID       UNIQUE,
    &firstCriticality       Criticality,
    &FirstValue,
    &secondCriticality      Criticality,
    &SecondValue,
    &presence               Presence
}
WITH SYNTAX {
    ID                      &id
    FIRST CRITICALITY       &firstCriticality
    FIRST TYPE              &FirstValue
    SECOND CRITICALITY      &secondCriticality
    SECOND TYPE             &SecondValue
    PRESENCE                &presence
}

-- **************************************************************
--
-- Class Definition for Protocol Extensions
--
-- **************************************************************

XNAP-PROTOCOL-EXTENSION ::= CLASS {
    &id                 ProtocolIE-ID       UNIQUE,
    &criticality        Criticality,
    &Extension,
    &presence           Presence
}
WITH SYNTAX {
    ID                  &id
    CRITICALITY         &criticality
    EXTENSION           &Extension
    PRESENCE            &presence
}

-- **************************************************************
--
-- Class Definition for Private IEs
--
-- **************************************************************

XNAP-PRIVATE-IES ::= CLASS {
    &id                 PrivateIE-ID,
    &criticality        Criticality,
    &Value,
    &presence           Presence
}
WITH SYNTAX {
    ID                  &id
    CRITICALITY         &criticality
    TYPE                &Value
    PRESENCE            &presence
}

-- **************************************************************
--
-- Container for Protocol IEs
--
-- **************************************************************

ProtocolIE-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=
    SEQUENCE (SIZE (0..maxProtocolIEs)) OF
    ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container {XNAP-PROTOCOL-IES : IEsSetParam} ::=   ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {XNAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {
    id              XNAP-PROTOCOL-IES.&id                   ({IEsSetParam}),
    criticality     XNAP-PROTOCOL-IES.&criticality          ({IEsSetParam}{@id}),
    value           XNAP-PROTOCOL-IES.&Value                ({IEsSetParam}{@id})
}

-- **************************************************************
--
-- Container for Protocol IE Pairs
--
-- **************************************************************

ProtocolIE-ContainerPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=
    SEQUENCE (SIZE (0..maxProtocolIEs)) OF
    ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {
    id                  XNAP-PROTOCOL-IES-PAIR.&id                  ({IEsSetParam}),
    firstCriticality    XNAP-PROTOCOL-IES-PAIR.&firstCriticality    ({IEsSetParam}{@id}),
    firstValue          XNAP-PROTOCOL-IES-PAIR.&FirstValue          ({IEsSetParam}{@id}),
    secondCriticality   XNAP-PROTOCOL-IES-PAIR.&secondCriticality   ({IEsSetParam}{@id}),
    secondValue         XNAP-PROTOCOL-IES-PAIR.&SecondValue         ({IEsSetParam}{@id})
}

-- **************************************************************
--
-- Container Lists for Protocol IE Containers
--
-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES : IEsSetParam} ::=
    SEQUENCE (SIZE (lowerBound..upperBound)) OF
    ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, XNAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=
    SEQUENCE (SIZE (lowerBound..upperBound)) OF
    ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************
--
-- Container for Protocol Extensions
--
-- **************************************************************

ProtocolExtensionContainer {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=    SEQUENCE (SIZE (1..maxProtocolExtensions)) OF
    ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {XNAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {
    id                  XNAP-PROTOCOL-EXTENSION.&id             ({ExtensionSetParam}),
    criticality         XNAP-PROTOCOL-EXTENSION.&criticality    ({ExtensionSetParam}{@id}),
    extensionValue      XNAP-PROTOCOL-EXTENSION.&Extension      ({ExtensionSetParam}{@id})
}

-- **************************************************************
--
-- Container for Private IEs
--
-- **************************************************************

PrivateIE-Container {XNAP-PRIVATE-IES : IEsSetParam} ::=
    SEQUENCE (SIZE (1..maxPrivateIEs)) OF
    PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {XNAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {
    id              XNAP-PRIVATE-IES.&id            ({IEsSetParam}),
    criticality     XNAP-PRIVATE-IES.&criticality   ({IEsSetParam}{@id}),
    value           XNAP-PRIVATE-IES.&Value         ({IEsSetParam}{@id})
}

END
-- ASN1STOP