3GPP TS 29.551 V19.2.0 (2025-06)
Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Packet Flow Description Management Service;

Stage 3

(Release 19)

[image: image1.emf] 


[image: image2.png]=

A GLOBAL INITIATIVE




The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

5Foreword

1
Scope
5
2
References
5
3
Definitions, symbols and abbreviations
6
3.1
Definitions
6
3.2
Abbreviations
6
4
Packet Flow Description Management Service
7
4.1
Service Description
7
4.1.1
Overview
7
4.1.2
Service Architecture
7
4.1.3
Network Functions
8
4.1.3.1
Packet Flow Description Function (PFDF)
8
4.1.3.2
NF Service Consumers
8
4.2
Service Operations
8
4.2.1
Introduction
8
4.2.2
Nnef_PFDmanagement_Fetch Service Operation
8
4.2.2.1
General
8
4.2.2.1.1
When the NF service consumer is SMF
8
4.2.2.1.2
Void
9
4.2.2.2
Retrieval of PFDs by the full pull
9
4.2.2.3
Retrieval of PFDs by the partial pull
10
4.2.3
Nnef_PFDmanagement_Subscribe Service Operation
11
4.2.3.1
General
11
4.2.3.2
Subscription for event notifications on PFDs change
11
4.2.3.3
Subscription update for event notifications on PFDs change
12
4.2.4
Nnef_PFDmanagement_Notify Service Operation
13
4.2.4.1
General
13
4.2.4.2
Notification of PFD change
14
4.2.4.3
Notification PUSH
14
4.2.5
Nnef_PFDmanagement_Unsubscribe Service Operation
15
4.2.5.1
General
15
4.2.5.2
Unsubscribe from event notifications on PFDs change
15
5
Nnef_PFDmanagement API
16
5.1
Introduction
16
5.2
Usage of HTTP
16
5.2.1
General
16
5.2.2
HTTP standard headers
16
5.2.2.1
General
16
5.2.2.2
Content type
16
5.2.3
HTTP custom headers
17
5.2.3.1
General
17
5.3
Resources
17
5.3.1
Resource Structure
17
5.3.2
Resource: PFD of applications
18
5.3.2.1
Description
18
5.3.2.2
Resource definition
18
5.3.2.3
Resource Standard Methods
18
5.3.2.3.1
GET
18
5.3.2.4
Resource Custom Operations
20
5.3.2.4.1
Overview
20
5.3.2.4.2
Operation: PartialPull
20
5.3.2.4.2.1
Description
20
5.3.2.4.2.2
Operation Definition
20
5.3.3
Resource: Individual application PFD
21
5.3.3.1
Description
21
5.3.3.2
Resource definition
21
5.3.3.3
Resource Standard Methods
21
5.3.3.3.1
GET
21
5.3.3.4
Resource Custom Operations
22
5.3.4
Resource: PFD subscriptions
23
5.3.4.1
Description
23
5.3.4.2
Resource definition
23
5.3.4.3
Resource Standard Methods
23
5.3.4.3.1
POST
23
5.3.4.4
Resource Custom Operations
23
5.3.5
Resource: Individual PFD subscription
24
5.3.5.1
Description
24
5.3.5.2
Resource definition
24
5.3.5.3
Resource Standard Methods
24
5.3.5.3.1
DELETE
24
5.3.5.3.2
PUT
25
5.3.5.4
Resource Custom Operations
26
5.4
Custom Operations without associated resources
26
5.5
Notifications
27
5.5.1
General
27
5.5.2
PFD Change Notification
27
5.5.2.1
Description
27
5.5.2.2
Target URI
27
5.5.2.3
Standard Methods
27
5.5.2.3.1
POST
27
5.5.3
Notification Push
28
5.5.3.1
Description
28
5.5.3.2
Target URI
29
5.5.3.3
Standard Methods
29
5.5.3.3.1
POST
29
5.6
Data Model
30
5.6.1
General
30
5.6.2
Structured data types
31
5.6.2.1
Introduction
31
5.6.2.2
Type: PfdDataForApp
31
5.6.2.3
Type: PfdSubscription
32
5.6.2.4
Type: PfdChangeNotification
32
5.6.2.5
Type: PfdContent
33
5.6.2.6
Type: PfdChangeReport
33
5.6.2.7
Type: NotificationPush
34
5.6.2.8
Type: ApplicationForPfdRequest
34
5.6.3
Simple data types and enumerations
34
5.6.3.1
Introduction
34
5.6.3.2
Simple data types
34
5.6.3.3
Enumeration: PfdOperation
34
5.7
Error handling
35
5.7.1
General
35
5.7.2
Protocol Errors
35
5.7.3
Application Errors
35
5.8
Feature negotiation
35
5.9
Security
36
Annex A (normative):
OpenAPI specification
37
A.1
General
37
A.2
Nnef_PFDmanagement API
37
Annex B (informative):
Change history
46


Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x
the first digit:

1
presented to TSG for information;

2
presented to TSG for approval;

3
or greater indicates TSG approved document under change control.

y
the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z
the third digit is incremented when editorial only changes have been incorporated in the document.

1
Scope

The present document provides the stage 3 specification of the PFD Management Service of the 5G system.

The stage 2 definition and related procedures of the PFD Management Service are contained in 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4]. The 5G System Architecture is defined in 3GPP TS 23.501 [2].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition of the 5G System are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

The Packet Flow Description Function (PFDF) provides the PFD Management Service to NF service consumers (e.g. Session Management Function). The PFDF is functionality within the NEF.
2
References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-
References are either specific (identified by date of publication, edition number, version number, etc.) or non‑specific.

-
For a specific reference, subsequent revisions do not apply.

-
For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]
3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]
3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]
3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]
3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]
3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]
3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]
IETF RFC 9113: "HTTP/2".

[8]
IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".
[9]
OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.
[10]
3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".
[11]
3GPP TS 29.122: "T8 reference point for Northbound APIs".

[12]
3GPP TS 29.251: "Gw and Gwn reference points for sponsored data connectivity".
[13]
3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".
[14]
3GPP TS 33.501: "Security architecture and procedures for 5G system".

[15]
IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[16]
IETF RFC 9457: "Problem Details for HTTP APIs".

[17]
3GPP TR 21.900: "Technical Specification Group working methods".

[18]
IETF RFC 6733: "Diameter Base Protocol".

[19]
3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".
3
Definitions, symbols and abbreviations

3.1
Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2
Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

JSON
JavaScript Object Notation

NEF
Network Exposure Function
NRF
Network Repository Function

NWDAF
Network Data Analytics Function
PFD
Packet Flow Description

PFDF
Packet Flow Description Function

SBI
Service Based Interface

SMF
Session Management Function

4
Packet Flow Description Management Service

4.1
Service Description

4.1.1
Overview

The PFD Management Service, as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3] and 3GPP TS 23.503 [4], and the NWDAF subscribe to PFD notifications from the NEF(PFDF) as described in 3GPP TS 23.288 [19], is provided by the Packet Flow Description Function (PFDF).

The known NF service consumers are:
-
Session Management Function (SMF);
-
Network Data Analytics Function (NWDAF).
This service:
-
allows a NF service consumer (e.g. SMF, NWDAF) to subscribe to and unsubscribe from PFD changes;
-
notifies a NF service consumer (e.g.SMF, NWDAF) about changes of PFDs;

-
notifies a NF service consumer (e.g.SMF) to retrieve the PFDs; and

-
allows a NF service consumer (e.g. SMF) to retrieve PFDs.
4.1.2
Service Architecture
The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Policy and Charging related 5G architecture is also described in 3GPP TS 23.503 [4].

The PFD Management Service is provided by the PFDF to NF service consumers (e.g. SMF) and shown in the SBI representation model in Figure 4.1.2-1. The PFDF is a functionality within the NEF.
 SHAPE  \* MERGEFORMAT 


Figure 4.1.2-1: Reference Architecture for the Nnef_PFDmanagement Service; SBI representation

[image: image4]
Figure 4.1.2-2: Reference Architecture for the Nnef_PFDmanagement Service; reference point representation
4.1.3
Network Functions
4.1.3.1
Packet Flow Description Function (PFDF)

The Packet Flow Description Function (PFDF):

-
provides PFDs associated with one or more Application Identifiers;
-
notifies a NF service consumer to retrieve the PFDs; and
-
allows NF service consumers to subscribe to and unsubscribe from notifications on changes of PFDs for Application Identifier.
4.1.3.2
NF Service Consumers

The SMF shall support:

-
requesting and receiving the PFD(s) for one or more Application Identifiers.

The NWDAF shall support:

-
(un)subscription of the PFD(s) notifications for the known Application Identifiers.
4.2
Service Operations

4.2.1
Introduction
Service operations defined for the Nnef_PFDmanagement Service are shown in table 4.2.1-1.

Table 4.2.1-1: Nnef_PFDmanagement Service Operations
	Service Operation Name
	Description
	Initiated by

	Nnef_PFDmanagement_Fetch
	Provides the PFDs for application identifier(s) to the NF service consumer by the full pull or partial pull.
	NF service consumer (e.g. SMF)

	Nnef_PFDmanagement_Subscribe
	Allows NF service consumers to subscribe to notifications on events when the PFDs for application identifier(s) change.
	NF service consumer (e.g. SMF, NWDAF)

	Nnef_PFDmanagement_Notify
	Notifies NF service consumers to update and/or delete the PFDs for application identifier(s) or notifies NF service consumer to retrieve the PFDs for application identifier(s).
	PFDF

	Nnef_PFDmanagement_Unsubscribe
	Allows NF service consumers to unsubscribe from notifications on PFDs change events.
	NF service consumer (e.g. SMF, NWDAF)


4.2.2
Nnef_PFDmanagement_Fetch Service Operation
4.2.2.1
General

The Nnef_PFDmanagement_Fetch service operation provides means for the NF service consumer to retrieve the PFDs for one or more application identifier(s).
4.2.2.1.1
When the NF service consumer is SMF

This service operation enables the NF service consumer to retrieve PFDs for an Application Identifier(s) from the PFDF when:

-
a PCC rule with this application identifier is provided/activated by the PCF and the PFDs provided by the PFDF are not available at the NF service consumer; or
-
the caching timer for an application identifier elapses and a PCC rule for this application identifier is still active.
When the NF service consumer removes the last PCC rule that refers to the corresponding application identifier, or when the caching timer expires and no PCC rule refers to the application identifier, the NF service consumer may remove the PFD(s) related with the application identifier.

The PFDs retrieved from PFDF take precedence over any PFDs pre-configured in the NF service consumer. If all PFDs retrieved from the PFDF are removed for an application identifier, the pre-configured PFDs shall be applied again for the application identifier.

The PFDF may provide caching time value via the "cachingTime" attribute or, if the feature CachingTimer is supported, via the "cachingTimer" attribute, together with the PFDs for an application identifier. The caching time value retrieved from the PFDF takes precedence over the default caching time value configured in the NF service consumer. If no caching time value is received from the PFDF, the configured default caching time value shall be applied.
NOTE 1:
The NF service consumer(s) and the PFDF(s) within an operator network are configured with the same default caching time value to be applied for all application identifiers. 
NOTE 2:
The configuration of a caching time value per application identifier in the PFDF is based on the SLA between the operator and the ASP.
The following procedures using the Nnef_PFDmanagement_Fetch service operation are supported:

-
Retrieval of PFDs by the full pull.

-
Retrieval of PFDs by the partial pull.

4.2.2.1.2
Void
4.2.2.2
Retrieval of PFDs by the full pull

This procedure, as shown in Figure 4.2.2.2-1, is used to retrieve PFDs from the PFDF by the full pull for requested application identifier(s).


[image: image5.emf]1a.GET 

…

/applications/{appId}

1b. GET 

…

/applications

2. 200 OK

NF Service 

Consumer

PFDF


Figure 4.2.2.2-1: Retrieval of PFDs by the full pull
1.
The NF service consumer (e.g. SMF) shall send a GET request to the resource representing the PFDs for the requested application identifier(s):

-
for PFDs of an individual application identifier, the request URI shall be set to "{apiRoot}/nnef‑pfdmanagement/v1/applications/{appId}" (as shown in figure 4.2.2.2-1, step 1a); and

-
for PFD of a collection of application identifiers, the request URI shall be set to "{apiRoot}/nnef‑pfdmanagement/v1/applications" (as shown in figure 4.2.2.2-1, step 1b) with query parameters indicating the requested application identifier(s).
2.
On success, an HTTP "200 OK" response shall be returned, with the content containing a representation of an "Individual application PFD" resource or a "PFD of applications" resource for the requested application identifier(s). When the NF service consumer is SMF, it shall replace the stored PFD(s) retrieved from the PFDF with the new received PFD(s) for the requested application identifier(s). If the PFD(s) of one or more requested application identifier(s) are not provided in the response, the NF service consumer shall remove the PFD(s) of these requested application identifier(s) and re-apply the pre-configured PFDs.

If errors occur when processing the HTTP GET request, the PFDF shall send an HTTP error response as specified in clause 5.7. For "404 Not Found", when the NF service consumer is SMF, it shall remove the PFD(s) of the requested application identifier(s) in the NF service consumer and re-apply the pre-configured PFDs.

If the feature "ES3XX" is supported, and the PFDF determines the received HTTP GET request needs to be redirected, the PFDF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

4.2.2.3
Retrieval of PFDs by the partial pull

This procedure, as shown in Figure 4.2.2.3-1, is used to retrieve PFDs from the PFDF by the partial pull for requested application identifier(s) if the "PartialPull" feature defined in clause 5.8 is supported both by NF service consumer and PFDF.


[image: image6.emf]   

NF  Service Consumer   PFDF  

POST   …/ applications / partialpull  

2a.  200 OK   2b.  204 No Content  


Figure 4.2.2.3-1: Retrieval of PFDs by the partial pull
1.
The NF service consumer (e.g. SMF) shall send an HTTP POST message to the resource "{apiRoot}/nnef‑pfdmanagement/v1/applications/partialpull". The NF service consumer shall include one or more ApplicationForPfdRequest data structure in the content of the HTTP POST. With the ApplicationForPfdRequest data structure, the NF service consumer shall include the application identifier within the "applicationId" attribute and the timestamp of the PFDs received in the last provisioning for the application identifier within the "pfdTimestamp" attribute. The NF service consumer may also request full set of PFD(s) of an application identifier without including the timestamp if it is not available.
2.
If the PFDF accepted the HTTP POST request, the PFDF shall send to the NF service consumer:

-
the HTTP "200 OK" response (as shown in figure 4.2.2.3-1, step 2a) including one or more PfdDataForApp data structure if the NF service consumer requests the PFD(s) for an application identifier(s) without the timestamp or if the NF service consumer requests PFD(s) for an application identifier(s) with timestamp and the PFDF determines that corresponding PFD(s) is changed since the last request based on the timestamp received in the request for the application identifier. Within the PfdDataForApp data type, the PFDF shall include the application identifier within the "applicationId" attribute, the new timestamp within the "pfdTimestamp" attirbute, the "partialFlag" attribute if applicable and create/update/remove the PFDs as follows:

-
include the full list of the PFD(s) within the "pfds" attribute for the application identifier which is requested without the timestamp;

-
include the full list of the PFD(s) within the "pfds" attribute for the application identifier if all the PFD(s) are changed for the application identifier since the last request based on the timestamp;
-
for the application identifier whose PFD(s) are partially changed:
-
include the new PFD(s) with new PFD identifier(s) within the "pfds" attribute if the new PFD(s) is added for the application identifier and the "partialFlag" attribute set to true; 
-
include the new PFD(s) with existing PFD identifier(s) within the "pfds" attribute if the existing PFD(s) is updated for the application identifier and the "partialFlag" attribute set to true;
-
include the existing PFD identifier(s) without any content within the "pfds" attribute if the existing PFD(s) is removed for the application identifier and the "partialFlag" attribute set to true;

-
not include the "pfds" attribute if the PFD(s) is removed for the application identifier.
NOTE 1:
The PFDF does not include the PfdDataForApp data type for the application identifier whose PFD(s) is not updated since last request.

NOTE 2
If the PFDF determines that the PFDs are changed since the last request but cannot determine what changes have been applied to the individual PFD(s) for an application identifier, the PFDF can include the full list of the PFD(s) in the PfdDataForApp data type.
-
the HTTP "204 No Content" response (as shown in figure 4.2.2.3-1, step 2b) if the PFD(s) for all the requested application identifier(s) are not changed since last request.

When the NF service consumer receives the response with "200 OK" status code, the NF service consumer shall
-
remove the all existing PFD(s) (if available) and install all the new provided PFD(s) for an application identifier if full list of PFD(s) is received but "partialFlag" attribute is not received;
-
delete the existing PFD(s) for an application identifier(s) where no PFD(s) is received;
-
for an application identifier(s) where the PFD(s) is provided and "partialFlag" attribute is also provided and set to true:
-

install a new PFD(s) if the new PFD(s) with a new PFD identifier(s) is received;
-
update an existing PFD(s) if a new PFD(s) with the same PFD identifier(s) is received;
-
delete an existing PFD(s) if the same PFD identifier(s) without any content is received;

-
reserve an existing PFD(s) if the PFD identifier(s) is not received.

If the "ES3XX" feature is supported, and the PFDF determines the received HTTP POST request needs to be redirected, the PFDF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].
4.2.3
Nnef_PFDmanagement_Subscribe Service Operation
4.2.3.1
General

The Nnef_PFDmanagement_Subscribe service operation enables the NF service consumer to subscribe to notifications on events when the PFDs for application identifier(s) change.

The following procedures using the Nnef_PFDmanagement_Subscribe service operation are supported:

-
Subscription for event notifications on PFDs change;

-
Subscription update for event notifications on PFD change.

4.2.3.2
Subscription for event notifications on PFDs change

This procedure, as shown in Figure 4.2.3.2-1, is used to subscribe to notifications on events when the PFDs for application identifier(s) change.


[image: image7.emf]2. 201 Created

1. POST

…

/subscriptions

NF service 

consumer

PFDF


Figure 4.2.3.2-1: Creation of a subscription for event notifications on PFDs change
1.
The NF service consumer (e.g. SMF, NWDAF) shall send a POST request to the request URI representing the collection of PFD subscriptions resource "{apiRoot}/nnef‑pfdmanagement/v1/subscriptions". The NF service consumer shall include the PfdSubscription data type in the request content. Within the PfdSubscription data type, the NF service consumer shall include:

-
an URI where to receive the requested notifications as "notifyUri" attribute;
and may include:

-
subscribed application identifier(s) within the "applicationIds" attribute;
-
the immediate reporting indication within the "immRep" attribute, if the "PfdDetermination" feature is supported.

2.
If the request is accepted, the PFDF shall: 

-
create a new subscription;

-
assign a subscriptionId;

-
store the subscription;

-
send an HTTP "201 Created" response, with the content containing a representation of the created subscription, and the Location header containing the resource URI of the created subscription "{apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}"; and
-
if the "PfdDetermination" feature is supported, the "immRep" attribute value was set to "true" in the request, and there are currently existing PFDs matching the subscription, then the PFDF shall provide these PFDs in the response by including the "pfds" attribute of the "PfdSubscription" data type.

Otherwise, one of the HTTP status codes listed in table 5.3.4.3.1-3 shall be returned.

NOTE:
The PFDs that have been provisioned to the PFDF before the NF service consumer performs the subscription are not notified to the NF service consumer as a result of this subscription when the PfdDetermination feature is not supported, but the NF service consumer can retrieve them before performing the subscription by invoking Nnef_PFDmanagement_Fetch Service Operation.
4.2.3.3
Subscription update for event notifications on PFDs change

This procedure, as shown in Figure 4.2.3.3-1, is used to update an existing subscription to notifications on events when the PFDs for application identifier(s) change.


[image: image8.emf]2. 200OK

1. PUT

…

/subscriptions/{subscriptionId}

NF service 

consumer

PFDF


Figure 4.2.3.3-1: Update of a subscription for event notifications on PFDs change
1.
If the feature PfdChgSubsUpdate is supported, the NF service consumer (e.g. SMF) shall send a PUT request to the resource URI representing the targeted PFD subscription resource "{apiRoot}/nnef‑pfdmanagement/v1/subscriptions/{subscriptionId}". The NF service consumer shall include the PfdSubscription data type in the request content. Within the PfdSubscription data type, the NF service consumer shall include:

-
an URI where to receive the requested notifications as "notifyUri" attribute;
and may include:

-
subscribed application identifier(s) within the "applicationIds" attribute;
-
the immediate reporting indication within the "immRep" attribute, if the "PfdDetermination" feature is supported.
NOTE 1:
The "notifyUri" attribute within the PfdSubscription data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.
2.
If the feature PfdChgSubsUpdate is supported and the request is accepted, the PFDF shall: 

-
update the subscription;

-
send an HTTP "200 OK" response with the content containing a representation of the updated subscription;
-
if the "PfdDetermination" feature is supported, the "immRep" attribute value was set to "true" in the request, and there are currently existing PFDs matching the subscription, then the PFDF shall provide these PFDs in the response by including the "pfds" attribute of the "PfdSubscription" data type.

Otherwise, if errors occur when processing the HTTP PUT request, the PFDF shall send an HTTP error response as specified in clause 5.7. If the feature "ES3XX" is supported, and the PFDF determines the received HTTP PUT request needs to be redirected, the PFDF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

NOTE 2:
The PFDs that have been provisioned to the PFDF before the NF service consumer performs the subscription are not notified to the NF service consumer as a result of this subscription when the PfdDetermination feature is not supported, but the NF service consumer can retrieve them before performing the subscription by invoking Nnef_PFDmanagement_Fetch Service Operation.
4.2.4
Nnef_PFDmanagement_Notify Service Operation
4.2.4.1
General

The Nnef_PFDmanagement_Notify service operation notifies the NF service consumer to update, delete or retrieve the PFDs for application identifier(s).

The following procedures using the Nnef_PFDmanagement_Notify service operation are supported:

-
Notification of PFD change.

-
Notification PUSH

4.2.4.2
Notification of PFD change


[image: image9.emf]2a. 200 OK

2b. 204 No Content

1. POST{notifyUri}(array(PfdChangeNotification))

NF service 

consumer

PFDF


Figure 4.2.4.2-1: Notification of PFD change
1.
The PFDF shall send a POST request to the NF service consumer (e.g. SMF, NWDAF) targeting the URI "{notifyUri}, where {notifyUri} is the notification URI provided during the creation or modification of the subscription resource as specified in clause 4.2.3. The content of the POST request shall contain one or more PfdChangeNotification data structure(s).

2
If the notification is accepted, the NF service consumer shall reply with:

-
"204 No Content" indicating the successful provisioning of all PFDs; or

-
"200 OK" and the content of the response shall contain "PfdChangeReport" data structure with detailed information of failed application(s).

Otherwise, if errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.7. If the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].
4.2.4.3
Notification PUSH


[image: image10.emf]2. 204 No Content

1. POST {notifyUri}/notifypush

NF service 

consumer

PFDF


Figure 4.2.4.3-1: Notification PUSH

1.
If the NotificationPush feature defined in clause 5.8 is supported, and when the PFDF only notifies the NF service consumer to retrieve or remove the PFDs for application identifier(s), then the PFDF shall send a POST request to the NF service consumer (e.g. SMF) with "{notifyUri}/notifypush" as URI (where the "notifyUri" was previously supplied by the NF service consumer) and one or more NotificationPush data structure as request body. Each NotificationPush data structure shall include the application identifier(s) within the "appIds" attribute, the "pfdOp" attribute set to the applicable value and may include the "allowedDelay" attribute containing the allowed delay time if received when the "pfdOp" attribute is set to "RETRIEVE", "FULLPULL" or "PARTIALPULL". 
2
If the NF service consumer accepts the received POST request, the NF service consumer shall send an HTTP "204 No Content" response.

After the successful processing of the HTTP POST request, 

-
if the PFDF requests the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "RETRIEVE" or without the "pfdOp" attribute, the NF service consumer shall determine to invoke the full pull procedure defined in clause 4.2.2.2 or invoke the partial pull procedure defined in clause 4.2.2.3 if the "PartialPull" feature is supported to retrieve the PFD(s) for the application identifier(s).

-
if the "PartialPull" feature is supported and if the PFDF requests the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "FULLPULL", the NF service consumer shall invoke the full pull procedure defined in clause 4.2.2.2.

-
if the "PartialPull" feature is supported and if the PFDF indicates the NF service consumer to retrieve the PFD(s) with the "pfdOp" attribute set to the value "PARTIALPULL", the NF service consumer may invoke the partial full procedure defined in clause 4.2.2.3.

-
for all above cases, if the "allowedDelay" attribute is provided for one or more application(s), the NF service consumer shall retrieve the PFD(s) within the allowed delay time. 
-
if the PFDF requests the NF service consumer to remove the PFD(s) with the "pfdOp" attribute set to the value "REMOVE", the NF service consumer shall remove the PFD(s) for the application identifier(s) and re-apply the pre-configured PFDs.

If errors occur when processing the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

4.2.5
Nnef_PFDmanagement_Unsubscribe Service Operation
4.2.5.1
General

The Nnef_PFDmanagement_Unsubscribe service operation is used by the NF service consumer to unsubscribe from notifications on PFD change events.

The following procedures using the Nnef_PFDmanagement_Unsubscribe service operation are supported:

-
Unsubscribe from event notifications on PFDs change.
4.2.5.2
Unsubscribe from event notifications on PFDs change


[image: image11.emf]2. 204 No Content

1. DELETE

…

/subscriptions/{subscriptionId}

NF service 

consumer

PFDF


Figure 4.2.5.2-1: Unsubscribe from event notifications on PFDs change

1.
The NF service consumer (e.g. SMF, NWDAF) shall send a DELETE request to the resource URI representing the individual PFD subscription. The request body shall be empty.

2.
If the request is accepted, an HTTP "204 No Content" response shall be returned. The response body shall be empty.

Otherwise, if errors occur when processing the HTTP DELETE request, the PFDF consumer shall send an HTTP error response as specified in clause 5.7. If the feature "ES3XX" is supported, and the PFDF determines the received HTTP DELETE request needs to be redirected, the PFDF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

5
Nnef_PFDmanagement API

5.1
Introduction

The Packet Flow Description Management Service shall use the Nnef_PFDmanagement API. 

The API URI of the Nnef_PFDmanagement API shall be: 
{apiRoot}/<apiName>/<apiVersion>
The request URIs used in HTTP requests from the NF service consumer towards the PFDF shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-
The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-
The <apiName> shall be "nnef-pfdmanagement".

-
The <apiVersion> shall be "v1".

-
The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.
5.2
Usage of HTTP

5.2.1
General

HTTP/2, IETF RFC 9113 [7], shall be used as specified in clause 5.2 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

The OpenAPI [9] specification of HTTP messages and content bodies for the Nnef_PFDmanagement service is contained in Annex A.

5.2.2
HTTP standard headers

5.2.2.1
General

See clause 5.2.2 of 3GPP TS 29.500 [5] for the usage of HTTP standard headers.

5.2.2.2
Content type

JSON, IETF RFC 8259 [8], shall be used as content type of the HTTP bodies specified in the present specification, as specified in clause 5.4 of 3GPP TS 29.500 [5]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 9457 [16].

5.2.3
HTTP custom headers

5.2.3.1
General

The Nnef_PFDmanagement API shall support HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [5] and may support HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [5].

In this Release of the specification, no specific custom headers are defined for the Nnef_PFDmanagement API.
5.3
Resources

5.3.1
Resource Structure


[image: image12.emf]/applications

/{appId}

{apiRoot}/nnef-pfdmanagement/v1

/subscriptions

/{subscriptionId}

/partialpull


Figure 5.3.1-1: Resource URI structure of the Nnef_PFDmanagement API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

	Resource name
	Resource URI
	HTTP method or custom operation
	Description

	PFD of applications
	/applications
	GET
	Nnef_PFDmanagement_Fetch.

Retrieve PFDs for one or multiple applications using query parameters.

	
	/applications/partialpull
	PartialPull


	Request PFDs for one or multiple application identifier(s) by the partial update.

	Individual application PFD
	/applications/{appId}
	GET
	Nnef_PFDmanagement_Fetch.

Retrieve the PFD for an application.

	PFD subscriptions
	/subscriptions
	POST
	Nnef_PFDmanagement_Subscribe.

Subscribe the notification of PFD changes.

	Individual PFD subscription
	/subscriptions/{subscriptionId}
	PUT
	Nnef_PFDmanagement_Subscribe.

Update a subscription to PFD change notifications.

	Individual PFD subscription
	/subscriptions/{subscriptionId}
	DELETE
	Nnef_PFDmanagement_Unsubscribe.

Delete a subscription to PFD change notifications.


5.3.2
Resource: PFD of applications

5.3.2.1
Description

This resource represents PFDs for all applications.
5.3.2.2
Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications
This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource
	Name
	Data type
	Definition

	apiRoot
	string
	See clause 5.1


5.3.2.3
Resource Standard Methods

5.3.2.3.1
GET

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the GET method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	application-ids
	array(ApplicationId)
	M
	1..N
	The requested application identifier(s) for which PFD(s) shall be returned.

	supported-features
	SupportedFeatures
	O
	0..1
	To filter irrelevant responses related to unsupported features.


This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

	Data type
	P
	Cardinality
	Description

	n/a
	
	
	


Table 5.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

	Data type
	P
	Cardinality
	Response

codes
	Description

	array(PfdDataForApp)
	M
	0..N
	200 OK
	The PFDs for one or more application identifier(s) provided in the request URI are returned.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply.

NOTE 2:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Contains the identifier of the target PFDF (service) instance towards which the request should be redirected


Table 5.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Contains the identifier of the target PFDF (service) instance towards which the request should be redirected


5.3.2.4
Resource Custom Operations

5.3.2.4.1
Overview

Table 5.3.2.4.1-1: Custom operations

	Operation Name
	Custom operation URI
	Mapped HTTP method
	Description

	PartialPull
	/applications/partialpull
	POST
	Request PFDs for one or multiple application identifier(s) by the partial update.


5.3.2.4.2
Operation: PartialPull
5.3.2.4.2.1
Description

5.3.2.4.2.2
Operation Definition

This custom operation requests PFDs for one ore multiple application identifier(s) by the partial update..

This operation shall support the request data structures specified in table 5.3.2.4.2.2-1 and the response data structure and response codes specified in table 5.3.2.4.2.2-2.

Table 5.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

	Data type
	P
	Cardinality
	Description

	array(ApplicationForPfdRequest)
	M
	1..N
	Contains the application Identifier(s) for PFDs request.


Table 5.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

	Data type
	P
	Cardinality
	Response codes
	Description

	array(PfdDataForApp)
	M
	1..N
	200 OK
	Successful case. The partial pull request is successfully received and processed, and PFDs for one or more application identifier(s) are returned.

	n/a
	
	
	204 No Content
	Successful case. The partial pull request is successfully received and processed, and the PFD(s) for all the requested application identifier(s) have not changed since last request.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	NOTE:
The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [5] shall also apply.


Table 5.3.2.4.2.2-3: Headers supported by the 307 Response Code on this resource custom operation

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative target URI of the resource custom operation located in an alternative PFDF (service) instance towards which the request should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Contains the identifier of the target PFDF (service) instance towards which the request should be redirected


Table 5.3.2.4.2.2-4: Headers supported by the 308 Response Code on this resource custom operation

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative target URI of the resource custom operation located in an alternative PFDF (service) instance towards which the request should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Contains the identifier of the target PFDF (service) instance towards which the request should be redirected


5.3.3
Resource: Individual application PFD

5.3.3.1
Description

This resource represents the PFD identified by an application identifier.
5.3.3.2
Resource definition
Resource URI: {apiRoot}/nnef-pfdmanagement/v1/applications/{appId}
This resource shall support the resource URI variables defined in table 5.3.3.2-1.

Table 5.3.3.2-1: Resource URI variables for this resource
	Name
	Data type
	Definition

	apiRoot
	string
	See clause 5.1.

	appId
	string
	Identifies a set of PFD for an application identifier.


5.3.3.3
Resource Standard Methods

5.3.3.3.1
GET

This method shall support the URI query parameters specified in table 5.3.3.3.1-1.

Table 5.3.3.3.1-1: URI query parameters supported by the GET method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	supported-features
	SupportedFeatures
	O
	0..1
	To filter irrelevant responses related to unsupported features.


This method shall support the request data structures specified in table 5.3.3.3.1-2 and the response data structures and response codes specified in table 5.3.3.3.1-3.

Table 5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

	Data type
	P
	Cardinality
	Description

	n/a
	
	
	


Table 5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

	Data type
	P
	Cardinality
	Response

codes
	Description

	PfdDataForApp
	M
	1
	200 OK
	A representation of PFDs for an application in the request URI is returned.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection, during Individual PFD subscription retrieval.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection, during Individual PFD subscription retrieval.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the GET method shall also apply.

NOTE 2:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


Table 5.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


5.3.3.4
Resource Custom Operations

None.

5.3.4
Resource: PFD subscriptions
5.3.4.1
Description

This resource represents a collection of subscriptions created by NF service consumers of Nnef_PFDmanagement service.

5.3.4.2
Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions
This resource shall support the resource URI variables defined in table 5.3.4.2-1.

Table 5.3.4.2-1: Resource URI variables for this resource
	Name
	Data type
	Definition

	apiRoot
	string
	See clause 5.1


5.3.4.3
Resource Standard Methods

5.3.4.3.1
POST

This method shall support the URI query parameters specified in table 5.3.4.3.1-1.

Table 5.3.4.3.1-1: URI query parameters supported by the POST method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	n/a
	
	
	
	


This method shall support the request data structures specified in table 5.3.4.3.1-2 and the response data structures and response codes specified in table 5.3.4.3.1-3.

Table 5.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

	Data type
	P
	Cardinality
	Description

	PfdSubscription
	M
	1
	Create a PfdSubscription resource.


Table 5.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

	Data type
	P
	Cardinality
	Response

codes
	Description

	PfdSubscription
	M
	1
	201 Created
	The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned.

	NOTE:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply.


Table 5.3.4.3.1-4: Headers supported by the 201 Response Code on this resource 

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}


5.3.4.4
Resource Custom Operations

None.

5.3.5
Resource: Individual PFD subscription
5.3.5.1
Description

This resource represents an individual PFD subscription created by an NF service consumer of the Nnef_PFDmanagement service.
5.3.5.2
Resource definition

Resource URI: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}
This resource shall support the resource URI variables defined in table 5.3.5.2-1.

Table 5.3.5.2-1: Resource URI variables for this resource
	Name
	Data type
	Definition

	apiRoot
	string
	See clause 5.1

	subscriptionId
	string
	Identifies an individual subscription to the PFD management service


5.3.5.3
Resource Standard Methods

5.3.5.3.1
DELETE

This method shall support the URI query parameters specified in table 5.3.5.3.1-1.

Table 5.3.5.3.1-1: URI query parameters supported by the DELETE method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	n/a
	
	
	
	


This method shall support the request data structures specified in table 5.3.5.3.1-2 and the response data structures and response codes specified in table 5.3.5.3.1-3.

Table 5.3.5.3.1-2: Data structures supported by the DELETE Request Body on this resource

	Data type
	P
	Cardinality
	Description

	n/a
	
	
	


Table 5.3.5.3.1-3: Data structures supported by the DELETE Response Body on this resource

	Data type
	P
	Cardinality
	Response

codes
	Description

	n/a
	
	
	204 No Content
	The PfdSubscription resource matching the subscriptionId was deleted successfully.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection, during Individual PFD subscription deletion.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection, during Individual PFD subscription deletion.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the DELETE method shall also apply.

NOTE 2:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.3.5.3.1-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


Table 5.3.5.3.1-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


5.3.5.3.2
PUT

This method shall support the URI query parameters specified in table 5.3.5.3.2-1.

Table 5.3.5.3.2-1: URI query parameters supported by the PUT method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	n/a
	
	
	
	


This method shall support the request data structures specified in table 5.3.5.3.2-2 and the response data structures and response codes specified in table 5.3.5.3.2-3.

Table 5.3.5.3.2-2: Data structures supported by the PUT Request Body on this resource

	Data type
	P
	Cardinality
	Description

	PfdSubscription
	M
	1
	Update a PfdSubscription resource.


Table 5.3.5.3.2-3: Data structures supported by the PUT Response Body on this resource

	Data type
	P
	Cardinality
	Response

codes
	Description

	PfdSubscription
	M
	1
	200 OK
	The update of a PfdSubscription resource is confirmed and a representation of that resource is returned.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection, during Individual PFD subscription modification.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection, during Individual PFD subscription modification.

Applicable if the feature "ES3XX" is supported.

(NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the PUT method shall also apply.

NOTE 2:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.3.5.3.2-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains a alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


Table 5.3.5.3.2-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI of the resource located in an alternative PFDF (service) instance towards which the request is redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target PFDF (service) instance towards which the request is redirected


5.3.5.4
Resource Custom Operations

None.

5.4
Custom Operations without associated resources

None

5.5
Notifications

5.5.1
General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [5] and clause 4.6.2.3 of 3GPP TS 29.501 [6].

Table 5.5.1-1: Notifications overview

	Notification
	Callback URI
	HTTP method or custom operation
	Description (service operation)

	PFD Change Notification
	{notifyUri}
	POST
	Notification of PFD change.

	Notification Push
	{notifyUri}/notifypush
	notifypush (POST)
	Notifies NF service consumer to retrieve the PFDs for application identifier(s).


5.5.2
PFD Change Notification

5.5.2.1
Description

The PFD Change Notification is used by the PFDF to inform the NF service consumer, which has subscribed to this Notification via the PFD subscriptions resource.
5.5.2.2
Target URI

The Callback URI "{notifyUri}" shall be used with the callback URI variables defined in table 5.5.2.2-1.

Table 5.5.2.2-1: Callback URI variables
	Name
	Data type
	Definition

	notifyUri
	Uri
	The Notification Uri as assigned within the PFD subscriptions resource and described within the PfdSubscription data type (see table 5.6.2.3-1).


5.5.2.3
Standard Methods

5.5.2.3.1
POST
This method shall support the URI query parameters specified in table 5.5.2.3.1-1.

Table 5.5.2.3.1-1: URI query parameters supported by the POST method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	n/a
	
	
	
	


This method shall support the request data structures specified in table 5.5.2.3.1-2 and the response data structures and response codes specified in table 5.5.2.3.1-3.

Table 5.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

	Data type
	P
	Cardinality
	Description

	array(PfdChangeNotification)
	M
	1..N
	Provides PFD change information.


Table 5.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

	Data type
	P
	Cardinality
	Response codes
	Description

	n/a
	
	
	204 No Content
	The PFD operation in the notification is performed successfully, i.e. all PFD changes are accepted by the NF service consumer.

	array(PfdChangeReport)
	M
	1..N
	200 OK
	The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason for each failed application in the partial success.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection, during PFD Change Notification.

Applicable if the feature "ES3XX" is supported.

(NOTE 3)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection, during PFD Change Notification.

Applicable if the feature "ES3XX" is supported.

(NOTE 3)

	ProblemDetails
	O
	0..1
	500 Internal Server Error
	The server encountered an unexpected condition that prevented it from fulfilling the request. PFDs for all applications are not accepted by the NF service consumer. (NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply.

NOTE 2:
Failure cases are described in clause 5.7.

NOTE 3:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target NF (service) instance towards which the notification request is redirected


Table 5.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target NF (service) instance towards which the notification request is redirected


5.5.3
Notification Push

5.5.3.1
Description

The Notification Push is used by the PFDF to inform the NF service consumer, which has subscribed to this Notification via the PFD subscriptions resource.
5.5.3.2
Target URI

The Callback URI "{notifyUri}/notifypush" shall be used with the callback URI variables defined in table 5.5.3.2-1.

Table 5.5.3.2-1: Callback URI variables
	Name
	Data type
	Definition

	notifyUri
	Uri
	The Notification Uri as assigned within the PFD subscriptions resource and described within the PfdSubscription data type (see table 5.6.2.3-1).


5.5.3.3
Standard Methods

5.5.3.3.1
POST
This method shall support the URI query parameters specified in table 5.5.3.3.1-1.

Table 5.5.3.3.1-1: URI query parameters supported by the POST method on this resource
	Name
	Data type
	P
	Cardinality
	Description

	n/a
	
	
	
	


This method shall support the request data structures specified in table 5.5.3.3.1-2 and the response data structures and response codes specified in table 5.5.3.3.1-3.

Table 5.5.3.3.1-2: Data structures supported by the POST Request Body on this resource

	Data type
	P
	Cardinality
	Description

	array(NotificationPush)
	M
	1..N
	Provides the information for the NF service consumer to retrieve and/or remove the PFDs.


Table 5.5.3.3.1-3: Data structures supported by the POST Response Body on this resource

	Data type
	P
	Cardinality
	Response codes
	Description

	n/a
	
	
	204 No Content
	Notification PUSH is accepted.

	RedirectResponse
	O
	0..1
	307 Temporary Redirect
	Temporary redirection, during PFD Change Notification.

Applicable if the feature "ES3XX" is supported.

(NOTE 3)

	RedirectResponse
	O
	0..1
	308 Permanent Redirect
	Permanent redirection, during PFD Change Notification.

Applicable if the feature "ES3XX" is supported.

(NOTE 3)

	ProblemDetails
	O
	0..1
	500 Internal Server Error
	(NOTE 2)

	NOTE 1:
In addition, the HTTP status codes which are specified as mandatory in table 5.2.7.1-1 of 3GPP TS 29.500 [5] for the POST method shall also apply.

NOTE 2:
Failure cases are described in clause 5.7.

NOTE 3:
The RedirectResponse data structure may be provided by an SCP (cf. clause 6.10.9.1 of 3GPP TS 29.500 [5]).


Table 5.5.3.3.1-4: Headers supported by the 307 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target NF (service) instance towards which the notification request is redirected


Table 5.5.3.3.1-5: Headers supported by the 308 Response Code on this resource

	Name
	Data type
	P
	Cardinality
	Description

	Location
	string
	M
	1
	Contains an alternative URI representing the end point of an alternative NF consumer (service) instance towards which the notification should be redirected.

For the case where the request is redirected to the same target via a different SCP, refer to clause 6.10.9.1 of 3GPP TS 29.500 [5].

	3gpp-Sbi-Target-Nf-Id
	string
	O
	0..1
	Identifier of the target NF (service) instance towards which the notification request is redirected


5.6
Data Model

5.6.1
General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nnef_PFDmanagement service based interface protocol.

Table 5.6.1-1: Nnef_PFDmanagement specific Data Types

	Data type
	Section defined
	Description
	Applicability

	ApplicationForPfdRequest
	5.6.2.8
	Contains the application identifier(s) for the PFDs request.
	PartialPull

	NotificationPush
	5.6.2.7
	Provides the information for the NF service consumer to retrieve the PFDs and/or remove the PFDs of the applicable application identifier(s).
	NotificationPush

	PfdChangeNotification
	5.6.2.4
	Represents PFD change information.
	

	PfdChangeReport
	5.6.2.6
	Represents error of PFD change.
	

	PfdContent
	5.6.2.5
	Represents the content of a PFD for an application identifier.
	

	PfdDataForApp
	5.6.2.2
	Represents the PFDs for an application identifier.
	

	PfdOperation
	5.6.3.3
	Indicates the operation of the PFDs.
	NotificationPush

	PfdSubscription
	5.6.2.3
	Represents a PFD subscription.
	


Table 5.6.1-2 specifies data types re-used by the Nnef_PFDmanagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_PFDmanagement service based interface. 

Table 5.6.1-2: Nnef_PFDmanagement re-used Data Types

	Data type
	Reference
	Comments
	Applicability

	ApplicationId
	3GPP TS 29.571 [10]
	
	

	DateTime
	3GPP TS 29.571 [10]
	
	

	DomainNameProtocol
	3GPP TS 29.122 [11]
	Indicates the additional protocol and protocol field for domain names to be matched.
	DomainNameProtocol

	DurationSec
	3GPP TS 29.571 [10]
	Identifies a period of time in units of seconds.
	NotificationPush

	NFType
	3GPP TS 29.510 [13]
	Identifies the NF type.
	PfdDetermination

	RedirectResponse
	3GPP TS 29.571 [10]
	Contains redirection related information.
	ES3XX

	SupportedFeatures
	3GPP TS 29.571 [10]
	
	

	Uri
	3GPP TS 29.571 [10]
	
	


5.6.2
Structured data types

5.6.2.1
Introduction

This clause defines the structures to be used in resource representations. 

Allowed structures are: array, object.

5.6.2.2
Type: PfdDataForApp
Table 5.6.2.2-1: Definition of type PfdDataForApp

	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	applicationId
	ApplicationId
	M
	1
	Identifier of an application.
	

	pfds
	array(PfdContent)
	C
	1..N
	PFDs for the application identifier.
	

	cachingTime
	DateTime
	O
	0..1
	Caching time for an application identifier.
	

	cachingTimer
	DurationSec
	O
	0..1
	Caching timer for an application identifer expressed as duration with unit of second.
	CachingTimer

	pfdTimestamp
	DateTime
	O
	0..1
	The value represents the UTC time set for the PFD(s) of the application identifier provisioning by the PFDF.
	PartialPull

	partialFlag
	boolean
	O
	0..1
	Indication of partial update of PFDs for an existing application identifier if it is included and set to "true". Otherwise set to "false" indicates not supporting partial update of PFDs for an existing application identifier. The default value "false" applies if the attribute is not present.
	PartialPull

	supportedFeatures
	SupportedFeatures
	C
	0..1
	Used to negotiate the applicability of the optional features.
This attribute shall be present in in the HTTP GET response if the "supported-features" attribute query parameter is included in the HTTP GET request.
	

	NOTE:
The "pfds" attribute shall be included if the "PartialPull" feature is not supported.


5.6.2.3
Type: PfdSubscription
Table 5.6.2.3-1: Definition of type PfdSubscription
	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	applicationIds
	array(ApplicationId)
	O
	1..N
	Identifiers of applications with PFDs change.
	

	immRep
	boolean
	O
	0..1
	Indication of immediate reporting.

-
Set to "true" indicates immediate reporting of current stored PFDs of the subscription, if available.

-
Set to "false" indicates no immediate reporting of current stored PFS of the subscriptions.

-
Default value is "false" if omitted.
	PfdDetermination

	notifyUri
	Uri
	M
	1
	Identifies the recipient of notifications sent by PFDF for this subscription.
	

	pfds
	array(PfdContent)
	C
	1..N
	Identifies the currently available PFDs matching the subscription.

Shall be included in the subscription response, when the PfdDetermination feature is supported, the "immRep" attribute was set to "true" in the request, and there are currently available PFDs matching the subscription.
	PfdDetermination

	supportedFeatures
	SupportedFeatures
	M
	1
	List of supported features used as described in clause 5.8.
	


5.6.2.4
Type: PfdChangeNotification

Table 5.6.2.4-1: Definition of type PfdChangeNotification

	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	applicationId
	ApplicationId
	M
	1
	Identifier of an application.
	

	removalFlag
	boolean
	O
	0..1
	Indication of removal of PFDs for an existing application identifier.

Set to "true" indicates the PFDs are removed.
Set to "false" indicates the PFDs are not removed.
Default value is "false" if omitted.
	

	partialFlag
	boolean
	O
	0..1
	Indication of partial update of PFDs for an existing application identifier if this operation is supported according to feature negotiation.

Set to "true" indicates partial update of PFDs for the included application identifier.
Set to "false" indicates not partial update of PFDs for the included application identifier.
Default value is "false" if omitted.
	PartialUpdate

	pfds
	array(PfdContent)
	O
	1..N
	PFD creation/update for the application identifier as specified in clause 6.3.3.5 of 3GPP TS 29.251 [12].
	


5.6.2.5
Type: PfdContent

Table 5.6.2.5-1: Definition of type PfdContent
	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	pfdId
	string
	C
	0..1
	Identifies a PFD of an application identifier. If PartialUpdate or PartialPull feature is supported, this attribute shall be provided by the PFDF. (NOTE 2)
	

	flowDescriptions
	array(string)
	O
	1..N
	Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic (e.g., the destination IP address and port for UL and/or the source IP address and port for DL). The content of the string has the same encoding as the IPFilterRule AVP value as defined in IETF RFC 6733 [18]. (NOTE 1)
	

	urls
	array(string)
	O
	1..N
	Indicates a URL or a regular expression which is used to match the significant parts of the URL. (NOTE 1)
	

	domainNames
	array(string)
	O
	1..N
	Indicates an FQDN or a regular expression as a domain name matching criteria. (NOTE 1)
	

	dnProtocol
	DomainNameProtocol
	C
	0..1
	Indicates the additional protocol and protocol field for domain names to be matched, it may only be provided when domainNames attribute is present.
	DomainNameProtocol

	sourceNfType
	NFType
	O
	0..1
	Indicates the source NF type of this PFD.

The absence of this attribute indicates that this PFD was generated by the AF.
	PfdDetermination

	NOTE 1:
If a PFD contains multiple filter types, the PFD is only matched when every filter type contained in the PFD has a matching value.

NOTE 2:
If the pfdId is not provided to the NWDAF, then the NWDAF will not be able to update the corresponding entries.


5.6.2.6
Type: PfdChangeReport

Table 5.6.2.6-1: PfdChangeReport
	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	pfdError
	ProblemDetails
	M
	1
	More information on the error shall be provided in the "cause" attribute of the "ProblemDetails" structure.

The "cause" attribute in the ProblemDetails shall be set to one of following application errors (see table 5.2.7.1-1 of 3GPP TS 29.500 [5]):

- SYSTEM_FAILURE
- INSUFFICIENT_RESOURCES
- UNSPECIFIED_NF_FAILURE
	

	applicationId
	array(ApplicationId)
	M
	1..N
	Indicates the application identifier(s) which PFD(s) are failed to be added or modified.
	


5.6.2.7
Type: NotificationPush

Table 5.6.2.7-1: Definition of type NotificationPush
	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	appIds
	array(ApplicationId)
	M
	1..N
	Identifiers of one or more applications.
	

	allowedDelay
	DurationSec
	O
	0..1
	Indicates the time limit that the NF service consumer shall retrieve the PFD(s).
	

	pfdOp
	PfdOperation
	O
	0..1
	Indication the operation of the PFDs.
	


5.6.2.8
Type: ApplicationForPfdRequest
Table 5.6.2.8-1: Definition of ApplicationForPfdRequest
	Attribute name
	Data type
	P
	Cardinality
	Description
	Applicability

	applicationId
	ApplicationId
	M
	1
	Identifier of an application.
	

	pfdTimestamp
	DateTime
	O
	0..1
	The value represents the UTC time set for the PFD(s) of the application identifier provisioning by the PFDF.
	


5.6.3
Simple data types and enumerations

5.6.3.1
Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.6.3.2
Simple data types

The simple data types defined in table 5.6.3.2-1 shall be supported.

Table 5.6.3.2-1: Simple data types

	Type Name
	Type Definition
	Description
	Applicability

	
	
	
	


5.6.3.3
Enumeration: PfdOperation

Table 5.6.3.3-1: Enumeration PfdOperation

	Enumeration value
	Description
	Applicability

	RETRIEVE
	Indicates to retrieve the PFD(s) for the application identifier(s). The NF service consumer determines which pull procedure can be applied to retrieve the PFD(s).
	

	FULLPULL
	Indicates to retrieve the PFD(s) for the application identifier(s) by the full pull procedure. 
	

	PARTIALPULL
	Indicates to retrieve the PFD(s) for the application identifier(s) by the partial pull procedure. The NF consumer may decide to use full pull procedure instead if needed. 
	

	REMOVE
	Indicates to remove the PFD(s) for the application identifier(s).
	


5.7
Error handling

5.7.1
General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [5].

For the Nnef_PFDmanagement API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [6].

Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [5] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [5].

In addition, the requirements in the following clauses shall apply.
5.7.2
Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the Nnef_PFDmanagement API.

5.7.3
Application Errors

The application errors defined for the Nnef_PFDmanagement service are listed in table 5.7.3-1.

Table 5.7.3-1: Application errors

	Application Error
	HTTP status code
	Description

	SYSTEM_FAILURE
	500 Internal Server Error
	Something functions wrongly in PFD provisioning or the PFD provisioning does not function at all. (NOTE)

	INSUFFICIENT_RESOURCE
	500 Internal Server Error
	There is limitation for resource storage. (NOTE)

	UNSPECIFIED_NF_FAILURE
	500 Internal Server Error
	Unspecified reason. (NOTE)

	NOTE:
This application error is included in the responses to the POST request of PFD change notification/Notification PUSH.


5.8
Feature negotiation

The optional features in table 5.8-1 are defined for the Nnef_PFDmanagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

	Feature number
	Feature Name
	Description

	1
	PartialUpdate
	The PFDF can use this feature for partial update of PFDs.

	2
	DomainNameProtocol
	This feature supports the additional protocol matching condition for the domain name in PFD data.

	3
	PfdChgSubsUpdate
	The NF service consumer can use this feature for updating the PFD change subscription.

	4
	ES3XX
	Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

	5
	PartialPull
	The NF service consumer can use this feature to retrieve the PFDs by the partial update.

	6
	NotificationPush
	The PFDF can use this feature to notify the NF service consumer to retrieve the PFDs.

	7
	CachingTimer
	The PFDF and the NF service consumer can use this feature to support caching timer with unit of duration second.

	8
	PfdDetermination
	The PFDF can use this feature to provide the source NF type of the PFD to the consumer. This feature also indicates the support of immediate report within the subscription response.


5.9
Security

As indicated in 3GPP TS 33.501 [14] and 3GPP TS 29.500 [5], the access to the Nnef_PFDmanagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [15]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [13]) plays the role of the authorization server.

If OAuth2 is used, an NF service consumer, prior to consuming services offered by the Nnef_PFDmanagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [13], clause 5.4.2.2.

NOTE:
When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF service consumer used for discovering the Nnef_PFDmanagement service.

The Nnef_PFDmanagement API defines a single scope "nnef-pfdmanagement" for OAuth2 authorization (as specified in 3GPP TS 33.501 [14]) for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

A.1
General

The present Annex contains an OpenAPI [9] specification of HTTP messages and content bodies used by the Nnef_PFDmanagement API.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:
The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [17] and clause 5.3.1 of the 3GPP TS 29.501 [6] for further information).:

A.2
Nnef_PFDmanagement API

openapi: 3.0.0

info:

  title: Nnef_PFDmanagement Service API

  version: 1.4.0-alpha.2
  description: |

    Packet Flow Description Management Service.  
    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).  
    All rights reserved.

#

externalDocs:

  description: 3GPP TS 29.551 v19.2.0, 5G System; Packet Flow Description Management Service

  url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.551/'

#

servers:

  - url: '{apiRoot}/nnef-pfdmanagement/v1'

    variables:

      apiRoot:

        default: https://example.com

        description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

  - {}

  - oAuth2ClientCredentials:

    - nnef-pfdmanagement
paths:

  /applications:

    get:

      summary: Retrieve PFDs for all applications or for one or multiple applications with query parameter.

      tags:

        - PFD of applications

      operationId: Nnef_PFDmanagement_AllFetch

      parameters:

          - name: application-ids

            description: The required application identifier(s) for the returned PFDs.

            in: query

            required: true

            style: form

            explode: false

            schema:

              type: array

              items:

                $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

              minItems: 1

          - name: supported-features

            in: query

            description: To filter irrelevant responses related to unsupported features

            schema:

               $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

      responses:

        '200':

          description: >

            The PFDs for one or more application identifier(s) in the request URI are returned.

          content:

            application/json:

              schema:

                type: array

                items:

                  $ref: '#/components/schemas/PfdDataForApp'

                minItems: 0

        '307':

          $ref: 'TS29571_CommonData.yaml#/components/responses/307'
        '308':

          $ref: 'TS29571_CommonData.yaml#/components/responses/308'
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'

        '406':

          $ref: 'TS29571_CommonData.yaml#/components/responses/406'

        '414':

          $ref: 'TS29571_CommonData.yaml#/components/responses/414'

        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'

        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'

        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /applications/partialpull:

    post:

      summary: retrieve the PFD(s) by partial update

      operationId: Nnef_PFDmanagement_AppFetchPartialUpdate
      tags:

        - PFD of applications by partial update
      requestBody:

        required: true

        content:

          application/json:

            schema:

              type: array

              items:

                $ref: '#/components/schemas/ApplicationForPfdRequest'

              minItems: 1
      responses:

        '200':

          description: OK. Changed PFD(s) is returned

          content:

            application/json:

              schema:

                type: array

                items:

                  $ref: '#/components/schemas/PfdDataForApp'

                minItems: 1

        '204':

          description: The PFD(s) is not changed

        '307':

          $ref: 'TS29571_CommonData.yaml#/components/responses/307'
        '308':

          $ref: 'TS29571_CommonData.yaml#/components/responses/308'
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'

        '411':

          $ref: 'TS29571_CommonData.yaml#/components/responses/411'

        '413':

          $ref: 'TS29571_CommonData.yaml#/components/responses/413'

        '415':

          $ref: 'TS29571_CommonData.yaml#/components/responses/415'

        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'

        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'

        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /applications/{appId}:

    get:

      summary: Retrieve the PFD for an application.

      tags:

        - Individual application PFD

      operationId: Nnef_PFDmanagement_IndAppFetch

      parameters:

        - name: appId

          description: The required application identifier(s) for the returned PFDs.

          in: path

          required: true

          schema:

            type: string

        - name: supported-features

          in: query

          description: To filter irrelevant responses related to unsupported features

          schema:

             $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

      responses:

        '200':

          description: A representation of PFDs for an application in the request URI is returned.

          content:

            application/json:

              schema:

                $ref: '#/components/schemas/PfdDataForApp'

        '307':

          $ref: 'TS29571_CommonData.yaml#/components/responses/307'
        '308':

          $ref: 'TS29571_CommonData.yaml#/components/responses/308'
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'

        '406':

          $ref: 'TS29571_CommonData.yaml#/components/responses/406'

        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '414':

          $ref: 'TS29571_CommonData.yaml#/components/responses/414'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'

        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'

        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /subscriptions:

    post:

      summary: Subscribe the notification of PFD changes.

      tags:

        - PFD subscriptions

      operationId: Nnef_PFDmanagement_CreateSubscr

      requestBody:

        description: a PfdSubscription resource to be created.

        required: true

        content:

          application/json:

            schema:

              $ref: '#/components/schemas/PfdSubscription'

      callbacks:

        PfdChangeNotification:

          '{$request.body#/notifyUri}':

            post:

              summary: Notification of PFD change.

              tags:

                - PfdChangeNotification data

              operationId: Nnef_PFDmanagement_Notify

              requestBody:

                required: true

                content:

                  application/json:

                    schema:

                      type: array

                      items:

                        $ref: '#/components/schemas/PfdChangeNotification'

                      minItems: 1

              responses:

                '200':

                  description: >

                    The PFD operation in the notification is performed and the
                    PfdChangeReport indicates failure reason.

                  content:

                    application/json:

                      schema:

                        type: array

                        items:

                          $ref: '#/components/schemas/PfdChangeReport'

                        minItems: 1

                '204':

                  description: The PFD operation in the notification is performed successfully.

                '307':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/307'
                '308':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/308'
                '400':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/400'

                '401':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/401'

                '403':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/403'

                '404':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/404'

                '411':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/411'

                '413':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/413'

                '415':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/415'

                '429':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/429'

                '500':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/500'

                '502':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/502'

                '503':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/503'

                default:

                  $ref: 'TS29571_CommonData.yaml#/components/responses/default'

        NotificationPush:

          '{$request.body#/notifyUri}/notifypush':

            post:

              summary: Notification Push.

              tags:

                - NotificationPush data

              operationId: Nnef_PFDmanagement_PushNotify

              requestBody:

                required: true

                content:

                  application/json:

                    schema:

                      type: array

                      items:

                        $ref: '#/components/schemas/NotificationPush'

                      minItems: 1

              responses:

                '204':

                  description: Notificaiton PUSH is accepted.

                '307':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/307'
                '308':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/308'
                '400':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/400'

                '401':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/401'

                '403':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/403'

                '404':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/404'

                '411':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/411'

                '413':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/413'

                '415':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/415'

                '429':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/429'

                '500':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/500'

                '502':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/502'

                '503':

                  $ref: 'TS29571_CommonData.yaml#/components/responses/503'
                default:

                  $ref: 'TS29571_CommonData.yaml#/components/responses/default'

      responses:

        '201':

          description: >

            The creation of a PfdSubscription resource is confirmed and a representation of
            that resource is returned.

          content:

            application/json:

              schema:

                $ref: '#/components/schemas/PfdSubscription'

          headers:

            Location:

              description: >

                Contains the URI of the newly created resource, according to the structure
                {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}

              required: true

              schema:

                type: string
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'

        '411':

          $ref: 'TS29571_CommonData.yaml#/components/responses/411'

        '413':

          $ref: 'TS29571_CommonData.yaml#/components/responses/413'

        '415':

          $ref: 'TS29571_CommonData.yaml#/components/responses/415'

        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'
        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'
        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

  /subscriptions/{subscriptionId}:

    put:

      summary: Updates/replaces an existing subscription resource

      tags:

        - Individual PFD subscription

      operationId: Nnef_PFDmanagement_ModifySubscr

      parameters:

        - name: subscriptionId

          description: Identify the subscription.

          in: path

          required: true

          schema:

            type: string

      requestBody:

        description: Parameters to update/replace the existing subscription

        required: true

        content:

          application/json:

            schema:

              $ref: '#/components/schemas/PfdSubscription'

      responses:

        '200':

          description: OK (Successful update of the subscription)

          content:

            application/json:

              schema:

                $ref: '#/components/schemas/PfdSubscription'

        '307':

          $ref: 'TS29571_CommonData.yaml#/components/responses/307'
        '308':

          $ref: 'TS29571_CommonData.yaml#/components/responses/308'
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'

        '411':

          $ref: 'TS29571_CommonData.yaml#/components/responses/411'

        '413':

          $ref: 'TS29571_CommonData.yaml#/components/responses/413'

        '415':

          $ref: 'TS29571_CommonData.yaml#/components/responses/415'

        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'

        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'

        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'

    delete:

      summary: Delete a subscription of PFD change notification.

      tags:

        - Individual PFD subscription

      operationId: Nnef_PFDmanagement_Unsubscribe

      parameters:

        - name: subscriptionId

          description: Identify the subscription.

          in: path

          required: true

          schema:

            type: string

      responses:

        '204':

          description: >

            The PfdSubscription resource matching the subscriptionId was deleted successfully.

        '307':

          $ref: 'TS29571_CommonData.yaml#/components/responses/307'
        '308':

          $ref: 'TS29571_CommonData.yaml#/components/responses/308'
        '400':

          $ref: 'TS29571_CommonData.yaml#/components/responses/400'

        '401':

          $ref: 'TS29571_CommonData.yaml#/components/responses/401'

        '403':

          $ref: 'TS29571_CommonData.yaml#/components/responses/403'

        '404':

          $ref: 'TS29571_CommonData.yaml#/components/responses/404'
        '429':

          $ref: 'TS29571_CommonData.yaml#/components/responses/429'

        '500':

          $ref: 'TS29571_CommonData.yaml#/components/responses/500'

        '502':

          $ref: 'TS29571_CommonData.yaml#/components/responses/502'
        '503':

          $ref: 'TS29571_CommonData.yaml#/components/responses/503'

        default:

          $ref: 'TS29571_CommonData.yaml#/components/responses/default'
components:

  securitySchemes:

    oAuth2ClientCredentials:

      type: oauth2

      flows:

        clientCredentials:

          tokenUrl: '{nrfApiRoot}/oauth2/token'

          scopes:

            nnef-pfdmanagement: Access to the Nnef_PFDmanagement API

  schemas:

#

# STRUCTURED DATA TYPES

#

    PfdContent:

      description: Represents the content of a PFD for an application identifier.
      type: object

      properties:

        pfdId:

          type: string

          description: Identifies a PFD of an application identifier.
        flowDescriptions:

          type: array

          items:

            type: string

          minItems: 1

          description: >

            Represents a 3-tuple with protocol, server ip and server port for UL/DL
            application traffic.
        urls:

          type: array

          items:

            type: string

          minItems: 1

          description: >

            Indicates a URL or a regular expression which is used to match the significant parts
            of the URL.

        domainNames:

          type: array

          items:

            type: string

          minItems: 1

          description: Indicates an FQDN or a regular expression as a domain name matching criteria.

        dnProtocol:

          $ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

        sourceNfType:

          $ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

    PfdDataForApp:

      description: Represents the PFDs for an application identifier.
      type: object

      properties:

        applicationId:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

        pfds:

          type: array

          items:

            $ref: '#/components/schemas/PfdContent'

          minItems: 1
        cachingTime:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

        cachingTimer:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        pfdTimestamp:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

        partialFlag:

          type: boolean

          description: >
            Indication of partial update of PFDs for an existing application identifier if it is 
            included and set to true. Otherwise set to false indicates not supporting partial 
            update of PFDs for an existing application identifier. The default value false 
            applies if the attribute is not present.
          default: false

        supportedFeatures:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'
      required:

        - applicationId

    PfdSubscription:

      description: Represents a PFD subscription.
      type: object

      properties:

        applicationIds:

          type: array

          items:

            $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

          minItems: 1
        immRep: 

          type: boolean
        notifyUri:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

        pfds:

          type: array

          items:

            $ref: '#/components/schemas/PfdContent'

          minItems: 1
        supportedFeatures:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

      required:

        - notifyUri

        - supportedFeatures

    PfdChangeNotification:

      description: Represents information related to a notification of PFD change.
      type: object

      properties:

        applicationId:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

        removalFlag:

          type: boolean

          description: >
            Indication of removal of PFDs for an existing application identifier.

            Set to true indicates the PFDs are removed.

            Set to false indicates the PFDs are not removed.

            Default value is false if omitted.
          default: false

        partialFlag:

          type: boolean

          description: >

            Indication of partial update of PFDs for an existing application identifier 

            if this operation is supported according to feature negotiation.

            Set to true indicates partial update PFDs for the included application identifier.

            Set to false indicates not partial update PFDs for the included application identifier.

            Default value is "false" if omitted.
          default: false

        pfds:

          type: array

          items:

            $ref: '#/components/schemas/PfdContent'

          minItems: 1
      required:

        - applicationId

    NotificationPush:

      description: >

        Represents the information to be used by the NF service consumer to retrieve the
        PFDs and/or remove the PFDs of the applicable application identifier(s).
      type: object

      properties:

        appIds:

          type: array

          items:

            $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

          minItems: 1
        allowedDelay:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

        pfdOp:

          $ref: '#/components/schemas/PfdOperation'

      required:

        - appIds

    PfdChangeReport:

      description: Represents an error report on PFD change.
      type: object

      properties:

        pfdError:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

        applicationId:

          type: array

          items:

            $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

          minItems: 1

      required:

        - pfdError

        - applicationId

    ApplicationForPfdRequest:

      description: Contains the application identifier(s) for the PFD(s) request.
      type: object

      properties:

        applicationId:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

        pfdTimestamp:

          $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

      required:

        - applicationId

#

# ENUMERATIONS

#
    PfdOperation:

      description: Indicates the operation to be applied on PFD(s).
      anyOf:

      - type: string

        enum:

          - RETRIEVE

          - FULLPULL

          - PARTIALPULL

          - REMOVE

      - type: string

        description: >

          This string provides forward-compatibility with future extensions to the enumeration

          and is not used to encode content defined in the present version of this API.
Annex B (informative):
Change history

	Change history

	Date
	Meeting
	TDoc
	CR
	Rev
	Cat
	Subject/Comment
	New version

	2018-01
	
	
	
	
	
	TS skeleton of PFD Management Service specification
	0.0.0

	2018-01
	CT3#94
	
	
	
	
	Includes the following contribution agreed by CT3 at CT3#94:
C3-180045, C3-180270, C3-180271.
	0.1.0

	2018-03
	CT3#95
	
	
	
	
	Includes the following contribution agreed by CT3 at CT3#95:

C3-181100, C3-181262, C3-181102, C3-181103, C3-181314.
	0.2.0

	2018-04
	CT3#96
	
	
	
	
	Includes the following contribution agreed by CT3 at CT3#96:

C3-182153, C3-182411, C3-182412, C3-182413, C3-182414, C3-182477, C3-182478.
	0.3.0

	2018-05
	CT3#97
	
	
	
	
	Includes the following contribution agreed by CT3 at CT3#97:

C3-183115, C3-183557, C3-183558, C3-183560.
	0.4.0

	2018-06
	CT#80
	CP-181029
	
	
	
	TS sent to plenary for approval
	1.0.0

	2018-06
	CT#80
	CP-181029
	
	
	
	TS approved by plenary
	15.0.0

	2018-09
	CT#81
	CP-182015
	0001
	2
	F
	Protocol error statement
	15.1.0

	2018-09
	CT#81
	CP-182015
	0002
	1
	F
	Description of Structured data types
	15.1.0

	2018-12
	CT#82
	CP-183205
	0003
	
	F
	Default value for apiRoot
	15.2.0

	2018-12
	CT#82
	CP-183205
	0004
	3
	F
	Cardinality
	15.2.0

	2018-12
	CT#82
	CP-183205
	0005
	1
	F
	Correct Nnef_PFDmanagement API
	15.2.0

	2018-12
	CT#82
	CP-183205
	0006
	
	F
	Location Header
	15.2.0

	2018-12
	CT#82
	CP-183205
	0007
	1
	F
	Alignment of openAPI
	15.2.0

	2018-12
	CT#82
	CP-183205
	0008
	
	F
	API version update
	15.2.0

	2018-12
	CT#82
	CP-183205
	0009
	
	F
	Security
	15.2.0

	2018-12
	CT#82
	CP-183205
	0010
	
	F
	Content type
	15.2.0

	2019-03
	CT#83
	CP-190116
	0011
	1
	F
	Fetch PFD for all applications
	15.3.0

	2019-03
	CT#83
	CP-190116
	0014
	1
	F
	Correction of resource URIs
	15.3.0

	2019-03
	CT#83
	CP-190130
	0010
	1
	B
	PUT for PFD change subscription
	16.0.0

	2019-03
	CT#83
	CP-190121
	0012
	3
	B
	PFD extension
	16.0.0

	2019-03
	CT#83
	
	
	
	
	Open API version update by MCC
	16.0.0

	2019-06
	CT#84
	CP-191083
	0016
	2
	A
	Precedence of OpenAPI file
	16.1.0

	2019-06
	CT#84
	CP-191083
	0019
	
	A
	Correction to Notification of PFD change
	16.1.0

	2019-06
	CT#84
	CP-191083
	0021
	
	A
	Copyright Note in YAML file
	16.1.0

	2019-06
	CT#84
	CP-191101
	0023
	2
	F
	API version Update
	16.1.0

	2019-09
	CT#85
	CP-192149
	0025
	2
	A
	Correct presence condition in PFD definition
	16.2.0

	2020-03
	CT#87e
	CP-200215
	0026
	
	F
	Reference of Error code
	16.3.0

	2020-03
	CT#87e
	CP-200216
	0027
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	16.3.0

	2020-06
	CT#88e
	CP-201244
	0028
	
	F
	Non-unique operation identifiers
	16.4.0

	2020-06
	CT#88e
	CP-201244
	0029
	1
	F
	Storage of YAML files in ETSI Forge
	16.4.0

	2020-06
	CT#88e
	CP-201256
	0030
	1
	F
	URI of the Nnef_PFDmanagement service
	16.4.0

	2020-06
	CT#88e
	CP-201244
	0031
	1
	F
	Optionality of ProblemDetails
	16.4.0

	2020-06
	CT#88e
	CP-201244
	0032
	1
	F
	Supported headers, Resource Data type and yaml mapping
	16.4.0

	2020-06
	CT#88e
	CP-201255
	0033
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	16.4.0

	2020-09
	CT#89e
	CP-202056
	0035
	
	A
	Correction to the PFD change notification
	16.5.0

	2020-09
	CT#89e
	CP-202205
	0038
	2
	B
	Notification PUSH
	17.0.0

	2020-09
	CT#89e
	CP-202085
	0039
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.0.0

	2020-12
	CT#90e
	CP-203078
	0041
	2
	A
	Essential corrections and alignments
	17.1.0

	2020-12
	CT#90e
	CP-203135
	0042
	1
	F
	Corrections to Notification Push procedure
	17.1.0

	2020-12
	CT#90e
	CP-203120
	0046
	1
	A
	Correction to PFD retrieval in PULL mode
	17.1.0

	2020-12
	CT#90e
	CP-203120
	0049
	
	A
	Correction to notification URI of PFD change notification
	17.1.0

	2020-12
	CT#90e
	CP-203135
	0050
	1
	B
	PULL mode enhancement
	17.1.0

	2020-12
	CT#90e
	CP-203135
	0051
	1
	B
	Remove the EN within the PushNotification
	17.1.0

	2020-12
	CT#90e
	CP-203139
	0053
	1
	A
	Storage of YAML files in 3GPP Forge
	17.1.0

	2020-12
	CT#90e
	CP-203135
	0054
	
	F
	NotificationPush data type definition
	17.1.0

	2020-12
	CT#90e
	CP-203141
	0055
	1
	F
	Non-unique operation identifiers
	17.1.0

	2020-12
	CT#90e
	CP-203153
	0056
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.1.0

	2021-03
	CT#91e
	CP-210191
	0058
	1
	A
	Support of stateless NFs
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0059
	5
	B
	Resolve the FFS for notification push
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0060
	3
	B
	Partial pull update
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0061
	5
	F
	Corrections on partial pull
	17.2.0

	2021-03
	CT#91e
	CP-210218
	0062
	
	F
	OpenAPI reference
	17.2.0

	2021-03
	CT#91e
	CP-210222
	0064
	1
	A
	notifyUri used by notification
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0065
	
	F
	Invalid Nnef_PFDmanagement OpenAPI file
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0066
	
	B
	Correction to the name of custom operation
	17.2.0

	2021-03
	CT#91e
	CP-210215
	0067
	1
	B
	Overall description of notification push
	17.2.0

	2021-03
	CT#91e
	CP-210219
	0069
	
	F
	Adding some missing description fields to data type definitions in OpenAPI specification files
	17.2.0

	2021-03
	CT#91e
	CP-210200
	0072
	
	A
	Datatype and figure corrections
	17.2.0

	2021-03
	CT#91e
	CP-210220
	0073
	
	F
	Custom header clarification
	17.2.0

	2021-03
	CT#91e
	CP-210221
	0074
	1
	F
	Terminology alignment usage of NF service consumer
	17.2.0

	2021-03
	CT#91e
	CP-210240
	0076
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.2.0

	2021-06
	CT#92e 
	CP-211208
	0079
	1
	A
	Correction of request URI in 4.2.2.2
	17.3.0

	2021-06
	CT#92e 
	CP-211200
	0081
	1
	A
	Temporary and Permanent Redirection
	17.3.0

	2021-06
	CT#92e 
	CP-211265
	0083
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.3.0

	2021-09
	CT#93e
	CP-212084
	0084
	1
	F
	Correction to custom operation partialpull
	17.4.0

	2021-09
	CT#93e
	CP-212217
	0085
	1
	F
	mandate the attribute pfdOp
	17.4.0

	2021-09
	CT#93e
	CP-212190
	0088
	
	A
	default caching time value
	17.4.0

	2021-09
	CT#93e
	CP-212190
	0091
	
	A
	Presentation condition of pfdId attribute
	17.4.0

	2021-09
	CT#93e
	CP-212223
	0092
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.4.0

	2021-12
	CT#94e
	CP-213239
	0093
	
	F
	Aligning API URI with SBI template
	17.5.0

	2021-12
	CT#94e
	CP-213243
	0094
	
	F
	Correction to presence condition of supportedFeatures in PfdSubscription
	17.5.0

	2021-12
	CT#94e
	CP-213215
	0097
	1
	A
	Correction to PFD management in push mode
	17.5.0

	2021-12
	CT#94e
	CP-213224
	0099
	1
	A
	Adding supported features in GET response
	17.5.0

	2021-12
	CT#94e
	CP-213246
	0101
	
	F
	Update of OpenAPI version and TS version in externalDocs field
	17.5.0

	2022-03
	CT#95e
	CP-220201
	0102
	1
	F
	Description fields format alignment
	17.6.0

	2022-03
	CT#95e
	CP-220194
	0103
	
	F
	Update of info and externalDocs fields
	17.6.0

	2022-06
	CT#96
	CP-221119
	0106
	1
	F
	Correcting the description of the encoding used for flow descriptions
	17.7.0

	2022-06
	CT#96
	CP-221152
	0107
	
	F
	Update of info and externalDocs fields
	17.7.0

	2022-09
	CT#97e
	CP-222122
	0108
	1
	F
	Corrections to caching timer
	17.8.0

	2022-09
	CT#97e
	CP-222121
	0109
	
	F
	Update of info and externalDocs fields
	17.8.0

	2022-12
	CT#98e
	CP-223191
	0110
	
	F
	Adding the mandatory error code 502 Bad Gateway
	18.0.0

	2022-12
	CT#98e
	CP-223192
	0111
	1
	F
	PfdOperation enumeration definition in the OpenAPI file
	18.0.0

	2022-12
	CT#98e
	CP-223189
	0112
	
	F
	Update of info and externalDocs fields
	18.0.0

	2023-03
	CT#99
	CP-230147
	0113
	1
	B
	Support NWDAF fetches PFD information in Nnef_PFDmanagement API
	18.1.0

	2023-06
	CT#100
	CP-231131
	0114
	1
	F
	Corrections to the redirection mechanism description
	18.2.0

	2023-06
	CT#100
	CP-231133
	0115
	1
	F
	Error response for Notification PUSH
	18.2.0

	2023-12
	CT#102
	CP-233278
	0117
	
	A
	Corrections to boolean type definitions
	18.3.0

	2023-12
	CT#102
	CP-233229
	0118
	1
	F
	Updating the obsoleted IETF HTTP RFCs
	18.3.0

	2023-12
	CT#102
	CP-233237
	0120
	
	F
	Update of info and externalDocs fields
	18.3.0

	2024-03
	CT#103
	CP-240162
	0121
	1
	B
	Support of NF type in the PFD Notification
	18.4.0

	2024-03
	CT#103
	CP-240166
	0123
	
	F
	Update of info and externalDocs fields
	18.4.0

	2024-06
	CT#104
	CP-241093
	0124
	
	F
	Callback correction to Nnef_PFDmanagement API
	18.5.0

	2024-06
	CT#104
	CP-241077
	0125
	
	F
	Clarification for the source NF type of PFD
	18.5.0

	2024-06
	CT#104
	CP-241095
	0126
	1
	F
	Cardinality correction
	18.5.0

	2024-06
	CT#104
	CP-241132
	0128
	
	A
	Essential corrections to the definition of the "Retrieval of PFDs by the partial pull" procedure
	18.5.0

	2024-06
	CT#104
	CP-241079
	0129
	1
	B
	Updates to support PFD Determination
	18.5.0

	2024-06
	CT#104
	CP-241095
	0130
	
	F
	Corrections on the cardinality and the reference number
	18.5.0

	2024-06
	CT#104
	CP-241086
	0132
	
	F
	Update of info and externalDocs fields
	18.5.0

	2024-12
	CT#106
	CP-243091
	0133
	3
	F
	Updates to PFD Management
	19.0.0

	2024-12
	CT#106
	CP-243089
	0134
	1
	F
	Corrections related to PfdDetermination
	19.0.0

	2025-03
	CT#107
	CP-250086
	0135
	1
	F
	Correction to the definition of query parameter application-ids
	19.1.0

	2025-03
	CT#107
	CP-250129
	0136
	
	F
	Update of info and externalDocs fields
	19.1.0

	2025-06
	CT#108
	C3-252598
	0137
	
	F
	Update of info and externalDocs fields
	19.2.0


NEF





PFDF





Nnef





SMF





Nnef_PFDmanagement





NWDAF





NEF





PFDF





SMF





N29





NWDAF








_1676811856.vsd
2. 201 Created


1. POST… /subscriptions


NF service consumer


PFDF



_1732714428.doc
[image: image1.png]~

5G







_1779028439.doc




POST …/applications/partialpull







2a. 200 OK



2b. 204 No Content







NF Service Consumer







PFDF














_1683967131.vsd
PFDF


1a.GET …/applications/{appId}
1b. GET …/applications


2. 200 OK


NF Service Consumer



/applications
/{appId}
{apiRoot}/nnef-pfdmanagement/v1
/subscriptions
/{subscriptionId}
/partialpull



_1676811055.vsd
PFDF


2a. 200 OK
2b. 204 No Content


1. POST{notifyUri}(array(PfdChangeNotification))


NF service consumer



_1676811057.vsd
PFDF


2. 204 No Content


1. POST {notifyUri}/notifypush


NF service consumer



_1676811059.vsd
2. 204 No Content


1. DELETE… /subscriptions/{subscriptionId}


NF service consumer


PFDF



_1676811053.vsd
2. 200 OK


1. PUT… /subscriptions/{subscriptionId}


NF service consumer


PFDF



