| 3GPP TS 29.482 V1.1.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service Enabler Architecture Layer for Verticals (SEAL); Artificial Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	22

1	Scope	24

2	References	25

3	Definitions, symbols and abbreviations	26

3.1	Definitions	26

3.2	Symbols	26

3.3	Abbreviations	26

4	Overview	27

5	Services offered by AIMLE	28

5.1	Introduction	28

5.2	Services offered by the AIMLE Server	28

5.2.1	AIMLES_ContextTransfer Service	28

5.2.1.1	Service Description	28

5.2.1.2	Service Operations	28

5.2.1.2.1	Introduction	28

5.2.1.2.2	AIMLES_ContextTransfer_Request	29

5.2.1.2.2.1	General	29

5.2.1.2.2.2	AIMLE Context Transfer Request	29

5.2.2	AIMLES_DataManagement Service	30

5.2.2.1	Service Description	30

5.2.2.2	Service Operations	30

5.2.2.2.1	Introduction	30

5.2.2.2.2	AIMLES_DataManagement_Subscribe	30

5.2.2.2.3	AIMLES_DataManagement_Update	31

5.2.2.2.4	AIMLES_DataManagement_Unsubscribe	32

5.2.2.2.5	AIMLES_DataManagement_Notify	33

5.2.3	AIMLES_FLMemberGroupSupport Service	34

5.2.3.1	Service Description	34

5.2.3.2	Service Operations	34

5.2.3.2.1	Introduction	34

5.2.3.2.2	AIMLES_FLMemberGroupSupport_Create	34

5.2.3.2.3	AIMLES_FLMemberGroupSupport_Query	35

5.2.3.2.4	AIMLES_FLMemberGroupSupport_Update	35

5.2.3.2.5	AIMLES_FLMemberGroupSupport_Delete	36

5.2.4	AIMLES_AIMLEServiceOperationsManagement Service	38

5.2.4.1	Service Description	38

5.2.4.2	Service Operations	38

5.2.4.2.1	Introduction	38

5.2.4.2.2	AIMLES_AIMLEServiceOperationsManagement_Request	38

5.2.5	AIMLES_HierarchicalComputingAssist Service	40

5.2.5.1	Service Description	40

5.2.5.2	Service Operations	40

5.2.5.2.1	Introduction	40

5.2.5.2.2	AIMLES_HierarchicalComputingAssist_Request	40

5.2.6	AIMLES_AIMLEClientDiscovery Service	42

5.2.6.1	Service Description	42

5.2.6.2	Service Operations	42

5.2.6.2.1	Introduction	42

5.2.6.2.2	AIMLES_AIMLEClientDiscovery_Request	42

5.2.6.2.2.1	General	42

5.2.6.2.2.2	AIMLE Clients discovery request	42

5.2.7	AIMLES_AIMLEClientSelection Service	43

5.2.7.1	Service Description	43

5.2.7.2	Service Operations	43

5.2.7.2.1	Introduction	43

5.2.7.2.2	AIMLES_ClientSelection_Subscribe	43

5.2.7.2.3	AIMLES_DataManagement_Update	44

5.2.7.2.4	AIMLES_DataManagement_Unsubscribe	45

5.2.7.2.5	AIMLES_DataManagement_Notify	46

5.2.7.2.6	AIMLES_AIMLEClientSelection_Request	46

5.2.8	AIMLES_MLModelPerfMonitor Service	49

5.2.8.1	Service Description	49

5.2.8.2	Service Operations	49

5.2.8.2.1	Introduction	49

5.2.8.2.2	AIMLES_MLModelPerfMonitor_Subscribe	49

5.2.8.2.3	AIMLES_MLModelPerfMonitor_Update	50

5.2.8.2.4	AIMLES_MLModelPerfMonitor_Unsubscribe	51

5.2.8.2.5	AIMLES_MLModelPerfMonitor_Notify	51

5.2.9	AIMLES_AssistedMLModelSelection Service	53

5.2.9.1	Service Description	53

5.2.9.2	Service Operations	53

5.2.9.2.1	Introduction	53

5.2.9.2.2	AIMLES_AssistedMLModelSelection_Subscribe	53

5.2.9.2.3	AIMLES_AssistedMLModelSelection_Update	54

5.2.9.2.4	AIMLES_AssistedMLModelSelection_Unsubscribe	55

5.2.9.2.5	AIMLES_AssistedMLModelSelection_Notify	56

5.2.10	Aimles_MLModelRetrieval Service	57

5.2.10.1	Service Description	57

5.2.10.2	Service Operations	57

5.2.10.2.1	Introduction	57

5.2.10.2.2	Aimles_MLModelRetrieval_Request	57

5.2.10.2.3	Aimles_MLModelRetrieval_Subscribe	58

5.2.10.2.4	Aimles_MLModelRetrieval_Update	59

5.2.10.2.5	Aimles_MLModelRetrieval_Unsubscribe	60

5.2.10.2.6	Aimles_MLModelRetrieval_Notify	60

5.2.11	AIMLES_TLModelSelectionAssistance Service	62

5.2.11.1	Service Description	62

5.2.11.2	Service Operations	62

5.2.11.2.1	Introduction	62

5.2.11.2.2	AIMLES_TLModelSelectionAssistance_Request	62

5.2.12	AIMLES_SplitOpNodeRegistration	64

5.2.12.1	Service Description	64

5.2.12.2	Service Operations	64

5.2.12.2.1	Introduction	64

5.2.12.2.2	Aimles_SplitOpNodeRegistration_Request	64

5.2.12.2.3	Aimles_ SplitOpNodeRegistration_Update service operation	65

5.2.12.2.3.1	General	65

5.2.12.2.3.2	AIMLE split operation node registration update	65

5.2.12.2.4	Aimles_ SplitOpNodeRegistration_Deregister service operation	66

5.2.12.2.4.1	General	66

5.2.12.2.4.2	AIML split operation node deregistration	66

5.3	Services offered by the AIMLE Repository	67

5.3.1	MLR_MLModelManagement Service	67

5.3.1.1	Service Description	67

5.3.1.2	Service Operations	67

5.3.1.2.1	Introduction	67

5.3.1.2.2	MLR_MLModelManagement_Store	67

5.3.1.2.2.2	ML Models Storage Creation	67

5.3.1.2.2.3	ML Models Storage Update	68

5.3.1.2.2.4	ML Models Storage Deletion	68

5.3.2	MLR_ModelInformationDiscovery Service	70

5.3.2.1	Service Description	70

5.3.2.2	Service Operations	70

5.3.2.2.1	Introduction	70

5.3.2.2.2	MLR_MLModelInformationDiscovery_Request	70

5.3.2.2.2.1	General	70

5.3.2.2.2.2	ML model discovery request	70

5.3.3	MLR_FLEvents Service	71

5.3.3.1	Service Description	71

5.3.3.2	Service Operations	71

5.3.3.2.1	Introduction	71

5.3.3.2.2	MLR_FLEvents_Subscribe	71

5.3.3.2.3	MLR_FLEvents_Update	72

5.3.3.2.4	MLR_FLEvents_Unsubscribe	73

5.3.3.2.5	MLR_FLEvents_Notify	73

5.3.4	MLR_FLMember Service	75

5.3.4.1	Service Description	75

5.3.4.2	Service Operations	75

5.3.4.2.1	Introduction	75

5.3.4.2.2	MLR_FLMember_Register	75

5.3.4.2.2.1	General	75

5.3.4.2.2.2	MLR FL Member Register	75

5.3.4.2.3	MLR_FLMember_Query	76

5.3.4.2.3.1	General	76

5.3.4.2.3.2	MLR FL Member Query	76

5.3.4.2.4	MLR_FLMember_Update_Register	76

5.3.4.2.4.1	General	76

5.3.4.2.4.2	MLR FL Member Update Register	76

5.3.4.2.5	MLR_FLMember_Deregister	77

5.3.4.2.5.1	General	77

5.3.4.2.5.2	MLR FL Member Deregister	77

6	API Definitions	79

6.1	AIMLE Server APIs	79

6.1.1	AIMLES_ContextTransfer API	79

6.1.1.1	Introduction	79

6.1.1.2	Usage of HTTP	79

6.1.1.3	Resources	79

6.1.1.4	Custom Operations without associated resources	79

6.1.1.4.1	Overview	79

6.1.1.4.2	Operation: RequestTrans	80

6.1.1.4.2.2	Operation Definition	80

6.1.1.5	Notifications	81

6.1.1.6	Data Model	81

6.1.1.6.1	General	81

6.1.1.6.2	Structured data types	81

6.1.1.6.2.2	Type: TransReq	81

6.1.1.6.2.3	Type: AimleServStatus	82

6.1.1.6.3	Simple data types and enumerations	82

6.1.1.6.3.2	Simple data types	82

6.1.1.6.3.3	Enumeration: AimleServOpStatus	82

6.1.1.6.4	Data types describing alternative data types or combinations of data types	83

6.1.1.6.5	Binary data	83

6.1.1.7	Error Handling	83

6.1.1.7.1	General	83

6.1.1.7.2	Protocol Errors	83

6.1.1.7.3	Application Errors	83

6.1.1.8	Feature negotiation	83

6.1.1.9	Security	84

6.1.2	AIMLES_DataManagement API	85

6.1.2.1	Introduction	85

6.1.2.2	Usage of HTTP	85

6.1.2.3	Resources	85

6.1.2.3.1	Overview	85

6.1.2.3.2	Resource: AIMLE Data Management Assistance Subscriptions	86

6.1.2.3.3	Resource: Individual AIMLE Data Management Assistance Subscription	87

6.1.2.4	Custom Operations without associated resources	92

6.1.2.5	Notifications	92

6.1.2.5.1	General	92

6.1.2.5.2	AIMLE Data Management Assistance Event Notification	92

6.1.2.6	Data Model	93

6.1.2.6.1	General	93

6.1.2.6.2	Structured data types	94

6.1.2.6.3	Simple data types and enumerations	97

6.1.2.6.3.3	Enumeration: DataMgmtOp	97

6.1.2.6.4	Data types describing alternative data types or combinations of data types	98

6.1.2.6.5	Binary data	98

6.1.2.7	Error Handling	98

6.1.2.7.1	General	98

6.1.2.7.2	Protocol Errors	98

6.1.2.7.3	Application Errors	98

6.1.2.8	Feature negotiation	98

6.1.2.9	Security	98

6.1.3	AIMLES_FLMemberGroupSupport API	99

6.1.3.1	Introduction	99

6.1.3.2	Usage of HTTP and common API related aspects	99

6.1.3.3	Resources	99

6.1.3.3.1	Overview	99

6.1.3.3.2	Resource: FL Member Group Support Configurations	100

6.1.3.3.3	Resource: Individual FL Member Group Support Configuration	101

6.1.3.4	Custom Operations without associated resources	106

6.1.3.4.1	Overview	106

6.1.3.5	Notifications	106

6.1.3.6	Data Model	106

6.1.3.6.1	General	106

6.1.3.6.2	Structured data types	106

6.1.3.6.3	Simple data types and enumerations	108

6.1.3.6.4	Data types describing alternative data types or combinations of data types	109

6.1.3.6.5	Binary data	109

6.1.3.7	Error Handling	109

6.1.3.7.1	General	109

6.1.3.7.2	Protocol Errors	109

6.1.3.7.3	Application Errors	109

6.1.3.8	Feature negotiation	109

6.1.3.9	Security	110

6.1.4	AIMLES_AIMLEServiceOperationsManagement API	111

6.1.4.1	Introduction	111

6.1.4.2	Usage of HTTP and common API related aspects	111

6.1.4.3	Resources	111

6.1.4.4	Custom Operations without associated resources	111

6.1.4.4.1	Overview	111

6.1.4.4.2	Operation: RequestServOpMngt	111

6.1.4.5	Notifications	112

6.1.4.6	Data Model	112

6.1.4.6.1	General	112

6.1.4.6.2	Structured data types	113

6.1.4.6.3	Simple data types and enumerations	115

6.1.4.6.4	Data types describing alternative data types or combinations of data types	115

6.1.4.6.5	Binary data	116

6.1.4.7	Error Handling	116

6.1.4.7.1	General	116

6.1.4.7.2	Protocol Errors	116

6.1.4.7.3	Application Errors	116

6.1.4.8	Feature negotiation	116

6.1.4.9	Security	116

6.1.5	AIMLES_HierarchicalComputingAssist API	117

6.1.5.1	Introduction	117

6.1.5.2	Usage of HTTP and common API related aspects	117

6.1.5.3	Resources	117

6.1.5.4	Custom Operations without associated resources	117

6.1.5.4.1	Overview	117

6.1.5.4.2	Operation: RequestServOpMngt	117

6.1.5.5	Notifications	118

6.1.5.6	Data Model	118

6.1.5.6.1	General	118

6.1.5.6.2	Structured data types	119

6.1.5.6.3	Simple data types and enumerations	120

6.1.5.6.4	Data types describing alternative data types or combinations of data types	121

6.1.5.6.5	Binary data	121

6.1.5.7	Error Handling	121

6.1.5.7.1	General	121

6.1.5.7.2	Protocol Errors	121

6.1.5.7.3	Application Errors	121

6.1.5.8	Feature negotiation	121

6.1.5.9	Security	122

6.1.6	AIMLES_AIMLEClientDiscovery API	123

6.1.6.1	Introduction	123

6.1.6.2	Usage of HTTP and common API related aspects	123

6.1.6.3	Resources	123

6.1.6.3.1	Overview	123

6.1.6.3.2	Resource: AIMLE Clients	124

6.1.6.3.2.1	Description	124

6.1.6.3.2.2	Resource Definition	124

6.1.6.3.2.3	Resource Standard Methods	124

6.1.6.3.2.3.1	GET	124

6.1.6.3.2.4	Resource Custom Operations	125

6.2.2.4	Custom Operations without associated resources	125

6.2.2.5	Notifications	125

6.2.2.6	Data Model	125

6.2.2.6.1	General	125

6.2.2.6.2	Structured data types	126

6.2.2.6.2.1	Introduction	126

6.2.2.6.2.2	Type: ClientDiscCriteria	126

6.2.2.6.2.3	Type: ServiceRequirement	127

6.2.2.6.2.4	Type: ClientAppCapability	127

6.2.2.6.2.5	Type: DatasetRequirement	127

6.2.2.6.2.6	Type: DatasetAvailablity	127

6.2.2.6.2.7	Type: DatasetCapability	128

6.2.2.6.2.8	Type: ClientTaskCapability	128

6.2.2.6.2.9	Type: ClientDiscResp	128

6.2.2.6.3	Simple data types and enumerations	128

6.2.2.6.3.1	Introduction	128

6.2.2.6.3.2	Simple data types	128

6.2.2.6.4	Data types describing alternative data types or combinations of data types	130

6.2.2.6.5	Binary data	130

6.2.2.7	Error Handling	130

6.2.2.7.1	General	130

6.2.2.7.2	Protocol Errors	130

6.2.2.7.3	Application Errors	131

6.2.2.8	Feature Negotiation	131

6.2.2.9	Security	131

6.1.7	AIMLES_AIMLEClientSelection API	132

6.1.7.1	Introduction	132

6.1.7.2	Usage of HTTP and common API related aspects	132

6.1.7.3	Resources	132

6.1.7.3.1	Overview	132

6.1.7.3.2	Resource: AIMLE Client Selection Subscriptions	133

6.1.7.3.3	Resource: Individual AIMLE Client Selection Subscription	134

6.1.7.4	Custom Operations without associated resources	139

6.1.7.4.1	Overview	139

6.1.7.4.2	Operation: Select	139

6.1.7.5	Notifications	140

6.1.7.5.1	General	140

6.1.7.5.2	AIMLE Client Selection Event Notification	140

6.1.7.6	Data Model	141

6.1.7.6.1	General	141

6.1.7.6.2	Structured data types	142

6.1.7.6.3	Simple data types and enumerations	144

6.1.7.6.4	Data types describing alternative data types or combinations of data types	144

6.1.7.6.5	Binary data	144

6.1.7.7	Error Handling	145

6.1.7.7.1	General	145

6.1.7.7.2	Protocol Errors	145

6.1.7.7.3	Application Errors	145

6.1.7.8	Feature negotiation	145

6.1.7.9	Security	145

6.1.8	AIMLES_MLModelTraining API	146

6.1.8.1	Introduction	146

6.1.8.2	Usage of HTTP and common API related aspects	146

6.1.8.3	Resources	146

6.1.8.4	Custom Operations without associated resources	146

6.1.8.4.1	Overview	146

6.1.8.4.2	Operation: RequestTrain	147

6.1.8.5	Notifications	148

6.1.8.5.1	General	148

6.1.8.5.2	ML Model Training Notification	148

6.1.8.6	Data Model	149

6.1.8.6.1	General	149

6.1.8.6.2	Structured data types	150

6.1.8.6.2.2	Type: TrainRequest	150

6.1.8.6.2.3	Type: MlModelTrainNotif	151

6.1.8.6.2.4	Type: MemberSelCriteria	151

6.1.8.6.2.5	Type: MlModelInfo	151

6.1.8.6.2.6	Type: MemberInfo	151

6.1.8.6.2.7	Type: PerfParam	151

6.1.8.6.3	Simple data types and enumerations	152

6.1.8.6.3.3	Enumeration: TrainingType	152

6.1.8.6.3.4	Enumeration: TrainingErr	152

6.1.8.6.4	Data types describing alternative data types or combinations of data types	153

6.1.8.6.5	Binary data	153

6.1.8.7	Error Handling	153

6.1.8.7.1	General	153

6.1.8.7.2	Protocol Errors	153

6.1.8.7.3	Application Errors	153

6.1.8.8	Feature negotiation	153

6.1.8.9	Security	154

6.1.9	AIMLES_MLModelPerfMonitor API	156

6.1.9.1	Introduction	156

6.1.9.2	Usage of HTTP	156

6.1.9.3	Resources	156

6.1.9.3.1	Overview	156

6.1.9.3.2	Resource: AIMLE ML Model Performance Monitor Subscriptions	157

6.1.9.3.3	Resource: Individual AIMLE ML Model Performance Monitor Subscription	158

6.1.9.4	Custom Operations without associated resources	163

6.1.9.5	Notifications	163

6.1.9.5.1	General	163

6.1.9.5.2	AIMLE ML Model Performance Monitor Event Notification	163

6.1.9.6	Data Model	164

6.1.9.6.1	General	164

6.1.9.6.2	Structured data types	165

6.1.9.6.3	Simple data types and enumerations	168

6.1.9.6.4	Data types describing alternative data types or combinations of data types	169

6.1.9.6.5	Binary data	169

6.1.9.7	Error Handling	169

6.1.9.7.1	General	169

6.1.9.7.2	Protocol Errors	169

6.1.9.7.3	Application Errors	169

6.1.9.8	Feature negotiation	170

6.1.9.9	Security	170

6.1.10	AIMLES_TLModelSelectionAssistance API	171

6.1.10.1	Introduction	171

6.1.10.2	Usage of HTTP and common API related aspects	171

6.1.10.3	Resources	171

6.1.10.3.1	Overview	171

6.1.10.3.2	Resource: AIMLE TL Model Selection Assistance	172

6.1.10.3.2.1	Description	172

6.1.10.3.2.2	Resource Definition	172

6.1.10.3.2.3	Resource Standard Methods	172

6.1.10.3.2.3.1	GET	172

6.1.10.3.2.4	Resource Custom Operations	173

6.1.10.4	Custom Operations without associated resources	173

6.1.10.5	Notifications	173

6.1.10.6	Data Model	173

6.1.10.6.1	General	173

6.1.10.6.2	Structured data types	174

6.1.10.6.2.1	Introduction	174

6.1.10.6.3	Simple data types and enumerations	174

6.1.10.6.3.1	Introduction	174

6.1.10.6.3.2	Simple data types	174

6.1.10.6.4	Data types describing alternative data types or combinations of data types	174

6.1.10.6.5	Binary data	174

6.1.10.7	Error Handling	174

6.1.10.7.1	General	174

6.1.10.7.2	Protocol Errors	174

6.1.10.7.3	Application Errors	175

6.1.10.8	Feature Negotiation	175

6.1.10.9	Security	175

6.1.11	AIMLES_AssistedMLModelSelection API	176

6.1.11.1	Introduction	176

6.1.11.2	Usage of HTTP	176

6.1.11.3	Resources	176

6.1.11.3.1	Overview	176

6.1.11.3.2	Resource: AIMLE Assisted ML Model Selection Subscriptions	177

6.1.11.3.3	Resource: Individual AIMLE Assisted ML Model Selection Subscription	178

6.1.11.4	Custom Operations without associated resources	183

6.1.11.5	Notifications	183

6.1.11.5.1	General	183

6.1.11.5.2	AIMLE Assisted ML Model Selection Event Notification	183

6.1.11.6	Data Model	184

6.1.11.6.1	General	184

6.1.11.6.2	Structured data types	185

6.1.11.6.3	Simple data types and enumerations	188

6.1.11.6.4	Data types describing alternative data types or combinations of data types	189

6.1.11.6.5	Binary data	189

6.1.11.7	Error Handling	189

6.1.11.7.1	General	189

6.1.11.7.2	Protocol Errors	190

6.1.11.7.3	Application Errors	190

6.1.11.8	Feature negotiation	190

6.1.11.9	Security	190

6.1.12	AIMLES_SplitOpEvent API	191

6.1.12.1	Introduction	191

6.1.12.2	Usage of HTTP	191

6.1.12.3	Resources	191

6.1.12.3.1	Overview	191

6.1.12.3.2	Resource: AIMLE Split Operation Event Subscriptions	192

6.1.12.3.2.1	Description	192

6.1.12.3.2.2	Resource Definition	192

6.1.12.3.2.3	Resource Standard Methods	192

6.1.12.3.2.3.1	POST	192

6.1.12.3.2.4	Resource Custom Operations	193

6.1.12.3.3	Resource: Individual AIMLE Split Operation Event Subscription	193

6.1.12.3.3.1	Description	193

6.1.12.3.3.2	Resource Definition	193

6.1.12.3.3.3	Resource Standard Methods	193

6.1.12.3.3.3.1	GET	193

6.1.12.3.3.3.2	PUT	194

6.1.12.3.3.3.3	PATCH	195

6.1.12.3.3.3.4	DELETE	196

6.1.12.4	Custom Operations without associated resources	197

6.1.12.5	Notifications	197

6.1.12.5.1	General	197

6.1.12.5.2	AIMLE Split Operation Event Notification	198

6.1.12.6	Data Model	199

6.1.12.6.1	General	199

6.1.12.6.2	Structured data types	200

6.1.12.6.2.1	Introduction	200

6.1.12.6.2.2	Type: SplitOpEventSub	200

6.1.12.6.2.3	Type: SplitOpEventSubPatch	201

6.1.12.6.2.4	Type: SplitOpEventNotif	201

6.1.12.6.2.5	Type: DiscFilters	201

6.1.12.6.2.6	Type: SplitOpPipelineInfo	202

6.1.12.6.2.7	Type: AssistanceInfo	202

6.1.12.6.2.8	Type: AvailabilityInfo	202

6.1.12.6.2.9	Type: SplitOpProfile	202

6.1.12.6.2.10	Type: StageInfo	202

6.1.12.6.3	Simple data types and enumerations	203

6.1.12.6.3.2	Simple data types	203

6.1.12.6.3.3	Enumeration: SplitOpEventId	203

6.1.12.6.3.3	Enumeration: SubEventId	203

6.1.12.6.4	Data types describing alternative data types or combinations of data types	203

6.1.12.6.5	Binary data	204

6.1.12.7	Error Handling	204

6.1.12.7.1	General	204

6.1.12.7.2	Protocol Errors	204

6.1.12.7.3	Application Errors	204

6.1.12.8	Feature negotiation	204

6.1.12.9	Security	204

6.1.13	Aimles_MLModelRetrieval API	205

6.1.13.1	Introduction	205

6.1.13.2	Usage of HTTP	205

6.1.13.3	Resources	205

6.1.13.3.1	Overview	205

6.1.13.3.2	Resource: AIMLE ML Model Retrieval Subscriptions	206

6.1.13.3.3	Resource: Individual AIMLE ML Model Retrieval Subscription	207

6.1.13.4	Custom Operations without associated resources	212

6.1.13.4.1	Overview	212

6.1.13.4.2	Operation: Retrieve	212

6.1.13.5	Notifications	213

6.1.13.5.1	General	213

6.1.13.5.2	AIMLE ML Model Retrieval Notification	213

6.1.13.6	Data Model	215

6.1.13.6.1	General	215

6.1.13.6.2	Structured data types	215

6.1.13.6.3	Simple data types and enumerations	217

6.1.13.6.4	Data types describing alternative data types or combinations of data types	218

6.1.13.6.5	Binary data	218

6.1.13.7	Error Handling	218

6.1.13.7.1	General	218

6.1.13.7.2	Protocol Errors	218

6.1.13.7.3	Application Errors	218

6.1.13.8	Feature negotiation	218

6.1.13.9	Security	218

6.1.14	Aimles_SplitOpNodeRegistration API	220

6.1.14.1	Introduction	220

6.1.14.2	Usage of HTTP and common API related aspects	220

6.1.14.3	Resources	220

6.1.14.3.1	Overview	220

6.1.14.3.2	Resource: AIMLE Split Operation Node Register Configurations	221

6.1.14.3.3	Resource: Individual AIMLE Split Operation Node Register Configuration	222

6.1.14.4	Custom Operations without associated resources	227

6.1.14.5	Notifications	227

6.1.14.5.1	General	227

6.1.14.6	Data Model	227

6.1.14.6.1	General	227

6.1.14.6.2	Structured data types	227

6.1.14.6.3	Simple data types and enumerations	228

6.1.14.6.4	Data types describing alternative data types or combinations of data types	229

6.1.14.6.5	Binary data	229

6.1.14.7	Error Handling	230

6.1.14.7.1	General	230

6.1.14.7.2	Protocol Errors	230

6.1.14.7.3	Application Errors	230

6.1.14.8	Feature negotiation	230

6.1.14.9	Security	230

6.2	AIMLE Repository APIs	231

6.2.1	MLR_MLModelManagement Service API	231

6.2.1.1	Introduction	231

6.2.1.2	Usage of HTTP	231

6.2.1.3	Resources	231

6.2.1.3.1	Overview	231

6.2.1.3.2	Resource: ML Models Storages	232

6.2.1.3.2.1	Description	232

6.2.1.3.2.2	Resource Definition	232

6.2.1.3.2.3	Resource Standard Methods	232

6.2.1.3.2.3.1	GET	232

6.2.1.3.2.3.2	POST	233

6.2.1.3.2.4	Resource Custom Operations	234

6.2.1.3.3	Resource: Individual ML Models Storage	234

6.2.1.3.3.1	Description	234

6.2.1.3.3.2	Resource Definition	234

6.2.1.3.3.3	Resource Standard Methods	234

6.2.1.3.3.3.1	GET	234

6.2.1.3.3.3.2	PUT	235

6.2.1.3.3.3.3	PATCH	236

6.2.1.3.3.3.4	DELETE	237

6.2.1.3.3.4	Resource Custom Operations	238

6.2.1.4	Custom Operations without associated resources	238

6.2.1.5	Notifications	238

6.2.1.6	Data Model	238

6.2.1.6.1	General	238

6.2.1.6.2	Structured data types	239

6.2.1.6.2.1	Introduction	239

6.2.1.6.2.2	Type: MLModelsStorage	239

6.2.1.6.2.3	Type: MLModelProfile	240

6.2.1.6.2.4	Type: MLModel	240

6.2.1.6.2.5	Type: MLModelPhaseInfo	241

6.2.1.6.2.6	Type: MLModelStoreDiscReqs	242

6.2.1.6.2.7	Type: MLModelUsageReqs	242

6.2.1.6.2.8	Type: MLModelPerf	242

6.2.1.6.2.9	Type: MLModelTrainingInfo	243

6.2.1.6.2.10	Type: MLModelAccessReqs	243

6.2.1.6.2.11	Type: MLModelsStoragePatch	244

6.2.1.6.3	Simple data types and enumerations	244

6.2.1.6.3.1	Introduction	244

6.2.1.6.3.2	Simple data types	244

6.2.1.6.3.3	Enumeration: MLModelDomain	245

6.2.1.6.3.4	Enumeration: MLModelPhase	245

6.2.1.6.3.5	Enumeration: MLModelUsage	245

6.2.1.6.3.6	Enumeration: AccessReqs	245

6.2.1.6.3.7	Enumeration: DataFreshness	246

6.2.1.6.4	Data types describing alternative data types or combinations of data types	246

6.2.1.6.5	Binary data	246

6.2.1.6.5.1	Binary Data Types	246

6.2.1.7	Error Handling	246

6.2.1.7.1	General	246

6.2.1.7.2	Protocol Errors	246

6.2.1.7.3	Application Errors	246

6.2.1.8	Feature negotiation	247

6.2.1.9	Security	247

6.2.2	MLR_ModelInformationDiscovery API	248

6.2.2.1	Introduction	248

6.2.2.2	Usage of HTTP and common API related aspects	248

6.2.2.3	Resources	248

6.2.2.3.1	Overview	248

6.2.2.3.2	Resource: ML Models	249

6.2.2.3.2.1	Description	249

6.2.2.3.2.2	Resource Definition	249

6.2.2.3.2.3	Resource Standard Methods	249

6.2.2.3.2.3.1	GET	249

6.2.2.3.2.4	Resource Custom Operations	250

6.2.2.4	Custom Operations without associated resources	250

6.2.2.5	Notifications	250

6.2.2.6	Data Model	250

6.2.2.6.1	General	250

6.2.2.6.2	Structured data types	251

6.2.2.6.2.1	Introduction	251

6.2.2.6.2.2	Type: DiscoveryResp	251

6.2.2.6.2.3	Type: MlModel	251

6.2.2.6.3	Simple data types and enumerations	252

6.2.2.6.3.1	Introduction	252

6.2.2.6.3.2	Simple data types	252

6.2.2.6.3.3	Enumerations	252

6.2.2.6.4	Data types describing alternative data types or combinations of data types	252

6.2.2.6.5	Binary data	252

6.2.2.7	Error Handling	252

6.2.2.7.1	General	252

6.2.2.7.2	Protocol Errors	252

6.2.2.7.3	Application Errors	252

6.2.2.8	Feature Negotiation	252

6.2.2.9	Security	253

6.2.3	MLR_FLEvents API	254

6.2.3.1	Introduction	254

6.2.3.2	Usage of HTTP	254

6.2.3.3	Resources	254

6.2.3.3.1	Overview	254

6.2.3.3.2	Resource: MLR FL Events Subscriptions	255

6.2.3.3.3	Resource: Individual MLR FL Events Subscription	256

6.2.3.4	Custom Operations without associated resources	260

6.2.3.5	Notifications	260

6.2.3.5.1	General	260

6.2.3.5.2	MLR FL Events Event Notification	261

6.2.3.6	Data Model	262

6.2.3.6.1	General	262

6.2.3.6.2	Structured data types	263

6.2.3.6.3	Simple data types and enumerations	266

6.2.3.6.4	Data types describing alternative data types or combinations of data types	266

6.2.3.6.5	Binary data	267

6.2.3.7	Error Handling	267

6.2.3.7.1	General	267

6.2.3.7.2	Protocol Errors	267

6.2.3.7.3	Application Errors	267

6.2.3.8	Feature negotiation	267

6.2.3.9	Security	267

6.2.4	MLR_FLMember API	268

6.2.4.1	Introduction	268

6.2.4.2	Usage of HTTP and common API related aspects	268

6.2.4.3	Resources	268

6.2.4.3.1	Overview	268

6.2.4.3.2	Resource: FL Member Configurations	269

6.2.4.3.3	Resource: Individual FL Member Configuration	270

6.2.4.4	Custom Operations without associated resources	274

6.2.4.5	Notifications	274

6.2.4.6	Data Model	275

6.2.4.6.1	General	275

6.2.4.6.2	Structured data types	275

6.2.4.6.3	Simple data types and enumerations	277

6.2.4.6.4	Data types describing alternative data types or combinations of data types	278

6.2.4.6.5	Binary data	278

6.2.4.7	Error Handling	278

6.2.4.7.1	General	278

6.2.4.7.2	Protocol Errors	278

6.2.4.7.3	Application Errors	278

6.2.4.8	Feature negotiation	278

6.2.4.9	Security	279

7	Using Common API Framework	280

Annex A (normative): OpenAPI specification	281

A.1	General	281

A.2	AIMLES_ContextTransfer API	282

A.3	AIMLES_DataManagement API	284

A.4	MLR_MLModelManagement API	290

A.5	AIMLES_AIMLEClientDiscovery API	299

A.6	MLR_ModelInformationDiscovery API	304

A.7	AIMLES_AIMLEClientSelection API	306

A.8	AIMLES_AIMLEServiceOperationsManagement API	312

A.9	AIMLES_HierarchicalComputingAssist API	315

A.10	AIMLES_AssistedMLModelSelection API	318

A.11	Aimles_MLModelRetrieval API	325

A.12	Aimles_SplitOpNodeRegistration API	331

Annex B (informative): Withdrawn API versions	336

B.1	General	336

B.2	AIMLES_ContextTransfer API	336

B.3	AIMLES_DataManagement API	336

B.4	MLR_MLModelManagement API	336

B.5	AIMLES_AIMLEClientDiscovery API	336

B.6	MLR_ModelInformationDiscovery API	337

B.7	AIMLES_AIMLEClientSelection API	337

B.8	AIMLES_AIMLEServiceOperationsManagement API	337

B.9	AIMLES_HierarchicalComputingAssist API	337

B.10	AIMLES_AssistedMLModelSelection API	337

B.11	Aimles_MLModelRetrieval API	337

B.12	Aimles_SplitOpNodeRegistration API	338

Annex C (informative): Change history	339

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the SEAL AIMLE Services, for enabling the support of AIMLE services for vertical applications. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the AIMLE Layer entities.

The stage 2 architecture and procedures are specified in 3GPP TS 23.482 [13].

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[14]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 24.560: "Artificial Intelligence Machine Learning (AIML) Services - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3".

[17]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in 3GPP TS 23.482 [13] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIML	Artificial Intelligence Machine Learning

AIMLE	Artificial Intelligence Machine Learning Enablement

FL	Federated Learning

MLR	Machine Learning Repository

TL	Transfer Learning

SEAL	Service Enabler Architecture Layer for Verticals

VAL	Vertical Application Layer

# 4	Overview

The AIMLE Layer forms part of the SEAL Enabler Layer defined in 3GPP TS 23.434 [17] and aims to ensure the efficient use and deployment of AIML capabilities for vertical applications. The AIMLE Layer, via the AIMLE Server and ML Repository, support for this purpose the functionalities defined in clause 6 of 3GPP TS 23.482 [13].

Figure 4-1 shows the reference model of the AIMLE service, with a focus on the AIMLE Server and ML Repository.

Figure 4-1: AIMLE service functional model

# 5	Services offered by AIMLE

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| AIMLES_ContextTransfer | 6.1.1 | AIMLE Context Transfer Information Service | TS29482_AIMLES_ContextTransfer.yaml | aimles-ct | A.2 |
| AIMLES_DataManagement | 6.1.2 | AIMLE Data Management assistance Service | TS29482_AIMLES_DataManagement.yaml | aimles-dm | A.3 |
| AIMLES_FLMemberGroupSupport | 6.1.3 | FL Member Grouping Service | TS29482_AIMLES_FLMemberGroupSupport.yaml | aimles-fl | A.X |
| AIMLES_AIMLEServiceOperationsManagement | 6.1.4 | AIMLE Operation Management service | TS29482_AIMLES_AIMLEServiceOperationsManagement API.yaml | aimles-opm | A.8 |
| AIMLES_HierarchicalComputingAssist | 6.1.5 | AIMLE Hierarchical Computing Assist service | TS29482_AIMLES_HierarchicalComputingAssist.yaml | aimles-hca | A.9 |
| AIMLES_AIMLEClientDiscovery | 6.1.6 | AIMLE Client discovery service | TS29482_AIMLES_AIMLEClientDiscovery.yaml | aimles-disc | A.5 |
| AIMLES_AIMLEClientSelection | 6.1.7 | AIMLE Client Selection service | TS29482_AIMLES_AIMLEClientSelection.yaml | aimles-sel | A.7 |
| MLR_MLModelManagement | 6.2.1 | MLR Model Management Service API | TS29482_MLR_MLModelManagement.yaml | mlr-mlmm | A.4 |
| MLR_ModelInformationDiscovery | 6.2.2 | MLR Model Information Discovery service | TS29482_MLR_ModelInformationDiscovery.yaml | mlr-mid | A.6 |
| AIMLES_TLModelSelectionAssistance | 5.2.11 | TL enablement service | TS29482_AIMLES_TLModelSelectionAssistance.yaml | aimles-tlmsa | A.X |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (e.g., AIMLE Server, AIMLE Repository) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

## 5.2	Services offered by the AIMLE Server

### 5.2.1	AIMLES_ContextTransfer Service

#### 5.2.1.1	Service Description

The AIMLES_ContextTransfer service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE context information transfer to the AIMLE Server.

#### 5.2.1.2	Service Operations

##### 5.2.1.2.1	Introduction

The service operations defined for AIMLES_ContextTransfer API are shown in the table 5.2.1.2.1-1.

Table 5.2.1.2.1-1: AIMLES_ContextTransfer Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_ContextTransfer_Request | This service operation is used by a service consumer to request the AIMLE context information transfer. | e.g., AIMLE Server |

##### 5.2.1.2.2	AIMLES_ContextTransfer_Request

###### 5.2.1.2.2.1	General

This service operation is used by a service consumer to request AIMLE context transfer to the AIMLE Server.

The following procedures are supported by the "AIMLES_ContextTransfer_Request" service operation:

-	AIMLE Context Transfer Request.

###### 5.2.1.2.2.2	AIMLE Context Transfer Request

Figure 5.2.1.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE context information transfer (see also clause 8.24.3.1 of 3GPP°TS°23.482°[13]).

Figure 5.2.1.2.2.2-1: Procedure for AIMLE Context Transfer Request

1.	In order to request AIMLE context information transfer, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation (i.e., "RequestTrans"), with the request body including the TransReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "204 No Content" status code to indicate that the AIMLE context transfer request is successfully received and processed.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.1.7.

### 5.2.2	AIMLES_DataManagement Service

#### 5.2.2.1	Service Description

The AIMLES_DataManagement service exposed by the AIMLE Server enables a service consumer to:

-	create/update/delete a AIMLE Data Management Assistance Subscription; and

-	receive AIMLE Data Management Assistance Notifications.

#### 5.2.2.2	Service Operations

##### 5.2.2.2.1	Introduction

The service operations defined for the AIMLES_DataManagement API are shown in the table 5.2.2.10.2.1-1.

Table 5.2.2.2.1-1: Service operations of the Aimles_DataManagement API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_DataManagement_Subscribe | This service operation enables a service consumer to create a AIMLE data management assistance Subscription. | e.g. VAL Server |
| AIMLES_DataManagement_Update | This service operation enables a service consumer to update an existing AIMLE data management assistance Subscription. | e.g. VAL Server |
| AIMLES_DataManagement_Unsubscribe | This service operation enables a service consumer to delete an existing AIMLE data management assistance Subscription. | e.g. VAL Server |
| AIMLES_DataManagement_Notify | This service operation enables a service consumer to receive AIMLE data management assistance notifications. | AIMLE Server |

##### 5.2.2.2.2	AIMLES_DataManagement_Subscribe

5.2.2.2.2.1	General

This service operation is used by a service consumer to request the creation of AIMLE Data Management Assistance Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_DataManagement_Subscribe" service operation:

-	AIMLE Data Management Assistance Subscription Creation.

This service operation is used by the analytics consumer for AIMLE Data Management Assistance Subscription to the AIMLE Server.

5.2.2.2.2.2	AIMLE Data Management Assistance Subscription Creation

Figure 5.2.2.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the creation of AIMLE Data Management Assistance Subscription (see also clause 8.15 of 3GPP°TS°23.482°[13]).

Figure 5.2.2.2.2.2-1: Procedure for AIMLE Data Management Assistance Subscription Creation

1.	In order to subscribe to AIMLE Data Management Assistance reporting, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "AIMLE Data Management Assistance Subscriptions" collection resource, with the request body including the DataMgmtAssistSubsc data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIMLE Data Management Assistance Subscription" resource within the DataMgmtAssistSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.2.7.

##### 5.2.2.2.3	AIMLES_DataManagement_Update

5.2.2.2.3.1	General

This service operation is used by a service consumer to request the update of a AIMLE Data Management Assistance Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_DataManagement_Subscribe" service operation:

-	AIMLE Data Management Assistance Subscription Update.

This service operation is used by the analytics consumer for AIMLE Data Management Assistance Subscription to the AIMLE Server.

5.2.2.2.3.2	AIMLE Data Management Assistance Subscription Update

Figure 5.2.2.2.5.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the update of AIMLE Data Management Assistance Subscription (see also clause 8.15 of 3GPP°TS°23.482°[13]).

Figure 5.2.2.2.3.2-1: Procedure for AIMLE Data Management Assistance Subscription Creation

1.	In order to request the update of an existing AIMLE Data Management Assistance reporting, the service consumer shall send an HTTP PUT/PATCH request to the AIMEL Server, targeting the URI of the corresponding "Individual AIMLE Data Management Assistance Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the DataMgmtAssistSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the DataMgmtAssistSubscPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall update the targeted "AIMLE Data Management Assistance Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIMLE Data Management Assistance Subscriptions" resource within the DataMgmtAssistSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.2.7.

##### 5.2.2.2.4	AIMLES_DataManagement_Unsubscribe

5.2.2.2.4.1	General

This service operation is used by a service consumer to request the deletion of AIMLE Data Management Assistance Subscription at the ADAE Server.

The following procedures are supported by the "AIMLES_DataManagement_Unsubscribe" service operation:

-	AIMLE Data Management Assistance Subscription Deletion.

5.2.2.2.4.2	AIMLE Data Management Assistance Subscription Deletion

Figure 5.2.2.2.4.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to delete an existing AIML Member Capability Analytics Subscription (see also clause 8.15 of 3GPP°TS°23.482°[13]).

Figure 5.2.2.2.4.2-1: Procedure for AIMLE Data Management Assistance Subscription Deletion

1.	In order to request the deletion of an existing AIMLE Data Management Assistance Subscription, the service consumer shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding "Individual AIMLE Data Management Assistance Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.2.7.

##### 5.2.2.2.5	AIMLES_DataManagement_Notify

5.2.2.2.5.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE Data Management Assistance report(s).

The following procedures are supported by the "AIMLE Data Management Assistance_Notify" service operation:

-	AIMLE Data Management Assistance Notification.

5.2.2.2.5.2	AIMLE Data management Assistance Notification

Figure 5.2.2.2.5.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE Data Management Assistance report(s) (see also clause 8.15 of 3GPP°TS°23.482°[13]).

Figure 5.2.2.2.5.2-1: Procedure for AIMLE Data Management Assistance Notification

1.	In order to notify a previously subscribed service consumer on AIMLE Data Management Assistance report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE Data Management Assistance Subscription using the procedures defined in clauses 5.2.2.2.2.2, and the request body including the DataMgmtAssistNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.2.7.

### 5.2.3	AIMLES_FLMemberGroupSupport Service

#### 5.2.3.1	Service Description

The AIMLES_FLMemberGroupSupport service, exposed by the AIMLE Server, enables a service consumer to:

-	create, update, and delete an FL Member Support Group for an FL process as defined in clause 8.17 of 3GPP TS 23.482 [13].

#### 5.2.3.2	Service Operations

##### 5.2.3.2.1	Introduction

The service operation defined for AIMLES_FLMemberGroupSupport API for is shown in the table 5.2.3.2.1-1.

Table 5.2.3.2.1-1: Operations for AIMLES_FLMemberGroupSupport API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_FLMemberGroupSupport_Create | This service operation is used to create an Individual FL Member Support Group for an FL process. | Service consumer e.g., VAL Server |
| AIMLES_FLMemberGroupSupport_Query | This service operation is used to query for an Individual FL Member Support Group for an FL process. | Service consumer e.g, VAL Server |
| AIMLES_FLMemberGroupSupport_Update | This service operation is used to update an Individual FL Member Support Group for an FL process. | Service consumer e.g., VAL Server |
| AIMLES_FLMemberGroupSupport_Delete | This service operation is used to delete an Individual FL Member Support Group for an FL process. | Service consumer e.g., VAL Server |

##### 5.2.3.2.2	AIMLES_FLMemberGroupSupport_Create

5.2.3.2.2.1	General

This service operation is used by a service consumer e.g., VAL Server to request the AIMLE Server to create an Individual FL Member Support Group for an FL process.

The following procedure is supported by the "AIMLES_FLMemberGroupSupport_Create" service operation:

-	AIMLES FL Member Group Support Create.

5.2.3.2.2.2	AIMLES FL Member Group Support Create

Figure 5.2.3.2.2.2-1 depicts a scenario where a service consumer e.g., VAL Server sends a request to the AIMLE Server to create an Individual FL Member Support Group for an FL process. (see also clause 8.17.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.3.2.2.2-1: Procedure for AIMLES FL Member Group Support Create

1.	In order to create an Individual FL Member Support Group for an FL process, the service consumer e.g., VAL Server shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "FL Member Group Support Configurations"), with the request body including the FlMbrSuppGrp data structure.

2a.	Upon success that the request to create the Individual FL Member Support Group for an FL process is successfully received and processed, the AIMLE Server shall respond with an HTTP "201 Created" status code with the request body including the FlMbrSuppGrp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.3.7.

##### 5.2.3.2.3	AIMLES_FLMemberGroupSupport_Query

5.2.3.2.3.1	General

This service operation is used by a service consumer e.g., VAL Server to request the AIMLE Server to query an Individual FL Member Support Group for an FL process.

The following procedure is supported by the "AIMLES_FLMemberGroupSupport_Query" service operation:

-	AIMLES FL Member Group Support Query.

5.2.3.2.3.2	AIMLES FL Member Group Support Query

Figure 5.2.3.2.3.2-1 depicts a scenario where a service consumer e.g., VAL Server sends a request to the AIMLE Server to query an Individual FL Member Support Group for an FL process. (see also clause 8.17.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.3.2.3.2-1: Procedure for AIMLES FL Member Group Support Query

1.	In order to query an Individual FL Member Support Group for an FL process, the service consumer e.g., VAL Server shall send an HTTP GET request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "Individual FL Member Group Support Configuration").

2a.	Upon success that the request to query the Individual FL Member Support Group for an FL process is successfully received and processed, the AIMLE Server shall respond with an HTTP "200 OK" status code with the request body including the FlMbrSuppGrp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.3.7.

##### 5.2.3.2.4	AIMLES_FLMemberGroupSupport_Update

5.2.3.2.4.1	General

This service operation is used by a service consumer e.g., VAL Server to request the AIMLE Server to update an Individual FL Member Support Group for an FL process.

The following procedure is supported by the "AIMLES_FLMemberGroupSupport_Update" service operation:

-	AIMLES FL Member Group Support Update.

5.2.3.2.4.2	AIMLES FL Member Group Support Update

Figure 5.2.3.2.4.2-1 depicts a scenario where a service consumer e.g., VAL Server sends a request to the AIMLE Server to update an Individual FL Member Support Group for an FL process. (see also clause 8.17.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.3.2.4.2-1: Procedure for AIMLES FL Member Group Support Update

1.	In order to update an Individual FL Member Support Group for an FL process, the service consumer e.g., VAL Server shall send an HTTP PUT/PATCH request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "Individual FL Member Group Support Configuration"), with the request body including either:

-	the updated representation of the resource within the FlMbrSuppGrp data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the FlMbrSuppGrpPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success that the request to update the Individual FL Member Support Group for an FL process is successfully received and processed, the AIMLE Server shall respond with an HTTP "200 OK" status code with the request body including the FlMbrSuppGrp data structure.

-	an HTTP "200 OK" status code with the response body containing a representation of the updated " Individual FL Member Group Support Configuration" resource within the FlMbrSuppGrp data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.3.7.

##### 5.2.3.2.5	AIMLES_FLMemberGroupSupport_Delete

5.2.3.2.5.1	General

This service operation is used by a service consumer e.g., VAL Server to request the AIMLE Server to delete an Individual FL Member Support Group for an FL process.

The following procedure is supported by the "AIMLES_FLMemberGroupSupport_Delete" service operation:

-	AIMLES FL Member Group Support Delete.

5.2.3.2.5.2	AIMLES FL Member Group Support Delete

Figure 5.2.3.2.5.2-1 depicts a scenario where a service consumer e.g., VAL Server sends a request to the AIMLE Server to delete an Individual FL Member Support Group for an FL process. (see also clause 8.17.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.3.2.5.2-1: Procedure for AIMLES FL Member Group Support Delete

1.	In order to delete an Individual FL Member Support Group for an FL process, the service consumer e.g., VAL Server shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "Individual FL Member Group Support Configuration").

2a.	Upon success that the request to delete the Individual FL Member Support Group for an FL process is successfully received and processed, the AIMLE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.3.7.

### 5.2.4	AIMLES_AIMLEServiceOperationsManagement Service

#### 5.2.4.1	Service Description

The AIMLES_AIMLEServiceOperationsManagement service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE server to provide AIMLE service operation management.

#### 5.2.4.2	Service Operations

##### 5.2.4.2.1	Introduction

The service operation defined for AIMLES_AIMLEServiceOperationsManagement API is shown in the table 5.2.4.2.1-1.

Table 5.2.4.2.1-1: AIMLES_AIMLEServiceOperationsManagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_AIMLEServiceOperationsManagement_Request | This service operation is used by a service consumer to request AIMLE service operation management. | e.g., VAL Server. |

##### 5.2.4.2.2	AIMLES_AIMLEServiceOperationsManagement_Request

5.2.4.2.2.1	General

This service operation is used by a service consumer to request AIMLE Service Operations Management to the AIMLE server.

The following procedures are supported by the "AIMLES_AIMLEServiceOperationsManagement_Request " service operation:

-	AIMLE Service Operations Management Request.

5.2.4.2.2.2	AIMLE Service Operations Management Request

Figure 5.2.4.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE Service Operations Management (see also clause 8.20 of 3GPP°TS°23.482°[13]).

Figure 5.2.4.2.2.2-1: Procedure for AIMLE Service Operations Management Request

1.	In order to request AIMLE service operations management, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation (i.e., "RequestServOpMngt"), with the request body including the AimlServOperReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code including the AimlServOperResp data type to indicate that the request is successfully received and processed.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.4.7.

### 5.2.5	AIMLES_HierarchicalComputingAssist Service

#### 5.2.5.1	Service Description

The AIMLES_HierarchicalComputingAssist service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE server to request the hierarchical computing assistance.

#### 5.2.5.2	Service Operations

##### 5.2.5.2.1	Introduction

The service operation defined for AIMLES_HierarchicalComputingAssist API is shown in the table 5.2.5.2.1-1.

Table 5.2.5.2.1-1: AIMLES_HierarchicalComputingAssist Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_HierarchicalComputingAssist_Request | This service operation is used by a service consumer to request the hierarchical computing assistance. | e.g., AIMLE Server. |

##### 5.2.5.2.2	AIMLES_HierarchicalComputingAssist_Request

5.2.5.2.2.1	General

This service operation is used by a service consumer to request AIMLE hierarchical computing assist to the AIMLE server.

The following procedures are supported by the "AIMLES_HierarchicalComputingAssist_Request" service operation:

-	AIMLE Hierarchical Computing Assist Request.

5.2.5.2.2.2	AIMLE Hierarchical Computing Assist Request

Figure 5.2.5.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE hierarchical computing assist (see also clause 8.25 of 3GPP°TS°23.482°[13]).

Figure 5.2.5.2.2.2-1: Procedure for AIMLE Hierarchical Computing Assist Request

1.	In order to request AIMLE hierarchical computing assist, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation (i.e., "RequestServOpMngt"), with the request body including the AimlHierAssitReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code including the AimlHierAssitResp data type to indicate that the request is successfully received and processed.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.5.7.

### 5.2.6	AIMLES_AIMLEClientDiscovery Service

#### 5.2.6.1	Service Description

The AIMLES_AIMLEClientDiscovery service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE server to discover the AIMLE Clients according to the filtering criteria

#### 5.2.6.2	Service Operations

##### 5.2.6.2.1	Introduction

The service operation defined for AIMLES_AIMLEClientDiscovery API is shown in the table 5.2.6.2.1-1.

Table 5.2.6.2.1-1: AIMLES_AIMLEClientDiscovery Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_AIMLEClientDiscovery_Request | This service operation is used by a service consumer to discover the AIMLE clients. | e.g., VAL Server. |

#### 5.2.6.2.2	AIMLES_AIMLEClientDiscovery_Request

##### 5.2.6.2.2.1	General

This service operation is used by a service consumer to request the discovery of the AIMLE Clients.

The following procedures are supported by the "AIMLES_AIMLEClientDiscovery_Request" service operation:

-	AIMLE Clients discovery request.

##### 5.2.6.2.2.2	AIMLE Clients discovery request

Figure 5.2.6.2.2.2.2-1 depicts a scenario where a service consumer sends a request to the MLR to request the discovery of the ML model(s) (see also clause 8.8 of 3GPP°TS°23.482°[13]).

Figure 5.2.6.2.2.2-1: Procedure for AIMLE Clients discovery request

1.	In order to discover the AIMLE Client(s), the service consumer shall send an HTTP GET request to the AIMLE Server targeting the URI of the "AIMLE Clients" resource.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code with the response body containing the ClientDiscResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.6.7.

### 5.2.7	AIMLES_AIMLEClientSelection Service

#### 5.2.7.1	Service Description

The AIMLES_AIMLEClientSelection service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE server to select the AIMLE Clients according to the filtering criteria; and

-	request AIMLE server to select the AIMLE Clients and subscribe to the notifications.

#### 5.2.7.2	Service Operations

##### 5.2.7.2.1	Introduction

The service operation defined for AIMLES_AIMLEClientSelection API is shown in the table 5.2.7.2.1-1.

Table 5.2.7.2.1-1: AIMLES_AIMLEClientSelection Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_AIMLEClientSelection_Request | This service operation is used by a service consumer to select the AIMLE clients. | e.g., VAL Server. |
| AIMLES_ClientSelection_Subscribe | This service operation is used by a service consumer to subscribe for monitoring and selection of AIMLE clients for AI/ML operations. | e.g., VAL Server. |
| AIMLES_ClientSelection_Update | This service operation is used by a service consumer to update the subscription for monitoring and selection of AIMLE clients for AI/ML operations. | e.g., VAL Server. |
| AIMLES_ClientSelection_Unsubscribe | This service operation is used by a service consumer to delete the subscription for monitoring and selection of AIMLE clients for AI/ML operations. | e.g., VAL Server. |
| AIMLES_ClientSelection_Notify | This service operation enables a service consumer to receive AIMLE client selection notifications. | AIMLE Server |

##### 5.2.7.2.2	AIMLES_ClientSelection_Subscribe

5.2.7.2.2.1	General

This service operation is used by a service consumer to request the creation of AIMLE Client Selection Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_ClientSelection_Subscribe" service operation:

-	AIMLE Client Selection Subscription Creation.

This service operation is used by the analytics consumer for AIMLE Client Selection Subscription to the AIMLE Server.

5.2.7.2.2.2	AIMLE Client Selection Subscription Creation

Figure 5.2.7.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the creation of AIMLE Client Selection Subscription (see also clause 8.13 of 3GPP°TS°23.482°[13]).

Figure 5.2.7.2.2.2-1: Procedure for AIMLE Client Selection Subscription Creation

1.	In order to subscribe to AIMLE Client Selection reporting, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "AIMLE Client Selection Subscriptions" collection resource, with the request body including the ClientSelSub data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIMLE Client Selection Subscription" resource within the DataMgmtAssistSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.7.

##### 5.2.7.2.3	AIMLES_DataManagement_Update

5.2.7.2.3.1	General

This service operation is used by a service consumer to request the update of a AIMLE Client Selection Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_ClientSelection_Subscribe" service operation:

-	AIMLE Client Selection Subscription Update.

This service operation is used by the analytics consumer for AIMLE Client Selection Subscription to the AIMLE Server.

5.2.7.2.3.2	AIMLE Client Selection Subscription Update

Figure 5.2.7.2.5.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the update of AIMLE Client Selection Subscription (see also clause 8.13 of 3GPP°TS°23.482°[13]).

Figure 5.2.7.2.3.2-1: Procedure for AIMLE Client Selection Subscription Update

1.	In order to request the update of an existing AIMLE Client Selection reporting, the service consumer shall send an HTTP PUT/PATCH request to the AIMEL Server, targeting the URI of the corresponding "Individual AIMLE Client Selection Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the ClientSelSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the ClientSelSubPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall update the targeted "AIMLE Client Selection Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIMLE Client Selection Subscriptions" resource within the ClientSelSub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.7.7.

##### 5.2.7.2.4	AIMLES_DataManagement_Unsubscribe

5.2.7.2.4.1	General

This service operation is used by a service consumer to request the deletion of AIMLE Client Selection Subscription at the ADAE Server.

The following procedures are supported by the "AIMLES_ClientSelection_Unsubscribe" service operation:

-	AIMLE Client Selection Subscription Deletion.

5.2.7.2.4.2	AIMLE Client Selection Subscription Deletion

Figure 5.2.7.2.4.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to delete an existing AIMLE Client Selection Subscription (see also clause 8.13 of 3GPP°TS°23.482°[13]).

Figure 5.2.7.2.4.2-1: Procedure for AIMLE Client Selection Subscription Deletion

1.	In order to request the deletion of an existing AIMLE Client Selection Subscription, the service consumer shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding "Individual AIMLE Client Selection Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.7.7.

##### 5.2.7.2.5	AIMLES_DataManagement_Notify

5.2.7.2.5.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE Client Selection report(s).

The following procedures are supported by the "AIMLE Client Selection_Notify" service operation:

-	AIMLE Client Selection Notification.

5.2.7.2.5.2	AIMLE Client Selection Notification

Figure 5.2.7.2.5.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE Client Selection report(s) (see also clause 8.13 of 3GPP°TS°23.482°[13]).

Figure 5.2.7.2.5.2-1: Procedure for AIMLE Client Selection Notification

1.	In order to notify a previously subscribed service consumer on AIMLE Client Selection report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE Client Selection Subscription using the procedures defined in clauses 5.2.7.2.2.2, and the request body including the DataMgmtAssistNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.7.

##### 5.2.7.2.6	AIMLES_AIMLEClientSelection_Request

5.2.7.2.6.1	General

This service operation is used by a service consumer to request AIMLE Client selection to the AIMLE server.

The following procedures are supported by the "AIMLES_AIMLEClientSelection_Request" service operation:

-	AIMLE Client Selection Request.

5.2.7.2.6.2	AIMLE Client Selection Request

Figure 5.2.7.2.6.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE context transfer (see also clause 8.24.3.1 of 3GPP°TS°23.482°[13]).

Figure 5.2.7.2.6.2-1: Procedure for AIMLE Client Selection Request

1.	In order to request AIMLE context transfer, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation (i.e., "Select"), with the request body including the SelectReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code including the SelectResp data type to indicate that the request is successfully received and processed.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.7.

### 5.2.8	AIMLES_MLModelPerfMonitor Service

#### 5.2.8.1	Service Description

The AIMLES_MLModelPerfMonitor Service, exposed by the AIMLE Server, enables a service consumer to:

-	create/update/delete an AIMLE ML Model Performance Monitor Subscription; and

-	receive AIMLE ML Model Performance Monitor Notifications.

#### 5.2.8.2	Service Operations

##### 5.2.8.2.1	Introduction

The service operations defined for the AIMLES_ MLModelPerfMonitor API are shown in the table 5.2.8.2.1-1.

Table 5.2.8.2.1-1: Service operations of the AIMLES_MLModelPerfMonitor API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_MLModelPerfMonitor_Subscribe | This service operation enables a service consumer to create an AIMLE ML model performance monitor subscription. | e.g., VAL Server |
| AIMLES_MLModelPerfMonitor_Update | This service operation enables a service consumer to update an existing AIMLE ML model performance monitor subscription. | e.g., VAL Server |
| AIMLES_MLModelPerfMonitor_Unsubscribe | This service operation enables a service consumer to delete an existing AIMLE ML model performance monitor subscription. | e.g., VAL Server |
| AIMLES_MLModelPerfMonitor_Notify | This service operation enables a service consumer to receive AIMLE notifications. | AIMLE Server |

##### 5.2.8.2.2	AIMLES_MLModelPerfMonitor_Subscribe

5.2.8.2.2.1	General

This service operation is used by a service consumer to request the creation of AIMLE ML Model Performance Monitor Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_MLModelPerfMonitor_Subscribe" service operation:

-	AIMLE ML Model Performance Monitor Subscription Creation.

5.2.8.2.2.2	AIMLE ML Model Performance Monitor Subscription Creation

Figure 5.2.8.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the creation of AIMLE ML Model Performance Monitor Subscription (see also clause 8.22 of 3GPP°TS°23.482°[13]).

Figure 5.2.8.2.2.2-1: Procedure for AIMLE ML Model Performance Monitor Subscription Creation

1.	In order to subscribe to AIMLE ML model performance monitoring, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "AIMLE ML Model Performance Monitor Subscriptions" collection resource, with the request body including the MlMdlPerfMonitSub data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIMLE ML Model Performance Monitor Subscription" resource within the MlMdlPerfMonitSub data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.9.7.

##### 5.2.8.2.3	AIMLES_MLModelPerfMonitor_Update

5.2.8.2.3.1	General

This service operation is used by a service consumer to request the update of an AIMLE ML Model Performance Monitor Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_MLModelPerfMonitor_Update" service operation:

-	AIMLE ML Model Performance Monitor Subscription Update.

5.2.8.2.3.2	AIMLE ML Model Performance Monitor Subscription Update

Figure 5.2.8.2.5.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the update of AIMLE ML Model Performance Monitor Subscription (see also clause 8.22 of 3GPP°TS°23.482°[13]).

Figure 5.2.8.2.3.2-1: Procedure for AIMLE ML Model Performance Monitor Subscription Update

1.	In order to request the update of an existing AIMLE ML model performance monitoring subscription, the service consumer shall send an HTTP PUT/PATCH request to the AIMEL Server, targeting the URI of the corresponding "Individual AIMLE ML Model Performance Monitor Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the MlMdlPerfMonitSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the MlMdlPerfMonitSubPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the AIMLE Server shall update the targeted "AIMLE ML Model Performance Monitor Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIMLE ML Model Performance Monitor Subscriptions" resource within the MlMdlPerfMonitSub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.9.7.

##### 5.2.8.2.4	AIMLES_MLModelPerfMonitor_Unsubscribe

5.2.8.2.4.1	General

This service operation is used by a service consumer to request the deletion of AIMLE ML Model Performance Monitor Subscription at the ADAE Server.

The following procedures are supported by the "AIMLES_MLModelPerfMonitor_Unsubscribe" service operation:

-	AIMLE ML Model Performance Monitor Subscription Deletion.

5.2.8.2.4.2	AIMLE ML Model Performance Monitor Subscription Deletion

Figure 5.2.8.2.4.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to delete an existing AIML ML Model Performance Monitor Subscription (see also clause 8.22 of 3GPP°TS°23.482°[13]).

Figure 5.2.8.2.4.2-1: Procedure for AIMLE ML Model Performance Monitor Subscription Deletion

1.	In order to request the deletion of an existing AIMLE ML model performance monitoring subscription, the service consumer shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding "Individual AIMLE ML Model Performance Subscription" resource.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.9.7.

##### 5.2.8.2.5	AIMLES_MLModelPerfMonitor_Notify

5.2.8.2.5.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE ML Model Performance Monitor report(s).

The following procedures are supported by the "AIMLE_MLModelPerfMonitor_Notify" service operation:

-	AIMLE ML Model Performance Monitor Notification.

5.2.8.2.5.2	AIMLE ML Model Performance Monitor Notification

Figure 5.2.8.2.5.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE ML Model Performance Monitor report(s) (see also clause 8.22 of 3GPP°TS°23.482°[13]).

Figure 5.2.8.2.5.2-1: Procedure for AIMLE ML Model Performance Monitor Notification

1.	In order to notify a previously subscribed service consumer on AIMLE ML Model Performance Monitor report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE ML Model Performance Monitor Subscription using the procedures defined in clauses 5.2.8.2.2.2, and the request body including the MlMdlPerfMonitNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.9.7.

### 5.2.9	AIMLES_AssistedMLModelSelection Service

#### 5.2.9.1	Service Description

The AIMLES_AssistedMLModelSelection service exposed by the AIMLE Server enables a service consumer to:

-	create/update/delete a AIMLE Assisted ML Model Selection Subscription; and

-	receive AIMLE Assisted ML Model Selection Notifications.

#### 5.2.9.2	Service Operations

##### 5.2.9.2.1	Introduction

The service operations defined for the AIMLES_AssistedMLModelSelection API are shown in the table 5.2.9.10.2.1-1.

Table 5.2.9.2.1-1: Service operations of the AIMLES_AssistedMLModelSelection API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_AssistedMLModelSelection_Subscribe | This service operation enables a service consumer to create a AIMLE Assisted ML Model Selection Subscription. | e.g. VAL Server |
| AIMLES_AssistedMLModelSelection_Update | This service operation enables a service consumer to update an existing AIMLE Assisted ML Model Selection Subscription. | e.g. VAL Server |
| AIMLES_AssistedMLModelSelection_Unsubscribe | This service operation enables a service consumer to delete an existing AIMLE Assisted ML Model Selection Subscription. | e.g. VAL Server |
| AIMLES_AssistedMLModelSelection_Notify | This service operation enables a service consumer to receive AIMLE Assisted ML Model Selection notifications. | AIMLE Server |

##### 5.2.9.2.2	AIMLES_AssistedMLModelSelection_Subscribe

5.2.9.2.2.1	General

This service operation is used by a service consumer to request the creation of AIMLE Assisted ML Model Selection Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_AssistedMLModelSelection_Subscribe" service operation:

-	AIMLE Assisted ML Model Selection Subscription Creation.

This service operation is used by the analytics consumer for AIMLE Assisted ML Model Selection Subscription to the AIMLE Server.

5.2.9.2.2.2	AIMLE Assisted ML Model Selection Subscription Creation

Figure 5.2.9.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the creation of AIMLE Assisted ML Model Selection Subscription (see also clause 8.23 of 3GPP°TS°23.482°[13]).

Figure 5.2.9.2.2.2-1: Procedure for AIMLE Assisted ML Model Selection Subscription Creation

1.	In order to subscribe to AIMLE Assisted ML Model Selection notification, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "AIMLE Assisted ML Model Selection Subscriptions" collection resource, with the request body including the AssistMLMdlSelSubsc data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIMLE Assisted ML Model Selection Subscription" resource within the AssistMLMdlSelSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.10.7.

##### 5.2.9.2.3	AIMLES_AssistedMLModelSelection_Update

5.2.9.2.3.1	General

This service operation is used by a service consumer to request the update of a AIMLE Assisted ML Model Selection Subscription at the AIMLE server.

The following procedures are supported by the "AIMLES_AssistedMLModelSelection_Subscribe" service operation:

-	AIMLE Assisted ML Model Selection Subscription Update.

This service operation is used by the analytics consumer for AIMLE Assisted ML Model Selection Subscription to the AIMLE Server.

5.2.9.2.3.2	AIMLE Assisted ML Model Selection Subscription Update

Figure 5.2.9.2.5.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the update of AIMLE Assisted ML Model Selection Subscription (see also clause 8.23 of 3GPP°TS°23.482°[13]).

Figure 5.2.9.2.3.2-1: Procedure for AIMLE Assisted ML Model Selection Subscription Creation

1.	In order to request the update of an existing AIMLE Assisted ML Model Selection notification, the service consumer shall send an HTTP PUT/PATCH request to the AIMEL Server, targeting the URI of the corresponding "Individual AIMLE Assisted ML Model Selection Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the AssistMLMdlSelSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the AssistMLMdlSelSubscPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall update the targeted "AIMLE Assisted ML Model Selection Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIMLE Assisted ML Model Selection Subscriptions" resource within the AssistMLMdlSelSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.10.7.

##### 5.2.9.2.4	AIMLES_AssistedMLModelSelection_Unsubscribe

5.2.9.2.4.1	General

This service operation is used by a service consumer to request the deletion of AIMLE Assisted ML Model Selection Subscription at the AIMLE Server.

The following procedures are supported by the "AIMLES_AssistedMLModelSelection_Unsubscribe" service operation:

-	AIMLE Assisted ML Model Selection Subscription Deletion.

5.2.9.2.4.2	AIMLE Assisted ML Model Selection Subscription Deletion

Figure 5.2.9.2.4.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to delete an existing AIML Member Capability Analytics Subscription (see also clause 8.23 of 3GPP°TS°23.482°[13]).

Figure 5.2.9.2.4.2-1: Procedure for AIMLE Assisted ML Model Selection Subscription Deletion

1.	In order to request the deletion of an existing AIMLE Assisted ML Model Selection Subscription, the service consumer shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding "Individual AIMLE Assisted ML Model Selection Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.10.7.

##### 5.2.9.2.5	AIMLES_AssistedMLModelSelection_Notify

5.2.9.2.5.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE Assisted ML Model Selection report(s).

The following procedures are supported by the "AIMLE Assisted ML Model Selection_Notify" service operation:

-	AIMLE Assisted ML Model Selection Notification.

5.2.9.2.5.2	AIMLE Assisted ML Model Selection Notification

Figure 5.2.9.2.5.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE Assisted ML Model Selection report(s) (see also clause 8.23 of 3GPP°TS°23.482°[13]).

Figure 5.2.9.2.5.2-1: Procedure for AIMLE Assisted ML Model Selection Notification

1.	In order to notify a previously subscribed service consumer on AIMLE Assisted ML Model Selection report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE Assisted ML Model Selection Subscription using the procedures defined in clauses 5.2.9.2.2.2, and the request body including the AssistMLMdlSelNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.10.7.

### 5.2.10	Aimles_MLModelRetrieval Service

#### 5.2.10.1	Service Description

The Aimles_MLModelRetrieval service exposed by the AIMLE Server enables a service consumer to:

-	request AIMLE server for AIMLE ML Model retrieval according to filtering criteria;

-	request AIMLE server to create/update/delete a AIMLE ML Model retrieval subscription; and

-	receive AIMLE ML Model retrieval notifications.

#### 5.2.10.2	Service Operations

##### 5.2.10.2.1	Introduction

The service operations defined for the Aimles_MLModelRetrieval API are shown in the table 5.2.10.10.2.1-1.

Table 5.2.10.2.1-1: Service operations of the Aimles_MLModelRetrieval API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| Aimles_MLModelRetrieval_Request | This service operation enables a service consumer to request for AIMLE ML Model Retrieval. | e.g. VAL Server, AIMLE client |
| Aimles_MLModelRetrieval_Subscribe | This service operation enables a service consumer to create a AIMLE ML Model Retrieval Subscription. | e.g. VAL Server, AIMLE client |
| Aimles_MLModelRetrieval_Update | This service operation enables a service consumer to update an existing AIMLE ML Model Retrieval Subscription. | e.g. VAL Server, AIMLE client |
| Aimles_MLModelRetrieval_Unsubscribe | This service operation enables a service consumer to delete an existing AIMLE ML Model Retrieval Subscription. | e.g. VAL Server, AIMLE client |
| Aimles_MLModelRetrieval_Notify | This service operation enables a service consumer to receive AIMLE ML Model Retrieval notifications. | AIMLE Server, AIMLE client |

##### 5.2.10.2.2	Aimles_MLModelRetrieval_Request

5.2.10.2.2.1	General

This service operation is used by a service consumer to request for one time ML AIMLE Model retrieval at the AIMLE server.

The following procedures are supported by the "Aimles_MLModelRetrieval_Request" service operation:

-	AIMLE ML Model Retrieval Request.

5.2.10.2.2.2	AIMLE ML Model Retrieval Request

Figure 5.2.10.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server for one time AIMLE ML Model Retrieval (see also clause 8.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.10.2.2.2-1: Procedure for AIMLE ML Model Retrieval Request

1.	In order to request for one time AIMLE ML Model Retrieval, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "Retrieve" custom operation (i.e., "{apiRoot}/aimless-mlmr<apiVersion>/retrieve") as specified in clause 6.1.13.4, with the request body including the MLMdlReq data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code with the response body including the MLMdlRsp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.13.7.

##### 5.2.10.2.3	Aimles_MLModelRetrieval_Subscribe

5.2.10.2.3.1	General

This service operation is used by a service consumer to request the creation of AIMLE ML Model Retrieval Subscription at the AIMLE server.

The following procedures are supported by the "Aimles_MLModelRetrieval_Subscribe" service operation:

-	AIMLE ML Model Retrieval Subscription Creation.

5.2.10.2.3.2	AIMLE ML Model Retrieval Subscription Creation

Figure 5.2.10.2.3.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the creation of AIMLE ML Model Retrieval Subscription (see also clause 8.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.10.2.3.2-1: Procedure for AIMLE ML Model Retrieval Subscription Creation

1.	In order to subscribe to AIMLE ML Model Retrieval, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the "AIMLE ML Model Retrieval Subscriptions" collection resource, with the request body including the MLMdlRetSub data structure.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIMLE ML Model Retrieval Subscription" resource within the MLMdlRetSub data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.x.7.

##### 5.2.10.2.4	Aimles_MLModelRetrieval_Update

5.2.10.2.4.1	General

This service operation is used by a service consumer to request the update of a AIMLE ML Model Retrieval Subscription at the AIMLE server.

The following procedures are supported by the "Aimles_MLModelRetrieval_Subscribe" service operation:

-	AIMLE ML Model Retrieval Subscription Update.

5.2.10.2.4.2	AIMLE ML Model Retrieval Subscription Update

Figure 5.2.10.2.4.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request the update of AIMLE ML Model Retrieval Subscription (see also clause 8.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.10.2.4.2-1: Procedure for AIMLE ML Model Retrieval Subscription Creation

1.	In order to request the update of an existing AIMLE ML Model Retrieval Subscription, the service consumer shall send an HTTP PUT/PATCH request to the AIMLE Server, targeting the URI of the corresponding "Individual AIMLE ML Model Retrieval Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the MLMdlRetSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the MLMdlRetSubPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall update the targeted "AIMLE ML Model Retrieval Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIMLE ML Model Retrieval Subscriptions" resource within the MLMdlRetSub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.1.13.7.

##### 5.2.10.2.5	Aimles_MLModelRetrieval_Unsubscribe

5.2.10.2.5.1	General

This service operation is used by a service consumer to request the deletion of AIMLE ML Model Retrieval Subscription at the AIMLE Server.

The following procedures are supported by the "Aimles_MLModelRetrieval_Unsubscribe" service operation:

-	AIMLE ML Model Retrieval Subscription Deletion.

5.2.10.2.5.2	AIMLE ML Model Retrieval Subscription Deletion

Figure 5.2.10.2.4.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to delete an existing AIMLE ML Model Retrieval Subscription (see also clause 8.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.10.2.5.2-1: Procedure for AIMLE ML Model Retrieval Subscription Deletion

1.	In order to request the deletion of an existing AIMLE ML Model Retrieval Subscription, the service consumer shall send an HTTP DELETE request to the AIMLE Server targeting the URI of the corresponding "Individual AIMLE ML Model Retrieval Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.x.7.

##### 5.2.10.2.6	Aimles_MLModelRetrieval_Notify

5.2.10.2.6.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE ML Model Retrieval report(s).

The following procedures are supported by the "ML Model Retrieval_Notify" service operation:

-	AIMLE ML Model Retrieval Notification.

5.2.10.2.6.2	AIMLE ML Model Retrieval Notification

Figure 5.2.10.2.6.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE ML Model Retrieval report(s) (see also clause 8.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.10.2.6.2-1: Procedure for AIMLE ML Model Retrieval Notification

1.	In order to notify a previously subscribed service consumer on AIMLE ML Model Retrieval report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE ML Model Retrieval Subscription using the procedures defined in clauses 5.2.10.2.2.2, and the request body including the MLMdlRetNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.x.7.

### 5.2.11	AIMLES_TLModelSelectionAssistance Service

#### 5.2.11.1	Service Description

The AIMLES_TLModelSelectionAssistance Service, exposed by the AIMLE Server, enables a service consumer to:

-	request AIMLE Server to provide pre-trained ML models for a target ML task by using filter criteria.

#### 5.2.11.2	Service Operations

##### 5.2.11.2.1	Introduction

The service operations defined for the AIMLES_TLModelSelectionAssistance API are shown in the table 5.2.11.2.1-1.

Table 5.2.11.2.1-1: Service operations of the AIMLES_TLModelSelectionAssistance API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_TLModelSelectionAssistance_Request | This service operation enables a service consumer to perform an ML model training by using pre-trained models. | e.g., VAL Server |

##### 5.2.11.2.2	AIMLES_TLModelSelectionAssistance_Request

5.2.11.2.2.1	General

This service operation is used by a service consumer to perform AIMLE TL Model Selection Assistance Request at the AIMLE Server.

The following procedures are supported by the "AIMLES_TLModelSelectionAssistance_Request" service operation:

-	AIMLE TL Model Selection Assistance Request.

5.2.11.2.2.2	AIMLE TL Model Selection Assistance Request

Figure 5.2.11.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE TL Model Selection Assistance (see also clause 8.16 of 3GPP°TS°23.482°[13]).

Figure 5.2.11.2.2.2-1: Procedure for AIMLE TL Model Selection Assistance Request

1.	In order to request to AIMLE ML model selection assistance, the service consumer shall send an HTTP GET request to the AIMLE Server targeting the URI of the "AIMLE TL Model Selection Assistance" resource.

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code with the response body containing the TlModelSelectAssistResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.10.7.

### 5.2.12	AIMLES_SplitOpNodeRegistration

#### 5.2.12.1	Service Description

The AIMLES_SplitOpNodeRegistration service exposed by the AIMLE Server enables a service consumer to:

-	request, update, and deregister for split AI/ML operation node registration.

#### 5.2.12.2	Service Operations

##### 5.2.12.2.1	Introduction

The service operation defined for AIMLES_SplitOpNodeRegistration API is shown in Table 5.2.12.2.1-1.

Table 5.2.1.2.1-1: AIMLES_ContextTransfer Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_SplitOpNodeRegistration_Request | This service operation is used by a service consumer to request for one time split AI/ML node registration. | e.g., VAL server |
| AIMLES_SplitOpNodeRegistration_Update | The service operation is used by a service consumer to update split AI/ML node registration. | e.g., VAL server |
| AIMLES_SplitOpNodeRegistration_Deregister | The service operation is used by a service consumer to deregister split AI/ML node registration. | e.g. VAL server |

##### 5.2.12.2.2	Aimles_SplitOpNodeRegistration_Request

5.2.12.2.2.1	General

This service operation is used by a service consumer to request AIMLE split operation node registration with the AIMLE server.

The following procedures are supported by the "AIMLES_SplitOpNodeRegistration_Request" service operation:

-	AIMLE Split Operation Node Registration.

5.2.12.2.2.2	AIMLE split operation node registration request

Figure 5.2.12.2.2.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE Split Operation Node Registration Request (see also clause 8.14.2.4.2 of 3GPP°TS°23.482°[13]).

Figure 5.2.1.2.2.2-1: Procedure for AIMLE Split Operation Node Registration Request

1.	In order to register for AIMLE Split Operation Node Registration, the VAL server shall send an HTTP POST request to the AIMLE Server targeting the URI of the corresponding custom operation, with the request body including the SplitOpNodeRegInfo data structure as specified in clause 6.1.X.6.2.3.

2a.	Upon reception of the HTTP POST registration request, the AIMLE server shall perform an authentication and authorization check to determine if the service consumer is permitted to register at the AIMLE server and participate in AIML operations. If the VAL server is authorized to register at the AIMLE server, the AIMLE server shall:

a)	create a new "Individual AIMLE split operation node registration" resource with the received registration information; and

b)	respond with an HTTP "201 Created" status code with the response body including the SplitOpNodeReg data structure and an HTTP "Location" header field containing the URI of the created resource.

2b. If the VAL server is not authorized to register at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.X.7, and respond with an appropriate error status code.

##### 5.2.12.2.3	Aimles_ SplitOpNodeRegistration_Update service operation

###### 5.2.12.2.3.1	General

The Aimles_ SplitOpNodeRegistration_Update service operation is used by the VAL server to update its registration information at the AIMLE server.

###### 5.2.12.2.3.2	AIMLE split operation node registration update

Figure 5.2.12.2.3.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE Split Operation Node Registration Update (see also clause 8.14.2.4.3 of 3GPP°TS°23.482°[13]).

Figure 5.2.12.2.3.2-1: Procedure for AIMLE Split Operation Node Registration Update

1.	In order to update an Individual Registered AIMLE Split Operation Node Register, the service consumer shall send an HTTP PUT/PATCH request to the AIMLE Server targeting the URI of the corresponding resource (i.e., "Individual Registered AIMLE Split Operation Node Register Configuration"), with the request body including either:

-	the updated representation of the resource within the SplitOpNodeReg data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the SplitOpNodeRegPatch data structure, in case the HTTP PATCH method is used.

2a. Upon reception of the HTTP PUT/PATCH request, the AIMLE server shall perform an authentication and authorization check to determine if the VAL server is permitted to update the targeted registration. If the VAL server is authorized to update the targeted registration at the AIMLE server, the AIMLE server shall:

a)	accordingly update the targeted "Individual AIMLE split operation node registration" resource; and

b)	respond with either:

-	an HTTP "204 No Content" status code; or

-	an HTTP "200 OK" status code with the response body including a representation of the updated resource within the SplitOpNodeReg data structure.

2b. If the VAL server is not authorized to update the targeted registration at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.X.7, and respond with an appropriate error status code.

##### 5.2.12.2.4	Aimles_ SplitOpNodeRegistration_Deregister service operation

###### 5.2.12.2.4.1	General

The Aimles_ SplitOpNodeRegistration_Deregister service operation is used by the VAL server to deregister itself from the Split AI/ML operation.

###### 5.2.12.2.4.2	AIML split operation node deregistration

Figure 5.2.12.2.4.2-1 depicts a scenario where a VAL server sends a request to the AIMLE Server for AIMLE split operation node deregistration (see also clause 8.14.2.4.4 of 3GPP°TS°23.482°[13]).

Figure 5.2.12.2.4.2-1: Procedure for AIMLE Split Operation Node Deregistration

1. To deregister itself at the AIMLE server, the VAL server shall send an HTTP DELETE request to the AIMLE server targeting the "Individual AIMLE split operation registration" resource, as specified in clause 6.1.X.3.3.3.2.

2a. Upon reception of the HTTP DELETE request, the AIMLE server shall perform an authentication and authorization check to determine if the VAL server is permitted to deregister at the AIMLE server. If the VAL server	is authorized to deregister at the AIMLE server, the AIMLE server shall:

a)	delete the corresponding "Individual AIMLE Split Operation Node registration" resource; and

b)	respond with an HTTP "204 Not Content" status code.

2b. If the VAL server is not authorized to deregister at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.1.X.7, and respond with an appropriate error status code.

## 5.3	Services offered by the AIMLE Repository

### 5.3.1	MLR_MLModelManagement Service

#### 5.3.1.1	Service Description

The MLR_MLModelManagement service exposed by the AIMLE Repository enables an NF service consumer to:

-	request to create/update/delete an ML Model(s) Storage.

#### 5.3.1.2	Service Operations

##### 5.3.1.2.1	Introduction

The service operations defined for the MLR_MLModelManagement service are shown in table 5.3.1.2.1-1.

Table 5.3.1.2.1-1: MLR_MLModelManagement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| MLR_MLModelManagement_Store | This service operation enables the NF service consumer to request to create/update/delete an ML Model(s) Storage. | e.g., AIMLE Server |

##### 5.3.1.2.2	MLR_MLModelManagement_Store

5.3.1.2.2.1	General

This service operation is used by a service consumer to request to create/update/delete an ML Model(s) Storage at the AIMLE Repository.

The following procedures are supported by the "MLR_MLModelManagement_Store" service operation:

-	ML Models Storage Creation.

-	ML Models Storage Update.

-	ML Models Storage Deletion.

###### 5.3.1.2.2.2	ML Models Storage Creation

Figure 5.3.1.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to request the creation of an ML Model(s) Storage (see also clause 8.11 of 3GPP°TS°23.482°[13]).

Figure 5.3.1.2.2.2-1: Procedure for ML Models Storage Creation

1.	In order to create or reserve a new ML Model(s) Storage, the service consumer shall send an HTTP POST request to the AIMLE Repository targeting the URI of the "ML Models Storages" collection resource, with the request body including the MLModelsStorage data structure.

2a.	Upon success, the AIMLE Repository shall respond with an HTTP "201 Created" status code, with the response body containing a representation of the created "Individual ML Models Storage" resource within the MLModelsStorage data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.1.7.

###### 5.3.1.2.2.3	ML Models Storage Update

Figure 5.3.1.2.2.3-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to request the update of an existing ML Model(s) Storage (see also clause 8.11 of 3GPP°TS°23.482°[13]).

Figure 5.3.1.2.2.3-1: Procedure for ML Models Storage Update

1.	In order to update an existing ML Model(s) Storage, the service consumer shall send an HTTP PUT/PATCH request to the AIMLE Repository, targeting the URI of the corresponding "Individual ML Models Storage" resource, with the request body including either:

-	the updated representation of the resource within the MLModelsStorage data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the MLModelsStoragePatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e., other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Repository shall update the targeted "Individual ML Models Storage" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual ML Models Storage" resource within the MLModelsStorage data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.2.1.7.

###### 5.3.1.2.2.4	ML Models Storage Deletion

Figure 5.3.1.2.2.4-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to request the deletion of an existing ML Model(s) Storage (see also clause 8.11 of 3GPP°TS°23.482°[13]).

Figure 5.3.1.2.2.4-1: Procedure for ML Models Storage Deletion

1.	In order to request the deletion of an existing ML Models Storage, the service consumer shall send an HTTP DELETE request to the AIMLE Repository targeting the corresponding "Individual ML Models Storage" resource.

NOTE:	An alternative service consumer (i.e., other than the one that requested the creation/update of the targeted resource) can initiate this request.

2a.	Upon success, the AIMLE Repository shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.2.1.7.

### 5.3.2	MLR_ModelInformationDiscovery Service

#### 5.3.2.1	Service Description

The MLR_ModelInformationDiscovery service exposed by the MLR enables a service consumer to:

-	request MLR the model information discovery.

#### 5.3.2.2	Service Operations

##### 5.3.2.2.1	Introduction

The service operation defined for MLR_ModelInformationDiscovery API is shown in the table 5.3.2.2.1-1.

Table 5.3.2.2.1-1: MLR_ModelInformationDiscovery Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| MLR_MLModelInformationDiscovery_Request | This service operation is used by a service consumer to request the model information discovery. | AIMLE Server |

#### 5.3.2.2.2	MLR_MLModelInformationDiscovery_Request

###### 5.3.2.2.2.1	General

This service operation is used by a service consumer to request the discovery of the ML model.

The following procedures are supported by the "MLR_MLModelInformationDiscovery_Request" service operation:

-	ML model discovery request.

###### 5.3.2.2.2.2	ML model discovery request

Figure 5.3.2.2.2.2-1 depicts a scenario where a service consumer sends a request to the MLR to request the discovery of the ML model(s) (see also clause 8.11 of 3GPP°TS°23.482°[13]).

Figure 5.3.2.2.2.2-1: Procedure for ML model discovery request

1.	In order to discover the ML model(s), the service consumer shall send an HTTP GET request to the MLR targeting the URI of the "ML Models" resource.

2a.	Upon success, the MLR shall respond with an HTTP "200 OK" status code with the response body containing the DiscoveryResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.2.2.7.

### 5.3.3	MLR_FLEvents Service

#### 5.3.3.1	Service Description

The MLR_FLEvents Service, exposed by the AIMLE Server or the VAL Server via the AIMLE Server, enables a service consumer to:

-	create/update/delete an MLR FL Events Subscription; and

-	receive MLR FL Events Notifications.

#### 5.3.3.2	Service Operations

##### 5.3.3.2.1	Introduction

The service operations defined for the MLR_FLEvents API are shown in the table 5.3.3.2.1-1.

Table 5.3.3.2.1-1: Service operations of the MLR_FLEvents API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| MLR_FLEvents_Subscribe | This service operation enables a service consumer to create an FL related event subscription. | e.g., AIMLE Server |
| MLR_FLEvents_Update | This service operation enables a service consumer to update an existing FL related event subscription. | e.g., AIMLE Server |
| MLR_FLEvents_Unsubscribe | This service operation enables a service consumer to delete an existing FL related event subscription. | e.g., AIMLE Server |
| MLR_FLEvents_Notify | This service operation enables a service consumer to receive FL related events notifications. | ML Repository |

##### 5.3.3.2.2	MLR_FLEvents_Subscribe

5.3.3.2.2.1	General

This service operation is used by an e.g., AIMLE Server to request the creation of an MLR FL Events Subscription at the ML Repository.

The following procedures are supported by the "MLR_FLEvents_Subscribe" service operation:

-	MLR FL Events Subscription Creation.

This service operation is used by the analytics consumer e.g., AIMLE Server for MLR FL Events Subscription to the ML Repository.

5.3.3.2.2.2	MLR FL Events Subscription Creation

Figure 5.3.3.2.2.2-1 depicts a scenario where a service consumer sends a request to the ML Repository to request the creation of MLR FL Events Subscription (see also clause 8.5 of 3GPP°TS°23.482°[13]).

Figure 5.3.3.2.2.2-1: Procedure for MLR FL Events Subscription Creation

1.	In order to subscribe to MLR FL events, the service consumer shall send an HTTP POST request to the ML Repository targeting the URI of the "MLR FL Events Subscriptions" collection resource, with the request body including the FlEvtsSub data structure.

2a.	Upon success, the ML Repository shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual MLR FL Events Subscription" resource within the FlEvtsSub data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.3.7.

##### 5.3.3.2.3	MLR_FLEvents_Update

5.3.3.2.3.1	General

This service operation is used by an e.g., the AIMLE Server to request the update of an MLR FL Events Subscription at the ML Repository.

The following procedures are supported by the "MLR_FLEvents_Update" service operation:

-	MLR FL Events Subscription Update.

This service operation is used by the analytics consumer for MLR FL Events Subscription Update to the ML Repository.

5.3.3.2.3.2	MLR FL Events Subscription Update

Figure 5.3.3.2.5.2-1 depicts a scenario where a service consumer sends a request to the ML Repository to request the update of ML FL Events Subscription (see also clause 8.5 of 3GPP°TS°23.482°[13]).

Figure 5.3.3.2.3.2-1: Procedure for MLR FL Events Subscription Update

1.	In order to request the update of an existing MLR FL events subscription, the service consumer shall send an HTTP PUT/PATCH request to the ML Repository, targeting the URI of the corresponding "Individual MLR FL Events Subscriptions" resource, with the request body including either:

-	the updated representation of the resource within the FlEvtsSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the FlEvtsSubPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the ML Repository shall update the targeted "MLR FL Events Subscriptions" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual MLR FL Events Subscriptions" resource within the FlEvtsSub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.2.3.7.

##### 5.3.3.2.4	MLR_FLEvents_Unsubscribe

5.3.3.2.4.1	General

This service operation is used by an e.g., the AIMLE Server to request the deletion of MLR FL Events Subscription at the ML Repository.

The following procedures are supported by the "MLR_FLEvents_Unsubscribe" service operation:

-	MLR FL Events Subscription Deletion.

5.3.3.2.4.2	MLR FL Events Subscription Deletion

Figure 5.3.3.2.4.2-1 depicts a scenario where a service consumer sends a request to the ML Repository to delete an existing MLR FL Events Subscription (see also clause 8.5 of 3GPP°TS°23.482°[13]).

Figure 5.3.3.2.4.2-1: Procedure for MLR FL Events Subscription Deletion

1.	In order to request the deletion of an existing MLR FL events subscription, the service consumer shall send an HTTP DELETE request to the ML Repository targeting the URI of the corresponding "Individual MLR FL Events Subscription" resource.

2a.	Upon success, the ML Repository shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.2.3.7.

##### 5.3.3.2.5	MLR_FLEvents_Notify

5.3.3.2.5.1	General

This service operation is used by an ML Repository to notify a previously subscribed service consumer on:

-	MLR FL Events report(s).

The following procedures are supported by the "MLR_FlEvents_Notify" service operation:

-	MLR FL Events Notification.

5.3.3.2.5.2	MLR FL Events Notification

Figure 5.3.3.2.5.2-1 depicts a scenario where the ML Repository sends a request to notify a previously subscribed service consumer on MLR FL Events report(s) (see also clause 8.5 of 3GPP°TS°23.482°[13]).

Figure 5.3.3.2.5.2-1: Procedure for MLR FL Events Notification

1.	In order to notify a previously subscribed service consumer on MLR FL Events report(s), the MLR repository shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding MLR FL Events Subscription using the procedures defined in clauses 5.3.3.2.2.2, and the request body including the FlEvtsNotif data structure.

2a.	Upon success, the service consumer shall respond to the ML Repository with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.3.7.

### 5.3.4	MLR_FLMember Service

#### 5.3.4.1	Service Description

The MLR_FLMember Service, exposed by the AIMLE Repository, enables a service consumer to:

-	register/update/deregister/retrieve FL members consisting of AIMLE Clients.

#### 5.3.4.2	Service Operations

##### 5.3.4.2.1	Introduction

The service operations defined for MLR_FLMember API for are shown in the table 5.3.4.2.1-1.

Table 5.3.4.2.1-1: Operations for MLR_FLMember API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MLR_FLMember_Register | This service operation is used to register an Individual FL Member. | e.g., AIMLE Server |
| MLR_FLMember_Query | This service operation is used to query for an Individual registered FL Member. | e.g., AIMLE Server |
| MLR_FLMember_Update | This service operation is used to update registration of an Individual registered FL Member. | e.g., AIMLE Server |
| MLR_FLMember_Deregister | This service operation is used to deregister an Individual registered FL Member. | e.g., AIMLE Server |

##### 5.3.4.2.2	MLR_FLMember_Register

##### 5.3.4.2.2.1	General

This service operation is used by a service consumer to request the AIMLE Repository to register an Individual FL Member.

The following procedure is supported by the "MLR_FLMember_Register" service operation:

-	MLR FL Member Register.

###### 5.3.4.2.2.2	MLR FL Member Register

Figure 5.3.4.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to register an Individual FL Member. (see clause 8.4 of 3GPP°TS°23.482°[13]).

Figure 5.3.4.2.2.2-1: Procedure for MLR FL Member Register

1.	In order to register an Individual FL Member, the service consumer shall send an HTTP POST request to the AIMLE Repository targeting the URI of the corresponding resource (i.e., "FL Member Configurations"), with the request body including the FlMbr data structure.

2a.	Upon success that the request to register the Individual FL Member is successfully received and processed, the AIMLE Repository shall respond with an HTTP "201 Created" status code with the request body including the FlMbr data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.4.7.

##### 5.3.4.2.3	MLR_FLMember_Query

###### 5.3.4.2.3.1	General

This service operation is used by a service consumer to request the AIMLE Repository to query an Individual FL Member.

The following procedure is supported by the "MLR_FLMember_Query" service operation:

-	MLR FL Member Query.

###### 5.3.4.2.3.2	MLR FL Member Query

Figure 5.3.4.2.3.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to query an Individual FL Member. (see clause 8.4 of 3GPP°TS°23.482°[13]).

Figure 5.3.4.2.3.2-1: Procedure for MLR FL Member Query

1.	In order to query an Individual Registered FL Member, the service consumer shall send an HTTP GET request to the AIMLE Repository targeting the URI of the corresponding resource (i.e., "Query Registered FL Member").

2a.	Upon success that the request to query the Registered FL Member is successfully received and processed, the AIMLE Repository shall respond with an HTTP "200 OK" status code with the request body including the FlMbr data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.2.4.7.

##### 5.3.4.2.4	MLR_FLMember_Update_Register

###### 5.3.4.2.4.1	General

This service operation is used by a service consumer to request the AIMLE Repository to update registration of an Individual Registered FL Member.

The following procedure is supported by the "MLR_FLMember_Update_Register" service operation:

-	MLR FL Member Update Register.

###### 5.3.4.2.4.2	MLR FL Member Update Register

Figure 5.3.4.2.4.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to update registration of an Individual Registered FL Member. (see clause 8.4 of 3GPP°TS°23.482°[13]).

Figure 5.3.4.2.4.2-1: Procedure for MLRFL Member Update

1.	In order to update an Individual Registered FL Member, the service consumer shall send an HTTP PUT/PATCH request to the AIMLE Repository targeting the URI of the corresponding resource (i.e., "Individual Registered FL Member Configuration"), with the request body including either:

-	the updated representation of the resource within the FlMbr data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the FlMbrPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success that the request to update the Individual Registered FL Member is successfully received and processed, the AIMLE Repository shall respond with an HTTP "200 OK" status code with the request body including the FlMbr data structure.

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Registered FL Member Configuration" resource within the FlMbr data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.2.4.7.

##### 5.3.4.2.5	MLR_FLMember_Deregister

###### 5.3.4.2.5.1	General

This service operation is used by a service consumer to request the AIMLE Repository to deregister an Individual Registered FL Member.

The following procedure is supported by the "MLR_FLMember_Deregister" service operation:

-	MLR FL Member Deregister.

###### 5.3.4.2.5.2	MLR FL Member Deregister

Figure 5.3.4.2.5.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Repository to deregister an Individual Registered FL Member. (see also clause 8.4 of 3GPP°TS°23.482°[13]).

Figure 5.3.4.2.5.2-1: Procedure for MLR FL Member Deregister

1.	In order to delete an Individual Registered FL Member, the service consumer shall send an HTTP DELETE request to the AIMLE Repository targeting the URI of the corresponding resource (i.e., "Individual Registered FL Member Configuration").

2a.	Upon success that the request to delete the Individual Registered FL Member is successfully received and processed, the AIMLE Repository shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.2.4.7.

# 6	API Definitions

## 6.1	AIMLE Server APIs

### 6.1.1	AIMLES_ContextTransfer API

#### 6.1.1.1	Introduction

The AIMLES_ContextTransfer service shall use the AIMLES_ContextTransfer API.

The API URI of the AIMLES_ContextTransfer API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be " aimles-ct".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.1.1.3 and 6.1.1.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.1, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.1.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_ContextTransfer API.

#### 6.1.1.3	Resources

There are no resources defined for this API in this release of the specification.

#### 6.1.1.4	Custom Operations without associated resources

##### 6.1.1.4.1	Overview

The structure of the custom operation URIs of the AIMLES_ContextTransfer API is shown in Figure 6.1.1.4.1-1.

Figure 6.1.1.4.1-1: Custom operation URI structure of the AIMLES_ContextTrasnfer API

Table 6.1.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the AIMLES_ContextTransfer API.

Table 6.1.1.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestTrans | /request-trans | POST | Enables a service consumer to send AIMLE context transfer information to the AIMLE Server. |

The custom operations shall support the URI variables defined in table 6.1.1.4.1-2.

Table 6.1.1.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1.1. |

##### 6.1.1.4.2	Operation: RequestTrans

6.1.1.4.2.1	Description

The custom operation enables a service consumer to send AIMLE context transfer information to the AIMLE Server.

###### 6.1.1.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.1.4.2.2-1 and 6.1.1.4.2.2-2.

Table 6.1.1.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransReq | M | 1 | Contains the parameters to request AIMLE context transfer. |

Table 6.1.1.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE context transfer request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.1.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

Table 6.1.1.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

#### 6.1.1.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.1.1.6	Data Model

##### 6.1.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.1.6.1-1 specifies the data types defined for the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-1: AIMLES_ContextTransfer API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| TransReq | Clause 6.1.1.6.2.2 | Represents the AIMLE context information transfer request. |  |
| AimleServOpStatus | Clause 6.1.1.6.3.3 | Represents the AIMLE service operation status information. |  |
| AimleServStatus | Clause 6.1.1.6.2.3 | Represents the AIMLE service status information. |  |

Table 6.1.1.6.1-2 specifies data types re-used by the AIMLES_ContextTransfer API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_ContextTransfer API.

Table 6.1.1.6.1-2: AIMLES_ContextTransfer API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

##### 6.1.1.6.2	Structured data types

6.1.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.1.1.6.2.2	Type: TransReq

Table 6.1.1.6.2.2-1: Definition of type TransReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleClientId | AimleClientId | M | 1 | Contains the identifier of the concerned AIMLE Client associated with the context. |  |
| aimleServerId | string | O | 0..1 | Contains the identifier of the concerned AIMLE Server associated with service area that the AIMLE Client is currently in. |  |
| aimleServerIds | array(string) | O | 1..N | Contains the list of identifier(s) of the AIMLE Server(s) maintained by the AIMLE Client as the list of source AIMLE Server(s) whenever the UE transitioned from a source edge area to a target edge area. |  |
| aimleServStatus | AimleServStatus | O | 0..1 | Contains the status of the AIML operations at the AIMLE Client. |  |
| aimleServRes | FFS | O | 0..1 | Contains the result of the AIML operations performed by the AIMLE Client. |  |
| aimleServApp | FFS | O | 0..1 | Contains the AIMLE Client's applicability information for the performed AIML operations. |  |

Editor's Note:	The content of the AimleServRes and the aimleServApp attribute is FFS.

###### 6.1.1.6.2.3	Type: AimleServStatus

Table 6.1.1.6.2.2-1: Definition of type AimleServStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleServOpStatus | AimleServOpStatus | M | 1 | Identifies the AIMLE service status of AIML operation at the AIMLE Client. |  |
| aimleServOpPercent | integer | C | 0..1 | Contains the percentage of completion of the AIML operation at the AIMLE Client. Minimum = 0. Maximum = 100. This attribute shall be present only when the "AimleServOpStatus" attribute value is either "ACTIVE" or "PAUSED". |  |

##### 6.1.1.6.3	Simple data types and enumerations

6.1.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.1.1.6.3.2	Simple data types

The simple data types defined in table 6.1.1.6.3.2-1 shall be supported.

Table 6.1.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AimleClientId | string | Represents the unique identifier of a AIMLE Client. |  |

###### 6.1.1.6.3.3	Enumeration: AimleServOpStatus

The enumeration AimleServOpStatus represents the AIMLE service operation status information. It shall comply with the provisions defined in table 6.1.1.6.3.3-1.

Table 6.1.1.6.3.3-1: Enumeration AimleServOpStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVE | Indicates that the status of the AIML operations at the AIMLE Client is active. |  |
| PAUSED | Indicates that the status of the AIML operations at the AIMLE Client is paused. |  |

##### 6.1.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.1.6.5	Binary data

6.1.1.6.5.1	Binary Data Types

Table 6.1.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.1.7	Error Handling

##### 6.1.1.7.1	General

For the AIMLES_ContextTransfer API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_ContextTransfer API.

##### 6.1.1.7.2	Protocol Errors

No specific protocol errors for the AIMLES_ContextTransfer API are specified.

##### 6.1.1.7.3	Application Errors

The application errors defined for the AIMLES_ContextTransfer API are listed in Table 6.1.1.7.3-1.

Table 6.1.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.1.8	Feature negotiation

The optional features in table 6.1.1.8-1 are defined for the AIMLES_ContextTransfer API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.1.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_ContextTransfer API API.

### 6.1.2	AIMLES_DataManagement API

#### 6.1.2.1	Introduction

The AIMLES_DataManagement service shall use the AIMLES_DataManagement API.

The API URI of the AIMLES_DataManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-dm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.1.2.3 and 6.1.2.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.2, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.2.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_DataManagement API.

#### 6.1.2.3	Resources

##### 6.1.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.2.3.1-1 depicts the resource URIs structure for the AIMLES_DataManagement API.

Figure 6.1.2.3.1-1: Resource URI structure of the AIMLES_DataManagement API

Table 6.1.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE Data Management Assistance Subscriptions | /subscriptions | POST | Request the creation of a AIMLE Data Management Assistance Subscription resource. |
| Individual AIMLE Data Management Assistance Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE Data Management Assistance Subscription" resource. |

##### 6.1.2.3.2	Resource: AIMLE Data Management Assistance Subscriptions

6.1.2.3.2.1	Description

This resource represents the collection of AIMLE Data Management Assistance Subscriptions managed by the AIMLE Server.

6.1.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-dm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.2.3.2.2-1.

Table 6.1.2.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.2.1. |

6.1.2.3.2.3	Resource Standard Methods

6.1.2.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual AIMLE Data Management Assistance Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.2.3.1-1.

Table 6.1.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.2.3.2.3.1-3.

Table 6.1.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | Create a new Individual AIMLE Data Management Assistance Subscription resource. |

Table 6.1.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE Data Management Assistance Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-dm/<apiVersion>/subscriptions{subscriptionId} |

6.1.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.2.3.3	Resource: Individual AIMLE Data Management Assistance Subscription

6.1.2.3.3.1	Description

6.1.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-dm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.2.3.3.2-1.

Table 6.1.2.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.2.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE Data Management Assistance Subscription" resource. |

6.1.2.3.3.3	Resource Standard Methods

6.1.2.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual AIMLE Data Management Assistance Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.3.3.1-1.

Table 6.1.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.1-3.

Table 6.1.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE Data Management Assistance Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.2.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual AIMLE Data Management Assistance Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.3.3.2-1.

Table 6.1.2.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.2-3.

Table 6.1.2.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | Represents the updated representation of the "Individual AIMLE Data Management Assistance Subscription" resource. |

Table 6.1.2.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 200 OK | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.2.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual AIMLE Data Management Assistance Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.2.3.3.3.3-1.

Table 6.1.2.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.3-3.

Table 6.1.2.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual AIMLE Data Management Assistance Subscription" resource. |

Table 6.1.2.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataMgmtAssistSubsc | M | 1 | 200 OK | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.2.3.3.3.4	DELETE

The HTTP PATCH method allows a AIMLE service consumer to request the deletion of an existing "Individual AIMLE Data Management Assistance Subscription" resource.

Table 6.1.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.2.3.3.3.4-3.

Table 6.1.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Data Management Assistance Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.2.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.2.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 6.1.2.5	Notifications

##### 6.1.2.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Data Management Assistance Event Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed service consumer on AIML Data Management Assistance related event(s). |

##### 6.1.2.5.2	AIMLE Data Management Assistance Event Notification

6.1.2.5.2.1	Description

The AIMLE Data Management Assistance Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML Data Management Assistance related event(s).

6.1.2.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.2.5.2.2-1.

Table 6.1.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification Uri is assigned within the Individual AIMLE Data Management Assistance Subscription and described within the DataMgmtAssistSubsc type |

6.1.2.5.2.3	Standard Methods

6.1.2.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.2.5.2.3-1 and the response data structures and response codes specified in table 6.1.2.5.2.3-2.

Table 6.1.2.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataMgmtAssistNotif | M | 1 | Represents the AIMLE Data Management Assistance Event Notification. |

Table 6.1.2.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Data Management Assistance Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.2.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.2.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.2.6	Data Model

##### 6.1.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.2.6.1-1 specifies the data types defined for the AIMLES_DataManagement API.

Table 6.1.2.6.1-1: AIMLES_DataManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DataMgmtAssistSubsc | 6.1.2.6.2.2 | Represents the AIMLE Data Management Assistance subscription information. |  |
| DataMgmtAssistSubscPatch | 6.1.2.6.2.7 | Represents the requested modifications to a AIMLE Data Management Assistance subscription information. |  |
| DataMgmtAssistNotif | 6.1.2.6.2.3 | Represents the AIMLE Data Management Assistance notification. |  |
| DataMgmtOp | 6.1.2.6.3.3 | Represents the data management operation type. |  |
| DataPreparationRequirement | 6.1.2.6.2.4 | Represents the data preparation requirement. |  |
| DataAnalysisRequirement | 6.1.2.6.2.5 | Represents the data analysis requirement. |  |
| DataOperRequirement | 6.1.2.6.2.6 | Represents the data requirement. |  |

Table 6.1.2.6.1-2 specifies data types re-used by the AIMLES_DataManagement API service.

Table 6.1.2.6.1-2: AIMLES_DataManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleClientId | Clause 6.1.1.6.3.2 | Represents the unique identifier of an AIMLE Client. |  |
| DateTime | 3GPP TS 29.122 [2] | Used to indicate a timestamp. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |

##### 6.1.2.6.2	Structured data types

6.1.2.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.1.2.6.2.2	Type: DataMgmtAssistSubsc

Table 6.1.2.6.2.2-1: Definition of type DataMgmtAssistSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientList | array(AimleClientId) | C | 1..N | Contains the list of AIMLE client set identifier for data management operations. (NOTE 1) |  |
| selCriteria | FFS | C | 0..1 | Represents the selection criteria for finding suitable AIMLE clients for data management operations. (NOTE 1) |  |
| dataMgmtOp | DataMgmtOp | M | 1 | Contains the requested data management operation type. |  |
| dataPrepReqs | DataPrepReqs | C | 0..1 | Represents the requirement for the data preparation operation type. This attribute shall be present if the requested data management type is "DATA_PREPARATION". |  |
| dataAnalysisReqs | DataAnalysisReqs | C | 0..1 | Represents the requirement for the data analysis operation type. This attribute shall be present if the requested data management is "DATA_ANALYSIS". |  |
| notifUri | Uri | M | 1 | Indicates the URI towards which the notification should be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE 1: At least one of the "clientList" attribute and the "selCriteria" attribute shall be present. |  |  |  |  |  |

Editor’s NOTE: The content of the clientCriteria attribute is FFS.

6.1.2.6.2.3	Type: DataMgmtAssistNotif

Table 6.1.2.6.2.3-1: Definition of type DataMgmtAssistNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataMgmtOp | DataMgmtOp | M | 1 | Contains the received data management operation type. |  |
| aggregatedDataPrepOutputs | FFS | C | 0..1 | Contains the list of output for data preparation. This attribute shall be present if the received data management operation type is "DATA_PREPARATION". (NOTE) |  |
| aggregatedDataAnaOutputs | FFS | C | 0..1 | Contains the outputs for data analysis. This attribute shall be present if the requested data management operation type is "DATA_ANALYSIS". (NOTE) |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp of the data management notification. |  |
| NOTE: At least one of the "aggregatedDataPrepOutputs" attribute and the "aggregatedDataAnaOutputs" attribute shall be present. |  |  |  |  |  |

Editor’s NOTE: The content of the aggregatedDataPrepOutputs attribute and the aggregatedDataAnaOutputs attribute is FFS.

6.1.2.6.2.4	Type: DataPreparationRequirement

Table 6.1.2.6.2.4-1: Definition of type DataPreparationRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSetId | string | M |  | Contains the identifier of the dataset. |  |
| dataOperReqs | arra(DataOperRequirement) | M |  | Contains the requirements for data preparation.. |  |

6.1.2.6.2.5	Type: DataAnalysisRequirement

Table 6.1.2.6.2.4-1: Definition of type DataAnalysisRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSetId | string | M | 1 | Contains the identifier of the dataset. |  |
| dataOperRequirement | array(DataRequirement) | M | 1 | Contains the requirements for data analysis. |  |

6.1.2.6.2.6	Type: DataOperRequirement

Table 6.1.2.6.2.4-1: Definition of type DataOperRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataId | string | M | 1 | Contains the identifier of the dataset. |  |
| dataSetFeatureId | string | M | 1 | Contains the identifier of the data set feature. |  |
| dataOperationFunc | FFS | M | 1 | Contains the function that performs the data analysis. It may be an identifier of a function if the function is available locally at the UE or an executable included in the request. |  |

Editor’s NOTE: The content of the dataOperationFunc attribute is FFS.

6.1.2.6.2.7	Type: DataMgmtAssistSubscPatch

Table 6.1.2.6.2.2-1: Definition of type DataMgmtAssistSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataPrepReqs | DataPrepReqs | O | 0..1 | Represents the requirement for the data preparation operation type. |  |
| dataAnalysisReqs | DataAnalysisReqs | O | 0..1 | Represents the requirement for the data analysis operation type. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI towards which the notification should be delivered. |  |

##### 6.1.2.6.3	Simple data types and enumerations

6.1.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.2.6.3.2	Simple data types

The simple data types defined in table 6.1.2.6.3.2-1 shall be supported.

Table 6.1.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 6.1.2.6.3.3	Enumeration: DataMgmtOp

The enumeration DataMgmtOp represents the data management operation type. It shall comply with the provisions defined in table 6.1.2.6.3.3-1.

Table 6.1.2.6.3.3-1: Enumeration DataMgmtOp

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DATA_PREPARATION | Indicates that the data management operation type is data preparation. |  |
| DATA_ANALYSIS | Indicates that the data management operation type is data analysis. |  |

##### 6.1.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.2.6.5	Binary data

6.1.2.6.5.1	Binary Data Types

Table 6.1.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.2.7	Error Handling

##### 6.1.2.7.1	General

For the AIMLES_DataManagement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the Aimles_DataManagement API.

##### 6.1.2.7.2	Protocol Errors

No specific protocol errors for the AIMLES_DataManagement API are specified.

##### 6.1.2.7.3	Application Errors

The application errors defined for AIMLES_DataManagement API are listed in table 6.1.2.7.3-1.

Table 6.1.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.2.8	Feature negotiation

The optional features in table 6.1.2.8-1 are defined for the AIMLES_DataManagement API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.2.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_DataManagement API.

### 6.1.3	AIMLES_FLMemberGroupSupport API

#### 6.1.3.1	Introduction

The AIMLES_FLMemberGroupSupport Service shall use the AIMLES_FLMemberGroupSupport API.

The API URI of the AIMLES_FLMemberGroupSupport API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-fl".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.1.3.3 and clause 6.1.3.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.3, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.3.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_FLMemberGroupSupport API.

#### 6.1.3.3	Resources

##### 6.1.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.3.1-1 depicts the resource URIs structure for the AIMLES_FLMemberGroupSupport API.

Figure 6.1.3.3.1-1: Resource URI structure of the AIMLES_FLMemberGroupSupport API

Table 6.1.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| FL Member Group Support Configurations | /configurations | POST | Create a new Individual FL Member Support Group resource |
| Individual FL Member Group Support Configuration | /configurations/{configurationId} | GET | Retrieve an existing Individual FL Member Support Group resource. |
|  |  | PUT | Update an existing Individual FL Member Support Group resource. |
|  |  | PATCH | Modify an existing Individual FL Member Support Group resource. |
|  |  | DELETE | Delete an existing Individual FL Member Support Group resource. |

Editor's Note:	The HTTP GET method on "FL Member Group Support Configurations" resource collection is FFS.

##### 6.1.3.3.2	Resource: FL Member Group Support Configurations

6.1.3.3.2.1	Description

This resource represents the FL Member Group Support Configurations resource managed by the AILME Server.

6.1.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-fl/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 6.1.3.3.2.2-1.

Table 6.1.3.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.3.1 |

6.1.3.3.2.3	Resource Standard Methods

6.1.3.3.2.3.1	POST

The HTTP POST method enables the AIMLE service consumer to create a new Individual FL Member Support Group for an FL process at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.3.3.2.3.1-1.

Table 6.1.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.2.3.1-3.

Table 6.1.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | Create a new Individual FL Member Support Group for an FL process. |

Table 6.1.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | 201 Created | Successful case. The creation of the new Individual FL Member Support Group for the FL process is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-fl/<apiVersion>/configurations{configurationId} |

6.1.3.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.3.3.3	Resource: Individual FL Member Group Support Configuration

6.1.3.3.3.1	Description

This resource represents the individual FL Member Group Support Configuration resource managed by the AILME Server.

6.1.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-fl/<apiVersion>/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.3.1 |
| configurationId | string | Represents the identifier of the Individual FL Member Group Support Configuration resource. |

6.1.3.3.3.3	Resource Standard Methods

6.1.3.3.3.3.1	GET

The HTTP GET method enables the AIMLE service consumer to query an existing Individual FL Member Support Group for an FL process at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3.1-1.

Table 6.1.3.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.3.1-3.

Table 6.1.3.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | 200 OK | Successful case. The requested Individual FL Member Support Group for the FL process is confirmed and a representation of that resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.3.3.3.3.2	PUT

The HTTP PUT method enables the AIMLE service consumer to update an existing Individual FL Member Support Group for an FL process at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3.2-1.

Table 6.1.3.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.3.2-3.

Table 6.1.3.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | Represents the updated representation of an existing Individual FL Member Support Group for an FL process. |

Table 6.1.3.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | 200 OK | Successful case. The requested update of the existing Individual FL Member Support Group for the FL process is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested update of the existing Individual FL Member Support Group for the FL process is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.3.3.3.3.3	PATCH

The HTTP PATCH method enables the AIMLE service consumer to modify an existing Individual FL Member Support Group for an FL process at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3.3-1.

Table 6.1.3.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3.3-3.

Table 6.1.3.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbrSuppGrpPatch | M | 1 | Represents the parameters to modify of an existing Individual FL Member Support Group for an FL process. |

Table 6.1.3.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbrSuppGrp | M | 1 | 200 OK | Successful case. The requested modification of the existing Individual FL Member Support Group for the FL process is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested modification of the existing Individual FL Member Support Group for the FL process is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.3.3.3.3.4	DELETE

The HTTP DELETE method enables the AIMLE service consumer to delete an existing Individual FL Member Support Group for an FL process at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.3.3.3.3.4-1.

Table 6.1.3.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.3.4-3.

Table 6.1.3.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. Deletion of the existing Individual FL Member Support Group for the FL process is confirmed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.3.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.3.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.3.4	Custom Operations without associated resources

##### 6.1.3.4.1	Overview

There are no custom operations without associated resources defined for this resource in this release of the specification.

#### 6.1.3.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.1.3.6	Data Model

##### 6.1.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.3.6.1-1 specifies the data types defined for the AIMLES_FLMemberGroupSupport API.

Table 6.1.3.6.1-1: AIMLES_FLMemberGroupSupport API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| FlMbrSuppGrp | 6.1.3.6.2.2 | Create a new Individual FL Member Support Group for an FL process. |  |
| FlMbrSuppGrpPatch | 6.1.3.6.2.3 | Individual FL Member Support Group to be requested, replaced, modified, or deleted. |  |
| MlTaskInfo | 6.1.3.6.2.4 | Represents information on the identity and type of the ML task. |  |
| FlCandidateType | 6.1.3.6.2.5 | Represents information on candidate FL member. |  |
| MlTaskType | 6.1.3.6.3.3 | Represents information on the identity of the ML model. |  |
| FlMbrType | 6.1.3.6.3.4 | Represents information on the type of the FL member. |  |
| FlMbrStatus | 6.1.3.6.3.5 | Represents information on whether the FL member is a candidate or is selected. |  |

Table 6.1.3.6.1-2 specifies data types re-used by the AIMLES_FLMemberGroupSupport API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_FLMemberGroupSupport API.

Table 6.1.3.6.1-2: Aimles_FLMemberGroupSupport API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| FlGroupInfo | 3GPP TS 24.560 [16] | Created Individual FL Member Support Group for the FL process. |  |
| FlMemberData | 3GPP TS 24.560 [16] | FL member data e.g. FL member identifier, address. |  |
| SupportedFetaures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

##### 6.1.3.6.2	Structured data types

6.1.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.3.6.2.2	Type: FlMbrSuppGrp

Table 6.1.3.6.2.2-1: Definition of type FlMbrSuppGrp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | C | 0..1 | Identifies the VAL Service, for which the request applies. (NOTE) |  |
| mlModelId | string | C | 0..1 | Identifies the ML model, for which the request applies. (NOTE) |  |
| adaeAnalyticsId | string | C | 0..1 | Identifies the ADAE analytics, for which the request applies. (NOTE) |  |
| mlTask | MlTaskInfo | O | 0..1 | Identifies the ML task, for which the request applies. |  |
| mlModelProfile | TBD | O | 0..1 | Identifies the ML model profile, for which the Individual FL Member Support Group is requested. |  |
| flCandidates | FlCandidateType | O | 0..N | Identifies one or more selected candidates for the Individual FL Member Support Group. |  |
| flMbrSuppGrp | FlGroupInfo | M | 1..N | Identifies the created Individual FL Member Support Group for the FL process. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.3.8. |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

Editor's Note:	Data types for mlModelProfile is FFS and is aligned with ML model management as defined in 3GPP TS 23.482 [13].

6.1.3.6.2.3	Type: FlMbrSuppGrpPatch

Table 6.1.3.6.2.3-1: Definition of type FlMbrSuppGrpPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | O | 0..1 | Identifies the VAL Service, for which the request applies. |  |
| mlModelId | string | O | 0..1 | Identifies the ML model, for which the request applies. |  |
| adaeAnalyticsId | string | O | 0..1 | Identifies the ADAE analytics, for which the request applies. |  |
| mlTask | MlTaskInfo | O | 0..1 | Identifies the ML task, for which the request applies. |  |
| mlModelProfile | TBD | O | 0..1 | Identifies the ML model profile, for which the Individual FL Member Support Group is requested. |  |
| flCandidates | FlCandidateType | O | 0..N | Identifies one or more selected candidates for the Individual FL Member Support Group. |  |

Editor's Note:	Data types for mlModelProfile is FFS and is aligned with ML model management as defined in 3GPP TS 23.482 [13].

6.1.3.6.2.4	Type: MlTaskInfo

Table 6.1.3.6.2.4-1: Definition of type MlTaskInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlTaskId | string | M | 1 | Identity of the ML Task. |  |
| mlTaskType | MlTaskType | M | 1 | Identifies the type of the ML Task. |  |

6.1.3.6.2.5	Type: FlCandidateType

Table 6.1.3.6.2.5-1: Definition of type FlCandidateType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrId | string | M | 1 | Identifies the FL member which may be an FL client or FL server. |  |
| flMbrType | FlMbrType | M | 1 | Identifies the type of the FL member. |  |
| flMbrStatus | FlMbrStatus | M | 1 | Identifies the status of the FL member. |  |

##### 6.1.3.6.3	Simple data types and enumerations

6.1.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.3.6.3.2	Simple data types

The simple data types defined in table 6.1.3.6.3.2-1 shall be supported.

Table 6.1.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.3.6.3.3	Enumeration: MlTaskType

The enumeration MlTaskType represents information regarding the training identity or inference identity of the ML model. It shall comply with the provisions defined in table 6.1.3.6.3.3-1.

Table 6.1.3.6.3.3-1: Enumeration MlTaskType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_TRAINING | Identifies an ML model training |  |
| FL_INFERENCE | Identifies an ML model inference |  |

6.1.3.6.3.4	Enumeration: FlMbrType

The enumeration FlMbrType represents information regarding the FL member being an FL server or an FL client. It shall comply with the provisions defined in table 6.1.3.6.3.4-1.

Table 6.1.3.6.3.4-1: Enumeration FlMbrType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_SERVER | Identifies an FL server |  |
| FL_CLIENT | Identifies an FL client |  |

6.1.3.6.3.5	Enumeration: FlMbrStatus

The enumeration FlMbrStatus represents information regarding the FL member being a candidate or being selected. It shall comply with the provisions defined in table 6.1.3.6.3.5-1.

Table 6.1.3.6.3.5-1: Enumeration FlMbrStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_CANDIDATE | Identifies an FL member being a candidate |  |
| FL_SELECTED | Identifies an FL member being selected |  |

##### 6.1.3.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combination of data types for AIMLES_FLMemberGroupSupport API in this release of the specification.

##### 6.1.3.6.5	Binary data

6.1.3.6.5.1	Binary Data Types

The binary data types defined for the AIMLES_FLMemberGroupSupport API are listed in Table 6.1.3.6.5.1-1.

Table 6.1.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.3.7	Error Handling

##### 6.1.3.7.1	General

For the AIMLES_FLMemberGroupSupport API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_FLMemberGroupSupport API.

##### 6.1.3.7.2	Protocol Errors

No specific procedures for the AIMLES_FLMemberGroupSupport API are specified.

##### 6.1.3.7.3	Application Errors

The application errors defined for the AIMLES_FLMemberGroupSupport API are listed in Table 6.1.3.7.3-1.

Table 6.1.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

#### 6.1.3.8	Feature negotiation

The optional features in table 6.1.3.8-1 are defined for the AIMLES_FLMemberGroupSupport API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.3.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_FLMemberGroupSupport API.

### 6.1.4	AIMLES_AIMLEServiceOperationsManagement API

#### 6.1.4.1	Introduction

The AIMLES_AIMLEServiceOperationsManagement service shall use the AIMLES_AIMLEServiceOperationsManagement API.

The API URI of the AIMLES_AIMLEServiceOperationsManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-opm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.1.4.3 and 6.1.4.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.4, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.4.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AIMLEServiceOperationsManagement API.

#### 6.1.4.3	Resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 6.1.4.4	Custom Operations without associated resources

##### 6.1.4.4.1	Overview

Table 6.1.4.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestServOpMngt | /request | POST | Request AIMLE service operations management. |

##### 6.1.4.4.2	Operation: RequestServOpMngt

6.1.4.4.2.1	Description

The custom operation enables a service consumer to send AIMLE service operations control and management request to the AIMLE Server.

6.1.4.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.4.4.2.2-1 and 6.1.4.4.2.2-2.

Table 6.1.4.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimlServOperReq | M | 1 | Contains the parameters to request AIMLE service operations control and management. |

Table 6.1.4.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimlServOperResp | M | 1 | 200 OK | Successful case. The AIMLE service operations control and management request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.4.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

Table 6.1.4.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

#### 6.1.4.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.1.4.6	Data Model

##### 6.1.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.4.6.1-1 specifies the data types defined for the AIMLES_AIMLEServiceOperationsManagement API.

Table 6.1.4.6.1-1: AIMLES_AIMLEServiceOperationsManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleConfigMode | Clause 6.1.4.6.2.7 | Represent the AIMLE service operation configuration mode information. |  |
| AimleOperAssist | Clause 6.1.4.6.2.6 | Represent the AIMLE service operation assistance information. |  |
| AimleOperId | Clause 6.1.4.6.2.4 | Represents the AIMLE service operation identifier. |  |
| AimleOperInfo | Clause 6.1.4.6.2.5 | Represents the AIMLE service operation information. |  |
| AimleOperMode | Clause 6.1.4.6.3.3 | Represents the AIMLE service operation mode. |  |
| AimlServOperReq | Clause 6.1.4.6.2.2 | Represents the AIMLE service operations control and management request. |  |
| AimlServOperResp | Clause 6.1.4.6.3.3 | Represents the AIMLE service operations control and management response. |  |
| AimleServStatus | Clause 6.1.4.6.2.3 | Represents the AIMLE service status information. |  |
| MeasurementData | Clause 6.1.4.6.2.8 | Represent the measurement data for the AIMLE service operation. |  |

Table 6.1.4.6.1-2 specifies data types re-used by the AIMLES_AIMLEServiceOperationsManagement API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_AIMLEServiceOperationsManagement API.

Table 6.1.4.6.1-2: AIMLES_AIMLEServiceOperationsManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

##### 6.1.4.6.2	Structured data types

6.1.4.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.4.6.2.2	Type: AimlServOperReq

Table 6.1.4.6.2.2-1: Definition of type AimlServOperReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 1 | Contains the VAL service identifier. |  |
| clients | array(string) | C | 0..1 | Contains the list of AIMLE Client IDs. (NOTE) |  |
| setId | string | C | 0..1 | Contains the AIMLE Client set ID. (NOTE) |  |
| operId | AimleOperId | O | 0..1 | Contains AIMLE operation ID. |  |
| operInfo | AimleOperInfo | O | 0..1 | Contains AIMLE operation information. |  |
| operMode | AimleOperMode | M | 1 | Contains AIMLE operation mode, |  |
| confModes | array(AimleConfigMode) | O | 1..N | Contains AIMLE operation configuration modes. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

6.1.4.6.2.3	Type: AimlServOperResp

Table 6.1.4.6.2.3-1: Definition of type AimlServOperResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 1 | Contains the VAL service identifier. |  |
| operId | AimleOperId | M | 1 | Contains AIMLE operation ID. |  |
| operMode | AimleOperMode | M | 1 | Contains AIMLE operation status. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.4.6.2.4	Type: AimleOperId

Table 6.1.4.6.2.4-1: Definition of type AimleOperId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainingId | string | C | 0..1 | Contains the ML model training ID. (NOTE) |  |
| taskId | string | C | 0..1 | Contains the ML taks ID. (NOTE) |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

6.1.4.6.2.5	Type: AimleOperInfo

Table 6.1.4.6.2.5-1: Definition of type AimleOperInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| container | string | M | 1 | Contains the AIMLE service operation container. |  |
| fetchUri | Uri | M | 1 | Contains the URI to fetch model from a repository. |  |
| agrUri | Uri | M | 1 | Contains the AIMLE service aggregator URI to send model updates. |  |
| assitInfo | AimleOperAssist | M | 1 | Represent the AIMLE service operation assistance information. |  |

6.1.4.6.2.6	Type: AimleOperAssist

Table 6.1.4.6.2.6-1: Definition of type AimleOperAssist

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxConvTime | DurationSec | M | 1 | Contains the maximum convergence time. |  |

6.1.4.6.2.7	Type: AimleConfigMode

Table 6.1.4.6.2.7-1: Definition of type AimleConfigMode

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mode | AimleOperMode | M | 1 | Indicated the requiered mode to be activiated if the conditions are matched. |  |
| measThrValues | MeasurementData | M | 1 | Indicates the value(s) for the measurement threshold index(es). |  |
| thrDirection | MatchingDirection | M | 1 | Indicates the threshold matching direction for the measurement threshold index(es) provided in the "measThrValues" attribute. |  |

6.1.4.6.2.8	Type: MeasurementData

Table 6.1.4.6.2.8-1: Definition of type MeasurementData

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| latency | Uinteger | O | 0..1 | Indicates the latency in milliseconds. |  |
| time | DateTime | O | 0..1 | Indicates the time limit. |  |
| accuracy | integer | O | 0..1 | Indicates the accuracy. Min: 0, Max: 100 |  |

##### 6.1.4.6.3	Simple data types and enumerations

6.1.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.4.6.3.2	Simple data types

The simple data types defined in table 6.1.4.6.3.2-1 shall be supported.

Table 6.1.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.1.4.6.3.3	Enumeration: AimleOperMode

Table 8.1.0.6.3-1: Enumeration AimleOperMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| START | Indicates the AIMLE service operation mode is start. |  |
| STOP | Indicates the AIMLE service operation mode is stop. |  |

##### 6.1.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.4.6.5	Binary data

6.1.4.6.5.1	Binary Data Types

Table 6.1.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.4.7	Error Handling

##### 6.1.4.7.1	General

For the AIMLES_AIMLEServiceOperationsManagement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_AIMLEServiceOperationsManagement API.

##### 6.1.4.7.2	Protocol Errors

No specific protocol errors for the AIMLES_AIMLEServiceOperationsManagement API are specified.

##### 6.1.4.7.3	Application Errors

The application errors defined for the AIMLES_AIMLEServiceOperationsManagement API are listed in Table 6.1.4.7.3-1.

Table 6.1.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.4.8	Feature negotiation

The optional features in table 6.1.4.8-1 are defined for the AIMLES_AIMLEServiceOperationsManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.4.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AIMLEServiceOperationsManagement API.

### 6.1.5	AIMLES_HierarchicalComputingAssist API

#### 6.1.5.1	Introduction

The AIMLES_HierarchicalComputingAssist service shall use the AIMLES_HierarchicalComputingAssist API.

The API URI of the AIMLES_HierarchicalComputingAssist API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-hca".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.1.5.3 and 6.1.5.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.5, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.5.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_HierarchicalComputingAssist API.

#### 6.1.5.3	Resources

There are no resources defined for this API in this release of the specification.

#### 6.1.5.4	Custom Operations without associated resources

##### 6.1.5.4.1	Overview

Table 6.1.5.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestAssistance | /assist | POST | Request the hierarchical computing assistance. |

##### 6.1.5.4.2	Operation: RequestServOpMngt

6.1.5.4.2.1	Description

The custom operation enables a service consumer to send AIMLE hierarchical computing assist request to the AIMLE Server.

6.1.5.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.5.4.2.2-1 and 6.1.5.4.2.2-2.

Table 6.1.5.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimlHierAssitReq | M | 1 | Contains the parameters to request AIMLE hierarchical computing assist. |

Table 6.1.5.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimlHierAssitResp | M | 1 | 200 OK | Successful case. The AIMLE hierarchical computing assist request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.5.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

Table 6.1.5.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

#### 6.1.5.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.1.5.6	Data Model

##### 6.1.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.5.6.1-1 specifies the data types defined for the AIMLES_HierarchicalComputingAssist API.

Table 6.1.5.6.1-1: AIMLES_HierarchicalComputingAssist API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlHierAssitReq | Clause 6.1.5.6.2.2 | Represents the AIMLE hierarchical computing assist request. |  |
| AimlHierAssitResp | Clause 6.1.5.6.3.3 | Represents the AIMLE hierarchical computing assist response. |  |
| AssitInfo | Clause 6.1.5.6.2.5 | Represents the assistance information. |  |
| ExecutionNode | Clause 6.1.5.6.2.6 | Represents the execution node information. |  |
| RequestorId | Clause 6.1.5.6.2.4 | Represents the AIMLE service operation identifier. |  |
| Role | Clause 6.1.5.6.3.3 | Represents the role of the node. |  |
| TaskType | Clause 6.1.5.6.3.4 | Represents the type of task. |  |

Table 6.1.5.6.1-2 specifies data types re-used by the AIMLES_HierarchicalComputingAssist API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the AIMLES_HierarchicalComputingAssist API.

Table 6.1.5.6.1-2: AIMLES_HierarchicalComputingAssist API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| EasAnalytics | 3GPP TS 29.549 [13] | Represents the EAS status. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of optional features. |  |

##### 6.1.5.6.2	Structured data types

6.1.5.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.5.6.2.2	Type: AimlHierAssitReq

Table 6.1.5.6.2.2-1: Definition of type AimlHierAssitReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| origReqId | RequestorId | O | 0..1 | Contains the original requestor identifier. |  |
| role | Role | M | 1 | Contains the role of the VAL server in the hiearchitical computing architecture. |  |
| task | TaskType | M | 1 | Contains the type of computing task. |  |
| assistSets | array(AssitInfo) | M | 1..N | Contains the assistance information. |  |
| exeNodes | array(ExecutionNode) | O | 1..N | Contains the execution nodes information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.5.6.2.3	Type: AimlHierAssitResp

Table 6.1.5.6.2.3-1: Definition of type AimlHierAssitResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| assistSets | array(AssitInfo) | M | 1..N | Contains the assistance information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.5.6.2.4	Type: RequestorId

Table 6.1.5.6.2.4-1: Definition of type RequestorId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | C | 0..1 | Contains the VAL Server ID. (NOTE) |  |
| easId | string | C | 0..1 | Contains the EAS ID. (NOTE) |  |
| casId | string | C | 0..1 | Contains the CAS ID. (NOTE) |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

6.1.5.6.2.5	Type: AssitInfo

Table 6.1.5.6.2.5-1: Definition of type AssitInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Contains the EAS identifier. |  |
| status | EasAnalytics | M | 1 | Contains the EAS status. |  |

6.1.5.6.2.6	Type: ExecutionNode

Table 6.1.5.6.2.6-1: Definition of type ExecutionNode

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | O | 0..1 | Contains the EAS ID. |  |

##### 6.1.5.6.3	Simple data types and enumerations

6.1.5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.5.6.3.2	Simple data types

The simple data types defined in table 6.1.5.6.3.2-1 shall be supported.

Table 6.1.5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

6.1.5.6.3.3	Enumeration: Role

Table 8.1.0.6.3-1: Enumeration Role

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ROOT_NODE | Indicates the role is root node. |  |
| SUB_ROOT_NODE | Indicates the role is sub root node. |  |
| LEAF_NODE | Indicates the role is leaf node. |  |

6.1.5.6.3.4	Enumeration: TaskType

Table 8.1.0.6.4-1: Enumeration TaskType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VFL | Indicates the task is VFL. |  |
| HFL | Indicates the task is HFL. |  |

##### 6.1.5.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.5.6.5	Binary data

6.1.5.6.5.1	Binary Data Types

Table 6.1.5.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.5.7	Error Handling

##### 6.1.5.7.1	General

For the AIMLES_HierarchicalComputingAssist API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_HierarchicalComputingAssist API.

##### 6.1.5.7.2	Protocol Errors

No specific protocol errors for the AIMLES_HierarchicalComputingAssist API are specified.

##### 6.1.5.7.3	Application Errors

The application errors defined for the AIMLES_HierarchicalComputingAssist API are listed in Table 6.1.5.7.3-1.

Table 6.1.5.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.5.8	Feature negotiation

The optional features in table 6.1.5.8-1 are defined for the AIMLES_HierarchicalComputingAssist API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.5.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_HierarchicalComputingAssist API.

### 6.1.6	AIMLES_AIMLEClientDiscovery API

#### 6.1.6.1	Introduction

The AIMLES_AIMLEClientDiscovery service shall use the AIMLES_AIMLEClientDiscovery API.

The API URI of the AIMLES_AIMLEClientDiscovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "aimles-disc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (e.g. AIMLE Server) takes the role of the SCEF and the service consumer (i.e. AIMLE service consumer, e.g. VAL server) takes the role of the SCS/AS.

#### 6.1.6.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2.1 of 3GPP TS 29.122 [2] shall apply for the AIMLES_AIMLEClientDiscovery API.

#### 6.1.6.3	Resources

##### 6.1.6.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.6.3.1-1 depicts the resource URIs structure for the AIMLES_AIMLEClientDiscovery Service API.

Figure 6.1.6.3.1-1: Resource URI structure of the AIMLES_AIMLEClientDiscovery Service API

Table 6.1.6.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.6.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Clients | /clients | GET | Discover the AIMLE Clients according to the filtering crieria. |

##### 6.1.6.3.2	Resource: AIMLE Clients

###### 6.1.6.3.2.1	Description

The "AIMLE Clients" resource represents the AIMLE Clients.

###### 6.1.6.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-disc/<apiVersion>/clients

This resource shall support the resource URI variables defined in the table 6.1.6.3.2.2-1.

Table 6.1.6.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [14]. |

###### 6.1.6.3.2.3	Resource Standard Methods

6.1.6.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.6.3.2.3.1-1.

Table 6.1.6.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| filt-criteria | ClientDiscCriteria | M | 1 | Represents the AIMLE Client(s) filtering criteria. |
| clNumber | Uinteger | O | 0..1 | Represents the required number of the discovered AIMLE Clients. |
| supported-features | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |

This method shall support the request data structures specified in table 6.1.6.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.6.3.2.3.1-3.

Table 6.1.6.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.6.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientDiscResp | M | 1 | 200 OK | Successful case. The response body contains the result of the search over the list of AIMLE Clients. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.6.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.6.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

###### 6.1.6.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.2.4	Custom Operations without associated resources

There are no custom operations without associated resources in the present release of the document.

#### 6.2.2.5	Notifications

There are no notifications in the present release of the document.

#### 6.2.2.6	Data Model

##### 6.2.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.2.6.1-1 specifies the data types defined for the AIMLES_AIMLEClientDiscovery API.

Table 6.2.2.6.1-1: AIMLES_AIMLEClientDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlOperationRole | 6.2.2.6.3.5 | Represents the AIMLE operation role. |  |
| ClientTaskCapability | 6.2.2.6.2.8 | Represents the AIMLE Client task capability information. |  |
| ClientVelocity | 6.2.2.6.3.10 | Represents the AIMLE Client velocity level. |  |
| ComputeCapability | 6.2.2.6.3.9 | Represents the AIMLE Client compute capability type. |  |
| DataType | 6.2.2.6.3.7 | Represents the type of the collected data. |  |
| DatasetAvailablity | 6.2.2.6.2.6 | Represents the dataset availability information. |  |
| DatasetCapability | 6.2.2.6.2.7 | Represents the dataset capability informatiion. |  |
| DatasetRequirement | 6.2.2.6.2.5 | Represents the dataset requirements. |  |
| ClientAppCapability | 6.2.2.6.2.4 | Represents the AIMLE Client Application layer capabilities. |  |
| ClientDiscCriteria | 6.2.2.6.2.2 | Represents the AIMLE Client discovery criteria. |  |
| ClientDiscResp | 6.2.2.6.2.9 | Represents the AIMLE Client discovery response. |  |
| MlAppType | 6.2.2.6.3.6 | Represents the ML application type. |  |
| MlModelType | 6.2.2.6.3.4 | Represents the ML model type. |  |
| PerformanceCapability | 6.2.2.6.3.8 | Represents the performance capabilities. |  |
| ServicePermLevel | 6.2.2.6.3.3 | Represents the service permision level. |  |
| ServiceRequirement | 6.2.2.6.2.3 | Represents the AIMLE service requirements. |  |

Table 6.2.2.6.1-2 specifies data types re-used by the AIMLES_AIMLEClientDiscovery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_AIMLEClientDiscovery API.

Table 6.2.2.6.1-2: AIMLES_AIMLEClientDiscovery API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| LocationArea5G | 3GPP TS 29.122 [2] | Represents the location. |  |
| TransQualMeasCriteriaSet | 3GPP TS 29.548 [A] | Represents QoS criteria. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the supported features, used to negotiate the supported optional features of the API. |  |

##### 6.2.2.6.2	Structured data types

###### 6.2.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.2.2.6.2.2	Type: ClientDiscCriteria

Table 6.2.2.6.2.2-1: Definition of type ClientDiscCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceReq | ServiceRequirement | M | 1 | Represents the service requirements. |  |
| mlModelTypes | array(MlModelType) | O | 1..N | Represents the requested ML model types. |  |
| aimlOpers | array(AimlOperationRole) | M | 1..N | Represents the roles for AI/ML operations. |  |
| clientAppCap | ClientAppCapability | M | 1 | Represents the AIMLE Client application layer capabilities. |  |
| datasetReq | DatasetRequirement | O | 0..1 | Represents the dataset requirements. |  |
| clientTaskCap | ClientTaskCapability | O | 0..1 | Represents the AIMLE Client task capabilities. |  |
| clientVel | ClientVelocity | O | 0..1 | Represents the AIMLE Client velocity. |  |
| location | LocationArea5G | O | 0..1 | Represents the location information for the AIMLE Client discovery. |  |
| clientQosReqs | array(TransQualMeasCriteriaSet) | O | 1..N | Represents the requirements on AIMLE Client QoS for discovery. |  |

###### 6.2.2.6.2.3	Type: ServiceRequirement

Table 6.2.2.6.2.3-1: Definition of type ServiceRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | M | 1 | Represents the VAL Service identifier. |  |
| permLevel | ServicePermLevel | O | 0..1 | Represents the service permission level. |  |

###### 6.2.2.6.2.4	Type: ClientAppCapability

Table 6.2.2.6.2.4-1: Definition of type ClientAppCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appType | MlAppType | M | 1 | Represents the ML application identifier. |  |
| avail | array(ScheduledCommunicationTime) | O | 1..N | Represents the availablility of AIMLE Client to support opertations. |  |
| dropOfRate | integer | O | 0..1 | Represents the drop off rate in percent. Minumum: 0 Maximum: 100 |  |

###### 6.2.2.6.2.5	Type: DatasetRequirement

Table 6.2.2.6.2.5-1: Definition of type DatasetRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| avail | DatasetAvailablity | O | 0..1 | Represents the dataset availablity information. |  |
| cap | DatasetCapability | O | 0..1 | Represents the dataset capability information. |  |

###### 6.2.2.6.2.6	Type: DatasetAvailablity

Table 6.2.2.6.2.6-1: Definition of type DatasetAvailablity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ids | array(string) | O | 1..N | Represents the list dataset identifiers. |  |
| age | DateTime | O | 0..1 | Represents the age of the dataset, i.e., when the data set was created. |  |
| size | Uinteger | O | 0..1 | Represents the dataset size in bytes. |  |
| features | array(string) | O | 1..N | Represents the list of dataset features identifiers. |  |

###### 6.2.2.6.2.7	Type: DatasetCapability

Table 6.2.2.6.2.7-1: Definition of type DatasetCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataType | DataType | O | 0..1 | Represents the type of data. |  |
| functions | array(string) | O | 1..N | Represents the list of the exploratory data analysis functions. |  |

###### 6.2.2.6.2.8	Type: ClientTaskCapability

Table 6.2.2.6.2.8-1: Definition of type ClientTaskCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| compCap | ComputeCapability | O | 0..1 | Represents the compute capabilities. |  |
| perfCap | PerformanceCapability | O | 0..1 | Represents the performance capabilities. |  |

###### 6.2.2.6.2.9	Type: ClientDiscResp

Table 6.2.2.6.2.9-1: Definition of type ClientDiscResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clients | array(string) | M | 1..N | Represents the AIMLE client identifiers that matches the discovery criteria. |  |
| suppTasks | array(ClientTaskCapability) | O | 1..N | Represents the supported AIML tasks. |  |
| suppFeats | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |  |

##### 6.2.2.6.3	Simple data types and enumerations

###### 6.2.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.2.2.6.3.2	Simple data types

None.

6.2.2.6.3.3	Enumeration: ServicePermLevel

The enumeration ServicePermLevel represents the service permission level requested. It shall comply with the provisions defined in table 6.2.2.6.3.3-1.

Table 6.2.2.6.3.3-1: Enumeration ServicePermLevel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PREMIUM | Indicates that the service permission level is premium. |  |
| STANDARD | Indicates that the service permission level is standard. |  |
| LIMITED | Indicates that the service permission level is limited. |  |

6.2.2.6.3.4	Enumeration: MlModelType

The enumeration MlModelType represents the ML model type requested. It shall comply with the provisions defined in table 6.2.2.6.3.4-1.

Table 6.2.2.6.3.4-1: Enumeration MlModelType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DECISION_TREES | Indicates that the ML model type is decision trees. |  |
| LINEAR_REGRESSION | Indicates that the ML model type is linear regression. |  |
| NEURAL_NETWORKS | Indicates that the ML model type is neural networks. |  |

6.2.2.6.3.5	Enumeration: AimlOperationRole

The enumeration AimlOperationRole represents the AIML operation role requested. It shall comply with the provisions defined in table 6.2.2.6.3.5-1.

Table 6.2.2.6.3.5-1: Enumeration AimlOperationRole

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MODEL_TRAINING | Indicates that the supported AIML operation role is model training. |  |
| MODEL_TRANSFER | Indicates that the supported AIML operation role is model transfer. |  |
| MODEL_INFERENCE | Indicates that the supported AIML operation role is model inference. |  |
| MODEL_OFFLOAD | Indicates that the supported AIML operation role is model offload. |  |
| MODEL_SPLIT | Indicates that the supported AIML operation role is model split. |  |

6.2.2.6.3.6	Enumeration: MlAppType

The enumeration MlAppType represents the ML application type requested. It shall comply with the provisions defined in table 6.2.2.6.3.6-1.

Table 6.2.2.6.3.6-1: Enumeration MlAppType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL | Indicates that the supported ML application type is Federated Learning. |  |
| TL | Indicates that the supported ML application type is Transfer Learning. |  |
| SL | Indicates that the supported ML application type is Split Learning. |  |

6.2.2.6.3.7	Enumeration: DataType

The enumeration DataType represents the type of collected data requested. It shall comply with the provisions defined in table 6.2.2.6.3.7-1.

Table 6.2.2.6.3.7-1: Enumeration DataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| RAW | Indicates that the type of the collected data is raw. |  |
| PROCESSED | Indicates that the type of the collected data is processed. |  |

6.2.2.6.3.8	Enumeration: PerformanceCapability

The enumeration PerformanceCapability represents the performance capability requested. It shall comply with the provisions defined in table 6.2.2.6.3.8-1.

Table 6.2.2.6.3.8-1: Enumeration PerformanceCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GREEN_TASK | Indicates that the performance capability is green task. |  |
| ENERGY_EFFICIENT | Indicates that the performance capability is energy-efficient. |  |
| LOW_COSTS | Indicates that the performance capability is low costs. |  |

6.2.2.6.3.9	Enumeration: ComputeCapability

The enumeration ComputeCapability represents the compute capability requested. It shall comply with the provisions defined in table 6.2.2.6.3.9-1.

Table 6.2.2.6.3.9-1: Enumeration ComputeCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOW | Indicates that the compute capability is low. |  |
| HIGH | Indicates that the compute capability is high. |  |

6.2.2.6.3.10	Enumeration: ClientVelocity

The enumeration ClientVelocity represents the AIMLE Client velocity requested. It shall comply with the provisions defined in table 6.2.2.6.3.10-1.

Table 6.2.2.6.3.10-1: Enumeration ClientVelocity

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOW | Indicates that the AIMLE Client velocity is low. |  |
| HIGH | Indicates that the AIMLE Client velocity is high. |  |

##### 6.2.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types and combinations of data types in this release of the specification.

##### 6.2.2.6.5	Binary data

There are no binary data defined in this release of the specification.

#### 6.2.2.7	Error Handling

##### 6.2.2.7.1	General

For the AIMLES_AIMLEClientDiscovery API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_AIMLEClientDiscovery API.

##### 6.2.2.7.2	Protocol Errors

No specific procedures for the AIMLES_AIMLEClientDiscovery API are specified.

##### 6.2.2.7.3	Application Errors

The application errors defined for AIMLES_AIMLEClientDiscovery API are listed in table 6.2.2.7.3-1.

Table 6.2.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.2.8	Feature Negotiation

The optional features in table 6.2.2.8-1 are defined for the AIMLES_AIMLEClientDiscovery API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.2.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.2.2.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AIMLEClientDiscovery API.

### 6.1.7	AIMLES_AIMLEClientSelection API

#### 6.1.7.1	Introduction

The AIMLES_AIMLEClientSelection service shall use the AIMLES_AIMLEClientSelection API.

The API URI of the AIMLES_AIMLEClientSelection API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "aimles-sel".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (e.g. AIMLE Server) takes the role of the SCEF and the service consumer (i.e. AIMLE service consumer, e.g. VAL server) takes the role of the SCS/AS.

#### 6.1.7.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2.1 of 3GPP TS 29.122 [2] shall apply for the AIMLES_AIMLEClientSelection API.

#### 6.1.7.3	Resources

##### 6.1.7.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.7.3.1-1 depicts the resource URIs structure for the AIMLES_AIMLEClientSelection API.

Figure 6.1.7.3.1-1: Resource URI structure of the AIMLES_AIMLEClientSelection API

Table 6.1.7.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.7.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Client Selection Subscriptions | /subscriptions | POST | Create a new "Individual AIMLE Client Selection Subscription" resource. |
| Individual AIMLE Client Selection Subscription | /subscriptions/{subscriptionId} | GET | Read "Individual AIMLE Client Selection Subscription" resource. |
|  |  | PUT | Update "Individual AIMLE Client Selection Subscription" resource. |
|  |  | PATCH | Modify "Individual AIMLE Client Selection Subscription" resource. |
|  |  | DELETE | Delete "Individual AIMLE Client Selection Subscription" resource. |

##### 6.1.7.3.2	Resource: AIMLE Client Selection Subscriptions

6.1.7.3.2.1	Description

This resource represents the collection of AIMLE Client Selection Subscriptions managed by the AIMLE Server.

6.1.7.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-sel/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.7.3.2.2-1.

Table 6.1.7.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.7.1. |

6.1.7.3.2.3	Resource Standard Methods

6.1.7.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual AIMLE Client Selection Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.7.3.2.3.1-1.

Table 6.1.7.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.7.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.7.3.2.3.1-3.

Table 6.1.7.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ClientSelSub | M | 1 | Create a new Individual AIMLE Client Selection Subscription resource. |

Table 6.1.7.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientSelSub | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE Client Selection Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.7.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-sel/<apiVersion>/subscriptions{subscriptionId} |

6.1.7.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.7.3.3	Resource: Individual AIMLE Client Selection Subscription

6.1.7.3.3.1	Description

6.1.7.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-sel/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.7.3.3.2-1.

Table 6.1.7.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.7.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE Client Selection Subscription" resource. |

6.1.7.3.3.3	Resource Standard Methods

6.1.7.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual AIMLE Client Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.7.3.3.3.1-1.

Table 6.1.7.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.7.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.7.3.3.3.1-3.

Table 6.1.7.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.7.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientSelSub | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE Client Selection Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.7.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.7.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.7.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual AIMLE Client Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.7.3.3.3.2-1.

Table 6.1.7.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.7.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.7.3.3.3.2-3.

Table 6.1.7.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ClientSelSub | M | 1 | Represents the updated representation of the "Individual AIMLE Client Selection Subscription" resource. |

Table 6.1.7.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientSelSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE Client Selection Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Client Selection Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.7.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.7.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.7.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual AIMLE Client Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.7.3.3.3.3-1.

Table 6.1.7.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.7.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.7.3.3.3.3-3.

Table 6.1.7.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ClientSelSubPatch | M | 1 | Represents the parameters to request the modification of the "Individual AIMLE Client Selection Subscription" resource. |

Table 6.1.7.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientSelSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE Client Selection Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Client Selection Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.7.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.7.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.7.3.3.3.4	DELETE

The HTTP DELETE method allows a AIMLE service consumer to request the deletion of an existing "Individual AIMLE Client Selection Subscription" resource.

Table 6.1.7.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.7.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.7.3.3.3.4-3.

Table 6.1.7.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.7.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Client Selection Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.7.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.7.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.7.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.7.4	Custom Operations without associated resources

##### 6.1.7.4.1	Overview

Table 6.1.7.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| /select | POST | Select AIMLE Clients according to the provided criteria. |

##### 6.1.7.4.2	Operation: Select

6.1.7.4.2.1	Description

The custom operation enables a service consumer to select AIMLE Clients according to the provided criteria.

6.1.7.4.2.2	Operation Definition

This operation shall support the request data structures and the response data structures and response codes specified in tables 6.1.7.4.2.2-1 and 6.1.7.4.2.2-2.

Table 6.1.7.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SelectReq | M | 1 | Contains the parameters to request AIMLE Client selection. |

Table 6.1.7.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SelectResp | M | 1 | 200 OK | Successful case. The AIMLE Client selection request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.7.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

Table 6.1.7.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE Server. |

#### 6.1.7.5	Notifications

##### 6.1.7.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.7.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Client Selection Event Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed service consumer on AIMLE Client selection related event(s). |

##### 6.1.7.5.2	AIMLE Client Selection Event Notification

6.1.7.5.2.1	Description

The AIMLE Client Selection Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIMLE Client selection related event(s).

6.1.7.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.7.5.2.2-1.

Table 6.1.7.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The notification Uri is assigned within the Individual AIMLE Client Selection Subscription and described within the ClientSelSub type |

6.1.7.5.2.3	Standard Methods

6.1.7.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.7.5.2.3-1 and the response data structures and response codes specified in table 6.1.7.5.2.3-2.

Table 6.1.7.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ClientSelNotif | M | 1 | Represents the AIMLE Client Selection Event Notification. |

Table 6.1.7.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Client Selection Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.7.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.7.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.7.6	Data Model

##### 6.1.7.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.7.6.1-1 specifies the data types defined for the AIMLES_AIMLEClientSelection API.

Table 6.1.7.6.1-1: AIMLES_AIMLEClientSelection API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ClientSelSub | 6.1.7.6.2.2 | Represents the AIMLE Client selection subscription information. |  |
| ClientSelSubPatch | 6.1.7.6.2.4 | Represents the requested modifications to a AIMLE Client selection subscription information. |  |
| ClientSelNotif | 6.1.7.6.2.3 | Represents the AIMLE Client selection notification. |  |
| SelectReq | 6.1.7.6.2.5 | Represents the AIMLE Client selection request. |  |
| SelectResp | 6.1.7.6.2.6 | Represents the AIMLE Client selection response. |  |
| SelUpdate | 6.1.7.6.2.7 | Represents the update of the selected client. |  |

Table 6.1.7.6.1-2 specifies data types re-used by the AIMLES_AIMLEClientSelection API service.

Table 6.1.7.6.1-2: AIMLES_AIMLEClientSelection API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleClientId | Clause 6.1.7.6.3.2 | Represents unique identifier of a AIMLE client. |  |
| ClientDiscCriteria | Clause 6.2.2.6.2.2 | Represents the AIMLE Client discovery criteria. |  |
| DateTime | 3GPP TS 29.122 [2] | Used to indicate a timestamp. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |

##### 6.1.7.6.2	Structured data types

6.1.7.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.1.7.6.2.2	Type: ClientSelSub

Table 6.1.7.6.2.2-1: Definition of type ClientSelSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| selCriteria | ClientDiscCriteria | M | 1 | Represents the selection criteria for finding suitable AIMLE clients. |  |
| reqNum | Uinteger | O | 0..1 | Represents the requested number of the selected clients. |  |
| notifUri | Uri | M | 1 | Indicates the URI towards which the notification should be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.7.6.2.3	Type: ClientSelNotif

Table 6.1.7.6.2.3-1: Definition of type ClientSelNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(SelUpdate) | M | 1..N | Contains the AIMLE Client selection update reports. |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp of the data management notification. |  |

6.1.7.6.2.4	Type: ClientSelSubPatch

Table 6.1.7.6.2.4-1: Definition of type ClientSelSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| selCriteria | ClientDiscCriteria | O | 0..1 | Represents the selection criteria for finding suitable AIMLE clients for data management operations. |  |
| num | Uinteger | O | 0..1 | Represents the requested number of the selected clients. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI towards which the notification should be delivered. |  |

6.1.7.6.2.5	Type: ClientSelReq

Table 6.1.7.6.2.5-1: Definition of type ClientSelReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Represents the VAL service identifier. |  |
| clients | array(string) | C | 1..N | Represents the list of AIMLE Client IDs. (NOTE 1) |  |
| selCriteria | ClientDiscCriteria | C | 0..1 | Represents the selection criteria for finding suitable AIMLE clients for data management operations. (NOTE 1) |  |
| reqNum | Uinteger | O | 0..1 | Represents the required number of the selected clients. (NOTE 2) |  |
| minNum | Uinteger | O | 0..1 | Represents the minimal number of the selected clients. (NOTE 2) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE 1: These attributes are mutual exclusive and only one shall be provided. NOTE 2: This attribute may only be present if the "selCriteria" attribue is present. |  |  |  |  |  |

6.1.7.6.2.6	Type: ClientSelResp

Table 6.1.7.6.2.6-1: Definition of type ClientSelResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clients | array(string) | O | 1..N | Represents the list of selected AIMLE Client IDs. |  |
| clientSet | string | O | 1..N | Represents the selected AIMLE Client set identifier. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.7.6.2.7	Type: SelUpdate

Table 6.1.7.6.2.7-1: Definition of type SelUpdate

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| replaced | string | M | 1 | Represents the replaced AIMLE Client ID. |  |
| selected | string | M | 1 | Represents the newly selected AIMLE Client ID instead of the replaced one. |  |

##### 6.1.7.6.3	Simple data types and enumerations

6.1.7.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.7.6.3.2	Simple data types

The simple data types defined in table 6.1.7.6.3.2-1 shall be supported.

Table 6.1.7.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.7.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.7.6.5	Binary data

6.1.7.6.5.1	Binary Data Types

Table 6.1.7.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.7.7	Error Handling

##### 6.1.7.7.1	General

For the AIMLES_AIMLEClientSelection API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_AIMLEClientSelection API.

##### 6.1.7.7.2	Protocol Errors

No specific protocol errors for the AIMLES_AIMLEClientSelection API are specified.

##### 6.1.7.7.3	Application Errors

The application errors defined for AIMLES_AIMLEClientSelection API are listed in table 6.1.7.7.3-1.

Table 6.1.7.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.7.8	Feature negotiation

The optional features in table 6.1.7.8-1 are defined for the AIMLES_AIMLEClientSelection API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.7.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.7.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AIMLEClientSelection API.

### 6.1.8	AIMLES_MLModelTraining API

#### 6.1.8.1	Introduction

The ML model training service shall use the AIMLES_MLModelTraining API.

The API URI of the AIMLES_MLModelTraining API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.1.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-trn".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. AIMLE Server) takes the role of the SCEF and the service consumer (i.e. VAL Server) takes the role of the SCS/AS.

#### 6.1.8.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2.1 of 3GPP TS 29.122 [2] shall apply for the AIMLES_MLModelTraining API.

#### 6.1.8.3	Resources

There are no resources defined for this API in this release of the specification.

#### 6.1.8.4	Custom Operations without associated resources

##### 6.1.8.4.1	Overview

The structure of the custom operation URIs of the AIMLES_ContextTransfer API is shown in Figure 6.1.8.4.1-1.

Figure 6.1.8.4.1-1: Custom operation URI structure of the AIMLES_MLModelTraining API

Table 6.1.8.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the AIMLES_MLModelTraining API.

Table 6.1.8.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestTrain | /request-train | POST | Enables a service consumer to send AIMLE ML Model training request to the AIMLE server. |

The custom operations shall support the URI variables defined in table 6.1.8.4.1-2.

Table 6.1.8.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.8. |

##### 6.1.8.4.2	Operation: RequestTrain

6.1.8.4.2.1	Description

The custom operation enables a service consumer to send AIMLE ML Model training request to the AIMLE server.

6.1.8.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.1.8.4.2.2-1 and 6.1.8.4.2.2-2.

Table 6.1.8.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainRequest | M | 1 | Contains the parameters to request AIMLE ML Model training. |

Table 6.1.8.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimleModelId | M | 1 | 200 OK | Successful case. The ML Model training request is successfully received and processed. The identifier of the ML model selected by AIMLE server for training shall be included in the response message. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the <e.g. HTTP POST> method listed in table 5.2.1.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

#### 6.1.8.5	Notifications

##### 6.1.8.5.1	General

Notifications shall comply to clause 5.2.1.5 of 3GPP TS 29.122 [2].

Table 6.1.8.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ML Model Training Notification | {notifUri} | POST | Notification about the ML Model training. |

##### 6.1.8.5.2	ML Model Training Notification

6.1.8.5.2.1	Description

The Event Notification is used by the service producer to report the ML Model Training events to the service consumer that has subscribed to such Notifications.

6.1.8.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.8.5.2.1.2-1.

Table 6.1.8.5.2.1.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback Uri |

6.1.8.5.2.3	Standard Methods

6.1.8.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.8.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.8.5.2.3.1-1.

Table 6.1.8.5.2.3.1-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlModelTrainNotif | M | 1 | Notification information on the ML Model Training events. |

Table 6.1.8.5.2.3.1-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.1.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

#### 6.1.8.6	Data Model

##### 6.1.8.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.8.6.1-1 specifies the data types defined for the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-1: AIMLES_MLModelTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MemberInfo | 6.1.8.6.2.7 | Represents the list of AIMLE clients selected or de-selected for the ML model training. |  |
| MemberSelCriteria | 6.1.8.6.2.4 | Represents the criteria to be continuously monitored for selecting the member clients. |  |
| MlModelInfo | 6.1.8.6.2.5 | Represents the ML model information. |  |
| MlModelTrainNotif | 6.1.8.6.2.3 | Represents the ML Model training notification |  |
| ModelDomain | 6.1.8.6.3.6 | Represents the domain for which the model can be used. |  |
| TrainRequest | 6.1.8.6.2.2 | Represents the ML Model training request |  |
| PerfParam | 6.1.8.6.2.8 | Represents the output of training, e.g., ML model performance parameters for the training. |  |

Table 6.1.8.6.1-2 specifies data types re-used by the AIMLES_MLModelTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-2: AIMLES_MLModelTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| LocationArea5G | 3GPP TS 29.122 [5] | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |
| Uri | 3GPP TS 29.122 [2] | Represent an URI, used to indicate the notification URI. |  |
| ClientCapability | 3GPP TS 29.560 [16] | Represents the client capability information. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicates the time window. |  |
| Endpoint | 3GPP TS 29.558 [19] | Represents the endpoint information. |  |
| Float | 3GPP TS 29.571 [15] | Used to represent the fractional part of the proximity range in the reference UE details. |  |
| MlModel | Clause 6.2.1.6.2.4 | Represents the ML model that has to be distributed to the selected member clients for training. |  |

##### 6.1.8.6.2	Structured data types

6.1.8.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.1.8.6.2.2	Type: TrainRequest

Table 6.1.8.6.2.2-1: Definition of type TrainRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trnType | TrainingType | M | 1 | Contains the type of training to be performed. |  |
| members | array(AimleClientId) | C | 1..N | Contains the list of member clients to be utilized for training the ML model. (NOTE 1) |  |
| memSelCrit | MemberSelCriteria | C | 0..1 | Contains the criteria that needs to be continuously monitored for selecting the member clients. (NOTE 1) |  |
| modelInf | MlModelInfo | C | 0..1 | Contains the ML model that has to be distributed to the selected member clients for training. (NOTE 2) |  |
| modelReq | MlModel | C | 0..1 | Contains the requirement for selecting a model to be trained and the filtering criteria for selecting the model. (NOTE 2) |  |
| memUpdNotif | Boolean | O | 0..1 | Indicates whether the requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Set to "true" to indicate that requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Default value is "false" if omitted. |  |
| notifUri | Uri | C | 0..1 | Contains the notification URI where notifications should be sent. This attribute shall be present when the attribute "memUpdNotif" is set to "true". |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.3	Type: MlModelTrainNotif

Table 6.1.8.6.2.3-1: Definition of type MlModelTrainNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| members | array(MemberInfo) | C | 1..N | Contains the list of AIMLE clients selected or de-selected for the ML model training. (NOTE) |  |
| trainOut | PerfParams | C | 0..1 | Contains the output of training, e.g., ML model parameters for the training. (NOTE) |  |
| trainErr | TrainingErr | C | 0..1 | Contains the list of errors, if any, encountered during training process. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

.

###### 6.1.8.6.2.4	Type: MemberSelCriteria

Table 6.1.8.6.2.4-1: Definition of type MemberSelCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientLoc | LocationArea5G | C | 0..1 | Contains the location of the AIMLE client for the AIML service that needs to be monitored. |  |
| clientAvailability | TimeWindow | C | 0..1 | Contains the required availability duration of the AIMLE client. |  |
| clientCapability | ClientCapability | C | 0..1 | Contains the required client capability information. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.5	Type: MlModelInfo

Table 6.1.8.6.2.5-1: Definition of type MlModelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | C | 0..1 | Contains the identifier for ML model. |  |
| mlModelLoc | Endpoint | C | 0..1 | Contains the URI, fqdn or address that maps to the resource where ML model is stored. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.6	Type: MemberInfo

Table 6.1.8.6.2.6-1: Definition of type MemberInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientSel | boolean | C | 0..1 | Indicates whether the AIMLE client is selected or not. 0 represents the client is not selected. 1 represents the client is selected. |  |
| clientUri | Uri | C | 0..1 | Contains the URI information of the AIMLE clients. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.7	Type: PerfParam

Table 6.1.8.6.2.7-1: Definition of type PerfParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelAccuracy | integer | C | 0..1 | Indicates the achieved ML model accuracy, expressed as a percentage. Minimum: 0, maximum: 100. |  |
| modelPrecision | Float | C | 0..1 | Represents the accuracy for the positive predictions made by the model. Minimum: 0, maximum: 1. |  |
| modelRecall | Float | C | 0..1 | Represents model's ability to identify all the actual positive instances within a dataset. Minimum: 0, maximum: 1. |  |
| modelF1Score | Float | C | 0..1 | Represents the combined metric for precision and recall. Minimum: 0, maximum: 1. |  |
| errorMeanSquare | Float | C | 0..1 | Represents the mean squared error between predicted values and actual values. |  |
| errorMeanAbs | Float | C | 0..1 | Represents the mean absolute error between predicted values and actual values. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 6.1.8.6.3	Simple data types and enumerations

6.1.8.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.8.6.3.2	Simple data types

The simple data types defined in table 6.1.8.6.3.2-1 shall be supported.

Table 6.1.8.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AimleModelId | string | Represents the ML model identifier. |  |

###### 6.1.8.6.3.3	Enumeration: TrainingType

The enumeration TrainingType represents the type training requested. It shall comply with the provisions defined in table 6.1.8.6.3.3-1.

Table 6.1.8.6.3.3-1: Enumeration TrainingType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRAIN_VFL | Indicates that the training type is horizontal federated learning |  |
| TRAIN_HFL | Indicates that the training type is vertical federated learning |  |

###### 6.1.8.6.3.4	Enumeration: TrainingErr

The enumeration TrainingErr represents the training error encountered during ML model training. It shall comply with the provisions defined in table 6.1.8.6.3.4-1.

Table 6.1.8.6.3.4-1: Enumeration TrainingErr

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNDERFITTING | Indicates that the trained model is underfitting the training data. |  |
| OVERFITTING | Indicates that the trained mode is overfitting the training data. |  |
| PERFORMANCE_ERRORS | Indicates that the trained model is unable to meet the desired performance. |  |
| DATA_LEAKAGE | Indicates that there is data leakage from evaluation data set to the training data. |  |

##### 6.1.8.6.4	Data types describing alternative data types or combinations of data types

6.1.8.6.4.1	Type: <TypeName 1>

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.8.6.5	Binary data

6.1.8.6.5.1	Binary Data Types

Table 6.1.8.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.8.7	Error Handling

##### 6.1.8.7.1	General

For the AIMLES_MLModelTraining API, HTTP error responses shall be supported as specified in clause 5.2.1.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.1.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.1.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the AIMLES_MLModelTraining API.

##### 6.1.8.7.2	Protocol Errors

No specific procedures for the AIMLES_MLModelTraining API are specified.

##### 6.1.8.7.3	Application Errors

The application errors defined for the AIMLES_MLModelTraining API are listed in Table 6.1.8.7.3-1.

Table 6.1.8.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

#### 6.1.8.8	Feature negotiation

The optional features in table 6.1.8.8-1 are defined for the AIMLES_MLModelTraining API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.1.7 of 3GPP TS 29.122 [2].

Table 6.1.8.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.8.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the AIMLES_MLModelTraining API.

### 6.1.9	AIMLES_MLModelPerfMonitor API

#### 6.1.9.1	Introduction

The AIMLES_MLModelPerfMonitor Service shall use the AIMLES_MLModelPerfMonitor API.

The API URI of the AIMLES_MLModelPerfMonitor API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-mlmpm".

-	The <apiVersion> shall be "v1".

-	-	The <apiSpecificSuffixes> shall be set as described in clause 6.1.9.3 and clause 6.1.9.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.9, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.9.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_MLModelPerfMonitor API.

#### 6.1.9.3	Resources

##### 6.1.9.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.9.3.1-1 depicts the resource URIs structure for the AIMLES_MLModelPerfMonitor API.

Figure 6.1.9.3.1-1: Resource URI structure of the AIMLES_MLModelPerfMonitor API

Table 6.1.9.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.9.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE ML Model Performance Monitor Subscriptions | /subscriptions | POST | Request the creation of an AIMLE ML Model Performance Monitor Subscription resource. |
| Individual AIMLE ML Model Performance Monitor Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource. |

##### 6.1.9.3.2	Resource: AIMLE ML Model Performance Monitor Subscriptions

6.1.9.3.2.1	Description

This resource represents the collection of AIMLE ML Model Performance Monitor Subscriptions managed by the AIMLE Server.

6.1.9.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-mlmpm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.9.3.2.2-1.

Table 6.1.9.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.9.1. |

6.1.9.3.2.3	Resource Standard Methods

6.1.9.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual AIMLE ML Model Performance Monitor Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.9.3.2.3.1-1.

Table 6.1.9.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.9.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.9.3.2.3.1-3.

Table 6.1.9.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | Create a new Individual AIMLE ML Model Performance Monitor Subscription resource. |

Table 6.1.9.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE ML Model Performance Monitor Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.9.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-mlmpm/<apiVersion>/subscriptions{subscriptionId} |

6.1.9.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.9.3.3	Resource: Individual AIMLE ML Model Performance Monitor Subscription

6.1.9.3.3.1	Description

6.1.9.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-mlmpm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.9.3.3.2-1.

Table 6.1.9.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.9.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE ML Model Performance Monitor Subscription" resource. |

6.1.9.3.3.3	Resource Standard Methods

6.1.9.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual AIMLE ML Model Performance Monitor Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.9.3.3.3.1-1.

Table 6.1.9.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.9.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.9.3.3.3.1-3.

Table 6.1.9.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.9.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE ML Model Performance Monitor Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.9.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.9.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.9.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.9.3.3.3.2-1.

Table 6.1.9.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.9.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.9.3.3.3.2-3.

Table 6.1.9.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | Represents the updated representation of the "Individual AIMLE ML Model Performance Monitor Subscription" resource. |

Table 6.1.9.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE ML Model Performance Monitor Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Performance Monitor Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.9.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.9.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.9.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.9.3.3.3.3-1.

Table 6.1.9.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.9.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.9.3.3.3.3-3.

Table 6.1.9.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlMdlPerfMonitSubPatch | M | 1 | Represents the parameters to request the modification of the "Individual AIMLE ML Model Performance Monitor Subscription" resource. |

Table 6.1.9.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MlMdlPerfMonitSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE ML Model Performance Monitor Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Performance Monitor Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.9.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.9.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.9.3.3.3.4	DELETE

The HTTP PATCH method allows a AIMLE service consumer to request the deletion of an existing "Individual AIMLE ML Model Performance Monitor Subscription" resource.

Table 6.1.9.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.9.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.9.3.3.3.4-3.

Table 6.1.9.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.9.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Performance Monitor Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.9.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.9.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.9.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.9.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 6.1.9.5	Notifications

##### 6.1.9.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.9.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE ML Model Performance Monitor Event Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML ML Model Performance Monitor related event(s). |

##### 6.1.9.5.2	AIMLE ML Model Performance Monitor Event Notification

6.1.9.5.2.1	Description

The AIMLE ML Model Performance Monitor Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML ML Model Performance Monitor related event(s).

6.1.9.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.9.5.2.2-1.

Table 6.1.9.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification URI is assigned within the Individual AIMLE ML Model Performance Monitor Subscription and described within the MlMdlPerfMonitSub type |

6.1.9.5.2.3	Standard Methods

6.1.9.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.9.5.2.3-1 and the response data structures and response codes specified in table 6.1.9.5.2.3-2.

Table 6.1.9.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlMdlPerfMonitNotif | M | 1 | Represents the AIMLE ML Model Performance Monitor Event Notification. |

Table 6.1.9.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE ML Model Performance Monitor Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.9.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.9.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.9.6	Data Model

##### 6.1.9.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.9.6.1-1 specifies the data types defined for the AIMLES_MLModelPerfMonitor API.

Table 6.1.9.6.1-1: AIMLES_MLModelPerfMonitor API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| MlMdlPerfMonitSub | 6.1.9.6.2.2 | Represents the AIMLE ML Model Performance Monitor subscription information. |  |
| MlMdlPerfMonitSubPatch | 6.1.9.6.2.3 | Represents the requested modifications to the AIMLE ML Model Performance Monitor subscription information. |  |
| MlMdlPerfMonitSubNotif | 6.1.9.6.2.4 | Represents the AIMLE ML Model Performance Monitor notification. |  |
| MontReportConfig | 6.1.9.6.2.5 | Represents the configuration of the monitoring report. |  |
| AimlOperInfo | 6.1.9.6.2.6 | Represents the information on the AIML operation. |  |
| MlMdlDegraded | 6.1.9.6.2.7 | Represents the metrics of the degraded ML model. |  |
| ServPerfKpi | 6.1.9.6.2.8 | Represents the key performance indicators for the AIMLE service performance. |  |
| AimlOperType | 6.1.9.6.3.3 | Represents the AIML operation type. |  |
| ReportType | 6.1.9.6.3.4 | Represents the type of the monitoring report. |  |
| TrigActReqType | 6.1.9.6.3.5 | Represents the types of the trigger action requirements on the monitoring event. |  |
| MlMdlDegradedParam | 6.1.9.6.3.6 | Represents the parameters of the degraded ML model. |  |

Table 6.1.9.6.1-2 specifies data types re-used by the AIMLES_MLModelPerfMonitor API service.

Table 6.1.9.6.1-2: AIMLES_MLModelPerfMonitor API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |
| GeographicArea | 3GPP TS 29.572 [18] | Identifies the geographical information of the FL member. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Identifies the start time and the end time for the validity time. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of optional features. |  |
| AimleClientId | Clause 6.1.2.6.3.2 | Represents unique identifier of a AIMLE client. |  |
| QosMonitoringInformation | 3GPP TS 29.122 [2] | Identifies the QoS monitoring information. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |

##### 6.1.9.6.2	Structured data types

6.1.9.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.1.9.6.2.2	Type: MlMdlPerfMonitSub

Table 6.1.9.6.2.2-1: Definition of type MlMdlPerfMonitSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlMdlId | string | M | 1 | Identifies the ML model, for which the request applies. |  |
| notifUri | Uri | M | 1 | Identifies the URI, towards which the notification should be delivered. |  |
| montReportConfig | MontReportConfig | M | 1 | Configuration of the monitoring report, for which the request applies. |  |
| aimlOperInfo | AimlOperInfo | O | 0..1 | Information on the AIML operation, for which the ML model is used. |  |
| trigActReq | array(TrigActReqType) | O | 0..3 | Identifies the requirement for triggering an action on the monitoring event. |  |
| areaValidity | GeographicArea | O | 0..1 | Identifies the coordinate of the area of interest, for which the request applies. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the time of interest, for which the request applies. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Identifies the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.9.6.2.3	Type: MlMdlPerfMonitSubPatch

Table 6.1.9.6.2.3-1: Definition of type MlMdlPerfMonitSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Identifies the URI, towards which the notification should be delivered. |  |
| montReportConfig | MontReportConfig | O | 0..1 | Configuration of the monitoring report, for which the request applies. |  |
| aimlOperInfo | AimlOperInfo | O | 0..1 | Information on the AIML operation, for which the ML model is used. |  |
| areaValidity | GeographicArea | O | 0..1 | Identifies the coordinate of the area of interest, for which the request applies. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the time of interest, for which the request applies. |  |

6.1.9.6.2.4	Type: MlMdlPerfMonitNotif

Table 6.1.9.6.2.4-1: Definition of type MlMdlPerfMonitNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlMdlDegraded | MlMdlDegraded | M | 1 | Identitifies the type of degradation of the ML Model performance |  |
| trigActReq | TrigActReqType | O | 1 | Identifies the requirement for triggering an action on the monitoring event. |  |

6.1.9.6.2.5	Type: MontReportConfig

Table 6.1.9.6.2.5-1: Definition of type MontReportConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reportType | ReportType | M | 1 | Identifies the type of the report. |  |
| delay | Uinteger | C | 0’’1 | Identifies delay time in units of micro seconds, that triggers the monitoring event. |  |
| minAccuracy | Uinteger | C | 0..1 | Identifies an unsigned integer between 0 and 100 representing the percentage of minimum accuracy, that triggers the monitoring event. |  |
| period | Uinteger | C | 0..1 | Identifies an unsigned integer, that represents the a time interval in seconds. |  |

6.1.9.6.2.6	Type: AimlOperInfo

Table 6.1.9.6.2.6-1: Definition of type AimlOperInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlOper | AimlOperType | M | 1 | Identifies the type of the AIML operation. |  |
| servId | string | M | 1 | Identifies the AIMLE service using the ML model. |  |
| clientList | array(AimleClientId) | M | 1..N | Identifies the list of AIMLE client identifiers for training ML Model. |  |
| servPerfKpi | ServPerfKpi | M | 1 | Identifies the key performance indicators for the AIMLE service performance. |  |

6.1.9.6.2.7	Type: MlMdlDegraded

Table 6.1.9.6.2.7-1: Definition of type MlMdlDegraded

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlMdlDegradeInd | Boolean | M | 1 | Identifies whether the ML model has degraded (TRUE) or not (FALSE). |  |
| mlMdlDegradedParam | Array(MlMdleDegradedParam) | O | 0..N | Identifies the ML model metrics that have degraded. |  |
| mlMdlDegradedCause | string | O | 0..1 | Identifies the cause for the ML model degradation. |  |

6.1.9.6.2.8	Type: ServPerfKpi

Table 6.1.9.6.2.8-1: Definition of type ServPerfKpi

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| latency | Uinteger | M | 1 | Identifies a period of time in units of micro seconds. |  |
| accuracy | Uinteger | M | 1 | Identifies an unsigned integer between 0 and 100 representing the percentage of accuracy. |  |
| qos | QosMonitoringInformation | M | 1 | Idenfying the QoS Monitoring information |  |

6.1.9.6.2.9	Type: MlMdlDegradedParam

Table 6.1.9.6.2.9-1: Definition of type MlMdlDegradedParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| recall | Uinteger | M | 1 | Identifies an unsigned integer between 0 and 100 representing the percentage of recall. |  |
| precision | Uinteger | M | 1 | Identifies an unsigned integer between 0 and 100 representing the percentage of precision. |  |
| accuracy | Uinteger | M | 1 | Identifies an unsigned integer between 0 and 100 representing the percentage of accuracy. |  |

##### 6.1.9.6.3	Simple data types and enumerations

6.1.9.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.9.6.3.2	Simple data types

The simple data types defined in table 6.1.9.6.3.2-1 shall be supported.

Table 6.1.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.9.6.3.3	Enumeration: AimlOperType

Table 6.1.9.6.3.3-1: Enumeration AimlOperType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ML_MODEL | Identifies that AIML operation is ML model training. |  |
| HFL | Identifies that AIML operation is Horizontal Federated Learning (HFL) training. |  |
| VFL | Identifies that AIML operation is Vertical Federated Learning (VFL) training. |  |
| TL | Identifies that AIML operation is Transfer Learning (TL) training. |  |

6.1.9.6.3.4	Enumeration: ReportType

Table 6.1.9.6.3.4-1: Enumeration ReportType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ONE_TIME | Identifies the report is of one time type. |  |
| PERIODIC | Identifies the report is of periodic type. |  |
| EVENT_TRIGGERED | Identifies the report is of event triggered type. |  |

6.1.9.6.3.5	Enumeration: TrigActReqType

Table 6.1.9.6.3.4-1: Enumeration TrigActReqType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ADAPTATION | Identifies the trigger action requirement is training of a new ML model for the AIMLE service by the same or a new AIMLE client. |  |
| RETRAINING | Identifies the trigger action requirement is retraining of the existing ML model for the AIMLE service by the same or a new AIMLE client. |  |
| TERMINATION | Identifies the trigger action requirement is terminating of the existing ML model for AIMLE service and initiating to train a new ML model by the same or a new AIMLE client. |  |

##### 6.1.9.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.9.6.5	Binary data

6.1.9.6.5.1	Binary Data Types

Table 6.1.9.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.9.7	Error Handling

##### 6.1.9.7.1	General

For the AIMLES_MLModelPerfMonitor API, HTTP error responses shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_MLModelPerfMonitor API.

##### 6.1.9.7.2	Protocol Errors

No specific protocol errors for the AIMLES_MLModelPerfMonitor API are specified.

##### 6.1.9.7.3	Application Errors

The application errors defined for AIMLES_MLModelPerfMonitor API are listed in table 6.1.9.7.3-1.

Table 6.1.9.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.9.8	Feature negotiation

The optional features in table 6.1.9.8-1 are defined for the AIMLES_MLModelPerfMonitor API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.9.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.9.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_MLModelPerfMonitor API.

### 6.1.10	AIMLES_TLModelSelectionAssistance API

#### 6.1.10.1	Introduction

The AIMLES_TLModelSelectionAssistance service shall use the AIMLES_TLModelSelectionAssistance API.

The API URI of the AIMLES_TLModelSelectionAssistance API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-tlmsa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (e.g. AIMLE Server) takes the role of the SCEF and the service consumer (i.e. AIMLE service consumer, e.g. VAL server) takes the role of the SCS/AS.

#### 6.1.10.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2.1 of 3GPP TS 29.122 [2] shall apply for the AIMLES_TLModelSelectionAssistance API.

#### 6.1.10.3	Resources

##### 6.1.10.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.10.3.1-1 depicts the resource URIs structure for the AIMLES_TLModelSelectionAssistance Service API.

Figure 6.1.10.3.1-1: Resource URI structure of the AIMLES_TLModelSelectionAssistance Service API

Table 6.1.10.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.10.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE TL Model Selection Assistance | /pre-trained-models | GET | Request the AIMLE TL Model Selection Assistance according to the filtering criteria. |

##### 6.1.10.3.2	Resource: AIMLE TL Model Selection Assistance

###### 6.1.10.3.2.1	Description

The "AIMLE TL Model Selection Assistance" resource represents the AIMLE TL Model Selection Assistance.

###### 6.1.10.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-tlmsa/<apiVersion>/pre-trained-models

This resource shall support the resource URI variables defined in the table 6.1.10.3.2.2-1.

Table 6.1.10.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [14]. |

###### 6.1.10.3.2.3	Resource Standard Methods

6.1.10.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.10.3.2.3.1-1.

Table 6.1.10.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| filt-criteria | TlModelSelectAssistReq | M | 1 | Represents the AIMLE Model Selection Assistance filtering criteria. |
| supported-features | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |

This method shall support the request data structures specified in table 6.1.10.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.10.3.2.3.1-3.

Table 6.1.10.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.10.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TlModelSelectAssistResp | M | 1 | 200 OK | Successful case. The response body contains one or more pre-trained ML models for the target ML task. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.10.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.10.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

###### 6.1.10.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.10.4	Custom Operations without associated resources

There are no custom operations without associated resources in the present release of the document.

#### 6.1.10.5	Notifications

There are no notifications in the present release of the document.

#### 6.1.10.6	Data Model

##### 6.1.10.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.10.6.1-1 specifies the data types defined for the AIMLES_TLModelSelectionAssistance API.

Table 6.1.10.6.1-1: AIMLES_TLModelSelectionAssistance API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
|  |  | . |  |

Table 6.1.10.6.1-2 specifies data types re-used by the AIMLES_TLModelSelectionAssistance API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_TLModelSelectionAssistance API.

Table 6.1.10.6.1-2: AIMLES_TLModelSelectionAssistance API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| TlModelSelectAssistReq | 3GPP TS 24.560 [16] | Represents the filter criteria. |  |
| TlModelSelectAssistResp | 3GPP TS 24.560 [16] | Represents the pre-trained models for the target ML task. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the supported features, used to negotiate the supported optional features of the API. |  |

##### 6.1.10.6.2	Structured data types

###### 6.1.10.6.2.1	Introduction

There is not any structure to be used in resource representations.

##### 6.1.10.6.3	Simple data types and enumerations

###### 6.1.10.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.1.10.6.3.2	Simple data types

The simple data types defined in table 6.1.10.6.3.2-1 shall be supported.

Table 6.1.10.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.10.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types and combinations of data types in this release of the specification.

##### 6.1.10.6.5	Binary data

There are no binary data defined in this release of the specification.

#### 6.1.10.7	Error Handling

##### 6.1.10.7.1	General

For the AIMLES_TLModelSelectionAssistance API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_TLModelSelectionAssistance API.

##### 6.1.10.7.2	Protocol Errors

No specific procedures for the AIMLES_TLModelSelectionAssistance API are specified.

##### 6.1.10.7.3	Application Errors

The application errors defined for AIMLES_TLModelSelectionAssistance API are listed in table 6.1.10.7.3-1.

Table 6.2.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.10.8	Feature Negotiation

The optional features in table 6.1.10.8-1 are defined for the AIMLES_TLModelSelectionAssistance API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.10.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.10.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_TLModelSelectionAssistance API.

### 6.1.11	AIMLES_AssistedMLModelSelection API

#### 6.1.11.1	Introduction

The AIMLES_AssistedMLModelSelection service shall use the AIMLES_AssistedMLModelSelection API.

The API URI of the AIMLES_AssistedMLModelSelection API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-amlmsel".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.1.11.3 and 6.1.11.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.11, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.11.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AssistedMLModelSelection API.

#### 6.1.11.3	Resources

##### 6.1.11.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.11.3.1-1 depicts the resource URIs structure for the AIMLES_AssistedMLModelSelection API.

Figure 6.1.11.3.1-1: Resource URI structure of the AIMLES_AssistedMLModelSelection API

Table 6.1.11.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.11.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE Assisted ML Model Selection Subscriptions | /subscriptions | POST | Request the creation of a AIMLE Assisted ML Model Selection Subscription resource. |
| Individual AIMLE Assisted ML Model Selection Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource. |

##### 6.1.11.3.2	Resource: AIMLE Assisted ML Model Selection Subscriptions

6.1.11.3.2.1	Description

This resource represents the collection of AIMLE Assisted ML Model Selection Subscriptions managed by the AIMLE Server.

6.1.11.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-amlmsel/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.11.3.2.2-1.

Table 6.1.11.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.11.1. |

6.1.11.3.2.3	Resource Standard Methods

6.1.11.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual AIMLE Assisted ML Model Selection Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.11.3.2.3.1-1.

Table 6.1.11.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.11.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.11.3.2.3.1-3.

Table 6.1.11.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | Create a new Individual AIMLE Assisted ML Model Selection Subscription resource. |

Table 6.1.11.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE Assisted ML Model Selection Subscription resource is confirmed and a representation of that resource is returned in the response body. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.11.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-amlmsel/<apiVersion>/subscriptions{subscriptionId} |

6.1.11.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.11.3.3	Resource: Individual AIMLE Assisted ML Model Selection Subscription

6.1.11.3.3.1	Description

6.1.11.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-amlmsel/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.11.3.3.2-1.

Table 6.1.11.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.11.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE Assisted ML Model Selection Subscription" resource. |

6.1.11.3.3.3	Resource Standard Methods

6.1.11.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual AIMLE Assisted ML Model Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.11.3.3.3.1-1.

Table 6.1.11.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.11.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.11.3.3.3.1-3.

Table 6.1.11.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.11.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE Assisted ML Model Selection Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.11.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.11.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.11.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.11.3.3.3.2-1.

Table 6.1.11.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.11.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.11.3.3.3.2-3.

Table 6.1.11.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | Represents the updated representation of the "Individual AIMLE Assisted ML Model Selection Subscription" resource. |

Table 6.1.11.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | 200 OK | Successful case. The "Individual AIMLE Assisted ML Model Selection Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Assisted ML Model Selection Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.11.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.11.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.11.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.11.3.3.3.3-1.

Table 6.1.11.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.11.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.11.3.3.3.3-3.

Table 6.1.11.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AssistMLMdlSelSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual AIMLE Assisted ML Model Selection Subscription" resource. |

Table 6.1.11.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AssistMLMdlSelSubsc | M | 1 | 200 OK | Successful case. The "Individual AIMLE Assisted ML Model Selection Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Assisted ML Model Selection Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. NOTE 2: Failure causes are described in clause 6.4.7. |  |  |  |  |

Table 6.1.11.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.11.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.11.3.3.3.4	DELETE

The HTTP PATCH method allows a AIMLE service consumer to request the deletion of an existing "Individual AIMLE Assisted ML Model Selection Subscription" resource.

Table 6.1.11.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.11.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.11.3.3.3.4-3.

Table 6.1.11.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.11.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Assisted ML Model Selection Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.11.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.11.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.11.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.11.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 6.1.11.5	Notifications

##### 6.1.11.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.11.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Assisted ML Model Selection Event Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed service consumer on AIML Assisted ML Model Selection related event(s). |

##### 6.1.11.5.2	AIMLE Assisted ML Model Selection Event Notification

6.1.11.5.2.1	Description

The AIMLE Assisted ML Model Selection Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML Assisted ML Model Selection related event(s).

6.1.11.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.11.5.2.2-1.

Table 6.1.11.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification Uri is assigned within the Individual AIMLE Assisted ML Model Selection Subscription and described within the AssistMLMdlSelSubsc type |

6.1.11.5.2.3	Standard Methods

6.1.11.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.11.5.2.3-1 and the response data structures and response codes specified in table 6.1.11.5.2.3-2.

Table 6.1.11.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AssistMLMdlSelNotif | M | 1 | Represents the AIMLE Assisted ML Model Selection Event Notification. |

Table 6.1.11.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Assisted ML Model Selection Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.11.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.11.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.11.6	Data Model

##### 6.1.11.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.11.6.1-1 specifies the data types defined for the AIMLES_AssistedMLModelSelection API.

Table 6.1.11.6.1-1: AIMLES_AssistedMLModelSelection API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlProfile | Clause 6.1.11.6.2.4 | Represents the ML model selection operation. |  |
| AssistMLMdlSelSubsc | Clause 6.1.11.6.2.2 | Represents the AIMLE Assisted ML Selection subscription information. |  |
| AssistMLMdlSelNotif | Clause 6.1.11.6.2.3 | Represents the AIMLE Assisted ML Selection notification. |  |
| AssistMLMdlSelSubscPatch | Clause 6.1.11.6.2.7 | Represents the requested modifications to a AIMLE Assisted ML Selection subscription information. |  |
| CandMLMdl | Clause 6.1.11.6.2.9 | Contains the candidate ML model selection information. |  |
| PerformanceMetric | Clause 6.1.11.6.3.3 | Represents the performance metric for training the ML model. |  |
| PerformanceRequirement | Clause 6.1.11.6.2.6 | Represents the performance requirements for ML model selection. |  |
| ReportingInformation | Clause 6.1.11.6.2.8 | Represents the reporting requirements for ML model selection. |  |
| TrainingRequirement | Clause 6.1.11.6.2.5 | Represents the training requirements for ML model selection. |  |

Table 6.1.11.6.1-2 specifies data types re-used by the AIMLES_AssistedMLModelSelection API service.

Table 6.1.11.6.1-2: AIMLES_AssistedMLModelSelection API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleClientId | Clause 6.1.2.6.3.2 | Represents unique identifier of a AIMLE client. |  |
| ClientDiscCriteria | Clause 6.2.2.6.2.2 | Represents the AIMLE Client selection criteria. |  |
| DataMgmtOp | Clause 6.1.2.6.3.3 | Represents the data management operation type. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| DurationSec | 3GPP TS 29.122 [2] | Unsigned integer identifying a period of time in units of seconds. |  |
| MlModelInfo | Clause 6.2.1.6.2.B | Represents the ML model Information. |  |
| MLModelTrainingInfo | Clause 6.2.1.6.2.9 | Represents the ML Model training information. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |

##### 6.1.11.6.2	Structured data types

6.1.11.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.1.11.6.2.2	Type: AssistMLMdlSelSubsc

Table 6.1.11.6.2.2-1: Definition of type AssistMLMdlSelSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlProfile | AimlProfile | M | 1 | Contains the requirements for the ML model selection operation. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI towards which the notification should be delivered. |  |
| repInfo | ReportingInformation | C | 0..1 | Contains the type of reporting that the subscription requires. Shall be provided if the "notifUri" attribute is present. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

6.1.11.6.2.3	Type: AssistMLMdlSelNotif

Table 6.1.11.6.2.3-1: Definition of type AssistMLMdlSelNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| opStatus | DataMgmtOp | M | 1 | Contains the received data management operation type. |  |
| trainMLModel | MLModelTrainingInfo | M | 1 | Contains the result of the ML model training. |  |
| elapseTime | DurationSec | O | 0..1 | Contains the duration of elapsed time in seconds for the ML model selection operation. |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp of the AIMLE assisted ML model selection notification. |  |

6.1.11.6.2.4	Type: AimlProfile

Table 6.1.11.6.2.4-1: Definition of type AimlProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| candMLMdls | array(CandMLMdl) | M | 1..N | Contains the list of candidate ML models and initial model parameters for training. |  |
| mlMdlReq | MlModelInfo | O | 0..N | Contains the additional list of candidate ML models for training. |  |
| dataSetIds | array(string) | M | 1..N | Represents the list dataset identifiers. |  |
| trainReq | array(TrainingRequirement) | M | 1..N | Contains the parameters for training requirements. |  |
| clientList | array(AimleClientId) | C | 1..N | Contains the list of AIMLE client set identifier to train the ML model. (NOTE 1) |  |
| clientSelCriteria | ClientDiscCriteria | C | 0..N | Contains the selection criteria for finding suitable AIMLE clients for training the ML model. (NOTE 1) |  |
| clNumber | Uinteger | C | 0..1 | Represents the required number of the AIMLE Clients for training the ML model. This attribute shall be provided if the "clientSelCriteria" attribute is present. |  |
| NOTE1: At least one of the information elements shall be provided. |  |  |  |  |  |

6.1.11.6.2.5	Type: TrainingRequirement

Table 6.1.11.6.2.5-1: Definition of type TrainingRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| perfReq | array(PerformanceRequirement) | M | 1..N | Identifies the performance metrics to evaluate ML model training. |  |
| trainCount | Unteger | M | 1 | Contains number of training rounds for the ML training. |  |
| sampleCount | Unteger | M | 1 | Contains number of data samples for the ML training. |  |

6.1.11.6.2.6	Type: PerformanceRequirement

Table 6.1.11.6.2.6-1: Definition of type PerformanceRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| perfMetric | PerformanceMetric | M | 1 | Identifies the performance metrics to evaluate ML model training. |  |
| perfTarget | Uinteger | O | 0..1 | Indicates the target value acceptable performance is reached and training can be stopped. Identifies an unsigned integer between 0 and 100 representing the target value. |  |

6.1.11.6.2.7	Type: AssistMLMdlSelSubscPatch

Table 6.1.11.6.2.7-1: Definition of type AssistMLMdlSelSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelReq | MlModelInfo | O | 0..1 | Contains the additional list of candidate ML models for training. |  |
| trainReq | array(TrainingRequirement) | O | 0..N | Contains the parameters for training requirements. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI towards which the notification should be delivered. |  |

6.1.11.6.2.8	Type: ReportingInformation

Table 6.1.11.6.2.8-1: Definition of type ReportingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifMethod | NotificationMethod | O | 0..1 | Represents the type of notification moethod. If "notifMethod" attribute is not supplied, the default value "ON_EVENT_DETECTION" applies. |  |
| jobPercentage | Uinteger | O | 0..1 | Represents the requirement job percentage completion. Identifies an unsigned integer between 0 and 100 representing the percentage of accuracy. Shall be provided if the notification method is set to "ON_JOB_COMPLETION". |  |
| timeWindow | TimeWindow | O | 0..1 | Identifies the starting and ending reporting time period for the event. Shall be provided if the notification method is set to "PERIODIC". |  |

6.1.11.6.2.9	Type: CandMLMdl

Table 6.1.11.6.2.9-1: Definition of type CandMLMdl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlMdlId | string | M | 1 | Identifies the ML model to train. |  |
| mlMdlParam | string | M | 1 | Indicates the initial model parameters to train. |  |

##### 6.1.11.6.3	Simple data types and enumerations

6.1.11.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.11.6.3.2	Simple data types

The simple data types defined in table 6.1.11.6.3.2-1 shall be supported.

Table 6.1.11.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.11.6.3.3	Enumeration: PerformanceMetric

Table 6.1.11.6.3.3-1: Enumeration PerformanceMetric

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACCURACY | Indicates the performance metric is accuracy. |  |
| PRECISION | Indicates the performance metric is precision. |  |
| RECALL | Indicates the performance metric is recall. |  |
| MEAN_SQUARED_ERROR | Indicates the performance metric is mean squared error. |  |
| MEAN_ABSOLUTE_ERROR | Indicates the performance metric is mean absolute error. |  |

6.1.11.6.3.4	Enumeration: NotificationMethod

Table 6.1.11.6.3.4-1: Enumeration NotificationMethod

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PERIODIC | The notification of the ML model status is periodically sent. |  |
| ON_JOB_COMPLETION | The notification is sent only after the entire ML model selection job is completed. |  |
| ON_PCT_COMPLETION | The notification is sent after the certain job percentage completion. |  |
| ON_EVENT_DETECTION | The notification is sent each time the event is detected. |  |

##### 6.1.11.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.11.6.5	Binary data

6.1.11.6.5.1	Binary Data Types

Table 6.1.11.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.11.7	Error Handling

##### 6.1.11.7.1	General

For the AIMLES_AssistedMLModelSelection API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_AssistedMLModelSelection API.

##### 6.1.11.7.2	Protocol Errors

No specific protocol errors for the AIMLES_AssistedMLModelSelection API are specified.

##### 6.1.11.7.3	Application Errors

The application errors defined for AIMLES_AssistedMLModelSelection API are listed in table 6.1.11.7.3-1.

Table 6.1.11.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.11.8	Feature negotiation

The optional features in table 6.1.11.8-1 are defined for the AIMLES_AssistedMLModelSelection API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.11.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.11.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AssistedMLModelSelection API.

### 6.1.12	AIMLES_SplitOpEvent API

#### 6.1.12.1	Introduction

The AIMLES_SplitOpEvent Service shall use the AIMLES_SplitOpEvent API.

The API URI of the AIMLES_SplitOpEvent API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-splitopevent".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.1.12.3 and clause 6.1.12.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.12, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.12.2	Usage of HTTP

The provisions of clause 5.2 of 3GPP TS 29.122 [2] shall apply for the AIMLES_SplitOpEvent API.

#### 6.1.12.3	Resources

##### 6.1.12.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.12.3.1-1 depicts the resource URIs structure for the AIMLES_SplitOpEvent API.

Figure 6.1.12.3.1-1: Resource URI structure of the AIMLES_SplitOpEvent API

Table 6.1.12.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.12.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE Split Operation Event Subscriptions | /subscriptions | POST | Request the creation of an AIMLE Split Operation Event Subscription resource. |
| Individual AIMLE Split Operation Event Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE Split Operation Event" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE Split Operation Event" resource. |
|  |  | PATCH | Modifies an "Individual AIMLE Split Operation Event” Subscription resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE Split Operation Event Subscription" resource. |

##### 6.1.12.3.2	Resource: AIMLE Split Operation Event Subscriptions

###### 6.1.12.3.2.1	Description

This resource represents the collection of AIMLE Split Operation Event Subscriptions managed by the AIMLE Server.

###### 6.1.12.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-splitopevent/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in Table 6.1.12.3.2.2-1.

Table 6.1.12.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.12.1. |

###### 6.1.12.3.2.3	Resource Standard Methods

6.1.12.3.2.3.1	POST

The HTTP POST method enables an AIMLE service consumer to request the creation of a new Individual AIMLE Split Operation Event Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.12.3.2.3.1-1.

Table 6.1.12.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.12.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.12.3.2.3.1-3.

Table 6.1.12.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | Create a new Individual AIMLE Split Operation Event Subscription resource. |

Table 6.1.12.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE Split Operation Event Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.12.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-mlmpm/<apiVersion>/subscriptions{subscriptionId} |

###### 6.1.12.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.12.3.3	Resource: Individual AIMLE Split Operation Event Subscription

###### 6.1.12.3.3.1	Description

The Individual Split Operation Event Subscription resource represents an individual event subscription of VAL server or AIMLE client.

###### 6.1.12.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-splitopevent/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in Table 6.1.12.3.3.2-1.

Table 6.1.12.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.12.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE Split Operation Event Subscription" resource. |

###### 6.1.12.3.3.3	Resource Standard Methods

6.1.12.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Split Operation Event Subscription" resource at the AIMLE server.

This method shall support the URI query parameters specified in table 6.1.12.3.3.3.1-1.

Table 6.1.12.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.12.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.12.3.3.3.1-3.

Table 6.1.12.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.12.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | 200 OK | Successful case. The requested "Individual Split Operation Event Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.12.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE server. |

Table 6.1.12.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE server. |

6.1.12.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Split Operation Event Subscription" resource at the AIMLE server.

This method shall support the URI query parameters specified in table 6.1.12.3.3.3.2-1.

Table 6.1.12.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.12.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.12.3.3.3.2-3.

Table 6.1.12.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | Represents the updated representation of the "Individual Split Operation Event Subscription" resource. |

Table 6.1.12.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | 200 OK | Successful case. The "Individual Split Operation Event Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Split Operation Event Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.12.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE server. |

Table 6.1.12.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE server. |

6.1.12.3.3.3.3	PATCH

This method shall support the URI query parameters specified in table 6.1.12.3.3.3.3-1.

Table 6.1.12.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.12.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.12.3.3.3.3-3.

Table 6.1.12.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpEventSubPatch | M | 1 | Contains the modifications to be applied to the Split Operation Event subscription resource. |

Table 6.1.12.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpEventSub | M | 1 | 200 OK | Split Operation Events Subscription resource is modified successfully and the representation of the modified Split Operation Event subscription is returned. |
| n/a |  |  | 204 No Content | The Split Operation Events Subscription is updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 6.1.12.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 6.1.12.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

6.1.12.3.3.3.4	DELETE

The HTTP DELETE method allows an AIMLE service consumer to request the deletion of an existing "Individual AIMLE Split Operation Event Subscription" resource.

Table 6.1.12.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in Table 6.1.12.3.3.3.2-2 and the response data structures and response codes specified in Table 6.1.12.3.3.3.2-3.

Table 6.1.12.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.12.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE Split Operation Event Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.12.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.12.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.12.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.12.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 6.1.12.5	Notifications

##### 6.1.12.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.122 [2].

Table 6.1.12.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Split Operation Event Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML Split Operation related event(s). |

##### 6.1.12.5.2	AIMLE Split Operation Event Notification

6.1.12.5.2.1	Description

The AIMLE Split Operation Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML Split Operation related event(s).

6.1.12.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.12.5.2.2-1.

Table 6.1.12.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification URI is assigned within the Individual AIMLE Split Operation Event Subscription and described within the SplitOpEventSub type |

6.1.12.5.2.3	Standard Methods

6.1.12.5.2.3.1	POST

This method shall support the request data structures specified in Table 6.1.12.5.2.3-1 and the response data structures and response codes specified in Table 6.1.12.5.2.3-2.

Table 6.1.12.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpEventNotif | M | 1 | Represents the AIMLE Split Operation Event Notification. |

Table 6.1.12.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE Split Operation Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the endpoint of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the endpoint of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.12.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.12.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.12.6	Data Model

##### 6.1.12.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.12.6.1-1 specifies the data types defined for the AIMLES_SplitOpEvent API.

Table 6.1.12.6.1-1: AIMLES_SplitOpEvent API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AssistanceInfo | 6.1.12.6.2.7 | Represents the split operation assistance information. |  |
| AvailabilityInfo | 6.1.12.6.2.8 | Represents the split operation availability information. |  |
| DiscFilters | 6.1.12.6.2.5 | Represents the discovery filters to determine matching split operation profile or nodes. |  |
| SplitOpEventSub | 6.1.12.6.2.2 | Represents the AIMLE Split Operation Event subscription information. |  |
| SplitOpEventSubPatch | 6.1.12.6.2.3 | Represents the requested modifications to the AIMLE Split Operation Event subscription information. |  |
| SplitOpEventSubNotif | 6.1.12.6.2.4 | Represents the AIMLE Split Operation Event notification. |  |
| SplitOpPipelineInfo | 6.1.12.6.2.6 | Represents split operation pipeline information. |  |
| SplitOpProfile | 6.1.12.6.2.9 | Represents the split operation profile that VAL server participates to. |  |
| StageInfo | 6.1.12.6.2.10 | Represnts the information related to each stage in split operation pipeline. |  |

Table 6.1.12.6.1-2 specifies data types re-used by the AIMLES_SplitOpEvent API service.

Table 6.1.12.6.1-2: AIMLES_SplitOpEvent API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [3] | Represents the subscription duration. |  |
| Endpoint | 3GPP TS 29.558[19] | Represent the endpoint information of a node. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Identifies the start time and the end time for the validity time. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| UsageInformation | Clause 6.1.12.6.2.6 | Represents the usage information for the split AI/ML model. |  |

##### 6.1.12.6.2	Structured data types

###### 6.1.12.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 6.1.12.6.2.2	Type: SplitOpEventSub

Table 6.1.12.6.2.2-1: Definition of type SplitOpEventSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| splitOpPipelineId | string | M | 1 | Contains the identifier for split operation pipeline. |  |
| notifUri | Uri | M | 1 | Contains the URI, towards which the notification should be delivered. |  |
| reportReq | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| splitOpEventId | SplitOpEventId | M | 1 | Contains the event identifier for subscription. |  |
| discFilters | DiscFilters | O | 0..1 | Contains the set of characteristics to determine matching split operation profiles or nodes. |  |
| assistInfo | AssistanceInfo | O | 0..1 | Contains the assistance information for subscription. |  |
| expTime | DateTime | O | 0..1 | Contains the proposed expiration time of the subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 6.1.12.6.2.3	Type: SplitOpEventSubPatch

Table 6.1.12.6.2.3-1: Definition of type SplitOpEventSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Contains the URI, towards which the notification should be delivered. |  |
| reportReq | ReportingInformation | O | 0..1 | Contains the updated reporting requirements of the subscription. |  |
| splitOpEventId | SplitOpEventId | O | 0..1 | Contains the event identifier for subscription. |  |
| discFilters | DiscFilters | O | 0..1 | Contains the set of characteristics to determine matching split operation profiles or nodes. |  |
| expTime | DateTime | O | 0..1 | Contains the proposed expiration time of the subscription. |  |

###### 6.1.12.6.2.4	Type: SplitOpEventNotif

Table 6.1.12.6.2.4-1: Definition of type SplitOpEventNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| splitOpEventId | SplitOpEventId | M | 1 | Contains the event identifier for subscription. |  |
| availabilityInfo | AvailabilityInfo | C | 0..1 | Contains the availability information. This shall be present for SPLIT_OP_AVAILABILITY event. |  |
| splitOpPipelineInfo | SplitOpPipelineInfo | C | 0..1 | Contains split operation pipeline information. This shall be present for SPLIT_OP_PIPELINE_INFO event. |  |
| assistanceInfo | AssistanceInfo | C | 0..1 | Contains split operation assistance information. This shall be present for SPLIT_OP_ASSISTANCE_INFO event. |  |

###### 6.1.12.6.2.5	Type: DiscFilters

Table 6.1.12.6.2.5-1: Definition of type DiscFilters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| stageInfo | array(StageInfo) | M | N | Contains information about split operation stages. |  |
| usageInfo | UsageInformation | O | 0..1 | Contains information about planned usage of the split operation. |  |
| minNodes | string | O | 0..1 | Contains information about minimum number of nodes required to support AIMl operation splitting. |  |

###### 6.1.12.6.2.6	Type: SplitOpPipelineInfo

Table 6.1.12.6.2.6-1: Definition of type SplitOpPipelineInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| splitOpProfile | SplitOpProfile | O | 0..1 | Contains the split operation profile that service consumer participates to. |  |
| subEventId | SubEventId | O | 0..1 | Indicates the sub event. |  |

###### 6.1.12.6.2.7	Type: AssistanceInfo

Table 6.1.12.6.2.7-1: Definition of type AssistanceInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deliveryTime | TimeWindow | O | 0..1 | Contains the time to deliver the task or data for the split operation. |  |
| achievableQoS | string | O | 0..1 | Contains the achievable QoS for the current configuration for the task or data delivery. |  |
| qosSuggestion | string | O | 0..1 | Contains the suggestion of QoS for the task or data delivery. |  |

###### 6.1.12.6.2.8	Type: AvailabilityInfo

Table 6.1.12.6.2.8-1: Definition of type AvailabilityInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| splitOpProfile | array(SplitOpProfile) | O | 0..N | The list of newly available split operation profiles. |  |
| availableNodes | string | O | 0..1 | The list of newly available nodes. |  |

###### 6.1.12.6.2.9	Type: SplitOpProfile

Table 6.1.12.6.2.9-1: Definition of type SplitOpProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| splitOpPipelineId | string | M | 1 | Contains the identifier for split operation pipeline. |  |
| headEp | Endpoint | M | 1 | Contains the endpoint information of the head node. |  |
| tailEp | Endpoint | M | 1 | Contains the endpoint information of the tail node. |  |
| usageInfo | UsageInformation | O | 0..1 | Contains the usage information of the AIML split operation. |  |
| stageInfo | array(StageInfo) | M | 1 | Contains the stage information for each stage in the split operation pipeline. |  |

###### 6.1.12.6.2.10	Type: StageInfo

Table 6.1.12.6.2.10-1: Definition of type StageInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| stageId | string | M | 1 | Contains the identifier of the stage in split operation pipeline. |  |
| numNodes | string | O | 0..1 | Contains the number of nodes included in the stage. |  |
| headEp | Endpoint | M | 1 | Contains the endpoint of the head node for providing initial inference data. |  |
| tailEp | Endpoint | M | 1 | Contains the endpoint of the tail node for obtaining inference results. |  |
| nodeOrder | array(Endpoint) | M | N | Contains the order of nodes in the AI/ML split operation. |  |
| mlModelId | string | M | 1 | Contains ML model information used in the stage. |  |

##### 6.1.12.6.3	Simple data types and enumerations

6.1.12.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.1.12.6.3.2	Simple data types

The simple data types defined in table 6.1.12.6.3.2-1 shall be supported.

Table 6.1.12.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 6.1.12.6.3.3	Enumeration: SplitOpEventId

Table 6.1.12.6.3.3-1: Enumeration SplitOpEventId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SPLIT_OP_AVAILABILITY | Indicates split operation availability event |  |
| SPLIT_OP_PIPELINE_INFO | Indicates split operation pipeline information event. |  |
| SPLIT_OP_ASSISTANCE_INFO | Indicates split operation assistance information event. |  |

###### 6.1.12.6.3.3	Enumeration: SubEventId

Table 6.1.12.6.3.3-1: Enumeration SubEventId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CREATED | Indicates that a new split operation profile is created. |  |
| UPDATED | Indicates that an existing split operation profile is updated. |  |
| DELETED | Indicates that an existing split operation profile is deleted. |  |

##### 6.1.12.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.12.6.5	Binary data

6.1.12.6.5.1	Binary Data Types

Table 6.1.12.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.12.7	Error Handling

##### 6.1.12.7.1	General

For the AIMLES_SplitOpEvent API, HTTP error responses shall be supported as specified in clause 5.2.1.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.1.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.1.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the AIMLES_SplitOpEvent API.

##### 6.1.12.7.2	Protocol Errors

No specific protocol errors for the AIMLES_SplitOpEvent API are specified.

##### 6.1.12.7.3	Application Errors

The application errors defined for AIMLES_SplitOpEvent API are listed in table 6.1.12.7.3-1.

Table 6.1.12.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.12.8	Feature negotiation

The optional features in table 6.1.12.8-1 are defined for the AIMLES_SplitOpEvent API. They shall be negotiated using the extensibility mechanism defined clause 5.2.1.7 of 3GPP TS 29.122 [2].

Table 6.1.12.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.12.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the AIMLES_SplitOpEvent API.

### 6.1.13	Aimles_MLModelRetrieval API

#### 6.1.13.1	Introduction

The Aimles_MLModelRetrieval Service shall use the Aimles_MLModelRetrieval API.

The API URI of the Aimles_MLModelRetrieval API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-mlmr".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.1.13.3 and clause 6.1.13.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.13, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.13.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the Aimles_MLModelRetrieval API.

#### 6.1.13.3	Resources

##### 6.1.13.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.13.3.1-1 depicts the resource URIs structure for the Aimles_MLModelRetrieval API.

Figure 6.1.13.3.1-1: Resource URI structure of the Aimles_MLModelRetrieval API

Table 6.1.13.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.13.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIMLE ML Model Retrieval Subscriptions | /subscriptions | POST | Request the creation of an AIMLE ML Model Retrieval Subscription resource. |
| Individual AIMLE ML Model Retrieval Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual AIMLE ML Model Retrieval Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIMLE ML Model Retrieval Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIMLE ML Model Retrieval Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual AIMLE ML Model Retrieval Subscription" resource. |

##### 6.1.13.3.2	Resource: AIMLE ML Model Retrieval Subscriptions

6.1.13.3.2.1	Description

This resource represents the collection of AIMLE ML Model Retrieval Subscriptions managed by the AIMLE Server.

6.1.13.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-mlmr/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.13.3.2.2-1.

Table 6.1.13.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.13.1. |

6.1.13.3.2.3	Resource Standard Methods

6.1.13.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual AIMLE ML Model Retrieval Subscription at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.13.3.2.3.1-1.

Table 6.1.13.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.13.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.13.3.2.3.1-3.

Table 6.1.13.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | Create a new Individual AIMLE ML Model Retrieval Subscription resource. |

Table 6.1.13.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | 201 Created | Successful case. The creation of an Individual AIMLE ML Model Retrieval Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.13.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimles-mlmr/<apiVersion>/subscriptions{subscriptionId} |

6.1.13.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.13.3.3	Resource: Individual AIMLE ML Model Retrieval Subscription

6.1.13.3.3.1	Description

6.1.13.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-mlmr/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.13.3.3.2-1.

Table 6.1.13.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.13.1 |
| subscriptionId | string | Represents the identifier of an "Individual AIMLE ML Model Retrieval Subscription" resource. |

6.1.13.3.3.3	Resource Standard Methods

6.1.13.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual AIMLE ML Model Retrieval Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.13.3.3.3.1-1.

Table 6.1.13.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.13.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.13.3.3.3.1-3.

Table 6.1.13.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.13.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | 200 OK | Successful case. The requested "Individual AIMLE ML Model Retrieval Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.13.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.13.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.13.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual AIMLE ML Model Retrieval Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.13.3.3.3.2-1.

Table 6.1.13.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.13.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.13.3.3.3.2-3.

Table 6.1.13.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | Represents the updated representation of the "Individual AIMLE ML Model Retrieval Subscription" resource. |

Table 6.1.13.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE ML Model Retrieval Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Retrieval Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.13.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.13.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.13.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual AIMLE ML Model Retrieval Subscription" resource at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.13.3.3.3.3-1.

Table 6.1.13.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.13.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.13.3.3.3.3-3.

Table 6.1.13.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLMdlRetSubPatch | M | 1 | Represents the parameters to request the modification of the "Individual AIMLE ML Model Retrieval Subscription" resource. |

Table 6.1.13.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLMdlRetSub | M | 1 | 200 OK | Successful case. The "Individual AIMLE ML Model Retrieval Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Retrieval Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.13.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.13.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.13.3.3.3.4	DELETE

The HTTP PATCH method allows a AIMLE service consumer to request the deletion of an existing "Individual AIMLE ML Model Retrieval Subscription" resource.

Table 6.1.13.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.13.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.13.3.3.3.4-3.

Table 6.1.13.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.13.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIMLE ML Model Retrieval Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.13.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.13.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.13.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.13.4	Custom Operations without associated resources

##### 6.1.13.4.1	Overview

The structure of the custom operation URIs of the Aimles_MLModelRetrieval API is shown in Figure 6.1.13.4.1-1.

Figure 6.1.13.4.1-1: Custom operation URI structure of the Aimles_MLModelRetrieval API

Table 6.1.13.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Aimles_MLModelRetrieval API.

Table 6.1.13.4.1-1: Custom operations without associated resources

| Custom Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Retrieve | /retrieve | POST | Enables a service consumer to send AIMLE ML Model retrieval request to the AIMLE server. |

The custom operations shall support the URI variables defined in table 6.1.13.4.1-2.

Table 6.1.13.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.8. |

##### 6.1.13.4.2	Operation: Retrieve

6.1.13.4.2.1	Description

The custom operation enables a service consumer to send AIMLE ML Model retrieval request to the AIMLE server.

6.1.13.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.1.13.4.2.2-1 and 6.1.13.4.2.2-2.

Table 6.1.13.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLMdlRetReq | M | 1 | Contains the parameters to request AIMLE ML Model retrieval. |

Table 6.1.13.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLMdlRetRsp | M | 1 | 200 OK | Successful case. The ML Model retrieval request is successfully received and processed. The identifier of the ML model selected by AIMLE server for retrieval shall be included in the response message. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the <e.g. HTTP POST> method listed in table 5.2.1.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

#### 6.1.13.5	Notifications

##### 6.1.13.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.1.13.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE ML Model Retrieval Notification | {notifUri} | POST | This service operation enables an AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML ML Model Retrieval report(s). |

##### 6.1.13.5.2	AIMLE ML Model Retrieval Notification

6.1.13.5.2.1	Description

The AIMLE ML Model Retrieval Event Notification is used by the AIMLE Server to notify a previously subscribed AIMLE service consumer on AIML ML Model Retrieval report(s).

6.1.13.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.13.5.2.2-1.

Table 6.1.13.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification URI is assigned within the Individual AIMLE ML Model Retrieval Subscription and described within the MLMdlRetSub type |

6.1.13.5.2.3	Standard Methods

6.1.13.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.13.5.2.3-1 and the response data structures and response codes specified in table 6.1.13.5.2.3-2.

Table 6.1.13.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MlMdlRetNotif | M | 1 | Represents the AIMLE ML Model Retrieval Notification. |

Table 6.1.13.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE ML Model Retrieval Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.13.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.1.13.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.1.13.6	Data Model

##### 6.1.13.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.13.6.1-1 specifies the data types defined for the Aimles_MLModelRetrieval API.

Table 6.1.13.6.1-1: Aimles_MLModelRetrieval API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| MLMdlRetReq | 6.1.13.6.2.5 | Represents the AIMLE ML Model Retrieval request information. |  |
| MLMdlRetRsp | 6.1.13.6.2.6 | Represents the AIMLE ML Model Retrieval response information. |  |
| MLMdlRetSub | 6.1.13.6.2.2 | Represents the AIMLE ML Model Retrieval subscription information. |  |
| MLMdlRetSubPatch | 6.1.13.6.2.3 | Represents the requested modifications to the AIMLE ML Model Retrieval subscription information. |  |
| MLMdlRetNotif | 6.1.13.6.2.4 | Represents the AIMLE ML Model Retrieval notification. |  |
| MlModelDetail | 6.1.13.6.2.7 | Represents the ML Model information. |  |

Table 6.1.13.6.1-2 specifies data types re-used by the Aimles_MLModelRetrieval API service.

Table 6.1.13.6.1-2: Aimles_MLModelRetrieval API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [15] | Represents a date and a time. |  |
| EndPoint | 3GPP TS 29.558 [19] | Represents the endpoint information. |  |
| Fqdn | 3GPP TS 29.571 [15] | Represents an FQDN. |  |
| Ipv4Addr | 3GPP TS 29.122 [2] | Represents the IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.122 [2] | Represents the IPv6 address. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |

##### 6.1.13.6.2	Structured data types

6.1.13.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.1.13.6.2.2	Type: MLMdlRetSub

Table 6.1.13.6.2.2-1: Definition of type MLMdlRetSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| notifUri | Uri | M | 1 | Identifies the URI, towards which the notification should be delivered. |  |
| modelSelCrit | MlModelInfo | O | 0..1 | Contains the filtering criteria for selecting the ML model information. It may be included in a subscription request. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the subscription, after which the AIMLE Server shall not send any notifications and the subscription becomes invalid. It may be included in a subscription request and may be included in a subscription response based on operator policies. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. If the expiry time is not included in the response, the NF service consumer shall not associate an expiry time for the subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Identifies the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

Editor's note:	the definition of MlModelInfo data type to be aligned with the MLR_MLModelManagement API.

6.1.13.6.2.3	Type: MLMdlRetSubPatch

Table 6.1.13.6.2.3-1: Definition of type MLMdlRetSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| subId | string | M | 0..1 | Represents a unique identifier corresponding to the subscription |  |
| notifUri | Uri | O | 0..1 | Identifies the URI, towards which the notification should be delivered. |  |
| modelSelCrit | MlModelInfo | O | 0..1 | Contains the filtering criteria for selecting the ML model information. It may be included in a subscription request. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the subscription, after which the AIMLE Server shall not send any notifications and the subscription becomes invalid. It may be included in a subscription request and may be included in a subscription response based on operator policies. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. If the expiry time is not included in the response, the NF service consumer shall not associate an expiry time for the subscription. |  |

6.1.13.6.2.4	Type: MlMdlRetNotif

Table 6.1.13.6.2.4-1: Definition of type MlMdlRetNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 0..1 | Represents a unique identifier corresponding to the subscription |  |
| mlModels | array(MLModelDetail) | O | 0..N | Identifies the type of the retrieved ML Model. |  |

6.1.13.6.2.5	Type: MLMdlRetReq

Table 6.1.13.6.2.5-1: Definition of type MLMdlRetReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer that is sending the request. |  |
| modelSelCrit | MlModelInfo | O | 0..1 | Contains the filtering criteria for selecting the ML model information. It may be included in a subscription request. |  |

6.1.13.6.2.6	Type: MLMdlRetRsp

Table 6.1.13.6.2.6-1: Definition of type MLMdlRetRsp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModels | array(MlModelDetail) | M | 1..N | Contains the list of ML models. |  |

6.1.13.6.2.7	Type: MlModelDetail

Table 6.1.13.6.2.7-1: Definition of type MlModelDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | M | 1 | Represents the ML model identifier. |  |
| mlMdlEndpoint | EndPoint | C | 0..1 | Represents the ML model endpoint. (NOTE) |  |
| mlModel | Bytes | C | 0..1 | Represents the ML model. (NOTE) |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

##### 6.1.13.6.3	Simple data types and enumerations

6.1.13.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.13.6.3.2	Simple data types

The simple data types defined in table 6.1.13.6.3.2-1 shall be supported.

Table 6.1.13.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.13.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.1.13.6.5	Binary data

6.1.13.6.5.1	Binary Data Types

Table 6.1.13.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.13.7	Error Handling

##### 6.1.13.7.1	General

For the Aimles_MLModelRetrieval API, HTTP error responses shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the Aimles_MLModelRetrieval API.

##### 6.1.13.7.2	Protocol Errors

No specific protocol errors for the Aimles_MLModelRetrieval API are specified.

##### 6.1.13.7.3	Application Errors

The application errors defined for Aimles_MLModelRetrieval API are listed in table 6.1.13.7.3-1.

Table 6.1.13.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.1.13.8	Feature negotiation

The optional features in table 6.1.13.8-1 are defined for the Aimles_MLModelRetrieval API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.13.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.13.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the Aimles_MLModelRetrieval API.

### 6.1.14	Aimles_SplitOpNodeRegistration API

#### 6.1.14.1	Introduction

The Aimles_SplitOpNodeRegistration Service shall use the Aimles_SplitOpNodeRegistration API.

The API URI of the Aimles_SplitOpNodeRegistration API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "aimles-sonreg".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.1.14.3 and clause 6.1.14.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.1.14, the AIMLE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.1.14.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the Aimles_SplitOpNodeRegistration API.

#### 6.1.14.3	Resources

##### 6.1.14.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.14.3.1-1 depicts the resource URIs structure for the Aimles_SplitOpNodeRegistration API.

Figure 6.1.14.3.1-1: Resource URI structure of the Aimles_SplitOpNodeRegistration API

Table 6.1.14.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.14.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIMLE Split Operation Node Register Configurations | /configurations | POST | Register a new Individual AIMLE Split Operation Node Register resource |
| Individual AIMLE Split Operation Node Register Configuration | /configurations/{configurationId} | GET | Query an Individual Registered AIMLE Split Operation Node Register resource. |
|  |  | PUT | Update an Individual Registered AIMLE Split Operation Node Register resource. |
|  |  | PATCH | Modify an Individual Registered AIMLE Split Operation Node Register resource. |
|  |  | DELETE | Deregister an Individual Registered AIMLE Split Operation Node Register resource. |

##### 6.1.14.3.2	Resource: AIMLE Split Operation Node Register Configurations

6.1.14.3.2.1	Description

This resource represents the AIMLE Split Operation Node Register Configurations resource managed by the AIMLE Server.

6.1.14.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-sonreg/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 6.1.14.3.2.2-1.

Table 6.1.14.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.14.1 |

6.1.14.3.2.3	Resource Standard Methods

6.1.14.3.2.3.1	POST

The HTTP POST method enables the service consumer to register a AIMLE Split Operation Node Register at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.14.3.2.3.1-1.

Table 6.1.14.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.14.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.14.3.2.3.1-3.

Table 6.1.14.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | Register a new Individual AIMLE Split Operation Node Register. |

Table 6.1.14.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | 201 Created | Successful case. The registration of the new Individual AIMLE Split Operation Node Register is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.14.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimless-sonreg/<apiVersion>/configurations{configurationId} |

6.1.14.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.1.14.3.3	Resource: Individual AIMLE Split Operation Node Register Configuration

6.1.14.3.3.1	Description

This resource represents the individual AIMLE Split Operation Node Register Configuration resource managed by the AIMLE Server.

6.1.14.3.3.2	Resource Definition

Resource URI: {apiRoot}/aimles-sonreg/<apiVersion>/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 6.1.14.3.3.2-1.

Table 6.1.14.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.14.1 |
| configurationId | string | Represents the identifier of the Individual AIMLE Split Operation Node Register Configuration resource. |

6.1.14.3.3.3	Resource Standard Methods

6.1.14.3.3.3.1	GET

The HTTP GET method enables the service consumer e.g., the VAL Server to query an Individual Registered AIMLE Split Operation Node Register at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.14.3.3.3.1-1.

Table 6.1.14.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.14.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.14.3.3.3.1-3.

Table 6.1.14.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.14.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | 200 OK | Successful case. The requested information on the Individual Resgietered AIMLE Split Operation Node Register is confirmed and a representation of that resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.14.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.14.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.14.3.3.3.2	PUT

The HTTP PUT method enables the service consumer e.g., the VAL Server to update an Individual Registered AIMLE Split Operation Node Register at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.14.3.3.3.2-1.

Table 6.1.14.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.14.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.14.3.3.3.2-3.

Table 6.1.14.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | Represents the updated representation of an Individual Registered AIMLE Split Operation Node Register. |

Table 6.1.14.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | 200 OK | Successful case. The requested update of the Individual Registered AIMLE Split Operation Node Register is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested update of the Individual Registered AIMLE Split Operation Node Register is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.14.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.14.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.14.3.3.3.3	PATCH

The HTTP PATCH method enables the service consumer e.g., the VAL Server to modify an Individual Registered AIMLE Split Operation Node Register at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.14.3.3.3.3-1.

Table 6.1.14.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.14.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.14.3.3.3.3-3.

Table 6.1.14.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SplitOpNodeRegPatch | M | 1 | Represents the parameters to modify of an Individual Registered AIMLE Split Operation Node Register. |

Table 6.1.14.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SplitOpNodeReg | M | 1 | 200 OK | Successful case. The requested modification of the Individual Registered AIMLE Split Operation Node Register is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested modification of the Individual Registered AIMLE Split Operation Node Register is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.14.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.14.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.14.3.3.3.4	DELETE

The HTTP DELETE method enables the service consumer e.g., VAL Server to deregister an Individual Registered AIMLE Split Operation Node Register at the AIMLE Server.

This method shall support the URI query parameters specified in table 6.1.14.3.3.3.4-1.

Table 6.1.14.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.14.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.14.3.3.3.4-3.

Table 6.1.14.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.14.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. Deregistration of the Individual Registered AIMLE Split Operation Node Register is confirmed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.14.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.14.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

6.1.14.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.14.4	Custom Operations without associated resources

There is not any custom operation defined for the Aimles_SplitOpNodeRegistration API in this release of the specification.

#### 6.1.14.5	Notifications

##### 6.1.14.5.1	General

There is not any notification defined for the Aimles_SplitOpNodeRegistration API in this release of the specification.

#### 6.1.14.6	Data Model

##### 6.1.14.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.14.6.1-1 specifies the data types defined for the Aimles_SplitOpNodeRegistration API.

Table 6.1.14.6.1-1: Aimles_SplitOpNodeRegistration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SplitOpNodeReg | 6.1.14.6.2.2 | Represents the Split Operation Node Register information. |  |
| SplitOpNodeRegPatch | 6.1.14.6.2.3 | Represents the Split Operation Node Register information to be modified |  |

Table 6.1.14.6.1-2 specifies data types re-used by the Aimles_SplitOpNodeRegistration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimles_SplitOpNodeRegistration API.

Table 6.1.14.6.1-2: Aimles_SplitOpNodeRegistration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ConnInfo | 3GPP TS 29.548 [A] | Represents node information. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Identifies the start time and the end time for the validity time. |  |

##### 6.1.14.6.2	Structured data types

6.1.14.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.1.14.6.2.2	Type: SplitOpNodeReg

Table 6.1.14.6.2.2-1: Definition of type SplitOpNodeReg

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nodeInfo | ConnInfo | M | 1 | Contains the endpoint information of the requestor. |  |
| sonRegCapability | SplitOpCapabilities | M | 1 | Identifies the split operation capabilities of the requestor. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the expiration time of the registration. |  |

6.1.14.6.2.3	Type: SplitOpNodeRegPatch

Table 6.1.14.6.2.3-1: Definition of type SplitOpNodeRegPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sonregCapability | SplitOpCapabilities | O | 0..1 | Identifies the split operation capabilities of the requestor. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the expiration time of the registration. |  |

6.1.14.6.2.4	Type: SplitOpCapabilities

Table 6.1.14.6.2.4-1: Definition of type SplitOpCapabilities

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelInfo | ModelInformation | M | 1 | Identifies the ML model capabilities for split operation. |  |
| usageInfo | UsageInformation | O | 0..1 | Identifies the usage capabilities for split operation. |  |

6.1.14.6.2.5	Type: ModelInformation

Table 6.1.14.6.2.5-1: Definition of type ModelInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelId | string | M | 1 | Identifies the ML model identifier. |  |
| modelName | string | O | 0..1 | Identifies the name of the ML model. |  |
| modelVersion | string | M | 1 | Identifies the version of the ML model. |  |
| suppOperation | array(string) | M | 1..N | Contains the list of supported split operations. |  |

6.1.14.6.2.6	Type: UsageInformation

Table 6.1.14.6.2.6-1: Definition of type UsageInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| inputFreq | Uinteger | O | 0..1 | Contains the input frequency in bytes per second at which data is being fed to the ML model. |  |
| inputSize | Uinteger | O | 0..1 | Contains the size in bytes of input data being fed to the ML model. |  |
| outputFreq | Uinteger | O | 0..1 | Contains the output frequency in bytes per second at which data is being fed to the ML model. |  |
| outputSize | Uinteger | O | 0..1 | Contains the size in bytes of output data from the ML model. |  |

##### 6.1.14.6.3	Simple data types and enumerations

6.1.14.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.1.14.6.3.2	Simple data types

The simple data types defined in table 6.1.14.6.3.2-1 shall be supported.

Table 6.1.14.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.1.14.6.3.3	Enumeration: SuppAimlRoleType

The enumeration SuppAimlRoleType represents information regarding the supported AIML role identity of the AIMLE Split Operation Node Register. It shall comply with the provisions defined in table 6.1.14.6.3.3-1.

Table 6.1.14.6.3.3-1: Enumeration SuppMlTaskType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_CLIENT | Identifies the supported AIML role of AIMLE Split Operation Node Register is used as FL client. |  |
| FL_SERVER | Identifies the supported AIML role of AIMLE Split Operation Node Register is used as FL server. |  |

6.1.14.6.3.4	Enumeration: MLAppType

The enumeration MLAppType represents information regarding the supported ML application related to the capability of the AIMLE Split Operation Node Register. It shall comply with the provisions defined in table 6.1.14.6.3.4-1.

Table 6.1.14.6.3.4-1: Enumeration MLAppType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REINFORCEMENT_LEARNING | Identifies the ML application of reinforcement learning type related to the capability of AIMLE Split Operation Node Register. |  |
| SUPERVISED_LEARNING | Identifies the ML application of supervised learning type related to the capability of AIMLE Split Operation Node Register. |  |
| TRANSFER_LEARNING | Identifies the ML application of transfer learning type related to the capability of AIMLE Split Operation Node Register. |  |

6.1.14.6.3.5	Enumeration: AvailabilityType

The enumeration AvailabilityType represents information regarding the availability of the AIMLE Split Operation Node Register. It shall comply with the provisions defined in table 6.1.14.6.3.5-1.

Table 6.1.14.6.3.-1: Enumeration AvailabilityType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AVAILABLE | Identifies the AIMLE Split Operation Node Register is available. |  |
| NOT_AVAILABLE | Identifies the AIMLE Split Operation Node Register is not available. |  |

##### 6.1.14.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combination of data types for Aimles_SplitOpNodeRegistration API in this release of the specification.

##### 6.1.14.6.5	Binary data

6.1.14.6.5.1	Binary Data Types

The binary data types defined for the Aimles_SplitOpNodeRegistration API are listed in Table 6.1.14.6.5.1-1.

Table 6.1.14.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.1.14.7	Error Handling

##### 6.1.14.7.1	General

For the Aimles_SplitOpNodeRegistration API, HTTP error responses shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the Aimles_SplitOpNodeRegistration API.

##### 6.1.14.7.2	Protocol Errors

No specific procedures for the Aimles_SplitOpNodeRegistration API are specified.

##### 6.1.14.7.3	Application Errors

The application errors defined for the Aimles_SplitOpNodeRegistration API are listed in Table 6.1.14.7.3-1.

Table 6.1.14.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

#### 6.1.14.8	Feature negotiation

The optional features in table 6.1.14.8-1 are defined for the Aimles_SplitOpNodeRegistration API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.14.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.1.14.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the Aimles_SplitOpNodeRegistration API.

## 6.2	AIMLE Repository APIs

### 6.2.1	MLR_MLModelManagement Service API

#### 6.2.1.1	Introduction

The MLR_MLModelManagement service shall use the MLR_MLModelManagement API.

The API URI of the MLR_MLModelManagement Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "mlr-mlmm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2.1, the AIMLE Repository takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.2.1.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the MLR_MLModelManagement API.

#### 6.2.1.3	Resources

##### 6.2.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.1.3.1-1 depicts the resource URIs structure for the MLR_MLModelManagement API.

Figure 6.2.1.3.1-1: Resource URIs structure of the MLR_MLModelManagement API

Table 6.2.1.3.1-1 provides an overview of the resources and applicable HTTP methods for the MLR_MLModelManagement API.

Table 6.2.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| ML Models Storages | /storages | GET | Retrieve one or several existing ML Models Storage(s). |
|  |  | POST | Request the creation of a ML Models Storage. |
| Individual ML Models Storage | /storages/{storageId} | GET | Retrieve an existing ML Models Storage. |
|  |  | PUT | Request the update of an existing "Individual ML Models Storage" resource. |
|  |  | PATCH | Request the modification of an existing "Individual ML Models Storage" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual ML Models Storage" resource. |

##### 6.2.1.3.2	Resource: ML Models Storages

###### 6.2.1.3.2.1	Description

This resource represents the collection of ML Models Storage(s) managed by the AIMLE Repository.

###### 6.2.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/mlr-mlmm/<apiVersion>/storages

This resource shall support the resource URI variables defined in table 6.2.1.3.2.2-1.

Table 6.2.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1.1. |

###### 6.2.1.3.2.3	Resource Standard Methods

6.2.1.3.2.3.1	GET

The HTTP GET method allows a service consumer to retrieve one or several existing "Individual ML Models Storage" resource(s) managed by the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.2.3.1-1.

Table 6.2.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name |  | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- | --- |
| storage-ids |  | array(string) | O | 1..N | Contains the identifier(s) of the targeted ML Models Storage resource(s). (NOTE) |  |
| profile-ids | array(string) |  | O | 1..N | Contains the identifier(s) of the profile(s) of the targeted stored ML Model(s). (NOTE) |  |
| supp-feats |  | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.1.8. This query parameter shall be present only when feature negotiation is required. |  |
| NOTE: These query parameters are mutually exclusive and only one of them may be present. |  |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.1.3.2.3.1-3.

Table 6.2.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(MLModelsStorage) | M | 0..N | 200 OK | Successful case. The requested "Individual ML Models Storage" resource(s) shall be returned. If there are no available "Individual ML Models Storage" resource(s) fulfilling the request, an empty array shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

Table 6.2.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

6.2.1.3.2.3.2	POST

The HTTP POST method allows a service consumer to request the creation of a ML Models Storage at the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.2.3.2-1.

Table 6.2.1.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.2.1.3.2.3.2-3.

Table 6.2.1.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLModelsStorage | M | 1 | Represents the parameters to request the creation of a ML Models Storage resource. |

Table 6.2.1.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLModelsStorage | M | 1 | 201 Created | Successful case. The ML Models Storage is successfully created and a representation of the created "Individual ML Models Storage" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.2.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/mlr-mlmm/<apiVersion>/storages/{storageId} |

###### 6.2.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.2.1.3.3	Resource: Individual ML Models Storage

###### 6.2.1.3.3.1	Description

This resource represents an "Individual ML Models Storage" resource managed by the AIMLE Repository.

###### 6.2.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/mlr-mlmm/<apiVersion>/storages/{storageId}

This resource shall support the resource URI variables defined in table 6.2.1.3.3.2-1.

Table 6.2.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1.1. |
| storageId | string | Represents the identifier of the "Individual ML Models Storage" resource. |

###### 6.2.1.3.3.3	Resource Standard Methods

6.2.1.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual ML Models Storage" resource at the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.3.3.1-1.

Table 6.2.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.1.3.3.3.1-3.

Table 6.2.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLModelsStorage | M | 1 | 200 OK | Successful case. The requested "Individual ML Models Storage" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

Table 6.2.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

6.2.1.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual ML Models Storage" resource at the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.3.3.2-1.

Table 6.2.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.1.3.3.3.2-3.

Table 6.2.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLModelsStorage | M | 1 | Represents the updated representation of the "Individual ML Models Storage" resource. |

Table 6.2.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLModelsStorage | M | 1 | 200 OK | Successful case. The "Individual ML Models Storage" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual ML Models Storage" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

Table 6.2.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

6.2.1.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual ML Models Storage" resource at the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.3.3.3-1.

Table 6.2.1.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.1.3.3.3.3-3.

Table 6.2.1.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MLModelsStoragePatch | M | 1 | Represents the parameters to request the modification of the "Individual ML Models Storage" resource. |

Table 6.2.1.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MLModelsStorage | M | 1 | 200 OK | Successful case. The "Individual ML Models Storage" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual ML Models Storage" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

Table 6.2.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

6.2.1.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual ML Models Storage" resource at the AIMLE Repository.

This method shall support the URI query parameters specified in table 6.2.1.3.3.3.4-1.

Table 6.2.1.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.1.3.3.3.4-3.

Table 6.2.1.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.1.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual ML Models Storage" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

Table 6.2.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Repository. |

###### 6.2.1.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 6.2.1.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.2.1.6	Data Model

##### 6.2.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.1.6.1-1 specifies the data types defined for the SDD_DataStorage API.

Table 6.2.1.6.1-1: SDD_DataStorage API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AccessReqs | 6.2.1.6.3.6 | Represents the ML Model access requirements. |  |
| DataFreshness | 6.2.1.6.3.7 | Represents the data freshness level. |  |
| MLModel | 6.2.1.6.2.4 | Represents a ML Model. |  |
| MLModelAccessReqs | 6.2.1.6.2.10 | Represents ML Model access requirements. |  |
| MLModelDomain | 6.2.1.6.3.3 | Represents the domain of the ML Model. |  |
| MLModelPhase | 6.2.1.6.3.4 | Represents the ML Model phase. |  |
| MLModelPerf | 6.2.1.6.2.8 | Represents the performance information of the ML Model. |  |
| MLModelPhaseInfo | 6.2.1.6.2.5 | Represents the ML Model phase related information. |  |
| MLModelProfile | 6.2.1.6.2.3 | Represents the ML Model Profile. |  |
| MLModelStoreDiscReqs | 6.2.1.6.2.6 | Represents the ML Model storage and discovery requirements. |  |
| MLModelTrainingInfo | 6.2.1.6.2.9 | Represents the ML Model training information. |  |
| MLModelUsage | 6.2.1.6.3.5 | Represents the ML Model usage. |  |
| MLModelUsageReqs | 6.2.1.6.2.7 | Represents the ML Model usage requirements. |  |
| MLModelsStorage | 6.2.1.6.2.2 | Represents a ML Models Storage. |  |
| MLModelsStoragePatch | 6.2.1.6.2.11 | Represents the requested modifications to a ML Models Storage. |  |

Table 6.2.1.6.1-2 specifies data types re-used by the SDD_DataStorage API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_DataStorage API.

Table 6.2.1.6.1-2: SDD_DataStorage API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DurationSec | 3GPP TS 29.122 [2] | Represents a time duration in seconds. |  |
| EndPoint | 3GPP TS 29.558 [19] | Represent an endpoint. |  |
| LocationArea5G | 3GPP TS 29.122 [2] | Represents a location. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| Volume | 3GPP TS 29.122 [2] | Represents a data volume. |  |

##### 6.2.1.6.2	Structured data types

###### 6.2.1.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 6.2.1.6.2.2	Type: MLModelsStorage

Table 6.2.1.6.2.2-1: Definition of type DataStorage

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModels | array(MLModelProfile) | C | 1..N | Contains the profile(s) of the ML Model(s). (NOTE) |  |
| mlModelsAddresses | array(EndPoint) | C | 1..N | Contains the address(es) (e.g., URI(s)) via which the ML Model(s) shall be retrieved. This attribute may be present only in the initial request to store ML Model(s) for the first time, i.e., it shall be used only to retrieve the ML Model(s) related information and thus not be present in the "ML Models Storage" resource representation. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.1.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.3	Type: MLModelProfile

Table 6.2.1.6.2.3-1: Definition of type MLModelProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelProfId | string | C | 0..1 | Contains the identifier of the ML Model profile. This value of this attribute is assigned by the AIMLE Repository and shall hence not be present in the initial request to store the ML Model for the first time. |  |
| aimleServId | string | C | 0..1 | Contains the identifier of the service consumer that stored the ML Model. This attribute shall be added by the AIMLE Repository and shall hence not be present in the initial request to store the ML Model for the first time. |  |
| aimleRepId | string | C | 0..1 | Contains the identifier of the AIMLE Repository where the ML Model is stored. This attribute shall be added by the AIMLE Repository and shall hence not be present in the initial request to store the ML Model for the first time. |  |
| mlModelInfo | MLModel | M | 1 | Contains the ML Model related information. |  |
| mlModelUri | EndPoint | O | 0..1 | Contains the endpoint information via which the ML Model profile can be retrieved. |  |

###### 6.2.1.6.2.4	Type: MLModel

Table 6.2.1.6.2.4-1: Definition of type MLModel

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | M | 1 | Contains the identifier of the ML Model. |  |
| adaeAnalyticsId | string | O | 0..1 | Contains the ADAE analytics ID with which the ML Model can be used. |  |
| mlModelSize | Volume | O | 0..1 | Contains the size of the ML Model. |  |
| mlModelSrcId | string | O | 0..1 | Contains the identifier of the source entity that stored the ML Model. This attribute shall be added by the AIMLE Repository and shall hence not be present in the initial request to store the ML Model for the first time. |  |
| valServiceIds | array(string) | O | 1..N | Contains the identifier(s) of the VAL service(s) for which the ML Model applies. |  |
| domain | MLModelDomain | O | 0..1 | Contains the domain of the ML Model. |  |
| customDomain | string | O | 0..1 | Contains the customized domain information of the ML Model. This attribute may be present only if the "domain" attribute is present and set to "CUSTOM". |  |
| vendors | array(string) | C | 1..N | Contains the vendor(s) that are allowed to use (and thus interoperable with) the ML Model. (NOTE) |  |
| interopInfo | string | C | 0..1 | Contains the ML Model interoperability information. (NOTE) |  |
| phaseInfo | MLModelPhaseInfo | C | 0..1 | Contains the ML Model interoperability information. (NOTE) |  |
| storeDiscReqs | MLModelStoreDiscReqs | C | 0..1 | Contains the ML Model storage and discovery requirements. (NOTE) |  |
| usageReqs | MLModelUsageReqs | O | 0..1 | Contains the ML Model usage requirements. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.5	Type: MLModelPhaseInfo

Table 6.2.1.6.2.5-1: Definition of type MLModelPhaseInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| phase | MLModelPhase | M | 1 | Contains the ML Model phase. |  |
| observedPerf | MLModelPerf | O | 0..1 | Contains the observed performance of the ML Model. This attribute may be present only if the "phase" attribute is set to "TRAINED" or "DEPLOYED". |  |
| trainingInfo | MLModelTrainingInfo | O | 0..1 | Contains information on the data that has been used to train the ML Model. This attribute may be present only if the "phase" attribute is set to "TRAINED" or "DEPLOYED". |  |
| contTrainInd | boolean | O | 0..1 | Indicates whether the ML Model can be trained continuously or not. - "true": Indicates that the ML Model can be trained continuously. - "false": Indicates that the ML Model cannot be trained continuously. - The default value is "false" when this attribute is omitted. |  |
| contTrainParams | string | O | 0..1 | Contains the parameters required for continuous model training. This attribute may be present only if the "contTrainInd" attribute is present and set to "true". |  |

###### 6.2.1.6.2.6	Type: MLModelStoreDiscReqs

Table 6.2.1.6.2.6-1: Definition of type MLModelStoreDiscReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| duration | DurationSec | C | 0..1 | Contains the duration of the ML Model storage, i.e., the time duration after which the ML Model storage shall be deleted. (NOTE) |  |
| accessReqs | MLModelAccessReqs | C | 0..1 | Contains the access requirements of the ML Model. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.7	Type: MLModelUsageReqs

Table 6.2.1.6.2.7-1: Definition of type MLModelUsageReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| usages | array(MLModelUsage) | M | 1..N | Contains the usage(s) of the ML Model. |  |

###### 6.2.1.6.2.8	Type: MLModelPerf

Table 6.2.1.6.2.8-1: Definition of type MLModelPerf

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceAddr | integer | C | 0..1 | Contains the accuracy level of the ML Model, expressed as a percentage. Minimum: 0, Maximum: 100. (NOTE) |  |
| appSpecPerfData | string | C | 0..1 | Contains application-specific performance metrics of the ML Model. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.9	Type: MLModelTrainingInfo

Table 6.2.1.6.2.9-1: Definition of type MLModelTrainingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataSources | string | C | 0..1 | Contains information on the data sources used to train the ML Model. (NOTE) |  |
| dataVolume | Volume | C | 0..1 | Contains the data volume used to train the ML Model. (NOTE) |  |
| freshness | DataFreshness | C | 0..1 | Contains the freshness level of the data used to train the ML Model. (NOTE) |  |
| baseModelId | string | C | 0..1 | Contains the identifier of the base model. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.10	Type: MLModelAccessReqs

Table 6.2.1.6.2.10-1: Definition of type MLModelAccessReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accessReqs | array(AccessReqs) | M | 1 | Contains the ML Model access requirements. |  |
| valServerIds | array(string) | O | 1..N | Contains the identifier(s) of the VAL Server(s) that are allowed to access and use the ML Model. This attribute may be present only if the "accessReqs" attribute is set to "RESTRICTED". (NOTE) |  |
| valClientIds | array(string) | O | 1..N | Contains the identifier(s) of the VAL Client(s) that are allowed to access and use the ML Model. This attribute may be present only if the "accessReqs" attribute is set to "RESTRICTED". (NOTE) |  |
| timePeriod | DurationSec | O | 0..1 | Contains the time period during which the ML Model can be accessed and used. This attribute may be present only if the "accessReqs" attribute is set to "RESTRICTED". (NOTE) |  |
| location | LocationArea5G | O | 0..1 | Contains the location/area within which the ML Model can be accessed and used. This attribute may be present only if the "accessReqs" attribute is set to "RESTRICTED". (NOTE) |  |
| NOTE: When the "accessReqs" attribute is set to "RESTRICTED", then at least one of these attributes shall be present. |  |  |  |  |  |

###### 6.2.1.6.2.11	Type: MLModelsStoragePatch

Table 6.2.1.6.2.11-1: Definition of type DataStoragePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModels | array(MLModelProfile) | O | 1..N | Contains the profile(s) of the ML Model(s). |  |
| mlModelsAddresses | array(EndPoint) | O | 1..N | Contains the address(es) (e.g., URI(s)) via which the ML Model(s) shall be retrieved. |  |

##### 6.2.1.6.3	Simple data types and enumerations

###### 6.2.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.2.1.6.3.2	Simple data types

The simple data types defined in table 6.2.1.6.3.2-1 shall be supported.

Table 6.2.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 6.2.1.6.3.3	Enumeration: MLModelDomain

The enumeration MLModelDomain represents the domain of the ML Model. It shall comply with the provisions defined in table 6.2.1.6.3.3-1.

Table 6.2.1.6.3.3-1: Enumeration MLModelDomain

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SPEECH_RECOGNITION | Indicates that the domain of the ML Model is speech recognition. |  |
| IMAGE_RECOGNITION | Indicates that the domain of the ML Model is image recognition. |  |
| IMAGE_PROCESSING | Indicates that the domain of the ML Model is image processing. |  |
| LOCATION_PREDICTION | Indicates that the domain of the ML Model is location prediction. |  |
| CUSTOM | Indicates that the domain of the ML Model is a custom domain. |  |

###### 6.2.1.6.3.4	Enumeration: MLModelPhase

The enumeration MLModelPhase represents the ML Model phase. It shall comply with the provisions defined in table 6.2.1.6.3.4-1.

Table 6.2.1.6.3.4-1: Enumeration MLModelPhase

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NOT_TRAINED | Indicates that the ML Model phase is that ML Model is not trained. |  |
| IN_TRAINING | Indicates that the ML Model phase is that the ML Model is in training. |  |
| TRAINED | Indicates that the ML Model phase is that the ML Model is trained. |  |
| IN_RETRAINING | Indicates that the ML Model phase is that the ML Model is in re-training. |  |
| DEPLOYED | Indicates that the ML Model phase is that the ML Model is deployed. |  |

###### 6.2.1.6.3.5	Enumeration: MLModelUsage

The enumeration MLModelUsage represents the ML Model usage. It shall comply with the provisions defined in table 6.2.1.6.3.5-1.

Table 6.2.1.6.3.5-1: Enumeration MLModelUsage

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRAINING | Indicates that the ML Model usage is training. |  |
| INFERENCE | Indicates that the ML Model usage is inference. |  |

###### 6.2.1.6.3.6	Enumeration: AccessReqs

The enumeration AccessReqs represents the ML Model access requirements. It shall comply with the provisions defined in table 6.2.1.6.3.6-1.

Table 6.2.1.6.3.6-1: Enumeration AccessReqs

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PUBLICLY_AVAILABLE | Indicates that the ML Model is publicly available. |  |
| RESTRICTED | Indicates that the ML Model is restricted. |  |
| PRIVATE_USE_ONLY | Indicates that the ML Model is for private use only. |  |

###### 6.2.1.6.3.7	Enumeration: DataFreshness

The enumeration DataFreshness represents the data freshness level. It shall comply with the provisions defined in table 6.2.1.6.3.7-1.

Table 6.2.1.6.3.7-1: Enumeration DataFreshness

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REAL_TIME | Indicates that the data freshness level is real-time. |  |
| NEAR_REAL_TIME | Indicates that the data freshness level is near real-time. |  |
| HOURLY | Indicates that the data freshness level is hourly. |  |
| DAILY | Indicates that the data freshness level is daily. |  |
| WEEKLY | Indicates that the data freshness level is weekly. |  |
| MONTHLY | Indicates that the data freshness level is monthly. |  |

##### 6.2.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.2.1.6.5	Binary data

###### 6.2.1.6.5.1	Binary Data Types

Table 6.2.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.2.1.7	Error Handling

##### 6.2.1.7.1	General

For the MLR_MLModelManagement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the MLR_MLModelManagement API.

##### 6.2.1.7.2	Protocol Errors

No specific protocol errors for the MLR_MLModelManagement API are specified.

##### 6.2.1.7.3	Application Errors

The application errors defined for the MLR_MLModelManagement API are listed in Table 6.2.1.7.3-1.

Table 6.2.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.1.8	Feature negotiation

The optional features listed in table 6.2.1.8-1 are defined for the MLR_MLModelManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.2.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.2.1.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the MLR_MLModelManagement API.

### 6.2.2	MLR_ModelInformationDiscovery API

#### 6.2.2.1	Introduction

The MLR_ModelInformationDiscovery service shall use the MLR_ModelInformationDiscovery API.

The API URI of the MLR_ModelInformationDiscovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "mlr-mid".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 6.2.2.3 and 6.2.2.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2.2, the ML Repository takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.2.2.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the MLR_ModelInformationDiscovery API.

#### 6.2.2.3	Resources

##### 6.2.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.2.3.1-1 depicts the resource URIs structure for the MLR_ModelInformationDiscovery Service API.

Figure 6.2.2.3.1-1: Resource URI structure of the MLR_ModelInformationDiscovery Service API

Table 6.2.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ML Models | /models | GET | Discover the ML model information according to the filtering criteria. |

##### 6.2.2.3.2	Resource: ML Models

###### 6.2.2.3.2.1	Description

The "ML Models" resource represents the ML Models.

###### 6.2.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/mlr-mid/<apiVersion>/models

This resource shall support the resource URI variables defined in the table 6.2.2.3.2.2-1.

Table 6.2.2.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [14]. |

###### 6.2.2.3.2.3	Resource Standard Methods

6.2.2.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.2.2.3.2.3.1-1.

Table 6.2.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| filt-criteria | MLModel | M | 1 | Represents the ML model filtering criteria. |
| supported-features | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |

This method shall support the request data structures specified in table 6.2.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.2.3.2.3.1-3.

Table 6.2.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DiscoveryResp | M | 1 | 200 OK | Successful case. The response body contains the result of the search over the list of stored ML Models. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MLR. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative MLR. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MLR. |

Table 6.2.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MLR. |

###### 6.2.2.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.2.4	Custom Operations without associated resources

There are no custom operations without associated resources in the present release of the document.

#### 6.2.2.5	Notifications

There are no notifications in the present release of the document.

#### 6.2.2.6	Data Model

##### 6.2.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.2.6.1-1 specifies the data types defined for the MLR_ModelInformationDiscovery API.

Table 6.2.2.6.1-1: MLR_ModelInformationDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DiscoveryResp | 6.2.2.6.2.2 | Represents the ML model discovery response. |  |
| MlModel | 6.2.2.6.2.3 | Represents the ML model. |  |

Table 6.2.2.6.1-2 specifies data types re-used by the MLR_ModelInformationDiscovery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MLR_ModelInformationDiscovery API.

Table 6.2.2.6.1-2: MLR_ModelInformationDiscovery API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| MLModel | Clause 6.2.1.6.2.4 | Represents the ML model information. |  |
| MLModelProfile | Clause 6.2.1.6.2.3 | Represents the ML model profile. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the supported features, used to negotiate the supported optional features of the API. |  |
| Uri | 3GPP TS 29.122 [2] | Represent an URI, used to indicate the notification URI. |  |

##### 6.2.2.6.2	Structured data types

###### 6.2.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.2.2.6.2.2	Type: DiscoveryResp

Table 6.2.2.6.2.2-1: Definition of type DiscoveryResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profiles | array(MLModelProfile) | C | 1..N | Contains the list of ML model profiles. (NOTE) |  |
| mlModels | array(MlModel) | C | 1..N | Contains the list of ML models. (NOTE) |  |
| indicator | boolean | O | 0..1 | Represents the continuously training indicator. - "true": Indicates that the ML Model can be trained continuously. - "false": Indicates that the ML Model cannot be trained continuously. - The default value is "false" when this attribute is omitted. |  |
| NOTE: One of these attributes shall be provided. |  |  |  |  |  |

###### 6.2.2.6.2.3	Type: MlModel

Table 6.2.2.6.2.3-1: Definition of type MlModel

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlModelId | string | M | 1 | Represents the ML model identifier. |  |
| mlModel | Bytes | O | 0..1 | Represents the ML model. |  |

##### 6.2.2.6.3	Simple data types and enumerations

###### 6.2.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 6.2.2.6.3.2	Simple data types

None.

###### 6.2.2.6.3.3	Enumerations

There are no enumerations in this release of the specification.

##### 6.2.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types and combinations of data types in this release of the specification.

##### 6.2.2.6.5	Binary data

There are no binary data defined in this release of the specification.

#### 6.2.2.7	Error Handling

##### 6.2.2.7.1	General

For the MLR_ModelInformationDiscovery API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the MLR_ModelInformationDiscovery API.

##### 6.2.2.7.2	Protocol Errors

No specific procedures for the MLR_ModelInformationDiscovery API are specified.

##### 6.2.2.7.3	Application Errors

The application errors defined for MLR_ModelInformationDiscovery API are listed in table 6.2.2.7.3-1.

Table 6.2.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.2.8	Feature Negotiation

The optional features in table 6.2.2.8-1 are defined for the MLR_ModelInformationDiscovery API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.2.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.2.2.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the MLR_ModelInformationDiscovery API.

### 6.2.3	MLR_FLEvents API

#### 6.2.3.1	Introduction

The MLR_FLEvents Service shall use the MLR_FLEvents API.

The API URI of the MLR_FLEvents API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "mlr-fle".

-	The <apiVersion> shall be "v1".

-	-	The <apiSpecificSuffixes> shall be set as described in clause 6.2.3.3 and clause 6.2.3.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2.3, the ML Repository takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.2.3.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the MLR_FLEvents API.

#### 6.2.3.3	Resources

##### 6.2.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.3.1-1 depicts the resource URIs structure for the MLR_FLEvents API.

Figure 6.2.3.3.1-1: Resource URI structure of the MLR_FLEvents API

Table 6.2.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| MLR FL Events Subscriptions | /subscriptions | POST | Request the creation of an MLR FL Events Subscription resource. |
| Individual MLR FL Events Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual MLR FL Events Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual MLR FL Events Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual MLR FL Events Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual MLR FL Events Subscription" resource. |

##### 6.2.3.3.2	Resource: MLR FL Events Subscriptions

6.2.3.3.2.1	Description

This resource represents the collection of MLR FL Events Subscriptions managed by the ML Repository.

6.2.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/mlr-fle/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.2.3.3.2.2-1.

Table 6.2.3.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.3.1. |

6.2.3.3.2.3	Resource Standard Methods

6.2.3.3.2.3.1	POST

The HTTP POST method enables a AIMLE service consumer to request the creation of a new Individual MLR FL Events Subscription at the ML Repository.

This method shall support the URI query parameters specified in table 6.2.3.3.2.3.1-1.

Table 6.2.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.2.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.2.3.1-3.

Table 6.2.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlEvtsSub | M | 1 | Create a new Individual MLR FL Events Subscription resource. |

Table 6.2.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlEvtsSub | M | 1 | 201 Created | Successful case. The creation of an Individual MLR Fl Events Subscription resource is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/mlr-fle/<apiVersion>/subscriptions{subscriptionId} |

6.2.3.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.2.3.3.3	Resource: Individual MLR FL Events Subscription

6.2.3.3.3.1	Description

6.2.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/mlr-fle/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.3.1 |
| subscriptionId | string | Represents the identifier of an "Individual MLR FL Events Subscription" resource. |

6.2.3.3.3.3	Resource Standard Methods

6.2.3.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing " Individual MLR FL Events Subscription" resource at the ML Repository.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3.1-1.

Table 6.2.3.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.3.1-3.

Table 6.2.3.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlEvtsSub | M | 1 | 200 OK | Successful case. The requested "Individual MLR FL Events Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

Table 6.2.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

6.2.3.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual MLR FL Events Subscription" resource at the ML Repository.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3.2-1.

Table 6.2.3.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.3.2-3.

Table 6.2.3.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlEvtsSub | M | 1 | Represents the updated representation of the "Individual MLR FL Events Subscription" resource. |

Table 6.2.3.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlEventsSub | M | 1 | 200 OK | Successful case. The "Individual MLR FL Events Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual MLR FL Events Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

Table 6.2.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

6.2.3.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual MLR FL Events Subscription" resource at the ML Repository.

This method shall support the URI query parameters specified in table 6.2.3.3.3.3.3-1.

Table 6.2.3.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3.3-3.

Table 6.2.3.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlEvtsSubPatch | M | 1 | Represents the parameters to request the modification of the "Individual MLR FL Events Subscription" resource. |

Table 6.2.3.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlEvtsSub | M | 1 | 200 OK | Successful case. The "Individual MLR FL Events Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual MLR FL Events Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

Table 6.2.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

6.2.3.3.3.3.4	DELETE

The HTTP PATCH method allows a AIMLE service consumer to request the deletion of an existing "Individual MLR FL Events Subscription" resource.

Table 6.2.3.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.3.4-3.

Table 6.2.3.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual MLR FL Events Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML Repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.3.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

Table 6.2.3.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML Repository. |

6.2.3.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 6.2.3.5	Notifications

##### 6.2.3.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [14].

Table 6.2.3.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| MLR FL Events Event Notification | {notifUri} | POST | This service operation enables an ML Repository to notify a previously subscribed AIMLE service consumer on MLR FL Events related event(s). |

##### 6.2.3.5.2	MLR FL Events Event Notification

6.2.3.5.2.1	Description

The MLR FL Events Event Notification is used by the ML Repository to notify a previously subscribed AIMLE service consumer on MLR FL Events related event(s).

6.2.3.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.2.3.5.2.2-1.

Table 6.2.3.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | The Notification URI is assigned within the Individual MLR FL Events Subscription and described within the FlEvtsSub type |

6.2.3.5.2.3	Standard Methods

6.2.3.5.2.3.1	POST

This method shall support the request data structures specified in table 6.2.3.5.2.3-1 and the response data structures and response codes specified in table 6.2.3.5.2.3-2.

Table 6.2.3.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlEvtsNotif | M | 1 | Represents the AIMLE ML Model Performance Monitor Event Notification. |

Table 6.2.3.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The MLR FL Events Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative AIMLE service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.2.3.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

Table 6.2.3.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative AIMLE service consumer towards which the notification should be redirected. |

#### 6.2.3.6	Data Model

##### 6.2.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.3.6.1-1 specifies the data types defined for the MLR_FLEvents API.

Table 6.2.3.6.1-1: MLR_FLEvents API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| FlEvtsSub | 6.2.3.6.2.2 | Represents the MLR FL Events subscription information. |  |
| FlEvtsSubPatch | 6.2.3.6.2.3 | Represents the requested modifications to the MLR FL Events subscription information. |  |
| FlEvtsSubNotif | 6.2.3.6.2.4 | Represents the MLR FL Events notification. |  |
| FlMbrInfo | 6.2.3.6.2.5 | Represents the FL member information. |  |
| EvtInfo | 6.2.3.6.2.6 | Represents the event information. |  |
| EvtType | 6.2.3.6.2.7 | Represent the event type. |  |
| FlMdlInfo | 6.2.3.6.2.8 | Represents FL modle information. |  |
| EvtContent | 6.2.3.6.2.9 | Represents event content. |  |
| FlMbrType | 6.2.3.6.3.3 | Represent FL member type. |  |
| AvailFlMbr | 6.2.3.6.3.4 | Represent FL member presence and availability. |  |

Table 6.2.3.6.1-2 specifies data types re-used by the MLR_FLEvents API service.

Table 6.2.3.6.1-2: MLR_FLEvents API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [2] | Used to indicate the notification URI. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Identifies the start time and the end time for the validity time. |  |
| LocationArea | 3GPP TS 29.122 [2] | Identifies the location area. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [2] | Represents an offered scheduled communication time. |  |

##### 6.2.3.6.2	Structured data types

6.2.3.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

6.2.3.6.2.2	Type: FlEvtsSub

Table 6.2.3.6.2.2-1: Definition of type FlEvtsSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrInfo | FlMbrInfo | M | 1 | Identifies the FL member information, for which the Individual FL Event is requested. |  |
| notifUri | Uri | M | 1 | Identifies the URI, towards which the notification should be delivered. |  |
| mlMdlInfo | TBD | M | 1 | Identifies the ML model information, for which the Individual FL Event is requested. |  |
| evtInfo | EvtInfo | M | 1 | Identifies the event information, for which the Individual FL Event is requested. |  |
| timeValidity | TimeWindow | M | 1 | Identifies the time of interest, for which the Individual FL Events is requested. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Identifies the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

Editor's Note:	Data types for flMdlInfo is FFS and is aligned with ML model management as defined in 3GPP TS 23.482 [13].

6.2.3.6.2.3	Type: FlEvtsSubPatch

Table 6.2.3.6.2.3-1: Definition of type FlEvtsSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrInfo | FlMbrInfo | O | 0..1 | Identifies the FL member information, for which the Individual FL Event is requested. |  |
| notifUri | Uri | O | 0..1 | Identifies the URI, towards which the notification should be delivered. |  |
| mlMdlInfo | TBD | O | 0..1 | Identifies the ML model information, for which the Individual FL Event is requested. |  |
| evtInfo | EvtInfo | O | 0..1 | Identifies the event information, for which the Individual FL Event is requested. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the time of interest, for which the Individual FL Events is requested. |  |

Editor's Note:	Data types for flMdlInfo is FFS and is aligned with ML model management as defined in 3GPP TS 23.482 [13].

6.2.3.6.2.4	Type: FlEvtsNotif

Table 6.2.3.6.2.4-1: Definition of type FlEvtsNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| evtId | string | M | 1 | Identifies the identity of the event. |  |
| evtTime | Uinteger | M | 1 | Identifies the time of interest, for which the event applies. |  |
| evtArea | LocationArea | O | 0..1 | Identifies the location of interest, for which the event applies. |  |
| evtContentList | array (EvtContent) | M | 1..N | Identifies the list of one or more Event Contents. |  |

6.2.3.6.2.5	Type: FlMbrInfo

Table 6.2.3.6.2.5-1: Definition of type FlMbrInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrType | FlMbrType | C | 0..1 | Identifies the type of the FL member, for which the individual FL Event is requested. (NOTE) |  |
| flMbrId | string | C | 0..1 | Identifies the identity of the FL member, for which the individual FL Event is requested. (NOTE) |  |
| NOTE: At least one of these shall be present. |  |  |  |  |  |

6.2.3.6.2.6	Type: EvtInfo

Table 6.2.3.6.2.6-1: Definition of type EvtInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| evtId | string | O | 0..1 | Identifies the identity of the event, for which the individual FL Event is requested. |  |
| evtType | EvtType | M | 1 | Identifies the type of the event, for which the individual FL Event is requested. |  |

6.2.3.6.2.7	Type: EvtType

Table 6.2.3.6.2.7-1: Definition of type EvtType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| availFlMmbr | AvailFlMbr | M | 1 | Identifies the availability of the FL member. |  |
| flMdlInfo | FlMdlInfo | M | 1 | Identifies the information of the FL model. |  |
| flMbrLdInfo | Uinteger | M | 1 | Identifies the information of the FL member computational load. |  |

6.2.3.6.2.8	Type: FlMdlInfo

Table 6.2.3.6.2.8-1: Definition of type FlMdlInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accuracy | Uinteger | M | 1 | Identifies an unsigned integer between 0 and 100 representing the percentage of the accuracy of the FL model training. |  |
| timeSchedule | array (ScheduledCommunicationTime) | M | 1..N | Identifies the time schedule of the FL model training. |  |
| latency | Uinteger | M | 1 | Identifies an unsigned integer in time unit representing when the FL model training shall be completed. |  |

6.2.3.6.2.9	Type: EvtContent

Table 6.2.3.6.2.9-1: Definition of type EvtContent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrInfo | FlMbrInfo | M | 1 | Identifies the FL member information, for which the Individual FL Event is executed. |  |
| availFlMbr | AvailFlMbr | O | 0..1 | Indicates whether the FL member is present and available. |  |
| flMbrCapability | TBD | M | 1 | Identifies an unsigned integer in time unit representing when the FL model training shall be completed. |  |
| flMbrTimeAvail | TimeWindow | O | 0..1 | Identifies the time period the FL member is available. |  |

Editor's Note:	Data types for lMbrCapability is FFS and is aligned with AIMLE client profile as defined in Table 8.7.3.2-2 of 3GPP TS 23.482 [13].

##### 6.2.3.6.3	Simple data types and enumerations

6.2.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.3.6.3.2	Simple data types

The simple data types defined in table 6.2.3.6.3.2-1 shall be supported.

Table 6.2.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.2.3.6.3.3	Enumeration: FlMbrType

Table 6.2.3.6.3.3-1: Enumeration FlMbrType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_CLIENT | Identifies the FL Member Type is FL Client. |  |
| FL_SERVER | Identifies the FL Member Type is FL Server. |  |

6.2.3.6.3.4	Enumeration: AvailFlMbr

Table 6.2.3.6.3.4-1: Enumeration AvailFlMbr

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AVAILABLE | Identifies the FL Member is available. |  |
| UNAVAILABLE | Identifies the FL Member is unavailable. |  |

##### 6.2.3.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.2.3.6.5	Binary data

6.2.3.6.5.1	Binary Data Types

Table 6.2.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.2.3.7	Error Handling

##### 6.2.3.7.1	General

For the MLR_FLEvents API, HTTP error responses shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the MLR_FLEvents API.

##### 6.2.3.7.2	Protocol Errors

No specific protocol errors for the MLR_FLEvents API are specified.

##### 6.2.3.7.3	Application Errors

The application errors defined for MLR_FLEvents API are listed in table 6.2.3.7.3-1.

Table 6.2.3.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 6.2.3.8	Feature negotiation

The optional features in table 6.2.3.8-1 are defined for the MLR_FLEvents API. They shall be negotiated using the extensibility mechanism defined clause 6.8 of 3GPP TS 29.549 [14].

Table 6.2.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.2.3.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the MLR_FLEvents API.

### 6.2.4	MLR_FLMember API

#### 6.2.4.1	Introduction

The MLR_FLMember Service shall use the MLR_FLMember API.

The API URI of the MLR_FLMember API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [14].

-	The <apiName> shall be "mlr-fl".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.2.4.3 and clause 6.2.4.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.2.4, the ML Repository takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.2.4.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [14] shall apply for the MLR_FLMember API.

#### 6.2.4.3	Resources

##### 6.2.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.4.3.1-1 depicts the resource URIs structure for the MLR_FLMember API.

Figure 6.2.4.3.1-1: Resource URI structure of the MLR_FLMember API

Table 6.2.4.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| FL Member Configurations | /configurations | POST | Register a new Individual FL Member resource |
| Individual FL Member Configuration | /configurations/{configurationId} | GET | Query an Individual Registered FL Member resource. |
|  |  | PUT | Update an Individual Registered FL Member resource. |
|  |  | PATCH | Modify an Individual Registered FL Member resource. |
|  |  | DELETE | Deregister an Individual Registered FL Member resource. |

##### 6.2.4.3.2	Resource: FL Member Configurations

6.2.4.3.2.1	Description

This resource represents the FL Member Configurations resource managed by the ML repository.

6.2.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/mlr-fl/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 6.2.4.3.2.2-1.

Table 6.2.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.4.1. |

6.2.4.3.2.3	Resource Standard Methods

6.2.4.3.2.3.1	POST

The HTTP POST method enables the AIMLE service consumer to register an FL Member at the ML repository.

This method shall support the URI query parameters specified in table 6.2.4.3.2.3.1-1.

Table 6.2.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.4.3.2.3.1-3.

Table 6.2.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbr | M | 1 | Register a new Individual FL member. |

Table 6.2.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbr | M | 1 | 201 Created | Successful case. The registration of the new Individual FL Member is confirmed and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/mlr-fl/<apiVersion>/configurations{configurationId} |

6.2.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 6.2.4.3.3	Resource: Individual FL Member Configuration

6.2.4.3.3.1	Description

This resource represents the individual FL Member Configuration resource managed by the ML repository.

6.2.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/mlr-fl/<apiVersion>/configurations/{configurationId}

This resource shall support the resource URI variables defined in table 6.2.4.3.3.2-1.

Table 6.2.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.4.1. |
| configurationId | string | Represents the identifier of the Individual FL Member Configuration resource. |

6.2.4.3.3.3	Resource Standard Methods

6.2.4.3.3.3.1	GET

The HTTP GET method enables the service consumer e.g., the AIMLE Server to query an Individual Registered FL Member at the ML repository.

This method shall support the URI query parameters specified in table 6.2.4.3.3.3.1-1.

Table 6.2.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.4.3.3.3.1-3.

Table 6.2.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbr | M | 1 | 200 OK | Successful case. The requested information on the Individual Resgietered FL Member is confirmed and a representation of that resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

Table 6.2.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

6.2.4.3.3.3.2	PUT

The HTTP PUT method enables the service consumer e.g., the AIMLE Server to update an Individual Registered FL Member at the ML repository.

This method shall support the URI query parameters specified in table 6.2.4.3.3.3.2-1.

Table 6.2.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.4.3.3.3.2-3.

Table 6.2.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbr | M | 1 | Represents the updated representation of an Individual Registered FL Member. |

Table 6.2.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbr | M | 1 | 200 OK | Successful case. The requested update of the Individual Registered FL Member is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested update of the Individual Registered FL Member is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

Table 6.2.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

6.2.4.3.3.3.3	PATCH

The HTTP PATCH method enables the service consumer e.g., the AIMLE Server to modify an Individual Registered FL Member at the ML repository.

This method shall support the URI query parameters specified in table 6.2.4.3.3.3.3-1.

Table 6.2.4.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.4.3.3.3.3-3.

Table 6.2.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| FlMbrPatch | M | 1 | Represents the parameters to modify of an Individual Registered FL Member. |

Table 6.2.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| FlMbr | M | 1 | 200 OK | Successful case. The requested modification of the Individual FL Registered Member is confirmed and a representation of that resource is returned. |
| n/a |  |  | 204 No Content | Successful case. The requested modification of the Individual Registered FL Member is confirmed and no content is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

Table 6.2.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

6.2.4.3.3.3.4	DELETE

The HTTP DELETE method enables the service consumer e.g., AIMLE Server to deregister an Individual Registered FL Member at the ML repository.

This method shall support the URI query parameters specified in table 6.2.4.3.3.3.4-1.

Table 6.2.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.4.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.4.3.3.3.4-3.

Table 6.2.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. Deregistration of the Individual Registered FL Member is confirmed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ML repository. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

Table 6.2.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ML repository. |

6.2.4.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.4.4	Custom Operations without associated resources

There is not any custom operation without associated resources defined for this API in this release of the specification.

#### 6.2.4.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.2.4.6	Data Model

##### 6.2.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.4.6.1-1 specifies the data types defined for the MLR_FLMember API.

Table 6.2.4.6.1-1: MLR_FLMember API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| FlMbr | 6.2.4.6.2.2 | Registration of a new individual FL Member and Individual Registered FL Member to be updated |  |
| FlMbrPatch | 6.2.4.6.2.3 | Individual Registered FL Member to be modified |  |
| FlCapabilityType | 6.2.4.6.2.4 | The capability of the FL member |  |
| FlMbrAvailSchedule | 6.2.4.6.2.5 | The availability schedule of the FL member |  |
| SuppMlTaskType | 6.2.4.6.3.3 | The supported AIML role identity of the FL member |  |
| MLAppType | 6.2.4.6.3.4 | The supported ML application related to the capability of the FL member |  |
| AvailabilityType | 6.2.4.6.3.5 | The availability of the FL member |  |

Table 6.2.4.6.1-2 specifies data types re-used by the MLR_FLMember API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MLR_FLMember API.

Table 6.2.4.6.1-2: MLR_FLMember API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| FlGroupInfo | 3GPP TS 24.560 [16] | Created Individual FL Member Support Group for the FL process. |  |
| VelocityEstimate | 3GPP TS 29.572 [18] | Identifies the velocity information of the FL member. |  |
| LocationArea | 3GPP TS 29.122 [2] | Identifies the location area of the FL member. |  |
| GeographicArea | 3GPP TS 29.572 [18] | Identifies the geographical information of the FL member. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Identifies the start time and the end time for the validity time. |  |

##### 6.2.4.6.2	Structured data types

6.2.4.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.4.6.2.2	Type: FlMbr

Table 6.2.4.6.2.2-1: Definition of type FlMbr

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrId | string | M | 1 | Identifies the FL member, for which the request applies. |  |
| flMbrSuppGrp | FlGroupInfo | M | 1..N | Identifies the FL Member Support Group for an FL process. |  |
| suppAimlRole | SuppAimlRoleType | M | 1 | Identifies the supported AIML role, for which the request applies. |  |
| flMbrCapability | FlCapabilityType | O | 0..N | Identifies the FL member capabilities, for which the request applies. (NOTE) |  |
| flMbrVelocity | VelocityEstimate | O | 0..1 | Identifies the FL member velocity, for which the request applies. |  |
| flMbrLocation | LocationArea | O | 0..1 | Identifies the FL member location, for which the request applies. |  |
| flMbrAvailSch | FlMbrAvailSchedule | O | 0..N | Identifies the FL member availability schedule, for which the request applies. (NOTE) |  |
| supMlModelId | string | O | 0..N | Identifies the one or more supported ML models by the FL member, for which the request applies. |  |
| areaValidity | GeographicArea | O | 0..1 | Identifies the coordinate of the area of interest, for which the request applies. |  |
| timeValidity | TimeWindow | O | 0..1 | Identifies the time of interest, for which the request applies. |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

6.2.4.6.2.3	Type: FlMbrPatch

Table 6.2.4.6.2.3-1: Definition of type FlMbrPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrId | string | M | 1 | Identifies the FL member, for which the request applies. |  |
| suppAimlRole | SuppAimlRoleType | C | 0..1 | Identifies the supported AIML role, for which the request applies. (NOTE) |  |
| flMbrAvailSch | FlMbrAvailSchedule | C | 0..1 | Identifies the FL member availability schedule, for which the request applies. (NOTE) |  |
| supMlModelId | string | C | 0..N | Identifies the one or more supported ML models by the FL member, for which the request applies. (NOTE) |  |
| areaValidity | GeographicArea | C | 0..1 | Identifies the coordinate of the area of interest, for which the request applies. (NOTE) |  |
| timeValidity | TimeWindow | C | 0..1 | Identifies the time of interest, for which the request applies. (NOTE) |  |
| NOTE: At least one of these attributes shall be included. |  |  |  |  |  |

6.2.4.6.2.4	Type: FlCapabilityType

Table 6.2.4.6.2.4-1: Definition of type FlCapabilityType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlApp | MLAppType | M | 1 | Identifies the ML application type related to the capability of the FL member. |  |
| alResourLvl | AlResourLvlType | M | 1 | Identifies the allowed resource usage level related to the capabilitiy of the FL member. |  |

6.2.4.6.2.5	Type: FlMbrAvailSchedule

Table 6.2.4.6.2.5-1: Definition of type FlMbrAvailSchedule

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flMbrAvailability | AvailabilityType | M | 1 | Identifies the availability of the FL member. |  |
| Time | TimeWindow | M | 1 | Identifies the time of availability of the FL member. |  |

##### 6.2.4.6.3	Simple data types and enumerations

6.2.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.4.6.3.2	Simple data types

The simple data types defined in table 6.2.4.6.3.2-1 shall be supported.

Table 6.2.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

6.2.4.6.3.3	Enumeration: SuppAimlRoleType

The enumeration SuppAimlRoleType represents information regarding the supported AIML role identity of the FL member. It shall comply with the provisions defined in table 6.2.4.6.3.3-1.

Table 6.2.4.6.3.3-1: Enumeration SuppMlTaskType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL_CLIENT | Identifies the supported AIML role of FL member is used as FL client. |  |
| FL_SERVER | Identifies the supported AIML role of FL member is used as FL server. |  |

6.2.4.6.3.4	Enumeration: MLAppType

The enumeration MLAppType represents information regarding the supported ML application related to the capability of the FL member. It shall comply with the provisions defined in table 6.2.4.6.3.4-1.

Table 6.2.4.6.3.4-1: Enumeration MLAppType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REINFORCEMENT_LEARNING | Identifies the ML application of reinforcement learning type related to the capability of FL member. |  |
| SUPERVISED_LEARNING | Identifies the ML application of supervised learning type related to the capability of FL member. |  |
| TRANSFER_LEARNING | Identifies the ML application of transfer learning type related to the capability of FL member. |  |

6.2.4.6.3.5	Enumeration: AvailabilityType

The enumeration AvailabilityType represents information regarding the availability of the FL member. It shall comply with the provisions defined in table 6.2.4.6.3.5-1.

Table 6.2.4.6.3.-1: Enumeration AvailabilityType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AVAILABLE | Identifies the FL member is available. |  |
| NOT_AVAILABLE | Identifies the FL member is not available. |  |

##### 6.2.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combination of data types for MLR_FLMember API in this release of the specification.

##### 6.2.4.6.5	Binary data

6.2.4.6.5.1	Binary Data Types

The binary data types defined for the MLR_FLMember API are listed in Table 6.2.4.6.5.1-1.

Table 6.2.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 6.2.4.7	Error Handling

##### 6.2.4.7.1	General

For the MLR_FLMember API, HTTP error responses shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the MLR_FLMember API.

##### 6.2.4.7.2	Protocol Errors

No specific procedures for the MLR_FLMember API are specified.

##### 6.2.4.7.3	Application Errors

The application errors defined for the MLR_FLMember API are listed in Table 6.2.4.7.3-1.

Table 6.2.4.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

#### 6.2.4.8	Feature negotiation

The optional features in table 6.2.4.8-1 are defined for the MLR_FLMember API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.2.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 6.2.4.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the MLR_FLMember API.

# 7	Using Common API Framework

The provisions of clause 8 of 3GPP TS 29.549 [14] shall apply for the AIMLE APIs defined in this specification.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the APIs defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the APIs.

NOTE:	The semantics and procedures, as well as conditions, e.g., for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [3] and clause 5B of 3GPP TR 21.900 [5]).

# A.2	AIMLES_ContextTransfer API

openapi: 3.0.0

info:

title: AIMLE Context Transfer Information Service

version: 1.0.0-alpha.3

description: |

AIMLE Context Trnaser Information Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.0.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-ct/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request-trans:

post:

summary: Request to send AIMLE context transfer information.

operationId: RequestTrans

tags:

- AIMLE Context Transfer Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransReq'

responses:

'204':

description: >

No Content. The AIMLE context transfer information is successfully received and

processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TransReq:

description: >

Represents the AIMLE context transfer information.

type: object

properties:

aimleClientId:

$ref: '#/components/schemas/AimleClientId'

aimleServerId:

type: string

aimleServerIds:

type: array

items:

type: string

minItems: 1

aimleServStatus:

$ref: '#/components/schemas/AimleServStatus'

required:

- aimleClientId

AimleServStatus:

description: >

Represents the AIMLE service status information.

type: object

properties:

aimleServOpStatus:

$ref: '#/components/schemas/AimleServOpStatus'

aimleServOpPercent:

type: integer

minimum: 0

maximum: 100

required:

- aimleServOpStatus

# SIMPLE DATA TYPES

#

AimleClientId:

description: Represents the unique identifier of a AIMLE Client.

type: string

#

# ENUMERATIONS

#

AimleServOpStatus:

anyOf:

- type: string

enum:

- ACTIVE

- PAUSED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation status.

Possible values are:

- ACTIVE: Indicates that the status of the AIML operations at the AIMLE Client is active.

- PAUSED: Indicates that the status of the AIML operations at the AIMLE Client is paused.

# A.3	AIMLES_DataManagement API

openapi: 3.0.0

info:

title: Aimles_DataManagement

description: |

API for AIMLE Data Management assistance Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.3"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-dm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of a Individual AIMLE Data Management Assistance Subscription.

operationId: CreateAimleDmAssistSubscription

tags:

- AIMLE Data Management Assistance Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

responses:

'201':

description: >

The requested Individual AIMLE Data Management Assistance Subscription is successfully

created and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual AIMLE Data Management Assistance Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing the Individual AIMLE Data Management Assistance Subscription resource.

operationId: GetIndAimleDmAssistSubscription

tags:

- Individual AIMLE Data Management Assistance Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual AIMLE Data Management Assistance Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE Data Management Assistance Subscription resource.

operationId: UpdateIndAimleDmAssistSubscription

tags:

- Individual AIMLE Data Management Assistance Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

responses:

'200':

description: >

OK. The Individual AIMLE Data Management Assistance Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

'204':

description: >

No Content. The Individual AIMLE Data Management Assistance Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE Data Management Assistance Subscription resource.

operationId: ModifyIndAimleDmAssistSubscription

tags:

- Individual AIMLE Data Management Assistance Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubscPatch'

responses:

'200':

description: >

OK. The Individual AIMLE Data Management Assistance Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataMgmtAssistSubsc'

'204':

description: >

No Content. The Individual AIMLE Data Management Assistance Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE Data Management Assistance Subscription.

operationId: UnsubscribeAimleDmAssistSubscription

tags:

- Individual AIMLE Data Management Assistance Subscription (Document).

responses:

'204':

description: >

The individual AIMLE Data Management Assistance Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DataMgmtAssistSubsc:

description: Represents the AIMLE Data Management Assistance subscription information.

type: object

properties:

clientList:

type: array

items:

$ref: 'TS29482_AIMLES_ContextTransfer.yaml#/components/schemas/AimleClientId'

minItems: 1

dataMgmtOp:

$ref: '#/components/schemas/DataMgmtOp'

dataPrepReqs:

$ref: '#/components/schemas/DataPrepReqs'

dataAnalysisReqs:

$ref: '#/components/schemas/DataAnalysisReqs'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- dataMgmtOp

- notifUri

DataMgmtAssistNotif:

description: Represents the AIMLE Data Management Assistance notification.

type: object

properties:

dataMgmtOp:

$ref: '#/components/schemas/DataMgmtOp'

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- dataMgmtOp

DataPrepReqs:

description: Represents the data preparation requirement.

type: object

properties:

dataSetId:

type: string

dataOperRequirement:

type: array

items:

$ref: '#/components/schemas/DataOperRequirement'

minItems: 1

required:

- dataSetId

- dataOperRequirement

DataAnalysisReqs:

description: Represents the data analysis requirement.

type: object

properties:

dataSetId:

type: string

dataOperRequirement:

type: array

items:

$ref: '#/components/schemas/DataOperRequirement'

minItems: 1

required:

- dataSetId

- dataOperRequirement

DataOperRequirement:

description: Represents the data analysis requirement.

type: object

properties:

dataId:

type: string

dataSetFeatureId:

type: string

required:

- dataId

- dataSetFeatureId

DataMgmtAssistSubscPatch:

description: >

Represents the requested modifications to a AIMLE Data Management Assistance subscription.

type: object

properties:

dataPrepReqs:

$ref: '#/components/schemas/DataPrepReqs'

dataAnalysisReqs:

$ref: '#/components/schemas/DataAnalysisReqs'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

DataMgmtOp:

anyOf:

- type: string

enum:

- DATA_PREPARATION

- DATA_ANALYSIS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the data management operation type.

Possible values are:

- DATA_PREPARATION: Indicates that the data management operation type is data preparation.

- DATA_ANALYSIS: Indicates that the data management operation type is data analysis.

#

# A.4	MLR_MLModelManagement API

openapi: 3.0.0

info:

title: AIMLE Server ML Model Management

version: 1.0.0-alpha.1

description: |

AIMLE Server ML Model Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 V1.1.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/mlr-mlmm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/storages:

get:

summary: Retrieve one or several existing Individual ML Models Storage resource(s).

operationId: GetMLModelsStorages

tags:

- ML Models Storages (Collection)

parameters:

- name: storage-ids

in: query

description: Contains the identifier(s) of the targeted ML Models Storage resource(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: profile-ids

in: query

description: Contains identifier(s) of the profile(s) of the targeted stored ML Model(s).

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feats

in: query

description: >

Contains the list of supported features among the ones defined in clause 6.2.1.8.

This query parameter shall be present only when feature negotiation is required.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested Individual ML Models Storage resource(s) shall be returned.

If there are no available Individual ML Models Storage resource(s) fulfilling the

request, an empty array shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MLModelsStorage'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a ML Models Storage.

operationId: CreateMLModelsStorage

tags:

- ML Models Storages (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

responses:

'201':

description: >

Created. The ML Models Storage is successfully created and a representation of the

created Individual ML Models Storage resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

headers:

Location:

description: >

Contains the URI of the created Individual ML Models Storage resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/storages/{storageId}:

parameters:

- name: storageId

in: path

description: >

Represents the identifier of the Individual ML Models Storage resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual ML Models Storage resource.

operationId: GetIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

responses:

'200':

description: >

OK. The requested Individual ML Models Storage resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual ML Models Storage resource.

operationId: UpdateIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

responses:

'200':

description: >

OK. The Individual ML Models Storage resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual ML Models Storage resource.

operationId: ModifyIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MLModelsStoragePatch'

responses:

'200':

description: >

OK. The Individual ML Models Storage resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual ML Models Storage resource.

operationId: DeleteIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

responses:

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

MLModelsStorage:

description: >

Represents a ML Models Storage.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelProfile'

minItems: 1

mlModelsAddresses:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [mlModels]

- required: [mlModelsAddresses]

MLModelProfile:

description: >

Represents the ML Model Profile.

type: object

properties:

mlModelProfId:

type: string

aimleServId:

type: string

aimleRepId:

type: string

mlModelInfo:

$ref: '#/components/schemas/MLModel'

mlModelUri:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- mlModelInfo

MLModel:

description: >

Represents a ML Model.

type: object

properties:

mlModelId:

type: string

adaeAnalyticsId:

type: string

mlModelSize:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

mlModelSrcId:

type: string

valServiceIds:

type: array

items:

type: string

minItems: 1

domain:

$ref: '#/components/schemas/MLModelDomain'

customDomain:

type: string

vendors:

type: array

items:

type: string

minItems: 1

interopInfo:

type: string

phaseInfo:

$ref: '#/components/schemas/MLModelPhaseInfo'

storeDiscReqs:

$ref: '#/components/schemas/MLModelStoreDiscReqs'

usageReqs:

$ref: '#/components/schemas/MLModelUsageReqs'

required:

- mlModelId

anyOf:

- required: [vendors]

- required: [interopInfo]

- required: [phaseInfo]

- required: [storeDiscReqs]

MLModelPhaseInfo:

description: >

Represents the ML Model phase related information.

type: object

properties:

phase:

$ref: '#/components/schemas/MLModelPhase'

observedPerf:

$ref: '#/components/schemas/MLModelPerf'

trainingInfo:

$ref: '#/components/schemas/MLModelTrainingInfo'

contTrainInd:

type: boolean

default: false

description: >

Indicates whether the ML Model can be trained continuously or not.

true indicates that the ML Model can be trained continuously.

false indicates that the ML Model cannot be trained continuously.

The default value is false when this attribute is omitted.

contTrainParams:

type: string

required:

- phase

MLModelStoreDiscReqs:

description: >

Represents the ML Model storage and discovery requirements.

type: object

properties:

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

accessReqs:

$ref: '#/components/schemas/MLModelAccessReqs'

anyOf:

- required: [duration]

- required: [accessReqs]

MLModelUsageReqs:

description: >

Represents the ML Model usage requirements.

type: object

properties:

usages:

type: array

items:

$ref: '#/components/schemas/MLModelUsage'

minItems: 1

required:

- usages

MLModelPerf:

description: >

Represents the performance information of the ML Model.

type: object

properties:

resourceAddr:

type: integer

minimum: 0

maximum: 100

appSpecPerfData:

type: string

anyOf:

- required: [resourceAddr]

- required: [appSpecPerfData]

MLModelTrainingInfo:

description: >

Represents the ML Model training information.

type: object

properties:

dataSources:

type: string

dataVolume:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

freshness:

$ref: '#/components/schemas/DataFreshness'

baseModelId:

type: string

anyOf:

- required: [dataSources]

- required: [dataVolume]

- required: [freshness]

- required: [baseModelId]

MLModelAccessReqs:

description: >

Represents ML Model access requirements.

type: object

properties:

accessReqs:

type: array

items:

$ref: '#/components/schemas/AccessReqs'

minItems: 1

valServerIds:

type: array

items:

type: string

minItems: 1

valClientIds:

type: array

items:

type: string

minItems: 1

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- accessReqs

MLModelsStoragePatch:

description: >

Represents the requested modifications to a ML Models Storage.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelProfile'

minItems: 1

mlModelsAddresses:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

#

# Simple data types

#

#

# ENUMERATIONS

#

MLModelDomain:

anyOf:

- type: string

enum:

- SPEECH_RECOGNITION

- IMAGE_RECOGNITION

- IMAGE_PROCESSING

- LOCATION_PREDICTION

- CUSTOM

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the domain of the ML Model.

Possible values are:

- SPEECH_RECOGNITION: Indicates that the domain of the ML Model is speech recognition.

- IMAGE_RECOGNITION: Indicates that the domain of the ML Model is image recognition.

- IMAGE_PROCESSING: Indicates that the domain of the ML Model is image processing.

- LOCATION_PREDICTION: Indicates that the domain of the ML Model is location prediction.

- CUSTOM: Indicates that the domain of the ML Model is a custom domain.

MLModelPhase:

anyOf:

- type: string

enum:

- NOT_TRAINED

- IN_TRAINING

- TRAINED

- IN_RETRAINING

- DEPLOYED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model phase.

Possible values are:

- NOT_TRAINED: Indicates that the ML Model phase is that ML Model is not trained.

- IN_TRAINING: Indicates that the ML Model phase is that the ML Model is in training.

- TRAINED: Indicates that the ML Model phase is that the ML Model is trained.

- IN_RETRAINING: Indicates that the ML Model phase is that the ML Model is in re-training.

- DEPLOYED: Indicates that the ML Model phase is that the ML Model is deployed.

MLModelUsage:

anyOf:

- type: string

enum:

- TRAINING

- INFERENCE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model usage.

Possible values are:

- TRAINING: Indicates that the ML Model usage is training.

- INFERENCE: Indicates that the ML Model usage is inference.

AccessReqs:

anyOf:

- type: string

enum:

- PUBLICLY_AVAILABLE

- RESTRICTED

- PRIVATE_USE_ONLY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model access requirements.

Possible values are:

- PUBLICLY_AVAILABLE: Indicates that the ML Model is publicly available.

- RESTRICTED: Indicates that the ML Model is restricted.

- PRIVATE_USE_ONLY: Indicates that the ML Model is for private use only.

DataFreshness:

anyOf:

- type: string

enum:

- REAL_TIME

- NEAR_REAL_TIME

- HOURLY

- DAILY

- WEEKLY

- MONTHLY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the data freshness level.

Possible values are:

- REAL_TIME: Indicates that the data freshness level is real-time.

- NEAR_REAL_TIME: Indicates that the data freshness level is near real-time.

- HOURLY: Indicates that the data freshness level is hourly.

- DAILY: Indicates that the data freshness level is daily.

- WEEKLY: Indicates that the data freshness level is weekly.

- MONTHLY: Indicates that the data freshness level is monthly.

# Data types describing alternative data types or combinations of data types:

#

# A.5	AIMLES_AIMLEClientDiscovery API

openapi: 3.0.0

info:

title: AIMLES_AIMLEClientDiscovery

description: |

API for AIMLE Client Discovery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/clients:

get:

summary: Retrieve an existing the Individual AIMLE Data Management Assistance Subscription resource.

operationId: GetAimleClients

tags:

- AIMLE Clients (Collection)

parameters:

- name: filt-criteria

in: query

description: >

Represents the AIMLE Client(s) filtering criteria.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscCriteria'

- name: cl-number

in: query

description: >

Represents the required number of the discovered AIMLE Clients.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: supported-features

in: query

description: >

Contains supported features information, used to negotiate the applicability

of optional features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The response body contains the result of the search over the list of AIMLE Clients.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ClientDiscCriteria:

description: Represents the AIMLE Client discovery criteria.

type: object

properties:

serviceReq:

$ref: '#/components/schemas/ServiceRequirement'

mlModelTypes:

type: array

items:

$ref: '#/components/schemas/MlModelType'

minItems: 1

aimlOpers:

type: array

items:

$ref: '#/components/schemas/AimlOperationRole'

minItems: 1

clientAppCap:

$ref: '#/components/schemas/ClientAppCapability'

datasetReq:

$ref: '#/components/schemas/DatasetRequirement'

clientTaskCap:

$ref: '#/components/schemas/ClientTaskCapability'

clientVel:

$ref: '#/components/schemas/ClientVelocity'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

clientQosReqs:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteriaSet'

minItems: 1

required:

- serviceReq

- aimlOpers

- clientAppCap

ServiceRequirement:

description: Represents the AIMLE service requirements.

type: object

properties:

valServId:

type: string

permLevel:

$ref: '#/components/schemas/ServicePermLevel'

required:

- valServId

ClientAppCapability:

description: Represents the AIMLE Client compute capability type.

type: object

properties:

appType:

$ref: '#/components/schemas/ServicePermLevel'

avail:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

dropOfRate:

type: integer

minimum: 0

maximum: 100

required:

- appType

DatasetRequirement:

description: Represents the dataset requirements.

type: object

properties:

avail:

$ref: '#/components/schemas/DatasetCapability'

cap:

$ref: '#/components/schemas/DatasetCapability'

DatasetAvailablity:

description: Represents the dataset availability information.

type: object

properties:

ids:

type: array

items:

type: string

minItems: 1

age:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

size:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

features:

type: array

items:

type: string

minItems: 1

DatasetCapability:

description: Represents the dataset capability information.

type: object

properties:

dataType:

$ref: '#/components/schemas/DataType'

functions:

type: array

items:

type: string

minItems: 1

ClientTaskCapability:

description: Represents the performance capabilities.

type: object

properties:

compCap:

$ref: '#/components/schemas/ComputeCapability'

perfCap:

$ref: '#/components/schemas/PerformanceCapability'

ClientDiscResp:

description: Represents the AIMLE Client discovery response.

type: object

properties:

clients:

type: array

items:

type: string

minItems: 1

suppTasks:

type: array

items:

$ref: '#/components/schemas/ClientTaskCapability'

minItems: 1

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

# SIMPLE DATA TYPES

#

AimleClientId:

description: >

Represents unique identifier of a AIMLE client

type: string

# ENUMERATIONS

ServicePermLevel:

anyOf:

- type: string

enum:

- PREMIUM

- STANDARD

- LIMITED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the  service permision level.

Possible values are:

- PREMIUM: Indicates that the service permission level is premium.

- STANDARD: Indicates that the service permission level is standard.

- LIMITED: Indicates that the service permission level is limited.

MlModelType:

anyOf:

- type: string

enum:

- DECISION_TREES

- LINEAR_REGRESSION

- NEURAL_NETWORKS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- DECISION_TREES: Indicates that the ML model type is decision trees.

- LINEAR_REGRESSION: Indicates that the ML model type is linear regression.

- NEURAL_NETWORKS: Indicates that the ML model type is neural networks.

AimlOperationRole:

anyOf:

- type: string

enum:

- MODEL_TRAINING

- MODEL_TRANSFER

- MODEL_INFERENCE

- MODEL_OFFLOAD

- MODEL_SPLIT

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE operation role.

Possible values are:

- MODEL_TRAINING: Indicates that the supported AIML operation role is model training.

- MODEL_TRANSFER: Indicates that the supported AIML operation role is model transfer.

- MODEL_INFERENCE: Indicates that the supported AIML operation role is model inference.

- MODEL_OFFLOAD: Indicates that the supported AIML operation role is model offload.

- MODEL_SPLIT: Indicates that the supported AIML operation role is model split.

MlAppType:

anyOf:

- type: string

enum:

- FL

- TL

- SL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- FL: Indicates that the supported ML application type is Federated Learning.

- TL: Indicates that the supported ML application type is Transfer Learning.

- SL: Indicates that the supported ML application type is Split Learning.

DataType:

anyOf:

- type: string

enum:

- RAW

- PROCESSED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of the collected data.

Possible values are:

- RAW: Indicates that the type of the collected data is raw.

- PROCESSED: Indicates that the type of the collected data is processed.

PerformanceCapability:

anyOf:

- type: string

enum:

- GREEN_TASK

- ENERGY_EFFICIENT

- LOW_COSTS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the performance capabilities.

Possible values are:

- GREEN_TASK: Indicates that the performance capability is green task.

- ENERGY_EFFICIENT: Indicates that the performance capability is energy-efficient.

- LOW_COSTS: Indicates that the performance capability is low costs.

ComputeCapability:

anyOf:

- type: string

enum:

- LOW

- HIGH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client compute capability type.

Possible values are:

- LOW: Indicates that the compute capability is low.

- HIGH: Indicates that the compute capability is high.

ClientVelocity:

anyOf:

- type: string

enum:

- LOW

- HIGH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client velocity level.

Possible values are:

- LOW: Indicates that the AIMLE Client velocity is low.

- HIGH: Indicates that the AIMLE Client velocity is high.

# A.6	MLR_ModelInformationDiscovery API

openapi: 3.0.0

info:

title: MLR_ModelInformationDiscovery

description: |

API for MLR Model Information Discovery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/mlr-mid/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/models:

get:

summary: Discover the ML model information according to the filtering criteria.

operationId: GetMlModelInfo

tags:

- ML Models (Collection)

parameters:

- name: filt-criteria

in: query

description: >

Represents the ML model filtering criteria.

required: true

content:

application/json:

schema:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

- name: supported-features

in: query

description: >

Contains supported features information, used to negotiate the applicability

of optional features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The response body contains the result of the search over the list

of stored ML Models.

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DiscoveryResp:

description: Represents the ML model discovery repsonse.

type: object

properties:

profiles:

type: array

items:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelProfile'

minItems: 1

mlModels:

type: array

items:

$ref: '#/components/schemas/MlModel'

minItems: 1

indicator:

type: boolean

oneOf:

- required: [profiles]

- required: [mlModels]

MlModel:

description: Represents the ML model.

type: object

properties:

mlModelId:

type: string

mlModel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

required:

- mlModelId

# SIMPLE DATA TYPES

#

# ENUMERATIONS

# A.7	AIMLES_AIMLEClientSelection API

openapi: 3.0.0

info:

title: AIMLES_AIMLEClientSelection

description: |

API for AIMLE Client Selection Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-sel/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of a Individual AIMLE Client Selection Subscription.

operationId: CreateAimleClSelSubscription

tags:

- AIMLE Client Selection Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

responses:

'201':

description: >

The requested Individual AIMLE Client Selection Subscription is successfully

created and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual AIMLE Client Selection Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing the Individual AIMLE Client Selection Subscription resource.

operationId: GetIndAimleClSelSubscription

tags:

- Individual AIMLE Client Selection Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual AIMLE Client Selection Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE Client Selection Subscription resource.

operationId: UpdateIndAimleClSelSubscription

tags:

- Individual AIMLE Client Selection Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

responses:

'200':

description: >

OK. The Individual AIMLE Client Selection Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

'204':

description: >

No Content. The Individual AIMLE Client Selection Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE Client Selection Subscription resource.

operationId: ModifyIndAimleClSelSubscription

tags:

- Individual AIMLE Client Selection Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ClientSelSubPatch'

responses:

'200':

description: >

OK. The Individual AIMLE Client Selection Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelSub'

'204':

description: >

No Content. The Individual AIMLE Client Selection Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE Client Selection Subscription.

operationId: UnsubscribeAimleClSelSubscription

tags:

- Individual AIMLE Client Selection Subscription (Document).

responses:

'204':

description: >

The individual AIMLE Client Selection Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/select:

post:

summary: Request to select AIMLE Clients.

operationId: Select

tags:

- AIMLE Client Selection Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelReq'

responses:

'200':

description: >

The requested Individual AIMLE Client Selection Subscription is successfully

created and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientSelResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ClientSelSub:

description: Represents the AIMLE Client selection subscription information.

type: object

properties:

selCriteria:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ClientDiscCriteria'

reqNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- selCriteria

- notifUri

ClientSelNotif:

description: Represents the AIMLE Client selection notification.

type: object

properties:

reports:

type: array

items:

$ref: '#/components/schemas/SelUpdate'

minItems: 1

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- reports

ClientSelSubPatch:

description: >

Represents the requested modifications to a AIMLE Client

selection subscription information.

type: object

properties:

selCriteria:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ClientDiscCriteria'

reqNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

ClientSelReq:

description: Represents the AIMLE Client selection request.

type: object

properties:

valSvcId:

type: string

clients:

type: array

items:

type: string

minItems: 1

selCriteria:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ClientDiscCriteria'

reqNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ClientSelResp:

description: Represents the AIMLE Client selection response.

type: object

properties:

clients:

type: array

items:

type: string

minItems: 1

clientSet:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

SelUpdate:

description: Represents the update of the selected client.

type: object

properties:

replaced:

type: string

selected:

type: string

required:

- replaced

- selected

# SIMPLE DATA TYPES

#

# A.8	AIMLES_AIMLEServiceOperationsManagement API

openapi: 3.0.0

info:

title: AIMLE Service Operations Control and Management Service

version: 1.0.0-alpha.1

description: |

AIMLE Service Operations Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-ct/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: Request AIMLE service operations control and management.

operationId: Request

tags:

- AIMLE Service Operation Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlServOperReq'

responses:

'200':

description: >

The AIMLE service operations control and management request is successfully

received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlServOperResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AimlServOperReq:

description: >

Represents the AIMLE service operations control and management request.

type: object

properties:

valSvcId:

type: string

clients:

type: array

items:

type: string

minItems: 1

setId:

type: string

operId:

$ref: '#/components/schemas/AimleOperId'

operInfo:

$ref: '#/components/schemas/AimleOperInfo'

operMode:

$ref: '#/components/schemas/AimleOperMode'

confModes:

type: array

items:

$ref: '#/components/schemas/AimleConfigMode'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- operMode

AimlServOperResp:

description: >

Represents the AIMLE service operations control and management response.

type: object

properties:

valSvcId:

type: string

operId:

$ref: '#/components/schemas/AimleOperId'

operMode:

$ref: '#/components/schemas/AimleOperMode'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- operId

- operMode

AimleOperId:

description: >

Represents the AIMLE service operation identifier.

type: object

properties:

trainingId:

type: string

taskId:

type: string

oneOf:

- required: [trainingId]

- required: [taskId]

AimleOperInfo:

description: >

Represents the AIMLE service operation information.

type: object

properties:

container:

type: string

fetchUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

agrUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

assitInfo:

$ref: '#/components/schemas/AimleOperAssist'

required:

- container

- fetchUri

- agrUri

- assitInfo

AimleOperAssist:

description: >

Represent the AIMLE service operation assistance information.

type: object

properties:

maxConvTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- maxConvTime

AimleConfigMode:

description: >

Represent the AIMLE service operation configuration mode information.

type: object

properties:

mode:

$ref: '#/components/schemas/AimleOperMode'

measThrValues:

$ref: '#/components/schemas/MeasurementData'

thrDirection:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- mode

- measThrValues

- thrDirection

MeasurementData:

description: Represent the measurement data for the AIMLE service operation.

type: object

properties:

latency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

time:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

accuracy:

type: integer

minimum: 0

maximum: 100

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AimleOperMode:

anyOf:

- type: string

enum:

- START

- STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation mode.

Possible values are:

- START: Indicates the AIMLE service operation mode is start.

- STOP: Indicates the AIMLE service operation mode is stop.

# A.9	AIMLES_HierarchicalComputingAssist API

openapi: 3.0.0

info:

title: AIMLE Service Operations Control and Management Service

version: 1.0.0-alpha.1

description: |

AIMLE Hierarchical Computing Assist Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-hca/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: Request AIMLE hierarchical computing assist.

operationId: Request

tags:

- AIMLE Hierarchical Computing Assist Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlHierAssitReq'

responses:

'200':

description: >

The AIMLE hierarchical computing assist request is successfully

received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlHierAssitResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AimlHierAssitReq:

description: >

Represents the AIMLE hierarchical computing assist request.

type: object

properties:

origReqId:

$ref: '#/components/schemas/RequestorId'

role:

$ref: '#/components/schemas/Role'

task:

$ref: '#/components/schemas/TaskType'

assistSets:

type: array

items:

$ref: '#/components/schemas/AssitInfo'

minItems: 1

exeNodes:

type: array

items:

$ref: '#/components/schemas/ExecutionNode'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- role

- task

- assistSets

AimlHierAssitResp:

description: >

Represents the AIMLE hierarchical computing assist response.

type: object

properties:

assistSets:

type: array

items:

$ref: '#/components/schemas/AssitInfo'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- assistSets

RequestorId:

description: >

Represents the requestor identifier.

type: object

properties:

valServId:

type: string

easId:

type: string

casId:

type: string

oneOf:

- required: [valServId]

- required: [easId]

- required: [casId]

AssitInfo:

description: >

Represents the assistance information.

type: object

properties:

easId:

type: string

status:

$ref: 'TS29549_SS_ADAE_EdgeLoadAnalytics.yaml#/components/schemas/EasAnalytics'

required:

- easId

- status

ExecutionNode:

description: >

Represent the execution node information.

type: object

properties:

easId:

type: string

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

Role:

anyOf:

- type: string

enum:

- ROOT_NODE

- SUB_ROOT_NODE

- LEAF_NODE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the role of the node.

Possible values are:

- ROOT_NODE: Indicates the role is root node.

- SUB_ROOT_NODE: Indicates the role is sub root node.

- LEAF_NODE: Indicates the role is leaf node.

TaskType:

anyOf:

- type: string

enum:

- VFL

- HFL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the type of task.

Possible values are:

- VFL: Indicates the task is VFL.

- HFL: Indicates the task is HFL.

# A.10	AIMLES_AssistedMLModelSelection API

openapi: 3.0.0

info:

title: AIMLES_AssistedMLModelSelection

description: |

API for AIMLE Assisted ML Model Selection Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-amlmsel/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of a AIMLE Assisted ML Model Selection Subscription resource.

operationId: CreateAimleAssistMLMdlSelSubscription

tags:

- AIMLE Assisted ML Model Selection Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

responses:

'201':

description: >

Successful case. The creation of an Individual AIMLE Assisted ML Model Selection

Subscription resource is confirmed and a representation of that resource is returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: notify a previously subscribed service consumer on AIML Assisted ML Model Selection related event(s).

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an Individual AIMLE Assisted ML Model Selection Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual AIMLE Assisted ML Model Selection Subscription resource.

operationId: GetAimleAssistMLMdlSelSubscription

tags:

- Individual AIMLE Assisted ML Model Selection Subscription (Document)

responses:

'200':

description: >

Successful case. The requested Individual AIMLE Assisted ML Model Selection

Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE Assisted ML Model Selection Subscription resource.

operationId: UpdateAimleAssistMLMdlSelSubscription

tags:

- Individual AIMLE Assisted ML Model Selection Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

responses:

'200':

description: >

Successful case. The Individual AIMLE Assisted ML Model Selection Subscription resource

is successfully updated and a representation of the updated resource shall be returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

'204':

description: >

No Content. The Individual AIMLE Assisted ML Model Selection Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE Assisted ML Model Selection Subscription resource.

operationId: ModifyAimleAssistMLMdlSelSubscription

tags:

- Individual AIMLE Assisted ML Model Selection Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubscPatch'

responses:

'200':

description: >

Successful case. The Individual AIMLE Assisted ML Model Selection Subscription resource

is successfully modified and a representation of the updated resource shall be returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AssistMLMdlSelSubsc'

'204':

description: >

No Content. The Individual AIMLE Assisted ML Model Selection Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual AIMLE Assisted ML Model Selection Subscription resource.

operationId: UnsubscribeAimleAssistMLMdlSelSubscription

tags:

- Individual AIMLE Assisted ML Model Selection Subscription (Document).

responses:

'204':

description: >

No Content. The Individual AIMLE Assisted ML Model Selection Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AssistMLMdlSelSubsc:

description: Represents the AIMLE Assisted ML Model Selection subscription information.

type: object

properties:

aimlProfile:

$ref: '#/components/schemas/AimlProfile'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

repInfo:

$ref: '#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- aimlProfile

AssistMLMdlSelNotif:

description: Represents the AIMLE Assisted ML Selection notification.

type: object

properties:

opStatus:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/DataMgmtOp'

trainMLModel:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelTrainingInfo'

elapseTime:

type: integer

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- opStatus

- trainMLModel

AimlProfile:

description: Represents the ML model selection operation.

type: object

properties:

candMLMdls:

type: array

items:

$ref: '#/components/schemas/CandMLMdl'

minItems: 1

mlMdlReq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MlModelInfo'

dataSetIds:

type: array

items:

type: string

minItems: 1

trainReq:

type: array

items:

$ref: '#/components/schemas/TrainingRequirement'

minItems: 1

clientList:

type: array

items:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/AimleClientId'

minItems: 1

clNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

TrainingRequirement:

description: Represents the training requirements for ML model selection.

type: object

properties:

perfReq:

type: array

items:

$ref: '#/components/schemas/PerformanceRequirement'

minItems: 1

trainCount:

type: integer

sampleCount:

type: integer

PerformanceRequirement:

description: Represents the performance requirements for ML model selection.

type: object

properties:

perfMetric:

$ref: '#/components/schemas/PerformanceMetric'

perfTarget:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

AssistMLMdlSelSubscPatch:

description: >

Represents the requested modifications to a AIMLE Assisted ML Selection subscription

information.

type: object

properties:

trainReq:

type: array

items:

$ref: '#/components/schemas/TrainingRequirement'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

ReportingInformation:

description: Represents the reporting requirements for ML model selection.

type: object

properties:

immRep:

type: boolean

notifMethod:

$ref: '#/components/schemas/NotificationMethod'

jobPercentage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

CandMLMdl:

description: Contains the candidate ML model selection information.

type: object

properties:

mlMdlId:

type: string

mlMdlParam:

type: string

# SIMPLE DATA TYPES

#

# ENUMERATIONS

PerformanceMetric:

anyOf:

- type: string

enum:

- ACCURACY

- PRECISION

- RECALL

- MEAN_SQUARED_ERROR

- MEAN_ABSOLUTE_ERROR

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the performance metric for training the ML model.

Possible values are:

- ACCURACY: Indicates the performance metric is accuracy.

- PRECISION: Indicates the performance metric is precision.

- RECALL: Indicates the performance metric is recall.

- MEAN_SQUARED_ERROR: Indicates the performance metric is mean squared error.

- MEAN_ABSOLUTE_ERROR: Indicates the performance metric is mean absolute error.

NotificationMethod:

anyOf:

- type: string

enum:

- PERIODIC

- ON_JOB_COMPLETION

- ON_PCT_COMPLETION

- ON_EVENT_DETECTION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the notification methods that can be subscribed.

Possible values are:

- PERIODIC: The notification of the ML model status is periodically sent.

- ON_JOB_COMPLETION: The notification is sent only after the entire ML model selection job

is completed.

- ON_PCT_COMPLETION: The notification is sent after the certain job percentage completion.

- ON_EVENT_DETECTION: The notification is sent each time the event is detected.

# A.11	Aimles_MLModelRetrieval API

openapi: 3.0.0

info:

title: Aimles_MLModelRetrieval

description: |

API for AIMLE ML Model Retrieval Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-mlmr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/retrieve:

post:

summary: Enables a service consumer to send AIMLE ML Model retrieval request to the AIMLE server.

operationId: RetrieveMLModel

tags:

- Retrieve

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetReq'

responses:

'200':

description: OK (The requested location information)

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Request the creation of a Individual AIMLE ML Model Retrieval Subscription.

operationId: CreateAimleMLModelRetrievalSubscription

tags:

- AIMLE ML Model Retrieval Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

responses:

'201':

description: >

The requested Individual AIMLE ML Model Retrieval Subscription is successfully

created and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MlMdlRetNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual AIMLE ML Model Retrieval Subscription resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing the Individual AIMLE ML Model Retrieval Subscription resource.

operationId: GetIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual AIMLE ML Model Retrieval Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE ML Model Retrieval Subscription resource.

operationId: UpdateIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

responses:

'200':

description: >

OK. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'204':

description: >

No Content. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE ML Model Retrieval Subscription resource.

operationId: ModifyIndAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MLMdlRetSubPatch'

responses:

'200':

description: >

OK. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLMdlRetSub'

'204':

description: >

No Content. The Individual AIMLE ML Model Retrieval Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE ML Model Retrieval Subscription.

operationId: UnsubscribeAimleMLModelRetrievalSubscription

tags:

- Individual AIMLE ML Model Retrieval Subscription (Document).

responses:

'204':

description: >

The individual AIMLE ML Model Retrieval Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MLMdlRetSub:

description: Represents the AIMLE ML Model Retrieval subscription information.

type: object

properties:

requestorId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

- notifUri

MLMdlRetSubPatch:

description: Represents the requested modifications to the AIMLE ML Model Retrieval subscription information.

type: object

properties:

requestorId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MlMdlRetNotif:

description: Represents the AIMLE ML Model Retrieval notification.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelDetail'

minItems: 1

MLMdlRetReq:

description: Represents the AIMLE ML Model Retrieval request information.

type: object

properties:

requestorId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

MLMdlRetRsp:

description: Represents the AIMLE ML Model Retrieval response information.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelDetail'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

MLModelDetail:

description: Represents the ML Model information.

type: object

properties:

mlModelId:

type: string

mlMdlEndpoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

mlModel:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bytes'

required:

- mlModelId

oneOf:

- required: [mlMdlEndpoint]

- required: [mlModel]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# A.12	Aimles_SplitOpNodeRegistration API

openapi: 3.0.0

info:

title: Aimles_SplitOpNodeRegistration

description: |

API for AIMLE Split Operation Node Register Configurations.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-sonreg/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Individual AIMLE Split Operation Node Register.

operationId: CreateAimleSplitOpNodeRegSubscription

tags:

- AIMLE Split Operation Node Register Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

responses:

'201':

description: >

The registration of the new Individual AIMLE AIMLE Split Operation Node Register is confirmed

and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configurationId}:

get:

summary: Retrieve an existing Individual AIMLE Split Operation Node Register resource.

operationId: GetIndAimleSplitOpNodeRegSubscription

tags:

- Individual AIMLE Split Operation Node Register Configuration (Document)

parameters:

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

responses:

'200':

description: >

OK. The requested information on the Individual Resgietered AIMLE Split

Operation Node Register is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIMLE Split Operation Node Register resource.

operationId: UpdateIndAimleSplitOpNodeRegSubscription

tags:

- Individual AIMLE Split Operation Node Register Configuration (Document)

parameters:

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

responses:

'200':

description: >

OK. The requested update of the Individual Registered AIMLE Split Operation

Node Register is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

'204':

description: >

No Content. The Individual AIMLE Split Operation Node Register resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIMLE Split Operation Node Register resource.

operationId: ModifyIndAimleSplitOpNodeRegSubscription

tags:

- Individual AIMLE Split Operation Node Register Configuration (Document)

parameters:

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SplitOpNodeRegPatch'

responses:

'200':

description: >

OK. The requested modification of the Individual Registered AIMLE Split Operation Node

Register is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpNodeReg'

'204':

description: >

No Content. The requested modification of the Individual Registered AIMLE Split

Operation Node Register is confirmed and no content is returned.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the Individual AIMLE Split Operation Node Register.

operationId: UnsubscribeAimleSplitOpNodeRegSubscription

tags:

- Individual AIMLE Split Operation Node Register Configuration (Document).

parameters:

- name: configurationId

in: path

description: Identifier of the configuration resource

required: true

schema:

type: string

responses:

'204':

description: >

Deregistration of the Individual Registered AIMLE Split Operation Node Register is

confirmed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SplitOpNodeReg:

description: Represents the Split Operation Node Register information.

type: object

properties:

nodeInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

sonRegCapability:

$ref: '#/components/schemas/SplitOpCapabilities'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- nodeInfo

SplitOpNodeRegPatch:

description: Represents the split operation node register information to be modified.

type: object

properties:

sonregCapability:

$ref: '#/components/schemas/SplitOpCapabilities'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

SplitOpCapabilities:

description: Represents the split operation capabilities.

type: object

properties:

modelInfo:

$ref: '#/components/schemas/ModelInformation'

usageInfo:

$ref: '#/components/schemas/UsageInformation'

required:

- modelInfo

ModelInformation:

description: Represents the ML model information.

type: object

properties:

modelId:

type: string

modelName:

type: string

modelVersion:

type: string

suppOperation:

type: array

items:

type: string

minItems: 1

required:

- modelId

- modelVersion

- suppOperation

UsageInformation:

description: Represents the data analysis requirement.

type: object

properties:

inputFreq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

inputSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

outputFreq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

outputSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# B.2	AIMLES_ContextTransfer API

The API versions listed in table B.2-1 are withdrawn for the AIMLES_ContextTransfer API.

Table B.2-1: Withdrawn API versions of the AIMLES_ContextTransfer service

| API version number | Remarks |
| --- | --- |

# B.3	AIMLES_DataManagement API

The API versions listed in table B.3-1 are withdrawn for the AIMLES_DataManagement API.

Table B.3-1: Withdrawn API versions of the AIMLES_DataManagement service

| API version number | Remarks |
| --- | --- |

# B.4	MLR_MLModelManagement API

The API versions listed in table B.4-1 are withdrawn for the MLR_MLModelManagement API.

Table B.4-1: Withdrawn API versions of the MLR_MLModelManagement service

| API version number | Remarks |
| --- | --- |

# B.5	AIMLES_AIMLEClientDiscovery API

The API versions listed in table B.5-1 are withdrawn for the AIMLES_AIMLEClientDiscovery API.

Table B.5-1: Withdrawn API versions of the AIMLES_AIMLEClientDiscovery service

| API version number | Remarks |
| --- | --- |

# B.6	MLR_ModelInformationDiscovery API

The API versions listed in table B.6-1 are withdrawn for the MLR_ModelInformationDiscovery API.

Table B.6-1: Withdrawn API versions of the MLR_ModelInformationDiscovery service

| API version number | Remarks |
| --- | --- |

# B.7	AIMLES_AIMLEClientSelection API

The API versions listed in table B.7-1 are withdrawn for the AIMLES_AIMLEClientSelection API.

Table B.7-1: Withdrawn API versions of the AIMLES_AIMLEClientSelection service

| API version number | Remarks |
| --- | --- |

# B.8	AIMLES_AIMLEServiceOperationsManagement API

The API versions listed in table B.8-1 are withdrawn for the AIMLES_AIMLEServiceOperationsManagement API.

Table B.8-1: Withdrawn API versions of the AIMLES_AIMLEServiceOperationsManagement service

| API version number | Remarks |
| --- | --- |

# B.9	AIMLES_HierarchicalComputingAssist API

The API versions listed in table B.9-1 are withdrawn for the AIMLES_HierarchicalComputingAssist API.

Table B.9-1: Withdrawn API versions of the AIMLES_HierarchicalComputingAssist service

| API version number | Remarks |
| --- | --- |

# B.10	AIMLES_AssistedMLModelSelection API

The API versions listed in table B.10-1 are withdrawn for the AIMLES_AssistedMLModelSelection API.

Table B.10-1: Withdrawn API versions of the AIMLES_AssistedMLModelSelection service

| API version number | Remarks |
| --- | --- |

# B.11	Aimles_MLModelRetrieval API

The API versions listed in table B.12-1 are withdrawn for the Aimles_MLModelRetrieval API.

Table B.12-1: Withdrawn API versions of the Aimles_MLModelRetrieval service

| API version number | Remarks |
| --- | --- |

# B.12	Aimles_SplitOpNodeRegistration API

The API versions listed in table B.13-1 are withdrawn for the AIMLES_AssistedMLModelSelection API.

Table B.13-1: Withdrawn API versions of the Aimles_SplitOpNodeRegistration service

| API version number | Remarks |
| --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | CT3#139 |  |  |  |  | Based on skeleton C3-250618 Incorporates agreed pCRs C3-250513, C3-250514, C3-250517, C3-250519, C3-250520, C3-250521, C3-250522, C3-250523, C3-250524 | 0.1.0 |
| 2025-04 | CT3#140 |  |  |  |  | Incorporates agreed pCRs C3-251150, C3-251152, C3-251153, C3-251154, C3-251317, C3-251318, C3-251321, C3-251521, C3-251523, C3-251524, C3-251525, C3-251533, C3-251534, C3-251535,C3-251610, C3-251611, C3-251612, C3-251613 | 0.2.0 |
| 2025-05s | CT3#141 |  |  |  |  | Incorporates agreed pCRs C3-252088, C3-252089, C3-252090, C3-252172, C3-252173, C3-252174, C3-252175, C3-252452, C3-252453, C3-252454, C3-252455, C3-252456, C3-252457, C3-252459, C3-252460, C3-252461, C3-252462, C3-252463, C3-252464, C3-252465, C3-252466, C3-252467, C3-252468 | 0.3.0 |
| 2025-06 | CT#108 | CP-251137 |  |  |  | Presentation to TSG CT for information. | 1.0.0 |
| 2025-09 | CT3#142 |  |  |  |  | Incorporates agreed pCRs C3-253049, C3-253193, C3-253199, C3-253217, C3-253218, C3-253219, C3-253220, C3-253221, C3-253222, C3-253223, C3-253224, C3-253332, C3-253332, C3-253333, C3-253334, C3-253335, C3-253336, C3-253337, C3-253429, C3-253430, C3-253431, C3-253432, C3-253559, C3-253561, C3-253562, C3-253563, C3-253564, C3-253565, C3-253566, C3-253567, C3-253568, C3-253673, C3-253674, C3-253675 | 1.1.0 |