| 3GPP TS 29.538 V19.4.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Enabling MSGin5G Service; Application Programming Interfaces (API) specification; Stage 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions of terms, symbols and abbreviations	12

3.1	Terms	12

3.2	Symbols	12

3.3	Abbreviations	12

4	Overview	13

5	Services offered by the MSGin5G Servers	13

5.1	Introduction	13

5.2	MSGS_ASRegistration Service	14

5.2.1	Service Description	14

5.2.2	Service Operations	14

5.2.2.1	Introduction	14

5.2.2.2	MSGS_ASRegistration_Request	14

5.2.2.2.1	General	14

5.2.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation	14

5.2.2.3	MSGS_ASRegistration_Deregister	15

5.2.2.3.1	General	15

5.2.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation	15

5.3	MSGS_MSGDelivery Service	16

5.3.1	Service Description	16

5.3.2	Service Operations	16

5.3.2.1	Introduction	16

5.3.2.2	MSGS_MSGDelivery_ASODelivery	17

5.3.2.2.1	General	17

5.3.2.2.2	AS Originating MSGin5G Message Delivery	17

5.3.2.3	MSGS_MSGDelivery_ASODeliveryReport	18

5.3.2.3.1	General	18

5.3.2.3.2	AS Originating Message Delivery Status Report	18

5.3.2.4	MSGS_MSGDelivery_UEODelivery	19

5.3.2.4.1	General	19

5.3.2.4.2	UE Originating Message Delivery	19

5.3.2.5	MSGS_MSGDelivery_UEODeliveryReport	20

5.3.2.5.1	General	20

5.3.2.5.2	UE Originating Message Delivery Status Report	21

5.4	MSGS_TopiclistEvent Service	21

5.4.1	Service Description	21

5.4.2	Service Operations	22

5.4.2.1	Introduction	22

5.4.2.2	MSGS_TopiclistEvent_SubscribeMSGTopiclist	22

5.4.2.2.1	General	22

5.4.2.2.2	MSGin5G Server subscribing to MSGin5G Messaging Topic List	22

5.4.2.3	MSGS_TopiclistEvent_UnsubscribeMSGTopiclist	23

5.4.2.3.1	General	23

5.4.2.3.2	MSGin5G Server Unsubscribing to MSGin5G Messaging Topic List	23

5.4.2.4	MSGS_TopiclistEvent_NotifyMSGTopiclist	23

5.4.2.4.1	General	23

5.4.2.4.2	Notification about MSGin5G Messaging Topic List	24

5.4.2.5	MSGS_TopiclistEvent_SubscribeMSGTopic	24

5.4.2.5.1	General	24

5.4.2.5.2	MSGin5G Server Subscribing to MSGin5G Messaging Topic	24

5.4.2.6	MSGS_TopiclistEvent_UnsubscribeMSGTopic	25

5.4.2.6.1	General	25

5.4.2.6.2	MSGin5G Server Unsubscribing to MSGin5G Messaging Topic	25

6	Services offered by the Message Gateway	26

6.1	Introduction	26

6.2	MSGG_L3GDelivery Service	26

6.2.1	Service Description	26

6.2.2	Service Operations	26

6.2.2.1	Introduction	26

6.2.2.2	MSGG_L3GDelivery_GTDelivery	27

6.2.2.2.1	General	27

6.2.2.2.2	Legacy 3GPP Message Gateway Terminating Message Delivery	27

6.2.2.3	MSGG_L3GDelivery_GTDeliveryReport	28

6.2.2.3.1	General	28

6.2.2.3.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report	28

6.3	MSGG_N3GDelivery Service	29

6.3.1	Service Description	29

6.3.2	Service Operations	29

6.3.2.1	Introduction	29

6.3.2.2	MSGG_N3GDelivery_GTDelivery	29

6.3.2.2.1	General	29

6.3.2.2.2	Non-3GPP Message Gateway Terminating Message Delivery	29

6.3.2.3	MSGG_N3GDelivery_GTDeliveryReport	30

6.3.2.3.1	General	30

6.3.2.3.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report	30

6.4	MSGG_BGDelivery Service	31

6.4.1	Service Description	31

6.4.2	Service Operations	31

6.4.2.1	Introduction	31

6.4.2.2	MSGG_BGDelivery_GTDelivery	31

6.4.2.2.1	General	31

6.4.2.2.2	Broadcast Message Gateway Terminating Message Delivery	32

7	Common information applicable to several APIs	33

7.1	General	33

7.2	Data Types	33

7.2.1	General	33

7.2.2	Referenced structured data types	33

7.2.3	Referenced Simple data types and enumerations	33

7.3	Usage of HTTP	34

7.4	Content type	34

7.5	URI structure	34

7.5.1	Resource URI structure	34

7.5.2	Custom operations URI structure	34

7.6	Notifications	34

7.7	Error Handling	34

7.8	Feature negotiation	34

7.9	HTTP headers	34

7.10	Conventions for Open API specification files	34

8	Message Server API definition	34

8.1	MSGS_ASRegistration API	34

8.1.1	API URI	34

8.1.2	Resources	35

8.1.2.1	Overview	35

8.1.2.2	Resource: AS Registrations	35

8.1.2.2.1	Description	35

8.1.2.2.2	Resource Definition	35

8.1.2.2.3	Resource Standard Methods	36

8.1.2.2.3.1	POST	36

8.1.2.3	Resource: AS DeRegistration	36

8.1.2.3.1	Description	36

8.1.2.3.2	Resource Definition	36

8.1.2.3.3	Resource Standard Methods	37

8.1.2.3.3.1	DELETE	37

8.1.3	Custom Operations without associated resources	38

8.1.4	Notifications	38

8.1.5	Data Model	38

8.1.5.1	General	38

8.1.5.2	Structured data types	39

8.1.5.2.1	Introduction	39

8.1.5.2.2	Type: ASRegistration	39

8.1.5.2.3	Type: ASRegistrationAck	39

8.1.5.2.4	Type: ASProfile	39

8.1.5.3	Simple data types and enumerations	39

8.1.6	Error Handling	39

8.1.6.1	General	39

8.1.6.2	Protocol Errors	40

8.1.6.3	Application Errors	40

8.1.7	Feature negotiation	40

8.2	MSGS_MSGDelivery API	40

8.2.1	API URI	40

8.2.2	Resources	40

8.2.3	Custom Operations without associated resources	40

8.2.3.1	Overview	40

8.2.3.2	Operation: deliver-as-message	41

8.2.3.2.1	Description	41

8.2.3.2.2	Operation Definition	41

8.2.3.3	Operation: deliver-ue-message	42

8.2.3.3.1	Description	42

8.2.3.3.2	Operation Definition	42

8.2.3.4	Operation: deliver-report	43

8.2.3.4.1	Description	43

8.2.3.4.2	Operation Definition	43

8.2.4	Notifications	44

8.2.5	Data Model	44

8.2.5.1	General	44

8.2.5.2	Structured data types	46

8.2.5.2.1	Introduction	46

8.2.5.2.2	Type: ASMessageDelivery	46

8.2.5.2.3	Type:UEMessageDelivery	47

8.2.5.2.4	Type: MessageDeliveryAck	47

8.2.5.2.5	Type:MessageSegmentParameters	48

8.2.5.2.6	Type:StoreAndForwardParameters	48

8.2.5.2.7	Type:DeliveryStatusReport	48

8.2.5.3	Simple data types and enumerations	48

8.2.5.3.1	Introduction	48

8.2.5.3.2	Simple data types	48

8.2.5.3.3	Enumeration: DeliveryStatus	49

8.2.5.3.4	Enumeration: ReportDeliveryStatus	49

8.2.5.3.5	Enumeration:Priority	49

8.2.6	Error Handling	49

8.2.6.1	General	49

8.2.6.2	Protocol Errors	49

8.2.6.3	Application Errors	49

8.2.7	Feature negotiation	49

8.3	MSGS_TopiclistEvent API	50

8.3.1	API URI	50

8.3.2	Resources	50

8.3.2.1	Overview	50

8.3.2.2	Resource: Topic List Subscriptions	50

8.3.2.2.1	Description	50

8.3.2.2.2	Resource Definition	50

8.3.2.2.3	Resource Standard Methods	51

8.3.2.2.3.1	POST	51

8.3.2.3	Resource: Individual Topic List Subscription	52

8.3.2.3.1	Description	52

8.3.2.3.2	Resource Definition	52

8.3.2.3.3	Resource Standard Methods	52

8.3.2.3.3.1	DELETE	52

8.3.3	Custom Operations without associated resources	53

8.3.3.1	Overview	53

8.3.3.2	Operation: request-topic-subscription	54

8.3.3.2.1	Description	54

8.3.3.2.2	Operation Definition	54

8.3.3.3	Operation: request-topic-unsubscription	55

8.3.3.3.1	Description	55

8.3.3.3.2	Operation Definition	55

8.3.4	Notifications	56

8.3.4.2	Topiclist Notification	56

8.3.4.2.1	Description	56

8.3.4.2.2	Target URI	56

8.3.4.2.3	Standard Methods	56

8.3.4.2.3.1	POST	56

8.3.5	Data Model	57

8.3.5.1	General	57

8.3.5.2	Structured data types	58

8.3.5.2.1	Introduction	58

8.3.5.2.2	Type: TopicListSubscription	58

8.3.5.2.3	Void	59

8.3.5.2.4	Void	59

8.3.5.2.5	Type: TopicListUnsubscriptionAck	59

8.3.5.2.6	Type: TopicSubscription	59

8.3.5.2.7	Type: TopicSubscriptionAck	59

8.3.5.2.8	Type: TopicUnsubscription	59

8.3.5.2.9	Type: TopicListNotification	59

8.3.5.2.10	Type: MessagingTopic	60

8.3.5.3	Simple data types and enumerations	60

8.3.5.3.1	Introduction	60

8.3.5.3.2	Enumeration: UpdateStatus	60

8.3.6	Error Handling	60

8.3.6.1	General	60

8.3.6.2	Protocol Errors	60

8.3.6.3	Application Errors	60

8.3.7	Feature negotiation	60

9	Message Gateway API definition	61

9.1	MSGG_L3GDelivery API	61

9.1.1	API URI	61

9.1.2	Resources	61

9.1.3	Custom Operations without associated resources	61

9.1.3.1	Overview	61

9.1.3.2	Operation: deliver-message	62

9.1.3.2.1	Description	62

9.1.3.2.2	Operation Definition	62

9.1.3.3	Operation: deliver-report	62

9.1.3.3.1	Description	62

9.1.3.3.2	Operation Definition	63

9.1.4	Notifications	63

9.1.5	Data Model	63

9.1.5.1	General	63

9.1.5.2	Structured data types	65

9.1.5.2.1	Introduction	65

9.1.5.2.2	Type: L3gMessageDelivery	65

9.1.5.2.3	Type: Address	65

9.1.5.3	Simple data types and enumerations	66

9.1.5.3.1	Introduction	66

9.1.5.3.2	Enumeration: AddressType	66

9.1.6	Error Handling	66

9.1.6.1	General	66

9.1.6.2	Protocol Errors	66

9.1.6.3	Application Errors	66

9.1.7	Feature negotiation	66

9.2	MSGG_N3GDelivery API	66

9.2.1	API URI	66

9.2.2	Resources	67

9.2.3	Custom Operations without associated resources	67

9.2.3.1	Overview	67

9.2.3.2	Operation: deliver-message	67

9.2.3.2.1	Description	67

9.2.3.2.2	Operation Definition	67

9.2.3.3	Operation: deliver-report	68

9.2.3.3.1	Description	68

9.2.3.3.2	Operation Definition	68

9.2.4	Notifications	69

9.2.5	Data Model	69

9.2.5.1	General	69

9.2.5.2	Structured data types	70

9.2.5.2.1	Introduction	70

9.2.5.2.2	Type: N3gMessageDelivery	70

9.2.6	Error Handling	71

9.2.6.1	General	71

9.2.6.2	Protocol Errors	71

9.2.6.3	Application Errors	71

9.2.7	Feature negotiation	71

9.3	MSGG_BGDelivery API	71

9.3.1	API URI	71

9.3.2	Resources	71

9.3.3	Custom Operations without associated resources	71

9.3.3.1	Overview	71

9.3.3.2	Operation: deliver-message	72

9.3.3.2.1	Description	72

9.3.3.2.2	Operation Definition	72

9.3.4	Notifications	73

9.3.5	Data Model	73

9.3.5.1	General	73

9.3.5.2	Structured data types	74

9.3.5.2.1	Introduction	74

9.3.5.2.2	Type: BgMessageDelivery	74

9.3.5.3	Simple data types and enumerations	74

9.3.6	Error Handling	75

9.3.6.1	General	75

9.3.6.2	Protocol Errors	75

9.3.6.3	Application Errors	75

9.3.7	Feature negotiation	75

10	Security	75

11	Using Common API Framework	75

11.1	General	75

11.2	Security	76

12	Usage of Network Capabilities	76

Annex A (normative): OpenAPI specification	78

A.1	General	78

A.2	MSGS_ASRegistration API	78

A.3	MSGS_MSGDelivery API	80

A.4	MSGG_L3GDelivery API	85

A.5	MSGG_N3GDelivery API	88

Annex B (informative): Change history	97

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specified the Application Programming Interface (API) for enabling the MSGin5G Service over MSGin5G-2/3/4/7/8 interfaces. The application layer architecture, functional requirements, procedures and information flows necessary for MSGin5G Service are contained in 3GPP TS 23.554 [2]. The requirements for MSGin5G are specified in 3GPP TS 22.262 [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service".

[3]	3GPP TS 22.262: "Message Service within the 5G System".

[4]	Void.

[5]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[8]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[9]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[10]	IETF RFC 9112: "HTTP/1.1".

[11]	IETF RFC 9110: "HTTP Semantics"

[12]	Void.

[13]	Void.

[14]	IETF RFC 9111: Caching".

[15]	Void

[16]	IETF RFC 9113: HTTP/2".

[17]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[18]	3GPP TR 21.900: "Technical Specification Group working methods".

[19]	3GPP TR 33.862: "Study on security aspects of the Message Service for MIoT over the 5G System (MSGin5G)".

[20]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[21]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[22]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[23]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[24]	3GPP TS 29.122: "T8 reference point for northbound APIs".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and its definitions given in 3GPP TS 23.554 [2] shall apply:

MSGin5G Service

MSGin5G message

MSGin5G UE

MSGin5G Group

MSGin5G Client

MSGin5G Server

Legacy 3GPP Message Gateway

Non-3GPP Message Gateway

Broadcast Message Gateway

Legacy 3GPP UE

Non-3GPP UE

Point-to-Point messaging

Point-to-Application messaging

Application-to-Point messaging

Group messaging

Broadcast messaging

Messaging Topic

Message Gateway

Broadcast Area

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

BC	Broadcast

BMG	Broadcast Message Gateway

CAPIF	Common API Framework

L3G	Legacy 3GPP Message Gateway

N3G	Non-3GPP Message Gateway

4	Overview

The MSGin5G Service is designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication and provides messaging capability in 5GS with messaging communication models including Point-to-Point, Application-to-Point/Point-to-Application, Group and Broadcast messaging.

3GPP TS 23.554 [2] has specified the application layer architecture, architectural requirements, procedures, information flows and some APIs, in order to support the MSGin5G Service. Various features are defined to ensure the efficient use and deployment of MSGin5G Service, including configuration, registration, message delivery, message aggregation, segmentation and reassembly, topic messaging.

The present document specifies MSGin5G Services offered by MSGin5G Servers and MSGin5G Gateway, and APIs in detail, needed over MSGin5G-2/3/4/7/8 interfaces for interworking between MSGin5G Server and Legacy 3GPP UE, Non-3GPP UE, Broadcast Message Gateway or Application Server, with following functionalities need to be supported:

1.	Server-side functionality with the sending and receiving of messages to/from Application Servers and/or other MSGin5G Service endpoints on other UEs, provided by MSGin5G Server.

2.	Interconnecting two different messaging delivery mechanisms and assure the message integrity between different message delivery mechanisms, provided by Message Gateway.

And the definition of APIs specified in TS 23.554 [2] clause 9 is introduced in present document.

5	Services offered by the MSGin5G Servers

5.1	Introduction

The Table 5.1-1 lists the services provided by the MSGin5G Server and corresponding service operations. A service description clause for each API gives a general description of the related API.

Table 5.1-1 List of services provided by the MSGin5G Servers

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MSGS_ASRegistration | MSGS_ASRegistration_Request | Request/Response | AS |
|  | MSGS_ASRegistration_Deregister |  |  |
| MSGS_MSGDelivery | MSGS_MSGDelivery_ASODelivery | Request/ Response | AS, Legacy 3GPP Message Gateway, Non-3GPP Message Gateway |
|  | MSGS_MSGDelivery_ASODeliveryReport |  |  |
|  | MSGS_MSGDelivery_UEODelivery |  |  |
|  | MSGS_MSGDelivery_UEODeliveryReport |  |  |
| MSGS_TopiclistEvent | MSGS_TopiclistEvent_SubscribeMSGTopiclist | Subscribe/Notify | MSGin5G Server |
|  | MSGS_TopiclistEvent_UnsubscribeMSGTopiclist |  |  |
|  | MSGS_TopiclistEvent_NotifyMSGTopiclist |  |  |
|  | MSGS_TopiclistEvent_SubscribeMSGTopic | Request/ Response | MSGin5G Server |
|  | MSGS_TopiclistEvent_UnsubscribeMSGTopic |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| MSGS_ASRegistration | 8.1 | AS Registration Service | TS29538_MSGS_ASRegistration.yaml | Msgs-asregistration | A.2 |
| MSGS_MSGDelivery | 8.2 | Message Delivery Service | TS29538_MSGS_MSGDelivery.yaml | Msgs-msgdelivery | A.3 |
| MSGS_TopiclistEvent | 8.3 | Topic Messaging Service | TS29538_MSGS_TopiclistEvent.yaml | Msgs-topiclistevent | A.7 |

5.2	MSGS_ASRegistration Service

5.2.1	Service Description

The MSGS_ASRegistration API, as defined in 3GPP TS 23.554 [2], allows an AS via Mm5s interface to register, and deregister at a given MSGin5G Server.

5.2.2	Service Operations

5.2.2.1	Introduction

The service operation defined for MSGS_ASRegistration API is shown in the Table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the MSGS_ASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_ASRegistration_Request | This service operation is used by the AS to register itself to MSGin5G Server. | AS |
| MSGS_ASRegistration_Deregister | This service operation is used by the AS to deregister itself from a MSGin5G Server. | AS |

5.2.2.2	MSGS_ASRegistration_Request

5.2.2.2.1	General

This service operation is used by the AS to register itself to MSGin5G Server.

5.2.2.2.2	Application Server registering to MSGin5G Server using MSGS_ASRegistration_Request operation

Figure 5.2.2.2.2-1: AS Registering to MSGin5G Server

To register itself at the MSGin5G Server, the AS shall send an HTTP POST message to the MSGin5G Server on the "AS Registrations" collection resource. The body of the HTTP POST message shall include ASRegistration data structure that shall include:

-	the Application Server Identifier within the "asSvcId" attribute; and

may include:

-	the Application Identifier within the "appId" attribute;

-	the notification target URI within the "targetUri" attribute;

-	the Requested expiration time within the "reqExprTime" attribute;

-	the Expected MSGin5G group list within the "exGroupList" attribute; and

-	the Application Server Profile Information within the "asProf" attribute, that may include:

-	the Application Server name within the "appName" attribute;

-	the list of Application Providers name within the "appProviders" attribute;

-	the list of Application scenarios within the "appScenarios" attribute;

-	the list of Application Server category within the "appCategory" attribute; and

-	the list of Application Server status within the "asStatus" attribute.

Upon receiving the HTTP POST message from the AS, the MSGin5G Server shall:

1.	process the AS registration request information;

2.	verify the identity of the AS and check if the AS is authorized to register itself at MSGin5G Server; and

3.	if the AS is authorized to register to MSGin5G Server, then the MSGin5G Server shall:

a.	store the AS registration information and create a new resource with the AS registration information as specified in clause 8.1.2.1; and

b.	If the registration is successful, the MSGin5G Server shall respond to the AS with an HTTP "201 Created" status code, including an HTTP Location header field containing the URI of the created "AS Registration" resource and the response body containing ASRegistrationAck data structure that shall contain:

-	the Application Server Identifier within the "asSvcId" attribute; and

-	the registration result within the "result" attribute.

may include:

-	the Registration expiration time within the "regExprTime" attribute;

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.1.6.

5.2.2.3	MSGS_ASRegistration_Deregister

5.2.2.3.1	General

This service operation is used by the AS to deregister itself from a MSGin5G Server.

5.2.2.3.2	Application Server deregistering from MSGin5G Server using MSGS_ASRegistration_Deregister operation

Figure 5.2.2.3.2-1: AS Deregistering from MSGin5G Server

To deregister itself from the MSGin5G Server, the AS shall send HTTP DELETE message to the MSGin5G Server on the "AS DeRegistration" collection resource, as specified in clause 8.1.2.3.3.2.

Upon receiving the HTTP DELETE request, the MSGin5G Server shall:

1.	verify the identity of the AS and check if the AS is authorized to deregister the AS registration information;

2.	if the AS is authorized to deregister the AS registration information, then the MSGin5G Server shall deregister the AS profile from the MSGin5G Server and delete the resource representing AS registration information; and

3.	return the HTTP "200 OK" status code to the AS, indicating the successful deregistration of the AS information and shall include ASRegistrationAck data structure within the response body or HTTP "204 No Content" status code to the AS.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [24].

If errors occur when processing the HTTP DELETE request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.1.6.

5.3	MSGS_MSGDelivery Service

5.3.1	Service Description

The MSGS_MSGDelivery Service corresponding to Mm5s as defined in 3GPP TS 23.554 [2], is provided by the MSGin5G Server.

This service:

-	allows AS invokes services provided by MSGin5G Server to send MSGin5G Messages and MSGin5G message delivery status reports to MSGin5G Server; and

-	allows L3G/N3G invokes services provided by MSGin5G Server to send MSGin5G Messages and MSGin5G message delivery status reports to MSGin5G Server on behalf of Legacy 3GPP UE or Non-3GPP UE.

-	allows BMG invokes services provided by MSGin5G Server to send MSGin5G message delivery status reports to MSGin5G Server.

-	allows other MSGin5G Server invokes services provided by MSGin5G Server to send MSGin5G Messages and MSGin5G message delivery status reports to MSGin5G Server.

5.3.2	Service Operations

5.3.2.1	Introduction

The service operation defined for MSGS_MSGDelivery Service is shown in the Table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the MSGS_MSGDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_MSGDelivery_ASODelivery | This service operation is used by AS to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to clause 9.1.1.1.2 as defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_ASODeliveryReport | This service operation is used by AS to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to clause 9.1.1.3.2 as defined in 3GPP TS 23.554 [2]. | AS |
| MSGS_MSGDelivery_UEODelivery | This service operation is used by Legacy 3GPP Message Gateway or Non-3GPP Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver MSGin5G message to the MSGin5G Server. This service operation corresponds to clause 9.1.1.2.2 as defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway MSGin5G Server |
| MSGS_MSGDelivery_UEODeliveryReport | This service operation is used by Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) to deliver the delivery status report to the MSGin5G Server. This service operation corresponds to clause 9.1.1.4.2 as defined in 3GPP TS 23.554 [2]. | Legacy 3GPP Message Gateway Non-3GPP Message Gateway Broadcast Message Gateway MSGin5G Server |

5.3.2.2	MSGS_MSGDelivery_ASODelivery

5.3.2.2.1	General

This service operation corresponding to clause 9.1.1.1.2 as defined in 3GPP TS 23.554 [2], is used by AS to deliver MSGin5G message to the MSGin5G Server.

5.3.2.2.2	AS Originating MSGin5G Message Delivery

Figure 5.3.2.2.2-1: AS Originating MSGin5G Message Delivery

When the AS needs to send the message to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the Figure 5.3.2.2.2-1.

The AS shall include ASMessageDelivery data structure in the content of the HTTP POST request.

The ASMessageDelivery data structure shall include:

-	the AS Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the store and forward flag within the "stoAndFwInd" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the priority type within the "priority" attribute;

-	the message segment flag within the "segInd" attribute;

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute;

-	the store and forward parameters within the "stoAndFwParams" attribute, this attribute may include:

-	the message expiration time within the "exprTime" attribute;

-	The latency within the "latency" attribute.

-	The Notification URL within the "notiUrl" attribute.

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G Server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall respond to the AS with a 200 OK message.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [24].

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.2.6.

5.3.2.3	MSGS_MSGDelivery_ASODeliveryReport

5.3.2.3.1	General

This service operation corresponds to clause 9.1.1.3.2 as defined in 3GPP TS 23.554 [2], is used by AS to deliver the delivery status report to the MSGin5G Server.

5.3.2.3.2	AS Originating Message Delivery Status Report

Figure 5.3.2.3.2-1: AS Originating MSGin5G Delivery Report

When the AS needs to send the delivery report to the MSGin5G Server, the AS shall send the HTTP POST method as step 1 of the Figure 5.3.2.3.2-1.

The AS shall include DeliveryStatusReport data structure in the content of the HTTP POST request.

The DeliveryStatusReport data structure shall include:

-	the AS Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the AS, the MSGin5G Server shall make an authorization based on the information received from the AS. If the authorization is successful, the MSGin5G Server shall respond to the AS with a 200 OK message.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [24].

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.2.6.

5.3.2.4	MSGS_MSGDelivery_UEODelivery

5.3.2.4.1	General

This service operation corresponds to clause 9.1.1.2.2 as defined in 3GPP TS 23.554 [2], is used by Legacy 3GPP Message Gateway (on behalf of Legacy 3GPP UE), Non-3GPP Message Gateway (on behalf of Non-3GPP UE) or other MSGin5G Server to deliver MSGin5G message to the MSGin5G Server.

5.3.2.4.2	UE Originating Message Delivery

Figure 5.3.2.4.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Message Delivery

When the Legacy 3GPP Message Gateway (on behalf of Legacy 3GPP UE), Non-3GPP Message Gateway (on behalf of Non-3GPP UE) or other MSGin5G Server needs to send the message to the MSGin5G Server, the Legacy 3GPP Message Gateway, Non-3GPP Message Gateway or other MSGin5G Server shall send the HTTP POST method as step 1of the Figure 5.3.2.4.2-1.

The Legacy 3GPP Message Gateway, Non-3GPP Message Gateway or other MSGin5G Server shall include UEMessageDelivery data structure in the content of the HTTP POST request.

The UEMessageDelivery data structure shall include:

-	the Originating UE Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

-	the store and forward flag within the "stoAndFwInd" attribute;

and may include:

-	the Application ID within the "appId" attribute;

-	the Payload within the "payload" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute; and

-	the message segment flag within the "segInd" attribute;

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute;

-	the last segment flag within the "lastSegFlag" attribute;

-	the store and forward parameters within the "stoAndFwParams" attribute, this attribute may include:

-	the message expiration time within the "exprTime" attribute;

When the MSGin5G Server receives the HTTP POST request from the Legacy 3GPP Message Gateway, Non-3GPP Message Gateway or other MSGin5G Server, the MSGin5G Server shall make an authorization based on the information received from the Legacy 3GPP Message Gateway, Non-3GPP Message Gateway or other MSGin5G Server. If the authorization is successful, the MSGin5G Server shall respond to the Legacy 3GPP Message Gateway, Non-3GPP Message Gateway or other MSGin5G Server with a 200 OK message.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.2.6.

5.3.2.5	MSGS_MSGDelivery_UEODeliveryReport

5.3.2.5.1	General

This service operation corresponds to clause 9.1.1.4.2 as defined in 3GPP TS 23.554 [2], is used by Message Gateway (on behalf Legacy 3GPP UE or Non-3GPP UE) or other MSGin5G Server to deliver the delivery status report to the MSGin5G Server.

5.3.2.5.2	UE Originating Message Delivery Status Report

Figure 5.3.2.5.2-1: Legacy 3GPP UE or Non-3GPP UE Originating MSGin5G Delivery Report

When the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or other MSGin5G Server needs to send the delivery report to the MSGin5G Server, the Message Gateway or other MSGin5G Server shall send the HTTP POST method as step 1 of the Figure 5.3.2.5.2-1.

The Message Gateway or other MSGin5G Server shall include DeliveryStatusReport data structure in the content of the HTTP POST request.

The DeliveryStatusReport data structure shall include:

-	the Originating UE Service ID within the "oriAddr" attribute;

-	the Recipient Address within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

-	the delivery status within the "delivSt" attribute;

and may include:

-	The failure cause within the "failureCause" attribute;

When the MSGin5G Server receives the HTTP POST request from the Message Gateway or other MSGin5G Server, the MSGin5G Server shall make an authorization based on the information received from the Message Gateway or other MSGin5G Server. If the authorization is successful, the MSGin5G Server shall respond to the Message Gateway or other MSGin5G Server with a 200 OK message.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the MSGin5G Server shall apply error handling procedures as specified in clause 8.2.6.

5.4	MSGS_TopiclistEvent Service

5.4.1	Service Description

The MSGS_TopiclistEvent API Service corresponding to Mm5s as defined in 3GPP TS 23.554 [2], is provided by the MSGin5G Server.

This service:

-	allows another MSGin5G Server invokes services provided by MSGin5G Server to subscribe or unsubscribe to MSGin5G Messaging Topic;

-	allows another MSGin5G Server invokes services provided by MSGin5G Server to subscribe or unsubscribe to MSGin5G Messaging Topic list;

-	allows another MSGin5G Server invokes services provided by MSGin5G Server to notify changes of MSGin5G Messaging Topic list;

5.4.2	Service Operations

5.4.2.1	Introduction

The service operation defined for MSGS_TopiclistEvent service is shown in the Table 5.4.2.1-1, corresponds to clause 9.1.1.6 as defined in 3GPP TS 23.554 [2].

Table 5.4.2.1-1: Operations of the MSGS_TopiclistEvent Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGS_TopiclistEvent_SubscribeMSGTopiclist | This service operation is used by MSGin5G Server to subscribe to Messaging Topic list on another MSGin5G Server. | MSGin5G Server |
| MSGS_TopiclistEvent_UnsubscribeMSGTopiclist | This service operation is used by MSGin5G Server to unsubscribe to Messaging Topic list on another MSGin5G Server. | MSGin5G Server |
| MSGS_TopiclistEvent_NotifyMSGTopiclist | This service operation is used by MSGin5G Server, to deliver the notification of Messaging Topic list changes. | MSGin5G Server |
| MSGS_TopiclistEvent_SubscribeMSGTopic | This service operation is used by MSGin5G Server (on behalf of Application Server or MSGin5G Client) to subscribe to Messaging Topic on the MSGin5G Server. | MSGin5G Server |
| MSGS_TopiclistEvent_UnsubscribeMSGTopic | This service operation is used by MSGin5G Server(on behalf of Application Server or MSGin5G Client) to unsubscribe to Messaging Topic on the MSGin5G Server. | MSGin5G Server |

5.4.2.2	MSGS_TopiclistEvent_SubscribeMSGTopiclist

5.4.2.2.1	General

This service operation is used by MSGin5G Server to subscribe to Messaging Topic list on another MSGin5G Server, corresponds to clause 9.1.1.6.2 as defined in 3GPP TS 23.554 [2].

5.4.2.2.2	MSGin5G Server subscribing to MSGin5G Messaging Topic List

Figure 5.4.2.2.2-1: MSGin5G Server Subscribing to Messaging Topic List

To subscribe MSGin5G Messaging Topic list on another MSGin5G Server 2, the MSGin5G Server 1 shall send an HTTP POST message to the MSGin5G Server 2 on the "Topic List Subscriptions" collection resource. The body of the HTTP POST message shall include TopicListSubscription data structure.

Upon receiving the HTTP POST message from the MSGin5G Server 1, the MSGin5G Server 2 shall:

1.	make an authorization based on the information received from from MSGin5G Server 1;

2.	checks the locally stored Messaging Topic list subscription(s):

a.	If the MSGin5G Server 1's subscription has already been created, the MSGin5G Server 2 updates the validity time of this subscription;

b.	If the MSGin5G Server 1's subscription has not been created, the MSGin5G Server 2 creates new subscription; and

If the subscription request is successfully processed, the MSGin5G Server 2 shall respond to the MSGin5G Server 1 with an HTTP "201 Created" status code, including an HTTP Location header field containing the URI of the created "Topic List Subscription" resource and the response body containing TopicListSubscription data structure that shall contain the representation of the created resource within the TopicListSubscription data structure.

If errors occur when processing the HTTP POST request, the MSGin5G Server 2 shall apply error handling procedures as specified in clause 8.3.6.

5.4.2.3	MSGS_TopiclistEvent_UnsubscribeMSGTopiclist

5.4.2.3.1	General

This service operation is used by MSGin5G Server to subscribe to Messaging Topic list on another MSGin5G Server, corresponds to clause 9.1.1.6.6 as defined in 3GPP TS 23.554 [2].

5.4.2.3.2	MSGin5G Server Unsubscribing to MSGin5G Messaging Topic List

Figure 5.4.2.3.2-1: MSGin5G Server Unsubscribing to Messaging Topic List

As shown in Figure 5.4.2.3.2-1, to unsubscribe MSGin5G Messaging Topic list on the MSGin5G Server 2, the MSGin5G Server 1 shall send HTTP DELETE request message to the MSGin5G Server 2 on the "Individual Topic List Subscription" collection resource, where "{subscriptionId}" is the identifier of the existing Messaging Topic list subscription that is to be deleted.

Upon receiving the HTTP DELETE request message from the MSGin5G Server 1, the MSGin5G Server 2 shall make an authorization based on the information received. If the authorization is successful, the MSGin5G Server 2 shall remove the corresponding subscription. If the unsubscription request is successfully processed, the MSGin5G Server 2 shall respond to the MSGin5G Server 1 with an HTTP "204 No Content" status code.

If the MSGin5G Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [3].

If errors occur when processing the HTTP POST request, the MSGin5G Server 2 shall apply error handling procedures as specified in clause 8.3.6.

5.4.2.4	MSGS_TopiclistEvent_NotifyMSGTopiclist

5.4.2.4.1	General

This service operation is used by MSGin5G Server 1 to notify another MSGin5G Server 2 about changes of Messaging Topic list, corresponds to clause 9.1.1.6.3 as defined in 3GPP TS 23.554 [2].

5.4.2.4.2	Notification about MSGin5G Messaging Topic List

Figure 5.4.2.4.2-1:MSGin5G Server Notification about Messaging Topic List

As shown in Figure 5.4.2.4.2-1, the MSGin5G Server 1 shall invoke the MSGS_TopiclistEvent_NotifyMSGTopiclist service operation to notify about subscribed Messaging Topic list events. The MSGin5G Server 1 shall send HTTP POST request to the MSGin5G Server 2 with "{notificationURI}" previously received in the subscription. The TopicListNotification data structure provided in the request body shall include:

-	The Messaging Topic list exists in MSGin5G Server 1within the "msgTopics" attribute, this attribute may include:

-	the unique topic identifier within the "msgTopic" attribute;

-	the topic update status indicating if such topic is newly created or deleted within the "updateStat" attribute; and

may include:

-	the Expiration within the "exprTime" attribute;

Upon receiving the HTTP POST message from the MSGin5G Server 1, the MSGin5G Server 2 shall make an authorization based on the information received. If the authorization is successful, the MSGin5G Server 2 shall update locally stored Messaging Topic list, and respond to the MSGin5G Server 1 with an HTTP "204 No Content" status code.

If the MSGin5G Server 2 is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent, as defined in clause 5.2.10 of 3GPP TS 29.122 [3].

If errors occur when processing the HTTP POST request, the MSGin5G Server 2 shall apply error handling procedures as specified in clause 8.3.6.

5.4.2.5	MSGS_TopiclistEvent_SubscribeMSGTopic

5.4.2.5.1	General

This service operation is used by MSGin5G Server 1 to subscribe to Messaging Topic on another MSGin5G Server 2, corresponding to clause 9.1.1.6.4 as defined in 3GPP TS 23.554 [2].

5.4.2.5.2	MSGin5G Server Subscribing to MSGin5G Messaging Topic

Figure 5.4.2.5.2-1:Subscribing to Messaging Topic List

As shown in Figure 5.4.2.5.2-1, to subscribe MSGin5G Messaging Topic on the MSGin5G Server 2, the MSGin5G Server 1 shall send the HTTP POST method. The body of the HTTP POST message shall include TopicSubscription data structure.

Upon receiving the HTTP POST message, the MSGin5G Server 2 shall:

1.	 make an authorization based on the information received;

2.	checks the locally stored Messaging Topic subscription(s):

a.	If the requested subscription has already been created, the MSGin5G Server 2 updates the validity time of this subscription;

b.	If the requested subscription has not been created, the MSGin5G Server 2 creates new subscription; and

If the subscription request is successfully processed, the MSGin5G Server 2 shall respond with an HTTP "200 OK" status code with the response body containing TopicSubscriptionAck data structure.

If the MSGin5G Server 2 is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the MSGin5G Server 2 shall apply error handling procedures as specified in clause 8.3.6.

5.4.2.6	MSGS_TopiclistEvent_UnsubscribeMSGTopic

5.4.2.6.1	General

This service operation is used by MSGin5G Server to unsubscribe to Messaging Topic(s) on another MSGin5G Server, corresponding to clause 9.1.1.6.7 as defined in 3GPP TS 23.554 [2].

5.4.2.6.2	MSGin5G Server Unsubscribing to MSGin5G Messaging Topic

Figure 5.4.2.6.2-1:Unsubscribing to Messaging Topic(s)

As shown in Figure 5.4.2.6.2-1, to unsubscribe one or more MSGin5G Messaging Topic(s) on the MSGin5G Server 2, the MSGin5G Server 1 shall send HTTP POST message to the MSGin5G Server 2. The body of the HTTP POST message shall include TopicUnsubscription data structure.

Upon receiving the HTTP POST message, the MSGin5G Server 2 shall make an authorization based on the information received. If the authorization is successful, the MSGin5G Server 2 shall remove the corresponding subscription and respond with an HTTP "204 No Content" status code.

If the MSGin5G Server 2 is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the MSGin5G Server 2 shall apply error handling procedures as specified in clause 8.3.6.

6	Services offered by the Message Gateway

6.1	Introduction

The Table 6.1-1 lists the services provided by the MSGin5G Message Gateway and corresponding service operations. A service description clause for each API gives a general description of the related API.

Table 6.1-1 List of services provided by the MSGin5G Message Gateway

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| MSGG_L3GDelivery | MSGG_L3GDelivery_GTDelivery | Request/Response | MSGin5G Server |
|  | MSGG_L3GDelivery_GTDeliveryReport |  |  |
| MSGG_N3GDelivery | MSGG_N3GDelivery_GTDelivery | Request/Response | MSGin5G Server |
|  | MSGG_N3GDelivery_GTDeliveryReport |  |  |
| MSGG_BGDelivery | MSGG_BGDelivery_GTDelivery | Request/Response | MSGin5G Server |

Table 6.1-2 summarizes the corresponding APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| MSGG_L3GDelivery | 9.1 | L3G Message Delivery Service | TS29538_MSGG_ L3GDelivery.yaml | msgg-l3gdelivery | A.4 |
| MSGG_N3GDelivery | 9.2 | N3G Message Delivery Service | TS29538_MSGG_ N3GDelivery.yaml | msgg-n3gdelivery | A.5 |
| MSGG_BGDelivery | 9.3 | BMG Message Delivery Service | TS29538_MSGG_ BGDelivery.yaml | msgg-bgdelivery | A.6 |

6.2	MSGG_L3GDelivery Service

6.2.1	Service Description

The MSGG_L3GDelivery Service corresponding to Ml3g as defined in 3GPP TS 23.554 [2], is provided by the Legacy 3GPP Message Gateway.

This service:

-	allows MSGin5G Server invokes services provided by Legacy 3GPP Message Gateway to send MSGin5G Messages to Legacy 3GPP Message Gateway.

6.2.2	Service Operations

6.2.2.1	Introduction

The service operation defined for MSGG_L3GDelivery Service is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the MSGG_L3GDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGG_L3GDelivery_GTDelivery | This service operation is used by MSGin5G Server to deliver MSGin5G message to Legacy 3GPP Message Gateway. This service operation corresponds to clause 9.2.1.1.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |
| MSGG_L3GDelivery_GTDeliveryReport | This service operation is used by MSGin5G Server to deliver the delivery status report to Legacy 3GPP Message Gateway. This service operation corresponds to clause 9.2.1.3.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |

6.2.2.2	MSGG_L3GDelivery_GTDelivery

6.2.2.2.1	General

This service operation corresponds to clause 9.2.1.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Legacy 3GPP Message Gateway.

6.2.2.2.2	Legacy 3GPP Message Gateway Terminating Message Delivery

Figure 6.2.2.2.2-1: Legacy 3GPP Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Legacy 3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in Figure 6.2.2.2.2-1.

The MSGin5G Server shall send a POST request to the resource with an L3gMessageDelivery object in the request body.

The L3gMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the Message is segmented within the "segInd" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

When the Legacy 3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Legacy 3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If the Legacy Message Gateway is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Message Gateway, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the Legacy 3GPP Message Gateway shall apply error handling procedures as specified in clause 9.1.6.

6.2.2.3	MSGG_L3GDelivery_GTDeliveryReport

6.2.2.3.1	General

This service operation corresponds to clause 9.2.1.3.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver the delivery status report to the Legacy 3GPP Message Gateway.

6.2.2.3.2	Legacy 3GPP Message Gateway Terminating Message Delivery Status Report

Figure 6.2.2.3.2-1: Legacy 3GPP Message Gateway Terminating Delivery Status Report

When the MSGin5G Server needs to send the delivery status report to the Legacy 3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-report" resource as shown in Figure 6.2.2.3.2-1.

The MSGin5G Server shall send a POST request to the resource with a DeliveryStatusReport object in the request body.

The DeliveryStatusReport data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the failure cause within the "failureCause" attribute.

When the Legacy 3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Legacy 3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If the Legacy Message Gateway is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Message Gateway, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the Legacy 3GPP Message Gateway shall apply error handling procedures as specified in clause 9.1.6.

6.3	MSGG_N3GDelivery Service

6.3.1	Service Description

The MSGG_N3GDelivery Service corresponding to Mn3g as defined in 3GPP TS 23.554 [2], is provided by the Non-3GPP Message Gateway.

This service:

-	allows MSGin5G Server invokes services provided by Non-3GPP Message Gateway to send MSGin5G Messages to Non-3GPP Message Gateway.

6.3.2	Service Operations

6.3.2.1	Introduction

The service operation defined for MSGG_N3GDelivery Service is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations of the MSGG_N3GDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGG_N3GDelivery_GTDelivery | This service operation is used by MSGin5G Server to deliver MSGin5G message to Non-3GPP Message Gateway. This service operation corresponds to clause 9.2.2.1.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |
| MSGG_N3GDelivery_GTDeliveryReport | This service operation is used by MSGin5G Server to deliver the delivery status report to Non-3GPP Message Gateway. This service operation corresponds to clause 9.2.2.2.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |

6.3.2.2	MSGG_N3GDelivery_GTDelivery

6.3.2.2.1	General

This service operation corresponds to clause 9.2.2.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Non-3GPP Message Gateway.

6.3.2.2.2	Non-3GPP Message Gateway Terminating Message Delivery

Figure 6.3.2.2.2-1: Non-3GPP Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in figure 6.3.2.2.2-1.

The MSGin5G Server shall send a POST request to the resource with an N3gMessageDelivery object in the request body.

The N3gMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the Payload within the "payload" attribute;

-	the Message is segmented within the "segInd" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If the Non-3GPP Message Gateway is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Message Gateway, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in clause 9.2.6.

6.3.2.3	MSGG_N3GDelivery_GTDeliveryReport

6.3.2.3.1	General

This service operation corresponds to clause 9.2.2.2.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver the delivery status report to the Non-3GPP Message Gateway.

6.3.2.3.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report

Figure 6.3.2.3.2-1: Non-3GPP Message Gateway Terminating Delivery Status Report

When the MSGin5G Server needs to send the delivery status report to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-report" resource as shown in figure 6.3.2.3.2-1.

The MSGin5G Server shall send a POST request to the resource with a DeliveryStatusReport object in the request body.

The DeliveryStatusReport data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;

-	the delivery status within the "delivSt" attribute; and

may include:

-	the failure cause within the "failureCause" attribute.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If the Non-3GPP Message Gateway is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Message Gateway, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in clause 9.2.6.

6.4	MSGG_BGDelivery Service

6.4.1	Service Description

The MSGG_BGDelivery Service corresponding to Mbg as defined in 3GPP TS 23.554 [2], is provided by the Broadcast Message Gateway.

This service:

-	allows MSGin5G Server invokes services provided by Broadcast Message Gateway to deliver MSGin5G messages to Broadcast Message Gateway.

6.4.2	Service Operations

6.4.2.1	Introduction

The service operation defined for MSGG_BGDelivery Service is shown in the table 6.4.2.1-1.

Table 6.4.2.1-1: Operations of the MSGG_BGDelivery Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MSGG_BGDelivery_GTDelivery | This service operation is used by MSGin5G Server to deliver MSGin5G message to Broadcast Message Gateway. This service operation corresponds to clause 9.2.3.1.2 as defined in 3GPP TS 23.554 [2]. | MSGin5G Server |

6.4.2.2	MSGG_BGDelivery_GTDelivery

6.4.2.2.1	General

This service operation corresponds to clause 9.2.3.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Broadcast Message Gateway.

6.4.2.2.2	Broadcast Message Gateway Terminating Message Delivery

Figure 6.4.2.2.2-1: Broadcast Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Broadcast Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in Figure 6.4.2.2.2-1.

The MSGin5G Server shall send a POST request to the resource with an BgMessageDelivery object in the request body.

The BgMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Message ID within the "msgId" attribute;

may include:

-	the Recipient ID within the "destAddr" attribute;

-	the Payload within the "payload" attribute; and

-	the Application ID within the "appId" attribute; and

-	the indication whether the message delivery status report is required within the "delivStReqInd" attribute;

-	the priority type within the "priority" attribute;

-	the message segment flag within the "segInd" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

When the Broadcast Message Gateway receives the HTTP POST request from the MSGin5G Server, the Broadcast Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If the Broadcast Message Gateway is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative target URI of the custom operation located in an alternative MSGin5G Message Gateway, as defined in clause 5.2.10 of 3GPP TS 29.122 [2].

If errors occur when processing the HTTP POST request, the Broadcast Message Gateway shall apply error handling procedures as specified in clause 9.3.6.

7	Common information applicable to several APIs

7.1	General

MSGin5G APIs allow secure access to the capabilities provided by the MSGin5G.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.554 [2].

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.122 [24] and referenced by this specification.

7.2	Data Types

7.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI Specification [6] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data types in the present specification are written with an upper-case letter in the beginning. Parameters are written with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI Specification [6] can use a lower-case case letter in the beginning for consistency.

Table 7.2.1-1 specifies data types re-used by the MSGin5G from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the APIs of this specification.

Table 7.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |

7.2.2	Referenced structured data types

Table 7.2.2-1 lists structured data types defined in this specification referenced by multiple services.

Table 7.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| Address | Clause 9.1.5.2.3 | Represent an address |
| DeliveryStatusReport | Clause 8.2.5.2.7 | The message delivery status report request information. |
| MessageSegmentParameters | Clause 8.2.5.2.5 | Parameters for message segmentation |

7.2.3	Referenced Simple data types and enumerations

Following simple data types defined in Table 7.2.3.1-1 are applicable to several APIs in this document:

Table 7.2.3.1-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

7.3	Usage of HTTP

For MSGin5G APIs, support of HTTP/1.1 (IETF RFC 9110 [11], IETF RFC 9111 [14] and IETF RFC 9112 [10]) over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [16]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 9113 [16].

7.4	Content type

JSON, IETF RFC 8259 [17], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.2.3 of 3GPP TS 29.122 [24]. The use of the JSON format shall be signalled by the content type "application/json".

7.5	URI structure

7.5.1	Resource URI structure

The resource URI structure of all the APIs specified in this document shall be as specified in clause 5.2.4.1 of 3GPP TS 29.122 [24].

7.5.2	Custom operations URI structure

The URI of a custom operation which is associated with a resource shall be as specified in clause 5.2.4.2 of 3GPP TS 29.122 [24].

7.6	Notifications

None.

7.7	Error Handling

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [24].

7.8	Feature negotiation

The procedures in clause 5.2.7 of 3GPP TS 29.122 [24] shall be applicable for the APIs defined in the present specification. For each of the APIs defined, the applicable list of features is contained in the related API definition.

7.9	HTTP headers

The MSGin5G API shall support HTTP custom header fields specified in clause 5.2.8 of 3GPP TS 29.122 [24]. No specific custom headers are defined for the MSGin5G API in the present specification.

7.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [24] shall be applicable for all APIs in this document.

8	Message Server API definition

8.1	MSGS_ASRegistration API

8.1.1	API URI

The MSGS_ASRegistration service shall use the MSGS_ASRegistration API.

The request URIs used in HTTP requests from the AS towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs-asregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.1.2.

8.1.2	Resources

8.1.2.1	Overview

Figure 8.1.2.1-1: Resource URI structure of the MSGS_ASRegistration API

Table 8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AS Registrations | /registrations | POST | Registers a new AS at the MSGin5G Server. |
| AS DeRegistration | /registrations/{registrationId} | DELETE | Removes an AS registration resource. |

8.1.2.2	Resource: AS Registrations

8.1.2.2.1	Description

This resource represents all the Application Servers that are registered at a given MSGin5G Server.

8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/msgs-asregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in the table 8.1.2.2.2-1.

Table 8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |

8.1.2.2.3	Resource Standard Methods

8.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.1.2.2.3.1-1.

Table 8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.2.3.1-2 and the response data structures and response codes specified in table 8.1.2.2.3.1-3.

Table 8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASRegistration | M | 1 | AS registration request information. |

Table 8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | 201 Created | AS information is registered successfully at MSGin5G Server. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.1.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.1.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/msgs-asregistration/<apiVersion>/registrations/{registrationId} |

Table 8.1.2.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

8.1.2.3	Resource: AS DeRegistration

8.1.2.3.1	Description

This resource represents all the Application Servers that are deregistered at a given MSGin5G Server.

8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/msgs-asregistration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 8.1.2.3.2-1.

Table 8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.1.1 |
| registrationId | string | The AS registration resource id |

8.1.2.3.3	Resource Standard Methods

8.1.2.3.3.1	DELETE

This method deregisters an AS registration from the MSGin5G Server. This method shall support the URI query parameters specified in the table 8.1.2.3.3.1-1.

Table 8.1.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.1.2.3.3.1-2 and the response data structures and response codes specified in table 8.1.2.3.3.1-3.

Table 8.1.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.1.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ASRegistrationAck | M | 1 | 200 OK | The AS DeRegistration information matching the registrationId is deleted. |
| n/a |  |  | 204 No Content | Successful response. The individual AS registration matching the registrationId is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.1.3	Custom Operations without associated resources

None.

8.1.4	Notifications

None.

8.1.5	Data Model

8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API Table 8.1.5.1-1 specifies the data types defined specifically for the MSGS_ASRegistration API service.

Table 8.1.5.1-1: MSGS_ASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASRegistration | 8.1.5.2.2 | The AS registration request information. |  |
| ASRegistrationAck | 8.1.5.2.3 | The AS registration response information. |  |
| ASProfile | 8.1.5.2.4 | The profile information related to the AS in the ASRegistration data type. |  |

Table 8.1.5.1-2 specifies data types re-used by the MSGS_ASRegistration API service.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 3GPP TS 29.571 [5] |  |  |
| Uri | 3GPP TS 29.571 [5] |  |  |

8.1.5.2	Structured data types

8.1.5.2.1	Introduction

8.1.5.2.2	Type: ASRegistration

Table 8.1.5.2.2-1: Definition of type ASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSvcId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| appId | string | O | 0..1 | The identifier of the application specified by the application provider |  |
| targetUri | Uri | O | 0..1 | The URL for receiving message, message delivery status report, etc. The MSGin5G Server uses this URL to interact to AS. |  |
| reqExprTime | DateTime | O | 0..1 | The requested expiration time for the registration. |  |
| exGroupList | array(string) | O | 1..N | The list of group service id of MSGin5G groups which the Application Server will control. |  |
| asProf | ASProfile | O | 0..1 | The profile information of the AS. |  |

8.1.5.2.3	Type: ASRegistrationAck

Table 8.1.5.2.3-1: Definition of type ASRegistrationAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| asSvcId | string | M | 1 | The MSGin5G identifier of the Application Server. |  |
| result | ProblemDetails | M | 1 | The result of the registration. |  |
| regExprTime | DateTime | O | 0..1 | The expiration time of the registration. |  |

8.1.5.2.4	Type: ASProfile

Table 8.1.5.2.4-1: Definition of type ASProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appName | string | O | 1 | The name of the Application Server |  |
| appProviders | array(string) | O | 1..N | The provider of the Application Server |  |
| appSenarios | array(string) | O | 1..N | The application scenario description. |  |
| appCategory | string | O | 0..1 | The category or type of Application Server. |  |
| asStatus | string | O | 0..1 | AS status (e.g. Enabled, Disabled etc.) |  |

8.1.5.3	Simple data types and enumerations

None.

8.1.6	Error Handling

8.1.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

8.1.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGS_ASRegistration API.

8.1.6.3	Application Errors

The application errors defined for the MSGS_ASRegistration API are listed in table 8.1.6.3-1.

Table 8.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

8.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.1.7-1 lists the supported features for MSGS_ASRegistration API.

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

8.2	MSGS_MSGDelivery API

8.2.1	API URI

The MSGS_MSGDelivery service shall use the MSGS_MSGDelivery API, The MSGS_MSGDelivery API corresponding to Mm5s APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server towards the MSGin5G Server shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs-msgdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.2.2.

8.2.2	Resources

None.

8.2.3	Custom Operations without associated resources

8.2.3.1	Overview

The structure of the custom operation URIs of the MSGS_MSGDelivery service is shown in Figure 8.2.3.1-1.

Figure 8.2.3.1-1: Custom operation URI structure of the MSGS_MSGDelivery API

Table 8.2.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 8.2.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-as-message | POST | Request of AS to deliver message to a given MSGin5G Server. |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-ue-message | POST | Request of Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message to a given MSGin5G Server. |
| {apiRoot}/msgs-msgdelivery/<apiVersion>/deliver-report | POST | Request of Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver status report to a given MSGin5G Server. |

8.2.3.2	Operation: deliver-as-message

8.2.3.2.1	Description

This operation is used by the Application Server to deliver message to a given MSGin5G Server.

8.2.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.2.2-1 and Table 8.2.3.2.2-2.

Table 8.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ASMessageDelivery | M | 1 | Represents the data to be used for AS to deliver message. |

Table 8.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | AS Message is delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.2.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.2.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.2.3.3	Operation: deliver-ue-message

8.2.3.3.1	Description

This operation is used by the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message to a given MSGin5G Server.

8.2.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.3.2-1 and Table 8.2.3.3.2-2.

Table 8.2.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UEMessageDelivery | M | 1 | Represents the data to be used for Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) to deliver message. |

Table 8.2.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | UE Message is delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.2.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.2.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.2.3.4	Operation: deliver-report

8.2.3.4.1	Description

This operation is used by the Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server to deliver status report to a given MSGin5G Server.

8.2.3.4.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 8.2.3.4.2-1 and Table 8.2.3.4.2-2.

Table 8.2.3.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for Message Gateway (on behalf of Legacy 3GPP UE or Non-3GPP UE) or the Application Server to deliver status report. |

Table 8.2.3.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MessageDeliveryAck | M | 1 | 200 OK | The status report is delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.2.3.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.2.3.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.2.4	Notifications

None.

8.2.5	Data Model

8.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 8.2.5.1-1 specifies the data types defined specifically for the MSGS_MSGDelivery API service.

Table 8.2.5.1-1: API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ASMessageDelivery | 8.2.5.2.2 | The AS message delivery request information. |  |
| DeliveryStatus | 8.2.5.3.3 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| DeliveryStatusReport | 8.2.5.2.7 | The message delivery status report request information. |  |
| MessageDeliveryAck | 8.2.5.2.4 | The message delivery response information. |  |
| MessageSegmentParameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |
| Priority | 8.2.5.3.5 | Application priority level requested for this message. |  |
| ReportDeliveryStatus | 8.2.5.3.4 | The delivery status description, including success or failure in delivery. |  |
| StoreAndForwardParameters | 8.2.5.2.6 | Contains the store forward information of the message. |  |
| UEMessageDelivery | 8.2.5.2.3 | The UE message delivery request information. |  |

Table 8.2.5.1-2 specifies data types re-used by the MSGS_MSGDelivery API service.

Table 8.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Address | 9.1.5.2.3 | The data type of the oriAddr and destAddr. |  |
| DateTime | 3GPP TS 29.571 [5] | String with format "date-time" as defined in OpenAPI Specification [6]. |  |

8.2.5.2	Structured data types

8.2.5.2.1	Introduction

8.2.5.2.2	Type: ASMessageDelivery

Table 8.2.5.2.2-1: Definition of type ASMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Application Server (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or MSGin5G UE. The service identifier of the target MSGin5G Group. The service identifier of the Broadcast Service Area where the message needs to be broadcast. Indicates which Messaging Topic this message is related to. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the content is intended. This list of Application IDs attribute is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| priority | Priority | O | 0..1 | Application priority level requested for this message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | C | 0..1 | The message segment parameters. This attribute shall be included only if the value of the message Segment Flag attribute indicates that message Segment services are requested. |  |
| stoAndFwInd | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. Set to "true" if it is required to store and forward services for this message. otherwise set to "false". |  |
| stoAndFwParams | StoreAndForwardParameters | C | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This attribute shall be included only if the value of the Store and forward flag attribute indicates that store and forward services are requested. |  |
| latency | integer | O | 0..1 | The latency requirement for the message which only applies to AS Originating MSGin5G Message. Unit: millisecond. |  |
| notiUrl | string | O | 0..1 | The URL for MSGin5G Server to send the MSGin5G response notification |  |
| NOTE: Only "AS" is applicable to the addrType attribute in the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.3	Type:UEMessageDelivery

Table 8.2.5.2.3-1: Definition of type UEMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE or Non-3GPP UE (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Application Server or MSGin5G UE. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the content is intended. This list of Application IDs attribute is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | C | 0..1 | The message segment parameters. This attribute shall be included only if the value of the message Segment Flag attribute indicates that message Segment services are requested. |  |
| stoAndFwInd | boolean | M | 1 | An indicator of whether store and forward services are requested for this message. Set to "true" if it is required to store and forward services for this message. otherwise set to "false". |  |
| stoAndFwParams | StoreAndForwardParameters | C | 0..1 | Parameters used by MSGin5G Server for providing store and forward services, This attribute shall be included only if the value of the Store and forward flag attribute indicates that store and forward services are requested. |  |
| NOTE: Only "UE" is applicable to the addrType attribute in the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.4	Type: MessageDeliveryAck

Table 8.2.5.2.4-1: Definition of type MessageDeliveryAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server (NOTE). |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| status | DeliveryStatus | O | 0..1 | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |  |
| failureCause | string | C | 0..1 | The reason for failure. May only be present if the "status" attribute is set to "DELY_FAILED". |  |
| NOTE: Either "UE" or "AS" shall be included in the "addrType" attribute within the Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.2.5	Type:MessageSegmentParameters

Table 8.2.5.2.5-1: Definition of type MessageSegmentParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| segId | string | O | 0..1 | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. |  |
| totalSegCount | integer | O | 0..1 | Indicates the total number of segments for the message. |  |
| segNumb | integer | O | 0..1 | Indicates segmented message number of each segmented message within a set of segmented messages. |  |
| lastSegFlag | boolean | O | 0..1 | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. Set to "true" if the segmented message is the last segment in the set of segmented messages. otherwise set to "false". Default value is "false". |  |

8.2.5.2.6	Type:StoreAndForwardParameters

Table 8.2.5.2.6-1: Definition of type StoreAndForwardParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exprTime | DateTime | O | 0..1 | Indicates message expiration time. |  |

8.2.5.2.7	Type:DeliveryStatusReport

Table 8.2.5.2.7-1: Definition of type DeliveryStatusReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the sending Legacy 3GPP UE, Non-3GPP UE or the sending Application Server (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server (NOTE). |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| failureCause | string | C | 0..1 | The Failure Cause indicates the failure reason, if applicable. May only be present if the ReportDeliveryStatus sets to "REPT_DELY_FAILED". |  |
| delivSt | ReportDeliveryStatus | M | 1 | The delivery status description, including success or failure in delivery. |  |
| NOTE: Either "UE" or "AS" shall be included in the addrType attribute in Address data type to represent the originating type of message request. |  |  |  |  |  |

8.2.5.3	Simple data types and enumerations

8.2.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.2.5.3.2	Simple data types

None.

8.2.5.3.3	Enumeration: DeliveryStatus

Table 8.2.5.3.3-1: Enumeration DeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DELY_FAILED | Indicates that the message delivery is failed. |  |
| DELY_STORED | Indicates that the message is stored for deferred delivery. |  |

8.2.5.3.4	Enumeration: ReportDeliveryStatus

Table 8.2.5.3.4-1: Enumeration ReportDeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REPT_DELY_SUCCESS | Indicates that the report delivery is successful. |  |
| REPT_DELY_FAILED | Indicates that the report delivery is failed. |  |

8.2.5.3.5	Enumeration:Priority

Table 8.2.5.3.5-1: Enumeration Priority

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HIGH | Indicates the messages should be sent in high priority. |  |
| MIDDLE | Indicates the messages should be sent in middle priority. |  |
| LOW | Indicates the messages should be sent in low priority. |  |

8.2.6	Error Handling

8.2.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

8.2.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGS_MSGDelivery API.

8.2.6.3	Application Errors

The application errors defined for the MSGS_MSGDelivery API are listed in table 8.2.6.3-1.

Table 8.2.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

8.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.2.7-1 lists the supported features for MSGS_MSGDelivery API.

Table 8.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

8.3	MSGS_TopiclistEvent API

8.3.1	API URI

The MSGS_TopiclistEvent service shall use the MSGS_TopiclistEvent API.

The request URIs used in HTTP requests from the MSGin5G Server 1 towards the MSGin5G Server 2 shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgs-topiclistevent".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 8.3.2.

8.3.2	Resources

8.3.2.1	Overview

Figure 8.3.2.1-1: Resource URI structure of the MSGS_TopiclistEvent API

Table 8.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Topic List Subscriptions | /topiclist-subscriptions | POST | Subscribe to a Messaging Topic list on a MSGin5G Server. |
| Individual Topic List Subscription | /topiclist-subscriptions/{subscriptionId} | DELETE | Delete an existing "Individual Topic List Subscription” resource. |

8.3.2.2	Resource: Topic List Subscriptions

8.3.2.2.1	Description

This resource represents all the MSGin5G Servers that are subscribed at a given Messaging Topic list on a MSGin5G Server.

8.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/msgs-topiclistevent/<apiVersion>/topiclist-subscriptions

This resource shall support the resource URI variables defined in the table 8.3.2.2.2-1.

Table 8.3.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.3.1 |

8.3.2.2.3	Resource Standard Methods

8.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 8.3.2.2.3.1-1.

Table 8.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.3.2.2.3.1-2 and the response data structures and response codes specified in table 8.3.2.2.3.1-3.

Table 8.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TopicListSubscription | M | 1 | Messaging Topic list subscription request information. |

Table 8.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TopicListSubscription | M | 1 | 201 Created | Successful case. The creation of an Messaging Topic list subscription resource is confirmed, and the representation of the created "Individual Topic List Subscription" resource is returned in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.3.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 8.3.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/msgs-topiclistevent/<apiVersion>/topiclist-subscriptions/{subscriptionId} |

8.3.2.3	Resource: Individual Topic List Subscription

8.3.2.3.1	Description

The Individual Topic List Subscription resource represents single Messaging Topic list subscription at a given MSGin5G Server. The resource allows an MSGin5G Server to delete Individual Topic List Subscription resource.

8.3.2.3.2	Resource Definition

Resource URI: {apiRoot}/msgs-topiclistevent/<apiVersion>/topiclist-subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 8.3.2.3.2-1.

Table 8.3.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.5 |
| apiVersion | string | See clause 8.3.1 |
| subscriptionId | string | The MSGin5G Server Messaging Topic list subscription resource id |

8.3.2.3.3	Resource Standard Methods

8.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 8.3.2.3.3.1-1.

Table 8.3.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 8.3.2.3.3.1-2 and the response data structures and response codes specified in table 8.3.2.3.3.1-3.

Table 8.3.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.3.2.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful response. The "Individual Topic List Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.3.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.3.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.3.3	Custom Operations without associated resources

8.3.3.1	Overview

The structure of the custom operation URIs of the MSGS_MSGTopiclistEvent service is shown in Figure 8.3.3.1-1.

Figure 8.3.3.1-1: Custom operation URI structure of the MSGS_TopiclistEvent API

Table 8.3.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 8.3.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgs-topiclistevent/<apiVersion>/request-topic-subscription | POST | Request of MSGin5G Server to deliver Messaging Topic subscription to another given MSGin5G Server. |
| {apiRoot}/msgs-topiclistevent/<apiVersion>/request-topic-unsubscription | POST | Request of MSGin5G Server to deliver Messaging Topic unsubscription to another given MSGin5G Server. |

8.3.3.2	Operation: request-topic-subscription

8.3.3.2.1	Description

The operation is used by the MSGin5G Server to request the other MSGin5G Server to create a subscription for one or more Messaging Topic(s).

8.3.3.2.2	Operation Definition

This operation shall support the request data structures shown in Table 8.3.3.2.2-1 and the response data structures and error codes specified in Tables 8.3.3.2.2-2.

Table 8.3.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TopicSubscription | M | 1 | Information about the Messaging Topic subscription that the MSGin5G Server shall create. |

Table 8.3.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TopicSubscriptionAck | M | 1 | 200 OK | Successful request to trigger the creation of a subscription for Messaging Topic at the MSGin5G Server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.3.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.3.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.3.3.3	Operation: request-topic-unsubscription

8.3.3.3.1	Description

The operation is used by the MSGin5G Server to request the other MSGin5G Server to remove a subscription for one or more Messaging Topic(s).

8.3.3.3.2	Operation Definition

This operation shall support the request data structures shown in Table 8.3.3.3.2-1 and the response data structures and error codes specified in Tables 8.3.3.3.2-2.

Table 8.3.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TopicUnsubscription | M | 1 | Reference used to identify the Messaging Topic subscription that the MSGin5G Server shall remove. |

Table 8.3.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful request to trigger the removal of a subscription for Messaging Topic(s) on the MSGin5G Server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.3.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

Table 8.3.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Server. |

8.3.4	Notifications

Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [24].

Table 8.3.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| TopicList Notification | {notificationURI} | POST | Notify about Messaging Topic list changes from MSGin5G Server. |

8.3.4.2	Topiclist Notification

8.3.4.2.1	Description

The Topiclist Notification is used by the MSGin5G Server to notify Messaging Topic list change events to another MSGin5G Server that has subscribed to such Messaging Topic list.

8.3.4.2.2	Target URI

The Callback URI "{notificationURI}" shall be used with the callback URI variables defined in table 8.3.4.2.2-1.

Table 8.3.4.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationURI | String formatted as URI with the Callback Uri |

8.3.4.2.3	Standard Methods

8.3.4.2.3.1	POST

This method shall support the request data structures specified in table 8.3.4.2.3.1-1 and the response data structures and response codes specified in table 8.3.4.2.3.1-2.

Table 8.3.4.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TopicListNotification | M | 1 | Provides information about subscribed Messaging Topic list. |

Table 8.3.4.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 8.3.4.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 8.3.4.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

8.3.5	Data Model

8.3.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API Table 8.3.5.1-1 specifies the data types defined specifically for the MSGS_TopiclistEvent API service.

Table 8.3.5.1-1: TopiclistEvent API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| TopicListSubscription | 8.3.5.2.2 | The Messaging Topic list subscription request information. |  |
| TopicListUnsubscriptionAck | 8.3.5.2.5 | The Messaging Topic list unsubscription response information, indicating the subscription result. |  |
| TopicSubscription | 8.3.5.2.6 | The Messaging Topic subscription request information. |  |
| TopicSubscriptionAck | 8.3.5.2.7 | The Messaging Topic subscription response information. |  |
| TopicUnsubscription | 8.3.5.2.8 | The Messaging Topic list unsubscription request information. |  |
| TopicListNotification | 8.3.5.2.9 | The Messaging Topic list notification information, indicating changed Messaging Topics. |  |
| MessagingTopic | 8.3.5.2.10 | The individual Messaging Topic information |  |

Table 8.3.5.1-2 specifies data types re-used by the MSGS_TopiclistEvent API service.

Table 8.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.571 [5] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [5] | Used to negotiate the optional features defined in table 8.3.7-1. |  |

8.3.5.2	Structured data types

8.3.5.2.1	Introduction

8.3.5.2.2	Type: TopicListSubscription

Table 8.3.5.2.2-1: Definition of type TopicListSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the MSGin5G Server which requests the Messaging Topic list subscription. |  |
| destAddr | Address | M | 1 | The service identity of the receiving MSGin5G Server. |  |
| notificationURI | Uri | M | 1 | Notification target address. |  |
| exprTime | DateTime | O | 0..1 | Contains the expiration time of this subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features as described in clause 8.3.7. It shall be present if at least one feature defined in clause 8.3.7 is supported. |  |

8.3.5.2.3	Void

8.3.5.2.4	Void

8.3.5.2.5	Type: TopicListUnsubscriptionAck

Table 8.3.5.2.5-1: Definition of type TopicListSubscriptionAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subStat | string | M | 1 | The result of the subscription. |  |

8.3.5.2.6	Type: TopicSubscription

Table 8.3.5.2.6-1: Definition of type TopicSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | string | M | 1 | The Service identifier of the originating MSGin5G Client or Application Server, or MSGin5G Server |  |
| msgTopics | array(string) | M | 1..N | Information of Messaging Topic(s) to be subscribed |  |
| exprTime | DateTime | O | 0..1 | The expiration time of this subscription requested, and should be larger than 0 if included. If not included, the expiration time subjects to operator policy. |  |

8.3.5.2.7	Type: TopicSubscriptionAck

Table 8.3.5.2.7-1: Definition of type TopicSubscriptionAck

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subStat | string | M | 1 | Indicating update status of the Messaging Topic if newly created or deleted. |  |
| exprTime | DateTime | O | 0..1 | The expiration time of this subscription set by MSGin5G Server. |  |

8.3.5.2.8	Type: TopicUnsubscription

Table 8.3.5.2.8-1: Definition of type TopicUnsubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | string | M | 0..1 | The Service identifier of the originating MSGin5G Client or Application Server, or MSGin5G Server. |  |
| msgTopics | array(string) | M | 1..N | Information of Messaging Topic(s) to be unsubscribed. |  |

8.3.5.2.9	Type: TopicListNotification

Table 8.3.5.2.9-1: Definition of type TopicListNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exprTime | DateTime | O | 0..1 | The expiration time of this subscription set by MSGin5G Server. |  |
| msgTopics | array(MessagingTopic) | M | 1..N | List of Messaging Topic(s) information notified by MSGin5G Server. |  |

8.3.5.2.10	Type: MessagingTopic

Table 8.3.5.2.10-1: Definition of type MessagingTopic

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| msgTopic | string | M | 1 | The identifier of the Messaging Topic. |  |
| updateStat | UpdateStatus | M | 1 | Indicating update status of the Messaging Topic if newly created or deleted. |  |

8.3.5.3	Simple data types and enumerations

8.3.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

8.3.5.3.2	Enumeration: UpdateStatus

Table 8.3.5.3.2-1: Enumeration UpdateStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CREATED | The Messaging Topic is newly created. |  |
| DELETED | The Messaging Topic is newly deleted. |  |

8.3.6	Error Handling

8.3.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

8.3.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGS_TopiclistEvent API.

8.3.6.3	Application Errors

The application errors defined for the MSGS_TopiclistEvent API are listed in table 8.3.6.3-1.

Table 8.3.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

8.3.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 8.3.7-1 lists the supported features for MSGS_TopiclistEvent API.

Table 8.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9	Message Gateway API definition

9.1	MSGG_L3GDelivery API

9.1.1	API URI

The MSGG_L3GDelivery service shall use the MSGG_L3GDelivery API, The MSGG_L3GDelivery API corresponding to Ml3g APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the MSGin5G Server towards the Legacy 3GPP Message Gateway shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgg-l3gdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.1.3.

9.1.2	Resources

None.

9.1.3	Custom Operations without associated resources

9.1.3.1	Overview

The structure of the custom operation URIs of the MSGG_L3GDelivery service is shown in Figure 9.1.3.1-1.

Figure 9.1.3.1-1: Custom operation URI structure of the MSGG_L3GDelivery API

Table 9.1.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 9.1.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgg-l3gdelivery/<apiVersion>/deliver-message | POST | Request of MSGin5G Server to deliver message to a given Legacy 3GPP Message Gateway. |
| {apiRoot}/msgg-l3gdelivery/<apiVersion>/deliver-report | POST | Request of MSGin5G Server to deliver status report to a given Legacy 3GPP Message Gateway. |

9.1.3.2	Operation: deliver-message

9.1.3.2.1	Description

This operation is used by the MSGin5G Server to deliver message to a given Legacy 3GPP Message Gateway.

9.1.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.1.3.2.2-1 and table 9.1.3.2.2-2.

Table 9.1.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| L3gMessageDelivery | M | 1 | Represents the data to be used for MSGin5G Server to deliver message. |

Table 9.1.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Message is Delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 9.1.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

Table 9.1.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

9.1.3.3	Operation: deliver-report

9.1.3.3.1	Description

This operation is used by the MSGin5G Server to deliver status report to a given Legacy 3GPP Message Gateway.

9.1.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in Table 9.1.3.3.2-1 and Table 9.1.3.3.2-2.

Table 9.1.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for MSGin5G Server to deliver status report. |

Table 9.1.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The status report is Delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 9.1.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

Table 9.1.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

9.1.4	Notifications

None.

9.1.5	Data Model

9.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 9.1.5.1-1 specifies the data types defined specifically for the MSGG_L3GDelivery API service.

Table 9.1.5.1-1: MSGG_L3GDelivery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| L3gMessageDelivery | 9.1.5.2.2 | Information within message delivery request. |  |

Table 9.1.5.1-2 specifies data types re-used by the MSGG_L3GDelivery API service.

Table 9.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DeliveryStatusReport | 8.2.5.2.7 | The message delivery status report request information. |  |
| MessageSegmentParameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |

9.1.5.2	Structured data types

9.1.5.2.1	Introduction

9.1.5.2.2	Type: L3gMessageDelivery

Table 9.1.5.2.2-1: Definition of type L3gMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the originating MSGin5G Client or the originating Application Server. This attribute is copied from the associated inbound message (NOTE). |  |
| destAddr | Address | M | 1 | The service identity of the receiving entity. The receiving entity can only be Legacy 3GPP UE Service ID in MSGG_L3GDelivery API. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the content is intended. This list of Application IDs attribute is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. This attribute is copied from the associated inbound message request |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. This attribute is copied from the associated inbound message. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. This attribute is copied from the associated inbound message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | C | 0..1 | The message segment parameters. This attribute shall be included only if the value of segInd is true to indicate that message Segment services are requested. |  |
| NOTE: The addrType in Address data type shall only include AS or UE to represent the originating of message request. |  |  |  |  |  |

9.1.5.2.3	Type: Address

Table 9.1.5.2.3-1: Definition of type Address

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| addrType | AddressType | M | 1 | Represent the type of message request. |  |
| addr | string | M | 1 | Refer to UE Service ID or AS Service ID or Group Service ID or Broadcast Area ID or Messaging Topic. |  |

9.1.5.3	Simple data types and enumerations

9.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

9.1.5.3.2	Enumeration: AddressType

Table 9.1.5.3.2-1: Enumeration AddressType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE | The address type is UE. |  |
| AS | The address type is AS. |  |
| GROUP | The address type is GROUP. |  |
| BC | The address type is BC. |  |
| TOPIC | The address type is TOPIC. |  |

9.1.6	Error Handling

9.1.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

9.1.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGG_L3GDelivery API.

9.1.6.3	Application Errors

The application errors defined for the MSGG_L3GDelivery API are listed in table 9.1.6.3-1.

Table 9.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

9.1.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.1.7-1 lists the supported features for MSGG_L3GDelivery API.

Table 9.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9.2	MSGG_N3GDelivery API

9.2.1	API URI

The MSGG_N3GDelivery service shall use the MSGG_N3GDelivery API, The MSGG_N3GDelivery API corresponding to Mn3g APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the MSGin5G Server towards the Non-3GPP Message Gateway shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgg-n3gdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.2.3.

9.2.2	Resources

None.

9.2.3	Custom Operations without associated resources

9.2.3.1	Overview

The structure of the custom operation URIs of the MSGG_N3GDelivery service is shown in Figure 9.2.3.1-1.

Figure 9.2.3.1-1: Custom operation URI structure of the MSGG_N3GDelivery API

Table 9.2.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 9.2.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgg-n3gdelivery/<apiVersion>/deliver-message | POST | Request of MSGin5G Server to deliver message to a given Non-3GPP Message Gateway |
| {apiRoot}/msgg-n3gdelivery/<apiVersion>/deliver-report | POST | Request of MSGin5G Server to deliver status report to a given Non-3GPP Message Gateway |

9.2.3.2	Operation: deliver-message

9.2.3.2.1	Description

This operation is used by the MSGin5G Server to deliver message to a given Non-3GPP Message Gateway.

9.2.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.2.3.2.2-1 and table 9.2.3.2.2-2.

Table 9.2.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N3gMessageDelivery | M | 1 | Represents the data to be used for MSGin5G Server to deliver message. |

Table 9.2.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Message is Delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 9.2.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

Table 9.2.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

9.2.3.3	Operation: deliver-report

9.2.3.3.1	Description

This operation is used by the MSGin5G Server to deliver status report to a given Non-3GPP Message Gateway.

9.2.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.2.3.3.2-1 and table 9.2.3.3.2-2.

Table 9.2.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DeliveryStatusReport | M | 1 | Represents the data to be used for MSGin5G Server to deliver status report. |

Table 9.2.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The status report is Delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 9.2.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

Table 9.2.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

9.2.4	Notifications

None.

9.2.5	Data Model

9.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 9.2.5.1-1 specifies the data types defined specifically for the MSGG_N3GDelivery API service.

Table 9.2.5.1-1: MSGG_N3GDelivery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| N3gMessageDelivery | 9.2.5.2.2 | Information within message delivery request. |  |

Table 9.2.5.1-2 specifies data types re-used by the MSGG_N3GDelivery API service.

Table 9.2.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DeliveryStatusReport | 8.2.5.2.7 | The message delivery status report request information. |  |
| MessageSegmentParameters | 8.2.5.2.5 | Contains the message segment information of the message. |  |
| Address | 9.1.5.2.3 | The data type of the oriAddr and destAddr. |  |

9.2.5.2	Structured data types

9.2.5.2.1	Introduction

9.2.5.2.2	Type: N3gMessageDelivery

Table 9.2.5.2.2-1: Definition of type N3gMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The oriAddr is the service identity of the originating MSGin5G Client or the originating Application Server. This attribute is copied from the associated inbound message (NOTE). |  |
| destAddr | Address | M | 1 | The destAddr is the service identity of the receiving entity. The receiving entity can only be Non-3GPP UE Service ID in MSGG_N3GDelivery API. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the content is intended. This list of Application IDs attribute is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. This attribute is copied from the associated inbound message request. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. This attribute is copied from the associated inbound message. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. This attribute is copied from the associated inbound message. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | C | 0..1 | The message segment parameters. This attribute shall be included only if the value of segInd is true to indicate that message Segment services are requested. |  |
| NOTE: The addrType in Address data type shall only include AS or UE to represent the originating of message request. |  |  |  |  |  |

9.2.6	Error Handling

9.2.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

9.2.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGG_N3GDelivery API.

9.2.6.3	Application Errors

The application errors defined for the MSGG_N3GDelivery API are listed in table 9.2.6.3-1.

Table 9.2.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

9.2.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.2.7-1 lists the supported features for MSGG_N3GDelivery API.

Table 9.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

9.3	MSGG_BGDelivery API

9.3.1	API URI

The MSGG_BGDelivery service shall use the MSGG_BGDelivery API, The MSGG_BGDelivery API corresponding to Mbg APIs as defined in in 3GPP TS 23.554 [2].

The request URIs used in HTTP requests from the MSGin5G Server towards the Broadcast Message Gateway shall have the Resource URI structure as defined in clause 7.5 with the following clarifications:

-	The <apiName> shall be "msgg-bgdelivery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 9.3.3.

9.3.2	Resources

None.

9.3.3	Custom Operations without associated resources

9.3.3.1	Overview

The structure of the custom operation URIs of the MSGG_BGDelivery service is shown in Figure 9.3.3.1-1.

Figure 9.3.3.1-1: Custom operation URI structure of the MSGG_BGDelivery API

Table 9.3.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 9.3.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/msgg-bgdelivery/<apiVersion>/deliver-message | POST | Request of MSGin5G Server to deliver message to a given Broadcast Message Gateway. |

9.3.3.2	Operation: deliver-message

9.3.3.2.1	Description

This operation is used by the MSGin5G Server to deliver message to a given Broadcast Message Gateway.

9.3.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in table 9.3.3.2.2-1 and table 9.3.3.2.2-2.

Table 9.3.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BgMessageDelivery | M | 1 | Represents the data to be used for MSGin5G Server to deliver message. |

Table 9.3.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Message is delivered successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MSGin5G Message Gateway. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [24]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [24] shall also apply. |  |  |  |  |

Table 9.3.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

Table 9.3.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative MSGin5G Message Gateway. |

9.3.4	Notifications

None.

9.3.5	Data Model

9.3.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API. Table 9.3.5.1-1 specifies the data types defined specifically for the MSGG_BGDelivery API service.

Table 9.3.5.1-1: MSGG_BGDelivery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| BgMessageDelivery | 9.3.5.2.2 | Information within broadcast message delivery request. |  |

Table 9.3.5.1-2 specifies data types re-used by the MSGG_BGDelivery API service.

Table 9.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Address | 9.1.5.2.3 | The data type of the oriAddr and destAddr. |  |
| MessageSegmentParameters | 8.2.5.2.5 | The data type of the segParams |  |
| Priority | 8.2.5.3.5 | The data type of the priority |  |

9.3.5.2	Structured data types

9.3.5.2.1	Introduction

9.3.5.2.2	Type: BgMessageDelivery

Table 9.3.5.2.2-1: Definition of type BgMessageDelivery

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| oriAddr | Address | M | 1 | The service identity of the originating MSGin5G Client or the originating Application Server. (NOTE). |  |
| destAddr | Address | O | 0..1 | The service identity of the Broadcast Service Area where the message needs to be broadcast. |  |
| appId | string | O | 0..1 | Identifies the application(s) for which the content is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. |  |
| msgId | string | M | 1 | Unique identifier of this message. |  |
| delivStReqInd | boolean | O | 0..1 | Indicates if delivery acknowledgement from the recipient is requested. Set to "true" if delivery acknowledgement from the recipient is requested. otherwise set to "false". Default value is "false". |  |
| payload | string | O | 0..1 | Payload of the message. |  |
| grpSrvId | string | O | 0..1 | Identifies Service Identifier of a Group. |  |
| msgTopic | string | O | 0..1 | Indicates the related Messaging Topic. |  |
| segInd | boolean | O | 0..1 | Indicates this message is part of a segmented message. Set to "true" if the message is part of a segmented message. otherwise set to "false". Default value is "false". |  |
| segParams | MessageSegmentParameters | C | 0..1 | The message segment parameters. This IE shall be included only if the value of the attribute segInd is set to “true”. |  |
| priority | Priority | O | 0..1 | Application priority level requested for this message. |  |
| NOTE: The addrType in Address data type shall only include AS or UE to represent the originating of message request. |  |  |  |  |  |

9.3.5.3	Simple data types and enumerations

None.

9.3.6	Error Handling

9.3.6.1	General

HTTP error handling shall be supported as specified in clause 7.7.

In addition, the requirements in the following clauses shall apply.

9.3.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the MSGG_BGDelivery API.

9.3.6.3	Application Errors

The application errors defined for the MSGG_BGDelivery API are listed in table 9.3.6.3-1.

Table 9.3.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

9.3.7	Feature negotiation

General feature negotiation procedures are defined in clause 7.8. Table 9.3.7-1 lists the supported features for MSGG_BGDelivery API.

Table 9.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

10	Security

TLS shall be used to support the security communication between the service producer (e.g., MSGin5G Server, Message Gateway) and the service consumer as specified in 3GPP TS 33.862 [19] and 3GPP TS 33.501 [20].

When CAPIF is not supported and/or not used, the access to the MSGin5G Service APIs shall be authorized by means of OAuth2 protocol (see IETF RFC 6749 [21]), based on local configuration, using the "Client Credentials" authorization grant. When OAuth2 is used, a client shall obtain a "token" from the authorization server prior to consuming services offered by any MSGin5G Service API.

When the service producer (e.g., MSGin5G Server, Message Gateway) supports CAPIF for external exposure and CAPIF is used, the provisions of clause 11.2 shall apply for authorizing the access to the MSGin5G APIs.

11	Using Common API Framework

11.1	General

When CAPIF is used with a MSGin5G service, the service producer (e.g., MSGin5G Server, Message Gateway) shall support the following as defined in 3GPP TS 29.222 [8]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [7], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a MSGin5G service, the service producer (e.g., MSGin5G Server, Message Gateway) shall register all the features for northbound APIs in the CAPIF Core Function.

11.2	Security

When CAPIF is used for managing the exposure of the MSGin5G APIs, before invoking an API exposed by an MSGin5G Enabler Layer entity (e.g., MSGin5G Server, Message Gateway), the service consumer, acting as an API Invoker, shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with the CAPIF Core Function and ensure that the MSGin5G Enabler Layer entity has enough credential to authenticate the service consumer, as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [8].

If PKI or TLS-PSK is used as the selected security method between the service consumer and the MSGin5G Enabler Layer entity, upon API invocation, the MSGin5G Enabler Layer entity shall retrieve the authorization information from the CAPIF Core Function as described in clause 5.6.2.4 of 3GPP TS 29.222 [8].

As indicated in 3GPP TS 33.122 [22], the access to the MSGin5G Enabler Layer entity APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [21]), where the CAPIF Core Function (see 3GPP TS 29.222 [8]) plays the role of the authorization server.

If OAuth2 is used as the selected security method between the service consumer and the MSGin5G Enabler Layer entity, the service consumer, prior to consuming services offered by the MSGin5G Enabler Layer entity APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [8].

The MSGin5G APIs do not define any scopes for OAuth2 authorization in the present specification. For the definition and handling of scopes for OAuth2 authorization in CAPIF, see 3GPP TS 29.222 [8].

It is the MSGin5G Enabler Layer entity's responsibility to check whether the service consumer is authorized to use an API based on the "token". Once the MSGin5G Enabler Layer entity verifies the "token", it shall check whether the MSGin5G Enabler Layer entity's identifier in the "token" matches its own published identifier, whether the API name in the "token" matches its own published API name and whether the granted scope (see 3GPP TS 29.222 [8]) in the "token" is authorized. If those checks are passed, the service consumer has full authority to access any resource(s) and/or operation(s) for the invoked service API and that are within the limits of the granted scope in the "token".

NOTE:	For the aforementioned security methods, the MSGin5G Enabler Layer entity needs to apply admission control according to access control policies after performing the authorization checks.

12	Usage of Network Capabilities

As specified in 3GPP TS 23.554 [2], MSGin5G Server may perform procedures of UE reachability monitoring and device triggering by consuming the 3GPP core network capabilities from SCEF/NEF as specified in 3GPP TS 29.522 [23], with description of the AF applies to the MSGin5G Server. With usage of network capabilities, the following procedure for UE reachability monitoring and device triggering procedure could be supported. Upon the MSGin5G Server receiving a request to send MSGin5G message to a MSGin5G UE, the MSGin5G Server may determine whether the recipient MSGin5G Client is not reachable:

-	by using the recipient's information received when performing the procedures specified in clause 8.9.2.2 of 3GPP TS 23.554 [2]; or

-	by using the recipient's availability information provided by MSGin5G Client at registration as specified in clause 8.2.1 of 3GPP TS 23.554 [2].

If the recipient MSGin5G Client is not reachable, the MSGin5G Server may send a request for device triggering to SCEF/NEF as specified in clause 4.4.3 of 3GPP TS 29.522 [23]. The request uses the information provided by the MSGin5G Client at registration in the MSGin5G Client Triggering Information IE. And the MSGin5G Server may use the MSGin5G Client communication availability and/or pre-configured information to determine the timing of the device triggering request, e.g. the trigger request may be sent to ensure that the target UE is reachable prior to resuming MSGin5G communications.

As specified in clause 4.4.6 of 3GPP TS 29.122 [24], upon the MSGin5G Server receiving an HTTP POST request from SCEF/NEF indicating the result of the triggering delivery, the MSGin5G Server shall respond with an HTTP 200 OK or 204 No Content response.

Annex A (normative):
OpenAPI specification

A.1	General

This Annex is based on the OpenAPI Specification [6] and provides corresponding representations of all APIs defined in the present specification.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [18] and clause 5.3.1 of the 3GPP TS 29.501 [9] for further information).

A.2	MSGS_ASRegistration API

openapi: 3.0.0

info:

title: MSGS_ASRegistration

version: 1.2.0

description: |

API for MSGS AS Registration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgs-asregistration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/registrations:

post:

summary: Registers a new AS at a MSGin5G Server

tags:

- AS registration

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistration'

responses:

'201':

description: AS information is registered successfully at MSGin5G Server

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistrationAck'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

delete:

summary: Delete an existing AS registration at MSGin5G Server

tags:

- AS DeRegistration

parameters:

- name: registrationId

in: path

description: AS registration Id

required: true

schema:

type: string

responses:

'200':

description: The individual AS registration is deleted successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ASRegistrationAck'

'204':

description: >

No Content. The individual AS registration resource is deleted successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ASRegistration:

description: AS registration data

type: object

required:

- asSvcId

properties:

asSvcId:

type: string

appId:

type: string

targetUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reqExprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

exGroupList:

type: array

items:

type: string

minItems: 1

description: The group list which the AS will control.

asProf:

$ref: '#/components/schemas/ASProfile'

ASRegistrationAck:

description: AS registration response data

type: object

required:

- asSvcId

- result

properties:

asSvcId:

type: string

result:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ProblemDetails'

regExprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ASProfile:

description: AS profile information

type: object

properties:

appName:

type: string

appProviders:

type: array

items:

type: string

minItems: 1

description: The provider of the AS.

appSenarios:

type: array

items:

type: string

minItems: 1

description: The application scenario.

appCategory:

type: string

asStatus:

type: string

A.3	MSGS_MSGDelivery API

openapi: 3.0.0

info:

title: MSGS_MSGDelivery

version: 1.2.0

description: |

API for MSGG MSGin5G Server Message Delivery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgs-msgdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/deliver-as-message:

post:

summary: AS deliver message to MSGin5G Server

tags:

- AS Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ASMessageDelivery'

responses:

'200':

description: OK, AS Message delivery successful

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/deliver-ue-message:

post:

summary: UE deliver message to MSGin5G Server

tags:

- UE Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UEMessageDelivery'

responses:

'200':

description: OK, UE Message delivery successful

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: AS or UE deliver status report to MSGin5G Server

tags:

- AS/UE status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DeliveryStatusReport'

responses:

'200':

description: OK, status report delivery successfully

content:

application/json:

schema:

$ref: '#/components/schemas/MessageDeliveryAck'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ASMessageDelivery:

description: Contains the AS message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

- stoAndFwInd

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

priority:

$ref: '#/components/schemas/Priority'

segInd:

type: boolean

segParams:

$ref: '#/components/schemas/MessageSegmentParameters'

stoAndFwInd:

type: boolean

stoAndFwParams:

$ref: '#/components/schemas/StoreAndForwardParameters'

latency:

type: integer

notiUrl:

type: string

UEMessageDelivery:

description: Contains the UE message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

- stoAndFwInd

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: '#/components/schemas/MessageSegmentParameters'

stoAndFwInd:

type: boolean

stoAndFwParams:

$ref: '#/components/schemas/StoreAndForwardParameters'

MessageDeliveryAck:

description: Contains the message delivery ack data

type: object

required:

- oriAddr

- msgId

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgId:

type: string

status:

$ref: '#/components/schemas/DeliveryStatus'

failureCause:

type: string

MessageSegmentParameters:

description: Contains the message segment parameters data

type: object

properties:

segId:

type: string

totalSegCount:

type: integer

segNumb:

type: integer

lastSegFlag:

type: boolean

StoreAndForwardParameters:

description: Contains the store and forward parameters data

type: object

properties:

exprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

DeliveryStatusReport:

description: Contains the delivery status report data

type: object

required:

- oriAddr

- destAddr

- msgId

- delivSt

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgId:

type: string

failureCause:

type: string

delivSt:

$ref: '#/components/schemas/ReportDeliveryStatus'

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

DeliveryStatus:

anyOf:

- type: string

enum:

- DELY_FAILED

- DELY_STORED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates if delivery is a failure, or if the message is stored for deferred delivery.

Possible values are:

- DELY_FAILED: Indicates that the message delivery is failed.

- DELY_STORED: Indicates that the message is stored for deferred delivery.

ReportDeliveryStatus:

anyOf:

- type: string

enum:

- REPT_DELY_SUCCESS

- REPT_DELY_FAILED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

The delivery status description, including success or failure in delivery.

Possible values are:

- REPT_DELY_SUCCESS: Indicates that the report delivery is successful.

- REPT_DELY_FAILED: Indicates that the report delivery is failed.

Priority:

anyOf:

- type: string

enum:

- HIGH

- MIDDLE

- LOW

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Application priority level requested for this message.

Possible values are:

- HIGH: Indicates the messages should be sent in high priority.

- MIDDLE: Indicates the messages should be sent in middle priority.

- LOW: Indicates the messages should be sent in low priority.

A.4	MSGG_L3GDelivery API

openapi: 3.0.0

info:

title: MSGG_L3GDelivery

version: 1.2.0

description: |

API for MSGG L3G Message Delivery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgg-l3gdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/deliver-message:

post:

summary: deliver message to Legacy 3GPP Message Gateway

tags:

- L3G Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/L3gMessageDelivery'

responses:

'204':

description: No Content, Message delivery successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: deliver status report to Legacy 3GPP Message Gateway

tags:

- L3G status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/DeliveryStatusReport'

responses:

'204':

description: No Content, status report delivery successfully

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

L3gMessageDelivery:

description: Contains the L3G message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

properties:

oriAddr:

$ref: '#/components/schemas/Address'

destAddr:

$ref: '#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/MessageSegmentParameters'

Address:

description: Contains the Message type data

type: object

required:

- addrType

- addr

properties:

addrType:

$ref: '#/components/schemas/AddressType'

addr:

type: string

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AddressType:

anyOf:

- type: string

enum:

- UE

- AS

- GROUP

- BC

- TOPIC

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represent the type of message request.

Possible values are:

- UE: The address type is UE.

- AS: The address type is AS.

- GROUP: The address type is GROUP.

- BC: The address type is BC.

- TOPIC: The address type is TOPIC.

A.5	MSGG_N3GDelivery API

openapi: 3.0.0

info:

title: MSGG_N3GDelivery

version: 1.2.0

description: |

API for MSGG N3G Message Delivery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgg-n3gdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/deliver-message:

post:

summary: deliver message to NON-3GPP Message Gateway

tags:

- N3G Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/N3gMessageDelivery'

responses:

'204':

description: No Content,Message delivery successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/deliver-report:

post:

summary: deliver status report to NON-3GPP Message Gateway

tags:

- N3G status report delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/DeliveryStatusReport'

responses:

'204':

description: No Content, status report delivery successfully

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

N3gMessageDelivery:

description: N3G message delivery data

type: object

required:

- oriAddr

- destAddr

- msgId

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

payload:

type: string

segInd:

type: boolean

segParams:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/MessageSegmentParameters'

# A.6	MSGG_BGDelivery API

openapi: 3.0.0

info:

title: MSGG_BGDelivery

version: 1.1.0

description: |

API for Broadcast Message Delivery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgg-bgdelivery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/deliver-message:

post:

summary: deliver message to Broadcast Message Gateway

tags:

- Broadcast Message delivery

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BgMessageDelivery'

responses:

'204':

description: No Content, Message delivery successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

BgMessageDelivery:

description: Broadcast message delivery data

type: object

required:

- oriAddr

- msgId

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

appId:

type: string

msgId:

type: string

delivStReqInd:

type: boolean

grpSrvId:

type: string

msgTopic:

type: string

payload:

type: string

segInd:

type: boolean

segParams:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/MessageSegmentParameters'

priority:

$ref: 'TS29538_MSGS_MSGDelivery.yaml#/components/schemas/Priority'

# A.7	MSGS_TopiclistEvent API

openapi: 3.0.0

info:

title: MSGS_TopiclistEvent

version: 1.1.0

description: |

API for Topic Messaging Event Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.538 V19.4.0; Enabling MSGin5G Service; Application Programming Interfaces (API)

specification; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.538/

servers:

- url: '{apiRoot}/msgs-topiclistevent/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials:

- msgs-topiclistevent

paths:

/topiclist-subscriptions:

post:

summary: subscribe to Messaging Topic list on a MSGin5G Server

tags:

- Topic List Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TopicListSubscription'

responses:

'201':

description: Topic list on the MSGin5G Server is successfully subscribed

content:

application/json:

schema:

$ref: '#/components/schemas/TopicListSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

MessagingTopicListNotification:

'{$request.body#/notificationURI}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TopicListNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/topiclist-subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Topic List Subscription resource.

required: true

schema:

type: string

delete:

summary: Request the deletion of an existing Individual Topic List Subscription resource.

operationId: DeleteIndTopicListSubsc

tags:

- Individual Topic List Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Topic List Subscription resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-topic-subscription:

post:

summary: subscribe to Messaging Topic on a MSGin5G Server

tags:

- Messaging Topic Subscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TopicSubscription'

responses:

'200':

description: The Messaging Topic subscription is created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/TopicSubscriptionAck'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-topic-unsubscription:

post:

summary: unsubscribe to Messaging Topic on a MSGin5G Server

tags:

- Messaging Topic Unsubscription

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TopicUnsubscription'

responses:

'204':

description: >

No Content. The Messaging Topic subscription is removed successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

TopicListSubscription:

description: Messaging Topic list subscription data

type: object

required:

- oriAddr

- destAddr

- notificationURI

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

notificationURI:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

exprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

TopicListUnsubscription:

description: Messaging Topic list unsubscription data

type: object

required:

- oriAddr

- destAddr

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

destAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

TopicListUnsubscriptionAck:

description: Response of Messaging Topic list unsubscription

type: object

required:

- subStat

properties:

subStat:

type: string

TopicSubscription:

description: Messaging Topic subscription data

type: object

required:

- oriAddr

- msgTopics

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgTopics:

type: array

items:

type: string

minItems: 1

description: A list of Messaging Topics to be subscribed.

exprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

TopicSubscriptionAck:

description: Response of Messaging Topic subscription

type: object

required:

- subStat

properties:

subStat:

type: string

exprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

TopicUnsubscription:

description: Messaging Topic unsubscription data

type: object

required:

- oriAddr

- msgTopics

properties:

oriAddr:

$ref: 'TS29538_MSGG_L3GDelivery.yaml#/components/schemas/Address'

msgTopics:

type: array

items:

type: string

minItems: 1

description: A list of Messaging Topics to be unsubscribed.

TopicListNotification:

description: Messaging Topic list changes notification data

type: object

required:

- msgTopics

properties:

exprTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

msgTopics:

type: array

items:

$ref: '#/components/schemas/MessagingTopic'

minItems: 1

description: list of Messaging Topics for notification.

MessagingTopic:

description: Represent Messaging Topic information

type: object

required:

- msgTopic

- updateStat

properties:

msgTopic:

type: string

updateStat:

$ref: '#/components/schemas/UpdateStatus'

#

# ENUMERATIONS DATA TYPES

#

UpdateStatus:

anyOf:

- type: string

enum:

- CREATED

- DELETED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the Messaging Topic Update status.

Possible values are:

- CREATED: Indicates Topic newly created.

- DELETED: Indicates Topic newly deleted.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-10 | CT3#118-e | C3-215030 |  |  |  | Draft skeleton provided by the rapporteur | 0.0.0 |
| 2021-10 | CT3#118-e | C3-215479 |  |  |  | Inclusion of documents agreed in CT3#118e C3-215116, C3-215165, C3-215206, C3-215432, C3-215441, C3-215462. Editorial change from the rapporteur. Correction from the rapporteur. | 0.1.0 |
| 2021-11 | CT3#119-e | C3-216552 |  |  |  | Inclusion of documents agreed in CT3#119e C3-216175, C3-216525, C3-216528, C3-216529, C3-216545, C3-216546, C3-216581. Correction from the rapporteur. | 0.2.0 |
| 2022-01 | CT3#119bis-e | C3-220458 |  |  |  | Inclusion of documents agreed in CT3#119bis-e C3-220384, C3-220385, C3-220386, C3-220387, C3-220389, C3-220391, C3-220410, C3-220411, C3-220412 and C3-220413. Correction from the rapporteur. | 0.3.0 |
| 2022-02 | CT3#120e | C3-221559 |  |  |  | Inclusion of documents agreed in CT3#120e C3-221245, C3-221246, C3-221247, C3-221248, C3-221249, C3-221529, C3-221530, C3-221531, C3-221532, C3-221533, C3-222534, C3-221535, C3-221536, C3-221537, C3-221538, C3-221541, C3-221542, C3-221543, C3-221544, C3-221545, C3-221546, C3-221547, C3-221548 and C3-221549. Correction from the rapporteur. | 0.4.0 |
| 2022-03 | CT#95e | CP-220163 |  |  |  | Presentation to TSG CT for approval | 1.0.0 |
| 2022-03 | CT#95e | CP-220163 |  |  |  | Approved by TSG CT | 17.0.0 |
| 2022-06 | CT#96 | CP-221118 | 0001 | 1 | B | Add Usage of Network Capabilities in MSGin5G Server | 17.1.0 |
| 2022-06 | CT#96 | CP-221118 | 0002 | 1 | F | Update of abbreviations and terms | 17.1.0 |
| 2022-06 | CT#96 | CP-221118 | 0003 | 1 | F | Update the Presence condition of appId in Table 8.1.5.2.2-1 | 17.1.0 |
| 2022-06 | CT#96 | CP-221151 | 0004 |  | F | Update of info and externalDocs fields | 17.1.0 |
| 2022-09 | CT#97e | CP-222092 | 0005 | 2 | F | Update the Presence condition of Store and forward flag | 17.2.0 |
| 2022-12 | CT#98-e | CP-223199 | 0007 | 1 | F | MSGS_ASRegistration_Deregister operation related corrections in the description clause | 18.0.0 |
| 2022-12 | CT#98-e | CP-223199 | 0008 | 1 | F | MSGS_ASRegistration_Request operation related corrections in the description clause | 18.0.0 |
| 2022-12 | CT#98-e | CP-223185 | 0009 |  | F | "Error handling" clause: alignment with other NBI and 5GS APIs | 18.0.0 |
| 2022-12 | CT#98-e | CP-223200 | 0010 | 1 | F | Rewording some description of data structure in clasue 5.3.2 | 18.0.0 |
| 2022-12 | CT#98-e | CP-223200 | 0011 | 1 | F | Change the underline to hyphen of the apiname in clause 5.1 | 18.0.0 |
| 2022-12 | CT#98-e | CP-223189 | 0012 |  | F | Update of info and externalDocs fields | 18.0.0 |
| 2023-03 | CT#99 | CP-230156 | 0013 | 1 | F | Correction of the description fields in enumerations | 18.1.0 |
| 2023-03 | CT#99 | CP-230161 | 0014 |  | F | Update of info and externalDocs fields | 18.1.0 |
| 2023-06 | CT#100 | CP-231150 | 0015 | 1 | B | Update of Scope for Broadcast Messaging | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0016 | 1 | D | Editorial Fixes of Words and Numbers | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0017 | 1 | B | Update the terms and overview | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0018 | 1 | B | MSGG_BGDelivery Service introduction | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0019 | 1 | B | MSGG_BGDelivery service description and operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0020 | 1 | B | MSGG_BGDelivery API | 18.2.0 |
| 2023-06 | CT#100 | CP-231337 | 0021 | 1 | B | New OpenAPI file for MSGG_BGDelivery API | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0022 | 1 | B | Update Scope for Topic Messaging | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0023 | 1 | B | Add service of Topic Messaging | 18.2.0 |
| 2023-06 | CT#100 | CP-231150 | 0024 | 1 | D | Reference Clause Number Correction | 18.2.0 |
| 2023-08 | CT#101 | CP-232111 | 0026 | 1 | F | Remove the IE of security credentials in message delivery procedures | 18.3.0 |
| 2023-08 | CT#101 | CP-232111 | 0027 |  | F | Corrections on MSGS_BGDelivery API | 18.3.0 |
| 2023-08 | CT#101 | CP-232111 | 0028 | 1 | F | Update the service operations of message delivery status report | 18.3.0 |
| 2023-08 | CT#101 | CP-232085 | 0029 |  | F | Update of info and externalDocs fields | 18.3.0 |
| 2023-12 | CT#102 | CP-233251 | 0030 | 1 | F | Correction on the openAPI files | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0032 | 1 | F | Corrections to BgMessageDelivery data type | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0033 | 1 | B | Introduction update for Messaging Topic Events | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0034 | 1 | B | Service Description of Messaging Topic Events | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0035 | 1 | B | Service Operation for Messaging Topic List Subscription | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0036 | 1 | B | Service Operation for Messaging Topic Subscription | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0037 | 2 | B | Messaging Topic Events API | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0039 | 2 | F | Correction on Terms and Abbreviations | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0040 | 1 | F | Corrections to BgMessageDelivery data type | 18.4.0 |
| 2023-12 | CT#102 | CP-233251 | 0041 |  | F | Correction on Messaging Topic Events API | 18.4.0 |
| 2023-12 | CT#102 | CP-233229 | 0042 | 1 | F | IETF HTTP RFCs obsoleted by RFCs 9110, 9111, 9112 and 9113 | 18.4.0 |
| 2023-12 | CT#102 | CP-233237 | 0044 |  | F | Update of info and externalDocs fields | 18.4.0 |
| 2024-03 | CT#103 | CP-240196 | 0045 | 1 | F | Corrections in MSGG_BGDelivery API | 18.5.0 |
| 2024-03 | CT#103 | CP-240196 | 0046 |  | F | Updates API provided by MSGin5G Server | 18.5.0 |
| 2024-03 | CT#103 | CP-240196 | 0047 | 1 | B | OpenAPI and Corrections for TopiclistEvent | 18.5.0 |
| 2024-03 | CT#103 | CP-240166 | 0048 |  | F | Update of info and externalDocs fields | 18.5.0 |
| 2024-06 | CT#104 | CP-241123 | 0049 | 1 | F | Callback correction to MSGS_TopicListEvent API | 18.6.0 |
| 2024-06 | CT#104 | CP-241123 | 0050 | 1 | F | Editorial fixes on service and API descriptions | 18.6.0 |
| 2024-06 | CT#104 | CP-241123 | 0051 |  | F | EN resolutions within MSGG_BGDelivery API | 18.6.0 |
| 2024-06 | CT#104 | CP-241251 | 0052 | 2 | F | Correct references and clarify regarding NBI usage | 18.6.0 |
| 2024-06 | CT#104 | CP-241086 | 0053 |  | F | Update of info and externalDocs fields | 18.6.0 |
| 2024-07 | CT#104 |  |  |  |  | Correction to fix OpenAPI parsing errors | 18.6.1 |
| 2024-09 | CT#105 | CP-242152 | 0055 |  | F | Corrections to add the missing resource representation in resource creation response | 18.7.0 |
| 2024-09 | CT#105 | CP-242152 | 0056 |  | F | Corrections to the resource deletion definition | 18.7.0 |
| 2024-09 | CT#105 | CP-242120 | 0057 |  | F | Update of info and externalDocs fields | 18.7.0 |
| 2024-09 | CT#105 | CP-242113 | 0054 | 1 | F | Correct presence field for some of the attributes | 19.0.0 |
| 2024-12 | CT#106 | CP-243085 | 0059 |  | F | Introduce the support of the 3xx redirection mechanism where missing | 19.1.0 |
| 2024-12 | CT#106 | CP-243147 | 0061 |  | F | Update of info and externalDocs fields | 19.1.0 |
| 2025-01 | CT#106 | - | - | - | - | Corrects YAML file of MSGS_ASRegistration API | 19.1.1 |
| 2025-02 | CT#107 | CP-250109 | 0062 | 1 | F | Updates to ASRegistration Service | 19.2.0 |
| 2025-02 | CT#107 | CP-250109 | 0063 |  | F | Updates to MSGDelivery Service | 19.2.0 |
| 2025-02 | CT#107 | CP-250120 | 0065 |  | A | Necessary corrections to the MSGS_TopiclistEvent API | 19.2.0 |
| 2025-02 | CT#107 | CP-250109 | 0066 | 1 | F | Description updates on the redirection mechanism | 19.2.0 |
| 2025-02 | CT#107 | CP-250129 | 0068 |  | F | Update of info and externalDocs fields | 19.2.0 |
| 2025-06 | CT#108 | CP-251131 | 0069 | 1 | B | Update on registration for controlling AS | 19.3.0 |
| 2025-12 | CT#110 | CP-253036 | 0070 | 1 | F | TS reference correction | 19.4.0 |
| 2025-12 | CT#110 | CP-253026 | 0071 | 1 | B | Updates and corrections to the CAPIF related Security clause | 19.4.0 |
| 2025-12 | CT#110 | CP-253064 | 0072 |  | F | Update of info and externalDocs fields | 19.4.0 |