| 3GPP TS 29.571 V19.4.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Common Data Types for Service Based Interfaces; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	12

2	References	12

3	Definitions and abbreviations	15

3.1	Definitions	15

3.2	Abbreviations	15

4	Overview	15

5	Common Data Types	16

5.1	Introduction	16

5.2	Data Types for Generic Usage	16

5.2.1	Introduction	16

5.2.1A	Re-used Data Types	16

5.2.2	Simple Data Types	16

5.2.3	Enumerations	21

5.2.3.1	Enumeration: PatchOperation	21

5.2.3.2	Enumeration: UriScheme	21

5.2.3.3	Enumeration: ChangeType	22

5.2.3.4	Enumeration: HttpMethod	25

5.2.3.5	Enumeration: NullValue	25

5.2.3.6	Enumeration: MatchingOperator	25

5.2.3.7	Enumeration: UpfPacketInspectionFunctionality	26

5.2.4	Structured Data Types	27

5.2.4.1	Type: ProblemDetails	27

5.2.4.2	Type: Link	29

5.2.4.3	Type PatchItem	29

5.2.4.4	Type: LinksValueSchema	29

5.2.4.5	Type: SelfLink	29

5.2.4.6	Type: InvalidParam	30

5.2.4.7	Type: LinkRm	30

5.2.4.8	Type ChangeItem	31

5.2.4.9	Type NotifyItem	31

5.2.4.10	Type: ComplexQuery	32

5.2.4.11	Type: Cnf	32

5.2.4.12	Type: Dnf	32

5.2.4.13	Type: CnfUnit	32

5.2.4.14	Type: DnfUnit	32

5.2.4.15	Type: Atom	33

5.2.4.16	Void	33

5.2.4.17	Type: PatchResult	33

5.2.4.18	Type: ReportItem	33

5.2.4.19	Type: HalTemplate	34

5.2.4.20	Type: Property	34

5.2.4.21	Type: RedirectResponse	34

5.2.4.22	Type: TunnelAddress	35

5.2.4.23	Type: FqdnPatternMatchingRule	35

5.2.4.24	Type: StringMatchingRule	35

5.2.4.25	Type: StringMatchingCondition	36

5.2.4.26	Type: Ipv4AddressRange	36

5.2.4.27	Type: Ipv6AddressRange	36

5.2.4.28	Type: Ipv6PrefixRange	36

5.2.4.29	Type NfSignallingInfo	37

5.2.4.30	Type NfSignallingInfoPerTimeWindow	37

5.2.4.31	Type NfSignallingInfoPerService	38

5.2.4.32	Type NfHeartbeatInfo	38

5.3	Data Types related to Subscription, Identification and Numbering	38

5.3.1	Introduction	38

5.3.2	Simple Data Types	38

5.3.3	Enumerations	42

5.3.3.1	Enumeration: GroupServiceId	42

5.3.4	Structured Data Types	43

5.3.4.1	Type: Guami	43

5.3.4.2	Type: NetworkId	43

5.3.4.3	Type: GuamiRm	43

5.4	Data Types related to 5G Network	43

5.4.1	Introduction	43

5.4.2	Simple Data Types	43

5.4.3	Enumerations	51

5.4.3.1	Enumeration: AccessType	51

5.4.3.2	Enumeration: RatType	52

5.4.3.3	Enumeration: PduSessionType	52

5.4.3.4	Enumeration: UpIntegrity	53

5.4.3.5	Enumeration: UpConfidentiality	53

5.4.3.6	Enumeration: SscMode	53

5.4.3.7	Enumeration: DnaiChangeType	53

5.4.3.8	Enumeration: RestrictionType	54

5.4.3.9	Enumeration: CoreNetworkType	54

5.4.3.10	Enumeration: AccessTypeRm	54

5.4.3.11	Enumeration: RatTypeRm	54

5.4.3.12	Enumeration: PduSessionTypeRm	54

5.4.3.13	Enumeration: UpIntegrityRm	54

5.4.3.14	Enumeration: UpConfidentialityRm	54

5.4.3.15	Enumeration: SscModeRm	54

5.4.3.17	Enumeration: DnaiChangeTypeRm	55

5.4.3.18	Enumeration: RestrictionTypeRm	55

5.4.3.19	Enumeration: CoreNetworkTypeRm	55

5.4.3.20	Enumeration: PresenceState	55

5.4.3.21	Enumeration: StationaryIndication	55

5.4.3.22	Enumeration: StationaryIndicationRm	55

5.4.3.23	Enumeration: ScheduledCommunicationType	55

5.4.3.24	Enumeration: ScheduledCommunicationTypeRm	55

5.4.3.25	Enumeration: TrafficProfile	56

5.4.3.26	Enumeration: TrafficProfileRm	56

5.4.3.27	Enumeration: LcsServiceAuth	56

5.4.3.28	Enumeration: UeAuth	56

5.4.3.29	Enumeration: DlDataDeliveryStatus	57

5.4.3.30	Enumeration: DlDataDeliveryStatusRm	57

5.4.3.31	Void	57

5.4.3.32	Enumeration: AuthStatus	57

5.4.3.33	Enumeration: LineType	57

5.4.3.34	Enumeration: LineTypeRm	57

5.4.3.35	Void	58

5.4.3.36	Void	58

5.4.3.37	Enumeration: NotificationFlag	58

5.4.3.38	Enumeration: TransportProtocol	58

5.4.3.39	Enumeration: SatelliteBackhaulCategory	58

5.4.3.40	Enumeration: SatelliteBackhaulCategoryRm	58

5.4.3.41	Enumeration: BufferedNotificationsAction	59

5.4.3.42	Enumeration: SubscriptionAction	59

5.4.3.43	Enumeration: SnssaiStatus	59

5.4.3.44	Enumeration: TerminationIndication	59

5.4.3.45	Enumeration: DelayMeasurementProtocol	60

5.4.3.46	Enumeration: CagProvisionOperation	60

5.4.4	Structured Data Types	60

5.4.4.1	Type: SubscribedDefaultQos	60

5.4.4.2	Type: Snssai	61

5.4.4.3	Type: PlmnId	61

5.4.4.4	Type: Tai	62

5.4.4.5	Type: Ecgi	62

5.4.4.6	Type: Ncgi	62

5.4.4.7	Type: UserLocation	63

5.4.4.8	Type: EutraLocation	64

5.4.4.9	Type: NrLocation	65

5.4.4.10	Type: N3gaLocation	66

5.4.4.11	Type: UpSecurity	68

5.4.4.12	Type: NgApCause	68

5.4.4.13	Type: BackupAmfInfo	69

5.4.4.14	Type: RefToBinaryData	69

5.4.4.15	Type RouteToLocation	69

5.4.4.16	Type RouteInformation	69

5.4.4.17	Type: Area	70

5.4.4.18	Type: ServiceAreaRestriction	70

5.4.4.19	Type: PlmnIdRm	70

5.4.4.20	Type: TaiRm	70

5.4.4.21	Type: EcgiRm	70

5.4.4.22	Type: NcgiRm	70

5.4.4.23	Type: EutraLocationRm	70

5.4.4.24	Type: NrLocationRm	71

5.4.4.25	Type: UpSecurityRm	71

5.4.4.26	Type: RefToBinaryDataRm	71

5.4.4.27	Type: PresenceInfo	72

5.4.4.28	Type: GlobalRanNodeId	73

5.4.4.29	Type: GNbId	74

5.4.4.30	Type: PresenceInfoRm	74

5.4.4.31	Void	74

5.4.4.32	Type: AtsssCapability	75

5.4.4.33	Type: PlmnIdNid	75

5.4.4.34	Type: PlmnIdNidRm	76

5.4.4.35	Type: SmallDataRateStatus	76

5.4.4.36	Type: HfcNodeId	76

5.4.4.37	Type: HfcNodeIdRm	76

5.4.4.38	Type: WirelineArea	77

5.4.4.39	Type: WirelineServiceAreaRestriction	77

5.4.4.40	Type: ApnRateStatus	78

5.4.4.41	Type: ScheduledCommunicationTime	78

5.4.4.42	Type: ScheduledCommunicationTimeRm	78

5.4.4.43	Type: BatteryIndication	79

5.4.4.44	Type: BatteryIndicationRm	79

5.4.4.45	Type: AcsInfo	79

5.4.4.46	Type: AcsInfoRm	79

5.4.4.47	Type: NrV2xAuth	79

5.4.4.48	Type: LteV2xAuth	80

5.4.4.49	Type: Pc5QoSPara	80

5.4.4.50	Type: Pc5QosFlowItem	80

5.4.4.51	Type: Pc5FlowBitRates	80

5.4.4.52	Type: UtraLocation	81

5.4.4.53	Type: GeraLocation	82

5.4.4.54	Type: CellGlobalId	82

5.4.4.55	Type: ServiceAreaId	83

5.4.4.56	Type: LocationAreaId	83

5.4.4.57	Type: RoutingAreaId	83

5.4.4.58	Type: DddTrafficDescriptor	83

5.4.4.59	Type: MoExpDataCounter	83

5.4.4.60	Type: NssaaStatus	84

5.4.4.61	Type: NssaaStatusRm	84

5.4.4.62	Type: TnapId	84

5.4.4.63	Type: TnapIdRm	84

5.4.4.64	Type: TwapId	85

5.4.4.65	Type: TwapIdRm	85

5.4.4.66	Type: SnssaiExtension	85

5.4.4.67	Type: SdRange	85

5.4.4.68	Type: ProseServiceAuth	87

5.4.4.69	Type: EcsServerAddr	89

5.4.4.70	Type: EcsServerAddrRm	89

5.4.4.71	Type: IpAddr	89

5.4.4.72	Type: SACInfo	90

5.4.4.73	Type: SACEventStatus	93

5.4.4.74	Type: SpatialValidityCond	93

5.4.4.75	Type: SpatialValidityCondRm	93

5.4.4.76	Type: ServerAddressingInfo	93

5.4.4.77	Type PcfUeCallbackInfo	94

5.4.4.78	Type PduSessionInfo	94

5.4.4.79	Type EasIpReplacementInfo	94

5.4.4.80	Type EasServerAddress	94

5.4.4.81	Type RoamingRestrictions	95

5.4.4.82	Type: GeoServiceArea	95

5.4.4.83	Type: MutingExceptionInstructions	95

5.4.4.84	Type: MutingNotificationsSettings	95

5.4.4.85	Type: VplmnOffloadingInfo	96

5.4.4.86	Type: PartiallyAllowedSnssai	96

5.4.4.87	Type: VarRepPeriod	97

5.4.4.88	Type: RangingSlPosAuth	97

5.4.4.89	Type: NrA2xAuth	97

5.4.4.90	Type: LteA2xAuth	97

5.4.4.91	Type: SliceUsageControlInfo	98

5.4.4.92	Type: CombGciAndHfcNIds	98

5.4.4.93	Type: SnssaiDnnItem	98

5.4.4.94	Type: NtnTaiInfo	98

5.4.4.95	Type: MitigationInfo	99

5.4.4.96	Type: VplmnDlAmbr	99

5.4.4.97	Type: LocalOffloadingManagementInfo	100

5.4.4.98	Type: CagProvisionInformation	101

5.4.5	Data types describing alternative data types or combinations of data types	101

5.4.5.1	Type: ExtSnssai	101

5.4.5.2	Type: SnssaiReplaceInfo	102

5.5	Data Types related to 5G QoS	102

5.5.1	Introduction	102

5.5.2	Simple Data Types	102

5.5.3	Enumerations	106

5.5.3.1	Enumeration: PreemptionCapability	106

5.5.3.2	Enumeration: PreemptionVulnerability	106

5.5.3.3	Enumeration: ReflectiveQosAttribute	106

5.5.3.4	Void	106

5.5.3.5	Enumeration: NotificationControl	106

5.5.3.6	Enumeration: QosResourceType	107

5.5.3.7	Enumeration: PreemptionCapabilityRm	107

5.5.3.8	Enumeration: PreemptionVulnerabilityRm	107

5.5.3.9	Enumeration: ReflectiveQosAttributeRm	107

5.5.3.10	Enumeration: NotificationControlRm	107

5.5.3.11	Enumeration: QosResourceTypeRm	107

5.5.3.12	Enumeration: AdditionalQosFlowInfo	107

5.5.3.13	Enumeration: PartitioningCriteria	108

5.5.3.14	Enumeration: PartitioningCriteriaRm	108

5.5.3.15	Enumeration: PduSetHandlingInfo	108

5.5.3.16	Enumeration: MediaTransportProto	108

5.5.3.17	Enumeration: RtpHeaderExtType	108

5.5.3.18	Enumeration: RtpPayloadFormat	109

5.5.3.19	Enumeration: MediaTransportProtoRm	109

5.5.3.20	Enumeration: RtpHeaderExtTypeRm	109

5.5.3.21	Enumeration: RtpPayloadFormatRm	109

5.5.3.22	Enumeration: PduSetHandlingInfoRm	109

5.5.3.23	Enumeration: MriTransferMethod	109

5.5.3.24	Enumeration: MriTransferMethodRm	109

5.5.4	Structured Data Types	110

5.5.4.1	Type: Arp	110

5.5.4.2	Type: Ambr	110

5.5.4.3	Type: Dynamic5Qi	111

5.5.4.4	Type: NonDynamic5Qi	112

5.5.4.5	Type: ArpRm	112

5.5.4.6	Type: AmbrRm	112

5.5.4.7	Void	112

5.5.4.8	Void	113

5.5.4.9	Type: SliceMbr	113

5.5.4.10	Type: SliceMbrRm	113

5.5.4.11	Type: PduSetQosPara	113

5.5.4.12	Type: PduSetQosParaRm	113

5.5.4.13	Type ProtocolDescription	114

5.5.4.13A	Type ProtocolDescriptionRm	115

5.5.4.14	Type RtpHeaderExtInfo	117

5.5.4.14A	Type RtpHeaderExtInfoRm	119

5.5.4.15	Type RtpPayloadInfo	121

5.5.4.16	Type RtpPayloadInfoRm	121

5.5.4.17	Type MriTransferInfo	122

5.5.4.18	Type MriTransferInfoRm	122

5.6	Data Types related to 5G Trace	122

5.6.1	Introduction	122

5.6.2	Simple Data Types	122

5.6.3	Enumerations	123

5.6.3.1	Enumeration: TraceDepth	123

5.6.3.2	Enumeration: TraceDepthRm	123

5.6.3.3	Enumeration: JobType	123

5.6.3.4	Enumeration: ReportTypeMdt	124

5.6.3.5	Enumeration: MeasurementLteForMdt	124

5.6.3.6	Enumeration: MeasurementNrForMdt	124

5.6.3.7	Enumeration: SensorMeasurement	125

5.6.3.8	Enumeration: ReportingTrigger	125

5.6.3.9	Enumeration: ReportIntervalMdt	125

5.6.3.10	Enumeration: ReportAmountMdt	126

5.6.3.11	Enumeration: EventForMdt	126

5.6.3.12	Enumeration: LoggingIntervalMdt	126

5.6.3.13	Enumeration: LoggingDurationMdt	127

5.6.3.14	Enumeration: PositioningMethodMdt	127

5.6.3.15	Enumeration: CollectionPeriodRmmLteMdt	127

5.6.3.16	Enumeration: MeasurementPeriodLteMdt	128

5.6.3.17	Enumeration: ReportIntervalNrMdt	128

5.6.3.18	Enumeration: LoggingIntervalNrMdt	128

5.6.3.19	Enumeration: CollectionPeriodRmmNrMdt	129

5.6.3.20	Enumeration: LoggingDurationNrMdt	129

5.6.3.21	Enumeration: QoeServiceType	129

5.6.3.22	Enumeration: AvailableRanVisibleQoeMetric	130

5.6.3.23	Enumeration: MeasurementType	130

5.6.3.24	Enumeration: BluetoothRssi	130

5.6.3.25	Enumeration: WlanRssi	130

5.6.3.26	Enumeration: WlanRtt	130

5.6.4	Structured Data Types	132

5.6.4.1	Type: TraceData	132

5.6.4.2	Type: MdtConfiguration	135

5.6.4.3	Type: AreaScope	140

5.6.4.4	Type: TacInfo	140

5.6.4.5	Type: MbsfnArea	140

5.6.4.6	Type: InterFreqTargetInfo	141

5.6.4.7	Type: QmcConfigInfo	142

5.6.4.8	Type: QmcAreaScope	143

5.6.4.9	Type: QoeTarget	143

5.6.4.10	Type: CagInfo	143

5.6.4.11	Type: NidInfo	143

5.6.4.12	Type: UeLevelMeasurementsConfiguration	144

5.6.4.13	Type: CellIdNidInfo	144

5.6.4.14	Type: CellIdNid	144

5.6.4.15	Type: TacNidInfo	144

5.6.4.16	Type: TacNid	144

5.6.4.17	Type: BluetoothMeasurement	145

5.6.4.18	Type: WlanMeasurement	145

5.6.4.19	Type: MeasurementName	145

5.6.4.20	Type: SliceScopePerPlmn	145

5.7	Data Types related to 5G Operator Determined Barring	146

5.7.1	Introduction	146

5.7.2	Simple Data Types	146

5.7.3	Enumerations	146

5.7.3.1	Enumeration: RoamingOdb	146

5.7.3.2	Enumeration: OdbPacketServices	146

5.7.4	Structured Data Types	146

5.7.4.1	Type: OdbData	146

5.8	Data Types related to Charging	147

5.8.1	Introduction	147

5.8.2	Simple Data Types	147

5.8.3	Enumerations	147

5.8.4	Structured Data Types	147

5.8.4.1	Type: SecondaryRatUsageReport	147

5.8.4.2	Type: QoSFlowUsageReport	148

5.8.4.3	Type: SecondaryRatUsageInfo	148

5.8.4.4	Type: VolumeTimedReport	148

5.9	Data Types related to MBS	148

5.9.1	Introduction	148

5.9.2	Simple Data Types	148

5.9.3	Enumerations	149

5.9.3.1	Enumeration: MbsServiceType	149

5.9.3.2	Enumeration: MbsSessionActivityStatus	149

5.9.3.3	Enumeration: MbsSessionEventType	150

5.9.3.4	Enumeration: BroadcastDeliveryStatus	150

5.9.3.5	Enumeration: NrRedCapUeInfo	150

5.9.4	Structured Data Types	150

5.9.4.1	Type: MbsSessionId	150

5.9.4.2	Type: Tmgi	151

5.9.4.3	Type: Ssm	151

5.9.4.4	Type: MbsServiceArea	151

5.9.4.5	Type: NcgiTai	152

5.9.4.6	Type: MbsSession	153

5.9.4.7	Type: MbsSessionSubscription	157

5.9.4.8	Type: MbsSessionEventReportList	157

5.9.4.9	Type: MbsSessionEvent	158

5.9.4.10	Type: MbsSessionEventReport	158

5.9.4.11	Type: ExternalMbsServiceArea	158

5.9.4.12	Type: MbsSecurityContext	158

5.9.4.13	Type: MbsKeyInfo	159

5.9.4.14	Type: IngressTunAddrInfo	159

5.9.4.15	Type: MbsServiceAreaInfo	160

5.9.4.16	Type: MbsServiceInfo	160

5.9.4.17	Type: MbsMediaComp	160

5.9.4.18	Type: MbsMediaCompRm	160

5.9.4.19	Type: MbsQoSReq	161

5.9.4.20	Type: MbsMediaInfo	161

5.9.4.21	Data types describing alternative data types or combinations of data types	161

5.9.4.21.1	Type: AssociatedSessionId	161

5.10	Data Types related to Time Synchronization	161

5.10.1	Introduction	161

5.10.2	Simple Data Types	161

5.10.3	Enumerations	162

5.10.3.1	Enumeration: SynchronizationState	162

5.10.3.2	Enumeration: TimeSource	162

5.10.3.3	Enumeration: ClockQualityDetailLevel	162

5.10.3.4	Enumeration: ClockQualityDetailLevelRm	162

5.10.4	Structured Data Types	162

5.10.4.1	Type: ClockQualityAcceptanceCriterion	162

5.10.4.1A	Type: ClockQualityAcceptanceCriterionRm	163

5.10.4.2	Type: ClockQuality	163

5.10.4.2A	Type: ClockQualityRm	163

5.11	Data Types related to IMS SBA	164

5.11.1	Introduction	164

5.11.2	Simple Data Types	164

5.11.3	Enumerations	165

5.11.3.1	Enumeration: MediaResourceType	165

5.11.3.2	Enumeration: MediaProxy	165

5.11.3.3	Enumeration: SecuritySetup	166

5.11.3.4	Enumeration: BdcUsedBy	166

5.11.3.5	Enumeration: AdcEndpointType	166

5.11.3.6	Enumeration: ImsEvent	166

5.11.3.7	Type: AdcStatus	167

5.11.3.8	Type: BdcStatus	167

5.11.3.9	Enumeration: ImsReportMode	167

5.11.3.10	Enumeration: RenderingMode	167

5.11.4	Structured Data Types	168

5.11.4.1	Type: DcEndpoint	168

5.11.4.2	Type: DcStream	169

5.11.4.3	Type: ReplaceHttpUrl	169

5.11.4.4	Type: Endpoint	170

5.11.4.5	Type: AppBindingInfo	170

5.11.4.6	Type: AppDcInfo	170

5.11.4.7	Type: MdcEndpoint	171

5.11.4.8	Type: ImsReportingOptions	172

5.11.4.9	Type: ImsEventConfiguration	173

5.11.4.10	Type: EventInformation	173

5.11.4.11	Type: ImsEventFilter	173

5.11.4.12	Type: ImsEventReport	174

5.11.4.13	Type: ImsEventReportInfo	174

5.11.4.14	Type: AdcReport	174

5.11.4.15	Type: BdcReport	174

5.11.4.16	Type: RcdProperties	174

5.11.4.17	Type: RcdInformation	175

5.11.4.18	Type: IdentificationProperties	175

5.11.4.19	Type: DeliveringAddrProperties	176

5.11.4.20	Type: CommunicationsProperties	176

5.11.4.21	Type: GeographicalProperties	176

5.11.4.22	Type: OrganizationalProperties	177

5.11.4.23	Type: ExplanatoryProperties	178

5.12	Data Types related to Ambient IoT	178

5.12.1	Introduction	178

5.12.2	Simple Data Types	178

5.12.3	Enumerations	179

5.12.4	Structured Data Types	179

5.12.4.1	Type: AiotArea	179

5.12.4.2	Type: AiotAreaId	179

Annex A (normative): OpenAPI specification	180

A.1	General	180

A.2	Data related to Common Data Types	180

Annex B (informative): Change history	279

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for common data types that are used or may be expected to be used by multiple Service Based Interface APIs supported by the same or different Network Function(s).

The Principles and Guidelines for Services Definition are specified in 3GPP TS 29.501 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[3]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[4]	IETF RFC 1166: "Internet Numbers".

[5]	IETF RFC 5952: "A recommendation for IPv6 address text representation".

[6]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[9]	IETF RFC 9457: "Problem Details for HTTP APIs".

[10]	IETF RFC 3339: "Date and Time on the Internet: Timestamps".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP) ".

[12]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[13]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[14]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[15]	IETF RFC 9562: "Universally Unique IDentifier (UUID)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[18]	IETF RFC 6733: "Diameter Base Protocol".

[19]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) Protocol for 5G System (5GS); Stage 3".

[21]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[22]	Void.

[23]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[24]	ITU-T Recommendation Q.763 (1999): "Specifications of Signalling System No.7; Formats and codes".

[25]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[26]	3GPP TS 23.015: "Technical Realization of Operator Determined Barring".

[27]	3GPP TR 21.900: "Technical Specification Group working methods".

[28]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[29]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[30]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[31]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[32]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[33]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access; Stage 2".

[34]	BBF TR-069: "CPE WAN Management Protocol".

[35]	BBF TR-369: "User Services Platform (USP)".

[36]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[37]	BBF TR-470: "5G Wireless Wireline Convergence Architecture".

[38]	IEEE "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf

[39]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[40]	IETF RFC 5580: "Carrying Location Objects in RADIUS and Diameter".

[41]	BBF TR-456: "AGF Functional Requirements".

[42]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[43]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[44]	ECMA-262: "ECMAScript® Language Specification", https://www.ecma-international.org/ecma-262/5.1/.

[45]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[46]	3GPP TS 33.501: "Security architecture and procedures for 5G system; Stage 2".

[47]	IETF RFC 7542: "The Network Access Identifier".

[48]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[49]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[50]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[51]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[52]	3GPP TS 29.573: "5G System: Public Land Mobile Network (PLMN) Interconnection; Stage 3".

[53]	IETF RFC 8122: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[54]	IETF RFC 8842: "Session Description Protocol (SDP) Offer/Answer Considerations for Datagram Transport Layer Security (DTLS) and Transport Layer Security (TLS)".

[55]	IETF RFC 8841: "Session Description Protocol (SDP) Offer/Answer Procedures for Stream Control Transmission Protocol (SCTP) over Datagram Transport Layer Security (DTLS) Transport".

[56]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[57]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[58]	3GPP TS 32.255: "Charging management; 5G data connectivity domain charging; stage 2".

[59]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[60]	IETF RFC 8285: "A General Mechanism for RTP Header Extensions".

[61]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[62]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[63]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[64]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[65]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[66]	3GPP TS 28.558: "Management and orchestration; User Equipment (UE) level measurements for 5G system".

[67]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

Editor's note:	The above reference will be revised to RFC when finalized by IETF.

[68]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[69]	IETF RFC 5357: "A Two-Way Active Measurement Protocol (TWAMP)".

[70]	IETF RFC 4656: "A One-way Active Measurement Protocol (OWAMP)".

[71]	IETF RFC 8762: "Simple Two-Way Active Measurement Protocol".

[72]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[73]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[74]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[75]	IETF RFC 9796: "SIP Call-Info Parameters for Rich Call Data".

[76]	IETF RFC 6350: "vCard Format Specification".

[77]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[78]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AIoT	Ambient IoT

DNAI	Data Network Access Identifier

eRedCap UE	a UE with enhanced reduced capabilities as specified in clause 4.2.22.1 in 3GPP TS 38.306 [65].

EUI	Extended Unique Identifier

GEO	Geosynchronous Orbit

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber Coax

LEO	Low Earth Orbit

MEO	Medium Earth Orbit

N5GC	Non-5G Capable

NSSAA	Network Slice- Specific Authentication and Authorization

NTN	Non-Terrestrial Network

PEI	Permanent Equipment Identifier

RedCap UE	a UE with reduced capabilities as specified in clause 4.2.21.1 in 3GPP TS 38.306 [65].

SBI	Service Based Interface

SUPI	Subscription Permanent Identifier

UAV	Uncrewed Aerial Vehicle

# 4	Overview

For the different 5GC SBI API, data types shall be defined. Data types identified as common data types shall be defined in this Technical specification and should be referenced from individual 5GC SBI API specifications.

Data types applicable or intended to be applicable to several 5GC SBI API specifications should be interpreted as common data types.

# 5	Common Data Types

## 5.1	Introduction

In the following clauses, common data types for the following areas are defined:

-	Data types for generic usage;

-	Data types for Subscription, Identification and Numbering;

-	Data types related to 5G Network;

-	Data types related to 5G QoS;

-	Data types related to 5G Trace;

-	Data types related to 5G ODBs.

## 5.2	Data Types for Generic Usage

### 5.2.1	Introduction

This clause defines common data types for generic usage.

### 5.2.1A	Re-used Data Types

This clause specifies the re-used data types from other specifications.

# Table 5.2.1A-1: Re-used Data Types

| Data Type | Reference | Comments |
| --- | --- | --- |
| NFType | 3GPP TS 29.510 [29] |  |
| ServiceName | 3GPP TS 29.510 [29] |  |
| DataSetId | 3GPP TS 29.510 [29] |  |
| PlmnSnssai | 3GPP TS 29.510 [29] |  |
| GeographicArea | 3GPP TS 29.572 [43] |  |
| CivicAddress | 3GPP TS 29.572 [43] |  |
| NoProfileMatchInfo | 3GPP TS 29.510 [29] |  |
| ValidTimePeriod | 3GPP TS 29.503 [68] |  |
| TimeWindow | 3GPP TS 29.122 [74] | Time Window |

### 5.2.2	Simple Data Types

This clause specifies common simple data types.

Table 5.2.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Binary | string | String with format "binary" as defined in OpenAPI Specification [3] |
| BinaryRm | string | This data type is defined in the same way as the "Binary" data type, but with the OpenAPI "nullable: true" property. |
| Bytes | string | String with format "byte" as defined in OpenAPI Specification [3], i.e, base64-encoded characters, |
| BytesRm | string | This data type is defined in the same way as the "Bytes" data type, but with the OpenAPI "nullable: true" property. |
| Date | string | String with format "date" as defined in OpenAPI Specification [3] |
| DateRm | string | This data type is defined in the same way as the "Date" data type, but with the OpenAPI "nullable: true" property. |
| DateTime | string | String with format "date-time" as defined in OpenAPI Specification [3] |
| DateTimeRm | string | This data type is defined in the same way as the "DateTime" data type, but with the OpenAPI "nullable: true" property. |
| DiameterIdentity | Fqdn | String containing a Diameter Identity (FQDN), according to clause 4.3 of IETF RFC 6733 [18]. DiameterIdentity is defined as a simple data type because Fqdn is also a simple data type (string). |
| DiameterIdentityRm | FqdnRm | This data type is defined in the same way as the "DiameterIdentity" data type, but with the OpenAPI "nullable: true" property. DiameterIdentityRm is defined as a simple data type because FqdnRm is also a simple data type (either a string or null). |
| Double | number | Number with format "double" as defined in OpenAPI Specification [3] |
| DoubleRm | number | This data type is defined in the same way as the "Double" data type, but with the OpenAPI "nullable: true" property. |
| DurationSec | integer | Unsigned integer identifying a period of time in units of seconds. |
| DurationSecRm | integer | This data type is defined in the same way as the "DurationSec" data type, but with the OpenAPI "nullable: true" property. |
| Float | number | Number with format "float" as defined in OpenAPI Specification [3] |
| FloatRm | number | This data type is defined in the same way as the "Float" data type, but with the OpenAPI "nullable: true" property. |
| Uint16 | integer | Integer where the allowed values correspond to the value range of an unsigned 16-bit integer, i.e. 0 to 65535. Minimum = 0. Maximum = 65535. |
| Uint16Rm | integer | This data type is defined in the same way as the "Uint16" data type, but with the OpenAPI "nullable: true" property. |
| Int32 | integer | Integer with format "int32" as defined in OpenAPI Specification [3] |
| Int32Rm | integer | This data type is defined in the same way as the "Int32" data type, but with the OpenAPI "nullable: true" property. |
| Int64 | integer | Integer with format "int64" as defined in OpenAPI Specification [3] |
| Int64Rm | integer | This data type is defined in the same way as the "Int64" data type, but with the OpenAPI "nullable: true" property. |
| Ipv4Addr | string | String identifying a IPv4 address formatted in the "dotted decimal" notation as defined in IETF RFC 1166 [4]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])$' |
| Ipv4AddrRm | string | This data type is defined in the same way as the "Ipv4Addr" data type, but with the OpenAPI "nullable: true" property. |
| Ipv4AddrMask | string | String identifying a IPv4 address mask formatted in the "dotted decimal" notation as defined in IETF RFC 1166 [4]. Pattern: '^(([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])\.){3}([0-9]¦[1-9][0-9]¦1[0-9][0-9]¦2[0-4][0-9]¦25[0-5])(\/([0-9]¦[1-2][0-9]¦3[0-2]))$' |
| Ipv4AddrMaskRm | string | This data type is defined in the same way as the "Ipv4AddrMask" data type, but with the OpenAPI "nullable: true" property. |
| Ipv6Addr | string | String identifying an IPv6 address formatted according to clause 4 of IETF RFC 5952 [5]. The mixed IPv4 IPv6 notation according to clause 5 of IETF RFC 5952 [5] shall not be used. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$' |
| Ipv6AddrRm | string | This data type is defined in the same way as the "Ipv6Addr" data type, but with the OpenAPI "nullable: true" property. |
| Ipv6Prefix | string | String identifying an IPv6 address prefix formatted according to clause 4 of IETF RFC 5952 [5]. IPv6Prefix data type may contain an individual /128 IPv6 address. Pattern: '^((:¦(0?¦([1-9a-f][0-9a-f]{0,3}))):)((0?¦([1-9a-f][0-9a-f]{0,3})):){0,6}(:¦(0?¦([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])¦([0-9]{2})¦(1[0-1][0-9])¦(12[0-8])))$' and Pattern: '^((([^:]+:){7}([^:]+))¦((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$' |
| Ipv6PrefixRm | string | This data type is defined in the same way as the "Ipv6Prefix" data type, but with the OpenAPI "nullable: true" property. |
| MacAddr48 | string | String identifying a MAC address formatted in the hexadecimal notation according to clause 1.1 and clause 2.1 of IETF RFC 7042 [17]. Pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$' |
| MacAddr48Rm | string | This data type is defined in the same way as the "MacAddr48" data type, but with the OpenAPI "nullable: true" property. |
| SupportedFeatures | string | A string used to indicate the features supported by an API that is used as defined in clause 6.6 in 3GPP TS 29.500 [25]. The string shall contain a bitmask indicating supported features in hexadecimal representation: Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent the support of 4 features as described in table 5.2.2-3. The most significant character representing the highest-numbered features shall appear first in the string, and the character representing features 1 to 4 shall appear last in the string. The list of features and their numbering (starting with 1) are defined separately for each API. If the string contains a lower number of characters than there are defined features for an API, all features that would be represented by characters that are not present in the string are not supported. |
| Uinteger | integer | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. Minimum = 0. |
| UintegerRm | integer | This data type is defined in the same way as the "Uinteger" data type, but with the OpenAPI "nullable: true" property. |
| Uint32 | integer | Integer where the allowed values correspond to the value range of an unsigned 32-bit integer, i.e. 0 to (2^32)-1. Minimum = 0. Maximum = 4294967295. |
| Uint32Rm | integer | This data type is defined in the same way as the "Uint32" data type, but with the OpenAPI "nullable: true" property. |
| Uint64 | integer | Integer where the allowed values correspond to the value range of an unsigned 64-bit integer, i.e. 0 to (2^64)-1. Minimum = 0. Maximum = 18446744073709551615. |
| Uint64Rm | integer | This data type is defined in the same way as the "Uint64" data type, but with the OpenAPI "nullable: true" property. |
| Uri | string | String providing an URI formatted according to IETF RFC 3986 [6]. If the URI fields intended to convey generic data (e.g., in the value part of a query parameter, or in the URI path segments) contain reserved characters, these reserved characters shall be percent-encoded as defined in clause 5.2.10.2 of 3GPP TS 29.500 [25]. |
| UriRm | string | This data type is defined in the same way as the "Uri" data type, but with the OpenAPI "nullable: true" property. |
| VarUeId | string | String represents the SUPI or GPSI. Pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦msisdn-[0-9]{5,15}¦extid-[^@]+@[^@]+¦gci-.+¦gli-.+¦.+)$". |
| VarUeIdRm | string | This data type is defined in the same way as the "VarUeId" data type, but with the OpenAPI "nullable: true" property. |
| TimeZone | string | String with format "<time-numoffset>" optionally appended by "<daylightSavingTime>", where: - <time-numoffset> shall represent the time zone adjusted for daylight saving time and be encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339 [10]; - <daylightSavingTime> shall represent the adjustment that has been made and shall be encoded as "+1" or "+2" for a +1 or +2 hours adjustment. Example: "-08:00+1" (for 8 hours behind UTC, +1 hour adjustment for Daylight Saving Time). |
| TimeZoneRm | string | This data type is defined in the same way as the "TimeZone" data type, but with the OpenAPI "nullable: true" property. |
| StnSr | string | String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003 [7]. |
| StnSrRm | string | This data type is defined in the same way as the "StnSr" data type, but with the OpenAPI "nullable: true" property. |
| CMsisdn | string | String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003 [7]). Pattern: "^[0-9]{5,15}$". |
| CMsisdnRm | string | This data type is defined in the same way as the "CMsisdn" data type, but with the OpenAPI "nullable: true" property. |
| MonthOfYear | integer | Integer between and including 1 and 12 denoting a month. "1" shall indicate "January", and the subsequent months shall be indicated with the next higher numbers. "12" shall indicate "December". |
| DayOfWeek | integer | Integer between and including 1 and 7 denoting a weekday. "1" shall indicate "Monday", and the subsequent weekdays shall be indicated with the next higher numbers. "7" shall indicate "Sunday". |
| TimeOfDay | string | String with format "partial-time" or "full-time" as defined in clause 5.6 of IETF RFC 3339 [10]. Examples: "20:15:00", "20:15:00-08:00" (for 8 hours behind UTC). |
| EmptyObject | object | Empty JSON object: { } It is defined with the keyword: "additionalProperties: false". |
| Fqdn | string | Fully Qualified Domain Name Pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$' minLength: 4 maxLength: 253 As specified for FQDNs in 3GPP TS 23.003 [7], clause 19.4.2.1, FQDNs shall be handled as case-insensitive strings. |
| FqdnRm | string | This data type is defined in the same way as the "Fqdn" data type, but it also allows the null value. |

Table 5.2.2-2: Reused OpenAPI data types

| Type Name | Description |
| --- | --- |
| boolean | As defined in OpenAPI Specification [3] |
| integer | As defined in OpenAPI Specification [3] |
| number | As defined in OpenAPI Specification [3] |
| string | As defined in OpenAPI Specification [3] |
| object | As defined in OpenAPI Specification [3] |
| array | As defined in OpenAPI Specification [3] |
| NOTE Data types defined in OpenAPI Specification [3] do not follow the UpperCamel convention for data types in 3GPP TS 29.501 [2] |  |

Table 5.2.2-3: Meaning of a Hexadecimal Character in SupportedFeatures Type

| Character | Feature n+3 supported | Feature n+2 supported | Feature n+1 supported | Feature n supported |
| --- | --- | --- | --- | --- |
| "0" | no | no | no | no |
| "1" | no | no | no | yes |
| "2" | no | no | yes | no |
| "3" | no | no | yes | yes |
| "4" | no | yes | no | no |
| "5" | no | yes | no | yes |
| "6" | no | yes | yes | no |
| "7" | no | yes | yes | yes |
| "8" | yes | no | no | no |
| "9" | yes | no | no | yes |
| "A" | yes | no | yes | no |
| "B" | yes | no | yes | yes |
| "C" | yes | yes | no | no |
| "D" | yes | yes | no | yes |
| "E" | yes | yes | yes | no |
| "F" | yes | yes | yes | yes |
| NOTE 1 "n" shall be i * 4 + 1, where "i" is zero or a natural number, i.e permissible values of "n" are 1, 5, 9, … NOTE 2 In this table if a feature is not defined, it shall also be indicated with value "no". |  |  |  |  |

For example, if only the first feature defined in the feature list is set to 1, the corresponding SupportedFeatures attribute would have a hexadecimal character value of "1", or a string of hexadecimal characters with value of "001" (any amount of 0's to the left of the 1 would result into an equivalent feature list). If we have 32 features defined, and only the last feature in a feature list is set to 1, the corresponding SupportedFeatures attribute would have a string of hexadecimal characters with value of "80000000"(see the description of the SupportedFeatures encoding in Table 5.2.2-1).

### 5.2.3	Enumerations

#### 5.2.3.1	Enumeration: PatchOperation

Table 5.2.3.1-1: Enumeration PatchOperation

| Enumeration value | Description |
| --- | --- |
| "add" | Add operation as defined in IETF RFC 6902 [14]. |
| "copy" | Copy operation as defined in IETF RFC 6902 [14]. |
| "move" | Move operation as defined in IETF RFC 6902 [14]. |
| "remove" | Remove operation as defined in IETF RFC 6902 [14]. |
| "replace" | Replace operation as defined in IETF RFC 6902 [14]. |
| "test" | Test operation as defined in IETF RFC 6902 [14]. |

#### 5.2.3.2	Enumeration: UriScheme

Table 5.2.3.2-1: Enumeration UriScheme

| Enumeration value | Description |
| --- | --- |
| "http" | HTTP URI scheme |
| "https" | HTTPS URI scheme |

#### 5.2.3.3	Enumeration: ChangeType

Table 5.2.3.3-1: Enumeration ChangeType

| Enumeration value | Description |
| --- | --- |
| "ADD" | This value indicates new attribute has been added to the resourceThe "ADD" operation performs one of the following functions, depending upon what the target location references: -If the target location specifies an array index, a new value is inserted into the array at the specified index. -If the target location specifies an object member that does not already exist, a new member is added to the object. -If the target location specifies an object member that does exist, that member's value is replaced. The operation object shall contain a "value" member whose content specifies the value to be added. For example: { "op": "ADD", "path": "/a/b/c", "value": [ "foo", "bar" ] } When the operation is applied, the target location shall reference one of: -The root of the target document - whereupon the specified value becomes the entire content of the target document. -A member to add to an existing object - whereupon the supplied value is added to that object at the indicated location. If the member already exists, it is replaced by the specified value. -An element to add to an existing array - whereupon the supplied value is added to the array at the indicated location. Any elements at or above the specified index are shifted one position to the right. The specified index shall not be greater than the number of elements in the array. If the "-" character is used to index the end of the array (see IETF RFC 6901 [12]), this has the effect of appending the value to the array. Because this operation is designed to add to existing objects and arrays, its target location will often not exist. Although the pointer's error handling algorithm will thus be invoked, this specification defines the error handling behavior for "ADD" pointers to ignore that error and add the value as specified. However, the object itself or an array containing it does need to exist, and it remains an error for that not to be the case. For example, an "ADD" with a target location of "/a/b" starting with this document: { "a": { "foo": 1 } } is not an error, because "a" exists, and "b" will be added to its value. It is an error in this document: { "q": { "bar": 2 } } because "a" does not exist. |
| "MOVE" | This value indicates existing attribute has been moved to a different path in the resource. The "MOVE" operation removes the value at a specified location and adds it to the target location. The operation object shall contain a "from" member, which is a string containing a JSON Pointer value that references the location in the target document to move the value from. The "from" location shall exist for the operation to be successful. For example: { "op": "MOVE", "from": "/a/b/c", "path": "/a/b/d" } This operation is functionally identical to a "REMOVE" operation on the "from" location, followed immediately by an "ADD" operation at the target location with the value that was just removed. The "from" location shall not be a proper prefix of the "path" location; i.e., a location cannot be moved into one of its children. |
| "REMOVE" | This value indicates existing attribute has been deleted from the resource. The "REMOVE" operation removes the value at the target location. The target location shall exist for the operation to be successful. For example: { "op": "REMOVE", "path": "/a/b/c" } If removing an element from an array, any elements above the specified index are shifted one position to the left. |
| "REPLACE" | This value indicates existing attribute has been updated with new value. The "REPLACE" operation replaces the value at the target location with a new value. The operation object shall contain a "value" member whose content specifies the replacement value. The target location shall exist for the operation to be successful. For example: { "op": "REPLACE", "path": "/a/b/c", "value": 42 } This operation is functionally identical to a "REMOVE" operation for a value, followed immediately by an "ADD" operation at the same location with the replacement value. |

#### 5.2.3.4	Enumeration: HttpMethod

# Table 5.2.3.4-1: Enumeration HttpMethod

| Enumeration value | Description |
| --- | --- |
| "GET" | HTTP GET method. |
| "POST" | HTTP POST method. |
| "PUT" | HTTP PUT method. |
| "DELETE" | HTTP DELETE method. |
| "PATCH" | HTTP PATCH method. |
| "OPTIONS" | HTTP OPTIONS method. |
| "HEAD" | HTTP HEAD method. |
| "CONNECT" | HTTP CONNECT method. |
| "TRACE" | HTTP TRACE method. |

#### 5.2.3.5	Enumeration: NullValue

# Table 5.2.3.5-1: Enumeration NullValue

| Enumeration value | Description |
| --- | --- |
| null | JSON's null value |

#### 5.2.3.6	Enumeration: MatchingOperator

Table 5.2.3.6-1: Enumeration MatchingOperator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FULL_MATCH | Indicates a full match between the string against which the matching applies and the provided matching string. |  |
| MATCH_ALL | Indicate a match for any string |  |
| STARTS_WITH | Indicates a match when the string against which the matching applies starts with the provided matching string (e.g. the string "smartmeter-01.company.com" matches the matching string "smartmeter-"). |  |
| NOT_START_WITH | Indicates a match when the string against which the matching applies does not start with the provided matching string (e.g. the string "smartmeter-01.company.com" matches the matching string "metersmart-"). |  |
| ENDS_WITH | Indicates a match when the string against which the matching applies ends with the matching string (e.g. the string "somehost.company.com" matches the matching string "company.com"). |  |
| NOT_END_WITH | Indicates a match when the string against which the matching applies does not end with the matching string (e.g. the string "somehost.company.com" matches the matching string "company.se"). |  |
| CONTAINS | Indicates a match when the string against which the matching applies contains the matching string (e.g. the string "media.news.com" matches the matching string "media"). |  |
| NOT_CONTAIN | Indicates a match when the string against which the matching applies does not contain the matching string (e.g. the string "media.news.com" matches the matching string "aidem"). |  |

#### 5.2.3.7	Enumeration: UpfPacketInspectionFunctionality

Table 5.2.3.7-1: Enumeration UpfPacketInspectionFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "DEEP_PACKET_INSPECTION" | Higher Layer Packet Inspection |  |
| "MAC_FILTER_BASED_PACKET_DETECTION" | MAC Filter based Packet Detection |  |
| "IP_FILTER_BASED_PACKET_DETECTION" | IP Filter based Packet Detection |  |

### 5.2.4	Structured Data Types

#### 5.2.4.1	Type: ProblemDetails

Table 5.2.4.1-1: Definition of type ProblemDetails

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| type | Uri | O | 0..1 | A URI reference according to IETF RFC 3986 [6] that identifies the problem type. |
| title | string | O | 0..1 | A short, human-readable summary of the problem type. It should not change from occurrence to occurrence of the problem. |
| status | integer | O | 0..1 | The HTTP status code for this occurrence of the problem. |
| detail | string | O | 0..1 | A human-readable explanation specific to this occurrence of the problem. |
| instance | Uri | O | 0..1 | A URI reference that identifies the specific occurrence of the problem. |
| cause | string | C | 0..1 | A machine-readable application error cause specific to this occurrence of the problem This IE should be present and provide application-related error information, if available. |
| invalidParams | array(InvalidParam) | O | 1..N | Description of invalid or missing parameters, for a request rejected due to invalid or missing parameters. |
| supportedFeatures | SupportedFeatures | C | 0..1 | Features supported by the NF Service Producer. This IE shall be present when rejecting a request due to an unsupported query parameter, if at least one feature is defined for the corresponding service in the version of the specification that the NF Service Producer implements (see clause 5.2.9 of 3GPP TS 29.500 [25]). When present, this IE shall indicate the features supported by the NF Service Producer; if the NF Service Producer supports no features, this IE shall be set to the character "0". |
| accessTokenError | AccessTokenErr | C | 0..1 | This IE should be present if an SCP request to get an access token was rejected by the NRF. When present, it should contain the Access Token Error content received from the NRF. |
| accessTokenRequest | AccessTokenReq | O | 0..1 | This IE may be present if an SCP request to get an access token was rejected by the NRF. When present, it shall contain the Access Token Request that was sent by the SCP. |
| nrfId | Fqdn | O | 0..1 | This IE may be present if an SCP request to get an access token was rejected by the NRF. When present, it shall contain the Identity (i.e. FQDN) of the NRF that rejected the access token request. |
| supportedApiVersions | array(string) | O | 1..N | This IE may be present if the SCP did not find NF service producers matching the MAJOR API version of the incoming service request and MAJOR API version(s) are known to be supported by NF service producers for the corresponding service. When present, it shall contain MAJOR API version(s) known to be supported by NF service producers for the corresponding service. The API version shall be encoded as the apiVersionInUri defined in NFServiceVersion defined in 3GPP TS 29.510 [29] (e.g. "v1"). |
| noProfileMatchInfo | NoProfileMatchInfo | O | 0..1 | This IE may be present if an SCP discovery returned no profiles. |
| NOTE 1: See IETF RFC 9457 [9] for detailed information and guidance for each attribute, and 3GPP TS 29.501 [2] for guidelines on error handling support by 5GC SBI APIs. NOTE 2: Additional attributes may be defined per API. |  |  |  |  |

#### 5.2.4.2	Type: Link

Table 5.2.4.2-1: Definition of type link

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| href | Uri | M | 1 | It contains the URI of the linked resource. |

#### 5.2.4.3	Type PatchItem

Table 5.2.4.3-1: Definition of type PatchItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| op | PatchOperation | M | 1 | This IE indicates the patch operation as defined in IETF RFC 6902 [14] to be performed on resource. |  |
| path | string | M | 1 | This IE contains a JSON pointer value (as defined in IETF RFC 6901 [12]) that references a location of a resource on which the patch operation shall be performed. |  |
| from | string | C | 0..1 | This IE indicates the path of the source JSON element (according to JSON Pointer syntax) being moved or copied to the location indicated by the "path" attribute. It shall be present if the patch operation is "move" or "copy". |  |
| value | Any Type | C | 0..1 | This IE indicates a new value for the resource specified in the path attribute. It shall be present if the patch operation is "add", "replace" or "test". The data type of this attribute shall be the same as the type of the resource on which the patch operation shall be performed. The null value shall be allowed. |  |

#### 5.2.4.4	Type: LinksValueSchema

Table 5.2.4.4-1: Definition of type LinksValueSchema as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| array(Link) | 1..N | Array of links |
| Link | 1 | link |

#### 5.2.4.5	Type: SelfLink

Table 5.2.4.5-1: Definition of type SelfLink

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| self | Link | M | 1 | It contains the URI of the linked resource. |

#### 5.2.4.6	Type: InvalidParam

Table 5.2.4.6-1: Definition of type InvalidParam

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| param | string | M | 1 | If the invalid parameter is an attribute in a JSON body, this IE shall contain the attribute's name and shall be encoded as a JSON Pointer. If the invalid parameter is an HTTP header, this IE shall be formatted as the concatenation of the string "header: " plus the name of such header. If the invalid parameter is a query parameter, this IE shall be formatted as the concatenation of the string "query: " plus the name of such query parameter. If the invalid parameter is a variable part in the path of a resource URI, this IE shall contain the name of the variable, including the symbols "{" and "}" used in OpenAPI specification as the notation to represent variable path segments. If the invalid parameter is a missing claim name in an OAuth2 access token, this IE shall be formatted as the concatenation of the string "access-token-claim: " plus the name of the missing claim. Example: "access-token-claim: producerSnssaiList" |
| reason | string | O | 0..1 | A human-readable reason, e.g. "must be a positive integer". In cases involving failed operations in a PATCH request, the reason string should identify the operation that failed using the operation's array index to assist in correlation of the invalid parameter with the failed operation, e.g." Replacement value invalid for attribute [failed operation index: 4]". |

#### 5.2.4.7	Type: LinkRm

This data type is defined in the same way as the "Link" data type, but with the OpenAPI "nullable: true" property.

#### 5.2.4.8	Type ChangeItem

Table 5.2.4.8-1: Definition of type ChangeItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| op | ChangeType | M | 1 | This IE indicates the operation to be performed on the resource. |  |
| path | string | M | 1 | This IE contains a JSON pointer value (as defined in IETF RFC 6901 [12]) that references a target location within the resource on which the change has been applied. (See Note) |  |
| from | string | C | 0..1 | This IE indicates the path of the source JSON element (according to JSON Pointer syntax) being moved or copied to the location indicated by the "path" attribute. It shall be present if the "op" attribute is of value "MOVE". |  |
| origValue | Any Type | O | 0..1 | This IE indicates the original value at the target location within the resource specified in the path attribute. This attribute only applies when the "op" attribute is of value "REMOVE", "REPLACE" or "MOVE" Based on the use case, this attribute may be included. |  |
| newValue | Any Type | C | 0..1 | This IE indicates a new value at the target location within the resource specified in the path attribute. It shall be present if the "op" attribute is of value "ADD", "REPLACE". The data type of this attribute shall be the same as the type of the resource on which the change has happened. The null value shall be allowed. |  |
| NOTE: As described in IETF RFC 6901 [12], the value "" (empty JSON string) is the JSON Pointer expression to represent "the whole JSON document"; therefore, when the attribute "path" takes value "" and attribute "op" takes values "ADD" or "REMOVE", this shall be interpreted as the creation or deletion respectively of the resource to which this "ChangeItem" refers to. |  |  |  |  |  |

#### 5.2.4.9	Type NotifyItem

Table 5.2.4.9-1: Definition of type NotifyItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceId | Uri | M | 1 | This IE contains the URI of the resource which has been changed. |  |
| changes | array(ChangeItem) | M | 1..N | This IE contains the changes which have been applied on the resource identified by the resourceId attribute. See NOTE. |  |
| NOTE: There may be more than one way to express a given modification of a resource's representation. E.g. removing one attribute from an object can be done by a) a change item with op set to "REMOVE" and path pointing to the attribute to be removed, or b) a change item with op set to "REPLACE" and path pointing to the object, and a newValue of the object i.e. without the attribute that has been removed. It is up to sending nodes decision to select one of the available ways to express the modification and the receiving node shall support all possible ways. |  |  |  |  |  |

#### 5.2.4.10	Type: ComplexQuery

# Table 5.2.4.10-1: Definition of type ComplexQuery as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| Cnf | 1 | A conjunctive normal form |
| Dnf | 1 | A disjunctive normal form |

The ComplexQuery data type is either a conjunctive normal form or a disjunctive normal form. The attribute names "cnfUnits" and "dnfUnits" (see clause 5.2.4.11 and clause 5.2.4.12) serve as discriminator.

#### 5.2.4.11	Type: Cnf

Table 5.2.4.11-1: Definition of type Cnf

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cnfUnits | array(CnfUnit) | M | 1..N | During the processing of cnfUnits attribute, all the members in the array shall be interpreted as logically concatenated with logical "AND". |  |

#### 5.2.4.12	Type: Dnf

Table 5.2.4.12-1: Definition of type Dnf

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnfUnits | array(DnfUnit) | M | 1..N | During the processing of dnfUnits attribute, all the members in the array shall be interpreted as logically concatenated with logical "OR". |  |

#### 5.2.4.13	Type: CnfUnit

Table 5.2.4.13-1: Definition of type CnfUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cnfUnit | array(Atom) | M | 1..N | During the processing of cnfUnit attribute, all the members in the array shall be interpreted as logically concatenated with logical "OR". |  |

#### 5.2.4.14	Type: DnfUnit

Table 5.2.4.14-1: Definition of type DnfUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnfUnit | array(Atom) | M | 1..N | During the processing of dnfUnit attribute, all the members in the array shall be interpreted as logically concatenated with logical "AND". |  |

#### 5.2.4.15	Type: Atom

Table 5.2.4.15-1: Definition of type Atom

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| attr | string | M | 1 | This attribute contains the name of a defined query parameter. |  |
| value | Any Type | M | 1 | This attribute contains the value of the query parameter as indicated by attr attribute. |  |
| negative | boolean | O | 0..1 | This attribute indicates whether the negative condition applies for the query condition. |  |

#### 5.2.4.16	Void

#### 5.2.4.17	Type: PatchResult

# Table 5.2.4.17-1: Definition of type PatchResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| report | array(ReportItem) | M | 1..N | The execution report contains an array of report items. Each report item indicates one failed modification. |  |

#### 5.2.4.18	Type: ReportItem

# Table 5.2.4.18-1: Definition of type ReportItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| path | string | M | 1 | This attribute contains a JSON pointer value (as defined in IETF RFC 6901 [12]) that references a location of a resource to which the modification is subject. |  |
| reason | string | O | 0..1 | A human-readable reason providing details on the reported modification failure. The reason string should identify the operation that failed using the operation's array index to assist in correlation of the invalid parameter with the failed operation, e.g. "Replacement value invalid for attribute [failed operation index: 4]". |  |

#### 5.2.4.19	Type: HalTemplate

# Table 5.2.4.19-1: Definition of type HalTemplate

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| title | string | O | 0..1 | A human-readable string that can be used to identify this template. |
| method | HttpMethod | M | 1 | The HTTP method that should be applied for the corresponding link. If the value is not understood, the value shall be treated as an HTTP GET. |
| contentType | string | O | 0..1 | The media type that should be used for the corresponding request. If the attribute is missing, or contains an unrecognized value, the client should act as if the contentType is set to "application/json". |
| properties | array(Property) | O | 1..N | The properties that should be included in the body of the corresponding request. If the contentType attribute is set to "application/json", then this attribute describes the attributes of the JSON object of the body. |

#### 5.2.4.20	Type: Property

# Table 5.2.4.20-1: Definition of type Property

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| name | string | M | 1 | The name of the property. |
| required | boolean | O | 0..1 | Indicates whether the property is required: - true: required - false(default): not required |
| regex | string | O | 0..1 | A regular expression string to be applied to the value of the property. |
| value | string | O | 0..1 | The property value. When present, it shall be a valid JSON string. |

#### 5.2.4.21	Type: RedirectResponse

Table 5.2.4.21-1: Definition of type RedirectResponse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cause | string | C | 0..1 | A machine-readable cause string, specific to this occurrence of the redirection. If the redirection is initiated by an SCP towards another SCP, this IE shall be present and set to "SCP_REDIRECTION" (see clause 6.10.9 of 3GPP TS 29.500 [25]). If the redirection is initiated by an SEPP towards another SEPP over an non N32 interface, this IE shall be present and set to "SEPP_REDIRECTION" (see clause 6.10.9 of 3GPP TS 29.500 [25] and clause 6.1.8 of 3GPP TS 29.573 [52]). |
| targetScp | Uri | O | 0..1 | ApiRoot of the SCP towards which an HTTP request is redirected (see clause 6.10.9 of 3GPP TS 29.500 [25]). |
| targetSepp | Uri | O | 0..1 | ApiRoot of the SEPP towards which an non N32 interface HTTP request is redirected (see clause 6.10.9 of 3GPP TS 29.500 [25]) and clause 6.1.8 of 3GPP TS 29.573 [52]). |

#### 5.2.4.22	Type: TunnelAddress

Table 5.2.4.22-1: Definition of type TunnelAddress

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | IPv4 address (NOTE) |  |
| Ipv6Addr | Ipv6Addr | C | 0..1 | IPv6 address (NOTE) |  |
| portNumber | Uinteger | M | 1 | UDP Port |  |
| NOTE: At least one of these IEs shall be present. |  |  |  |  |  |

#### 5.2.4.23	Type: FqdnPatternMatchingRule

Table 5.2.4.23-1: Definition of type FqdnPatternMatchingRule

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| regex | string | C | 0..1 | One FQDN pattern, defined as a regular expression according to the ECMA-262 dialect [44]. (NOTE) |
| stringMatchingRule | StringMatchingRule | C | 0..1 | One FQDN pattern, described as a string match rule. (NOTE) |
| NOTE: When provisioning an FQDN pattern, the StringMatchingRule shall be preferred over regular expression and used whenever possible (i.e. if the pattern can be described by a string matching rule) to optimize the matching process and reduce the processing load, since the use of regular expressions can be more computing intensive than using string matching rule. Either the regex or the stringMatchingRule shall be present. |  |  |  |  |

EXAMPLE 1:	A FQDN pattern described by a string matching rule matching all FQDNs with "smartmeter-{factoryID}.company.com" where "{factoryID}" can be any string
JSON: {"stringMatchingRule": {stringMatchingConditions:[{"matchingString": "smartmeter-","matchingOperator": "STARTS_WITH"},{"matchingString": ".company.com","matchingOperator": "ENDS_WITH"}]}}

EXAMPLE 2:	A FQDN pattern described by a regular expression matching all FQDNs with "smartmeter-{factoryID}.company.com" where "{factoryID}" can be any string.
JSON: {"regex": "^smartmeter-.+\.company\.com$"}

#### 5.2.4.24	Type: StringMatchingRule

Table 5.2.4.24-1: Definition of type StringMatchingRule

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stringMatchingConditions | array(StringMatchingCondition) | M | 1..N | Contains a list of conditions which shall be evaluated for string matching. |
| NOTE: The conditions in the stringMatchingConditions array shall be evaluated as "and" logical relationship. |  |  |  |  |

#### 5.2.4.25	Type: StringMatchingCondition

Table 5.2.4.25-1: Definition of type StringMatchingCondition

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| matchingString | string | C | 0..1 | This IE shall be present to identify the string against which the matching is performed except when the matchingOperator is MATCH_ALL. |
| matchingOperator | MatchingOperator | M | 1 | Identifies the matching operation. |

#### 5.2.4.26	Type: Ipv4AddressRange

Table 5.2.4.26-1: Definition of type Ipv4AddressRange

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| start | Ipv4Addr | M | 1 | First value identifying the start of an IPv4 address range |
| end | Ipv4Addr | M | 1 | Last value identifying the end of an IPv4 address range |

#### 5.2.4.27	Type: Ipv6AddressRange

Table 5.2.4.27-1: Definition of type Ipv6AddressRange

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| start | Ipv6Addr | M | 1 | First value identifying the start of an IPv6 address range |
| end | Ipv6Addr | M | 1 | Last value identifying the end of an IPv6 address range |

#### 5.2.4.28	Type: Ipv6PrefixRange

Table 5.2.4.28-1: Definition of type Ipv6PrefixRange

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| start | Ipv6Prefix | M | 1 | First value identifying the start of an Ipv6 prefix range |
| end | Ipv6Prefix | M | 1 | Last value identifying the end of an Ipv6 prefix range |
| NOTE: When Ipv6PrefixRange is used to identify a range of Ipv6 addresses served by certain NF (e.g. BSF), the range of Ipv6 addresses identified by the Ipv6PrefixRange shall include the entire Ipv6 addresses represented by the "start" and "end" Ipv6 prefixes. For example, if the "start" attribute is set to "240e:006a:0000:0000::/32" and the "end" attribute is set to "250e:006a:0000:0000::/32", the Ipv6PrefixRange identifies all the Ipv6 addresses from the start Ipv6 address "240e:006a:0000:0000::/32" to the end Ipv6 address "250e:006a:ffff:ffff:ffff:ffff:ffff:ffff/32". |  |  |  |  |

#### 5.2.4.29	Type NfSignallingInfo

Table 5.2.4.29-1: Definition of type NfSignallingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nfInstanceId | NfInstanceId | M | 1 | The NF instance ID of the NF. |  |
| nfSigInfoPerWndw | array(NfSignallingInfoPerTimeWindow) | O | 1..N | Contains NF signalling information per time window. |  |
| nfHeartbeatInfo | NfHeartbeatInfo | O | 0..1 | Contains NF heartbeat-related information. |  |
| unexpStatusInd | boolean | O | 0..1 | Indicates whether the NF is at an unexpected status (i.e. deviates from the normal operations, based on thresholds or rules configured by operator). - "true": the NF is at an unexpected status. - "false": the NF is not at an unexpected status. The default value is "false" if omitted. |  |

#### 5.2.4.30	Type NfSignallingInfoPerTimeWindow

Table 5.2.4.30-1: Definition of type NfSignallingInfoPerTimeWindow

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tw | TimeWindow | M | 1 | The time window in which the provided signalling information occurred. |  |
| nfSigInfoPerService | array(NfSignallingInfoPerService) | M | 1..N | NF signalling information per NF service instance in the NF instance. |  |

#### 5.2.4.31	Type NfSignallingInfoPerService

Table 5.2.4.31-1: Definition of type NfSignallingInfoPerService

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nfServiceInstanceId | string | M | 1 | The NF sevice instance ID as specified in table 6.22.2-1 of 3GPP TS 23.288 [77] to which the provided signalling information refers. |  |
| serviceName | ServiceName | M | 1 | The name of the service of the NF service instance as specified in clause 6.1.6.3.11 of 3GPP TS 29.510 [29]. |  |
| avgProcTime | Float | O | 0..1 | The average processing time (in milliseconds) of each request, i.e. the time duration between receiving the request from an NF and sending the response to the NF. |  |
| numOfReq | Uinteger | O | 0..1 | The number of requests received for this service by the NF service instance. (NOTE) |  |
| numOfReqUnresp | Uinteger | O | 0..1 | The number of requests received for this service by the NF service instance which were not responded. (NOTE) |  |
| numOfReqReject | Uinteger | O | 0..1 | The number of requests received for this service by the NF service instance which were rejected. (NOTE) |  |
| numOfRedMessages | Uinteger | O | 0..1 | The number of redundant messages received by the NF service instance, i.e. messages which were transmitted multiple times. |  |
| numOfPosteriorReq | Uinteger | O | 0..1 | The number of posterior requests, i.e. requests that were triggered by a previous request received by the NFservice instance. |  |
| NOTE: The number of failed requests equals to "numOfReqUnresp" plus "numOfReqReject". The number of successful requests equals to "numOfReq" minus the number of failed requests. |  |  |  |  |  |

#### 5.2.4.32	Type NfHeartbeatInfo

Table 5.2.4.32-1: Definition of type NfHeartbeatInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| numOfRetrans | Uinteger | O | 0..1 | Number of retransmissions performed. |  |
| hbIntvl | Uinteger | O | 0..1 | NF heartbeat interval in milliseconds. |  |

## 5.3	Data Types related to Subscription, Identification and Numbering

### 5.3.1	Introduction

This clause defines common data types related to subscription, identification and numbering information.

### 5.3.2	Simple Data Types

This clause specifies common simple data types.

Table 5.3.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Dnn | string | String representing a Data Network as defined in clause 9A of 3GPP TS 23.003 [7]; it shall contain either a DNN Network Identifier, or a full DNN with both the Network Identifier and Operator Identifier, as specified in 3GPP TS 23.003 [7] clauses 9.1.1 and 9.1.2. It shall be coded as string in which the labels are separated by dots (e.g. "Label1.Label2.Label3"). See NOTE 2. As specified for APNs in 3GPP TS 23.003 [7], clause 9.1, DNNs shall be handled as case-insensitive strings. |
| DnnRm | string | This data type is defined in the same way as the "Dnn" data type, but with the OpenAPI "nullable: true" property. |
| WildcardDnn | string | String representing the Wildcard DNN. It shall contain the string "*". Pattern: '^[*]$' |
| WildcardDnnRm | string | This data type is defined in the same way as the "WildcardDnn" data type, but with the OpenAPI "nullable: true" property. |
| Gpsi | string | String representing a Gpsi shall contain either an External Id or an MSISDN. It shall be formatted as follows: -External Identifier: "extid-<extid>, where <extid> shall be formatted according to clause 19.7.2 of 3GPP TS 23.003 [7] that describes an External Identifier. -MSISDN: "msisdn-<msisdn>, where <msisdn> shall be formatted according to clause 3.3 of 3GPP TS 23.003 [7] that describes an MSISDN. Pattern: '^(msisdn-[0-9]{5,15}¦extid-.+@.+¦.+)$' |
| GpsiRm | string | This data type is defined in the same way as the "Gpsi" data type, but with the OpenAPI "nullable: true" property. |
| GroupId | string | String representing a group of devices network internal globally unique ID which identifies a set of IMSIs, as specified in clause 19.9 of 3GPP TS 23.003 [7]. For 5G related service, the GroupServiceId shall identify the specific service for which the IMSI-Group-Id is used, as specified in clause 5.3.3.1. Pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'. |
| GroupIdRm | string | This data type is defined in the same way as the "GroupId" data type, but with the OpenAPI "nullable: true" property. |
| ExternalGroupId | string | String representing External Group Identifier that identifies a group made up of one or more subscriptions associated to a group of IMSIs, as specified in clause 19.7.3 of 3GPP TS 23.003 [7]. Pattern: "^extgroupid-[^@]+@[^@]+$" |
| ExternalGroupIdRm | string | This data type is defined in the same way as the "ExternalGroupId" data type, but with the OpenAPI "nullable: true" property. |
| Pei | string | String representing a Permanent Equipment Identifier that may contain: - an IMEI or IMEISV, as specified in clause 6.2 of 3GPP TS 23.003 [7]; - a MAC address for a 5G-RG or FN-RG via wireline access, with an indication that this address cannot be trusted for regulatory purpose if this address cannot be used as an Equipment Identifier of the FN-RG, as specified in clause 4.7.7 of 3GPP TS 23.316 [30]. - an IEEE Extended Unique Identifier (EUI-64), for UEs not supporting any 3GPP access technologies, as defined in IEEE "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)" [38]. Pattern: '^(imei-[0-9]{15}¦imeisv-[0-9]{16}¦mac((-[0-9a-fA-F]{2}){6})(-untrusted)?¦eui((-[0-9a-fA-F]{2}){8})¦.+)$'. See NOTE 1. Examples: imei-012345678901234 imeisv-0123456789012345 mac-00-00-5E-00-53-00 mac-00-00-5E-00-53-00-untrusted eui-AC-DE-48-23-45-67-01-9F |
| PeiRm | string | This data type is defined in the same way as the "Pei" data type, but with the OpenAPI "nullable: true" property. |
| Supi | string | String representing a Supi that shall contain either an IMSI, a network specific identifier, a Global Cable Identifier (GCI) or a Global Line Identifier (GLI) as specified in clause 2.2A of 3GPP TS 23.003 [7]. It shall be formatted as follows: - for an IMSI "imsi-<imsi>", where <imsi> shall be formatted according to clause 2.2 of 3GPP TS 23.003 [7] that describes an IMSI. - for a network specific identifier "nai-<nai>, where <nai> shall be formatted according to clause 28.7.2 of 3GPP TS 23.003 [7] that describes an NAI. - for a GCI: "gci-<gci>", where <gci> shall be formatted according to clause 28.15.2 of 3GPP TS 23.003 [7]. - for a GLI: "gli-<gli>", where <gli> shall be formatted according to clause 28.16.2 of 3GPP TS 23.003 [7]. To enable that the value is used as part of an URI, the string shall only contain characters allowed according to the "lower-with-hyphen" naming convention defined in 3GPP TS 29.501 [2]. Pattern: '^(imsi-[0-9]{5,15}¦nai-.+¦ gci-.+¦gli-.+¦.+)$' (NOTE 1). |
| SupiRm | string | This data type is defined in the same way as the "Supi" data type, but with the OpenAPI "nullable: true" property. |
| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| AmfId | string | String representing the AMF ID composed of AMF Region ID (8 bits), AMF Set ID (10 bits) and AMF Pointer (6 bits) as specified in clause 2.10.1 of 3GPP TS 23.003 [7]. It is encoded as a string of 6 hexadecimal characters (i.e., 24 bits). Pattern: '^[A-Fa-f0-9]{6}$' |
| AmfRegionId | string | String representing the AMF Region ID (8 bits), as specified in clause 2.10.1 of 3GPP TS 23.003 [7]. It is encoded as a string of 2 hexadecimal characters (i.e. 8 bits). Pattern: '^[A-Fa-f0-9]{2}$' |
| AmfSetId | string | String representing the AMF Set ID (10 bits) as specified in clause 2.10.1 of 3GPP TS 23.003 [7]. It is encoded as a string of 3 hexadecimal characters where the first character is limited to values 0 to 3 (i.e. 10 bits). Pattern: '^[0-3][A-Fa-f0-9]{2}$' |
| RfspIndex | integer | Unsigned integer representing the "Subscriber Profile ID for RAT/Frequency Priority" as specified in 3GPP TS 36.413 [16]. Minimum = 1. Maximum = 256. |
| RfspIndexRm | integer | This data type is defined in the same way as the "RfspIndex" data type, but with the OpenAPI "nullable: true" property. |
| NfGroupId | string | Identifier of a group of NFs |
| MtcProviderInformation | string | String uniquely identifying MTC provider information. |
| CagId | string | String containing a Closed Access Group Identifier. Pattern: "^[A-Fa-f0-9]{8}$" |
| SupiOrSuci | string | String representing a SUPI or a SUCI. Pattern: "^(imsi-[0-9]{5,15}¦nai-.+¦gli-.+¦gci-.+¦suci-(0-[0-9]{3}-[0-9]{2,3}¦[1-7]-.+)-[0-9]{1,4}-(0-0-.*¦[a-fA-F1-9]-([1-9]¦[1-9][0-9]¦1[0-9]{2}¦2[0-4][0-9]¦25[0-5])-[a-fA-F0-9]+)¦.+)$" |
| Imsi | string | String representing an IMSI Pattern: ^[0-9]{5,15}$ |
| ApplicationlayerId | string | String representing an application layer ID. The format of the application layer ID parameter is same as the Application layer ID defined in clause 11.3.4 of 3GPP TS 24.554 [57]. |
| NsacSai | string | String that uniquely identifies the NSAC Service Area Identifier. Reserved value(s): "ENTIRE_PLMN", it indicates the NSACF serves the entire PLMN. |
| NOTE 1: The encoding of 3GPP defined identifiers (e.g. IMSI, NAI, IMEI, GCI, GLI) shall be prefixed with its corresponding prefix (e.g. 'imsi-','nai-', 'imei-', 'gci-', 'gli-'). NOTE 2: Whether the Dnn data type contains just the DNN Network Identifier, or the Network Identifier plus the Operator Identifier, shall be documented in each API where this data type is used. NOTE 3: NFs shall be able to receive a NF Instance Id in any UUID format. |  |  |

### 5.3.3	Enumerations

#### 5.3.3.1	Enumeration: GroupServiceId

The enumeration GroupServiceId is a part of IMSI-Group-Id (see clause 19.9 of 3GPP TS 23.003 [7]) and indicates the specific service for which the IMSI-Group-Id is used. Values greater than 1000 are reserved for home operator specific use. IMSI-Group-IDs with a Group-Service-Id in this range shall not be sent outside the HPLMN unless roaming agreements allow so.

Table 5.3.3.1-1: Enumeration GroupServiceId

| Enumeration value | Description |
| --- | --- |
| 1 | Group specific NAS level congestion control |
| 2 | Group specific Monitoring of Number of UEs present in a geographical area |
| 3 | Group specific for 5G LAN-type service |

### 5.3.4	Structured Data Types

#### 5.3.4.1	Type: Guami

Table 5.3.4.1-1: Definition of type Guami

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnIdNid | M | 1 | PLMN Identity and Network Identity |
| amfId | AmfId | M | 1 | AMF Identity |

#### 5.3.4.2	Type: NetworkId

Table 5.3.4.2-1: Definition of type NetworkId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mcc | Mcc | C | 0..1 | Mobile Country Code |
| mnc | Mnc | C | 0..1 | Mobile Network Code |
| NOTE: At least one MNC or MCC shall be included. |  |  |  |  |

#### 5.3.4.3	Type: GuamiRm

This data type is defined in the same way as the "Guami" data type, but with the OpenAPI "nullable: true" property.

## 5.4	Data Types related to 5G Network

### 5.4.1	Introduction

This clause defines common data types related to 5G Network (other than related to 5G QoS).

### 5.4.2	Simple Data Types

This clause specifies common simple data types.

Table 5.4.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| ApplicationId | string | String providing an application identifier. |
| ApplicationIdRm | string | This data type is defined in the same way as the "ApplicationId" data type, but with the OpenAPI "nullable: true" property. |
| PduSessionId | integer | Unsigned integer identifying a PDU session, within the range 0 to 255, as specified in clause 11.2.3.1b, bits 1 to 8, of 3GPP TS 24.007 [13]. If the PDU Session ID is allocated by the Core Network for UEs not supporting N1 mode, reserved range 64 to 95 is used. PDU Session ID within the reserved range is only visible in the Core Network (NOTE 1). |
| Mcc | string | Mobile Country Code part of the PLMN, comprising 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413 [11]. Pattern: '^[0-9]{3}$' |
| MccRm | string | This data type is defined in the same way as the "Mcc" data type, but with the OpenAPI "nullable: true" property. |
| Mnc | string | Mobile Network Code part of the PLMN, comprising 2 or 3 digits, as defined in clause 9.3.3.5 of 3GPP TS 38.413 [11]. (NOTE 2) Pattern: '^[0-9]{2,3}$' |
| MncRm | string | This data type is defined in the same way as the "Mnc" data type, but with the OpenAPI "nullable: true" property. |
| Tac | string | 2 or 3-octet string identifying a tracking area code as specified in clause 9.3.3.10 or 9.3.3.16 of 3GPP TS 38.413 [11], in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TAC shall appear first in the string, and the character representing the 4 least significant bit of the TAC shall appear last in the string. Examples: A legacy TAC 0x4305 shall be encoded as "4305". An extended TAC 0x63F84B shall be encoded as "63F84B" |
| TacRm | string | This data type is defined in the same way as the "Tac" data type, but with the OpenAPI "nullable: true" property. |
| EutraCellId | string | 28-bit string identifying an E-UTRA Cell Id as specified in clause 9.3.1.9 of 3GPP TS 38.413 [11], in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Cell Id shall appear first in the string, and the character representing the 4 least significant bit of the Cell Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{7}$' Example: An E-UTRA Cell Id 0x5BD6007 shall be encoded as "5BD6007". |
| EutraCellIdRm | string | This data type is defined in the same way as the "EutraCellId" data type, but with the OpenAPI "nullable: true" property. |
| NrCellId | string | 36-bit string identifying an NR Cell Id as specified in clause 9.3.1.7 of 3GPP TS 38.413 [11], in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Cell Id shall appear first in the string, and the character representing the 4 least significant bit of the Cell Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{9}$' Example: An NR Cell Id 0x225BD6007 shall be encoded as "225BD6007". |
| NrCellIdRm | string | This data type is defined in the same way as the "NrCellId" data type, but with the OpenAPI "nullable: true" property. |
| Dnai | string | DNAI (Data network access identifier), see clause 5.6.7 of 3GPP TS 23.501 [8]. |
| DnaiRm | string | This data type is defined in the same way as the "Dnai" data type, but with the OpenAPI "nullable: true" property. |
| 5GMmCause | Uinteger | This represents the 5GMM cause code values as specified in 3GPP TS 24.501 [20]. |
| AreaCodeRm | string | This data type is defined in the same way as the "AreaCode" data type, but with the OpenAPI "nullable: true" property. |
| AmfName | Fqdn | FQDN (Fully Qualified Domain Name) of the AMF as defined in clause 28.3.2.5 of 3GPP TS 23.003 [7]. |
| AreaCode | string | Values are operator specific. |
| N3IwfId | string | This represents the identifier of the N3IWF ID as specified in clause 9.3.1.57 of 3GPP TS 38.413 [11] in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N3IWF ID shall appear first in the string, and the character representing the 4 least significant bit of the N3IWF ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]+$' Example: The N3IWF Id 0x5BD6 shall be encoded as "5BD6". |
| WAgfId | string | This represents the identifier of the W-AGF ID as specified in clause 9.3.1.162 of 3GPP TS 38.413 [11] in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the W-AGF ID shall appear first in the string, and the character representing the 4 least significant bit of the W-AGF ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]+$' Example: The W-AGF Id 0x5BD6 shall be encoded as "5BD6". |
| TngfId | string | This represents the identifier of the TNGF ID as specified in clause 9.3.1.161 of 3GPP TS 38.413 [11] in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TNGF ID shall appear first in the string, and the character representing the 4 least significant bit of the TNGF ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]+$' Example: The TNGF Id 0x5BD6 shall be encoded as "5BD6". |
| NgeNbId | string | This represents the identifier of the ng-eNB ID as specified in clause 9.3.1.8 of 3GPP TS 38.413 [11]. The string shall be formatted with following pattern: Pattern: '^('MacroNGeNB-[A-Fa-f0-9]{5}¦ LMacroNGeNB-[A-Fa-f0-9]{6}¦ SMacroNGeNB-[A-Fa-f0-9]{5})$' The value of the ng-eNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, so the most significant character representing the padding 0 if required together with the 4 most significant bits of the ng-eNB ID shall appear first in the string, and the character representing the 4 least significant bit of the ng-eNB ID (to form a nibble) shall appear last in the string. Examples: " SMacroNGeNB-34B89" indicates a Short Macro NG-eNB ID with value 0x34B89. |
| Nid | string | This represents the Network Identifier, which together with a PLMN ID is used to identify an SNPN (see 3GPP TS 23.003 [7] and 3GPP TS 23.501 [8] clause 5.30.2.1). Pattern: '^[A-Fa-f0-9]{11}$' |
| NidRm | string | This data type is defined in the same way as the "Nid" data type, but with the OpenAPI "nullable: true" property. |
| NfSetId | string | NF Set Identifier (see clause 28.12 of 3GPP TS 23.003 [7]), formatted as the following string: " set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or "set<SetID>.<NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with <MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition) <MNC> encoding the Mobile Network Code part of the PLMN, comprising 3 digits. If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC. Pattern: '^[0-9]{3}$ <NFType> encoded as a value defined in Table 6.1.6.3.3-1 of 3GPP TS 29.510 [29] but with lower case characters <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit. Pattern: '^([A-Za-z0-9\-]*[A-Za-z0-9])$' Examples: "setxyz.smfset.5gc.mnc012.mcc345" "set12.pcfset.5gc.mnc012.mcc345" As specified for NF Set ID in 3GPP TS 23.003 [7], clause 28.12, NF Set ID shall be handled as case-insensitive string. |
| NfServiceSetId | string | NF Service Set Identifier (see clause 28.13 of 3GPP TS 23.003 [7]) formatted as the following string: " set<Set ID>.sn<Service Name>.nfi<NF Instance ID>.5gc.mnc<MNC>.mcc<MCC>">", or "set<SetID>.sn<ServiceName>.nfi<NFInstanceID>.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with <MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition) <MNC> encoding the Mobile Network Code part of the PLMN, comprising 3 digits. If there are only 2 significant digits in the MNC, one "0" digit shall be inserted at the left side to fill the 3 digits coding of MNC. Pattern: '^[0-9]{3}$' <NID> encoded as defined in clause 5.4.2 ("Nid" data type definition) <NFInstanceId> encoded as defined in clause 5.3.2 <ServiceName> encoded as defined in 3GPP TS 29.510 [29] <Set ID> encoded as a string of characters consisting of alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end with either an alphabetic character or a digit. Pattern: '^([A-Za-z0-9\-]*[A-Za-z0-9])$ Examples: "setxyz.snnsmf-pdusession.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345" "set2.snnpcf-smpolicycontrol.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345" As specified for NF Service Set ID in 3GPP TS 23.003 [7], clause 28.13, NF Service Set ID shall be handled as case-insensitive string. |
| PlmnAssiUeRadioCapId | Bytes | String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, encoding the "UE radio capability ID" IE as specified in clause 9.11.3.68 of 3GPP TS 24.501 [20] (starting from octet 1). |
| ManAssiUeRadioCapId | Bytes | String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, encoding the "UE radio capability ID" IE as specified in clause 9.11.3.68 of 3GPP TS 24.501 [20] (starting from octet 1). |
| TypeAllocationCode | string | Type Allocation Code (TAC) of the UE, comprising the initial eight-digit portion of the 15-digit IMEI and 16-digit IMEISV codes. See clause 6.2 of 3GPP TS 23.003 [7]. Pattern: '^[0-9]{8}$' |
| HfcNId | string | This IE represents the identifier of the HFC node Id as specified in CableLabs WR-TR-5WWC-ARCH [32]. It is provisioned by the wireline operator as part of wireline operations and may contain up to six characters. |
| HfcNIdRm | string | This data type is defined in the same way as the "HfcNId" data type, but with the OpenAPI "nullable: true" property. |
| ENbId | string | This represents the identifier of the eNB ID as specified in clause 9.2.1.37 of 3GPP TS 36.413 [16]. The string shall be formatted with following pattern: Pattern: '^('MacroeNB-[A-Fa-f0-9]{5}¦LMacroeNB-[A-Fa-f0-9]{6}¦SMacroeNB-[A-Fa-f0-9]{5}¦HomeeNB-[A-Fa-f0-9]{7})$' The value of the eNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, so the most significant character representing the padding 0 if required together with the 4 most significant bits of the eNB ID shall appear first in the string, and the character representing the 4 least significant bit of the eNB ID (to form a nibble) shall appear last in the string. Examples: "SMacroeNB-34B89" indicates a Short Macro eNB ID with value 0x34B89. |
| Gli | Bytes | Global Line Identifier uniquely identifying the line connecting the 5G-BRG or FN-BRG to the 5GS. See clause 28.16.3 of 3GPP TS 23.003 [7]. This shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the GLI value (up to 150 bytes) encoded as specified in BBF WT-470 [37]. |
| Gci | string | Global Cable Identifier uniquely identifying the connection between the 5G-CRG or FN-CRG to the 5GS. See clause 28.15.4 of 3GPP TS 23.003 [7]. This shall be encoded as a string per clause 28.15.4 of 3GPP TS 23.003 [7], and compliant with the syntax specified in clause 2.2 of IETF RFC 7542 [47] for the username part of a NAI. The GCI value is specified in CableLabs WR-TR-5WWC-ARCH [32]. |
| NsSrg | string | String representing Network Slice Simultaneous Registration Group (see clause 5.15.12 of 3GPP TS 23.501 [8]) |
| NsSrgRm | string | This data type is defined in the same way as the "NsSrg" data type, but with the OpenAPI "nullable: true" property. |
| RelayServiceCode | integer | Relay Service Code to identify a connectivity service provided by the UE-to-Network relay or the UE-to-UE relay. Integer type as defined in OpenAPI Specification [3], with value range from 0 to 16777215 (decimal). Minimum = 0. Maximum = 16777215. |
| 5GPrukId | string | Prose Remote User Key ID over Control Plane A string carrying the CP-PRUK ID of the the 5G ProSe Remote UE or the 5G ProSe End UE as specified in 3GPP TS 33.503 [50]. The CP-PRUK ID is a string in NAI format as specified in clause 28.7.11 of 3GPP TS 23.003 [7]. pattern: "^rid[0-9]{1,4}\.pid[0-9a-fA-F]+\@prose-cp\.5gc\.mnc[0-9]{2,3}\.mcc[0-9]{3}\.3gppnetwork\.org$" |
| NsagId | integer | Containing a Network Slice AS Group ID, see 3GPP TS 38.413 [11]. Values between 0 and 255 are allowed for this data type in this release. |
| NsagIdRm | integer | This data type is defined in the same way as the "NsagId" data type, but with the OpenAPI "nullable: true" property. |
| GeoSatelliteId | string | Unique identifier of a GEO satellite. See e.g. clause 5.43.2 in 3GPP TS 23.501 [8]. |
| OffloadIdentifier | string | Offload identifier uniquely identifying a VPLMN offloading policy information instance of the HPLMN (for HR-SBO) or uniquely identifying a Local Offloading Management Policy information instance of the PLMN (for I-SMF based local offloading management). For HR-SBO, it shall comprise the PLMN ID of the HPLMN providing the VPLMN offloading policy and a unique identifier of the VPLMN offloading policy instance in the HPLMN. For I-SMF based Local Offloading management, it shall comprise the PLMN ID of the PLMN providing the local offloading management policy information and a unique identifier of the Local Offloading Management Policy information in this PLMN. The PLMN ID shall be composed of three digits "mcc" followed by "-" and two or three digits "mnc" and shall match the following pattern: '[0-9]{3}-[0-9]{2,3}' The unique identifier shall match the following pattern: '[A-Fa-f0-9]{8}' It may further contain the version number (between 0 and 99) of the VPLMN or Local offloading policy instance in the HPLMN or PLMN. A VPLMN or Local Offloading Information provided by the H-SMF with a higher version number will overwrite the one with lower version number. When present, the version number shall match the following pattern: '-v[0-9]{1,2}' Pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{8}(-v[0-9]{1,2}){0,1}$' Examples (with and without a version number): "262-01-00A17C01-v3" "302-720-00A17C01" |
| OpConfiguredCapability | string | The operator configurable capability supported by the UPF. |
| SatelliteId | string | Unique identifier of a satellite. See e.g. clause 5.4.14 in 3GPP TS 23.501 [8]. |
| EnergySavingIndicator | Uinteger | Energy Saving Indicator. See clause 4.2.2.2.2 of 3GPP TS 23.502 [28] and clause 5.51.5 of 3GPP TS 23.501 [8]. |
| OverloadControlInfo | string | String representing the Overload Control Information. The string shall be encoded as the value part of the "Overload Control Information" header defined in clause 5.2.3.2.9 of 3GPP TS 29.500 [25]. EXAMPLE: "Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 75s; Overload-Reduction-Metric: 50%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8" |
| NOTE 1: For a PDN connection established via MME/S4-SGSN, the PDU Session ID value is set to 64 plus the EPS bearer ID of the default EPS bearer of the PDN connection; for a PDN connection established via ePDG, the PDU Session ID value is set to 80 plus the EPS bearer ID of the default EPS bearer of the PDN connection; for a PDN connection established via Gn-SGSN, the PDU Session ID value is set to 64 plus the NSAPI of the primary PDP Context of the PDN connection. NOTE 2: When sending a 2-digit MNC value over the SBI interface it can happen that the sender erroneously adds a leading "0". The receiver can detect such error by analysing the MCC value: If only 2-digit MNCs are allocated for the country the MCC is pointing to, then the receiver can remove the leading "0" and continue processing with the 2-digit MNC. |  |  |

### 5.4.3	Enumerations

#### 5.4.3.1	Enumeration: AccessType

Table 5.4.3.1-1: Enumeration AccessType

| Enumeration value | Description |
| --- | --- |
| "3GPP_ACCESS" | 3GPP access |
| "NON_3GPP_ACCESS" | Non-3GPP access |

#### 5.4.3.2	Enumeration: RatType

Table 5.4.3.2-1: Enumeration RatType

| Enumeration value | Description |
| --- | --- |
| "NR" | New Radio |
| "EUTRA" | (WB) Evolved Universal Terrestrial Radio Access |
| "WLAN" | Untrusted Wireless LAN (IEEE 802.11) access |
| "VIRTUAL" | Virtual (see NOTE 1) |
| "NBIOT" | NB IoT |
| "WIRELINE" | Wireline access |
| "WIRELINE_CABLE" | Wireline Cable access |
| "WIRELINE_BBF" | Wireline BBF access |
| "LTE-M" | LTE-M (see NOTE 2) |
| "NR_U" | New Radio in unlicensed bands |
| "EUTRA_U" | (WB) Evolved Universal Terrestrial Radio Access in unlicensed bands |
| "TRUSTED_N3GA" | Trusted Non-3GPP access |
| "TRUSTED_WLAN" | Trusted Wireless LAN (IEEE 802.11) access |
| "UTRA" | UMTS Terrestrial Radio Access |
| "GERA" | GSM EDGE Radio Access Network |
| "NR_LEO" | NR (LEO) satellite access type |
| "NR_MEO" | NR (MEO) satellite access type |
| "NR_GEO" | NR (GEO) satellite access type |
| "NR_OTHER_SAT" | NR (OTHERSAT) satellite access type |
| "NR_REDCAP" | NR RedCap access type (see NOTE 3) |
| "WB_E_UTRAN_LEO" | WB-E-UTRAN (LEO) satellite access type |
| "WB_E_UTRAN_MEO" | WB-E-UTRAN (MEO) satellite access type |
| "WB_E_UTRAN_GEO" | WB-E-UTRAN (GEO) satellite access type |
| "WB_E_UTRAN_OTHERSAT" | WB-E-UTRAN (OTHERSAT) satellite access type |
| "NB_IOT_LEO" | NB-IoT (LEO) satellite access type (NOTE 5) |
| "NB_IOT_MEO" | NB-IoT (MEO) satellite access type (NOTE 5) |
| "NB_IOT_GEO" | NB-IoT (GEO) satellite access type (NOTE 5) |
| "NB_IOT_OTHERSAT" | NB-IoT (OTHERSAT) satellite access type |
| "LTE_M_LEO" | LTE-M (LEO) satellite access type (NOTE 6) |
| "LTE_M_MEO" | LTE-M (MEO) satellite access type (NOTE 6) |
| "LTE_M_GEO" | LTE-M (GEO) satellite access type (NOTE 6) |
| "LTE_M_OTHERSAT" | LTE-M (OTHERSAT) satellite access type (NOTE 6) |
| "NR_EREDCAP" | NR eRedCap access type (NOTE 4) |
| "HSPA_EVOLUTION" | HSPA Evolution |
| NOTE 1: Virtual shall be used if the N3IWF does not know the access technology used for an untrusted non-3GPP access. NOTE 2: This RAT type value is used only in the Core Network; it shall be used when a Category M UE using E-UTRA has provided a Category M indication to the NG-RAN. NOTE 3: This RAT type value is used only in the Core Network; it shall be used only for an UE using NR with reduced radio capability (RedCap) provided to the NG-RAN. NOTE 4: This RAT type value is used only in the Core Network; it shall be used only for an UE using NR with enhanced reduced radio capability (eRedCap) provided to the NG-RAN. NOTE 5: This RAT type value shall not be used as list of restricted RAT when the UDM provide the mobility restriction information to AMF. NOTE 6: This RAT type value is used only in the Core Network; it shall be used only to allow the AMF to provide it to MME in the case of mobility from 5GS to EPS. |  |

#### 5.4.3.3	Enumeration: PduSessionType

The enumeration PduSessionType indicates the type of a PDU session. It shall comply with the provisions defined in table 5.4.3.3-1.

Table 5.4.3.3-1: Enumeration PduSessionType

| Enumeration value | Description |
| --- | --- |
| "IPV4" | IPv4 |
| "IPV6" | IPv6 |
| "IPV4V6" | IPv4v6 (see clause 5.8.2.2.1 of 3GPP TS 23.501 [8]) |
| "UNSTRUCTURED" | Unstructured |
| "ETHERNET" | Ethernet |

#### 5.4.3.4	Enumeration: UpIntegrity

The enumeration UpIntegrity indicates whether UP integrity protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.4-1.

Table 5.4.3.4-1: Enumeration UpIntegrity

| Enumeration value | Description |
| --- | --- |
| "REQUIRED" | UP integrity protection shall apply for all the traffic on the PDU Session. |
| "PREFERRED" | UP integrity protection should apply for all the traffic on the PDU Session. |
| "NOT_NEEDED" | UP integrity protection shall not apply on the PDU Session. |

#### 5.4.3.5	Enumeration: UpConfidentiality

The enumeration UpConfidentiality indicates whether UP confidentiality protection is required, preferred or not needed for all the traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.5-1.

Table 5.4.3.5-1: Enumeration UpConfidentiality

| Enumeration value | Description |
| --- | --- |
| "REQUIRED" | UP confidentiality protection shall apply for all the traffic on the PDU Session. |
| "PREFERRED" | UP confidentiality protection should apply for all the traffic on the PDU Session. |
| "NOT_NEEDED" | UP confidentiality protection shall not apply on the PDU Session. |

#### 5.4.3.6	Enumeration: SscMode

The enumeration SscMode represents the service and session continuity mode.

Table 5.4.3.6-1: Enumeration SscMode

| Enumeration value | Description |
| --- | --- |
| "SSC_MODE_1" | see 3GPP TS 23.501 [8] |
| "SSC_MODE_2" | see 3GPP TS 23.501 [8] |
| "SSC_MODE_3" | see 3GPP TS 23.501 [8] |

#### 5.4.3.7	Enumeration: DnaiChangeType

The enumeration DnaiChangeType represents the type of a DNAI change. A NF service consumer may subscribe to "EARLY", "LATE" or "EARLY_LATE" types of DNAI change. The types of observed DNAI change the SMF may notify are "EARLY" or "LATE". The DnaiChangeType data type shall comply with the provisions defined in table 5.4.3.7-1.

Table 5.4.3.7-1: Enumeration DnaiChangeType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EARLY | Early notification of UP path reconfiguration. |  |
| EARLY_LATE | Early and late notification of UP path reconfiguration. This value shall only be present in the subscription to the DNAI change event. |  |
| LATE | Late notification of UP path reconfiguration. |  |

#### 5.4.3.8	Enumeration: RestrictionType

Table 5.4.3.8-1: Enumeration RestrictionType

| Enumeration value | Description |
| --- | --- |
| "ALLOWED_AREAS" | This value indicates that areas are allowed. |
| "NOT_ALLOWED_AREAS" | This value indicates that areas are not allowed. |

#### 5.4.3.9	Enumeration: CoreNetworkType

Table 5.4.3.9-1: Enumeration CoreNetworkType

| Enumeration value | Description |
| --- | --- |
| "5GC" | 5G Core |
| "EPC" | Evolved Packet Core |

#### 5.4.3.10	Enumeration: AccessTypeRm

This enumeration is defined in the same way as the "AccessType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.11	Enumeration: RatTypeRm

This enumeration is defined in the same way as the "RatType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.12	Enumeration: PduSessionTypeRm

This enumeration is defined in the same way as the "PduSessionType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.13	Enumeration: UpIntegrityRm

This enumeration is defined in the same way as the "UpIntegrity" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.14	Enumeration: UpConfidentialityRm

This enumeration is defined in the same way as the "UpConfidentiality" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.15	Enumeration: SscModeRm

This data type is defined in the same way as the "SscMode" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.17	Enumeration: DnaiChangeTypeRm

This data type is defined in the same way as the "DnaiChangeType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.18	Enumeration: RestrictionTypeRm

This data type is defined in the same way as the "RestrictionType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.19	Enumeration: CoreNetworkTypeRm

This data type is defined in the same way as the "CoreNetworkType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.20	Enumeration: PresenceState

Table 5.4.3.20-1: Enumeration PresenceState

| Enumeration value | Description |
| --- | --- |
| "IN_AREA" | Indicates that the UE is inside or enters the presence reporting area. |
| "OUT_OF_AREA" | Indicates that the UE is outside or leaves the presence reporting area. |
| "UNKNOWN" | Indicates it is unknown whether the UE is in the presence reporting area or not. |
| "INACTIVE" | Indicates that the presence reporting area is inactive in the serving node. |

#### 5.4.3.21	Enumeration: StationaryIndication

Table 5.4.3.21-1: Enumeration StationaryIndication

| Enumeration value | Description |
| --- | --- |
| "STATIONARY" | Identifies the UE is stationary |
| "MOBILE" | Identifies the UE is mobile |

#### 5.4.3.22	Enumeration: StationaryIndicationRm

This enumeration is defined in the same way as the "StationaryIndication" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.23	Enumeration: ScheduledCommunicationType

Table 5.4.3.23-1: Enumeration ScheduledCommunicationType

| Enumeration value | Description |
| --- | --- |
| "DOWNLINK_ONLY" | Downlink only |
| "UPLINK_ONLY" | Uplink only |
| "BIDIRECTIONAL" | Bi-directional |

#### 5.4.3.24	Enumeration: ScheduledCommunicationTypeRm

This enumeration is defined in the same way as the "ScheduledCommunicationType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.25	Enumeration: TrafficProfile

Table 5.4.3.25-1: Enumeration TrafficProfile

| Enumeration value | Description |
| --- | --- |
| "SINGLE_TRANS_UL" | Uplink single packet transmission. |
| "SINGLE_TRANS_DL" | Downlink single packet transmission. |
| "DUAL_TRANS_UL_FIRST" | Dual packet transmission, firstly uplink packet transmission with subsequent downlink packet transmission. |
| "DUAL_TRANS_DL_FIRST" | Dual packet transmission, firstly downlink packet transmission with subsequent uplink packet transmission. |
| "MULTI_TRANS" | Multiple packet transmission. |

#### 5.4.3.26	Enumeration: TrafficProfileRm

This enumeration is defined in the same way as the "TrafficProfile" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.27	Enumeration: LcsServiceAuth

Table 5.4.3.27-1: Enumeration LcsServiceAuth

| Enumeration value | Description |
| --- | --- |
| "LOCATION_ALLOWED_WITH_NOTIFICATION" | Location allowed with notification |
| "LOCATION_ALLOWED_WITHOUT_NOTIFICATION" | Location allowed without notification |
| "LOCATION_ALLOWED_WITHOUT_RESPONSE" | Location with notification and privacy verification; location allowed if no response |
| "LOCATION_RESTRICTED_WITHOUT_RESPONSE" | Location with notification and privacy verification; location restricted if no response |
| "NOTIFICATION_ONLY" | Notification only |
| "NOTIFICATION_AND_VERIFICATION_ONLY" | Notification and privacy verification only |

#### 5.4.3.28	Enumeration: UeAuth

Table 5.4.3.28-1: Enumeration UeAuth

| Enumeration value | Description |
| --- | --- |
| "AUTHORIZED" | Indicates that the UE is authorized. |
| "NOT_AUTHORIZED" | Indicates that the UE is not authorized. |

#### 5.4.3.29	Enumeration: DlDataDeliveryStatus

Table 5.4.3.29-1: Enumeration DlDataDeliveryStatus

| Enumeration value | Description |
| --- | --- |
| "BUFFERED" | The first downlink data is buffered with extended buffering matching the source of the downlink traffic. |
| "TRANSMITTED" | The first downlink data matching the source of the downlink traffic is transmitted after previous buffering or discarding of corresponding packet(s) because the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE. |
| "DISCARDED" | The first downlink data matching the source of the downlink traffic is discarded because the Extended Buffering time, as determined by the SMF, expires or the amount of downlink data to be buffered is exceeded. |

#### 5.4.3.30	Enumeration: DlDataDeliveryStatusRm

This enumeration is defined in the same way as the "DlDataDeliveryStatus" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.31	Void

#### 5.4.3.32	Enumeration: AuthStatus

Table 5.4.3.32-1: Enumeration AuthStatus

| Enumeration value | Description |
| --- | --- |
| "EAP_SUCCESS" | The NSSAA status is EAP-Success. |
| "EAP_FAILURE" | The NSSAA status is EAP-Failure. |
| "PENDING" | The NSSAA status is Pending, i.e. the NSSAA procedure is ongoing. |

#### 5.4.3.33	Enumeration: LineType

Table 5.4.3.33-1: Enumeration LineType

| Enumeration value | Description |
| --- | --- |
| "DSL" | DSL line |
| "PON" | PON line |

#### 5.4.3.34	Enumeration: LineTypeRm

This enumeration is defined in the same way as the "LineType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.4.3.35	Void

#### 5.4.3.36	Void

#### 5.4.3.37	Enumeration: NotificationFlag

Table 5.4.3.37-1: Enumeration NotificationFlag

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVATE | The event notification is activated. |  |
| DEACTIVATE | The event notification is deactivated and shall be muted. The available event(s) shall be stored. |  |
| RETRIEVAL | The event notification shall be sent to the NF service consumer(s), after that, is muted again. (See NOTE) |  |
| NOTE: The value "RETRIEVAL" shall not be provided during the creating subscription procedure. |  |  |

#### 5.4.3.38	Enumeration: TransportProtocol

Table 5.4.3.38-1: Enumeration TransportProtocol

| Enumeration value | Description |
| --- | --- |
| "UDP" | User Datagram Protocol |
| "TCP" | Transmission Control Protocol |

#### 5.4.3.39	Enumeration: SatelliteBackhaulCategory

Table 5.4.3.39-1: Enumeration SatelliteBackhaulCategory

| Enumeration value | Description |
| --- | --- |
| "GEO" | Indicates Geostationary satellite backhaul category. |
| "MEO" | Indicates Medium Earth Orbit satellite backhaul category. |
| "LEO" | Indicates Low Earth Orbit satellite backhaul category. |
| "OTHER_SAT" | Indicates other satellite backhaul category. |
| "DYNAMIC_GEO" | Indicates dynamic Geostationary satellite backhaul category. |
| "DYNAMIC_MEO" | Indicates dynamic Medium Earth Orbit satellite backhaul category. |
| "DYNAMIC_LEO" | Indicates dynamic Low Earth Orbit satellite backhaul category. |
| "DYNAMIC_OTHER_SAT" | Indicates dynamic other satellite backhaul category. |
| "NON_SATELLITE" | Indicates non satellite backhaul. (NOTE) |
| NOTE: This value indicates that there is no longer any satellite backhaul towards the 5G AN currently serving the UE. |  |

#### 5.4.3.40	Enumeration: SatelliteBackhaulCategoryRm

This data type is defined in the same way as the "SatelliteBackhaulCategory" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.3.41	Enumeration: BufferedNotificationsAction

Table 5.4.3.41-1: Enumeration BufferedNotificationsAction

| Enumeration value | Description |
| --- | --- |
| "SEND_ALL" | The NF Service Producer should send all the reports of the stored events to the NF service consumer. |
| "DISCARD_ALL" | The NF Service Producer should discard all the reports of the stored events for the NF service consumer. |
| "DROP_OLD" | The NF Service Producer should drop the old reports of the stored events for the NF service consumer when new event reports need to be stored. |

#### 5.4.3.42	Enumeration: SubscriptionAction

Table 5.4.3.42-1: Enumeration SubscriptionAction

| Enumeration value | Description |
| --- | --- |
| "CLOSE" | The subscription to the event notification should be terminated if an exception occurs at the NF Service Producer. |
| "CONTINUE_WITH_MUTING" | The subscription to the event notification should be continued with muting if an exception occurs at the NF Service Producer. |
| "CONTINUE_WITHOUT_MUTING" | The subscription to the event notification should be continued without muting if an exception occurs at the NF Service Producer. |

#### 5.4.3.43	Enumeration: SnssaiStatus

Table 5.4.3.43-1: Enumeration SnssaiStatus

| Enumeration value | Description |
| --- | --- |
| "AVAILABLE" | This value is used when the S-NSSAI becomes available. |
| "UNAVAILABLE" | This value is used when the S-NSSAI becomes unavailable. |

#### 5.4.3.44	Enumeration: TerminationIndication

Table 5.4.3.44-1: Enumeration TerminationIndication

| Enumeration value | Description |
| --- | --- |
| "NEW_UES_TERMINATION" | It indicates that Network Slice Replacement is terminated for new UEs. |
| "ALL_UES_TERMINATION" | It indicates that Network Slice Replacement is terminated for all UEs and PDU sessions shall move back from the alternative S-NSSAI to the S-NSSAI. |

#### 5.4.3.45	Enumeration: DelayMeasurementProtocol

Table 5.4.3.45-1: Enumeration DelayMeasurementProtocol

| Enumeration value | Description |
| --- | --- |
| "TWAMP" | A Two-Way Active Measurement Protocol (see IETF RFC 5357 [69]) |
| "OWAMP" | A One-way Active Measurement Protocol (see IETF RFC 4656 [70] |
| "STAMP" | Simple Two-Way Active Measurement Protocol (see IETF RFC 8762 [71]) |
| "OTHER" | Measurement protocol for which no specific value exists in this enumeration |

#### 5.4.3.46	Enumeration: CagProvisionOperation

Table 5.4.3.46-1: Enumeration CagProvisionOperation

| Enumeration value | Description |
| --- | --- |
| "ADD" | To add/append the indicated CAG ID list to the previously configured CAG ID list (if any). |
| "REPLACE" | To replace the previously configured CAG ID list (if any) with the newly provisioned CAG ID list. |
| "REMOVE" | To remove the indicated CAG ID list from the previously configured CAG ID list. |

### 5.4.4	Structured Data Types

#### 5.4.4.1	Type: SubscribedDefaultQos

Table 5.4.4.1-1: Definition of type SubscribedDefaultQos

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Default 5G QoS identifier see 3GPP TS 23.501 [8] clause 5.7.2.7. |
| arp | Arp | M | 1 | Default Allocation and Retention Priority see 3GPP TS23.501 [8] clause 5.7.2.7. |
| priorityLevel | 5QiPriorityLevel | O | 0..1 | Defines the 5QI Priority Level. When present, it contains the 5QI Priority Level value that overrides the standardized or pre-configured value as described in 3GPP TS 23.501 [8]. |

#### 5.4.4.2	Type: Snssai

Table 5.4.4.2-1: Definition of type Snssai

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sst | integer | M | 1 | Integer, within the range 0 to 255, representing the Slice/Service Type. It indicates the expected Network Slice behaviour in terms of features and services. Values 0 to 127 correspond to the standardized SST range. Values 128 to 255 correspond to the Operator-specific range. See clause 28.4.2 of 3GPP TS 23.003 [7]. Standardized values are defined in clause 5.15.2.2 of 3GPP TS 23.501 [8]. |
| sd | string | O | 0..1 | 3-octet string, representing the Slice Differentiator, in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the SD shall appear first in the string, and the character representing the 4 least significant bit of the SD shall appear last in the string. This is an optional parameter that complements the Slice/Service type(s) to allow to differentiate amongst multiple Network Slices of the same Slice/Service type. This IE shall be absent if no SD value is associated with the SST. Pattern: '^[A-Fa-f0-9]{6}$' |

When Snssai needs to be converted to string (e.g. when used in maps as key), the string shall be composed of one to three digits "sst" optionally followed by "-" and 6 hexadecimal digits "sd", and shall match the following pattern:

^([0-9]|[1-9][0-9]|1[0-9][0-9]|2([0-4][0-9]|5[0-5]))(-[A-Fa-f0-9]{6})?$

Example 1:	"255-19CDE0"

Example 2:	"29"

#### 5.4.4.3	Type: PlmnId

Table 5.4.4.3-1: Definition of type PlmnId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mcc | Mcc | M | 1 | Mobile Country Code |
| mnc | Mnc | M | 1 | Mobile Network Code |

When PlmnId needs to be converted to string (e.g. when used in maps as key), the string shall be composed of three digits "mcc" followed by "-" and two or three digits "mnc", and shall match the following pattern:

^[0-9]{3}-[0-9]{2,3}$

Example 1:	"262-01"

Example 2:	"302-720"

#### 5.4.4.4	Type: Tai

Table 5.4.4.4-1: Definition of type Tai

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| tac | Tac | M | 1 | Tracking Area Code |
| nid | Nid | O | 0..1 | Network Identifier, shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the TA belongs to. |

NOTE:	The "nid" attribute is used to convey the Network Identifier (NID) of the SNPN as part of the "Tai" JSON object data type definition; this is a protocol aspect that does not imply any change on the system-wide definition of the TAI, as described in 3GPP 23.003 [7].

#### 5.4.4.5	Type: Ecgi

Table 5.4.4.5-1: Definition of type Ecgi

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| eutraCellId | EutraCellId | M | 1 | E-UTRA Cell Identity |
| nid | Nid | O | 0..1 | Network Identifier |

NOTE:	The "nid" attribute is used to convey the Network Identifier (NID) of the SNPN as part of the "Ecgi" JSON object data type definition; this is a protocol aspect that does not imply any change on the system-wide definition of the ECGI, as described in 3GPP 23.003 [7].

#### 5.4.4.6	Type: Ncgi

Table 5.4.4.6-1: Definition of type Ncgi

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| nrCellId | NrCellId | M | 1 | NR Cell Identity |
| nid | Nid | C | 0..1 | Network Identifier, shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the NR cell belongs to. |

NOTE:	The "nid" attribute is used to convey the Network Identifier (NID) of the SNPN as part of the "Ncgi" JSON object data type definition; this is a protocol aspect that does not imply any change on the system-wide definition of the NCGI, as described in 3GPP 23.003 [7].

#### 5.4.4.7	Type: UserLocation

Table 5.4.4.7-1: Definition of type UserLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eutraLocation | EutraLocation | C | 0..1 | E-UTRA user location (see NOTE). |
| nrLocation | NrLocation | C | 0..1 | NR user location (see NOTE). |
| n3gaLocation | N3gaLocation | C | 0..1 | Non-3GPP access user location (see NOTE). |
| utraLocation | UtraLocation | C | 0..1 | UTRAN access user location (see NOTE). |
| geraLocation | GeraLocation | C | 0..1 | GERAN access user location (see NOTE). |
| NOTE: At least one of eutraLocation, nrLocation, n3gaLocation, utraLocation and geraLocation shall be present. Several of them may be present. |  |  |  |  |

#### 5.4.4.8	Type: EutraLocation

Table 5.4.4.8-1: Definition of type EutraLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tai | Tai | M | 1 | Tracking Area Identity. The TAC of the TAI shall be set to one reserved value (e.g. 0x0000, see clause 19.4.2.3 of 3GPP TS 23.003 [7]) if the TAI information is not available. |
| ignoreTai | boolean | O | 0..1 | This flag when present shall indicate that the Tai shall be ignored. When present, it shall be set as follows: - true: tai shall be ignored. - false (default): tai shall not be ignored. |
| ecgi | Ecgi | M | 1 | E-UTRA Cell Identity |
| ignoreEcgi | boolean | O | 0..1 | This flag when present shall indicate that the Ecgi shall be ignored. When present, it shall be set as follows: - true: ecgi shall be ignored. - false (default): ecgi shall not be ignored. |
| ageOfLocationInformation | integer | O | 0 1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the eNB when the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [21] clause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [23] clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24] clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| globalNgenbId | GlobalRanNodeId | O | 0..1 | It indicates the global identity of the ng-eNodeB in which the UE is currently located. See 3GPP TS 38.413 [11] clause 9.3.1.8. |
| globalENbId | GlobalRanNodeId | O | 0..1 | It indicates the global identity of the eNodeB in which the UE is currently located. See 3GPP TS 36.413 [16] clause 9.2.1.37. |
| NOTE: Either the "globalNgenbId" attribute or the "globalENbId" attribute shall be included in the "EutraLocation" data type. |  |  |  |  |

#### 5.4.4.9	Type: NrLocation

Table 5.4.4.9-1: Definition of type NrLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tai | Tai | M | 1 | Tracking Area Identity When NTN TAI Information with UE Location Derived TAC was received from satellite NG-RAN, this IE shall carry the value of the UE Location Derived TAC; if NTN TAI Information without UE Location Derived TAC was received, this IE shall carry the value of the first TAC that is not restricted to the UE, in the list of TAC(s) broadcast in the cell. |
| ncgi | Ncgi | M | 1 | NR Cell Identity |
| ignoreNcgi | boolean | O | 0..1 | This flag when present shall indicate that the Ncgi shall be ignored. When present, it shall be set as follows: - true: ncgi shall be ignored. - false (default): ncgi shall not be ignored. |
| ageOfLocationInformation | integer | O | 0 1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful NG-RAN location reporting procedure with the gNB when the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [21] clause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [23] clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24] clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| globalGnbId | GlobalRanNodeId | O | 0..1 | It indicates the global identity of the gNodeB in which the UE is currently located. See 3GPP TS 38.413 [11] clause 9.3.1.6. |
| ntnTaiInfo | NtnTaiInfo | O | 0..1 | Contains NR NTN TAI Information. When this IE is present and supported, the tai attribute shall be ignored by the receiver, see clause 9.3.3.53 of 3GPP TS 38.413 [11]. |

#### 5.4.4.10	Type: N3gaLocation

Table 5.4.4.10-1: Definition of type N3gaLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n3gppTai | Tai | C | 0..1 | This IE shall be present over the 3GPP PLMN internal interfaces, but it shall not be present over the N5 interface, nor on the N7/N40 interface in EPC interworking scenario. When present, it shall contain the TAI reported by the N3IWF, TNGF, TWIF or W-AGF for the non-3GPP access. |
| n3IwfId | string | C | 0..1 | This IE shall contain the N3IWF identifier received over NGAP and shall be encoded as a string of hexadecimal characters. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N3IWF ID shall appear first in the string, and the character representing the 4 least significant bit of the N3IWF ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]+$' Example: The N3IWF Id 0x5BD6 shall be encoded as "5BD6". It shall be present over the 3GPP PLMN internal interfaces if the UE is accessing the 5GC via an untrusted non-3GPP access, but shall not be present over the N5 interface, nor on the N7/N40 interface in EPC interworking scenario. |
| ueIpv4Addr | Ipv4Addr | C | 0..1 | UE/N5CW device local IPv4 address (used to reach the N3IWF, TNGF or TWIF, or ePDG). The ueIpv4Addr or the ueIpv6Addr shall be present if the UE is accessing the 5GC via a trusted or untrusted non-3GPP access and the information is available. |
| ueIpv6Addr | Ipv6Addr | C | 0..1 | UE/N5CW device local IPv6 address (used to reach the N3IWF, TNGF or TWIF, or ePDG). The ueIpv4Addr or the ueIpv6Addr shall be present if the UE is accessing the 5GC via a trusted or untrusted non-3GPP access and the information is available. |
| portNumber | Uinteger | C | 0..1 | UDP or TCP source port number. It shall be present if the UE/N5CW is accessing the 5GC via a trusted or untrusted non-3GPP access and NAT is detected and the information is available. |
| protocol | TransportProtocol | O | 0..1 | This IE may be present if portNumber is present. When present, this IE shall indicate the transport protocol used by the UE to access the core network via a trusted or untrusted non-3GPP access and NAT is detected. The absence of this IE indicates that the transport protocol used by the UE to access the core network via a trusted or untrusted non-3GPP access is not specified, i.e. could be UDP or TCP. |
| tnapId | TnapId | C | 0..1 | This IE shall contain the TNAP Identifier, see clause 5.6.2 of 3GPP TS 23.501 [8]. |
| twapId | TwapId | C | 0..1 | In the scenario of accessing 5GC from N5CW device, this IE shall contain the TWAP Identifier, see clause 4.2.8.5.3 of 3GPP TS 23.501 [8]. In the scenario of interworking between ePDG/EPC and 5GS, this IE shall contain the WLAN location information, see clause 4.5.7.2.8 of 3GPP TS 23.402 [48]. |
| hfcNodeId | HfcNodeId | C | 0..1 | This IE shall contain the HFC Node Identifier received over NGAP. It shall be present for a 5G-CRG/FN-CRG accessing the 5GC via wireline access network, and for a AUN3 device connected behind the 5G-CRG (see clause 7.2.8.1 of 3GPP TS 23.316 [30]). |
| gli | Gli | C | 0..1 | This IE shall contain the Global Line Identifier. It shall be present for a 5G-BRG/FN-BRG accessing the 5GC via wireline access network, and for a AUN3 device connected behind a 5G-BRG. |
| w5gbanLineType | LineType | O | 0..1 | This IE may be present for a 5G-BRG/FN-BRG accessing the 5GC via wireline access network. When present, it shall indicate the type of the wireline (DSL or PON). |
| gci | Gci | C | 0..1 | This IE shall contain the Global Cable Identifier. It shall be present for the N5GC device accessing the 5GC via wireline access network(see clause 4.10a of 3GPP TS 23.316 [30]), and for a AUN3 device connected behind the 5G-CRG (see clause 7.2.8.1 of 3GPP TS 23.316 [30]). |

#### 5.4.4.11	Type: UpSecurity

Table 5.4.4.11-1: Definition of type UpSecurity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| upIntegr | UpIntegrity | M | 1 | This IE shall indicate whether UP integrity protection is required, preferred or not needed for all the traffic on the PDU Session. |
| upConfid | UpConfidentiality | M | 1 | This IE shall indicate whether UP confidentiality protection is required, preferred or not needed for all the traffic on the PDU Session. |

#### 5.4.4.12	Type: NgApCause

Table 5.4.4.12-1: Definition of type NgApCause

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| group | Uinteger | M | 1 | This IE shall indicate the group of the NGAP cause. The value of this IE shall equal to the ASN.1 value of the specified NGAP cause group. NGAP supports following cause groups defined as separate enumerations, as specified in clause 9.4.5 of 3GPP TS 38.413 [11], with following values: 0 – radioNetwork 1 – transport 2 – nas 3 – protocol 4 – misc |
| value | Uinteger | M | 1 | This IE shall carry the NG AP cause value in specific cause group identified by the "group" attribute, as specified in clause 9.4.5 of 3GPP TS 38.413 [11]. |

#### 5.4.4.13	Type: BackupAmfInfo

Table 5.4.4.13-1: Definition of type BackupAmfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| backupAmf | AmfName | M | 1 | This IE shall contain the AMF name of the backup AMF that can serve the specific GUAMI(s) supported by the primary AMF (see clause 5.21.2.3 of 3GPP TS 23.501 [8]). |
| guamiList | array(Guami) | C | 1..N | If present, this IE shall contain the list of GUAMI(s) (supported by the primary AMF) which the backup AMF can serve. If this IE is absent, it indicates that the backup AMF can serve all the GUAMI(s) supported by the primary AMF. |

#### 5.4.4.14	Type: RefToBinaryData

Table 5.4.4.14-1: Definition of type RefToBinaryData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| contentId | string | M | 1 | This IE shall contain the value of the Content-ID header of the referenced binary body part. |

#### 5.4.4.15	Type RouteToLocation

Table 5.4.4.15-1: Definition of type RouteToLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dnai | Dnai | M | 1 | Identifies the location of the application. |
| routeInfo | RouteInformation | C | 0..1 | Includes the traffic routing information. |
| routeProfId | string | C | 0..1 | Identifies the routing profile Id. |
| NOTE: At least one of the "routeInfo" attribute and the "routeProfId" attribute shall be included in the "RouteToLocation" data type.. |  |  |  |  |

#### 5.4.4.16	Type RouteInformation

Table 5.4.4.16-1: Definition of type RouteInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | Ipv4address of the tunnel end point in the data network. |
| ipv6Addr | Ipv6Addr | C | 0..1 | Ipv6 address of the tunnel end point in the data network. |
| portNumber | Uinteger | M | 1 | UDP port number of the tunnel end point in the data network. |
| NOTE: At least one of the "ipv4Addr" attribute and the "ipv6Addr" attribute shall be included in the "RouteInformation" data type. |  |  |  |  |

#### 5.4.4.17	Type: Area

Table 5.4.4.17-1: Definition of type Area

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tacs | array(Tac) | C | 1..N | List of TACs; shall be present if and only if areaCode is absent. |
| areaCode | AreaCode | C | 0..1 | Area Code; shall be present if and only if tacs is absent. |

#### 5.4.4.18	Type: ServiceAreaRestriction

Table 5.4.4.18-1: Definition of type ServiceAreaRestriction

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| restrictionType | RestrictionType | C | 0..1 | string "ALLOWED_AREAS" or "NOT_ALLOWED_AREAS" shall be present if and only if the areas attribute is present |
| areas | array(Area) | O | 0..N (NOTE) | A list of Areas. These areas are: - allowed areas if RestrictionType is "ALLOWED_AREAS" - not allowed areas if RestrictionType is "NOT_ALLOWED_AREAS" |
| maxNumOfTAs | Uinteger | C | 0..1 | Maximum number of allowed tracking areas for use when restrictionType indicates "ALLOWED_AREAS". This attribute shall be absent when attribute "restrictionType" takes the value "NOT_ALLOWED_AREAS". |
| maxNumOfTAsForNotAllowedAreas | Uinteger | C | 0..1 | Maximum number of allowed tracking areas for use when restrictionType indicates "NOT_ALLOWED_AREAS". This attribute shall be absent when attribute "restrictionType" takes the value "ALLOWED_AREAS". |
| NOTE: The empty array is used when service is allowed/restricted nowhere. |  |  |  |  |

#### 5.4.4.19	Type: PlmnIdRm

This data type is defined in the same way as the "PlmnId" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.20	Type: TaiRm

This data type is defined in the same way as the "Tai" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.21	Type: EcgiRm

This data type is defined in the same way as the "Ecgi" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.22	Type: NcgiRm

This data type is defined in the same way as the "Ncgi" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.23	Type: EutraLocationRm

This data type is defined in the same way as the "EutraLocation" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.24	Type: NrLocationRm

This data type is defined in the same way as the "NrLocation" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.25	Type: UpSecurityRm

This data type is defined in the same way as the "UpSecurity" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.26	Type: RefToBinaryDataRm

This data type is defined in the same way as the "RefToBinaryData" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.27	Type: PresenceInfo

Table 5.4.4.27-1: Definition of type PresenceInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| praId | string | C | 0..1 | Represents an identifier of the Presence Reporting Area (see clause 28.10 of 3GPP TS 23.003 [7]). This IE shall be present if the Area of Interest subscribed or reported is a Presence Reporting Area or a Set of Core Network predefined Presence Reporting Areas. When present, it shall be encoded as a string representing an integer in the following ranges: 0 to 8 388 607 for UE-dedicated PRA 8 388 608 to 16 777 215 for Core Network predefined PRA. Examples: PRA ID 123 is encoded as "123" PRA ID 11 238 660 is encoded as "11238660" |
| additionalPraId | string | C | 0..1 | This IE may be present if the praId IE is present and if it contains a PRA identifier referring to a set of Core Network predefined Presence Reporting Areas. When present, this IE shall contain a PRA Identifier of an individual PRA within the Set of Core Network predefined Presence Reporting Areas indicated by the praId IE. |
| presenceState | PresenceState | C | 0..1 | Indicates whether the UE is inside or outside of the area of interest (e.g presence reporting area or the LADN area), or if the presence reporting area is inactive in the serving node. (NOTE) |
| trackingAreaList | array(Tai) | C | 1..N | Represents the list of tracking areas that constitutes the area. This IE shall be present if the subscription or the event report is for tracking UE presence in the tracking areas. For non 3GPP access the TAI shall be the N3GPP TAI. |
| ecgiList | array(Ecgi) | C | 1..N | Represents the list of EUTRAN cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of EUTRAN cell Ids. |
| ncgiList | array(Ncgi) | C | 1..N | Represents the list of NR cell Ids that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NR cell Ids. |
| globalRanNodeIdList | array(GlobalRanNodeId) | C | 1..N | Represents the list of NG RAN node identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of NG RAN node identifiers. |
| globaleNbIdList | array(GlobalRanNodeId) | C | 1..N | Represents the list of eNodeB identifiers that constitutes the area. This IE shall be present if the Area of Interest subscribed is a list of eNodeB identifiers. |
| NOTE: If the additionalPraId IE is present, this IE shall state the presence information of the UE for the individual PRA identified by the additionalPraId IE; If the additionalPraId IE is not present, this IE shall state the presence information of the UE for the PRA identified by the praId IE. |  |  |  |  |

#### 5.4.4.28	Type: GlobalRanNodeId

Table 5.4.4.28-1: Definition of type GlobalRanNodeId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | Indicates the identity of the PLMN that the RAN node belongs to. |
| n3IwfId | N3IwfId | C | 0..1 | This IE shall be included if the AN node represents a N3IWF. When present, this IE shall contain the identifier of the N3IWF. (NOTE 1). |
| gNbId | GNbId | C | 0..1 | This IE shall be included if the RAN Node Id represents a gNB. When present, this IE shall contain the identifier of the gNB. (NOTE 1). |
| ngeNbId | NgeNbId | C | 0..1 | This IE shall be included if the RAN Node Id represents a NG-eNB. When present, this IE shall contain the identifier of an NG-eNB. (NOTE 1). |
| wagfId | WAgfId | C | 0..1 | This IE shall be included if the AN node represents a W-AGF. When present, this IE shall contain the identifier of the W-AGF. (NOTE 1). |
| tngfId | TngfId | C | 0..1 | This IE shall be included if the AN node represents a TNGF. When present, this IE shall contain the identifier of the TNGF. (NOTE 1). |
| nid | Nid | O | 0..1 | Network Identifier shall be present in case of SNPN, PlmnId together with Nid indicates the identity of the SNPN to which the RanNode belongs to. |
| eNbId | ENbId | C | 0..1 | This IE shall be included if the RAN Node Id represents an eNB. When present, this IE shall contain the identifier of an eNB. (NOTE 1, NOTE 2). |
| NOTE 1: One of the six attributes n3IwfId, gNbIdm, ngeNbId, wagfId, tngfId, eNbId shall be present. NOTE 2: For UEs with 5GS subscription but without 5G NAS support, eNbId is used on N7 instead of n3IwfId, gNbIdm, ngeNbId. |  |  |  |  |

#### 5.4.4.29	Type: GNbId

Table 5.4.4.29-1: Definition of type GNbId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bitLength | integer | M | 1 | Unsigned integer representing the bit length of the gNB ID as defined in clause 9.3.1.6 of 3GPP TS 38.413 [11], within the range 22 to 32 |
| gNBValue | string | M | 1 | This represents the identifier of the gNB. The string shall be formatted with following pattern: '^[A-Fa-f0-9]{6,8}$' The value of the gNB ID shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, the most significant character representing the padding 0 if required together with the 4 most significant bits of the gNB ID shall appear first in the string, and the character representing the 4 least significant bit of the gNB ID shall appear last in the string. Examples: A 30 bit value "382A3F47" indicates a gNB ID with value 0x382A3F47 A 22 bit value "2A3F47" indicates a gNB ID with value 0x2A3F47 |

#### 5.4.4.30	Type: PresenceInfoRm

This data type is defined in the same way as the "PresenceInfo" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.31	Void

#### 5.4.4.32	Type: AtsssCapability

Table 5.4.4.32-1: Definition of type AtsssCapability

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| atsssLL | boolean | O | 0..1 | Indicates the support of Access Traffic Steering, Switching and Splitting procedures using the ATSSS-LL steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501 [8]). true: Supported false (default): Not Supported |
| mptcp | boolean | O | 0..1 | Indicates the support of Access Traffic Steering, Switching and Splitting procedures using the MPTCP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501 [8]). true: Supported false (default): Not Supported |
| mpquic | boolean | O | 0..1 | Indicates the support of Access Traffic Steering, Switching and Splitting procedures using the MPQUIC-UDP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501 [8]). true: Supported false (default): Not Supported |
| rttWithoutPmf | boolean | O | 0..1 | This IE is only used by the UPF to indicate whether the UPF supports RTT measurement without PMF (see clauses 5.32.2, 6.3.3.3 of 3GPP TS 23.501 [8]). If this attribute is present and set to true, the mptcp attribute shall also be present and set to true. true: Supported false (default): Not Supported. |
| mpquicIp | boolean | O | 0..1 | Indicates the support of Access Traffic Steering, Switching and Splitting procedures using the MPQUIC-IP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501 [8]). true: Supported false (default): Not Supported |
| mpquicE | boolean | O | 0..1 | Indicates the support of Access Traffic Steering, Switching and Splitting procedures using the MPQUIC-E steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501 [8]). true: Supported false (default): Not Supported |

#### 5.4.4.33	Type: PlmnIdNid

Table 5.4.4.33-1: Definition of type PlmnIdNid

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mcc | Mcc | M | 1 | Mobile Country Code |
| mnc | Mnc | M | 1 | Mobile Network Code |
| nid | Nid | C | 0..1 | Network Identity; Shall be present if PlmnIdNid identifies an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in 3GPP TS 23.501 [2]). Otherwise, this attribute shall be absent. |

When PlmnIdNid needs to be converted to string (e.g. when used in maps as key), the string shall be composed of three digits "mcc" followed by "-" and two or three digits "mnc" followed by "-" and 11 digits "nid", and shall match the following pattern:

^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{11}$

Example 1:	"262-01-000007ed9d5"

Example 2:	"302-720-000007ed9d5"

#### 5.4.4.34	Type: PlmnIdNidRm

This data type is defined in the same way as the "PlmnIdNid" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.35	Type: SmallDataRateStatus

Table 5.4.4.35-1: Definition of type SmallDataRateStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| remainPacketsUl | integer | C | 0..1 | This IE shall be included if available. When present, it shall contain the number of packets the UE is allowed to send uplink in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501 [8]). |
| remainPacketsDl | integer | C | 0..1 | This IE shall be included if available. When present it shall contain the number of packets the AF is allowed to send downlink in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501 [8]). |
| validityTime | DateTime | C | 0..1 | This IE shall be included if available. When present, it shall indicate the period of time during which the small data rate control status will remain valid (see clause 5.31.14.3 of 3GPP TS 23.501 [8]). |
| remainExReportsUl | integer | C | 0..1 | This IE shall be included if available. When present, it shall indicate number of additional exception reports the UE is allowed to send uplink in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501 [8]). |
| remainExReportsDl | integer | C | 0..1 | This IE shall be included if available. When present, it shall indicate number of additional exception reports the AF is allowed to send downlink in the given time unit for the given PDU session (see clause 5.31.14.3 in 3GPP TS 23.501 [8]). |

#### 5.4.4.36	Type: HfcNodeId

Table 5.4.4.36-1: Definition of type HfcNodeId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| hfcNId | HfcNId | M | 1 | HFC Node Id. |  |

#### 5.4.4.37	Type: HfcNodeIdRm

This data type is defined in the same way as the "HfcNodeId" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.38	Type: WirelineArea

Table 5.4.4.38-1: Definition of type WirelineArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| globalLineIds | array(Gli) | C | 1..N | List of Global Line Identifiers, for a 5G-BRG or an AUN3 device behind 5G-BRG accessing the 5GC via wireline access network. |  |
| hfcNIds | array(HfcNId) | C | 1..N | List of HFC Node Ids, for a 5G-CRG/FN-CRG is accessing the 5GC via wireline access network. |  |
| areaCodeB | AreaCode | C | 0..1 | Area Code for 5G-BRG or an AUN3 device behind 5G-BRG accessing via wireline access network |  |
| areaCodeC | AreaCode | C | 0..1 | Area Code for 5G-CRG/FN-CRG is accessing via wireline access network |  |
| combGciAndHfcNIds | array(CombGciAndHfcNIds) | C | 1..N | List of combinations of GCI and HFC Node ID, for a 5G-CRG or an AUN3 device behind the 5G-CRG accessing the 5GC via wireline access network. |  |
| NOTE: One and only one of the "globLineIds", "hfcNIds", "areaCodeB", "areaCodeC" and combGciAndHfcNIds attributes shall be included in a WirelineArea data structure. |  |  |  |  |  |

#### 5.4.4.39	Type: WirelineServiceAreaRestriction

Table 5.4.4.39-1: Definition of type WirelineServiceAreaRestriction

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| restrictionType | RestrictionType | C | 0..1 | string "ALLOWED_AREAS" or "NOT_ALLOWED_AREAS" (NOTE 1) |
| areas | array(WirelineArea) | C | 0..N | A list of Areas. These areas are: - allowed areas if RestrictionType is "ALLOWED_AREAS" - not allowed areas if RestrictionType is "NOT_ALLOWED_AREAS" (NOTE 1) (NOTE 2) |
| NOTE 1: The "restrictionType" attribute and the "areas" attribute shall be either both present or absent. NOTE 2: The empty array is used when service is allowed/restricted nowhere. |  |  |  |  |

#### 5.4.4.40	Type: ApnRateStatus

Table 5.4.4.40-1: Definition of type ApnRateStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| remainPacketsUl | integer | C | 0..1 | This IE shall be included if available. When present, it shall contain the number of packets the UE is allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN see clause 4.7.7.3 in 3GPP TS 23.401 [33]). |
| remainPacketsDl | integer | C | 0..1 | This IE shall be included if available. When present, it shall contain the number of packets, which the UE is allowed to send downlink for the given time unit period of time and for the given APN (all PDN connections of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401 [33]). |
| validityTime | DateTime | C | 0..1 | This IE shall be included if available. When present, it shall indicate the period of time during which the APN rate control status will remain valid. |
| remainExReportsUl | integer | C | 0..1 | This IE shall be included if available. When present, it shall indicate the number of additional exception reports the UE is allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401 [33]). |
| remainExReportsDl | integer | C | 0..1 | This IE shall be included if available. When present, it shall indicate the number of additional exception reports the AF is allowed to send downlink in the given time unit for the given APN (all PDN connections of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401 [33]). |

#### 5.4.4.41	Type: ScheduledCommunicationTime

Table 5.4.4.41-1: Definition of type ScheduledCommunicationTime

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| daysOfWeek | array(DayOfWeek) | O | 1..6 | Identifies the day(s) of the week. If absent, it indicates every day of the week. |
| timeOfDayStart | TimeOfDay | O | 0..1 | Identifies the start time of the day. |
| timeOfDayEnd | TimeOfDay | O | 0..1 | Identifies the end time of the day. |

#### 5.4.4.42	Type: ScheduledCommunicationTimeRm

This data type is defined in the same way as the "ScheduledCommunicationTime" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.43	Type: BatteryIndication

Table 5.4.4.43-1: Definition of type BatteryIndication

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| batteryInd | boolean | O | 0..1 | When present, this IE shall indicate whether the UE is battery powered or not. true: the UE is battery powered; false or absent: the UE is not battery powered. |
| replaceableInd | boolean | O | 0..1 | When present, this IE shall indicate whether the battery of the UE is replaceable or not. true: the battery of the UE is replaceable; false or absent: the battery of the UE is not replaceable. |
| rechargeableInd | boolean | O | 0..1 | When present, this IE shall indicate whether the battery of the UE is rechargeable or not. true: the battery of UE is rechargeable; false or absent: the battery of the UE is not rechargeable. |
| NOTE: Parameters "replaceableInd" and "rechargeableInd" are only included if the value of Parameter "batteryInd" is true. |  |  |  |  |

#### 5.4.4.44	Type: BatteryIndicationRm

This data type is defined in the same way as the "BatteryIndication" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.45	Type: AcsInfo

Table 5.4.4.45-1: Definition of type AcsInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| acsUrl | Uri | O | 0..1 | This IE may contain the URL of the ACS, see BBF TR-069 [34] or BBF TR-369 [35]. (NOTE) |
| acsIpv4Addr | Ipv4Addr | O | 0..1 | This IE may contain the IPv4 address of the ACS, see BBF TR-069 [34] or BBF TR-369 [35]. (NOTE) |
| acsIpv6Addr | Ipv6Addr | O | 0..1 | This IE may contain the IPv6 address of the ACS, see BBF TR-069 [34] or BBF TR-369 [35]. (NOTE) |
| NOTE: At least one of acsUrl, acsIpv4Addr, acsIpv6Addr shall be included. |  |  |  |  |

#### 5.4.4.46	Type: AcsInfoRm

This data type is defined in the same way as the "AcsInfo" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.47	Type: NrV2xAuth

Table 5.4.4.47-1: Definition of type NrV2xAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vehicleUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Vehicle UE. |
| pedestrianUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Pedestrian UE. |

#### 5.4.4.48	Type: LteV2xAuth

Table 5.4.4.48-1: Definition of type LteV2xAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vehicleUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Vehicle UE. |
| pedestrianUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as Pedestrian UE. |

#### 5.4.4.49	Type: Pc5QoSPara

Table 5.4.4.49-1: Definition of type Pc5QoSPara

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pc5QosFlowList | array(Pc5QosFlowItem) | M | 1..N | This IE shall contain the set of PC5 flow(s). |
| pc5LinkAmbr | BitRate | C | 0..1 | This IE shall be present if available. When present, it shall represent the PC5 Link Aggregated Bit Rates for all the Non-GBR QoS Flows (see clause 5.4.2.3 of 3GPP TS 23.287 [36]). |

#### 5.4.4.50	Type: Pc5QosFlowItem

Table 5.4.4.50-1: Definition of type Pc5QosFlowItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pqi | 5Qi | M | 1 | PQI is a special 5QI (see clause 5.4.2.1 of 3GPP TS 23.287 [36]). |
| pc5FlowBitRates | Pc5FlowBitRates | C | 0..1 | This IE shall be present if available. When present, it shall represent the PC5 Flow Bit Rates (see clause 5.4.2.2 of 3GPP TS 23.287 [36]). |
| range | Uinteger | C | 0..1 | This IE shall be present if available. When present, it shall represent the Range in the unit of meters (see clause 5.4.2.4 of 3GPP TS 23.287 [36]). |

#### 5.4.4.51	Type: Pc5FlowBitRates

Table 5.4.4.51-1: Definition of type Pc5FlowBitRates

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| guaFbr | BitRate | C | 0..1 | This IE shall be present if available. When present, it shall contain the guaranteed Bit Rate for the PC5 QoS flow. |
| maxFbr | BitRate | C | 0..1 | This IE shall be present if available. When present, it shall contain the maximum Bit Rate for the PC5 QoS flow. |

#### 5.4.4.52	Type: UtraLocation

Table 5.4.4.52-1: Definition of type UtraLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cgi | CellGlobalId | O | 0..1 | Cell Global Identification. See 3GPP TS 23.003 [7], clause 4.3.1 (NOTE 1) |
| sai | ServiceAreaId | O | 0..1 | Service Area Identifier. See 3GPP TS 23.003 [7], clause 12.5 (NOTE 1) |
| lai | LocationAreaId | O | 0..1 | Location area identification. See 3GPP TS 23.003 [7], clause 4.1 (NOTE 1) |
| rai | RoutingAreaId | O | 0..1 | Routing Area Identification. See 3GPP TS 23.003 [7], clause 4.2 |
| ageOfLocationInformation | integer | O | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful location reporting procedure the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [21] clause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [23] clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24] clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| NOTE 1: Exactly one of cgi, sai or lai shall be present. |  |  |  |  |

#### 5.4.4.53	Type: GeraLocation

Table 5.4.4.53-1: Definition of type GeraLocation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationNumber | string | O | 0..1 | Location number within the PLMN. See 3GPP TS 23.003 [7], clause 4.5. |
| cgi | CellGlobalId | O | 0..1 | Cell Global Identification. See 3GPP TS 23.003 [7], clause 4.3.1 (NOTE 1) |
| sai | ServiceAreaId | O | 0..1 | Service Area Identifier. See 3GPP TS 23.003 [7], clause 12.5 (NOTE 1) |
| lai | LocationAreaId | O | 0..1 | Location Area identification. See 3GPP TS 23.003 [7], clause 4.1 (NOTE 1) |
| rai | RoutingAreaId | O | 0..1 | Routing Area Identification. See 3GPP TS 23.003 [7], clause 4.2 |
| vlrNumber | string | O | 0..1 | VLR number. See 3GPP TS 23.003 [7] clause 5.1. |
| mscNumber | string | O | 0..1 | MSC number. See 3GPP TS 23.003 [7] clause 5.1. |
| ageOfLocationInformation | integer | O | 0..1 | The value represents the elapsed time in minutes since the last network contact of the mobile station. Value "0" indicates that the location information was obtained after a successful paging procedure for Active Location Retrieval when the UE is in idle mode or after a successful location reporting procedure the UE is in connected mode. Any other value than "0" indicates that the location information is the last known one. See 3GPP TS 29.002 [21] clause 17.7.8. |
| ueLocationTimestamp | DateTime | O | 0..1 | The value represents the UTC time when the UeLocation information was acquired. |
| geographicalInformation | string | O | 0..1 | Refer to geographical Information. See 3GPP TS 23.032 [23] clause 7.3.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F; |
| geodeticInformation | string | O | 0..1 | Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24] clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed to be used. Allowed characters are 0-9 and A-F. |
| NOTE 1: Exactly one of cgi, sai or lai shall be present. |  |  |  |  |

#### 5.4.4.54	Type: CellGlobalId

Table 5.4.4.54-1: Definition of type CellGlobalId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| lac | string | M | 1 | Location Area Code Pattern: '^[A-Fa-f0-9]{4}$' |
| cellId | string | M | 1 | Cell Identity Pattern: '^[A-Fa-f0-9]{4}$' |

#### 5.4.4.55	Type: ServiceAreaId

Table 5.4.4.55-1: Definition of type ServiceAreaId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| lac | string | M | 1 | Location Area Code Pattern: '^[A-Fa-f0-9]{4}$' |
| sac | string | M | 1 | Service Area Code Pattern: '^[A-Fa-f0-9]{4}$' |

#### 5.4.4.56	Type: LocationAreaId

Table 5.4.4.56-1: Definition of type LocationAreaId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| lac | string | M | 1 | Location Area Code Pattern: '^[A-Fa-f0-9]{4}$' |

#### 5.4.4.57	Type: RoutingAreaId

Table 5.4.4.57-1: Definition of type RoutingAreaId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN Identity |
| lac | string | M | 1 | Location Area Code Pattern: '^[A-Fa-f0-9]{4}$' |
| rac | string | M | 1 | Routing Area Code Pattern: '^[A-Fa-f0-9]{2}$' |

#### 5.4.4.58	Type: DddTrafficDescriptor

Table 5.4.4.58-1: Definition of type DddTrafficDescriptor

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | Ipv4 address of the source of downlink data. |
| ipv6Addr | Ipv6Addr | C | 0..1 | Ipv6 address of the source of downlink data. |
| portNumber | Uinteger | O | 0..1 | Port number of the source of downlink data. |
| macAddr | MacAddr48 | C | 0..1 | Source MAC address. |
| NOTE: Either IP address (at least one of the "ipv4Addr" attribute or the "ipv6Addr" attribute) or MAC address (the "macAddr" attribute) shall be included. |  |  |  |  |

#### 5.4.4.59	Type: MoExpDataCounter

Table 5.4.4.59-1: Definition of type MoExpDataCounter

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| counter | integer | M | 1 | Unsigned integer identifying the MO Exception Data Counter, as specified in clause 5.31.14.3 of 3GPP TS 23.501 [8]. |
| timeStamp | DateTime | O | 0..1 | UTC time indicating the time at which the counter value increased from 0 to 1. |

#### 5.4.4.60	Type: NssaaStatus

Table 5.4.4.60-1: Definition of type NssaaStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Subscribed S-NSSAI |
| status | AuthStatus | M | 1 | This flag when present shall indicate the NSSAA status of the related Snssai. |

#### 5.4.4.61	Type: NssaaStatusRm

This data type is defined in the same way as the "NssaaStatus" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.62	Type: TnapId

Table 5.4.4.62-1: Definition of type TnapId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ssId | string | C | 0..1 | This IE shall be present if the UE is accessing the 5GC via a trusted WLAN access network. When present, it shall contain the SSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012 [31]. |
| bssId | string | C | 0..1 | This IE shall be present if available and if the UE is accessing the 5GC via a trusted WLAN access network. When present, it shall contain the BSSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012 [31]. |
| civicAddress | Bytes | C | 0..1 | This IE shall be present if available and if the UE is accessing the 5GC via a trusted WLAN access network. This IE shall also be present if the UE behind the 5G-RG is accessing the 5GC via a trusted WLAN access network. When present, it shall contain the civic address information of the TNAP/5G-RG to which the UE is attached, including the Location-Information Attribute and / or Location-Data Attribute as defined in IETF RFC 5580 [40]. |

#### 5.4.4.63	Type: TnapIdRm

This data type is defined in the same way as the "TnapId" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.64	Type: TwapId

Table 5.4.4.64-1: Definition of type TwapId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ssId | string | M | 1 | This IE shall contain the SSID of the access point to which the UE is attached, that is received over NGAP, see IEEE Std 802.11-2012 [31]. |
| bssId | string | C | 0..1 | This IE shall be present if available. When present, it shall contain the BSSID of the access point to which the UE is attached, for trusted WLAN access, see IEEE Std 802.11-2012 [31]. |
| civicAddress | Bytes | C | 0..1 | This IE shall be present if available. When present, it shall contain the civic address information of the TWAP to which the UE is attached, for trusted WLAN access. This IE shall include the Location-Information Attribute and / or Location-Data Attribute as defined in IETF RFC 5580 [40]. |

#### 5.4.4.65	Type: TwapIdRm

This data type is defined in the same way as the "TwapId" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.66	Type: SnssaiExtension

Table 5.4.4.66-1: Definition of type SnssaiExtension

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sdRanges | array(SdRange) | C | 1..N | When present, it shall contain the range(s) of Slice Differentiator values supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2). |
| wildcardSd | boolean | C | 0..1 | When present, it shall be set to true, to indicate that all SD values are supported for the Slice/Service Type value indicated in the sst attribute of the Snssai data type (see clause 5.4.4.2). |
| NOTE 1: sdRanges and wildcardSd shall not be present simultaneously. NOTE 2: An SdRange may include the value “FFFFFF”; similarly, if wildcardSd is set to true, the SD value “FFFFFF” is one of the supported values. In both cases the SST without associated SD is one of the supported SNSSAIs, as the value “FFFFFF” indicates "no SD value associated with the SST" (see 3GPP TS 23.003 [7]). |  |  |  |  |

#### 5.4.4.67	Type: SdRange

Table 5.4.4.67-1: Definition of type SdRange

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| start | string | M | 1 | First value identifying the start of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2. |
| end | string | M | 1 | Last value identifying the end of an SD range. This string shall be formatted as specified for the sd attribute of the Snssai data type in clause 5.4.4.2. |

EXAMPLE:	SD range from 023400 to 023499 (hexadecimal)
JSON: { "start": "023400", "end": "023499" }

#### 5.4.4.68	Type: ProseServiceAuth

Table 5.4.4.68-1: Definition of type ProseServiceAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| proseDirectDiscoveryAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to use ProSe Direct Discovery. |
| proseDirectCommunicationAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to use ProSe Direct Communication. |
| proseL2RelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 UE-to-Network Relay. |
| proseL3RelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 UE-to-Network Relay. |
| proseL2RemoteAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 Remote UE. |
| proseL3RemoteAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 Remote UE. |
| proseMultipathComL2RemoteAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE. |
| proseL2UeRelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 UE-to-UE Relay. |
| proseL3UeRelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 UE-to-UE Relay. |
| proseL2EndAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 End UE. |
| proseL3EndAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 End UE. |
| proseL3IntermediateRelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 Intermediate UE-to-Network Relay. |
| proseL3RemoteMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 Remote UE supporting Multi-hop 5G ProSe Layer-3 UE-to-Network Relay. |
| proseL3RelayMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-Network Relay. |
| proseL3UeRelayMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 UE-to-UE Relay supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay. |
| proseL3EndMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-3 End UE supporting Multi-hop 5G ProSe Layer-3 UE-to-UE Relay. |
| proseL2IntermediateRelayAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 Intermediate UE-to-Network Relay. |
| proseL2RemoteMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 Remote UE supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay. |
| proseL2RelayMultihopAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as 5G ProSe Layer-2 UE-to-Network Relay supporting Multi-hop 5G ProSe Layer-2 UE-to-Network Relay. |

#### 5.4.4.69	Type: EcsServerAddr

Table 5.4.4.69-1: Definition of type EcsServerAddr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ecsFqdnList | array(Fqdn) | C | 1..N | This IE shall be included if available. When present, it shall contain the list of FQDN(s) of Edge Configuration Server(s). |
| ecsIpAddressList | array(IpAddr) | C | 1..N | This IE shall be included if available. When present, it shall contain the list of IP Address (es) of Edge Configuration Server(s). |
| ecsUriList | array(Uri) | C | 1..N | This IE shall be included if available. When present, it shall contain the list of URI(s) of the Edge Configuration Server(s). |
| ecsProviderId | string | C | 0..1 | This IE shall be included if available. When present, it shall contain the identifier of the Edge Configuration Server Provider. |

#### 5.4.4.70	Type: EcsServerAddrRm

This data type is defined in the same way as the "EcsServerAddr" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.71	Type: IpAddr

Table 5.4.4.71-1: Definition of type IpAddr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | When present, it shall contain the IPv4 address. |
| ipv6Addr | Ipv6Addr | C | 0..1 | When present, it shall contain the IPv6 address. |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | When present, it shall contain the IPv6 Prefix. |
| NOTE: Either ipv4Addr, or ipv6Addr, or ipv6Prefix shall be present. |  |  |  |  |

#### 5.4.4.72	Type: SACInfo

Table 5.4.4.72-1: Definition of type SACInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| numericValNumUes | integer | C | 0..1 | This attribute may be present in the following cases: - to configure the monitoring threshold for the reporting of the number of registered UEs for a network slice identified by an S-NSSAI; - to report the network slice status for the current number of registered UEs. When used to configure the monitoring threshold for an S-NSSAI, it shall contain the configured event monitoring threshold value for monitoring the number of registered UEs expressed in a numerical value. When used to report the network slice status for an S-NSSAI, it shall contain the current number of registered UEs in the concerned network slice expressed in a numerical value. For threshold based reporting and the threshold value for the number of registered UEs in the concerned network slice was previously configured in the form of a numerical value, this attribute shall contain the current number of registered UEs in the concerned network slice expressed in a numerical value. |
| numericValNumPduSess | integer | C | 0..1 | This attribute may be present in the following cases: - to configure the monitoring threshold for the reporting of the number established PDU session for a network slice identified by an S-NSSAI; - to report the network slide status for the number of established PDU sessions. When used to configure the monitoring threshold for an S-NSSAI, it shall contain the configured event monitoring threshold value for monitoring the number of established PDU sessions expressed in a numerical value. When used to report the network slice status for an S-NSSAI, it shall contain the current number of established PDU sessions in the concerned network slice expressed in a numerical value. For threshold based reporting and the threshold value for the number of established PDU sessions in the concerned network slice was previously configured in the form of a numerical value,, this attribute shall contain the current number of established PDU sessions in the concerned network slice expressed in a numerical value. |
| percValueNumUes | integer | C | 0..1 | This attribute may be present in the following cases: - to configure the monitoring threshold for the reporting of the number of registered UEs for a network slice identified by an S-NSSAI; - to report the network slice status for the number of registered UEs . When used to configure the monitoring threshold for an S-NSSAI, it shall contain an unsigned integer indicating the event monitoring threshold value for the number of registered UEs expressed in percentage format. When used to report the network slice status for an S-NSSAI, it shall contain the current number of registered UEs in the concerned network slice expressed as a percentage. For threshold based reporting and the threshold value for the number of registered UEs in the concerned network slice was previously configured as a percentage, this attribute shall contain the current number of registered UEs in the concerned network slice expressed as a percentage. Minimum = 0. Maximum = 100. |
| percValueNumPduSess | integer | C | 0..1 | This IE may be present in the following cases: - to configure the monitoring threshold for the reporting of the number of established PDU sessions for a network slice identified by an S-NSSAI; - to report the network slice status for the number of established PDU sessions. When used to configure the monitoring threshold for an S-NSSAI, it shall contain an unsigned integer indicating the event monitoring threshold value for the number of established PDU sessions expressed in percentage format. When used to report the status of an S-NSSAI, it shall contain the current number of established PDU sessions in the concerned network slice expressed as a percentage. For threshold based reporting and the threshold value for the number of established PDU sessions in the concerned network slice was previously configured as a percentage, this attribute shall contain the current number of established PDU sessions in the concerned network slice expressed as a percentage. Minimum = 0. Maximum = 100. |
| uesWithPduSessionInd | boolean | C | 0..1 | This IE may be present if the numericValNumUes IE or the percValueNumUes IE is present, when reporting the network slice status for an S-NSSAI. When present, it shall be set as follows: - True: the numericValNumUes and percValueNumUes report the number of UEs with at least one PDU session/PDN connection. - False (default): the numericValNumUes and percValueNumUes report the current number of registered UEs. |

#### 5.4.4.73	Type: SACEventStatus

Table 5.4.4.73-1: Definition of type SACEventStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reachedNumUes | SACInfo | O | 0..1 | Contains a confirmation that the requested threshold for the number of registered UEs in the concerned network slice was reached, when threshold based reporting is used, or the current number of registered UEs in the concerned network slice, when periodic reporting / immediate reporting is used. |
| reachedNumPduSess | SACInfo | O | 0..1 | Contains a confirmation that the requested threshold for the number of established PDU session in the concerned network slice was reached, when threshold based reporting is used, or the current number of established PDU sessions in the concerned network slice, when periodic reporting / immediate reporting is used. |

#### 5.4.4.74	Type: SpatialValidityCond

Table 5.4.4.74-1: Definition of type SpatialValidityCond

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| trackingAreaList | array(Tai) | C | 1..N | This IE shall be included if available. When present, it shall contain the list of tracking areas identities. |
| countries | array(Mcc) | O | 1..N | When present, it shall contain the list of Mobile Country Codes. |
| geographicalServiceArea | GeoServiceArea | O | 0..1 | Geographical Service Area; see 3GPP TS 23.558 [49] clause 7.3.3.3 |

#### 5.4.4.75	Type: SpatialValidityCondRm

This data type is defined in the same way as the "SpatialValidityCond" data type, but with the OpenAPI "nullable: true" property.

#### 5.4.4.76	Type: ServerAddressingInfo

Table 5.4.4.76 -1: Definition of type ServerAddressingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Addresses | array(Ipv4Addr) | C | 1..N | IPv4 address(es) of the server (NOTE). |
| ipv6Addresses | array(Ipv6Addr) | C | 1..N | IPv6 address(es) of the server (NOTE). |
| fqdnList | array(Fqdn) | C | 1..N | List of FQDNs (Fully Qualified Domain Names) of the server (NOTE). |
| NOTE: At least one of the addressing parameters (ipv4addresses, ipv6adresses or fqdnList) shall be included in the ServerAddressingInfo; all addressing parameters in this data type shall be understood as referring to a same sever. |  |  |  |  |

#### 5.4.4.77	Type PcfUeCallbackInfo

Table 5.4.4.77-1: Definition of type PcfUeCallbackInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| callbackUri | Uri | M | 1 | This IE shall contain the Callback URI on the PCF for a UE to receive the SM Policy Association Establishment and Termination Event Notifications from the PCF for a PDU session. |  |
| bindingInfo | string | O | 0..1 | This IE shall be present, if available. When present, this IE shall contain the Binding indications of the Callback URI on the PCF for a UE indicated by callbackUri IE and set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [25], without the header name. |  |

#### 5.4.4.78	Type PduSessionInfo

Table 5.4.4.78-1: Definition of type PduSessionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | This IE shall indicate the S-NSSAI in the serving PLMN of a PDU session. |  |
| dnn | Dnn | M | 1 | This IE shall Indicate the DNN of a PDU session. If DNN replacement if applicable for the PDU session, this IE shall indicate the DNN of the PDU session after DNN replacement. |  |

#### 5.4.4.79	Type EasIpReplacementInfo

Table 5.4.4.79-1: Definition of type EasIpReplacementInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| source | EasServerAddress | M | 1 | Address of the source EAS, i.e., address that shall be used for the traffic on the N3 side of the UPF(s). |  |
| target | EasServerAddress | M | 1 | Address of the target EAS, i.e., address that shall be used for the traffic on the N6 side of the UPF(s). |  |

#### 5.4.4.80	Type EasServerAddress

Table 5.4.4.80-1: Definition of type EasServerAddress

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ip | IpAddr | M | 1 | IP address information. |  |
| port | Uinteger | M | 1 | IP port number. |  |

#### 5.4.4.81	Type RoamingRestrictions

Table 5.4.4.81-1: Definition of type RoamingRestrictions

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accessAllowed | boolean | C | 0..1 | Indicates if access is allowed to a given serving network, e.g. a PLMN (MCC, MNC) or an SNPN (MCC, MNC, NID). NOTE |  |
| NOTE: The actual query determines if the 'accessAllowed' attribute refers to an SNPN or to a PLMN. |  |  |  |  |  |

#### 5.4.4.82	Type: GeoServiceArea

Table 5.4.4.82-1: Definition of type GeoServiceArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| geographicAreaList | array(GeographicArea) | O | 1..N | Identifies a list of geographic area specified by different shapes. |
| civicAddressList | array(CivicAddress) | O | 1..N | Identifies a list of civic address. |

#### 5.4.4.83	Type: MutingExceptionInstructions

Table 5.4.4.83-1: Definition of type MutingExceptionInstructions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bufferedNotifs | BufferedNotificationsAction | O | 0..1 | When present, it shall indicate the action required by the NF Service Consumer to the NF Service Producer on the buffered notifications if an exception occurs while the event is muted. |
| subscription | SubscriptionAction | O | 0..1 | When present, it shall indicate the action required by the NF Service Consumer to the NF Service Producer on the subscription if an exception occurs while the event is muted. |

#### 5.4.4.84	Type: MutingNotificationsSettings

Table 5.4.4.84-1: Definition of type MutingNotificationsSettings

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| maxNoOfNotif | integer | O | 0..1 | Maximum number of notifications that can be stored by the Event producer NF. |
| durationBufferedNotif | DurationSec | O | 0..1 | Maximum duration during which notifications can be buffered by the Event producer NF. |

#### 5.4.4.85	Type: VplmnOffloadingInfo

Table 5.4.4.85-1: Definition of type VplmnOffloadingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| offloadIdentifier | OffloadIdentifier | O | 0..1 | Offload Identifier uniquely identifying the VPLMN Offloading information provided by the HPLMN. |
| vplmnId | PlmnId | O | 0..1 | V-PLMN ID. When absent, the PLMN ID of the PLMN serving the UE shall be assumed. |
| allowedTraffic | boolean | O | 0..1 | When present, this IE shall be set as follows: - true (default): the VplmnOffloadingInfo describes the traffic allowed to be offloaded - false: the VplmnOffloadingInfo describes the traffic disallowed to be offloaded |
| ipv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| ipv4AddrMasks | array(Ipv4AddrMask) | O | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN, whereby each range of IPv4 addresses corresponds to the IPv4 addresses of an IPv4 subnet defined by an IPv4 address and subnet mask. |
| ipv6AddressRanges | array(Ipv6AddressRange) | O | 1..N | List of ranges of IPv6 addresses allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| ipv6PrefixRanges | array(Ipv6PrefixRange) | O | 1..N | List of ranges of IPv6 prefixes allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| fqdnList | array(Fqdn) | O | 1..N | List of FQDNs allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| fqdnPatterns | array(FqdnPatternMatchingRule) | O | 1..N | List of FQDN patterns of FQDNs allowed (or disallowed) to be routed to the local part of DN in the VPLMN |
| NOTE: If none of the ipv4AddressRanges, ipv4AddrMasks, ipv6AddressRanges, ipv6PrefixRanges, fqdnList and fqdnPatterns IEs is present, all the traffic of the PDU session is allowed to be routed to the local part of DN in the VPLMN. At least one of these IEs shall be present when the allowedTraffic IE is set to false. |  |  |  |  |

#### 5.4.4.86	Type: PartiallyAllowedSnssai

Table 5.4.4.86-1: Definition of type PartiallyAllowedSnssai

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | This IE shall indicate the S-NSSAI that is partially allowed in the Registration Area. |
| allowedTaiList | array(Tai) | M | 1..N | This IE shall contain the list of TAI(s) in the Registration Area where the indicated S-NSSAI is allowed. |

#### 5.4.4.87	Type: VarRepPeriod

Table 5.4.4.87-1: Definition of type VarRepPeriod

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| repPeriod | DurationSec | M | 1 | This IE describes the period time for the variable event reports. |
| percValueNfLoad | Uinteger | C | 0..1 | This IE shall be present if the variable reporting periodicity is based on the load of NF service producer (see 3GPP TS 23.502 [28], clause 4.15.1). When present, this IE indicates the load percentage of NF service producer, within the range 0 to 100. |
| NOTE: The reporting periodicity is changed depending on the load of NF service producer, if the load of NF service producer is equal or greater than several values in array of VarRepPeriod, the repPeriod related to the highest value of nfLoad shall be applied. |  |  |  |  |

#### 5.4.4.88	Type: RangingSlPosAuth

Table 5.4.4.88-1: Definition of type RangingSlPosAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rgSlPosPc5Auth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to perform Ranging/Sidelink Positioning over PC5. |
| rgSlPosLocAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as Located UE. |
| rgSlPosClientAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as SL Positioning Client UE. |
| rgSlPosServerAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized to act as SL Positioning Server UE. |

#### 5.4.4.89	Type: NrA2xAuth

Table 5.4.4.89-1: Definition of type NrA2xAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uavUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as UAV UE. |

#### 5.4.4.90	Type: LteA2xAuth

Table 5.4.4.90-1: Definition of type LteA2xAuth

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uavUeAuth | UeAuth | C | 0..1 | This IE shall be present if available. When present, it shall indicate whether the UE is authorized as UAV UE. |

#### 5.4.4.91	Type: SliceUsageControlInfo

Table 5.4.4.91-1: Definition of type SliceUsageControlInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sNssai | Snssai | M | 1 | S-NSSAI |
| deregInactTimer | DurationSec | C | 0..1 | Identifies the slice deregistration inactivity timer for the Network Slice identified by the sNssai IE (see 3GPP TS 23.501 [8], clause 5.15.15.3) The value shall be applied to both 3GPP access and Non-3GPP access. (NOTE) |
| sessInactTimer | DurationSec | C | 0..1 | Identifies the PDU Session inactivity timer for the Network Slice identified by the sNssai IE (see 3GPP TS 23.501 [8], clause 5.15.15.3) The value shall be applied to both 3GPP access and Non-3GPP access. (NOTE) |
| NOTE: At least deregInactTimer or sessInactTimer shall be present. |  |  |  |  |

#### 5.4.4.92	Type: CombGciAndHfcNIds

Table 5.4.4.92-1: Definition of type CombGciAndHfcNIds

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| globalCableId | Gci | C | 0..1 | Global Cable Identifier, for an AUN3 device behind 5G-CRG accessing the 5GC via wireline access network. |  |
| hfcNId | HfcNId | C | 0..1 | HFC Node Id, for an AUN3 device behind 5G-CRG is accessing the 5GC via wireline access network. |  |

#### 5.4.4.93	Type: SnssaiDnnItem

Table 5.4.4.93-1: Definition of type SnssaiDnnItem

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssaiList | array(ExtSnssai) | C | 1..N | List of S-NSSAIs |
| dnnList | array(Dnn) | C | 1..N | List of DNNs |
| NOTE: At least one of the snssaiList and dnnList IEs shall be present. If the dnnList IE is absent, this indicates that all DNNs of the provided S-NSSAIs are considered. If the sNssaiList IE is absent, this indicates that all S-NSSAIs of the provided DNNs are considered. If both IEs are present, this indicates that the provided DNNs for the provided S-NSSAIs are considered. |  |  |  |  |

#### 5.4.4.94	Type: NtnTaiInfo

Table 5.4.4.94-1: Definition of type NtnTaiInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnIdNid | M | 1 | UE's serving PLMN Identity |
| tacList | array(Tac) | M | 1..N | TAC list received from satellite NG-RAN without the forbidden TAIs for the UE |
| derivedTac | Tac | O | 0..1 | This attribute may be present if Derived TAC is received from satellite NG-RAN |

#### 5.4.4.95	Type: MitigationInfo

Table 5.4.4.95-1: Definition of type MitigationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| percValueNumUes | integer | C | 0..1 | This IE shall be present if available. When present, it shall contain an unsigned integer indicating the number of registered UEs expressed in percentage format applied for Network Slice Replacement. Minimum = 0. Maximum = 100. |
| newUesInd | boolean | C | 0..1 | This IE shall be present if available. When present, it shall be set to true to indicate the Network Slice Replacement is applied for new UEs. |
| NOTE: Either the percValueNumUes IE or the newUesInd IE shall be present. |  |  |  |  |

#### 5.4.4.96	Type: VplmnDlAmbr

Table 5.4.4.96-1: VplmnDlAmbr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vplmnId | PlmnId | M | 1 | V-PLMN ID. |
| sessionDlAmbr | BitRate | M | 1 | Authorized DL Session AMBR for Offloading, i.e. DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session authorized for offloading to the local part of DN in VPLMN. |

#### 5.4.4.97	Type: LocalOffloadingManagementInfo

Table 5.4.4.97-1: Definition of type LocalOffloadingManagementInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| offloadIdentifier | OffloadIdentifier | C | 0..1 | When present, this IE shall uniquely identify the Local Offloading Management information within a PLMN. (NOTE 1) (NOTE 2) |
| allowedTraffic | boolean | O | 0..1 | When present, this IE shall be set as follows: - true (default): the LocalOffloadingInfo describes the traffic allowed to be offloaded - false: the LocalOffloadingInfo describes the traffic disallowed to be offloaded |
| ipv4AddressRanges | array(Ipv4AddressRange) | C | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN. (NOTE 2) |
| ipv4AddrMasks | array(Ipv4AddrMask) | C | 1..N | List of ranges of IPv4 addresses allowed (or disallowed) to be routed to the local part of DN, whereby each range of IPv4 addresses corresponds to the IPv4 addresses of an IPv4 subnet defined by an IPv4 address and subnet mask. (NOTE 2) |
| ipv6AddressRanges | array(Ipv6AddressRange) | C | 1..N | List of ranges of IPv6 addresses allowed (or disallowed) to be routed to the local part of DN. (NOTE 2) |
| ipv6PrefixRanges | array(Ipv6PrefixRange) | C | 1..N | List of ranges of IPv6 prefixes allowed (or disallowed) to be routed to the local part of DN. (NOTE 2) |
| fqdnList | array(Fqdn) | C | 1..N | List of FQDNs allowed (or disallowed) to be routed to the local part of DN. (NOTE 2) |
| fqdnPatterns | array(FqdnPatternMatchingRule) | C | 1..N | List of FQDN patterns of FQDNs allowed (or disallowed) to be routed to the local part of DN. (NOTE 2) |
| NOTE 1: If none of the ipv4AddressRanges, ipv4AddrMasks, ipv6AddressRanges, ipv6PrefixRanges, fqdnList and fqdnPatterns IEs is present, whether the traffic of the PDU session is allowed to be routed to the local part of DN is determined based on the stored or pre-configured Local Offloading Management Policy identified by offload identifier. NOTE 2: At least one of the "offloadIdentifier", "ipv4AddressRanges", "ipv4AddrMasks", "ipv6AddressRanges", "ipv6PrefixRanges", "fqdnList" and "fqdnPatterns" attributes shall be included. |  |  |  |  |

#### 5.4.4.98	Type: CagProvisionInformation

Table 5.4.4.98-1: Definition of type CagProvisionInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cagInfo | array(CagId) | M | 1..N | Indicates a list of CAG ID(s) identifying the CAG(s) which are allowed for a UE or for a list of UEs. |
| commonValidTimePeriodList | array(ValidTimePeriod) | C | 0..N | This IE shall be present in the following cases: - the provisionOperation attribute is not present; or - the provisionOperation attribute is present with the value of "ADD" or "REPLACE". When present, it indicates the time periods during which the UE can access CAG cells. The time periods present in this IE shall be applied to all CAG ID(s) listed in the cagInfo attribute. An empty array indicates no common time periods are configured to those CAG ID(s). |
| additionalValidTimePeriodList | map(array(ValidTimePeriod)) | O | 1..M(1..N) | A map (list of key-value pairs) where CAG ID converted to string serves as key, and the value is an array of time periods associated with the CAG ID. This IE may be present if some CAG ID(s) are configured with additional time periods that are different from the time periods in the commonValidTimePeriodList attribute. |
| provisionOperation | CagProvisionOperation | O | 0..1 | Indicates the CAG provision operation applied to the CAG ID list present in the cagInfo attribute. If this attribute is not present, it indicates the CAG ID list indicated in the cagInfo attribute is used to replace the previously configured CAG ID list, if any. |
| NOTE: If a CAG ID is not associated to any validity time period(s) (i.e. neither the commonValidTimePeriodList is present with non-empty time periods nor the corresponding CAG ID is present in the additionalValidTimePeriodList attribute), the CAG shall be permanently allowed. |  |  |  |  |

### 5.4.5	Data types describing alternative data types or combinations of data types

#### 5.4.5.1	Type: ExtSnssai

Table 5.4.5.1-1: Definition of type ExtSnssai as a list of to be combined data types

| Data type | Cardinality | Description |
| --- | --- | --- |
| Snssai | 1 | Common data type defined in clause 5.4.4.2. |
| SnssaiExtension | 1 | Extensions to the Snssai common data type defined in clause 5.4.4.66. |
| NOTE: The sdRanges and wildcardSd attributes shall be exclusive from each other. If one of these attributes is present, the sd attribute shall also be present and it shall contain one Slice Differentiator value within the range of SD (if the sdRanges attribute is present) or with any value (if the wildcardSd attribute is present). |  |  |

#### 5.4.5.2	Type: SnssaiReplaceInfo

Table 5.4.5.2-1: Definition of type SnssaiReplaceInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | Indicates the impacted S-NSSAI. |
| status | SnssaiStatus | C | 0..1 | It shall be present, if the status of the S-NSSAI indicated in the snssai IE changes. When present, it indicates the availability status of the S-NSSAI indicated in the snssai IE. |
| altSnssai | Snssai | C | 0..1 | It shall be present, if the alternative S-NSSAI is requested to replace the S-NSSAI indicated in snssai IE. When present, this IE shall indicate the alternative S-NSSAI NSSAI to the impacted S-NSSAI indicated by the "snssai" attribute. In the case of roaming it shall indicate: - the alternative VPLMN S-NSSAI for replacement of the impacted VPLMN S-NSSAI, when the snssai IE contains a VPLMN S-NSSAI; or - the alternative HPLMN S-NSSAI for replacement of the impacted HPLMN S-NSSAI, when the snssai IE contains an HPLMN S-NSSAI. |
| nsReplTerminInd | TerminationIndication | C | 0..1 | This IE shall be present for a notification of termination of Network Slice Replacement. |
| plmnId | PlmnId | C | 0..1 | This IE shall be present in roaming scenarios, if the impacted S-NSSAI indicated by the snssai IE is an HPLMN S-NSSAI. It may be present otherwise. When present, it shall indicate the PLMN ID of the impacted S-NSSAI (and alternative S-NSSAI). |
| mitigationInfo | MitigationInfo | O | 0..1 | This IE may be present for a notification of Network Slice Replacement. When present, it shall include the congestion mitigation information (see clause 5.2.16.3.3 of 3GPP TS 23.502 [28]). If absent, Network Slice Replacement shall be applied for all the registered UEs and new UEs. |

## 5.5	Data Types related to 5G QoS

### 5.5.1	Introduction

This clause defines common data types related to 5G QoS.

### 5.5.2	Simple Data Types

This clause specifies common simple data types.

Table 5.5.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Qfi | integer | Unsigned integer identifying a QoS flow, within the range 0 to 63. |
| QfiRm | integer | This data type is defined in the same way as the "Qfi" data type, but with the OpenAPI "nullable: true" property. |
| 5Qi | integer | Unsigned integer representing a 5G QoS Identifier (see clause 5.7.2.1 of 3GPP TS 23.501 [8]), within the range 0 to 255. |
| 5QiRm | integer | This data type is defined in the same way as the "5Qi" data type, but with the OpenAPI "nullable: true" property. |
| BitRate | string | String representing a bit rate that shall be formatted as follows: Pattern: '^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$' (NOTE) Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps" |
| BitRateRm | string | This data type is defined in the same way as the "BitRate" data type, but with the OpenAPI "nullable: true" property. |
| PacketRate | string | String representing a packet rate, i.e. packets per second, that shall be formatted as follows: Pattern: '^\d+(\.\d+)? (pps¦kpps¦Mpps¦Gpps¦Tpps)$' (NOTE) Examples: "125 Mpps", "0.125 Gpps", "125000 kpps" |
| PacketRateRm | string | This data type is defined in the same way as the "PacketRate" data type, but with the OpenAPI "nullable: true" property. |
| TrafficVolume | string | String representing a traffic volume measured in bytes that shall be formatted as follows: Pattern: '^\d+(\.\d+)? (B¦kB¦MB¦GB¦TB)$' (NOTE) Examples: "125 MB", "0.125 GB", "125000 kB" |
| TrafficVolumeRm | string | This data type is defined in the same way as the "TrafficVolume" data type, but with the OpenAPI "nullable: true" property. |
| ArpPriorityLevel | integer | Unsigned integer indicating the ARP Priority Level (see clause 5.7.2.2 of 3GPP TS 23.501 [8]), within the range 1 to 15. Values are ordered in decreasing order of priority, i.e. with 1 as the highest priority and 15 as the lowest priority. |
| ArpPriorityLevelRm | integer | This data type is defined in the same way as the "ArpPriorityLevel" data type, but with the OpenAPI "nullable: true" property. |
| 5QiPriorityLevel | integer | Unsigned integer indicating the 5QI Priority Level (see clauses 5.7.3.3 and 5.7.4 of 3GPP TS 23.501 [8]), within the range 1 to 127. Values are ordered in decreasing order of priority, i.e. with 1 as the highest priority and 127 as the lowest priority. |
| 5QiPriorityLevelRm | integer | This data type is defined in the same way as the "5QiPriorityLevel" data type, but with the OpenAPI "nullable: true" property. |
| PacketDelBudget | integer | Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501 [8])), expressed in milliseconds. Minimum = 1. |
| PacketDelBudgetRm | integer | This data type is defined in the same way as the "PacketDelBudget" data type, but with the OpenAPI "nullable: true" property. |
| PacketErrRate | string | String representing Packet Error Rate (see clause 5.7.3.5 and 5.7.4 of 3GPP TS 23.501 [8]), expressed as a "scalar x 10-k" where the scalar and the exponent k are each encoded as one decimal digit. Pattern: '^([0-9]E-[0-9])$' Examples: Packer Error Rate 4x10-6 shall be encoded as "4E-6". Packer Error Rate 10-2 shall be encoded as "1E-2". |
| PacketErrRateRm | string | This data type is defined in the same way as the "PacketErrRate" data type, but with the OpenAPI "nullable: true" property. |
| PacketLossRate | integer | Unsigned integer indicating Packet Loss Rate (see clauses 5.7.2.8 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in tenth of percent. Minimum = 0. Maximum = 1000. |
| PacketLossRateRm | integer | This data type is defined in the same way as the "PacketLossRate" data type, but with the OpenAPI "nullable: true" property. |
| AverWindow | integer | Unsigned integer indicating Averaging Window (see clause 5.7.3.6 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in milliseconds. Minimum = 1. Maximum = 4095. Default = 2000.. |
| AverWindowRm | integer | This data type is defined in the same way as the "AverWindow" data type, but with the OpenAPI "nullable: true" property. |
| MaxDataBurstVol | integer | Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in Bytes. Minimum = 1. Maximum = 4095. |
| MaxDataBurstVolRm | integer | This data type is defined in the same way as the "MaxDataBurstVol" data type, but with the OpenAPI "nullable: true" property. |
| SamplingRatio | integer | Unsigned integer indicating Sampling Ratio (see clauses 4.15.1 of 3GPP TS 23.502 [28], expressed in percent. Minimum = 1. Maximum = 100 |
| SamplingRatioRM | integer | This data type is defined in the same way as the "SamplingRatio" data type, but with the OpenAPI "nullable: true" property. |
| RgWirelineCharacteristics | Bytes | RG Level Wireline Access Characteristics (see BBF TR-456 [41] and BBF TR-470 [37]). It shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64 encoded characters, representing the RG-Level Wireline Access Characteristics encoded as specified in clause 7.5 of BBF TR-470 [37]. |
| RgWirelineCharacteristicsRm | Bytes | This data type is defined in the same way as the "RgWirelineCharacteristics" data type, but with the OpenAPI "nullable: true" property. |
| ExtMaxDataBurstVol | integer | Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in Bytes. Minimum = 4096. Maximum = 2000000. |
| ExtMaxDataBurstVolRm | integer | This data type is defined in the same way as the "ExtMaxDataBurstVol" data type, but with the OpenAPI "nullable: true" property. |
| ExtPacketDelBudget | integer | Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501 [8])), expressed in 0.01 milliseconds. Minimum = 1. |
| ExtPacketDelBudgetRm | integer | This data type is defined in the same way as the "ExtPacketDelBudget" data type, but with the OpenAPI "nullable: true" property. |
| Metadata | string | This datatype contains information that is transparently passed to UPF and the UPF provides it to the service functions in N6-LAN. When present, this IE shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the Metadata. |
| NOTE: The prefixes used for bit rate unit "bps", packet rate unit "pps" and traffic volume in byte unit "B" shall be taken as x1000 multipliers and were meant to follow the standard symbols from "The International System of Units" (https://www.bipm.org/en/measurement-units/si-prefixes). However, even when the standard symbol for 10^3 multiplier is "k", in the present specification it has been defined as "K", and has been kept as such due to backwards-compatibility with earlier versions of this specification. |  |  |

### 5.5.3	Enumerations

#### 5.5.3.1	Enumeration: PreemptionCapability

The enumeration PreemptionCapability indicates the pre-emption capability of a request on other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501 [8]. It shall comply with the provisions defined in table 5.5.3.1-1.

Table 5.5.3.1-1: Enumeration PreemptionCapability

| Enumeration value | Description |
| --- | --- |
| "NOT_PREEMPT" | Shall not trigger pre-emption. |
| "MAY_PREEMPT" | May trigger pre-emption. |

#### 5.5.3.2	Enumeration: PreemptionVulnerability

The enumeration PreemptionVulnerability indicates the pre-emption vulnerability of the QoS flow to pre-emption from other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501 [8]. It shall comply with the provisions defined in table 5.5.3.2-1.

Table 5.5.3.2-1: Enumeration PreemptionVulnerability

| Enumeration value | Description |
| --- | --- |
| "NOT_PREEMPTABLE" | Shall not be pre-empted. |
| "PREEMPTABLE" | May be pre-empted. |

#### 5.5.3.3	Enumeration: ReflectiveQosAttribute

The enumeration ReflectiveQosAttribute indicates whether certain traffic of the QoS flow may be subject to Reflective QoS (see clause 5.7.2.3 of 3GPP TS 23.501 [8]). It shall comply with the provisions defined in table 5.5.3.3-1.

Table 5.5.3.3-1: Enumeration ReflectiveQosAttribute

| Enumeration value | Description |
| --- | --- |
| "RQOS" | Certain traffic of the Qos flow may be subject to Reflective QoS. |
| "NO_RQOS" | Traffic of the Qos flow is not subject to Reflective QoS. |

#### 5.5.3.4	Void

#### 5.5.3.5	Enumeration: NotificationControl

The enumeration NotificationControl indicates whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS Flow during the lifetime of the QoS Flow (see clause 5.7.2.4 of 3GPP TS 23.501 [8]). It shall comply with the provisions defined in table 5.5.3.5-1.

Table 5.5.3.5-1: Enumeration NotificationControl

| Enumeration value | Description |
| --- | --- |
| "REQUESTED" | Notifications are requested from the RAN. |
| "NOT_REQUESTED" | Notifications are not requested from the RAN. |

#### 5.5.3.6	Enumeration: QosResourceType

The enumeration QosResourceType indicates whether a QoS Flow is non-GBR, delay critical GBR, or non-delay critical GBR (see clauses 5.7.3.4 and 5.7.3.5 of 3GPP TS 23.501 [8]). It shall comply with the provisions defined in table 5.5.3.6-1.

Table 5.5.3.6-1: Enumeration QosResourceType

| Enumeration value | Description |
| --- | --- |
| "NON_GBR" | Non-GBR QoS Flow. |
| "NON_CRITICAL_GBR" | Non-delay critical GBR QoS flow. |
| "CRITICAL_GBR" | Delay critical GBR QoS flow. |

#### 5.5.3.7	Enumeration: PreemptionCapabilityRm

This enumeration is defined in the same way as the "PreemptionCapability" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.8	Enumeration: PreemptionVulnerabilityRm

This enumeration is defined in the same way as the "PreemptionVulnerability" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.9	Enumeration: ReflectiveQosAttributeRm

This enumeration is defined in the same way as the "ReflectiveQosAttribute" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.10	Enumeration: NotificationControlRm

This enumeration is defined in the same way as the "NotificationControl" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.11	Enumeration: QosResourceTypeRm

This enumeration is defined in the same way as the "QosResourceType" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.12	Enumeration: AdditionalQosFlowInfo

The enumeration AdditionalQosFlowInfo provides additional QoS flow information (see clause 9.3.1.12 3GPP TS 38.413 [11]). It shall comply with the provisions defined in table 5.5.3.12-1.

Table 5.5.3.12-1: Enumeration AdditionalQosFlowInfo

| Enumeration value | Description |
| --- | --- |
| "MORE_LIKELY" | Traffic for the QoS flow is likely to appear more often than traffic for other flows established for the PDU session. |

#### 5.5.3.13	Enumeration: PartitioningCriteria

The enumeration PartitioningCriteria indicates criteria for grouping the UEs (see clause 4.15.1 of 3GPP TS 23.502 [28]). It shall comply with the provisions defined in table 5.5.3.13-1.

Table 5.5.3.13-1: Enumeration PartitioningCriteria

| Enumeration value | Description |
| --- | --- |
| "TAC" | Type Allocation Code |
| "SUBPLMN" | Subscriber PLMN ID |
| "GEOAREA" | Geographical area |
| "SNSSAI" | S-NSSAI |
| "DNN" | DNN |

#### 5.5.3.14	Enumeration: PartitioningCriteriaRm

This enumeration is defined in the same way as the "PartitioningCriteria" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.15	Enumeration: PduSetHandlingInfo

The enumeration PduSetHandlingInfo indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. It shall comply with the provisions defined in table 5.5.3.15-1.

Table 5.5.3.15-1: Enumeration PduSetHandlingInfo

| Enumeration value | Description |
| --- | --- |
| "ALL_PDUS_NEEDED" | All PDUs of the PDU Set are needed. |
| "ALL_PDUS_NOT_NEEDED" | All PDUs of the PDU Set are not needed. |

#### 5.5.3.16	Enumeration: MediaTransportProto

Table 5.5.3.16-1: Enumeration MediaTransportProto

| Enumeration value | Description |
| --- | --- |
| "RTP" | Real-time Transport Protocol (see IETF RFC 3550 [61]) |
| "SRTP" | Secure Real-time Transport Protocol (see IETF RFC 3711 [62]) |
| "MOQT" | Media Over QUIC Transport protocol (see IETF draft-ietf-moq-transport [67]). |

#### 5.5.3.17	Enumeration: RtpHeaderExtType

Table 5.5.3.17-1: Enumeration RtpHeaderExtType

| Enumeration value | Description |
| --- | --- |
| "PDU_SET_MARKING" | RTP header extension for PDU Set Marking (see clause 4.4.2 of 3GPP TS 26.522 [59]) |
| "DYN_CHANGING_TRAFFIC_CHAR" | RTP header extension for Dynamically changing traffic characteristics (see clause 4.5 of 3GPP TS 26.522 [59]) |
| "EXPEDITED_TRANSFER_IND" | RTP header extension for Expedited Transfer Indication (see clause 4.7 of 3GPP TS 26.522 [59]) |

#### 5.5.3.18	Enumeration: RtpPayloadFormat

Table 5.5.3.18-1: Enumeration RtpPayloadFormat

| Enumeration value | Description |
| --- | --- |
| "H264" | RTP payload format for H.264/AVC codec (see IETF RFC 6184 [63]) |
| "H265" | RTP payload format for H.265/HEVC codec (see IETF RFC 7798 [64]) |

#### 5.5.3.19	Enumeration: MediaTransportProtoRm

This data type is defined in the same way as the "MediaTransportProto" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.3.20	Enumeration: RtpHeaderExtTypeRm

This data type is defined in the same way as the "RtpHeaderExtType" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.3.21	Enumeration: RtpPayloadFormatRm

This data type is defined in the same way as the "RtpPayloadFormat" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.3.22	Enumeration: PduSetHandlingInfoRm

This enumeration is defined in the same way as the "PduSetHandlingInfo" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.5.3.23	Enumeration: MriTransferMethod

Table 5.5.3.23-1: Enumeration MriTransferMethod

| Enumeration value | Description |
| --- | --- |
| "UDP_OPTION" | Media related information is transferred using the UDP Option method. |

#### 5.5.3.24	Enumeration: MriTransferMethodRm

This data type is defined in the same way as the "MriTransferMethod" data type, but with the OpenAPI "nullable: true" property.

### 5.5.4	Structured Data Types

#### 5.5.4.1	Type: Arp

Table 5.5.4.1-1: Definition of type Arp

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| priorityLevel | ArpPriorityLevel | M | 1 | Defines the relative importance of a resource request. |
| preemptCap | PreemptionCapability | M | 1 | Defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level. |
| preemptVuln | PreemptionVulnerability | M | 1 | Defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level. |

#### 5.5.4.2	Type: Ambr

Table 5.5.4.2-1: Definition of type Ambr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uplink | BitRate | M | 1 | AMBR for uplink |
| downlink | BitRate | M | 1 | AMBR for downlink |

#### 5.5.4.3	Type: Dynamic5Qi

Table 5.5.4.3-1: Definition of type Dynamic5Qi

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resourceType | QosResourceType | M | 1 | Defines the 5QI resource type. See clause 5.5.3.6. |  |
| priorityLevel | 5QiPriorityLevel | M | 1 | Defines the 5QI Priority Level. See clause 5.5.2. |  |
| packetDelayBudget | PacketDelBudget | M | 1 | Defines the packet delay budget. See clause 5.5.2. See NOTE 3. |  |
| packetErrRate | PacketErrRate | M | 1 | Defines the packet error rate. See clause 5.5.2. |  |
| averWindow | AverWindow | C | 0..1 | Defines the averaging window. See clause 5.5.2. This IE shall be present only for a GBR QoS flow or a Delay Critical GBR QoS flow. |  |
| maxDataBurstVol | MaxDataBurstVol | C | 0..1 | Defines the maximum data burst volume. See clause 5.5.2. See NOTE 1, NOTE 2. This IE shall be present for a Delay Critical GBR QoS flow. |  |
| extMaxDataBurstVol | ExtMaxDataBurstVol | C | 0..1 | Defines the maximum data burst volume. See clause 5.5.2. See NOTE 1, NOTE 2. |  |
| extPacketDelBudget | ExtPacketDelBudget | O | 0..1 | Defines the packet delay budget. See clause 5.5.2. See NOTE 3. |  |
| cnPacketDelayBudgetDl | ExtPacketDelBudget | O | 0..1 | Defines the Core Network Packet Delay Budget for downlink. See clause 5.5.2. |  |
| cnPacketDelayBudgetUl | ExtPacketDelBudget | O | 0..1 | Defines the Core Network Packet Delay Budget for uplink. See clause 5.5.2. |  |
| NOTE 1: Unless specified otherwise in an API: if the maximum data burst volume value to be transmitted is lower than or equal to 4095 Bytes, the maxDataBurst Vol IE shall be set to the maximum data burst volume value to be transmitted and the extMaxDataBurstVol IE shall be omitted. If the maximum data burst volume value to be transmitted is greater than 4095 Bytes, the maxDataBurst Vol IE shall be set to 4095 Bytes and, if ExtMaxDataBurstVol data type is supported by the sender, the extMaxDataBurstVol IE shall be set to the maximum data burst volume value to be transmitted. NOTE 2: Unless specified otherwise in an API: if both the maxDataBurstVol IE and the extMaxDataBurstVol IE are received, the value in the extMaxDataBurstVol IE shall be used if the receiver supports ExtMaxDataBurstVol data type, otherwise the value in the maxDataBurstVol IE shall be used. NOTE 3: Unless specified otherwise in an API: if both the packetDelayBudget IE and the extPacketDelBudget IE are received, the value in the extPacketDelBudget IE shall be used if the receiver supports ExtPacketDelBudget data type, otherwise the value in the packetDelayBudget IE shall be used. |  |  |  |  |  |

#### 5.5.4.4	Type: NonDynamic5Qi

Table 5.5.4.4-1: Definition of type NonDynamic5Qi

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| priorityLevel | 5QiPriorityLevel | O | 0..1 | Defines the 5QI Priority Level. See clause 5.5.2. When present, it contains the 5QI Priority Level value that overrides the standardized or pre-configured value. |  |
| averWindow | AverWindow | O | 0..1 | Defines the averaging window. See clause 5.5.2. This IE may be present for a GBR QoS flow or a Delay Critical GBR QoS flow. When present, it contains the Averaging Window that overrides the standardized or pre-configured value. |  |
| maxDataBurstVol | MaxDataBurstVol | O | 0..1 | Defines the maximum data burst volume. See clause 5.5.2. This IE may be present for a Delay Critical GBR QoS flow. When present, it contains the Maximum Data Burst Volume value that overrides the standardized or pre-configured value. See NOTE 1, NOTE 2. |  |
| extMaxDataBurstVol | ExtMaxDataBurstVol | C | 0..1 | Defines the maximum data burst volume. See clause 5.5.2. This IE may be present for a Delay Critical GBR QoS flow. When present, it contains the Maximum Data Burst Volume value that overrides the standardized or pre-configured value See NOTE 1, NOTE 2. |  |
| cnPacketDelayBudgetDl | ExtPacketDelBudget | O | 0..1 | Defines the Core Network Packet Delay Budget for downlink. See clause 5.5.2. |  |
| cnPacketDelayBudgetUl | ExtPacketDelBudget | O | 0..1 | Defines the Core Network Packet Delay Budget for uplink. See clause 5.5.2. |  |
| NOTE 1: Unless specified otherwise in an API: if the maximum data burst volume value to be transmitted is lower than or equal to 4095 Bytes, the maxDataBurst Vol IE shall be set to the maximum data burst volume value to be transmitted and the extMaxDataBurstVol IE shall be omitted. If the maximum data burst volume value to be transmitted is greater than 4095 Bytes, the maxDataBurst Vol IE shall be set to 4095 Bytes and, if ExtMaxDataBurstVol data type is supported by the sender, the extMaxDataBurstVol IE shall be set to the maximum data burst volume value to be transmitted. NOTE 2: Unless specified otherwise in an API: if both the maxDataBurstVol IE and the extMaxDataBurstVol IE are received, the value in the extMaxDataBurstVol IE shall be used if the receiver supports ExtMaxDataBurstVol data type, otherwise the value in the maxDataBurstVol IE shall be used. |  |  |  |  |  |

#### 5.5.4.5	Type: ArpRm

This data type is defined in the same way as the "Arp" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.4.6	Type: AmbrRm

This data type is defined in the same way as the "Ambr" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.4.7	Void

#### 5.5.4.8	Void

#### 5.5.4.9	Type: SliceMbr

Table 5.5.4.9-1: Definition of type SliceMbr

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| uplink | BitRate | M | 1 | MBR for uplink |
| downlink | BitRate | M | 1 | MBR for downlink |

#### 5.5.4.10	Type: SliceMbrRm

This data type is defined in the same way as the "SliceMbr" data type, but with the OpenAPI "nullable: true" property.

#### 5.5.4.11	Type: PduSetQosPara

Table 5.5.4.11-1: Definition of type PduSetQosPara

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pduSetDelayBudget | ExtPacketDelBudget | C | 0..1 | Indicates the PDU Set Delay Budget (PSDB) (see clause 5.7.7.2 of 3GPP TS 23.501 [8]). (NOTE) |  |
| pduSetErrRate | PacketErrRate | C | 0..1 | Indicates the PDU Set Error Rate (PSER) (see clause 5.7.7.3 3GPP TS 23.501 [8]). (NOTE) |  |
| pduSetHandlingInfo | PduSetHandlingInfo | C | 0..1 | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. (NOTE) |  |
| NOTE: At least one of the following shall be present: 1) pduSetHandlingInfo and/or 2) both pduSetDelayBudget and pduSetErrRate. |  |  |  |  |  |

#### 5.5.4.12	Type: PduSetQosParaRm

This data type is defined in the same way as the "PduSetQosPara" data type, but with the OpenAPI "nullable: true" property and with the "pduSetDelayBudget" attribute with the removable data type "ExtPacketDelBudgetRm", the "pduSetErrRate" attribute with the removable data type "PacketErrRateRm" and the "pduSetHandlingInfo" attribute with the removable data type "PduSetHandlingInfoRm".

Table 5.5.4.12-1: Definition of type PduSetQosParaRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pduSetDelayBudget | ExtPacketDelBudgetRm | C | 0..1 | Indicates the PDU Set Delay Budget (PSDB) (see clause 5.7.7.2 of 3GPP TS 23.501 [8]). (NOTE 1, NOTE 2) |
| pduSetErrRate | PacketErrRateRm | C | 0..1 | Indicates the PDU Set Error Rate (PSER) (see clause 5.7.7.3 3GPP TS 23.501 [8]). (NOTE 1, NOTE 2) |
| pduSetHandlingInfo | PduSetHandlingInfoRm | C | 0..1 | Indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. (NOTE 2) |
| NOTE 1: The pduSetDelayBudget and pduSetErrRate shall only be removed at the same time. NOTE 2: When the PduSetQosParaRm is used to initially provision the PDU Set QoS parameters the PduSetQosParamRm shall contain at least one of the following: 1) pduSetHandlingInfo and/or 2) both pduSetDelayBudget and pduSetErrRate. When the PduSetQosParaRm is used to modify the previously provisioned value(s), only the attribute(s) to be modified shall be present, e.g. only the pduSetErrRate is present when the value of the pduSetErrRate is to be changed but there is no change for the pduSetDelayBudget and the pduSetHandlingInfo attributes. |  |  |  |  |

.

#### 5.5.4.13	Type ProtocolDescription

Table 5.5.4.13-1: Definition of type ProtocolDescription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| transportProto | MediaTransportProto | O | 0..1 | When present, this IE shall indicate the transport protocol used by the media flow. |
| rtpHeaderExtInfo | RtpHeaderExtInfo | C | 0..1 | This IE shall be present if RTP or SRTP is used and the RTP payload packets contains a RTP Header Extension that can be used for PDU Set identification, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. When present, this IE shall contain information on the RTP header extension that can be used for PDU Set identification, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE 1) (NOTE 4) |
| addRtpHeaderExtInfo | array(RtpHeaderExtInfo) | O | 1..N | This IE may be present if the rtpHeaderExtInfo IE is present. When present, it shall contain information on additional RTP header extensions that can be used for PDU Set identification, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE 4) |
| rtpPayloadInfoList | array(RtpPayloadInfo) | O | 1..N | When present, it shall contain RTP Payload information for the RTP stream, which can be used to derive the PDU Set information, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE 1) (NOTE 2) |
| mriTransferInfo | MriTransferInfo | O | 0..1 | This IE may be present, for end-to-end encrypted traffic, to indicate how media related information is transferred. |
| NOTE 1: If the rtpPayloadInfoList is present and contains one or more Payload Type values, the UPF may only parse the RTP packets with an RTP header containing any of these Payload Type value(s). Otherwise, if the rtpPayloadInfoList is absent or does not contain any Payload Type value, the UPF should parseall the RTP packets of the media flow and use either the RTP Header Extension if included, or the Payload format to derive the PDU set information (see Guidelines for PDU Set identification in clauses A.1 and A.2 of 3GPP TS 26.522 [59]). NOTE 2: In this release of the specification, the rtpPayloadInfoList contains only one RtpPayloadInfo element. NOTE 3: Vendor/operator specific attributes may be supported as defined in clause 6.6.3 of 3GPP TS 29.500 [25]. NOTE 4: If at least two RTP header extensions are signaled in the ProtocolDescription, the RTP header extension for PDU Set identification and/or End of Data Burst marking (if any) should be signaled in the rtpHeaderExtInfo attribute (implementations complying with an earlier release of the specification may support PDU set identification and/or End of Data Burst marking without supporting the addRtpHeaderExtInfo attribute). |  |  |  |  |

EXAMPLE 1:	For a media flow using RTP transport with:
- the RTP Header Extension for PDU Set Marking (see clause 4.4.2 of 3GPP TS 26.522 [59]);
- the RTP header extension Id "3";
- RTP packets with different PTs, where packets with PT 96 contain the RTP Header Extension,

the Protocol Description is set to:

{ "transportProto": "RTP", "rtpHeaderExtInfo": { "rtpHeaderExtType": "PDU_SET_MARKING", "rtpHeaderExtId": 3}, "rtpPayloadInfoList": [{ "rtpPayloadTypeList": [ 96 ]}]}

EXAMPLE 2:	For a media flow using RTP transport:
- not using any RTP Header Extension for PDU Set identication;
- H.265 payload format with Payload Types 96 and 97 (see clause A.2.3 (RTP with HEVC payload format) of 3GPP TS 26.522 [59]);

the Protocol Description is set to:

{ "transportProto": "RTP", "rtpPayloadInfoList": [{"rtpPayloadFormat": "H265", "rtpPayloadTypeList": [96, 97]}]}

EXAMPLE 3:	For a media flow using RTP transport with:
- the RTP Header Extension for PDU Set Marking (see clause 4.4.2 of 3GPP TS 26.522 [59]);
- the RTP Header Extension for Dynamically changing traffic characteristics (see clause 4.5 of 3GPP TS 26.522 [59]);
- the RTP header extension Id "3" and "4" for the above RTP header extensions respectively;
- RTP packets with different PTs, where packets with PT 96 contain the RTP Header Extensions,

the Protocol Description is set to:

{ "transportProto": "RTP", "rtpHeaderExtInfo": { "rtpHeaderExtType": "PDU_SET_MARKING", "rtpHeaderExtId": 3}, "addRtpHeaderExtInfo": [{ "rtpHeaderExtType": "DYN_CHANGING_TRAFFIC_CHAR", "rtpHeaderExtId": 4}], "rtpPayloadInfoList": [{ "rtpPayloadTypeList": [ 96 ]}]}

#### 5.5.4.13A	Type ProtocolDescriptionRm

Describes the modifications to the "ProtocolDescription" data type. This data type is defined in the same way as the "ProtocolDescription" data type, but:

-	with the OpenAPI "nullable: true" property;

-	the removable attributes "transportProto", "rtpHeaderExtInfo" and "mriTransferInfo" with the removable data types "MediaTransportProtoRm", RtpHeaderExtInfoRm" and "MriTransferInfoRm" respectively; and

-	the removalble attribute "rtpPayloadInfoList" with the OpenAPI "nullable: true" property.

Table 5.5.4.13A-1: Definition of type ProtocolDescriptionRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| transportProto | MediaTransportProtoRm | O | 0..1 | When present, this IE shall indicate the transport protocol used by the media flow. |
| rtpHeaderExtInfo | RtpHeaderExtInfoRm | O | 0..1 | When present, this IE shall contain information on the RTP header extension that can be used for PDU Set identification, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking, and/or Expedited Transfer Indication marking. |
| addRtpHeaderExtInfo | array(RtpHeaderExtInfo) | O | 1..N | This IE may be present if the rtpHeaderExtInfo IE is present. When present, it shall contain information on additional RTP header extensions that can be used for PDU Set identification, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE) |
| rtpPayloadInfoList | array(RtpPayloadInfo) | O | 1..N | When present, it shall contain RTP Payload information for the RTP stream, which can be used to derive the PDU Set information, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking, and/or Expedited Transfer Indication marking. |
| mriTransferInfo | MriTransferInfoRm | O | 0..1 | This IE may be present, for end-to-end encrypted traffic, to indicate how media related information is transferred. |
| NOTE: If at least two RTP header extensions are signaled in the ProtocolDescriptionRm, the RTP header extension for PDU Set identification and/or End of Data Burst marking (if any) should be signaled in the rtpHeaderExtInfo attribute (implementations complying with an earlier release of the specification may support PDU set identification and/or End of Data Burst marking without supporting the addRtpHeaderExtInfo attribute). |  |  |  |  |

#### 5.5.4.14	Type RtpHeaderExtInfo

Table 5.5.4.14-1: Definition of type RtpHeaderExtInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rtpHeaderExtType | RtpHeaderExtType | C | 0..1 | This IE shall be present if RTP or SRTP is used and the RTP payload packets contains a RTP Header Extension that can be used for: - PDU Set identification and/or End of Data Burst marking; - Data Burst Size marking and/or Time to Next Burst marking; or - Expedited Transfer Indication marking. When present, it shall indicate the RTP header extension type. |
| rtpHeaderExtId | integer | C | 0..1 | Integer between and including 1 and 255. This IE shall be present if the rtpHeaderExtType IE is present. When present, the rtpHeaderExtId shall be set to the Id of the RTP header extension identified by the rtpHeaderExtType IE, as defined in IETF RFC 8285 [60]. |
| longFormat | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING", "DYN_CHANGING_TRAFFIC_CHAR" or "EXPEDITED_TRANSFER_IND". When present, this IE shall indicate if a short or a long header extension format is used. true: 2-byte (long) format is used false: 1-byte (short) format is used The absence of this IE means that this information is not known. |
| pduSetSizeActive | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING". When present, this IE shall indicate if the PDU Set size in bytes is present in the RTP header extension of every RTP packet. true: PDU Set size is present false: PDU Set size is not present The absence of this IE means that this information is not known. |
| pduSetPduCountActive | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING". When present, this IE shall indicate if the number of PDUs of a PDU Set is present in the RTP header extension of every RTP packet. true: Number of PDUs of PDU Set is present false: Number of PDUs of PDU Set is not present The absence of this IE means that this information is not known. |
| NOTE: Vendor/operator specific attributes may be supported as defined in clause 6.6.3 of 3GPP TS 29.500 [25]. |  |  |  |  |

#### 5.5.4.14A	Type RtpHeaderExtInfoRm

Describes the modifications to "RtpHeaderExtInfo" data type. This data type is defined in the same way as the "RtpHeaderExtInfo" data type, but:

-	with the OpenAPI "nullable: true" property;

-	the removable attribute "rtpHeaderExtType" with the removable data type "RtpHeaderExtTypeRm"; and

-	the removalble attributes "rtpHeaderExtId", "longFormat" and "pduSetSizeActive" with the OpenAPI "nullable: true" property.

Table 5.5.4.14A-1: Definition of type RtpHeaderExtInfoRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rtpHeaderExtType | RtpHeaderExtTypeRm | O | 0..1 | When present, it shall indicate the RTP header extension type. |
| rtpHeaderExtId | integer | C | 0..1 | Integer between and including 1 and 255. This IE shall be present if the rtpHeaderExtType IE is present. When present, the rtpHeaderExtId shall be set to the Id of the RTP header extension identified by the rtpHeaderExtType IE, as defined in IETF RFC 8285 [60]. |
| longFormat | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING", "DYN_CHANGING_TRAFFIC_CHAR" or "EXPEDITED_TRANSFER_IND". When present, this IE shall indicate if a short or a long header extension format is used. true: 2-byte (long) format is used false: 1-byte (short) format is used |
| pduSetSizeActive | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING". When present, this IE shall indicate if the PDU Set size in bytes is present in the RTP header extension of every RTP packet. true: PDU Set size is present false: PDU Set size is not present |
| pduSetPduCountActive | boolean | O | 0..1 | This IE may be present if the rtpHeaderExtType is set to "PDU_SET_MARKING". When present, this IE shall indicate if the number of PDUs of a PDU Set is present in the RTP header extension of every RTP packet. true: Number of PDUs of PDU Set is present false: Number of PDUs of PDU Set is not present The absence of this IE means that this information is not known. |
| NOTE: Vendor/operator specific attributes may be supported as defined in clause 6.6.3 of 3GPP TS 29.500 [25]. |  |  |  |  |

#### 5.5.4.15	Type RtpPayloadInfo

Table 5.5.4.15-1: Definition of type RtpPayloadInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rtpPayloadTypeList | array(integer) | C | 1..N | Integer between and including 0 and 127. This IE shall be present when the rtpPayloadFormat is present, otherwise it may be present. When present, this IE shall contain the list of Payload Type (PT) values in the RTP header of RTP packets the UPF may parse to derive the PDU Set Information, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE) |
| rtpPayloadFormat | RtpPayloadFormat | O | 0..1 | When present, it shall indicate the RTP Payload format as defined in 3GPP TS 26.522 [59]. (NOTE) |
| NOTE: The rtpPayloadType(s) shall correspond to the RTP Payload Format if the rtpPayloadFormat is present. |  |  |  |  |

#### 5.5.4.16	Type RtpPayloadInfoRm

Describes the modifications to the "ProtocolDescription" data type. This data type is defined in the same way as the "RtpPayloadInfo" data type, but:

-	with the OpenAPI "nullable: true" property;

-	the removable attribute "rtpPayloadFormat" with the removable data type "RtpPayloadFormatRm"; and

-	the removalble attribute "rtpPayloadTypeList" with the OpenAPI "nullable: true" property.

Table 5.5.4.16-1: Definition of type RtpPayloadInfoRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rtpPayloadTypeList | array(integer) | O | 1..N | Integer between and including 0 and 127. When present, this IE shall contain the list of Payload Type (PT) values in the RTP header of RTP packets the UPF may parse to derive the PDU Set Information, End of Data Burst marking, Data Burst Size marking, Time to Next Burst marking and/or Expedited Transfer Indication marking. (NOTE) |
| rtpPayloadFormat | RtpPayloadFormatRm | O | 0..1 | When present, it shall indicate the RTP Payload format as defined in 3GPP TS 26.522 [59]. (NOTE) |
| NOTE: The rtpPayloadType(s) shall correspond to the RTP Payload Format if the rtpPayloadFormat is present. |  |  |  |  |

#### 5.5.4.17	Type MriTransferInfo

Table 5.5.4.17-1: Definition of type MriTransferInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| transferMethod | MriTransferMethod | O | 0..1 | Method for transferring media related information, e.g. using the UDP Option method. |

#### 5.5.4.18	Type MriTransferInfoRm

Describes the modifications to the "MriTransferInfo" data type. This data type is defined in the same way as the "MriTransferInfo" data type, but:

-	with the OpenAPI "nullable: true" property; and

-	the removable attribute "transferMethod" with the removable data type "MriTransferMethodRm".

Table 5.5.4.18-1: Definition of type MriTransferInfoRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| transferMethod | MriTransferMethodRm | O | 0..1 | Method for transferring media related information, e.g. using the UDP Option method. |

## 5.6	Data Types related to 5G Trace

### 5.6.1	Introduction

This clause defines common data types related to 5G Trace.

### 5.6.2	Simple Data Types

This clause specifies common simple data types.

Table 5.6.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| PhysCellId | integer | integer value identifying the physical cell identity (PCI), as definition of "PhysCellId" IE in clause 6.3.2 of 3GPP TS 38.331 [42]. Minimum = 0. Maximum = 1007. |
| ArfcnValueNR | integer | Integer value indicating the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as definition of "ARFCN-ValueNR" IE in clause 6.3.2 of 3GPP TS 38.331 [42]. Minimum = 0. Maximum = 3279165. |
| QoeReference | string | String containing MCC (3 digits), MNC (2 or 3 digits) and QMC ID (3 octets, encoded as 6 hexadecimal digits). Each value is separated by the "-" character. See 3GPP TS 28.405 [56], clause 5.2. Pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{6}$' |
| MdtAlignmentInfo | string | String containing: - Trace Reference: MCC (3 digits), MNC (2 or 3 digits), Trace ID (3 octets, encoded as 6 hexadecimal digits) - Trace Recording Session Reference (2 octets, encoded as 4 hexadecimal digits). Each value is separated by the "-" character. See 3GPP TS 28.405 [56], clause 5.13. Pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{6}-[A-Fa-f0-9]{4}$' |

### 5.6.3	Enumerations

#### 5.6.3.1	Enumeration: TraceDepth

The enumeration TraceDepth defines how detailed information should be recorded in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.1-1.

Table 5.6.3.1-1: Enumeration TraceDepth

| Enumeration value | Description |
| --- | --- |
| "MINIMUM" | Minimum |
| "MEDIUM" | Medium |
| "MAXIMUM" | Maximum |
| "MINIMUM_WO_VENDOR_EXTENSION" | Minimum without vendor specific extension |
| "MEDIUM_WO_VENDOR_EXTENSION" | Medium without vendor specific extension |
| "MAXIMUM_WO_VENDOR_EXTENSION" | Maximum without vendor specific extension |

#### 5.6.3.2	Enumeration: TraceDepthRm

This enumeration is defined in the same way as the "TraceDepth" enumeration, but with the OpenAPI "nullable: true" property.

#### 5.6.3.3	Enumeration: JobType

The enumeration JobType defines Job Type in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration JobType

| Enumeration value | Description |
| --- | --- |
| "IMMEDIATE_MDT_ONLY" | Immediate MDT only |
| "LOGGED_MDT_ONLY" | Logged MDT only |
| "TRACE_ONLY" | Trace only |
| "IMMEDIATE_MDT_AND_TRACE" | Immediate MDT and Trace |
| "LOGGED_MBSFN_MDT" | Logged MBSFN MDT |
| "5GC_UE_LEVEL_MEASUREMENTS_ONLY" | 5GC UE level measurements only |
| "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS_ONLY" | Trace and 5GC UE level measurements |
| "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" | Immediate MDT and 5GC UE level measurements |
| "TRACE_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" | Trace, Immediate MDT and 5GC UE level measurements |

#### 5.6.3.4	Enumeration: ReportTypeMdt

The enumeration ReportTypeMdt defines Report Type for logged MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.4-1.

Table 5.6.3.4-1: Enumeration ReportTypeMdt

| Enumeration value | Description |
| --- | --- |
| "PERIODICAL" | Periodical |
| "EVENT_TRIGGED" | Event triggered |

#### 5.6.3.5	Enumeration: MeasurementLteForMdt

The enumeration MeasurementLteForMdt defines Measurements used for MDT in LTE in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.5-1.

Table 5.6.3.5-1: Enumeration MeasurementLteForMdt

| Enumeration value | Description |
| --- | --- |
| "M1" | M1 |
| "M2" | M2 |
| "M3" | M3 |
| "M4_DL" | M4 for DL |
| "M4_UL" | M4 for UL |
| "M5_DL" | M5 for DL |
| "M5_UL" | M5 for UL |
| "M6_DL" | M6 for DL |
| "M6_UL" | M6 for UL |
| "M7_DL" | M7 for DL |
| "M7_UL" | M7 for UL |
| "M8" | M8 |
| "M9" | M9 |

#### 5.6.3.6	Enumeration: MeasurementNrForMdt

The enumeration MeasurementNrForMdt defines Measurements used for MDT in NR in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.6-1.

Table 5.6.3.6-1: Enumeration MeasurementNrForMdt

| Enumeration value | Description |
| --- | --- |
| "M1" | M1 |
| "M2" | M2 |
| "M3" | M3 |
| "M4_DL" | M4 for DL |
| "M4_UL" | M4 for UL |
| "M5_DL" | M5 for DL |
| "M5_UL" | M5 for UL |
| "M6_DL" | M6 for DL |
| "M6_UL" | M6 for UL |
| "M7_DL" | M7 for DL |
| "M7_UL" | M7 for UL |
| "M8" | M8 |
| "M9" | M9 |

#### 5.6.3.7	Enumeration: SensorMeasurement

The enumeration SensorMeasurement defines sensor measurement type for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.7-1.

Table 5.6.3.7-1: Enumeration SensorMeasurement

| Enumeration value | Description |
| --- | --- |
| "BAROMETRIC_PRESSURE" | Barometric pressure |
| "UE_SPEED" | UE speed |
| "UE_ORIENTATION" | UE orientation |

#### 5.6.3.8	Enumeration: ReportingTrigger

The enumeration ReportingTrigger defines Reporting Triggers for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.8-1.

Table 5.6.3.8-1: Enumeration ReportingTrigger

| Enumeration value | Description |
| --- | --- |
| "PERIODICAL" | Periodical |
| "EVENT_A2" | Event A2 for LTE and NR |
| "EVENT_A2_PERIODIC" | A2 event triggered periodic for LTE and NR |
| "ALL_RRM_EVENT_TRIGGERS" | All configured RRM event triggers for LTE |

#### 5.6.3.9	Enumeration: ReportIntervalMdt

The enumeration ReportIntervalMdt defines Report Interval for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.9-1.

Table 5.6.3.9-1: Enumeration ReportIntervalMdt

| Enumeration value | Description |
| --- | --- |
| "120" | 120 ms |
| "240" | 240 ms |
| "480" | 480 ms |
| "640" | 640 ms |
| "1024" | 1024 ms |
| "2048" | 2048 ms |
| "5120" | 5120 ms |
| "10240" | 10240ms |
| "60000" | 1 min=60000 ms |
| "360000" | 6 min=360000 ms |
| "720000" | 12 min=720000 ms |
| "1800000" | 30 min=1800000 ms |
| "3600000" | 60 min=3600000 ms |

#### 5.6.3.10	Enumeration: ReportAmountMdt

The enumeration ReportAmountMdt defines Report Amount for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.10-1.

Table 5.6.3.10-1: Enumeration ReportAmountMdt

| Enumeration value | Description |
| --- | --- |
| "1" | 1 |
| "2" | 2 |
| "4" | 4 |
| "8" | 8 |
| "16" | 16 |
| "32" | 32 |
| "64" | 64 |
| "infinity" | Infinity |

#### 5.6.3.11	Enumeration: EventForMdt

The enumeration EventForMdt defines events triggered measurement for logged MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.11-1.

Table 5.6.3.11-1: Enumeration EventForMdt

| Enumeration value | Description |
| --- | --- |
| "OUT_OF_COVERAGE" | Out of coverage |
| "A2_EVENT" | A2 event |

#### 5.6.3.12	Enumeration: LoggingIntervalMdt

The enumeration LoggingIntervalMdt defines Logging Interval for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.12-1.

Table 5.6.3.12-1: Enumeration LoggingIntervalMdt

| Enumeration value | Description |
| --- | --- |
| "128" | 1280 ms |
| "256" | 2560 ms |
| "512" | 5120 ms |
| "1024" | 10240 ms |
| "2048" | 20480 ms |
| "3072" | 30720 ms |
| "4096" | 40960 ms |
| "6144" | 61440 ms |

#### 5.6.3.13	Enumeration: LoggingDurationMdt

The enumeration LoggingDurationMdt defines Logging Duration for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.13-1.

Table 5.6.3.13-1: Enumeration LoggingDurationMdt

| Enumeration value | Description |
| --- | --- |
| "600" | 600 sec |
| "1200" | 1200 sec |
| "2400" | 2400 sec |
| "3600" | 3600 sec |
| "5400" | 5400 sec |
| "7200" | 7200 sec |

#### 5.6.3.14	Enumeration: PositioningMethodMdt

The enumeration PositioningMethodMdt defines Positioning Method for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.14-1.

Table 5.6.3.14-1: Enumeration PositioningMethodMdt

| Enumeration value | Description |
| --- | --- |
| "GNSS" | GNSS |
| "E_CELL_ID" | E-Cell ID |

#### 5.6.3.15	Enumeration: CollectionPeriodRmmLteMdt

The enumeration CollectionPeriodRmmLteMdt defines Collection period for RRM measurements LTE for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.15-1.

Table 5.6.3.15-1: Enumeration CollectionPeriodRmmLteMdt

| Enumeration value | Description |
| --- | --- |
| "1024" | 1024 ms |
| "1280" | 1280 ms |
| "2048" | 2048 ms |
| "2560" | 2560 ms |
| "5120" | 5120 ms |
| "10240" | 10240 ms |
| "60000" | 1 min |

#### 5.6.3.16	Enumeration: MeasurementPeriodLteMdt

The enumeration MeasurementPeriodLteMdt defines Measurement period LTE for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.16-1.

Table 5.6.3.16-1: Enumeration MeasurementPeriodLteMdt

| Enumeration value | Description |
| --- | --- |
| "1024" | 1024 ms |
| "1280" | 1280 ms |
| "2048" | 2048 ms |
| "2560" | 2560 ms |
| "5120" | 5120 ms |
| "10240" | 10240 ms |
| "60000" | 1 min |

#### 5.6.3.17	Enumeration: ReportIntervalNrMdt

The enumeration ReportIntervalNrMdt defines Report Interval in NR for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.17-1.

Table 5.6.3.17-1: Enumeration ReportIntervalNrMdt

| Enumeration value | Description |
| --- | --- |
| "120" | 120 ms |
| "240" | 240 ms |
| "480" | 480 ms |
| "640" | 640 ms |
| "1024" | 1024 ms |
| "2048" | 2048 ms |
| "5120" | 5120 ms |
| "10240" | 10240ms |
| "20480" | 20480ms |
| "40960" | 40960ms |
| "60000" | 1 min=60000 ms |
| "360000" | 6 min=360000 ms |
| "720000" | 12 min=720000 ms |
| "1800000" | 30 min=1800000 ms |
| "3600000" | 60 min=3600000 ms |

#### 5.6.3.18	Enumeration: LoggingIntervalNrMdt

The enumeration LoggingIntervalNrMdt defines Logging Interval in NR for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.18-1.

Table 5.6.3.18-1: Enumeration LoggingIntervalNrMdt

| Enumeration value | Description |
| --- | --- |
| "1280" | 1280 ms |
| "2560" | 2560 ms |
| "5120" | 5120 ms |
| "10240" | 10240 ms |
| "20480" | 20480 ms |
| "30720" | 30720 ms |
| "40960" | 40960 ms |
| "61440" | 61440 ms |
| "320" | 320 ms |
| "640" | 640 ms |
| "infinity" | Infinity |

#### 5.6.3.19	Enumeration: CollectionPeriodRmmNrMdt

The enumeration CollectionPeriodRmmNrMdt defines Collection period for RRM measurements NR for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.19-1.

Table 5.6.3.19-1: Enumeration CollectionPeriodRmmNrMdt

| Enumeration value | Description |
| --- | --- |
| "1024" | 1024 ms |
| "2048" | 2048 ms |
| "5120" | 5120 ms |
| "10240" | 10240 ms |
| "60000" | 1 min |

#### 5.6.3.20	Enumeration: LoggingDurationNrMdt

The enumeration LoggingDurationMdt defines Logging Duration in NR for MDT in the trace. See 3GPP TS 32.422 [19] for further description of the values. It shall comply with the provisions defined in table 5.6.3.20-1.

Table 5.6.3.20-1: Enumeration LoggingDurationNrMdt

| Enumeration value | Description |
| --- | --- |
| "600" | 600 sec |
| "1200" | 1200 sec |
| "2400" | 2400 sec |
| "3600" | 3600 sec |
| "5400" | 5400 sec |
| "7200" | 7200 sec |

#### 5.6.3.21	Enumeration: QoeServiceType

The enumeration QoeServiceType indicates the kind of service that shall be recorded for QMC. It shall comply with the provisions defined in Table 5.6.3.21-1.

Table 5.6.3.21-1: Enumeration QoeServiceType

| Enumeration value | Description |
| --- | --- |
| "DASH" | Dynamic Adaptive Streaming over HTTP |
| "MTSI" | Multimedia Telephony Service for IMS |
| "VR" | Virtual Reality |

#### 5.6.3.22	Enumeration: AvailableRanVisibleQoeMetric

The enumeration AvailableRanVisibleQoeMetric indicates different available RAN-visible QoE metrics to the gNB. It shall comply with the provisions defined in Table 5.6.3.22-1.

Table 5.6.3.22-1: Enumeration AvailableRanVisibleQoeMetric

| Enumeration value | Description |
| --- | --- |
| "APPLICATION_LAYER_BUFFER_LEVEL_LIST" | See 3GPP TS 28.405 [56], clause 5.12. |
| "PLAYOUT_DELAY_FOR_MEDIA_STARTUP" | See 3GPP TS 28.405 [56], clause 5.12. |

#### 5.6.3.23	Enumeration: MeasurementType

The enumeration MeasurementType defines Measurement Type in the 5GC UE level measurements trace. See 3GPP TS 32.422 [19] and 3GPP TS 28.558 [66] for further description of the values.

Table 5.6.3.23-1: Enumeration MeasurementType

| Enumeration value | Description |
| --- | --- |
| "GTP_DELAYDLPSAUPFUEMEAN_SNSSAI_QFI" | GTP.DelayDlPsaUpfUeMean.SNSSAI.QFI. |
| "GTP_DELAYULPSAUPFUEMEANEXCD1_SNSSAI_QFI" | GTP.DelayUlPsaUpfUeMeanExcD1.SNSSAI.QFI. |
| "GTP_DELAYDLPSAUPFUEMEANINCD1_SNSSAI_QFI" | GTP.DelayDlPsaUpfUeMeanIncD1.SNSSAI.QFI. |
| "GTP_DELAYULPSAUPFNGRANMEAN_SNSSAI_QFI" | GTP.DelayUlPsaUpfNgranMean.SNSSAI.QFI. |
| "GTP_DELAYDLPSAUPFNGRANMEAN_SNSSAI_QFI" | GTP.DelayDlPsaUpfNgranMean.SNSSAI.QFI. |
| "GTP_RTTDELAYPSAUPFNGRANMEAN" | GTP.RttDelayPsaUpfNgranMean. |
| "GTP_RTTDELAYPSAUPFNGRANMEAN_SNSSAI" | GTP.RttDelayPsaUpfNgranMean_SNSSAI. |
| "GTP_RTTDELAYPSAUPFNGRANMEAN_QFI" | GTP.RttDelayPsaUpfNgranMean_QFI. |
| "GTP_RTTDELAYPSAUPFNGRANMEAN_SNSSAI_QFI" | GTP.RttDelayPsaUpfNgranMean_SNSSAI_QFI. |

#### 5.6.3.24	Enumeration: BluetoothRssi

The enumeration BluetoothRssi defines RSSI measurement by UE for Bluetooth. See 3GPP TS 38.413 [11] for further description of the values. It shall comply with the provisions defined in table 5.6.3.24-1.

Table 5.6.3.24-1: Enumeration BluetoothRssi

| Enumeration value | Description |
| --- | --- |
| "TRUE" | RSSI measurement by UE for Bluetooth is required. |

#### 5.6.3.25	Enumeration: WlanRssi

The enumeration WlanRssi defines RSSI measurement by UE for WLAN. See 3GPP TS 38.413 [11] for further description of the values. It shall comply with the provisions defined in table 5.6.3.25-1.

Table 5.6.3.25-1: Enumeration WlanRssi

| Enumeration value | Description |
| --- | --- |
| "TRUE" | RSSI measurement by UE for WLAN is required. |

#### 5.6.3.26	Enumeration: WlanRtt

The enumeration WlanRtt defines RTT measurement by UE for WLAN. See 3GPP TS 38.413 [11] for further description of the values. It shall comply with the provisions defined in table 5.6.3.26-1.

Table 5.6.3.26-1: Enumeration WlanRtt

| Enumeration value | Description |
| --- | --- |
| "TRUE" | RTT measurement by UE for WLAN is required. |

### 5.6.4	Structured Data Types

#### 5.6.4.1	Type: TraceData

Table 5.6.4.1-1: Definition of type TraceData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| traceRef | string | M | 1 | Trace Reference (see 3GPP TS 32.422 [19]). It shall be encoded as the concatenation of MCC, MNC and Trace ID as follows: <MCC><MNC>-<Trace ID> The Trace ID shall be encoded as a 3 octet string in hexadecimal representation. Each character in the Trace ID string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the Trace ID shall appear first in the string, and the character representing the 4 least significant bit of the Trace ID shall appear last in the string. Pattern: '^[0-9]{3}[0-9]{2,3}-[A-Fa-f0-9]{6}$' |
| traceDepth | TraceDepth | M | 1 | Trace Depth (see 3GPP TS 32.422 [19]). |
| neTypeList | string | M | 1 | List of NE Types (see 3GPP TS 32.422 [19]). It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string. Octets shall be coded according to 3GPP TS 32.422 [19]. Pattern: '^[A-Fa-f0-9]+$' |
| eventList | string | M | 1 | Triggering events (see 3GPP TS 32.422 [19]). It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string. Octets shall be coded according to 3GPP TS 32.422 [19]. Pattern: '^[A-Fa-f0-9]+$' |
| collectionEntityIpv4Addr | Ipv4Addr | C | 0..1 | IPv4 Address of the Trace Collection Entity (see 3GPP TS 32.422 [19]. At least one of the collectionEntityIpv4Addr or collectionEntityIpv6Addr attributes shall be present. |
| collectionEntityIpv6Addr | Ipv6Addr | C | 0..1 | IPv6 Address of the Trace Collection Entity (see 3GPP TS 32.422 [19]. At least one of the collectionEntityIpv4Addr or collectionEntityIpv6Addr attributes shall be present. |
| traceReportingConsumerUri | Uri | O | 0..1 | URI of the Trace Reporting Consumer if streaming based report is supported (see 3GPP TS 32.422 [19]). If both the IP Address of the Trace Collection Entity and URI of the Trace Reporting Consumer are present, the URI of the Trace Reporting Consumer shall take the higher precedence. |
| interfaceList | string | O | 0..1 | List of Interfaces (see 3GPP TS 32.422 [19]). It shall be encoded as an octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string. Octets shall be coded according to 3GPP TS 32.422 [19]. If this attribute is not present, all the interfaces applicable to the list of NE types indicated in the neTypeList attribute should be traced. Pattern: '^[A-Fa-f0-9]+$' |
| jobType | JobType | C | 0..1 | This IE shall be present if it is available. When present, the IE shall be set to the value "TRACE_ONLY". |

#### 5.6.4.2	Type: MdtConfiguration

Table 5.6.4.2-1: Definition of type MdtConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| jobType | JobType | M | 1 | This IE shall indicate the Job type for MDT, see 3GPP TS 32.422 [19]. This IE shall be set to one of the following values: "IMMEDIATE_MDT_ONLY" "LOGGED_MDT_ONLY" "IMMEDIATE_MDT_AND_TRACE" "LOGGED_MBSFN_MDT" "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" "TRACE_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" |
| reportType | ReportTypeMdt | C | 0..1 | This IE shall be present for logged MDT. When present, this IE shall indicate the report type for logged MDT, see 3GPP TS 32.422 [19]. |
| areaScope | AreaScope | O | 0..1 | When present, this IE shall contain the area in Cells or Tracking Areas where the MDT data collection shall take place, see 3GPP TS 32.422 [19]. |
| measurementLteList | array(MeasurementLteForMdt) | C | 1..N | This IE shall be present if the Job type is configured for Immediate MDT or combined Immediate MDT and Trace. When present, this IE shall contain a list of the measurements that shall be collected for LTE. |
| measurementNrList | array(MeasurementNrForMdt) | C | 1..N | This IE shall be present if the Job type is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall contain a list of the measurements that shall be collected for NR. |
| sensorMeasurementList | array(SensorMeasurement) | O | 1..N | When present, this IE shall include a list of the sensor measurements to be collected for UE in NR if they are available. |
| sensorMeasurementLteList | array(SensorMeasurement) | O | 1..N | When present, this IE shall include a list of the sensor measurements to be collected for UE in LTE if they are available. |
| reportingTriggerList | array(ReportingTrigger) | C | 1..N | This IE shall be present if MeasurementList is configured for UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall contain a list of the reporting triggers. For LTE and NR, this IE shall not have the combination of periodical, event based and event based periodic reporting at the same time. |
| reportInterval | ReportIntervalMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT or combined Immediate MDT and Trace. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in LTE. |
| reportIntervalNr | ReportIntervaLNrMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in NR) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall indicate the interval between the periodical measurements to be taken when UE is in connected in NR. |
| reportAmount | ReportAmountMdt | C | 0..1 | This IE shall be present if the reportingTriggerList is configured for Periodic UE side measurements (such as M1 measurement in LTE) and the jobType is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, this IE shall indicate the overall number of measurement reports that shall be taken for periodical reporting while UE is in connected. |
| reportAmountPerMeasurementLte | map(ReportAmountMdt) | C | 1..N | A map (list of key-value pairs where MeasurementLteForMdt serves as key; see clause 5.6.3.5) of ReportAmountMdt). May be present if reportAmount is present. When present, this IE shall indicate the number of measurement reports per measurement that shall be taken for periodical reporting while UE is in connected in LTE. |
| reportAmountPerMeasurementNR | map(ReportAmountMdt) | C | 1..N | A map (list of key-value pairs where MeasurementNrForMdt serves as key; see clause 5.6.3.6) of ReportAmountMdt). May be present if reportAmount is present. When present, this IE shall indicate the number of measurement reports per measurement that shall be taken for periodical reporting while UE is in connected in NR. |
| eventThresholdRsrp | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE. When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-97. |
| eventThresholdRsrpNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements in NR. When present, this IE shall indicate the Event Threshold for RSRP, and the value shall be between 0-127. |
| eventThresholdRsrq | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in LTE. When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-34. |
| eventThresholdRsrqNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements in NR. When present, this IE shall indicate the Event Threshold for RSRQ, and the value shall be between 0-127. |
| eventList | array(EventForMdt) | C | 1..N | This IE shall be present for event triggered measurement in the case of logged MDT. When present, this IE shall contain a list of events triggered measurement in NR. |
| loggingInterval | LoggingIntervalMdt | C | 0..1 | This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in LTE. When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in LTE when UE is in Idle. |
| loggingIntervalNr | LoggingIntervalNrMdt | C | 0..1 | This IE shall be present if the job type is configured for Logged MDT or Logged MBSFN MDT in NR. When present, this IE shall contain the periodicity for logging MDT measurement results for periodic downlink pilot strength measurement in NR when UE is in Idle. |
| loggingDuration | LoggingDurationMdt | O | 0..1 | This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT. When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in LTE |
| loggingDurationNr | LoggingDurationNrMdt | O | 0..1 | This IE shall be present if the job type parameter is configured for Logged MDT or Logged MBSFN MDT. When present, this IE shall indicate the validity time of MDT logged configuration for IDLE in NR. |
| positioningMethod | PositioningMethodMdt | O | 0..1 | This IE may be present if the job type is set to Immediate MDT, Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements. When present, it shall indicate the positioning method that shall be used for the MDT job. For LTE the value "GNSS" may be selected only if the M1 measurement is selected in measurementList. |
| addPositioningMethodList | array(PositioningMethodMdt) | O | 1..N | This IE may be present if positioningMethod is present. When present, it shall indicate a list of the additional positioning methods that shall be used for the MDT job. For LTE, the value "GNSS" may be selected only if the M1 measurement is selected in measurementList. |
| collectionPeriodRmmLte | CollectionPeriodRmmLteMdt | C | 0..1 | This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and any of the "M2" or "M3" is contained in measurementList attribute in LTE. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job. |
| collectionPeriodRmmNr | CollectionPeriodRmmNrMdt | C | 0..1 | This IE shall be present if the job type is set to Immediate MDT, Immediate MDT and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC UE level measurements and any of the "M4" or "M5" is contained in measurementList attribute in NR. When present, it shall contain the collection period that should be used to collect available measurement samples in case of RRM configured measurements. The same collection period should be used for all such measurements that are requested in the same MDT or combined Trace and MDT job. |
| measurementPeriodLte | MeasurementPeriodLteMdt | C | 0..1 | This IE shall be present if the job type is set to Immediate MDT or Immediate MDT and Trace and either the value "M4_DL" or "M4_UL" or "M5_DL" or "M5_UL" is contained in measurementList attribute in LTE. When present, it shall contain the collection period that should be used for the Data Volume and Scheduled IP Throughput measurements made by the eNB. The same measurement period should be used for the UL and DL. |
| mdtAllowedPlmnIdList | array(PlmnId) | O | 1..N | When present, this IE shall contain the PLMNs where measurement collection, status indication and log reporting is allowed. E.g. the UE performs these actions for Logged MDT when the RPLMN is part of this set of PLMNs. Maximum of 16 PLMNs can be contained. |
| mbsfnAreaList | array(MbsfnArea) | O | 1..N | When present, this IE shall contain MBSFN Area(s) for MBSFN measurement logging. Maximum of 8 MBSFN area(s) can be contained. This parameter is applicable only if the job type is Logged MBSFN MDT and for eUTRAN only. |
| interFreqTargetList | array(InterFreqTargetInfo) | O | 1..8 | When present, this IE shall indicate Inter Frequency Target(s) for which the UE is requested to perform measurement logging. |
| mnOnlyInd | boolean | O | 0..1 | When present, this IE shall indicate whether the MDT Configuration is only applicable for the master node or not if the UE is configured with MR-DC. This IE shall be set as follows: - true: MDT Configuration is for master node only - false (default): MDT Configuration is for both master node and secondary node. |
| eventThresholdSinrNr | integer | C | 0..1 | This IE shall be present if the report trigger parameter is configured for A2 event reporting or A2 event triggered periodic reporting and the job type parameter is configured for Immediate MDT or combined Immediate MDT and Trace in NR. When present, this IE shall indicate the Event Threshold for SINR, and the value shall be between 0-127. |
| bluetoothMeasurementNr | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in NR if they are available. |
| bluetoothMeasurementLte | BluetoothMeasurement | O | 0..1 | When present, this IE shall include the bluetooth measurements to be collected for UE in LTE if they are available. |
| wlanMeasurementNr | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in NR if they are available. |
| wlanMeasurementLte | WlanMeasurement | O | 0..1 | When present, this IE shall include the WLAN Measurements to be collected for UE in LTE if they are available. |
| sliceAreaScope | array(SliceScopePerPlmn) | O | 1..16 | When present, this IE shall contain the list of network slices for MDT. |

#### 5.6.4.3	Type: AreaScope

Table 5.6.4.3-1: Definition of type AreaScope

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eutraCellIdList | array(EutraCellId) | O | 1..N | When present, this IE shall contain a list of the E-UTRAN Cell Identifications where the MDT data collection shall take place. |
| nrCellIdList | array(NrCellId) | O | 1..N | When present, this IE shall contain a list of the NR Cell Identities where the MDT data collection shall take place. |
| tacList | array(Tac) | O | 1..N | When present, this IE shall contain a list of the tracking area codes where the MDT data collection shall take place. |
| tacInfoPerPlmn | map(TacInfo) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see clause 5.4.4.3) of TacInfo |
| cagInfoPerPlmn | map(CagInfo) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see clause 5.4.4.3) of CagInfo |
| nidInfoPerPlmn | map(NidInfo) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see clause 5.4.4.3) of NidInfo |
| cellIdNidInfoPerPlmn | map(CellIdNidInfo) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see clause 5.4.4.3) of CellIdNidInfo |
| tacNidInfoPerPlmn | map(TacNidInfo) | O | 1..N | A map (list of key-value pairs where PlmnId converted to string serves as key; see clause 5.4.4.3) of TacNidInfo |
| cagList | array(CagId) | O | 1..N | When present, this IE shall contain a list of the CAG IDs where the MDT data collection shall take place. |

#### 5.6.4.4	Type: TacInfo

Table 5.6.4.4-1: Definition of type TacInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tacList | array(Tac) | M | 1..N | This IE shall contain a list of the tracking area codes. |

#### 5.6.4.5	Type: MbsfnArea

Table 5.6.4.5-1: Definition of type MbsfnArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsfnAreaId | integer | O | 0..1 | This IE shall contain the MBSFN Area ID. The range of the value is from 0 to 255, see 3GPP TS 36.331 [39]. |
| carrierFrequency | integer | O | 0..1 | When present, this IE shall contain the Carrier Frequency (EARFCN). The range of the value is from 0 to 262143, see 3GPP TS 36.331 [39]. |
| NOTE If both mbsfnAreaId and carrierFrequency values are present, a specific MBSFN area is indicated. If carrierFrequency is present, but mbsfnAreaId is absent, all MBSFN areas on that carrier frequency are indicated. If both mbsfnAreaId and carrierFrequency are absent, any MBSFN area is indicated. |  |  |  |  |

#### 5.6.4.6	Type: InterFreqTargetInfo

Table 5.6.4.6-1: Definition of type InterFreqTargetInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dlCarrierFreq | ArfcnValueNr | M | 1 | This IE shall indicate the value of frequency for download for measurement logging. |
| cellIdList | array(PhysCellId) | O | 1..32 | When present, this IE shall contain a list of the physical cell identities where the UE is requested to perform measurement logging for the indicated frequency. If absent, the UE shall perform measurement logging on all physical cells. |

#### 5.6.4.7	Type: QmcConfigInfo

Table 5.6.4.7-1: Definition of type QmcConfigInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qoeReference | QoeReference | M | 1 | This IE contains the Quality of Experience (QoE) Reference. See 3GPP TS 28.405 [56], clause 5.2. |
| serviceType | QoeServiceType | O | 0..1 | This IE contains the Service Type of QoE measurements. See 3GPP TS 28.405 [56], clause 5.8. |
| sliceScope | array(Snssai) | O | 1..N | This IE contains a list of S-NSSAIs. See 3GPP TS 28.405 [56], clause 5.9. |
| areaScope | QmcAreaScope | O | 0..1 | This IE contains the area in Cells or Tracking Areas where the QMC data collection shall take place. See 3GPP TS 28.405 [56], clause 5.4. |
| qoeCollectionEntityAddress | IpAddr | O | 0..1 | This IE contains the IP address (IPv4 or IPv6) of the entity to which the QMC records shall be transferred. See 3GPP TS 28.405 [56], clause 5.1. |
| qoeTarget | QoeTarget | O | 0..1 | This parameter specifies the target object (individual UE) for the QMC in case of signalling based QMC. The qoeTarget parameter shall be able to carry an IMSI or a SUPI. See 3GPP TS 28.405 [56], clause 5.10. |
| mdtAlignmentInfo | MdtAlignmentInfo | O | 0..1 | This parameter indicates the MDT measurements with which alignment of QoE measurement is required. It contains the Trace Reference and Trace Recording Session Reference. See 3GPP TS 28.405 [56], clause 5.13. |
| availableRanVisibleQoeMetrics | array(AvailableRanVisibleQoeMetric) | O | 1..N | A list of RAN-visible QoE metrics configured by gNB to collect all or some of the available RAN visible QoE metrics, where the indication of metric availability is indicated by UDM. |
| containerForAppLayerMeasConfig | Bytes | O | 0..1 | This parameter contains application layer measurement configuration. See 3GPP TS 28.405 [56], clause 5.5. |
| mbsCommunicationServiceType | MbsServiceType | O | 0..1 | This parameter indicates for which type of MBS communication service the QoE measurement configuration pertains to (i.e. "MULTICAST", "BROADCAST"). |

#### 5.6.4.8	Type: QmcAreaScope

Table 5.6.4.8-1: Definition of type QmcAreaScope

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nrCellIdList | array(NrCellId) | O | 1..N | When present, this IE shall contain a list of the NR Cell Identities where the QMC shall take place. |
| tacList | array(Tac) | O | 1..N | When present, this IE shall contain a list of the tracking area codes where the QMC shall take place. |
| taiList | array(Tai) | O | 1..N | When present, this IE shall contain a list of the TAIs where the QMC shall take place. |
| plmnList | array(PlmnId) | O | 1..N | When present, this IE shall contain a list of the PLMNs where the QMC shall take place. |

#### 5.6.4.9	Type: QoeTarget

Table 5.6.4.9-1: Definition of type QoeTarget

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supi | Supi | O | 0..1 | When present, this IE shall contain the SUPI of the target object (individual UE) for the QMC in case of signalling based QMC. |
| imsi | Imsi | O | 0..1 | When present, this IE shall contain the IMSI of the target object (individual UE) for the QMC in case of signalling based QMC. |

#### 5.6.4.10	Type: CagInfo

Table 5.6.4.10-1: Definition of type CagInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cagList | array(CagId) | M | 1..N | This IE shall contain a list of the CAG IDs. |

#### 5.6.4.11	Type: NidInfo

Table 5.6.4.11-1: Definition of type NidInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nidList | array(Nid) | M | 1..N | This IE shall contain a list of the NIDs. |

#### 5.6.4.12	Type: UeLevelMeasurementsConfiguration

Table 5.6.4.12-1: Definition of type UeLevelMeasurementsConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| jobType | JobType | M | 1 | This IE shall indicate the Job type for 5GC UE level measurements, see 3GPP TS 32.422 [19]. This IE shall be set to one of the following values: "5GC_UE_LEVEL_MEASUREMENTS_ONLY" "TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS_ONLY" "IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" "TRACE_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS" |
| ueLevelMeasurementsList | array(MeasurementType) | M | 1..N | List of 5GC UE level Measurement Type (see 3GPP TS 28.558 [66]). |
| granularityPeriod | DurationSec | O | 0..1 | This IE identifies the period time for every Trace Recording Session of 5GC UE level measurement (see 3GPP TS 32.422 [19]). |

#### 5.6.4.13	Type: CellIdNidInfo

Table 5.6.4.13-1: Definition of type CellIdNidInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cellIdNidList | array(CellIdNid) | M | 1..N | This IE shall contain a list of the NR Cell Identities in SNPN. |

#### 5.6.4.14	Type: CellIdNid

Table 5.6.4.14-1: Definition of type CellIdNid

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cellId | NrCellId | M | 1 | This IE shall contain the NR Cell Identity. |
| nid | Nid | M | 1 | This IE shall contain the Network Identity. |

#### 5.6.4.15	Type: TacNidInfo

Table 5.6.4.15-1: Definition of type TacNidInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tacNidList | array(TacNid) | M | 1..N | This IE shall contain a list of the tracking area codes in SNPN. |

#### 5.6.4.16	Type: TacNid

Table 5.6.4.16-1: Definition of type TacNid

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tac | Tac | M | 1 | This IE shall contain the tracking area code. |
| nid | Nid | M | 1 | This IE shall contain the Network Identity. |

#### 5.6.4.17	Type: BluetoothMeasurement

Table 5.6.4.17-1: Definition of type BluetoothMeasurement

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| measurementNameList | array(MeasurementName) | C | 1..4 | This IE shall be present if available. When present, this IE shall contain the list of Bluetooth Measurement Configuration Name. |
| bluetoothRssi | BluetoothRssi | O | 0..1 | When present, this IE shall contain the RSSI measurement by UE for Bluetooth for M8 measurement. |

#### 5.6.4.18	Type: WlanMeasurement

Table 5.6.4.18-1: Definition of type WlanMeasurement

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| measurementNameList | array(MeasurementName) | C | 1..4 | This IE shall be present if available. When present, this IE shall contain the list of WLAN Measurement Configuration Name. |
| wlanRssi | WlanRssi | O | 0..1 | When present, this IE shall contain the RSSI measurement by UE for WLAN for M8 measurement. |
| wlanRtt | WlanRtt | O | 0..1 | When present, this IE shall contain the RTT measurement by UE for WLAN for M9 measurement. |

#### 5.6.4.19	Type: MeasurementName

Table 5.6.4.19-1: Definition of type MeasurementName

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bluetoothName | string | C | 0..1 | This IE shall be present for Bluetooth Measurement Configuration. When present, this IE shall contain the Bluetooth local name used for Bluetooth measurement collection, see clause 9.3.1.177 of 3GPP TS 38.413 [11]. |
| wlanName | string | C | 0..1 | This IE shall be present for WLAN Measurement Configuration. When present, this IE shall contain the WLAN SSID used for WLAN measurement collection, see clause 9.3.1.178 of 3GPP TS 38.413 [11]. |

#### 5.6.4.20	Type: SliceScopePerPlmn

Table 5.6.4.20-1: Definition of type SliceScopePerPlmn

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmn | PlmnId | M | 1 | This IE shall contain the PLMN where the MDT data collection shall take place. |
| sliceScope | array(Snssai) | M | 1..1024 | This IE shall contain a list of S-NSSAIs. |

## 5.7	Data Types related to 5G Operator Determined Barring

### 5.7.1	Introduction

This clause defines common data types related to 5G Operator Determined Barring.

### 5.7.2	Simple Data Types

This clause specifies common simple data types.

Table 5.7.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |

### 5.7.3	Enumerations

#### 5.7.3.1	Enumeration: RoamingOdb

The enumeration RoamingOdb defines the Barring of Roaming as. See 3GPP TS 23.015 [26] for further description. It shall comply with the provisions defined in table 5.7.3.1-1.

Table 5.7.3.1-1: Enumeration RoamingOdb

| Enumeration value | Description |
| --- | --- |
| "OUTSIDE_HOME_PLMN" | Barring of roaming outside the home PLMN |
| "OUTSIDE_HOME_PLMN_COUNTRY" | Barring of roaming outside the home PLMN country |

#### 5.7.3.2	Enumeration: OdbPacketServices

The enumeration OdbPacketServices defines the Barring of Packet Oriented Services. See 3GPP TS 23.015 [26] for further description. It shall comply with the provisions defined in table 5.7.3.2-1.

Table 5.7.3.2-1: Enumeration OdbPacketServices

| Enumeration value | Description |
| --- | --- |
| "ALL_PACKET_SERVICES" | Barring of all Packet Oriented Services |
| "ROAMER_ACCESS_HPLMN_AP" | Barring of Packet Oriented Services from access points that are within the HPLMN whilst the subscriber is roaming in a VPLMN |
| "ROAMER_ACCESS_VPLMN_AP" | Barring of Packet Oriented Services from access points that are within the roamed to VPLMN. |

### 5.7.4	Structured Data Types

#### 5.7.4.1	Type: OdbData

Table 5.7.4.1-1: Definition of type OdbData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| roamingOdb | RoamingOdb | O | 0..1 | Barring of Roaming (see 3GPP TS 23.015 [26]). |

## 5.8	Data Types related to Charging

### 5.8.1	Introduction

This clause defines common data types related to Charging.

### 5.8.2	Simple Data Types

This clause specifies common simple data types.

Table 5.8.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| ChargingId | Uint32 | Charging identifier allowing correlation of charging information (NOTE) |
| SmfChargingId | string | String based Charging ID as specified in 3GPP TS 32.255 [58]. The String based Charging ID shall include a Uint32 base charging identifier (decimal encoded value within the values range: 0 to 4294967295 included) as the first segment, which shall be unique within the SMF assigning the Charging ID. The String based Charging ID shall include the NF Instance ID (UUID format) of the SMF that assigned the Charging ID, as the second segment. Pattern: '^(0¦([1-9]{1}[0-9]{0,9}))\.smf-([0-9a-f]{8}-[0-9a-f]{4}-[0-9a-f]{4}-[0-9a-f]{4}-[0-9a-f]{12})$' Example: Base Charging ID: "9387" SMF NF Instance ID: "4947a69a-f61b-4bc1-b9da-47c9c5d14b64" String based Charging ID: "9387.smf-4947a69a-f61b-4bc1-b9da-47c9c5d14b64" |
| ApplicationChargingId | string | Application provided charging identifier allowing correlation of charging information. |
| RatingGroup | Uint32 | Identifier of a Rating Group |
| ServiceId | Uint32 | Identifier of a Service |
| NOTE: This data type is deprecated and shall not be used by any new API definition. To ensure the uniqueness of the charging identifier, the SmfChargingId data type shall be used for new attributes defined in APIs carrying a charging identifier. |  |  |

### 5.8.3	Enumerations

### 5.8.4	Structured Data Types

#### 5.8.4.1	Type: SecondaryRatUsageReport

Table 5.8.4.1-1: Definition of type SecondaryRatUsageReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| secondaryRatType | RatType | M | 1 | Secondary RAT type |
| qosFlowsUsageData | array(QosFlowUsageReport) | M | 1..N | QoS flows usage data |

#### 5.8.4.2	Type: QoSFlowUsageReport

Table 5.8.4.2-1: Definition of type QoSFlowUsageReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | QoS Flow Indicator |
| startTimeStamp | DateTime | M | 1 | UTC time indicating the start time of the collection period of the included usage data for DL and UL. |
| endTimeStamp | DateTime | M | 1 | UTC time indicating the end time of the collection period of the included usage data for DL and UL. |
| downlinkVolume | Int64 | M | 1 | Data usage for DL, encoding a number of octets |
| uplinkVolume | Int64 | M | 1 | Data usage for UL, encoding a number of octets |

#### 5.8.4.3	Type: SecondaryRatUsageInfo

Table 5.8.4.3-1: Definition of type SecondaryRatUsageInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| secondaryRatType | RatType | M | 1 | Secondary RAT type |
| qosFlowsUsageData | array(QosFlowUsageReport) | O | 1..N | QoS flows usage data |
| pduSessionUsageData | array(VolumeTimedReport) | O | 1..N | PDU session usage data |

#### 5.8.4.4	Type: VolumeTimedReport

Table 5.8.4.4-1: Definition of type VolumeTimedReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| startTimeStamp | DateTime | M | 1 | UTC time indicating the start time of the collection period of the included usage data for DL and UL. |
| endTimeStamp | DateTime | M | 1 | UTC time indicating the end time of the collection period of the included usage data for DL and UL. |
| downlinkVolume | Int64 | M | 1 | Data usage for DL, encoding a number of octets |
| uplinkVolume | Int64 | M | 1 | Data usage for UL, encoding a number of octets |

## 5.9	Data Types related to MBS

### 5.9.1	Introduction

This clause defines common data types related to MBS.

### 5.9.2	Simple Data Types

This clause specifies common simple data types.

Table 5.9.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| AreaSessionId | Uint16 | Area Session Identifier used for MBS session with location dependent content. When present, the Area Session ID together with the TMGI uniquely identifies the MBS session in a specific MBS service area. |
| AreaSessionPolicyId | Uint16 | Area Session Policy ID used for MBS session with location dependent content. |
| MbsFsaId | string | MBS Frequency Selection Area ID, for a broadcast MBS session The value of the MbsFsaId shall be encoded in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the MBS FSA Id shall appear first in the string, and the character representing the 4 least significant bit of the MBS FSA Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{6}$' |
| IntendedServiceArea | Bytes | String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, encoding the value of the Intended Service Area IE defined in clause 9.3.1.x of 3GPP TS 38.413 [11]. |

Editor's note: the encoding of the IntendedServiceArea needs to be confirmed/aligned on TS 38.413.

### 5.9.3	Enumerations

#### 5.9.3.1	Enumeration: MbsServiceType

The enumeration MbsServiceType indicates the type of MBS session. It shall comply with the provisions defined in Table 5.9.3.1-1.

Table 5.9.3.1-1: Enumeration MbsServiceType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "MULTICAST" | Multicast MBS session |  |
| "BROADCAST" | Broadcast MBS session |  |

#### 5.9.3.2	Enumeration: MbsSessionActivityStatus

The enumeration MbsSessionActivityStatus indicates the MBS session's activity status. It shall comply with the provisions defined in Table 5.9.3.2-1.

Table 5.9.3.2-1: Enumeration MbsSessionActivityStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ACTIVE" | Active MBS session |  |
| "INACTIVE" | Inactive MBS session |  |

#### 5.9.3.3	Enumeration: MbsSessionEventType

Table 5.9.3.3-1: Enumeration MbsSessionEventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "MBS_REL_TMGI_EXPIRY" | Subscription to be notified or notification request about the MBS session release due to TMGI expiry. |  |
| "BROADCAST_DELIVERY_STATUS" | Subscription to be notified or notification request about the MBS session broadcast delivery status. |  |
| "INGRESS_TUNNEL_ADD_CHANGE" | Subscription to be notified or notification request about change of the Ingress Tunnel Address, when using unicast transport over N6mb/Nmb9. |  |

#### 5.9.3.4	Enumeration: BroadcastDeliveryStatus

Table 5.9.3.4-1: Enumeration BroadcastDeliveryStatus

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "STARTED" | The MBS session has been started. |  |
| "TERMINATED" | The MBS session has been terminated. |  |

#### 5.9.3.5	Enumeration: NrRedCapUeInfo

The enumeration NrRedCapUeInfo indicates NR RedCap Information of the broadcast MBS session. It shall comply with the provisions defined in Table 5.9.3.5-1.

Table 5.9.3.5-1: Enumeration NrRedCapUeInfo

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "NR_REDCAP_UE_ONLY" | Indicates that the MBS session is expected to be received only by NR (e)RedCap UEs. |  |
| "BOTH_NR_REDCAP_UE_AND_NON_REDCAP_UE" | Indicates that the MBS session is expected to be received by any kind of UEs. |  |
| "NON_REDCAP_UE_ONLY" | Indicates that the MBS session is expected to be received only by non-RedCap UEs, i.e. UEs that are neither NR RedCap UEs nor NR eRedCap UEs. |  |

### 5.9.4	Structured Data Types

#### 5.9.4.1	Type: MbsSessionId

Table 5.9.4.1-1: Definition of type MbsSessionId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tmgi | Tmgi | C | 0..1 | TMGI identifying the MBS session (NOTE) |
| ssm | Ssm | C | 0..1 | Source specific IP multicast address identifying the MBS session (NOTE) |
| nid | Nid | O | 0..1 | Network Identity used together with the TMGI to identify an MBS session in an SNPN |
| NOTE: At least one of the tmgi IE and ssm IE shall be present. |  |  |  |  |

#### 5.9.4.2	Type: Tmgi

Table 5.9.4.2-1: Definition of type Tmgi

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsServiceId | string | M | 1 | MBS Service ID consisting of a 6-digit fixed-length hexadecimal number between 000000 and FFFFFF. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the MBS Service ID shall appear first in the string, and the character representing the 4 least significant bit of the MBS Service ID shall appear last in the string. Pattern: '^[A-Fa-f0-9]{6}$' |
| plmnId | PlmnId | M | 1 | PLMN ID |

#### 5.9.4.3	Type: Ssm

Table 5.9.4.3-1: Definition of type Ssm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sourceIpAddr | IpAddr | M | 1 | IP unicast address used as source address in IP packets for identifying the source of the multicast service (e.g. AF/AS). |
| destIpAddr | IpAddr | M | 1 | IP multicast address used as destination address in related IP packets for identifying the multicast service associated with the source. |

#### 5.9.4.4	Type: MbsServiceArea

Table 5.9.4.4-1: Definition of type MbsServiceArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ncgiList | array(NcgiTai) | C | 1..N | List of NR cell ids with their pertaining TAIs (NOTE 1, NOTE 3). |
| taiList | array(Tai) | C | 1..N | List of tracking area Ids (NOTE 1). |
| intendedServAreaList | array(IntendedServiceArea) | O | 1..N | List of NR NTN Intended Service Areas (NOTE 2) |
| NOTE 1: The MBS Service Area consists of the union of the cells in the tracking areas listed in the taiList IE and the cells listed in the ncgiList IE. At least one of the ncgiList IE and taiList IE shall be present. NOTE 2: The NR NTN Intended Service Area may be sent by the NEF/MBSF to the NG-RAN, via the MB-SMF and AMF, to further restrict the MBS service area. It contains information that shall only be used by the NG-RAN, i.e. the MB-SMF and AMF shall transfer this information transparently. See 3GPP TS 23.247 [72]. NOTE 3: In case of NR NTN, the cell IDs contained in the MBS service area may correspond to mapped cell IDs as defined in 3GPP TS 38.300 [73]. |  |  |  |  |

#### 5.9.4.5	Type: NcgiTai

Table 5.9.4.5-1: Definition of type NcgiTai

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tai | Tai | M | 1 | TAI of the cells in cellList (NOTE) |
| cellList | array(Ncgi) | M | 1..N | List of NR cell ids |
| NOTE: The NcgiTai consists of the list of cells listed in the cellList IE. These cells pertain to the TAI indicated in the tai IE. The TAI may be used e.g. to discover and select an AMF that serves NG-RAN nodes supporting the corresponding cells. |  |  |  |  |

#### 5.9.4.6	Type: MbsSession

Table 5.9.4.6-1: Definition of type MbsSession

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | C | 0..1 | MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE 1) |
| tmgiAllocReq | boolean | C | 0..1 | TMGI allocation request indication. This IE shall be present if the mbsSessionId IE is absent. This IE may also be present if the mbsSessionId IE is present and it does not contain a TMGI. When present, it shall be set as follows: - true: a TMGI is requested to be allocated - false (default): no TMGI is requested to be allocated Write-Only: true (NOTE 1) |
| tmgi | Tmgi | C | 0..1 | This IE shall be present in an MBS session creation response if the tmgiAllocReq IE was present and set to "true" in the MBS session creation request. When present, it shall indicate the TMGI allocated to the MBS session. Read-Only: true |
| expirationTime | DateTime | C | 0..1 | This IE shall be present in an MBS session creation response if the tmgiAllocReq IE was present and set to "true" in the in the MBS session creation request. When present, it shall indicate the expiration time for the TMGI allocated to the MBS session. Read-Only: true |
| serviceType | MbsServiceType | M | 1 | MBS Service Type (either multicast or broadcast service) Write-Only: true |
| locationDependent | boolean | C | 0..1 | Location dependent MBS session indication. This IE shall be present and set to true for a Location dependent MBS session. It may be present otherwise. When present, it shall be set as follows: - true: this is a Location dependent MBS session - false (default): this is not a Location dependent MBS session |
| areaSessionId | AreaSessionId | C | 0..1 | This IE shall be present in a successful response to a request to create an instance of a Location dependent MBS session i.e. when the "locationDependent" attribute is present and set to "true" in the MBS session creation request. When present, it shall contain the Area Session ID assigned by the MB-SMF to the location dependent MBS session instance in the MBS Service Area. Read-Only: true |
| ingressTunAddrReq | boolean | O | 0..1 | Ingress transport address request indication (for unicast transport over N6mb/Nmb9). When present, it shall be set as follows: - true: an ingress transport address is requested - false (default): no request Write-Only: true |
| ingressTunAddr | array(TunnelAddress) | C | 1..N | Ingress tunnel address (UDP/IP tunnel). This IE shall be present in an MBS session creation response if the ingressTunAddrReq IE was present and set to "true" in the corresponding MBS session creation request. When present, it shall indicate the allocated ingress tunnel address(es). Read-Only: true (NOTE 2) |
| ssm | Ssm | C | 0..1 | Source specific IP multicast address This IE shall be present if multicast transport applies over N6mb and the MBS session is not identified by the SSM, e.g. for a location-dependent MBS session with multicast transport over N6mb. Write-Only: true |
| mbsServiceArea | MbsServiceArea | O | 0..1 | Contains the MBS Service Area This attribute shall be present only for a location dependent MBS session or a local MBS session. Write-Only: true |
| extMbsServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the MBS service area. This attribute shall be present only for a location dependent MBS session or a local MBS session. This IE may be present only over the N33 interface; it shall not be present over other interfaces. When present, it shall indicate the MBS Service Area information which shall either be geographical area information or civic address information. Write-Only: true |
| redMbsServArea | MbsServiceArea | C | 0..1 | This attribute shall be present if the requested MBS service area is not entirely contained within the service area of a single MB-SMF. When present, it shall contain the reduced MBS Service Area in which the MBS session has been created. Read-Only: true (NOTE 3) |
| extRedMbsServArea | ExternalMbsServiceArea | C | 0..1 | This attribute shall be present if the requested MBS service area is not entirely contained within the service area of a single MB-SMF. This IE may be present only over the N33 interface; it shall not be present over other interfaces. When present, it shall contain the reduced MBS Service Area in which the MBS session has been created, which shall either be geographical area information or civic address information. Read-Only: true (NOTE 3) |
| dnn | Dnn | O | 0..1 | Represents the DNN Write-Only: true |
| snssai | Snssai | O | 0..1 | Represents the S-NSSAI Write-Only: true |
| activationTime | DateTime | O | 0..1 | Represents the MBS session start time. This attribute is deprecated and replaced by the "startTime" attribute. |
| startTime | DateTime | O | 0..1 | Represents the MBS session start time. |
| terminationTime | DateTime | O | 0..1 | Represents the MBS session termination time. |
| mbsServInfo | MbsServiceInfo | O | 0..1 | Contains the MBS Service Information for the MBS session. |
| mbsSessionSubsc | MbsSessionSubscription | O | 0..1 | Contains the parameters to request the creation of a subscription to one or more MBS session status event(s). |
| activityStatus | MbsSessionActivityStatus | O | 0..1 | Contains the session activity status (active or inactive). This IE may be provided if the "serviceType" attribute indicates a multicast MBS session. |
| anyUeInd | boolean | O | 0..1 | Indication that any UE may join the MBS session. This IE may be provided if the "serviceType" attribute indicates a multicast MBS session. When present, it shall be set as follows: - true: any UE may join the MBS session - false (default): the MBS session is not open to any UE Write-Only: true |
| mbsFsaIdList | array(MbsFsaId) | O | 1..N | List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. This attribute may be present if the "serviceType" attribute indicates a broadcast MBS session. |
| associatedSessionId | AssociatedSessionId | O | 0..1 | Associated Session ID to enable NG-RAN to identify the multiple MBS sessions delivering the same content when AF creates multiple broadcast MBS Sessions via different Core Networks to deliver the same content. |
| nrRedCapUeInfo | NrRedCapUeInfo | O | 0..1 | Indicates whether the broadcast MBS session is intended only for NR (e)RedCap UEs, only for UEs that are neither NR RedCap UEs nor NR eRedCap UEs, or for any kind of UE. |
| NOTE 1: At least one of the mbsSessionId IE and tmgiAllocReq IE shall be present. Both may be present if the mbsSessionId IE does not contain a TMGI (i.e. if it only contains a SSM). NOTE 2: In a scenario where an MB-UPF covers a small service area (i.e. a list of TAIs that is a subset of the MBS service area), the MB-SMF needs to find other MB-UPF(s) to cover the whole MBS service area for the MBS session. In such scenarios, multiple ingress addresses of all MB-UPFs need to be allocated for an MBS session. These multiple ingress tunnel addresses are used to receive the same copy of the MBS session data from the AF/MBSTF. In such scenarios, the MBS service area served by an MB-UPF shall be larger than the MBS service area served by an AMF (set), i.e. an AMF in an AMF set shall receive only one MBS Session Information Request Transfer for an MBS session in the MBS Session Context Create/Update Request message. NOTE 3: These attributes are sent on different interfaces. Accordingly, they are mutually exclusive. |  |  |  |  |

#### 5.9.4.7	Type: MbsSessionSubscription

Table 5.9.4.7-1: Definition of type MbsSessionSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsSessionId | MbsSessionId | C | 0..1 | Identifier of the MBS Session for which the subscription is created. This IE shall be present, except for an MBS session subscription request that is sent within an MBS session creation request. |
| areaSessionId | AreaSessionId | C | 0..1 | Area Session ID, for a location dependent MBS session, identifying the part of the MBS session in an MBS service area for which the subscription is created. It shall be present for a location dependent MBS session, except for an MBS session subscription request that is sent within an MBS session creation request |
| eventList | array(MbsSessionEvent) | M | 1..N | List of MBS session events subscribed |
| notifyUri | Uri | M | 1 | URI where the NF service consumer requests to receive MBS session notifications. Write-Only: true |
| notifyCorrelationId | string | O | 0..1 | Notification Correlation ID Write-Only: true |
| expiryTime | DateTime | O | 0..1 | When present in an MBS Session subscription creation request, it shall indicate the time up to which the subscription is desired to be kept active and after which the subscribed events shall stop generating notifications. When present in a subscription response, it shall indicate the expiry time after which the subscription becomes invalid. |
| nfcInstanceId | NfInstanceId | C | 0..1 | NF Instance ID of the NF Service Consumer This IE shall be present if available. Write-Only: true |
| mbsSessionSubscUri | Uri | C | 0..1 | This IE shall be present in the response to an MBS session creation request that includes a subscription to events about the MBS session and the subscription was created successfully. When present, it shall contain the URI of the individual subscription resource. Read-Only: true (NOTE) |
| NOTE: When an MBS session status subscription is created separately (i.e. after) an MBS session creation, the Location header returned in the MBS session status subscription creation response contains the URI of the created subscription. |  |  |  |  |

#### 5.9.4.8	Type: MbsSessionEventReportList

Table 5.9.4.8-1: Definition of type MbsSessionEventReportList

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventReportList | array(MbsSessionEventReport) | M | 1..N | List of MBS session events to report |
| notifyCorrelationId | string | C | 0..1 | Notification Correlation ID. This IE shall be present if a Notification Correlation ID is available in the subscription. |

#### 5.9.4.9	Type: MbsSessionEvent

Table 5.9.4.9-1: Definition of type MbsSessionEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | MbsSessionEventType | M | 1 | MBS session event type |  |

#### 5.9.4.10	Type: MbsSessionEventReport

Table 5.9.4.10-1: Definition of type MbsSessionEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | MbsSessionEventType | M | 1 | MBS session event type |  |
| timeStamp | DateTime | C | 0..1 | This IE shall contain the time at which the event is generated. This IE should be present, if available. |  |
| ingressTunAddrInfo | IngressTunAddrInfo | C | 0..1 | This IE shall be present if the eventType IE indicates "INGRESS_TUNNEL_ADD_CHANGE". |  |
| broadcastDelStatus | BroadcastDeliveryStatus | C | 0..1 | This IE shall be present if the eventType IE indicates "BROADCAST_DELIVERY_STATUS". |  |

#### 5.9.4.11	Type: ExternalMbsServiceArea

Table 5.9.4.11-1: Definition of type ExternalMbsServiceArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| geographicAreaList | array(GeographicArea) | C | 1..N | Identifies a list of geographic area specified by different shapes. |
| civicAddressList | array(CivicAddress) | C | 1..N | Identifies a list of civic address. |
| NOTE: Either the geographicAreaList IE or the civicAddressList IE shall be present. |  |  |  |  |

#### 5.9.4.12	Type: MbsSecurityContext

Table 5.9.4.12-1: Definition of type MbsSecurityContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| keyList | map(MbsKeyInfo) | M | 1..N | One or more MSK/MTK(s) and associated IDs. The key of the map shall be a (unique) valid JSON string per clause 7 of IETF RFC 8259 [78], with a maximum of 32 characters |

#### 5.9.4.13	Type: MbsKeyInfo

Table 5.9.4.13-1: Definition of type MbsKeyInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| keyDomainId | Bytes | M | 1 | Key Domain ID = MCC ¦¦ MNC as defined in 3GPP TS 33.246 [45]. (NOTE) It shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the Key Domain ID (encoded in 3 bytes). |
| mskID | Bytes | M | 1 | MSK ID as defined in 3GPP TS 33.246 [45]. It shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the MSK ID (encoded in 4 bytes). |
| msk | Bytes | C | 0..1 | MSK as defined in 3GPP TS 33.246 [45]. The IE shall not be present when MBSTF requests updated MSK from MBSF after, e.g. lifetime expiry. Shall be present otherwise. When present, it shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the MSK (encoded in 16 bytes). |
| mskLifetime | DateTime | O | 0..1 | MSK Lifetime as defined in 3GPP TS 33.501 [46]. |
| mtkID | Bytes | C | 0..1 | MTK ID as defined in 3GPP TS 33.246 [45]. Shall be present if available. It shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the MTK ID (encoded in 2 bytes). |
| mtk | Bytes | C | 0..1 | MTK as defined in 3GPP TS 33.246 [45]. Shall be present if available. It shall be encoded as a string with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters, representing the MTK (encoded in 16 bytes). |
| NOTE: For a multicast MBS session in a SNPN, the Key Domain ID for the MBS session should be encoded using MCC and MNC, in this case, it may not be unique. |  |  |  |  |

#### 5.9.4.14	Type: IngressTunAddrInfo

Table 5.9.4.14-1: Definition of type IngressTunAddrInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ingressTunAddr | array(TunnelAddress) | M | 1..N | Ingress Tunnel Address(es) to use to send MBS session data over N6mb/Nmb9 and that replace any earlier provided Ingress Tunnel Address(es). |  |

#### 5.9.4.15	Type: MbsServiceAreaInfo

Table 5.9.4.15-1: Definition of type MbsServiceAreaInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| areaSessionId | AreaSessionId | M | 1 | Area Session Identifier used for MBS session with location dependent content. |
| mbsServiceArea | MbsServiceArea | M | 1 | MBS Service Area for MBS session with location dependent content. |

#### 5.9.4.16	Type: MbsServiceInfo

Table 5.9.4.16-1: Definition of type MbsServiceInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsMediaComps | map(MbsMediaCompRm) | M | 1..N | Contains the information of one or several media component(s). The key of the map is the "mbsMedCompNum" attribute of the corresponding MbsMediaCompRm data structure provided as a map entry. |
| mbsSdfResPrio | ReservPriority | O | 0..1 | Indicates the reservation priority of the MBS service data flow(s) identified by the "mbsMediaComps" attribute. |
| afAppId | AfAppId | O | 0..1 | Contains the AF application identifier. |
| mbsSessionAmbr | BitRate | O | 0..1 | Contains the required MBS Session-AMBR. |

#### 5.9.4.17	Type: MbsMediaComp

Table 5.9.4.17-1: Definition of type MbsMediaComp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsMedCompNum | integer | M | 1 | Contains the ordinal number of the MBS media component. |  |
| mbsFlowDescs | array(FlowDescription) | O | 1..N | Contains the flow description for the MBS Downlink IP flow(s). |  |
| mbsSdfResPrio | ReservPriority | O | 0..1 | Indicates the reservation priority for the MBS service data flow(s) identified by the "mbsFlowDescs" attribute. (NOTE 2) |  |
| mbsMediaInfo | MbsMediaInfo | O | 0..1 | Indicates the MBS media information. (NOTE 1) |  |
| qosRef | string | O | 0..1 | Contains the identifier to pre-defined MBS QoS. (NOTE 1) |  |
| mbsQoSReq | MbsQoSReq | O | 0..1 | Contains the MBS QoS requirements. (NOTE 1) |  |
| NOTE 1: Only one of these attributes may be present. NOTE 2: When present, the value of this attribute shall apply for the MBS service data flow(s) identified by this MBS Media Component. It shall take precedence over the value of the same attribute within the parent MbsServiceInfo data structure. |  |  |  |  |  |

#### 5.9.4.18	Type: MbsMediaCompRm

This data type is defined in the same way as the MbsMediaComp data type defined in clause 5.9.4.17, but with the OpenAPI "nullable: true" property.

#### 5.9.4.19	Type: MbsQoSReq

Table 5.9.4.19-1: Definition of type MbsQoSReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | Represents the required 5QI. |  |
| guarBitRate | BitRate | O | 0..1 | Contain the required 5GS guaranteed bit rate. |  |
| maxBitRate | BitRate | O | 0..1 | Contain the required 5GS maximum bit rate. |  |
| averWindow | AverWindow | C | 0..1 | Indicates the averaging window. This attribute shall be present only for a GBR QoS flow or a Delay Critical GBR QoS flow. |  |
| reqMbsArp | Arp | O | 0..1 | Indicates the requested allocation and retention priority. |  |

#### 5.9.4.20	Type: MbsMediaInfo

Table 5.9.4.20-1: Definition of type MbsMediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsMedType | MediaType | O | 0..1 | Indicates the MBS media type. |  |
| maxReqMbsBwDl | BitRate | O | 0..1 | Contains the Maximum requested bandwidth. |  |
| minReqMbsBwDl | BitRate | O | 0..1 | Contains the Minimum requested bandwidth. |  |
| codecs | array(CodecData) | O | 1..2 | Indicates the codec data. |  |

#### 5.9.4.21	Data types describing alternative data types or combinations of data types

##### 5.9.4.21.1	Type: AssociatedSessionId

5.9.6.21.1-1: Definition of type AssociatedSessionId as a list of non-exclusive alternative data types

| Data type | Cardinality | Description |
| --- | --- | --- |
| Ssm | 1 | AssociatedSessionId encoded as an SSM. |
| string | 1 | AssociatedSessionId encoded as a string. |

## 5.10	Data Types related to Time Synchronization

### 5.10.1	Introduction

This clause defines common data types related to Time Synchronization.

### 5.10.2	Simple Data Types

This clause specifies common simple data types.

Table 5.10.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| n/a |  |  |

### 5.10.3	Enumerations

#### 5.10.3.1	Enumeration: SynchronizationState

Table 5.10.3.1-1: Enumeration SynchronizationState

| Enumeration value | Description |
| --- | --- |
| "LOCKED" | Locked, see 3GPP TS 23.501 [2] |
| "HOLDOVER" | Holdover, see 3GPP TS 23.501 [2] |
| "FREERUN" | Freerun, see 3GPP TS 23.501 [2] |

#### 5.10.3.2	Enumeration: TimeSource

Table 5.10.3.2-1: Enumeration TimeSource

| Enumeration value | Description |
| --- | --- |
| "SYNC_E" | SyncE, see 3GPP TS 23.501 [2] |
| "PTP" | PTP, see 3GPP TS 23.501 [2] |
| "GNSS" | GNSS, see 3GPP TS 23.501 [2] |
| "ATOMIC_CLOCK" | atomic clock, see 3GPP TS 23.501 [2] |
| "TERRESTRIAL_RADIO" | terrestrial radio, see 3GPP TS 23.501 [2] |
| "SERIAL_TIME_CODE" | serial time code, see 3GPP TS 23.501 [2] |
| "NTP" | NTP, see 3GPP TS 23.501 [2] |
| "HAND_SET" | hand_set, see 3GPP TS 23.501 [2] |
| "OTHER" | other, see 3GPP TS 23.501 [2] |

#### 5.10.3.3	Enumeration: ClockQualityDetailLevel

Table 5.10.3.3-1: Enumeration ClockQualityDetailLevel

| Enumeration value | Description |
| --- | --- |
| "CLOCK_QUALITY_METRICS" | Clock Quality Metrics are to be provided to the UE |
| "ACCEPT_INDICATION" | Acceptable/not acceptable indication is to be provided to the UE |

#### 5.10.3.4	Enumeration: ClockQualityDetailLevelRm

This enumeration is defined in the same way as the "ClockQualityDetailLevel" enumeration, but with the OpenAPI "nullable: true" property.

### 5.10.4	Structured Data Types

#### 5.10.4.1	Type: ClockQualityAcceptanceCriterion

Table 5.10.4.1-1: Definition of type ClockQualityAcceptanceCriterion

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| synchronizationState | array(SynchronizationState) | O | 1..N | Indicates the state of the node synchronization, represented by the values "LOCKED", "HOLDOVER", or "FREERUN" |
| clockQuality | ClockQuality | O | 0..1 | Clock Quality |
| parentTimeSource | array(TimeSource) | O | 1..N | Parent Time Source. |

#### 5.10.4.1A	Type: ClockQualityAcceptanceCriterionRm

Describes the modifications to the "ClockQualityAcceptanceCriterion" data type. This data type is defined in the same way as the "ClockQualityAcceptanceCriterion" data type, but:

-	with the OpenAPI "nullable: true" property;

-	the removable attributes "clockQuality" with the removable data types "ClockQualityRm"

-	the removalble attributes "synchronizationState" and "parentTimeSource" with the OpenAPI "nullable: true" property.

Table 5.10.4.1A-1: Definition of type ClockQualityAcceptanceCriterionRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| synchronizationState | array(SynchronizationState) | O | 1..N | Indicates the state of the node synchronization, represented by the values "LOCKED", "HOLDOVER", or "FREERUN" (NOTE) |
| clockQuality | ClockQualityRm | O | 0..1 | Clock Quality |
| parentTimeSource | array(TimeSource) | O | 1..N | Parent Time Source (NOTE) |
| NOTE: The attribute may be removed and need to be defined as nullable:true in the openAPI file. |  |  |  |  |

#### 5.10.4.2	Type: ClockQuality

Table 5.10.4.2-1: Definition of type ClockQuality

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| traceabilityToGnss | boolean | O | 0..1 | true indicates yes false indicates no" |
| traceabilityToUtc | boolean | O | 0..1 | true indicates yes false indicates no |
| frequencyStability | Uint16 | O | 0..1 | see 3GPP TS 23.501 [2] |
| clockAccuracyIndex | string | O | 0..1 | string of two hexadecimal digits; see table 5 of IEEE Std 1588 [51]. |
| clockAccuracyValue | integer | O | 0..1 | Indicates the absolute clock accuracy value. Unit in 25ns. Min: 1 Max: 40000000 |

#### 5.10.4.2A	Type: ClockQualityRm

Describes the modifications to the "ClockQuality" data type. This data type is defined in the same way as the "ClockQuality" data type, but:

-	with the OpenAPI "nullable: true" property;

-	the removable attribute "frequencyStability" with the removable data types "Uint16Rm"

-	the removalble attributes "clockAccuracyValue" and "clockAccuracyIndex" with the OpenAPI "nullable: true" property.

Table 5.10.4.2A-1: Definition of type ClockQualityRm

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| traceabilityToGnss | boolean | O | 0..1 | true indicates yes false indicates no |
| traceabilityToUtc | boolean | O | 0..1 | true indicates yes false indicates no |
| frequencyStability | Uint16Rm | O | 0..1 | see 3GPP TS 23.501 [2] |
| clockAccuracyIndex | string | O | 0..1 | string of two hexadecimal digits; see table 5 of IEEE Std 1588 [51]. (NOTE) |
| clockAccuracyValue | integer | O | 0..1 | Indicates the absolute clock accuracy value. Unit in 25ns. Min: 1 Max: 40000000 (NOTE) |
| NOTE: The attribute may be removed and need to be defined as nullable:true in the openAPI file. |  |  |  |  |

## 5.11	Data Types related to IMS SBA

### 5.11.1	Introduction

This clause defines common data types related to IMS SBA.

### 5.11.2	Simple Data Types

This clause specifies common simple data types.

Table 5.11.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Fingerprint | string | Represents the certificate fingerprint for the DTLS association. The fingerprint is formatted as the pattern defined in IETF RFC 8122 [53]. Pattern: '^(SHA-1¦SHA-224¦SHA-256¦SHA-384¦SHA-512¦MD5¦MD2)\s[0-9A-F]{2}(:[0-9A-F]{2})+$' For example: 'SHA-1 4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB' |
| MediaId | string | Media ID uniquely identifies one media flow within an IMS session. |
| MaxMessageSize | integer | The attribute can be associated with an "m=" line to indicate the maximum SCTP user message size (indicated in bytes) that an SCTP endpoint is willing to receive on the SCTP association associated with the "m=" line. Different attribute values can be used in each direction. The MaxMessageSize is specified in IETF RFC 8841 [55]. |
| SessionId | string | Session ID is used for IMS session identification. When present, the Session ID uniquely identifies the IMS session in a specific IMS service area. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| TlsId | string | Represents the TLS ID for the media stream. The TlsId is formatted as the pattern defined in IETF RFC 8842 [54]. Pattern: '^[A-Za-z0-9+/_-]{20,255}$' For example: 'abc3de65cddef001be82'. |
| ImsTranspInfo | object | To carry transparent objects for IMS exposure services. |

### 5.11.3	Enumerations

#### 5.11.3.1	Enumeration: MediaResourceType

The enumeration MediaResourceType indicates the type of media resource. It shall comply with the provisions defined in Table 5.11.3.1-1.

Table 5.11.3.1-1: Enumeration MediaResourceType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "DC" | Data Channel. |  |
| "AUDIO" | Audio |  |
| "VIDEO" | Video |  |

#### 5.11.3.2	Enumeration: MediaProxy

The enumeration MediaProxy represents the media proxy configuration applicable to the media flow. It shall comply with the provisions defined in Table 5.11.3.2-1.

Table 5.11.3.2-1: Enumeration MediaProxy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "HTTP_PROXY" | Represents the media proxy configuration is HTTP Proxy. |  |
| "UDP_PROXY" | Represents the media proxy configuration is UDP Proxy. |  |
| "DC_APPLICATION_PROXY" | Represents the media proxy configuration is DC Application Proxy. |  |

#### 5.11.3.3	Enumeration: SecuritySetup

The enumeration SecuritySetup represents the security setup of the DTLS connection. It shall comply with the provisions defined in Table 5.11.3.3-1.

Table 5.11.3.3-1: Enumeration SecuritySetup

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ACTIVE" | Represents the endpoint will initiate an outgoing connection. |  |
| "PASSIVE" | Represents the endpoint will accept an incoming connection. |  |
| "ACTPASS" | Represents the endpoint is willing to accept an incoming connection or to initiate an outgoing connection. |  |

#### 5.11.3.4	Enumeration: BdcUsedBy

The enumeration BdcUsedBy represents the party uses the bootstrap data channel in the media description. It shall comply with the provisions defined in Table 5.11.3.4-1.

Table 5.11.3.4-1: Enumeration BdcUsedBy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "SENDER" | Represents the bootstrap data channel is used by the UE sending this SDP. |  |
| "RECEIVER" | Represents the bootstrap data channel is used for the UE receiving the SDP. |  |

#### 5.11.3.5	Enumeration: AdcEndpointType

The enumeration AdcEndpointType represents the remote endpoint type of the application data channel. It shall comply with the provisions defined in Table 5.11.3.5-1.

Table 5.11.3.5-1: Enumeration AdcEndpointType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "UE" | Represents the remote endpoint type of the application data channel is the peer UE. |  |
| "SERVER" | Represents the remote endpoint type of the application data channel is the application server. |  |

#### 5.11.3.6	Enumeration: ImsEvent

The enumeration ImsEvent represents the IMS events. It shall comply with the provisions defined in Table 5.11.3.6-1.

Table 5.11.3.6-1: Enumeration ImsEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ADC_ESTABLISHMENT" | The application data channel establishment event. |  |
| "ADC_RELEASE" | The application data channel release event. |  |
| "BDC_ESTABLISHMENT" | The bootstrap data channel establishment event. |  |
| "BDC_RELEASE" | The bootstrap data channel release event. |  |
| "ADC_IWK_ESTABLISHMENT" | The application data channel requiring interworking establishment event. |  |

NOTE:	Values in table 5.11.3.6-1 include the standardized IMS event types. However, the actual values of IMS event types and corresponding event filters are transparent for the different NF service producers (e.g. HSS). This allows that additional standardized as well as non-standardized event types can be accepted even if the semantics are unknown by e.g. the NRF, HSS, NEF.

#### 5.11.3.7	Type: AdcStatus

Table 5.11.3.7-1: Enumeration AdcStatus

| Enumeration value | Description |
| --- | --- |
| "ADC_ESTABLISHED" | Application data channel established. |
| "ADC_RELEASED" | Application data channel released. |
| "ADC_IWK_ESTABLISHED" | Application data channel requiring interworking established. |

#### 5.11.3.8	Type: BdcStatus

Table 5.11.3.8-1: Enumeration BdcStatus

| Enumeration value | Description |
| --- | --- |
| "BDC_ESTABLISHED" | Bootstrap data channel established. |
| "BDC_RELEASED" | Bootstrap data channel released. |

#### 5.11.3.9	Enumeration: ImsReportMode

Table 5.11.3.9-1: Enumeration ImsReportMode

| Enumeration value | Description |
| --- | --- |
| "ON_EVENT_DETECTION" | The report is sent based on event detection. |
| "PERIODIC" | The report is periodically sent. |

#### 5.11.3.10	Enumeration: RenderingMode

The enumeration RenderingMode represents the rendering mode for avatar media. It shall comply with the provisions defined in Table 5.11.3.10-1.

Table 5.11.3.10-1: Enumeration RenderingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "UE_CENTRIC" | The UE centric rendering mode for avatar media. |  |
| "NET_CENTRIC_MF" | The network centric with MF rendering mode for avatar media. |  |
| "NET_CENTRIC_DCAS" | The network centric with DCAS rendering mode for avatar media. |  |

### 5.11.4	Structured Data Types

#### 5.11.4.1	Type: DcEndpoint

Table 5.11.4.1-1: Definition of type DcEndpoint

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| sctpPort | integer | M | 1 | Represent the local or remote port for the Data Channel. The SctpPort is formatted as the pattern defined in IETF RFC 8841 [55]. Minimum = 0. Maximum = 65535. |
| fingerprint | Fingerprint | O | 0..1 | Represents the local or remote certificate fingerprint for the DTLS association. This attribute is deprecated and replaced by the "fingerprints" attribute.. |
| fingerprints | array(Fingerprint) | O | 1..N | Represents the certificate fingerprints for the DTLS association. |
| tlsId | TlsId | O | 0..1 | Represents the local or remote TLS ID for the media stream. |
| securitySetup | SecuritySetup | O | 0..1 | Represents the security set up of the DTLS association. |

#### 5.11.4.2	Type: DcStream

Table 5.11.4.2-1: Definition of type DcStream

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| streamId | integer | M | 1 | Identifies the data channel stream. The value range of the stream id is an unsigned 16-bit integer, i.e. 0 to 65535. Minimum = 0. Maximum = 65535. The 0-999 is used for Bootstrap Data Channel. The 1000-65535 is used for Application Data Channel. |
| subprotocol | string | O | 0..1 | Represents the subprotocol of the SCTP stream. It defaults to "http" if the mediaId represents bootstrap data channel. (NOTE 1) |
| order | boolean | O | 0..1 | Represents the stream is ordered or not, "true" for ordered delivery and "false" for unordered delivery. (NOTE 1) |
| maxRetry | integer | O | 0..1 | Represents the maximal number of the times a message will be retransmitted. Default value: 0 (NOTE 1) (NOTE 2) |
| maxTime | integer | O | 0..1 | Represents the maximal lifetime in milliseconds after which a message will no longer be transmitted or retransmitted. Default value: 0. (NOTE 1) (NOTE 2) |
| priority | integer | O | 0..1 | Represents the priority of data channel relative to other data channels. Default value: 256. (NOTE 1) |
| NOTE 1: The IE cannot be changed once the media has been established. NOTE 2: At most one of the two attributes shall be present. |  |  |  |  |

#### 5.11.4.3	Type: ReplaceHttpUrl

Table 5.11.4.3-1: Definition of type ReplaceHttpUrl

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| replaceHttpUrl | Uri | M | 1 | Represents the replacement HTTP URL per stream ID allocated by the application layer for the specific IMS subscriber when requesting the application list (e.g., graphical user interface) via the MDC1 interface. |
| streamId | integer | M | 1 | Represents the stream ID that the replaceHttpUrl apply to. The value range of the stream id is an unsigned 16-bit integer, i.e. 0 to 65535. Minimum = 0. Maximum = 65535. This attribute can only set to 0 or 100 here for Bootstrap Data Channel. |

#### 5.11.4.4	Type: Endpoint

Table 5.11.4.4-1: Definition of type Endpoint

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ip | IpAddr | M | 1 | Represents the IP address of the endpoint. |  |
| transport | TransportProtocol | M | 1 | Represents the transport protocol. |  |
| portNumber | Uinteger | M | 1 | Represents the TCP or UDP port number of the endpoint. |  |

#### 5.11.4.5	Type: AppBindingInfo

Table 5.11.4.5-1: Definition of type AppBindingInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| applicationId | string | M | 1 | Identifies the application. |
| appDcInfo | AppDcInfo | O | 0..1 | Represents the application data channel is intened towards to a server or the remote UE. This attribute is deprecated; the attribute "appDcInfoList" should be used instead. |
| appDcInfoList | array(AppDcInfo) | O | 1..N | This attribute provides information of the remote endpoint type of application data channels. |

#### 5.11.4.6	Type: AppDcInfo

Table 5.11.4.6-1: Definition of type AppDcInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| streamId | integer | M | 1 | Represents the stream ID of the data channel. |
| adcEndpointType | AdcEndpointType | O | 0..1 | Represents the remote endpoint type of the application data channel |

#### 5.11.4.7	Type: MdcEndpoint

Table 5.11.4.7-1: Definition of type MdcEndpoint

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ip | IpAddr | M | 1 | Represents the IP address of the endpoint. |  |
| portNumber | Uinteger | M | 1 | Represents the TCP or UDP or SCTP port number over IP layer. |  |
| sctpPort | Uinteger | C | 0..1 | Represents the SCTP port number over DTLS. The sctpPort is formatted as the pattern defined in IETF RFC 8841 [55]. Minimum = 0. Maximum = 65535. This attribute shall be present for SCTP connection over DTLS. |  |
| fingerprint | Fingerprint | C | 0..1 | Represents the certificate fingerprint for the DTLS association. This attribute shall be present for DTLS connection. This attribute is deprecated and replaced by the "fingerprints" attribute. |  |
| fingerprints | array(Fingerprint) | C | 1..N | Represents the certificate fingerprints for the DTLS association. This attribute shall be present for DTLS connection. |  |
| tlsId | TlsId | C | 0..1 | Represents the TLS ID for the DTLS connection. |  |
| securitySetup | SecuritySetup | O | 0..1 | Represents the security set up of the DTLS association. This attribute may be present for DTLS connection. |  |

#### 5.11.4.8	Type: ImsReportingOptions

Table 5.11.4.8: Definition of type ImsReportingOptions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportMode | ImsReportMode | O | 0..1 | Indicates the mode of report (e.g, periodic reporting along with periodicity, reporting based on event detection). See clause 4.15.1 of 3GPP TS23.502 [3]. If this attribute is absent, the default reportMode shall be "ON_EVENT_DETECTION". It shall not be included if oneTimeReport is included. |
| oneTimeReport | boolean | O | 0..1 | This IE may be included only if ImmediateReport is requested. - true: Indicates that a single notification or status report is requested. - false or absent: Indicates that continuous notification is requested after the immediate report (if any) with the current associated status is returned in the response. |
| oneTimeNotification | boolean | O | 0..1 | - true: Indicates that a single notification (one-time notification) is requested. - false or absent: Indicates that continuous notification is requested. (NOTE 1) |
| maxNumOfReports | Integer | C | 0..1 | Maximum number of reports. If the event subscription is for a group of UEs, this parameter shall be applied to each individual member UE of the group. (NOTE 2) |
| reportPeriod | DurationSec | C | 0..1 | Indicates the interval time between which the event notification is reported, shall be present if reportMode is "PERIODIC". |
| expiry | DateTime | C | 0..1 | This IE shall be included in an event subscription response, if, based on operator policy, the IMS AS needs to include an expiry time, and may be included in an event subscription request. When present, this IE shall represent the time at which monitoring shall cease and the subscription becomes invalid. If the maxNumOfReports included in an event subscription response is 1 and if an event report is included in the subscription response, then the value of the expiry included in the response shall be an immediate timestamp. (NOTE 2) |
| NOTE 1: If immediate report is requested and the current status associated to the event is returned, it shall not be considered a notification report about the event detection for e.g. maximum number of reports. NOTE 2: If both "maxNumOfReports" and "expiry" are included, the subscription will expire as soon as one of the conditions is met. If the reportMode attribute is set to "PERIODIC", at least one of the "maxNumOfReports" and "expiry" attributes shall be included. If “oneTimeReport” is included, “maxNumOfReports” shall not be included. |  |  |  |  |

#### 5.11.4.9	Type: ImsEventConfiguration

Table 5.11.4.9-1: Definition of type ImsEventConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsEvent | ImsEvent | M | 1 | The type of the subscribed event. |
| eventInfo | EventInformation | O | 0..1 | Additional configuration information related to the specific event type. |
| reqCurrentStatus | boolean | O | 0..1 | - true: Current status related to the event subscribed (if available) is requested to be provided in the response. - false (default): Current status related to the event subscribed (if available) is not requested to be provided in the response. (NOTE 1, NOTE 2) |
| imsEventFilters | array(ImsEventFilter) | O | 1..N | List of IMS event filters. |
| NOTE 1: The current status related to the subscribed event shall not be considered as a notification report about the event detection for e.g. Maximum number of reports. NOTE 2: This IE correspond to Immediate Report Indication, see 3GPP TS 23.502 [28], table 4.15.1-1 |  |  |  |  |

#### 5.11.4.10	Type: EventInformation

Table 5.11.4.10-1: Definition of type EventInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| imsTranspEventConfig | ImsTranspInfo | O | 0..1 | Transparent event configuration for the IMS event |

#### 5.11.4.11	Type: ImsEventFilter

Table 5.11.4.11-1: Definition of type ImsEventFilter

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| callingId | string | O | 0..1 | Represents the public identity of the calling subscriber. String containing an IMS Public Identity in SIP URI format or TEL URI format, as specified 3GPP TS 23.003 [7] Pattern: "^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦tel\:\+[0-9]{5,15})$" |
| calledId | string | O | 0..1 | Represents the public identity of the called subscriber. String containing an IMS Public Identity in SIP URI format or TEL URI format, as specified 3GPP TS 23.003 [7] Pattern: "^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦tel\:\+[0-9]{5,15})$" |
| appBinInfo | AppBindingInfo | O | 0..1 | Represents the data channel application binding information. It may be included if the event is "ADC_ESTABLISHMENT", "ADC_RELEASE" or "ADC_IWK_ESTABLISHMENT". |
| sessionId | SessionId | O | 0..1 | The IMS session ID. |
| imsTranspFilter | ImsTranspInfo | O | 0..1 | Transparent filter for the IMS event |

Editor's Note:	It is FFS to determine whether calling and called IDs should allow multiple identities (i.e. if they should be modelled as an array of PublicIdentities).

#### 5.11.4.12	Type: ImsEventReport

Table 5.11.4.12-1: Definition of type ImsEventReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| event | ImsEvent | M | 1 | Event that is reported. |
| eventReport | ImsEventReportInfo | O | 0..1 | Report Information per event type. |
| sessionId | SessionId | O | 0..1 | Identifies the session ID. |

#### 5.11.4.13	Type: ImsEventReportInfo

Table 5.11.4.13-1: Definition of type ImsEventReportInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| adcReport | AdcReport | O | 0..1 | Report application data channel. |
| bdcReport | BdcReport | O | 0..1 | Report bootstrap data channel. |
| imsTranspReport | ImsTranspInfo | O | 0..1 | Transparent Report for the IMS event |

#### 5.11.4.14	Type: AdcReport

Table 5.11.4.14-1: Definition of type AdcReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| adcStatus | AdcStatus | M | 1 | ADC event status. |

#### 5.11.4.15	Type: BdcReport

Table 5.11.4.15-1: Definition of type BdcReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bdcStatus | BdcStatus | M | 1 | BDC event status. |

#### 5.11.4.16	Type: RcdProperties

Table 5.11.4.16-1: Definition of type RcdProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdSrvAddr | ServerAddressingInfo | O | 0..1 | Represents the address of an RCD server in a third party network storing IMS subscriber specific RCD information. |
| rcdUrl | Uri | O | 0..1 | Represents the URL from where RCD information of a specific IMS subscriber stored at an RCD server can be retrieved from. |
| rcdInfo | RcdInformation | O | 0..1 | Indicates a collection of RCD properties associated with an IMS subscriber e.g. caller name, company name, e-mail address, telephone number, job title as specified in IETF RFC 9796 [75] and represented as a jCard JSON object. |

#### 5.11.4.17	Type: RcdInformation

Table 5.11.4.17-1: Definition of type RcdInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| identifProps | IdentificationProperties | M | 1 | Represents the identity information of the entity associated with the jCard. |
| delvAddrProps | DeliveringAddrProperties | O | 0..1 | Represents the information related to the delivery address of the entity associated with the jCard. |
| commsProps | CommunicationsProperties | O | 0..1 | Indicates how to communicate with the entity associated with the jCard. |
| geogrProps | GeographicalProperties | O | 0..1 | Represents the geographical information associated with the entity associated with the jCard. |
| organProps | OrganizationalProperties | O | 0..1 | Represents the information associated with characteristics of the organization or organizational units of the entity associated with the jCard. |
| explanProps | ExplanatoryProperties | O | 0..1 | Indicates an additional information such as an application category information or notes that are specific to the entity associated with the jCard. |

#### 5.11.4.18	Type: IdentificationProperties

Table 5.11.4.18-1: Definition of type IdentificationProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| fns | array(string) | M | 1..N | Represent the "fn" property which provides a formatted text corresponding to the name of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.2.1. |
| nProp | string | O | 0..1 | Represent the "n" property which provides the components of the name of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.2.2. Value type: A single structured text value. The text components are separated by a semicolon character. Each component may have multiple values separated by a comma character. |
| nickNames | array(string) | O | 1..N | Represent the "nickname" property which provides the text corresponding to the nickname of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.2.3. Value type: One or more text values separated by a comma character. |
| photos | array(Uri) | O | 1..N | Represent the "photo" property which provides image or photograph information that annotates some aspect of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.2.4. |

#### 5.11.4.19	Type: DeliveringAddrProperties

Table 5.11.4.19-1: Definition of type DeliveringAddrProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| adrs | array(string) | O | 1..N | Represent the "adr" property which provides the delivery address of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.3.1. Value type: A single structured text value separated by a semicolon character. Each component may have multiple values separated by a comma character. |

#### 5.11.4.20	Type: CommunicationsProperties

Table 5.11.4.20-1: Definition of type CommunicationsProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tels | array(string) | O | 1..N | Represent the "tel" property which provides the telephone number for the object the jCard represents as defined in IETF RFC 6350 [76], section 6.4.1. Value type: By default, it is a single free-form text value. |
| emails | array(string) | O | 1..N | Represent the "email" property which provides the electronic mail address of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.4.2. Value type: A single text value. |
| langs | array(string) | O | 1..N | Represent the "lang" property which provides the language(s) that may be used for communicating with the object the jCard represents as defined in IETF RFC 6350 [76], section 6.4.4. Value type: A single language-tag value. |

#### 5.11.4.21	Type: GeographicalProperties

Table 5.11.4.21-1: Definition of type GeographicalProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| tzs | array(string) | O | 1..N | Represent the "tz" property which provides the time zone of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.5.1. Value type: The default is a single text value. It can also be reset to a single URI or a UTC-offset value. |
| geos | array(Uri) | O | 1..N | Represent the "geo" property which provides the global positioning of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.5.2. Value type: A single URI. |

#### 5.11.4.22	Type: OrganizationalProperties

Table 5.11.4.22-1: Definition of type OrganizationalProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| titles | array(string) | O | 1..N | Represent the "title" property which has the intent of providing the position or job of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.6.1. Value type: A single text value. |
| roles | array(string) | O | 1..N | Represent the "role" property which has the intent of providing the position or job of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.6.2. Value type: A single text value. |
| logos | array(Uri) | O | 1..N | Represent the "logo" property which has the intent of specifying a graphic image of a logo associated with the object the jCard represents as defined in IETF RFC 6350 [76], section 6.6.3. Value type: A single URI. |
| orgs | array(string) | O | 1..N | Represent the "org" property which has the intent of specifying the organizational name and units of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.6.4. Value type: A single structured text value consisting of components separated by a semicolon character. |

#### 5.11.4.23	Type: ExplanatoryProperties

Table 5.11.4.23-1: Definition of type ExplanatoryProperties

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| categories | array(string) | O | 1..N | Represent the "categories" property which specifies application category information about the object the jCard represents as defined in IETF RFC 6350 [76], section 6.7.1. Value type: One or more text values separated by a comma character. |
| notes | array(string) | O | 1..N | Represent the "note" property which specifies supplemental information or a comment about the object the jCard represents as defined in IETF RFC 6350 [76], section 6.7.2. Value type: A single text value. |
| sounds | array(Uri) | O | 1..N | Represent the "sound" property which specifies digital sound content information that annotates some aspect of the object the jCard represents as defined in IETF RFC 6350 [76], section 6.7.5. Value type: A single URI. |
| uid | Uri | O | 0..1 | Represent the "uid" property which specifies a globally unique identifier corresponding to the object the jCard represents as defined in IETF RFC 6350 [76], section 6.7.6. Value type: A single URI value. It MAY also be reset to free-form text. |
| urls | array(Uri) | O | 1..N | Represent the "url" property which specifies a uniform resource locator associated with the object the jCard represents as defined in IETF RFC 6350 [76], section 6.7.8. Value type: A single uri value. |
| version | string | M | 1 | Represent the "version" property which specifies the version of the vCard specification used to format this vCard as defined in IETF RFC 6350 [76], section 6.7.9. Value type: A single text value. |

## 5.12	Data Types related to Ambient IoT

### 5.12.1	Introduction

This clause defines common data types related to Ambient IoT.

### 5.12.2	Simple Data Types

This clause specifies common simple data types.

Table 5.12.2-1: Simple Data Types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| AiotAreaCode | string | Represents the AIoT area code encoded as a three octet string in hexadecimal representation. Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits shall appear first in the string, and the character representing the 4 least significant bit shall appear last in the string. Octets shall be coded according to 3GPP TS 32.422 [19]. Pattern: '^[A-Fa-f0-9]{6}$' . |
| AiotFilteringInformation | Bytes | String that uniquely represents an AIoT Filtering Information which is used to identify or filter multiple AIoT Devices. See clause 31.3 of 3GPP TS 23.003 [7]. String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters. |
| AiotDevPermId | Bytes | String that uniquely represents an AIoT Device Permanent Identifier, as specified in clause 31.2 of 3GPP TS 23.003 [7]. String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters |

### 5.12.3	Enumerations

### 5.12.4	Structured Data Types

#### 5.12.4.1	Type: AiotArea

Table 5.12.4.1-1: Definition of type AiotArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| areaIds | array(AiotAreaId) | M | 1..N | Contains a list of AIoT Area ID(s) which represent the AIoT Service Area. |

#### 5.12.4.2	Type: AiotAreaId

Table 5.12.4.2-1: Definition of type AiotAreaId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnId | PlmnId | M | 1 | PLMN ID |
| nid | Nid | O | 0..1 | Network identity |
| aiotAreaCode | AiotAreaCode | M | 1 | AIoT area code |

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of common data types. It consists of an OpenAPI 3.0.0 specification, in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository, that uses the GitLab software version control system (see 3GPP TS 29.501 [2] clause 5.3.1 and 3GPP TR 21.900 [27] clause 5B)

# A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.6.0-alpha.5'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.571 Common Data Types for Service Based Interfaces, version 19.4.0

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.571/'

paths: {}

components:

schemas:

#

# Common Data Types for Generic usage definitions as defined in clause 5.2

#

#

# COMMON SIMPLE DATA TYPES

#

Binary:

format: binary

type: string

description: string with format 'binary' as defined in OpenAPI.

BinaryRm:

format: binary

type: string

nullable: true

description: >

"string with format 'binary' as defined in OpenAPI OpenAPI with 'nullable: true' property."

Bytes:

format: byte

type: string

description: string with format 'bytes' as defined in OpenAPI

BytesRm:

format: byte

type: string

nullable: true

description: >

string with format 'bytes' as defined in OpenAPI OpenAPI with 'nullable: true' property.

Date:

format: date

type: string

description: string with format 'date' as defined in OpenAPI.

DateRm:

format: date

type: string

nullable: true

description: >

string with format 'date' as defined in OpenAPI OpenAPI with 'nullable: true' property.

DateTime:

format: date-time

type: string

description: string with format 'date-time' as defined in OpenAPI.

DateTimeRm:

format: date-time

type: string

nullable: true

description: >

string with format 'date-time' as defined in OpenAPI with 'nullable:true' property.

DiameterIdentity:

$ref: '#/components/schemas/Fqdn'

DiameterIdentityRm:

$ref: '#/components/schemas/FqdnRm'

Double:

format: double

type: number

description: string with format 'double' as defined in OpenAPI

DoubleRm:

format: double

type: number

nullable: true

description: >

string with format 'double' as defined in OpenAPI with 'nullable: true' property.

DurationSec:

type: integer

description: indicating a time in seconds.

DurationSecRm:

type: integer

nullable: true

description: "indicating a time in seconds with OpenAPI defined 'nullable: true' property."

Float:

format: float

type: number

description: string with format 'float' as defined in OpenAPI.

FloatRm:

format: float

type: number

nullable: true

description: >

string with format 'float' as defined in OpenAPI with the OpenAPI defined

'nullable: true' property.

Int32:

format: int32

type: integer

description: string with format 'int32' as defined in OpenAPI.

Int32Rm:

format: int32

type: integer

nullable: true

description: >

string with format 'int32' as defined in OpenAPI with the OpenAPI defined

'nullable: true' property.

Int64:

type: integer

format: int64

description: string with format 'int64' as defined in OpenAPI.

Int64Rm:

format: int64

type: integer

nullable: true

description: >

string with format 'int64' as defined in OpenAPI with the OpenAPI defined

'nullable: true' property.

Ipv4Addr:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

description: >

String identifying a IPv4 address formatted in the 'dotted decimal' notation

as defined in RFC 1166.

Ipv4AddrRm:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

nullable: true

description: >

String identifying a IPv4 address formatted in the 'dotted decimal' notation

as defined in RFC 1166 with the OpenAPI defined 'nullable: true' property.

Ipv4AddrMask:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])(\/([0-9]|[1-2][0-9]|3[0-2]))$'

example: '198.51.0.0/16'

description: >

"String identifying a IPv4 address mask formatted in the 'dotted decimal' notation

as defined in RFC 1166."

Ipv4AddrMaskRm:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])(\/([0-9]|[1-2][0-9]|3[0-2]))$'

example: '198.51.0.0/16'

nullable: true

description: >

String identifying a IPv4 address mask formatted in the 'dotted decimal' notation

as defined in RFC 1166 with the OpenAPI defined 'nullable: true' property.

Ipv6Addr:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

description: >

String identifying an IPv6 address formatted according to clause 4 of RFC5952.

The mixed IPv4 IPv6 notation according to clause 5 of RFC5952 shall not be used.

Ipv6AddrRm:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

nullable: true

description: >

String identifying an IPv6 address formatted according to clause 4 of RFC5952 with the

OpenAPI 'nullable: true' property.

The mixed IPv4 IPv6 notation according to clause 5 of RFC5952 shall not be used.

Ipv6Prefix:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

description: >

String identifying an IPv6 address prefix formatted according to clause 4 of RFC 5952.

IPv6Prefix data type may contain an individual /128 IPv6 address.

Ipv6PrefixRm:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

nullable: true

description: >

String identifying an IPv6 address prefix formatted according to clause 4 of RFC 5952 with

the OpenAPI 'nullable: true' property. IPv6Prefix data type may contain an individual

/128 IPv6 address.

MacAddr48:

type: string

pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'

description: >

String identifying a MAC address formatted in the hexadecimal notation

according to clause 1.1 and clause 2.1 of RFC 7042.

MacAddr48Rm:

type: string

pattern: '^([0-9a-fA-F]{2})((-[0-9a-fA-F]{2}){5})$'

nullable: true

description: >

"String identifying a MAC address formatted in the hexadecimal notation according to

clause 1.1 and clause 2.1 of RFC 7042 with the OpenAPI 'nullable: true' property."

SupportedFeatures:

type: string

pattern: '^[A-Fa-f0-9]*$'

description: >

A string used to indicate the features supported by an API that is used as defined in clause

6.6 in 3GPP TS 29.500. The string shall contain a bitmask indicating supported features in

hexadecimal representation Each character in the string shall take a value of "0" to "9",

"a" to "f" or "A" to "F" and shall represent the support of 4 features as described in

table 5.2.2-3. The most significant character representing the highest-numbered features

shall  appear first in the string, and the character representing features 1 to 4

shall appear last  in the string. The list of features and their numbering (starting with 1)

are defined  separately for each API. If the string contains a lower number of characters

than there are  defined features for an API, all features that would be represented by

characters that are not  present in the string are not supported.

Uinteger:

type: integer

minimum: 0

description: Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

UintegerRm:

type: integer

minimum: 0

description: >

Unsigned Integer, i.e. only value 0 and integers above 0 are permissible with

the OpenAPI 'nullable: true' property.

nullable: true

Uint16:

type: integer

minimum: 0

maximum: 65535

description: >

Integer where the allowed values correspond to the value range of an

unsigned 16-bit integer.

Uint16Rm:

type: integer

minimum: 0

maximum: 65535

nullable: true

description: >

Integer where the allowed values correspond to the value range of an unsigned

16-bit integer with the OpenAPI 'nullable: true' property.

Uint32:

type: integer

minimum: 0

maximum: 4294967295 #(2^32)-1

description: >

Integer where the allowed values correspond to the value range of an unsigned

32-bit integer.

Uint32Rm:

format: int32

type: integer

minimum: 0

maximum: 4294967295 #(2^32)-1

nullable: true

description: >

Integer where the allowed values correspond to the value range of an unsigned

32-bit integer with the OpenAPI 'nullable: true' property.

Uint64:

type: integer

minimum: 0

maximum: 18446744073709551615 #(2^64)-1

description: >

Integer where the allowed values correspond to the value range of an

unsigned 64-bit integer.

Uint64Rm:

type: integer

minimum: 0

maximum: 18446744073709551615 #(2^64)-1

nullable: true

description: >

Integer where the allowed values correspond to the value range of an unsigned

16-bit integer with the OpenAPI 'nullable: true' property.

Uri:

type: string

description: String providing an URI formatted according to RFC 3986.

UriRm:

type: string

nullable: true

description: >

String providing an URI formatted according to RFC 3986 with the OpenAPI

'nullable: true' property.

VarUeId:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|gci-.+|gli-.+|.+)$'

description: String represents the SUPI or GPSI

VarUeIdRm:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|gci-.+|gli-.+|.+)$'

nullable: true

description: "String represents the SUPI or GPSI with the OpenAPI 'nullable: true' property."

TimeZone:

type: string

example: '-08:00+1'

description: |

String with format "time-numoffset" optionally appended by "daylightSavingTime", where

- "time-numoffset" shall represent the time zone adjusted for daylight saving time and be

encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339;

- "daylightSavingTime" shall represent the adjustment that has been made and shall be

encoded as "+1" or "+2" for a +1 or +2 hours adjustment.

The example is for 8 hours behind UTC, +1 hour adjustment for Daylight Saving Time.

TimeZoneRm:

type: string

nullable: true

description: |

"String with format 'time-numoffset' optionally appended by '<daylightSavingTime>', where

- 'time-numoffset' shall represent the time zone adjusted for daylight saving time and be

encoded as time-numoffset as defined in clause 5.6 of IETF RFC 3339;

- 'daylightSavingTime' shall represent the adjustment that has been made and shall be

encoded as '+1' or '+2' for a +1 or +2 hours adjustment.

But with the OpenAPI 'nullable: true' property."

StnSr:

type: string

description: String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003.

StnSrRm:

type: string

nullable: true

description: >

String representing the STN-SR as defined in clause 18.6 of 3GPP TS 23.003

with the OpenAPI 'nullable: true' property.

CMsisdn:

type: string

pattern: '^[0-9]{5,15}$'

description: String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003.

CMsisdnRm:

type: string

pattern: '^[0-9]{5,15}$'

nullable: true

description: >

String representing the C-MSISDN as defined in clause 18.7 of 3GPP TS 23.003 with

the OpenAPI 'nullable: true' property.

MonthOfYear:

type: integer

minimum: 1

maximum: 12

description: >

integer between and including 1 and 12 denoting a month. 1 shall indicate January,

and the subsequent months shall be indicated with the next higher numbers.

12 shall indicate December.

DayOfWeek:

type: integer

minimum: 1

maximum: 7

description: >

integer between and including 1 and 7 denoting a weekday. 1 shall indicate Monday,

and the subsequent weekdays shall be indicated with the next higher numbers.

7 shall indicate Sunday.

TimeOfDay:

type: string

description: >

String with format partial-time or full-time as defined in clause 5.6 of IETF RFC 3339.

Examples, 20:15:00, 20:15:00-08:00 (for 8 hours behind UTC).

EmptyObject:

description: Empty JSON object { }, it is defined with the keyword additionalProperties false

type: object

additionalProperties: false

Fqdn:

description: Fully Qualified Domain Name

type: string

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

FqdnRm:

description: Fully Qualified Domain Name, but it also allows the null value

anyOf:

- $ref: '#/components/schemas/Fqdn'

- $ref: '#/components/schemas/NullValue'

#

#   COMMON ENUMERATED DATA TYPES

#

PatchOperation:

anyOf:

- type: string

enum:

- add

- copy

- move

- remove

- replace

- test

- type: string

description: Operations as defined in IETF RFC 6902.

UriScheme:

anyOf:

- type: string

enum:

- http

- https

- type: string

description: HTTP and HTTPS URI scheme.

ChangeType:

anyOf:

- type: string

enum:

- ADD

- MOVE

- REMOVE

- REPLACE

- type: string

description: Indicates the type of change to be performed.

HttpMethod:

anyOf:

- type: string

enum:

- GET

- POST

- PUT

- DELETE

- PATCH

- OPTIONS

- HEAD

- CONNECT

- TRACE

- type: string

description: HTTP methodes.

NullValue:

enum:

- null

description: JSON's null value.

MatchingOperator:

anyOf:

- type: string

enum:

- FULL_MATCH

- MATCH_ALL

- STARTS_WITH

- NOT_START_WITH

- ENDS_WITH

- NOT_END_WITH

- CONTAINS

- NOT_CONTAIN

- type: string

description: the matching operation.

UpfPacketInspectionFunctionality:

description: Indicates a packet inspection functionality of the UPF.

anyOf:

- type: string

enum:

- DEEP_PACKET_INSPECTION

- MAC_FILTER_BASED_PACKET_DETECTION

- IP_FILTER_BASED_PACKET_DETECTION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration.

#

#  COMMON STRUCTURED DATA TYPES

#

ProblemDetails:

description: Provides additional information in an error response.

type: object

properties:

type:

$ref: '#/components/schemas/Uri'

title:

type: string

status:

type: integer

detail:

type: string

description: A human-readable explanation specific to this occurrence of the problem.

instance:

$ref: '#/components/schemas/Uri'

cause:

type: string

description: >

A machine-readable application error cause specific to this occurrence of the problem.

This IE should be present and provide application-related error information, if

available.

invalidParams:

type: array

items:

$ref: '#/components/schemas/InvalidParam'

minItems: 1

supportedFeatures:

$ref: '#/components/schemas/SupportedFeatures'

accessTokenError:

$ref: 'TS29510_Nnrf_AccessToken.yaml#/components/schemas/AccessTokenErr'

accessTokenRequest:

$ref: 'TS29510_Nnrf_AccessToken.yaml#/components/schemas/AccessTokenReq'

nrfId:

$ref: '#/components/schemas/Fqdn'

supportedApiVersions:

type: array

items:

type: string

minItems: 1

noProfileMatchInfo:

$ref: 'TS29510_Nnrf_NFDiscovery.yaml#/components/schemas/NoProfileMatchInfo'

Link:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

description: It contains the URI of the linked resource.

LinkRm:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

nullable: true

description: >

It contains the URI of the linked resource with the OpenAPI 'nullable: true' property.

PatchItem:

type: object

properties:

op:

$ref: '#/components/schemas/PatchOperation'

path:

type: string

description: >

contains a JSON pointer value (as defined in IETF RFC 6901) that references

a location of a resource on which the patch operation shall be performed.

from:

type: string

description: >

indicates the path of the source JSON element (according to JSON Pointer syntax)

being moved or copied to the location indicated by the "path" attribute.

value: {}

required:

- op

- path

description: it contains information on data to be changed.

LinksValueSchema:

oneOf:

- type: array

items:

$ref: '#/components/schemas/Link'

minItems: 1

- $ref: '#/components/schemas/Link'

description: A list of mutually exclusive alternatives of 1 or more links.

SelfLink:

type: object

properties:

self:

$ref: '#/components/schemas/Link'

required:

- self

description: It contains the URI of the linked resource.

InvalidParam:

type: object

properties:

param:

type: string

description: >

If the invalid parameter is an attribute in a JSON body, this IE shall contain the

attribute's name and shall be encoded as a JSON Pointer. If the invalid parameter is

an HTTP header, this IE shall be formatted as the concatenation of the string "header "

plus the name of such header. If the invalid parameter is a query parameter, this IE

shall be formatted as the concatenation of the string "query " plus the name of such

query parameter. If the invalid parameter is a variable part in the path of a resource

URI, this IE shall contain the name of the variable, including the symbols "{" and "}"

used in OpenAPI specification as the notation to represent variable path segments.

reason:

type: string

description: >

A human-readable reason, e.g. "must be a positive integer". In cases involving failed

operations in a PATCH request, the reason string should identify the operation that

failed using the operation's array index to assist in correlation of the invalid

parameter with the failed operation, e.g." Replacement value invalid for attribute

(failed operation index= 4)"

required:

- param

description: It contains an invalid parameter and a related description.

ChangeItem:

type: object

properties:

op:

$ref: '#/components/schemas/ChangeType'

path:

type: string

description: >

contains a JSON pointer value (as defined in IETF RFC 6901) that references a target

location within the resource on which the change has been applied.

from:

type: string

description: >

indicates the path of the source JSON element (according to JSON Pointer syntax)

being moved or copied to the location indicated by the "path" attribute. It shall

be present if the "op" attribute is of value "MOVE".

origValue: {}

newValue: {}

required:

- op

- path

description: It contains data which need to be changed.

NotifyItem:

type: object

required:

- resourceId

- changes

properties:

resourceId:

$ref: '#/components/schemas/Uri'

changes:

type: array

items:

$ref: '#/components/schemas/ChangeItem'

minItems: 1

description: Indicates changes on a resource.

ComplexQuery:

oneOf:

- $ref: '#/components/schemas/Cnf'

- $ref: '#/components/schemas/Dnf'

description: >

The ComplexQuery data type is either a conjunctive normal form or a disjunctive normal form.

The attribute names "cnfUnits" and "dnfUnits" (see clause 5.2.4.11 and clause 5.2.4.12)

serve as discriminator.

Cnf:

type: object

required:

- cnfUnits

properties:

cnfUnits:

type: array

items:

$ref: '#/components/schemas/CnfUnit'

minItems: 1

description: A conjunctive normal form

Dnf:

type: object

required:

- dnfUnits

properties:

dnfUnits:

type: array

items:

$ref: '#/components/schemas/DnfUnit'

minItems: 1

description: A disjunctive normal form.

CnfUnit:

type: object

required:

- cnfUnit

properties:

cnfUnit:

type: array

items:

$ref: '#/components/schemas/Atom'

minItems: 1

description: >

During the processing of cnfUnits attribute, all the members in the array shall be

interpreted as logically concatenated with logical "AND".

DnfUnit:

type: object

required:

- dnfUnit

properties:

dnfUnit:

type: array

items:

$ref: '#/components/schemas/Atom'

minItems: 1

description: >

During the processing of dnfUnits attribute, all the members in the array shall be

interpreted as logically concatenated with logical "OR".

Atom:

description: contains a search parameter and its positive or negative content.

type: object

required:

- attr

- value

properties:

attr:

type: string

description: contains the name of a defined query parameter.

value: {}

negative:

type: boolean

description: indicates whether the negative condition applies for the query condition.

PatchResult:

description: The execution report result on failed modification.

type: object

required:

- report

properties:

report:

type: array

items:

$ref: '#/components/schemas/ReportItem'

minItems: 1

description: >

The execution report contains an array of report items. Each report item indicates one

failed modification.

ReportItem:

type: object

required:

- path

properties:

path:

type: string

description: >

Contains a JSON pointer value (as defined in IETF RFC 6901) that references a

location of a resource to which the modification is subject.

reason:

type: string

description: >

A human-readable reason providing details on the reported modification failure.

The reason string should identify the operation that failed using the operation's

array index to assist in correlation of the invalid parameter with the failed

operation, e.g. "Replacement value invalid for attribute (failed operation index= 4)".

description: indicates performed modivications.

HalTemplate:

description: >

Hypertext Application Language (HAL) template contains the extended 3GPP hypermedia format.

type: object

required:

- method

properties:

title:

type: string

description: A human-readable string that can be used to identify this template

method:

$ref: '#/components/schemas/HttpMethod'

contentType:

type: string

description: >

The media type that should be used for the corresponding request. If the attribute

is missing, or contains an unrecognized value, the client should act as if the

contentType is set to "application/json".

properties:

type: array

items:

$ref: '#/components/schemas/Property'

minItems: 1

description: >

The properties that should be included in the body of the corresponding request.

If the contentType attribute is set to "application/json", then this attribute

describes the attributes of the JSON object of the body.

Property:

description: >

If the contentType attribute is set to "application/json", then this attribute describes

the attributes of the JSON object of the body.

type: object

required:

- name

properties:

name:

type: string

description: The name of the property

required:

type: boolean

description: >

Indicates whether the property is required – true= required –

false(default)= not required.

regex:

type: string

description: A regular expression string to be applied to the value of the property.

value:

type: string

description: The property value. When present, it shall be a valid JSON string.

RedirectResponse:

description: >

The response shall include a Location header field containing a different URI

(pointing to a different URI of an other service instance), or the same URI if a request

is redirected to the same target resource via a different SCP.

type: object

properties:

cause:

type: string

targetScp:

$ref: '#/components/schemas/Uri'

targetSepp:

$ref: '#/components/schemas/Uri'

TunnelAddress:

description: Tunnel address

type: object

properties:

ipv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

portNumber:

$ref: '#/components/schemas/Uinteger'

required:

- portNumber

anyOf:

- required: [ ipv4Addr ]

- required: [ ipv6Addr ]

FqdnPatternMatchingRule:

description: a matching rule for a FQDN pattern

type: object

oneOf:

- required: [ regex ]

- required: [ stringMatchingRule ]

properties:

regex:

type: string

stringMatchingRule:

$ref: '#/components/schemas/StringMatchingRule'

StringMatchingRule:

description: A list of conditions for string matching

type: object

properties:

stringMatchingConditions:

type: array

items:

$ref: '#/components/schemas/StringMatchingCondition'

minItems: 1

required:

- stringMatchingConditions

StringMatchingCondition:

description: A String with Matching Operator

type: object

properties:

matchingString:

type: string

matchingOperator:

$ref: '#/components/schemas/MatchingOperator'

required:

- matchingOperator

Ipv4AddressRange:

description: Range of IPv4 addresses

type: object

properties:

start:

$ref: '#/components/schemas/Ipv4Addr'

end:

$ref: '#/components/schemas/Ipv4Addr'

required:

- start

- end

Ipv6AddressRange:

description: Range of IPv6 addresses

type: object

properties:

start:

$ref: '#/components/schemas/Ipv6Addr'

end:

$ref: '#/components/schemas/Ipv6Addr'

required:

- start

- end

Ipv6PrefixRange:

description: Range of IPv6 prefixes

type: object

properties:

start:

$ref: '#/components/schemas/Ipv6Prefix'

end:

$ref: '#/components/schemas/Ipv6Prefix'

required:

- start

- end

NfSignallingInfo:

description: NF Signalling Information

type: object

properties:

nfInstanceId:

$ref: '#/components/schemas/NfInstanceId'

nfSigInfoPerWndw:

type: array

items:

$ref: '#/components/schemas/NfSignallingInfoPerTimeWindow'

minItems: 1

nfHeartbeatInfo:

$ref: '#/components/schemas/NfHeartbeatInfo'

unexpStatusInd:

type: boolean

default: false

required:

- nfInstanceId

NfSignallingInfoPerTimeWindow:

description: NF Signalling Information Per time window

type: object

properties:

tw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

nfSigInfoPerService:

type: array

items:

$ref: '#/components/schemas/NfSignallingInfoPerService'

minItems: 1

required:

- tw

- nfSigInfoPerService

NfSignallingInfoPerService:

description: NF signalling information per service.

type: object

properties:

nfServiceInstanceId:

type: string

serviceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

avgProcTime:

$ref: '#/components/schemas/Float'

numOfReq:

$ref: '#/components/schemas/Uinteger'

numOfReqUnresp:

$ref: '#/components/schemas/Uinteger'

numOfReqReject:

$ref: '#/components/schemas/Uinteger'

numOfRedMessages:

$ref: '#/components/schemas/Uinteger'

numOfPosteriorReq:

$ref: '#/components/schemas/Uinteger'

required:

- nfServiceInstanceId

- serviceName

NfHeartbeatInfo:

description: NF heart beat information.

type: object

properties:

numOfRetrans:

$ref: '#/components/schemas/Uinteger'

hbIntvl:

$ref: '#/components/schemas/Uinteger'

#

# Data Types related to Subscription, Identification and Numbering as defined in clause 5.3

#

#

# SIMPLE DATA TYPES

#

Dnn:

type: string

description: >

String representing a Data Network as defined in clause 9A of 3GPP TS 23.003;

it shall contain either a DNN Network Identifier, or a full DNN with both the Network

Identifier and Operator Identifier, as specified in 3GPP TS 23.003 clause 9.1.1 and 9.1.2.

It shall be coded as string in which the labels are separated by dots

(e.g. "Label1.Label2.Label3").

DnnRm:

type: string

nullable: true

description: >

String representing a Data Network as defined in clause 9A of 3GPP TS 23.003;

it shall contain either a DNN Network Identifier, or a full DNN with both the

Network Identifier and Operator Identifier, as specified in 3GPP TS 23.003 clause 9.1.1

and 9.1.2. It shall be coded as string in which the labels are separated by dots

(e.g. 'Label1.Label2.Label3') with the OpenAPI 'nullable: true' property.

WildcardDnn:

type: string

pattern: '^[*]$'

description: String representing the Wildcard DNN. It shall contain the string "*".

WildcardDnnRm:

type: string

pattern: '^[*]$'

nullable: true

description: >

String representing the Wildcard DNN. It shall contain the string '*' but with the

OpenAPI 'nullable: true' property.

Gpsi:

type: string

pattern: '^(msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|.+)$'

description: >

String identifying a Gpsi shall contain either an External Id or an MSISDN.

It shall be formatted as follows -External Identifier= "extid-'extid', where 'extid'

shall be formatted according to clause 19.7.2 of 3GPP TS 23.003 that describes an

External Identifier.

GpsiRm:

type: string

pattern: '^(msisdn-[0-9]{5,15}|extid-[^@]+@[^@]+|.+)$'

nullable: true

description: >

String identifying a Gpsi shall contain either an External Id or an MSISDN. It shall be

formatted as follows -External Identifier= 'extid-'extid', where 'extid' shall be formatted

according to clause 19.7.2 of 3GPP TS 23.003 that describes an External Identifier with the

OpenAPI 'nullable: true' property.

GroupId:

type: string

pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'

description: >

String identifying a group of devices network internal globally unique ID which identifies

a set of IMSIs, as specified in clause 19.9 of 3GPP TS 23.003.

GroupIdRm:

type: string

pattern: '^[A-Fa-f0-9]{8}-[0-9]{3}-[0-9]{2,3}-([A-Fa-f0-9][A-Fa-f0-9]){1,10}$'

nullable: true

description: >

String identifying a group of devices network internal globally unique ID which

identifies a set of IMSIs, as specified in clause 19.9 of 3GPP TS 23.003 with the

OpenAPI 'nullable: true' property.

ExternalGroupId:

type: string

pattern: '^extgroupid-[^@]+@[^@]+$'

description: >

String identifying External Group Identifier that identifies a group made up of one or

more  subscriptions associated to a group of IMSIs, as specified in clause 19.7.3 of 3GPP

TS 23.003.

ExternalGroupIdRm:

type: string

pattern: '^extgroupid-[^@]+@[^@]+$'

nullable: true

description: >

String identifying External Group Identifier that identifies a group made up of one or

more  subscriptions associated to a group of IMSIs, as specified in clause 19.7.3 of

3GPP TS 23.003  with the OpenAPI 'nullable: true' property.

Pei:

type: string

pattern: '^(imei-[0-9]{15}|imeisv-[0-9]{16}|mac((-[0-9a-fA-F]{2}){6})(-untrusted)?|eui((-[0-9a-fA-F]{2}){8})|.+)$'

description: >

String representing a Permanent Equipment Identifier that may contain - an IMEI or IMEISV,

as  specified in clause 6.2 of 3GPP TS 23.003; a MAC address for a 5G-RG or FN-RG via

wireline  access, with an indication that this address cannot be trusted for regulatory

purpose if this  address cannot be used as an Equipment Identifier of the FN-RG, as

specified in clause 4.7.7  of 3GPP TS23.316. Examples are imei-012345678901234 or

imeisv-0123456789012345.

PeiRm:

type: string

pattern: '^(imei-[0-9]{15}|imeisv-[0-9]{16}|mac((-[0-9a-fA-F]{2}){6})(-untrusted)?|eui((-[0-9a-fA-F]{2}){8})|.+)$'

nullable: true

description: >

This data type is defined in the same way as the 'Pei' data type but with

the OpenAPI 'nullable: true' property.

Supi:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gci-.+|gli-.+|.+)$'

description: |

String identifying a Supi that shall contain either an IMSI, a network specific identifier,

a Global Cable Identifier (GCI) or a Global Line Identifier (GLI) as specified in clause

2.2A of 3GPP TS 23.003. It shall be formatted as follows

- for an IMSI "imsi-<imsi>", where <imsi> shall be formatted according to clause 2.2

of 3GPP TS 23.003 that describes an IMSI.

- for a network specific identifier "nai-<nai>, where <nai> shall be formatted

according to clause 28.7.2 of 3GPP TS 23.003 that describes an NAI.

- for a GCI "gci-<gci>", where <gci> shall be formatted according to clause 28.15.2

of 3GPP TS 23.003.

- for a GLI "gli-<gli>", where <gli> shall be formatted according to clause 28.16.2 of

3GPP TS 23.003.To enable that the value is used as part of an URI, the string shall

only contain characters allowed according to the "lower-with-hyphen" naming convention

defined in 3GPP TS 29.501.

SupiRm:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gci-.+|gli-.+|.+)$'

nullable: true

description: >

This data type is defined in the same way as the 'Supi' data type, but with the

OpenAPI 'nullable: true' property.

NfInstanceId:

type: string

format: uuid

description: >

String uniquely identifying a NF instance. The format of the NF Instance ID shall be a

Universally Unique Identifier (UUID) version 4.

AmfId:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: >

String identifying the AMF ID composed of AMF Region ID (8 bits), AMF Set ID (10 bits)

and AMF  Pointer (6 bits) as specified in clause 2.10.1 of 3GPP TS 23.003. It is encoded

as a string of  6 hexadecimal characters (i.e., 24 bits).

AmfRegionId:

type: string

pattern: '^[A-Fa-f0-9]{2}$'

description: >

String identifying the AMF Region (8 bits) as specified in clause 2.10.1 of

3GPP TS 23.003.

It is encoded as a string of 2 hexadecimal characters (i.e. 8 bits).

AmfSetId:

type: string

pattern: '^[0-3][A-Fa-f0-9]{2}$'

description: >

String identifying the AMF Set ID (10 bits) as specified in clause 2.10.1 of 3GPP TS 23.003.

It is encoded as a string of 3 hexadecimal characters where the first character is limited

to values 0 to 3 (i.e. 10 bits).

RfspIndex:

type: integer

minimum: 1

maximum: 256

description: >

Unsigned integer representing the "Subscriber Profile ID for RAT/Frequency Priority"

as specified in 3GPP TS 36.413.

RfspIndexRm:

type: integer

minimum: 1

maximum: 256

nullable: true

description: >

Unsigned integer representing the 'Subscriber Profile ID for RAT/Frequency Priority'

as specified in 3GPP TS 36.413 with the OpenAPI 'nullable: true' property.

NfGroupId:

type: string

description: Identifier of a group of NFs.

MtcProviderInformation:

type: string

description: String uniquely identifying MTC provider information.

CagId:

type: string

pattern: '^[A-Fa-f0-9]{8}$'

description: String containing a Closed Access Group Identifier.

SupiOrSuci:

type: string

pattern: '^(imsi-[0-9]{5,15}|nai-.+|gli-.+|gci-.+|suci-(0-[0-9]{3}-[0-9]{2,3}|[1-7]-.+)-[0-9]{1,4}-(0-0-.*|[a-fA-F1-9]-([1-9]|[1-9][0-9]|1[0-9]{2}|2[0-4][0-9]|25[0-5])-[a-fA-F0-9]+)|.+)$'

description: String identifying a SUPI or a SUCI.

Imsi:

description: String identifying an IMSI

type: string

pattern: '^[0-9]{5,15}$'

ApplicationlayerId:

type: string

description: >

String identifying an UE with application layer ID. The format of the application

layer ID parameter is same as the Application layer ID defined in clause 11.3.4 of

3GPP TS 24.554.

NsacSai:

type: string

description: >

String identifying the Network Slice Admission Control Service Area Identifier.

#

# ENUMERATED DATA TYPES

#

GroupServiceId:

anyOf:

- type: integer

enum:

- 1

- 2

- 3

- type: integer

description: >

This integer provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- 1: Group specific NAS level congestion control

- 2: Group specific Monitoring of Number of UEs present in a geographical area

- 3: Group specific Group specific for 5G LAN Type service

#

# STRUCTURED DATA TYPES

#

Guami:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnIdNid'

amfId:

$ref: '#/components/schemas/AmfId'

required:

- plmnId

- amfId

description: Globally Unique AMF Identifier constructed out of PLMN, Network and AMF identity.

GuamiRm:

anyOf:

- $ref: '#/components/schemas/Guami'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Guami' data type, but with the OpenAPI

'nullable: true' property.

NetworkId:

type: object

properties:

mnc:

$ref: '#/components/schemas/Mnc'

mcc:

$ref: '#/components/schemas/Mcc'

description: contains PLMN and Network identity.

#

# Data Types related to 5G Network as defined in clause 5.4

#

#

# SIMPLE DATA TYPES

#

ApplicationId:

type: string

description: String providing an application identifier.

ApplicationIdRm:

type: string

nullable: true

description: >

String providing an application identifier with the OpenAPI 'nullable: true' property.

PduSessionId:

type: integer

minimum: 0

maximum: 255

description: >

Unsigned integer identifying a PDU session, within the range 0 to 255, as specified in

clause 11.2.3.1b, bits 1 to 8, of 3GPP TS 24.007. If the PDU Session ID is allocated by the

Core Network for UEs not supporting N1 mode, reserved range 64 to 95 is used. PDU Session ID

within the reserved range is only visible in the Core Network.

Mcc:

type: string

pattern: '^\d{3}$'

description: >

Mobile Country Code part of the PLMN, comprising 3 digits, as defined in clause 9.3.3.5

of 3GPP TS 38.413.

MccRm:

type: string

pattern: '^\d{3}$'

nullable: true

description: >

Mobile Country Code part of the PLMN, comprising 3 digits, as defined in clause 9.3.3.5 of

3GPP TS 38.413 with the OpenAPI 'nullable: true' property.

Mnc:

type: string

pattern: '^\d{2,3}$'

description: >

Mobile Network Code part of the PLMN, comprising 2 or 3 digits, as defined in

clause 9.3.3.5 of 3GPP TS 38.413.

MncRm:

type: string

pattern: '^\d{2,3}$'

nullable: true

description: >

Mobile Network Code part of the PLMN, comprising 2 or 3 digits, as defined in clause

9.3.3.5 of 3GPP TS 38.413 with the OpenAPI 'nullable: true' property.

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

description: >

2 or 3-octet string identifying a tracking area code as specified in clause 9.3.3.16 or

9.3.3.10 of 3GPP TS 38.413, in hexadecimal representation. Each character in the string

shall take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits.

The most significant character representing the 4 most significant bits of the TAC shall

appear first in the string, and the character representing the 4 least significant bit

of the TAC shall appear last in the string.

TacRm:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

nullable: true

description: >

This data type is defined in the same way as the 'Tac' data type, but with the

OpenAPI 'nullable: true' property.

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

description: >

28-bit string identifying an E-UTRA Cell Id as specified in clause 9.3.1.9 of

3GPP TS 38.413, in hexadecimal representation. Each character in the string shall take a

value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most

significant character representing the 4 most significant bits of the Cell Id shall appear

first in the string, and the character representing the 4 least significant bit of the

Cell Id shall appear last in the string.

EutraCellIdRm:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

nullable: true

description: >

This data type is defined in the same way as the 'EutraCellId' data type, but with

the OpenAPI 'nullable: true' property.

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

description: >

36-bit string identifying an NR Cell Id as specified in clause 9.3.1.7 of 3GPP TS 38.413,

in hexadecimal representation. Each character in the string shall take a value of "0" to

"9",  "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character

representing the 4 most significant bits of the Cell Id shall appear first in the string,

and  the character representing the 4 least significant bit of the Cell Id shall appear last

in the  string.

NrCellIdRm:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

nullable: true

description: >

This data type is defined in the same way as the 'NrCellId' data type, but with the

OpenAPI 'nullable: true' property.

Dnai:

type: string

description: DNAI (Data network access identifier), see clause 5.6.7 of 3GPP TS 23.501.

DnaiRm:

type: string

nullable: true

description: >

This data type is defined in the same way as the 'Dnai' data type, but with the

OpenAPI 'nullable: true' property.

5GMmCause:

$ref: '#/components/schemas/Uinteger'

AmfName:

$ref: '#/components/schemas/Fqdn'

AreaCode:

type: string

description: Values are operator specific.

AreaCodeRm:

type: string

nullable: true

description: >

This data type is defined in the same way as the 'AreaCode' data type, but with the

OpenAPI 'nullable: true' property.

N3IwfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

This represents the identifier of the N3IWF ID as specified in clause 9.3.1.57 of

3GPP TS 38.413 in hexadecimal representation. Each character in the string shall take a value

of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant

character representing the 4 most significant bits of the N3IWF ID shall appear first in the

string, and the character representing the 4 least significant bit of the N3IWF ID shall

appear last in the string.

WAgfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

This represents the identifier of the W-AGF ID as specified in clause 9.3.1.162 of

3GPP TS 38.413 in hexadecimal representation. Each character in the string shall take a

value  of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most

significant  character representing the 4 most significant bits of the W-AGF ID shall

appear first in the  string, and the character representing the 4 least significant bit

of the W-AGF ID shall  appear last in the string.

TngfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

This represents the identifier of the TNGF ID as specified in clause 9.3.1.161 of

3GPP TS 38.413  in hexadecimal representation. Each character in the string shall take a value

of "0" to "9", "a"  to "f" or "A" to "F" and shall represent 4 bits. The most significant

character representing the  4 most significant bits of the TNGF ID shall appear first in

the string, and the character  representing the 4 least significant bit of the TNGF ID

shall appear last in the string.

NgeNbId:

type: string

pattern: '^(MacroNGeNB-[A-Fa-f0-9]{5}|LMacroNGeNB-[A-Fa-f0-9]{6}|SMacroNGeNB-[A-Fa-f0-9]{5})$'

description: >

This represents the identifier of the ng-eNB ID as specified in clause 9.3.1.8 of

3GPP TS 38.413. The value of the ng-eNB ID shall be encoded in hexadecimal representation.

Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F" and

shall represent 4 bits. The padding 0 shall be added to make multiple nibbles, so the most

significant character representing the padding 0 if required together with the 4 most

significant bits of the ng-eNB ID shall appear first in the string, and the character

representing the 4 least significant bit of the ng-eNB ID (to form a nibble) shall appear

last  in the string.

example: SMacroNGeNB-34B89

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

description: >

This represents the Network Identifier, which together with a PLMN ID is used to identify

an SNPN (see 3GPP TS 23.003 and 3GPP TS 23.501 clause 5.30.2.1).

NidRm:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

nullable: true

description: >

This data type is defined in the same way as the 'Nid' data type, but with the OpenAPI

'nullable: true' property."

NfSetId:

type: string

description: >

NF Set Identifier (see clause 28.12 of 3GPP TS 23.003), formatted as the following string

"set<Set ID>.<nftype>set.5gc.mnc<MNC>.mcc<MCC>", or

"set<SetID>.<NFType>set.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with

<MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition)

<MNC> encoding the Mobile Network Code part of the PLMN, comprising 3 digits.

If there are only 2 significant digits in the MNC, one "0" digit shall be inserted

at the left side to fill the 3 digits coding of MNC.  Pattern: '^[0-9]{3}$'

<NFType> encoded as a value defined in Table 6.1.6.3.3-1 of 3GPP TS 29.510 but

with lower case characters <Set ID> encoded as a string of characters consisting of

alphabetic characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that

shall end with either an alphabetic character or a digit.

NfServiceSetId:

type: string

description: >

NF Service Set Identifier (see clause 28.12 of 3GPP TS 23.003) formatted as the following

string "set<Set ID>.sn<Service Name>.nfi<NF Instance ID>.5gc.mnc<MNC>.mcc<MCC>", or

"set<SetID>.sn<ServiceName>.nfi<NFInstanceID>.5gc.nid<NID>.mnc<MNC>.mcc<MCC>" with

<MCC> encoded as defined in clause 5.4.2 ("Mcc" data type definition)

<MNC> encoding the Mobile Network Code part of the PLMN, comprising 3 digits.

If there are only 2 significant digits in the MNC, one "0" digit shall be inserted

at the left side to fill the 3 digits coding of MNC.  Pattern: '^[0-9]{3}$'

<NID> encoded as defined in clause 5.4.2 ("Nid" data type definition)

<NFInstanceId> encoded as defined in clause 5.3.2

<ServiceName> encoded as defined in 3GPP TS 29.510

<Set ID> encoded as a string of characters consisting of alphabetic

characters (A-Z and a-z), digits (0-9) and/or the hyphen (-) and that shall end

with either an alphabetic character or a digit.

PlmnAssiUeRadioCapId:

$ref: '#/components/schemas/Bytes'

ManAssiUeRadioCapId:

$ref: '#/components/schemas/Bytes'

TypeAllocationCode:

type: string

pattern: '^[0-9]{8}$'

description: >

Type Allocation Code (TAC) of the UE, comprising the initial eight-digit portion of the

15-digit IMEI and 16-digit IMEISV codes. See clause 6.2 of 3GPP TS 23.003.

HfcNId:

type: string

maxLength: 6

description: >

This IE represents the identifier of the HFC node Id as specified in CableLabs

WR-TR-5WWC-ARCH. It is provisioned by the wireline operator as part of wireline

operations and may contain up to six characters.

HfcNIdRm:

type: string

maxLength: 6

nullable: true

description: >

This data type is defined in the same way as the 'HfcNId' data type, but with the

OpenAPI 'nullable: true' property.

ENbId:

type: string

pattern: '^(MacroeNB-[A-Fa-f0-9]{5}|LMacroeNB-[A-Fa-f0-9]{6}|SMacroeNB-[A-Fa-f0-9]{5}|HomeeNB-[A-Fa-f0-9]{7})$'

description: >

This represents the identifier of the eNB ID as specified in clause 9.2.1.37 of

3GPP TS 36.413. The string shall be formatted with the following pattern

'^('MacroeNB-[A-Fa-f0-9]{5}|LMacroeNB-[A-Fa-f0-9]{6}|SMacroeNB-[A-Fa-f0-9]{5}

|HomeeNB-[A-Fa-f0-9]{7})$'. The value of the eNB ID shall be encoded in hexadecimal

representation. Each character in the  string shall take a value of "0" to "9", "a" to "f"

or "A" to "F" and shall represent 4 bits.  The padding 0 shall be added to make multiple

nibbles, so the most significant character  representing the padding 0 if required together

with the 4 most significant bits of the eNB ID  shall appear first in the string, and the

character representing the 4 least significant bit  of the eNB ID (to form a nibble) shall

appear last in the string.

Gli:

$ref: '#/components/schemas/Bytes'

Gci:

type: string

description: >

Global Cable Identifier uniquely identifying the connection between the 5G-CRG or FN-CRG

to the 5GS. See clause 28.15.4 of 3GPP TS 23.003. This shall be encoded as a string per

clause 28.15.4 of 3GPP TS 23.003, and compliant with the syntax specified  in clause 2.2

of IETF RFC 7542 for the username part of a NAI. The GCI value is specified in

CableLabs WR-TR-5WWC-ARCH.

NsSrg:

type: string

description: >

String providing a Network Slice Simultaneous Registration Group. See clause 5.15.12 of

3GPP TS 23.501

NsSrgRm:

type: string

nullable: true

description: >

String providing a Network Slice Simultaneous Registration Group with the OpenAPI

"nullable: true" property. See clause 5.15.12 of 3GPP TS 23.501

RelayServiceCode:

type: integer

minimum: 0

maximum: 16777215

description: >

Relay Service Code to identify a connectivity service provided by the UE-to-Network relay or

the UE-to-UE relay.

5GPrukId:

type: string

description: >

A string carrying the CP-PRUK ID of the 5G ProSe Remote UE or the 5G ProSe End UE.

The CP-PRUK ID is a string in NAI format as specified in clause 28.7.11 of 3GPP TS 23.003.

pattern: '^rid[0-9]{1,4}\.pid[0-9a-fA-F]+\@prose-cp\.5gc\.mnc[0-9]{2,3}\.mcc[0-9]{3}\.3gppnetwork\.org$'

NsagId:

type: integer

description: >

The Network Slice AS Group ID, see 3GPP TS 38.413

NsagIdRm:

type: integer

nullable: true

description: >

This data type is defined in the same way as the "NsagId" data type, but with the OpenAPI

"nullable: true" property

GeoSatelliteId:

type: string

description: >

A string carrying the GEO Satellite ID.

OffloadIdentifier:

type: string

description: >

Offload identifier uniquely identifying a VPLMN offloading policy information instance

pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{8}(-v[0-9]{1,2}){0,1}$'

OpConfiguredCapability:

type: string

description: >

The operator configurable capability supported by the UPF.

SatelliteId:

type: string

description: >

Unique identifier of a satellite.

EnergySavingIndicator:

$ref: '#/components/schemas/Uinteger'

OverloadControlInfo:

description: >

String representing the Overload Control Information as defined in clause 5.2.3.2.9 of 3GPP

TS 29.500.

type: string

#

# ENUMERATED DATA TYPES

#

AccessType:

type: string

enum:

- 3GPP_ACCESS

- NON_3GPP_ACCESS

description: Indicates whether the access is  via 3GPP or via non-3GPP.

AccessTypeRm:

anyOf:

- $ref: '#/components/schemas/AccessType'

- $ref: '#/components/schemas/NullValue'

description: >

Indicates wether the access is via 3GPP or via non-3GPP but with the OpenAPI

'nullable: true' property."

RatType:

anyOf:

- type: string

enum:

- NR

- EUTRA

- WLAN

- VIRTUAL

- NBIOT

- WIRELINE

- WIRELINE_CABLE

- WIRELINE_BBF

- LTE-M

- NR_U

- EUTRA_U

- TRUSTED_N3GA

- TRUSTED_WLAN

- UTRA

- GERA

- NR_LEO

- NR_MEO

- NR_GEO

- NR_OTHER_SAT

- NR_REDCAP

- WB_E_UTRAN_LEO

- WB_E_UTRAN_MEO

- WB_E_UTRAN_GEO

- WB_E_UTRAN_OTHERSAT

- NB_IOT_LEO

- NB_IOT_MEO

- NB_IOT_GEO

- NB_IOT_OTHERSAT

- LTE_M_LEO

- LTE_M_MEO

- LTE_M_GEO

- LTE_M_OTHERSAT

- NR_EREDCAP

- HSPA_EVOLUTION

- type: string

description: Indicates the radio access used.

RatTypeRm:

anyOf:

- $ref: '#/components/schemas/RatType'

- $ref: '#/components/schemas/NullValue'

description: >

Provides information about the radio access but with the OpenAPI 'nullable: true' property.

PduSessionType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- IPV4V6

- UNSTRUCTURED

- ETHERNET

- type: string

description: >

PduSessionType indicates the type of a PDU session. It shall comply with the provisions

defined in table 5.4.3.3-1.

PduSessionTypeRm:

anyOf:

- $ref: '#/components/schemas/PduSessionType'

- $ref: '#/components/schemas/NullValue'

description: >

PduSessionType indicates the type of a PDU session. It shall comply with the provisions

defined in table 5.4.3.3-1 but with the OpenAPI "nullable: true" property.

UpIntegrity:

anyOf:

- type: string

enum:

- REQUIRED

- PREFERRED

- NOT_NEEDED

- type: string

description: >

indicates whether UP integrity protection is required, preferred or not needed for all

the traffic on the PDU Session. It shall comply with the provisions defined in

table 5.4.3.4-1.

UpIntegrityRm:

anyOf:

- $ref: '#/components/schemas/UpIntegrity'

- $ref: '#/components/schemas/NullValue'

description: >

indicates whether UP integrity protection is required, preferred or not needed for all

the traffic on the PDU Session. It shall comply with the provisions defined in

table 5.4.3.4-1.

UpConfidentiality:

anyOf:

- type: string

enum:

- REQUIRED

- PREFERRED

- NOT_NEEDED

- type: string

description: >

indicates whether UP confidentiality protection is required, preferred or not needed for

all the traffic on the PDU Session. It shall comply with the provisions defined in

table 5.4.3.5-1.

UpConfidentialityRm:

anyOf:

- $ref: '#/components/schemas/UpConfidentiality'

- $ref: '#/components/schemas/NullValue'

description: >

indicates whether UP integrity protection is required, preferred or not needed for all the

traffic on the PDU Session. It shall comply with the provisions defined in table 5.4.3.4-1,

but with the OpenAPI 'nullable: true' property.

SscMode:

anyOf:

- type: string

enum:

- SSC_MODE_1

- SSC_MODE_2

- SSC_MODE_3

- type: string

description: >

represents the service and session continuity mode It shall comply with the provisions

defined in table 5.4.3.6-1.

SscModeRm:

anyOf:

- $ref: '#/components/schemas/SscMode'

- $ref: '#/components/schemas/NullValue'

description: >

represents the service and session continuity mode It shall comply with the provisions

defined in table 5.4.3.6-1 but with the OpenAPI 'nullable: true' property.

DnaiChangeType:

anyOf:

- type: string

enum:

- EARLY

- EARLY_LATE

- LATE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode  content defined in the present version of this API.

description: |

Possible values are:

- EARLY: Early notification of UP path reconfiguration.

- EARLY_LATE: Early and late notification of UP path reconfiguration. This value shall

only be present in the subscription to the DNAI change event.

- LATE: Late notification of UP path reconfiguration.

DnaiChangeTypeRm:

anyOf:

- $ref: '#/components/schemas/DnaiChangeType'

- $ref: '#/components/schemas/NullValue'

description: >

It can take the values  as specified for DnaiChangeType but with the OpenAPI

'nullable: true' property.

RestrictionType:

anyOf:

- type: string

enum:

- ALLOWED_AREAS

- NOT_ALLOWED_AREAS

- type: string

description: It contains the restriction type ALLOWED_AREAS or NOT_ALLOWED_AREAS.

RestrictionTypeRm:

anyOf:

- $ref: '#/components/schemas/RestrictionType'

- $ref: '#/components/schemas/NullValue'

description: >

It contains the restriction type ALLOWED_AREAS or NOT_ALLOWED_AREAS but with the

OpenAPI 'nullable: true' property.

CoreNetworkType:

anyOf:

- type: string

enum:

- 5GC

- EPC

- type: string

description: It contains the Core Network type 5GC or EPC.

CoreNetworkTypeRm:

anyOf:

- $ref: '#/components/schemas/CoreNetworkType'

- $ref: '#/components/schemas/NullValue'

description: >

It contains the Core Network type 5GC or EPC but with the OpenAPI

'nullable: true' property.

PresenceState:

anyOf:

- type: string

enum:

- IN_AREA

- OUT_OF_AREA

- UNKNOWN

- INACTIVE

- type: string

description: |

Possible values are:

-IN_AREA: Indicates that the UE is inside or enters the presence reporting area.

-OUT_OF_AREA: Indicates that the UE is outside or leaves the presence reporting area

-UNKNOW: Indicates it is unknown whether the UE is in the presence reporting area or not

-INACTIVE: Indicates that the presence reporting area is inactive in the serving node.

StationaryIndication:

anyOf:

- type: string

enum:

- STATIONARY

- MOBILE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- STATIONARY: Identifies the UE is stationary

- MOBILE: Identifies the UE is mobile

StationaryIndicationRm:

anyOf:

- $ref: '#/components/schemas/StationaryIndication'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'StationaryIndication' enumeration,

but with the OpenAPI 'nullable: true' property."

ScheduledCommunicationType:

anyOf:

- type: string

enum:

- DOWNLINK_ONLY

- UPLINK_ONLY

- BIDIRECTIONAL

- type: string

description: |

Possible values are:

-DOWNLINK_ONLY: Downlink only

-UPLINK_ONLY: Uplink only

-BIDIRECTIONA: Bi-directional

ScheduledCommunicationTypeRm:

anyOf:

- $ref: '#/components/schemas/ScheduledCommunicationType'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'ScheduledCommunicationTypen'

enumeration, but with the OpenAPI 'nullable: true' property."

TrafficProfile:

anyOf:

- type: string

enum:

- SINGLE_TRANS_UL

- SINGLE_TRANS_DL

- DUAL_TRANS_UL_FIRST

- DUAL_TRANS_DL_FIRST

- MULTI_TRANS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- SINGLE_TRANS_UL: Uplink single packet transmission.

- SINGLE_TRANS_DL: Downlink single packet transmission.

- DUAL_TRANS_UL_FIRST: Dual packet transmission, firstly uplink packet transmission

with subsequent downlink packet transmission.

- DUAL_TRANS_DL_FIRST: Dual packet transmission, firstly downlink packet transmission

with subsequent uplink packet transmission.

TrafficProfileRm:

anyOf:

- $ref: '#/components/schemas/TrafficProfile'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'TrafficProfile' enumeration, but

with the OpenAPI 'nullable: true' property.

LcsServiceAuth:

anyOf:

- type: string

enum:

- "LOCATION_ALLOWED_WITH_NOTIFICATION"

- "LOCATION_ALLOWED_WITHOUT_NOTIFICATION"

- "LOCATION_ALLOWED_WITHOUT_RESPONSE"

- "LOCATION_RESTRICTED_WITHOUT_RESPONSE"

- "NOTIFICATION_ONLY"

- "NOTIFICATION_AND_VERIFICATION_ONLY"

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- "LOCATION_ALLOWED_WITH_NOTIFICATION": Location allowed with notification

- "LOCATION_ALLOWED_WITHOUT_NOTIFICATION": Location allowed without notification

- "LOCATION_ALLOWED_WITHOUT_RESPONSE": Location with notification and privacy

verification; location allowed if no response

- "LOCATION_RESTRICTED_WITHOUT_RESPONSE": Location with notification and privacy

verification; location restricted if no response

- "NOTIFICATION_ONLY": Notification only

- "NOTIFICATION_AND_VERIFICATION_ONLY": Notification and privacy verification only

UeAuth:

anyOf:

- type: string

enum:

- AUTHORIZED

- NOT_AUTHORIZED

- type: string

description: |

Possible values are:

- AUTHORIZED: Indicates that the UE is authorized.

- NOT_AUTHORIZED: Indicates that the UE is not authorized.

DlDataDeliveryStatus:

anyOf:

- type: string

enum:

- BUFFERED

- TRANSMITTED

- DISCARDED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- BUFFERED: The first downlink data is buffered with extended buffering matching the

source of the downlink traffic.

- TRANSMITTED: The first downlink data matching the source of the downlink traffic is

transmitted after previous buffering or discarding of corresponding packet(s) because

the UE of the PDU Session becomes ACTIVE, and buffered data can be delivered to UE.

- DISCARDED: The first downlink data matching the source of the downlink traffic is

discarded because the Extended Buffering time, as determined by the SMF, expires or

the amount of downlink data to be buffered is exceeded.

DlDataDeliveryStatusRm:

anyOf:

- $ref: '#/components/schemas/DlDataDeliveryStatus'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the ' DlDataDeliveryStatus ' data type,

but with the OpenAPI 'nullable: true' property.

AuthStatus:

anyOf:

- type: string

enum:

- EAP_SUCCESS

- EAP_FAILURE

- PENDING

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- "EAP_SUCCESS": The NSSAA status is EAP-Success.

- "EAP_FAILURE": The NSSAA status is EAP-Failure.

- "PENDING": The NSSAA status is Pending.

LineType:

anyOf:

- type: string

enum:

- DSL

- PON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- DSL: Identifies a DSL line

- PON: Identifies a PON line

LineTypeRm:

anyOf:

- $ref: '#/components/schemas/LineType'

- $ref: '#/components/schemas/NullValue'

description: >
        This data type is defined in the same way as the 'LineType' data type, but with the

OpenAPI 'nullable: true' property.

NotificationFlag:

anyOf:

- type: string

enum:

- ACTIVATE

- DEACTIVATE

- RETRIEVAL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- ACTIVATE: The event notification is activated.

- DEACTIVATE: The event notification is deactivated and shall be muted. The available

event(s) shall be stored.

- RETRIEVAL: The event notification shall be sent to the NF service consumer(s),

after that, is muted again.

TransportProtocol:

anyOf:

- type: string

enum:

- UDP

- TCP

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- UDP: User Datagram Protocol.

- TCP: Transmission Control Protocol.

SatelliteBackhaulCategory:

anyOf:

- type: string

enum:

- GEO

- MEO

- LEO

- OTHER_SAT

- DYNAMIC_GEO

- DYNAMIC_MEO

- DYNAMIC_LEO

- DYNAMIC_OTHER_SAT

- NON_SATELLITE

- type: string

description: Indicates the satellite backhaul used.

SatelliteBackhaulCategoryRm:

anyOf:

- $ref: '#/components/schemas/SatelliteBackhaulCategory'

- $ref: '#/components/schemas/NullValue'

description: >
        Provides information about the satellite backhaul but with the OpenAPI

'nullable: true' property.

BufferedNotificationsAction:

anyOf:

- type: string

enum:

- SEND_ALL

- DISCARD_ALL

- DROP_OLD

- type: string

description: >

Indicates the required action by the event producer NF on the buffered Notifications.

SubscriptionAction:

anyOf:

- type: string

enum:

- CLOSE

- CONTINUE_WITH_MUTING

- CONTINUE_WITHOUT_MUTING

- type: string

description: >

Indicates the required action by the event producer NF on the event subscription if an

exception occurs while the event is muted.

SnssaiStatus:

anyOf:

- type: string

enum:

- AVAILABLE

- UNAVAILABLE

- type: string

description: Indicates the S-NSSAI availability.

TerminationIndication:

description: Indicates the termination of Network Slice Replacement.

anyOf:

- type: string

enum:

- NEW_UES_TERMINATION

- ALL_UES_TERMINATION

- type: string

DelayMeasurementProtocol:

description: Delay measurement protocol

anyOf:

- type: string

enum:

- TWAMP

- OWAMP

- STAMP

- OTHER

- type: string

CagProvisionOperation:

description: CAG provision operation

anyOf:

- type: string

enum:

- ADD

- REPLACE

- REMOVE

- type: string

#

# STRUCTURED DATA TYPES

#

SubscribedDefaultQos:

type: object

required:

- 5qi

- arp

properties:

5qi:

$ref: '#/components/schemas/5Qi'

arp:

$ref: '#/components/schemas/Arp'

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

description: Provides the subsribed 5QI and the ARP, it may contain the priority level.

Snssai:

type: object

properties:

sst:

type: integer

minimum: 0

maximum: 255

description: >

Unsigned integer, within the range 0 to 255, representing the Slice/Service Type.

It indicates the expected Network Slice behaviour in terms of features and services.

Values 0 to 127 correspond to the standardized SST range. Values 128 to 255 correspond

to the Operator-specific range. See clause 28.4.2 of 3GPP TS 23.003.

Standardized values are defined in clause 5.15.2.2 of 3GPP TS 23.501.

sd:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: >

3-octet string, representing the Slice Differentiator, in hexadecimal representation.

Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F"

and shall represent 4 bits. The most significant character representing the 4 most

significant bits of the SD shall appear first in the string, and the character

representing the 4 least significant bit of the SD shall appear last in the string.

This is an optional parameter that complements the Slice/Service type(s) to allow to

differentiate amongst multiple Network Slices of the same Slice/Service type. This IE

shall be absent if no SD value is associated with the SST.

description: >

When Snssai needs to be converted to string (e.g. when used in maps as key), the string shall

be composed of one to three digits "sst" optionally followed by "-" and 6 hexadecimal digits

"sd".

required:

- sst

PlmnId:

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

description: >

When PlmnId needs to be converted to string (e.g. when used in maps as key), the string

shall be composed of three digits "mcc" followed by "-" and two or three digits "mnc".

required:

- mcc

- mnc

PlmnIdRm:

anyOf:

- $ref: '#/components/schemas/PlmnId'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'PlmnId' data type, but with the

OpenAPI 'nullable: true' property.

Tai:

description: Contains the tracking area identity as described in 3GPP 23.003

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

tac:

$ref: '#/components/schemas/Tac'

nid:

$ref: '#/components/schemas/Nid'

required:

- plmnId

- tac

TaiRm:

anyOf:

- $ref: '#/components/schemas/Tai'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Tai' data type, but with the OpenAPI

'nullable: true' property.

Ecgi:

description: Contains the ECGI (E-UTRAN Cell Global Identity), as described in 3GPP 23.003

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

eutraCellId:

$ref: '#/components/schemas/EutraCellId'

nid:

$ref: '#/components/schemas/Nid'

required:

- plmnId

- eutraCellId

EcgiRm:

anyOf:

- $ref: '#/components/schemas/Ecgi'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Ecgi' data type, but with the

OpenAPI 'nullable: true' property.

Ncgi:

description: Contains the NCGI (NR Cell Global Identity), as described in 3GPP 23.003

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

nrCellId:

$ref: '#/components/schemas/NrCellId'

nid:

$ref: '#/components/schemas/Nid'

required:

- plmnId

- nrCellId

NcgiRm:

anyOf:

- $ref: '#/components/schemas/Ncgi'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Ncgi' data type, but with the

OpenAPI 'nullable: true' property.

UserLocation:

type: object

properties:

eutraLocation:

$ref: '#/components/schemas/EutraLocation'

nrLocation:

$ref: '#/components/schemas/NrLocation'

n3gaLocation:

$ref: '#/components/schemas/N3gaLocation'

utraLocation:

$ref: '#/components/schemas/UtraLocation'

geraLocation:

$ref: '#/components/schemas/GeraLocation'

description: >

At least one of eutraLocation, nrLocation and n3gaLocation shall be present. Several

of them may be present.

EutraLocation:

description: Contains the E-UTRA user location.

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

ignoreTai:

type: boolean

default: false

ecgi:

$ref: '#/components/schemas/Ecgi'

ignoreEcgi:

type: boolean

default: false

description: >

This flag when present shall indicate that the Ecgi shall be ignored

When present, it shall be set as follows:

- true: ecgi shall be ignored.

- false (default): ecgi shall not be ignored.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the

mobile station.  Value "0" indicates that the location information was obtained after a

successful paging procedure for Active Location Retrieval when the UE is in idle mode

or after a successful NG-RAN location reporting procedure with the eNB when the UE is

in connected mode.  Any other value than "0" indicates that the location information is

the last known one.  See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: >

Refer to geographical Information. See 3GPP TS 23.032 clause 7.3.2. Only the

description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: >

Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24]

clause 3.88.2. Only the description of an ellipsoid point with uncertainty circle

is allowed to be used.

globalNgenbId:

$ref: '#/components/schemas/GlobalRanNodeId'

globalENbId:

$ref: '#/components/schemas/GlobalRanNodeId'

required:

- tai

- ecgi

EutraLocationRm:

anyOf:

- $ref: '#/components/schemas/EutraLocation'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'EutraLocation' data type, but with

the OpenAPI 'nullable: true' property.

NrLocation:

description: Contains the NR user location.

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

ncgi:

$ref: '#/components/schemas/Ncgi'

ignoreNcgi:

type: boolean

default: false

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the mobile

station. Value "0" indicates that the location information was obtained after a

successful paging procedure for Active Location Retrieval when the UE is in idle mode

or after a successful  NG-RAN location reporting procedure with the eNB when the UE is

in connected mode. Any other value than "0" indicates that the location information is

the last known one. See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: >

Refer to geographical Information. See 3GPP TS 23.032 clause 7.3.2. Only the description

of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: >

Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) [24] clause

3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed

to be used.

globalGnbId:

$ref: '#/components/schemas/GlobalRanNodeId'

ntnTaiInfo:

$ref: '#/components/schemas/NtnTaiInfo'

required:

- tai

- ncgi

NrLocationRm:

anyOf:

- $ref: '#/components/schemas/NrLocation'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'NrLocation' data type, but with the

OpenAPI 'nullable: true' property."

N3gaLocation:

description: Contains the Non-3GPP access user location.

type: object

properties:

n3gppTai:

$ref: '#/components/schemas/Tai'

n3IwfId:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

This IE shall contain the N3IWF identifier received over NGAP and shall be encoded as a

string of hexadecimal characters. Each character in the string shall take a value of "0"

to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant

character representing the 4 most significant bits of the N3IWF ID shall appear first in

the string, and the character representing the 4 least significant bit of the N3IWF ID

shall appear last in the string.

ueIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ueIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

portNumber:

$ref: '#/components/schemas/Uinteger'

protocol:

$ref: '#/components/schemas/TransportProtocol'

tnapId:

$ref: '#/components/schemas/TnapId'

twapId:

$ref: '#/components/schemas/TwapId'

hfcNodeId:

$ref: '#/components/schemas/HfcNodeId'

gli:

$ref: '#/components/schemas/Gli'

w5gbanLineType:

$ref: '#/components/schemas/LineType'

gci:

$ref: '#/components/schemas/Gci'

UpSecurity:

description: Contains Userplain security information.

type: object

properties:

upIntegr:

$ref: '#/components/schemas/UpIntegrity'

upConfid:

$ref: '#/components/schemas/UpConfidentiality'

required:

- upIntegr

- upConfid

UpSecurityRm:

anyOf:

- $ref: '#/components/schemas/UpSecurity'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'UpSecurity' data type, but with the

OpenAPI 'nullable: true' property.

NgApCause:

description: Represents the NGAP cause.

type: object

properties:

group:

$ref: '#/components/schemas/Uinteger'

value:

$ref: '#/components/schemas/Uinteger'

required:

- group

- value

BackupAmfInfo:

description: Provides details of the Backup AMF.

type: object

properties:

backupAmf:

$ref: '#/components/schemas/AmfName'

guamiList:

type: array

items:

$ref: '#/components/schemas/Guami'

minItems: 1

description: >

If present, this IE shall contain the list of GUAMI(s) (supported by the AMF) for

which the backupAmf IE applies.

required:

- backupAmf

RefToBinaryData:

description: This parameter provides information about the referenced binary body data.

type: object

properties:

contentId:

type: string

description: >

This IE shall contain the value of the Content-ID header of the referenced binary

body part.

required:

- contentId

RefToBinaryDataRm:

anyOf:

- $ref: '#/components/schemas/RefToBinaryData'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the ' RefToBinaryData ' data type,

but with the OpenAPI 'nullable: true' property.

RouteToLocation:

type: object

properties:

dnai:

$ref: '#/components/schemas/Dnai'

routeInfo:

$ref: '#/components/schemas/RouteInformation'

routeProfId:

type: string

nullable: true

description: Identifies the routing profile Id.

required:

- dnai

anyOf:

- required: [ routeInfo ]

- required: [ routeProfId ]

nullable: true

description: >

At least one of the "routeInfo" attribute and the "routeProfId" attribute shall be included

in the "RouteToLocation" data type.

RouteInformation:

type: object

properties:

ipv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

portNumber:

$ref: '#/components/schemas/Uinteger'

required:

- portNumber

nullable: true

description: >

At least one of the "ipv4Addr" attribute and the "ipv6Addr" attribute shall be included in

the "RouteInformation" data type.

Area:

description: Provides area information.

type: object

oneOf:

- required:

- tacs

- required:

- areaCode

properties:

tacs:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

areaCode:

$ref: '#/components/schemas/AreaCode'

ServiceAreaRestriction:

description: Provides information about allowed or not allowed areas.

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

areas:

type: array

items:

$ref: '#/components/schemas/Area'

maxNumOfTAs:

$ref: '#/components/schemas/Uinteger'

maxNumOfTAsForNotAllowedAreas:

$ref: '#/components/schemas/Uinteger'

allOf:

#

# 1st condition: restrictionType and areas attributes shall be either both absent

#                or both present

#

- oneOf:

- not:

required: [ restrictionType ]

- required: [ areas ]

#

# 2nd condition: if restrictionType takes value NOT_ALLOWED_AREAS,

#                then maxNumOfTAs shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ NOT_ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAs ]

#

# 3rd condition: if restrictionType takes value ALLOWED_AREAS,

#                then maxNumOfTAsForNotAllowedAreas shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAsForNotAllowedAreas ]

PresenceInfo:

type: object

properties:

praId:

type: string

description: >

Represents an identifier of the Presence Reporting Area (see clause 28.10 of 3GPP

TS 23.003.  This IE shall be present  if the Area of Interest subscribed or reported is

a Presence Reporting Area or a Set of Core Network predefined Presence Reporting Areas.

When present, it shall be encoded as a string representing an integer in the following

ranges:

0 to 8 388 607 for UE-dedicated PRA

8 388 608 to 16 777 215 for Core Network predefined PRA

Examples:

PRA ID 123 is encoded as "123"

PRA ID 11 238 660 is encoded as "11238660"

additionalPraId:

type: string

description: >

This IE may be present if the praId IE is present and if it contains a PRA identifier

referring to a set of Core Network predefined Presence Reporting Areas. When present,

this IE shall contain a PRA Identifier of an individual PRA within the Set of Core

Network predefined Presence Reporting Areas indicated by the praId IE.

presenceState:

$ref: '#/components/schemas/PresenceState'

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

description: >

Represents the list of tracking areas that constitutes the area. This IE shall be

present if the subscription or  the event report is for tracking UE presence in the

tracking areas. For non 3GPP access the TAI shall be the N3GPP TAI.

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 1

description: >

Represents the list of EUTRAN cell Ids that constitutes the area. This IE shall

be present if the Area of Interest subscribed is a list of EUTRAN cell Ids.

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 1

description: >

Represents the list of NR cell Ids that constitutes the area. This IE shall be

present if the Area of Interest subscribed is a list of NR cell Ids.

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: >

Represents the list of NG RAN node identifiers that constitutes the area. This IE shall

be present if the Area of Interest subscribed is a list of NG RAN node identifiers.

globaleNbIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: >

Represents the list of eNodeB identifiers that constitutes the area. This IE shall be

present if the Area of Interest subscribed is a list of eNodeB identifiers.

description: >

If the additionalPraId IE is present, this IE shall state the presence information of the

UE for the individual PRA identified by the additionalPraId IE;  If the additionalPraId IE

is not present, this IE shall state the presence information of the UE for the PRA

identified by the praId IE.

PresenceInfoRm:

type: object

properties:

praId:

type: string

description: |

Represents an identifier of the Presence Reporting Area (see clause 28.10 of

3GPP TS 23.003. This IE shall be present  if the Area of Interest subscribed or

reported is a Presence Reporting Area or a Set of Core Network predefined Presence

Reporting Areas. When present, it shall be encoded as a string representing an integer

in the following ranges:

- 0 to 8 388 607 for UE-dedicated PRA

- 8 388 608 to 16 777 215 for Core Network predefined PRA

Examples:

PRA ID 123 is encoded as "123"

PRA ID 11 238 660 is encoded as "11238660"

additionalPraId:

type: string

description: >

This IE may be present if the praId IE is present and if it contains a PRA identifier

referring to a set of Core Network predefined Presence Reporting Areas.

When present, this IE shall contain a PRA Identifier of an individual PRA within the Set

of Core Network predefined Presence Reporting Areas indicated by the praId IE.

presenceState:

$ref: '#/components/schemas/PresenceState'

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 0

description: >

Represents the list of tracking areas that constitutes the area. This IE shall be

present if the subscription or the event report  is for tracking UE presence in the

tracking areas. For non 3GPP access the TAI shall be the N3GPP TAI.

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 0

description: >

Represents the list of EUTRAN cell Ids that constitutes the area. This IE shall be

present if the Area of Interest subscribed is a list of EUTRAN cell Ids.

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 0

description: >

Represents the list of NR cell Ids that constitutes the area. This IE shall be present

if the Area of Interest subscribed is a list of NR cell Ids.

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

description: >

Represents the list of NG RAN node identifiers that constitutes the area. This IE shall be

present if the Area of Interest subscribed is a list of NG RAN node identifiers.

globaleNbIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

description: >

Represents the list of eNodeB identifiers that constitutes the area. This IE shall be present

if the Area of Interest subscribed is a list of eNodeB identifiers.

nullable: true

description: >

This data type is defined in the same way as the 'PresenceInfo' data type, but with the

OpenAPI 'nullable: true' property.  If the additionalPraId IE is present, this IE shall state

the presence information of the UE for the individual PRA identified by the additionalPraId

IE;  If the additionalPraId IE is not present, this IE shall state the presence information

of the UE for the PRA identified by the praId IE.

GlobalRanNodeId:

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

n3IwfId:

$ref: '#/components/schemas/N3IwfId'

gNbId:

$ref: '#/components/schemas/GNbId'

ngeNbId:

$ref: '#/components/schemas/NgeNbId'

wagfId:

$ref: '#/components/schemas/WAgfId'

tngfId:

$ref: '#/components/schemas/TngfId'

nid:

$ref: '#/components/schemas/Nid'

eNbId:

$ref: '#/components/schemas/ENbId'

oneOf:

- required: [ n3IwfId ]

- required: [ gNbId ]

- required: [ ngeNbId ]

- required: [ wagfId ]

- required: [ tngfId ]

- required: [ eNbId ]

description: >

One of the six attributes n3IwfId, gNbIdm, ngeNbId, wagfId, tngfId, eNbId shall be present.

required:

- plmnId

GNbId:

description: Provides the G-NB identifier.

type: object

properties:

bitLength:

type: integer

minimum: 22

maximum: 32

description: >

Unsigned integer representing the bit length of the gNB ID as defined in clause

9.3.1.6 of 3GPP TS 38.413 [11], within the range 22 to 32.

gNBValue:

type: string

pattern: '^[A-Fa-f0-9]{6,8}$'

description: >

This represents the identifier of the gNB. The value of the gNB ID shall be encoded

in hexadecimal representation. Each character in the string shall take a value of

"0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits. The padding 0 shall

be added to make multiple nibbles,  the most significant character representing the

padding 0 if required together with the 4 most significant bits of the gNB ID shall

appear first in the string, and the character representing the 4 least significant bit

of the gNB ID shall appear last in the string.

required:

- bitLength

- gNBValue

AtsssCapability:

description: >

Containes Capability to support procedures related to Access Traffic Steering, Switching,

Splitting.

type: object

properties:

atsssLL:

type: boolean

default: false

description: >

Indicates the support of Access Traffic Steering, Switching and Splitting procedures

using the ATSSS-LL steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501).

true: Supported

false (default): Not Supported

mptcp:

type: boolean

default: false

description: >

Indicates the support of Access Traffic Steering, Switching and Splitting procedures

using the MPTCP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501

true: Supported

false (default): Not Supported

mpquic:

type: boolean

default: false

description: >

Indicates the support of Access Traffic Steering, Switching and Splitting procedures

using the MPQUIC-UDP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501)

true: Supported

false (default): Not Supported

rttWithoutPmf:

type: boolean

default: false

description: >

This IE is only used by the UPF to indicate whether the UPF supports RTT measurement

without PMF (see clauses 5.32.2, 6.3.3.3 of 3GPP TS 23.501

true: Supported

false (default): Not Supported

mpquicIp:

type: boolean

default: false

description: >

Indicates the support of Access Traffic Steering, Switching and Splitting procedures

using the MPQUIC-IP steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501)

true: Supported

false (default): Not Supported

mpquicE:

type: boolean

default: false

description: >

Indicates the support of Access Traffic Steering, Switching and Splitting procedures

using the MPQUIC-E steering functionality (see clauses 4.2.10, 5.32 of 3GPP TS 23.501)

true: Supported

false (default): Not Supported

PlmnIdNid:

description: >

Contains the serving core network operator PLMN ID and, for an SNPN, the NID that together

with the PLMN ID identifies the SNPN.

type: object

required:

- mcc

- mnc

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

nid:

$ref: '#/components/schemas/Nid'

PlmnIdNidRm:

anyOf:

- $ref: '#/components/schemas/PlmnIdNid'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'PlmnIdNid' data type, but with the

OpenAPI 'nullable: true' property.

SmallDataRateStatus:

description: It indicates theSmall Data Rate Control Status

type: object

properties:

remainPacketsUl:

type: integer

minimum: 0

description: >

When present, it shall contain the number of packets the UE is allowed to send uplink

in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501.

remainPacketsDl:

type: integer

minimum: 0

description: >

When present it shall contain the number of packets the AF is allowed to send downlink

in the given time unit for the given PDU session (see clause 5.31.14.3 of 3GPP TS 23.501.

validityTime:

$ref: '#/components/schemas/DateTime'

remainExReportsUl:

type: integer

minimum: 0

description: >

When present, it shall indicate number of additional exception reports the UE is allowed

to send uplink in the given time  unit for  the given PDU session (see clause 5.31.14.3

of 3GPP TS 23.501.

remainExReportsDl:

type: integer

minimum: 0

description: >

When present, it shall indicate number of additional exception reports the AF is allowed

to send downlink  in the given time unit for the given PDU session (see clause 5.31.14.3

in 3GPP TS 23.501

HfcNodeId:

description: REpresents the HFC Node Identifer received over NGAP.

type: object

required:

- hfcNId

properties:

hfcNId:

$ref: '#/components/schemas/HfcNId'

HfcNodeIdRm:

anyOf:

- $ref: '#/components/schemas/HfcNodeId'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'HfcNodeId' data type, but with the

OpenAPI 'nullable: true' property.

WirelineArea:

type: object

properties:

globalLineIds:

type: array

items:

$ref: '#/components/schemas/Gli'

minItems: 1

hfcNIds:

type: array

items:

$ref: '#/components/schemas/HfcNId'

minItems: 1

areaCodeB:

$ref: '#/components/schemas/AreaCode'

areaCodeC:

$ref: '#/components/schemas/AreaCode'

combGciAndHfcNIds:

type: array

items:

$ref: '#/components/schemas/CombGciAndHfcNIds'

minItems: 1

description: >

One and only one of the "globLineIds", "hfcNIds", "areaCodeB",d "areaCodeC" and

combGciAndHfcNIds attributes shall be included in a WirelineArea data structure

WirelineServiceAreaRestriction:

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

areas:

type: array

items:

$ref: '#/components/schemas/WirelineArea'

description: >

The "restrictionType" attribute and the "areas" attribute shall be either both present

or absent. The empty array of areas is used when service is allowed/restricted nowhere.

ApnRateStatus:

description: Contains the APN rate control status e.g. of the AMF.

type: object

properties:

remainPacketsUl:

type: integer

minimum: 0

description: >

When present, it shall contain the number of packets the UE is allowed to send uplink

in the given time unit for the given APN (all PDN connections of the UE to this APN

see clause 4.7.7.3 in 3GPP TS 23.401.

remainPacketsDl:

type: integer

minimum: 0

description: >

When present, it shall contain the number of packets the UE is allowed to send uplink

in the given time unit for the given APN (all PDN connections of the UE to this APN

see clause 4.7.7.3 in 3GPP TS 23.401.

validityTime:

$ref: '#/components/schemas/DateTime'

remainExReportsUl:

type: integer

minimum: 0

description: >

When present, it shall indicate the number of additional exception reports the UE is

allowed to send uplink in the given time unit for the given APN (all PDN connections of the UE to this APN,

see clause 4.7.7.3 in 3GPP TS 23.401.

remainExReportsDl:

type: integer

minimum: 0

description: >

When present, it shall indicate the number of additional exception reports the AF is

allowed to send downlink in the given time unit for the given APN (all PDN connections

of the UE to this APN, see clause 4.7.7.3 in 3GPP TS 23.401.

ScheduledCommunicationTime:

description: Identifies time and day of the week when the UE is available for communication.

type: object

properties:

daysOfWeek:

type: array

items:

$ref: '#/components/schemas/DayOfWeek'

minItems: 1

maxItems: 6

description: >

Identifies the day(s) of the week. If absent, it indicates every day of the week.

timeOfDayStart:

$ref: '#/components/schemas/TimeOfDay'

timeOfDayEnd:

$ref: '#/components/schemas/TimeOfDay'

ScheduledCommunicationTimeRm:

anyOf:

- $ref: '#/components/schemas/ScheduledCommunicationTime'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'ScheduledCommunicationTime' data type,

but with the OpenAPI 'nullable: true' property.

BatteryIndication:

type: object

properties:

batteryInd:

type: boolean

description: >

This IE shall indicate whether the UE is battery powered or not.

true: the UE is battery powered;

false or absent: the UE is not battery powered

replaceableInd:

type: boolean

description: >

This IE shall indicate whether the battery of the UE is replaceable or not.

true: the battery of the UE is replaceable;

false or absent: the battery of the UE is not replaceable.

rechargeableInd:

type: boolean

description: >

This IE shall indicate whether the battery of the UE is rechargeable or not.

true: the battery of UE is rechargeable;

false or absent: the battery of the UE is not rechargeable.

description: >

Parameters "replaceableInd" and "rechargeableInd" are only included if the value of

Parameter "batteryInd" is true.

BatteryIndicationRm:

anyOf:

- $ref: '#/components/schemas/BatteryIndication'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'BatteryIndication' data type, but

with the OpenAPI 'nullable: true' property.

AcsInfo:

description: The ACS information for the 5G-RG is defined in BBF TR-069 [42] or in BBF TR-369

type: object

properties:

acsUrl:

$ref: '#/components/schemas/Uri'

acsIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

acsIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

AcsInfoRm:

anyOf:

- $ref: '#/components/schemas/AcsInfo'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'AcsInfo' data type, but with the

OpenAPI 'nullable: true' property.

NrV2xAuth:

description: Contains NR V2X services authorized information.

type: object

properties:

vehicleUeAuth:

$ref: '#/components/schemas/UeAuth'

pedestrianUeAuth:

$ref: '#/components/schemas/UeAuth'

LteV2xAuth:

description: Contains LTE V2X services authorized information.

type: object

properties:

vehicleUeAuth:

$ref: '#/components/schemas/UeAuth'

pedestrianUeAuth:

$ref: '#/components/schemas/UeAuth'

Pc5QoSPara:

description: Contains policy data on the PC5 QoS parameters.

type: object

required:

- pc5QosFlowList

properties:

pc5QosFlowList:

type: array

items:

$ref: '#/components/schemas/Pc5QosFlowItem'

minItems: 1

pc5LinkAmbr:

$ref: '#/components/schemas/BitRate'

Pc5QosFlowItem:

description: Contains a PC5 QOS flow.

type: object

required:

- pqi

properties:

pqi:

$ref: '#/components/schemas/5Qi'

pc5FlowBitRates:

$ref: '#/components/schemas/Pc5FlowBitRates'

range:

$ref: '#/components/schemas/Uinteger'

Pc5FlowBitRates:

description: it shall represent the PC5 Flow Bit Rates

type: object

properties:

guaFbr:

$ref: '#/components/schemas/BitRate'

maxFbr:

$ref: '#/components/schemas/BitRate'

UtraLocation:

type: object

oneOf:

- required:

- cgi

- required:

- sai

- required:

- rai

description: Exactly one of cgi, sai or lai shall be present.

properties:

cgi:

$ref: '#/components/schemas/CellGlobalId'

sai:

$ref: '#/components/schemas/ServiceAreaId'

lai:

$ref: '#/components/schemas/LocationAreaId'

rai:

$ref: '#/components/schemas/RoutingAreaId'

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the

mobile station.  Value "0" indicates that the location information was obtained after a

successful paging procedure for Active Location Retrieval when the UE is in idle mode

or after a successful location reporting procedure the UE is in connected mode. Any

other value than "0" indicates that the location information is the last known one.

See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: >

Refer to geographical Information.See 3GPP TS 23.032 clause 7.3.2. Only the

description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: >

Refers to Calling Geodetic Location. See ITU-T Recommendation Q.763 (1999) clause

3.88.2. Only the description of an ellipsoid point with uncertainty circle is allowed

to be used.

GeraLocation:

type: object

oneOf:

- required:

- cgi

- required:

- sai

- required:

- lai

description: Exactly one of cgi, sai or lai shall be present.

properties:

locationNumber:

type: string

description: Location number within the PLMN. See 3GPP TS 23.003, clause 4.5.

cgi:

$ref: '#/components/schemas/CellGlobalId'

sai:

$ref: '#/components/schemas/ServiceAreaId'

lai:

$ref: '#/components/schemas/LocationAreaId'

rai:

$ref: '#/components/schemas/RoutingAreaId'

vlrNumber:

type: string

description: VLR number. See 3GPP TS 23.003 clause 5.1.

mscNumber:

type: string

description: MSC number. See 3GPP TS 23.003 clause 5.1.

ageOfLocationInformation:

type: integer

minimum: 0

maximum: 32767

description: >

The value represents the elapsed time in minutes since the last network contact of the

mobile station. Value "0" indicates that the location information was obtained after a

successful paging procedure for  Active Location Retrieval when the UE is in idle mode

or after a successful location reporting procedure the UE is in connected mode. Any

other value than "0" indicates that the location information is the last known one.

See 3GPP TS 29.002 clause 17.7.8.

ueLocationTimestamp:

$ref: '#/components/schemas/DateTime'

geographicalInformation:

type: string

pattern: '^[0-9A-F]{16}$'

description: >

Refer to geographical Information.See 3GPP TS 23.032 clause 7.3.2. Only the

description of an ellipsoid point with uncertainty circle is allowed to be used.

geodeticInformation:

type: string

pattern: '^[0-9A-F]{20}$'

description: >

Refers to Calling Geodetic Location.See ITU-T Recommendation Q.763 (1999) clause 3.88.2.

Only the description of an ellipsoid point with uncertainty circle is allowed to be

used.

CellGlobalId:

description: Contains a Cell Global Identification as defined in 3GPP TS 23.003, clause 4.3.1.

type: object

required:

- plmnId

- lac

- cellId

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

lac:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

cellId:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

ServiceAreaId:

description: Contains a Service Area Identifier as defined in 3GPP TS 23.003, clause 12.5.

type: object

required:

- plmnId

- lac

- sac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

lac:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

description: Location Area Code.

sac:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

description: Service Area Code.

LocationAreaId:

description: Contains a Location area identification as defined in 3GPP TS 23.003, clause 4.1.

type: object

required:

- plmnId

- lac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

lac:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

description: Location Area Code.

RoutingAreaId:

description: Contains a Routing Area Identification as defined in 3GPP TS 23.003, clause 4.2.

type: object

required:

- plmnId

- lac

- rac

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

lac:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

description: Location Area Code

rac:

type: string

pattern: '^[A-Fa-f0-9]{2}$'

description: Routing Area Code

DddTrafficDescriptor:

description: Contains a Traffic Descriptor.

type: object

properties:

ipv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

portNumber:

$ref: '#/components/schemas/Uinteger'

macAddr:

$ref: '#/components/schemas/MacAddr48'

MoExpDataCounter:

description: Contain the MO Exception Data Counter.

type: object

required:

- counter

properties:

counter:

type: integer

description: >

Unsigned integer identifying the MO Exception Data Counter, as specified in clause

5.31.14.3 of 3GPP TS 23.501.

timeStamp:

$ref: '#/components/schemas/DateTime'

NssaaStatus:

description: contains the Subscribed S-NSSAI subject to NSSAA procedure and the status.

type: object

required:

- snssai

- status

properties:

snssai:

$ref: '#/components/schemas/Snssai'

status:

$ref: '#/components/schemas/AuthStatus'

NssaaStatusRm:

anyOf:

- $ref: '#/components/schemas/NssaaStatus'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'NssaaStatus' data type, but with

the OpenAPI 'nullable: true' property.

TnapId:

description: Contain the TNAP Identifier see clause5.6.2 of 3GPP TS 23.501.

type: object

properties:

ssId:

type: string

description: >

This IE shall be present if the UE is accessing the 5GC via a trusted WLAN access

network.When present, it shall contain the SSID of the access point to which the UE

is attached, that is received over NGAP,  see IEEE Std 802.11-2012.

bssId:

type: string

description: >

When present, it shall contain the BSSID of the access point to which the UE is

attached, that is received over NGAP, see IEEE Std 802.11-2012.

civicAddress:

$ref: '#/components/schemas/Bytes'

TnapIdRm:

anyOf:

- $ref: '#/components/schemas/TnapId'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'TnapId' data type, but with the

OpenAPI 'nullable: true' property.

TwapId:

description: >

Contain the TWAP Identifier as defined in clause 4.2.8.5.3 of 3GPP TS 23.501

or the WLAN location information as defined in clause 4.5.7.2.8 of 3GPP TS 23.402.

type: object

required:

- ssId

properties:

ssId:

type: string

description: >

This IE shall contain the SSID of the access point to which the UE is attached, that is

received over NGAP, see IEEE Std 802.11-2012.

bssId:

type: string

description: >

When present, it shall contain the BSSID of the access point to which the UE is

attached, for trusted WLAN access, see IEEE Std 802.11-2012.

civicAddress:

$ref: '#/components/schemas/Bytes'

TwapIdRm:

anyOf:

- $ref: '#/components/schemas/TwapId'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'TwapId' data type, but with the

OpenAPI 'nullable: true' property.

SnssaiExtension:

description: >

Extensions to the Snssai data type, sdRanges and wildcardSd shall not be present

simultaneously

type: object

not:

required:

- sdRanges

- wildcardSd

properties:

sdRanges:

description: >

When present, it shall contain the range(s) of Slice Differentiator values supported for

the Slice/Service Type value indicated in the sst attribute of the Snssai data type

type: array

items:

$ref: '#/components/schemas/SdRange'

minItems: 1

wildcardSd:

description: >

When present, it shall be set to true, to indicate that all SD values are supported for

the Slice/Service Type value indicated in the sst attribute of the Snssai data type.

type: boolean

enum:

- true

SdRange:

description: A range of SDs (Slice Differentiators)

type: object

properties:

start:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: >

First value identifying the start of an SD range. This string shall be formatted as

specified for the sd attribute of the Snssai data type in clause 5.4.4.2.

end:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: >

Last value identifying the end of an SD range. This string shall be formatted as

specified for the sd attribute of the Snssai data type in clause 5.4.4.2.

ProseServiceAuth:

description: >

Indicates whether the UE is authorized to use related services.

type: object

properties:

proseDirectDiscoveryAuth:

$ref: '#/components/schemas/UeAuth'

proseDirectCommunicationAuth:

$ref: '#/components/schemas/UeAuth'

proseL2RelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL3RelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL2RemoteAuth:

$ref: '#/components/schemas/UeAuth'

proseL3RemoteAuth:

$ref: '#/components/schemas/UeAuth'

proseMultipathComL2RemoteAuth:

$ref: '#/components/schemas/UeAuth'

proseL2UeRelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL3UeRelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL2EndAuth:

$ref: '#/components/schemas/UeAuth'

proseL3EndAuth:

$ref: '#/components/schemas/UeAuth'

proseL3IntermediateRelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL3RemoteMultihopAuth:

$ref: '#/components/schemas/UeAuth'

proseL3RelayMultihopAuth:

$ref: '#/components/schemas/UeAuth'

proseL3UeRelayMultihopAuth:

$ref: '#/components/schemas/UeAuth'

proseL3EndMultihopAuth:

$ref: '#/components/schemas/UeAuth'

proseL2IntermediateRelayAuth:

$ref: '#/components/schemas/UeAuth'

proseL2RemoteMultihopAuth:

$ref: '#/components/schemas/UeAuth'

proseL2RelayMultihopAuth:

$ref: '#/components/schemas/UeAuth'

EcsServerAddr:

description: >

Contains the Edge Configuration Server Address Configuration Information as defined in

clause 5.2.3.6.1 of 3GPP TS 23.502.

type: object

properties:

ecsFqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

ecsIpAddressList:

type: array

items:

$ref: '#/components/schemas/IpAddr'

minItems: 1

ecsUriList:

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

ecsProviderId:

type: string

EcsServerAddrRm:

anyOf:

- $ref: '#/components/schemas/EcsServerAddr'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the ' EcsServerAddr ' data type, but with

the OpenAPI 'nullable: true' property.

IpAddr:

description: Contains an IP adresse.

type: object

oneOf:

- required:

- ipv4Addr

- required:

- ipv6Addr

- required:

- ipv6Prefix

properties:

ipv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: '#/components/schemas/Ipv6Prefix'

SACInfo:

description: >

Represents threshold(s) to control the triggering of network slice reporting notifications

or the information contained in the network slice reporting notification.

type: object

properties:

numericValNumUes:

type: integer

numericValNumPduSess:

type: integer

percValueNumUes:

type: integer

minimum: 0

maximum: 100

percValueNumPduSess:

type: integer

minimum: 0

maximum: 100

uesWithPduSessionInd:

type: boolean

default: false

SACEventStatus:

description: >

Contains the network slice status information in terms of the current number of UEs

registered  with a network slice, the current number of PDU Sessions established on a

network slice or both.

type: object

properties:

reachedNumUes:

$ref: '#/components/schemas/SACInfo'

reachedNumPduSess:

$ref: '#/components/schemas/SACInfo'

SpatialValidityCond:

description: Contains the Spatial Validity Condition.

type: object

properties:

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

countries:

type: array

items:

$ref: '#/components/schemas/Mcc'

minItems: 1

geographicalServiceArea:

$ref: '#/components/schemas/GeoServiceArea'

SpatialValidityCondRm:

description: Contains the Spatial Validity Condition or the null value.

anyOf:

- $ref: '#/components/schemas/SpatialValidityCond'

- $ref: '#/components/schemas/NullValue'

ServerAddressingInfo:

description: Contains addressing information (IP addresses and/or FQDNs) of a server.

type: object

anyOf:

- required:

- ipv4Addresses

- required:

- ipv6Addresses

- required:

- fqdnList

properties:

ipv4Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv4Addr'

minItems: 1

ipv6Addresses:

type: array

items:

$ref: '#/components/schemas/Ipv6Addr'

minItems: 1

fqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

PcfUeCallbackInfo:

description: >

Contains the PCF for the UE information necessary for the PCF for the PDU session to send

SM Policy Association Establishment and Termination events.

type: object

properties:

callbackUri:

$ref: '#/components/schemas/Uri'

bindingInfo:

type: string

nullable: true

required:

- callbackUri

PduSessionInfo:

description: indicates the DNN and S-NSSAI combination of a PDU session.

type: object

properties:

snssai:

$ref: '#/components/schemas/Snssai'

dnn:

$ref: '#/components/schemas/Dnn'

required:

- dnn

- snssai

EasIpReplacementInfo:

description: Contains EAS IP replacement information for a Source and a Target EAS.

type: object

properties:

source:

$ref: '#/components/schemas/EasServerAddress'

target:

$ref: '#/components/schemas/EasServerAddress'

required:

- source

- target

EasServerAddress:

description: Represents the IP address and port of an EAS server.

type: object

properties:

ip:

$ref: '#/components/schemas/IpAddr'

port:

$ref: '#/components/schemas/Uinteger'

required:

- ip

- port

RoamingRestrictions:

description: >

Indicates if access is allowed to a given serving network, e.g. a PLMN (MCC, MNC) or an

SNPN (MCC, MNC, NID).

type: object

properties:

accessAllowed:

type: boolean

GeoServiceArea:

description: List of geographic area or list of civic address info

type: object

properties:

geographicAreaList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

civicAddressList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

MutingExceptionInstructions:

description: >

Indicates to an Event producer NF instructions for the subscription and stored events when

an exception (e.g. full buffer) occurs at the Event producer NF while the event is muted.

type: object

properties:

bufferedNotifs:

$ref: '#/components/schemas/BufferedNotificationsAction'

subscription:

$ref: '#/components/schemas/SubscriptionAction'

MutingNotificationsSettings:

description: >

Indicates the Event producer NF settings to the Event consumer NF

type: object

properties:

maxNoOfNotif:

type: integer

durationBufferedNotif:

$ref: '#/components/schemas/DurationSec'

CombGciAndHfcNIds:

description: >

Combined Global Cable Identifier and HFC Node Id

type: object

properties:

globalCableId:

$ref: '#/components/schemas/Gci'

hfcNId:

$ref: '#/components/schemas/HfcNId'

VplmnOffloadingInfo:

description: VPLMN Specific Offloading Information

type: object

nullable: true

properties:

offloadIdentifier:

$ref: '#/components/schemas/OffloadIdentifier'

vplmnId:

$ref: '#/components/schemas/PlmnId'

allowedTraffic:

type: boolean

default: true

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv4AddrMasks:

type: array

items:

$ref: '#/components/schemas/Ipv4AddrMask'

minItems: 1

ipv6AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

fqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

fqdnPatterns:

type: array

items:

$ref: '#/components/schemas/FqdnPatternMatchingRule'

minItems: 1

PartiallyAllowedSnssai:

description: >

Indicates a S-NSSAI that is partially allowed in the Registration Area.

type: object

properties:

snssai:

$ref: '#/components/schemas/Snssai'

allowedTaiList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

required:

- snssai

- allowedTaiList

VarRepPeriod:

description: >

Indicates the Variable reporting periodicity for event reporting

type: object

properties:

repPeriod:

$ref: '#/components/schemas/DurationSec'

percValueNfLoad:

allOf:

- $ref: '#/components/schemas/Uinteger'

minimum: 0

maximum: 100

required:

- repPeriod

RangingSlPosAuth:

description: >

Indicates whether the UE is authorized to use related services.

type: object

properties:

rgSlPosPc5Auth:

$ref: '#/components/schemas/UeAuth'

rgSlPosLocAuth:

$ref: '#/components/schemas/UeAuth'

rgSlPosClientAuth:

$ref: '#/components/schemas/UeAuth'

rgSlPosServerAuth:

$ref: '#/components/schemas/UeAuth'

NrA2xAuth:

description: Contains NR A2X services authorized information.

type: object

properties:

uavUeAuth:

$ref: '#/components/schemas/UeAuth'

LteA2xAuth:

description: Contains LTE A2X services authorized information.

type: object

properties:

uavUeAuth:

$ref: '#/components/schemas/UeAuth'

SliceUsageControlInfo:

description: The network slice usage control related information

type: object

required:

- sNssai

properties:

sNssai:

$ref: '#/components/schemas/Snssai'

deregInactTimer:

$ref: '#/components/schemas/DurationSec'

sessInactTimer:

$ref: '#/components/schemas/DurationSec'

anyOf:

- required: [ deregInactTimer ]

- required: [ sessInactTimer ]

SnssaiDnnItem:

description: Combination of S-NSSAIs and DNNs

type: object

properties:

snssaiList:

type: array

items:

$ref: '#/components/schemas/ExtSnssai'

minItems: 1

dnnList:

type: array

items:

$ref: '#/components/schemas/Dnn'

minItems: 1

anyOf:

- required: [ snssaiList ]

- required: [ dnnList ]

NtnTaiInfo:

description: Contains NR NTN TAI Information.

type: object

required:

- plmnId

- tacList

properties:

plmnId:

$ref: '#/components/schemas/PlmnIdNid'

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

derivedTac:

$ref: '#/components/schemas/Tac'

MitigationInfo:

description: include the congestion mitigation information.

type: object

properties:

percValueNumUes:

type: integer

minimum: 0

maximum: 100

newUesInd:

type: boolean

VplmnDlAmbr:

description: an Authorized DL Session AMBR for Offloading for the VPLMN

type: object

nullable: true

properties:

vplmnId:

$ref: '#/components/schemas/PlmnId'

sessionDlAmbr:

$ref: '#/components/schemas/BitRate'

required:

- vplmnId

- sessionDlAmbr

LocalOffloadingManagementInfo:

description: Local Offloading Management Information

type: object

nullable: true

properties:

offloadIdentifier:

$ref: '#/components/schemas/OffloadIdentifier'

allowedTraffic:

type: boolean

default: true

ipv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv4AddrMasks:

type: array

items:

$ref: '#/components/schemas/Ipv4AddrMask'

minItems: 1

ipv6AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6AddressRange'

minItems: 1

ipv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

fqdnList:

type: array

items:

$ref: '#/components/schemas/Fqdn'

minItems: 1

fqdnPatterns:

type: array

items:

$ref: '#/components/schemas/FqdnPatternMatchingRule'

minItems: 1

CagProvisionInformation:

description: CAG Provision Information

type: object

required:

- cagInfo

properties:

cagInfo:

type: array

items:

$ref: '#/components/schemas/CagId'

minItems: 1

commonValidTimePeriodList:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ValidTimePeriod'

minItems: 0

additionalValidTimePeriodList:

description: >

A map (list of key-value pairs) where CAG ID coverted to string serves the key;

and the value is an array of time periods associted with the CAG ID.

type: object

additionalProperties:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ValidTimePeriod'

minItems: 1

minProperties: 1

provisionOperation:

$ref: '#/components/schemas/CagProvisionOperation'

#

# Data types describing alternative data types or combinations of data types

#

ExtSnssai:

allOf:

- $ref: '#/components/schemas/Snssai'

- $ref: '#/components/schemas/SnssaiExtension'

description: >

The sdRanges and wildcardSd attributes shall be exclusive from each other. If one of these

attributes is present,  the sd attribute shall also be present and it shall contain one Slice

Differentiator value within the range of SD  (if the sdRanges attribute is present) or with

any value (if the wildcardSd attribute is present).

SnssaiReplaceInfo:

description: Indicates the status of an S-NSSAI and an alternative S-NSSAI optionally.

type: object

properties:

snssai:

$ref: '#/components/schemas/Snssai'

status:

$ref: '#/components/schemas/SnssaiStatus'

altSnssai:

$ref: '#/components/schemas/Snssai'

nsReplTerminInd:

$ref: '#/components/schemas/TerminationIndication'

plmnId:

$ref: '#/components/schemas/PlmnId'

mitigationInfo:

$ref: '#/components/schemas/MitigationInfo'

required:

- snssai

#

# Data Types related to 5G QoS as defined in clause 5.5

#

#

# SIMPLE DATA TYPES

#

#

Qfi:

type: integer

minimum: 0

maximum: 63

description: Unsigned integer identifying a QoS flow, within the range 0 to 63.

QfiRm:

type: integer

minimum: 0

maximum: 63

nullable: true

description: >

This data type is defined in the same way as the 'Qfi' data type, but with the

OpenAPI 'nullable: true' property.

5Qi:

type: integer

minimum: 0

maximum: 255

description: >

Unsigned integer representing a 5G QoS Identifier (see clause 5.7.2.1 of 3GPP TS 23.501,

within the range 0 to 255.

5QiRm:

type: integer

minimum: 0

maximum: 255

nullable: true

description: >

This data type is defined in the same way as the '5QiPriorityLevel' data type, but with

the OpenAPI 'nullable: true' property. "

BitRate:

type: string

pattern: '^\d+(\.\d+)? (bps|Kbps|Mbps|Gbps|Tbps)$'

description: >

String representing a bit rate; the prefixes follow the standard symbols from The International

System of Units, and represent x1000 multipliers, with the exception that prefix "K" is

used to represent the standard symbol "k".

BitRateRm:

type: string

pattern: '^\d+(\.\d+)? (bps|Kbps|Mbps|Gbps|Tbps)$'

nullable: true

description: >

This data type is defined in the same way as the 'BitRate' data type, but with the OpenAPI

'nullable: true' property.

PacketRate:

type: string

pattern: '^\d+(\.\d+)? (pps|kpps|Mpps|Gpps|Tpps)$'

description: >

String representing a packet rate, i.e., packet per second; the prefixes follow the symbols

from The International System of Units, and represent x1000 multipliers.

PacketRateRm:

type: string

pattern: '^\d+(\.\d+)? (pps|kpps|Mpps|Gpps|Tpps)$'

nullable: true

description: >

This data type is defined in the same way as the 'PacketRate' data type, but with the

OpenAPI 'nullable: true' property.

TrafficVolume:

type: string

pattern: '^\d+(\.\d+)? (B|kB|MB|GB|TB)$'

description: >

String representing a Traffic Volume measured in bytes; the prefixes follow the symbols
        from The International System of Units, and represent x1000 multipliers.

TrafficVolumeRm:

type: string

pattern: '^\d+(\.\d+)? (B|kB|MB|GB|TB)$'

nullable: true

description: >

This data type is defined in the same way as the 'TrafficVolume' data type, but with the

OpenAPI 'nullable: true' property.

ArpPriorityLevelRm:

type: integer

minimum: 1

maximum: 15

nullable: true

description: >

This data type is defined in the same way as the 'ArpPriorityLevel' data type, but with

the OpenAPI 'nullable: true' property.

ArpPriorityLevel:

type: integer

minimum: 1

maximum: 15

nullable: true

description: >

nullable true shall not be used for this attribute. Unsigned integer indicating the ARP

Priority Level (see clause 5.7.2.2 of 3GPP TS 23.501, within the range 1 to 15.Values are

ordered in decreasing order of priority, i.e. with 1 as the highest priority and 15 as

the lowest priority.

5QiPriorityLevel:

type: integer

minimum: 1

maximum: 127

description: >

Unsigned integer indicating the 5QI Priority Level (see clauses 5.7.3.3 and 5.7.4 of 3GPP

TS 23.501, within the range 1 to 127.Values are ordered in decreasing order of priority,

i.e. with 1 as the highest priority and 127 as the lowest priority.

5QiPriorityLevelRm:

type: integer

minimum: 1

maximum: 127

nullable: true

description: >

This data type is defined in the same way as the '5QiPriorityLevel' data type, but with

the OpenAPI 'nullable: true' property.

PacketDelBudget:

type: integer

minimum: 1

description: >

Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP

TS 23.501), expressed in milliseconds.

PacketDelBudgetRm:

type: integer

minimum: 1

nullable: true

description: >

This data type is defined in the same way as the 'PacketDelBudget' data type, but with

the OpenAPI 'nullable: true' property.

PacketErrRate:

type: string

pattern: '^([0-9]E-[0-9])$'

description: >

String representing Packet Error Rate (see clause 5.7.3.5 and 5.7.4 of 3GPP TS 23.501,

expressed as a "scalar x 10-k" where the scalar and the exponent k are each encoded as

one decimal digit.

PacketErrRateRm:

type: string

pattern: '^([0-9]E-[0-9])$'

nullable: true

description: >

This data type is defined in the same way as the 'PacketErrRate' data type, but with

the OpenAPI 'nullable: true' property.

PacketLossRate:

type: integer

minimum: 0

maximum: 1000

description: >

Unsigned integer indicating Packet Loss Rate (see clauses 5.7.2.8 and 5.7.4 of 3GPP

TS 23.501), expressed in tenth of percent.

PacketLossRateRm:

type: integer

minimum: 0

maximum: 1000

nullable: true

description: >

This data type is defined in the same way as the 'PacketLossRate' data type, but with

the OpenAPI 'nullable: true' property.

AverWindow:

type: integer

minimum: 1

maximum: 4095

default: 2000

description: >

Unsigned integer indicating Averaging Window (see clause 5.7.3.6 and 5.7.4 of

3GPP TS 23.501), expressed in milliseconds.

AverWindowRm:

type: integer

maximum: 4095

default: 2000

minimum: 1

nullable: true

description: >

This data type is defined in the same way as the 'AverWindow' data type, but with

the OpenAPI 'nullable: true' property.

MaxDataBurstVol:

type: integer

minimum: 1

maximum: 4095

description: >

Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of

3GPP TS 23.501), expressed in Bytes.

MaxDataBurstVolRm:

type: integer

minimum: 1

maximum: 4095

nullable: true

description: >

This data type is defined in the same way as the 'MaxDataBurstVol' data type, but with

the OpenAPI 'nullable: true' property.

SamplingRatio:

type: integer

minimum: 1

maximum: 100

description: >

Unsigned integer indicating Sampling Ratio (see clauses 4.15.1 of 3GPP TS 23.502),

expressed in percent.

SamplingRatioRm:

type: integer

minimum: 1

maximum: 100

nullable: true

description: >

This data type is defined in the same way as the 'SamplingRatio' data type, but with the

OpenAPI 'nullable: true' property.

#

RgWirelineCharacteristics:

$ref: '#/components/schemas/Bytes'

RgWirelineCharacteristicsRm:

anyOf:

- $ref: '#/components/schemas/RgWirelineCharacteristics'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'RgWirelineCharacteristics' data type,

but with the OpenAPI 'nullable: true' property.

ExtMaxDataBurstVol:

type: integer

minimum: 4096

maximum: 2000000

description: >

Unsigned integer indicating Maximum Data Burst Volume (see clauses 5.7.3.7 and 5.7.4 of

3GPP TS 23.501), expressed in Bytes.

ExtMaxDataBurstVolRm:

type: integer

minimum: 4096

maximum: 2000000

nullable: true

description: >

This data type is defined in the same way as the 'ExtMaxDataBurstVol' data type, but

with the OpenAPI 'nullable: true' property.

ExtPacketDelBudget:

type: integer

minimum: 1

description: >

Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP

TS 23.501 [8])), expressed in 0.01 milliseconds.

ExtPacketDelBudgetRm:

type: integer

minimum: 1

nullable: true

description: >

This data type is defined in the same way as the 'ExtPacketDelBudget' data type, but

with the OpenAPI 'nullable: true' property. "

Metadata:

format: byte

type: string

nullable: true

description: >

A String which is transparently passed to the UPF to be applied for traffic to SFC.

#

# ENUMERATED DATA TYPES

#

PreemptionCapability:

anyOf:

- type: string

enum:

- NOT_PREEMPT

- MAY_PREEMPT

- type: string

description: >

The enumeration PreemptionCapability indicates the pre-emption capability of a request on

other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501. It shall comply with the provisions

defined in table 5.5.3.1-1.

PreemptionCapabilityRm:

anyOf:

- $ref: '#/components/schemas/PreemptionCapability'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'PreemptionCapability' enumeration,

but with the OpenAPI 'nullable: true' property.

PreemptionVulnerability:

anyOf:

- type: string

enum:

- NOT_PREEMPTABLE

- PREEMPTABLE

- type: string

description: >

The enumeration PreemptionVulnerability indicates the pre-emption vulnerability of the QoS

flow to pre-emption from other QoS flows. See clause 5.7.2.2 of 3GPP TS 23.501. It shall

comply with the provisions defined in table 5.5.3.2-1

PreemptionVulnerabilityRm:

anyOf:

- $ref: '#/components/schemas/PreemptionVulnerability'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'PreemptionVulnerability' enumeration,

but with the OpenAPI 'nullable: true' property."

ReflectiveQoSAttribute:

anyOf:

- type: string

enum:

- RQOS

- NO_RQOS

- type: string

description: >

The enumeration ReflectiveQosAttribute indicates whether certain traffic of the QoS flow may

be subject to Reflective QoS (see clause 5.7.2.3 of 3GPP TS 23.501). It shall comply with

the provisions defined in table 5.5.3.3-1.

ReflectiveQoSAttributeRm:

anyOf:

- $ref: '#/components/schemas/ReflectiveQoSAttribute'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'ReflectiveQosAttribute' enumeration,

but with the OpenAPI 'nullable: true' property. "

NotificationControl:

anyOf:

- type: string

enum:

- REQUESTED

- NOT_REQUESTED

- type: string

description: >

The enumeration NotificationControl indicates whether notifications are requested from the

RAN when the GFBR can no longer  (or again) be fulfilled for a QoS Flow during the lifetime

of the QoS Flow (see clause 5.7.2.4 of 3GPP TS 23.501).

It shall comply with the provisions defined in table 5.5.3.5-1.

NotificationControlRm:

anyOf:

- $ref: '#/components/schemas/NotificationControl'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'NotificationControl' enumeration, but

with the OpenAPI 'nullable: true' property.

QosResourceType:

anyOf:

- type: string

enum:

- NON_GBR

- NON_CRITICAL_GBR

- CRITICAL_GBR

- type: string

description: >

The enumeration QosResourceType indicates whether a QoS Flow is non-GBR, delay critical GBR,

or non-delay critical GBR (see clauses 5.7.3.4 and 5.7.3.5 of 3GPP TS 23.501). It shall

comply with the provisions defined in table 5.5.3.6-1.

QosResourceTypeRm:

anyOf:

- $ref: '#/components/schemas/QosResourceType'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'QosResourceType' enumeration, but

with the OpenAPI 'nullable: true' property. "

AdditionalQosFlowInfo:

anyOf:

- anyOf:

- type: string

enum:

- MORE_LIKELY

- type: string

- $ref: '#/components/schemas/NullValue'

description: >

The enumeration AdditionalQosFlowInfo provides additional QoS flow information (see clause

9.3.1.12 3GPP TS 38.413 [11]). It shall comply with the provisions defined in table

5.5.3.12-1.

PartitioningCriteria:

anyOf:

- type: string

enum:

- TAC

- SUBPLMN

- GEOAREA

- SNSSAI

- DNN

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- "TAC": Type Allocation Code

- "SUBPLMN": Subscriber PLMN ID

- "GEOAREA": Geographical area, i.e. list(s) of TAI(s)

- "SNSSAI": S-NSSAI

- "DNN": DNN

PartitioningCriteriaRm:

anyOf:

- $ref: '#/components/schemas/PartitioningCriteria'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the ' PartitioningCriteria ' data type, but

with the OpenAPI 'nullable: true' property.

PduSetHandlingInfo:

anyOf:

- type: string

enum:

- ALL_PDUS_NEEDED

- ALL_PDUS_NOT_NEEDED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

- "ALL_PDUS_NEEDED": All PDUs of the PDU Set are needed

- "ALL_PDUS_NOT_NEEDED": All PDUs of the PDU Set are not needed

MediaTransportProto:

anyOf:

- type: string

enum:

- RTP

- SRTP

- MOQT

- type: string

description: >

The enumeration MediaTransportProto indicates the transport protocol used for a media flow.

MediaTransportProtoRm:

anyOf:

- $ref: '#/components/schemas/MediaTransportProto'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'MediaTransportProto' data type,

but with the OpenAPI 'nullable: true' property.

RtpHeaderExtType:

anyOf:

- type: string

enum:

- PDU_SET_MARKING

- DYN_CHANGING_TRAFFIC_CHAR

- EXPEDITED_TRANSFER_IND

- type: string

description: >

The enumeration indicates the type of Rtp Header Extension type

RtpHeaderExtTypeRm:

anyOf:

- $ref: '#/components/schemas/RtpHeaderExtType'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'RtpHeaderExtType' data type,

but with the OpenAPI 'nullable: true' property.

RtpPayloadFormat:

anyOf:

- type: string

enum:

- H264

- H265

- type: string

description: >

The enumeration RtpPayloadFormat indicates the RTP Payload format

RtpPayloadFormatRm:

anyOf:

- $ref: '#/components/schemas/RtpPayloadFormat'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'RtpPayloadFormat' data type, but with the

OpenAPI 'nullable: true' property.

MriTransferMethod:

anyOf:

- type: string

enum:

- UDP_OPTION

- type: string

description: >

This data type indicates the method used for transferring Media Related Information.

MriTransferMethodRm:

anyOf:

- $ref: '#/components/schemas/MriTransferMethod'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'MriTransferMethod' data type,

but with the OpenAPI 'nullable: true' property.

#

#

# STRUCTURED DATA TYPES

#

Arp:

description: Contains Allocation and Retention Priority information.

type: object

properties:

priorityLevel:

$ref: '#/components/schemas/ArpPriorityLevel'

preemptCap:

$ref: '#/components/schemas/PreemptionCapability'

preemptVuln:

$ref: '#/components/schemas/PreemptionVulnerability'

required:

- priorityLevel

- preemptCap

- preemptVuln

Ambr:

description: Contains the maximum aggregated uplink and downlink bit rates.

type: object

properties:

uplink:

$ref: '#/components/schemas/BitRate'

downlink:

$ref: '#/components/schemas/BitRate'

required:

- uplink

- downlink

Dynamic5Qi:

description: >

It indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI

for downlink and uplink.

type: object

properties:

resourceType:

$ref: '#/components/schemas/QosResourceType'

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

packetDelayBudget:

$ref: '#/components/schemas/PacketDelBudget'

packetErrRate:

$ref: '#/components/schemas/PacketErrRate'

averWindow:

$ref: '#/components/schemas/AverWindow'

maxDataBurstVol:

$ref: '#/components/schemas/MaxDataBurstVol'

extMaxDataBurstVol:

$ref: '#/components/schemas/ExtMaxDataBurstVol'

extPacketDelBudget:

$ref: '#/components/schemas/ExtPacketDelBudget'

cnPacketDelayBudgetDl:

$ref: '#/components/schemas/ExtPacketDelBudget'

cnPacketDelayBudgetUl:

$ref: '#/components/schemas/ExtPacketDelBudget'

required:

- resourceType

- priorityLevel

- packetDelayBudget

- packetErrRate

NonDynamic5Qi:

description: >

It indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink

and uplink.

type: object

properties:

priorityLevel:

$ref: '#/components/schemas/5QiPriorityLevel'

averWindow:

$ref: '#/components/schemas/AverWindow'

maxDataBurstVol:

$ref: '#/components/schemas/MaxDataBurstVol'

extMaxDataBurstVol:

$ref: '#/components/schemas/ExtMaxDataBurstVol'

cnPacketDelayBudgetDl:

$ref: '#/components/schemas/ExtPacketDelBudget'

cnPacketDelayBudgetUl:

$ref: '#/components/schemas/ExtPacketDelBudget'

minProperties: 0

ArpRm:

anyOf:

- $ref: '#/components/schemas/Arp'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Arp' data type, but with the

OpenAPI 'nullable: true' property.

AmbrRm:

anyOf:

- $ref: '#/components/schemas/Ambr'

- $ref: '#/components/schemas/NullValue'

description: >

This data type is defined in the same way as the 'Ambr' data type, but with the

OpenAPI 'nullable: true' property."

SliceMbr:

description: MBR related to slice

type: object

properties:

uplink:

$ref: '#/components/schemas/BitRate'

downlink:

$ref: '#/components/schemas/BitRate'

required:

- uplink

- downlink

SliceMbrRm:

description: "SliceMbr with nullable: true"

anyOf:

- $ref: '#/components/schemas/SliceMbr'

- $ref: '#/components/schemas/NullValue'

PduSetQosPara:

description: Represents the PDU Set level QoS parameters.

type: object

properties:

pduSetDelayBudget:

$ref: '#/components/schemas/ExtPacketDelBudget'

pduSetErrRate:

$ref: '#/components/schemas/PacketErrRate'

pduSetHandlingInfo:

$ref: '#/components/schemas/PduSetHandlingInfo'

anyOf:

- required: [ pduSetDelayBudget, pduSetErrRate ]

- required: [ pduSetHandlingInfo ]

#

PduSetQosParaRm:

description: "PduSetQosPara contains removable attributes"

type: object

nullable: true

properties:

pduSetDelayBudget:

$ref: '#/components/schemas/ExtPacketDelBudgetRm'

pduSetErrRate:

$ref: '#/components/schemas/PacketErrRateRm'

pduSetHandlingInfo:

$ref: '#/components/schemas/PduSetHandlingInfoRm'

ProtocolDescription:

description: ProtocolDescription contains information to derive PDU set information.

type: object

properties:

transportProto:

$ref: '#/components/schemas/MediaTransportProto'

rtpHeaderExtInfo:

$ref: '#/components/schemas/RtpHeaderExtInfo'

addRtpHeaderExtInfo:

type: array

items:

$ref: '#/components/schemas/RtpHeaderExtInfo'

minItems: 1

rtpPayloadInfoList:

type: array

items:

$ref: '#/components/schemas/RtpPayloadInfo'

minItems: 1

mriTransferInfo:

$ref: '#/components/schemas/MriTransferMethod'

ProtocolDescriptionRm:

description: Describes the modifications to the ProtocolDescription data type.

type: object

nullable: true

properties:

transportProto:

$ref: '#/components/schemas/MediaTransportProtoRm'

rtpHeaderExtInfo:

$ref: '#/components/schemas/RtpHeaderExtInfoRm'

addRtpHeaderExtInfo:

type: array

nullable: true

items:

$ref: '#/components/schemas/RtpHeaderExtInfo'

minItems: 1

rtpPayloadInfoList:

type: array

nullable: true

items:

$ref: '#/components/schemas/RtpPayloadInfo'

minItems: 1

mriTransferInfo:

$ref: '#/components/schemas/MriTransferMethodRm'

RtpHeaderExtInfo:

description: RTP Header Extension information

type: object

properties:

rtpHeaderExtType:

$ref: '#/components/schemas/RtpHeaderExtType'

rtpHeaderExtId:

type: integer

minimum: 1

maximum: 255

longFormat:

type: boolean

pduSetSizeActive:

type: boolean

pduSetPduCountActive:

type: boolean

RtpHeaderExtInfoRm:

description: Describes the modifications to RtpHeaderExtInfo data type

type: object

nullable: true

properties:

rtpHeaderExtType:

$ref: '#/components/schemas/RtpHeaderExtTypeRm'

rtpHeaderExtId:

type: integer

nullable: true

minimum: 1

maximum: 255

longFormat:

type: boolean

nullable: true

pduSetSizeActive:

type: boolean

nullable: true

pduSetPduCountActive:

type: boolean

nullable: true

RtpPayloadInfo:

description: RtpPayloadInfo contains Rtp payload type and format.

type: object

properties:

rtpPayloadTypeList:

type: array

items:

type: integer

minimum: 0

maximum: 127

minItems: 1

rtpPayloadFormat:

$ref: '#/components/schemas/RtpPayloadFormat'

RtpPayloadInfoRm:

nullable: true

description: Describes the modifications to the RtpPayloadInfo data type.

type: object

properties:

rtpPayloadTypeList:

type: array

nullable: true

items:

type: integer

minimum: 0

maximum: 127

minItems: 1

rtpPayloadFormat:

$ref: '#/components/schemas/RtpPayloadFormatRm'

PduSetHandlingInfoRm:

anyOf:

- $ref: '#/components/schemas/PduSetHandlingInfo'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'PduSetHandlingInfo' enumeration,

but with the OpenAPI 'nullable: true' property.

MriTransferInfo:

description: Describes how media related information is transferred

type: object

properties:

transferMethod:

$ref: '#/components/schemas/MriTransferMethod'

MriTransferInfoRm:

description: Describes how media related information is transferred

type: object

nullable: true

properties:

transferMethod:

$ref: '#/components/schemas/MriTransferMethodRm'

#

# Data Types related to 5G Trace as defined in clause 5.6

#

#

# SIMPLE DATA TYPES

#

PhysCellId:

type: integer

minimum: 0

maximum: 1007

description: >

Integer value identifying the physical cell identity (PCI), as definition of "PhysCellId" IE

in clause 6.3.2 of 3GPP TS 38.331.

ArfcnValueNR:

type: integer

minimum: 0

maximum: 3279165

description: >

Integer value indicating the ARFCN applicable for a downlink, uplink or bi-directional (TDD)

NR global frequency raster,

as definition of "ARFCN-ValueNR" IE in clause 6.3.2 of 3GPP TS 38.331.

QoeReference:

description: >

String containing MCC (3 digits), MNC (2 or 3 digits)

and QMC ID (3 octets, encoded as 6 hexadecimal digits).

type: string

pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{6}$'

MdtAlignmentInfo:

description: |

String containing:

- Trace Reference: MCC (3 digits), MNC (2 or 3 digits),

Trace ID (3 octets, encoded as 6 hexadecimal digits)

- Trace Recording Session Reference (2 octets, encoded as 4 hexadecimal digits)

format: string

pattern: '^[0-9]{3}-[0-9]{2,3}-[A-Fa-f0-9]{6}-[A-Fa-f0-9]{4}$'

#

#

# Enumerations

#

TraceDepth:

anyOf:

- type: string

enum:

- MINIMUM

- MEDIUM

- MAXIMUM

- MINIMUM_WO_VENDOR_EXTENSION

- MEDIUM_WO_VENDOR_EXTENSION

- MAXIMUM_WO_VENDOR_EXTENSION

- type: string

description: >

The enumeration TraceDepth defines how detailed information should be recorded

in the trace. See 3GPP TS 32.422 for further description of the values.

It shall comply with the provisions defined in table 5.6.3.1-1

TraceDepthRm:

anyOf:

- $ref: '#/components/schemas/TraceDepth'

- $ref: '#/components/schemas/NullValue'

description: >

This enumeration is defined in the same way as the 'TraceDepth' enumeration, but with

the OpenAPI 'nullable: true' property.

JobType:

anyOf:

- type: string

enum:

- IMMEDIATE_MDT_ONLY

- LOGGED_MDT_ONLY

- TRACE_ONLY

- IMMEDIATE_MDT_AND_TRACE

- LOGGED_MBSFN_MDT

- 5GC_UE_LEVEL_MEASUREMENTS_ONLY

- TRACE_AND_5GC_UE_LEVEL_MEASUREMENTS_ONLY

- IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS

- TRACE_IMMEDIATE_MDT_AND_5GC_UE_LEVEL_MEASUREMENTS

- type: string

description: >

The enumeration JobType defines Job Type in the trace. See 3GPP TS 32.422 for further

description of the values. It shall comply with the provisions defined in table 5.6.3.3-1.

ReportTypeMdt:

anyOf:

- type: string

enum:

- PERIODICAL

- EVENT_TRIGGED

- type: string

description: >

The enumeration ReportTypeMdt defines Report Type for logged MDT in the trace. See 3GPP TS

32.422 for further description of the values. It shall comply with the provisions defined

in table 5.6.3.4-1.

MeasurementLteForMdt:

anyOf:

- type: string

enum:

- M1

- M2

- M3

- M4_DL

- M4_UL

- M5_DL

- M5_UL

- M6_DL

- M6_UL

- M7_DL

- M7_UL

- M8

- M9

- type: string

description: >

The enumeration MeasurementLteForMdt defines Measurements used for MDT in LTE in the trace.

See 3GPP TS 32.422 for further description of the values. It shall comply with the

provisions defined in table 5.6.3.5-1.

MeasurementNrForMdt:

anyOf:

- type: string

enum:

- M1

- M2

- M3

- M4_DL

- M4_UL

- M5_DL

- M5_UL

- M6_DL

- M6_UL

- M7_DL

- M7_UL

- M8

- M9

- type: string

description: >

The enumeration MeasurementNrForMdt defines Measurements used for MDT in NR in the trace.

See 3GPP TS 32.422 for further description of the values. It shall comply with the

provisions defined in table 5.6.3.6-1.

SensorMeasurement:

anyOf:

- type: string

enum:

- BAROMETRIC_PRESSURE

- UE_SPEED

- UE_ORIENTATION

- type: string

description: >

The enumeration SensorMeasurement defines sensor measurement type for MDT in the trace.

See 3GPP TS 32.422 for further description of the values. It shall comply with the

provisions defined in table 5.6.3.7-1.

ReportingTrigger:

anyOf:

- type: string

enum:

- PERIODICAL

- EVENT_A2

- EVENT_A2_PERIODIC

- ALL_RRM_EVENT_TRIGGERS

- type: string

description: >

The enumeration ReportingTrigger defines Reporting Triggers for MDT in the trace. See 3GPP

TS 32.42] for further  description of the values. It shall comply with the provisions

defined in table 5.6.3.8-1.

ReportIntervalMdt:

anyOf:

- type: string

enum:

- 120

- 240

- 480

- 640

- 1024

- 2048

- 5120

- 10240

- 60000

- 360000

- 720000

- 1800000

- 3600000

- type: string

description: >

The enumeration ReportIntervalMdt defines Report Interval for MDT in the trace. See 3GPP

TS 32.422 for further description of the values. It shall comply with

the provisions defined in table 5.6.3.9-1.

ReportAmountMdt:

anyOf:

- type: string

enum:

- 1

- 2

- 4

- 8

- 16

- 32

- 64

- infinity

- type: string

description: >

The enumeration ReportAmountMdt defines Report Amount for MDT in the trace. See 3GPP

TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.10-1.

EventForMdt:

anyOf:

- type: string

enum:

- OUT_OF_COVERAG

- A2_EVENT

- type: string

description: >

The enumeration EventForMdt defines events triggered measurement for logged MDT in the

trace. See 3GPP TS 32.422 for further description of the values. It shall comply with

the provisions defined in table 5.6.3.11-1

LoggingIntervalMdt:

anyOf:

- type: string

enum:

- 128

- 256

- 512

- 1024

- 2048

- 3072

- 4096

- 6144

- type: string

description: >

The enumeration LoggingIntervalMdt defines Logging Interval for MDT in the trace. See 3GPP

TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.12-1.

LoggingDurationMdt:

anyOf:

- type: string

enum:

- 600

- 1200

- 2400

- 3600

- 5400

- 7200

- type: string

description: >

The enumeration LoggingIntervalMdt defines Logging Interval for MDT in the trace. See 3GPP

TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.12-1.

PositioningMethodMdt:

anyOf:

- type: string

enum:

- GNSS

- E_CELL_ID

- type: string

description: >

The enumeration LoggingDurationMdt defines Logging Duration for MDT in the trace. See 3GPP

TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.13-1.

CollectionPeriodRmmLteMdt:

anyOf:

- type: string

enum:

- 1024

- 1280

- 2048

- 2560

- 5120

- 10240

- 60000

- type: string

description: >

The enumeration CollectionPeriodRmmLteMdt defines Collection period for RRM measurements

LTE for MDT in the trace. See 3GPP TS 32.422 for further description of the values.

It shall comply with the provisions defined in table 5.6.3.15-1.

MeasurementPeriodLteMdt:

anyOf:

- type: string

enum:

- 1024

- 1280

- 2048

- 2560

- 5120

- 10240

- 60000

- type: string

description: >

The enumeration MeasurementPeriodLteMdt defines Measurement period LTE for MDT in the

trace.  See 3GPP TS 32.422 for further description of the values. It shall comply

with the provisions defined in table 5.6.3.16-1.

ReportIntervalNrMdt:

anyOf:

- type: string

enum:

- 120

- 240

- 480

- 640

- 1024

- 2048

- 5120

- 10240

- 20480

- 40960

- 60000

- 360000

- 720000

- 1800000

- 3600000

- type: string

description: >

The enumeration ReportIntervalNrMdt defines Report Interval in NR for MDT in the trace. See

3GPP TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.17-1.

LoggingIntervalNrMdt:

anyOf:

- type: string

enum:

- 128

- 256

- 512

- 1024

- 2048

- 3072

- 4096

- 6144

- 320

- 640

- infinity

- type: string

description: >

The enumeration LoggingIntervalNrMdt defines Logging Interval in NR for MDT in the trace.

See 3GPP TS 32.422 for further description of the values. It shall comply with the

provisions defined in table 5.6.3.18-1.

CollectionPeriodRmmNrMdt:

anyOf:

- type: string

enum:

- 1024

- 2048

- 5120

- 10240

- 60000

- type: string

description: >

The enumeration CollectionPeriodRmmNrMdt defines Collection period for RRM measurements NR

for MDT in the trace. See 3GPP TS 32.422 for further description of the values. It shall comply with the provisions defined in table 5.6.3.19-1

LoggingDurationNrMdt:

anyOf:

- type: string

enum:

- 600

- 1200

- 2400

- 3600

- 5400

- 7200

- type: string

description: >

The enumeration LoggingDurationMdt defines Logging Duration in NR for MDT in the trace. See

3GPP TS 32.422 for further description of the values. It shall comply with the provisions

defined in table 5.6.3.20-1.

QoeServiceType:

description: >

The enumeration QoeServiceType indicates the kind of service that shall be recorded for

QMC. It shall comply with the provisions defined in TS 29.571, table 5.6.3.21-1.

anyOf:

- type: string

enum:

- DASH

- MTSI

- VR

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

AvailableRanVisibleQoeMetric:

description: >

The enumeration AvailableRanVisibleQoeMetric indicates different available

RAN-visible QoE metrics to the gNB. It shall comply with the provisions defined

in TS 29.571, table 5.6.3.22-1.

anyOf:

- type: string

enum:

- APPLICATION_LAYER_BUFFER_LEVEL_LIST

- PLAYOUT_DELAY_FOR_MEDIA_STARTUP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

MeasurementType:

anyOf:

- type: string

enum:

- GTP_DELAYDLPSAUPFUEMEAN_SNSSAI_QFI

- GTP_DELAYULPSAUPFUEMEANEXCD1_SNSSAI_QFI

- GTP_DELAYDLPSAUPFUEMEANINCD1_SNSSAI_QFI

- GTP_DELAYULPSAUPFNGRANMEAN_SNSSAI_QFI

- GTP_DELAYDLPSAUPFNGRANMEAN_SNSSAI_QFI

- GTP_RTTDELAYPSAUPFNGRANMEAN

- GTP_RTTDELAYPSAUPFNGRANMEAN_SNSSAI

- GTP_RTTDELAYPSAUPFNGRANMEAN_QFI

- GTP_RTTDELAYPSAUPFNGRANMEAN_SNSSAI_QFI

- type: string

description: >

The enumeration MeasurementType defines Measurement Type in the

5GC UE level measurements trace.

BluetoothRssi:

anyOf:

- type: string

enum:

- TRUE

- type: string

description: >

The enumeration BluetoothRssi defines RSSI measurement by UE for Bluetooth.

WlanRssi:

anyOf:

- type: string

enum:

- TRUE

- type: string

description: >

The enumeration WlanRssi defines RSSI measurement by UE for WLAN.

WlanRtt:

anyOf:

- type: string

enum:

- TRUE

- type: string

description: >

The enumeration WlanRtt defines RTT measurement by UE for WLAN.

#

# STRUCTURED DATA TYPES

#

TraceData:

description: contains Trace control and configuration parameters.

type: object

nullable: true

properties:

traceRef:

type: string

pattern: '^[0-9]{3}[0-9]{2,3}-[A-Fa-f0-9]{6}$'

description: >

Trace Reference (see 3GPP TS 32.422).It shall be encoded as the concatenation

of MCC, MNC and Trace ID as follows: <MCC><MNC>-<Trace ID>

The Trace ID shall be encoded as a 3 octet string in hexadecimal

representation. Each character in the Trace ID string shall

take a value of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits.

The most significant character representing the 4 most significant bits of the

Trace ID shall appear first in the string, and the character representing the

4 least significant bit of the Trace ID shall appear last in the string.

traceDepth:

$ref: '#/components/schemas/TraceDepth'

neTypeList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

List of NE Types (see 3GPP TS 32.422). It shall be encoded as an octet string

in hexadecimal representation. Each character in the string shall take a value

of "0" to "9", "a" to "f" or "A" to "F" and shall represent 4 bits.

The most significant character representing the 4 most significant bits shall

appear first in the string, and the character representing the 4 least

significant bit shall appear last in the string. Octets shall be coded

according to 3GPP TS 32.422.

eventList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

Triggering events (see 3GPP TS 32.422). It shall be encoded as an octet string in

hexadecimal representation. Each character in the string shall take a value of "0"

to "9", "a" to "f" or "A" to "F" and shall represent 4 bits.

The most significant character representing the 4 most significant bits shall

appear first in the string, and the character representing the 4 least

significant bit shall appear last in the string. Octets shall be coded

according to 3GPP TS 32.422.

collectionEntityIpv4Addr:

$ref: '#/components/schemas/Ipv4Addr'

collectionEntityIpv6Addr:

$ref: '#/components/schemas/Ipv6Addr'

traceReportingConsumerUri:

$ref: '#/components/schemas/Uri'

interfaceList:

type: string

pattern: '^[A-Fa-f0-9]+$'

description: >

List of Interfaces (see 3GPP TS 32.422). It shall be encoded as an octet string in

hexadecimal representation.

Each character in the string shall take a value of "0" to "9", "a" to "f" or "A" to "F"

and shall represent 4 bits. The most significant character representing the 4 most

significant bits shall appear first in the string, and the character representing the

4 least significant bit shall appear last in the string. Octets shall be coded

according to 3GPP TS 32.422. If this attribute is not present, all the interfaces

applicable to the list of NE types indicated in the neTypeList attribute should

be traced.

jobType:

$ref: '#/components/schemas/JobType'

required:

- traceRef

- traceDepth

- neTypeList

- eventList

MdtConfiguration:

description: contains contain MDT configuration data.

type: object

required:

- jobType

properties:

jobType:

$ref: '#/components/schemas/JobType'

reportType:

$ref: '#/components/schemas/ReportTypeMdt'

areaScope:

$ref: '#/components/schemas/AreaScope'

measurementLteList:

type: array

items:

$ref: '#/components/schemas/MeasurementLteForMdt'

measurementNrList:

type: array

items:

$ref: '#/components/schemas/MeasurementNrForMdt'

minItems: 1

sensorMeasurementList:

type: array

items:

$ref: '#/components/schemas/SensorMeasurement'

minItems: 1

sensorMeasurementLteList:

type: array

items:

$ref: '#/components/schemas/SensorMeasurement'

minItems: 1

reportingTriggerList:

type: array

items:

$ref: '#/components/schemas/ReportingTrigger'

minItems: 1

reportInterval:

$ref: '#/components/schemas/ReportIntervalMdt'

reportIntervalNr:

$ref: '#/components/schemas/ReportIntervalNrMdt'

reportAmount:

$ref: '#/components/schemas/ReportAmountMdt'

reportAmountPerMeasurementLte:

type: object

additionalProperties:

$ref: '#/components/schemas/ReportAmountMdt'

minProperties: 1

description: >

A map (list of key-value pairs) where MeasurementLteForMdt serves as key;

reportAmountPerMeasurementNr:

type: object

additionalProperties:

$ref: '#/components/schemas/ReportAmountMdt'

minProperties: 1

description: >

A map (list of key-value pairs) where MeasurementNrForMdt serves as key;

eventThresholdRsrp:

type: integer

minimum: 0

maximum: 97

description: >

This IE shall be present if the report trigger parameter is configured for A2 event

reporting or A2 event triggered periodic reporting and the job type parameter is

configured for Immediate MDT or combined Immediate MDT and Trace in LTE.

When present, this IE shall indicate the Event Threshold for RSRP, and the value shall

be between 0-97.

mnOnlyInd:

type: boolean

default: false

eventThresholdRsrpNr:

type: integer

minimum: 0

maximum: 127

description: >

This IE shall be present if the report trigger parameter is configured for A2 event

reporting or A2 event triggered periodic reporting and the job type parameter is

configured for Immediate MDT, combined Immediate MDT

and Trace, Immediate MDT and 5GC UE level measurements or Trace, Immediate MDT and 5GC

UE level measurements in NR. When present,

this IE shall indicate the Event Threshold for RSRP, and the value shall be

between 0-127.

eventThresholdRsrq:

type: integer

minimum: 0

maximum: 34

description: >

This IE shall be present if the report trigger parameter is configured for A2 event

reporting or A2 event triggered periodic reporting and the job type parameter is

configured for Immediate MDT or combined Immediate MDT and Trace in LTE.When present,

this IE shall indicate the Event Threshold for RSRQ, and the value shall be

between 0-34.

eventThresholdRsrqNr:

type: integer

minimum: 0

maximum: 127

description: >

This IE shall be present if the report trigger parameter is configured for A2 event

reporting or A2 event triggered periodic reporting and the job type parameter is

configured for Immediate MDT, combined Immediate MDT and Trace, Immediate MDT and 5GC

UE level measurements or Trace, Immediate MDT and 5GC UE level measurements in NR.

When present,

this IE shall indicate the Event Threshold for RSRQ, and the value shall be

between 0-127.

eventList:

type: array

items:

$ref: '#/components/schemas/EventForMdt'

minItems: 1

loggingInterval:

$ref: '#/components/schemas/LoggingIntervalMdt'

loggingIntervalNr:

$ref: '#/components/schemas/LoggingIntervalNrMdt'

loggingDuration:

$ref: '#/components/schemas/LoggingDurationMdt'

loggingDurationNr:

$ref: '#/components/schemas/LoggingDurationNrMdt'

positioningMethod:

$ref: '#/components/schemas/PositioningMethodMdt'

addPositioningMethodList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodMdt'

minItems: 1

collectionPeriodRmmLte:

$ref: '#/components/schemas/CollectionPeriodRmmLteMdt'

collectionPeriodRmmNr:

$ref: '#/components/schemas/CollectionPeriodRmmNrMdt'

measurementPeriodLte:

$ref: '#/components/schemas/MeasurementPeriodLteMdt'

mdtAllowedPlmnIdList:

type: array

items:

$ref: '#/components/schemas/PlmnId'

minItems: 1

maxItems: 16

mbsfnAreaList:

type: array

items:

$ref: '#/components/schemas/MbsfnArea'

minItems: 1

maxItems: 8

interFreqTargetList:

type: array

items:

$ref: '#/components/schemas/InterFreqTargetInfo'

minItems: 1

maxItems: 8

eventThresholdSinrNr:

type: integer

minimum: 0

maximum: 127

bluetoothMeasurementNr:

$ref: '#/components/schemas/BluetoothMeasurement'

bluetoothMeasurementLte:

$ref: '#/components/schemas/BluetoothMeasurement'

wlanMeasurementNr:

$ref: '#/components/schemas/WlanMeasurement'

wlanMeasurementLte:

$ref: '#/components/schemas/WlanMeasurement'

sliceAreaScope:

type: array

items:

$ref: '#/components/schemas/SliceScopePerPlmn'

minItems: 1

maxItems: 16

AreaScope:

description: Contain the area based on Cells or Tracking Areas.

type: object

properties:

eutraCellIdList:

type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

nrCellIdList:

type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

tacInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/TacInfo'

minProperties: 1

description: >

A map (list of key-value pairs) where PlmnId converted to a string serves as key

cagInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/CagInfo'

minProperties: 1

description: >

A map (list of key-value pairs) where PlmnId converted to a string serves as key

nidInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/NidInfo'

minProperties: 1

description: >

A map (list of key-value pairs) where PlmnId converted to a string serves as key

cellIdNidInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/CellIdNidInfo'

minProperties: 1

description: >

A map (list of key-value pairs) where PlmnId converted to a string serves as key

tacNidInfoPerPlmn:

type: object

additionalProperties:

$ref: '#/components/schemas/TacNidInfo'

minProperties: 1

description: >

A map (list of key-value pairs) where PlmnId converted to a string serves as key

cagList:

type: array

items:

$ref: '#/components/schemas/CagId'

minItems: 1

TacInfo:

description: contains tracking area information (tracking area codes).

type: object

required:

- tacList

properties:

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

CagInfo:

description: contains CAG IDs.

type: object

required:

- cagList

properties:

cagList:

type: array

items:

$ref: '#/components/schemas/CagId'

minItems: 1

NidInfo:

description: contains NIDs.

type: object

required:

- nidList

properties:

nidList:

type: array

items:

$ref: '#/components/schemas/Nid'

minItems: 1

CellIdNidInfo:

description: contains a list of the NR Cell Identities in SNPN.

type: object

required:

- cellIdNidList

properties:

cellIdNidList:

type: array

items:

$ref: '#/components/schemas/CellIdNid'

minItems: 1

CellIdNid:

description: contains a NR Cell Identity and Network Identity.

type: object

required:

- cellId

- nid

properties:

cellId:

$ref: '#/components/schemas/NrCellId'

nid:

$ref: '#/components/schemas/Nid'

TacNidInfo:

description: contains a list of the tracking area codes in SNPN.

type: object

required:

- tacNidList

properties:

tacNidList:

type: array

items:

$ref: '#/components/schemas/TacNid'

minItems: 1

TacNid:

description: contains a tracking area code and Network Identity.

type: object

required:

- tac

- nid

properties:

tac:

$ref: '#/components/schemas/Tac'

nid:

$ref: '#/components/schemas/Nid'

MbsfnArea:

description: Contains an MBSFN area information.

type: object

properties:

mbsfnAreaId:

type: integer

minimum: 0

maximum: 255

description: This IE shall contain the MBSFN Area ID.

carrierFrequency:

type: integer

minimum: 0

maximum: 262143

description: When present, this IE shall contain the Carrier Frequency (EARFCN).

InterFreqTargetInfo:

description: Indicates the Inter Frequency Target information.

required:

- dlCarrierFreq

type: object

properties:

dlCarrierFreq:

$ref: '#/components/schemas/ArfcnValueNR'

cellIdList:

type: array

items:

$ref: '#/components/schemas/PhysCellId'

minItems: 1

maxItems: 32

description: >

When present, this IE shall contain a list of the physical cell identities where the

UE is requested to perform measurement logging for the indicated frequency.

QmcConfigInfo:

description: >

It contains the configuration information for signaling-based activation of the

Quality of Experience (QoE) Measurements Collection (QMC) functionality.

type: object

required:

- qoeReference

properties:

qoeReference:

$ref: '#/components/schemas/QoeReference'

serviceType:

$ref: '#/components/schemas/QoeServiceType'

sliceScope:

type: array

items:

$ref: '#/components/schemas/Snssai'

minItems: 1

areaScope:

$ref: '#/components/schemas/QmcAreaScope'

qoeCollectionEntityAddress:

$ref: '#/components/schemas/IpAddr'

qoeTarget:

$ref: '#/components/schemas/QoeTarget'

mdtAlignmentInfo:

$ref: '#/components/schemas/MdtAlignmentInfo'

availableRanVisibleQoeMetrics:

type: array

items:

$ref: '#/components/schemas/AvailableRanVisibleQoeMetric'

minItems: 1

containerForAppLayerMeasConfig:

$ref: '#/components/schemas/Bytes'

mbsCommunicationServiceType:

$ref: '#/components/schemas/MbsServiceType'

QmcAreaScope:

description: >

This IE contains the area in Cells or Tracking Areas where the QMC data collection

shall take place.

type: object

properties:

nrCellIdList:

type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

tacList:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

taiList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

plmnList:

type: array

items:

$ref: '#/components/schemas/PlmnId'

minItems: 1

QoeTarget:

description: >

This parameter specifies the target object (individual UE) for the QMC in case of

signalling based QMC. It shall be able to carry an IMSI or a SUPI.

type: object

properties:

supi:

$ref: '#/components/schemas/Supi'

imsi:

$ref: '#/components/schemas/Imsi'

UeLevelMeasurementsConfiguration:

description: 5GC UE Level Measurements configuration.

type: object

required:

- jobType

- ueLevelMeasurementsList

properties:

jobType:

$ref: '#/components/schemas/JobType'

ueLevelMeasurementsList:

type: array

items:

$ref: '#/components/schemas/MeasurementType'

minItems: 1

granularityPeriod:

$ref: '#/components/schemas/DurationSec'

BluetoothMeasurement:

description: contains the bluetooth measurements to be collected for UE.

type: object

properties:

measurementNameList:

type: array

items:

$ref: '#/components/schemas/MeasurementName'

minItems: 1

maxItems: 4

bluetoothRssi:

$ref: '#/components/schemas/BluetoothRssi'

WlanMeasurement:

description: contains the WLAN measurements to be collected for UE.

type: object

properties:

measurementNameList:

type: array

items:

$ref: '#/components/schemas/MeasurementName'

minItems: 1

maxItems: 4

wlanRssi:

$ref: '#/components/schemas/WlanRssi'

wlanRtt:

$ref: '#/components/schemas/WlanRtt'

MeasurementName:

description: contains the measurement name for measurement collection.

type: object

properties:

bluetoothName:

type: string

wlanName:

type: string

SliceScopePerPlmn:

description: contains the list of network slices for MDT per PLMN.

type: object

required:

- plmn

- sliceScope

properties:

plmn:

$ref: '#/components/schemas/PlmnId'

sliceScope:

type: array

items:

$ref: '#/components/schemas/Snssai'

minItems: 1

maxItems: 1024

# Data Types related to 5G ODB as defined in clause 5.7

#

# SIMPLE DATA TYPES

#

#

#

# Enumerations

#

RoamingOdb:

anyOf:

- type: string

enum:

- OUTSIDE_HOME_PLMN

- OUTSIDE_HOME_PLMN_COUNTRY

- type: string

description: >

The enumeration RoamingOdb defines the Barring of Roaming as. See 3GPP TS 23.015 for further

description. It shall comply with the provisions defined in table 5.7.3.1-1.

OdbPacketServices:

anyOf:

- anyOf:

- type: string

enum:

- ALL_PACKET_SERVICES

- ROAMER_ACCESS_HPLMN_AP

- ROAMER_ACCESS_VPLMN_AP

- type: string

- $ref: '#/components/schemas/NullValue'

description: >

The enumeration OdbPacketServices defines the Barring of Packet Oriented Services.

See 3GPP TS 23.015 for further description. It shall comply with the provisions defined

in table 5.7.3.2-1

#

# STRUCTURED DATA TYPES

#

OdbData:

description: Contains information regarding operater  determined  barring.

type: object

properties:

roamingOdb:

$ref: '#/components/schemas/RoamingOdb'

#

# Data Types related to Charging as defined in clause 5.8

#

#

# SIMPLE DATA TYPES

#

#

ChargingId:

deprecated: true

type: integer

minimum: 0

maximum: 4294967295 #(2^32)-1

description: >

Integer where the allowed values correspond to the value range of an unsigned 32-bit

integer.

SmfChargingId:

description: String based Charging ID

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))\.smf-([0-9a-f]{8}-[0-9a-f]{4}-[0-9a-f]{4}-[0-9a-f]{4}-[0-9a-f]{12})$'

ApplicationChargingId:

type: string

description: >

Application provided charging identifier allowing correlation of charging information.

RatingGroup:

$ref: '#/components/schemas/Uint32'

ServiceId:

$ref: '#/components/schemas/Uint32'

#

# Enumerations

#

#

# STRUCTURED DATA TYPES

#

SecondaryRatUsageReport:

description: Secondary RAT Usage Report to report usage data for a secondary RAT for QoS flows.

type: object

properties:

secondaryRatType:

$ref: '#/components/schemas/RatType'

qosFlowsUsageData:

type: array

items:

$ref: '#/components/schemas/QosFlowUsageReport'

minItems: 1

required:

- secondaryRatType

- qosFlowsUsageData

QosFlowUsageReport:

description: Contains QoS flows usage data information.

type: object

properties:

qfi:

$ref: '#/components/schemas/Qfi'

startTimeStamp:

$ref: '#/components/schemas/DateTime'

endTimeStamp:

$ref: '#/components/schemas/DateTime'

downlinkVolume:

$ref: '#/components/schemas/Int64'

uplinkVolume:

$ref: '#/components/schemas/Int64'

required:

- qfi

- startTimeStamp

- endTimeStamp

- downlinkVolume

- uplinkVolume

SecondaryRatUsageInfo:

description: >

Secondary RAT Usage Information to report usage data for a secondary RAT for QoS flows

and/or the whole PDU session.

type: object

properties:

secondaryRatType:

$ref: '#/components/schemas/RatType'

qosFlowsUsageData:

type: array

items:

$ref: '#/components/schemas/QosFlowUsageReport'

minItems: 1

pduSessionUsageData:

type: array

items:

$ref: '#/components/schemas/VolumeTimedReport'

minItems: 1

required:

- secondaryRatType

VolumeTimedReport:

description: Contains Usage data information.

type: object

properties:

startTimeStamp:

$ref: '#/components/schemas/DateTime'

endTimeStamp:

$ref: '#/components/schemas/DateTime'

downlinkVolume:

$ref: '#/components/schemas/Int64'

uplinkVolume:

$ref: '#/components/schemas/Int64'

required:

- startTimeStamp

- endTimeStamp

- downlinkVolume

- uplinkVolume

# Data Types related to MBS as defined in clause 5.9

#

#

# SIMPLE DATA TYPES

#

#

AreaSessionId:

$ref: '#/components/schemas/Uint16'

AreaSessionPolicyId:

$ref: '#/components/schemas/Uint16'

MbsFsaId:

description: MBS Frequency Selection Area Identifier

type: string

pattern: '^[A-Fa-f0-9]{6}$'

IntendedServiceArea:

$ref: '#/components/schemas/Bytes'

#

# Enumerations

#

#

MbsServiceType:

description: Indicates the MBS service type of an MBS session

anyOf:

- type: string

enum:

- MULTICAST

- BROADCAST

- type: string

MbsSessionActivityStatus:

description: Indicates the MBS session's activity status

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

MbsSessionEventType:

description: MBS Session Event Type

anyOf:

- type: string

enum:

- MBS_REL_TMGI_EXPIRY

- BROADCAST_DELIVERY_STATUS

- INGRESS_TUNNEL_ADD_CHANGE

- type: string

BroadcastDeliveryStatus:

description: Broadcast MBS Session's Delivery Status

anyOf:

- type: string

enum:

- STARTED

- TERMINATED

- type: string

NrRedCapUeInfo:

description: >

Indicates whether the broadcast MBS session is intended only for NR (e)RedCap UEs,

only for UEs that are neither NR RedCap UEs nor NR RedCap UEs, or for any kind of UE.

anyOf:

- type: string

enum:

- NR_REDCAP_UE_ONLY # NR RedCap UEs and NR eRedCap UEs

- BOTH_NR_REDCAP_UE_AND_NON_REDCAP_UE # any kind of UEs

- NON_REDCAP_UE_ONLY # neither NR RedCap UEs nor NR eRedCap UEs

- type: string

#

# STRUCTURED DATA TYPES

#

MbsSessionId:

description: MBS Session Identifier

type: object

properties:

tmgi:

$ref: '#/components/schemas/Tmgi'

ssm:

$ref: '#/components/schemas/Ssm'

nid:

$ref: '#/components/schemas/Nid'

anyOf:

- required: [ tmgi ]

- required: [ ssm ]

Tmgi:

description: Temporary Mobile Group Identity

type: object

properties:

mbsServiceId:

type: string

pattern: '^[A-Fa-f0-9]{6}$'

description: MBS Service ID

plmnId:

$ref: '#/components/schemas/PlmnId'

required:

- mbsServiceId

- plmnId

Ssm:

description: Source specific IP multicast address

type: object

properties:

sourceIpAddr:

$ref: '#/components/schemas/IpAddr'

destIpAddr:

$ref: '#/components/schemas/IpAddr'

required:

- sourceIpAddr

- destIpAddr

MbsServiceArea:

description: MBS Service Area

type: object

properties:

ncgiList:

type: array

items:

$ref: '#/components/schemas/NcgiTai'

minItems: 1

description: List of NR cell Ids

taiList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

description: List of tracking area Ids

intendedServAreaList:

type: array

items:

$ref: '#/components/schemas/IntendedServiceArea'

minItems: 1

description: NR NTN Intended Service Area

anyOf:

- required: [ ncgiList ]

- required: [ taiList ]

NcgiTai:

description: List of NR cell ids, with their pertaining TAIs

type: object

properties:

tai:

$ref: '#/components/schemas/Tai'

cellList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 1

description: List of List of NR cell ids

required:

- tai

- cellList

MbsSession:

description: Individual MBS session

type: object

properties:

mbsSessionId:

$ref: '#/components/schemas/MbsSessionId'

tmgiAllocReq:

type: boolean

default: false

writeOnly: true

tmgi:

allOf:

- $ref: '#/components/schemas/Tmgi'

readOnly: true

expirationTime:

allOf:

- $ref: '#/components/schemas/DateTime'

readOnly: true

serviceType:

allOf:

- $ref: '#/components/schemas/MbsServiceType'

writeOnly: true

locationDependent:

type: boolean

default: false

areaSessionId:

allOf:

- $ref: '#/components/schemas/AreaSessionId'

readOnly: true

ingressTunAddrReq:

type: boolean

default: false

writeOnly: true

ingressTunAddr:

type: array

items:

$ref: '#/components/schemas/TunnelAddress'

minItems: 1

readOnly: true

ssm:

allOf:

- $ref: '#/components/schemas/Ssm'

writeOnly: true

mbsServiceArea:

allOf:

- $ref: '#/components/schemas/MbsServiceArea'

writeOnly: true

extMbsServiceArea:

allOf:

- $ref: '#/components/schemas/ExternalMbsServiceArea'

writeOnly: true

redMbsServArea:

allOf:

- $ref: '#/components/schemas/MbsServiceArea'

readOnly: true

extRedMbsServArea:

allOf:

- $ref: '#/components/schemas/ExternalMbsServiceArea'

readOnly: true

dnn:

allOf:

- $ref: '#/components/schemas/Dnn'

writeOnly: true

snssai:

allOf:

- $ref: '#/components/schemas/Snssai'

writeOnly: true

activationTime:

deprecated: true

format: date-time

type: string

startTime:

$ref: '#/components/schemas/DateTime'

terminationTime:

$ref: '#/components/schemas/DateTime'

mbsServInfo:

$ref: '#/components/schemas/MbsServiceInfo'

mbsSessionSubsc:

$ref: '#/components/schemas/MbsSessionSubscription'

activityStatus:

$ref: '#/components/schemas/MbsSessionActivityStatus'

anyUeInd:

type: boolean

default: false

writeOnly: true

mbsFsaIdList:

type: array

items:

$ref: '#/components/schemas/MbsFsaId'

minItems: 1

associatedSessionId:

$ref: '#/components/schemas/AssociatedSessionId'

nrRedCapUeInfo:

$ref: '#/components/schemas/NrRedCapUeInfo'

required:

- serviceType

anyOf:

- required: [ mbsSessionId ]

- required: [ tmgiAllocReq ]

not:

required: [redMbsServArea, extRedMbsServArea]

MbsSessionSubscription:

description: MBS session subscription

type: object

properties:

mbsSessionId:

$ref: '#/components/schemas/MbsSessionId'

areaSessionId:

$ref: '#/components/schemas/AreaSessionId'

eventList:

type: array

items:

$ref: '#/components/schemas/MbsSessionEvent'

minItems: 1

notifyUri:

$ref: '#/components/schemas/Uri'

notifyCorrelationId:

type: string

expiryTime:

$ref: '#/components/schemas/DateTime'

nfcInstanceId:

$ref: '#/components/schemas/NfInstanceId'

mbsSessionSubscUri:

allOf:

- $ref: '#/components/schemas/Uri'

readOnly: true

required:

- eventList

- notifyUri

MbsSessionEventReportList:

description: MBS session event report list

type: object

properties:

eventReportList:

type: array

items:

$ref: '#/components/schemas/MbsSessionEventReport'

minItems: 1

notifyCorrelationId:

type: string

required:

- eventReportList

MbsSessionEvent:

description: MBS session event

type: object

properties:

eventType:

$ref: '#/components/schemas/MbsSessionEventType'

required:

- eventType

MbsSessionEventReport:

description: MBS session event report

type: object

properties:

eventType:

$ref: '#/components/schemas/MbsSessionEventType'

timeStamp:

$ref: '#/components/schemas/DateTime'

ingressTunAddrInfo:

$ref: '#/components/schemas/IngressTunAddrInfo'

broadcastDelStatus:

$ref: '#/components/schemas/BroadcastDeliveryStatus'

required:

- eventType

ExternalMbsServiceArea:

description: List of geographic area or list of civic address info for MBS Service Area

type: object

properties:

geographicAreaList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

civicAddressList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

oneOf:

- required: [ geographicAreaList ]

- required: [ civicAddressList ]

MbsSecurityContext:

description: MBS security context consisting of MSK/MTK(s) and associated IDs

type: object

properties:

keyList:

description: >

A map (list of key-value pairs) where a (unique) valid JSON string serves

as key of MbsSecurityContext

type: object

additionalProperties:

$ref: '#/components/schemas/MbsKeyInfo'

minProperties: 1

required:

- keyList

MbsKeyInfo:

description: MBS Security Key Data Structure

type: object

properties:

keyDomainId:

$ref: '#/components/schemas/Bytes'

mskId:

$ref: '#/components/schemas/Bytes'

msk:

$ref: '#/components/schemas/Bytes'

mskLifetime:

$ref: '#/components/schemas/DateTime'

mtkId:

$ref: '#/components/schemas/Bytes'

mtk:

$ref: '#/components/schemas/Bytes'

required:

- keyDomainId

- mskId

IngressTunAddrInfo:

description: Ingress Tunnel Address Information

type: object

properties:

ingressTunAddr:

type: array

items:

$ref: '#/components/schemas/TunnelAddress'

minItems: 1

required:

- ingressTunAddr

MbsServiceAreaInfo:

description: MBS Service Area Information for location dependent MBS session

type: object

properties:

areaSessionId:

$ref: '#/components/schemas/AreaSessionId'

mbsServiceArea:

$ref: '#/components/schemas/MbsServiceArea'

required:

- areaSessionId

- mbsServiceArea

MbsServiceInfo:

description: Represent MBS Service Information.

type: object

properties:

mbsMediaComps:

description: >

The key of the map is the "mbsMedCompNum" attribute of the corresponding MbsMediaCompRm

data structure provided as a map entry.

type: object

additionalProperties:

$ref: '#/components/schemas/MbsMediaCompRm'

minProperties: 1

mbsSdfResPrio:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ReservPriority'

afAppId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId'

mbsSessionAmbr:

$ref: '#/components/schemas/BitRate'

required:

- mbsMediaComps

MbsMediaComp:

description: Represents an MBS Media Component.

type: object

properties:

mbsMedCompNum:

type: integer

mbsFlowDescs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

mbsSdfResPrio:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/ReservPriority'

mbsMediaInfo:

$ref: '#/components/schemas/MbsMediaInfo'

qosRef:

type: string

mbsQoSReq:

$ref: '#/components/schemas/MbsQoSReq'

required:

- mbsMedCompNum

MbsMediaCompRm:

description: >

This data type is defined in the same way as the MbsMediaComp data type, but with the

OpenAPI nullable property set to true.

anyOf:

- $ref: '#/components/schemas/MbsMediaComp'

- $ref: '#/components/schemas/NullValue'

MbsQoSReq:

description: Represent MBS QoS requirements.

type: object

properties:

5qi:

$ref: '#/components/schemas/5Qi'

guarBitRate:

$ref: '#/components/schemas/BitRate'

maxBitRate:

$ref: '#/components/schemas/BitRate'

averWindow:

$ref: '#/components/schemas/AverWindow'

reqMbsArp:

$ref: '#/components/schemas/Arp'

required:

- 5qi

MbsMediaInfo:

description: Represent MBS Media Information.

type: object

properties:

mbsMedType:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/MediaType'

maxReqMbsBwDl:

$ref: '#/components/schemas/BitRate'

minReqMbsBwDl:

$ref: '#/components/schemas/BitRate'

codecs:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/CodecData'

minItems: 1

maxItems: 2

AssociatedSessionId:

description: an associated Session Id used in MOCN

anyOf:

- $ref: '#/components/schemas/Ssm'

- type: string

# Data Types related to Time Synchronization as defined in clause 5.10

#

#

# SIMPLE DATA TYPES

#

#

#

# Enumerations

#

#

SynchronizationState:

description: Indicates the Synchronization State.

anyOf:

- type: string

enum:

- LOCKED

- HOLDOVER

- FREERUN

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

TimeSource:

description: Indicates the Time Source.

anyOf:

- type: string

enum:

- SYNC_E

- PTP

- GNSS

- ATOMIC_CLOCK

- TERRESTRIAL_RADIO

- SERIAL_TIME_CODE

- NTP

- HAND_SET

- OTHER

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

ClockQualityDetailLevel:

description: Indicates the Clock Quality Detail Level.

anyOf:

- type: string

enum:

- CLOCK_QUALITY_METRICS

- ACCEPT_INDICATION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

ClockQualityDetailLevelRm:

description: >

This data type is defined in the same way as the 'ClockQualityDetailLevel' data type,

but with the OpenAPI 'nullable: true' property.

anyOf:

- $ref: '#/components/schemas/ClockQualityDetailLevel'

- $ref: '#/components/schemas/NullValue'

#

# STRUCTURED DATA TYPES

#

ClockQualityAcceptanceCriterion:

description: Contains a Clock Quality Acceptance Criterion.

type: object

properties:

synchronizationState:

type: array

items:

$ref: '#/components/schemas/SynchronizationState'

minItems: 1

clockQuality:

$ref: '#/components/schemas/ClockQuality'

parentTimeSource:

type: array

items:

$ref: '#/components/schemas/TimeSource'

minItems: 1

ClockQualityAcceptanceCriterionRm:

description: Contains a Clock Quality Acceptance Criterion.

type: object

nullable: true

properties:

synchronizationState:

type: array

nullable: true

items:

$ref: '#/components/schemas/SynchronizationState'

minItems: 1

clockQuality:

$ref: '#/components/schemas/ClockQualityRm'

parentTimeSource:

type: array

nullable: true

items:

$ref: '#/components/schemas/TimeSource'

minItems: 1

ClockQuality:

description: Contains Clock Quality.

type: object

properties:

traceabilityToGnss:

type: boolean

traceabilityToUtc:

type: boolean

frequencyStability:

$ref: '#/components/schemas/Uint16'

clockAccuracyIndex:

type: string

pattern: '^[A-Fa-f0-9]{2}$'

clockAccuracyValue:

type: integer

minimum: 1

maximum: 40000000

ClockQualityRm:

description: "ClockQuality with 'nullable: true' property"

type: object

nullable: true

properties:

traceabilityToGnss:

type: boolean

traceabilityToUtc:

type: boolean

frequencyStability:

$ref: '#/components/schemas/Uint16Rm'

clockAccuracyIndex:

type: string

nullable: true

pattern: '^[A-Fa-f0-9]{2}$'

clockAccuracyValue:

type: integer

nullable: true

minimum: 1

maximum: 40000000

# Data Types related to IMS SBA as defined in clause 5.11

#

#

# SIMPLE DATA TYPES

#

#

SessionId:

description: IMS Session Identifier

type: string

Fingerprint:

description: The certificate fingerprint for the DTLS association.

type: string

pattern: '^(SHA-1|SHA-224|SHA-256|SHA-384|SHA-512|MD5|MD2)\s[0-9A-F]{2}(:[0-9A-F]{2})+$'

MediaId:

description: IMS Media Flow Identifier

type: string

MaxMessageSize:

description: Maximum SCTP user message size

type: integer

maximum: 64

default: 64

TlsId:

description: The TLS ID for the media stream.

type: string

pattern: '^[A-Za-z0-9+/_-]{20,255}$'

ImsTranspInfo:

description: IMS object to carry transparent information for IMS Exposure services.

type: object

#

# Enumerations

#

#

MediaResourceType:

description: Indicates the Media Resource type

anyOf:

- type: string

enum:

- DC

- AUDIO

- VIDEO

- type: string

MediaProxy:

description: Media Proxy Configuration applicable to the media flow

anyOf:

- type: string

enum:

- HTTP_PROXY

- UDP_PROXY

- DC_APPLICATION_PROXY

- type: string

SecuritySetup:

description: security setup of the DTLS connection

anyOf:

- type: string

enum:

- ACTIVE

- PASSIVE

- ACTPASS

- type: string

BdcUsedBy:

description: The party uses the bootstrap data channel in the media description

anyOf:

- type: string

enum:

- SENDER

- RECEIVER

- type: string

AdcEndpointType:

description: The remote endpoint type of the application data channel

anyOf:

- type: string

enum:

- UE

- SERVER

- type: string

ImsEvent:

description: The IMS events

anyOf:

- type: string

enum:

- ADC_ESTABLISHMENT

- ADC_RELEASE

- BDC_ESTABLISHMENT

- BDC_RELEASE

- ADC_IWK_ESTABLISHMENT

- type: string

AdcStatus:

description: ADC event status.

anyOf:

- type: string

enum:

- ADC_ESTABLISHED

- ADC_RELEASED

- ADC_IWK_ESTABLISHED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

BdcStatus:

description: BDC event status.

anyOf:

- type: string

enum:

- BDC_ESTABLISHED

- BDC_RELEASED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

ImsReportMode:

description: Ims event report mode.

anyOf:

- type: string

enum:

- ON_EVENT_DETECTION

- PERIODIC

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

RenderingMode:

description: The rendering mode for avatar media

anyOf:

- type: string

enum:

- UE_CENTRIC

- NET_CENTRIC_MF

- NET_CENTRIC_DCAS

- type: string

#

# STRUCTURED DATA TYPES

#

DcEndpoint:

description: Endpoint for Data Channel

type: object

properties:

sctpPort:

type: integer

maximum: 65535

minimum: 0

description: Local or remote port for Data Channel

fingerprint:

deprecated: true

allOf:

- $ref: '#/components/schemas/Fingerprint'

fingerprints:

type: array

items:

$ref: '#/components/schemas/Fingerprint'

minItems: 1

tlsId:

$ref: '#/components/schemas/TlsId'

securitySetup:

$ref: '#/components/schemas/SecuritySetup'

DcStream:

description: Data Channel mapping and configuration information

type: object

not:

required: [maxRetry, maxTime]

properties:

streamId:

type: integer

maximum: 65535

default: 0

description: Stream identifier for Data Channel

subprotocol:

type: string

pattern: '^[A-Fa-f0-9]{20}$'

description: Subprotocol of the SCTP stream

order:

type: boolean

maxRetry:

type: integer

default: 0

description: maximal number of the times a message will be retransmitted

maxTime:

type: integer

default: 0

description: >

maximal lifetime in milliseconds after which a message will no longer be

transmitted or retransmitted

priority:

type: integer

default: 256

description: priority of data channel relative to other data channels

ReplaceHttpUrl:

description: replacement HTTP URL per stream

type: object

properties:

replaceHttpUrl:

$ref: '#/components/schemas/Uri'

streamId:

type: integer

maximum: 65535

default: 0

description: Stream identifier for Data Channel

Endpoint:

description: Represents the IP endpoint.

type: object

required:

- ip

- transport

- portNumber

properties:

ip:

$ref: '#/components/schemas/IpAddr'

transport:

$ref: '#/components/schemas/TransportProtocol'

portNumber:

$ref: '#/components/schemas/Uinteger'

AppBindingInfo:

description: Represents the application binding information.

type: object

required:

- applicationId

properties:

applicationId:

type: string

description: application binding information of the Data Channel.

appDcInfo:

deprecated: true

allOf:

- $ref: '#/components/schemas/AppDcInfo'

appDcInfoList:

type: array

items:

$ref: '#/components/schemas/AppDcInfo'

minItems: 1

AppDcInfo:

description: Represents the application data channel is intened towards to a server or the remote UE.

type: object

required:

- streamId

properties:

streamId:

type: integer

adcEndpointType:

$ref: '#/components/schemas/AdcEndpointType'

MdcEndpoint:

description: Endpoint for MDC1 and MDC2 interface

type: object

required:

- ip

- portNumber

properties:

ip:

$ref: '#/components/schemas/IpAddr'

portNumber:

$ref: '#/components/schemas/Uinteger'

sctpPort:

type: integer

maximum: 65535

minimum: 0

description: Port number for SCTP connection over DTLS

fingerprint:

deprecated: true

allOf:

- $ref: '#/components/schemas/Fingerprint'

fingerprints:

type: array

items:

$ref: '#/components/schemas/Fingerprint'

minItems: 1

tlsId:

$ref: '#/components/schemas/TlsId'

securitySetup:

$ref: '#/components/schemas/SecuritySetup'

ImsReportingOptions:

description: Ims event reporting options.

type: object

properties:

reportMode:

$ref: '#/components/schemas/ImsReportMode'

oneTimeReport:

type: boolean

oneTimeNotification:

type: boolean

maxNumOfReports:

type: integer

reportPeriod:

$ref: '#/components/schemas/DurationSec'

expiry:

$ref: '#/components/schemas/DateTime'

ImsEventConfiguration:

description: Ims event configuration information.

type: object

required:

- imsEvent

properties:

imsEvent:

$ref: '#/components/schemas/ImsEvent'

eventInfo:

$ref: '#/components/schemas/EventInformation'

reqCurrentStatus:

type: boolean

imsEventFilters:

type: array

items:

$ref: '#/components/schemas/ImsEventFilter'

minItems: 1

EventInformation:

description: Ims event information.

type: object

properties:

imsTranspEventConfig:

$ref: '#/components/schemas/ImsTranspInfo'

ImsEventFilter:

description: Conditions to match specific Ims event.

type: object

properties:

callingId:

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|tel\:\+[0-9]{5,15})$'

calledId:

type: string

pattern: '^(sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}|tel\:\+[0-9]{5,15})$'

appBindingInfo:

$ref: '#/components/schemas/AppBindingInfo'

sessionId:

$ref: '#/components/schemas/SessionId'

imsTranspFilter:

$ref: '#/components/schemas/ImsTranspInfo'

ImsEventReport:

description: Ims event report.

type: object

required:

- imsEvent

properties:

imsEvent:

$ref: '#/components/schemas/ImsEvent'

eventReport:

$ref: '#/components/schemas/ImsEventReportInfo'

sessionId:

$ref: '#/components/schemas/SessionId'

ImsEventReportInfo:

description: Ims event report specific information.

type: object

properties:

adcReport:

$ref: '#/components/schemas/AdcReport'

bdcReport:

$ref: '#/components/schemas/BdcReport'

imsTranspEventReport:

$ref: '#/components/schemas/ImsTranspInfo'

AdcReport:

description: Application data channel report.

type: object

required:

- adcStatus

properties:

adcStatus:

$ref: '#/components/schemas/AdcStatus'

BdcReport:

description: Bootstrap data channel report.

type: object

required:

- bdcStatus

properties:

bdcStatus:

$ref: '#/components/schemas/BdcStatus'

RcdProperties:

description: Contains the RCD related parameters.

type: object

properties:

rcdSrvAddr:

$ref: '#/components/schemas/ServerAddressingInfo'

rcdUrl:

$ref: '#/components/schemas/Uri'

rcdInfo:

$ref: '#/components/schemas/RcdInformation'

RcdInformation:

description: >

Contains a collection of RCD properties as specified in IETF RFC 9796

and represented as a jCard JSON object.

type: object

required:

- identifProps

properties:

identifProps:

$ref: '#/components/schemas/IdentificationProperties'

delvAddrProps:

$ref: '#/components/schemas/DeliveringAddrProperties'

commsProps:

$ref: '#/components/schemas/CommunicationsProperties'

geogrProps:

$ref: '#/components/schemas/GeographicalProperties'

organProps:

$ref: '#/components/schemas/OrganizationalProperties'

explanProps:

$ref: '#/components/schemas/ExplanatoryProperties'

IdentificationProperties:

description: Represents the identity information of the entity associated with the jCard.

type: object

required:

- fns

properties:

fns:

description: >

Represent the "fn" property which provides a formatted text corresponding to the

name of the object the jCard represents as defined in IETF RFC 6350, section 6.2.1.

type: array

items:

type: string

minItems: 1

nProp:

description: >

Represent the "n" property which provides the components of the name of the object

the jCard represents as defined as defined in IETF RFC 6350, section 6.2.2.

type: string

nickNames:

description: >

Represent the "nickname" property which provides the text corresponding to the

nickname of the object the jCard represents as defined in IETF RFC 6350, section 6.2.3.

type: array

items:

type: string

minItems: 1

photos:

description: >

Represent the "photo" property which provides image or photograph information that

annotates some aspect of the object the jCard represents as defined in

IETF RFC 6350, section 6.2.4.

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

DeliveringAddrProperties:

description: >

Represents the information related to the delivery address of the entity associated with

the jCard.

type: object

properties:

adrs:

description: >

Represent the "adr" property which provides the delivery address of the object the

jCard represents as defined in IETF RFC 6350, section 6.3.1.

type: array

items:

type: string

minItems: 1

CommunicationsProperties:

description: Indicates how to communicate with the entity associated with the jCard.

type: object

properties:

tels:

description: >

Represent the "tel" property which provides the telephone number for the object the

jCard represents as defined in IETF RFC 6350, section 6.4.1.

type: array

items:

type: string

minItems: 1

emails:

description: >

Represent the "email" property which provides the electronic mail address of the

object the jCard represents as defined in IETF RFC 6350, section 6.4.2.

type: array

items:

type: string

minItems: 1

langs:

description: >

Represent the "lang" property which provides the language(s) that may be used for

communicating with the object the jCard represents as defined in IETF RFC 6350,

section 6.4.4.

type: array

items:

type: string

minItems: 1

GeographicalProperties:

description: >

Represents the geographical information associated with the entity associated with

the jCard.

type: object

properties:

tzs:

description: >

Represent the "tz" property which provides the time zone of the object the jCard

represents as defined in IETF RFC 6350, section 6.5.1.

type: array

items:

type: string

minItems: 1

geos:

description: >

Represent the "geo" property which provides the global positioning of the object the

jCard represents as defined in IETF RFC 6350, section 6.5.2.

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

OrganizationalProperties:

description: >

Represents the information associated with characteristics of the organization or

organizational units of the entity associated with the jCard.

type: object

properties:

titles:

description: >

Represent the "title" property which has the intent of providing the position or job

of the object the jCard represents as defined in IETF RFC 6350, section 6.6.1.

type: array

items:

type: string

minItems: 1

roles:

description: >

Represent the "role" property which has the intent of providing the position or job

of the object the jCard represents as defined in IETF RFC 6350, section 6.6.2.

type: array

items:

type: string

minItems: 1

logos:

description: >

Represent the "logo" property which has the intent of specifying a graphic image of

a logo associated with the object the jCard represents as defined in IETF RFC 6350,

section 6.6.3.

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

orgs:

description: >

Represent the "org" property which has the intent of specifying the organizational

name and units of the object the jCard represents as defined in IETF RFC 6350,

section 6.6.4.

type: array

items:

type: string

minItems: 1

ExplanatoryProperties:

description: >

Indicates an additional information such as an application category information or notes

that are specific to the entity associated with the jCard.

type: object

required:

- version

properties:

categories:

description: >

Represent the "categories" property which specifies application category information

about the object the jCard represents as defined in IETF RFC 6350, section 6.7.1.

type: array

items:

type: string

minItems: 1

notes:

description: >

Represent the "note" property which specifies supplemental information or a comment

about the object the jCard represents as defined in IETF RFC 6350, section 6.7.2.

type: array

items:

type: string

minItems: 1

sounds:

description: >

Represent the "sound" property which specifies digital sound content information

that annotates some aspect of the object the jCard represents as defined in

IETF RFC 6350, section 6.7.5.

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

uid:

$ref: '#/components/schemas/Uri'

urls:

description: >

Represent the "url" property which specifies a uniform resource locator associated

with the object the jCard represents as defined in IETF RFC 6350, section 6.7.8.

type: array

items:

$ref: '#/components/schemas/Uri'

minItems: 1

version:

description: >

Represent the "version" property which specifies the version of the vCard

specification used to format this vCard as defined in IETF RFC 6350, section 6.7.9.

type: string

# Data Types related to Ambient IoT as defined in clause 5.12

#

#

# SIMPLE DATA TYPES

#

#

AiotAreaCode:

description: Ambient IoT Area code.

type: string

pattern: '^[A-Fa-f0-9]{6}$'

AiotFilteringInformation:

description: Ambient IoT Filtering Information.

$ref: '#/components/schemas/Bytes'

AiotDevPermId:

$ref: '#/components/schemas/Bytes'

description: >

String identifying the AIoT Device Permanent Identifier.

#

# STRUCTURED DATA TYPES

#

AiotArea:

description: Contains a list of AIoT Area ID(s).

type: object

properties:

areaIds:

type: array

items:

$ref: '#/components/schemas/AiotAreaId'

minItems: 1

required:

- areaIds

AiotAreaId:

description: AIoT Area ID.

type: object

properties:

plmnId:

$ref: '#/components/schemas/PlmnId'

nid:

$ref: '#/components/schemas/Nid'

aiotAreaCode:

$ref: '#/components/schemas/AiotAreaCode'

required:

- plmnId

- aiotAreaCode

#

# HTTP responses

#

responses:

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: '#/components/schemas/RedirectResponse'

headers:

Location:

description: 'The URI pointing to the resource located on the redirect target'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: >

'Identifier of target NF (service) instance towards which the request is redirected'

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: '#/components/schemas/RedirectResponse'

headers:

Location:

description: 'The URI pointing to the resource located on the redirect target'

required: true

schema:

type: string

3gpp-Sbi-Target-Nf-Id:

description: >

'Identifier of target NF (service) instance towards which the request is redirected'

schema:

type: string

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'401':

description: Unauthorized

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'405':

description: Method Not Allowed

'408':

description: Request Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'406':

description: 406 Not Acceptable

'409':

description: Conflict

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'410':

description: Gone

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'411':

description: Length Required

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'412':

description: Precondition Failed

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'413':

description: Content Too Large

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'414':

description: URI Too Long

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'415':

description: Unsupported Media Type

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'429':

description: Too Many Requests

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'501':

description: Not Implemented

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'502':

description: Bad Gateway

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'503':

description: Service Unavailable

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

'504':

description: Gateway Timeout

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetails'

default:

description: Generic Error

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-10 | CT4#80 | C4-175048 |  |  |  | Initial Draft. | 0.1.0 |
| 2017-10 | CT4#80 | C4-175400 |  |  |  | Skeleton and scope | 0.2.0 |
| 2017-12 | CT4#81 | C4-176442 |  |  |  | After CT4#81 | 0.3.0 |
| 2018-01 | CT4#82 | C4-181395 |  |  |  | After CT4#82 | 0.4.0 |
| 2018-03 | CT4#83 | C4-182440 |  |  |  | After CT4#83 | 0.5.0 |
| 2018-04 | CT4#84 | C4-183521 |  |  |  | After CT4#84 | 0.6.0 |
| 2018-05 | CT4#85 | C4-184635 |  |  |  | After CT4#85 | 0.7.0 |
| 2018-06 | CT#80 | CP-181110 |  |  |  | Presented for information and approval | 1.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Approved in CT#80 | 15.0.0 |
| 2018-09 | CT#81 | CP-182065 | 0001 |  | F | ProblemDetails | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0002 |  | F | Structure of AmfId | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0012 |  | B | DNAI change notification type | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0015 |  | F | RatType | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0017 |  | B | Definition of DNAI | 15.1.0 |
| 2018-09 | CT#81 | CP-182068 | 0008 | 1 | B | Add support for 5G Trace | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0010 | 1 | F | OpenAPI Corrections | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0013 | 1 | B | Structure of ECGI and NCGI | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0007 | 1 | F | Averaging Window | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0020 | 1 | F | sd pattern | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0021 | 1 | F | Correction of the title of clauses 5.2.4.4 _LinksValueSchema and 5.2.4.5 _ SelfLink | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0023 |  | F | NAI format in 5G System | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0031 |  | F | GroupId Definition | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0009 | 1 | F | Removal of systematic references to the "format" keyword in data type definitions | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0033 |  | F | Naming Conventions | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0027 | 1 | F | 5GMMCause and NGAP Cause | 15.1.0 |
| 2018-09 | CT#81 | CP-182173 | 0006 | 3 | F | BackUp AMF Info | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0035 |  | F | URI Scheme | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0024 | 2 | F | Cleanup of the specification | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0025 | 1 | F | Correction to Regular Expression Pattern of GPSI | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0005 | 4 | F | Common data types: NonDynamic5qi and Dynamic5qi | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0028 | 1 | F | Common data type used in both TS 29.505 and TS 29.519 | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0029 | 1 | B | n6 Traffic Routing Information data type | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0019 | 4 | F | DefaultQosInformation | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0034 | 1 | F | Update of N3gaLocation data type | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0016 | 3 | F | Mobility Restriction | 15.1.0 |
| 2018-09 | CT#81 | CP-182042 | 0030 | 3 | F | Adding "nullable" property to OpenAPI definitions of data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182174 | 0026 | 3 | F | Presence Reporting Area | 15.1.0 |
| 2018-09 | CT#81 | CP-182011 | 0032 | 4 | F | Adding age of location, geographic information and other missing ones in the UserLocation type | 15.1.0 |
| 2018-09 | CT#81 | CP-182183 | 0036 | 1 | B | Common data type for data change notification | 15.1.0 |
| 2018-09 | CT#81 | CP-182065 | 0037 |  | F | API version number update | 15.1.0 |
| 2018-12 | CT#82 | CP-183024 | 0040 |  | F | Application ID | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0049 |  | F | Corrections to PDU Session Id, PDU Session Type and SupportedFeatures | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0038 | 1 | F | Area definition | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0047 | 1 | F | DNN | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0044 | 1 | F | Update of missing status code 429 in TS 29.571 | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0057 | 1 | F | 29571 CR cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0045 | 2 | F | The ARP in Default QoS | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0058 | 1 | F | Snssai pattern | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0039 | 1 | F | GroupId pattern | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0059 |  | F | Adding of HTTP status code "406 Not Acceptable" | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0041 | 1 | F | VarUeId definition | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0061 |  | F | ProblemDetails for 501 | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0063 |  | F | ChangeItem alignment | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0046 | 2 | F | Regular Expression Patterns | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0048 | 3 | F | Alignments with NGAP | 15.2.0 |
| 2018-12 | CT#82 | CP-183168 | 0065 | 1 | F | Secondary RAT usage data reporting | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0060 | 1 | F | Data types associated with Subscribed and Authorized Default QoS for Default QoS Flow | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0042 | 3 | F | Alignment of pattern for data types with "nullable" property | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0062 | 1 | F | NF Group Id | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0053 | 2 | F | data type for complex query expression | 15.2.0 |
| 2018-12 | CT#82 | CP-183161 | 0064 | 2 | F | NgRanIdentifier and PresenceInfo | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0068 |  | F | Addition of HTTP status code "412 Precondition Failed" | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0051 | 3 | F | Introduction of Barring of Roaming in 5GC | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0066 | 1 | F | Service Area Restriction | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0067 | 1 | F | Charging related types | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0070 |  | F | Correction of the reference for the SupportedFeatures Data Type | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0072 | 1 | F | Update open API version | 15.2.0 |
| 2018-12 | CT#82 | CP-183024 | 0073 |  | F | ExternalDoc update | 15.2.0 |
| 2019-03 | CT#83 | CP-190029 | 0075 | 3 | F | Corrections on subscribed Priority | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0076 | 1 | F | AmfRegionId and AmfSetId | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0077 | 2 | F | Supported features | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0078 | 2 | F | Corrections on n3iwf Id | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0079 | 2 | F | Corrections on the encoding of bit string | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0081 | 2 | F | Corrections on Type RouteToLocation | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0082 | 1 | F | ODB correction | 15.3.0 |
| 2019-03 | CT#83 | CP-190029 | 0083 |  | F | 3GPP TS 29.571 API version update | 15.3.0 |
| 2019-06 | CT#84 | CP-191041 | 0077 | 3 | F | CR not implemented – Supported Features | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0084 | 1 | F | Service Area Restriction | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0087 | 1 | F | ChangeItem Indicating Complete Resource Creation or Removal | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0089 | 2 | F | Storage of OpenAPI specification files | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0090 | 1 | F | Clarificaiton on Universal Matching Pattern Schema | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0086 | 2 | F | Correct the discription of 5qi in SubscribedDefaultQos | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0097 |  | F | AreaCode | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0094 | 1 | F | Required attributes in NotifyItem | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0095 | 1 | F | Regular Expression Pattern of DiameterIdentity | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0096 | 1 | F | Secondary RAT Usage reporting at PDU session level | 15.4.0 |
| 2019-06 | CT#84 | CP-191041 | 0099 | 2 | F | Copyright Note in YAML file | 15.4.0 |
| 2019-06 | CT#84 | CP-191048 | 0100 | 1 | B | 3GPP TS 29.571 API version update | 16.0.0 |
| 2019-06 | CT#84 | CP-191050 | 0093 |  | B | Definition of MTC provider Information | 16.0.0 |
| 2019-06 | CT#84 | CP-191050 | 0098 | 1 | B | Extend value of RAT Type to add NBIOT | 16.0.0 |
| 2019-06 | CT#84 | CP-191051 | 0088 | 3 | B | Common Data Type for ATSSS Capability | 16.0.0 |
| 2019-06 | CT#84 | CP-191052 | 0085 | 1 | B | Addition of Event Reporting Information Parameters for network data analytics | 16.0.0 |
| 2019-06 | CT#84 | CP-191055 | 0091 | 2 | B | NF discovery factors | 16.0.0 |
| 2019-09 | CT#85 | CP-192194 | 0102 | 3 | B | NF Set and NF Service Set | 16.1.0 |
| 2019-09 | CT#85 | CP-192133 | 0103 |  | B | PlmnId | 16.1.0 |
| 2019-09 | CT#85 | CP-192133 | 0104 | 1 | B | Closed Access Group | 16.1.0 |
| 2019-09 | CT#85 | CP-192028 | 0113 | 2 | B | Network Identifier for SNPN | 16.1.0 |
| 2019-09 | CT#85 | CP-192211 | 0105 | 2 | B | Common Data Type for 5G SRVCC | 16.1.0 |
| 2019-09 | CT#85 | CP-192115 | 0107 | 1 | A | PRA ID encoding | 16.1.0 |
| 2019-09 | CT#85 | CP-192123 | 0108 | 1 | F | DNN Format correction | 16.1.0 |
| 2019-09 | CT#85 | CP-192123 | 0111 | 2 | B | PatchResult data type | 16.1.0 |
| 2019-09 | CT#85 | CP-192120 | 0116 | 3 | F | Extended PDU Session ID used in Core Network | 16.1.0 |
| 2019-09 | CT#85 | CP-192195 | 0121 | 2 | B | Small Data Rate Control Status | 16.1.0 |
| 2019-09 | CT#85 | CP-192130 | 0122 | 2 | B | Updates for 5WWC with HFC wireline access | 16.1.0 |
| 2019-09 | CT#85 | CP-192120 | 0124 |  | F | 3GPP TS 29.571 API version update | 16.1.0 |
| 2019-09 | CT#85 | CP-192210 | 0125 |  | F | Correction and alignment of Sampling Ratio | 16.1.0 |
| 2019-12 | CT#86 | CP-193032 | 0130 |  | A | N3IWF ID encoding | 16.2.0 |
| 2019-12 | CT#86 | CP-193032 | 0138 |  | A | Correction to GNbId | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0126 | 1 | B | Format of NF (Service) Set ID | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0142 | 1 | F | MAC Address as PEI format | 16.2.0 |
| 2019-12 | CT#86 | CP-193050 | 0143 | 1 | F | Alternative 1 for global uniqueness of universally managed NID - simple data types correction | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0135 | 2 | B | Definition of TNAP ID | 16.2.0 |
| 2019-12 | CT#86 | CP-193063 | 0131 | 1 | B | HAL-forms data type | 16.2.0 |
| 2019-12 | CT#86 | CP-193057 | 0127 | 3 | B | Delegated Discovery Parameters Conveyance in HTTP/2 headers | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0149 |  | B | LTE-M RAT Type | 16.2.0 |
| 2019-12 | CT#86 | CP-193062 | 0148 | 1 | B | Common Data Type for RACS | 16.2.0 |
| 2019-12 | CT#86 | CP-193063 | 0161 | 1 | B | DNN Network Identifier and Operator Identifier | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0114 | 5 | B | Increasing the maximum MDBV value | 16.2.0 |
| 2019-12 | CT#86 | CP-193031 | 0160 | 1 | A | Wildcard DNN | 16.2.0 |
| 2019-12 | CT#86 | CP-193032 | 0163 | 1 | A | Correction to charging identifiers | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0156 | 2 | F | TAI and CGI in UserLocation | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0158 | 2 | B | Definition of HFC node Id and User Location information for HFC | 16.2.0 |
| 2019-12 | CT#86 | CP-193225 | 0159 | 3 | B | Wireline Service Area Restrictions | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0144 | 1 | B | Defining new data type for the Rate Control | 16.2.0 |
| 2019-12 | CT#86 | CP-193049 | 0153 | 1 | B | Expected UE Behaviour parameters | 16.2.0 |
| 2019-12 | CT#86 | CP-193036 | 0150 | 2 | B | Adding support for NR and E-UTRA accessing through unlicensed bands | 16.2.0 |
| 2019-12 | CT#86 | CP-193063 | 0152 | 3 | B | PRA for LTE UE | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0154 | 3 | B | ACS information | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0136 | 4 | B | QoS for wireline access network | 16.2.0 |
| 2019-12 | CT#86 | CP-193046 | 0165 |  | B | IPv4AddrMask | 16.2.0 |
| 2019-12 | CT#86 | CP-193063 | 0145 | 1 | B | InvalidParam Data Type | 16.2.0 |
| 2019-12 | CT#86 | CP-193044 | 0167 |  | F | API version and External doc update | 16.2.0 |
| 2020-03 | CT#87E | CP-200032 | 0168 | 1 | C | NID | 16.3.0 |
| 2020-03 | CT#87E | CP-200020 | 0170 | 1 | F | Enumerations and "nullable" keyword | 16.3.0 |
| 2020-03 | CT#87E | CP-200032 | 0176 | 1 | F | CAG-ID size | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0172 | 2 | B | New RAT Type values for Non-3GPP accesses | 16.3.0 |
| 2020-03 | CT#87E | CP-200033 | 0180 |  | B | External Group Identifier | 16.3.0 |
| 2020-03 | CT#87E | CP-200031 | 0182 |  | B | Remove Unused MaPduCapbility Data Type | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0185 |  | B | HFC NODE ID | 16.3.0 |
| 2020-03 | CT#87E | CP-200133 | 0190 | 1 | B | CS/PS location | 16.3.0 |
| 2020-03 | CT#87E | CP-200018 | 0192 |  | B | LCS service authorization | 16.3.0 |
| 2020-03 | CT#87E | CP-200033 | 0175 | 2 | F | Status type definition | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0194 |  | B | SupiOrSuci | 16.3.0 |
| 2020-03 | CT#87E | CP-200020 | 0191 | 1 | F | Pattern of Ipv4AddrMask | 16.3.0 |
| 2020-03 | CT#87E | CP-200267 | 0183 | 3 | B | Common data types for V2X service | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0173 | 4 | B | User Location for wireliness and trusted non-3GPP accesses | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0174 | 3 | B | PEI for 5G-RG/FN-RG and for UEs not supporting any 3GPP access technologies | 16.3.0 |
| 2020-03 | CT#87E | CP-200035 | 0189 | 1 | B | SUPI definition for 5G-RG and FN-RG | 16.3.0 |
| 2020-03 | CT#87E | CP-200021 | 0188 | 1 | B | Remove the common data type Software Version Number | 16.3.0 |
| 2020-03 | CT#87E | CP-200181 | 0179 | 4 | B | Downlink data delivery status | 16.3.0 |
| 2020-03 | CT#87E | CP-200033 | 0181 | 2 | B | MO Exception Data Counter | 16.3.0 |
| 2020-03 | CT#87E | CP-200052 | 0195 |  | F | API version and External doc update | 16.3.0 |
| 2020-06 | CT#88E | CP-201030 | 0198 |  | F | HTTP redirection for indirect communication | 16.4.0 |
| 2020-06 | CT#88E | CP-201066 | 0201 | 1 | F | Clarification of NF Instance ID encoding | 16.4.0 |
| 2020-06 | CT#88E | CP-201067 | 0196 | 1 | B | MDT Configuration data for 5G g | 16.4.0 |
| 2020-06 | CT#88E | CP-201047 | 0202 | 1 | B | Authentication and Authorization status | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0203 | 1 | F | User Location of TWAP ID or TNAP ID | 16.4.0 |
| 2020-06 | CT#88E | CP-201034 | 0199 | 3 | F | Slice Differentiator Ranges and Wildcard | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0197 | 1 | F | User Location for W-5GBAN | 16.4.0 |
| 2020-06 | CT#88E | CP-201066 | 0205 | 1 | F | Correction on unsigned integer types | 16.4.0 |
| 2020-06 | CT#88E | CP-201045 | 0207 | 1 | F | Nid shall be present in data types of Tai/Ncgi/GlobalRanNodeId in case of SNPN | 16.4.0 |
| 2020-06 | CT#88E | CP-201045 | 0206 | 2 | F | Identify for AMF in SNPN | 16.4.0 |
| 2020-06 | CT#88E | CP-201032 | 0208 | 1 | F | Revising the defination of LcsServiceAuth data type | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0209 | 1 | F | Extend GlobalRanNodeId to Support W-AGF and TNGF | 16.4.0 |
| 2020-06 | CT#88E | CP-201034 | 0210 | 1 | F | Nullvalue and "nullable" keyword | 16.4.0 |
| 2020-06 | CT#88E | CP-201034 | 0222 | 1 | F | Editorial corrections | 16.4.0 |
| 2020-06 | CT#88E | CP-201034 | 0223 | 1 | F | Correct the data type in Pc5QosFlowItem | 16.4.0 |
| 2020-06 | CT#88E | CP-201034 | 0212 | 1 | F | NotifyItem | 16.4.0 |
| 2020-06 | CT#88E | CP-201044 | 0214 | 3 | B | UPF Supports RTT Measurements without PMF | 16.4.0 |
| 2020-06 | CT#88E | CP-201045 | 0227 |  | F | Clarifications to TAI / ECGI / NCGI for SNPNs | 16.4.0 |
| 2020-06 | CT#88E | CP-201046 | 0225 | 1 | F | Aligning "MO Exception data" handling with stage 2 - Data types | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0218 | 1 | F | Removal of RG-TMBR | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0219 | 1 | F | Update the RAT type definition | 16.4.0 |
| 2020-06 | CT#88E | CP-201048 | 0217 | 1 | F | Reference for RgWirelineCharacteristics | 16.4.0 |
| 2020-06 | CT#88E | CP-201066 | 0220 |  | F | Storage of YAML files in ETSI Forge | 16.4.0 |
| 2020-06 | CT#88E | CP-201066 | 0221 |  | F | Binary IE Encoding | 16.4.0 |
| 2020-06 | CT#88E | CP-201066 | 0226 | 1 | F | Correcting wrong reference | 16.4.0 |
| 2020-06 | CT#88E | CP-201073 | 0228 |  | F | API version and External doc update | 16.4.0 |
| 2020-09 | CT#89E | CP-202107 | 0236 | 1 | F | Dynamic CN PDB | 16.5.0 |
| 2020-09 | CT#89E | CP-202100 | 0232 | 1 | F | Error corrections | 16.5.0 |
| 2020-09 | CT#89E | CP-202100 | 0234 | 1 | F | Additional PRA ID | 16.5.0 |
| 2020-09 | CT#89E | CP-202103 | 0233 | 1 | F | N5GC Location | 16.5.0 |
| 2020-09 | CT#89E | CP-202506 | 0231 | 1 | F | Ncgi typo correction | 16.5.0 |
| 2020-09 | CT#89E | CP-202109 | 0229 | 1 | F | Adding missing Reference to SUPI definition | 16.5.0 |
| 2020-09 | CT#89E | CP-202096 | 0237 |  | F | Rel-16 API version and External doc update | 16.5.0 |
| 2020-12 | CT#90E | CP-203035 | 0239 |  | F | Removal of the reference to ETSI forge | 16.6.0 |
| 2020-12 | CT#90E | CP-203031 | 0240 |  | F | Correction for implementation error 29.571 | 16.6.0 |
| 2020-12 | CT#90E | CP-203031 | 0243 |  | F | Incomplete references and wrong table header | 16.6.0 |
| 2020-12 | CT#90E | CP-203039 | 0245 |  | F | Alignment with TR-456 / TR-470 (BBF technical specifications) | 16.6.0 |
| 2020-12 | CT#90E | CP-203048 | 0241 | 1 | F | ssid typo in yaml | 16.6.0 |
| 2020-12 | CT#90E | CP-203031 | 0246 | 1 | F | MDT LTE Measurements | 16.6.0 |
| 2020-12 | CT#90E | CP-203068 | 0247 | 2 | F | MDT Parameters for NR | 16.6.0 |
| 2020-12 | CT#90E | CP-203036 | 0248 |  | F | Rel-16 API version and External doc update | 16.6.0 |
| 2020-12 | CT#90E | CP-203061 | 0238 | 1 | F | Clarification to IPv6Prefix type | 17.0.0 |
| 2021-03 | CT#91E | CP-210037 | 0255 |  | A | Error handling when the SCP fails to obtain an access token | 17.1.0 |
| 2021-03 | CT#91E | CP-210047 | 0254 |  | A | NF Set ID and NF Service Set ID Definition for SNPN | 17.1.0 |
| 2021-03 | CT#91E | CP-210058 | 0256 | 1 | A | Corrections on MDT parameters | 17.1.0 |
| 2021-03 | CT#91E | CP-210034 | 0257 | 1 | F | OpenAPI Reference and description field for map data types | 17.1.0 |
| 2021-03 | CT#91E | CP-210021 | 0257 | 1 | F | ProblemDetails content in responses to PATCH requests | 17.1.0 |
| 2021-03 | CT#91E | CP-210021 | 0260 |  | F | 29.571 Rel-17 API version and External doc update | 17.1.0 |
| 2021-06 | CT#92E | CP-211027 | 0265 |  | B | Non-3GPP TAI | 17.2.0 |
| 2021-06 | CT#92E | CP-211080 | 0267 |  | A | TAI in EutraLocation | 17.2.0 |
| 2021-06 | CT#92E | CP-211036 | 0272 | 1 | B | Support of Mute Reporting | 17.2.0 |
| 2021-06 | CT#92E | CP-211059 | 0273 | 1 | A | RedirectResponse data type definition | 17.2.0 |
| 2021-06 | CT#92E | CP-211040 | 0258 |  | B | Support for satellite access RAT types | 17.2.0 |
| 2021-06 | CT#92E | CP-211039 | 0268 | 2 | B | Add ProseServiceAuth | 17.2.0 |
| 2021-06 | CT#92E | CP-211036 | 0271 | 2 | B | Common Partitioning criteria added | 17.2.0 |
| 2021-06 | CT#92E | CP-211028 | 0262 | 1 | F | ChangeItem operation definition | 17.2.0 |
| 2021-06 | CT#92E | CP-211031 | 0269 | 1 | B | CS Address Information | 17.2.0 |
| 2021-06 | CT#92E | CP-211102 | 0274 | 1 | F | Remove double definition and cleanup of the OpenAPI part | 17.2.0 |
| 2021-06 | CT#92E | CP-211103 | 0278 | 1 | F | Additions of description in OpenAPI | 17.2.0 |
| 2021-06 | CT#92E | CP-211060 | 0280 |  | A | Essential Correction to GeraLocation, LAC/RAC/SAC and Cell ID data types | 17.2.0 |
| 2021-06 | CT#92E | CP-211028 | 0281 |  | B | EmptyObject definition | 17.2.0 |
| 2021-06 | CT#92E | CP-211048 | 0283 | 1 | B | Extention of userLocationInfo attribute to support GERAN/UTRAN access | 17.2.0 |
| 2021-06 | CT#92E | CP-211031 | 0284 | 1 | B | New NSAC related data types | 17.2.0 |
| 2021-06 | CT#92E | CP-211030 | 0277 | 1 | B | Definition of UE-slice-MBR | 17.2.0 |
| 2021-06 | CT#92E | CP-211034 | 0275 |  | F | Home Network Identifier for SNPN | 17.2.0 |
| 2021-06 | CT#92E | CP-211050 | 0285 |  | F | 29.571 Rel-17 API version and External doc update | 17.2.0 |
| 2021-09 | CT#93E | CP-212054 | 0287 | 1 | F | Adding missing descriptions | 17.3.0 |
| 2021-09 | CT#93E | CP-212030 | 0289 | 2 | B | Clarification to SACInfo | 17.3.0 |
| 2021-09 | CT#93E | CP-212031 | 0290 |  | B | Spatial Validity Condition | 17.3.0 |
| 2021-09 | CT#93E | CP-212035 | 0291 | 1 | B | Common Data Types for MBS | 17.3.0 |
| 2021-09 | CT#93E | CP-212030 | 0292 |  | B | NSSRG value | 17.3.0 |
| 2021-09 | CT#93E | CP-212079 | 0295 | 2 | A | UE Transport Protocol Indication for N3GPP Location | 17.3.0 |
| 2021-09 | CT#93E | CP-212035 | 0296 |  | B | ProseServiceAuth | 17.3.0 |
| 2021-09 | CT#93E | CP-212059 | 0298 |  | F | 29.571 Rel-17 API version and External doc update | 17.3.0 |
| 2021-12 | CT#94E | CP-213100 | 0302 | 1 | B | Provisioning Server Information | 17.4.0 |
| 2021-12 | CT#94E | CP-213097 | 0303 | 1 | B | Additional common data types for MBS | 17.4.0 |
| 2021-12 | CT#94E | CP-213097 | 0304 | 1 | B | NCGI list of MBS Service Area | 17.4.0 |
| 2021-12 | CT#94E | CP-213097 | 0305 |  | B | Missing 502 response and description property in common data types for MBS | 17.4.0 |
| 2021-12 | CT#94E | CP-213199 | 0308 | 2 | F | Remove Siblings of $ref attributes in OpenAPI | 17.4.0 |
| 2021-12 | CT#94E | CP-213108 | 0309 |  | B | Common Data Types for SM Policy Association Establishment/Termination Events | 17.4.0 |
| 2021-12 | CT#94E | CP-213103 | 0310 | 1 | B | Update the RAT Type to support NR RedCap | 17.4.0 |
| 2021-12 | CT#94E | CP-213093 | 0311 |  | F | Correction of Spatial Validity Condition | 17.4.0 |
| 2021-12 | CT#94E | CP-213124 | 0315 |  | F | Extention of userLocationInfo attribute to support GERAN/UTRAN access | 17.4.0 |
| 2021-12 | CT#94E | CP-213092 | 0316 |  | F | Immediate Report | 17.4.0 |
| 2021-12 | CT#94E | CP-213088 | 0319 |  | A | SEPP Redirection | 17.4.0 |
| 2021-12 | CT#94E | CP-213137 | 0317 |  | B | Adding EAS IP replacement information | 17.4.0 |
| 2021-12 | CT#94E | CP-213111 | 0312 | 1 | B | Common Data Type for Satellite Backhaul Category | 17.4.0 |
| 2021-12 | CT#94E | CP-213088 | 0313 | 1 | A | SnssaiExtension data type definition | 17.4.0 |
| 2021-12 | CT#94E | CP-213121 | 0320 |  | F | 29.571 Rel-17 API version and External doc update | 17.4.0 |
| 2022-03 | CT#95E | CP-220047 | 0323 | 4 | F | SNPN impacts - new common type RoamingRestrictions | 17.5.0 |
| 2022-03 | CT#95E | CP-220023 | 0325 |  | F | BitRate Units | 17.5.0 |
| 2022-03 | CT#95E | CP-220024 | 0326 | 2 | F | Fqdn data type definition | 17.5.0 |
| 2022-03 | CT#95E | CP-220023 | 0327 |  | F | PatchItem definition | 17.5.0 |
| 2022-03 | CT#95E | CP-220306 | 0328 | 4 | F | PVS Info | 17.5.0 |
| 2022-03 | CT#95E | CP-220030 | 0329 | 1 | F | SACInfo in periodic notificatio | 17.5.0 |
| 2022-03 | CT#95E | CP-220025 | 0330 | 1 | F | Alignment of desription fields | 17.5.0 |
| 2022-03 | CT#95E | CP-220079 | 0332 |  | A | Correction to wrong CR implementation | 17.5.0 |
| 2022-03 | CT#95E | CP-220035 | 0334 | 1 | F | MbsSession data type for MBS session creation response | 17.5.0 |
| 2022-03 | CT#95E | CP-220125 | 0335 | 2 | F | MBS Session Status subscriptions and notifications | 17.5.0 |
| 2022-03 | CT#95E | CP-220035 | 0336 |  | B | Extensions for Location dependent MBS session | 17.5.0 |
| 2022-03 | CT#95E | CP-220035 | 0337 | 1 | F | MbsServiceArea data type extension | 17.5.0 |
| 2022-03 | CT#95E | CP-220025 | 0338 | 1 | F | Clarifications to the SupportedFeatures Type encoding | 17.5.0 |
| 2022-03 | CT#95E | CP-220066 | 0340 |  | F | 29.571 Rel-17 API version and External doc update | 17.5.0 |
| 2022-06 | CT#96 | CP-221024 | 0342 | 4 | F | MBS Security Context (MSK/MTK) Definitions | 17.6.0 |
| 2022-06 | CT#96 | CP-221043 | 0343 | 2 | F | Relay Service Code | 17.6.0 |
| 2022-06 | CT#96 | CP-221023 | 0344 |  | F | MBS Frequency Selection Area Identifier | 17.6.0 |
| 2022-06 | CT#96 | CP-221023 | 0346 |  | F | MBS Service Area Information for Location dependent MBS session | 17.6.0 |
| 2022-06 | CT#96 | CP-22103 | 0347 |  | F | Broadcast Delivery Status event | 17.6.0 |
| 2022-06 | CT#96 | CP-221024 | 0348 | 1 | F | Ingress Tunnel Address Change Status Event | 17.6.0 |
| 2022-06 | CT#96 | CP-221036 | 0349 |  | F | SUCI Regular Expression Pattern | 17.6.0 |
| 2022-06 | CT#96 | CP-221028 | 0350 | 4 | F | Applying the agreed formatting to the 'description' fields in A.2 | 17.6.0 |
| 2022-06 | CT#96 | CP-221027 | 0351 |  | F | BitRate | 17.6.0 |  |
| 2022-06 | CT#96 | CP-221045 | 0352 |  | F | Obsolete ChargingId Data Type | 17.6.0 |
| 2022-06 | CT#96 | CP-221024 | 0353 | 3 | F | Correction to the 'ingressTunAddr' type | 17.6.0 |
| 2022-06 | CT#96 | CP-221029 | 0354 | 3 | F | MNC Encoding in NfSetId and NfServiceSetId | 17.6.0 |
| 2022-06 | CT#96 | CP-221055 | 0355 | 5 | B | NSAG ID | 17.6.0 |
| 2022-06 | CT#96 | CP-221071 | 0361 |  | F | Incomplete CR implementation for RouteToLocation | 17.6.0 |
| 2022-06 | CT#96 | CP-221034 | 0362 | 1 | B | FQDN Pattern Matching Rule | 17.6.0 |
| 2022-06 | CT#96 | CP-221051 | 0365 |  | F | 29.571 Rel-17 API version and External doc update | 17.6.0 |
| 2022-09 | CT#97 | CP-222031 | 0366 | 1 | F | Defining the MBS Service Requirements | 17.7.0 |
| 2022-09 | CT#97 | CP-222029 | 0368 | 1 | F | Spatial Validity Condition | 17.7.0 |
| 2022-09 | CT#97 | CP-222048 | 0369 | 1 | F | WLAN location information for interworking between ePDG connected to EPC and 5GS | 17.7.0 |
| 2022-09 | CT#97 | CP-222026 | 0370 |  | F | PlmnIdNid conversion to string (e.g. when used in maps as key) | 17.7.0 |
| 2022-09 | CT#97 | CP-222026 | 0371 | 1 | F | Clarification on GUAMI List in BackupAmfInfo | 17.7.0 |
| 2022-09 | CT#97 | CP-222031 | 0372 | 1 | F | Clarification for the keyDomainId with SNPN | 17.7.0 |
| 2022-09 | CT#97 | CP-222214 | 0373 | 2 | F | 5GPRUK ID Common Data Type | 17.7.0 |
| 2022-09 | CT#97 | CP-222069 | 0375 |  | F | Missing Reference RFC 7542 | 17.7.0 |
| 2022-09 | CT#97 | CP-22229 | 0376 | 1 | F | Correction of ECS Configuration Information | 17.7.0 |
| 2022-09 | CT#97 | CP-222031 | 0377 | 1 | F | Updates and corrections to the common MBS data model | 17.7.0 |
| 2022-09 | CT#97 | CP-222058 | 0378 |  | F | 29.571 Rel-17 API version and External doc update | 17.7.0 |
| 2022-12 | CT#98 | CP-223036 | 0382 |  | F | Corrections to MBS data types | 17.8.0 |
| 2022-12 | CT#98 | CP-223054 | 0384 | 2 | F | 5GPRUK Name Alignment | 17.8.0 |
| 2022-12 | CT#98 | CP-223066 | 0392 |  | F | 29.571 Rel-17 API version and External doc update | 17.8.0 |
| 2022-12 | CT#98 | CP-223052 | 0380 | 1 | F | Clarificaiton on Usage of RedCap RAT Type | 18.0.0 |
| 2022-12 | CT#98 | CP-223029 | 0383 | 2 | F | Extending the problem details with supported API versions | 18.0.0 |
| 2022-12 | CT#98 | CP-223040 | 0385 |  | F | Remove Uint16 and Uint16Rm | 18.0.0 |
| 2022-12 | CT#98 | CP-223040 | 0386 | 1 | F | Misspellings of array | 18.0.0 |
| 2022-12 | CT#98 | CP-223033 | 0391 |  | F | 29.571 Rel-18 API version and External doc update | 18.0.0 |
| 2023-03 | CT#99 | CP-230080 | 0396 |  | A | PduSessionInfo | 18.1.0 |
| 2023-03 | CT#99 | CP-230033 | 0398 |  | B | PLMN list in Spatial Validity Condition | 18.1.0 |
| 2023-03 | CT#99 | CP-230029 | 0407 |  | F | Lower case of UUIDs in URIs | 18.1.0 |
| 2023-03 | CT#99 | CP-230036 | 0399 | 1 | B | Add associated session ID for MOCN | 18.1.0 |
| 2023-03 | CT#99 | CP-230041 | 0397 | 1 | B | Adding GEO satellite ID type | 18.1.0 |
| 2023-03 | CT#99 | CP-230041 | 0409 | 1 | B | Support of dynamic Satellite backhaul category | 18.1.0 |
| 2023-03 | CT#99 | CP-230081 | 0405 | 1 | A | Update ProseServiceAuth to support the authorization of UE-to-Network relay | 18.1.0 |
| 2023-03 | CT#99 | CP-230044 | 0401 | 2 | B | Metadata for Service Function Chain | 18.1.0 |
| 2023-03 | CT#99 | CP-230046 | 0402 | 1 | B | Manage Event Muting Impact on NFp | 18.1.0 |
| 2023-03 | CT#99 | CP-230049 | 0394 | 3 | F | Correcting $ref in the MbsSession data type | 18.1.0 |
| 2023-03 | CT#99 | CP-230071 | 0410 |  | F | 29.571 Rel-18 API version and External doc update | 18.1.0 |
| 2023-06 | CT#100 | CP-231025 | 0412 |  | F | SnssaiExtension | 18.2.0 |
| 2023-06 | CT#100 | CP-231033 | 0415 |  | B | Remove PLMN Ids in the Spatial Condition | 18.2.0 |
| 2023-06 | CT#100 | CP-231035 | 0416 | 1 | B | Packet Rate and Traffic Volume | 18.2.0 |
| 2023-06 | CT#100 | CP-231033 | 0413 | 2 | B | VPLMN Specific Offloading Information | 18.2.0 |
| 2023-06 | CT#100 | CP-231042 | 0422 | 2 | F | Support of multi-path transmission | 18.2.0 |
| 2023-06 | CT#100 | CP-231027 | 0419 | 1 | B | Correction on readonly definition | 18.2.0 |
| 2023-06 | CT#100 | CP-231048 | 0417 | 2 | B | Support of Alternative S-NSSAI | 18.2.0 |
| 2023-06 | CT#100 | CP-231057 | 0420 | 1 | F | Support of PDU Set QoS Parameters | 18.2.0 |
| 2023-06 | CT#100 | CP-231048 | 0423 | 1 | B | Partially Allowed NSSAI | 18.2.0 |
| 2023-06 | CT#100 | CP-231028 | 0424 |  | F | Correction of the interger data type | 18.2.0 |
| 2023-06 | CT#100 | CP-231047 | 0426 | 2 | B | Variable reporting periodicity | 18.2.0 |
| 2023-06 | CT#100 | CP-231042 | 0428 |  | F | Update on Update on U2U relay capabilities and subscription | 18.2.0 |
| 2023-06 | CT#100 | CP-231052 | 0429 | 2 | B | Ranging Sidelink Positioning Subscription data | 18.2.0 |
| 2023-06 | CT#100 | CP-231069 | 0432 | 2 | F | Update on the format of NfInstanceId | 18.2.0 |
| 2023-06 | CT#100 | CP-231070 | 0434 |  | F | 29.571 Rel-18 API version and External doc update | 18.2.0 |
| 2023-09 | CT#101 | CP-232040 | 0436 |  | C | Common Time Sync Data | 18.3.0 |
| 2023-09 | CT#101 | CP-232043 | 0440 | 1 | F | Optionality of status attribute in SnssaiReplaceInfo | 18.3.0 |
| 2023-09 | CT#101 | CP-232054 | 0441 | 1 | B | PDU Set Integrated Handling Information | 18.3.0 |
| 2023-09 | CT#101 | CP-232065 | 0443 | 1 | F | Correct data type name MbsMediaComp | 18.3.0 |
| 2023-09 | CT#101 | CP-232037 | 0444 | 1 | B | Addition of flag within the SACInfo to indicate if the number of UEs reported are the ones with at least one PDU session/PDN connection | 18.3.0 |
| 2023-09 | CT#101 | CP-232043 | 0445 | 2 | B | Addition of the roaming requirements and Network Slice Replacement termination indication in SnssaiReplaceInfo | 18.3.0 |
| 2023-09 | CT#101 | CP-232045 | 0447 | 2 | B | Add CommonData to support IMS DC | 18.3.0 |
| 2023-09 | CT#101 | CP-232058 | 0448 | 2 | B | QoE Parameters | 18.3.0 |
| 2023-09 | CT#101 | CP-232046 | 0449 |  | B | Common data types for A2X service | 18.3.0 |
| 2023-09 | CT#101 | CP-232057 | 0450 | 1 | F | Resolve EN for Multi-Path Transmission Term | 18.3.0 |
| 2023-09 | CT#101 | CP-232036 | 0452 |  | B | Authorized Session-AMBR for Offloading | 18.3.0 |
| 2023-09 | CT#101 | CP-232058 | 0456 | 1 | F | RedirectResponse Update | 18.3.0 |
| 2023-09 | CT#101 | CP-232049 | 0457 | 2 | B | Update common data for Ranging Sidelink Positioning | 18.3.0 |
| 2023-09 | CT#101 | CP-232043 | 0458 | 1 | B | Slice usage control information | 18.3.0 |
| 2023-09 | CT#101 | CP-232156 | 0459 | 3 | F | Add on Group-Service-Id | 18.3.0 |
| 2023-09 | CT#101 | CP-232060 | 0460 |  | F | 29.571 Rel-18 API version and External doc update | 18.3.0 |
| 2023-12 | CT#102 | CP-233050 | 0465 |  | B | Resolve Editor’s note | 18.4.0 |
| 2023-12 | CT#102 | CP-233028 | 0462 | 1 | F | HTTP RFCs obsoleted by IETF RFC 9110, 9111 and 9113 | 18.4.0 |
| 2023-12 | CT#102 | CP-233064 | 0471 | 2 | B | Definition of NSAC Service Area. | 18.4.0 |
| 2023-12 | CT#102 | CP-233038 | 0463 | 1 | B | VPLMN offloading policy information | 18.4.0 |
| 2023-12 | CT#102 | CP-233056 | 0468 | 1 | F | String based Charging Id Support | 18.4.0 |
| 2023-12 | CT#102 | CP-233048 | 0464 | 1 | B | Update the DcStream to add application binding information | 18.4.0 |
| 2023-12 | CT#102 | CP-233055 | 0466 | 1 | F | Enhancement to support UE-to-UE relay | 18.4.0 |
| 2023-12 | CT#102 | CP-233041 | 0472 |  | F | Time Source | 18.4.0 |
| 2023-12 | CT#102 | CP-233031 | 0474 | 1 | F | Reserved characters in JSON attributes defined as URI | 18.4.0 |
| 2023-12 | CT#102 | CP-233063 | 0477 | 1 | A | Area Session Policy ID | 18.4.0 |
| 2023-12 | CT#102 | CP-233063 | 0479 | 1 | A | MBS Service Area not contained within the MB-SMF service area | 18.4.0 |
| 2023-12 | CT#102 | CP-233054 | 0480 |  | B | User Location Information of AUN3 device | 18.4.0 |
| 2023-12 | CT#102 | CP-233067 | 0482 | 2 | A | Preventing NR to LTE NTN mobility for users without LTE NTN subscription | 18.4.0 |
| 2023-12 | CT#102 | CP-233053 | 0483 | 5 | B | Protocol Description | 18.4.0 |
| 2023-12 | CT#102 | CP-233048 | 0484 | 1 | B | Add the Endpoint data type as common data for IMS SBA | 18.4.0 |
| 2023-12 | CT#102 | CP-233031 | 0486 | 1 | F | Case insensitive handling of DNN | 18.4.0 |
| 2023-12 | CT#102 | CP-233030 | 0487 |  | F | Corrections on MbsKeyInfo and MbsQoSReq data types | 18.4.0 |
| 2023-12 | CT#102 | CP-233030 | 0488 |  | F | ProblemDetails RFC 7807 obsoleted by 9457 | 18.4.0 |
| 2023-12 | CT#102 | CP-233056 | 0489 | 3 | B | Extend nrLocation to include NR NTN TAI information | 18.4.0 |
| 2023-12 | CT#102 | CP-233038 | 0490 | 1 | F | Correction of attribute Ipv6AddressRanges | 18.4.0 |
| 2023-12 | CT#102 | CP-233056 | 0491 |  | B | Addition of MBS parameters for QMC | 18.4.0 |
| 2023-12 | CT#102 | CP-233054 | 0492 | 1 | B | Service restriction of AUN3 device access 5GC via W-5GAN | 18.4.0 |
| 2023-12 | CT#102 | CP-233056 | 0493 |  | F | Addition of missing descriptions of data types | 18.4.0 |
| 2023-12 | CT#102 | CP-233063 | 0495 | 2 | A | Correction of the external MBS Service Area descriptiona | 18.4.0 |
| 2023-12 | CT#102 | CP-233041 | 0497 |  | F | New data type to represent a combination of S-NSSAI(s) and/or DNN(s) | 18.4.0 |
| 2023-12 | CT#102 | CP-233060 | 0498 |  | F | 29.571 Rel-18 API version and External doc update | 18.4.0 |
| 2024-03 | CT#103 | CP-240028 | 0501 |  | B | Report amount in MDT configuration | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0505 |  | F | N3gaLocation for Interworking between ePDG connected to EPC and 5GS | 18.5.0 |
| 2024-03 | CT#103 | CP-240040 | 0513 |  | B | MPQUIC support | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0515 |  | B | Update ProblemDetails for the HTTP status code 403 | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0519 |  | F | Case insensitive handling of FQDN | 18.5.0 |
| 2024-03 | CT#103 | CP-240073 | 0525 |  | A | Correction to UtraLocation and GeraLocation | 18.5.0 |
| 2024-03 | CT#103 | CP-240072 | 0528 |  | A | Trace Data | 18.5.0 |
| 2024-03 | CT#103 | CP-240053 | 0529 |  | F | NR Location with NTN TAI Information | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0530 |  | F | Case insensitive handling of NF Set ID or NF Service Set ID | 18.5.0 |
| 2024-03 | CT#103 | CP-240052 | 0531 |  | B | Format of the User Location Information of 5G-RG acting as a TNAP accessing 5GC via TNGF | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0532 |  | F | NoProfileMatchInfo caching in SCPs | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0500 | 1 | B | Area Scope in MDT configuration | 18.5.0 |
| 2024-03 | CT#103 | CP-240035 | 0516 | 1 | B | Definition of data types to support NR RedCap UEs in MBS Broadcast | 18.5.0 |
| 2024-03 | CT#103 | CP-240042 | 0520 | 1 | B | Congestion mitigation information in Network Slice Replacement | 18.5.0 |
| 2024-03 | CT#103 | CP-240034 | 0521 | 1 | B | Updates the PduSetQosPara definition | 18.5.0 |
| 2024-03 | CT#103 | CP-240034 | 0509 | 1 | F | Corrections to Protocol Description | 18.5.0 |
| 2024-03 | CT#103 | CP-240034 | 0510 | 1 | F | C Extensions to Protocol Description requested by SA4 | 18.5.0 |
| 2024-03 | CT#103 | CP-240034 | 0512 | 1 | B | RTP Header Extension Type in Protocol Description | 18.5.0 |
| 2024-03 | CT#103 | CP-240053 | 0522 | 1 | F | Editorial and Style Corrections | 18.5.0 |
| 2024-03 | CT#103 | CP-240048 | 0518 | 1 | F | Addition of new data type for MBSR operation allowed | 18.5.0 |
| 2024-03 | CT#103 | CP-240156 | 0502 | 2 | F | Clock Accuracy | 18.5.0 |
| 2024-03 | CT#103 | CP-240039 | 0507 | 1 | C | Clock Quality Acceptance Criteria | 18.5.0 |
| 2024-03 | CT#103 | CP-240028 | 0533 | 1 | F | Error description in 401 Unauthorized response | 18.5.0 |
| 2024-03 | CT#103 | CP-240031 | 0514 | 2 | F | Add nullvalue for VplmnOffloadingInfo to enable the removal | 18.5.0 |
| 2024-03 | CT#103 | CP-240029 | 0506 | 3 | F | Access control for users with eRedcap/Redcap subscriptions - enumeration | 18.5.0 |
| 2024-03 | CT#103 | CP-240056 | 0534 |  | F | 29.571 Rel-18 API version and External doc update | 18.5.0 |
| 2024-06 | CT#104 | CP-241029 | 0537 | 1 | F | Description of NR_EREDCAP RAT Type | 18.6.0 |
| 2024-06 | CT#104 | CP-241050 | 0538 | 2 | B | Update the 5G Trace to support UE level measurement | 18.6.0 |
| 2024-06 | CT#104 | CP-241049 | 0541 | 1 | F | PDU Set QoS parameters | 18.6.0 |
| 2024-06 | CT#104 | CP-241039 | 0544 | 3 | F | Make IEs related TRS_URLLC nullable | 18.6.0 |
| 2024-06 | CT#104 | CP-241044 | 0545 | 4 | F | Support of SDP attributes a=3gpp-bdc-used-by and a=3gpp-req-app | 18.6.0 |
| 2024-06 | CT#104 | CP-241046 | 0546 | 1 | F | Updates on RangingSlPosAuth | 18.6.0 |
| 2024-06 | CT#104 | CP-241028 | 0548 |  | F | Definition of MbsSession | 18.6.0 |
| 2024-06 | CT#104 | CP-241033 | 0549 | 2 | F | Aligning the default value of nrRedCapUeInfo with stage 2 specification | 18.6.0 |
| 2024-06 | CT#104 | CP-241049 | 0550 | 2 | F | Removable data types for XRM | 18.6.0 |
| 2024-06 | CT#104 | CP-241028 | 0551 | 1 | F | Removal of unused Job Types | 18.6.0 |
| 2024-06 | CT#104 | CP-241046 | 0552 | 1 | F | JobType update for UE level measurements | 18.6.0 |
| 2024-06 | CT#104 | CP-241031 | 0553 | 4 | F | dlAmbr for HR-SBO PDU session | 18.6.0 |
| 2024-06 | CT#104 | CP-241028 | 0554 | 1 | F | Clarification on MNC Encoding | 18.6.0 |
| 2024-06 | CT#104 | CP-241049 | 0555 | 1 | F | Granularity of PduSetQosParaRm | 18.6.0 |
| 2024-06 | CT#104 | CP-241028 | 0556 |  | B | Corrections for MDT enhancements to support NPN | 18.6.0 |
| 2024-06 | CT#104 | CP-241050 | 0557 | 1 | F | Description of mbsMediaComps attribute | 18.6.0 |
| 2024-06 | CT#104 | CP-241044 | 0558 | 2 | F | Clarification on the maxRetry and maxTime of DcStream | 18.6.0 |
| 2024-06 | CT#104 | CP-241044 | 0559 | 1 | F | Define the common data type for MDC interface | 18.6.0 |
| 2024-06 | CT#104 | CP-241044 | 0561 |  | F | Update the MediaProxy value and update the DcEndpoint data type | 18.6.0 |
| 2024-06 | CT#104 | CP-241028 | 0562 |  | F | Update on data type Any type | 18.6.0 |
| 2024-06 | CT#104 | CP-241050 | 0563 |  | F | Correct the NfServiceSetId description | 18.6.0 |
| 2024-06 | CT#104 | CP-241052 | 0564 |  | F | 29.571 Rel-18 API version and External doc update | 18.6.0 |
| 2024-09 | CT#105 | CP-242048 | 0568 | 4 | F | Correct the data type of AppDcInfo | 18.7.0 |
| 2024-09 | CT#105 | CP-242053 | 0569 | 1 | F | PDU Session Id range for 2G/3G access | 18.7.0 |
| 2024-09 | CT#105 | CP-242053 | 0572 | 1 | F | Correction on 5GC UE level measurements | 18.7.0 |
| 2024-09 | CT#105 | CP-242053 | 0573 | 2 | F | Correction on MDT configuration in MR-DC | 18.7.0 |
| 2024-09 | CT#105 | CP-242048 | 0577 | 3 | F | Corrections on DC endpoint parameters | 18.7.0 |
| 2024-09 | CT#105 | CP-242054 | 0579 |  | F | 29.571 Rel-18 API version and External doc update | 18.7.0 |
| 2024-09 | CT#105 | CP-242033 | 0567 |  | F | Correction of attribute and IE names | 19.0.0 |
| 2024-09 | CT#105 | CP-242033 | 0576 | 2 | F | MDT parameters for M1, M8 and M9 | 19.0.0 |
| 2024-09 | CT#105 | CP-242038 | 0578 |  | F | 29.571 Rel-19 API version and External doc update | 19.0.0 |
| 2024-12 | CT#106 | CP-243063 | 0584 | 6 | B | Common Data Type for 5G Femto Information | 19.1.0 |
| 2024-12 | CT#106 | CP-2430 | 0585 | 2 | B | Support of optional extended UPF functionalities | 19.1.0 |
| 2024-12 | CT#106 | CP-2430 | 0586 | 3 | B | Support operator configurable parameter in common data | 19.1.0 |
| 2024-12 | CT#106 | CP-2430 | 0587 | 1 | B | Support of multi hop U2N and U2U | 19.1.0 |
| 2024-12 | CT#106 | CP-2430 | 0589 | 1 | A | Update on access restrictions for satellite access | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0592 | 2 | F | Correction of RtpHeaderExtInfo | 19.1.0 |
| 2024-12 | CT#106 | CP-243040 | 0593 | 3 | B | Local Offloading Information | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0598 | 3 | F | Applicability of Slice Usage Control Info | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0599 | 1 | B | Update the MeasurementType to support UE level measurement | 19.1.0 |
| 2024-12 | CT#106 | CP-243061 | 0600 | 2 | B | MPQUIC-IP and MPQUIC-E capabilities | 19.1.0 |
| 2024-12 | CT#106 | CP-243045 | 0603 | 2 | B | Add ImsEvent to the IMS common data | 19.1.0 |
| 2024-12 | CT#106 | CP-243064 | 0608 | 1 | B | Support of MoQ Transport protocol | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0609 |  | F | Correction on Plmn Data type | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0610 | 1 | F | Add description in data types OpenAPI definition | 19.1.0 |
| 2024-12 | CT#106 | CP-243059 | 0611 | 1 | B | Support of Network Slice Area Scope of MDT | 19.1.0 |
| 2024-12 | CT#106 | CP-243069 | 0615 |  | F | 29.571 Rel-19 API version and External doc update | 19.1.0 |
| 2025-03 | CT#107 | CP-250032 | 0618 | 1 | D | Type name alignment | 19.2.0 |
| 2025-03 | CT#107 | CP-250031 | 0620 | 1 | A | Correction of MBS support for eRedCap UEs | 19.2.0 |
| 2025-03 | CT#107 | CP-250047 | 0621 | 2 | B | Add the ADC interworking related IMS DC exposure event | 19.2.0 |
| 2025-03 | CT#107 | CP-250043 | 0624 | 1 | B | Support of Layer-2 multi-hop U2N relaying | 19.2.0 |
| 2025-03 | CT#107 | CP-250021 | 0627 | 1 | A | Correction of N3IWF user location information | 19.2.0 |
| 2025-03 | CT#107 | CP-250046 | 0628 | 1 | F | Correction on data type definitions of Snssai, SnssaiDnnItem and PresenceInfo | 19.2.0 |
| 2025-03 | CT#107 | CP-250042 | 0629 | 1 | F | Correction on GEO Satellite ID | 19.2.0 |
| 2025-03 | CT#107 | CP-250046 | 0630 | 1 | B | MBS broadcast support for NTN | 19.2.0 |
| 2025-03 | CT#107 | CP-250037 | 0631 |  | B | N6 delay measurement protocols | 19.2.0 |
| 2025-03 | CT#107 | CP-250045 | 0632 | 2 | B | NF signalling information data model | 19.2.0 |
| 2025-03 | CT#107 | CP-2500 | 0633 |  | F | 29.571 Rel-19 API version and External doc update | 19.2.0 |
| 2025-06 | CT#108 | CP-251053 | 0638 |  | A | Essential correction on the RTP Payload Type range for Protocol Description | 19.3.0 |
| 2025-06 | CT#108 | CP-251071 | 0639 | 2 | B | RTP header extension for dynamically changing traffic characteristics | 19.3.0 |
| 2025-06 | CT#108 | CP-251071 | 0640 |  | B | ProtocolDescription extensions for media related info over N6 using UDP Option | 19.3.0 |
| 2025-06 | CT#108 | CP-251073 | 0641 | 3 | B | Energy Saving Indicator | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0642 | 1 | B | Update the MediaProxy value | 19.3.0 |
| 2025-06 | CT#108 | CP-251067 | 0643 | 2 | F | Update and replace obsoleted HTTP RFC | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0645 |  | B | Common data types for IMS EE APIs | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0646 | 1 | F | ImsEvent correction | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0647 | 1 | B | Define the rendering mode as common data | 19.3.0 |
| 2025-06 | CT#108 | CP-251055 | 0650 | 1 | F | Correction on data type name of CoreNetworkTypeRm | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0652 | 1 | F | MediaResourceType values | 19.3.0 |
| 2025-06 | CT#108 | CP-251068 | 0653 |  | B | Support of RCD | 19.3.0 |
| 2025-06 | CT#108 | CP-251049 | 0656 | 1 | A | Delete AR in MediaResourceType | 19.3.0 |
| 2025-06 | CT#108 | CP-251046 | 0657 | 4 | B | Add Ambient IoT related common Data Type | 19.3.0 |
| 2025-06 | CT#108 | CP-251067 | 0659 | 2 | D | Editorial Changes | 19.3.0 |
| 2025-06 | CT#108 | CP-251079 | 0662 |  | F | 29.571 Rel-19 API version and External doc update | 19.3.0 |
| 2025-09 | CT#109 | CP-252049 | 0663 | 1 | F | Correction for description of AMFRegionID | 19.4.0 |
| 2025-09 | CT#109 | CP-252053 | 0664 | 1 | B | Additional RTP header extensions in ProtocolDescriptionRm | 19.4.0 |
| 2025-09 | CT#109 | CP-252053 | 0665 | 1 | B | RTP header extension for Expedited Transfer Indication | 19.4.0 |
| 2025-09 | CT#109 | CP-252050 | 0666 | 1 | F | Definition of transparent containers for IMS Exposure | 19.4.0 |
| 2025-09 | CT#109 | CP-252052 | 0667 | 1 | F | Update or Removal of CAG List | 19.4.0 |
| 2025-09 | CT#109 | CP-252042 | 0668 | 1 | F | Correct the description of attributes included in LocalOffloadingManagementInfo | 19.4.0 |
| 2025-09 | CT#109 | CP-252049 | 0670 | 2 | F | RAT Types for 2G/3G interworking scenarios | 19.4.0 |
| 2025-09 | CT#109 | CP-252048 | 0671 |  | B | Addition of OverloadControlInfo data type | 19.4.0 |
| 2025-09 | CT#109 | CP-252178 | 0672 | 2 | F | Update the NF signalling information | 19.4.0 |
| 2025-09 | CT#109 | CP-252054 | 0673 |  | F | Removal of Editor’s Note for EnergySavingIndicator | 19.4.0 |
| 2025-09 | CT#109 | CP-252049 | 0674 | 2 | F | Correction on the definition of Tac and MbsSecurityContext attributes | 19.4.0 |
| 2025-09 | CT#109 | CP-252049 | 0675 | 1 | D | Editorial corrections | 19.4.0 |
| 2025-09 | CT#109 | CP-252200 | 0676 | 2 | B | Completing AIoT Area ID definition and correction of references | 19.4.0 |
| 2025-09 | CT#109 | CP-252050 | 0677 | 1 | F | Reference update: RFC 9796 | 19.4.0 |
| 2025-09 | CT#109 | CP-252057 | 0678 | 1 | F | Clarify the Encoding of AIoT Device Simple Data Types | 19.4.0 |
| 2025-09 | CT#109 | CP-252050 | 0680 | 1 | F | Correct the description of appBinInfo in ImsEventFilter | 19.4.0 |
| 2025-09 | CT#108 | CP-252176 | 0682 |  | F | 29.571 Rel-19 API version and External doc update | 19.4.0 |