&3GPP TSG-RAN2 Meeting#110e                                  R2-20XXXX

April, 2020

Agenda Item:	XXX

Source: 	Samsung

Title: 	LTE Rel-16 ASN.1 Review, Class 0 and Class 1 issues

Document for:	Discussion and decision

# Guidelines

- This file is used to log LTE 36331 ASN:1 Review Class 0 and Class 1 issues.

- Trivial e.g. editorials, commas, colon, misspelling, missing/ double spaces, italics etc.

- Minor e.g. quite straightforward changes e.g. correction/ addition of specification references or sub-clauses

- Fill in the columns, see example.

  - Make sure the inserted specification text is unique, such that the location of the issue is simple to find.

  - Avoid indicating duplicated issues by checking if the concerned specification text is already reported in the table.

  - Step the file name v(x) -> v(x+1) and upload to ftp server.

- The “status” column will be filled in by the ASN.1 review moderator.

# Class 0 and Class 1 issues

| Issue number | Copied existing specification text. Text should be unique, so that it can be easily found in the specification. If needed, add also the new text. | Comment/description/ correction | Email address |  | Status |
| --- | --- | --- | --- | --- | --- |
| Ex 1 | 2> derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11]; | Missing italics. | hakan.l.palm@ericsson.com |  |  |
| Ex 2 | PUSCH scheduled by RAR UL grant (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1). | Incorrect reference, should be 9.2.101. | hakan.l.palm@ericsson.com |  |  |
| Insert issues from here |  |  |  |  |  |
| 1 | 2> if UE has sent RRCConnectionResumeRequest message and has not received RRCConnectionResume message: 3> reset MAC; 3> if UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.1: | Reference is incorrect i.e. should be 5.3.3.18 |  | uphuyal@qti.qualcomm.com |  |
| 2 | NOTE 2 In case of DRB reconfiguration at a DAPS HO, the reconfiguration is applied to the entities/resources for the target PCell | A colon is missing (after 2) |  | uphuyal@qti.qualcomm.com |  |
| 3 | Upon receiving SystemInformationBlockType2, the UE shall: … NOTE: upperLayerIndication is an indication to upper layers that the UE has entered a coverage area that offers 5G capabilities. 1> to upper layers either forward rlos-Enabled, if present, or otherwise indicate absence of this field; 1> if up-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if up-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE without a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE without a suspended RRC connection is configured with pur-Config: 2> release pur-Config; 2> indicate to lower layers that pur-Config is released. Upon receiving SystemInformationBlockType2-NB, the UE shall: 1> apply the configuration included in the radioResourceConfigCommon; 1> apply the defaultPagingCycle included in the radioResourceConfigCommon; 1> if SystemInformationBlockType22-NB is scheduled: 2> read and act on information sent in SystemInformationBlockType22-NB; 1> apply the specified PCCH configuration defined in 9.1.1.3. 1> if in RRC_CONNECTED and UE is configured with RLF timers and constants values received within rlf-TimersAndConstants: 2> not update its values of the timers and constants in ue-TimersAndConstants except for the value of timer T300; | section 5.2.2.9 The highlighted part should also have been captured under ‘Upon receiving SystemInformationBlockType2-NB’ but was missed during the CR implementation. It is proposed to move it at the end of the section and make it common to both as shown below: Upon receiving SystemInformationBlockType2, the UE shall: … Upon receiving SystemInformationBlockType2-NB, the UE shall: … Upon receiving Upon receiving SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), , the UE shall: 1> if up-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if up-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE without a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE without a suspended RRC connection is configured with pur-Config: 2> release pur-Config; 2> indicate to lower layers that pur-Config is released. [Qualcomm]: suggest is ok (minor typos): Upon receiving Upon receiving SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), , the UE shall: |  | First comment by odile.rollinger at Huawei |  |
| 4 | 3> for NB-IoT: 4> if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report: 5> include rlf-InfoAvailable; 4> if the UE has ANR measurements results available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB: 5> include anr-InfoAvailable; | section 5.3.3.4, ‘-NB' is missing in the variable name |  |  |  |
| 5 | 3> if the UE is connected to EPC: 4> if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report-NB: 5> include the rlf-InfoAvailable; 4> if the UE has ANR measurements information available in VarANR-MeasurementReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasurementReport-NB: 5> include anr-InfoAvailable; | section 5.3.7.5, add ‘the’ before anr-InfoAvailable for consistency |  |  |  |
| 6 | 2> if the UE is a NB-IoT UE connected to 5GC: 2> if a DRB was configured with the same pdu-Session (fullConfig): 3> associate the established DRB with corresponding included pdu-Session; 2> else if the entry of drb-ToAddModList includes pdcp-config (establishment of bearer): 3> indicate the establishment of the DRB(s) and the pdu-Session of the established DRB(s) to upper layers; | section 5.3.10.3, the bullet numbering is incorrect |  |  |  |
| 7 | When initiating the procedure according to 5.6.23.2, the UE shall set the contents of the PURConfigurationRequest message as follows: 1> set requestedNumOccasions to the requested number of PUR occasions requested; 1> set requestedPeriodicity to the requested periodicity between consecutive PUR occasions; 1> set requestedTBS to the requested TBS for the PUR occasion(s); 1> if UE preference is that no RRC response message is needed for acknowledging the reception of a transmission using PUR, set l1-ACK to TRUE; 1> set requestedTimeOffset to the requested time gap with respect to current time until the first PUR occasion; | section 5.6.23.3, l1-ACK is defined as ENUMERATED {true}, should be changed to ‘include l1-ACK’ |  |  |  |
| 8 | While the UE is in RRC_IDLE, the UE shall: 1> store the measurement results for the serving cell in measResultServCell in VarANR-MeasReport-NB; 1> while the serving cell global cell identity is the same as stored in servCellIdentity in VarANR-MeasReport-NB: 2> perform the measurements once in accordance with the following: 3> for each carrier frequency indicated by an entry in anr-CarrierList, if present, within VarANR-MeasConfig; or … 1> release the VarANR-MeasConfig. The UE may discard the ANR measurements information, i.e. release the UE variables VarANR-MeasConfig and VarANR-MeasReport, [96] hours after the configuration was received, upon power off or upon detach. | section 5.6.24.1: 'NB' is missing in VarANR-MeasConfig / VarANR-MeasReport VarANR-MeasConfig should be italics in the last bullet |  |  |  |
| 9 | cp-CIoT-5GS-Optimisation This field indicates if the UE is allowed to establish the connection with Control plane CIoT 5GS optimisation, see TS 24.501 [95]. up-CIoT-5GS-Optimisation This field indicates if the UE is allowed to resume the connection with User plane CIoT 5GS Optimisation, see TS24.501 [95]. | section 6.2 SystemInformationBlockType1: remove ‘this field’ and change ‘if’ to ‘whether’ to align with other field description (e.g. ims-EmergencySupport) |  |  |  |
| 10 | #### – SystemInformationBlockType27 The IE SystemInformationBlockType27 contains assistance information relevant only for inter-RAT cell selection i.e. assistance information about NB-IoT frequencies for cell selection. | section 6.3.1 SystemInformationBlockType27, remove the first ‘assistance’ |  |  |  |
| 11 | #### – SystemInformationBlockType27-NB The IE SystemInformationBlockType27-NB contains assistance information relevant only for inter-RAT cell selection i.e. assistance information about E-UTRA frequencies and/ or GERAN frequencies for cell selection. | section 6.7.3.1 SystemInformationBlockType2-NB7, remove the first ‘assistance’ |  |  |  |
| 12 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. rlf-InfoAvailable This field is used to indicate the availability of radio link failure related information. | section 6.7.2 'RRCConnectionReestablishmentComplete-NB, remove 'the field is used to' , this does not align with other field description |  |  |  |
| 13 | RRCConnectionRelease-NB-v16xy-IEs ::= SEQUENCE { resumeIdentity-r16 I-RNTI-r15 OPTIONAL, -- Need OR anr-MeasConfig-r16 ANR-MeasConfig-NB-r16 OPTIONAL, -- Need ON pur-Config-r16 CHOICE { release NULL, setup PUR-Config-NB-r16 } OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } anr-MeasConfig Configuration of the measurements to be performed by the UE in RRC_IDLE for ANR. | section 6.7.2 'RRCConnectionRelease-NB, remove field description of anr-MeasConfig as a IE type is defined |  |  |  |
| 14 | releaseCause The releaseCause is used to indicate the reason for releasing the RRC Connection. E-UTRAN should not set the releaseCause to loadBalancingTAURequired if the extendedWaitTime is present. The network should not set the releaseCause to loadBalancingTAURequired if the UE is connected to 5GC. | section 6.7.2 'RRCConnectionRelease-NB the two sentences can be merged together |  |  |  |
| 15 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. rlf-InfoAvailable This field is used to indicate the availability of radio link failure related information. | section 6.7.2 RRCConnectionResumeComplete-NB remove 'the field is used to' , this does not align with other field description |  |  |  |
| 16 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. | section 6.7.2 RRCConnectionResumeRequest-NB remove 'the field is used to' , this does not align with other field description |  |  |  |
| 17 | ng-U-DataTransfer If present, the field indicates that the NG-U data transfer as specified in TS 24.501 [95] is supported. up-CIoT-5GS-Optimisation This field indicates if the UE is allowed to resume the connection with User plane CIoT 5GS Optimisation, see TS24.501 [95]. | section 6.7.2 SystemInformationBlockType1-NB Align with other field description: Change both to ‘Indicates whether’ |  |  |  |
| 18 | anr-ReportReq This field is used to indicate whether the UE shall report, if available, ANR measurement results. rach-ReportReq This field is used to indicate whether the UE shall report, if available, information about the random access procedure. rlf-ReportReq This field is used to indicate whether the UE shall report, if available, information about radio link failure. | section 6.7.2 UEInformationRequest-NB: remove 'the field is used to' , this does not align with other field description |  |  |  |
| 19 | anr-MeasReport This field indicates the ANR measurement information. failedPCellId This field is used to indicate the PCell in which RLF is detected. initialNRSRP-Level Indicates the NRSRP level of the NPRACH resource selected for the first preamble transmission. measResultLastServCell This field refers to the last measurement results taken in the PCell, where radio link failure happened. numberOfPreamblesSent This field is used to indicate the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6]. reestablishmentCellId This field is used to indicate the cell in which the re-establishment attempt was made after connection failure. timeSinceFailure This field is used to indicate the time that elapsed since the connection failure. Value in seconds. The maximum value 172800 means 172800s or longer. | section 6.7.2 UEInformationResponse-NB: remove ‘This field’, 'the field is used to' , this does not align with other field description |  |  |  |
| 20 | contentionDetected This field is used to indicate that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6]. edt-Fallback Value TRUE indicates that EDT fallback indication was received from the lower layers, see TS 36.321 [6]. | section 6.7.2 UEInformationResponse-NB: contentionDetected is a Boolean. Change ‘This field is used to indicate’ to ‘Value TRUE indicates’ |  |  |  |
| 21 | cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to EPC, see 5.3.3.1b. cp-EDT-5GC For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to 5GC, see 5.3.3.1b. cp-PUR-EPC, cp-PUR-5GC This field indicates whether transmission using PUR is enabled in the cell for the Control Plane CIoT EPS/5GS optimisations respectively. | section 6.7.3.1 SystemInformationBlockType2-NB 'PUR same as EDT only applies to FDD. Add 'For FDD:' at the beginning of the field description |  |  |  |
| 22 | gwus-ResourcePosition Indicates the position of the WUS resource corresponding to the first entry in gwus-NumGroupsList-r16 Value primary indicates that the end of the WUS resource is defined by the timeoffset value for the corresponding gap type, value secondary indicates that the end of the WUS resource is immediately before the WUS resource configured by wus-Config-r15. E-UTRAN may only configure secondary when there is only one entry exists in gwus-NumGroupsList-r16 and wus-Config-r15 is present in SystemInformationBlockType2-NB. If two entries exist in gwus-NumGroupsList-r16, the position for the second WUS resource corresponds to value secondary. | section 6.7.3.2 GWUS-Config-NB typo, remove ‘exists’ |  |  |  |
| 23 | gwus-TimeParameters-r16 WUS-Config-NB-r15 OPTIONAL, -- Cond No-WUS-Config-r15 No-WUS-Config-r15: The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2-NB; otherwise the field is not present, and the UE shall delete any existing value for this field. | section 6.7.3.2 GWUS-Config-NB there is no need for hyphen. Better to align with eMTC: noWUSr15 |  |  |  |
| 24 | pur-RNTI-r16 C-RNTI OPTIONAL, --Need ON p0-UE-NPUSCH-r16 UplinkPowerControlDedicated-NB-r13, p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1, unit dB. pur-RNTI PUR-RNTI. | section 6.7.3.2 PUR-Config-NB remove field descriptions as a IE types are defined |  |  |  |
| 25 | ANR-CarrierList-NB-r16 ::= SEQUENCE (SIZE (1.. maxFreqANR-NB-r16)) OF ANR-Carrier-NB-r16 ANR-Carrier-NB-r16::= SEQUENCE { carrierFreqIndex-r16 INTEGER (1.. maxFreqANR-NB-r16), blackCellList-r16 ANR-BlackCellList-NB-r16 OPTIONAL, -- Need OP ... } ANR-BlackCellList-NB-r16 ::= SEQUENCE (SIZE (1.. maxCellBlack)) OF PhysCellId | section 6.7.3.5 ANR-MeasConfig-NB no need for space after ‘..’ |  |  |  |
| 26 | measResultList-r16 SEQUENCE (SIZE (1.. maxFreqANR-NB-r16)) OF ANR-MeasResult-NB-r16, | section 6.7.3.5 ANR-MeasReport-NB no need for space after ‘..’ |  | Last comment by odile.rollinger at Huawei |  |
| 27 | Section 5.3.12, there seems to be redundancy. Also, if the UE is “leaving RRC_INACTIVE”, then “if configured” does not make sense for rrc-InactiveConfig inside the condition. 1> else: 2> upon leaving RRC_INACTIVE: 3> discard the UE Inactive AS context; 3> release rrc-InactiveConfig, if configured; 3> discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key; 2> release rrc-InactiveConfig, if configured; | Remove the first occurrence (consistent with NR spec). 1> else: 2> upon leaving RRC_INACTIVE: 3> discard the UE Inactive AS context; 3> release rrc-InactiveConfig, if configured; 3> discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key; 2> release rrc-InactiveConfig, if configured; |  | uphuyal@qti.qualcomm.com |  |
| 28 | In section 5.3.8.3, 2> should be 3> below. 1> if the RRCConnectionRelease message includes the pur-Config: 2> if pur-Config is set to setup: 3> store or replace the PUR configuration provided by the pur-Config; 3> configure MAC in accordance with the stored pur-Config; 2> else: 3> release pur-Config, if configured; 3> discard previously stored pur-Config, if any; 2> indicate to lower layers that pur-Config is released. | 1> if the RRCConnectionRelease message includes the pur-Config: 2> if pur-Config is set to setup: 3> store or replace the PUR configuration provided by the pur-Config; 3> configure MAC in accordance with the stored pur-Config; 2> else: 3> release pur-Config, if configured; 3> discard previously stored pur-Config, if any; 23> indicate to lower layers that pur-Config is released. |  | uphuyal@qti.qualcomm.com |  |
| 29 | [[ configurdGrantAssistanceInfoReport-r16 BOOLEAN OPTIONAL -- Need ON ]] | Section 6.3.6 OtherConfig typo in the parameter name. Change to configuredGrantAssistanceInfoReport-r16 |  | hyunjeong.kang@samsung.com |  |
| 30 | 2> if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event: 3> remove the concerned transmission resource pool(s) from the poolsTriggeredList or poolsTriggeredListNR defined within the VarMeasReportList for this measId; | Section 5.5.4.1 poolsTriggeredListNR is missing in the if condition text add ‘or poolsTriggeredListNR' between 'the poolsTriggeredList' and 'defined' |  | hyunjeong.kang@samsung.com |  |
| 31 | SidelinkUEInformationNR-r16 ::= SEQUENCE { criticalExtensions CHOICE { sidelinkUEInformationNR-r16 SidelinkUEInformationNR-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } SidelinkUEInformationNR-r16-IEs::= SEQUENCE { sidelinkUEInformationNR-r16 OCTET STRING, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } sidelinkUEInformationNR Container for the indication of NR sidelink information, this field includes the SidelinkUEInformationNR IE as specified in TS 38.331 [82]. | Section 6.2.2 SidelinkUEInformationNR sidelinkUEInformationNR-r16 is used twice as different fields, one of field name should be changed to differentiate the fields. - Change the first sidelinkUEInformationNR-r16 to sidelinkUEInfoNR-r16 - Add OPTIONAL for the second sidelinkUEInformationNR-r16 in order to support future proof. SidelinkUEInformationNR-r16 ::= SEQUENCE { criticalExtensions CHOICE { sidelinkUEInfomationNR-r16 SidelinkUEInfomationNR-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } SidelinkUEInfomationNR-r16-IEs::= SEQUENCE { sidelinkUEInformationNR-r16 OCTET STRING OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |  | hyunjeong.kang@samsung.com |  |
| 32 | MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need OR tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need OR ... } carrierFreq Indicates the carrier frequency of pools configured for CBR measurement and reporting for NR sidelink communication. | Section 6.3.5 MeasObjectNR-SL Change carrierFreq-r15 to carrierFreq-r16 |  | hyunjeong.kang@samsung.com |  |
| 33 | eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING } | Section 6.3.5 ReportConfigEUTRA The description for S1 and S2 is missing in the heading text of ReportConfigEUTRA. Add the description for Event S1 and S2 in the heading text of ReportConfigEUTRA IE as follows. The E-UTRA measurement reporting events concerning CBR for NR sidelink communication are labelled SN with N equal to 1 and 2. Event S1: The NR sidelink channel busy ratio is above a threshold. Event S2: The NR sidelink channel busy ratio is below a threshold. |  | hyunjeong.kang@samsung.com |  |
| 34 | – MeasObjectToAddModList MeasObjectToAddModList ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod MeasObjectToAddModListExt-r13 ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddModExt-r13 MeasObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod-v9e0 MeasObjectToAddMod ::= SEQUENCE { measObjectId MeasObjectId, measObject CHOICE { measObjectEUTRA MeasObjectEUTRA, measObjectUTRA MeasObjectUTRA, measObjectGERAN MeasObjectGERAN, measObjectCDMA2000 MeasObjectCDMA2000, ..., measObjectWLAN-r13 MeasObjectWLAN-r13, measObjectNR-r15 MeasObjectNR-r15, measObjectNR-SL-r16 MeasObjectNR-SL-r16 } } MeasObjectToAddModExt-r13 ::= SEQUENCE { measObjectId-r13 MeasObjectId-v1310, measObject-r13 CHOICE { measObjectEUTRA-r13 MeasObjectEUTRA, measObjectUTRA-r13 MeasObjectUTRA, measObjectGERAN-r13 MeasObjectGERAN, measObjectCDMA2000-r13 MeasObjectCDMA2000, ..., measObjectWLAN-v1320 MeasObjectWLAN-r13, measObjectNR-r15 MeasObjectNR-r15, measObjectNR-SL-r16 MeasObjectNR-SL-r16 } } | Section 6.3.5 MeasObjectToAddModList General comment: do we need to extend the measObjectID range (greater than 64) to support newly introduced measObjectID? |  | hyunjeong.kang@samsung.com |  |
| 35 | Figure 5.10.15-1: Sidelink UE information for NR sidelink communication | Section 5.10.15 Change SIB XX2 to SIB 28 |  | hyunjeong.kang@samsung.com |  |
| 36 | The initiation and the procedure for the transmission of SidelinkUEInformationNR follow the procedures specified for NR sidelink communication in subclause 5.X.3 of TS 38.331 [82]. | Section 5.10.15 Change 5.X.3 to 5.8.3 |  | hyunjeong.kang@samsung.com |  |
| 37 | NOTE: When applying the procedure in this subclause, SystemInformationBlockType28 corresponds to SIBX specified in TS 38.331 [82]. | Section 5.10.15 Change SIBX to SIB12 |  | hyunjeong.kang@samsung.com |  |
| 38 | Figure 5.10.16-1: Synchronisation information transmission for NR sidelink communication, in (partial) coverage | Section 5.10.16 Change SIBXX to SIB28 |  | hyunjeong.kang@samsung.com |  |
| 39 | The initiation and the procedure for the transmission of sidelink SSB follow the procedure specified for NR sidelink communication in subclause 5.X.5 of TS 38.331 [82]. | Section 5.10.16 Change 5.X.5 to 5.8.5 |  | hyunjeong.kang@samsung.com |  |
| 40 | NOTE: When applying the procedure in this subclause, SystemInformationBlockType28 correspond to SIBX specified in TS 38.331 [82]. | Section 5.10.15 Change SIBX to SIB12 |  | hyunjeong.kang@samsung.com |  |
| 41 | MeasObjectNR-SL information element MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need OR tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need OR ... } | Section 6.3.5 MeasObjectNR-SL Change the need codes of both tx-ResourcePoolToRemoveList-r16/tx-ResourcePoolToAddList-r16, i.e. from Need OR to Need ON because No action is required when this field is absent. MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need ON tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need ON ... } |  | hyunjeong.kang@samsung.com |  |
| 42 | ReportConfigEUTRA ::= SEQUENCE { triggerType CHOICE { event SEQUENCE { eventId CHOICE { ... eventV1-r14 SEQUENCE { v1-Threshold-r14 SL-CBR-r14 }, eventV2-r14 SEQUENCE { v2-Threshold-r14 SL-CBR-r14 }, ... eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING } }, ReportConfigEUTRA field descriptions s1-Threshold, s2-Threshold Threshold used for events s1 and s2 specified in subclauses 5.5.4.18 and 5.5.4.19, respectively. They are containers with contents being c1-Threshold IE and c2-Threshold IE respectively, as specified in TS 38.331 [82]. | Section 6.3.5 ReportConfigEUTRA The events (S1 and S2) are encoded by EUTRA but the threshold is specified by an octet string. Since EUTRA can configure both event and threshold for S1 and S2, we suggest to encode threshold without using a container. eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING SL-CBR-r16 }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING SL-CBR-r16 } }, ThresholdEUTRA-v1250 ::= CSI-RSRP-Range-r12 MeasRSSI-ReportConfig-r13 ::= SEQUENCE { channelOccupancyThreshold-r13 RSSI-Range-r13 OPTIONAL -- Need OR } SL-CBR-r16 ::= INTEGER(0..100) ReportConfigEUTRA field descriptions s1-Threshold, s2-Threshold Threshold used for events s1 and s2 specified in subclauses 5.5.4.18 and 5.5.4.19, respectively. They are containers with contents being c1-Threshold IE and c2-Threshold IE respectively, as specified in TS 38.331 [82]. These fields indicate the SL-CBR-r16. SL-CBR Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on. |  | hyunjeong.kang@samsung.com |  |
| 43 | 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> if timer T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.6.20.3; | Text can be simplified/ running check not needed for release 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> stop timer T331, if running; 2> perform the actions as specified in 5.6.20.3; |  | himke.vandervelde at Samsung |  |
| 44 | 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> if timer T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.6.20.3; And RRCConnectionRelease-v16xy-IEs ::= SEQUENCE { releaseIdleMeasConfig ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } | Modify name used in ASN.1 and add suffix. Same for field description RRCConnectionRelease-v16xy-IEs ::= SEQUENCE { releaseMeasIdleConfig-r16 ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } |  | himke.vandervelde at Samsung |  |
| 45 |  |  |  |  |  |
| 46 |  |  |  |  |  |
| 47 |  |  |  |  |  |
| 48 |  |  |  |  |  |
| 49 |  |  |  |  |  |
| 50 |  |  |  |  |  |
| 51 |  |  |  |  |  |
| 52 |  |  |  |  |  |
| 53 |  |  |  |  |  |
| 54 |  |  |  |  |  |
| 55 |  |  |  |  |  |
| 56 |  |  |  |  |  |
| 57 |  |  |  |  |  |
| 58 |  |  |  |  |  |
| 59 |  |  |  |  |  |
| 60 |  |  |  |  |  |
| 61 |  |  |  |  |  |
| 62 |  |  |  |  |  |
| 63 |  |  |  |  |  |
| 64 |  |  |  |  |  |
| 65 |  |  |  |  |  |
| 66 |  |  |  |  |  |
| 67 |  |  |  |  |  |
| 68 |  |  |  |  |  |
| 69 |  |  |  |  |  |
| 70 |  |  |  |  |  |
| 71 |  |  |  |  |  |
| 72 |  |  |  |  |  |
| 73 |  |  |  |  |  |
| 74 |  |  |  |  |  |
| 75 |  |  |  |  |  |
| 76 |  |  |  |  |  |
| 77 |  |  |  |  |  |
| 78 |  |  |  |  |  |
| 79 |  |  |  |  |  |
| 80 |  |  |  |  |  |
| 81 |  |  |  |  |  |
| 82 |  |  |  |  |  |
| 83 |  |  |  |  |  |
| 84 |  |  |  |  |  |
| 85 |  |  |  |  |  |
| 86 |  |  |  |  |  |
| 87 |  |  |  |  |  |
| 87 |  |  |  |  |  |
| 88 |  |  |  |  |  |
| 89 |  |  |  |  |  |
| 90 |  |  |  |  |  |
| 91 |  |  |  |  |  |
| 92 |  |  |  |  |  |
| 93 |  |  |  |  |  |
| 94 |  |  |  |  |  |
| 95 |  |  |  |  |  |
| 96 |  |  |  |  |  |
| 97 |  |  |  |  |  |
| 98 |  |  |  |  |  |
| 99 |  |  |  |  |  |
| 100 |  |  |  |  |  |
| 101 |  |  |  |  |  |
| 102 |  |  |  |  |  |
| 103 |  |  |  |  |  |
| 104 |  |  |  |  |  |
| 105 |  |  |  |  |  |
| 106 |  |  |  |  |  |
| 107 |  |  |  |  |  |
| 108 |  |  |  |  |  |
| 109 |  |  |  |  |  |
| 110 |  |  |  |  |  |
| 111 |  |  |  |  |  |
| 112 |  |  |  |  |  |
| 113 |  |  |  |  |  |
| 114 |  |  |  |  |  |
| 115 |  |  |  |  |  |
| 116 |  |  |  |  |  |
| 117 |  |  |  |  |  |
| 118 |  |  |  |  |  |
| 119 |  |  |  |  |  |
| 120 |  |  |  |  |  |
| 121 |  |  |  |  |  |
| 122 |  |  |  |  |  |
| 123 |  |  |  |  |  |
| 124 |  |  |  |  |  |
| 125 |  |  |  |  |  |
| 126 |  |  |  |  |  |
| 127 |  |  |  |  |  |
| 128 |  |  |  |  |  |