3GPP TSG-RAN2 Meeting#110e                                  R2-20XXXX

April, 2020

Agenda Item:	XXX

Source: 	Ericsson

Title: 	NR Rel-16 ASN.1 Review, Class 0 and Class 1 issues

Document for:	Discussion and decision

# Guidelines

- This file is used to log NR 38331 ASN:1 Review Class 0 and Class 1 issues.

- Trivial e.g. editorials, commas, colon, misspelling, missing/ double spaces, italics etc.

- Minor e.g. quite straightforward changes e.g. correction/ addition of specification references or sub-clauses

- Fill in the columns, see example.

  - Make sure the inserted specification text is unique, such that the location of the issue is simple to find.

  - Avoid indicating duplicated issues by checking if the concerned specification text is already reported in the table.

  - Step the file name v(x) -> v(x+1) and upload to ftp server.

- The “status” column will be filled in by the ASN.1 review moderator.

# Class 0 and Class 1 issues

| Issue number | Copied existing specification text. Text should be unique, so that it can be easily found in the specification. If needed, add also the new text. | Comment/description/ correction | Email address | Status |
| --- | --- | --- | --- | --- |
| Ex 1 | 2> derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11]; | Missing italics. | hakan.l.palm@ericsson.com |  |
| Ex 2 | PUSCH scheduled by RAR UL grant (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1). | Incorrect reference, should be 9.2.101. | hakan.l.palm@ericsson.com |  |
| 1 | Original text: Performs logging of available measurements together with location and time for logged measurement configured UEs. Proposal text: Performs logging of available measurements together with location and time for if logged measurements is configured UEs. | There are two places in section 4.2.1 with the said text and both needs to be reworded as proposed. The reason for changing is to phrase the sentence from a specific UE point of view rather than a group of UEs point of view. | pradeepa.ramachandra@ericsson.com |  |
| 2 | The UE may discard the connection establishment failure information, i.e. release the UE variable VarConnEsFailReport, 48 hours after the last connection establishment failure is detected. | Missing Italics | pradeepa.ramachandra@ericsson.com |  |
| 3 | Generic comment: In some places the term SSB is used and in some other SS/PBCH Block is used. It is better to align the text with a single terminology | Alignment between SSB and SS/PBCH Block | pradeepa.ramachandra@ericsson.com |  |
| 4 | 5> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells associated to the measObjectNR, other than the source PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected handover failure; | ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 5 | 4> if the SS/PBCH block-based measurement quantities are available; 5> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells associated to the measObjectNR, other than the source PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected handover failure; 6> for each neighbour cell included, include the optional fields that are available; | ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 6 | 4> if the CSI-RS based measurement quantities are available; 5> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected handover failure; 6> for each neighbour cell included, include the optional fields that are available; | ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 7 | 3> for each of the configured EUTRA frequencies in which measurements are available; 4> set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected radio link failure; | ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 8 | 3> set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync; | This field name, timeConnFailure is not very self-explanatory and could be improved. Instead we can change the field name to ‘timeBetweenHOCommandAndFailure’. The changes are applicable in all places including ASN.1 | pradeepa.ramachandra@ericsson.com |  |
| 9 | 7> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected radio link failure; 8> for each neighbour cell included, include the optional fields that are available; | Missing italics ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 10 | 7> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure; 8> for each neighbour cell included, include the optional fields that are available; | ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 11 | 5> set the connectionFailureType to rlf; 5> set the c-RNTI to the C-RNTI used in the PCell; 5> set the rlf-Cause to the trigger for detecting radio link failure; 5> if the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure: | Missing italics | pradeepa.ramachandra@ericsson.com |  |
| 12 | 8> set the numberOfPreamblesSentOnSSB to indicate the number of successive random access attempts associated to the SS/PBCH block; | Missing hyphen (-) between random access i.e., random-access. | pradeepa.ramachandra@ericsson.com |  |
| 13 | 2> if the UE has connection resume failure informaton available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport: | Missing italics | pradeepa.ramachandra@ericsson.com |  |
| 14 | 2> if the ul-DelayValueConfig is configured for the associated reportConfig: 3> ignore the measObject; 3> for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB [x5]; | Missing reference | pradeepa.ramachandra@ericsson.com |  |
| 15 | 1> if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows: 2> include the locationTimestamp; 2> include the locationCoordinate, if available; 2> include the velocityEstimate, if available; 2> include the locationError, if available; 2> include the locationSource, if available; 2> if available, include the gnss-TOD-msec, | In section 5.5.5.1 Missing italics ‘;’ instead of ‘,’ | pradeepa.ramachandra@ericsson.com |  |
| 16 | 1> if reportType is set to periodical: 2> if a single reporting quantity is set to true in reportQuantityRS-Indexes; 3> consider the configured single quantity as the sorting quantity; 2> else: 3> if rsrp is set to true; 4> consider RSRP as the sorting quantity; 3> else: 4> consider RSRQ as the sorting quantity; | In section 5.5.5.2 ‘:’ instead of ‘;’ | pradeepa.ramachandra@ericsson.com |  |
| 17 | I 1> if available, set the locationInfo as follows: 2> if available, set the commonLocationInfo to include the detailed location information; 2> if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 2> if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs. 2> if available, set the sensor-LocationInfo to include the sensor measurement results. | n section 5.7.3.5 and 5.7.3a.3 ‘;’ instead of ‘.’ | pradeepa.ramachandra@ericsson.com |  |
| 18 | 1> if the number of RA-Report stored in the RA-ReportList-r16 is less than 8, then append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report: | Missing italics | pradeepa.ramachandra@ericsson.com |  |
| 19 | 2> else: 3> set the plmn-Identity to the PLMN selected by upper layers from the PLMN(s) included in the plmn-IdentityList in SIB1; | Missing italics | pradeepa.ramachandra@ericsson.com |  |
| 20 | 2> set the parameters associated to individual random-access attempt in the chronological order of attmepts in the perRAInfoList as specified in 5.3.10.3: | ‘;’ instead of ‘:’ | pradeepa.ramachandra@ericsson.com |  |
| 21 | NG-RAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message. | ‘towards’ instead of ‘to’ NG-RAN initiates the logged measurement configuration procedure towards UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message. | pradeepa.ramachandra@ericsson.com |  |
| 22 | This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC IDLE state could continue in RRC INACTIVE state or vice versa. | Missing ‘and’ This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC IDLE state and could continue in RRC INACTIVE state or vice versa. | pradeepa.ramachandra@ericsson.com |  |
| 23 | Generic comment: The terminology reportType used in loggedMeasurementConfiguration and the associated UE variable VarLogMeasConfig. | The purpose of this field is to inform the UE ‘when’ to log, not ‘what’ to log. Therefore, the field name is misleading. It is proposed to change it to loggingType instead of reportType. | pradeepa.ramachandra@ericsson.com |  |
| 24 | 3> if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig: | Remove the ‘,’ | pradeepa.ramachandra@ericsson.com |  |
| 25 | 4> set the measResultServCell to include the quantities of the last logged cell the UE was camping on; 3> else: 4> set the servCellIdentity to indicate global cell identity of the cell the UE is camping on; 4> set the measResultServCell to include the quantities of the cell the UE is camping on; | There is no IE by the name ‘measResultServCell’. Missing part of the correct field name (‘ing’). It should have been measResultServingCell’ | pradeepa.ramachandra@ericsson.com |  |
| 26 | 4> if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements (excluding the resultsSSB-Indexes IE) that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following: | No need for the text in the brackets as the field measResultNeighCells does not contain resultsSSB-Indexes | pradeepa.ramachandra@ericsson.com |  |
| 27 | LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE { traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), absoluteTimeInfo-r16 AbsoluteTimeInfo-r16, areaConfiguration-r16 AreaConfiguration-r16 OPTIONAL, --Need R plmn-IdentityList-r16 PLMN-IdentityList3-r16 OPTIONAL, --Need R bt-NameList-r16 BT-NameListConfig-r16 OPTIONAL, --Need R wlan-NameList-r16 WLAN-NameListConfig-r16 OPTIONAL, --Need R sensor-NameList-r16 Sensor-NameListConfig-r16 OPTIONAL, --Need R loggingDuration-r16 LoggingDuration-r16, reportType CHOICE { periodical LoggedPeriodicalReportConfig-r16, eventTriggered LoggedEventTriggerConfig-r16 } } | Missing ‘-r16’ | pradeepa.ramachandra@ericsson.com |  |
| 28 | eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state. | Missing italics | pradeepa.ramachandra@ericsson.com |  |
| 29 | LogMeasInfo-r16 ::= SEQUENCE { locationInfo-r16 LocationInfo-r16 OPTIONAL, relativeTimeStamp-r16 INTEGER (0..7200), servCellIdentity-r16 CGI-InfoNR-Logging-r16, measResultServingCell-r16 MeasResultServingCell-r16 OPTIONAL, measResultNeighCells-r16 SEQUENCE { measResultNeighCellListNR MeasResultListLogging2NR-r16 OPTIONAL, measResultNeighCellListEUTRA MeasResultList2EUTRA-r16 OPTIONAL }, anyCellSelectionDetected-r16 ENUMERATED {true} OPTIONAL } | Missing -r16 | pradeepa.ramachandra@ericsson.com |  |
| 30 | timeStamp Includes time stamps for the waypoints that describe planned locations for the UE. | No such field. Remove the field description completely. | pradeepa.ramachandra@ericsson.com |  |
| 31 | numberOfConnFail This field is used to indicate the number of failed connection setup attempts after radio link failure. | There is no relation to RLF. Remove ‘after radio link failure’ | pradeepa.ramachandra@ericsson.com |  |
| 32 | numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted. | No such field. Remove the field description completely. | pradeepa.ramachandra@ericsson.com |  |
| 33 | maxTxPowerReached This field is used to indicate whether or not the maximum power level was used for the last transmitted preamble. | No such field. Remove the field description completely. | pradeepa.ramachandra@ericsson.com |  |
| 34 | cellIdentity-eutra-epc, cellIdentity-eutra-5GC Unambiguously identify a cell within a PLMN and it belongs the first PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1. | Missing ‘to’ cellIdentity-eutra-epc, cellIdentity-eutra-5GC Unambiguously identify a cell within a PLMN and it belongs to the first PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1. | pradeepa.ramachandra@ericsson.com |  |
| 35 | excessDelay Indicates excess queueing delay ratio in UL, according to excess delay ratio measurement report mapping table, as defined in TS 38.314 [x5], Table 4.2.1.1.1-1. | No such field. Remove the field description completely. | pradeepa.ramachandra@ericsson.com |  |
| 36 | -- ASN1START -- TAG-ULDELAYVALUECONFIG-START UL-DelayValueConfig-r16 ::= SEQUENCE { delay-DRBlist SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity } -- TAG-ULDELAYVALUECONFIG-STOP -- ASN1STOP | Missing ‘-r16’ | pradeepa.ramachandra@ericsson.com |  |
| 37 | Sensor-NameList-r16 ::= SEQUENCE { measUncomBarPre-r16 BOOLEAN OPTIONAL, -- Need R measUeSpeed BOOLEAN OPTIONAL, -- Need R measUeOrientation BOOLEAN OPTIONAL -- Need R } -- TAG-SENSORNAMELISTCONFIG-STOP -- ASN1STOP | Missing ‘-r16’ | pradeepa.ramachandra@ericsson.com |  |
| 38 | measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined in uncompensatedBarometricPressure-r16. | There is no IE called ‘uncompensatedBarometricPressure-r16’. But it is defined in 37.355. Therefore, propose to change the field description as; measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined by uncompensatedBarometricPressure is TS 37.355 [YY]. | pradeepa.ramachandra@ericsson.com |  |
| 39 | reportType CHOICE { periodical LoggedPeriodicalReportConfig-r16, eventTriggered LoggedEventTriggerConfig-r16 } | Missing ‘-r16’ | pradeepa.ramachandra@ericsson.com |  |
| 40 | -- ASN1START -- TAG-VARLOGMEAREPORT-START VarLogMeasReport-r16 ::= SEQUENCE { absoluteTimeStamp-r16 AbsoluteTimeInfo-r16, traceReference-r16 TraceReference-r16, traceRecordingSessionRef-r16 OCTET STRING (SIZE (2)), tce-Id-r16 OCTET STRING (SIZE (1)), logMeasInfoList-r16 LogMeasInfoList-r16, plmn-IdentityList-r16 PLMN-IdentityList3-r16 } -- TAG-VARLOGMEAREPORT-STOP -- ASN1STOP | Missing ‘S’ VARLOGMEASREPORT | pradeepa.ramachandra@ericsson.com |  |
| 41 | MeasResultServingCell-r16 ::= SEQUENCE { physCellId PhysCellId OPTIONAL, resultsSSB-Cell MeasQuantityResults OPTIONAL, resultsSSB SEQUENCE{ best-ssb-Index SSB-Index, best-ssb-Results MeasQuantityResults OPTIONAL, numberOfGoodSSB INTEGER (1..maxNrofSSBs) OPTIONAL } OPTIONAL, ... } | Missing ‘-r16’ | pradeepa.ramachandra@ericsson.com |  |
| 42 | [[ simultaneousTCI-UpdateList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R simultaneousTCI-UpdateListSecond-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R simultaneousSpatial-UpdatedListSecond-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL -- Need R ]] In CellGroupConfig IE | [[ tci-RelationCellList1-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R tci-RelationCellList2-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R spatialRelationCellList1-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need R spatialRelationCellList2-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL -- Need R ]] | Helka-liina.maattanen@ericsson.com |  |
| 43 | lte-CRS-PatternList-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Cond LTE-CRS lte-CRS-PatternListSecond-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Cond CORESETPool IN servingCellConfig IE | lte-CRS-PatternList1-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Cond LTE-CRS lte-CRS-PatternList2-r16 SetupRelease { LTE-CRS-PatternList-r16 } OPTIONAL, -- Cond CORESETPool | Helka-liina.maattanen@ericsson.com |  |
| 44 | The field description of sps-ConfigList in IE BWP-DownlinkDedicated “UE specific multiple SPS (Semi-Persistent Scheduling) configurations for one BWP.” should be “UE specific one or multiple SPS (Semi-Persistent Scheduling) configurations for one BWP.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 45 | The field description of configuredGrantConfigList in IE BWP-UplinkDedicated “A list of multiple configured grant configurations for one BWP.” should be “A list of one or multiple configured grant configurations for one BWP.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 46 | The definition of the IE ConfiguredGrantConfigList: “The IE ConfiguredGrantConfigList is used to configure multiple uplink Configured Grant configurations in one BWP.” should be “The IE ConfiguredGrantConfigList is used to configure one or multiple uplink Configured Grant configurations in one BWP.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 47 | In the field description of IE ConfiguredGrantConfigList: “Indicates a list of multiple UL Configured Grant configurations to be added or modified.” should be “Indicates a list of one or multiple UL Configured Grant configurations to be added or modified.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 48 | In the field description of IE ConfiguredGrantConfigList: “Indicates a list of multiple UL Configured Grant configurations to be released.” should be “Indicates a list of one or multiple UL Configured Grant configurations to be released.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 49 | In the definition of IE SPS-ConfigList “The IE SPS-ConfigList is used to configure multiple downlink SPS configurations in one BWP.” should be “The IE SPS-ConfigList is used to configure one or multiple downlink SPS configurations in one BWP.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 50 | In the field description of IE SPS-ConfigList “Indicates a list of multiple DL SPS configurations to be added or modified“ should be “Indicates a list of one or multiple DL SPS configurations to be added or modified” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 51 | In the field description of IE SPS-ConfigList “Indicates a list of multiple DL SPS configurations to be released.” should be “Indicates a list of one or multiple DL SPS configurations to be released.” | The list can have one element and the wording “multiple” is not precise | zhenhua.zou@ericsson.com |  |
| 52 | In the BeamFailureRecoveryConfig field descriptions, for the field description of ra-prioritization and ra-PrioritizationTwoStep, the reference to 38.321 should point to clause 5.1.1a instead of 5.1.1 ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1). ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure for BFR (see TS 38.321 [3], clause 5.1.1). | ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see TS 38.321 [3], clause 5.1.1a). ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access procedure for BFR (see TS 38.321 [3], clause 5.1.1a). | eswar.vutukuri@zte.com.cn |  |
| 53 | RACH-ConfigCommonTwoStepRA information element rach-ConfigGenericTwoStepRA-r16 RACH-ConfigCommonTwoStepRA-r16, The highlighted text should have been RACH-ConfigGenericTwoStepRA-r16 | Change as follows: rach-ConfigGenericTwoStepRA-r16 RACH-ConfigCommonTwoStepRA-r16 Rach-ConfigGenericTwoStepRA-r16, | eswar.vutukuri@zte.com.cn |  |
| 54 | 5.7.10.3 3> set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report; 3> discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers; 3> discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers; | 3> set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10]; 3> discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers; | fanjiangsheng@catt.cn |  |
| 55 | 5.5a.3.2 3> if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig: 4> perform the logging at regular time intervals, as defined by the loggingInterval in the LoggedEventTriggerConfig; | 3> if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig: 4> perform the logging at regular time intervals, as defined by the loggingInterval in the periodical; | fanjiangsheng@catt.cn |  |
| 56 | 5.5a.3.2 2> else if the reportType is set to eventTriggered, which indicates outOfCoverage: 3> perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state; 2> else if the reportType is set to eventType and eventL1 is indicated: 3> perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met; | 2> else if the reportType is set to eventTriggered, which indicates outOfCoverage: 3> perform the logging at regular time intervals as defined by the loggingInterval in eventTriggered only when the UE is in any cell selection state; 2> else if the reportType is set to eventType and eventL1 is indicated: 3> perform the logging at regular time intervals as defined by the loggingInterval in eventTriggered only when the conditions indicated by the eventL1 are met; | fanjiangsheng@catt.cn |  |
| 57 | #### RadioLinkMonitoringConfig The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.1.1. | #### RadioLinkMonitoringConfig The IE RadioLinkMonitoringConfig is used to configure radio link monitoring for detection of beam- and/or cell radio link failure. See also TS 38.321 [3], clause 5.17. | fanjiangsheng@catt.cn |  |
| 58 | 5.3.5.9 1> if the received otherConfig includes the BT-NameListConfig: 2> if BT-NameListConfig is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report; 1> if the received otherConfig includes the WLAN-NameListConfg: 2> if WLAN-NameListConfg is set to setup, attempt to have WLAN measurement results available for subsequent measurement report; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation. 1> if the received otherConfig includes the Sensor-NameListConfig: 2> if Sensor-NameListConfig is set to setup, attempt to have Sensor measurement results available for subsequent measurement report; | 1> if the received otherConfig includes the bt-NameList: 2> if bt-NameList is set to setup, attempt to have Bluetooth measurement results available for subsequent measurement report; 1> if the received otherConfig includes the wlan-NameList: 2> if wlan-NameList is set to setup, attempt to have WLAN measurement results available for subsequent measurement report; NOTE 2: The UE is requested to attempt to have valid Bluetooth measurements and WLAN measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth, are up to UE implementation. 1> if the received otherConfig includes the sensor-NameList: 2> if sensor-NameList is set to setup, attempt to have Sensor measurement results available for subsequent measurement report; | fanjiangsheng@catt.cn |  |
| 59 | 5.3.5.8.3 3> if detailed location information is available, set the content of the LocationInfo as follows: 4> if available, set the commonLocationInfo to include the detailed location information; 4> if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 4> if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 4> if available, set the sensor-LocationInfo to include the sensor measurement results; | 3> if available, set the content of the locationInfo as follows: 4> if available, set the commonLocationInfo to include the detailed location information; 4> if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 4> if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 4> if available, set the sensor-LocationInfo to include the sensor measurement results; | fanjiangsheng@catt.cn |  |
| 60 | 5.3.10.3 5> if detailed location information is available, set the content of locationInfo as follows: 6> if available, set the commonLocationInfo to include the detailed location information; 6> if available, set the bt-LocationInfo in locationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 6> if available, set the wlan-LocationInfo in locationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 6> if available, set the sensor-LocationInfo in locationInfo to include the sensor measurement results; | 5> if available, set the content of locationInfo as follows: 6> if available, set the commonLocationInfo to include the detailed location information; 6> if available, set the bt-LocationInfo in locationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 6> if available, set the wlan-LocationInfo in locationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; 6> if available, set the sensor-LocationInfo in locationInfo to include the sensor measurement results; | fanjiangsheng@catt.cn |  |
| 61 | 5.5.5.1 General 1> if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows: 2> include the locationTimestamp; 2> include the locationCoordinate, if available; 2> include the velocityEstimate, if available; 2> include the locationError, if available; 2> include the locationSource, if available; 2> if available, include the gnss-TOD-msec, 1> if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs; 1> if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons; 1> if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the sensor-MeasurementInformation; 2> if available, include the sensor-MotionInformation; | 1> if the includeCommonLocationInfo is configured in the corresponding reportConfigNR for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows: 2> include the locationTimeStamp; 2> include the locationCoordinate, if available; 2> include the velocityEstimate, if available; 2> include the locationError, if available; 2> include the locationSource, if available; 2> include the gnss-TOD-msec, if available; 1> if the includeWLAN-Meas is configured in the corresponding reportConfigNR for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the wlan-LocationInfo, in order of decreasing RSSI for WLAN APs; 1> if the includeBT-Meas is configured in the corresponding reportConfigNR for this measId, set the bt-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the bt-LocationInfo, in order of decreasing RSSI for Bluetooth beacons; 1> if the includeSensor-Meas is configured in the corresponding reportConfigNR for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows: 2> if available, include the sensor-MeasurementInformation; 2> if available, include the sensor-MotionInformation; | fanjiangsheng@catt.cn |  |
| 62 | 5.5a.3.2 4> include the locationCoordinates; 3> if WLAN-NameList is included in VarLogMeasConfig: 4> if detailed WLAN measurements are available: 5> include logMeasResultListWLAN, in order of decreasing RSSI for WLAN APs; 3> if BT-NameList is included in VarLogMeasConfig: 4> if detailed Bluetooth measurements are available: 5> include logMeasResultListBT, in order of decreasing RSSI for Bluetooth beacons; 3> if Sensor-NameList is included in VarLogMeasConfig: 4> if detailed Sensor measurements are available: 5> include Sensor-LocationInfo-r16 for sensors; | 4> include the locationCoordinate; 3> if wlan-NameList is included in VarLogMeasConfig: 4> if detailed WLAN measurements are available: 5> include wlan-LocationInfo, in order of decreasing RSSI for WLAN APs; 3> if bt-NameList is included in VarLogMeasConfig: 4> if detailed Bluetooth measurements are available: 5> include bt-LocationInfo, in order of decreasing RSSI for Bluetooth beacons; 3> if sensor-NameList is included in VarLogMeasConfig: 4> if detailed Sensor measurements are available: 5> include sensor-LocationInfofor sensors; | fanjiangsheng@catt.cn |  |
| 63 | LogMeasReport field descriptions logMeasResultListBT and logMeasResultListWLAN | Remove field description for logMeasResultListBT and logMeasResultListWLAN as no related IE | fanjiangsheng@catt.cn |  |
| 64 | CommonLocationInfo information element locationTimestamp-r16 OCTET STRING OPTIONAL, Field LocationTimeStamp | locationTimeStamp-r16 OCTET STRING OPTIONAL, Field locationTimeStamp | fanjiangsheng@catt.cn |  |
| 65 | Definition of BT-NameList | Change BT-NameList to BT-NameListConfig for the whole definition | fanjiangsheng@catt.cn |  |
| 66 | Definition of LogMeasResultListWLAN Field description name | Use lowercase letters for the whole Field description name. | fanjiangsheng@catt.cn |  |
| 67 | Definition of Sensor-NameListConfig measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined in uncompensatedBarometricPressure-r16 | measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined in TS 37.355 [x1]; | fanjiangsheng@catt.cn |  |
| 68 | Definition of WLAN-NameList | Change WLAN-NameList to WLAN-NameListConfig for the whole definition | fanjiangsheng@catt.cn |  |
| 69 | RACH-ConfigDedicated information element CFRA-TwoStep-r16 ::= SEQUENCE { occasionsTwoStepRA-r16 SEQUENCE { rach-ConfigGenericTwoStepRA-r16 RACH-ConfigGeneric, … -- Cond SSB-CFRA } | The IE rach-ConfigGenericTwoStepRA-r16 definition should change to rach-ConfigGenericTwoStepRA-r16 RACH-ConfigGeneric RACH-ConfigGenericTwoStepRA, | erlin.zeng@catt.cn |  |
| 70 | In the definition of PNI-NPN identity, PNI-NPN identity: an identifier of a PNI-NPN compromising of a PLMN ID and a CAG -ID combination. | “compromising” should be “comprising” here | zhourui@catt.cn |  |
| 71 | SIB-TypeInfo ::= SEQUENCE { type ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1,... }, | “sibType10” should be defined as SIB10 is newly added for NPN | zhourui@catt.cn |  |
| 72 | In the “NPN-Identity field descriptions” CAG-Identity A CAG-ID as specified in TS 23.003 [21]. The PLMN ID and a CAG ID in the NPN-Identity identifies a PNI-NPN. | Field identifiers shall start with a lowercase letter CAG-Identity cag-Identity | zhourui@catt.cn |  |
| 73 | In the “NPN-IdentityInfoList field descriptions” NPN-IdentityInfo The NPN-IdentityInfo contains one or more NPN identities and additional information associated with those NPNs. | Field identifiers shall start with a lowercase letter NPN-IdentityInfo npn-IdentityInfo | zhourui@catt.cn |  |
| 74 | Existing text: SIBs other than SIB1 and posSIBs are carried in SystemInformation (SI) messages New text: SIBs (other than SIB1) and posSIBs are carried in SystemInformation (SI) messages | Ambiguous text. It could be misread that posSIBs are not carried in SI messages. Placing the SIB1 inside parenthesis removes ambiguity. | mani.thyagarajan@nokia.com |  |
| 75 | sl-OffsetDFN-r16 INTEGER (0..1000) OPTIONAL, -- Need R t400 ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need R in SL-ConfigCommonNR-r16 | Suffix of T400 is missing, i.e., “-r16” | qianxi.lu@oppo.com |  |
| 76 | sl-TimeResourcePSCCH Indicates the number of sumbols of PSCCH in a resource pool. | Typo of “sumbols” should be corrected as “symbols” | qianxi.lu@oppo.com |  |
| 77 | #### – SL-CBR-TxConfigList The IE SL-CBR-CommonTxConfigList indicates | The name of the IE is not aligned with the description, should be corrected as #### – SL-CBR-CommonTxConfigList The IE SL-CBR-CommonTxConfigList indicates | qianxi.lu@oppo.com |  |
| 78 | 3. NR sidelink measurement identities: A list of NR sidelink measurement identities where each NR sidelink measurement identity links one NR sidelink measurement object with one NR sidelink reporting configuration. By configuring multiple NR sidelink measurement identities, it is possible to link more than one NR sidelink measurement object to the same NR sidelink reporting configuration, as well as to link more than one NR sidelink reporting configuration to the same NR sidelink measurement object. The NR sidelink measurement identity is also included in the NR sidelink measurement report that triggered the reporting, serving as a reference to the network. | This section is for measurement and report via PC5-RRC, so the report is not to network, but the associated peer UE, so it should be corrected to 3. NR sidelink measurement identities: A list of NR sidelink measurement identities where each NR sidelink measurement identity links one NR sidelink measurement object with one NR sidelink reporting configuration. By configuring multiple NR sidelink measurement identities, it is possible to link more than one NR sidelink measurement object to the same NR sidelink reporting configuration, as well as to link more than one NR sidelink reporting configuration to the same NR sidelink measurement object. The NR sidelink measurement identity is also included in the NR sidelink measurement report that triggered the reporting, serving as a reference to the associated peer UE. | qianxi.lu@oppo.com |  |
| 79 | In 5.5.4.11, Thresh is the threshold parameter for this event (i.e. s1-Threshold as defined within reportConfigNR-SL for this event). apparently this should be c1-threshold instead of s1-threshold | Correct it to c1-threshold | qianxi.lu@oppo.com |  |
| 80 | In 5.5.4.12, Thresh is the threshold parameter for this event (i.e. v2-Threshold as defined within reportConfigNR-SL for this event). apparently this should be c2-threshold instead of v2-threshold | Correct it to c2-threshold | qianxi.lu@oppo.com |  |
| 81 | In field description of “SL-PSSCH field descriptions” sl-BetaOffsets Configure beta-offset values for the second stage SCI mapping. There is no such IE of sl-BetaOffsets, so no need for this field description | Remove this field description | qianxi.lu@oppo.com |  |
| 82 | upon indication of consistent uplink LBT failures from SCG MAC: | "; or" is missing before "1> upon indication of consistent uplink LBT failures from SCG MAC:" | jack.jang@samsung.com |  |
| 83 | cg-minDFIDelay | It should be corrected to 'cg-minDFI-Delay' ('-' is missing) | jack.jang@samsung.com |  |
| 84 | channellAccessPriority | It should be corrected to ' channelAccessPriority' (i.e. double l) | jack.jang@samsung.com |  |
| 85 | dl-DCI-triggered-UL-ChannelAccess-CPext | It should be corrected to ' dl-DCI-triggered-UL-ChannelAccess-CP-ext-r16' (i.e. to add '-' after acronym CP). | jack.jang@samsung.com |  |
| 86 | ul-dci-triggered-UL-ChannelAccess-CPext-CAPC | It should be corrected to ' ul-dci-triggered-UL-ChannelAccessCP-ext-CAPC-r16' (i.e. to remove '-' after Access and add '-' after acronym CP). | jack.jang@samsung.com |  |
| 87 | 3> if the UE is in any cell seletion state (as specificed in TS 38.304 [20]): | Spelling errors Seletion selection Specified specified | pradeepa.ramachandra@ericsson.com |  |
| 88 | In section 5.3.3.7 and 5.3.13.5 3> set perRAInfoList to indicate random access failure information as specified in 5.3.10.3; | As the cause for connection establishment/resume failure might not be due to random access procedure, the term ‘random access failure’ here is nor appropriate. We propose to change it to ‘random access procedure related’ i.e., 3> set perRAInfoList to indicate random access failure procedure related information as specified in 5.3.10.3; | pradeepa.ramachandra@ericsson.com |  |
| 89 |  |  |  |  |
| 90 |  |  |  |  |
| 91 |  |  |  |  |
| 92 |  |  |  |  |
| 93 |  |  |  |  |
| 94 |  |  |  |  |
| 95 |  |  |  |  |
| 96 |  |  |  |  |
| 97 |  |  |  |  |
| 98 |  |  |  |  |
| 99 |  |  |  |  |
| 100 |  |  |  |  |
| 101 |  |  |  |  |
| 102 |  |  |  |  |
| 103 |  |  |  |  |
| 104 |  |  |  |  |
| 105 |  |  |  |  |
| 106 |  |  |  |  |
| 107 |  |  |  |  |
| 108 |  |  |  |  |
| 109 |  |  |  |  |
| 110 |  |  |  |  |
| 111 |  |  |  |  |
| 112 |  |  |  |  |
| 113 |  |  |  |  |
| 114 |  |  |  |  |
| 115 |  |  |  |  |
| 116 |  |  |  |  |
| 117 |  |  |  |  |
| 118 |  |  |  |  |
| 119 |  |  |  |  |
| 120 |  |  |  |  |
| 121 |  |  |  |  |
| 122 |  |  |  |  |
| 123 |  |  |  |  |
| 124 |  |  |  |  |
| 125 |  |  |  |  |
| 126 |  |  |  |  |
| 127 |  |  |  |  |
| 128 |  |  |  |  |