3GPP TSG-RAN2 Meeting#110e                                Draft R2-205782

Online, 1- 12 June, 2020

Agenda Item:	7.0.1

Source: 	Samsung

Title: 	LTE Rel-16 ASN.1 Review, Class 0 and Class 1 issues

Document for:	Discussion and decision

# Guidelines

- This file is used to log LTE 36331 ASN:1 Review Class 0 and Class 1 issues.

- Trivial e.g. editorials, commas, colon, misspelling, missing/ double spaces, italics etc.

- Minor e.g. quite straightforward changes e.g. correction/ addition of specification references or sub-clauses

- Fill in the columns, see example.

  - Make sure the inserted specification text is unique, such that the location of the issue is simple to find.

  - Avoid indicating duplicated issues by checking if the concerned specification text is already reported in the table.

  - Step the file name v(x) -> v(x+1) and upload to ftp server.

- The “status” column will be filled in by the ASN.1 review moderator.

- Note that issue 83 and onwards were provided during phase 2 of the review

# Class 0 and Class 1 issues

| Issue number | Copied existing specification text. Text should be unique, so that it can be easily found in the specification. If needed, add also the new text. | Comment/description/ correction | Email address |  | Status | CR |
| --- | --- | --- | --- | --- | --- | --- |
| Ex 1 | 2> derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11]; | Missing italics. | hakan.l.palm@ericsson.com |  |  |  |
| Ex 2 | PUSCH scheduled by RAR UL grant (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1). | Incorrect reference, should be 9.2.101. | hakan.l.palm@ericsson.com |  |  |  |
| Insert issues from here |  |  |  |  |  |  |
| 1 | 2> if UE has sent RRCConnectionResumeRequest message and has not received RRCConnectionResume message: 3> reset MAC; 3> if UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.1: | Reference is incorrect i.e. should be 5.3.3.18 |  | uphuyal@qti.qualcomm.com | OK | eMTC (CR4239) |
| 2 | NOTE 2 In case of DRB reconfiguration at a DAPS HO, the reconfiguration is applied to the entities/resources for the target PCell | A colon is missing (after 2) |  | uphuyal@qti.qualcomm.com | OK | ASN.1 |
| 3 | Upon receiving SystemInformationBlockType2, the UE shall: … NOTE: upperLayerIndication is an indication to upper layers that the UE has entered a coverage area that offers 5G capabilities. 1> to upper layers either forward rlos-Enabled, if present, or otherwise indicate absence of this field; 1> if up-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if up-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE without a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE without a suspended RRC connection is configured with pur-Config: 2> release pur-Config; 2> indicate to lower layers that pur-Config is released. Upon receiving SystemInformationBlockType2-NB, the UE shall: 1> apply the configuration included in the radioResourceConfigCommon; 1> apply the defaultPagingCycle included in the radioResourceConfigCommon; 1> if SystemInformationBlockType22-NB is scheduled: 2> read and act on information sent in SystemInformationBlockType22-NB; 1> apply the specified PCCH configuration defined in 9.1.1.3. 1> if in RRC_CONNECTED and UE is configured with RLF timers and constants values received within rlf-TimersAndConstants: 2> not update its values of the timers and constants in ue-TimersAndConstants except for the value of timer T300; | section 5.2.2.9 The highlighted part should also have been captured under ‘Upon receiving SystemInformationBlockType2-NB’ but was missed during the CR implementation. It is proposed to move it at the end of the section and make it common to both as shown below: Upon receiving SystemInformationBlockType2, the UE shall: … Upon receiving SystemInformationBlockType2-NB, the UE shall: … Upon receiving Upon receiving SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), , the UE shall: 1> if up-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if up-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE with a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-5GC is not included and the UE connected to 5GC in RRC_IDLE without a suspended RRC connection is configured with pur-Config; or 1> if cp-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE without a suspended RRC connection is configured with pur-Config: 2> release pur-Config; 2> indicate to lower layers that pur-Config is released. [Qualcomm]: suggest is ok (minor typos): Upon receiving Upon receiving SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), , the UE shall: [Rapporteur] It seems better to copy and paste the yellow highlight bullets in the NB-IoT case instead of making new common part. [Rapporteur] After further checking, suggested by Huawei and Qualcomm will also be better when in future we add more common parts, so it could be OK. |  | First comment by odile.rollinger at Huawei | OK | NB-IoT (CR4287) |
| 4 | 3> for NB-IoT: 4> if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report: 5> include rlf-InfoAvailable; 4> if the UE has ANR measurements results available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB: 5> include anr-InfoAvailable; | section 5.3.3.4, ‘-NB' is missing in the variable name |  |  | OK | NB-IoT (CR4287) |
| 5 | 3> if the UE is connected to EPC: 4> if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report-NB: 5> include the rlf-InfoAvailable; 4> if the UE has ANR measurements information available in VarANR-MeasurementReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasurementReport-NB: 5> include anr-InfoAvailable; | section 5.3.7.5, add ‘the’ before anr-InfoAvailable for consistency |  |  | OK | NB-IoT (CR4287) |
| 6 | 2> if the UE is a NB-IoT UE connected to 5GC: 2> if a DRB was configured with the same pdu-Session (fullConfig): 3> associate the established DRB with corresponding included pdu-Session; 2> else if the entry of drb-ToAddModList includes pdcp-config (establishment of bearer): 3> indicate the establishment of the DRB(s) and the pdu-Session of the established DRB(s) to upper layers; | section 5.3.10.3, the bullet numbering is incorrect |  |  | OK | NB-IoT (CR4287) |
| 7 | When initiating the procedure according to 5.6.23.2, the UE shall set the contents of the PURConfigurationRequest message as follows: 1> set requestedNumOccasions to the requested number of PUR occasions requested; 1> set requestedPeriodicity to the requested periodicity between consecutive PUR occasions; 1> set requestedTBS to the requested TBS for the PUR occasion(s); 1> if UE preference is that no RRC response message is needed for acknowledging the reception of a transmission using PUR, set l1-ACK to TRUE; 1> set requestedTimeOffset to the requested time gap with respect to current time until the first PUR occasion; | section 5.6.23.3, l1-ACK is defined as ENUMERATED {true}, should be changed to ‘include l1-ACK’ |  |  | OK | NB-IoT (CR4287) |
| 8 | While the UE is in RRC_IDLE, the UE shall: 1> store the measurement results for the serving cell in measResultServCell in VarANR-MeasReport-NB; 1> while the serving cell global cell identity is the same as stored in servCellIdentity in VarANR-MeasReport-NB: 2> perform the measurements once in accordance with the following: 3> for each carrier frequency indicated by an entry in anr-CarrierList, if present, within VarANR-MeasConfig; or … 1> release the VarANR-MeasConfig. The UE may discard the ANR measurements information, i.e. release the UE variables VarANR-MeasConfig and VarANR-MeasReport, [96] hours after the configuration was received, upon power off or upon detach. | section 5.6.24.1: 'NB' is missing in VarANR-MeasConfig / VarANR-MeasReport VarANR-MeasConfig should be italics in the last bullet |  |  | OK | NB-IoT (CR4287) |
| 9 | cp-CIoT-5GS-Optimisation This field indicates if the UE is allowed to establish the connection with Control plane CIoT 5GS optimisation, see TS 24.501 [95]. up-CIoT-5GS-Optimisation This field indicates if the UE is allowed to resume the connection with User plane CIoT 5GS Optimisation, see TS24.501 [95]. | section 6.2 SystemInformationBlockType1: remove ‘this field’ and change ‘if’ to ‘whether’ to align with other field description (e.g. ims-EmergencySupport) |  |  | OK | eMTC (CR4239) |
| 10 | #### – SystemInformationBlockType27 The IE SystemInformationBlockType27 contains assistance information relevant only for inter-RAT cell selection i.e. assistance information about NB-IoT frequencies for cell selection. | section 6.3.1 SystemInformationBlockType27, remove the first ‘assistance’ |  |  | OK | eMTC (CR4239) |
| 11 | #### – SystemInformationBlockType27-NB The IE SystemInformationBlockType27-NB contains assistance information relevant only for inter-RAT cell selection i.e. assistance information about E-UTRA frequencies and/ or GERAN frequencies for cell selection. | section 6.7.3.1 SystemInformationBlockType2-NB7, remove the first ‘assistance’ |  |  | OK | NB-IoT (CR4287) |
| 12 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. rlf-InfoAvailable This field is used to indicate the availability of radio link failure related information. | section 6.7.2 'RRCConnectionReestablishmentComplete-NB, remove 'the field is used to' , this does not align with other field description |  |  | OK | NB-IoT (CR4287) |
| 13 | RRCConnectionRelease-NB-v16xy-IEs ::= SEQUENCE { resumeIdentity-r16 I-RNTI-r15 OPTIONAL, -- Need OR anr-MeasConfig-r16 ANR-MeasConfig-NB-r16 OPTIONAL, -- Need ON pur-Config-r16 CHOICE { release NULL, setup PUR-Config-NB-r16 } OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } anr-MeasConfig Configuration of the measurements to be performed by the UE in RRC_IDLE for ANR. | section 6.7.2 'RRCConnectionRelease-NB, remove field description of anr-MeasConfig as a IE type is defined |  |  | OK | NB-IoT (CR4287) |
| 14 | releaseCause The releaseCause is used to indicate the reason for releasing the RRC Connection. E-UTRAN should not set the releaseCause to loadBalancingTAURequired if the extendedWaitTime is present. The network should not set the releaseCause to loadBalancingTAURequired if the UE is connected to 5GC. | section 6.7.2 'RRCConnectionRelease-NB the two sentences can be merged together [Rapporteur] Seems good and the revised text would be: The network should not set the releaseCause to loadBalancingTAURequired if the extendedWaitTime is present and/or if the UE is connected to 5GC. |  |  | OK | NB-IoT (CR4287) |
| 15 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. rlf-InfoAvailable This field is used to indicate the availability of radio link failure related information. | section 6.7.2 RRCConnectionResumeComplete-NB remove 'the field is used to' , this does not align with other field description |  |  | OK | NB-IoT (CR4287) |
| 16 | anr-InfoAvailable This field is used to indicate the availability of ANR measurement information. | section 6.7.2 RRCConnectionResumeRequest-NB remove 'the field is used to' , this does not align with other field description |  |  | OK | NB-IoT (CR4287) |
| 17 | ng-U-DataTransfer If present, the field indicates that the NG-U data transfer as specified in TS 24.501 [95] is supported. up-CIoT-5GS-Optimisation This field indicates if the UE is allowed to resume the connection with User plane CIoT 5GS Optimisation, see TS24.501 [95]. | section 6.7.2 SystemInformationBlockType1-NB Align with other field description: Change both to ‘Indicates whether’ |  |  | OK | NB-IoT (CR4287) |
| 18 | anr-ReportReq This field is used to indicate whether the UE shall report, if available, ANR measurement results. rach-ReportReq This field is used to indicate whether the UE shall report, if available, information about the random access procedure. rlf-ReportReq This field is used to indicate whether the UE shall report, if available, information about radio link failure. | section 6.7.2 UEInformationRequest-NB: remove 'the field is used to' , this does not align with other field description |  |  | OK | NB-IoT (CR4287) |
| 19 | anr-MeasReport This field indicates the ANR measurement information. failedPCellId This field is used to indicate the PCell in which RLF is detected. initialNRSRP-Level Indicates the NRSRP level of the NPRACH resource selected for the first preamble transmission. measResultLastServCell This field refers to the last measurement results taken in the PCell, where radio link failure happened. numberOfPreamblesSent This field is used to indicate the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6]. reestablishmentCellId This field is used to indicate the cell in which the re-establishment attempt was made after connection failure. timeSinceFailure This field is used to indicate the time that elapsed since the connection failure. Value in seconds. The maximum value 172800 means 172800s or longer. | section 6.7.2 UEInformationResponse-NB: remove ‘This field’, 'the field is used to' , this does not align with other field description |  |  | OK | NB-IoT (CR4287) |
| 20 | contentionDetected This field is used to indicate that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6]. edt-Fallback Value TRUE indicates that EDT fallback indication was received from the lower layers, see TS 36.321 [6]. | section 6.7.2 UEInformationResponse-NB: contentionDetected is a Boolean. Change ‘This field is used to indicate’ to ‘Value TRUE indicates’ |  |  | OK | NB-IoT (CR4287) |
| 21 | cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to EPC, see 5.3.3.1b. cp-EDT-5GC For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to 5GC, see 5.3.3.1b. cp-PUR-EPC, cp-PUR-5GC This field indicates whether transmission using PUR is enabled in the cell for the Control Plane CIoT EPS/5GS optimisations respectively. | section 6.7.3.1 SystemInformationBlockType2-NB 'PUR same as EDT only applies to FDD. Add 'For FDD:' at the beginning of the field description [Rapporteur] It is not minor issue, it requires RIL# and can be discussed during ASN.1 review. [Rapporteur] It is covered by RIL issue H134. |  |  | NOK |  |
| 22 | gwus-ResourcePosition Indicates the position of the WUS resource corresponding to the first entry in gwus-NumGroupsList-r16 Value primary indicates that the end of the WUS resource is defined by the timeoffset value for the corresponding gap type, value secondary indicates that the end of the WUS resource is immediately before the WUS resource configured by wus-Config-r15. E-UTRAN may only configure secondary when there is only one entry exists in gwus-NumGroupsList-r16 and wus-Config-r15 is present in SystemInformationBlockType2-NB. If two entries exist in gwus-NumGroupsList-r16, the position for the second WUS resource corresponds to value secondary. | section 6.7.3.2 GWUS-Config-NB typo, remove ‘exists’ |  |  | OK | NB-IoT (CR4287) |
| 23 | gwus-TimeParameters-r16 WUS-Config-NB-r15 OPTIONAL, -- Cond No-WUS-Config-r15 No-WUS-Config-r15: The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2-NB; otherwise the field is not present, and the UE shall delete any existing value for this field. | section 6.7.3.2 GWUS-Config-NB there is no need for hyphen. Better to align with eMTC: noWUSr15 |  |  | OK | NB-IoT (CR4287) |
| 24 | pur-RNTI-r16 C-RNTI OPTIONAL, --Need ON p0-UE-NPUSCH-r16 UplinkPowerControlDedicated-NB-r13, p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1, unit dB. pur-RNTI PUR-RNTI. | section 6.7.3.2 PUR-Config-NB remove field descriptions as a IE types are defined [Rapporteur] p0-UE-NPUSCH refers the value range abd IE is removed in the latest version, so no need for this change. Just remove the field description for pur-RNTI. |  |  | OK | NB-IoT (CR4287) |
| 25 | ANR-CarrierList-NB-r16 ::= SEQUENCE (SIZE (1.. maxFreqANR-NB-r16)) OF ANR-Carrier-NB-r16 ANR-Carrier-NB-r16::= SEQUENCE { carrierFreqIndex-r16 INTEGER (1.. maxFreqANR-NB-r16), blackCellList-r16 ANR-BlackCellList-NB-r16 OPTIONAL, -- Need OP ... } ANR-BlackCellList-NB-r16 ::= SEQUENCE (SIZE (1.. maxCellBlack)) OF PhysCellId | section 6.7.3.5 ANR-MeasConfig-NB no need for space after ‘..’ |  |  | OK | NB-IoT (CR4287) |
| 26 | measResultList-r16 SEQUENCE (SIZE (1.. maxFreqANR-NB-r16)) OF ANR-MeasResult-NB-r16, | section 6.7.3.5 ANR-MeasReport-NB no need for space after ‘..’ |  | Last comment by odile.rollinger at Huawei | OK | NB-IoT (CR4287) |
| 27 | Section 5.3.12, there seems to be redundancy. Also, if the UE is “leaving RRC_INACTIVE”, then “if configured” does not make sense for rrc-InactiveConfig inside the condition. 1> else: 2> upon leaving RRC_INACTIVE: 3> discard the UE Inactive AS context; 3> release rrc-InactiveConfig, if configured; 3> discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key; 2> release rrc-InactiveConfig, if configured; | Remove the first occurrence (consistent with NR spec). 1> else: 2> upon leaving RRC_INACTIVE: 3> discard the UE Inactive AS context; 3> release rrc-InactiveConfig, if configured; 3> discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key; 2> release rrc-InactiveConfig, if configured; |  | uphuyal@qti.qualcomm.com | OK | ASN.1 |
| 28 | In section 5.3.8.3, 2> should be 3> below. 1> if the RRCConnectionRelease message includes the pur-Config: 2> if pur-Config is set to setup: 3> store or replace the PUR configuration provided by the pur-Config; 3> configure MAC in accordance with the stored pur-Config; 2> else: 3> release pur-Config, if configured; 3> discard previously stored pur-Config, if any; 2> indicate to lower layers that pur-Config is released. | 1> if the RRCConnectionRelease message includes the pur-Config: 2> if pur-Config is set to setup: 3> store or replace the PUR configuration provided by the pur-Config; 3> configure MAC in accordance with the stored pur-Config; 2> else: 3> release pur-Config, if configured; 3> discard previously stored pur-Config, if any; 23> indicate to lower layers that pur-Config is released. |  | uphuyal@qti.qualcomm.com | OK | NB-IoT (CR4287) |
| 29 | [[ configurdGrantAssistanceInfoReport-r16 BOOLEAN OPTIONAL -- Need ON ]] | Section 6.3.6 OtherConfig typo in the parameter name. Change to configuredGrantAssistanceInfoReport-r16 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 30 | 2> if the triggerType is set to event and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event: 3> remove the concerned transmission resource pool(s) from the poolsTriggeredList or poolsTriggeredListNR defined within the VarMeasReportList for this measId; | Section 5.5.4.1 poolsTriggeredListNR is missing in the if condition text add ‘or poolsTriggeredListNR' between 'the poolsTriggeredList' and 'defined' |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 31 | SidelinkUEInformationNR-r16 ::= SEQUENCE { criticalExtensions CHOICE { sidelinkUEInformationNR-r16 SidelinkUEInformationNR-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } SidelinkUEInformationNR-r16-IEs::= SEQUENCE { sidelinkUEInformationNR-r16 OCTET STRING, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } sidelinkUEInformationNR Container for the indication of NR sidelink information, this field includes the SidelinkUEInformationNR IE as specified in TS 38.331 [82]. | Section 6.2.2 SidelinkUEInformationNR sidelinkUEInformationNR-r16 is used twice as different fields, one of field name should be changed to differentiate the fields. - Change the first sidelinkUEInformationNR-r16 to sidelinkUEInfoNR-r16 - Add OPTIONAL for the second sidelinkUEInformationNR-r16 in order to support future proof. SidelinkUEInformationNR-r16 ::= SEQUENCE { criticalExtensions CHOICE { sidelinkUEInfomationNR-r16 SidelinkUEInfomationNR-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } SidelinkUEInfomationNR-r16-IEs::= SEQUENCE { sidelinkUEInformationNR-r16 OCTET STRING OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 32 | MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need OR tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need OR ... } carrierFreq Indicates the carrier frequency of pools configured for CBR measurement and reporting for NR sidelink communication. | Section 6.3.5 MeasObjectNR-SL Change carrierFreq-r15 to carrierFreq-r16 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 33 | eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING } | Section 6.3.5 ReportConfigEUTRA The description for S1 and S2 is missing in the heading text of ReportConfigEUTRA. Add the description for Event S1 and S2 in the heading text of ReportConfigEUTRA IE as follows. The E-UTRA measurement reporting events concerning CBR for NR sidelink communication are labelled SN with N equal to 1 and 2. Event S1: The NR sidelink channel busy ratio is above a threshold. Event S2: The NR sidelink channel busy ratio is below a threshold. |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 34 | – MeasObjectToAddModList MeasObjectToAddModList ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod MeasObjectToAddModListExt-r13 ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddModExt-r13 MeasObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod-v9e0 MeasObjectToAddMod ::= SEQUENCE { measObjectId MeasObjectId, measObject CHOICE { measObjectEUTRA MeasObjectEUTRA, measObjectUTRA MeasObjectUTRA, measObjectGERAN MeasObjectGERAN, measObjectCDMA2000 MeasObjectCDMA2000, ..., measObjectWLAN-r13 MeasObjectWLAN-r13, measObjectNR-r15 MeasObjectNR-r15, measObjectNR-SL-r16 MeasObjectNR-SL-r16 } } MeasObjectToAddModExt-r13 ::= SEQUENCE { measObjectId-r13 MeasObjectId-v1310, measObject-r13 CHOICE { measObjectEUTRA-r13 MeasObjectEUTRA, measObjectUTRA-r13 MeasObjectUTRA, measObjectGERAN-r13 MeasObjectGERAN, measObjectCDMA2000-r13 MeasObjectCDMA2000, ..., measObjectWLAN-v1320 MeasObjectWLAN-r13, measObjectNR-r15 MeasObjectNR-r15, measObjectNR-SL-r16 MeasObjectNR-SL-r16 } } | Section 6.3.5 MeasObjectToAddModList General comment: do we need to extend the measObjectID range (greater than 64) to support newly introduced measObjectID? [Rapporteur] Need the WI decision. [Rapporteur] It is covered by RIL S004. |  | hyunjeong.kang@samsung.com | NOK |  |
| 35 | Figure 5.10.15-1: Sidelink UE information for NR sidelink communication | Section 5.10.15 Change SIB XX2 to SIB 28 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 36 | The initiation and the procedure for the transmission of SidelinkUEInformationNR follow the procedures specified for NR sidelink communication in subclause 5.X.3 of TS 38.331 [82]. | Section 5.10.15 Change 5.X.3 to 5.8.3 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 37 | NOTE: When applying the procedure in this subclause, SystemInformationBlockType28 corresponds to SIBX specified in TS 38.331 [82]. | Section 5.10.15 Change SIBX to SIB12 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 38 | Figure 5.10.16-1: Synchronisation information transmission for NR sidelink communication, in (partial) coverage | Section 5.10.16 Change SIBXX to SIB28 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 39 | The initiation and the procedure for the transmission of sidelink SSB follow the procedure specified for NR sidelink communication in subclause 5.X.5 of TS 38.331 [82]. | Section 5.10.16 Change 5.X.5 to 5.8.5 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 40 | NOTE: When applying the procedure in this subclause, SystemInformationBlockType28 correspond to SIBX specified in TS 38.331 [82]. | Section 5.10.15 Change SIBX to SIB12 |  | hyunjeong.kang@samsung.com | OK | V2X (CR4270) |
| 41 | MeasObjectNR-SL information element MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need OR tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need OR ... } | Section 6.3.5 MeasObjectNR-SL Change the need codes of both tx-ResourcePoolToRemoveList-r16/tx-ResourcePoolToAddList-r16, i.e. from Need OR to Need ON because No action is required when this field is absent. MeasObjectNR-SL-r16 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, tx-ResourcePoolToRemoveList-r16 Tx-PoolMeasToRemoveListNR-r16 OPTIONAL, -- Need ON tx-ResourcePoolToAddList-r16 Tx-PoolMeasToAddModListNR-r16 OPTIONAL, -- Need ON ... } [Rapporteur] Need RIL #. [Rapporteur] It is covered by RIL S045 |  | hyunjeong.kang@samsung.com | NOK |  |
| 42 | ReportConfigEUTRA ::= SEQUENCE { triggerType CHOICE { event SEQUENCE { eventId CHOICE { ... eventV1-r14 SEQUENCE { v1-Threshold-r14 SL-CBR-r14 }, eventV2-r14 SEQUENCE { v2-Threshold-r14 SL-CBR-r14 }, ... eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING } }, ReportConfigEUTRA field descriptions s1-Threshold, s2-Threshold Threshold used for events s1 and s2 specified in subclauses 5.5.4.18 and 5.5.4.19, respectively. They are containers with contents being c1-Threshold IE and c2-Threshold IE respectively, as specified in TS 38.331 [82]. | Section 6.3.5 ReportConfigEUTRA The events (S1 and S2) are encoded by EUTRA but the threshold is specified by an octet string. Since EUTRA can configure both event and threshold for S1 and S2, we suggest to encode threshold without using a container. eventS1-r16 SEQUENCE { s1-Threshold-r16 OCTET STRING SL-CBR-r16 }, eventS2-r16 SEQUENCE { s2-Threshold-r16 OCTET STRING SL-CBR-r16 } }, ThresholdEUTRA-v1250 ::= CSI-RSRP-Range-r12 MeasRSSI-ReportConfig-r13 ::= SEQUENCE { channelOccupancyThreshold-r13 RSSI-Range-r13 OPTIONAL -- Need OR } SL-CBR-r16 ::= INTEGER(0..100) ReportConfigEUTRA field descriptions s1-Threshold, s2-Threshold Threshold used for events s1 and s2 specified in subclauses 5.5.4.18 and 5.5.4.19, respectively. They are containers with contents being c1-Threshold IE and c2-Threshold IE respectively, as specified in TS 38.331 [82]. These fields indicate the SL-CBR-r16. SL-CBR Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on. [Rapporteur] Need the WI decision. [Rapporteur] It is covered by RIL S046. |  | hyunjeong.kang@samsung.com | NOK |  |
| 43 | 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> if timer T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.6.20.3; | Text can be simplified/ running check not needed for release 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> stop timer T331, if running; 2> perform the actions as specified in 5.6.20.3; |  | himke.vandervelde at Samsung | OK | DCCA (CR4260) |
| 44 | 1> if the RRCConnectionRelease message includes the releaseMeasIdleConfig: 2> if timer T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.6.20.3; And RRCConnectionRelease-v16xy-IEs ::= SEQUENCE { releaseIdleMeasConfig ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } | Modify name used in ASN.1 and add suffix. Same for field description RRCConnectionRelease-v16xy-IEs ::= SEQUENCE { releaseMeasIdleConfig-r16 ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL } |  | himke.vandervelde at Samsung | OK | DCCA (CR4260) |
| 45 | 5.5.3.1: 2> if the UL-DelayValueConfig is configured for the associated reportConfig: 3> ignore the measObject; 3> configure the PDCP layer to perform UL PDCP Packet Delay value per DRB measurement; | Should reference to field name, i.e. ul-DelayValueConfig. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 46 | 5.6.1.3: Upon receiving DLInformationTransfer message, the the IAB-MT shall: | Redundant “the” can be removed. |  | hchoi5@lenovo.com | OK | IAB |
| 47 | 5.6.21.1: Figure 5.6.21.1-1: Failure information | Empty object to be removed. It overlaps with Figure 5.6.21.1-1. [Rapporteur] There is no empty object. |  | hchoi5@lenovo.com | NOK |  |
| 48 | FailureInformation-r16-IEs | Late NCE container can be added. |  | hchoi5@lenovo.com | OK | feMob |
| 49 | MCGFailureInformation-r16-IEs ::= SEQUENCE { failureReportMCG FailureReportMCG OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } FailureReportMCG ::= SEQUENCE { failureType ENUMERATED {t310-Expiry, randomAccessProblem,rlc-MaxNumRetx, spare}, measResultFreqListEUTRA MeasResultList3EUTRA-r15 OPTIONAL, measResultFreqListNR MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG OCTET STRING OPTIONAL, ... } | Suffix "-r16" is missing for the new fields and Ies. Furthermore, late NCE container can be added in the IE. |  | hchoi5@lenovo.com | OK | DCCA (CR4260) |
| 50 | PURConfigurationRequest-r16-IEs | Late NCE container can be added. [Rapporteur] It is covered by RIL H099. |  | hchoi5@lenovo.com | NOK |  |
| 51 | RRCConnectionSetupComplete-v16xy-IEs ::= SEQUENCE { rlos-Request-r16 ENUMERATED {true} OPTIONAL, cp-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, up-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, lte-M-r16 ENUMERATED {true} OPTIONAL, iab-NodeIndication ENUMERATED {true} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } | Suffix “-r16” is missing for field iab-NodeIndication. |  | hchoi5@lenovo.com | OK | IAB |
| 52 | RRCEarlyDataRequest-5GC-r16-IEs | Late NCE container can be added. [Rapporteur] NCE container seems more class 2 issue but if there are no RIL, it can be covered here. |  | hchoi5@lenovo.com | OK (check RIL) | eMTC CR4239) |
| 53 | PLMN-IdentityInfo-v16xy ::= SEQUENCE { cp-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, -- Need OR up-CIoT-5GS-Optimisation-r16 ENUMERATED {true} OPTIONAL, -- Need OR iab-support ENUMERATED {true} OPTIONAL -- Need OR } | Suffix “-r16” is missing for iab-support. |  | hchoi5@lenovo.com | OK | IAB |
| 54 | UEAssistanceInformationNR-r16-IEs | Late NCE container can be added. [Rapporteur] Already have RIL: S042. |  | hchoi5@lenovo.com | NOK |  |
| 55 | UEInformationResponse-r9-IEs: 2x RACH-Report-r9 | Suffix should be “-r16” since the IE is introduced in Rel-16. |  | hchoi5@lenovo.com | OK | eMTC (CR4239) |
| 56 | SystemInformationBlockType13-r9: [[ mbsfn-AreaInfoList-r16 MBSFN-AreaInfoList-r16 OPTIONAL ]] | Need code “Need OR” is missing for field mbsfn-AreaInfoList-r16. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 57 | SystemInformationBlockType24 field descriptions: smtc2-LP-r16 | Suffix “-r16” can be removed from field name. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 58 | SystemInformationBlockType27 field descriptions: carrierFreqNBIOT NB-IoT carrier frequency. | Name should start with uppercase letter as it is an IE and not field. [Qualcomm]: Instead of correction, the description of CarrierFreqNBIOT can be deleted as both fields of this IE as well as the list which contains this IE are well described in the table already. carrierFreqNBIOT NB-IoT carrier frequency. [Rapporteur] Remove the field description as Qualcomm mentioned. |  | hchoi5@lenovo.com | OK | eMTC (CR4239) |
| 59 | LogicalChannelConfig field descriptions: bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 36.321 [6]. Value x40 indicates bit rate multiplier 40, value x60 indicates bit rate multiplier 60 and so on. | Value x60 does not exist, but x70. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 60 | PUR-Config field descriptions: pur-RSRP-ChangeThreshold Indicates the threshold of change in serving cell RSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB and so on. When rsrp-ChangeThresh is included, if rsrp-DecreaseThresh is absent the value of rsrp-IncreaseThresh is also used for rsrp-DecreaseThresh. | rsrp-ChangeThresh does not exist, but assumption is that it shall refer to pur-RSRP-ChangeThreshold. If this is the case then it needs to be corrected to pur-RSRP-ChangeThreshold. [Qualcomm]: Agree to above comment. Further correction: given the setup/release is present, the text should be “When pur-RSRP-ChangeTheshold is set to setup rsrp-ChangeThresh is included” [Rapporteur] Accept the both comments. |  | hchoi5@lenovo.com | OK | eMTC (CR4239) |
| 61 | CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16 CondReconfigurationAddMod-r16 ::= SEQUENCE { | A "To" is missing in the IE name CondReconfigurationAddMod-r16, i.e. it should say CondReconfigurationToAddMod-r16. |  | hchoi5@lenovo.com | OK | feMob |
| 62 | ReportConfigEUTRA IE: [[ purpose-v16xy ENUMERATED {sidelinkNR} OPTIONAL -- Need ON ]] | Suffix “-v16xy” should be added to new value sidelinkNR. [Qualcomm]: I think we had similar discussion before. For ENUMERATED and CHOICE, for the first time the fields are introduced, the values do not need the vxxyy or rxy. Not sure if this value (not the field) would be considered “first time” or “extension” [Rapporteur] Purpose is an existing field, so –v16xy is appropriate (although also clear from the field name). Accept the Lenovo’s comment above. |  | hchoi5@lenovo.com | OK | V2X |
| 63 | ReportConfigEUTRA field descriptions: s1-Threshold, s2-Threshold Threshold used for events s1 and s2 specified in subclauses 5.5.4.18 and 5.5.4.19, respectively. They are containers with contents being c1-Threshold IE and c2-Threshold IE respectively, as specified in TS 38.331 [82]. | “IE” can be removed as c1-Threshold/c2-Threshold are field names. [Rapporteur] It seems the contents are IE (i.e. SL-CBR-r16 in NR RRC). If an information structure is used in multiple places, an IE should be defined. So this issue requires RIL either class 2 or class 3. [Rapporteur] It is covered by RIL B002. |  | hchoi5@lenovo.com | NOK |  |
| 64 | IE UL-DelayValueConfig: The IE UL-DelayValueConfig IE specifies the configuration of the UL PDCP Packet Delay value per DRB measurements specified in TS 38.314 [103]. | Redundant IE can be removed. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 65 | RRCConnectionResumeComplete-v16xy-IEs ::= SEQUENCE { measResultListIdle-r15 MeasResultListIdle-r15 OPTIONAL, | Suffix should be “-r16” for measResultListIdle-r15. |  | hchoi5@lenovo.com | OK | DCCA |
| 66 | UE-EUTRA-Capability-v16xy-IEs: irat-ParametersNR-r16 IRAT-ParametersNR-r16 OPTIONAL, IRAT-ParametersNR-r16 ::= SEQUENCE { nr-HO-ToEN-DC-r16 ENUMERATED {supported} OPTIONAL } | Suffix for field and IE should be “-v16xy” and OPTIONAL on parent level can be removed as IE consists of a single entry. |  | hchoi5@lenovo.com | OK | ASN.1 |
| 67 | MBSFN-AreaInfo-r16: mcch-RepetitionPeriod-r16 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, spare7}, mcch-ModificationPeriod-r16 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024, spare5}, subcarrierSpacingMBMS-r16 ENUMERATED {kHz7dot5, kHz2dot5, kHz1dot25, kHz0dot37, spare4}, | Looks quite odd why a single spare has been added for mcch-RepetitionPeriod-r16, mcch-ModificationPeriod-r16, subcarrierSpacingMBMS-r16 although the respective value range allow more spares. So, either more spares should be added to fill the entire value range or the single spares should be removed. [Rapporteur] Same problem exist in mcch-ModificationPeriod-v1430. mcch-ModificationPeriod-v1430 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, spare7} OPTIONAL -- Need OR we can modify the ModificationPeriod-v1430 as well because change will not create difference from ASN.1 encoding perspective. [Rapporteur] This seems more of a RIL issue, so should be handled in ASN.1 review i.e. class 2. [Qualcomm] It was done consistent with existing fields as follows. mcch-ModificationPeriod-v1430 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, spare7} OPTIONAL -- Need OR But fine to have spare7, spare6, spare5 all the way to spare1 if rapp prefers. At the same time, we could update v1430 also for consistency. No change in encoding in any case. [Rapporteur] It is covered by RIL B003. |  | hchoi5@lenovo.com | NOK |  |
| 68 | PURConfigurationRequest-NB-r16-IEs | Late NCE container can be added. [Rapporteur] It is covered by RIL H117. |  | hchoi5@lenovo.com | NOK |  |
| 69 | RRCEarlyDataRequest-5GC-NB-r16-IEs | Late NCE container can be added. [Rapporteur] NCE container seems more class 2 issue but if there are no RIL, it can be covered here. |  | hchoi5@lenovo.com | OK (check RIL) | NB-IoT (CR4287) |
| 70 | UEInformationRequest -NB-r16-IEs | Late NCE container can be added. [Rapporteur] It is covered by RIL H129. |  | hchoi5@lenovo.com | NOK |  |
| 71 | A.6 Protection of RRC messages (informative) | The following messages are missing in the table: -FailureInformation2 (with same setting as for FailureInformation) -SidelinkUEInformationNR (with same setting as for SidelinkUEInformation) -UEAssistanceInformationNR (with same setting as for UEAssistanceInformation) |  | hchoi5@lenovo.com | OK | ASN.1 |
| 72 | NOTE 2: If the measIdleConfig does not contain measIdleCarrierListEUTRA or measIdleCarrierListNR, UE may receive measIdleCarrierListEUTRA or measIdleCarrierListNR as specified in 5.2.2.12. | 5.2.2.12 only mentions measIdleCarrierListEUTRA, it is better to refer to 5.6.20 instead of 5.2.2.12. |  | liu.jing30@zte.com.cn | OK | DCCA |
| 73 | This procedure specifies the measurements done by a UE in RRC_IDLE or RRC_INACTIVE when it has an idel/inactive measurement configuration and the storage of the available measurements by a UE in RRC_IDLE and RRC_INACTIVE. | “idle” |  | liu.jing30@zte.com.cn | OK | DCCA |
| 74 | NOTE 6: In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.4). | Should be “5.3.9.5” for conditional reconfiguration execution. [Rapporteur] it seems the change should be “5.3.5.9.5” and NOTE 2d in 5.3.5.4 can be also applied. |  | zhang.mengjie@zte.com.cn | OK | feMob (CR4290) |
| 75 | 2> perform conditional reconfiguration evaulation, as specified in 5.3.5.9.4; | Should be “evaluation”. |  | zhang.mengjie@zte.com.cn | OK | feMob (CR4290) |
| 76 | Editor's note: FFS whether there are issues with configuration of different events (e.g. A3+A5) and how to handle the "and" of two triggering events in RRC. | This Editor’s note can be deleted since the issues have been solved. |  | zhang.mengjie@zte.com.cn | OK | feMob |
| 77 | In the context above, "the configuration" includes state variables and parameters of each radio bearer. PDCP entities associtated with RLC UM and SRB bearers are reset after the successful RRC connection re-establishment procedure according to clause 5.2 in TS 36.323 [8]. | Should be “associated”. |  | zhang.mengjie@zte.com.cn | OK | feMob |
| 78 | daps-SourceRelease Indicates that the UE shall release the resources associated with source PCell at a DAPS HO, including reconfiguration of the DAPS PDCP entity to normal PDCP. | Should be “the normal PDCP entity”. [Rapporteur] It is already treated in the WI CR. |  | zhang.mengjie@zte.com.cn | NOK | eMob |
| 79 | The IE ConditionalReconfigurationId is used to identify a conditional reconfiguration (e.g. CHO). | Prefer to change to “(i.e. conditional handover)” to align with the description in the text and other filed description. |  | zhang.mengjie@zte.com.cn | OK | feMob |
| 80 | # 1 Scope The RRC protocol is also used to configure the radio interface between an IAB node and its parent nodes [9]. | The space between “IAB” and “node” be replaced with a hyphen. [Rapporteur] There is one more IAB node in the specification so can be changed for consistency. |  | ajmal.muhammad@ericsson.com | OK | IAB |
| 81 | In the References section | Add the following reference for TS 38.472: [x] 3GPP TS 38.472: " NG-RAN; F1 signalling transport.". |  | ajmal.muhammad@ericsson.com | OK | IAB |
| 82 | ### 5.3.16 Unified Access Control #### 5.3.16.1 General The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [95] or the RRC layer. | Add the following procedural text: “This procedure does not apply to IAB-nodes.” |  | ajmal.muhammad@ericsson.com | OK | IAB |
| 83 | #### DedicatedInfoF1AP The IE DedicatedInfoF1AP is used to transfer IAB-DU specific F1AP related information between the network and the IAB Node. The carried information consists of F1AP message encapsulated in SCTP/IP or F1-C related SCTP/IP packet [TS 38.472]. The RRC layer is transparent for this information. | Replace [TS 38.472] with the reference number for TS 38.472. See issue 81 above. |  | ajmal.muhammad@ericsson.com | OK | IAB |
| 84 | In 5.5.5.1: 2> set the channelOccupancyNR to the rounded percentage of sample values which are beyond to the channelOccupancyThresholdNR within all the sample values in the reportInterval; | channelOccupancyThresholdNR |  | uphuyal@qti.qualcomm.com | OK | NR_unlic |
| 85 | 5.6.20.4 1> if intra-RAT cell selection or reselection occurs while T331 is runing: | running |  | uphuyal@qti.qualcomm.com | OK | DCCA |
| 86 | harq-Offset-r15 Indicates a HARQ subframe offset that is applied to the subframes designated as UL in the associated subrame assignment, see TS 36.213 [23]. When configured in EN-DC with LTE TDD PCell, the network ensures it does not violate the TDD configuration in SIB1, and the value range of this field is {0,1,2,5,6}. | harq-Offset-r15 Indicates a HARQ subframe offset that is applied to the subframes designated as UL in the associated subrame assignment, see TS 36.213 [23]. When configured in EN-DC with LTE TDD PCell, the network ensures it does not violate the TDD configuration in SIB1, and the value range of this field is {0, <<space>>1, <<space>>2, <<space>>5, <<space>>6}. |  | uphuyal@qti.qualcomm.com | OK | DCCA |
| 87 | #### – SSB-PositionQCL-RelationshipNR The IE SSB-PositionQCL-RelationshipNR is used to indicate the QCL relationship between SSB positions on the indicated frequency or cell (see TS 38.213 [88], clause 4.1) for NR operation with shared spectrum channel access. Value n1 corresponds to 1, value n2 corresponds to 2 and so on. -- ASN1START SSB-PositionQCL-RelationshipNR-r16 ::= ENUMERATED {n1,n2,n4,n8} -- ASN1STOP | SSB-PositionQCL-RelationshipNR<<font color black>> SSB-PositionQCL-RelationshipNR-r16 ::= ENUMERATED<<font color black>> {n1, <<space>>n2, <<space>>n4, <<space>>n8} |  | uphuyal@qti.qualcomm.com | OK | NR_unlic |
| 88 | ul-DCCH-MessageNR Includes the UL-DCCH-Message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, NR RRC UEAssistanceInformation and the NR RRC FailureInformation messages. | Italics for the following message names ul-DCCH-MessageNR Includes the UL-DCCH-Message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport, NR RRC UEAssistanceInformation and the NR RRC FailureInformation messages. |  | uphuyal@qti.qualcomm.com | OK | ASN.1 |
| 89 | Conditional presence; Explanation. CondReconfigurationAdd; The field is mandatory present if a condReconfigurationId is being added. Otherwise the field is optionally present and the UE uses the stored value for this field if it is absent.. | Following should be non italic: Conditional presence; Explanation. CondReconfigurationAdd; The field is mandatory present if a condReconfigurationId is being added. Otherwise the field is optionally present and the UE uses the stored value for this field if it is absent.. | uphuyal@qti.qualcomm.com | OK | eMob |
| 90 | PCCH-Config-NB-v16xy ::= SEQUENCE { ue-SpecificDRX-Allowed-EPC-r16 ENUMERATED {true} } | Should be deleted (in NB-IoT CR) |  | uphuyal@qti.qualcomm.com | OK | NB-IoT |
| 91 | soundingRS-UL-ConfigDedicatedAdd-r16 SoundingRS-UL-ConfigDedicatedAdd-r16 OPTIONAL, -- Need ON [[ soundingRS-UL-ConfigDedicatedAdd-r16 SoundingRS-UL-ConfigDedicatedAdd-r16 OPTIONAL, -- Need ON SoundingRS-UL-ConfigDedicatedAdd-r16 ::= CHOICE { release NULL, setup SEQUENCE { srs-ConfigIndexAp-r16 INTEGER (0..31), srs-ConfigApDCI-Format4-r16 SEQUENCE (SIZE (1..3)) OF SRS-ConfigAdd-r16 OPTIONAL, --Need ON srs-ActivateAp-r13 CHOICE { release NULL, setup SEQUENCE { srs-ConfigApDCI-Format0-r16 SRS-ConfigAdd-r16, srs-ConfigApDCI-Format1a2b2c-r16 SRS-ConfigAdd-r16 } } OPTIONAL --Need ON } } | As agreed in N018: soundingRS-UL-ConfigDedicatedAdd-r16 SetupRelease{SoundingRS-UL-ConfigDedicatedAdd-r16} OPTIONAL, -- Need ON [[ soundingRS-UL-ConfigDedicatedAdd-r16 SetupRelease{SoundingRS-UL-ConfigDedicatedAdd-r16} OPTIONAL, -- Need ON SoundingRS-UL-ConfigDedicatedAdd-r16 ::= CHOICE { release NULL, setup SEQUENCE { srs-ConfigIndexAp-r16 INTEGER (0..31), srs-ConfigApDCI-Format4-r16 SEQUENCE (SIZE (1..3)) OF SRS-ConfigAdd-r16 OPTIONAL, --Need ON srs-ActivateAp-r13 CHOICE { release NULL, setup SEQUENCE { srs-ConfigApDCI-Format0-r16 SRS-ConfigAdd-r16, srs-ConfigApDCI-Format1a2b2c-r16 SRS-ConfigAdd-r16 } } OPTIONAL --Need ON } } [Rapporteur] Changes are fine but already implemented in ASN.1 review file. |  | uphuyal@qti.qualcomm.com | OK | ASN.1 |
| 92 | In 5.5.3.1 NOTE: The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.9.4. | Note number missing NOTE xx: The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.9.4. [Rapporteur] Changes are fine but already implemented in ASN.1 review file. |  | uphuyal@qti.qualcomm.com | NoAct | eMob |
| 93 | SystemInformationBlockType2-NB field descriptions. additionalSpectrumEmission The UE requirements related to IE AdditionalSpectrumEmission are defined in TS 36.101 [42], clause 6.2.4F.. cp-EDT For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to EPC, see 5.3.3.1b.. cp-EDT-5GC For FDD: This field indicates whether the UE is allowed to initiate CP-EDT when connected to 5GC, see 5.3.3.1b.. cp-PUR-EPC, cp-PUR-5GC For FDD: This field indicates whether CP transmission using PUR is allowed in the cell when connected to 5GC, see 5.3.3.1c.. cp-PUR-EPC For FDD: This field indicates whether CP transmission using PUR is allowed in the cell when connected to EPC, see 5.3.3.1c.. | New row has been added for cp-PUR-EPC – thus cp-PUR-EPC should be removed from previous entry in the table |  | tuomas.tirronen@ericsson.com | OK | NB-IoT (latest endorsed) |
| 94 | 6.7..3.2 PhysicalConfigDedicated-NB dl- ResourceReservationConfig Configuration of downlink reserved resources for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [22]. ul- ResourceReservationConfig Configuration of uplink reserved resources for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [22]. | RAN2 has agreed to make the feature more generic so NR coexistence is only an example. Also remove extra space after ‘-‘ dl-ResourceReservationConfig Configuration of downlink reserved resources, e.g. for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [22]. ul-ResourceReservationConfig Configuration of uplink reserved resources, e.g. for NB-IoT co-existence with NR, see TS 36.211 [21], TS 36.212 [22], and TS 36.213 [22]. |  | odile.rollinger@huawei,com | OK | NB-IoT |
| 95 | 5.3.8.3 3> configure MAC in accordance with the stored pur-Config; | sentence should be deleted [Rapporteur] It seems that RIL is need for this change but it is OK to apply this change as minor correction. |  | odile.rollinger@huawei.com | OK | NB-IoT |
| 96 | 6.7.3.1 SystemInformationBlockType2-NB rai-ActivationEnh This field indicates whether the UE is allowed to report the AS Release Assistance Indication (RAI) via the DCQR and AS RAI MAC CE as specified in TS 36.321 [6] when connected to EPC. | RAI should be deleted rai-ActivationEnh This field indicates whether the UE is allowed to report the AS Release Assistance Indication (RAI) via the DCQR and AS RAI MAC CE as specified in TS 36.321 [6] when connected to EPC. |  | odile.rollinger@huawei.com | OK | NB-IoT |
| 97 | 3.2 Abbreviations | CPC is used in section 5.6.2a.1, but there is no corresponding abbreviation defined. Suggest to add the following text in section 3.2: CPC Conditional PSCell Change |  |  | OK | feMoB |
| 98 | In 6.3.2 rach-Report Indicates whether the UE supports delivery of rachReport. | Hyphen missing and should be italics. rach-Report Indicates whether the UE supports delivery of rach-Report. [Rapporteur] Can be handled in ASN.1 instead of WI CR. |  | uphuyal@qti.qualcomm.com | OK | ASN.1 |
| 99 | restoreMCG-Scells Indicates that the UE shall restore the MCG Scell configurations from the UE AS Context or UE Inactive AS Context, if configured. | Align first part with existing field descriptions. The second part is clear from procedural description (otherwise “if configured” is confusing here). restoreMCG-Scells Indicates that whether the UE shall restore the MCG SCell configurations from the UE AS Context or UE Inactive AS Context as specified in 5.3.3.4a, if configured. |  | uphuyal@qti.qualcomm.com | OK | DCCA |
| 100 | restoreSCG If included, the UE shall restore the SCG configurations from the UE AS Context or UE Inactive AS Context, if configured. | Align first part with existing field descriptions. The second part is clear from procedural description (otherwise “if configured” is confusing here).. restoreSCG If included, Indicates whether the UE shall restore the SCG configurations from the UE AS Context or UE Inactive AS Context as specified in 5.3.3.4a, if configured. |  | uphuyal@qti.qualcomm.com | OK | DCCA |
| 101 |  |  |  |  |  |  |
| 102 |  |  |  |  |  |  |
| 103 |  |  |  |  |  |  |
| 104 |  |  |  |  |  |  |
| 105 |  |  |  |  |  |  |
| 106 |  |  |  |  |  |  |
| 107 |  |  |  |  |  |  |
| 108 |  |  |  |  |  |  |
| 109 |  |  |  |  |  |  |
| 110 |  |  |  |  |  |  |
| 111 |  |  |  |  |  |  |
| 112 |  |  |  |  |  |  |
| 113 |  |  |  |  |  |  |
| 114 |  |  |  |  |  |  |
| 115 |  |  |  |  |  |  |
| 116 |  |  |  |  |  |  |
| 117 |  |  |  |  |  |  |
| 118 |  |  |  |  |  |  |
| 119 |  |  |  |  |  |  |
| 120 |  |  |  |  |  |  |
| 121 |  |  |  |  |  |  |
| 122 |  |  |  |  |  |  |
| 123 |  |  |  |  |  |  |
| 124 |  |  |  |  |  |  |
| 125 |  |  |  |  |  |  |
| 126 |  |  |  |  |  |  |
| 127 |  |  |  |  |  |  |
| 128 |  |  |  |  |  |  |