3GPP TSG-RAN WG2 Meeting #125                                                                                                   	R2-2xxxxxx

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:	CATT，[xxx]

Title:	[C113][C114][C116]UE based TA measurement handling

Agenda Item:	7.4.3.1

Document for:	Discussion and Decision

1	Introduction

In this contribution, we discuss the RIL issues on UE based TA measurement handling for LTM introduced in R18 Mobility.

2	UE based TA measurement handling

For UE based TA measurement, the RAN2 agreements are as follows,

The UE performs TA measurements for candidate cell(s) after configured by RRC R2 assumes that the exact time the UE performs TA measurement is up to UE impl (no need to specify in R2 TS)

In addition to this, UE-MeasuredTA-ID is introduced for candidate cell the current serving cell, it is used to indicate whether the UE should perform UE-based TA measurements towards an LTM candidate (i.e., UE-based TA measurements towards an LTM candidate  can be performed only if it has the same UE-MeasuredTA-ID as the current serving cell),the signalling as follows,

LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond FirstLTM-Only …… ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond LTM ... } LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId, …… ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Need M ... }

Based on above, the exact time the UE performs TA measurement is up to UE implementation in lower layer. RRC should inform the lower layers on whether it is allowed to perform UE-based TA measurements for a candidate if it has the same UE-MeasuredTA-ID as the current serving cell. Since subsequent LTM is supported, so there are two cases in which RRC should inform lower layer on this,

- Case 1: upon receiving the LTM configuration from network.

- Case 2: when current serving cell is changed (e.g., when LTM is executed successfully)

Case 1: upon receiving the LTM configuration from network.

In case 1, UE should inform lower layer if the UE-MeasuredTA-ID of a candidate is same as the ServingCellUE-MeasuredTA-ID of the initial serving cell. This case has been captured in section 5.3.5.18.3(i.e., LTM candidate configuration addition/modification) as below,

2> if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID: 3> if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID: 4> inform lower layers that UE is configured with UE-based TA measurements if an LTM cell switch is executed for this LTM candidate configuration;

However, part of the description (“if an LTM cell switch is executed for this LTM candidate configuration”) above is not suitable as UE can inform low layer once it receive the LTM, the behaviour of informing low layer should  not  wait the LTM execution. A TP is provided in Annex 1

Case 2: when current serving cell is changed (e.g., when LTM is executed successfully)

In case 2, ServingCellUE-MeasuredTA-ID will be updated to the UE-MeasuredTA-ID of the target cell(i.e.,the new serving cell).the UE-MeasuredTA-IDs of other candidates are supposed to be compared with the new  ServingCellUE-MeasuredTA-ID again. Low layer should be informed, but it is absent in the current spec, as follows,

5.3.5.18.6 LTM cell switch execution …… 2> replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3; 1> if the LTM-Candidate IE in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID: 2> replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

One may argue that it can be covered by the description (“inform lower layers that UE is configured with UE-based TA measurements if an LTM cell switch is executed for this LTM candidate configuration”) in section 5.3.5.18.3.This does not work as section 5.3.5.18.3 is not called upon LTM execution. Section 5.3.5.18.3(i.e., LTM candidate configuration addition/modification) is only called upon UE receives the LTM configuration via RRC from network.

To address the issue above, multiple solutions have been suggested by companies,

- Option 1: Specify the UE behaviours in 5.3.5.18.3 and 5.3.5.18.6

Option 1 is suggested in RIL [C113], [C114], [C116], [L005]. An example of TP is provided is in Annex 1.

- Option 2: remove the description in 5.3.5.18.3 and add the description in the field description of ltm-UE-MeasureTA-Id

Option 2 is suggested in RIL [051].

Observation 1: To address the issue how to inform lower layer that UE is configured with UE-based TA measurements for the candidate, the possible options are as follows,

- Option 1: Specify the UE behaviours in 5.3.5.18.3 and 5.3.5.18.6

- Option 2:remove the description in 5.3.5.18.3 and add the description in the field description of ltm-UE-MeasureTA-Id

Proposal 1: To inform lower layer that UE is configured with UE-based TA measurements for the candidate, in the following cases, 1）Case 1: upon receiving the LTM configuration from network，2）Case 2: When current serving cell is changed (e.g., upon LTM execution) ，RAN2 to discuss which option is selected,

- Option 1: Specify the UE behaviours in 5.3.5.18.3 and 5.3.5.18.6

- Option 2:remove the description in 5.3.5.18.3 and add the description in the field description of ltm-UE-MeasureTA-Id

- Option x:

3	Conclusion

3	Annex 1: TP for option 1

5.3.5.18.3	LTM candidate configuration addition/modification

The UE shall:

1>	for each ltm-CandidateId value in the ltm-CandidateToAddModList:

……

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that UE is configured with UE-based TA measurements;

5.3.5.18.6	LTM cell switch execution

……

2>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

2>	for each ltm-CandidateId value in the ltm-CandidateToAddModList:

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that UE is configured with UE-based TA measurements

4	Annex 2: TP for option 2

5.3.5.18.3	LTM candidate configuration addition/modification

The UE shall:

1>	for each ltm-CandidateId value in the ltm-CandidateToAddModList:

……

6.3.2	Radio resource control information elements

LTM-Candidate field descriptions

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate cell.

ltm-CandidateId This field indicates an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the configuration contained in ltm-CandidateConfig.

ltm-ConfigComplete This field indicates whether the LTM candidate configuration within ltm-CandidateConfig is a complete configuration.

ltm-DL-OrJointTCI-StateToAddModList A list of TCI states for LTM to add and/or modify.

ltm-DL-OrJointTCI-StateToReleaseList A list of TCI states for LTM to remove.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-nzp-CSI-RS-ResourceSetToAddModList A list of nzp-CSI-RS-Resources set for LTM to add and/or modify.

ltm-nzp-CSI-RS-ResourceSetToReleaseList A list of nzp-CSI-RS-Resources set for LTM to remove.

ltm-nzp-CSI-RS-ResourceToAddModList A list of nzp-CSI-RS-Resources for LTM to add and/or modify.

ltm-nzp-CSI-RS-ResourceToReleaseList A list of nzp-CSI-RS-Resources for LTM to remove.

ltm-SSB-Config This field indicates the configuration of SS/PBCH blocks to be used for L1 measurements configured with ltm-CSI-ReportConfigToAddModList in CSI-MeasConfig and for TCI states configured in other fields in LTM-Candidate.

ltm-UE-MeasuredTA-ID This field indicates whether the UE should perform UE-based TA measurements towards an LTM candidate. The UE based TA measurement for the LTM candidate cell is allowed if the value of this field is equal to the value of the ltm-ServingCellUE-MeasuredTA-ID within the current VarLTM-ServingCellUE-MeasuredTA-ID.

ltm-UL-TCI-StatesToAddModList A list of uplink TCI states for LTM to add and/or modify.

ltm-UL-TCI-StatesToReleaseList A list of uplink TCI states for LTM to remove.

pathlossReferenceRS-ToAddModList A list of Reference Signals to be used for path loss estimation for unified TCI state for LTM to add and/or modify.

pathlossReferenceRS-ToReleaseList A list of Reference Signals to be used for path loss estimation for unified TCI state for LTM to add and/or modify.

4	Annex 3???

xxx

7	Reference

[1] xxx

[2] xxx