3GPP TSG-RAN WG2 Meeting #125	R2-24xxxxxx

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:	CATT, … ?

Title:	[C606] Further discussion on CHO in EMC

Agenda Item:	7.7.3

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss RIL [C606] and [C619] as follows:

[RIL]: C606 [Delegate]: CATT (Xiao)  [WI]: NTN [Class]: 2 [Status]: ToDo [TDoc]: R2-24xxxxx [Proposed Conclusion]: v02

[Description]: Missing these two fields in ntn-Config for neighbour cells in SIB19 (thus unable to support the agreement to use ephemeris/epochTime in SIB19 for EMC CHO)

[Proposed Change]: There was the below agreement that allows the UE to use ephemeris/epochTime of the corresponding neighbour cell in SIB19 to evaluate CondEventD2.

4. If ephemeris and epochTime information for candidate CHO cell is not provided in RRC Reconfiguration, the UE may use the corresponding neighbour information from SIB19.

However, in this case, the condEventD2 evaluation procedure currently specified in 5.5.4.15a still relies on the referenceLocation2 in reportConfigNR to calculate the candidate cell’s moving reference location. This is techinically infeasible, because the NW does not know when the UE actually acquires the epochTime in SIB19, thus unable to set an associated referenceLocation2 value in dedicated signalling.To support the above agreement, movingReferenceLocation also needs to be introduced into the ntn-Config in ntn-Config for neighbour cells in SIB19.

We will bring a contribution to address this issue.

[Comments]:

This contribution, therefore, proposes solutions to resolve these RIL, in order to support related agreement correctly.

Discussion

2.1	Discussion on [C606]

The RIL [C606] corresponds to the agreement made in RAN2#124 meeting [1] as follows:

Table 1: Agreements for CHO in EMC in RAN2 #124

- For CHO in EMC a new event (e.g. condEventD2) is introduced.

- New event comprises a reference location and distance threshold for source and target cell.

- Ephemeris and epochTime information for candidate CHO cell is also provided in RRC Reconfiguration (configuring the CHO) within the configuration prepared by the source cell (outside of the new event).

- If ephemeris and epochTime information for candidate CHO cell is not provided in RRC Reconfiguration, the UE may use the corresponding neighbour information from SIB19.

It can be seen that when the ephemeris and epochTime information for candidate CHO cell is not provided in RRC dedicated signalling, the UE may use the corresponding information in SIB19. During the discussion in the last meeting, the agreement was made based on the assumption that for the distance based event, i.e. condEventD2, of a candidate EMC target cell, the NW can rely on the ephemeris and epochTime for the corresponding neighbour cell in SIB19 without need to configure them again in the event configuration, and thus only needs to configure in condEventD2 an reference location information (i.e. referenceLocation2-r18) associated with the epochTime in SIB19. So in the current spec, the reference location for neighbour cells (i.e. referenceLocation2-r18) is only included in the condEventD2 configuration, specified as mandatorily present.

condEventD2-r18 SEQUENCE { distanceThreshFromReference1-r18 INTEGER(0.. 65525), distanceThreshFromReference2-r18 INTEGER(0.. 65525), referenceLocation1-r18 ReferenceLocation-r17, referenceLocation2-r18 ReferenceLocation-r17, hysteresisLocation-r18 HysteresisLocation-r17, timeToTrigger-r18 TimeToTrigger }

However, we find that configuring the ephemerisInfo/epochTime and associated reference location in different signalling does not work, so above agreement on using SIB19 neighbour cell info for EMC CHO cannot be supported correctly via signalling design in the current Spec.

See below example in Figure 1. As illustrated above, if the epochTime and ephemerisInfo are not provided in RRC Reconfiguration for condEventD2, the UE will refer to SIB19 to obtain the neighbor information for the corresponding candidate target cell. Then for a given neighbor cell, the UE_m may obtain SIB19 at t1, and the epochTime used by UE_m is EpochTime_m; while UE_n actually obtains SIB19 at t2, and the EpochTime used by UE_n is EpochTime_n. Since the gNB cannot know the exact moment when UE_m and UE_n really acquire SIB19 successfully, it cannot know what exact epochTime value UE_m and UE_n are actually using respectively. This further leads to the problem that the NW cannot figure out the referenceLocation2 value associated with the epochTime really used by UE_n and UE_m, thus unable to configure referenceLocation2 in condEventD2 for them at all. Same problem holds for any UE, if NW intends to rely on the neighbor cell assistance information in SIB19 but provide the referenceLocation2 via dedicated signaling as per the current Spec.

Figure 1: Unawareness of actual EpochTime value acquired by each UE from SIB19 by NW

Observation 1: Using referenceLocation2 in condEvent2 but ephemerisInfo/epochTime in SIB19 for a candidate target cell with condEvent D2 does not work, as NW does not know the exact epochTime value each UE acquires from SIB19, and thus cannot figure out an associated referenceLocation2 value configured in condEventD2.

To resolve the problem and support the above agreement in Table 1 correctly for neighbor cell, it is straightforward that the moving reference location of a neighbor cell should also be broadcasted in SIB19, if the epochTime and ephemerisInfo are not provided in RRC Reconfiguration. Because for EMC, only if ephemeris, epochTime and associated moving reference location are configured in the same place, can the UE figure out the real moving reference location associated with the epochTime.

Proposal 1: To support use of ephemerisInfo and epochTime information in SIB19 for candidate CHO target cell, moving reference location (which is associated with the epochTime) is introduced for each neighbor cell indicated in SIB19.

If Proposal 1 is agreed, then the issue is how to associate the information provided in SIB19 and condEventD2 provided via dedicated signaling. One solution is to define an index for the candidate target cell, referring to the neighbor list in SIB19. The UE can get the referenceLocation, epochTime and ephemerisInfo of the candidate cell from SIB19 according to the index indicated in the associated condEventD2. Then, the UE can perform the evaluation on the candidate cells configured with condEventD2, based on the distance threshold carried by dedicated signaling, and referenceLocation/epochTime and ephemerisInfo broadcasted in SIB19. That is, in order to get the moving reference location of the candidate cell, the NW should provide either the reference location directly or the neighbor cell index referring to SIB19 in condEventD2 via dedicated signalling. So, we suggest using a Choice structure to realize this purpose.

Proposal 2: Define index in condEventD2 for candidate target cell, referring to the neighbor list in SIB19.

Proposal 3: Using a Choice signaling structure, the NW configures in condEventD2 either a referenceLocation directly or the index in P2 referring to the corresponding neighbor cell in SIB19.

The corresponding TP is provided in Annex A, corresponding to Proposal 1 ~ 3, and should be adopted by RAN2.

Proposal 4: Adopt the TP in the Annex A, if Proposal 1~3 are agreeable.

If Proposal 2 and 3 are not agreed, RAN2 needs to clarify how the ntn-Config of the neighbor cell is associated with the condEventD2 of corresponding candidate target cell.

Proposal 5: If Proposal 2 and 3 are not agreed, RAN2 clarifies how to associate neighbor cell in SIB19 and condEventD2 of the corresponding candidate cell in EMC CHO..

Conclusion

In this document, we analyse RIL issues C606 for CHO in EMC and our observations andproposals are provided below:

Observation 1: Using referenceLocation2 in condEvent2 but ephemerisInfo/epochTime in SIB19 for a candidate target cell with condEvent D2 does not work, as NW does not know the exact epochTime value each UE acquires from SIB19, and thus cannot figure out an associated referenceLocation2 value configured in condEventD2.

Proposal 1: To support use of ephemerisInfo and epochTime information in SIB19 for candidate CHO target cell, moving reference location (which is associated with the epochTime) is introduced for each neighbor cell indicated in SIB19.

Proposal 2: Define index in condEventD2 for candidate target cell, referring to the neighbor list in SIB19.

Proposal 3: Using a Choice signaling structure, the NW configures in condEventD2 either a referenceLocation directly or the index in P2 referring to the corresponding neighbor cell in SIB19.

Proposal 4: Adopt the TP in the Annex A, if Proposal 1~3 are agreeable.

Proposal 5: If Proposal 2 and 3 are not agreed, RAN2 clarifies how to associate neighbor cell in SIB19 and condEventD2 of the corresponding candidate cell in EMC CHO.

Reference

- RAN2#124 meeting report;

- Annex A: Proposed TP for P1~3 based on TS38.331 V18.0.0

### 6.3.1	System information blocks

#### –	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL,        -- Need R

ntn-NeighCellConfigList-v18xy            NTN-NeighCellConfigList-v18xy                   OPTIONAL,       -- Need R

ntn-NeighCellConfigListExt-v18xy         NTN-NeighCellConfigList-v18xy                   OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfigList-v18xy ::=        SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-v18xy

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-NeighCellConfig-v18xy ::=              SEQUENCE {

movingNeighCellReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

movingReferenceLocation Reference location of the serving cell of an NTN Earth moving system at a time reference. It is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

movingNeighCellReferenceLocation Reference location of the neighbour cell of an NTN Earth moving system at a time reference. It is used for CondEventD2 measurement evaluation. The time reference of this field is indicated by epochTime in ntn-Config of the neighbour cell. This field is excluded when determining changes in system information, i.e., changes to movingNeighCellReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch. In a TN cell, this field is only present in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies. If ntn-NeighCellConfigList-v18xy/ntn-NeighCellConfigListExt-v18xy is present, it shall contain the same number of entries, listed in the same order as in ntn-NeighCellConfigList/ ntn-NeighCellConfigListExt (without suffix).

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. This field is only present in an NTN cell.

satSwitchWithReSync Provides parameters for the target satellite required to perform satellite switch with re-synchronization. This field is only present in an NTN cell and its presence indicates that satellite switch without PCI change is supported in the cell.

t-Service Indicates the time information on when a cell provided via NTN system is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed system and feeder link switches for both NTN quasi-Earth fixed and Earth moving system. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. This field is only present in an NTN cell. The reference point for t-Service is the uplink time synchronization reference point of the cell.

satSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point. It is given in number of subframes.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

[...]

### 6.3.2	Radio resource control information elements

#### –	ReportConfigNR

[...]

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

condEventA4-r17                  SEQUENCE {

a4-Threshold-r17                 MeasTriggerQuantity,

hysteresis-r17                   Hysteresis,

timeToTrigger-r17                TimeToTrigger

},

condEventD1-r17                  SEQUENCE {

distanceThreshFromReference1-r17 INTEGER(0.. 65525),

distanceThreshFromReference2-r17 INTEGER(0.. 65525),

referenceLocation1-r17           ReferenceLocation-r17,

referenceLocation2-r17           ReferenceLocation-r17,

hysteresisLocation-r17           HysteresisLocation-r17,

timeToTrigger-r17                TimeToTrigger

},

condEventT1-r17                  SEQUENCE {

t1-Threshold-r17                 INTEGER (0..549755813887),

duration-r17                     INTEGER (1..6000)

},

condEventD2-r18                  SEQUENCE {

distanceThreshFromReference1-r18 INTEGER(0.. 65525),

distanceThreshFromReference2-r18 INTEGER(0.. 65525),

referenceLocation1-r18           ReferenceLocation-r17,

referenceLocation2-r18           CHOICE {

referenceLocation-r18            ReferenceLocation-r17,

neighCellConfigIndex-r18         INTEGER(1..maxCellNTN-r18)

},

hysteresisLocation-r18           HysteresisLocation-r17,

timeToTrigger-r18                TimeToTrigger

}

},

rsType-r16                       NR-RS-Type,

...,

[[

nesEvent-r18                     ENUMERATED {true}                                OPTIONAL    -- Need R

]]

}

[...]

CellIndividualOffsetList-r18 ::=    SEQUENCE {

physCellId-r18                      PhysCellId,

cellIndividualOffset-r18            Q-OffsetRangeList

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a4-Threshold Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a4.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 or a moving reference location determined by the UE based on referenceLocation1 or referenceLocation2. Each step represents 50m.

duration This field is used for defining the leaving condition T1-2 for conditional HO event condEventT1. Each step represents 100ms.

nesEvent Indicates the event is a NES-specific CHO event and the event is only considered to be satisfied if indication from lower layers is received indicating the applicability of NES-specific CHO event and the related entry condition(s) is fulfilled. This field can only be configured for condEventA3, condEventA4 or condEventA5.

referenceLocation1, referenceLocation2 Reference locations used for condEventD1 and condEventD2. The referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell. For candidate target cell, if neighCellConfigIndex is configured, it refers to a neighbour cell included in the ntn-NeighCellConfigList and ntn-NeighCellConfigListExt in SIB19. Value 1 indicate the first entry of ntn-NeighCellConfigList, Value 2 indicate the second entry of ntn-NeighCellConfigList, and so on. Value 5 indicate the first entry of ntn-NeighCellConfigListExt, Value 6 indicate the second entry of ntn-NeighCellConfigListExt, and so on. For the neighbour cell indexed, the movingNeighCellReferenceLocation and corresponding epoch time and satellite ephemeris included in ntn-Config is used for the evaluation of condEventD2.

t1-Threshold The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

[...]

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAdditionalRACH-r17                   INTEGER ::= 256     -- Maximum number of additional RACH configurations.

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxCandidateBandIndex-r18               INTEGER ::= 8       -- Maximum number of band entry index for MUSIM capability

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxTwoPUCCH-Grp-ConfigList-r17          INTEGER ::= 16      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config} for PUCCH cell switching

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellATG-r18                          INTEGER ::= 8       -- Maximum number of ATG neighbour cells for which assistance information is

-- provided

maxCellExcluded                         INTEGER ::= 16      -- Maximum number of NR exclude-listed cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited PCells reported

maxPSCellHistory-r17                    INTEGER ::= 16      -- Maximum number of visited PSCells across all reported PCells

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellNTN-r17                          INTEGER ::= 4       -- Maximum number of NTN neighbour cells for which assistance information is

-- provided

maxCellNTN-r18                          INTEGER ::= 8       -- Maximum number of cell index referring to NTN neighbour cells in SIB19

maxCarrierTypePairList-r16              INTEGER ::= 16      -- Maximum number of supported carrier type pair of (carrier type on which

-- CSI measurement is performed, carrier type on which CSI reporting is

-- performed) for CSI reporting cross PUCCH group

maxCellAllowed                          INTEGER ::= 16      -- Maximum number of NR allow-listed cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellExcluded                   INTEGER ::= 16      -- Maximum number of E-UTRA exclude-listed physical cell identity ranges

-- in SIB5

[...]

maxNrofBWPsInSetOfCells-r18             INTEGER ::= 16      -- Maximum number of BWPs configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling

maxLowerMSD-r18                         INTEGER ::= 256     -- Maximum number of lower MSD capability sets for a victim band

maxLowerMSDInfo-r18                     INTEGER ::= 64      -- Maximum number of lower MSD capability sets for a band combination

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP