3GPP TSG-RAN2 Meeting # 131	R2-2506585

# Bengaluru, India, 25 – 29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5395 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of R19 XR enhancements for RRC spec |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-09-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Implements the agreements achieve during R19 XR discussion |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implements the agreements achieve during R19 XR discussion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements introduced in R19 for XR cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.9, 5.3.7.2, 5.3.7.3, 5.3.13.2, 5.7.4.1, 5.7.4.2, 5.7.4.3, 6.2.2, 6.3.2, 6.3.4, 6.4, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR 1007 TS 38.306 CR 1321 TS 38.331 CR 5403 TS 38.321 CR 2102 TS 38.322 CR 0065 TS 38.323 CR 0149 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================FIRST CHANGE====================================

5.	Procedures

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxBW-PreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes minSchedulingOffsetPreferenceConfigExt:

4>	consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation, successful handover report, and successful PSCell change or addition report (if received for the associated cell group);

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if successHO-Config is set to setup:

2>	consider itself to be configured to provide the successful handover information in accordance with 5.7.10.6;

1>	else:

2>	consider itself not to be configured to provide the successful handover information.

1>	if sn-initiatedPSCellChange is not included in the received otherConfig and if the successPSCell-Config in the received otherConfig is set to setup:

2>	consider itself to be configured by the corresponding cell group to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

1>	else:

2>	consider itself not to be configured by the corresponding cell group to provide the successful PSCell change or addition information.

1>	if sn-initiatedPSCellChange is included in the received otherConfig and if the received otherConfig includes successPSCell-Config set to setup and thresholdPercentageT304-SCG is not included; or

1>	if sn-initiatedPSCellChange is included in received otherConfig and successPSCell-Config is already configured for the SCG:

2>	consider itself to be configured by the source PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7;

1>	if successPSCell-Config in the received otherConfig is set to setup and thresholdPercentageT304-SCG is included:

2>	consider itself to be configured by the target PSCell to provide the successful PSCell change or addition information in accordance with 5.7.10.7

1>	if the successPSCell-Config received in otherConfig is set to release:

2>	consider itself not to be configured by the corresponding cell group to provide the successful PSCell change or addition information.

1>	if the received otherConfig includes the ul-GapFR2-PreferenceConfig:

2>	consider itself to be configured to provide its preference on FR2 UL gap in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide its preference on FR2 UL gap;

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

1>	if the received otherConfig includes the musim-GapPriorityAssistanceConfig:

2>	consider itself to be configured to provide MUSIM assistance information for gap(s) priority in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide MUSIM assistance information for gap(s) priority;

1>	if the received otherConfig includes the musim-CapabilityRestrictionConfig:

2>	if musim-CapabilityRestrictionConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for capability restriction in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for capability restriction and stop timer T348 and T346n, if running;

1>	if the received otherConfig includes the rlm-RelaxationReportingConfig:

2>	if rlm-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of RLM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of RLM measurements and stop timer T346j associated with the cell group, if running;

1>	if the received otherConfig includes the bfd-RelaxationReportingConfig:

2>	if bfd-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of BFD measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of BFD measurements and stop timer T346k associated with the cell group, if running;

1>	if the received otherConfig includes the scg-DeactivationPreferenceConfig:

2>	if the scg-DeactivationPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its SCG deactivation preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its SCG deactivation preference and stop timer T346i, if running.

1>	if the received otherConfig includes the propDelayDiffReportConfig:

2>	if the propDelayDiffReportConfig is set to setup:

3>	consider itself to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s) in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s).

1>	if the received otherConfig includes the rrm-MeasRelaxationReportingConfig:

2>	if the rrm-MeasRelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the fulfilment of the criterion for relaxing RRM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the fulfilment of the criterion for relaxing RRM measurements.

1>	if the received otherConfig includes the multiRx-PreferenceReportingConfigFR2:

2>	if the multiRx-PreferenceReportingConfigFR2 is set to setup:

3>	consider itself to be configured to provide its preference on multi-Rx operation for FR2 in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on multi-Rx operation for FR2 and stop timer T346m, if running.

1>	if the received otherConfig includes the aerial-FlightPathAvailabilityConfig:

2>	consider itself to be configured to indicate the availability of flight path information in accordance with 5.7.4;

1>	if the received otherConfig includes the ul-TrafficInfoReportingConfig:

2>	if ul-TrafficInfoReportingConfig is set to setup:

3>	consider itself to be configured to provide UL traffic information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide UL traffic information and stop all instances of timer T346l, if running;

1>	if the received otherConfig includes n3c-RelayUE-InfoReportConfig:

2>	consider itself to be configured to report relay UE information with non-3GPP connection(s).

1>	if the received OtherConfig includes gapOccasionCancelRatioReportConfig:

2>	if gapOccasionCancelRatioReportConfig is set to setup:

3>	consider itself to be configured to provide UE's preference for gap occasion cancellation ratio in accordance with Clause 5.7.4.

2>	else:

3>	consider itself not to be configured to provide UE's preference for gap occasion cancellation ratio;

3>	stop the timer T346o, if running.

==================================NEXT CHANGE======================================

5.3.7	RRC connection re-establishment

5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release for the serving L2 U2N Relay UE indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED which is not configured with MP while T301 is not running; or

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with clause 5.3.10 while the transmission of indirect path is suspended as specified in 5.3.5.17; or

1>	if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with 5.3.10 while MP indirect path addition or change is ongoing; or

1>	if MP is configured, upon detecting sidelink radio link failure of SL indirect path by L2 U2N Remote UE, in accordance with clause 5.8.9.3, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon reception of NotificationMessageSidelink including indicationType in accordance with clause 5.8.9.10, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE, while MCG transmission (i.e. direct path) is suspended as specified in clause 5.7.3b; or

1>	if MP is configured, upon detecting the failure of N3C indirect path by N3C remote UE in accordance with clause 5.7.3c, while MCG transmission (i.e. direct path) is suspended.

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	stop timer T421, if running;

1>	if UE is not configured with attemptCondReconfig; and

1>	if UE is not configured with attemptLTM-Switch:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-GapPriorityAssistanceConfig, if configured;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

2>	release multiRx-PreferenceReportingConfigFR2, if configured, and stop timer T346m, if running;

2>	release aerial-FlightPathAvailabilityConfig, if configured;

2>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

2>	release gapOccasionCancelRatioReportConfig, if configured, and stop timer T346o, if running.

1>	release successHO-Config, if configured;

1>	release successPSCell-Config configured by the PCell, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	release sl-L2RelayUE-Config for L2 U2N relay operation, if configured;

1>	release sl-L2RemoteUE-Config for L2 U2N relay operation, if configured;

1>	release the SRAP entity for L2 U2N relay operation, if configured;

1>	release ncr-FwdConfig, if configured;

1>	if the UE is NCR-MT:

2>	indicate to NCR-Fwd to cease forwarding;

1>	if SL indirect path is configured:

2>	release cell identity and relay UE ID configured in sl-IndirectPathAddChange;

2>	indicate upper layers to trigger PC5 unicast link release of the SL indirect path;

1>	if N3C indirect path is configured:

2>	release n3c-IndirectPathAddChange;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as a N3C relay UE:

2>	release n3c-IndirectPathConfigRelay;

2> consider the non-3GPP connection is not used;

1>	if the UE is acting as L2 U2N Remote UE and MP via L2 U2N Relay UE is not configured:

2>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

3>	indicate upper layers to trigger PC5 unicast link release;

3>	perform either cell selection in accordance with the cell selection process as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else (i.e., maintain the PC5 RRC connection):

3>	consider the connected L2 U2N Relay UE as suitable and perform actions as specified in clause 5.3.7.3a;

NOTE 1:	It is up to Remote UE implementation whether to release or keep the current PC5 unicast link.

1> else:

2>	if the UE is capable of L2 U2N Remote UE:

3>	perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

NOTE 2:	For L2 U2N Remote UE, if both a suitable cell and a suitable relay are available, the UE can select either one based on its implementation.

==================================NEXT CHANGE======================================

5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-GapPriorityAssistanceConfig, if configured;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	release aerial-FlightPathAvailabilityConfig, if configured;

3>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

3>	release gapOccasionCancelRatioReportConfig, if configured, and stop timer T346o, if running.

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2a:	This procedure applies also if the UE returns to the source PCell.

NOTE 3:	A L2 U2N Relay UE may re-establish (e.g. via release and establish) the SL-RLC0 and SL-RLC1 of the connected L2 U2N Remote UE(s).

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

==================================NEXT CHANGE======================================

5.3.13	RRC connection resume

5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, upon requesting multicast reception as specified in clause 5.3.13.1d, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a, for requesting configuration for SRS for positioning, for activation of preconfigured Positioning SRS in RRC_INACTIVE, for activation of non-preconfigured Positioning SRS with type semi-persistent in RRC_INACTIVE, upon receiving RRCRelease message including resumeIndication) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging; or

1> if the resumption of the RRC connection is triggered by receiving RRCRelease message including resumeIndication; or

1>	if the resumption of the RRC connection is triggered for multicast reception as specified in clause 5.3.13.1d:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

3>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and/or in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

NOTE 0:	If there are multiple NSAGs with the same highest NAS-provided NSAG priority identified for access attempt as above, it is left to UE implementation to select the NSAG to be applied in the Random Access procedure.

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else if the resumption of the RRC connection is triggered for activation of preconfigured SRS for positioning available in srs-PosRRC-InactiveValidityAreaPreConfigList and if the UE is camped in one of the cells indicated in one of srs-PosConfigValidityArea; or

2>	if the resumption of the RRC connection is triggered due to the need for SRS for positioning configuration and no stored srs-PosRRC-InactiveValidityAreaPreConfigList for the camped cell exists; or

2>	if the resumption of the RRC connection is triggered due to activation of non-preconfigured SRS for positioning with type semi-persistent available in srs-PosRRC-InactiveValidityAreaNonPreConfig and if the UE is camped in the cells indicated in srs-PosConfigValidityArea:

3>	if an emergency service is ongoing:

4>	select '2' as the Access Category;

4>	set the resumeCause to emergency;

3>	else:

4>	set the resumeCause to srs-PosConfigOrActivationReq;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE 1:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	else if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

2>	if the resumption of the RRC connection is triggered due to cell reselection as specified in clause 5.3.13.6:

3>	if an emergency service is ongoing:

4>	select '2' as the Access Category;

4>	set the resumeCause to emergency;

3>	else:

4>	select '8' as the Access Category;

4>	set the resumeCause to srs-PosConfigOrActivationReq;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection resume triggered either by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.13.1a, or by reception of the RemoteUEInformationSidelink message containing the connectionForMP as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as resumeCause, if the same cause value in the message received from the L2 U2N Remote UE via SL-RLC0.

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default PDCP configuration as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346a, if running;

1>	release maxBW-PreferenceConfig and maxBW-PreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346b, if running;

1>	release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig and maxMIMO-LayerPreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig and minSchedulingOffsetPreferenceConfigExt for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346e, if running;

1>	release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346j, if running;

1>	release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346k, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	release wlanNameList from the UE Inactive AS context, if stored;

1>	release btNameList from the UE Inactive AS context, if stored;

1>	release sensorNameList from the UE Inactive AS context, if stored;

1>	release obtainCommonLocation from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	stop timer T346i, if running;

1>	release referenceTimePreferenceReporting from the UE Inactive AS context, if stored;

1>	release sl-AssistanceConfigNR from the UE Inactive AS context, if stored;

1>	release musim-GapAssistanceConfig from the UE Inactive AS context, if stored and stop timer T346h, if running;

1>	release musim-GapConfig from the UE Inactive AS context, if stored;

1>	release musim-GapPriorityAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-LeaveAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-CapabilityRestrictionConfig from the UE Inactive AS context, if stored and stop timer T346n, if running;

1>	release propDelayDiffReportConfig from the UE Inactive AS context, if stored;

1>	release ul-GapFR2-PreferenceConfig, if configured;

1>	release rrm-MeasRelaxationReportingConfig from the UE Inactive AS context, if stored;

1>	release multiRx-PreferenceReportingConfigFR2 if configured, and stop timer T346m, if running;

1>	release aerial-FlightPathAvailabilityConfig from the UE Inactive AS context, if stored;

1>	release ul-TrafficInfoReportingConfig from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346l, if running;

1>	release gapOccasionCancelRatioReportConfig from the UE Inactive AS context, if stored, and stop timer T346o, if running.

1>	if the UE is acting as L2 U2N Remote UE:

2>	apply the specified configuration of SL-RLC0 used for the delivery of RRC message over SRB0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ncd-SSB-RedCapInitialBWP-SDT is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored ncd-SSB-RedCapInitialBWP-SDT:

3>	release the stored ncd-SSB-RedCapInitialBWP-SDT;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	set the variable pendingRNA-Update to false;

1>	release successHO-Config from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PCell from the UE Inactive AS context, if stored;

1>	release successPSCell-Config configured by the PSCell from the UE Inactive AS context, if stored;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

==================================NEXT CHANGE======================================

5.7	Other

5.7.4	UE Assistance Information

5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

-	its delay budget report carrying desired increment/decrement in the connected mode DRX cycle length; or

-	its overheating assistance information; or

-	its IDC assistance information; or

-	its preference on DRX parameters for power saving; or

-	its preference on the maximum aggregated bandwidth for power saving; or

-	its preference on the maximum number of secondary component carriers for power saving; or

-	its preference on the maximum number of MIMO layers for power saving; or

-	its preference on the minimum scheduling offset for cross-slot scheduling for power saving; or

-	its preference on the RRC state; or

-	configured grant assistance information for NR sidelink communication; or

-	its preference in being provisioned with reference time information; or

-	its preference for FR2 UL gap; or

-	its preference to transition out of RRC_CONNECTED state for MUSIM operation; or

-	its preference on the MUSIM gaps; or

-	its preference on the MUSIM gap priority; or

-	its preference on keeping the colliding MUSIM gaps; or

-	its preference on the MUSIM temporary capability restriction; or

-	its relaxation state for RLM measurements; or

-	its relaxation state for BFD measurements; or

-	availability of data and/or signalling mapped to radio bearers which are not configured for SDT; or

-	its preference for the SCG to be deactivated; or

-	availability of uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated; or

-	change of its fulfilment status for RRM measurement relaxation criterion; or

-	service link (specified in TS 38.300 [2]) propagation delay difference between serving cell and neighbour cell(s); or

-	its preference on multi-Rx operation for FR2; or

-	availability of flight path information for Aerial UE operation; or

-	UL traffic information; or

-	the information of the relay UE(s) with which it connects via a non-3GPP connection for MP; or

-	configured grant assistance information for NR sidelink positioning; or

-	its preference for gap occasion cancellation (specified in clause 10.6 in TS 38.213 [13]) ratio.

==================================NEXT CHANGE======================================

5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information.

A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters.

A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference.

A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference.

A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference.

A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference.

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns.

A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference.

A UE capable of providing an indication of its preference in FR2 UL gap may initiate the procedure if it was configured to do so, upon detecting the need of FR2 UL gap activation/deactivation.

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for gap priority preference and/or preference to keep the colliding MUSIM gaps may initiate the procedure if it was configured to do so, upon determining it has gap priority preference information and/or it has preference to keep the colliding MUSIM gaps.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure for providing an indication of its relaxation state for BFD measurements upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state.

A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing).

A UE capable of providing its preference for SCG deactivation may initiate the procedure if it was configured to do so, upon determining that it prefers or does no more prefer the SCG to be deactivated.

A UE that has uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated shall initiate the procedure.

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

A UE capable of providing service link propagation delay difference between serving cell and neighbour cell(s) shall initiate the procedure upon being configured to do so, and upon determining that service link propagation delay difference between serving cell and a neighbour cell has changed more than threshPropDelayDiff compared with the last reported value.

A UE capable of providing an indication of its preference on multi-Rx operation for FR2 may initiate the procedure if it was configured to do so, upon detecting having a preference on multi-Rx operation for FR2 and upon change of its preference on multi-Rx operation for FR2.

A UE capable of indicating the availability of flight path information may initiate the procedure, if it was configured to do so, upon determining that an initial or updated flight path information is available.

A UE capable of providing UL traffic information shall initiate the procedure when this information is available upon being configured to do so, and upon change of UL traffic information.

A UE capable of N3C remote UE operation initiates the procedure upon being configured to report relay UE information on the available non-3GPP connection(s), and upon change of its available non-3GPP connection(s).

A UE capable of providing configured grant assistance information including SL-PRS transmission periodicity, priority, bandwidth and delay budget for NR sidelink positioning in RRC_CONNECTED may initiate the procedure.

A UE capable of providing UE's preference for gap occasion cancellation ratio initiates the procedure upon being configured to do so when the UE has a preference for gap occasion cancellation ratio, or upon change of its preference.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

1>	if configured to provide IDC assistance information based on candidateServingFreqListNR included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FDM IDC assistance information including a list of affected frequencies and/or frequency combinations;

2>	else if the current idc-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC FDM assistance information including a list of affected frequencies and/or frequency combinations;

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC enhanced FDM assistance information including a list of affected frequency ranges and/or frequency range combinations;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC enhanced FDM assistance information including a list of affected frequency ranges and/or frequency range combinations;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC TDM assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC TDM assistance information;

NOTE 1:	The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE 2:	For the frequencies or frequency range(s) on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.
For frequencies or frequency range(s) on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself.
For a non-serving frequency or frequency range(s), reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies or frequency range(s) became a serving frequency or serving frequencies or frequency range(s) then this would result in interference issues that the UE would not be able to solve by itself.

1>	if configured to provide its preference on DRX parameters of a cell group for power saving:

2>	if the UE has a preference on DRX parameters of the cell group and the UE did not transmit a UEAssistanceInformation message with drx-Preference for the cell group since it was configured to provide its preference on DRX parameters of the cell group for power saving; or

2>	if the current drx-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including drx-Preference for the cell group and timer T346a associated with the cell group is not running:

3>	start the timer T346a with the timer value set to the drx-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current drx-Preference;

1>	if configured to provide its preference on the maximum aggregated bandwidth of a cell group for power saving:

2>	if the UE has a preference on the maximum aggregated bandwidth of the cell group and the UE did not transmit a UEAssistanceInformation message with maxBW-Preference and/or maxBW-PreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum aggregated bandwidth of the cell group for power saving; or

2>	if the current maxBW-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxBW-Preference and/or maxBW-PreferenceFR2-2for the cell group and timer T346b associated with the cell group is not running:

3>	start the timer T346b with the timer value set to the maxBW-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxBW-Preference and/or maxBW-PreferenceFR2-2;

1>	if configured to provide its preference on the maximum number of secondary component carriers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of secondary component carriers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxCC-Preference for the cell group since it was configured to provide its preference on the maximum number of secondary component carriers of the cell group for power saving; or

2>	if the current maxCC-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxCC-Preference for the cell group and timer T346c associated with the cell group is not running:

3>	start the timer T346c with the timer value set to the maxCC-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxCC-Preference;

1>	if configured to provide its preference on the maximum number of MIMO layers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of MIMO layers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum number of MIMO layers of the cell group for power saving; or

2>	if the current maxMIMO-LayerPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group and timer T346d associated with the cell group is not running:

3>	start the timer T346d with the timer value set to the maxMIMO-LayerPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2;

1>	if configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving:

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling of the cell group and the UE did not transmit a UEAssistanceInformation message with minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group since it was configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving; or

2>	if the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group and timer T346e associated with the cell group is not running:

3>	start the timer T346e with the timer value set to the minSchedulingOffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt;

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink communication;

1>	if configured to provide preference in being provisioned with reference time information:

2>	if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoPreference since it was configured to provide preference; or

2>	if the UE's preference changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoPreference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide preference in being provisioned with reference time information.

1>	if configured to provide its preference on FR2 UL gap:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-GapFR2-Preference since it was configured to provide its preference on FR2 UL gap information:

3>	if the UE has a preference on FR2 UL gap activation/deactivation:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference;

2>	else if the current FR2 UL gap preference is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference.

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

1>	if configured to provide MUSIM assistance information for gap preference:

2>	if configured to provide MUSIM assistance information for gap priority preference:

3>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference since it was configured to provide MUSIM assistance information for gap preference and gap priority preference and the timer T346h is not running; or

3>	if the current musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference and the timer T346h is not running:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference;

4>	start the timer T346h with the timer value set to the musim-GapProhibitTimer.

2>	else:

3>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or

3>	if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList;

4>	start the timer T346h with the timer value set to the musim-GapProhibitTimer.

NOTE 3:	The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap.

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the UE has temporary capability restriction on the current configuration and timer T348 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T348 with the timer value set to the musim-WaitTimer.

2>	if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList or if the UE has temporary capability restriction on the maximum CC number, and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction and timer T346n is not running; or

2>	if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands and if the current measurement gap requirement information is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-NeedForGapsInfoNR or RRCReconfigurationComplete message or RRCResumeComplete message including needForGapsInfoNR:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-NeedForGapsInfoNR;

2>	if the UE has included musim-CapRestrictionInd in the RRCSetupComplete message or RRCResumeComplete or RRCReestablishmentComplete message and the temporary capability restriction is not applicable when the UE is configured to provide MUSIM assistance information for temporary capability restriction:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that there is no temporary capability restriction;

1>	if configured to provide the relaxation state of RLM measurements of a cell group and RLM measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with rlm-MeasRelaxationState since it was configured to provide the relaxation state of RLM measurements for the cell group; or

2>	if the relaxation state of RLM measurements for the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including rlm-MeasRelaxationState of the cell group and timer T346j associated with the cell group is not running:

3>	start timer T346j with the timer value set to the rlm-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of RLM measurements of the cell group;

1>	if configured to provide the relaxation state of BFD measurements of serving cells of a cell group and BFD measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with bfd-MeasRelaxationState since it was configured to provide the relaxation state of BFD measurements for the cell group; or

2>	if the relaxation state of BFD measurements in any serving cell of the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including bfd-MeasRelaxationState of the cell group and timer T346k associated with the cell group is not running:

3>	start timer T346k with the timer value set to the bfd-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of BFD measurements of serving cells of the cell group.

1>	if data and/or signalling mapped to radio bearers not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing):

2>	if the UE did not transmit a UEAssistanceInformation message with nonSDT-DataIndication since the initiation of the current resume procedure for SDT:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide nonSDT-DataIndication.

1>	if configured to provide its preference for SCG deactivation and timer T346i is not running;

2>	if the UE prefers the SCG to be deactivated and did not transmit a UEAssistanceInformation message with scg-DeactivationPreference since it was configured to provide its SCG deactivation preference; or

2>	if the UE preference for SCG deactivation is different from the last indicated scg-DeactivationPreference:

3>	start timer T346i with the timer value set to the scg-DeactivationPreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for SCG deactivation;

1>	if the SCG is deactivated, and,

1>	the UE has uplink data to send for an SCG RLC entity while the UE previously did not have any uplink data to send for any SCG RLC entity:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the UE has uplink data to send for a DRB whose DRB-Identity is not included in any RLC-BearerConfig in the CellGroupConfig associated with the MCG.

1>	if configured to send indications of RRM measurement relaxation criterion fulfilment:

2>	if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected:

3>	if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or

3>	the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled;

2>	else:

3>	if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

1>	if configured to provide service link propagation delay difference between serving cell and neighbour cell(s);

2>	if the UE did not transmit a UEAssistanceInformation message with propagationDelayDifference since it was configured to provide service link propagation delay difference between serving cell and neighbour cell(s); or

2>	for any neighbour cell in neighCellInfoList, if the service link propagation delay difference between serving cell and the neighbour cell has changed more than threshPropDelayDiff since the last transmission of the UEAssistanceInformation message including propagationDelayDifference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide service link propagation delay difference between serving cell and each neighbour cell included in the neighCellInfoList;

1>	if configured to provide its preference for multi-Rx operation and timer T346m is not running;

2>	if the UE has a preference on multi-Rx operation for FR2 and did not transmit a UEAssistanceInformation message with multiRx-PreferenceFR2 since it was configured to provide its preference on multi-Rx operation; or

2>	if the UE has a different preference on multi-Rx operation for FR2 from the last indicated multiRx-PreferenceFR2:

3>	start timer T346m with the timer value set to the multiRx-PreferenceReportingConfigFR2ProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for multi-Rx operation for FR2.

1>	if configured to indicate the availability of flight path information and the UE has (updated) flight path information available:

2>	if the UE had neither provided a flight path information nor indicated the availability of flight path information since last entering RRC_CONNECTED state; or

2>	if at least one waypoint or a timestamp corresponding to a waypoint location that was not previously provided since last entering RRC_CONNECTED state is available; or

2>	if at least one upcoming waypoint or a timestamp corresponding to a waypoint location that was previously provided since last entering RRC_CONNECTED state is to be removed; or

2>	if flightPathUpdateDistanceThr is configured and, for at least one waypoint, the 3D distance between the previously provided location and the new location is more than the distance threshold configured by flightPathUpdateDistanceThr; or

2> if flightPathUpdateTimeThr is configured and, for at least one waypoint, the time difference between the previously provided timestamp and the new timestamp, if available, is more than the time threshold configured by flightPathUpdateTimeThr:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate the availability of flight path information;

NOTE 4:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to initiate transmission of the UEAssistanceInformation message when updated flight path information is available.

1>	if configured to provide UL traffic information:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information; or

2>	if UL traffic information included in the previous UEAssistanceInformation has changed since the last transmission of the UEAssistanceInformation message containing ul-TrafficInfo for at least one QoS flow for which timer T346l is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UL traffic information.

NOTE 5:	The UE only considers burstArrivalTime to have changed when it changes relative to the periodicity of the Data Burst arrival.

1>	if configured to report relay UE information with non-3GPP connection(s):

2>	if the UE did not transmit a UEAssistanceInformation message with n3c-relayUE-InfoList since it was configured to report available relay UE information with non-3GPP connection(s); or

2>	if the UE has new available non-3GPP connection(s); or

2>	if the non-3GPP connection(s) with the reported relay UE(s) is not available:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report relay UE information with non-3GPP connection(s) included in the n3c-relayUE-InfoList;

1>	if configured to provide configured grant assistance information for NR sidelink positioning:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink positioning;

1>	if configured to provide its preference for gap occasion cancellation ratio:

2>	if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or [RIL]: N091, XR

2>	if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running:

3>	start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio.

==================================NEXT CHANGE======================================

5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2 or 5.3.5.3;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the OverheatingAssistance IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-2;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-2:

4>	include reducedMaxMIMO-LayersFR2-2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxBW-FR2-2, reducedMaxMIMO-LayersFR1, reducedMaxMIMO-LayersFR2 or reducedMaxMIMO-LayersFR2-2 in OverheatingAssistance IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC FDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA or NR-DC combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC enhanced FDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC TDM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

NOTE 1:	When sending an UEAssistanceInformation message to inform the IDC problems, the UE includes all IDC assistance information in the idc-Assistance (IDC FDM assistance information) or idc-FDM-Assistance (IDC enhanced FDM assistance information) or idc-TDM-Assistance (IDC TDM assistance information) fields respectively (rather than providing e.g. the changed part(s) of the IDC assistance information in respective fields).

NOTE 2:	Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the UEAssistanceInformation message (e.g. by not including the IDC assistance information in the idc-Assistance or idc-FDM-Assistance or idc-TDM-Assistance fields).

1>	if transmission of the UEAssistanceInformation message is initiated to provide drx-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include drx-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on DRX parameters for the cell group:

3>	if the UE has a preference for the long DRX cycle:

4>	include preferredDRX-LongCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the DRX inactivity timer:

4>	include preferredDRX-InactivityTimer in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX cycle:

4>	include preferredDRX-ShortCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX timer:

4>	include preferredDRX-ShortCycleTimer in the DRX-Preference IE and set it to the preferred value;

2>	else (if the UE has no preference on DRX parameters for the cell group):

3>	do not include preferredDRX-LongCycle, preferredDRX-InactivityTimer, preferredDRX-ShortCycle and preferredDRX-ShortCycleTimer in the DRX-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum aggregated bandwidth for the cell group:

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR1 in the cell group;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-1 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR1 and reducedMaxBW-FR2 in the MaxBW-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-PreferenceFR2-2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-PreferenceFR2-2 in the UEAssistanceInformation message;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-2 in the cell group;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-2 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxCC-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxCC-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of secondary component carriers for the cell group:

3>	include reducedMaxCCs in the MaxCC-Preference IE;

3>	set reducedCCsDL to the number of maximum SCells the UE desires to have configured in downlink in the cell group;

3>	set reducedCCsUL to the number of maximum SCells the UE desires to have configured in uplink in the cell group;

2>	else (if the UE has no preference on the maximum number of secondary component carriers for the cell group):

3>	do not include reducedMaxCCs in the MaxCC-Preference IE;

NOTE 3:	The UE can implicitly indicate a preference for NR SCG release by reporting the maximum aggregated bandwidth preference for power saving of the cell group, if configured, as zero for both FR1 and FR2, and by reporting the maximum number of secondary component carriers for power saving of the cell group, if configured, as zero for both uplink and downlink.

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO-LayerPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR1-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR1-UL to the preferred maximum number of uplink MIMO layers of each FR1 serving cell that the UE operates on in the cell group;

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-UL to the preferred maximum number of uplink MIMO layers of each FR2-1 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO LayerPreferenceFR2 2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreferenceFR2-2 in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group for FR2-2:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2 2:

4>	include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2 2 IE;

4>	set reducedMIMO-LayersFR2-2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-2 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-2-UL to the preferred maximum number of uplink MIMO layers of each FR2-2 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group:

3>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK0-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK0-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK0-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK0-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K2 (TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK2-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK2-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK2-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK2-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

2>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

3>	do not include preferredK0 and preferredK2 in the MinSchedulingOffsetPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreferenceExt of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group for FR2-2:

3>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

4>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK0-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K0 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK0-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK2-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK2-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

3>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

4>	do not include preferredK0 and preferredK2 in the minSchedulingOffsetPreferenceExt IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference in being provisioned with reference time information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference in being provisioned with reference time information:

3>	set referenceTimeInfoPreference to true;

2>	else:

3>	set referenceTimeInfoPreference to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide preference on FR2 UL gap according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for FR2 UL gap configuration:

3>	set ul-GapFR2-PatternPreference to the preferred FR2 UL gap pattern;

2>	else (if the UE has no preference for the FR2 UL gap configuration):

3>	do not include ul-GapFR2-PatternPreference in the UL-GapFR2-Preference IE.

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference, or provide MUSIM assistance information for leaving RRC_CONNECTED according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

4>	if UE has a preference for MUSIM gap priority;

5>	include the musim-GapPriorityPreferenceList the UE prefers to be configured;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has a preference to keep all colliding MUSIM gaps:

3>	include the musim-GapKeepPreference;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList, musim-GapPriorityPreferenceList and musim-GapKeepPreference in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-CapRestriction according to 5.7.4.2 or 5.3.5.3:

2>	if UE has a preference for temporary capability restriction:

3>	if UE has a preference for serving cell(s), except PCell, and/or SCG to be released:

4>	include the musim-Cell-SCG-ToRelease;

5>	set musim-CellToRelease to include the serving cell(s) the UE prefers to be released;

5>	set scg-ReleasePreference to scgReleasePreferred if the UE prefers the SCG to be released;

3>	if UE has a preference to indicate the serving cells with restricted capabilities:

4>	include the musim-CellToAffectList the UE prefers to be configured;

5>	include the musim-ServCellIndex and the musim-MIMO-Layers-DL/ musim-MIMO-Layers-UL/ musim-SupportedBandwidth-DL/ musim-SupportedBandwidth-UL for the corresponding serving cell;

3>	if UE has a preference to indicate the maximum number of CCs:

4>	include the musim-CapRestriction for the musim-MaxCC the UE prefers to be configured;

5>	include the musim-MaxCC-TotalDL/ musim-MaxCC-TotalUL/ musim-MaxCC-FR1-DL/ musim-MaxCC-FR1-UL/ musim-MaxCC-FR2-1-DL/ musim-MaxCC-FR2-2-UL/ musim-MaxCC-FR2-2-DL/ musim-MaxCC-FR2-2-UL for the corresponding maximum number of CCs;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands with capabilities restricted which comprise of the band(s) that is/are indicated in musim-CandidateBandList:

4>	include the musim-AffectededBandsList the UE prefer to be configured with capabilities restricted;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) for which capabilities are restricted;

5>	include the musim-CapabilityRestricted for the corresponding band;

3>	if UE has a preference to indicate band(s) and/or combination(s) of bands to be avoided which comprise of band(s) that is indicated in musim-CandidateBandList:

4>	include the musim-AvoidedBandsList the UE prefers not to be configured;

5>	include the musim-bandEntryIndex for each band or each band of the combination(s) to be avoided;

2>	if UE has no longer preference for temporary capability restriction indicated by musim-Cell-SCG-ToRelease, musim-CellToAffectList, musim-MaxCC, musim-AffectededBandsList and/or musim-AvoidedBandsList:

3>	do not include the corresponding temporary capability restriction preference in the musim-CapRestriction;

1>	if transmission of the UEAssistanceInformation message is initiated to provide musim-NeedForGapsInfoNR according to 5.7.4.2 or 5.3.5.3:

2>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each supported NR serving cell;

2>	if the requestedTargetBandFilterNR-r16 of NeedForGapsConfigNR is configured:

3>	for each supported NR band included in requestedTargetBandFilterNR-r16, include an entry in interFreq-needForGap and set the measurement gap requirement information for that band;

2>	else:

3>	include an entry in interFreq-needForGap and set the measurement gap requirement information for each supported NR band;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of RLM measurements of a cell group according to 5.7.4.2:

2>	if the UE performs RLM measurement relaxation on the cell group according to TS 38.133 [14]:

3>	set the rlm-MeasRelaxationState to true;

2>	else:

3>	set the rlm-MeasRelaxationState to false;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of BFD measurements of a cell group:

2>	for each serving cell of the cell group:

3>	if the UE performs BFD measurement relaxation on this serving cell according to TS 38.133 [14]:

4>	set the n-th bit of bfd-MeasRelaxationState to '1', where n is equal to the servCellIndex value + 1 of the serving cell;

3>	else:

4>	set the n-th bit of bfd-MeasRelaxationState to '0', where n is equal to the servCellIndex value + 1 of the serving cell.

1>	if transmission of the UEAssistanceInformation message is initiated to indicate availability of data mapped to radio bearers not configured for SDT according to 5.7.4.2:

2>	include the nonSDT-DataIndication in the UEAssistanceInformation message;

2>	include and set the resumeCause according to the information received from the upper layers, if provided.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference for SCG deactivation according to 5.7.4.2:

2>	include scg-DeactivationPreference in the UEAssistanceInformation message;

2>	set the scg-DeactivationPreference to scg-DeactivationPreferred if the UE prefers the SCG to be deactivated, otherwise set it to noPreference;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication that the UE has uplink data related to a deactivated SCG according to 5.7.4.2:

2>	include uplinkData in the UEAssistanceInformation message.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication about whether the criterion for RRM relaxation for connected mode is fulfilled or not fulfilled:

2>	if the criterion for RRM measurement relaxation for connected mode is fulfilled:

3>	set the rrm-MeasRelaxationFulfilment to true;

2>	else:

3>	set the rrm-MeasRelaxationFulfilment to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide the service link propagation delay difference between serving cell and neighbour cell(s) according to 5.7.4.2;

2>	include the propagationDelayDifference for each neighbour cell in the neighCellInfoList;

1>	if transmission of the UEAssistanceInformation message is initiated to provide preference on multi-Rx operation for FR2 according to 5.7.4.2:

2>	if the UE has a preference for not operating on multi-Rx (i.e. not supporting simultaneous reception with different QCL-typeD) for FR2:

3>	set multiRx-PreferenceFR2 to single;

2>	else (if the UE has the preference for operating on multi-Rx for FR2):

3>	set multiRx-PreferenceFR2 to multiple.

1>	if transmission of the UEAssistanceInformation message is initiated to indicate the availability of flight path information according to 5.7.4.2 or 5.3.5.3;

2>	include the flightPathInfoAvailable;

1>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.7.4.2 or 5.3.5.3:

2>	for each PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

3>	set pdu-SessionID to the value of the concerned PDU session ID;

3>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.3.5.3:

4>	stop timer T346l for each QoS flow of this PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message;

3>	for each QoS flow of this PDU session for which timer T346l is not running and for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

4>	start timer T346l associated to this QoS flow with the timer value set to the value of ul-TrafficInfoProhibitTimer;

4>	set qfi to the value of the concerned QFI;

4>	if the jitter range measurement is available; and

4>	if the UE did not provide jitter range since it was configured to provide UL traffic information, or if the measured jitter range has changed since the last transmission of the UEAssistanceInformation message containing jitterRange:

5>	set jitterRange to the latest measured value of the jitter range;

4>	if the burst arrival time measurement is available; and

4>	if the UE did not provide burst arrival time since it was configured to provide UL traffic information, or if the measured burst arrival time has changed since the last transmission of the UEAssistanceInformation message containing burstArrivalTime:

5>	set burstArrivalTime to the latest measured value of the burst arrival time;

4>	if the traffic periodicity measurement is available; and

4>	if the UE did not provide traffic periodicity since it was configured to provide UL traffic information, or if the measured traffic periodicity has changed since the last transmission of the UEAssistanceInformation message containing trafficPeriodicity:

5>	set trafficPeriodicity to the latest measured value of the traffic periodicity;

4>	if the UE did not provide pdu-SetIdentification since it was configured to provide UL traffic information, or if the information previously provided in pdu-SetIdentification has changed since the last transmission of the UEAssistanceInformation message containing pdu-SetIdentification:

5>	if the UE is able to identify PDU Set(s) for the QoS flow:

6>	set pdu-SetIdentification to true;

5>	else:

6>	set pdu-SetIdentification to false.

4>	if the UE did not provide psi-Identification since it was configured to provide UL traffic information, or if the information previously provided in psi-Identification has changed since the last transmission of the UEAssistanceInformation message containing psi-Identification:

5>	if the UE is able to identify PSI(s) for the QoS flow:

6>	set psi-Identification to true;

5>	else:

6>	set psi-Identification to false.

1>	if transmission of the UEAssistanceInformation message is initiated to report relay UE information with non-3GPP connection(s) according to 5.7.4.2:

2>	include n3c-relayUE-InfoList in the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE's preference for gap occasion cancellation ratio according to 5.7.4.2:

2>	if gapUE is configured under measGapConfig; and

2>	if the UE has the preference for gap occasion cancellation ratio for per UE gap:

3>	set perUE to the preferred gap occasion cancellation ratio;

2>	if gapFR1 or gapFR2 is configured under measGapConfig:

3>	if the UE has the preference for gap occasion cancellation ratio for FR1:

4>	set fr1 to the prefered gap occasion cancellation ratio;

3>	if the UE has the preference for gap occasion cancellation ratio for FR2

3>	set fr2 to the preferred gap occasion cancellation ratio;

2>	if gapToAddModList is configured under measGapConfig:

3>	include the preferred gap occasion cancellation ratio for each measurement gap configuration with measGapId for which the UE has preferred gap occasion cancellation ratio.

The UE shall set the contents of the UEAssistanceInformation message for configured grant assistance information for NR sidelink communication or NR sidelink positioning:

1>	if configured to provide configured grant assistance information for NR sidelink:

2>	include the sl-UE-AssistanceInformationNR;

1>	if configured to provide configured grant assistance information for NR sidelink positioning:

2>	include the sl-PRS-UE-AssistanceInformationNR;

NOTE 4:	It is up to UE implementation when and how to trigger configured grant assistance information for NR sidelink communication or NR sidelink positioning.

The UE shall:

1>	if the procedure was triggered to provide configured grant assistance information for NR sidelink communication by an NR RRCReconfiguration message that was embedded within an E-UTRA RRCConnectionReconfiguration:

2>	submit the UEAssistanceInformation to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else if the procedure was triggered to provide UE preference for SCG deactivation or to indicate that the UE with a deactivate SCG has uplink data to send on a DRB for which there is no MCG RLC bearer:

2>	submit the UEAssistanceInformation via SRB1 to lower layers for transmission;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the UE assistance configuration that triggered this UE assistance information is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

3>	else:

4>	submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

=================================================NEXT CHANGE================================================================

6.2	RRC messages

6.2.2	Message definitions

–	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING (CONTAINING RRCReconfiguration-v15t0-IEs)                 OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

-- Regular non-critical extensions:

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v1700-IEs                                         OPTIONAL

}

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need S

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1800-IEs                                   OPTIONAL

}

RRCReconfiguration-v1800-IEs ::=        SEQUENCE {

needForInterruptionConfigNR-r18         ENUMERATED { disabled, enabled }                                   OPTIONAL, -- Need M

aerial-Config-r18                           SetupRelease { Aerial-Config-r18 }                             OPTIONAL, -- Need M

sl-IndirectPathAddChange-r18                SetupRelease { SL-IndirectPathAddChange-r18 }                  OPTIONAL, -- Need M

n3c-IndirectPathAddChange-r18               SetupRelease { N3C-IndirectPathAddChange-r18 }                 OPTIONAL, -- Need M

n3c-IndirectPathConfigRelay-r18             SetupRelease { N3C-IndirectPathConfigRelay-r18 }               OPTIONAL, -- Need M

otherConfig-v1800                           OtherConfig-v1800                                              OPTIONAL, -- Need M

srs-PosResourceSetAggBW-CombinationList-r18 SetupRelease { SRS-PosResourceSetAggBW-CombinationList-r18 }   OPTIONAL, -- Need M

ltm-Config-r18                              SetupRelease {LTM-Config-r18}                                  OPTIONAL, -- Need M

nonCriticalExtension                        RRCReconfiguration-v1830-IEs                                   OPTIONAL

}

RRCReconfiguration-v1830-IEs ::=        SEQUENCE {

otherConfig-v1830                       OtherConfig-v1830                                                  OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v19xy-IEs                            OPTIONAL

}

RRCReconfiguration-v19xy-IEs ::=       SEQUENCE {

otherConfig-v19xy                       OtherConfig-v19xy                                                  OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE{}                                                         OPTIONAL

}

-- Late non-critical Rel-15 extensions:

RRCReconfiguration-v15t0-IEs ::=        SEQUENCE {

-- Following field is only to be used for late REL-15 extensions

lateNonCriticalExtension                OCTET STRING                                                       OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v16k0-IEs                                       OPTIONAL

}

RRCReconfiguration-v16k0-IEs ::=        SEQUENCE {

sl-ConfigDedicatedNR-v16k0              SetupRelease {SL-ConfigDedicatedNR-v16k0}                          OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE{}                                                         OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

UE-TxTEG-RequestUL-TDOA-Config-r17 ::=  CHOICE {

oneShot-r17                             NULL,

periodicReporting-r17                   ENUMERATED { ms160, ms320, ms1280, ms2560, ms61440, ms81920, ms368640, ms737280 }

}

SRS-PosResourceSetAggBW-CombinationList-r18 ::= SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResSetComb-r18)) OF SRS-PosResourceSetLinkedForAggBW-List-r18

SRS-PosResourceSetLinkedForAggBW-List-r18 ::= SEQUENCE (SIZE(2..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access or if sl-L2RemoteUE-Config-r17 is configured or not released.

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured for the cell group to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover, conditional PSCell addition or conditional PSCell change. The field is absent if any DAPS bearer is configured, if the sl-L2RemoteUE-Config or sl-L2RelayUE-Config is configured, or if the RRCReconfiguration message is contained within condRRCReconfig. When the masterCellGroup and/or secondaryCellGroup includes ReconfigurationWithSync, if this field is present, it only includes configurations/fields specific to subsequent CPAC. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change or for conditional PSCell addition. The network does not include this field in an RRCReconfiguration message contained within a LTM-Config IE.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPagingDelivery This field is used to transfer Paging message for the associated L2 U2N Remote UE to the L2 U2N Relay UE in RRC_CONNECTED.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE (including L2 U2N Remote UE). The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB20, SIB21, SIB25 to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel either on the MCG or on the SCG depending on whether the MN or the SN configures this field.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

ltm-Config The network does not configure this field in an RRCReconfiguration message within an LTM-Config IE and ConditionalReconfiguration IE.

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration, ltm-Config, measConfig, bap-Config, IAB-IP-AddressConfigurationList and appLayerMeasConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

mrdc-SecondaryCellGroupConfig This field is used to configure and release an SCG in NR-DC and NE-DC. In an RRCReconfiguration message within an LTM-Config IE associated with the MCG, if this field is present its value can only be set to release.

musim-GapConfig Indicates the MUSIM gap configuration and controls setup/release of MUSIM gaps. In this version of the specification, the network does not configure MUSIM gap together preconfigured measurement gap for positioning. For the UE supporting musim-GapPriorityPreference, the network can configure MUSIM gap together with concurrent measurement gap. Otherwise, the network does not configure MUSIM gap together with concurrent measurement gap.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForInterruptionConfigNR Indicates whether the UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. The network sets this field to enabled only if the needForGapsConfigNR is configured. The network sets this field to disabled if the needForGapsConfigNR is released.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandSIB-Request Indicates that the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList, obtainCommonLocation, idc-AssistanceConfig, multiRx-PreferenceReportingConfigFR2, ul-TrafficInfoReportingConfig, n3c-RelayUE-InfoReportConfig, successPSCell-Config, sn-InitiatedPSCellChange and gapOccasionCancelRatioReportConfig can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP. In (NG)EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3. SRB4 should not be configured if sl-L2RemoteUE-Config-r17 is configured or not released.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

scg-State Indicates that the SCG is in deactivated state. This field is not used - in an RRCReconfiguration message received: - within mrdc-SecondaryCellGroup, or - in an E-UTRA RRCConnectionReconfiguration message, or - in an E-UTRA RRCConnectionResume message or - in an RRCReconfiguration message received via SRB3, except if the RRCReconfiguration message is included in DLInformationTransferMRDC. The field is absent if CPA, CPC, or subsequent CPAC is configured for the UE, or if the RRCReconfiguration message is contained in CondRRCReconfig, or PSCell is configured with tag2, or if the RRCReconfiguration message is included within an LTM-Config IE.

sl-L2RelayUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Relay UE or L2 U2U relay operation related configuration used by a UE acting as a L2 U2U Relay UE. In case of L2 U2N relay operation, the field is absent if conditionalReconfiguration is configured for CHO.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Remote UE or L2 U2U relay operation related configuration used by a UE acting as a L2 U2U Remote UE. In case of L2 U2N relay operation, the field is absent if conditionalReconfiguration is configured for CHO, or if appLayerMeasConfig or SRB4 is configured/not released.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary, or if the RRCReconfiguration message is contained in condRRCReconfig for subsequent CPAC.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery/positioning.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

srs-PosResourceSetLinkedForAggBWList This field indicates the SRS resource sets across two or three carriers which are linked for SRS bandwidth aggregation in RRC_CONNECTED state as defined in clause 6.2.1.4 of TS 38.214 [19].

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

ue-TxTEG-RequestUL-TDOA-Config Configures the periodicity of UE reporting for the association between Tx TEG and SRS Positioning resources. When configured with oneShot UE reports the association only one time. When configured with periodicReporting UE reports the association periodically and the periodicReporting indicates the periodicity. Value ms160 corresponds to 160ms, value ms320 corresponds to 320ms and so on.

ul-GapFR2-Config Indicates the FR2 UL gap configuration to UE. In EN-DC and NGEN-DC, the SN decides and configures the FR2 UL gap pattern. In NE-DC, the MN decides and configures the FR2 UL gap pattern. In NR-DC without FR2-FR2 band combination, the network entity which is configured with FR2 serving cell(s) decides and configures the FR2 UL gap pattern.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. If ReconfigurationWithSync is part of an RRCReconfiguration message within an LTM-Config IE associated with the MCG, the field is absent. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during a reconfiguration with sync which is not related to an LTM cell switch or subsequent CPAC, and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). Otherwise, the field is absent. |
| PagingRelay | For L2 U2N Relay UE, the field is optionally present, Need N. Otherwise, it is absent. |

=================================================NEXT CHANGE================================================================

–	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1610-IEs   OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       ReducedMaxCCs-r16                   OPTIONAL,

reducedMaxBW-FR1                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxBW-FR2                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

OverheatingAssistance-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17

} OPTIONAL,

reducedMaxMIMO-LayersFR2-2          SEQUENCE {

reducedMIMO-LayersFR2-2-DL          MIMO-LayersDL,

reducedMIMO-LayersFR2-2-UL          MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

ReducedAggregatedBandwidth-r17 ::= ENUMERATED {mhz0, mhz100, mhz200, mhz400, mhz800, mhz1200, mhz1600, mhz2000}

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {

ul-GapFR2-Preference-r17              UL-GapFR2-Preference-r17              OPTIONAL,

musim-Assistance-r17                  MUSIM-Assistance-r17                  OPTIONAL,

overheatingAssistance-r17             OverheatingAssistance-r17             OPTIONAL,

maxBW-PreferenceFR2-2-r17             MaxBW-PreferenceFR2-2-r17             OPTIONAL,

maxMIMO-LayerPreferenceFR2-2-r17      MaxMIMO-LayerPreferenceFR2-2-r17      OPTIONAL,

minSchedulingOffsetPreferenceExt-r17  MinSchedulingOffsetPreferenceExt-r17  OPTIONAL,

rlm-MeasRelaxationState-r17           BOOLEAN                               OPTIONAL,

bfd-MeasRelaxationState-r17           BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL,

nonSDT-DataIndication-r17             SEQUENCE {

resumeCause-r17                       ResumeCause                       OPTIONAL

}                                                                           OPTIONAL,

scg-DeactivationPreference-r17        ENUMERATED { scg-DeactivationPreferred, noPreference }    OPTIONAL,

uplinkData-r17                        ENUMERATED { true }                   OPTIONAL,

rrm-MeasRelaxationFulfilment-r17      BOOLEAN                               OPTIONAL,

propagationDelayDifference-r17        PropagationDelayDifference-r17        OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v1800-IEs     OPTIONAL

}

UEAssistanceInformation-v1800-IEs ::= SEQUENCE {

idc-FDM-Assistance-r18                IDC-FDM-Assistance-r18                          OPTIONAL,

idc-TDM-Assistance-r18                IDC-TDM-Assistance-r18                          OPTIONAL,

multiRx-PreferenceFR2-r18             ENUMERATED {single, multiple }                  OPTIONAL,

musim-Assistance-v1800                MUSIM-Assistance-v1800                          OPTIONAL,

flightPathInfoAvailable-r18           ENUMERATED {true}                               OPTIONAL,

ul-TrafficInfo-r18                    UL-TrafficInfo-r18                              OPTIONAL,

n3c-RelayUE-InfoList-r18              SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18  OPTIONAL,

sl-PRS-UE-AssistanceInformationNR-r18 SL-PRS-UE-AssistanceInformationNR-r18           OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v19xy-IEs             OPTIONAL

}

UEAssistanceInformation-v19xy-IEs ::= SEQUENCE {

gapOccasionCancelRatio-r19            GapOccasionCancelRatio-r19                       OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                      OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...,

[[

uwb-r18                     ENUMERATED {true}        OPTIONAL

]]

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {

ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512,

ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

MaxBW-PreferenceFR2-2-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL

} OPTIONAL

}

MaxCC-Preference-r16 ::=            SEQUENCE {

reducedMaxCCs-r16                   ReducedMaxCCs-r16                        OPTIONAL

}

MaxMIMO-LayerPreference-r16 ::=     SEQUENCE {

reducedMaxMIMO-LayersFR1-r16        SEQUENCE {

reducedMIMO-LayersFR1-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR1-UL-r16        INTEGER (1..4)

} OPTIONAL,

reducedMaxMIMO-LayersFR2-r16        SEQUENCE {

reducedMIMO-LayersFR2-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR2-UL-r16        INTEGER (1..4)

} OPTIONAL

}

MaxMIMO-LayerPreferenceFR2-2-r17 ::=    SEQUENCE {

reducedMaxMIMO-LayersFR2-2-r17          SEQUENCE {

reducedMIMO-LayersFR2-2-DL-r17          INTEGER (1..8),

reducedMIMO-LayersFR2-2-UL-r17          INTEGER (1..4)

} OPTIONAL

}

MinSchedulingOffsetPreference-r16 ::= SEQUENCE {

preferredK0-r16                       SEQUENCE {

preferredK0-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL,

preferredK0-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL

}                                                                                  OPTIONAL,

preferredK2-r16                       SEQUENCE {

preferredK2-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL,

preferredK2-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL

}                                                                                 OPTIONAL

}

MinSchedulingOffsetPreferenceExt-r17 ::=  SEQUENCE {

preferredK0-r17                           SEQUENCE {

preferredK0-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK0-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL,

preferredK2-r17                           SEQUENCE {

preferredK2-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK2-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL

}

MUSIM-Assistance-r17 ::=              SEQUENCE {

musim-PreferredRRC-State-r17          ENUMERATED {idle, inactive, outOfConnected}     OPTIONAL,

musim-GapPreferenceList-r17           MUSIM-GapPreferenceList-r17                     OPTIONAL

}

MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17

MUSIM-Assistance-v1800 ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18     MUSIM-GapPriorityPreferenceList-r18           OPTIONAL,

musim-GapKeepPreference-r18             ENUMERATED {true}                             OPTIONAL,

musim-CapRestriction-r18                MUSIM-CapRestriction-r18                      OPTIONAL,

musim-NeedForGapsInfoNR-r18             NeedForGapsInfoNR-r16                         OPTIONAL,

...

}

MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..3)) OF GapPriority-r17

MUSIM-CapRestriction-r18 ::=            SEQUENCE {

musim-Cell-SCG-ToRelease-r18            MUSIM-Cell-SCG-ToRelease-r18                  OPTIONAL,

musim-CellToAffectList-r18              MUSIM-CellToAffectList-r18                    OPTIONAL,

musim-AffectedBandsList-r18             MUSIM-AffectedBandsList-r18                   OPTIONAL,

musim-AvoidedBandsList-r18              MUSIM-AvoidedBandsList-r18                    OPTIONAL,

musim-MaxCC-r18                         MUSIM-MaxCC-r18                               OPTIONAL

}

MUSIM-Cell-SCG-ToRelease-r18 ::=        SEQUENCE {

musim-CellToRelease-r18                 MUSIM-CellToRelease-r18                       OPTIONAL,

scg-ReleasePreference-r18               ENUMERATED {true}                             OPTIONAL

}

MUSIM-CellToRelease-r18 ::=             SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex

MUSIM-CellToAffectList-r18::=           SEQUENCE (SIZE (1..maxNrofServingCells)) OF MUSIM-CellToAffect-r18

MUSIM-CellToAffect-r18 ::=              SEQUENCE {

musim-ServCellIndex-r18                 ServCellIndex,

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                                OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                                OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                      OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                      OPTIONAL

}

MUSIM-AffectedBandsList-r18  ::=        SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AffectedBands-r18

MUSIM-AffectedBands-r18 ::=             SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-CapabilityRestrictedBandParameters-r18

MUSIM-CapabilityRestrictedBandParameters-r18 ::= SEQUENCE {

musim-bandEntryIndex-r18                MUSIM-BandEntryIndex-r18,

musim-CapabilityRestricted-r18          SEQUENCE {

musim-MIMO-Layers-DL-r18                INTEGER (1..8)                            OPTIONAL,

musim-MIMO-Layers-UL-r18                INTEGER (1..4)                            OPTIONAL,

musim-SupportedBandwidth-DL-r18         SupportedBandwidth-v1700                  OPTIONAL,

musim-SupportedBandwidth-UL-r18         SupportedBandwidth-v1700                  OPTIONAL

}

}

MUSIM-AvoidedBandsList-r18 ::=          SEQUENCE (SIZE (1..maxBandComb-MUSIM-r18)) OF MUSIM-AvoidedBands-r18

MUSIM-AvoidedBands-r18 ::=              SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF MUSIM-BandEntryIndex-r18

MUSIM-BandEntryIndex-r18 ::=            INTEGER(1.. maxCandidateBandIndex-r18)

MUSIM-MaxCC-r18 ::=                     SEQUENCE {

musim-MaxCC-TotalDL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-TotalUL-r18                 INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-DL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR1-UL-r18                  INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-DL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-1-UL-r18                INTEGER (1..32)                               OPTIONAL,

musim-MaxCC-FR2-2-DL-r18                INTEGER (1..32)                                OPTIONAL,

musim-MaxCC-FR2-2-UL-r18                INTEGER (1..32)                                OPTIONAL

}

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

ReducedMaxBW-FRx-r16 ::=            SEQUENCE {

reducedBW-DL-r16                    ReducedAggregatedBandwidth,

reducedBW-UL-r16                    ReducedAggregatedBandwidth

}

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16

SL-TrafficPatternInfo-r16::=          SEQUENCE {

trafficPeriodicity-r16                ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

timingOffset-r16                      INTEGER (0..10239),

messageSize-r16                       BIT STRING (SIZE (8)),

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16

}

UL-GapFR2-Preference-r17::=           SEQUENCE {

ul-GapFR2-PatternPreference-r17       INTEGER (0..3)                     OPTIONAL

}

PropagationDelayDifference-r17 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (-270..270)

IDC-FDM-Assistance-r18 ::=            SEQUENCE {

affectedCarrierFreqRangeList-r18      AffectedCarrierFreqRangeList-r18               OPTIONAL,

affectedCarrierFreqRangeCombList-r18  AffectedCarrierFreqRangeCombList-r18           OPTIONAL,

...

}

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds-r18 INTEGER (1..31),

milliSeconds-r18    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

AffectedCarrierFreqRangeList-r18 ::=  SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18

AffectedCarrierFreqRange-r18 ::=      SEQUENCE {

affectedFreqRange-r18                 AffectedFreqRange-r18,

interferenceDirection-r18             ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedCarrierFreqRangeCombList-r18 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18

AffectedCarrierFreqRangeComb-r18 ::=  SEQUENCE {

affectedCarrierFreqRangeComb-r18      SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedFreqRange-r18,

interferenceDirection-r18             ENUMERATED {nr, other, both, spare},

victimSystemType-r18                  VictimSystemType-r16                           OPTIONAL

}

AffectedFreqRange-r18 ::=             SEQUENCE {

centerFreq-r18                        ARFCN-ValueNR,

affectedBandwidth-r18                 ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3, mhz4, mhz5, mhz6,

mhz8, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200,

mhz300, mhz400, spare10, spare9, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

}

UL-TrafficInfo-r18 ::=                SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18

PDU-SessionUL-TrafficInfo-r18 ::=     SEQUENCE {

pdu-SessionID-r18                     PDU-SessionID,

qos-FlowUL-TrafficInfoList-r18        SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18

}

QOS-FlowUL-TrafficInfo-r18 ::=        SEQUENCE {

qfi-r18                               QFI,

jitterRange-r18                       SEQUENCE {

lowerBound-r18                        JitterBound-r18,

upperBound-r18                        JitterBound-r18

}                                                                                    OPTIONAL,

burstArrivalTime-r18                  CHOICE {

referenceTime                         ReferenceTime-r16,

referenceSFN-AndSlot                  ReferenceSFN-AndSlot-r18

}                                                                                    OPTIONAL,

trafficPeriodicity-r18                INTEGER (1..640000)                            OPTIONAL,

pdu-SetIdentification-r18             BOOLEAN                                        OPTIONAL,

psi-Identification-r18                BOOLEAN                                        OPTIONAL,

...

}

ReferenceSFN-AndSlot-r18 ::= SEQUENCE {

referenceSFN-r18                 INTEGER (0..1023),

referenceSlot-r18                INTEGER (0..639)

}

JitterBound-r18 ::= ENUMERATED {ms0, ms0dot5, ms1, ms1dot5, ms2, ms2dot5, ms3, ms3dot5, ms4, ms4dot5, ms5, ms5dot5, ms6, ms6dot5, ms7, beyondMs7}

SL-PRS-UE-AssistanceInformationNR-r18 ::= SEQUENCE (SIZE (1..maxNrofSL-PRS-TxConfig-r18)) OF SL-PRS-TxInfo-r18

SL-PRS-TxInfo-r18 ::=                 SEQUENCE {

sl-PRS-Periodicity-r18                ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6,

spare5, spare4, spare3, spare2, spare1},

sl-PRS-Priority-r18                   INTEGER (1..8)                                                            OPTIONAL,

sl-PRS-DelayBudget-r18                INTEGER (0..1023)                                                         OPTIONAL,

sl-PRS-Bandwidth-r18                  ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40,

mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400,

spare15, spare14, spare13, spare12, spare11, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}       OPTIONAL,

...

}

GapOccasionCancelRatio-r19 ::= SEQUENCE {

gapOccasionCancelRatioPerFR-PerUE-r19     CHOICE {

perUE-r19                      GapOccasionRatio-r19,

perFR-r19                      SEQUENCE {

fr1-r19                        GapOccasionRatio-r19                                                       OPTIONAL,

fr2-r19                        GapOccasionRatio-r19                                                       OPTIONAL

}

}                                                                                                                 OPTIONAL,

gapConfigRatioList-r19             SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapOccasionRatioPerGapConfig-r19          OPTIONAL

}

GapOccasionRatioPerGapConfig-r19 ::= SEQUENCE{

measGapId-r19                       MeasGapId-r17,

gapOccasionRatio-r19                GapOccasionRatio-r19

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

activeDuration Indicates the UE's preferred active duration to resolve the IDC problem. Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

affectedBandwidth Indicates the bandwidth around the center frequency of the carrier frequency range which is affected by the IDC problem. Value mhz5 corresponds to 5 MHz, value mhz10 corresponds to 10 MHz and so on. If candidateBandwidth is not configured, the UE is allowed to report the frequency range for any bandwidth as indicated by affectedBandwidth, within the frequency band limitation as defined in TS 38.101-1 [15], TS 38.101-2 [39], TS 38.101-3 [34] and TS 38.101-5 [75].

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqRangeList Indicates a list of NR carrier frequency ranges that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequency combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC.

affectedCarrierFreqRangeCombList Indicates a list of NR carrier frequency range combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit, starting from MSB. A bit that is set to 1 indicates that the UE is performing BFD measurements relaxation on the serving cell mapped on the bit. A bit that is set to 0 indicates that the UE is not performing BFD measurements relaxation on the serving cell mapped on the bit. If a serving cell is not configured to the UE, the corresponding bit is set to 0.

centerFreq Indicates the center frequency of the carrier frequency range which is affected by the IDC problem.

cycleLength Indicates the UE's preferred cycle length to resolve the IDC problem. Value in ms. Value ms2 corresponds to 2 ms, value ms3 corresponds to 3 ms, and so on.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

minSchedulingOffsetPreference Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceExt Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving for SCS 480 kHz and/or 960 kHz.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

musim-AffectedBandsList Indicates the UE's preference on the band(s) and/or combination(s) of bands with restricted capability for MUSIM operation. If the MUSIM-CapabilityRestrictedBandParameters-r18 with same musim-bandEntryIndex appears more than once in the list of bands in a MUSIM-AffectedBands entry, the UE supports intra-band non-contiguous CA with restricted capability for MUSIM operation for this band. UE explicitly indicates each band and each combination of bands that are affected. The Network should respect these capability restrictions when configuring the UE with bands or band combinations that contain these bands and/or combination of bands. Fields musim-MIMO-Layers-DL/UL and musim-SupportedBandwidth-DL/UL indicate the max number of MIMO layers and max bandwidth on each CC of the band, respectively. The band(s) and/or combination(s) of bands are supported in UE capability, and the musim-MIMO-Layers-DL/UL and musim-SupportedBandwidth-DL/UL range up to the concerned capability of band(s) and/or combination(s) of bands in UE capability.

musim-AvoidedBandsList Indicates the UE's preference on band(s) and/or combination(s) of bands to be avoided for MUSIM purpose. UE explicitly indicates each band and each combination of bands to be avoided. The list may include the band of the PCell. The Network should respect these capability restrictions for the band combinations that contain these bands and/or combination of bands. The band(s) and/or combination(s) of bands is a subset of the band combination(s) in UE capability.

musim-bandEntryIndex Indicates an NR band by referring to the position of a band entry in musim-CandidateBandList IE. Value 1 identifies the first band in the musim-CandidateBandList IE, value 2 identifies the second band in the musim-CandidateBandList IE, and so on.

musim-CapabilityRestricted Indicates the UE's preference on the temporary capability restriction on the band for MUSIM operation.

musim-CapRestriction Indicates the UE's preference on SCell(s) or PSCell to be released, serving cell(s) with restricted capability, band(s) or combination(s) of bands with restricted capability, or band(s) or band combination(s) to be avoided for UE temporary capabilities restriction.

musim-Cell-SCG-ToRelease Indicates the UE's preference on any serving cell(s), except for Pcell, and/or SCG to be released for MUSIM operation.

musim-CellToAffectList Indicates the UE's preference on the temporary capability restriction on the serving cell(s) for MUSIM operation.

musim-CellToRelease Indicates the UE's preference on the temporary capability restriction on the serving cell(s) to release, except PCell, for MUSIM operation.

musim-GapKeepPreference Indicates the UE's preference to keep all colliding gaps for requested MUSIM gap(s). If the field is absent, the colliding MUSIM gaps with lower priority shall be dropped as specified in TS 38.133 [14].

musim-GapPreferenceList Indicates the UE's MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.133 [14] clause 9.1.10.

musim-GapPriorityPreferenceList Indicates the UE's MUSIM gap priority preference for periodic MUSIM gaps as specified in TS 38.133[14]. If the UE includes musim-GapPriorityPreferenceList-r18, it includes the same number of entries, and listed in the same order for periodic gaps, as in musim-GapPreferenceList-r17.

musim-MaxCC Indicates the UE's preference on the temporary capability restriction on maximum number of CCs per DL/UL in total, and per FR1/FR2-1/F2-2.

musim-NeedForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands when in MUSIM operation while NR-DC or NE-DC is not configured.

musim-PreferredRRC-State Indicates the UE's preferred RRC state when leaving RRC_CONNECTED.

n3c-RelayUE-InfoList Information of available N3C relay UE(s).

nonSDT-DataIndication Informs the network about the arrival of data and/or signaling mapped to radio bearers not configured for SDT while SDT procedure is ongoing.

preferredDRX-InactivityTimer Indicates the UE's preferred DRX inactivity timer length for power saving. Value in ms (milliSecond). ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the DRX inactivity timer. If secondary DRX group is configured, the preferredDRX-InactivityTimer only applies to the default DRX group.

preferredDRX-LongCycle Indicates the UE's preferred long DRX cycle length for power saving. Value in ms. ms10 corresponds to 10ms, ms20 corresponds to 20 ms, ms32 corresponds to 32 ms, and so on. If preferredDRX-ShortCycle is provided, the value of preferredDRX-LongCycle shall be a multiple of the preferredDRX-ShortCycle value. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the long DRX cycle.

preferredDRX-ShortCycle Indicates the UE's preferred short DRX cycle length for power saving. Value in ms. ms2 corresponds to 2ms, ms3 corresponds to 3 ms, ms4 corresponds to 4 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle.

preferredDRX-ShortCycleTimer Indicates the UE's preferred short DRX cycle timer for power saving. Value in multiples of preferredDRX-ShortCycle. A value of 1 corresponds to preferredDRX-ShortCycle, a value of 2 corresponds to 2 * preferredDRX-ShortCycle and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle timer. A preference for the short DRX cycle is indicated when a preference for the short DRX cycle timer is indicated.

preferredK0 Indicates the UE's preferred value of k0 (slot offset between DCI and its scheduled PDSCH - see TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k0 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k0 for cross-slot scheduling.

preferredK2 Indicates the UE's preferred value of k2 (slot offset between DCI and its scheduled PUSCH - see TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k2 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k2 for cross-slot scheduling.

preferredRRC-State Indicates the UE's preferred RRC state. The value idle is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_IDLE. The value inactive is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_INACTIVE. The value connected is indicated if the UE prefers to revert an earlier indication to leave RRC_CONNECTED state. The value outOfConnected is indicated if the UE prefers to be released from RRC_CONNECTED and has no preferred RRC state to transition to. The value connected can only be indicated if the UE is configured with connectedReporting.

propagationDelayDifference Indicates the one-way service link propagation delay difference between serving cell and each neighbour cell included in neighCellInfoList, defined as neighbour cell's service link propagation delay minus serving cell's service link propagation delay, in number of ms. First entry in propagationDelayDifference corresponds to first entry in neighCellInfoList, second entry in propagationDelayDifference corresponds to second entry in neighCellInfoList, and so on.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes SCells of the NR MCG, PSCell and SCells of the SCG. This maximum number only includes PSCell and SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell and SCells of the cell group that this UE assistance information is associated with. The maximum number of downlink SCells can only range up to the current active configuration when indicated to address power savings.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes SCells of the NR MCG, PSCell and SCells of the SCG. This maximum number only includes PSCell and SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell and SCells of the cell group that this UE assistance information is associated with. The maximum number of uplink SCells can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR1 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. The aggregated bandwidth across all downlink carrier(s) of FR1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR1. The aggregated bandwidth across all uplink carrier(s) of FR1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR1 of both the NR MCG and the SCG. This maximum aggregated bandwidth only includes carriers of FR1 of the SCG in (NG)EN-DC. Value mhz0 is not used when indicated to address overheating. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-1. The aggregated bandwidth across all downlink carrier(s) of FR2-1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-1. The aggregated bandwidth across all uplink carrier(s) of FR2-1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-1 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR2-2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-2, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-2. The aggregated bandwidth across all downlink carrier(s) of FR2-2 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-2. The aggregated bandwidth across all uplink carrier(s) of FR2-2 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-2. If the field is absent from the MaxBW-PreferenceFR2-2 IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-2. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-2 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMIMO-LayersFR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving (see NOTE 1). This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

referenceTimeInfoPreference Indicates whether the UE prefers being provisioned with the timing information specified in the IE ReferenceTimeInfo.

resumeCause Provides the resume cause based on the information received from the upper layers.

rlm-MeasRelaxationState Indicates the relaxation state of RLM measurements. Value true indicates that the UE is performing relaxation of RLM measurements, and value false indicates that the UE is not performing relaxation of RLM measurements.

rrm-MeasRelaxationFulfilment Indicates whether the UE fulfils the relaxed measurement criterion for stationary UE in 5.7.4.4. Value true indicates that the UE fulfils the criterion, and value false indicates that the UE does not fulfil the criterion.

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-PRS-Bandwidth Indicates the desired bandwidth of the requested SL-PRS resources provided by upper layers (see TS 38.355 [77]) in the unit of MHz.

sl-PRS-DelayBudget Indicates the SL-PRS delay budget provided by upper layers (see TS 38.355 [77]).

sl-PRS-Periodicity Indicates the periodicity of SL-PRS transmission.

sl-PRS-Priority Indicates the priority of SL-PRS provided by upper layers (see TS 38.355 [77]). Value 1 is the highest priority whereas value 8 is the lowest priority.

sl-UE-AssistanceInformationNR Indicates the traffic characteristic of sidelink logical channel(s), specified in the IE SL-TrafficPatternInfo, that are setup for NR sidelink communication.

slotOffset Indicates the UE's preferred slot offset to resolve the IDC problem, in multiples of 1/32 ms.

startOffset Indicates the UE's preferred start offset to resolve the IDC problem, in multiples of 1 ms.

type1 Indicates the preferred amount of increment/decrement to the long DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

ul-GapFR2-PatternPreference Indicates the UE's preference on FR2 UL gap pattern as defined in TS 38.133 [14].

victimSystemType Indicate the list of victim system types to which IDC interference is caused from NR. Value gps, glonass, bds, galileo and navIC indicates the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth. Value uwb indicates Ultra Wide Band.

NOTE 1:	The field may also indicate the UE's preference on reduced configuration corresponding to the maximum number of SRS ports (i.e. nrofSRS-Ports) of each serving cell operating on the associated frequency range.

SL-TrafficPatternInfo field descriptions

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 38.321 [3], table 6.1.3.1-2.

timingOffset This field indicates the estimated timing for a packet arrival in a sidelink logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficPeriodicity This field indicates the estimated data arrival periodicity in a sidelink logical channel. Value ms20 corresponds to 20 ms, ms50 corresponds to 50 ms and so on.

UL-TrafficInfo field descriptions

burstArrivalTime Indicates the expected arrival time of the first packet of the Data Burst for the concerned QoS flow. If the UE provides both burstArrivalTime and jitterRange, burstArrivalTime is used as a reference time for the indicated jitter range. If burstArrivalTime is indicated as referenceTime, the indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If burstArrivalTime is indicated as referenceSFN-AndSlot, it refers to the UL timing of the closest SFN and slot of the PCell with the indicated number.

jitterRange Indicates the maximum deviation of the arrival time of the first packet of a Data Burst compared to the time indicated with burstArrivalTime and the periodicity of the Data Bursts. lowerBound indicates the negative deviation while upperBound indicates the positive deviation. This field shall only be reported together with the burstArrivalTime or after the burstArrivalTime has been already reported. Value ms0 corresponds to 0 ms, value 0dot5 to 0.5 ms, value ms1 to 1 ms and so on. Value beyondMs7 indicates the jitter bound is higher than 7 ms. Value 0 ms means there is no Data Burst arrival time deviation from the indicated burstArrivalTime.

pdu-SetIdentification Indicates whether the UE is able to identify PDU Set(s) for the QoS flow. If set to true, the UE is able to identify PDU Set(s) for the associated QoS flow, otherwise, the UE is not able to do so. Before receiving this indication, the network assumes the value is set to false.

psi-Identification Indicates whether the UE is able to identify PSI(s) for the QoS flow. This field shall only be set to true if pdu-SetIdentification is also set to true (or was set to true previously for the same QoS flow). If set to true, the UE is able to identify PSI(s) for the associated QoS flow, otherwise, the UE is not able to do so. Before receiving this indication, the network assumes the value is set to false.

qfi Identity of the QoS flow to which this UL traffic information refers.

trafficPeriodicity Indicates the average time period between the start times of two data bursts, expressed in the number of microseconds.

GapOccasionCancelRatio field descriptions

perUE Includes the UE's perference for gap occasion cancellation ratio for the configured per UE measurement gap under measGapConfig.

fr1, fr2 Includes the UE's preference for gap occasion cancellation ratio for the configured per FR measurement gap under measGapConfig.

gapConfigRatioList Includes the UE's preference for gap occasion cancellation ratio for the configured measurement gap with measGapId in gapToAddModList under measGapConfig.

=================================================NEXT CHANGE================================================================

6.3	RRC information elements

6.3.2	Radio resource control information elements

–	GapOccasionRatio

Ratio of gap occasions that is recommended for cancellation during a time period of 1 second. Value 'pc0' corresponds to 0 percent, 'pc20' corresponds to 20 percent and so on.

GapOccasionRatio information element

-- ASN1START

-- TAG-GAPOCCASIONRATIO-START

GapOccasionRatio-r19 ::= ENUMERATED {pc0, pc20, pc40, pc60}

-- TAG-GAPOCCASIONRATIO-STOP

-- ASN1STOP

–	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICALCHANNELCONFIG-START

LogicalChannelConfig ::=            SEQUENCE {

ul-SpecificParameters               SEQUENCE {

priority                            INTEGER (1..16),

prioritisedBitRate                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

allowedServingCells                 SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex

OPTIONAL,   -- Cond PDCP-CADuplication

allowedSCS-List                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                   OPTIONAL,   -- Need R

maxPUSCH-Duration                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, ms0p01-v1700, spare1}

OPTIONAL,   -- Need R

configuredGrantType1Allowed         ENUMERATED {true}                                                   OPTIONAL,   -- Need R

logicalChannelGroup                 INTEGER (0..maxLCG-ID)                                              OPTIONAL,   -- Need R

schedulingRequestID                 SchedulingRequestId                                                 OPTIONAL,   -- Need R

logicalChannelSR-Mask               BOOLEAN,

logicalChannelSR-DelayTimerApplied  BOOLEAN,

...,

bitRateQueryProhibitTimer       ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}               OPTIONAL,    -- Need R

[[

allowedCG-List-r16                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL,   -- Need S

allowedPHY-PriorityIndex-r16        ENUMERATED {p0, p1}                                                 OPTIONAL    -- Need S

]],

[[

logicalChannelGroupIAB-Ext-r17      INTEGER (0..maxLCG-ID-IAB-r17)                                      OPTIONAL,   -- Need R

allowedHARQ-mode-r17                ENUMERATED {harqModeA, harqModeB}                                   OPTIONAL    -- Need R

]],

[[

enhancedLCP-r19                  SEQUENCE{

priorityAdjustmentThreshold-r19          INTEGER (1..64),

additionalPriority-r19                   INTEGER (1..16),

...

}                                                                                                        OPTIONAL     -- Need R[RIL]: O400, XR

]]

}                                                                                                       OPTIONAL,   -- Cond UL

...,

[[

channelAccessPriority-r16           INTEGER (1..4)                                                      OPTIONAL,   -- Need R

bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}                                   OPTIONAL    -- Need R

]]

}

-- TAG-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

additionalPriority The additional logical channel priority that overrides the logical channel priority configured by the field priority when the logical channel priority adjustment condition is satisfied as specified in TS 38.321 [3]. For the same logical channel configuration, the value of the field shall be smaller than that of the field priority.

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

allowedHARQ-mode Indicates the allowed HARQ mode of a HARQ process mapped to this logical channel. If the parameter is absent, there is no restriction for HARQ mode for the mapping. This field applies to SRB1, SRB2, SRB4 and DRBs.

allowedPHY-PriorityIndex This restriction applies only when the UL grant is a dynamic grant. If the field is present and the dynamic grant has a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to the dynamic grants indicating PHY-priority index equal to the values configured by this field. If the field is present and the dynamic grant does not have a PHY-priority index, UL MAC SDUs from this logical channel can only be mapped to this dynamic grant if the value of the field is p0, see TS 38.213 [13], clause 9. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any dynamic grants. Corresponds to "allowedPHY-PriorityIndex" as specified in TS 38.321 [3].

allowedSCS-List If present, UL MAC SDUs from this logical channel can only be mapped to the indicated numerology. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured numerology. Corresponds to 'allowedSCS-List' as specified in TS 38.321 [3]. Only the following values are applicable depending on the used frequency: FR1: 15, 30, or 60 kHz FR2-1/FR2-NTN: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz

allowedServingCells If present, UL MAC SDUs from this logical channel can only be mapped to the serving cells indicated in this list. Otherwise, UL MAC SDUs from this logical channel can be mapped to any configured serving cell of this cell group. Corresponds to 'allowedServingCells' in TS 38.321 [3].

bitRateMultiplier Bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [3]. Value x40 indicates bit rate multiplier 40, value x70 indicates bit rate multiplier 70 and so on.

bitRateQueryProhibitTimer The timer is used for bit rate recommendation query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on.

bucketSizeDuration Value in ms. ms5 corresponds to 5 ms, value ms10 corresponds to 10 ms, and so on.

channelAccessPriority Indicates the Channel Access Priority Class (CAPC), as specified in TS 38.300 [2], to be used on uplink transmissions for operation with shared spectrum channel access in FR1. The network configures this field only for SRB2 and DRBs.

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1. Corresponds to 'configuredGrantType1Allowed' in TS 38.321 [3]. This field is ignored when SDT procedure is ongoing.

logicalChannelGroup, logicalChannelGroupIAB-Ext ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to. The logicalChannelGroupIAB-Ext is only applicable to the IAB-MT. When logicalChannelGroupIAB-Ext is configured, logicalChannelGroup shall be ignored.

priorityAdjustmentThreshold Remaining time threshold for determining whether the additional logical channel priority configured by additionalPriority is applied for the logical channel, as specified in TS 38.321 [3]. Value in number of milliseconds.

logicalChannelSR-Mask Controls SR triggering when a configured uplink grant of type1 or type2 is configured. true indicates that SR masking is configured for this logical channel as specified in TS 38.321 [3].

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to false if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC SDUs from this logical channel can only be transmitted using uplink grants that result in a PUSCH duration shorter than or equal to the duration indicated by this field. Otherwise, UL MAC SDUs from this logical channel can be transmitted using an uplink grant resulting in any PUSCH duration. Corresponds to "maxPUSCH-Duration" in TS 38.321 [3]. The PUSCH duration is calculated based on the same length of all symbols, and the shortest length applies if the symbol lengths are different.

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. Value kBps0 corresponds to 0 kiloBytes/s, value kBps8 corresponds to 8 kiloBytes/s, value kBps16 corresponds to 16 kiloBytes/s, and so on. For SRBs, the value can only be set to infinity.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel, as specified in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| PDCP-CADuplication | The field is mandatory present if the DRB/SRB associated with this logical channel is configured with PDCP CA duplication in UL in the cell group in which this IE is included (i.e. the PDCP entity is associated with multiple RLC entities belonging to this cell group). Otherwise the field is optionally present, need R. |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB or multicast MRB. It is optionally present, Need R, for a logical channel with uplink if it serves an SRB. Otherwise it is absent. |

=================================================NEXT CHANGE================================================================

–	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup-r16 }                   OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL    -- Need R

]],

[[

intraCG-Prioritization-r17          ENUMERATED {enabled}                        OPTIONAL,    -- Cond LCH-PrioWithReTxTimer

drx-ConfigSL-r17                    SetupRelease { DRX-ConfigSL-r17 }           OPTIONAL,    -- Need M

drx-ConfigExt-v1700                 SetupRelease { DRX-ConfigExt-v1700 }        OPTIONAL,    -- Need M

schedulingRequestID-BFR-r17         SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestID-BFR2-r17        SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestConfig-v1700       SchedulingRequestConfig-v1700               OPTIONAL,    -- Need M

tar-Config-r17                      SetupRelease { TAR-Config-r17  }                                OPTIONAL,    -- Need M

g-RNTI-ConfigToAddModList-r17       SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17       OPTIONAL,    -- Need N

g-RNTI-ConfigToReleaseList-r17      SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17     OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToAddModList-r17    SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfig-r17    OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToReleaseList-r17   SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF MBS-RNTI-SpecificConfigId-r17  OPTIONAL,    -- Need N

allowCSI-SRS-Tx-MulticastDRX-Active-r17   BOOLEAN                                                           OPTIONAL     -- Need M

]],

[[

schedulingRequestID-PosMG-Request-r17 SchedulingRequestId                                                   OPTIONAL,    -- Need R

drx-LastTransmissionUL-r17          ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

posMG-Request-r17                   ENUMERATED {enabled}                                                    OPTIONAL     -- Need R

]],

[[

drx-ConfigExt2-v1800                SetupRelease { DRX-ConfigExt2-v1800 }                                   OPTIONAL,    -- Need M

additionalBS-TableAllowed-r18       BIT STRING (SIZE (maxNrofLCGs-r18))                                     OPTIONAL,    -- Need R

dsr-ConfigToAddModList-r18          SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-DSR-Config-r18              OPTIONAL,    -- Need N

dsr-ConfigToReleaseList-r18         SEQUENCE (SIZE (1..maxNrofLCGs-r18)) OF LCG-Id-r18                      OPTIONAL,    -- Need N

tar-Config-r18                      SetupRelease { TAR-Config-r18  }                                        OPTIONAL     -- Need M

]],

[[

ul-RateQuery-r19                        SEQUENCE {

ul-RateQueryConfigList-r19          SEQUENCE (SIZE (1..maxNrofRateQueryQFIs-r19)) OF QoS-FlowIdentity-r19,

ul-RateQueryProhibitTimer-r19       ENUMERATED {s0, s0dot1, s0dot2, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare1},

...

}                                                                                                         OPTIONAL,   -- Need R

ul-RateControlConfigList-r19        SEQUENCE (SIZE (1..maxNrofRateCtrlQFIs-r19)) OF QoS-FlowIdentity-r19      OPTIONAL     -- Need R

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

MBS-RNTI-SpecificConfig-r17 ::=        SEQUENCE {

mbs-RNTI-SpecificConfigId-r17          MBS-RNTI-SpecificConfigId-r17,

groupCommon-RNTI-r17                   CHOICE {

g-RNTI                                 RNTI-Value,

g-CS-RNTI                              RNTI-Value

},

drx-ConfigPTM-r17                      SetupRelease { DRX-ConfigPTM-r17 }                          OPTIONAL,   -- Need M

harq-FeedbackEnablerMulticast-r17      ENUMERATED {dci-enabler, enabled}                           OPTIONAL,   -- Need S

harq-FeedbackOptionMulticast-r17       ENUMERATED {ack-nack, nack-only}                            OPTIONAL,   -- Cond HARQFeedback

pdsch-AggregationFactor-r17            ENUMERATED {n2, n4, n8}                                     OPTIONAL    -- Cond G-RNTI

}

MBS-RNTI-SpecificConfigId-r17 ::= INTEGER (0..maxG-RNTI-1-r17)

LCG-DSR-Config-r18 ::= SEQUENCE {

lcg-Id-r18                      LCG-Id-r18,

remainingTimeThreshold-r18      INTEGER (1..64),

...,

[[

dsr-ReportingThresList-r19             SEQUENCE (SIZE (1..maxDSR-ReportingThres-r19)) OF DSR-ReportingThreshold-r19  OPTIONAL,    -- Need R

dsr-ReportNonDelayCriticalData-r19     ENUMERATED {enabled}                                        OPTIONAL    -- Cond ReportThresList

]] [RIL]: H200, WI-code

}

LCG-Id-r18 ::= INTEGER (0..maxLCG-ID)

DSR-ReportingThreshold-r19 ::= INTEGER (1..64)

QoS-FlowIdentity-r19 ::= SEQUENCE {

pdu-SessionID-r19                                PDU-SessionID,

qfi-r19                                          QFI

}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

additionalBS-TableAllowed Indicates whether a UE is allowed to utilize the refined buffer size levels, as specified in TS 38.321 [3], for a certain Logical Channel Group. The leftmost bit corresponds to LCG ID=0, second leftmost bit to LCG ID=1 and so on. The UE is allowed to utilize the refined buffer size levels for a Logical Channel Group only when the corresponding bit is set to 1.

allowCSI-SRS-Tx-MulticastDRX-Active Used to control the CSI/SRS transmission during MBS multicast DRX ActiveTime, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config, drx-ConfigExt, drx-ConfigExt2 Used to configure DRX as specified in TS 38.321 [3]. Network only configures drx-ConfigExt or drx-ConfigExt2 when drx-Config is configured.

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

drx-ConfigSL Used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3]. Network only configures this field if sl-ScheduledConfig is configured and drx-Config is configured.

drx-LastTransmissionUL If this field is present, the start of the drx-HARQ-RTT-TimerUL is after the last transmission within a bundle, see TS 38.321 [3].

dsr-ConfigToAddModList List of LCG-specific DSR configurations to add or modify.

dsr-ConfigToReleaseList List of LCG-specific DSR configurations to release.

g-RNTI-ConfigToAddModList List of G-RNTI configurations to add or modify. Up to 8 G-RNTIs can be configured in total in this release based on the UE capability.

g-RNTI-ConfigToReleaseList List of G-RNTI configurations to release.

g-CS-RNTI-ConfigToAddModList List of G-CS-RNTI configurations to add or modify. Up to 8 G-CS-RNTIs can be configured in total in this release based on the UE capability.

g-CS-RNTI-ConfigToReleaseList List of G-CS-RNTI configurations to release.

intraCG-Prioritization Used to enable HARQ process ID selection based on LCH-priority for one CG as specified in TS 38.321 [3].

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3]. The network does not configure lch-BasedPrioritization with enhancedSkipUplinkTxDynamic simultaneously nor lch-BasedPrioritization with enhancedSkipUplinkTxConfigured simultaneously.

posMG-Request Indicates whether UE is configured to send UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-BFR Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet1 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet2 of the same serving cell.

schedulingRequestID-BFR2 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in failureDetectionSet2 of a serving cell while beam failure is not detected on resources configured in failureDetectionSet1 of the same serving cell.

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

schedulingRequestID-PosMG-Request Indicates the scheduling request configuration applicable for Positioning Measurement Gap Activation/Deactivation Request, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1. The network does not configure enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true together with numberOfSlotsTBoMS-r17.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

ul-RateControlConfigList Includes the list of QoS flows for which the UL rate control is enabled.

ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is enabled. [RIL]: V051, XR

ul-RateQueryProhibitTimer This timer is used for uplink date rate query MAC CE as specified in TS 38.321 [3]. Value s0 means 0 s, s0dot1 means 0.1 s and so on.

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

MBS-RNTI-SpecificConfig field descriptions

drx-ConfigPTM Used to configure DRX for PTM transmission as specified in TS 38.321 [3].

g-CS-RNTI Used to scramble the SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH for one or more MBS multicast services.

g-RNTI Used to scramble the scheduling and transmission of PTM for one or more MBS multicast services.

groupCommon-RNTI Used to configure g-RNTI or g-CS-RNTI.

harq-FeedbackEnablerMulticast Indicates whether the UE shall provide HARQ feedback for MBS multicast. Value dci-enabler means that whether the UE shall provide HARQ feedback for MBS multicast is indicated by DCI as specified in TS 38.213 [13]. Value enabled means the UE shall always provide HARQ feedback for MBS multicast. When the field is absent, the UE behavior is specified in TS 38.213 [13].

harq-FeedbackOptionMulticast Indicates the feedback mode for MBS multicast dynamically scheduled PDSCH or SPS PDSCH.

mbs-RNTI-SpecificConfigId An identifier of the RNTI specific configuration for MBS multicast.

pdsch-AggregationFactor Number of repetitions for dynamically scheduled MBS multicast data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent and groupCommon-RNTI is set to g-RNTI, the UE applies the value 1.

LCG-DSR-Config field descriptions

lcg-Id Identifier of the Logical Channel Group which the DSR configuration refers to.

remainingTimeThreshold Remaining time threshold used for triggering Single Entry and Multiple Entry DSR for the logical channels belonging to this Logical Channel Group, as specified in TS 38.321 [3]. Value in number of milliseconds.

dsr-ReportingThresList List of remaining time thresholds configured in ascending order for reporting delay status information in the Multiple Entry DSR MAC CE, as specified in TS 38.321 [3]. At least one configured DSR reporting threshold should be no lower than the remainingTimeThreshold. If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with remainingTimeThreshold should be configured with dsr-ReportingThresList. The IE DSR-ReportingThreshold in number of milliseconds.

dsr-ReportNonDelayCriticalData Indicates whether the UE should consider the non-delay reporting data ahead of delay reporting data in the delay status reporting data volume calculation for the Logical Channel Group as specified in TS 38.323 [5].

QoS-FlowIdentity field descriptions

qfi Identifier of the QoS flow for which bit rate query or bit rate control is enabled.

pdu-SessionID Identifier of the PDU session to which the QoS flow idenfitied by the field qfi belongs.

| Conditional Presence | Explanation |
| --- | --- |
| G-RNTI | This field is optionally present, Need S, if groupCommon-RNTI is set to g-RNTI. The field is absent when groupCommon-RNTI is set to g-CS-RNTI. |
| HARQFeedback | The field is mandatory present when harq-FeedbackEnablerMulticast is present. It is absent otherwise. |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |
| LCH-PrioWithReTxTimer | This field is optionally present, Need R, if lch-BasedPrioritization-r16 is configured in this MAC entity and cg-RetransmissionTimer-r16 is configured for any configured grant configuration associated with this MAC entity. It is absent otherwise, Need R. |
| ReportThresList | This field is optionally present, Need R, if the field dsr-ReportingThresList is present. It is absent otherwise. |

=================================================NEXT CHANGE================================================================

–	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling, MBS multicast and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL    -- Need M

]],

[[

survivalTimeStateSupport-r17   ENUMERATED {true}                                            OPTIONAL,   -- Cond Drb-Duplication

uplinkDataCompression-r17      SetupRelease { UplinkDataCompression-r17 }                   OPTIONAL,   -- Cond Rlc-AM

discardTimerExt2-r17           SetupRelease { DiscardTimerExt2-r17 }                        OPTIONAL,   -- Need M

initialRX-DELIV-r17            BIT STRING (SIZE (32))                                       OPTIONAL    -- Cond MRB-Initialization

]],

[[

pdu-SetDiscard-r18             ENUMERATED {true}                                            OPTIONAL,   -- Need R

discardTimerForLowImportance-r18   SetupRelease { DiscardTimerForLowImportance-r18 }        OPTIONAL,   -- Cond DRB2

primaryPathOnIndirectPath-r18  ENUMERATED {true}                                            OPTIONAL,   -- Cond SplitBearerMP

sn-GapReport-r18               ENUMERATED {true}                                            OPTIONAL    -- Need R

]],

[[

remainingTimeThresholdRLC-ReTx-r19              RLC-AM-RemainingTimeThreshold-r19           OPTIONAL,   -- Cond RLC-AM

remainingTimeThresholdRLC-Polling-r19           RLC-AM-RemainingTimeThreshold-r19           OPTIONAL    -- Cond RLC-AM

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

DiscardTimerExt2-r17 ::= ENUMERATED {ms2000, spare3, spare2, spare1}

UplinkDataCompression-r17 ::= CHOICE {

newSetup                      SEQUENCE {

bufferSize-r17                ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r17                ENUMERATED {sip-SDP, operator}                            OPTIONAL    -- Need N

},

drb-ContinueUDC           NULL

}

DiscardTimerForLowImportance-r18 ::= ENUMERATED {ms0, ms2, ms4, ms6, ms8, ms10, ms12, ms14, ms18, ms22, ms26, ms30, ms40, ms50, ms75, ms100}

RLC-AM-RemainingTimeThreshold-r19 ::= INTEGER (1..64)

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

discardTimerExt2 Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

discardTimerForLowImportance Value in ms of discardTimerForLowImportance specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms2 corresponds to 2 ms and so on. The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB. The network configures at most 8 DRBs with discardTimerForLowImportance.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. For MP, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB or a bi-directional multicast MRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured. Network only configures this field when uplinkDataCompression is not configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB or a multicast MRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure DAPS bearer(s), and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured. Network only configures this field when uplinkDataCompression is not configured.

initialRX-DELIV Indicates the initial value of RX_DELIV during PDCP window initialization for multicast MRB as specified in TS 38.323 [5]. The first/leftmost bit of the bit string contains the most significant bit.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdu-SetDiscard If set to true, the UE shall perform PDU Set based discarding for this PDCP entity, as specified in TS 38.323 [5]. This field is only configured for a DRB.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and except when the UE is required to set the primaryPath to refer to the SCG or indirect path as specified in clause 5.7.3b.4. In this last case, if the network sends an RRCReconfiguration message (in NR-DC) or an EUTRA RRCConnectionReconfiguration message (in (NG)EN-DC) keeping SRB1 as split SRB, the network explicitly configures the primaryPath for the PDCP entity of SRB1 to refer to the MCG. In this version of the specification, only cell group ID corresponding to MCG is supported for DRBs when the SCG is deactivated. In MR-DC, the NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field. This field is also used for configuring Multi-path Primary Path defined in [5] for MP operation. In MP, when the primary path is indirect path, the field cellGroup and logicalChannel are absent, and the field primaryPathOnIndirectPath is set to true.

primaryPathOnIndirectPath Indicates that the primary path is SL indirect path, or primary path is associated with the N3C indirect path, for DRB when MP is configured.

remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. [RIL]: V050, XR

remainingTimeThresholdRLC-ReTx Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based retransmission as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.

sn-GapReport Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the uplink, as specified in TS 38.323 [5]. This field is only configured for DRBs. This field can be included only if outOfOrderDelivery is not configured.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath. This RLC entity belongs to the cell group of the direct path if primaryPathOnIndirectPath is set to true in MP case.

statusReportRequired For AM DRBs, AM MRBs and DAPS UM DRBs, indicates whether the DRB or the multicast MRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant addressed to CS-RNTI, as specified in TS 38.321 [3].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG and when the SCG is deactivated or for multi-path U2N Remote UEs not supporting splitDRB-WithUL-BothDirectIndirect. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB or a multicast MRB, the value of the field ehc-CID-Length for this DRB or multicast MRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

UplinkDataCompression field descriptions

bufferSize This field indicates the buffer size applied for UDC as specified in TS 38.323 [5]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary This field indicates which pre-defined dictionary is used for UDC as specified in TS 38.323 [5]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 38.323 [5], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB/multicast MRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs, this field is absent if duplication is not configured. Otherwise, this field is optional, need R. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer (except for multicast MRB) with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need M. Otherwise, the field is absent. |
| Rlc-AM-UM | In case of DRB, for RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. In case of multicast MRB, if multicast MRB is associated with at least one RLC AM entity, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of DRB setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer except MP split bearer with primary path on direct path. Otherwise the field is absent. |
| SplitBearerMP | The field is absent for SRBs. Otherwise, the field is optionally present, need R, when MP is configured. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC or if the UE supports user plane integrity protection when connected to E-UTRA/EPC (as specified in TS 33.401 [30]). Otherwise the field is absent. |
| Setup1 | This field is mandatory present in case of DRB setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| MRB-Initialization | This field is mandatory present in case of multicast MRB setup or in case UE configured with multicast reception in RRC_INACTIVE resumes the RRC connection. In case of PDCP re-establishment for multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |
| RLC-AM | For RLC AM, this field is optionally present, need R; Otherwise, the field is absent. |

=================================================NEXT CHANGE================================================================

–	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters. If this IE is used for MBS CFR, the following fields shall be absent: tci-StatesToAddModList, tci-StatesToReleaseList, zp-CSI-RS-ResourceToAddModList, minimumSchedulingOffsetK0, antennaPortsFieldPresenceDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2, aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, dmrs-SequenceInitializationDCI-1-2, harq-ProcessNumberSizeDCI-1-2, mcs-TableDCI-1-2, numberOfBitsForRV-DCI-1-2, pdsch-AggregationFactor, pdsch-TimeDomainAllocationListDCI-1-2, prb-BundlingTypeDCI-1-2, priorityIndicatorDCI-1-2, rateMatchPatternGroup1DCI-1-2, rateMatchPatternGroup2DCI-1-2, resourceAllocationType1GranularityDCI-1-2, vrb-ToPRB-InterleaverDCI-1-2, referenceOfSLIVDCI-1-2, resourceAllocationDCI-1-2, dataScramblingIdentityPDSCH2-r16, repetitionSchemeConfig, pdsch-ConfigDCI-1-3.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }           OPTIONAL,   -- Need S

bundleSizeSet2                      ENUMERATED { n4, wideband }                                     OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...,

[[

maxMIMO-Layers-r16                      SetupRelease { MaxMIMO-LayersDL-r16 }                               OPTIONAL,   -- Need M

minimumSchedulingOffsetK0-r16           SetupRelease { MinSchedulingOffsetK0-Values-r16 }                   OPTIONAL,   -- Need M

-- Start of the parameters for DCI format 1_2 introduced in V16.1.0

antennaPortsFieldPresenceDCI-1-2-r16            ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2-r16  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodicZP-CSI-RS-ResourceSetsToReleaseListDCI-1-2-r16 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16  SetupRelease { DMRS-DownlinkConfig }                        OPTIONAL,   -- Need M

dmrs-SequenceInitializationDCI-1-2-r16          ENUMERATED {enabled}                                        OPTIONAL,   -- Need S

harq-ProcessNumberSizeDCI-1-2-r16               INTEGER (0..4)                                              OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r16                            ENUMERATED {qam256, qam64LowSE}                             OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-2-r16                   INTEGER (0..2)                                              OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationListDCI-1-2-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

prb-BundlingTypeDCI-1-2-r16             CHOICE {

staticBundling-r16                      SEQUENCE {

bundleSize-r16                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling-r16                     SEQUENCE {

bundleSizeSet1-r16                      ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2-r16                      ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

}                                                                                                           OPTIONAL,   -- Need R

priorityIndicatorDCI-1-2-r16                ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

rateMatchPatternGroup1DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

rateMatchPatternGroup2DCI-1-2-r16           RateMatchPatternGroup                                           OPTIONAL,   -- Need R

resourceAllocationType1GranularityDCI-1-2-r16  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

vrb-ToPRB-InterleaverDCI-1-2-r16            ENUMERATED {n2, n4}                                             OPTIONAL,   -- Need S

referenceOfSLIVDCI-1-2-r16                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need S

resourceAllocationDCI-1-2-r16               ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 1_2 introduced in V16.1.0

priorityIndicatorDCI-1-1-r16             ENUMERATED {enabled}                                               OPTIONAL,   -- Need S

dataScramblingIdentityPDSCH2-r16         INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

pdsch-TimeDomainAllocationList-r16       SetupRelease { PDSCH-TimeDomainResourceAllocationList-r16 }        OPTIONAL,   -- Need M

repetitionSchemeConfig-r16               SetupRelease { RepetitionSchemeConfig-r16}                         OPTIONAL    -- Need M

]],

[[

repetitionSchemeConfig-v1630             SetupRelease { RepetitionSchemeConfig-v1630}                       OPTIONAL    -- Need M

]],

[[

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2-r17    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-1-2-r17           ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2-r17     ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-RetxDCI-1-2-r17               ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-2-r17                   ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

dl-OrJointTCI-StateList-r17                  CHOICE {

explicitlist                                 SEQUENCE {

dl-OrJointTCI-StateToAddModList-r17        SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-State

OPTIONAL,   -- Need N

dl-OrJointTCI-StateToReleaseList-r17       SEQUENCE (SIZE (1..maxNrofTCI-States)) OF TCI-StateId

OPTIONAL    -- Need N

},

unifiedTCI-StateRef-r17                  ServingCellAndBWP-Id-r17

}                                                                                                           OPTIONAL,   -- Need R

beamAppTime-r17                              ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336, spare2,

spare1}                                             OPTIONAL,   -- Need R

dummy                                        SetupRelease { Dummy-TDRA-List }                                    OPTIONAL,   -- Need M

dmrs-FD-OCC-DisabledForRank1-PDSCH-r17          ENUMERATED {true}                                           OPTIONAL,   -- Need R

minimumSchedulingOffsetK0-r17                   SetupRelease { MinSchedulingOffsetK0-Values-r17 }           OPTIONAL,   -- Need M

harq-ProcessNumberSizeDCI-1-2-v1700          INTEGER (0..5)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-1-r17            INTEGER (5)                                                    OPTIONAL,   -- Need R

mcs-Table-r17                                ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

mcs-TableDCI-1-2-r17                         ENUMERATED {qam1024}                                           OPTIONAL,   -- Need R

xOverheadMulticast-r17                       ENUMERATED {xOh6, xOh12, xOh18}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-4-2-r17                 ENUMERATED {enabled}                                           OPTIONAL,   -- Need S

sizeDCI-4-2-r17                              INTEGER (20..maxDCI-4-2-Size-r17)                              OPTIONAL    -- Need R

]],

[[

pdsch-TimeDomainAllocationListForMultiPDSCH-r17 SetupRelease { MultiPDSCH-TDRA-List-r17 }                   OPTIONAL    -- Need M

]],

[[

advancedReceiver-MU-MIMO-r18                 SetupRelease { AdvancedReceiver-MU-MIMO-r18 }                  OPTIONAL,   -- Need M

pdsch-ConfigDCI-1-3-r18                      SetupRelease { PDSCH-ConfigDCI-1-3-r18 }                       OPTIONAL    -- Need M

]],

[[

pdsch-ConfigDCI-1-3-v1860                    SetupRelease { PDSCH-ConfigDCI-1-3-v1860 }                     OPTIONAL    -- Need M

]],

[[

mg-CancellationDCI-1-1-r19                    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-1-2-r19                    ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-1-3-r19                    ENUMERATED {enabled}                                           OPTIONAL    -- Need R

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

MinSchedulingOffsetK0-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r16)

MinSchedulingOffsetK0-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK0-SchedulingOffset-r17)

MaxMIMO-LayersDL-r16 ::=                INTEGER (1..8)

PDSCH-ConfigDCI-1-3-r18 ::=                    SEQUENCE {

resourceAllocationDCI-1-3-r18                  ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-1-3-r18                            ENUMERATED {config1, config2, config3, spare1}               OPTIONAL, -- Cond DCI-1-3

resourceAllocationType1GranularityDCI-1-3-r18  ENUMERATED {n2,n4,n8,n16}                                    OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-1-3-r18                  INTEGER (0..2)                                               OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-1-3-r18              INTEGER (0..5)                                               OPTIONAL    -- Need R

}

PDSCH-ConfigDCI-1-3-v1860 ::=                  SEQUENCE {

enabledDefaultBeamForMultiCellScheduling-r18   ENUMERATED {enabled}

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

advancedReceiver-MU-MIMO A set of assistance information for R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions.

antennaPortsFieldPresenceDCI-1-2 Configure the presence of "Antenna ports" field in DCI format 1_2. When the field is configured, then the "Antenna ports" field is present in DCI format 1_2. Otherwise, the field size is set to 0 for DCI format 1_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 nor dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 is configured, this field is absent.

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList, aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2). The field aperiodic-ZP-CSI-RS-ResourceSetsToAddModList applies to DCI format 1_1 and the field aperiodic-ZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.2 and TS 38.212 [17] clause 7.3.1).

beamAppTime Indicates the first slot to apply the unified TCI indicated by DCI as specified in TS 38.214 Clause 5.1.5. The value n1 means 1 symbol, n2 two symbols and so on. The first slot is at least Y symbols indicated by beamAppTime parameter after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. The same value shall be configured for all serving cells in any one of the simultaneousU-TCI-UpdateListN configured in IE CellGroupConfig based on the smallest SCS of the active BWP.

dataScramblingIdentityPDSCH, dataScramblingIdentityPDSCH2 Identifier(s) used to initialize data scrambling (c_init) for PDSCH as specified in TS 38.211 [16], clause 7.3.1.1. The dataScramblingIdentityPDSCH2 is configured if coresetPoolIndex is configured with 1 for at least one CORESET in the same BWP.

dl-OrJointTCI-StateToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports, PDCCH DMRS ports, and CSI-RS, and in case of joint mode, also the PUSCH, PUCCH and SRS (see TS 38.214 [19], clause 5.1.5).

dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeA applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-DownlinkForPDSCH-MappingTypeB applies to DCI formats 1_1 and 1_3, and the field dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-FD-OCC-DisabledForRank1-PDSCH If configured, the UE may assume that the set of remaining orthogonal antenna ports, which are within the same code division multiplexing (CDM) group and have different frequency domain orthogonal cover codes (FD-OCC), are not associated with the PDSCH of another UE (see TS 38.214 [19], clause 5.1.6.2). It is applicable for PDSCH SCS of 480 and 960 kHz when rank 1 PDSCH with type-1 or type-2 DMRS is scheduled. If dmrs-TypeEnh-r18 is configured, this field is not configured.

dmrs-SequenceInitializationDCI-1_2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 1_2 If the field is absent, then the UE applies the value of 0 bit for the field "DMRS Sequence Initialization" in DCI format 1_2. If the field is present, then the UE applies the value of 1 bit as in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

harq-ProcessNumberSizeDCI-1-2 Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 [17], clause 7.3.1).

maxMIMO-Layers Indicates the maximum number of MIMO layers to be used for PDSCH in this DL BWP. If not configured, the UE uses the maxMIMO-Layers configuration in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs, when the UE operates in this BWP. The value of maxMIMO-Layers for a DL BWP shall be smaller than or equal to the value of maxMIMO-Layers configured in IE PDSCH-ServingCellConfig of the serving cell to which this BWP belongs. For MBS multicast, indicates the maximum number of MIMO layers to be used for group-common PDSCH of MBS multicast in this CFR. If not configured for CFR, the UE applies value 1. The value of maxMIMO-Layers for a CFR shall be smaller than or equal to the value of maxMIMO-Layers configured in PDSCH-ServingCellConfig IE of the serving cell to which this CFR belongs.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH for DCI formats 1_0, 1_1 and 1_3 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-Table-r17 is present for DCI formats 1_1 and 1_3, the network does not configure the field mcs-Table (without suffix). For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

mcs-TableDCI-1-2 Indicates which MCS table the UE shall use for PDSCH for DCI format 1_2 (see TS 38.214 [19], clause 5.1.3.1). If all fields are absent the UE applies the value 64QAM. If the field mcs-TableDCI-1-2-r17 is present, the network does not configure the field mcs-TableDCI-1-2-r16. For an (e)RedCap UE, the 256QAM MCS table for PDSCH is only supported if the UE indicates support of 256QAM for PDSCH.

mg-CancellationDCI-1-1 Indicates the presence of Measurement gap cancellation in DCI format 1_1 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-1-2 Indicates the presence of Measurement gap cancellation in DCI format 1_2 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-1-3 Indicates the presence of Measurement gap cancellation in DCI format 1_3 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17]. The field could only be configured if the co-scheduled cells are intra-band and have the same sub-carrier spacing.

minimumSchedulingOffsetK0 List of minimum K0 values. Minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s) (see TS 38.214 [19], clause 5.3.1).

numberOfBitsForRV-DCI-1-2 Configures the number of bits for "Redundancy version" in the DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent in PDSCH-Config which is not used for MBS CFR, the UE applies the value 1.

pdsch-HARQ-ACK-EnhType3DCI-1-2 When configured, enhanced Type 3 HARQ-ACK codebook triggering by DCI format 1_2 is enabled.

pdsch-HARQ-ACK-EnhType3DCI-Field-1-2 Enables the enhanced Type 3 codebook through a new DCI field to indicate the enhanced Type 3 HARQ-ACK codebook in DCI format 1_2 if the more than one enhanced Type 3 HARQ-ACK codebook is configured for the primary PUCCH cell group.

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2 When configured, DCI format 1_2 can request the UE to report A/N for all HARQ processes and all component carriers configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-RetxDCI-1-2 When configured, DCI format 1_2 can request the UE to perform a HARQ-ACK re-transmission on a PUCCH resource (see TS 38.213 [13], clause 9.1.5).

pdsch-TimeDomainAllocationList, pdsch-TimeDomainAllocationListDCI-1-2, pdsch-TimeDomainAllocationListForMultiPDSCH List of time-domain configurations for timing of DL assignment to DL data. The field pdsch-TimeDomainAllocationList (with or without suffix) applies to DCI format 1_0, DCI format 1_1 and DCI format 1_3 (see table 5.1.2.1.1-1 in TS 38.214 [19]), and if the field pdsch-TimeDomainAllocationListDCI-1-2 is not configured, to DCI format 1_2. If the field pdsch-TimeDomainAllocationListDCI-1-2 is configured, it applies to DCI format 1_2 (see table 5.1.2.1.1-1A in TS 38.214 [19]). The field pdsch-TimeDomainAllocationListForMultiPDSCH applies to DCI format 1_1. The network does not configure the pdsch-TimeDomainAllocationList-r16 simultaneously with the pdsch-TimeDomainAllocationList (without suffix) in the same PDSCH-Config.

prb-BundlingType, prb-BundlingTypeDCI-1-2 Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2. The field prb-BundlingType applies to DCI formats 1_1 and 1_3, and the field prb-BundlingTypeDCI-1-2 applies to DCI format 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.3).

priorityIndicatorDCI-1-1, priorityIndicatorDCI-1-2, priorityIndicatorDCI-4-2 Configure the presence of "priority indicator" in DCI format 1_1/1_2/4_2. When the field is absent in the IE, then 0 bit for "priority indicator" in DCI format 1_1/1_2/4_2. The field priorityIndicatorDCI-1-1 applies to DCI format 1_1, the field priorityIndicatorDCI-1-2 applies to DCI format 1_2 and the field priorityIndicatorDCI-4-2 applies to DCI format 4_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9).

pucch-sSCellDynDCI-1-2 When configured, PUCCH cell switching based on dynamic indication in DCI format 1_2 is enabled (see TS 38.213 [13], clause 9.A).

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set. If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config for MBS CFR and PDSCH-Config for the assoicated BWP, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for MBS CFR can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for the assoicated BWP.

rateMatchPatternGroup1, rateMatchPatternGroup1DCI-1-2 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup1 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup1DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2, rateMatchPatternGroup2DCI-1-2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1). The field rateMatchPatternGroup2 applies to DCI formats 1_1 and 1_3, and the field rateMatchPatternGroup2DCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the rate match patterns (see TS 38.214 [19], clause 5.1.4.1). If a RateMatchPattern with the same RateMatchPatternId is configured in both MBS CFR and its associated BWP, the entire RateMatchPattern configuration, including the set of RBs/REs indicated by the patterns for the rate matching around, shall be the same and they are counted as a single rate match pattern in the total configured rate match patterns as defined in TS 38.214 [19].

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH except PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

referenceOfSLIVDCI-1-2 Enable using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV for DCI format 1_2. When the RRC parameter enables the utilization of the new reference, the new reference is applied for TDRA entries with K0=0. For other entries (if any) in the same TDRA table, the reference is slot boundary as in Rel-15. PDSCH mapping type A is not supported with the new reference. The new reference of SLIV is not configured for a serving cell configured to be scheduled by cross-carrier scheduling on a scheduling cell with different numerology (see TS 38.212 [17] clause 7.3.1 and TS 38.214 [19] clause 5.1.2.1).

repetitionSchemeConfig Configure the UE with repetition schemes. The network does not configure repetitionSchemeConfig-r16 and repetitionSchemeConfig-v1630 simultaneously to setup in the same PDSCH-Config. The network does not configure this parameter and sfnSchemePDSCH in MIMOParam-r17 simultaneously in the same serving cell.

resourceAllocation, resourceAllocationDCI-1-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2). The field resourceAllocation applies to DCI format 1_1, and the field resourceAllocationDCI-1-2 applies to DCI format 1_2 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-2 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

sizeDCI-4-2 Indicates the size of DCI format 4-2 (see TS 38.213 [13], clause 10.1).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5). If unifiedTCI-StateType is configured for the serving cell, no element in this list is configured.

unifiedTCI-StateRef Provides the serving cell and BWP where the configuration for dl-OrJointTCI-StateToAddModList-r17 are defined. When this field is present, dl-OrJointTCI-StateToAddModList and dl-OrJointTCI-StateToReleaseList are not present. The value of unifiedTCI-StateType of current serving cell is the same in the serving cell indicated by unifiedTCI-StateRef.

vrb-ToPRB-Interleaver, vrb-ToPRB-InterleaverDCI-1-2 Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

xOverheadMulticast Accounts for an overhead from CSI-RS, CORESET etc. If the field is absent, the UE applies value xOh0 (see TS 38.214 [19]).

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).

PDSCH-ConfigDCI-1-3 field descriptions

enabledDefaultBeamForMultiCellScheduling This field indicates whether default beam selection for DCI format 1_3 scheduled PDSCH or aperiodic CSI-RS is enabled (see TS 38.214 [19], clause 5.1.5 and clause 5.2.1.5).

harq-ProcessNumberSizeDCI-1-3 Configure the number of bits for the field "HARQ process number" in DCI format 1_3 (see TS 38.212 [17], clause 7.3.1).

numberOfBitsForRV-DCI-1-3 Configures the number of bits for "Redundancy version" in the DCI format 1_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 5.1.2.1).

rbg-SizeDCI-1-3 Selection among config 1, config 2 and config 3 for RBG size for PDSCH scheduled by DCI format 1_3. The UE ignores this field if resourceAllocationDCI-1-3 is set to resourceAllocationType1. (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocationDCI-1-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 1_3 (see TS 38.214 [19], clause 5.1.2.2).

resourceAllocationType1GranularityDCI-1-3 Configure the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 1_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 5.1.2.2.2).

| Conditional Presence | Explanation |
| --- | --- |
| DCI-1-3 | This field is mandatory present when ScheduledCellListDCI-1-3 is configured to the serving cell. Otherwise, it is absent, Need R. |

=================================================NEXT CHANGE================================================================

–	PUSCH-Config

The IE PUSCH-Config is used to configure the UE specific PUSCH parameters applicable to a particular BWP.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPUSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

txConfig                                ENUMERATED {codebook, nonCodebook}                                  OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB        SetupRelease { DMRS-UplinkConfig }                                  OPTIONAL,   -- Need M

pusch-PowerControl                      PUSCH-PowerControl                                                  OPTIONAL,   -- Need M

frequencyHopping                        ENUMERATED {intraSlot, interSlot}                                   OPTIONAL,   -- Need S

frequencyHoppingOffsetLists             SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

OPTIONAL,   -- Need M

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pusch-TimeDomainAllocationList          SetupRelease { PUSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pusch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

mcs-TableTransformPrecoder              ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

transformPrecoder                       ENUMERATED {enabled, disabled}                                      OPTIONAL,   -- Need S

codebookSubset                          ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL, -- Cond codebookBased

maxRank                                 INTEGER (1..4)                                                OPTIONAL, -- Cond codebookBased

rbg-Size                                ENUMERATED { config2}                                         OPTIONAL, -- Need S

uci-OnPUSCH                             SetupRelease { UCI-OnPUSCH}                                   OPTIONAL, -- Need M

tp-pi2BPSK                              ENUMERATED {enabled}                                          OPTIONAL, -- Need S

...,

[[

minimumSchedulingOffsetK2-r16           SetupRelease { MinSchedulingOffsetK2-Values-r16 }             OPTIONAL,  -- Need M

ul-AccessConfigListDCI-0-1-r16          SetupRelease { UL-AccessConfigListDCI-0-1-r16 }               OPTIONAL,  -- Need M

-- Start of the parameters for DCI format 0_2 introduced in V16.1.0

harq-ProcessNumberSizeDCI-0-2-r16                       INTEGER (0..4)                                OPTIONAL,   -- Need R

dmrs-SequenceInitializationDCI-0-2-r16                  ENUMERATED {enabled}                          OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-2-r16                           INTEGER (0..2)                                OPTIONAL,   -- Need R

antennaPortsFieldPresenceDCI-0-2-r16                    ENUMERATED {enabled}                          OPTIONAL,   -- Need S

dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2-r16            SetupRelease { DMRS-UplinkConfig }            OPTIONAL,   -- Need M

frequencyHoppingDCI-0-2-r16                             CHOICE {

pusch-RepTypeA                                          ENUMERATED {intraSlot, interSlot},

pusch-RepTypeB                                          ENUMERATED {interRepetition, interSlot}

}                                                                                                     OPTIONAL,   -- Need S

frequencyHoppingOffsetListsDCI-0-2-r16  SetupRelease { FrequencyHoppingOffsetListsDCI-0-2-r16}        OPTIONAL,  -- Need M

codebookSubsetDCI-0-2-r16               ENUMERATED {fullyAndPartialAndNonCoherent, partialAndNonCoherent,nonCoherent}

OPTIONAL,   -- Cond codebookBased

invalidSymbolPatternIndicatorDCI-0-2-r16                ENUMERATED {enabled}                          OPTIONAL,   -- Need S

maxRankDCI-0-2-r16                                      INTEGER (1..4)                                OPTIONAL,   -- Cond codebookBased

mcs-TableDCI-0-2-r16                                    ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

mcs-TableTransformPrecoderDCI-0-2-r16                   ENUMERATED {qam256, qam64LowSE}               OPTIONAL,   -- Need S

priorityIndicatorDCI-0-2-r16                            ENUMERATED {enabled}                          OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-2-r16                       ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}  OPTIONAL,  -- Need R

resourceAllocationDCI-0-2-r16                           ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

resourceAllocationType1GranularityDCI-0-2-r16           ENUMERATED { n2,n4,n8,n16 }                   OPTIONAL,   -- Need S

uci-OnPUSCH-ListDCI-0-2-r16                             SetupRelease { UCI-OnPUSCH-ListDCI-0-2-r16}   OPTIONAL,   -- Need M

pusch-TimeDomainAllocationListDCI-0-2-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

-- End of the parameters for DCI format 0_2 introduced in V16.1.0

-- Start of the parameters for DCI format 0_1 introduced in V16.1.0

pusch-TimeDomainAllocationListDCI-0-1-r16               SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,   -- Need M

invalidSymbolPatternIndicatorDCI-0-1-r16          ENUMERATED {enabled}                                OPTIONAL,   -- Need S

priorityIndicatorDCI-0-1-r16                      ENUMERATED {enabled}                                OPTIONAL,   -- Need S

pusch-RepTypeIndicatorDCI-0-1-r16                 ENUMERATED { pusch-RepTypeA, pusch-RepTypeB}        OPTIONAL,   -- Need R

frequencyHoppingDCI-0-1-r16                 ENUMERATED {interRepetition, interSlot}                   OPTIONAL,   -- Cond RepTypeB

uci-OnPUSCH-ListDCI-0-1-r16                 SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }             OPTIONAL,  -- Need M

-- End of the parameters for DCI format 0_1 introduced in V16.1.0

invalidSymbolPattern-r16                    InvalidSymbolPattern-r16                                  OPTIONAL,   -- Need S

pusch-PowerControl-v1610                SetupRelease {PUSCH-PowerControl-v1610}                       OPTIONAL,   -- Need M

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpowerMode2}         OPTIONAL,   -- Need R

pusch-TimeDomainAllocationListForMultiPUSCH-r16  SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 }

OPTIONAL,  --  Need M

numberOfInvalidSymbolsForDL-UL-Switching-r16        INTEGER (1..4)                                    OPTIONAL    -- Cond RepTypeB2

]],

[[

ul-AccessConfigListDCI-0-2-r17          SetupRelease { UL-AccessConfigListDCI-0-2-r17 }               OPTIONAL,  -- Need M

betaOffsetsCrossPri0-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri1-r17                SetupRelease { BetaOffsetsCrossPriSel-r17 }                   OPTIONAL,  -- Need M

betaOffsetsCrossPri0DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

betaOffsetsCrossPri1DCI-0-2-r17         SetupRelease { BetaOffsetsCrossPriSelDCI-0-2-r17 }            OPTIONAL,  -- Need M

mappingPattern-r17                      ENUMERATED {cyclicMapping, sequentialMapping}                 OPTIONAL,  -- Cond SRSsets

secondTPCFieldDCI-0-1-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

secondTPCFieldDCI-0-2-r17               ENUMERATED {enabled}                                          OPTIONAL,  -- Need R

sequenceOffsetForRV-r17                 INTEGER (0..3)                                                OPTIONAL,  -- Need R

ul-AccessConfigListDCI-0-1-r17          SetupRelease { UL-AccessConfigListDCI-0-1-r17 }                OPTIONAL,  -- Need M

minimumSchedulingOffsetK2-r17           SetupRelease { MinSchedulingOffsetK2-Values-r17 }              OPTIONAL,  -- Need M

availableSlotCounting-r17               ENUMERATED { enabled }                                         OPTIONAL,  -- Need S

dmrs-BundlingPUSCH-Config-r17           SetupRelease { DMRS-BundlingPUSCH-Config-r17 }                 OPTIONAL,  -- Need M

harq-ProcessNumberSizeDCI-0-2-v1700     INTEGER (5)                                                    OPTIONAL,  -- Need R

harq-ProcessNumberSizeDCI-0-1-r17       INTEGER (5)                                                    OPTIONAL,  -- Need R

mpe-ResourcePoolToAddModList-r17       SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-Resource-r17    OPTIONAL,  -- Need N

mpe-ResourcePoolToReleaseList-r17      SEQUENCE (SIZE(1..maxMPE-Resources-r17)) OF MPE-ResourceId-r17  OPTIONAL   -- Need N

]],

[[

maxRank-v1810                           INTEGER (5..8)                                              OPTIONAL, -- Need R

sTx-2Panel-r18                          ENUMERATED {enabled}                                           OPTIONAL,  -- Need R

multipanelSchemeSDM-r18                 SDM-Scheme-r18                                                 OPTIONAL,  -- Need R

multipanelSchemeSFN-r18                 SFN-Scheme-r18                                                 OPTIONAL,  -- Need R

codebookTypeUL-r18                      SetupRelease { CodebookTypeUL-r18 }                            OPTIONAL,  -- Need M

applyIndicatedTCI-State-r18             ENUMERATED {first, second}                                     OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-1-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

dynamicTransformPrecoderFieldPresenceDCI-0-2-r18    ENUMERATED {enabled}                               OPTIONAL,  -- Need R

pusch-ConfigDCI-0-3-r18                 SetupRelease { PUSCH-ConfigDCI-0-3-r18 }                       OPTIONAL   -- Need M

]],

[[

mg-CancellationDCI-0-1-r19              ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-0-2-r19              ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

mg-CancellationDCI-0-3-r19              ENUMERATED {enabled}                                           OPTIONAL    -- Need R

]]

}

UCI-OnPUSCH ::=                         SEQUENCE {

betaOffsets                             CHOICE {

dynamic                             SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic                          BetaOffsets

}                                                                                                 OPTIONAL, -- Need M

scaling                                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

MinSchedulingOffsetK2-Values-r16 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r16)

MinSchedulingOffsetK2-Values-r17 ::=    SEQUENCE (SIZE (1..maxNrOfMinSchedulingOffsetValues-r16)) OF INTEGER (0..maxK2-SchedulingOffset-r17)

UCI-OnPUSCH-DCI-0-2-r16 ::=             SEQUENCE {

betaOffsetsDCI-0-2-r16                  CHOICE {

dynamicDCI-0-2-r16                      CHOICE {

oneBit-r16                              SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16                             SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticDCI-0-2-r16          BetaOffsets

}                                                                                                 OPTIONAL,   -- Need M

scalingDCI-0-2-r16                 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

FrequencyHoppingOffsetListsDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)

UCI-OnPUSCH-ListDCI-0-2-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH-DCI-0-2-r16

UCI-OnPUSCH-ListDCI-0-1-r16 ::=  SEQUENCE (SIZE (1..2)) OF UCI-OnPUSCH

UL-AccessConfigListDCI-0-1-r16 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

UL-AccessConfigListDCI-0-1-r17 ::= SEQUENCE (SIZE (1..3)) OF INTEGER (0..2)

UL-AccessConfigListDCI-0-2-r17 ::= SEQUENCE (SIZE (1..64)) OF INTEGER (0..63)

BetaOffsetsCrossPriSel-r17 ::= CHOICE {

dynamic-r17         SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17,

semiStatic-r17          BetaOffsetsCrossPri-r17

}

BetaOffsetsCrossPriSelDCI-0-2-r17 ::= CHOICE {

dynamicDCI-0-2-r17      CHOICE {

oneBit-r17              SEQUENCE (SIZE (2)) OF BetaOffsetsCrossPri-r17,

twoBits-r17             SEQUENCE (SIZE (4)) OF BetaOffsetsCrossPri-r17

},

semiStaticDCI-0-2-r17   BetaOffsetsCrossPri-r17

}

MPE-Resource-r17 ::=        SEQUENCE {

mpe-ResourceId-r17          MPE-ResourceId-r17,

cell-r17                    ServCellIndex                                                         OPTIONAL,    -- Need R

additionalPCI-r17           AdditionalPCIIndex-r17                                                OPTIONAL,    -- Need R

mpe-ReferenceSignal-r17     CHOICE {

csi-RS-Resource-r17         NZP-CSI-RS-ResourceId,

ssb-Resource-r17            SSB-Index

}

}

MPE-ResourceId-r17 ::=      INTEGER (1..maxMPE-Resources-r17)

SDM-Scheme-r18   ::=        SEQUENCE {

maxRankSDM-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSDM-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

SFN-Scheme-r18   ::=        SEQUENCE {

maxRankSFN-r18              INTEGER (1..2)                                                        OPTIONAL,    -- Need R

maxRankSFN-DCI-0-2-r18      INTEGER (1..2)                                                        OPTIONAL     -- Need R

}

CodebookTypeUL-r18 ::=      CHOICE {

codebook1-r18               ENUMERATED {ng1n4n1, ng1n2n2},

codebook2-r18               ENUMERATED {ng2},

codebook3-r18               ENUMERATED {ng4},

codebook4-r18               ENUMERATED {ng8}

}

PUSCH-ConfigDCI-0-3-r18 ::=                   SEQUENCE {

resourceAllocationDCI-0-3-r18                 ENUMERATED {resourceAllocationType0, resourceAllocationType1, dynamicSwitch}

OPTIONAL,   -- Need M

rbg-SizeDCI-0-3-r18                           ENUMERATED {config2, config3}                                  OPTIONAL,   -- Need S

resourceAllocationType1GranularityDCI-0-3-r18 ENUMERATED {n2,n4,n8,n16}                                      OPTIONAL,   -- Need S

numberOfBitsForRV-DCI-0-3-r18                 INTEGER (0..2)                                                 OPTIONAL,   -- Need R

harq-ProcessNumberSizeDCI-0-3-r18             INTEGER (0..5)                                                 OPTIONAL,   -- Need R

uci-OnPUSCH-ListDCI-0-3-r18                   SetupRelease { UCI-OnPUSCH-ListDCI-0-1-r16  }                  OPTIONAL    -- Need M

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

PUSCH-Config field descriptions

antennaPortsFieldPresenceDCI-0-2 Configure the presence of "Antenna ports" field in DCI format 0_2. When the field is configured, then the "Antenna ports" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17], clause 7.3.1.1.3). If neither dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 nor dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, this field is absent.

applyIndicatedTCI-State This field indicates, for a PUSCH transmission, if UE applies the first or the second "indicated" UL only TCI or joint TCI as specified in TS 38.214 [19], clause 6.1.

availableSlotCounting Indicate whether PUSCH repetitions counted on the basis of available slots is enabled. If the field is absent, PUSCH repetitions counted on the basis of available slots is disabled.

betaOffsetsCrossPri0, betaOffsetsCrossPri1, betaOffsetsCrossPri0DCI-0-2, betaOffsetsCrossPri1DCI-0-2 Selection between and configuration of dynamic and semi-static beta-offset for multiplexing HARQ-ACK on dynamically scheduled PUSCH with different priorities, see TS 38.213 [13], clause 9.3. The field betaOffsetsCrossPrio0 indicates multiplexing low priority (LP) HARQ-ACK on dynamically scheduled high priority (HP) PUSCH. The field betaOffsetsCrossPrio1 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH. The field betaOffsetsCrossPrio0DCI-0-2 indicates multiplexing LP HARQ-ACK on dynamically scheduled HP PUSCH by DCI format 0_2. The field betaOffsetsCrossPrio1DCI-0-2 indicates multiplexing HP HARQ-ACK on dynamically scheduled LP PUSCH by DCI format 0_2.

codebookSubset, codebookSubsetDCI-0-2 Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities (see TS 38.214 [19], clause 6.1.1.1). The field codebookSubset applies to DCI formats 0_1 and 0_3, and the field codebookSubsetDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1).

codebookTypeUL Configures a codebook and the corresponding number of antenna port groups for codebook-based transmission of PUSCH with 8 antenna ports, see TS 38.211 [16], tables 6.3.1.5-9 to 6.3.1.5-47, and table 6.3.1.5-8 respectively). The values ng1n4n1 and ng1n2n2 correspond to codebooks with one antenna port group (Ng=1), while ng2, ng4, and ng8 correspond to codebooks with Ng=2, 4, and 8 antenna port groups, respectively.

dataScramblingIdentityPUSCH Identifier used to initialise data scrambling (c_init) for PUSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 6.3.1.1).

dmrs-BundlingPUSCH-Config Configure the parameters for DMRS bundling for PUSCH (see TS 38.214 [19], clause 6.1.7). In this release, this is not applicable to FR2-2.

dmrs-SequenceInitializationDCI-0-2 Configure whether the field "DMRS Sequence Initialization" is present or not in DCI format 0_2. If the field is absent, then 0 bit for the field "DMRS Sequence Initialization" in DCI format 0_2. If the field is present, then the number of bits is determined in the same way as DCI format 0_1 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeA, dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type A (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeA applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dmrs-UplinkForPUSCH-MappingTypeB, dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 DMRS configuration for PUSCH transmissions using PUSCH mapping type B (chosen dynamically via PUSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B. The field dmrs-UplinkForPUSCH-MappingTypeB applies to DCI formats 0_1 and 0_3, and the field dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

dynamicTransformPrecoderFieldPresenceDCI-0-1 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_1. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_1. Otherwise, the field size is set to 0 for DCI format 0_1 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

dynamicTransformPrecoderFieldPresenceDCI-0-2 Configure the presence of "Dynamic Transform Precoder" field in DCI format 0_2. When the field is configured, then the "Dynamic Transform Precoder" field is present in DCI format 0_2. Otherwise, the field size is set to 0 for DCI format 0_2 (See TS 38.212 [17]). The network ensures dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 and twoPHRMode-r17 cannot be configured at the same time for a UE.

frequencyHopping The value intraSlot enables 'Intra-slot frequency hopping' and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for 'pusch-RepTypeA' (see TS 38.214 [19], clause 6.3). The field frequencyHopping applies to DCI formats 0_0, 0_1 and 0_3 for 'pusch-RepTypeA'.

frequencyHoppingDCI-0-1 Indicates the frequency hopping scheme for DCI format 0_1 when pusch-RepTypeIndicatorDCI-0-1 is set to 'pusch-RepTypeB', The value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. If the field is absent, frequency hopping is not configured for DCI format 0_1 for 'pusch-RepTypeB' (see TS 38.214 [19], clause 6.1).

frequencyHoppingDCI-0-2 Indicate the frequency hopping scheme for DCI format 0_2. The value intraSlot enables 'intra-slot frequency hopping', and the value interRepetition enables 'Inter-repetition frequency hopping', and the value interSlot enables 'Inter-slot frequency hopping'. When pusch-RepTypeIndicatorDCI-0-2 is not set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'intra-slot frequency hopping and 'inter-slot frequency hopping' if enabled. When pusch-RepTypeIndicatorDCI-0-2 is set to 'pusch-RepTypeB', the frequency hopping scheme can be chosen between 'inter-repetition frequency hopping' and 'inter-slot frequency hopping' if enabled. If the field is absent, frequency hopping is not configured for DCI format 0_2 (see TS 38.214 [19], clause 6.3).

frequencyHoppingOffsetLists, frequencyHoppingOffsetListsDCI-0-2 Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 configured grant activation (see TS 38.214 [19], clause 6.3). The field frequencyHoppingOffsetLists applies to DCI formats 0_0, 0_1 and 0_3, and the field frequencyHoppingOffsetListsDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.3).

harq-ProcessNumberSizeDCI-0-2 Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

invalidSymbolPattern Indicates one pattern for invalid symbols for PUSCH transmission repetition type B applicable to both DCI format 0_1 and 0_2. If InvalidSymbolPattern is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols (see TS 38.214 [19] clause 6.1).

invalidSymbolPatternIndicatorDCI-0-1, invalidSymbolPatternIndicatorDCI-0-2 Indicates the presence of an additional bit in the DCI format 0_1/0_2. If invalidSymbolPattern is absent, then both invalidSymbolPatternIndicatorDCI-0-1 and invalidSymbolPatternIndicatorDCI-0-2 are absent. The field invalidSymbolPatternIndicatorDCI-0-1 applies to the DCI format 0_1 and the field invalidSymbolPatternIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19] clause 6.1). If the field is absent, the UE behaviour is specified in TS 38.214 [19], clause 6.1.2.1.

mappingPattern Indicates whether the UE should follow Cyclical mapping pattern or Sequential mapping pattern for when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook' for PUSCH transmission and the PUSCH transmission occasions are associated with both SRS resource sets.

maxRank, maxRankDCI-0-2 Subset of PMIs addressed by TRIs from 1 to ULmaxRank (see TS 38.214 [19], clause 6.1.1.1). The field maxRank applies to DCI formats 0_1 and 0_3, and the field maxRankDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.1.1). If network configures maxRank-v1810 UE ignores maxRank (without suffix).

mcs-Table, mcs-TableFormat0-2 Indicates which MCS table the UE shall use for PUSCH without transform precoder (see TS 38.214 [19], clause 6.1.4.1). If the field is absent the UE applies the value 64QAM. The field mcs-Table applies to DCI formats 0_0, 0_1 and 0_3, and the field mcs-TableDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mcs-TableTransformPrecoder, mcs-TableTransformPrecoderDCI-0-2 Indicates which MCS table the UE shall use for PUSCH with transform precoding (see TS 38.214 [19], clause 6.1.4.1) If the field is absent the UE applies the value 64QAM. The field mcs-TableTransformPrecoder applies to DCI formats 0_0, 0_1 and 0_3, and the field mcs-TableTransformPrecoderDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.4.1).

mg-CancellationDCI-0-1 Indicates the presence of Measurement gap cancellation in DCI format 0_1 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-0-2 Indicates the presence of Measurement gap cancellation in DCI format 0_2 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17].

mg-CancellationDCI-0-3 Indicates the presence of Measurement gap cancellation in DCI format 0_3 to indicate whether TX/RX is enabled in the gap/restriction as specified in TS 38.212 [17]. The field could only be configured if the co-scheduled cells are intra-band and have the same sub-carrier spacing.

minimumSchedulingOffsetK2 List of minimum K2 values. Minimum K2 parameter denotes minimum applicable value(s) for the Time domain resource assignment table for PUSCH (see TS 38.214 [19], clause 6.1.2.1).

mpe-ResourcePoolToAddModList List of SSB/CSI-RS resources for P-MPR reporting. Each resource is configured with serving cell index where the resource is configured for the UE. The additionalPCI is configured only if the resource is SSB. For each resource, if neither cell nor additionalPCI is present, the SSB/CSI-RS resource is from the serving cell where the PUSCH-Config is configured.

multipanelSchemeSDM Configures UE with a multiple panel simultaneous uplink transmission SDM scheme for PUSCH, as specified in TS 38.214 [19], clause 6.1. Network does not configure multipanelSchemeSDM with multipanelSchemeSFN. When this parameter is configured, two SRS resource sets with usage for codebook or noncodebook are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2.

multipanelSchemeSFN Configures UE with a multiple panel simultaneous uplink transmission SFN scheme for PUSCH, as specified in TS 38.214 [19], clause 6.1. Network does not configure multipanelSchemeSFN with multipanelSchemeSDM. When this parameter is configured, two SRS resource sets with usage for codebook or noncodebook are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2.

numberOfBitsForRV-DCI-0-2 Configures the number of bits for "Redundancy version" in the DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

numberOfInvalidSymbolsForDL-UL-Switching Indicates the number of symbols after the last semi-static DL symbol that are invalid symbols for PUSCH repetition Type B. If it is absent, no symbol is explicitly defined for DL-to-UL switching (see TS 38.214 [19], clause 6.1).

priorityIndicatorDCI-0-1, priorityIndicatorDCI-0-2 Configures the presence of "priority indicator" in DCI format 0_1/0_2. When the field is absent in the IE, then the UE shall apply 0 bit for "Priority indicator" in DCI format 0_1/0_2. The field priorityIndicatorDCI-0-1 applies to DCI format 0_1 and the field priorityIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17] clause 7.3.1 and TS 38.213 [13] clause 9).

pusch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 6.1.2.1). If the field is absent the UE applies the value 1.

pusch-PowerControl Configures power control parameters PUSCH transmission.

pusch-RepTypeIndicatorDCI-0-1, pusch-RepTypeIndicatorDCI-0-2 Indicates whether UE follows the behavior for "PUSCH repetition type A" or the behavior for "PUSCH repetition type B" for the PUSCH scheduled by DCI format 0_1/0_2 and for Type 2 CG associated with the activating DCI format 0_1/0_2.The value pusch-RepTypeA enables the 'PUSCH repetition type A' and the value pusch-RepTypeB enables the 'PUSCH repetition type B'. The field pusch-RepTypeIndicatorDCI-0-1 applies to DCI format 0_1 and the field pusch-RepTypeIndicatorDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2.1).

pusch-TimeDomainAllocationList List of time domain allocations for timing of UL assignment to UL data (see TS 38.214 [19], table 6.1.2.1.1-1). The field pusch-TimeDomainAllocationList applies to DCI format 0_0, or DCI formats 0_1 and 0_3 when the field pusch-TimeDomainAllocationListDCI-0-1 is not configured (see TS 38.214 [19], table 6.1.2.1.1-1 and tables 6.1.2.1.1-1A and 6.1.2.1.1-1C). The network does not configure the pusch-TimeDomainAllocationList (without suffix) simultaneously with the pusch-TimeDomainAllocationListDCI-0-2-r16 or pusch-TimeDomainAllocationListDCI-0-1-r16 or pusch-TimeDomainAllocationListForMultiPUSCH-r16.

pusch-TimeDomainAllocationListDCI-0-1 Configuration of the time domain resource allocation (TDRA) table for DCI formats 0_1 and 0_3 (see TS 38.214 [19], clause 6.1, tables 6.1.2.1.1-1A and 6.1.2.1.1-1C).

pusch-TimeDomainAllocationListDCI-0-2 Configuration of the time domain resource allocation (TDRA) table for DCI format 0_2 (see TS 38.214 [19], clause 6.1.2, table 6.1.2.1.1-1B).

pusch-TimeDomainAllocationListForMultiPUSCH Configuration of the time domain resource allocation (TDRA) table for multiple PUSCH (see TS 38.214 [19], clause 6.1.2). The network configures at most 64 rows in this TDRA table in PUSCH-TimeDomainResourceAllocationList-r16 configured by this field. This field is not configured simultaneously with pusch-AggregationFactor if extendedK2 is not configured. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the pusch-TimeDomainAllocationListDCI-0-1-r16. The network does not configure the pusch-TimeDomainAllocationListForMultiPUSCH-r16 simultaneously with the numberOfSlotsTBoMS-r17.

rbg-Size Selection between configuration 1 and configuration 2 for RBG size for PUSCH except PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocation is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocation, resourceAllocationDCI-0-2 Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 6.1.2). The field resourceAllocation applies to DCI format 0_1 and the field resourceAllocationDCI-0-2 applies to DCI format 0_2 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-2 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_2. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

secondTPCFieldDCI-0-1, secondTPCFieldDCI-0-2 A second TPC field can be configured via RRC for DCI-0-1 and DCI-0-2. Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to "closedLoopIndex" value = 0 and 1,

sequenceOffsetForRV Configures the RV offset for the starting RV for the first repetition (first actual repetition in PUSCH repetition Type B) towards the second 'SRS resource set' for PUSCH configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

sTx-2Panel Parameter to enable PUSCH+PUSCH multiple panel simultaneous uplink transmission, as specified in TS 38.214 [19], clause 6.1.

tp-pi2BPSK Enables pi/2-BPSK modulation with transform precoding if the field is present and disables it otherwise.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList).

txConfig Whether UE uses codebook based or non-codebook based transmission (see TS 38.214 [19], clause 6.1.1). If the field is absent, the UE transmits PUSCH on one antenna port, see TS 38.214 [19], clause 6.1.1.

uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2. The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCH-ListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 List of the combinations of cyclic prefix extension, channel access priority class (CAPC), and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable for DCI format 0_1 and DCI format 0_2, respectively. The fields ul-AccessConfigListDCI-0-1-r16 and ul-AccessConfigListDCI-0-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.1.2-35). The field ul-AccessConfigListDCI-0-1-r17 only contains a list of UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.1.2-35A).

ul-FullPowerTransmission Configures the UE with UL full power transmission mode as specified in TS 38.213 [13]. This field is not configured if ul-powerControl is configured in the BWP-UplinkDedicated in which the PUCCH-Config is included.

PUSCH-ConfigDCI-0-3 field descriptions

harq-ProcessNumberSizeDCI-0-3 Configure the number of bits for the field "HARQ process number" in DCI format 0_3 (see TS 38.212 [17], clause 7.3.1).

numberOfBitsForRV-DCI-0-3 Configures the number of bits for "Redundancy version" in the DCI format 0_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.214 [19], clause 6.1.2.1).

rbg-SizeDCI-0-3 Selection among configuration 1, configuration 2 and configuration 3 for RBG size for PUSCH scheduled by DCI format 0_3. The UE does not apply this field if resourceAllocationDCI-0-3 is set to resourceAllocationType1. Otherwise, the UE applies the value config1 when the field is absent (see TS 38.214 [19], clause 6.1.2.2.1).

resourceAllocationDCI-0-3 Configuration of resource allocation type 0 and resource allocation type 1 for DCI format 0_3 (see TS 38.214 [19], clause 6.1.2).

resourceAllocationType1GranularityDCI-0-3 Configures the scheduling granularity applicable for both the starting point and length indication for resource allocation type 1 in DCI format 0_3. If this field is absent, the granularity is 1 PRB (see TS 38.214 [19], clause 6.1.2.2.2).

uci-OnPUSCH-ListDCI-0-3 Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_3 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

SDM-Scheme field descriptions

maxRankSDM, maxRankSDM-DCI-0-2 configure maximal number of MIMO layers of each panel in SDM scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

SFN-Scheme field descriptions

maxRankSFN, maxRankSFN-DCI-0-2 configure maximal number of MIMO layers of each panel in SFN scheme for codebook based PUSCH or for DCI 0_2 for codebook based PUSCH.

UCI-OnPUSCH field descriptions

betaOffsets Selection between and configuration of dynamic and semi-static beta-offset for DCI formats other than DCI format 0_2. If the field is not configured, the UE applies the value 'semiStatic' (see TS 38.213 [13], clause 9.3).

scaling Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI formats other than DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on. The value configured herein is applicable for PUSCH with configured grant (see TS 38.212 [17], clause 6.3).

UCI-OnPUSCH-DCI-0-2 field descriptions

betaOffsetsDCI-0-2 Configuration of beta-offset for DCI format 0_2. If semiStaticDCI-0-2 is chosen, the UE shall apply the value of 0 bit for the field of beta offset indicator in DCI format 0_2. If dynamicDCI-0-2 is chosen, the UE shall apply the value of 1 bit or 2 bits for the field of beta offset indicator in DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

dynamicDCI-0-2 Indicates the UE applies the value 'dynamic' for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.3).

semiStaticDCI-0-2 Indicates the UE applies the value 'semiStatic' for DCI format 0_2. (see TS 38.212 [17], clause 7.3.1 and see TS 38.213 [13], clause 9.3).

scalingDCI-0-2 Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH for DCI format 0_2. Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on (see TS 38.212 [17], clause 6.3).

| Conditional Presence | Explanation |
| --- | --- |
| codebookBased | The field is mandatory present if txConfig is set to codebook and absent otherwise. |
| RepTypeB | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 is set to pusch-RepTypeB. It is absent otherwise. |
| RepTypeB2 | The field is optionally present, Need S, if pusch-RepTypeIndicatorDCI-0-1 or pusch-RepTypeIndicatorDCI-0-2 is set to pusch-RepTypeB. It is absent otherwise. |
| SRSsets | This field is mandatory present when UE is configured with two SRS sets in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage codebook or non-codebook and none of multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel is configured. It is absent otherwise. |

=================================================NEXT CHANGE================================================================

–	RLC-BearerConfig

The IE RLC-BearerConfig is used to configure an RLC entity, a corresponding logical channel in MAC and the linking to a PDCP entity (served radio bearer).

RLC-BearerConfig information element

-- ASN1START

-- TAG-RLC-BEARERCONFIG-START

RLC-BearerConfig ::=                        SEQUENCE {

logicalChannelIdentity                      LogicalChannelIdentity,

servedRadioBearer                           CHOICE {

srb-Identity                                SRB-Identity,

drb-Identity                                DRB-Identity

}                                                                                               OPTIONAL,   -- Cond LCH-SetupOnly

reestablishRLC                              ENUMERATED {true}                                   OPTIONAL,   -- Need N

rlc-Config                                  RLC-Config                                          OPTIONAL,   -- Cond LCH-Setup

mac-LogicalChannelConfig                    LogicalChannelConfig                                OPTIONAL,   -- Cond LCH-Setup

...,

[[

rlc-Config-v1610                            RLC-Config-v1610                                    OPTIONAL    -- Need R

]],

[[

rlc-Config-v1700                            RLC-Config-v1700                                    OPTIONAL,   -- Need R

logicalChannelIdentityExt-r17               LogicalChannelIdentityExt-r17                       OPTIONAL,   -- Cond LCH-SetupModMRB

multicastRLC-BearerConfig-r17               MulticastRLC-BearerConfig-r17                       OPTIONAL,   -- Cond LCH-SetupOnlyMRB

servedRadioBearerSRB4-r17                   SRB-Identity-v1700                                  OPTIONAL    -- Cond LCH-SetupOnlySRB4

]],

[[

servedRadioBearerSRB5-r18                   SRB-Identity-v1800                                  OPTIONAL    -- Cond LCH-SetupOnlySRB5

]],

[[

rlc-Config-v19xy                            RLC-Config-v19xy                                    OPTIONAL    -- Need R

]]

}

MulticastRLC-BearerConfig-r17 ::=           SEQUENCE {

servedMBS-RadioBearer-r17                   MRB-Identity-r17,

isPTM-Entity-r17                            ENUMERATED {true}                                   OPTIONAL    -- Need S

}

LogicalChannelIdentityExt-r17 ::=           INTEGER (320..65855)

-- TAG-RLC-BEARERCONFIG-STOP

-- ASN1STOP

RLC-BearerConfig field descriptions

isPTM-Entity If configured, indicates that the RLC entity is used for PTM reception. When the field is absent the RLC entity is used for PTP transmission/reception.

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer.

logicalChannelIdentityExt Extended logical channel ID used commonly for the MAC logical channel and for the RLC bearer for PTM reception. If this field is configured, the UE shall ignore logicalChannelIdentity.

reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2, multicast MRBs and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true. The network does not include this field if servedRadioBearer is set to drb-Identity and the RLC-BearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE. For DRBs, network doesn't include this field if the RLC-BearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE. Network doesn't include this field if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE which is received within a MCG RRCReconfiguration message via SRB1.

rlc-Config Determines the RLC mode (UM, AM) and provides corresponding parameters. RLC mode reconfiguration can only be performed by DRB/multicast MRB release/addition or full configuration. The network may configure rlc-Config-v1610 only when rlc-Config (without suffix) is set to am.

servedMBS-RadioBearer Associates the RLC Bearer with a multicast MRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedMBS-RadioBearer.

servedRadioBearer, servedRadioBearerSRB4, servedRadioBearerSRB5 Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling restrictions (moreThanOneRLC in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-Setup | This field is mandatory present upon creation of a new logical channel for a DRB or a multicast MRB or SRB4 or SRB5. This field is optionally present, Need S, upon creation of a new logical channel for an SRB except SRB4 and SRB5. It is optionally present, Need M, otherwise. |
| LCH-SetupModMRB | This field is optionally present upon creation of a new logical channel for PTM reception for a multicast MRB. If this field is included upon creation of a new logical channel for PTM reception for a multicast MRB, it shall be present when modifying this logical channel. The field is absent for logical channels configured for an SRB and a DRB. |
| LCH-SetupOnly | This field is mandatory present upon creation of a new logical channel for a DRB or an SRB (servedRadioBearer). It is absent, Need M otherwise. |
| LCH-SetupOnlyMRB | This field is mandatory present upon creation of a new logical channel for a multicast MRB and upon modification of MRB-Identity of the served MRB. It is absent, Need M otherwise. |
| LCH-SetupOnlySRB4 | This field is mandatory present upon creation of a new logical channel for SRB4 (servedRadioBearerSRB4). It is absent, Need M otherwise. |
| LCH-SetupOnlySRB5 | This field is mandatory present upon creation of a new logical channel for SRB5 (servedRadioBearerSRB5). It is absent, Need M otherwise. |

=================================================NEXT CHANGE================================================================

–	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs, multicast MRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=                      CHOICE {

am                                  SEQUENCE {

ul-AM-RLC                           UL-AM-RLC,

dl-AM-RLC                           DL-AM-RLC

},

um-Bi-Directional                   SEQUENCE {

ul-UM-RLC                           UL-UM-RLC,

dl-UM-RLC                           DL-UM-RLC

},

um-Uni-Directional-UL               SEQUENCE {

ul-UM-RLC                           UL-UM-RLC

},

um-Uni-Directional-DL               SEQUENCE {

dl-UM-RLC                           DL-UM-RLC

},

...

}

UL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-PollRetransmit                    T-PollRetransmit,

pollPDU                             PollPDU,

pollByte                            PollByte,

maxRetxThreshold                    ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthAM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly,

t-StatusProhibit                    T-StatusProhibit

}

UL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL    -- Cond Reestab

}

DL-UM-RLC ::=                       SEQUENCE {

sn-FieldLength                      SN-FieldLengthUM                                    OPTIONAL,   -- Cond Reestab

t-Reassembly                        T-Reassembly

}

T-PollRetransmit ::=                ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610,

ms4-v1610, spare1}

PollPDU ::=                         ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384,p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=                        ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=                    ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=                ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=                ENUMERATED {size6, size12}

SN-FieldLengthAM ::=                ENUMERATED {size12, size18}

RLC-Config-v1610 ::=                SEQUENCE {

dl-AM-RLC-v1610                     DL-AM-RLC-v1610

}

RLC-Config-v1700 ::=                SEQUENCE {

dl-AM-RLC-v1700                     DL-AM-RLC-v1700,

dl-UM-RLC-v1700                     DL-UM-RLC-v1700

}

RLC-Config-v19xy ::=                   SEQUENCE {

dl-AM-RLC-v19xy                     DL-AM-RLC-v19xy,

ul-AM-RLC-v19xy                     UL-AM-RLC-v19xy

}

DL-AM-RLC-v1610 ::=                 SEQUENCE {

t-StatusProhibit-v1610              T-StatusProhibit-v1610                               OPTIONAL,   -- Need R

...

}

DL-AM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

DL-AM-RLC-v19xy ::=                    SEQUENCE {

t-RxDiscard-r19                      T-RxDiscard-r19                                     OPTIONAL    -- Need R

}

UL-AM-RLC-v19xy ::=                     SEQUENCE {

stopReTxDiscardedSDU-r19             ENUMERATED {enabled}                                 OPTIONAL   -- Need R

}

DL-UM-RLC-v1700 ::=                 SEQUENCE {

t-ReassemblyExt-r17                 T-ReassemblyExt-r17                                  OPTIONAL    -- Need R

}

T-StatusProhibit-v1610 ::=          ENUMERATED { ms1, ms2, ms3, ms4, spare4, spare3, spare2, spare1}

T-ReassemblyExt-r17 ::=             ENUMERATED {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}

T-RxDiscard-r19 ::=                 ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms400, ms500, ms750, ms1000, ms1500, ms2000, ms3000, spare1, spare2, spare3, spare4,

spare5, spare6, spare7, spare8, spare9, spare10, spare11, spare12, spare13}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Config field descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength of an RLC entity for the DRB/multicast MRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

stopReTxDiscardedSDU Indicates whether the Tx side of the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer as specified in TS 38.323 [5].

t-RxDiscard Timer for the AMD RLC PDU(s) discard at the Rx side of the RLC entity, see TS 38.322 [4]. For the value of the IE T-RxDiscard, value ms10 means 10 milliseconds, value ms20 means 20 milliseconds, and so on. The value of the field should not be lower than that configured by the field t-Reassembly or t-ReassemblyExt.

t-PollRetransmit Timer for RLC AM in TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.

t-Reassembly, t-ReassemblyExt Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-ReassemblyExt-r17 is configured, the UE shall ignore t-Reassembly (without suffix).

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-StatusProhibit-v1610 is present, the UE shall ignore t-StatusProhibit (without suffix).

| Conditional Presence | Explanation |
| --- | --- |
| Reestab | The field is mandatory present at RLC bearer setup. It is optionally present, need M, at RLC re-establishment. Otherwise it is absent. Need M. |

=================================================NEXT CHANGE================================================================

6.3.4	Other information elements

–	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

OtherConfig-v1610 ::=                   SEQUENCE {

idc-AssistanceConfig-r16                SetupRelease {IDC-AssistanceConfig-r16}                       OPTIONAL, -- Need M

drx-PreferenceConfig-r16                SetupRelease {DRX-PreferenceConfig-r16}                       OPTIONAL, -- Need M

maxBW-PreferenceConfig-r16              SetupRelease {MaxBW-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxCC-PreferenceConfig-r16              SetupRelease {MaxCC-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxMIMO-LayerPreferenceConfig-r16       SetupRelease {MaxMIMO-LayerPreferenceConfig-r16}              OPTIONAL, -- Need M

minSchedulingOffsetPreferenceConfig-r16 SetupRelease {MinSchedulingOffsetPreferenceConfig-r16}        OPTIONAL, -- Need M

releasePreferenceConfig-r16             SetupRelease {ReleasePreferenceConfig-r16}                    OPTIONAL, -- Need M

referenceTimePreferenceReporting-r16    ENUMERATED {true}                                             OPTIONAL, -- Need R

btNameList-r16                          SetupRelease {BT-NameList-r16}                                OPTIONAL, -- Need M

wlanNameList-r16                        SetupRelease {WLAN-NameList-r16}                              OPTIONAL, -- Need M

sensorNameList-r16                      SetupRelease {Sensor-NameList-r16}                            OPTIONAL, -- Need M

obtainCommonLocation-r16                ENUMERATED {true}                                             OPTIONAL, -- Need R

sl-AssistanceConfigNR-r16               ENUMERATED{true}                                              OPTIONAL  -- Need R

}

OtherConfig-v1700 ::=                   SEQUENCE {

ul-GapFR2-PreferenceConfig-r17          ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapAssistanceConfig-r17           SetupRelease {MUSIM-GapAssistanceConfig-r17}                  OPTIONAL, -- Need M

musim-LeaveAssistanceConfig-r17         SetupRelease {MUSIM-LeaveAssistanceConfig-r17}                OPTIONAL, -- Need M

successHO-Config-r17                    SetupRelease {SuccessHO-Config-r17}                           OPTIONAL, -- Need M

maxBW-PreferenceConfigFR2-2-r17         ENUMERATED {true}                                             OPTIONAL, -- Cond maxBW

maxMIMO-LayerPreferenceConfigFR2-2-r17  ENUMERATED {true}                                             OPTIONAL, -- Cond maxMIMO

minSchedulingOffsetPreferenceConfigExt-r17  ENUMERATED {true}                                         OPTIONAL, -- Cond minOffset

rlm-RelaxationReportingConfig-r17       SetupRelease {RLM-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

bfd-RelaxationReportingConfig-r17       SetupRelease {BFD-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

scg-DeactivationPreferenceConfig-r17    SetupRelease {SCG-DeactivationPreferenceConfig-r17}           OPTIONAL, -- Cond SCG

rrm-MeasRelaxationReportingConfig-r17   SetupRelease {RRM-MeasRelaxationReportingConfig-r17}          OPTIONAL, -- Need M

propDelayDiffReportConfig-r17           SetupRelease {PropDelayDiffReportConfig-r17}                  OPTIONAL  -- Need M

}

OtherConfig-v1800 ::=                   SEQUENCE {

idc-AssistanceConfig-v1800              SetupRelease {IDC-AssistanceConfig-v1800}                     OPTIONAL, -- Need M

multiRx-PreferenceReportingConfigFR2-r18 SetupRelease {MultiRx-PreferenceReportingConfigFR2-r18}      OPTIONAL, -- Need M

aerial-FlightPathAvailabilityConfig-r18 ENUMERATED {true}                                             OPTIONAL, -- Need R

ul-TrafficInfoReportingConfig-r18       SetupRelease {UL-TrafficInfoReportingConfig-r18}              OPTIONAL, -- Need M

n3c-RelayUE-InfoReportConfig-r18        ENUMERATED {true}                                             OPTIONAL, -- Need R

successPSCell-Config-r18                SetupRelease {SuccessPSCell-Config-r18}                       OPTIONAL, -- Need M

sn-InitiatedPSCellChange-r18            ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapPriorityAssistanceConfig-r18   ENUMERATED {true}                                          OPTIONAL, -- Cond musimGapConfig

musim-CapabilityRestrictionConfig-r18   SetupRelease {MUSIM-CapabilityRestrictionConfig-r18}          OPTIONAL  -- Need M

}

OtherConfig-v19xy ::=                   SEQUENCE {

gapOccasionCancelRatioReportConfig-r19  SetupRelease {GapOccasionCancelRatioReportConfig-r19}       OPTIONAL  -- Need M

}

OtherConfig-v1830 ::=                   SEQUENCE {

sl-PRS-AssistanceConfigNR-r18           ENUMERATED{true}                                              OPTIONAL  -- Need R

}

IDC-AssistanceConfig-v1800 ::=          SEQUENCE {

idc-FDM-AssistanceConfig-r18            SetupRelease {IDC-FDM-AssistanceConfig-r18}                   OPTIONAL, -- Need M

idc-TDM-AssistanceConfig-r18            ENUMERATED {setup}                                            OPTIONAL  -- Cond FDM

}

MultiRx-PreferenceReportingConfigFR2-r18 ::= SEQUENCE {

multiRx-PreferenceReportingConfigFR2ProhibitTimer-r18  ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

MUSIM-GapAssistanceConfig-r17 ::= SEQUENCE {

musim-GapProhibitTimer-r17        ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10}

}

MUSIM-LeaveAssistanceConfig-r17 ::=     SEQUENCE {

musim-LeaveWithoutResponseTimer-r17     ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1}

}

MUSIM-CapabilityRestrictionConfig-r18 ::= SEQUENCE {

musim-CandidateBandList-r18               MUSIM-CandidateBandList-r18                                           OPTIONAL, -- Need R

musim-WaitTimer-r18                       ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18                   ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8,

s9, s10}

}

MUSIM-CandidateBandList-r18::= SEQUENCE (SIZE (1..maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...

}

SuccessPSCell-Config-r18 ::=            SEQUENCE {

thresholdPercentageT304-SCG-r18         ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-SCG-r18         ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-SCG-r18         ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

...

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16                     OPTIONAL, -- Need R

...

}

DRX-PreferenceConfig-r16 ::=          SEQUENCE {

drx-PreferenceProhibitTimer-r16       ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxBW-PreferenceConfig-r16 ::=        SEQUENCE {

maxBW-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxCC-PreferenceConfig-r16 ::=        SEQUENCE {

maxCC-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxMIMO-LayerPreferenceConfig-r16 ::= SEQUENCE {

maxMIMO-LayerPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MinSchedulingOffsetPreferenceConfig-r16 ::=   SEQUENCE {

minSchedulingOffsetPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

ReleasePreferenceConfig-r16 ::=       SEQUENCE {

releasePreferenceProhibitTimer-r16    ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, infinity, spare1},

connectedReporting                    ENUMERATED {true}                                               OPTIONAL  -- Need R

}

RLM-RelaxationReportingConfig-r17 ::= SEQUENCE {

rlm-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

BFD-RelaxationReportingConfig-r17 ::= SEQUENCE {

bfd-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

SCG-DeactivationPreferenceConfig-r17 ::=       SEQUENCE {

scg-DeactivationPreferenceProhibitTimer-r17    ENUMERATED {

s0, s1, s2, s4, s8, s10, s15, s30,

s60, s120, s180, s240, s300, s600, s900, s1800}

}

RRM-MeasRelaxationReportingConfig-r17 ::= SEQUENCE {

s-SearchDeltaP-Stationary-r17             ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17             ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}

PropDelayDiffReportConfig-r17 ::= SEQUENCE {

threshPropDelayDiff-r17           ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, spare5,

spare4, spare3, spare2, spare1}                OPTIONAL,   -- Need M

neighCellInfoList-r17             SEQUENCE (SIZE (1..maxCellNTN-r17)) OF NeighbourCellInfo-r17         OPTIONAL    -- Need M

}

NeighbourCellInfo-r17  ::= SEQUENCE {

epochTime-r17                  EpochTime-r17,

ephemerisInfo-r17              EphemerisInfo-r17

}

IDC-FDM-AssistanceConfig-r18 ::=        SEQUENCE {

candidateServingFreqRangeListNR-r18     CandidateServingFreqRangeListNR-r18                     OPTIONAL, -- Need R

...

}

CandidateServingFreqRangeListNR-r18 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18

CandidateServingFreqRangeNR-r18 ::=     SEQUENCE {

candidateCenterFreq-r18                 ARFCN-ValueNR,

candidateBandwidth-r18                  ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3, mhz4, mhz5,

mhz6, mhz8, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100,

mhz200, mhz300, mhz400}                             OPTIONAL -- Need R

}

UL-TrafficInfoReportingConfig-r18 ::=   SEQUENCE {

pdu-SessionsToReportUL-TrafficInfoList-r18   SEQUENCE (SIZE (1.. maxNrofPDU-Sessions-r17)) OF PDU-SessionToReportUL-TrafficInfo-r18,

ul-TrafficInfoProhibitTimer-r18              ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

PDU-SessionToReportUL-TrafficInfo-r18 ::= SEQUENCE {

pdu-SessionID-r18                        PDU-SessionID,

qfi-ToReportUL-TrafficInfoList-r18       SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI

}

GapOccasionCancelRatioReportConfig-r19 ::= SEQUENCE {

gapOccasionCancelRatioProhibitTimer-r19              ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

aerial-FlightPathAvailabilityConfig Configuration for the UE to indicate the availability of flight path information for Aerial UE operation.

bfd-RelaxationReportingConfig Configuration for the UE to report the relaxation state of BFD measurements.

btNameList Configuration for the UE to report measurements from specific Bluetooth beacons. NG-RAN configures the field if includeBT-Meas is configured for one or more measurements.

candidateBandwidth Indicates the bandwidth of the candidate frequency range around the center frequency.

candidateCenterFreq Indicates the center frequency of the candidate frequency range.

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

candidateServingFreqRangeListNR Indicates the candidate frequency range with the combination of the center frequency and the candidate bandwidth, around which the UE is requested to report IDC issues.

connectedReporting Indicates that the UE can report a preference to remain in RRC_CONNECTED state following a report to leave RRC_CONNECTED state. If absent, the UE cannot report a preference to stay in RRC_CONNECTED state.

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot4 means prohibit timer is set to 0.4 seconds, and so on.

drx-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's DRX preferences for power saving.

drx-PreferenceProhibitTimer Prohibit timer for DRX preferences assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

idc-AssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected IDC problem.

maxBW-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred bandwidth for power saving.

maxBW-PreferenceProhibitTimer Prohibit timer for preferred bandwidth assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxCC-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of carriers for power saving.

maxBW-PreferenceConfigFR2-2 Configuration for the UE to report assistance information to inform the gNB about the UE's preferred bandwidth for power saving for FR2-2.

maxCC-PreferenceProhibitTimer Prohibit timer for preferred number of carriers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxMIMO-LayerPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of MIMO layers for power saving.

maxMIMO-LayerPreferenceConfigFR2-2 Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of MIMO layers for power saving for FR2-2.

maxMIMO-LayerPreferenceProhibitTimer Prohibit timer for preferred number of number of MIMO layers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

gapOccasionCancelRatioProhibitTimer Prohibit timer for transmitting the assistance information of gap occasion cancellation ratio. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

gapOccasionCancelRatioReportConfig Configuration for the UE to report preference for gap occasion cancellation ratio.

minSchedulingOffsetPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred minimumSchedulingOffset value for cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceConfigExt Configuration for the UE to report assistance information to inform the gNB about the UE's preferred minimumSchedulingOffset value for cross-slot scheduling for power saving for SCS 480 kHz and/or 960 kHz.

minSchedulingOffsetPreferenceProhibitTimer Prohibit timer for preferred minimumSchedulingOffset assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

multiRx-PreferenceReportingConfigFR2 Configuration for the UE to report assistance information to inform gNB about the UE's preference on multi-Rx operation for FR2.

multiRx-PreferenceReportingConfigFR2ProhibitTimer Prohibit timer for multi-Rx operation preference reporting for FR2. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

musim-CandidateBandList A list of candidate bands that the network intends to use, e.g., for serving cells and for which the UE is requested to provide information on temporary restricted capabilities for MUSIM operation as specified in clause 5.7.4.3.

musim-GapAssistanceConfig Configuration for the UE to report assistance information for gap preference.

musim-GapPriorityAssistanceConfig Indicates the UE is allowed to provide MUSIM assistance information for gap(s) priority and/or MUSIM gaps keep preference.

musim-GapProhibitTimer Prohibit timer for MUSIM assistance information reporting for gap preference.

musim-LeaveAssistanceConfig Configuration for the UE to report assistance information for leaving RRC_CONNECTED for MUSIM purpose.

musim-LeaveWithoutResponseTimer Indicates the timer for the UE to enter RRC_IDLE for MUSIM purpose as defined in clause 5.3.8.6.

musim-ProhibitTimer Indicates the prohibit timer for UE temporary restricted capabilities for MUSIM operation. Value in milliseconds. Value ms0 means prohibit timer is set to 0 milliseconds, value ms10 means prohibit timer is set to 10 milliseconds and so on.

musim-WaitTimer Indicates the wait timer for UE temporary restricted capabilities for MUSIM operation. Value in milliseconds. Value ms10 means wait timer is set to 10 milliseconds, value ms20 means wait timer is set to 20 milliseconds and so on.

obtainCommonLocation Requests the UE to attempt to have detailed location information available using GNSS. NR configures the field if includeCommonLocationInfo is configured for one or more measurements.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected internal overheating.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

pdu-SessionsToReportUL-TrafficInfoList A list of PDU sessions for which the UE shall report UL traffic information.

propDelayDiffReportConfig Configuration for the UE to report service link propagation delay difference between serving cell and neighbour cell(s).

qfi-ToReportUL-TrafficInfoList A list of QFIs of a PDU session for which the UE shall report UL traffic information.

referenceTimePreferenceReporting If present, the field indicates the UE is configured to provide reference time assistance information.

releasePreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preference to leave RRC_CONNECTED state.

rlm-RelaxationReportingConfig Configuration for the UE to report the relaxation state of RLM measurements.

releasePreferenceProhibitTimer Prohibit timer for release preference assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on. Value infinity means that once a UE has reported a release preference, the UE cannot report a release preference again during the RRC connection.

s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-StationaryConnected" in 5.7.4.4. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on.

scg-DeactivationPreferenceConfig Configuration of the UE to indicate its preference for SCG deactivation.

scg -StatePreferenceProhibitTimer Prohibit timer for UE indication of its preference for SCG deactivation. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s1 means prohibit timer is set to 1 second and so on.

sensorNameList Configuration for the UE to report measurements from specific sensors. NG-RAN configures the field if includeSensor-Meas is configured for one or more measurements.

sl-AssistanceConfigNR Indicate whether UE is configured to provide configured grant assistance information for NR sidelink communication.

sl-PRS-AssistanceConfigNR Indicate whether UE is configured to provide configured grant assistance information for NR sidelink positioning.

sn-InitiatedPSCellChange This field indicates whether the PSCell change procedure or the CPC included in the RRCReconfiguration message is SN initiated or not. In case of SN initiated inter-SN PSCell change procedure or SN configured inter-SN CPC, MN includes this field in the MCG RRC Reconfiguration message. In case of intra-SN PSCell change, or intra-SN CPC, source SN includes the field in the SCG RRC Reconfiguration.

sourceDAPS-FailureReporting This field indicates whether the UE shall generate the SHR upon successfully completing the DAPS handover to the target cell and if a radio link failure was experienced in the source PCell while executing the DAPS handover. This field is set in the otherConfig configured by the source cell of the DAPS handover.

successHO-Config Configuration for the UE to report the successful handover information to the network.

successPSCell-Config Configuration for the UE to report the successful PSCell change or addition information to the network. When this field is configured in CG-Config, the thresholdPercentageT304-SCG is absent.

t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-StationaryConnected" in 5.7.4.4. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

thresholdPercentageT304 This field indicates the threshold for the ratio in percentage between the elapsed T304 timer and the configured value of the T304 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the target cell of the handover.

thresholdPercentageT310 This field indicates the threshold for the ratio in percentage between the elapsed T310 timer and the configured value of the T310 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the source cell of the handover.

thresholdPercentageT312 This field indicates the threshold for the ratio in percentage between the elapsed T312 timer and the configured value(s) of the T312 timer. Value p20 corresponds to 20%, value p40 corresponds to 40% and so on. This field is set in the otherConfig configured by the source cell of the handover.

thresholdPercentageT304-SCG This field indicates the threshold for the ratio in percentage between the elapsed T304 timer associated to the target PSCell and the configured value of the T304 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the target PSCell of the PSCell change or addition.

thresholdPercentageT310-SCG This field indicates the threshold for the ratio in percentage between the elapsed T310 timer associated to the source PSCell and the configured value of the T310 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the source PSCell of the PSCell change or CPC, or in the otherConfig configured by the PCell for the PSCell change or CPC. This field is not configured at the time of PSCell change via SRB3.

thresholdPercentageT312-SCG This field indicates the threshold for the ratio in percentage between the elapsed T312 timer associated to the measurement identity of the target PSCell and the configured value of the T312 timer. Value p20 corresponds to 20%, value p40 corresponds to 40% and so on. This field is set in the otherConfig configured by the source PSCell of the PSCell change or CPC, or in the otherConfig configured by the PCell for the PSCell change or CPC. This field is not configured at the time of PSCell change via SRB3.

threshPropDelayDiff Threshold for one-way service link propagation delay difference report as specified in 5.7.4.2.

ul-GapFR2-PreferenceConfig Indicates whether UE is configured to request for FR2 UL gap activation/deactivation and preferred FR2 UL gap pattern.

wlanNameList Configuration for the UE to report measurements from specific WLAN APs. NG-RAN configures the field if includeWLAN-Meas is configured for one or more measurements.

ul-TrafficInfoProhibitTimer Prohibit timer for UL traffic information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

ul-TrafficInfoReportingConfig Configuration for the UE to report UL traffic information.

| Conditional Presence | Explanation |
| --- | --- |
| FDM | This field is optionally present, need R, if idc-AssistanceConfig-r16 or idc-FDM-AssistanceConfig is setup. Otherwise, it is absent, need R. |
| maxBW | This field is optionally present, need R, if maxBW-PreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| maxMIMO | This field is optionally present, need R, if maxMIMO-LayerPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| minOffset | This field is optionally present, need R, if minSchedulingOffsetPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| musimGapConfig | This field is optionally present, need R, if musim-GapAssistanceConfig-r17 is setup; otherwise it is absent, need R. |
| SCG | This field is optionally present, need M, in an RRCReconfiguration message not within mrdc-SecondaryCellGroup and received, either via SRB3 within DLInformationTransferMRDC or via SRB1. Otherwise, it is absent. |

=================================================NEXT CHANGE================================================================

6.4	RRC multiplicity and type constraint values

–	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAdditionalRACH-r17                   INTEGER ::= 256     -- Maximum number of additional RACH configurations.

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandComb-MUSIM-r18                   INTEGER ::= 64      -- Maximum number of MUSIM bands and/or band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxCandidateBandIndex-r18               INTEGER ::= 8       -- Maximum number of band entry index for MUSIM capability

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxTwoPUCCH-Grp-ConfigList-r17          INTEGER ::= 16      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config} for PUCCH cell switching

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellATG-r18                          INTEGER ::= 8       -- Maximum number of ATG neighbour cells for which assistance information is

-- provided

maxCellExcluded                         INTEGER ::= 16      -- Maximum number of NR exclude-listed cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited PCells reported

maxPSCellHistory-r17                    INTEGER ::= 16      -- Maximum number of visited PSCells across all reported PCells

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellNTN-r17                          INTEGER ::= 4       -- Maximum number of NTN neighbour cells for which assistance information is

-- provided

maxCarrierTypePairList-r16              INTEGER ::= 16      -- Maximum number of supported carrier type pair of (carrier type on which

-- CSI measurement is performed, carrier type on which CSI reporting is

-- performed) for CSI reporting cross PUCCH group

maxCellAllowed                          INTEGER ::= 16      -- Maximum number of NR allow-listed cell ranges in SIB3, SIB4

maxDSR-ReportingThres-r19               INTEGER ::= 4       -- Maximum number of DSR reporting thresholds configurable per LCG

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellExcluded                   INTEGER ::= 16      -- Maximum number of E-UTRA exclude-listed physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFeatureCombPreamblesPerRACHResource-r17 INTEGER ::= 256  -- Maximum number of feature combination preambles.

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCells + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16 INTEGER ::= 12

maxNrofAperiodicFwdTimeResource-r18     INTEGER ::= 112     -- Max number of aperiodic fowarding time resources for NCR

maxNrofAperiodicFwdTimeResource-1-r18   INTEGER ::= 111     -- Max number of aperiodic fowarding time resources for NCR minus 1

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAppLayerMeas-r17                 INTEGER ::= 16      -- Max number of simultaneous application layer measurements

maxNrofAppLayerMeas-1-r17               INTEGER ::= 15      -- Max number of simultaneous application layer measurements minus 1

maxNrofAppLayerReports-r18              INTEGER ::= 16      -- Max number of application layer measurement reports with the same

-- measConfigAppLayerId included in the same

-- MeasurementReportAppLayerMessage

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofIABResourceConfig-r17            INTEGER ::= 65536   -- Max number of IAB-ResourceConfigID used in MAC CE

maxNrofIABResourceConfig-1-r17          INTEGER ::= 65535   -- Max number of IAB-ResourceConfigID used in MAC CE minus 1

maxNrofPeriodicFwdResourceSet-r18       INTEGER ::= 32      -- Max number of periodic fowarding resource sets for NCR

maxNrofPeriodicFwdResourceSet-1-r18     INTEGER ::= 31      -- Max number of periodic fowarding resource sets for NCR minus 1

maxNrofPeriodicFwdResource-r18          INTEGER ::= 1024    -- Max number of periodic fowarding resources for NCR

maxNrofPeriodicFwdResource-1-r18        INTEGER ::= 1023    -- Max number of periodic fowarding resources for NCR minus 1

maxNrofSemiPersistentFwdResourceSet-r18 INTEGER ::= 32      -- Max number of semi-persistent fowarding resource sets for NCR

maxNrofSemiPersistentFwdResourceSet-1-r18 INTEGER ::= 31    -- Max number of semi-persistent fowarding resource sets for NCR minus 1

maxNrofSemiPersistentFwdResource-r18    INTEGER ::= 128     -- Max number of semi-persistent fowarding resources for NCR

maxNrofSemiPersistentFwdResource-1-r18  INTEGER ::= 127     -- Max number of semi-persistent fowarding resources for NCR minus 1

maxNrofSCellActRS-r17                   INTEGER ::= 255     -- Max number of RS configurations per SCell for SCell activation

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCRS-IM-InterfCell-r17            INTEGER ::= 8       -- Maximum number of LTE interference cells for CRS-IM per UE

maxNrofRelayMeas-r17                    INTEGER ::= 32      -- Maximum number of L2 U2N Relay UEs to measure for each measurement object

-- on sidelink frequency

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxSL-GC-BC-DRX-QoS-r17                 INTEGER ::= 16      -- Max number of sidelink DRX configurations for NR

-- sidelink groupcast/broadcast communication

maxNrofSL-RxInfoSet-r17                 INTEGER ::= 4       -- Max number of sidelink DRX configuration sets in sidelink DRX assistant

-- information

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCondCells-1-r17                  INTEGER ::= 7       -- Max number of conditional candidate SpCells minus 1

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofDL-AllocationsExt-r17            INTEGER ::= 64      -- Maximum number of PDSCH time domain resource allocations for multi-PDSCH

-- scheduling

maxNrofDL-Allocations-1-r18             INTEGER ::= 15      -- Maximum number of PDSCH time domain resource allocations minus 1

maxNrofPDU-Sessions-r17                 INTEGER ::= 256     -- Maximum number of PDU Sessions

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxNrofLCGs-r18                         INTEGER ::= 8       -- Maximum number of LCGs

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLCG-ID-IAB-r17                       INTEGER ::= 255     -- Maximum value of LCG ID for IAB-MT

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrOfLinkedSRS-CarriersInactive-1-r18 INTEGER ::= 2       -- Maximum number of carriers for positioning SRS CA in RRC_INACTIVE minus 1

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxNrofSearchSpacesLinks-1-r17          INTEGER ::= 39      -- Max number of Search Space links minus 1

maxNrofBFDResourcePerSet-r17            INTEGER ::= 64      -- Max number of reference signal in one BFD set

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18 INTEGER ::= 63      -- Maximum number of NZP CSI-RS resources per resource set minus 1

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofCSI-SSB-ResourceSetsPerConfigExt INTEGER ::= 2       -- Maximum number of CSI SSB resource sets per resource configuration

-- extended

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFailureDetectionResources-1-r17  INTEGER ::= 63      -- Maximum number of the enhanced failure detection resources minus 1

maxNrofRateCtrlQFIs-r19                 INTEGER ::= 16      -- Maximum number of QoS flows for which rate control can be performed

maxNrofRateQueryQFIs-r19                INTEGER ::= 16      -- Maximum number of QoS flows for which rate query can be performed

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequency for NR sidelink communication

maxNrofFreqSL-1-r18                     INTEGER ::= 7       -- Maximum number of carrier frequency for NR sidelink communication minus 1

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for NR sidelink communication

maxNrofSL-CarrierSetConfig-r18          INTEGER ::= 96      -- Maximum number of SCCH carrier set configuration for NR sidelink

-- communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequency for NR sidelink

-- communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resource pool for NR sidelink measurement to measure

-- for each measurement object (for CBR)

maxNrofDedicatedSL-PRS-PoolToMeas-r18   INTEGER ::= 8       -- Maximum number of SL-PRS dedicated resource pool for positioning

-- measurement to measure for each measurement object (for SL-PRS CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequency for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establishment

maxTAC-r17                              INTEGER ::= 12      -- Maximum number of Tracking Area Codes to which a cell belongs to

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

-- minus 1.

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication and discovery

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication and discovery

maxNrofSL-PRS-PerDest-r18               INTEGER ::= 8       -- Max number of SL-PRS transmission supported per destination UE

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE without duplication

maxSL-LCID-Plus1-r18                    INTEGER ::= 513     -- Maximum number of RLC bearer for NR sidelink communication per UE without duplication plus 1

maxSL-LCID-r18                          INTEGER ::= 1024    -- Maximum number of RLC bearer for NR sidelink communication per UE with duplication

maxSL-NonAnchorRBsets                   INTEGER ::= 4       -- Maximum number of non-anchor RB sets

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource pool for NR sidelink communication and

-- discovery

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resource pool for NR sidelink communication and

-- discovery

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication and

-- discovery

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control

-- minus 1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxSimultaneousBands-2-r18              INTEGER ::= 30      -- Maximum number of simultaneously aggregated bands minus 2.

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band

-- combination.

maxULTxSwitchingBetweenBandPairs-r18    INTEGER ::= 32      -- Maximum number of combinations of a band pair and another band pair/band

-- between which dynamic UL Tx switching requires additional switching

-- period.

maxSchedulingBandCombination-r18        INTEGER ::= 32      -- Maximum number of combinations of scheduling cell and co-scheduled cells

-- have same or different carrier type.

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r17 INTEGER ::= 7       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofPowerControlSetInfos-r17         INTEGER ::= 8       -- Maximum number of PUCCH power control set infos

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see TS 38.213 [13], clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see TS 38.213 [13], clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofPathlossReferenceRSs-r17         INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation

maxNrofPathlossReferenceRSs-1-r17       INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation minus 1

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModList).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqPlus1                            INTEGER ::= 9       -- Max number of frequencies for Slicing.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximum number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxUL-TCI-r17                           INTEGER ::= 64      -- Maximum number of TCI states.

maxUL-TCI-1-r17                         INTEGER ::= 63      -- Maximum number of TCI states minus 1.

maxNrofAdditionalPCI-r17                INTEGER ::= 7       -- Maximum number of additional PCI

maxNrofAdditionalPRACHConfigs-r18       INTEGER ::= 7       -- Maximum number of additional PRACH configurations for 2TA

maxNrofdelayD-r18                       INTEGER ::= 4       -- Maximum number of delayD values.

maxMPE-Resources-r17                    INTEGER ::= 64      -- Maximum number of pooled MPE resources

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks supported by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-ResourcesExt-r17          INTEGER ::= 8       -- Maximum number of codebook resources for fetype2R1 and fetype2R2

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxSIB-MessagePlus1-r17                 INTEGER::= 33       -- Maximum number of SIB messages plus 1

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identities in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxGIN-r17                              INTEGER ::= 24      -- Maximum number of broadcast GINs

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxSNPN-ConfigCellId-r18                INTEGER ::= 32      -- Maximum number of Cell ID subject for SNPNS for MDT scope

maxSNPN-ConfigID-r18                    INTEGER ::= 16      -- Maximum number of SNPNs subject for MDT scope

maxSNPN-ConfigTAI-r18                   INTEGER ::= 8       -- Maximum number of TA subject for MDT scope

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxK0-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-7-Size-r17                     INTEGER ::= 43      -- Maximum size of DCI format 2-7

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxDCI-2-9-Size-r18                     INTEGER ::= 140     -- Maximum DCI format 2-9 size

maxDCI-2-9-Size-1-r18                   INTEGER ::= 139     -- Maximum DCI format 2-9 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofUL-Allocations-1-r18             INTEGER ::= 63      -- Maximum number of PUSCH time domain resource allocations minus 1

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxUu-RelayRLC-ChannelID-r17            INTEGER ::= 32      -- Maximum value of Uu Relay RLC channel ID

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofCC-Group-r17                     INTEGER ::= 16      -- Maximum number of CC groups for DC location report

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18 INTEGER ::= 8 -- Maximum number of CSI report subconfigurations per CSI report

-- configuration

maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-1-r18 INTEGER ::= 7 -- Maximum number of CSI report subconfigurations per CSI report

-- configuration minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofPPW-Config-r17                   INTEGER ::= 4       -- Maximum number of Preconfigured PRS processing windows per DL BWP

maxNrofPPW-ID-1-r17                     INTEGER ::= 15      -- Maximum number of Preconfigured PRS processing windows minus 1

maxNrOfTxTEGReport-r17                  INTEGER ::= 256     -- Maximum number of UE Tx Timing Error Group Report

maxNrOfTxTEG-ID-1-r17                   INTEGER ::= 7       -- Maximum number of UE Tx Timing Error Group ID minus 1

maxNrofPagingSubgroups-r17              INTEGER ::= 8       -- Maximum number of paging subgroups per paging occasion

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3

maxNrofReqComDC-Location-r17            INTEGER ::= 128     -- Maximum number of requested carriers/BWPs combinations for DC location

-- report

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofRB-SetGroups-r17                 INTEGER ::= 8       -- Maximum number of RB set groups

maxNrofRB-Sets-r17                      INTEGER ::= 8       -- Maximum number of RB sets

maxNrofEnhType3HARQ-ACK-r17             INTEGER ::= 8       -- Maximum number of enhanced type 3 HARQ-ACK codebook

maxNrofEnhType3HARQ-ACK-1-r17           INTEGER ::= 7       -- Maximum number of enhanced type 3 HARQ-ACK codebook minus 1

maxNrofPRS-ResourcesPerSet-r17          INTEGER ::= 64      -- Maximum number of PRS resources for one set

maxNrofPRS-ResourcesPerSet-1-r17        INTEGER ::= 63      -- Maximum number of PRS resources for one set minus 1

maxNrofPRS-ResourceOffsetValue-1-r17    INTEGER ::= 511

maxNrofGapId-r17                        INTEGER ::= 8       -- Maximum number of measurement gap ID

maxNrofPreConfigPosGapId-r17            INTEGER ::= 16      -- Maximum number of preconfigured positioning measurement gap

maxNrOfGapPri-r17                       INTEGER ::= 16      -- Maximum number of gap priority level

maxCEFReport-r17                        INTEGER ::= 4       -- Maximum number of CEF reports by the UE

maxNrofMultiplePDSCHs-r17               INTEGER ::= 8       -- Maximum number of PDSCHs in PDSCH TDRA list

maxSliceInfo-r17                        INTEGER ::= 8       -- Maximum number of NSAGs

maxCellSlice-r17                        INTEGER ::= 16      -- Maximum number of cells supporting the NSAG

maxNrofTRS-ResourceSets-r17             INTEGER ::= 64      -- Maximum number of TRS resource sets

maxNrofSearchSpaceGroups-1-r17          INTEGER ::= 2       -- Maximum number of search space groups minus 1

maxNrofRemoteUE-r17                     INTEGER ::= 32      -- Maximum number of connected L2 U2N Remote UEs

maxDCI-4-2-Size-r17                     INTEGER ::= 140     -- Maximum size of DCI format 4-2

maxFreqMBS-r17                          INTEGER ::= 16      -- Maximum number of MBS frequencies reported in MBSInterestIndication

maxNrofDRX-ConfigPTM-r17                INTEGER ::= 64      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell

maxNrofDRX-ConfigPTM-1-r17              INTEGER ::= 63      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell minus 1

maxNrofMBS-ServiceListPerUE-r17         INTEGER ::= 16      -- Maximum number of services which the UE can include in the  MBS interest

-- indication

maxNrofMBS-Session-r17                  INTEGER ::= 1024    -- Maximum number of MBS sessions provided in MBS broadcast or multicast in

-- a cell

maxNrofMTCH-SSB-MappingWindow-r17       INTEGER ::= 16      -- Maximum number of MTCH to SSB beam mapping pattern

maxNrofMTCH-SSB-MappingWindow-1-r17     INTEGER ::= 15      -- Maximum number of MTCH to SSB beam mapping pattern minus 1

maxNrofMRB-Broadcast-r17                INTEGER ::= 4       -- Maximum number of broadcast MRBs configured for one MBS broadcast service

maxNrofPageGroup-r17                    INTEGER ::= 32      -- Maximum number of paging groups in a paging message

maxNrofPDSCH-ConfigPTM-r17              INTEGER ::= 16      -- Maximum number of PDSCH configuration groups for PTM

maxNrofPDSCH-ConfigPTM-1-r17            INTEGER ::= 15      -- Maximum number of PDSCH configuration groups for PTM minus 1

maxG-RNTI-r17                           INTEGER ::= 16      -- Maximum number of G-RNTI that can be configured for a UE.

maxG-RNTI-1-r17                         INTEGER ::= 15      -- Maximum number of G-RNTI that can be configured for a UE minus 1.

maxG-CS-RNTI-r17                        INTEGER ::= 8       -- Maximum number of G-CS-RNTI that can be configured for a UE.

maxG-CS-RNTI-1-r17                      INTEGER ::= 7       -- Maximum number of G-CS-RNTI that can be configured for a UE minus 1.

maxMRB-r17                              INTEGER ::= 32      -- Maximum number of multicast MRBs (that can be added in MRB-ToAddModLIst)

maxFSAI-MBS-r17                         INTEGER ::= 64      -- Maximum number of MBS frequency selection area identities

maxNeighCellMBS-r17                     INTEGER ::= 8       -- Maximum number of MBS broadcast neighbour cells

maxNrofPdcch-BlindDetectionMixed-1-r16  INTEGER ::= 7       -- Maximum number of combinations of mixed Rel-16 and Rel-15 PDCCH

-- monitoring capabilities minus 1

maxNrofPdcch-BlindDetection-r17         INTEGER ::= 16      -- Maximum number of combinations of PDCCH blind detection monitoring

-- capabilities

maxNrofAltitudeRanges-r18               INTEGER ::= 8       -- Maximum number of altitude ranges for altitude-based measurement configurations

maxWayPoint-r18                         INTEGER ::= 20      -- Maximum number of flight path information waypoints

maxAltitude-r18                         INTEGER ::= 10000   -- Maximum altitude in meters

minAltitude-r18                         INTEGER ::= -420    -- Minimum altitude in meters

maxMeasSequence-r18                     INTEGER ::= 64      -- Maximum number of configured sequence for measurement

maxNrofHops-1-r18                       INTEGER ::= 5       -- Maximum number of Hops that can be configured for Positioning SRS Transmission

maxNrOfCellsInVA-r18                    INTEGER ::= 16      -- Maximum number of cells in validity area for Positioning SRS

maxNrOfCellsInVA-Ext-r18                INTEGER ::= 16      -- Maximum number of additional cells in validity area for Positioning SRS

maxNrOfLinkedSRS-PosResourceSet-r18     INTEGER ::= 3       -- Maximum number of linked SRSPosResourceSets that can be aggregated across

-- CCs

maxNrOfLinkedSRS-PosResSetComb-r18       INTEGER ::= 32     -- Maximum number of combinations of linked SRSPosResourceSets that can be

-- aggregated in RRC_CONNECTED state

maxNrOfLinkedSRS-PosResSetCombInactive-r18 INTEGER ::= 16   -- Maximum number of combinations of linked SRSPosResourceSets that can be

-- aggregated in RRC_INACTIVE state

maxCBR-ConfigDedSL-PRS-1-r18            INTEGER ::= 7       -- Maximum number of CBR ranges for dedicated SL PRS resource pool

maxCBR-LevelDedSL-PRS-1-r18             INTEGER ::= 15      -- Maximum number of CBR levels for dedicated SL PRS resource pool

maxNrofSL-PRS-TxPool-r18                INTEGER ::= 8       -- Maximum number of Tx dedicated SL-PRS resource pool for NR sidelink positioning

maxNrofSL-PRS-TxConfig-r18              INTEGER ::= 64      -- Maximum number of SL PRS transmission parameter configurations

maxNrOfVA-r18                           INTEGER ::= 16      -- Maximum number of validity area

maxNrofLTM-Configs-r18                  INTEGER ::= 8       -- Maximum number of LTM candidate cells

maxNrofLTM-Configs-plus1-r18          INTEGER ::= 9       -- Maximum number of LTM candidate cells plus 1

maxNrofLTM-CSI-ReportConfigurations-r18     INTEGER ::= 48     -- Maximum number of LTM CSI reporting configurations

maxNrofLTM-CSI-ReportConfigurations-1-r18   INTEGER ::= 47     -- Maximum number of LTM CSI reporting configurations minus 1

maxNrofLTM-CSI-SSB-ResourcesPerSet-r18      INTEGER ::= 512    -- Maximum number of LTM CSI SSB resource per set

maxNrofLTM-CSI-ResourceConfigurations-r18   INTEGER ::= 112    -- Maximum number of LTM CSI resource configurations

maxNrofLTM-CSI-ResourceConfigurations-1-r18 INTEGER ::= 111    -- Maximum number of LTM CSI resource configurations minus 1

maxNrofCandidateTCI-State-r18           INTEGER ::= 128     -- Maximum number of LTM TCI states

maxNrofCandidateUL-TCI-r18              INTEGER ::= 64      -- Maximum number of LTM UL TCI states

maxSecurityCellSet-r18                  INTEGER ::= 9       -- Maximum number of cell sets for subsequent CPAC.

maxSK-Counter-r18                       INTEGER ::= 8       -- Maximum number of SK-counters configured for a cell set for subsequent CPAC.

maxNrofThresholdMBS-r18                 INTEGER ::= 8       -- Max number of thresholds of MBS sessions for RRC connection resume for a

-- UE receiving multicast in RRC_INACTIVE

maxNrofThresholdMBS-1-r18               INTEGER ::= 7       -- Max number of thresholds of MBS sessions for RRC connection resume for a

-- UE receiving multicast in RRC_INACTIVE minus 1

maxTN-AreaInfo-r18                      INTEGER ::= 32      -- Maximum number of TN coverage areas for which assistance info is

-- provided in an NTN cell

maxNrofSetsOfCells-r18                  INTEGER ::= 4       -- Maximum number of sets of cells for multi-cell PDSCH/PUSCH scheduling

maxNrofSetsOfCells-1-r18                INTEGER ::= 3       -- Maximum number of sets of cells for multi-cell PDSCH/PUSCH scheduling

-- minus 1

maxNrofCellsInSet-r18                   INTEGER ::= 4       -- Maximum number of cells configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling

maxNrofCellsInSet-1-r18                 INTEGER ::= 3       -- Maximum number of cells configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling minus 1

maxNrofCellCombos-r18                   INTEGER ::= 16      -- Maximum number of combinations of co-scheduled cells for multi-cell

-- PDSCH/PUSCH scheduling

maxNrofBWPsInSetOfCells-r18             INTEGER ::= 16      -- Maximum number of BWPs configured in a set of cells for multi-cell

-- PDSCH/PUSCH scheduling

maxLowerMSD-r18                         INTEGER ::= 256     -- Maximum number of lower MSD capability sets for a victim band

maxLowerMSDInfo-r18                     INTEGER ::= 64      -- Maximum number of lower MSD capability sets for a band combination

maxNrofIntraEndc-Components-r17         INTEGER ::= 4       -- Maximum number of intra-band (NG)EN-DC band components in an inter-band

-- (NG)EN-DC band combination

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

=================================================NEXT CHANGE================================================================

7.1	Timers

7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection, relay (re)selection or cell selection by a L2 U2N Remote UE, and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.7. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetup message as well as when the selected cell becomes unsuitable or the (re)selected L2 U2N Relay UE becomes unsuitable, upon reception of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection. | Go to RRC_IDLE |
| T302 | Upon reception of RRCReject while performing RRC connection establishment or resume, upon reception of RRCRelease with waitTime. | Upon entering RRC_CONNECTED or RRC_IDLE, upon cell re-selection, upon cell change due to relay (re)selection or cell selection by a L2 U2N Remote UE, and upon reception of RRCReject message. | Inform upper layers about barring alleviation as specified in 5.3.14.4 |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync for the MCG which does not include sl-PathSwitchConfig, or upon reception of RRCReconfiguration message including reconfigurationWithSync for the SCG not indicated as deactivated in the NR or E-UTRA message containing the RRCReconfiguration message or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. Also, for the MCG and SCG upon an indication from lower layer that an LTM cell switch procedure is triggered and, for the MCG, upon performing an LTM cell switch procedure following cell selection performed while timer T311 is running. | Upon successful completion of random access on the corresponding SpCell. Upon receiving an indication from lower layers of successful completion of Rach-less handover, or upon receiving an indication from lower layers of successful completion of an LTM RACH-less cell switch. For T304 of SCG, upon SCG release. | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, or an LTM cell switch procedure, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the connection re-establishment procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, upon an indication from lower layers that an LTM cell switch procedure is triggered, and upon initiating the MCG failure information procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If AS security is not activated: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure as specified in 5.3.7 or the procedure as specified in 5.3.10.3 if any DAPS bearer is configured. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Upon selection of a suitable NR cell, or upon selection of a suitable L2 U2N Relay UE, or a cell using another RAT. | Enter RRC_IDLE |
| T312 | If T312 is configured in MCG: Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 in PCell is running. If T312 is configured in SCG and useT312 has been set to true: Upon triggering a measurement report for a measurement identity for which T312 has been configured, while T310 in PSCell is running. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon initiating the connection re-establishment procedure, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the MCG failure information procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, upon an indication from lower layers that an LTM cell switch procedure is triggered, and upon the expiry of T310 in corresponding SpCell. Upon SCG release, if the T312 is kept in SCG | If the T312 is kept in MCG, initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure. If the T312 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure.as specified in 5.7.3. |
| T316 | Upon transmission of the MCGFailureInformation message | Upon receiving RRCRelease, RRCReconfiguration with reconfigurationwithSync for the PCell, MobilityFromNRCommand, or upon initiating the re-establishment procedure | Perform the actions as specified in 5.7.3b.5. |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1 when the resume procedure is not initiated for SDT. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, upon cell re-selection or upon relay (re)selection. | Perform the actions as specified in 5.3.13.5. |
| T319a | Upon transmission of RRCResumeRequest or RRCResumeRequest1 when the resume procedure is initiated for SDT. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCReject message or upon failure to resume RRC connection for SDT as specified in 5.3.13.5 or upon cell reselection. | Perform the actions as specified in 5.3.13.5. |
| T320 | Upon reception of t320 or upon cell (re)selection to NR from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, upon reception of RRCRelease, when PLMN selection or SNPN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT). | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the reportType set to reportCGI | Upon acquiring the information needed to set all fields of cgi-info, upon receiving measConfig that includes removal of the reportConfig with the reportType set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T322 | Upon receiving measConfig including reportConfigNR with the reportType set to reportSFTD and drx-SFTD-NeighMeas is set to true. | Upon acquiring the SFTD measurement results, upon receiving measConfig that includes removal of the reportConfig with the reportType set to reportSFTD. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T325 | Upon reception of RRCRelease message with deprioritisationTimer. | When PLMN selection or SNPN selection is performed on request by NAS according to TS 38.304 [20]. | Stop deprioritisation of all frequencies or NR signalled by RRCRelease and discard the stored deprioritisation request(s). |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.5a.1.4 |
| T331 | Upon receiving RRCRelease message with measIdleDuration | Upon receiving RRCSetup, RRCResume, RRCRelease with idle/inactive measurement configuration, upon cell selection/reselection to a cell that does not belong to the validityArea (if configured), or upon cell re-selection to another RAT. | Perform the actions as specified in 5.7.8.3. |
| T342 | Upon transmitting UEAssistanceInformation message with DelayBudgetReport. | Upon releasing delayBudgetReportingConfig during the connection re-establishment/resume procedures, and upon receiving delayBudgetReportingConfig set to release. | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |
| T346a (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with drx-Preference. | Upon releasing drx-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving drx-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346b (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxBW-Preference. | Upon releasing maxBW-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxBW-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346c (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxCC-Preference. | Upon releasing maxCC-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxCC-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346d (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxMIMO-LayerPreference. | Upon releasing maxMIMO-LayerPreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxMIMO-LayerPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346e (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with minSchedulingOffsetPreference. | Upon releasing minSchedulingOffsetPreferenceConfig during the connection re-establishment/resume procedures, upon receiving minSchedulingOffsetPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346f | Upon transmitting UEAssistanceInformation message with releasePreference. | Upon releasing releasePreferenceConfig during the connection re-establishment/resume procedures, or upon receiving releasePreferenceConfig set to release. | No action. |
| T346g | Upon transmitting UEAssistanceInformation message with musim-PreferredRRC-State. | Upon receiving RRCRelease, or upon receiving musim-LeaveAssistanceConfig set to release. | Perform the actions as specified in 5.3.8.6. |
| T346h | Upon transmitting UEAssistanceInformation message with musim-GapPreferenceList and/or musim-GapPriorityPreferenceList and/or musim-GapKeepPreference Information. | Upon releasing musim-GapAssistanceConfig during the connection re-establishment/resume procedures, or upon receiving musim-GapAssistanceConfig set to release. | No action. |
| T346i | Upon transmitting UEAssistanceInformation message with scg-DeactivationPreference | Upon releasing scg-DeactivationPreferenceConfig during RRC connection re-establishment/resume or upon receiving scg-DeactivationPreferenceConfig set to release. | No action. |
| T346j (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with rlm-RelaxationReportingConfig. | Upon releasing rlm-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving rlm-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T346k (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with bfd-RelaxationReportingConfig. | Upon releasing bfd-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving bfd-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T346l (The UE maintains one instance of this timer per QoS flow) | Upon transmitting UEAssistanceInformation message with ul-TrafficInfo for the concerned QoS flow. | Upon releasing ul-TrafficInfoReportingConfig during the connection re-establishment/resume procedures, or upon receiving ul-TrafficInfoReportingConfig set to release. | No action. |
| T346m | Upon transmitting UEAssistanceInformation message with multiRx-PreferenceFR2. | Upon releasing multiRx-PreferenceReportingConfigFR2 during the connection re-establishment/resume procedures, upon receiving multiRx-PreferenceReportingConfigFR2 set to release. | No action. |
| T346n | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for affected bands or combination of bands and/or avoided band(s) or combination of bands. | Upon releasing musim-CapabilityRestrictionConfig during the connection re-establishment/resume procedures, or upon receiving musim-CapabilityRestrictionConfig set to release. | No action. |
| T346o | Upon transmission of UEAssistanceInformation message with gapOccasionCancelRatio. | Upon releasing gapOccasionCancelRatioReportConfig during the connection re-establishment/resume procedure or upon receiving gapOccasionCancelRatioReportConfig set to release. | No action. |
| T348 | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for serving cell(s) with capabilities restricted, release of SCell or PSCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction indicated via musim-CapRestriction. | UE may apply the temporary UE capability restriction in accordance with the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction. UE may apply the temporary capability restriction that SCG is not supported if ServCellIndex of PSCell was included in indicated MUSIM-CellToRelease-r18. |
| T350 | Upon transmitting DedicatedSIBRequest message with requestedSIB-List and/or requestedPosSIB-List. | Upon acquiring the requested SIB(s) or posSIB(s), upon releasing onDemandSIB-Request during the connection re-establishment procedures, upon receiving onDemandSIB-Request set to release, upon reception of RRCRelease or upon successful change of PCell while in RRC_CONNECTED. | No action |
| T380 | Upon reception of t380 in RRCRelease. | Upon reception of RRCResume, RRCSetup or RRCRelease. | Perform the actions as specified in 5.3.13. |
| T390 | When access attempt is barred at access barring check for an Access Category. The UE maintains one instance of this timer per Access Category. | Upon cell (re)selection, upon relay (re)selection, upon entering RRC_CONNECTED, upon reception of RRCReconfiguration including reconfigurationWithSync, upon change of PCell while in RRC_CONNECTED, upon reception of MobilityFromNRCommand, or upon reception of RRCRelease. | Perform the actions as specified in 5.3.14.4. |
| T400 | Upon transmission of RRCReconfigurationSidelink | Upon reception of RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink | Perform the Sidelink radio link failure related actions as specified in 5.8.9.3. |
| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig where sl-IndirectPathMaintain is not included in reconfigurationWithSync | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |
| T421 | Upon reception of the RRCReconfiguration message including sl-IndirectPathAddChange where sl-IndirectPathMaintain is not included in reconfigurationWithSync | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) if split SRB1 with duplication is configured, or upon reception of RRCReconfigurationCompleteSidelink if split SRB1 with duplication is not configured, or upon initiation of indirect path failure information procedure. | Perform the Failure Information Reporting as specified in 5.7.3c. |
| T430 | Start or restart from the subframe indicated by epochTime upon reception of SIB19, or upon reception of RRCReconfiguration message for the target cell including reconfigurationWithSync, or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message for the target cell including reconfigurationWithSync, or upon satellite switch with resynchronization. | Stop T430, if it is running, for the source cell upon reception of RRCReconfiguration message including reconfigurationWithSync, or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync, or upon satellite switch with resynchronization. | Perform the actions as specified in 5.2.2.6. |

================================================= CHANGE ENDS================================================================