### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	NR-PositionCalculationAssistance

The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data including integrity information to enable UE-based downlink positioning.

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-TRP-LocationInfo-r16			NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-DL-PRS-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-RTD-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...,

[[

nr-TRP-BeamAntennaInfo-r17		NR-TRP-BeamAntennaInfo-r17			OPTIONAL,	-- Need ON

nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17

NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17

OPTIONAL,	-- Need ON

nr-DL-PRS-TRP-TEG-Info-r17		NR-DL-PRS-TRP-TEG-Info-r17			OPTIONAL	-- Need ON

]],

[[

nr-IntegrityServiceParameters-r18	NR-IntegrityServiceParameters-r18	OPTIONAL,	-- Need OR

nr-IntegrityServiceAlert-r18		NR-IntegrityServiceAlert-r18		OPTIONAL,	-- Need OR

nr-IntegrityRiskParameters-r18		NR-IntegrityRiskParameters-r18		OPTIONAL,	-- Need OR

nr-IntegrityParametersTRP-LocationInfo-r18																								NR-IntegrityParametersTRP-LocationInfo-r18

OPTIONAL, -- Cond Integrity1

nr-IntegrityParametersDL-PRS-BeamInfo-r18

NR-IntegrityParametersDL-PRS-BeamInfo-r18

OPTIONAL, -- Cond Integrity2

nr-IntegrityParametersRTD-Info-r18																										NR-IntegrityParametersRTD-Info-r18

OPTIONAL, -- Cond Integrity3

nr-IntegrityParametersTRP-BeamAntennaInfo-r18																							NR-IntegrityParametersTRP-BeamAntennaInfo-r18

OPTIONAL, -- Cond Integrity4

nr-PRU-DL-Info-r18					NR-PRU-DL-Info-r18				OPTIONAL	-- Need ON

]],

[[

nr-TRP-LocationInfo-Implicit-r19	NR-TRP-LocationInfo-Implicit-r19	OPTIONAL	-- Need ON

]]

}

NR-IntegrityParametersTRP-LocationInfo-r18 ::= SEQUENCE {

trp-ErrorCorrelationTime-r18					INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceSetARP-ErrorCorrelationTime-r18	INTEGER(0..255)		OPTIONAL, -- Need ON

dl-PRS-ResourceARP-ErrorCorrelationTime-r18		INTEGER(0..255)		OPTIONAL, -- Need ON

...

}

NR-IntegrityParametersDL-PRS-BeamInfo-r18 ::= SEQUENCE {

dl-PRS-BeamInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersRTD-Info-r18 ::= SEQUENCE {

rtd-ErrorCorrelationTime-r18		INTEGER (0..255),

...

}

NR-IntegrityParametersTRP-BeamAntennaInfo-r18 ::= SEQUENCE {

trp-BeamAntennaInfoErrorCorrelationTime-r18		INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is optionally present, need OR, if nr-TRP-LocationInfo is present and nr-IntegrityTRP-LocationBounds is present in IE NR-TRP-LocationInfo; otherwise it is not present. |
| Integrity2 | The field is optionally present, need OR, if nr-DL-PRS-BeamInfo is present and nr-IntegrityBeamInfoBounds is present in IE NR-DL-PRS-BeamInfo; otherwise it is not present. |
| Integrity3 | The field is optionally present, need OR, if nr-RTD-Info is present and nr-IntegrityRTD-InfoBounds is present in IE NR-RTD-Info; otherwise it is not present. |
| Integrity4 | The field is optionally present, need OR, if nr-TRP-BeamAntennaInfo is present and nr-IntegrityBeamPowerBounds is present in IE NR-TRP-BeamAntennaInfo; otherwise it is not present. |

NR-PositionCalculationAssistance field descriptions

nr-TRP-LocationInfo This field provides the location coordinates of the TRPs and location coordinates of antenna reference points for DL-PRS Resource Set(s) and DL-PRS Resources of the TRPs.

nr-DL-PRS-BeamInfo This field provides the spatial directions of DL-PRS Resources for TRPs.

nr-RTD-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs.

nr-TRP-BeamAntennaInfo This field provides the relative DL-PRS Resource power between DL-PRS Resources per angle per TRP.

nr-DL-PRS-ExpectedLOS-NLOS-Assistance This field provides the expected likelihood of a LOS propagation path from a TRP to the target device. The information is provided per TRP or per DL-PRS Resource.

nr-DL-PRS-TRP-TEG-Info This field provides the TRP Tx TEG ID associated with the transmission of each DL-PRS Resource of the TRP.

nr-IntegrityServiceParameters This field specifies the range of Integrity Risk (IR) for which the integrity assistance data are valid.

nr-IntegrityServiceAlert This field indicates whether the corresponding assistance data can be used for integrity related applications.

trp-ErrorCorrelationTime This field specifies the TRP Error Correlation Time which is the upper bound of the correlation time of the TRP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceSetARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource Set ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource Set ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

dl-PRS-ResourceARP-ErrorCorrelationTime This field, if present, specifies the DL-PRS Resource ARP Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource ARP error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

rtd-ErrorCorrelationTime This field specifies the inter-TRP synchronization error Correlation Time which is the upper bound of the correlation time of the inter-TRP synchronization error. The correlation time is calculated using: t=10i, 1≤&i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, 234<i [s] Where i is the value given by rtdErrorCorrelationTime. Range is 1-28,200 s.

dl-PRS-BeamInfoErrorCorrelationTime This field specifies the Beam Boresight Direction Angle Error Correlation Time which is the upper bound of the correlation time of the DL-PRS Resource angle error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

trp-BeamAntennaInfoErrorCorrelationTime This field specifies the Mean Beam Power Error Correlation Time which is the upper bound of the correlation time of the mean beam power error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

nr-PRU-DL-Info This field provides the measurements reported by a PRU to the target UE.

nr-TRP-LocationInfo-Implicit This field provides implicit information on location coordinates for a group of TRPs within a cell. If the cell contains only one TRP then this field provides implicit information on location coordinate of the single TRP in the cell. NOTE: This field is only applicable to NR DL AI/ML positioning.

[…]

#### –	NR-TRP-LocationInfo-Implicit

The IE NR-TRP-LocationInfo-Implicit provides information to enable a target device to determine whether the location coordinates of all TRPs within a cell are consistent between training and inference phases for NR DL AI/ML positioning.

-- ASN1START

NR-TRP-LocationInfo-Implicit-r19 ::= SEQUENCE (SIZE (1..maxCellIDs-r19)) OF

TRP-LocationInfo-Implicit-Element-r19

TRP-LocationInfo-Implicit-Element-r19 ::= SEQUENCE {

nr-CellGlobalID-r19				NCGI-r15					,

nr-AIML-AssociatedID-r19		INTEGER (0..255),

...

}

-- ASN1STOP

NR-TRP-LocationInfo-Implicit field descriptions

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR.

nr-AIML-AssociatedID This field provides an identity associated with the coordinates of all TRP(s) belonging to the cell indicated by nr-CellGlobalID. A cell may contain only a single TRP. The value of the nr-AIML-AssociatedID is changed if/when the coordinates of the TRP(s) belonging to the indicated cell is changed. NOTE 1: The target device is not expected to receive different values of nr-AIML-AssociatedID for TRPs belonging to the same cell. NOTE 2: Whether the nr-AIML-AssociatedID is also related to the location of ARPs of a specific TRP is left to network implementation. NOTE 3: The nr-AIML-AssociatedID is not used to implicitly indicate the location coordinates of PRS-only TPs.

[…]

### 6.5.13	NR DL AI/ML Positioning

This clause defines the information elements for NR DL AI/ML positioning (TS 38.305 [40]).

#### 6.5.13.1	NR DL AI/ML Positioning Assistance Data

#### –	NR-DL-AIML-ProvideAssistanceData

The IE NR-DL-AIML-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based DL AI/ML positioning. It may also be used to provide NR DL AI/ML positioning specific error reason.

-- ASN1START

NR-DL-AIML-ProvideAssistanceData-r19 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r19		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r19		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-r19

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r19

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17																				OPTIONAL,	-- Need ON

assistanceDataValidityArea-r19		AreaID-CellList-r17					OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r19

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Need ON

nr-DL-AIML-Positioning-Error-r19	NR-DL-AIML-Positioning-Error-r19	OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-DL-AIML-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. NOTE: If this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-DL-TDOA-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-AIML-ProvideAssistanceData message.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-DL-TDOA-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-AIML-ProvideAssistanceData message.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-AIML-ProvideAssistanceData is valid.

nr-PositionCalculationAssistance This field provides the position calculation assistance data.

nr-DL-AIML-Positioning-Error This field provides DL AI/ML positioning error reasons.

NOTE:	If any assistance data elements included in IE NR-DL-AIML-ProvideAssistanceData are provided for one or more TRPs identified by a DL-PRS ID, the optionally present NCGI – the globally unique identity of a cell in NR, as defined in TS 38.331 [35] – is also included, if applicable.

[…]

## 6.6	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

maxMBS-r14					INTEGER ::= 64

maxWLAN-AP-r13				INTEGER ::= 64

maxKnownAPs-r14				INTEGER ::= 2048

maxVisibleAPs-r14			INTEGER ::= 32

maxWLAN-AP-r14				INTEGER ::= 128

maxWLAN-DataSets-r14		INTEGER ::= 8

maxBT-Beacon-r13			INTEGER ::= 32

maxBT-BeaconAntElt-r18		INTEGER ::= 74

maxBT-BeaconAD-r18			INTEGER ::= 64

nrMaxBands-r16							INTEGER ::= 1024	-- Maximum number of supported bands in

-- UE capability.

nrMaxFreqLayers-r16						INTEGER ::= 4		-- Max freq layers

nrMaxFreqLayers-1-r16					INTEGER ::= 3

nrMaxNumDL-PRS-ResourcesPerSet-1-r16	INTEGER ::= 63

nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16	INTEGER ::= 7

nrMaxResourceIDs-r16					INTEGER ::= 64		-- Max Resource IDs

nrMaxResourceOffsetValue-1-r16			INTEGER ::= 511

nrMaxResourcesPerSet-r16				INTEGER ::= 64		-- Maximum resources for one set

nrMaxSetsPerTrpPerFreqLayer-r16			INTEGER ::= 2		-- Maximum resource sets for one TRP

nrMaxSetsPerTrpPerFreqLayer-1-r16		INTEGER ::= 1

nrMaxTRPs-r16							INTEGER ::= 256		-- Max TRPs per UE

nrMaxTRPsPerFreq-r16					INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxTRPsPerFreq-1-r16					INTEGER ::= 63

maxSimultaneousBands-r16				INTEGER ::= 4		-- Maximum number of simultaneously

-- measured bands

maxBandComb-r16							INTEGER ::= 1024

nrMaxConfiguredBands-r16				INTEGER ::= 16

maxNumOfRxTEGs-r17						INTEGER ::= 32

maxNumOfRxTEGs-1-r17					INTEGER ::= 31

maxNumOfTxTEGs-1-r17					INTEGER ::= 7

maxTxTEG-Sets-r17						INTEGER ::= 256		-- Maximum applicable number is 64

maxNumOfRxTxTEGs-1-r17					INTEGER ::= 255

maxNumOfTRP-TxTEGs-1-r17				INTEGER ::= 7

maxNumOfSRS-PosResources-r17			INTEGER ::= 64

maxNumOfSRS-PosResources-1-r17			INTEGER ::= 63

maxNumResourcesPerAngle-r17				INTEGER ::= 24

maxNumPrioResources-r17					INTEGER ::= 24

maxAddMeasTDOA-r17						INTEGER ::= 31

maxAddMeasAoD-r17						INTEGER ::= 23

maxAddMeasRTT-r17						INTEGER ::= 31

maxOD-DL-PRS-Configs-r17				INTEGER ::= 8

maxCellIDsPerArea-r17					INTEGER ::= 256

maxNrOfAreas-r17						INTEGER ::= 16

maxMeasInstances-r17					INTEGER ::= 32

nrMaxNumPRS-BandWidthAggregation-r18	INTEGER ::= 256		-- Max number of DL-PRS bandwidth

-- aggregation configurations that a

-- location server can provide to a UE

nrNumOfSamples-r18						INTEGER ::= 4		-- NSample of RSCP/RSCPD

nrNumOfSamples-1-r18					INTEGER ::= 3

maxCellIDs-r19							INTEGER ::= 256

-- ASN1STOP