3GPP TSG-RAN WG2 Meeting #101	R2-180xxxx

# Athens, Greece, 26th February - 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections identified during ASN.1 review (RAN2 NR AH 1801), and email discussions after the AH. This CR is based on R2-1801218 Baseline TS 38331 v1.0.1 for ASN.1 review |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be completed. Guidance for CR editors: - To avoid change marks for language formatting (typically happens when many users edit the same doc), please do the following word setting: Review panel => Language => Set proofing languge => Detect automatically => OFF - Set the “User name” to indicate the company name. - When storing the CR in 3GPP folder, companies should add their Company ID (one letter) to the file name (see RIL). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 38.331 V1.0.1 (2017-12)

Technical Specification

3rd Generation Partnership Project

Technical Specification Group Radio Access Network

NR

Radio Resource Control (RRC)

Protocol specification

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	12

1	Scope	13

2	References	13

3	Definitions, symbols and abbreviations	14

3.1	Definitions	14

3.2	Abbreviations	14

4	General	16

4.1	Introduction	16

4.2	Architecture	16

4.2.1	UE states and state transitions including inter RAT	16

4.2.2	Signalling radio bearers	19

4.3	Services	19

4.3.1	Services provided to upper layers	19

4.3.2	Services expected from lower layers	19

4.4	Functions	19

5	Procedures	20

5.1	General	20

5.1.1	Introduction	20

5.1.2	General requirements	20

5.2	System information	21

5.2.1	Introduction	21

5.2.2	System information acquisition	21

5.2.2.1	General UE requirements	21

5.2.2.2	SI validity and need to (re-)acquire SI	22

5.2.2.2.1	SI validity	22

5.2.2.2.2	SI change indication and PWS notification	22

5.2.2.3	Acquisition of System Information	23

5.2.2.3.1	Acquisition of MIB and SIB1	23

5.2.2.3.2	Acquisition of an SI message	23

5.2.2.3.3	Request for on demand system information	24

5.2.2.4	 Actions upon receipt of SI message	25

5.2.2.4.1	Actions upon reception of the MIB	25

5.2.2.4.2	Actions upon reception of the SystemInformationBlockType1	25

5.2.2.4.3	Actions upon reception of SystemInformationBlockTypeX	25

5.2.2.5	Essential system information missing	25

5.3	Connection control	26

5.3.1	Introduction	26

5.3.2	Paging	26

5.3.3	RRC connection establishment	26

5.3.4	Initial security activation	26

5.3.5	RRC reconfiguration	26

5.3.5.1	General	26

5.3.5.2	Initiation	27

5.3.5.3	Reception of an RRCReconfiguration by the UE	27

5.3.5.4	Secondary cell group release	28

5.3.5.5	Cell Group configuration	28

5.3.5.5.1	General	28

5.3.5.5.2	Reconfiguration with sync	29

5.3.5.5.3	RLC bearer release	29

5.3.5.5.4	RLC bearer addition/modification	30

5.3.5.5.5	MAC entity configuration	30

5.3.5.5.6	RLF Timers & Constants configuration	31

5.3.5.5.7	SPCell Configuration	31

5.3.5.5.8	SCell Release	31

5.3.5.5.9	SCell Addition/Modification	31

5.3.5.6	Radio Bearer configuration	32

5.3.5.6.1	General	32

5.3.5.6.2	SRB release	32

5.3.5.6.3	SRB addition/modification	32

5.3.5.6.4	DRB release	33

5.3.5.6.5	DRB addition/modification	33

5.3.5.7	Full configuration	34

5.3.5.8	Security key update	36

5.3.5.9	Reconfiguration failure	36

5.3.5.9.1	Integrity check failure	36

5.3.5.9.2	Inability to comply with RRCReconfiguration	36

5.3.5.9.3	T304 expiry (Reconfiguration with sync Failure)	36

5.3.5.9	Other configuration	37

5.3.5.10 EN-DC release	37

5.3.6	Counter check	37

5.3.7	RRC connection re-establishment	37

5.3.8	RRC connection release	37

5.3.9	RRC connection release requested by upper layers	37

5.3.10	Radio link failure related actions	37

5.3.10.1	Detection of physical layer problems in RRC_CONNECTED	37

5.3.10.2	Recovery of physical layer problems	37

5.3.10.3	Detection of radio link failure	38

5.3.11	UE actions upon leaving RRC_CONNECTED	38

5.3.12	UE actions upon PUCCH/SRS release request	38

5.4	Inter-RAT mobility	38

5.5	Measurements	39

5.5.1	Introduction	39

5.5.2	Measurement configuration	41

5.5.2.1	General	41

5.5.2.2	Measurement identity removal	41

5.5.2.3	Measurement identity addition/modification	42

5.5.2.4	Measurement object removal	42

5.5.2.5	Measurement object addition/modification	42

5.5.2.6	Reporting configuration removal	44

5.5.2.7	Reporting configuration addition/modification	44

5.5.2.8	Quantity configuration	44

5.5.2.9	Measurement gap configuration	44

5.5.2.10	Reference signal measurement timing configuration	45

5.5.3	Performing measurements	45

5.5.3.1	General	45

5.5.3.2	Layer 3 filtering	46

5.5.3.3	Derivation of cell measurement results	47

5.5.3.3a	Derivation of layer 3 beam filtered measurement	48

5.5.4	Measurement report triggering	48

5.5.4.1	General	48

5.5.4.2	Event A1 (Serving becomes better than threshold)	49

5.5.4.3	Event A2 (Serving becomes worse than threshold)	50

5.5.4.4	Event A3 (Neighbour becomes offset better than PCell/PSCell)	50

5.5.4.5	Event A4 (Neighbour becomes better than threshold)	51

5.5.4.6	Event A5 (PCell/PSCell becomes worse than threshold1 and neighbour becomes better than threshold2)	52

5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)	53

5.5.5	Measurement reporting	53

5.5.5.1	General	53

5.5.5.2	Reporting of beam measurement information	55

5.6	UE capabilities	56

5.6.1	UE capability transfer	56

5.6.1.1	General	56

5.6.1.3	Reception of the UECapabilityEnquiry by the UE	56

5.6.1.4	Compilation of band combinations supported by the UE	56

5.6.1.5	Compilation of baseband processing combinations supported by the UE	57

5.7	Other	57

5.7.1	DL information transfer	57

5.7.2	UL information transfer	57

5.7.3	SCG failure information	57

5.7.3.1	General	57

5.7.3.2	Initiation	58

5.7.3.3	Failure type determination	58

5.7.3.4	Setting the contents of MeasResultSCG-Failure	59

6	Protocol data units, formats and parameters (ASN.1)	60

6.1	General	60

6.1.1	Introduction	60

6.1.2	Need codes and conditions for optional downlink fields	60

6.2	RRC messages	61

6.2.1	General message structure	61

–	NR-RRC-Definitions	61

–	BCCH-BCH-Message	62

–	DL-DCCH-Message	62

–	UL-DCCH-Message	63

6.2.2	Message definitions	63

–	MIB	63

–	MeasurementReport	64

–	RRCReconfiguration	65

–	RRCReconfigurationComplete	67

–	SIB1	68

6.3	RRC information elements	69

6.3.0	Parameterized types	69

–	SetupRelease Information Element	69

6.3.1	System information blocks	70

6.3.2	Radio resource control information elements	70

–	AdditionalSpectrumEmission	70

–	Alpha	70

–	ARFCN-ValueNR	70

–	BandwidthPart-Config	71

–	BeamFailureDetectionConfig	73

–	BeamFailureRecoveryConfig	73

–	CellGroupConfig	74

–	ControlResourceSetId	76

–	CrossCarrierSchedulingConfig	77

–	CSI-MeasConfig	78

–	DMRS-DownlinkConfig	87

–	DMRS-UplinkConfig	88

–	DRB-Identity	89

–	MeasResultSCG-Failure	89

–	FrequencyInfoDL	90

–	SCS-SpecificVirtualCarrier	90

–	FrequencyInfoUL	91

–	GSCN-ValueNR	91

–	LogicalChannelConfig	92

–	MAC-CellGroupConfig	93

–	MeasConfig	98

–	MeasGapConfig	99

–	MeasId	100

–	MeasIdToAddModList	100

–	MeasObjectEUTRA	101

–	MeasObjectId	101

–	MeasObjectNR	101

–	MeasObjectToAddModList	108

–	MeasResults	108

–	PDCCH-ConfigCommon	112

–	PDCCH-Config	112

–	PDCP-Config	115

–	PDSCH-Config	118

–	PCI-List	121

–	PCI-Range	122

–	PCI-RangeIndex	122

–	PCI-RangeIndexList	123

–	PhysCellId	123

–	PRB-Id	123

–	PTRS-DownlinkConfig	123

–	PTRS-UplinkConfig	124

–	PUCCH-Config	125

–	PUSCH-Config	131

–	PUSCH-PowerControl	133

–	Q-OffsetRange	135

–	QuantityConfig	135

–	RACH-ConfigCommon	137

–	RACH-ConfigCommonGeneric	139

–	RACH-ConfigDedicated	139

–	RadioBearerConfig	140

–	ReportConfigId	142

–	ReportConfigNR	143

–	ReportConfigToAddModList	146

–	ReportInterval	147

–	RLC-Config	147

–	RLF-TimersAndConstants	150

–	RNTI-Value	151

–	RSRP-Range	151

–	RSRQ-Range	151

–	SINR-Range	152

–	SCellIndex	152

–	SchedulingRequest-Config	152

–	SchedulingRequestResourceConfig	153

–	SchedulingRequestResourceId	154

–	ScramblingId	154

–	SDAP-Config	155

–	SearchSpace	156

–	SlotFormatIndicatorSFI	158

–	DownlinkPreemption	158

–	SearchSpaceId	159

–	SecurityAlgorithmConfig	160

–	ServCellIndex	160

–	ServingCellConfigCommon	161

–	ServingCellConfig	163

–	SlotFormatCombinationsPerCell	164

–	SRB-Identity	165

–	SPS-Config	166

–	ConfiguredGrantConfig	166

–	SRS-Config	168

–	SRS-CarrierSwitching	171

–	SSB-Index	173

–	SubcarrierSpacing	173

–	TCI-State	174

–	TDD-UL-DL-Config	174

–	ZP-CSI-RS-Resource	176

6.3.3	UE capability information elements	177

–	BandCombinationList	177

–	RAT-Type	179

–	UE-CapabilityRAT-ContainerList	179

–	UE-MRDC-Capability	180

–	UE-NR-Capability	181

6.3.4	Other information elements	183

6.4	RRC multiplicity and type constraint values	183

–	Multiplicity and type constraint definitions	183

–	End of NR-RRC-Definitions	187

7	Variables and constants	188

7.1	Timers	188

7.1.1	Timers (Informative)	188

7.1.2	Timer handling	188

7.2	Counters	188

7.3	Constants	188

7.4	UE variables	189

–	NR-UE-Variables	189

–	VarMeasConfig	189

–	VarMeasReportList	190

–	End of NR-UE-Variables	190

8	Protocol data unit abstract syntax	192

8.1	General	192

8.2	Structure of encoded RRC messages	192

8.3	Basic production	192

8.4	Extension	193

8.5	Padding	193

9	Specified and default radio configurations	193

9.1	Specified configurations	193

9.1.1	Logical channel configurations	194

9.1.2	SRB configurations	194

9.1.2.1	SRB1/SRB1S	194

9.1..2.2	SRB2/SRB2S	194

9.1.2.3	SRB3	194

9.2	Default radio configurations	194

9.2.1	SRB configurations	194

9.2.1.1	SRB1/SRB1S	194

9.2.1.2	SRB2/SRB2S	195

9.2.1.3	SRB3	195

10	Generic error handling	196

10.1	General	196

10.2	ASN.1 violation or encoding error	196

10.3	Field set to a not comprehended value	196

10.4	Mandatory field missing	197

10.5	Not comprehended field	198

11	Radio information related interactions between network nodes	199

11.1	General	199

11.2	Inter-node RRC messages	199

11.2.1	General	199

11.2.2	Message definitions	200

–	HandoverCommand	200

–	HandoverPreparationInformation	200

–	CG-Config	202

–	CG-ConfigInfo	203

11.3	Inter-node RRC information element definitions	205

–	CandidateCellInfoList	205

11.4	Inter-node RRC multiplicity and type constraint values	207

–	End of NR-InterNodeDefinitions	207

12	Processing delay requirements for RRC procedures	208

Annex A (informative):	Guidelines, mainly on use of ASN.1	208

A.3.8	Guidelines on use of parameterised SetupRelease type	219

A.3.9	Guidelines on use of ToAddModList and ToReleaseList	220

–	ParentIE-WithEM	229

–	ChildIE1-WithoutEM	229

–	ChildIE2-WithoutEM	230

A.6	Guidelines regarding use of need codes	231

A.7	Guidelines regarding use of conditions	232

Annex <X> (informative): Change history	234

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Radio Resource Control protocol for the radio interface between UE and NG-RAN.

The scope of the present document also includes:

-	the radio related information transported in a transparent container between source gNB and target gNB upon inter gNB handover;

-	the radio related information transported in a transparent container between a source or target gNB and another system upon inter RAT handover.

-	the radio related information transported in a transparent container between a source eNB and target gNB during E-UTRA-NR Dual Connectivity.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[13]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception".

[16]	3GPP TS 38.211: “NR; Physical channels and modulation”.

[17]	3GPP TS 38.212: “NR; Multiplexing and channel coding”.

[19]	3GPP TS 38.214: “NR NR; Physical layer procedures for data”.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Field: The individual contents of an information element are referred as fields.

Floor: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Information element: A structural element containing a single or multiple fields is referred as information element.

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

SRB1S: The SCG part of MCG split SRB1 for EN-DC.

SRB2S: The SCG part of MCG split SRB2 for EN-DC.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CCCH	Common Control Channel

CG	Cell Group

CMAS	Commercial Mobile Alert Service

CP	Control Plane

C-RNTI	Cell RNTI

CSI	Channel State Information

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH                  Dedicated Traffic Channel

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

MAC	Medium Access Control

MCG	Master Cell Group

MIB	Master Information Block

N/A	Not Applicable

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PSCell	Primary Secondary Cell

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RNTI	Radio Network Temporary Identifier

ROHC	RObust Header Compression

RRC	Radio Resource Control

RS	Reference Signal

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SpCell	Special Cell

SRB	Signalling Radio Bearer

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TM	Transparent Mode

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

## 4.1	Introduction

This specification is organised as follows:

-	sub-clause 4.2 describes the RRC protocol model;

-	sub-clause 4.3 specifies the services provided to upper layers as well as the services expected from lower layers;

-	sub-clause 4.4 lists the RRC functions;

-	clause 5 specifies RRC procedures, including UE state transitions;

-	clause 6 specifies the RRC messages in ASN.1 and description;

-	clause 7 specifies the variables (including protocol timers and constants) and counters to be used by the UE;

-	clause 8 specifies the encoding of the RRC messages;

-	clause 9 specifies the specified and default radio configurations;

-	clause 10 specifies generic error handling;

-	clause 11 specifies the RRC messages transferred across network nodes;

-	clause 12 specifies the UE capability related constraints and performance requirements.

## 4.2	Architecture

Editor's note	The state model is still a subject for discussion.FFS

### 4.2.1	UE states and state transitions including inter RAT

Editor’s Note: For EN_DC, only RRC_CONNECTED is applicable.

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors a Paging channel;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information.

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	UE controlled mobility based on network configuration;

- 	The UE stores the AS context;

-	The UE:

-	Monitors a Paging channel;

-	Performs neighbouring cell measurements and cell (re-)selection;

- 	Performs RAN-based notification area updates when moving outside the RAN-based notification area;

Editor’s Note: FFS Whether a RAN-based notification area is always configured or not.

Editor’s Note: FFS UE behavior if it is decided that a RAN-based notification area is not always configured.

-	Acquires system information.

-	RRC_CONNECTED:

-	The UE stores the AS context.

-	Transfer of unicast data to/from UE.

-	At lower layers, the UE may be configured with a UE specific DRX.

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility, i.e. handover within NR and to/from E-UTRAN.

-	The UE:

-	Monitors a Paging channel;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell measurements and measurement reporting;

-	Acquires system information.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/NGC and E-UTRAN/EPC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/NGC and E-UTRAN/EPC

The UE state machine, state transition and mobility procedures between NR/NGC and E-UTRA/NGC is FFS.

### 4.2.2	Signalling radio bearers

## 4.3	Services

### 4.3.1	Services provided to upper layers

The RRC protocol offers the following services to upper layers:

-	Broadcast of common control information;

-	Notification of UEs in RRC_IDLE, e.g. about a terminating call [FFS, for ETWS, for CMAS];

-	Transfer of dedicated control information, i.e. information for one specific UE.

### 4.3.2	Services expected from lower layers

In brief, the following are the main services that RRC expects from lower layers:

-	PDCP: integrity protection, ciphering and in-sequence delivery of information without duplication [FFS if duplication need to be listed];

-	RLC: reliable transfer of information, without introducing duplicates and with support for segmentation.

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE and RRC_INACTIVE, e.g. cell (re-)selection parameters, neighbouring cell information and information (also) applicable for UEs in RRC_CONNECTED, e.g. common channel configuration information.

-	[FFS Including ETWS notification, CMAS notification]

-	RRC connection control:

-	Paging;

-	Establishment/modification/suspension/resumption/release of RRC connection, including e.g. assignment/modification of UE identity (C-RNTI), establishment/modification/release of SRBs, access class barring;

Editor’s note: The terminology for establishment/modification/suspension/resumption is FFS.

-	Initial security activation, i.e. initial configuration of AS integrity protection (SRBs) and AS ciphering (SRBs, DRBs);

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated security handling, i.e. key/algorithm change, specification of RRC context information transferred between network nodes;

-	Establishment/modification/release of RBs carrying user data (DRBs);

-	Radio configuration control including e.g. assignment/modification of ARQ configuration, HARQ configuration, DRX configuration;

-	In case of DC, cell management including e.g. change of PSCell, addition/modification/release of SCG cell(s).

-	Recovery from radio link failure;

-	Inter-RAT mobility including e.g. security activation, transfer of RRC context information;

-	Measurement configuration and reporting:

-	Establishment/modification/release of measurements (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting;

-	Other functions including e.g. transfer of dedicated NAS information, transfer of UE radio access capability information [FFS support for RAN sharing (multiple PLMN identities)];

# 5	Procedures

## 5.1	General

### 5.1.1	Introduction

This section covers the general requirements.

### 5.1.2	General requirements

The UE shall:

1>	process the received messages in order of reception by RRC, i.e. the processing of a message shall be completed before starting the processing of a subsequent message;

NOTE 1:  Networl may initiate a subsequent procedure prior to receiving the UE's response of a previously initiated procedure.

1>	within a sub-clause execute the steps according to the order specified in the procedural description;

1>	consider the term 'radio bearer' (RB) to cover SRBs and DRBs unless explicitly stated otherwise;

1>	set the rrc-TransactionIdentifier in the response message, if included, to the same value as included in the message received from NR that triggered the response message;

1>	upon receiving a choice value set to setup:

2>	apply the corresponding received configuration and start using the associated resources, unless explicitly specified otherwise;

1>	upon receiving a choice value set to release:

2>	clear the corresponding configuration and stop using the associated resources;

1>	in case the size of a list is extended, upon receiving an extension field comprising the entries in addition to the ones carried by the original field (regardless of whether NR signals more entries in total); apply the following generic behaviour unless explicitly stated otherwise:

2>	create a combined list by concatenating the additional entries included in the extension field to the original field while maintaining the order among both the original and the additional entries;

2>	for the combined list, created according to the previous, apply the same behaviour as defined for the original field;

## 5.2	System information

Editor’s Note: Targeted for completion in June 2018. For EN_DC, only parts related to MIB acquisition, in sub-clauses 5.2.2.3.1 and 5.2.2.4.1, are applicable.

### 5.2.1	Introduction

System Information (SI) is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs) where:

-	the MasterInformationBlock (MIB) is always transmitted on the BCH (refer Figure 5.2.2.X.X FFS_Ref) with a periodicity of 80 ms and repetitions made within 80 ms [X] and it includes parameters that are needed to acquire SystemInformationBlockType1 (SIB1) from the cell [FFS TBD-RAN1];

-	the SystemInformationBlockType1 (SIB1) is transmitted on the DL-SCH with a periodicity of [X] and repetitions made within [X]. SIB1 includes information regarding the availability and scheduling (e.g. periodicity, SI-window size) of other SIBs. It also indicates whether they (i.e. other SIBs) are provided via periodic broadcast basis or only on-demand basis (refer Figure 5.2.2.X.X FFS_Ref). If other SIBs are provided on-demand then SIB1 includes information for the UE to perform SI request;

-	SIBs other than SystemInformationBlockType1 are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows);

-	For PSCell and SCells, RAN provides the required SI by dedicated signalling. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI, RAN releases and adds the concerned SCell/PSCell.

Editor’s Note: Reference to RAN1 specification may be used for the MIB/SIB1 periodicities [X].FFS

### 5.2.2	System information acquisition

#### 5.2.2.1	General UE requirements

Figure 5.2.2.X-X: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS- and NAS information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MasterInformationBlock, SystemInformationBlockType1 as well as SystemInformationBlockTypeX through SystemInformationBlockTypeY (depending on support of the concerned RATs for UE controlled mobility).

The UE in RRC_CONNECTED shall ensure having a valid version of (at least) the MasterInformationBlock, SystemInformationBlockType1 as well as SystemInformationBlockTypeX (depending on support of mobility towards the concerned RATs).

The UE shall store relevant SI acquired from the currently camped/serving cell.  A version of the SI that the UE acquires and stores remains valid only for a certain time. The UE may use such a stored version of the SI e.g. after cell re-selection, upon return from out of coverage or after SI change indication.

Editor’s Note: [FFS_Standalone if the UE is required to store SI other than for the currently camped/serving cell].

Editor’s Note: [FFS_Standalone if different versions of SIBs are provided].

Editor’s Note: [FFS_Standalone UE may or shall store several versions of SI].

Editor’s Note: FFS_Standalone To be updated when above is resolved. Another sub-clause under 5.2.2.2 can be considered depending on the resolution of above.

#### 5.2.2.2	SI validity and need to (re)-acquire SI

The UE shall apply the SI acquisition procedure as defined in clause 5.2.2.3 upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync with key change completion, after entering RAN from another RAT; whenever the UE does not have a valid version in the stored SI.

Editor’s Note: [FFS_Standalone if upon receiving HO command the SI acquisition depend on stored SI]

When the UE acquires a MasterInformationBlock or a SystemInformationBlockType1 or a SI message in a currently camped/serving cell as described in clause 5.2.2.3, the UE shall store the acquired SI.

##### 5.2.2.2.1	SI validity

The UE shall:

1>	delete any stored version of SI after [FFS] hours from the moment it was successfully confirmed as valid;

1>	if the UE does not have in the stored SI a valid version for the required SI corresponding to the systemInfoAreaIdentifier and systemInfoValueTag/systemInfoConfigurationIndex of that SI in the currently camped/serving cell:

2> (re)acquire the SI as specified in clause 5.2.2.3.

NOTE: 	At the SI acquisition procedure, the UE may assume the acquired SI in the currently camped/serving cell to be valid in other cells than the currently camped/serving cell based on systemInfoAreaIdentifier and systemInfoValueTag/systemInfoConfigurationIndex.

Editor’s Note: [FFS_Standalone terminology to be used is systemInfoValueTag or systemInfoConfigurationIndex]

Editor’s Note: [FFS_Standalone terminology to be used for area ID is systemInfoAreaIdentifier]

Editor’s Note: [FFS_Standalone whether the area ID and valuetag is separately signalled or as a single identifier]

Editor’s Note: [FFS_Standalone whether the area ID is associated to each SIB/SI message or associated to a group of SIBs/SI messages or all SIBs/SI messages]

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI is provided in the modification period following the one where SI change indication is transmitted. RAN transmits SI change indication and PWS notification through paging. Repetitions of SI change indication may occur within preceding modification period.

Editor’s Note	: The above descriptive text can remain in this sub-clause or moved under 5.2.1. FFS_Standalone

If the UE is in RRC_CONNECTED or is configured to use a DRX cycle smaller than the modification period in RRC_IDLE or in RRC_INACTIVE and receives a Paging message:

1>	if the received Paging message includes the etws/cmasNotification;

2> the UE shall immediately re-acquire the SIB1 and apply the SI acquisition procedure as defined in sub-clause [X.X.X.X FFS_Ref].

1> else, if the received Paging message includes the systemInfoModification;

2>	the UE shall apply the SI acquisition procedure as defined in sub-clause [X.X.X.X FFS_Ref] from the start of the next modification period.

NOTE: For PWS notification the SIB1 is re-acquired to know the scheduling information for the PWS messages.

Editor’s Note: [FFS_Standalone if upon receiving a SI change indication the SI acquisition depend on stored SI]

Editor’s Note: [FFS_Standalone if value tags and area identifier included in paging message to reacquire SIB1]

Editor’s Note: [FFS_Standalone the update mechanism for access control notifications and other non-access control configuration updates]

#### 5.2.2.3	Acquisition of System Information

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	if the cell is a PSCell:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	perform the actions specified in section 5.2.2.4.1;

1>	else:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2> if the UE is unable to acquire the MIB;

3> follow the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in section 5.2.2.4.1;

2>	acquire the SystemInformationBlockType1 as specified in [X];

2>	if the UE is unable to acquire the SystemInformationBlockType1:

3> follow the actions as specified in clause 5.2.2.5;

2>	else:

3>perform the actions specified in section 5.2.2.4.2.

Editor’s Note: Reference to RAN1 [X] specification may be used for the scheduling of SIB1.FFS_Standalone

##### 5.2.2.3.2	Acquisition of an SI message

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

Editor’s Note: [FFS_Standalone the details of the mapping to subframes/slots where the SI messages are scheduled]

Editor’s Note: [FFS_Standalone if there are any exceptions on e.g. subframes where SI messages cannot be transmitted]

Editor’s Note: [FFS_Standalone if the SI-windows of different SI messages do not overlap].

Editor’s Note: [FFS_Standalone if multiple SI messages can be mapped to same SI window]

Editor’s Note: [FFS_Standalone if the length of SI-window is common for all SI messages or if it is configured per SI message]

Editor’s Note: [FFS_Standalone if the UE may accumulate the SI-Message transmissions across several SI-Windows within the Modification Period]

1> if SI message acquisition not triggered due to UE request:

2>	receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

2>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message;

1> if SI message acquisition triggered due to UE request:

2> [FFS_Standalone receive DL-SCH using the SI-RNTI from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received];

2>	[FFS_Standalone if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message];

Editor’s Note: [FFS_Standalone on the details of from which SI-window the UE shall receive the DL-SCH upon triggering the SI request.

Editor’s Note: [FFS_Standalone on the details of how many SI-windows the UE should monitor for SI message reception if transmission triggered by UE request]

Editor’s Note: [FFS_Standalone if UE need to monitor all the TTIs in SI window for receiving SI message]

1>	store the acquired SI message as specified in clause 5.2.2.2.

Editor’s Note: FFS_Standalone The procedural text for SI message acquisition triggered by UE request will be updated upon finalizing the details.

##### 5.2.2.3.3	Request for on demand system information

When acquiring an SI message, which according to the SystemInformationBlockType1 is indicated to be provided upon UE request, the UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE:

2>	if the [FFS_Standalone] field is received in SIB1:

3>	the UE shall trigger the lower layer to initiate the preamble transmission procedure in accordance with TS 38.321 [3] using the [indicated PRACH preamble] and [indicated PRACH resource];

3>	if acknowledgement for SI request is received from lower layer;

4>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

Editor’s Note: To be updated with details of the Msg1 request procedure.FFS_Standalone

2>	else

3>	the UE shall trigger the lower layer to initiate the random access procedure in accordance with TS 38.321 [3];

3>	if acknowledgement for SI request is received;

4>	acquire the requested SI message(s) as defined in sub-clause 5.2.2.3.2;

Editor’s Note: To be updated with details of the Msg3 request procedure. FFS_Standalone

1>	else (in RRC_CONNECTED):

2> [details FFS_Standalone]

Editor’s Note: To be updated with details of the on-demand request procedure in RRC_CONNECTED. FFS_Standalone

Editor’s Note: [FFS_Standalone if there is a need for a separate sub-clause to describe case where on demand SI is not successfully received by the UE and where it should initiate a new request]

#### 5.2.2.4		Actions upon receipt of SI message

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or if the UE is in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred;

3>	consider the cell as barred in accordance with TS 38.304 [FFS];

2>	else,

3>	apply the received parameter(s) [FFS] to acquire SIB1.

##### 5.2.2.4.2	Actions upon reception of the SystemInformationBlockType1

Upon receiving the SystemInformationBlockType1 the UE shall:

1>	store the acquired SIB1;

1>	if the UE has a stored valid version of the required SIB(s) associated with the systemInfoAreaIdentifier and systemInfoValueTag/systemInfoConfigurationIndex in the acquired SIB1:

2>	use that stored version of the SIB;

1>	else if the SIB1 message indicates that the SI message(s) is only provided on request:

2>	trigger a request to acquire the SI message(s) (if needed) as defined in sub-clause 5.2.2.3;

1>	else:

2>	acquire the SI message(s) (if needed) as defined in sub-clause 5.2.2.3.2, which are provided according to the schedulingInfoList in the SystemInformationBlockType1;

Editor’s Note: [FFS_Standalone Whether there is an additional indication that an on-demand SI is actually being broadcast at this instant in time]

Editor’s Note: To be updated when content of the SystemInformationBlockType1 has been agreed. FFS_Standalone.

Editor’s Note: To be updated how to capture the UE behaviour when some required SIBs are from broadcast and other required SIBs through SI request.

##### 5.2.2.4.3	Actions upon reception of SystemInformationBlockTypeX

Editor’s Note: To be extended with further sub-clauses as more SIBs are defined. FFS_Standalone

#### 5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE:

2>	if the UE is unable to acquire the MIB; or

2>	if the UE is unable to acquire the SIB1 and UE does not have a stored valid version of SIB1; or

2> [FFS_Standalone if the UE is unable to acquire the [FFS essential SystemInformationBlockTypeX] and UE does not have a stored valid version of SystemInformationBlockTypeX];

3>	consider the cell as barred in accordance with TS 38.304 [X]; and

3>	perform barring as if intraFreqReselection is set to allowed;

Editor’s Note: [FFS_Standalone on details of RRC connection re-establishment procedure and corresponding reading of SI in RRC_CONNECTED].

Editor’s Note: [FFS_Standalone whether all the information needed to access the cell is included in SIB1 or if both SIB1 and SIB2 are essential in NR].

## 5.3	Connection control

Editor's note:	FFS The structure and content of this subclause is a subject for discussion, e.g. potential merging of connection establishment and re-establishment messages, mobility aspects etc.

### 5.3.1	Introduction

### 5.3.2	Paging

Editor’s Note: Targeted for completion in June 2018.

### 5.3.3	RRC connection establishment

Editor’s Note: Targeted for completion in June 2018.

### 5.3.4	Initial security activation

Editor’s Note: Targeted for completion in June 2018.

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

In EN-DC, SRB3 can be used to configure measurements, MAC, RLC, PDCP, physical layer and RLF timers and constants.

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroupToAddModList only when at least one DRB is setup in SCG;

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>		perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2> if RRCReconfiguration was received via SRB1:

3> construct RRCReconfigurationComplete message and submit it via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10].

3> if reconfigurationWithSync was included in spCellConfig of an SCG:

4> initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

2> else (RRCReconfiguration was received via SRB3):

3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE:	In the case of SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case of SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>  if MAC of an NR cell group successfully completes a random access procedure triggered above;

2>  stop timer T304 for that cell group;

2>  apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>  apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>  the procedure ends;

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA:

2> reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2> release the SCG configuration;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running;

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

#### 5.3.5.5	Cell Group configuration

##### 5.3.5.5.1	General

The network configures the UE with one Secondary Cell Group (SCG). For EN-DC, the MCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2> perform Reconfiguration with sync according to 5.3.5.5.2;

2> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended;

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2> perform SCell addition/modification as specified in 5.3.5.5.9;

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell and acquire the MIB of the target SpCell as specified in 5.2.2.3.1;

NOTE X:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

Editor’s Note: Verify that this does not configure some common parameters which are later discarded due to e.g. SCell release or due to LCH release.

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync;

1>	perform the measurement related actions as specified in 5.5.3.1;

##### 5.3.5.5.3	RLC bearer release

The UE shall:

1>	for each logicalChannelIdentity value included in the rlc-BearerToReleaseList that is part of the current UE configuration (LCH release); or

1>	for each logicalChannelIdentity value that is to be released 	as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity (includes discarding all pending RLC PDUs and RLC SDUs);

2>	release the DTCH logical channel.

##### 5.3.5.5.4	RLC bearer addition/modification

For each LCH-Config received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE’s current configuration contains a RLC bearer with the received logicalChannelIdentity:

2> if reestablishRLC is received:

3> re-establish the RLC entity as specified in 38.322;

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2> reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured LCH with another radio bearer. Hence servedRadioBearer is not present in this case.

1> else (a logical channel with the given logicalChannelIdentity was not configured before):

2> if the logicalChannelIdentity corresponds to an SRB (i.e. ID less than or equal to 3) and rlc-Config is not included:

3> establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2> else:

3> establish an RLC entity in accordance with the received rlc-Config;

2> if the logicalChannelIdentity corresponds to an SRB (i.e. ID less than or equal to 3) and if mac-LogicalChannelConfig is not included:

3> configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer;

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig other than tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

##### 5.3.5.5.6	RLF Timers & Constants configuration

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

Editor’s Note: In EN-DC, rlf-TimersAndConstants cannot be released. Standalone part to be complete by June 2018.

2>

stop timer T310 for this cell group, if running, and

2>		release the value of timer t310 as well as constants n310 and n310 for this cell group;

1>	else:

2>	reconfigure the value of timers and constants in accordance with received rlf-TimersAndConstants;

##### 5.3.5.5.7	SPCell Configuration

The UE shall:

1>	if the SpCellConfig contains the rlf-TimersAndConstants

2>	configure the RLF timers for this cell group as specified in 5.3.5.5.6;

1>  if the SpCellConfig contains spCellConfigDedicated:

2> configure the SpCell in accordance with the spCellConfigDedicated;

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell;

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

Editor’s Note: FFS Check automatic measurement handling for SCells.

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb-ToReleaseList:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5;

##### 5.3.5.6.2	SRB release

Editor’s note: FFS / TODO: check handling during full configuration

For each SRB with SRB Identity corresponding to srb-ToReleaseList, the UE shall:

1>	release the PDCP entity of the SRB.

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity and configure it with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the KeNB/S-KgNB as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRAin TS 36.331 includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the specified configuration defined in 9.2.1 for the corresponding SRB;

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if reestablishPDCP is set:

3>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the KeNB/S-KgNB as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the KeNB/S-KgNB as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in 38.323 [5];

2>	else, if discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

##### 5.3.5.6.4	DRB release

Editor’s Note: FFS / TODO: Add handling for the new QoS concept (mapping of flows; configuration of QFI-to-DRB mapping; reflective QoS…) but keep also EPS-Bearer handling for the EN-DC case

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration (DRB release), or

1>	for each drb-identity value that is to be released as the result of full configuration option according to 5.3.5.7:

2>	release the PDCP entity;

Editor’s Note: FFS / TODO: handling of indication to higher layers in EN-DC

1> if a new bearer is not added either with NR or E-UTRA  with same eps-BearerIdentity:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers immediately;

NOTE:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KUPenc) associated with the KeNB/S-KgNB as indicated in keyToUse;

2>	else if no DRB was configured with the same eps-BearerIdentity either by NR or  E-UTRA prior to receiving this reconfiguration:

3>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	if reestablishPDCP is set:

3>	configure the PDCP entity of this RadioBearerConfig to apply the ciphering algorithm and KUPenc key associated with the KeNB/S-KgNB as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	re-establish the PDCP entity of this DRB as specified in 38.323 [5], section 5.1.2;

2>	else, if recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in 38.323;

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

Editor’s Note: verify that TS 38.323 covers case when more than one RLC entity is associated with the PDCP entity.

NOTE:	Removal and addition of the same drb-Identity in a single radioResourceConfig is not supported. In case drb-Identity is removed and added due to reconfiguration with sync or re-establishment with the full configuration option, the network can use the same value of drb-Identity.

NOTE:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in.  To re-associate a DRB with a different key (KeNB to S-KeNB or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE: 	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

#### 5.3.5.7	Full configuration

Editor’s Note: This subclause is not applicable for EN-DC, but is targeted for completion in June 2018. For EN-DC,NR RRCReconfiguration message does not include the fullConfig IE.

The UE shall:

1>	release/clear all current dedicated radio configurations except the MCG C-RNTI, the MCG security configuration and the PDCP, RLC, logical channel configurations for the RBs and the logged measurement configuration;

NOTE X:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig.

1>	if the spCellConfig in the masterCellGroupConfig includes the reconfigurationWithSync:

2>	release/clear all current common radio configurations;

2>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	else (full configuration after re-establishment):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SystemInformationBlockType2;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

2>	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2>	apply the corresponding default RLC configuration for the SRB specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2, 9.2.1.3 for SRB3;

2>	apply the corresponding default logical channel configuration for the SRB as specified in 9.2.1.1 for SRB1 or in 9.2.1.2 for SRB2, 9.2.1.3 for SRB3;

NOTE X: 	This is to get the SRBs (SRB1 and SRB2 for handover and SRB2 for reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

Editor’s Note: FFS_Standalone: Replace the following by corresponding handling of “PDU Sessions” and/or “Flows”. Remember: Current UE configuration refers to the current MCG and SCG configuration, i.e., it handles also DRBs associated with the S-KeNB prior to the HO/Re-establishment.

1>	for each eps-BearerIdentity value included in the drb-ToAddModList that is part of the current UE configuration:

2>	release the PDCP entity;

2>	release the RLC entity or entities;

2>	release the DTCH logical channel;

2>	release the drb-identity;

NOTE:	This will retain the eps-bearerIdentity but remove the DRBs including drb-identity of these bearers from the current UE configuration and trigger the setup of the DRBs within the AS using the new configuration. The eps-bearerIdentity acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1>	for each eps-BearerIdentity value that is part of the current UE configuration but not part of the drb-ToAddModList:

2>	perform DRB release as specified in 5.3.5.6.4;

#### 5.3.5.8	Security key update

Upon reception of sk-Counter as specified in TS 36.331 [10] the UE shall:

1>	update the S-KgNB key based on the KeNB key and using the received sk-Counter value, as specified in TS 33.501 [11];

1>	derive KRRCenc and KUPenc key as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint key as specified in TS 33.501 [11];

#### 5.3.5.9	Reconfiguration failure

Editor’s Note: Added sub-sections for the different failure cases that may occur during the RRCReconfiguration procedure.

##### 5.3.5.9.1	Integrity check failure

Editor’s Note: Removed “SIB3” from heading so that this sub-section can easily be expanded to stand-alone case (if considered necessary). FFS_Standalone

The UE shall:

1>	upon integrity check failure indication from NR lower layers for SRB3:

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SRB3 integrity check failure;

##### 5.3.5.9.2	Inability to comply with RRCReconfiguration

The UE shall:

1>	if the UE is operating in EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over MCG SRB1;

3> continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10, 5.3.7], upon which the connection reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

##### 5.3.5.9.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of a secondary cell group expires:

2>  release rach-ConfigDedicated;

2>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure;

#### 5.3.5.9	Other configuration

Editor’s Note: Targeted for completion in June 2018.

#### 5.3.5.10 EN-DC release

The UE shall:

1>	as a result of EN-DC release triggered by E-UTRA:

2> release SRB3 (configured according to radioBearerConfig), if present;

2>	release measConfig;

2> release the SCG configuration as specified in section 5.3.5.4.

### 5.3.6	Counter check

FFS

### 5.3.7	RRC connection re-establishment

Editor’s Note: Targeted for completion in June 2018.

### 5.3.8	RRC connection release

Editor’s Note: Targeted for completion in June 2018.

### 5.3.9	RRC connection release requested by upper layers

Editor’s Note: Targeted for completion in June 2018.

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while T311 is not running:

2>	start timer T310 for the corresponding SpCell;

Editor’s Note: FFS: Under which condition physical layer problems detection is performed, e.g. neither T300, T301, T304 nor T311 is running. It’s subject to the harmonization of the RRC procedures for RRC Connection establishment/resume/re-establishment and RRC connection reconfiguration.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell;

NOTE 1:		In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:		Periods in time where neither "in-sync" nor "out-of-sync" is reported by layer 1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while T311 is not running; or

Editor’s Note: FFS: Under which condition physical layer problems detection is performed, e.g. neither T300, T301, T304 nor T311 is running. It’s subject to the harmonization of the RRC procedures for RRC Connection establishment/resume/re-establishment and RRC connection reconfiguration.

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

Editor’s Note: FFS whether maximum ARQ retransmission is only criteria for RLC failure.

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

Editor’s Note: FFS: How to handle RLC failure in CA duplication for MCG DRB and SRB.

Editor’s Note: FFS: RLF related measurement reports e.g. VarRLF-Report is supported in NR.

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in x.x.x FFS_Ref, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in x.x.x FFS_Ref.

The UE shall:

1>	upon T310 expiry in SpCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

Editor’s Note: FFS: How to handle RLC failure in CA duplication for SCG DRB and SRB.

2>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure;

### 5.3.11	UE actions upon leaving RRC_CONNECTED

Editor’s Note: Targeted for completion in June 2018.

### 5.3.12	UE actions upon PUCCH/SRS release request

Editor’s Note: Targeted for completion in June 2018.

## 5.4	Inter-RAT mobility

Editor’s Note: Targeted for completion in June 2018.

## 5.5	Measurements

### 5.5.1	Introduction

Editor’s Note: FFS In the context of subclause 5.5., the term “beam” will be later aligned with the RAN1 terminology when stabilized. For the current version, the term “beam” refers to the reference signals SS/PBCH Blocks and/or CSI-RS resources. Beam level measurements refer to the output of the L1 filters as defined in 38.215, i.e., SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ and CSI-SINR.

The network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration.

The network may configure the UE to perform the following types of measurements:

-	NR measurements.

-	Inter-RAT measurements of E-UTRA frequencies.

The network may configure the UE to perform the following NR measurements, based on different RS types SS/PBCH Block or CSI-RS:

-	SS/PBCH Block based intra-frequency measurements: measurements at SSB(s) of neighbour cell(s) where both the center frequency(ies) and subcarrier spacing are the same as the cell-defining SSB of each serving cell.

-	SS/PBCH Block based inter-frequency measurements: measurements at SSB(s) of neighbour cell(s) that have different center frequency(ies) or different subcarrier spacing compared to the cell-defining SSB of each serving cell.

-	CSI-RS based intra-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are within the bandwidth(s) of the CSI-RS resource(s) on the serving cell(s) configured for measurements and having the same subcarrier spacing of the CSI-RS resource(s) on the serving cell(s) configured for measurements.

-	CSI-RS based inter-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are not within the bandwidth(s) or having different subcarrier spacing compared to the CSI-RS resource(s) on the serving cell(s) configured for measurements.

Editor’s Note: FFS Whether the definition of inter-frequency and intra-frequency measurements provided by RAN4 should be removed from 38.331.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block.

-	Measurement results per cell based on SS/PBCH block(s).

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource.

-	Measurement results per cell based on CSI-RS resource(s).

-	CSI-RS resource measurement identifiers.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object is associated to an NR carrier frequency. Associated with this NR carrier frequency, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

Editor’s Note: Revisit the formulation below, and as well as how to capture the following additional agreements:

2	More than one MO with  CSI-RS resources for measurement can be associated to the same SSB location in frequency. The SSB is at least used for timing reference.

3	In case that more than one MO with  CSI-RS resources for measurement is associated to the same SSB location in frequency the UE is indicated which MO corresponds to the serving carrier.

FFS whether the indication is in MO or serving cell configuration.

- 	UE determines which MO corresponds to the serving cell frequency from the frequency location of the cell-defining SSB that is contained within the serving cell configuration.

Editor’s Note: FFS Detailed definition of a measurement object based on RAN1/RAN4 input e.g. concerning SS Blocks transmissions. Revisit the procedures describing neighbouring cells on associated frequency and the concept of serving frequency. Consider summarizing the description if becomes lengthy.

-	For inter-RAT E-UTRA measurements a measurement object is a single EUTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and/or per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting of that measurement type. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements, i.e. no (UL, DL) transmissions are scheduled.

An RRC_CONNECTED UE maintains a single measurement object list, a single reporting configuration list, and a single measurement identities list. The measurement object list possibly includes NR intra-frequency object(s), NR inter-frequency object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The serving cell(s) - these are the SpCell and one or more SCells, if configured for a UE supporting CA.

2.	Listed cells - these are cells listed within the measurement object(s).

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the carrier frequency(ies) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells.

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for each serving frequency;

Editor’s Note: FFS How the procedure is used for CGI reporting.

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

Editor’s Note: FFS Whether we can simplify the procedural text and avoid using VarMeasConfig.

#### 5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer if running and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured;

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

#### 5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, cellsToRemoveList, blackCellsToRemoveList, whiteCellsToRemoveList, absThreshSS-BlocksConsolidation, absThreshCSI-RS-Consolidation, nrofSS-BlocksToAverage, nroCSI-RS-ResourcesToAverage;

Editor’s Note: FFS: Exceptions in handling measObject modification for other fields e.g. cells to add/remove from current cell list, measurement configuration for NR-SS and/or CSI-RS.

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId  included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId  from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId  value included in the cellsToAddModList:

5>	if an entry with the matching physCellId  exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId  to the cellsToAddModList;

3>	if the received measObject includes the blackCellsToRemoveList:

4>	for each pci-RangeIndex included in the blackCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the blackCellsToAddModList;

NOTE 1:	For each physCellId included in the blackCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the black list of cells only if all cell indexes containing it are removed.

3>	if the received measObject includes the blackCellsToAddModList:

4>	for each pci-RangeIndex included in the blackCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the blackCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the blackCellsToAddModList;

3>	if the received measObject includes the whiteCellsToRemoveList:

4>	for each pci-RangeIndex included in the whiteCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the whiteCellsToAddModList;

3>	if the received measObject includes the whiteCellsToAddModList:

4>	for each pci-RangeIndex included in the whiteCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the whiteCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the whiteCellsToAddModList;

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

Editor’s Note: FFS How cell indexes are encoded e.g. cell index range.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig;

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

#### 5.5.2.9	Measurement gap configuration

Editor’s Note: FFS How measurement gaps are configured.

Editor’s Note: FFS how to capture the e.g. following agreement: For the independent gap case where UE is able to apply a different gap pattern for LTE/FR1 and FR2:	a	NR RRC configures a measurement gap configuration for FR2.

#### 5.5.2.10	Reference signal measurement timing configuration

Editor’s Note: FFS How SS/PBCH block measurement timing is configured.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results in RRC_CONNECTED the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria and measurement reporting. For cell measurements, the network can configure RSRP, RSRQ or SINR as trigger quantity. Reporting quantities can be the same as trigger quantity or combinations of quantities (i.e. RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR).

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact layer 1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	if at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the associated reportConfig contains rsType set to ssb:

3>	if the measId contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the associated reportConfig contains rsType set to csi-rs:

3>	if the measId contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is not set to reportCGI:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the PCell (or PSCell when the UE is in EN-DC) RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the PCell (or PSCell when the UE is in EN-DC) RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRsIndexes for the associated reportConfig is configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRsIndexes for the associated reportConfig is configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject;

2>	perform the evaluation of reporting criteria as specified in 5.5.4.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity and for each beam measurement quantity that the UE performs measurements according to 5.5.3.1:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and

a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms;

Editor’s Note: FFS Exact value of the sampling rate (i.e. X) for layer 3 filtering.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

#### 5.5.3.3	Derivation of cell measurement results

The network may configure the UE to perform RSRP, RSRQ and SINR measurement results per cell associated to NR carrier frequencies based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block 	or CSI-RS).

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage in the associated measObject is not configured; or

2>	if absThreshSS-BlocksConsolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear average of the power values of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource on the associated frequency to be applicable for deriving RSRP when the concerned CSI-RS resource is included in the csi-rs-ResourceConfigMobility with the corresponding cellId and CSI-RS-ResourceId-RRM within the VarMeasConfig for this measId;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on CSI-RS as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear average of the power values of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nroCSI-RS-ResourcesToAverage;

#### 5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2;

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical;

3>	if the corresponding measObject concerns NR;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider the serving cell on the other frequency as a neighbouring cell;

5>	if useWhiteCellList is set to TRUE:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	if the reportAmount exceeds 1:

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell (or the PSCell when the UE is in EN-DC);

4>	else (i.e. the reportAmount is equal to 1):

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell (or the PSCell when the UE is in EN-DC) and for the strongest cell among the applicable cells;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the primary cell as an NR PCell, NR PSCell (when UE is in EN-DC), or secondary cell that are configured on the frequency indicated in the associated measObjectNR to be the serving cell;

Inequality A1-1 (Entering condition)

Inequality A1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the primary cell as an NR PCell, NR PSCell (when UE is in EN-DC), or secondary cell that is configured on the frequency indicated in the associated measObjectNR to be the serving cell;

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than PCell/PSCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	in EN-DC, use the PSCell for Mp, Ofp and Ocp;

NOTE	The cell(s) that triggers the event is on the frequency indicated in the associated measObjectNR which may be different from the frequency used by the PCell/PSCell (when UE is in EN-DC).

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the PCell/PSCell, not taking into account any offsets.

Ofp is the frequency specific offset of the frequency of the PCell/PSCell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the PCell/PSCell).

Ocp is the cell specific offset of the PCell/PSCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the PCell/PSCell), and is set to zero if not configured for the PCell/PSCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled;

Inequality A4-1 (Entering condition)

Inequality A4-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.6	Event A5 (PCell/PSCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	in EN-DC, use the PSCell for Mp;

NOTE:	The cell(s) that triggers the event is on the frequency indicated in the associated measObjectNR which may be different from the frequency used by the PCell/PSCell.

Inequality A5-1 (Entering condition 1)

Inequality A5-2 (Entering condition 2)

Inequality A5-3 (Leaving condition 1)

Inequality A5-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell/PSCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell that is configured on the frequency indicated in the associated measObjectNR to be the serving cell;

NOTE 1:	The neighbour(s) is on the same frequency as the SCell i.e. both are on the frequency indicated in the associated measObjectNR.

NOTE 2:	In EN-DC, The cell(s) that triggers the event is on the frequency indicated in the associated measObject shall be different from the frequency used by the PSCell.

Inequality A6-1 (Entering condition)

Inequality A6-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the serving frequency), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

Editor’s Note: FFS Details of B1/B2 inter-RAT events and periodical reporting for LTE measurements.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServingCell within measResultServingFreqList to include RSRP, RSRQ and the available SINR for each configured serving cell derived based on the rsType indicated in the associated reportConfig;

1>	in EN-DC, set the measResultServingCell within measResultServingFreqList to include for each NR SCell that is configured, if any, the servFreqId;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes:

2>	for each configured serving cell, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2> for each serving frequency for which measObjectId is referenced in the measIdList, other than the frequency corresponding with the measId that triggered the measurement reporting:

3>	set the measResultBestNeighCell within measResultServingFreqListmeasResultServingFreqList to include the physCellId and the available measurement quantities and rsType indicated in reportConfig of the best non-serving cell on the concerned serving frequency;

3>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes:

4>	for each best non-serving cell on the concerned serving frequency, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the reportType is set to eventTriggered:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns NR:

6>	if rsType in the associated reportConfig is set to ssb:

7> set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8>	if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

6>	else if rsType in the associated reportConfig is set to csi-rs:

7> set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8> if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1> if the UE is in EN-DC:

2> if SRB3 is configured:

3> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2> else:

3> submit the MeasurementReport message via the EUTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10];

1> else:

2>	 submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity;

1>	if reportType is set to periodical:

2> if a single reporting quantity is set to TRUE in reportQuantityRsIndexes;

3> consider the configured single quantity as the sorting quantity;

2> else:

3> if rsrp is set to TRUE;

4> consider RSRP as the sorting quantity;

3> else:

4> consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRsIndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if  includeBeamMeasurements is configured, include the SS/PBCH based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurements is configured, include the CSI-RS based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each CSI-RS index;

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

#### Editor’s Note: Targeted for completion in June 2018

#### 5.6.1.2	Initiation

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if includes requestedFreqBandList:

2>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in requestedFreqBandList, and prioritized in the order of requestedFreqBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates;

2>	include all band combinations in the candidate list into supportedBandCombination;

1>	else:

2> include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE;

#### 5.6.1.5	Compilation of baseband processing combinations supported by the UE

The UE shall:

1>	for each band combination included in supportedBandCombination:

2>	include the baseband processing combination supported for the band combination into supportedBasebandProcessingCombination, unless it is already included;

2>	if there are the fallback baseband processing combinations of this baseband processing combination as specified in TS 38.306 [xx] for which supported baseband capabilities are different from this baseband processing combination:

3>	include the fallback baseband processing combinations into supportedBasebandProcessingCombination;

## 5.7	Other

### 5.7.1	DL information transfer

Editor’s Note: Targeted for completion in June 2018.

### 5.7.2	UL information transfer

Editor’s Note: Targeted for completion in June 2018.

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform EUTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure, SCG configuration failure for RRC message on SRB3, SCG integrity check failure and exceeding the maximum uplink transmission timing difference.

Editor’s Note: SCG failure considers the case of exceeding the maximum uplink transmission timing difference if RAN1 decides that EN-DC supports the synchronised operation case. FFS how to capture

Editor’s Note: FFS whether to include the handling of SCell Failure in CA duplication case in SCGfailureinformation procedure and whether to rename SCGfailureinformation.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.9.3;

1>	upon stopping uplink transmission towards the SCG’s SpCell due to exceeding the maximum uplink transmission timing difference, in accordance with subclause x.x.x of TS 38.133 [14].

Editor’s Note: FFS on RAN1 decision on powerControlMode;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.9.2;

1>	upon integrity check failure indication from SCG lower layers, in accordance with subclause 5.3.5.9.1;

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG-MAC;

1>	stop T304, if running;

1>	if the UE is operating in EN-DC:

2>	initiate transmission of the SCGFailureInformation message as specified in TS 36.331 [10, 5.6.13a];

Editor’s Note: 
The section for transmission of SCGFailureInformation in NR RRC entity for SA is FFS_Standalone.

#### 5.7.3.3	Failure type determination

Editor’s Note: FFS / TODO: Either use this section also for NR-DC or change section title (add “for EN-DC”)

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2>	set the failureType as the trigger for detecting SCG radio link failure;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as scg-ChangeFailure;

Editor’s Note: FFS whether to change scg-ChangeFailure to synchronousReconfigurationFailure-SCG

1>	else if the UE initiates transmission of the SCGFailureInformation message due to exceeding maximum uplink transmission timing difference:

2>	set the failureType as maxUL-TimingDiff;

1>	else, if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1> else, if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

Editor’s Note: FFS: whether to include rrc-TransactionIdentifier information.

#### 5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failure as follows:

1>	set the measResultServFreqList to include for each SCG cell that is configured by the SN to be measured, if any, within measResultServingCell the quantities of the concerned SCell, if available, according to performance requirements in [FFS_Ref];

1>	for each SCG serving frequency included in measResultServFreqList include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements by the SN for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

2>	for each neighbour cell included:

3>	include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

### 6.1.1	Introduction

The contents of each RRC message is specified in sub-clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in sub-clause 6.3.

### 6.1.2	Need codes and conditions for optional downlink fields

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. All comment text tags are available for use in the downlink direction only. The meaning of each tag is specified in table 6.1-1.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is not present. The conditions clarify what the UE may expect regarding the setting of the message by the network. Violation of conditions is regarded as invalid network behaviour, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

For guidelines on the use of need codes and conditions, see Annex A.6 and A.7.

Table 6.1-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONSSTART

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::= SEQUENCE {

message											BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::= CHOICE {

mib												MIB,

messageClassExtension	SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::= SEQUENCE {

message											DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

rrcReconfiguration							RRCReconfiguration,

spare15 NULL, spare14 NULL, spare13 NULL,

spare12 NULL, spare11 NULL, spare10 NULL,

spare9 NULL, spare8 NULL, spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::= SEQUENCE {

message											UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

measurementReport							MeasurementReport,

rrcReconfigurationComplete					RRCReconfigurationComplete,

spare14 NULL, spare13 NULL, spare12 NULL,

spare11 NULL, spare10 NULL, spare9 NULL,

spare8 NULL, spare7 NULL, spare6 NULL,

spare5 NULL, spare4 NULL, spare3 NULL,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

#### –	MIB

The MIB includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: Network to UE

MIB

-- ASN1START

-- TAG-MIB-START

MIB ::= SEQUENCE {

-- The 6 most significant bit (MSB) of the 10 bit System Frame Number. The 4 LSB of the SFN are conveyed in the PBCH transport block

-- as well but outside the MIB.

systemFrameNumber					BIT STRING (SIZE (6)),

-- Subcarrier spacing for SIB1, Msg.2/4 for initial access and broadcast SI-messages.

-- If the UE acquires this MIB on a carrier frequency <6GHz, the values 15 and 30 kHz are applicable.

-- If the UE acquires this MIB on a carrier frequency >6GHz, the values 60 and 120 kHz are applicable.

subCarrierSpacingCommon				ENUMERATED {scs15or60, scs30or120},

-- The frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See 38.211, section 7.4.3.1)

-- Note: For frequencies <6 GHz a fith, this field may comprise only the 4 least significant bits of the ssb-SubcarrierOffset.

ssb-SubcarrierOffset				INTEGER (0..15),

-- Position of (first) DL DM-RS. Corresponds to L1 parameter 'DL-DMRS-typeA-pos' (see 38.211, section 7.4.1.1.1)

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

-- Determines a bandwidth for PDCCH/SIB, a common ControlResourceSet (CORESET) a common search space and necessary PDCCH parameters.

-- The codepoint "FFS_RAN1" indicates that this cell does not provide SIB1 and that there is hence no common CORESET.

-- Corresponds to L1 parameter 'RMSI-PDCCH-Config' (see FFS_Specification, section FFS_Section)

pdcch-ConfigSIB1					INTEGER (0..255),

-- Indicates that UE shall not camp on this cell

cellBarred							ENUMERATED {barred, notBarred},

-- Controls cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE,

-- as specified in TS 38.304.

intraFreqReselection				ENUMERATED {allowed, notAllowed},

-- FFS_CHECK with RAN1 whether 1 spare bit in MIB is the final value

spare								BIT STRING (SIZE (2))

}

-- TAG-MIB-STOP

-- ASN1STOP

MIB field descriptions

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=				SEQUENCE {

criticalExtensions					CHOICE {

measurementReport							MeasurementReport-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

MeasurementReport-IEs ::=		SEQUENCE {

measResults						MeasResults,

-- FFS

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including any associated dedicated NAS information and security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

-- In EN-DC this field may only be present if the RRCReconfiguration

-- is transmitted over SRB3.

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

-- Configuration of secondary cell group (EN-DC):

secondaryCellGroup						CellGroupConfig														OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration field descriptions

FFS FFS.

| Conditional presence | Explanation |
| --- | --- |
| FFS | FFS |

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to -Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::= 			SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

rrcReconfigurationComplete				RRCReconfigurationComplete-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::= SEQUENCE {

-- FFS

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete field descriptions

FFS FFS

#### –	SIB1

Editor’s Note: Discuss whether to keep SIB1 for the December version. FFS

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=		SEQUENCE {

-- FFS / TODO: Add other parameters.

-- Frequency offset for the SSB of -5kHz (M=-1) or +5kHz (M=1). When the field is absent, the UE applies no offset (M=0).

-- The offset is only applicable for the frequency range 0-2.65GHz. Corresponds to parameter 'M' (see 38.101, section FFS_Section)

frequencyOffsetSSB				FrequencyOffsetSSB																OPTIONAL,	-- Need R

-- Time domain positions of the transmitted SS-blocks in an SS-Burst-Set (see 38.213, section 4.1)

ssb-PositionsInBurst				SEQUENCE {

-- Indicates the presence of the up to 8 SSBs in one group

inOneGroup							BIT STRING (SIZE (8)),

-- For above 6 GHz: indicates which groups of SSBs is present

groupPresence						BIT STRING (SIZE (8))										OPTIONAL -- Cond above6GHzOnly

},

-- The SSB periodicity in msec for the rate matching purpose (see 38.211, section [7.4.3.1])

ssb-PeriodicityServingCell			ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2},

-- TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power.

-- (see 38.213, section 7.4)

ss-PBCH-BlockPower					INTEGER (-60..50),

uplinkConfigCommon						UplinkConfigCommon											OPTIONAL,

-- FFS: How to indicate the FrequencyInfoUL for the SUL

supplementaryUplink					SEQUENCE {

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL

-- FFS: Add additional (selection) criteria determining when/whether the UE shall use the SUL frequency

}																									OPTIONAL, -- Cond SUL

tdd-UL-DL-Configuration				TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

tdd-UL-DL-configurationCommon2		TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

pdcch-ConfigCommon					PDCCH-ConfigCommon												OPTIONAL,

pucch-ConfigCommon					PUCCH-ConfigCommon												OPTIONAL,

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-SIB1-STOP

-- ASN1STOP

## 6.3	RRC information elements

### 6.3.0	Parameterized types

### –	SetupRelease Information Element

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUP-RELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release			NULL,

setup			ElementTypeParam

}

-- TAG-SETUP-RELEASE-STOP

-- ASN1STOP

### 6.3.1	System information blocks

### 6.3.2	Radio resource control information elements

#### –	AdditionalSpectrumEmission

The IE AdditionalSpectrumEmission is used to indicate emission requirements to be fulfilled by the UE (see 38.101, section FFS_Section)

AdditionalSpectrumEmission information element

-- ASN1START

-- TAG-ADDITIONALSPECTRUMEMISSION-START

AdditionalSpectrumEmission ::=				INTEGER (0..7)

-- TAG-ADDITIONALSPECTRUMEMISSION-STOP

#### -- ASN1STOP

#### –	Alpha

The IE Alpha defines possible values for uplink power control.

-- ASN1START

-- TAG-ALPHA-START

Alpha ::=						ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1}

-- TAG-ALPHA-STOP

-- ASN1STOP

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101 [15].

-- ASN1START

-- TAG-ARFCN-VALUE-NR-START

ARFCN-ValueNR ::=				CHOICE {

-- Absolute carrier frequency in number of multiples of 5kHz. Applicable for the frequency range from 0 to 3GHz.

-- Corresponds to parameter 'N_REF' (see 38.101, section FFS_Section)

lowCarrierFrequency				INTEGER (0..599999),

-- Absolute carrier frequency in number of multiples of 15kHz. Applicable for the frequency range from 3GHz to 24GHz

-- Corresponds to parameter 'N_REF' (see 38.101, section FFS_Section)

midCarrierFrequency				INTEGER (600000..1999999),

-- Absolute carrier frequency in number of multiples of 60kHz. Applicable for the frequency range from 24GHz to 100GHz

-- Corresponds to parameter 'N_REF' (see 38.101, section FFS_Section)

highCarrierFrequency			INTEGER (2000000..3266667)

}

-- TAG-ARFCN-VALUE-NR-STOP

-- ASN1STOP

#### –	BandwidthPart-Config

The BandwidthPart-Config IE is used to configure a bandwidth part as defined in 38.211, section 4.2.2.

For each serving cell the network configures at least an initial bandwidth part comprising of at least a downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The bandwidth configuration is split into uplink and downlink parameters and into common and dedicated parameters. Common parameters (in UplinkBWP-Common and DownlinkBWPCommon) are ”cell specific” and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BandwidthPart-Config information element

-- ASN1START

-- TAG-BANDWIDTH-PART-START

-- Generic parameters used in Uplink- and Downlink bandwidth parts

BWP ::= 					SEQUENCE {

-- An identifier for this bandwidth part.

-- Corresponds to L1 parameter 'UL-BWP-index'. (see 38.211, 38.213, section 12)

bwp-Id							BWP-Id,

-- Frequency domain location and bandwidth of this bandwidth part defined commonly in a table (FFS_Section). The location is given as

-- distance (in number of PRBs) in relation to the lowest usable subcarrier defined by the SCS-SpecificVirtualCarrier

-- with the same subcarrier spacing as this BWP.

-- Corresponds to L1 parameter 'DL-BWP-loc'. (see 38.211, section FFS_Section).

-- In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same location (see 38.211, section REF)

-- FFS_Value: RAN1 seems to discuss the final range.

locationAndBandwidth			INTEGER (1..65536)							,

-- Subcarrier spacing to be used in this BWP. It is applied to at least PDCCH, PDSCH and corresponding DMRS.

-- The values provided here are converted into a subcarrier spacing as indicated in 38.211, Table 4.2-1.

subcarrierSpacing				ENUMERATED {n0, n1, n2, n3, n4, n5}											,

-- Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix.

-- Normal CP is supported for all numerologies and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing.

-- (see 38.211, section 4.2.2)

cyclicPrefix					ENUMERATED { extended }														OPTIONAL

}

UplinkBWP ::= 			SEQUENCE {

-- An identifier for this bandwidth part. BWP ID=0 is used for the initial BWP and may hence not be used here.

-- Corresponds to L1 parameter 'UL-BWP-index'. (see 38.211, 38.213, section 12)

bwp-Id								BWP-Id,

bwp-Common							UplinkBWP-Common														OPTIONAL,	-- Need M

bwp-Dedicated						UplinkBWP-Dedicated														OPTIONAL,	-- Need M

...

}

UplinkBWP-Common ::=				SEQUENCE {

genericParameters					BWP,

-- FFS: Consider adding conditions for the following fields:

rach-ConfigCommon					SetupRelease { RACH-ConfigCommon }										OPTIONAL, 	-- Need M

pusch-ConfigCommon					SetupRelease { PUSCH-ConfigCommon }										OPTIONAL, 	-- Need M

pucch-ConfigCommon					SetupRelease { PUCCH-ConfigCommon }										OPTIONAL, 	-- Need M

...

}

UplinkBWP-Dedicated ::= 	SEQUENCE {

-- PUCCH configuration for one BWP of the regular UL or SUL of a serving cell. If the UE is configured with SUL, the network

-- configures PUCCH only on one of the uplinks (UL or SUL).

pucch-Config						SetupRelease { PUCCH-Config }											OPTIONAL, 	-- Need M

-- PUSCH configuration for one BWP of the regular UL or SUL of a serving cell. If the UE is configured with SUL and

-- if it has a PUSCH-Config for both UL and SUL, a carrier indicator field in DCI indicates for which of the two to use an UL grant.

-- See also L1 parameter 'dynamicPUSCHSUL' (see 38.213, section FFS_Section)

pusch-Config						SetupRelease { PUSCH-Config }											OPTIONAL, 	-- Need M

-- A Configured-Grant of typ1 or type2. It may be configured for Ul or SUL but not for both at a time.

configuredGrantConfig				SetupRelease { ConfiguredGrantConfig }									OPTIONAL, 	-- Need M

srs-Config							SetupRelease { SRS-Config }												OPTIONAL, 	-- Need M

...

}

DownlinkBWP ::= 					SEQUENCE {

-- An identifier for this bandwidth part. BWP ID=0 is used for the initial BWP and may hence not be used here.

-- Corresponds to L1 parameter 'DL-BWP-index'. (see 38.211, 38.213, section 12)

bwp-Id								BWP-Id,

bwp-Common							DownlinkBWP-Common												OPTIONAL,	-- Need M

bwp-Dedicated						DownlinkBWP-Dedicated											OPTIONAL,	-- Need M

...

}

DownlinkBWP-Common ::=				SEQUENCE {

genericParameters					BWP,

pdcch-ConfigCommon					SetupRelease { PDCCH-ConfigCommon }												OPTIONAL,	-- Need M

...

}

DownlinkBWP-Dedicated ::= 			SEQUENCE {

pdcch-Config						SetupRelease { PDCCH-Config }											OPTIONAL,	-- Need M

pdsch-Config						SetupRelease { PDSCH-Config }											OPTIONAL,	-- Need M

sps-Config							SetupRelease { SPS-Config }												OPTIONAL, 	-- Need M

beamFailureDetectionConfig			SetupRelease { BeamFailureDetectionConfig }								OPTIONAL,	-- Need M

...

}

BWP-Id ::=					INTEGER (0..maxNrofBandwidthParts-1)

-- TAG-BANDWIDTH-PART-STOP

-- ASN1STOP

#### –	BeamFailureDetectionConfig

The BeamFailureDetectionConfig is used to configure the UE for monitoring detection of beam failure. See also 38.321, section 5.1.1.

BeamFailureDetectionConfig information element

-- ASN1START

-- TAG-BEAM-FAILURE-DETECTION-CONFIG-START

BeamFailureDetectionConfig ::=			SEQUENCE {

failureDetectionResources				SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index							NZP-CSI-RS-ResourceId

}																											OPTIONAL, 	--	Need M

beamFailureInstanceMaxCount				FFS_Value															OPTIONAL	--	Need M

}

-- TAG-BEAM-FAILURE-DETECTION-CONFIG-STOP

-- ASN1STOP

#### –	BeamFailureRecoveryConfig

The BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also 38.321, section 5.1.1.

Editor's Note: It is FFS whether this is configured per BWP, per cell, and FFS whether BFR needs to be performed on SCell

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-START

BeamFailureRecoveryConfig ::= 		SEQUENCE {

rootSequenceIndex-BFR				INTEGER (0..137)															OPTIONAL,	--	Need M

rach-ConfigCommon-BFR				RACH-ConfigCommonGeneric													OPTIONAL,	--	Need M

beamFailurerRecoveryTimer			FFS_Value																	OPTIONAL,	--	Need M

beamFailureCandidateBeamThreshold	RSRP-Range																	OPTIONAL,	--	Need M

candidateBeamRSList					SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR		OPTIONAL,	--	Need M

recoveryControlResourceSetId		ControlResourceSetId														OPTIONAL		--	Need M

}

-- NOTE: If the candidateBeamRSList includes both CSI-RS resource indexes and SSB indexes, AND only SSB indexes are associated with

-- PRACH resources then UE identifies PRACH resources for CSI-RS resource(s) in the candidateBeamRSList via spatial QCL indication

-- between SSBs and CSI-RS resources, if UE-identified new beam(s) is associated with CSI-RS resource(s).

PRACH-ResourceDedicatedBFR ::= 		SEQUENCE {

candidateBeam-RS					CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index							NZP-CSI-RS-ResourceId

},

ra-PreambleIndex					FFS_Value																	OPTIONAL,

prach-FreqOffset					FFS_Value																	OPTIONAL,

rach-ResourceMask					FFS_Value																	OPTIONAL

}

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-STOP

-- ASN1STOP

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELL-GROUP-CONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig	::= 						SEQUENCE {

cellGroupId									CellGroupId,

-- Logical Channel configuration and association with radio bearers:

rlc-BearerToAddModList 						SEQUENCE (SIZE(1..maxLCH)) OF RLC-Bearer-Config						OPTIONAL,   -- Need N

rlc-BearerToReleaseList						SEQUENCE (SIZE(1..maxLCH)) OF LogicalChannelIdentity			OPTIONAL,   -- Need N

-- Parameters applicable for the entire cell group:

mac-CellGroupConfig							MAC-CellGroupConfig												OPTIONAL,	-- Need M

physicalCellGroupConfig						PhysicalCellGroupConfig											OPTIONAL,	-- Need M

-- Serving Cell specific parameters (SpCell and SCells)

spCellConfig									SpCellConfig														OPTIONAL, 	-- Need M

sCellToAddModList								SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig						OPTIONAL,	-- Need N

-- List of seconary serving cells to be released (not applicable for SpCells)

sCellToReleaseList								SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex										OPTIONAL,	-- Need N

...

}

-- The ID of a cell group. 0 identifies the master cell group. Other values identify secondary cell groups.

-- In this version of the specification only values 0 and 1 are supported.

-- FFS: Should the constant anyway account for larger values? Extending it in the future will otherwise become very difficult.

CellGroupId ::=								INTEGER (0.. maxSecondaryCellGroups)

RLC-Bearer-Config ::=								SEQUENCE {

-- ID used commonly for the MAC logical channel and for the RLC bearer.

logicalChannelIdentity						LogicalChannelIdentity,

-- Associates the RLC Bearer with an SRB or a DRB. The UE shall deliver DL RLC SDUs received via the RLC entity of this

-- RLC bearer to the PDCP entity of the servedRadioBearer. Furthermore, the UE shall advertise and deliver uplink PDCP PDUs of the

-- uplink PDCP entity of the servedRadioBearer to the uplink RLC entity of this RLC bearer unless the uplink scheduling

-- restrictions ('moreThanOneRLC' in PDCP-Config and the restrictions in LogicalChannelConfig) forbid it to do so.

servedRadioBearer							CHOICE {

srb-Identity                           SRB-Identity,

drb-Identity                           DRB-Identity

}																											OPTIONAL,	-- Cond LCH-SetupOnly

reestablishRLC								ENUMERATED {true}												OPTIONAL, 	-- Need N

rlc-Config									RLC-Config														OPTIONAL,	-- Cond LCH-Setup

mac-LogicalChannelConfig					LogicalChannelConfig											OPTIONAL	-- Cond LCH-Setup

}

LogicalChannelIdentity ::= 					INTEGER (1..maxLC-ID)

-- Cell-Group specific L1 parameters

PhysicalCellGroupConfig ::=					SEQUENCE {

-- Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH

-- reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule.

-- Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

-- Absence indicates that spatial bundling is disabled.

harq-ACK-SpatialBundlingPUCCH				ENUMERATED {true}												OPTIONAL	,	-- Need R

-- Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH

-- reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule.

-- Corresponds to L1 parameter 'HARQ-ACK-spatial-bundling' (see 38.213, section FFS_Section)

-- Absence indicates that spatial bundling is disabled.

harq-ACK-SpatialBundlingPUSCH				ENUMERATED {true}												OPTIONAL,	-- Need R

p-NR									P-Max					OPTIONAL,

nonCriticalExtension					SEQUENCE{}															OPTIONAL

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=						SEQUENCE {

-- Serving cell ID of a PSCell (the PCell of the Master Cell Group uses ID = 0)

servCellIndex						ServCellIndex															OPTIONAL,	-- Cond SCG

-- Parameters for the synchronous reconfiguration to the target SpCell:

reconfigurationWithSync 			SEQUENCE {

spCellConfigCommon					ServingCellConfigCommon,

newUE-Identity						RNTI-Value,

t304								ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated				CHOICE {

uplink								RACH-ConfigDedicated,

supllementaryUplink					RACH-ConfigDedicated

}																										OPTIONAL	-- Need N

}																											OPTIONAL,	-- Cond ReconfWithSync

rlf-TimersAndConstants				RLF-TimersAndConstants													OPTIONAL,	-- Need M

spCellConfigDedicated				ServingCellConfig												OPTIONAL	-- Need M

}

SCellConfig ::=						SEQUENCE {

sCellIndex							SCellIndex,

sCellConfigCommon					ServingCellConfigCommon													OPTIONAL,	-- Cond SCellAdd

sCellConfigDedicated				ServingCellConfig												OPTIONAL	-- Cond SCellAddMod

}

-- TAG-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

logicalChannelIdentity The logical channel identity for both UL and DL.

| Conditional Presence | Explanation |
| --- | --- |
| LCH-SetupOnly | The field is mandatory present if the corresponding LCH is being set up; otherwise it is not present. |
| LCH-Setup | The field is mandatory present if the corresponding LCH is being set up for DRB; otherwise it is optionally present, need M. |
| ReconfWithSync | The field is mandatory present in case of SpCell change and security key change; otherwise it is optionally present, need M. |
| SCellAdd | The field is optionally present, need M, upon SCell addition; otherwise it is not present |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |

#### –	ControlResourceSetId

The ControlResourceSetId IE concerns a short identity, used to identify a control resource set within a serving cell. The ControlResourceSetId = 0 identifies the ControlResoruceSet configured via PBCH (MIB) and in ServingCellConfigCommon. The ID space is used across the BWPs of a Serving Cell. The number of CORESETs per BWP is limited to 3 (including the initial CORESET).

ControlResourceSetId information element

-- ASN1START

-- TAG-CONTROL-RESOURCE-SET-ID-START

ControlResourceSetId ::=				INTEGER (0..maxNrofControlResourceSets-1)

-- TAG-CONTROL-RESOURCE-SET-ID-STOP

-- ASN1STOP

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information elements

-- ASN1START

CrossCarrierSchedulingConfig ::=		SEQUENCE {

-- FFS: Indicate this separately for UL and DL (as done in LTE for LAA)

schedulingCellInfo						CHOICE {

own										SEQUENCE {					-- No cross carrier scheduling

cif-Presence							BOOLEAN

},

other									SEQUENCE {					-- Cross carrier scheduling

schedulingCellId						ServCellIndex,

-- FFS: pdsch-start is probably not needed since RAN1 agreed that the scheduling DCI can provide (an index into a

-- UE-specific table giving) the OFDM symbols used for the PDSCH (or PUSCH)  transmission. But what does “can provide” mean?

pdsch-Start								INTEGER (1..4),

-- FFS: cif-InSchedulingCell is probably still needed since RAN1 agreed that “If CIF is present in DCI, the bitwidth is fixed at 3 bit”.

cif-InSchedulingCell					INTEGER (1..7)

}

}

}

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

cif-Presence The field is used to indicate whether carrier indicator field is present (value TRUE) or not (value FALSE) in PDCCH/EPDCCH DCI formats, see TS 38.213 [REF, SECTION].

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [REF, SECTION]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

pdsch-Start The starting OFDM symbol of PDSCH for the concerned SCell, see TS [REF]. Values 1, 2, 3 are applicable when dl-Bandwidth for the concerned SCell is greater than 10 resource blocks, values 2, 3, 4 are applicable when dl-Bandwidth for the concerned SCell is less than or equal to 10 resource blocks, see TS [REF].

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell.

#### –	CSI-MeasConfig

The CSI-MeasConfig IE is used to configure the UE for measuring CSI-RS (reference signals) and for reporting those measurements on L1 (PUCCH, PUSCH) as channel state information. See also 38.214, section 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

CSI-MeasConfig ::=					SEQUENCE {

csi-ResourceConfigs 				SEQUENCE (SIZE (1..maxNrofCSI-ResrouceConfigurations)) OF CSI-ResourceConfig 			OPTIONAL,

csi-ReportConfigs					SEQUENCE (SIZE (1..maxNrofCSI-Reports)) OF CSI-ReportConfig								OPTIONAL,

csi-MeasIdToAddModList				SEQUENCE (SIZE (1..maxNrofCSI-MeasId)) OF CSI-MeasIdToAddMod								OPTIONAL,

-- Size of CSI request field in DCI (bits). Corresponds to L1 parameter 'ReportTriggerSize' (see 38.214, section 5.2)

reportTriggerSize				INTEGER (0..6)																				OPTIONAL,

-- Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations

-- and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement.

-- New description (still not clear and not fitting to other fields): Each trigger state is associated with one or multiple ReportConfigs where each ReportConfig is linked to one or two or three P/SP/AP CSI-RS resource setting(s). If a resource setting linked to a ReportConfig has multiple aperiodic resource sets and only a subset of the aperiodic resource sets is associated with the trigger state, a bitmap (with the bitwidth Nbit =number of resource sets in a resource setting.  Number of one(s) in the bitmap None = 1 (FFS on 2) for CSI acquisition) is RRC configured per trigger state per resource setting to select CSI-IM/NZP CSI-RS resource set(s) from the resource setting.

-- FFS_CHECK: Is this the appropriate place for the IE or should it be inside the resource configuration or in a set?

-- FFS_FIXME: This is just one report trigger. But of course it should be a list. Maximum number of configured triggers depends

-- on the trigger size

-- FFS: How to address the MAC-CE configuration

reportTrigger					SEQUENCE {

aperiodic						SEQUENCE {

-- The CSI-ReportConfig (their IDs) associated with this reportTrigger

associatedReportConfigs			SEQUENCE (SIZE (1..maxNrofReportConfigIdsPerTrigger)) OF CSI-ReportConfigId,

-- bitmap with the bitwidth Nbit =number of resource sets (max number Nbit = 64) in a linked resource setting per report trigger tate.

-- Number of one(s) in the bitmap None = 1 for CSI acquisition (FFS 1<= None <= 64 for beam management).

-- FFS: To enforce the number of linked resources, the linking information should instead be in the report that uses the resource

-- Corresponds to L1 parameter 'ResourceSetBitmap' (see 38.214, section FFS_Section)

-- FFS_FIXME: The following list assumes that all NZP- and IM resource sets use a common ID space. But that is not ensured

-- due to having separate lists of sets.

associatedResourceSets			SEQUENCE (SIZE (1..64)) OF NZP-CSI-ResourceSetId												OPTIONAL,

-- For a trigger state within aperiodicReportTrigger that triggers a ap-CSI-RS resource set, contains a list of

-- references to TCI-State elements configured in PDSCH-Config for providing the QCL source and QCL type for each ap-CSI-RS

-- resource within the triggered set of ap-CSI-RS resources. The length of the list is equal to the number of

-- aperiodic CSI-RS resources in the set (CSI-RS-ResourceSet). For a target aperiodic CSI-RS assoicated with each

-- triggering state, contains a reference to one TCI-RS-Set in TCI-States for providing the QCL source and QCL type.

-- Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section 5.2.1.5.1)

qcl-Info-aPeriodicReportingTrigger			SEQUENCE (SIZE(1..ffsValue)) OF TCI-StateId									OPTIONAL

},

semiPersistentOnPUSCH				SEQUENCE {

associatedReportConfig				CSI-ReportConfigId

}

}

}

-- TAG-CSI-MEAS-CONFIG-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig comprises of one or more NZP-CSI-RS-ResourceSets, CSI-IM-ResourceSet and/or CSI-SSB-Resource

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

-- One CSI resource configuration comprising of one or more resource sets

CSI-ResourceConfig ::= 				SEQUENCE {

csi-ResourceConfigId				CSI-ResourceConfigId,

-- Contains up to maxNrofCSI-ResourceSets resource CSI-ReosurceSets if ResourceConfigType is 'aperiodic' and 1 otherwise.

-- Corresponds to L1 parameter 'ResourceSetConfigList' (see 38.214, section 5.2.1.3.1)

csi-RS-ResourceSets 	 			CHOICE {

nzp-CSI-RS-ResourceSets				SEQUENCE (SIZE (1..maxNrofCSI-ResourceSets)) OF NZP-CSI-RS-ResourceSet,

csi-IM-ResourceSets					SEQUENCE (SIZE (1..maxNrofCSI-ResourceSets)) OF CSI-IM-ResourceSet

},

-- List of SSB resources used for beam measurement and reporting in a resource set

-- Corresponds to L1 parameter 'resource-config-SS-list' (see 38,214, section FFS_Section)

ssb-Resources						SEQUENCE (SIZE (1..maxNrofSSB-Resources)) OF	CSI-SSB-Resource		OPTIONAL, --Cond OnlyWithNZPResourceSets

-- The DL BWP which the CSI-RS assocaited with this CSI-ResourceConfig are located in.

-- Corresponds to L1 parameter 'BWP-Info' (see 38.214, section FFS_Section)

bwp-Id								BWP-Id,

-- Time domain behavior of resource configuration. Corresponds to L1 parameter 'ResourceConfigType' (see 38.214, section 5.2.2.3.5)

resourceType							CHOICE {

aperiodic								NULL,

semiPersistent 						NULL,

periodic								SEQUENCE {

-- For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and

-- QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS.

-- Corresponds to L1 parameter 'QCL-Info-PeriodicCSI-RS' (see 38.214, section FFS_Section)

qcl-InfoPeriodicCSI-RS					TCI-StateId																	OPTIONAL

}

},

-- Indication of which Serving Cell the configured CSI-RS is located in.

-- FFS_CHECK: RAN1 intended to enable cross-carrier scheduling of aperiodoic CSI-RS. This field would indicate on which ServingCell

-- the UE finds these resources. Discuss whether and how this works considering that currently a CSI-MeasConfig exists per ServingCell

-- Corresponds to L1 parameter 'CC_Info' (see 38.214, section 5.2.2.3.1)

crossCarrierInfo						FFS_Value																				OPTIONAL,

...

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::= INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::= 					SEQUENCE {

nzp-CSI-ResourceSetId					NZP-CSI-ResourceSetId,

-- NZP-CSI-RS-Resources assocaited with this NZP-CSI-RS resource set.

-- Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2)

-- FFS: Better make the csi-rs-Resources a common pool on CSI-MeasConfig level?

nzp-CSI-RS-Resources						SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-Resource,

-- Indicates whether repetition is on/off.

If set to set to 'OFF', the UE may not assume that the

-- NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter

-- and with same NrofPorts in every symbol.

-- Corresponds to L1 parameter 'CSI-RS-ResourceRep' (see 38.214, sections 5.2.2.3.1 and 5.1.6.1.2)

repetition									ENUMERATED { on, off },

-- Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the

-- CSI-RS resource set is transmitted. When the field is absent the UE applies the value 0.

-- Corresponds to L1 parameter 'Aperiodic-NZP-CSI-RS-TriggeringOffset' (see 38,214, section FFS_Section)

-- FFS_CHECK: Is this field at the correct place? Or should it be in the trigger configuration instead?

aperiodicTriggeringOffset			 		FFS_Value																		OPTIONAL	-- Need S

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

#### –	NZP-CSI-ResourceSetId

The IE NZP-CSI-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RESOURCESETID-START

NZP-CSI-ResourceSetId ::= INTEGER (0..maxNrofCSI-ResourceSets-1)

-- TAG-NZP-CSI-RESOURCESETID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS-Resource, which the UE may be configured to measure on (see 38.214, section 5.2.2.3.1).

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=					SEQUENCE {

-- Frequency domain allocation within a physical resource block in accordance with 38.211, section 7.4.1.5.3 including table 7.4.1.5.2-1.

-- The number of bits that may be set to one depend on the chosen row in that table.

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12)),

row3								BIT STRING (SIZE (6)),

row4								BIT STRING (SIZE (3)),

row5								BIT STRING (SIZE (6)),

row7								BIT STRING (SIZE (6)),

row8								BIT STRING (SIZE (6)),

row9								BIT STRING (SIZE (6)),

row10								BIT STRING (SIZE (6)),

row11								BIT STRING (SIZE (6)),

row12								BIT STRING (SIZE (6)),

row13								BIT STRING (SIZE (6)),

row14								BIT STRING (SIZE (6)),

row15								BIT STRING (SIZE (6)),

row16								BIT STRING (SIZE (6)),

row17								BIT STRING (SIZE (6)),

row18								BIT STRING (SIZE (6)),

row19								BIT STRING (SIZE (6))

},

-- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS.

-- Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

firstOFDMSymbolInTimeDomain			INTEGER (0..13)

,

-- CDM type (see 38.214, section 5.2.2.3.1)

cdm-Type								ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

-- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density'  (see 38.211, section 7.4.1.5.3)

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

density									CHOICE {

dot5									ENUMERATED {evenPRBs, oddPRBs},

one										NULL,

three									NULL,

spare									NULL

},

-- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1)

freqBand								CSI-FrequencyOccupation

,

-- Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, sections 5.2.2.3.1 and 4.1)

powerControlOffset						INTEGER(-8..15),

-- Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section FFS_Section)

powerControlOffsetSS					ENUMERATED{db-3, db0, db3, db6}													OPTIONAL,

-- Scrambling ID (see 38.214, section 5.2.2.3.1)

scramblingID							ScramblingId,

-- Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on.

-- The corresponding offset is also given in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1)

periodicityAndOffset					CHOICE {

sl4										INTEGER (0..3),

sl5										INTEGER (0..4),

sl8										INTEGER (0..7),

sl10									INTEGER (0..9),

sl16									INTEGER (0..15),

sl20									INTEGER (0..19),

sl32									INTEGER (0..31),

sl40									INTEGER (0..39),

sl64									INTEGER (0..63),

sl80									INTEGER (0..79),

sl160									INTEGER (0..159),

sl320									INTEGER (0..319),

sl640									INTEGER (0..639)

},

-- Indicates whether or not the antenna ports of NZP CSI-RS resources in the CSI-RS resource set is same

-- Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1)

trs-Info								ENUMERATED {true}																OPTIONAL

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=				SEQUENCE {

-- PRB where this CSI resource starts in relation to PRB 0 of the associated BWP.

-- Only multiples of 4 are allowed (0, 4, ...)

startingRB								INTEGER (0..maxNrofPhysicalResourceBlocks-1),

-- Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable

-- number is the minimum of 24 and the width of the associated BWP.

nrofRBs									INTEGER (24..maxNrofPhysicalResourceBlocks)

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::= 					INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=					SEQUENCE {

-- FFS: Where is the csi-im-ResourceSetId used?

csi-IM-ResourceSetId					CSI-IM-ResourceSetId,

-- CSI-IM-Resources associated with this CSI-IM-ResourceSet

-- Corresponds to L1 parameter 'CSI-IM-ResourceConfigList' (see 38.214, section 5.2)

csi-IM-Resources						SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-Resource

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=				INTEGER (0..maxNrof CSI-IM-ResourceSets)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::= 					SEQUENCE {

csi-IM-ResourceId						CSI-IM-ResourceId,

-- The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters.

-- Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4)

csi-IM-ResourceElementPattern				CHOICE {

pattern0									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p0						ENUMERATED { s0, s2, s4, s6, s8, s10 },

-- OFDM symbol location of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p0							INTEGER (0..12)

},

pattern1									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p1						ENUMERATED { s0, s4, s8 },

-- OFDM symbol location of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p1							INTEGER (0..13)

}

}																														OPTIONAL,	--Need M

-- Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2)

freqBand									CSI-FrequencyOccupation											OPTIONAL,	-- Need M

-- Periodicity and slot offset for periodic/semi-persistent CSI-IM. Corresponds to L1 parameter 'CSI-IM-timeConfig'

periodicityAndOffset					CHOICE {

sl4										INTEGER (0..3),

sl5										INTEGER (0..4),

sl8										INTEGER (0..7),

sl10									INTEGER (0..9),

sl16									INTEGER (0..15),

sl20									INTEGER (0..19),

sl32									INTEGER (0..31),

sl40									INTEGER (0..39),

sl64									INTEGER (0..63),

sl80									INTEGER (0..79),

sl160									INTEGER (0..159),

sl320									INTEGER (0..319),

sl640									INTEGER (0..639)

}

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::= 			INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-SSB-Resource

The IE CSI-SSB-Resource is used to configure one SSB resource.

CSI-SSB-Resource information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCE-START

CSI-SSB-Resource ::=						SEQUENCE {

-- FFS: Undefined what the IE CSI-SSB-Resource contains.

}

-- TAG-CSI-SSB-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure FFS

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

-- Configuration of a CSI-Report sent on L1 (e.g. PUCCH) (see 38.214, section 5.2.1)

CSI-ReportConfig ::=					SEQUENCE {

reportConfigId							CSI-ReportConfigId,

-- Time domain behavior of reporting configuration

reportConfigType						CHOICE {

periodic								SEQUENCE {

-- Periodicity and slot offset . Corresponds to L1 parameter 'ReportPeriodicity'and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentPUCCH						SEQUENCE {

-- Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity' and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentPUSCH						SEQUENCE {

-- Periodicity. Corresponds to L1 parameter 'Reportperiodicity-spCSI'. (see 38.214, section 5.2.1.1?FFS_Section)

reportSlotConfig						ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

-- RNTI for SP CSI-RNTI, Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section 5.2.1.5.2)

-- FFS: RAN1 models different RNTIs as different Search Spaces with independent configurations. Align the configuration

-- of this one (e.g. group with monitoring periodicity, PDCCH candidate configuration, DCI-Payload size...)?

csi-RNTI								RNTI-Value,

-- Index of the p0-alpha set determining the power control for this CSI report transmission.

-- Corresponds to L1 parameter 'SPCSI-p0alpha' (see 38.214, section FFS_Section)

p0alpha									P0-PUSCH-AlphaSetId

},

aperiodic								SEQUENCE {

-- Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. A particular value is indicated in DCI.

-- (see 38.214, section 5.2.3)

-- FFS_Value: Range wasn’t final in RAN1 table.

-- FFS_FIXME: How are the DCI codepoints mapped to the allowed offsets?

reportSlotOffset					SEQUENCE (SIZE (1..4)) OF INTEGER (0..8)

}

},

-- The CSI related quanities to report. Corresponds to L1 parameter 'ReportQuantity' (see 38.214, section REF)

reportQuantity							CHOICE {

none									NULL,

cri-RI-PMI-CQI							NULL,

cri-RI-i1								NULL,

cri-RI-i1-CQI							SEQUENCE {

-- PRB bundling size to assume for CQI calcuation when reportQuantity is CRI/RI/i1/CQI

-- Corresponds to L1 parameter 'PDSCH-bundle-size-for-CSI' (see 38.214, section 5.2.1.4)

pdsch-BundleSizeForCSI					ENUMERATED {n2, n4}		OPTIONAL

},

cri-RI-CQI								NULL,

cri-RSRP								NULL,

ssb-Index-RSRP							NULL,

cri-RI-LI-PMI-CQI						NULL

},

-- Reporting configuration in the frequency domain. (see 38.214, section 5.2.1.4)

reportFreqConfiguration					 SEQUENCE {

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see 38.214, section 5.2.1.4)

cqi-FormatIndicator						ENUMERATED { widebandCQI, subbandCQI },

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see 38.214, section 5.2.1.4)

pmi-FormatIndicator						ENUMERATED { widebandPMI, subbandPMI },

-- Indicates a contiguous or non-contigous subset of subbands in the bandwidth part which CSI shall be reported

-- for. FFS: Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the

-- lowest subband in the BWP. (see 38.214, section 5.2.1.4)

-- FFS: Size of the bitmap. Introduce a CHOICE with different bitmap lengths depening on number of subbands in carrier/BWP?

csi-ReportingBand						BIT STRING (SIZE (ffsValue))

},

-- Time domain measurement restriction for the channel (signal) measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-channel' (see 38.214, section 5.2.1.1)

timeRestrictionForChannelMeasurements				ENUMERATED {configured, notConfigured},

-- Time domain measurement restriction for interference measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-interference' (see 38.214, section 5.2.1.1)

timeRestrictionForInterferenceMeasurements			ENUMERATED {configured, notConfigured},

-- Codebook configuration for Type-1 or Type-II including codebook subset restriction

codebookConfig								CodebookConfig,

-- Maximum number of CQIs per CSI report (cf. 1 for 1-CW, 2 for 2-CW)

nrofCQIsPerReport							ENUMERATED {n1, n2},

-- Turning on/off group beam based reporting (see 38.214, section 5.2.1.4)

groupBasedBeamReporting						CHOICE {

enabled										SEQUENCE {

-- Number of beams to report for group based beam reporting (see 38.214, section REF)

nrofBeamsToReport						ENUMERATED {ffsTypeAndValue}

},

disabled 								SEQUENCE {

-- The number (N) of measured RS resources to be reported per report setting in a non-group-based report.

-- N <= N_max, where N_max is either 2 or 4 depending on UE capability.

-- FFS: The signaling mechanism for the gNB to select a subset of N beams for the UE to measure and report.

-- FFS: Note: this parameter may not be needed for certain resource and/or report settings

-- FFS_ASN1: Change groupBasedBeamReporting into a CHOICE and include this field into the “no” option?

-- (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

nrofReportedRS							ENUMERATED {n1, n2, n3, n4}												OPTIONAL	-- Need S

}

},

-- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-table' (see 38.214, section 5.2.2.1)

cqi-Table								ENUMERATED {table1, table2, spare2, spare1}										OPTIONAL,

-- Indicates one out of two possible BWP-dependent values for the subband size as indicated in 38.214 table 5.2.1.4-2

-- Corresponds to L1 parameter 'SubbandSize' (see 38.214, section 5.2.1.4)

subbandSize								ENUMERATED {value1, value2},

-- BLER target that the UE shall be assume in its CQI calculation.

-- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section 5.2.2.1)

-- FFS_Values (now filled with spares)

bler-Target								ENUMERATED {zerodot1, spare3, space2, spare1}										OPTIONAL,

-- Port indication for RI/CQI calculation. For each  CSI-RS resource in the linked ResourceConfig for channel measurement,

-- a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback.

-- Corresponds to L1 parameter 'Non-PMI-PortIndication' (see 38.214, section FFS_Section)

non-PMI-PortIndication					FFS_Value																		OPTIONAL,

-- Which DL BWP the CSI-ReportConfig is associated with. (see 38.214, section FFS_Section)

-- FFS_CHECK: Should it be possible to link a report to several BWPs? If not, shouldn’t the report configuration be in the BWP?

-- FFS_CHECK: Should it be possible to link a report to the initial BWP? If so, which ID does that have?

bandwidthPartId							BandwidthPartId																	OPTIONAL

}

PUCCH-CSI-Resource ::= 				CHOICE {

uplinkBandwidthPartId				BWP-Id,

-- PUCCH resource for the assocaited uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported.

pucch-Resource						PUCCH-Resource

}

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::= 					INTEGER (0..maxNrofCSI-ReportConfig-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see 38.214, section 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::= 						SEQUENCE {

-- Number of antenna ports in first dimension

codebookConfigN1						ENUMERATED {n1,n2,n3,n4,n6,n8,n12,n16},

-- Number of antenna ports in second dimension

codebookConfigN2						ENUMERATED {n1,n2,n3,n4},

-- Codebook subset restriction for the different codebooks

-- CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType'

-- (see 38.214, section 5.2.2.2)

codebookType 							CHOICE {

type1 									SEQUENCE {

subType									ENUMERATED {typeI-SinglePanel, typeI-MultiPanel},

-- Switch between Config 1 and Config 2

codebookMode							ENUMERATED {config1, config2},

-- Number of panels, Ng, used in multi-panel codebook

numberOfPanels							ENUMERATED {twopanels, fourpanels}					OPTIONAL,		-- Cond TypeI-MultiPanel

codebookSubsetRestrictionType1			CHOICE {

-- Codebook subset restriction for Type I Single-panel codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

-- FFS_Value: RAN1 indicated Bitmap of size N1*N2*O1*O2

singlePanel								BIT STRING (SIZE (ffsValue)),

-- Codebook subset restriction for 2TX codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

singlePanel2TX							BIT STRING (SIZE (6)),

-- Codebook subset restriction for Type I Multi-panel codebook

-- Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

multiPanel								BIT STRING (SIZE (ffsValue)),

-- i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214, section FFS_Section)

singlePanelCodebookSubsetRestriction-i2	BIT STRING (SIZE (16))

},

ri-Restriction							CHOICE {

-- Restriction for RI for TypeI-SinglePanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-SinglePanelRI-Restriction			BIT STRING (SIZE (8)),

-- Restriction for RI for TypeI-MultiPanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-MultiPanelRI-Restriction			BIT STRING (SIZE (4))

}

},

type2									SEQUENCE {

subType									ENUMERATED {typeII, typeII-PortSelection},

-- The size of the PSK alphabet, QPSK or 8-PSK

phaseAlphabetSize						ENUMERATED {n4, n8},

-- If subband amplitude reporting is activated (true)

subbandAmplitude						BOOLEAN,

-- Number of beams, L,  used for linear combination

numberOfBeams							ENUMERATED {two, three, four},

-- The size of the port selection codebook (parameter d)

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Cond TypeII-PortSelection

-- Codebook subset restriction for Type II codebook.

-- FFS: Clarify the meaning of the bitmap

-- FFS: The size of the bitmap is ceil(log2(nchoosek(O1*O2,4)))+8*N1*N2 ==> Clarify size. Present different bitmap sizes by CHOICE?

codebookSubsetRestrictionType2			BIT STRING (SIZE (ffsValue)),

ri-Restriction							CHOICE {

-- Restriction for RI for TypeII-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section 5.2.2.3)

typeII-RI-Restriction					BIT STRING (SIZE (2)),

-- Restriction for RI for TypeII-PortSelection-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section 5.2.2.4)

typeII-PortSelectionRI-Restriction		BIT STRING (SIZE (2))

}

}

}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

#### –	CSI-MeasIdToAddMod

The IE CSI-MeasIdToAddMod is used to link a CSI-RS-ResourceConfig to a CSI-ReportConfig (see 38.214, section 5.2)

CSI-MeasIdToAddMod information element

-- ASN1START

-- TAG-CSI-MEASIDTOADDMOD-START

CSI-MeasIdToAddMod ::=					SEQUENCE {

csi-measId								CSI-MeasId,

csi-RS-resourceConfigId					CSI-ResourceConfigId,

csi-ReportConfigId						CSI-ReportConfigId,

-- For CQI-Emulation, i.e., how to measure and compute the CQI.

-- CHECK: Clarify further what the values mean.

-- CHECK: Is there a need to inform the UE which resource to use for which measurement (signal, interference, ...)?

measQuantity							ENUMERATED {channel, interference}

}

-- TAG-CSI-MEASIDTOADDMOD-STOP

-- ASN1STOP

#### –	CSI-MeasId

The IE CSI-MeasId is used to identify one CSI-MeasIdToAddMod entry

CSI-MeasId information element

-- ASN1START

-- TAG-CSI-MEASID-START

CSI-MeasId ::= 							INTEGER (0..maxNrofCSI-MeasId-1)

-- TAG-CSI-MEASID-STOP

-- ASN1STOP

#### –	DMRS-DownlinkConfig

The IE DMRS-DownlinkConfig is used to configure downlink demodulation reference signals for PDSCH.

DMRS-DownlinkConfig information element

-- ASN1START

-- TAG-DMRS-DOWNLINKCONFIG-START

DMRS-DownlinkConfig ::=					SEQUENCE {

-- Selection of the DMRS type to be used for DL (see 38.211, section 7.4.1.1.1)

dmrs-Type								ENUMERATED {type1, type2}													OPTIONAL,	-- Need R

-- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211.

-- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL.

dmrs-AdditionalPosition					ENUMERATED {pos0, pos1, pos2, pos3}											OPTIONAL,	-- Need R

-- DM-RS groups that are QCL:ed, i.e. group 1 (see 38.214, section 5.1)

-- FFS CHECK: Clarify how to configure the DMRS groups and the relation to TCI.

dmrs-group1								INTEGER (1000..1011),

-- DM-RS groups that are QCL:ed, i.e. group 2 (see 38.214, section 5.1)

dmrs-group2								INTEGER (1000..1011),

-- The maximum number of OFDM symbols for DL front loaded DMRS

-- Corresponds to L1 parameter 'DL-DMRS-max-len' (see 38.214, section 5.1)

maxLength								ENUMERATED {len1, len2},

-- DL DMRS scrambling initalization

-- Corresponds to L1 parameter 'DL-DMRS-Scrambling-ID1' (see 38.214, section 5.1)

-- When the field is absent the UE applies the value Physical cell ID (physCellId) configured for this serving cell."

scramblingID1							INTEGER (0..65535)													OPTIONAL, 	-- Need S

-- DL DMRS scrambling initalization. Corresponds to L1 parameter 'DL-DMRS-Scrambling-ID2' (see 38.214, section 5.1)

-- When the field is absent the UE applies the value (physCellId) configured for this serving cell.

scramblingID2							INTEGER (0..65535)													OPTIONAL, 	-- Need S

...

}

-- TAG-DMRS-DOWNLINKCONFIG-STOP

#### -- ASN1STOP

#### –	DMRS-UplinkConfig

The IE DMRS-UplinkConfig is used to configure FFS

DMRS-UplinkConfig information element

-- ASN1START

-- TAG-DMRS-UPLINKCONFIG-START

DMRS-UplinkConfig ::=					SEQUENCE {

-- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.3)

dmrs-Type								ENUMERATED {type1, type2} 												OPTIONAL,

-- Position for additional DM-RS in UL. Corresponds to L1 parameter 'UL-DMRS-add-pos' (see Table 7.4.1.1.2-4 in 38.211)

-- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for UL.

dmrs-AdditionalPosition					ENUMERATED {pos0, pos1, pos2, pos3}										OPTIONAL,	-- Need R

-- Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref

phaseTrackingRS						SetupRelease { PTRS-UplinkConfig }										OPTIONAL,	-- Need M

-- The maximum number of OFDM symbols for UL front loaded DMRS.

-- Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2)

maxLength								ENUMERATED {len1, len2}													OPTIONAL,

modeSpecificParameters					CHOICE {

-- DMRS related parameters for Cyclic Prefix OFDM

cp-OFDM									SEQUENCE {

-- UL DMRS scrambling initalization for CP-OFDM

-- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID1' (see 38.214, section 6.4.1.1.2)

-- When the field is absent the UE applies the value Physical cell ID (physCellId)

-- FFS: Is this parameter also needed in cell specific signalling, e.g. to send PUSCH Msg3?

scramblingID1							INTEGER (0..65535)												OPTIONAL,	-- Need S

-- UL DMRS scrambling initalization for CP-OFDM.

-- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID2' (see 38.214, section 6.4.1.1.2)

-- When the field is absent the UE applies the value Physical cell ID (physCellId)

-- FFS: Is this parameter also needed in cell specific signalling, e.g. to send PUSCH Msg3?

scramblingID2							INTEGER (0..65535)												OPTIONAL		-- Need S

},

-- DMRS related parameters for DFT-s-OFDM (Transform Precoding)

dft-S-OFDM								SEQUENCE {

-- Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS

-- Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section)

-- FFS: Should we reaplace this explicit type by the type ScramblingId?

nPUSCH-Identity							INTEGER(0..1007)													OPTIONAL,

-- Sequence-group hopping for PUSCH can be disabled for a certain UE despite being enabled on a cell basis. For DFT-s-OFDM DMRS

-- Corresponds to L1 parameter 'Disable-sequence-group-hopping-Transform-precoding' (see 38.211, section FFS_Section)

disableSequenceGroupHopping				ENUMERATED {disabled}												OPTIONAL,

-- Determines if sequence hopping is enabled or not. For DFT-s-OFDM DMRS

-- Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section)

sequenceHoppingEnabled					ENUMERATED {enabled}												OPTIONAL

}

},

...

}

-- TAG-DMRS-UPLINKCONFIG-STOP

-- ASN1STOP

#### –	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

DRB-Identity information elements

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=					INTEGER (1..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of EN-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEAS-RESULT -SCG-FAILURE-START

-- FFS if failureType is needed

MeasResultSCG-Failure ::= 			SEQUENCE {

measResultServFreqList					MeasResultServFreqList2NR,

measResultNeighCells						MeasResultList2NR,

...

}

MeasResultServFreqList2NR ::=				SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServFreq2NR

MeasResultServFreq2NR ::=				SEQUENCE {

carrierFreq								ARFCN-ValueNR,

measResultServingCell						MeasResultNR,

measResultBestNeighCell				MeasResultNR		OPTIONAL

}

MeasResultList2NR ::=					SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

MeasResult2NR ::=						SEQUENCE {

carrierFreq								ARFCN-ValueNR,

measResultListNR								MeasResultListNR

}

-- TAG-MEAS-RESULT -SCG-FAILURE-STOP

-- ASN1STOP

#### –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-DL-START

FrequencyInfoDL ::= 				SEQUENCE {

-- Frequency of the SSB to be used for this serving cell.

absoluteFrequencySSB					GSCN-ValueNR,

-- The frequency domain offset between SSB and the overall resource block grid in number of subcarriers.

-- Absence of the field indicates that no offset is applied (offset = 0). See 38.211, section 7.4.3.1)

ssb-SubcarrierOffset				INTEGER (1..15)																	OPTIONAL,

-- Absolute frequency position of the lowest subcarrier (point A) of the reference PRB (Common PRB 0).

-- Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

absoluteFrequencyPointA						ARFCN-ValueNR																	OPTIONAL,

-- A set of virtual carriers for different subcarrier spacings (numerologies). Defined in relation to Point A.

-- Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

scs-SpecificCarriers				SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier							,

...

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

#### –	SCS-SpecificVirtualCarrier

The IE SCS-SpecificVirtualCarrier provides parameters determining the location and width of a carrier with a specific subcarrier spacing (SCS).

-- ASN1START

-- TAG-SCS-SPECIFIC-VIRTUAL-CARRIER-START

SCS-SpecificVirtualCarrier ::=			SEQUENCE {

-- Offset in frequency domain between Point A (lowest subcarrier of common PRB 0) and the lowest usable subcarrier on this virtual carrier.

-- The maximum value corresponds to 275*8-1. Corresponds to L1 parameter 'offset-pointA-low-scs' (see 38.211, section FFS_Section)

offsetToVirtualCarrier				INTEGER (0..2199)		,

-- Subcarrier spacing of this virtual carrier. It is used to convert the offsetToVirtualCarrier into an actual frequency.

-- Corresponds to L1 parameter 'ref-scs' (see 38.211, section FFS_Section)

subcarrierSpacing					SubcarrierSpacing																	OPTIONAL,

-- FFS_Description (see 38.211, section FFS_Section)

k0									FFS_Value																			OPTIONAL,

-- Width of this virtual carrier in number of PRBs (using the subcarrier spacing defined for this virtual carrier)

-- Corresponds to L1 parameter 'BW' (see 38.211, section FFS_Section)

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks)											OPTIONAL,

...

}

-- TAG-SCS-SPECIFIC-VIRTUAL-CARRIER-STOP

-- ASN1STOP

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-UL-START

FrequencyInfoUL ::= 				SEQUENCE {

-- Absolute frequency of the lowest subcarrier (point A) of the reference PRB (Common PRB 0).

-- Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

absoluteFrequencyPointA						ARFCN-ValueNR											,

-- A set of virtual carriers for different subcarrier spacings (numerologies). Defined in relation to Point A.

-- Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

scs-SpecificCarriers				SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier							,

additionalSpectrumEmission			AdditionalSpectrumEmission										OPTIONAL,	-- Need S

p-Max								P-Max															OPTIONAL,	-- Need S

-- Enable or disable the NR UL transmission with a 7.5KHz shift to the LTE raster

frequencyShift7p5khz				ENUMERATED {true}												OPTIONAL,	-- Cond FDD

...

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

#### –	GSCN-ValueNR

The IE GSCN-ValueNR is used to indicate the frequency positions of the SS/PBCH Blocks, as defined in TS 38.101 [15].

-- ASN1START

-- TAG-GSCN-VALUE-NR-START

GSCN-ValueNR ::=				CHOICE {

-- Frequency raster index and offset for 0 - 2,65 GHz. Corresponds to parameter 'N' (see 38.101, section FFS_Section)

lowCarrierFrequency				SEQUENCE {

frequencyIndex					INTEGER (1..2944),

-- An offset of -5kHz (M=-1) or +5kHz (M=1) to the absoluteFrequencySSB. When the field is absent, the UE applies no offset (M=0).

-- The offset is only applicable for the frequency range 0-2.65GHz. Corresponds to parameter 'M' (see 38.101, section FFS_Section)

frequencyOffsetSSB				FrequencyOffsetSSB																OPTIONAL	-- Need R

},

-- Frequency raster index for 2.4GHz - 24,25 GHz. Corresponds to parameter 'N' (see 38.101, section FFS_Section)

midCarrierFrequency				INTEGER (0..15173),

-- Frequency raster index for 24.25-100GHz range. Corresponds to parameter 'N' (see 38.101, section FFS_Section)

highCarrierFrequency			INTEGER (0..4383)

}

FrequencyOffsetSSB ::=				ENUMERATED { minus5kHz, plus5kHz }

-- TAG-GSCN-VALUE-NR-STOP

-- ASN1STOP

#### –	LogicalChannelConfig

The IE LogicalChannelConfig is used to configure the logical channel parameters.

LogicalChannelConfig information element

-- ASN1START

-- TAG-LOGICAL-CHANNEL-CONFIG-START

LogicalChannelConfig ::=		SEQUENCE {

ul-SpecificParameters			SEQUENCE {

priority						INTEGER (1..16),

prioritisedBitRate				ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

bucketSizeDuration				ENUMERATED {ms50, ms100, ms150, ms300, ms500, ms1000, spare2, spare1},

allowedServingCells				SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex					OPTIONAL,	-- Need R

allowedSCS-List				SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing															OPTIONAL,	-- Need R

maxPUSCH-Duration					ENUMERATED {ffsTypeAndValue}												OPTIONAL,	-- Need R

configuredGrantType1Allowed		ENUMERATED {true}															OPTIONAL,	-- Need R

logicalChannelGroup				INTEGER (0..maxLCG-ID)												OPTIONAL,

schedulingRequestID				SchedulingRequestId															OPTIONAL,	-- Need R

logicalChannelSR-Mask			BOOLEAN,

logicalChannelSR-DelayTimerApplied	BOOLEAN

}																												OPTIONAL,		-- Cond UL

-- other parameters

...

}

-- TAG-LOGICAL-CHANNEL-CONFIG-STOP

-- ASN1STOP

LogicalChannelConfig field descriptions

allowedSCS-List If present, UL MAC PDUs from this logical channel can only be mapped to the indicated numerology as specified in TS 38.321 [3].

allowedServingCells If present, the UE maps PDUs of this logical channel only to the serving cells indicated in this list. Corresponds to 'lcp-allowedServingCells' in TS 38.321 [3]

bucketSizeDuration Value in ms. ms50 corresponds to 50ms, ms100 corresponds to 100ms, and so on.

configuredGrantType1Allowed If present, UL MAC PDUs from this logical channel are allowed to be transmitted on a configured grant type 1. Corresponds to 'lcp-ConfiguredGrantType1Allowed' in TS 38.321 [3].

logicalChannelGroup ID of the logical channel group, as specified in TS 38.321 [3], which the logical channel belongs to.

logicalChannelSR-Mask Indicates whether SR masking is configured for this logical channel.

logicalChannelSR-DelayTimerApplied Indicates whether to apply the delay timer for SR transmission for this logical channel. Set to FALSE if logicalChannelSR-DelayTimer is not included in BSR-Config.

maxPUSCH-Duration If present, UL MAC PDUs from this logical channel can only be transmittedin using uplink grants that result in a PUSCH duration shorter than or equal to the the duration indicated by this field. Corresponds to "lcp-maxPUSCH-Duration' in TS 38.321 [3].

priority Logical channel priority, as specified in TS 38.321 [3].

prioritisedBitRate Value in kiloBytes/s. 0kBps corresponds to 0, 8kBps corresponds to 8 kiloBytes/s,16 kBps corresponds to 16 kiloBytes/s, and so on.

schedulingRequestId If present, it indicates the scheduling request configuration applicable for this logical channel.

| Conditional Presence | Explanation |
| --- | --- |
| UL | The field is mandatory present for a logical channel with uplink if it serves DRB. It is optionally present for a logical channel with uplink if it serves an SRB. otherwise it is not present. |

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELL-GROUP-CONFIG-START

MAC-CellGroupConfig ::= 			SEQUENCE {

drx-Config							SetupRelease { DRX-Config }																			OPTIONAL, -- Need M

schedulingRequestConfig				SchedulingRequestConfig																OPTIONAL,

bsr-Config							BSR-Config																	OPTIONAL,	-- Need M

tag-Config							TAG-Config																	OPTIONAL,	-- Need M

phr-Config							SetupRelease { PHR-Config }																			OPTIONAL,	-- Need M

-- FFS : configurable per SCell?

skipUplinkTxDynamic					BOOLEAN,

-- RNTI value for downlink SPS (see SPS-config) and uplink configured scheduling (see ConfiguredSchedulingConfig).

cs-RNTI								SetupRelease { RNTI-Value }															OPTIONAL	-- Need M

}

DRX-Config ::=

SEQUENCE {

drx-onDurationTimer					CHOICE {

subMilliSeconds	INTEGER (1..31),

milliSeconds	ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer					ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL				INTEGER (0..56),

drx-HARQ-RTT-TimerUL				INTEGER (0..56),

drx-RetransmissionTimerDL			ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL			ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset		CHOICE {

ms10							INTEGER(0..9),

ms20							INTEGER(0..19),

ms32							INTEGER(0..31),

ms40							INTEGER(0..39),

ms60							INTEGER(0..59),

ms64							INTEGER(0..63),

ms70							INTEGER(0..69),

ms80							INTEGER(0..79),

ms128							INTEGER(0..127),

ms160							INTEGER(0..159),

ms256							INTEGER(0..255),

ms320							INTEGER(0..319),

ms512							INTEGER(0..511),

ms640							INTEGER(0..639),

ms1024							INTEGER(0..1023),

ms1280							INTEGER(0..1279),

ms2048							INTEGER(0..2047),

ms2560							INTEGER(0..2559),

ms5120							INTEGER(0..5119),

ms10240							INTEGER(0..10239)

},

-- FFS need for finer offset granulary

-- FFS need for shorter values for long and short cycles

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}		OPTIONAL,													-- Need R

drx-SlotOffset

INTEGER (0..31)

}

PHR-Config ::=

SEQUENCE {

phr-PeriodicTimer					ENUMERATED {sf10, sf20, sf50, sf100, sf200,sf500, sf1000, infinity},

phr-ProhibitTimer					ENUMERATED {sf0, sf10, sf20, sf50, sf100,sf200, sf500, sf1000},

phr-Tx-PowerFactorChange			ENUMERATED {dB1, dB3, dB6, infinity},

multiplePHR							BOOLEAN,

phr-Type2PCell						BOOLEAN,

phr-Type2OtherCell					BOOLEAN,

phr-ModeOtherCG						ENUMERATED {real, virtual}

}

TAG-Config ::=				SEQUENCE {

tag-ToReleaseList			SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-Id																	OPTIONAL,	-- Need N

tag-ToAddModList			SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG-ToAddMod																	OPTIONAL		-- Need N

}

TAG-ToAddMod ::= 			SEQUENCE {

tag-Id						TAG-Id,

timeAlignmentTimer			TimeAlignmentTimer,

...

}

TAG-Id ::=					INTEGER (0..maxNrofTAGs-1)

TimeAlignmentTimer ::= 		ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120,ms10240, infinity}

BSR-Config ::=				SEQUENCE {

-- FFS: other values for periodicBSR-Timer, "every PDU" value

periodicBSR-Timer			ENUMERATED {

sf1, sf5, sf10, sf16, sf20, sf32, sf40, sf64, sf80, sf128, sf160, sf320, sf640, sf1280, sf2560, infinity},

retxBSR-Timer				ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, sf2560, sf5120, sf10240},

logicalChannelSR-DelayTimer		ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}	OPTIONAL

}

-- TAG-MAC-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-HARQ-RTT-TimerDL Value in number of symbols.

drx-HARQ-RTT-TimerUL Value in number of symbols.

drx-InactivityTimer Value in multiple integers of 1ms. ms0 corresponds to 0, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1ms.

drx-RetransmissionTimerDL Value in number of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle. A value of 1 corresponds to drx-ShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0ms, value 1 corresponds to 1/32ms, value 2 corresponds to 2/32ms, and so on.

logicalChannelSR-DelayTimer Value in number of subframes. sf1 corresponds to one subframe, sf2 corresponds to 2 subframes, and so on.

multiplePHR Indicates if power headroom shall be reported using the Single PHR MAC control element or Multiple PHR MAC control element defined in TS 38.321 [3]. True means to use Multiple PHR MAC control element and False means to use the Single PHR MAC control element defined in TS 38.321 [3].

phr-Tx-PowerFactorChange Value in dBf or PHR reporting as specified in TS 38.321 [3]. Value dB1 corresponds to 1 dB, dB3 corresponds to 3 dB and so on. The same value applies for each serving cell (although the associated functionality is performed independently for each cell).

phr-ModeOtherCG FFS

phr-PeriodicTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. sf10 corresponds to 10 subframes, sf20 corresonds to 20 subframes, and so on.

phr-ProhibitTimer Value in number of subframes for PHR reporting as specified in TS 38.321 [3]. sf0 corresponds to 0 subframe, sf10 corresponds to 10 subframes, sf20 corresponds to 20 subframes, and so on.

phr-Type2PCell Indicates whether or not PHR type 2 is reported for the PCell

phr-Type2OtherCell Indicates whether or not PHR type 2 is reported for the PSCell and PUCCH SCells.

skipUplinkTxDynmaic Indicates whether If configured, the UE skips UL transmissions for an uplink grant other than a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 38.321 [3].

timeAlignmentTimer Value in ms of the timeAlignmentTimer for TAG with ID tag-Id, as specified in TS 38.321 [3].

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL, 	-- Need M

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL, 	-- Need M

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL, 	-- Need M

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL, 	-- Need M

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL, 	-- Need M

measIdToAddModList						MeasIdToAddModList												OPTIONAL, 	-- Need M

-- Other parameters

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-RSRP								RSRP-Range,

csi-RSRP								RSRP-Range

} 																										OPTIONAL, 	-- Need M

quantityConfig						QuantityConfig														OPTIONAL, 	-- Need M

-- Placehold for measGapConfig

measGapConfig							SetupRelease{MeasGapConfig}													OPTIONAL 	-- Need M

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Whether UE speed based TTT scaling (e.g. speedStatePars) is supported in Rel-15.

Editor’s Note: FFS Whether measScaleFactor (or equivalent) is supported in Rel-15.

Editor’s Note: FFS How to support allowInterruptions in NR (RAN4 input needed) in Rel-15.

Editor’s Note: FFS where to add RLM related parameters: rlm-ResourceConfigCSI-RS, rlm-ResourceConfigSS

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for PCell or PSCell (when UE is in EN-DC) RSRP measurement controlling when the UE is required to perform measurements associated to neighbouring cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

-- ASN1START

MeasGapConfig ::=				SEQUENCE {

gapFR2 						GapConfig				OPTIONAL,

...

}

GapConfig ::=					SEQUENCE {

gapOffset 					INTEGER (0..159),

mgl 						ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp 						ENUMERATED {ms20, ms40, ms80, ms160},

...

}

-- ASN1STOP

MeasGapConfig field descriptions

gapFR2 Indicates measurement gap configuration applies to FR2 only. The applicability of the measurement gap is according to Table 9.1.2-2 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicates in the field mgrp. The value range should be from 0 to mgrp-1.

mgl Value mgl is the measurement gap length in (ms) of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14].

mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14].

#### –	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEAS-ID-START

MeasId ::=							INTEGER (1..maxNrofMeasId)

-- TAG-MEAS-ID-STOP

-- ASN1STOP

#### –	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-START

MeasIdToAddModList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=					SEQUENCE {

measId								MeasId,

measObjectId						MeasObjectId																	OPTIONAL,

reportConfigId						ReportConfigId

}

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

Editor’s Note: FFS Whether measObjectId is an OPTIONAL field or mandatory as in LTE (discuss the implication in procedural text).

#### –	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for intra-frequency or inter-frequency E-UTRA cells.

Editor’s Note: FFS Details of measObjectEUTRA that can be configured via NR.

#### –	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEAS-OBJECT-ID-START

MeasObjectId ::=							INTEGER (1..maxNrofObjectId)

-- TAG-MEAS-OBJECT-ID-STOP

-- ASN1STOP

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=							SEQUENCE {

ssbAbsoluteFreq								GSCN-ValueNR,

--FFS whether reference frequency represents pointA

refFreqCSI-RS								ARFCN-ValueNR															OPTIONAL,

--RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

referenceSignalConfig						ReferenceSignalConfig,

--Consolidation of L1 measurements per RS index

absThreshSS-BlocksConsolidation			ThresholdNR																	OPTIONAL, 	-- Need R

absThreshCSI-RS-Consolidation			ThresholdNR																	OPTIONAL, 	-- Need R

--Config for cell measurement derivation

nrofSS-BlocksToAverage					INTEGER (2..maxNrofSS-BlocksToAverage)										OPTIONAL, 	-- Need R

nrofCSI-RS-ResourcesToAverage			INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)								OPTIONAL, 	-- Need R

-- Filter coefficients applicable to this measurement object

quantityConfigIndex							INTEGER (1.. maxNrofQuantityConfig),

--Frequency-specific offsets

offsetFreq									Q-OffsetRangeList,

-- Cell list

cellsToRemoveList							PCI-List																OPTIONAL, 	-- Need M

cellsToAddModList							CellsToAddModList															OPTIONAL, 	-- Need M

-- Black list

blackCellsToRemoveList						PCI-RangeIndexList																OPTIONAL, 	-- Need M

blackCellsToAddModList						BlackCellsToAddModList														OPTIONAL, 	-- Need M

-- White list

whiteCellsToRemoveList						PCI-RangeIndexList																OPTIONAL, 	-- Need M

whiteCellsToAddModList						WhiteCellsToAddModList														OPTIONAL 	-- Need M

-- FFS: Where to include L1 parameters for RSSI measurements (SS-RSSI-MeasurementConfig in L1 table)

}

ReferenceSignalConfig ::=     			 SEQUENCE {

subcarrierSpacingSSB                     	SubcarrierSpacingSSB,

-- SSB configuration for mobility (nominal SSBs, timing configuration)

ssb-ConfigMobility						SSB-ConfigMobility				OPTIONAL, 	-- Need M

-- CSI-RS resources to be used for CSI-RS based RRM measurements

csi-rs-ResourceConfigMobility			CSI-RS-ResourceConfigMobility	OPTIONAL -- Need R

}

-- A measurement timing configuration

SSB-ConfigMobility ::= 	SEQUENCE {

subcarrierSpacingSSB                    SubcarrierSpacingSSB,

-- The set of SS blocks to be measured within the SMTC measurement duration.

-- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section)

-- When the field is absent the UE measures on all SS-blocks

-- FFS_CHECK: Is this IE placed correctly.

ssb-ToMeasure							SetupRelease {

CHOICE {

-- bitmap for sub 3 GHz

shortBitmap							BIT STRING (SIZE (4)),

-- bitmap for 3-6 GHz

mediumBitmap						BIT STRING (SIZE (8)),

-- bitmap for above 6 GHz

longBitmap							BIT STRING (SIZE (64))

}

} 																	OPTIONAL,	-- Need M

-- Indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell:

useServingCellTimingForSync				BOOLEAN,

-- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements.

smtc1									SEQUENCE {

-- Periodicity and offset of the measurement window in which to receive SS/PBCH blocks.

-- Periodicity and offset are given in number of subframes.

-- FFS_FIXME: This does not match the L1 parameter table! They seem to intend an index to a hidden table in L1 specs.

-- (see 38.213, section REF):

periodicityAndOffset					CHOICE {

sf5										INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160									INTEGER (0..159)

},

-- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes

-- (see 38.213, section 4.1)

duration								ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

-- The set of SS blocks to be measured within the SMTC measurement duration.

-- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section)

-- When the field is absent the UE measures on all SS-blocks

-- FFS_CHECK: Is this IE placed correctly.

ssb-ToMeasure							SetupRelease {

CHOICE {

-- bitmap for sub 3 GHz

shortBitmap							BIT STRING (SIZE (4)),

-- bitmap for 3-6 GHz

mediumBitmap						BIT STRING (SIZE (8)),

-- bitmap for above 6 GHz

longBitmap							BIT STRING (SIZE (64))

}

}

},

-- Secondary measurement timing confguration for explicitly signalled PCIs. It uses the offset and duration from smtc1.

-- It is supported only for intra-frequency measurements in RRC CONNECTED.

smtc2 									SEQUENCE {

-- PCIs that are known to follow this SMTC.

pci-List								SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId		OPTIONAL, 	-- Need M

-- Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1.

periodicity								ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, spare2, spare1}

}																										OPTIONAL -- Cond IntraFreqConnected

}

CSI-RS-ResourceConfigMobility ::= 		SEQUENCE {

-- MO specific values

isServingCellMO						BOOLEAN

,

-- Subcarrier spacing of CSI-RS.

-- Supported values are 15, 30 or 60 kHz  (<6GHz), 60 or 120 kHz (>6GHz).

-- Corresponds to L1 parameter 'Numerology' (see 38.211, section FFS_Section)

subcarrierSpacingCSI-RS						SubcarrierSpacingCSI-RS,

-- List of cells

csi-RS-CellList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM))	OF CSI-RS-CellMobility

}

CSI-RS-CellMobility ::=				SEQUENCE {

cellId									PhysCellId,

csi-rs-MeasurementBW					SEQUENCE {

-- Allowed size of the measurement BW in PRBs

-- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section)

nrofPRBs			ENUMERATED { size24, size48, size96, size192, size264},

-- Starting PRB index of the measurement bandwidth

-- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section)

-- FFS_Value: Upper edge of value range unclear in RAN1

startPRB			INTEGER(0..251)

},

-- Frequency domain density for the 1-port CSI-RS for L3 mobility

-- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section)

density									ENUMERATED {d1,d3}		OPTIONAL,

-- List of resources

csi-rs-ResourceList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=				SEQUENCE {

csi-rs-ResourceId-RRM					CSI-RS-ResourceId-RRM,

-- FFS_CHECK whether the following fields are supposed to be per resource (here) or in the resource config (above)

-- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)FFS_Ref

slotConfig								CHOICE {

ms5										INTEGER (0..79),

ms10									INTEGER (0..159),

ms20									INTEGER (0..319),

ms40									INTEGER (0..639)

},

-- Each CSI-RS resource may be associated with one SSB. If such SSB is indicated, the NW also indicates whether the UE may assume

-- quasi-colocation of this SSB with this CSI-RS reosurce.

-- Corresponds to L1 parameter 'Associated-SSB' (see FFS_Spec, section FFS_Section)

-- FFS: What does the UE do if it there is no such SSB-Index?

associatedSSB							SEQUENCE {

-- FFS_Value: Check the value range

ssb-Index								SSB-Index	,

-- The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters

-- Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section)

isQuasiColocated						BOOLEAN

}					OPTIONAL,

-- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref

resourceElementMappingPattern			ENUMERATED {ffsTypeAndValue},

-- Sequence generation parameter for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref

sequenceGenerationConfig				INTEGER (0..1023),

...

}

CSI-RS-ResourceId-RRM ::= 				INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

Q-OffsetRangeList ::=					SEQUENCE {

rsrpOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrqOffsetSSB							Q-OffsetRange				DEFAULT dB0,

sinrOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrpOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

rsrqOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

sinrOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0

}

ThresholdNR ::=							SEQUENCE{

thresholdRSRP							RSRP-Range					OPTIONAL,

thresholdRSRQ							RSRQ-Range					OPTIONAL,

thresholdSINR							SINR-Range					OPTIONAL

}

CellsToAddModList ::=					SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=						SEQUENCE {

physCellId								PhysCellId,

cellIndividualOffset					Q-OffsetRangeList

}

BlackCellsToAddModList ::=				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=					SEQUENCE {

pci-RangeIndex							PCI-RangeIndex,

pci-Range								PCI-Range

}

WhiteCellsToAddModList ::=				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF WhiteCellsToAddMod

WhiteCellsToAddMod ::=					SEQUENCE {

pci-RangeIndex							PCI-RangeIndex,

physCellIdRange							PhysCellIdRange

}

-- TAG-MEAS-OBJECT-NR-STOP

-- ASN1STOP

Editor’s Note: FFS How to support CGI reporting and whether changes are required in MeasObjectNR (e.g. introduction of cellForWhichToReportCGI)

Editor’s Note: FFS Whether alternative TTT is supported in Rel-15.

Editor’s Note: FFS measCycleSCell.

Editor’s Note: FFS reducedMeasPerformance.

Editor’s Note: FFS Whether offsetFreq within measObject can be set differently for CSI-RS and SS/PBCH block.

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per CSI-RS resource as described in 5.5.3.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per SS/PBCH block index as described in 5.5.3.2.

blackCellsToAddModList List of cells to add/modify in the black list of cells.

blackCellsToRemoveList List of cells to remove from the black list of cells.

carrierFreq Identifies NR carrier frequency for which this configuration is valid.

cellIndividualOffset Cell individual offsets applicable to a specific cell.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell in that carrierFreq.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell in that carrierFreq.

offsetFreq Offset values applicable to the carrier frequency.

physCellId Physical cell identity of a cell in the cell list.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

pci-Range Physical cell identity or a range of physical cell identities.

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacingCSI-RS is set to 15kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 4/9/19/39 slots. When subcarrierSpacingCSI-RS is set to 30kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 9/19/39/79 slots. When subcarrierSpacingCSI-RS is set to 60kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 19/39/79/159 slots. When subcarrierSpacingCSI-RS is set 120kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 39/79/159/319 slots. When subcarrierSpacingCSI-RS is set 240kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 79/159/319/639 slots.

whiteCellsToAddModList List of cells to add/modify in the white list of cells.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

#### –	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-START

MeasObjectToAddModList ::=					SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=						SEQUENCE {

measObjectId								MeasObjectId,

measObject									CHOICE {

measObjectNR								MeasObjectNR

}

}

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, and inter-RAT mobility.

MeasResults information element

-- ASN1START

-- TAG-MEAS-RESULTS-START

MeasResults ::=								SEQUENCE {

measId										MeasId,

measResultServingFreqList					MeasResultServFreqList,

measResultNeighCells						CHOICE {

measResultListNR							MeasResultListNR,

...

}																														OPTIONAL,

...

}

MeasResultServFreqList ::=					SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServFreq

MeasResultServFreq ::=						SEQUENCE {

servFreqId									ServCellIndex,

measResultServingCell						MeasResultNR,

measResultBestNeighCell						MeasResultNR,

...

}

MeasResultListNR ::=						SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=							SEQUENCE {

physCellId									PhysCellId																	OPTIONAL,

--FFS: Details of cgi info

cgi-Info									ENUMERATED {ffsTypeAndValue}												OPTIONAL,

measResult									SEQUENCE {

cellResults									SEQUENCE{

resultsSSB-Cell								ResultsSSB-Cell														OPTIONAL,

resultsCSI-RS-Cell 							ResultsCSI-RS-Cell													OPTIONAL

},

rsIndexResults								SEQUENCE{

resultsSSB-Indexes							ResultsPerSSB-IndexList												OPTIONAL,

resultsCSI-RS-Indexes						ResultsPerCSI-RS-IndexList 											OPTIONAL

}																													OPTIONAL

},

...

}

ResultsSSB-Cell ::= 						SEQUENCE {

ssb-Cellrsrp								RSRP-Range																	OPTIONAL,

ssb-Cellrsrq								RSRQ-Range																	OPTIONAL,

ssb-Cellsinr								SINR-Range																	OPTIONAL

}

ResultsCSI-RS-Cell ::= 						SEQUENCE {

csi-rs-CellRSRP								RSRP-Range																	OPTIONAL,

csi-rs-CellRSRQ								RSRQ-Range																	OPTIONAL,

csi-rs-CellSINR								SINR-Range																	OPTIONAL

}

ResultsPerSSB-IndexList ::= 					SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::= 						SEQUENCE {

ssb-Index									SSB-Index,

ss-RSRP										RSRP-Range																	OPTIONAL,

ss-RSRQ										RSRQ-Range																	OPTIONAL,

ss-SINR										SINR-Range																	OPTIONAL

}

ResultsPerCSI-RS-IndexList ::= 				SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::= 					SEQUENCE {

csi-		RS-Index								CSI-RS-Index,

csi-RSRP									RSRP-Range																	OPTIONAL,

csi-RSRQ									RSRQ-Range																	OPTIONAL,

csi-SINR									SINR-Range																	OPTIONAL

}

-- TAG-MEAS-RESULTS-STOP

-- ASN1STOP

Editor’s Note: FFS locationInfo.

MeasResults field descriptions

csi-rs-CellRSRP Measured RSRP result per NR cell based on CSI-RSRP value(s) from the L1 filter(s).

csi-rs-CellRSRQ Measured RSRQ result per NR cell based on CSI-RSRQ value(s) from the L1 filter(s).

csi-rs-CellSINR Measured SINR result per NR cell based on CSI-SINR value(s) from the L1 filter(s).

csi-rs-Index CSI-RS resource index associated to the measurement information to be reported.

csi-RSRP L3 filtered CSI-RSRP measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRP is defined in TS 38.215 [9].

csi-RSRQ L3 filtered CSI-RSRQ measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRQ is defined in TS 38.215 [9].

csi-SINR L3 filtered CSI-SINR measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-SINR is defined in TS 38.215 [9].

measId Identifies the measurement identity for which the reporting is being performed.

measResult Measured results of an NR cell.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultServingFreqList Measured results of the serving frequencies including measurement results of PCell, configured SCell(s) and best neighbouring cell on each serving frequency.

resultsCSI-RS-Indexes List of measurement information per CSI-RS resource index of an NR cell.

resultsSSB-Indexes List of measurement information per SS/PBCH index of an NR cell.

resultsCSI-RS-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived from CSI-RS measurements.

resultSSB-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived on SS/PBCH block measurements.

smtc2 Secondary measurement timing configuration for explicitly signalled PCIs. The timing offset is equal to SMTC1 offset mod SMTC2 periodicity.

ssb-CellRSRP Measured RSRP result per NR cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-CellRSRQ Measured RSRQ result of an NR Cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-CellSINR Measured SS-SINR result of an NR Cell based on SS-SINR value(s) from the L1 filter(s)..

ssb-Index SS/PBCH block index associated to the measurement information to be reported.

ss-rsrp L3 filtered SS-RSRP measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRP is defined in TS 38.215 [9].

ss-rsrq L3 filtered SS-RSRQ measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRQ is defined in TS 38.215 [9].

ss-sinr L3 filtered SS-SINR measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-SINR is defined in TS 38.215 [9].

#### –	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as during handover and PSCell/SCell addition.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=					SEQUENCE {

-- The initial CORESET configured via PBCH (MIB) and ServingCellConfigCommon. It has the ControlResoruceSetId = 0.

initialControlResourceSet					ControlResourceSet														OPTIONAL,	-- Need R

-- The initial Search Space configured via PBCH (MIB) and ServingCellConfigCommon. It has the SearchSpaceId = 0.

initialSearchSpace							SearchSpace																OPTIONAL,	-- Need R

-- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10)

-- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow

-- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs?

-- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present?

searchSpaceOtherSystemInformation			FFS_Value																OPTIONAL,

-- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10)

-- FFS: Which BWP and CORESET to assume?

-- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space?

pagingSearchSpace							FFS_Value																OPTIONAL,

-- CORESET configured for random access. When the field is absent the UE uses the CORESET according to pdcchConfigSIB1pdcch-ConfigSIB1

-- Corresponds to L1 parameter 'rach-coreset-configuration' (see 38.211?, section FFS_Section)

ra-ControlResourceSet					ControlResourceSetId																OPTIONAL, 	-- Need S

-- Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section)

ra-SearchSpace							SearchSpace																		OPTIONAL

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

#### –	PDCCH-Config

The PDCCH-Config IE is used to configure UE specific PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::= 						SEQUENCE {

-- List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE.

-- The network configures at most 3 CORESETs per BWP per cell (including the initial CORESET).

controlResourceSetToAddModList			SEQUENCE(SIZE (1..maxNrofControlResourceSets-1)) OF ControlResourceSet 		OPTIONAL,

controlResourceSetToReleaseList			SEQUENCE(SIZE (1..maxNrofControlResourceSets-1)) OF ControlResourceSetId			OPTIONAL,

-- List of UE specifically configured Control Resource Sets (CORESETs).

-- The network configures at most 10 Search Spaces per BWP per cell (including the initial Search Space).

-- FFS: RAN1 decided to model each RNTI which the UE monitors on PDCCH as a separate Search Space. Many of those are configured

-- in the respective feature/channel configurations (e.g. CSI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, SPS-RNTI). Others are configured

-- below (SFI-RNTI, INT-RNTI). ==> Aim to find a common generic structure.

searchSpacesToAddModList				SEQUENCE(SIZE (1..maxNrofSearchSpaces-1)) OF SearchSpace						OPTIONAL,

searchSpacesToReleaseList				SEQUENCE(SIZE (1..maxNrofSearchSpaces-1)) OF SearchSpaceId						OPTIONAL,

-- Configuration of Slot-Format-Indicators to be monitored in this cell

-- FFS: Can there be just one or multiple such configurations within a PDCCH-Config? How does it relate to BWP, CORESET(s)?

slotFormatIndicator						SlotFormatIndicator															OPTIONAL,

-- FFS: Is there a default timing (to be used at least until first reconfiguration). Are the fields optionally present?

timeDomainResourceAllocation 		SEQUENCE {

-- List of time-domain configurations for timing of DL assignment to DL data

pdsch-AllocationList			SEQUENCE (SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation,

-- Number of repetitions for data. Corresponds to L1 parameter 'aggregation-factor-DL' (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

pdsch-AggregationFactor					ENUMERATED { n2, n4, n8 }													OPTIONAL,	-- Need R

-- List of time domain allocations for timing of UL assignment to UL data

pusch-AllocationList					SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation,

-- Number of repetition for data. Corresponds to L1 parameter 'aggregation-factor-UL' (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

pusch-AggregationFactor					ENUMERATED { n2, n4, n8 }													OPTIONAL, 	-- Need R

-- List of timiing for given PDSCH to the DL ACK. In this version of the specification only the values [0..8] are applicable.

-- Corresponds to L1 parameter 'Slot-timing-value-K1' (see 38.213, section FFS_Section)

dl-data-to-UL-ACK					SEQUENCE (SIZE (8)) OF INTEGER (0..15)											OPTIONAL	-- Need M

}																														OPTIONAL	-- Need M

}

PDSCH-TimeDomainResourceAllocation ::= 	SEQUENCE {

-- Corresponds to L1 parameter 'K0' (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 0

k0									INTEGER (1..3)																OPTIONAL,	-- Need R

-- PDSCH mapping type. Corresponds to L1 parameter 'Mapping-type' (see 38.214, section FFS_Section)

mappingType							ENUMERATED {typeA, typeB}													,

-- An index into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded)

-- Corresponds to L1 parameter 'Index-start-len' (see 38.214, section FFS_Section)

startSymbolAndLength				BIT STRING (SIZE (6))

}

PUSCH-TimeDomainResourceAllocation ::= 	SEQUENCE {

-- Corresponds to L1 parameter 'K2' (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 0

k2									INTEGER (0..7)																OPTIONAL,

-- Mapping type. Corresponds to L1 parameter 'Mapping-type' (see 38.214, section FFS_Section)

mappingType							ENUMERATED {typeA, typeB},

-- An index into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded)

-- Corresponds to L1 parameter 'Index-start-len' (see 38.214, section FFS_Section)

startSymbolAndLength				BIT STRING (SIZE (6))

}

-- A time/frequency control resource set (CORESET) in which to search for downlink control information (see 38.213, section x.x.x.x)FFS_Ref

ControlResourceSet ::= 					SEQUENCE {

-- Corresponds to L1 parameter 'CORESET-ID'

-- Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon?

-- Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling?

controlResourceSetId					ControlResourceSetId,

-- Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from PRB 0, which is fully

-- contained in the bandwidth part within which the CORESET is configured.

-- The most significant bit corresponds to the group of lowest frequency which is fully contained in the bandwidth part within which the

-- CORESET is configured, each next subsequent lower significance bit corresponds to the next lowest frequency group fully contained within

-- the bandwidth part within which the CORESET is configured, if any.

-- Bits corresponding to a group not fully contained within the bandwidth part within which the CORESET is configured are set to zero.

-- Corresponds to L1 parameter 'CORESET-freq-dom'(see 38.211, section 7.3.2.2)

frequencyDomainResources				BIT STRING (SIZE (45)),

-- Contiguous time duration of the CORESET in number of symbols

-- Corresponds to L1 parameter 'CORESET-time-duration' (see 38.211, section 7.3.2.2FFS_Section)

duration								INTEGER (1..maxCoReSetDuration),

-- Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG).

-- Corresponds to L1 parameter 'CORESET-CCE-REG-mapping-type' (see 38.211Section sections 7.3.2.2 and 7.4.1.3.2)

cce-REG-MappingType							CHOICE {

interleaved								SEQUENCE {

-- Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles.

-- Corresponds to L1 parameter 'CORESET-REG-bundle-size' (see 38.211, section FFS_Section)

reg-BundleSize							ENUMERATED {n2, n3, n6},

-- Precoder granularity in frequency domain.

-- Corresponds to L1 parameter 'CORESET-precoder-granuality' (see 38.211, sections 7.3.2.2 and 7.4.1.3.2)

precoderGranularity						ENUMERATED {sameAsREG-bundle, allContiguousRBs},

-- Corresponds to L1 parameter 'CORESET-interleaver-size' (see 38.211, 38.213, section FFS_Section)

interleaverSize							ENUMERATED {n2, n3, n6}														OPTIONAL,

-- Corresponds to L1 parameter 'CORESET-shift-index' (see 38.211, section 7.3.2.2)

shiftIndex								INTEGER(0..maxNrofPhysicalResourceBlocks-1)									OPTIONAL

},

nonInterleaved 							NULL

},

-- A subset of the TCI states defined in TCI-States used for providing QCL relationships between the DL RS(s) in one RS Set

-- (TCI-State) and the PDCCH DMRS ports. Corresponds to L1 parameter 'TCI-StatesPDCCH' (see 38.214, section FFS_Section)

tci-StatesPDCCH							SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId					OPTIONAL,

-- If at least spatial QCL is configured/indicated, this field indicates if TCI field is present or not present in DL-related DCI.

-- When the field is absent the UE considers the TCI to be absent/disabled.

-- Corresponds to L1 parameter 'TCI-PresentInDCI' (see 38,213, section 5.1.5)

tci-PresentInDCI						ENUMERATED {enabled}														OPTIONAL, 	-- Need S

-- PDCCH DMRS scrambling initalization. Corresponds to L1 parameter 'PDCCH-DMRS-Scrambling-ID' (see 38.214, section 5.1)

-- When the field is absent the UE applies the value '0'.

pdcch-DMRS-ScramblingID					BIT STRING (SIZE (16))											OPTIONAL 	-- Need S

}

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=			SEQUENCE {

drb						SEQUENCE {

discardTimer			ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} 													OPTIONAL, -- Cond Setup

pdcp-SN-Size-UL			ENUMERATED {len12bits, len18bits},

pdcp-SN-Size-DL			ENUMERATED {len12bits, len18bits},

headerCompression		CHOICE {

notUsed					NULL,

rohc					SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0001			BOOLEAN,

profile0x0002			BOOLEAN,

profile0x0003			BOOLEAN,

profile0x0004			BOOLEAN,

profile0x0006			BOOLEAN,

profile0x0101			BOOLEAN,

profile0x0102			BOOLEAN,

profile0x0103			BOOLEAN,

profile0x0104			BOOLEAN

},

drb-ContinueROHC			BOOLEAN

},

uplinkOnlyROHC			SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0006			BOOLEAN

},

drb-ContinueROHC			BOOLEAN

},

...

},

integrityProtection		BOOLEAN,

statusReportRequired	BOOLEAN																			OPTIONAL,	-- Cond Rlc-AM

outOfOrderDelivery		BOOLEAN

}																											OPTIONAL,	-- Cond DRB

-- FFS / TODO: Handle more than two secondary cell groups

moreThanOneRLC			SEQUENCE {

primaryPath				SEQUENCE {

cellGroup				CellGroupId,

logicalChannel			LogicalChannelIdentity

},

ul-DataSplitThreshold	SetupRelease {

ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}									OPTIONAL, -- Cond SplitBearer

pdcpDuplication			BOOLEAN

}																										OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering				ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300,	ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000}		OPTIONAL, -- Need R

...

}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Configfield descriptions

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC should be configured at reconfiguration involving PDCP re-establsihment if the RB was previously configured with ROHC.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. FFS: text to indicate where to find the key.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5] FFS: need to specify something with respect to UE capabilities.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 clause 5.2.1 for UL data tranmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the upliink, as specified in TS 38.323 [5]. For UL DRBs, the value shall be ignored by the UE.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0ms, value ms20 corresponds to 20ms, value ms40 corresponds to 40ms, and so on.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bits, value b100 corresponds to 100 bits, value b200 corresponds to 200 bits, and so on. Value Infinity corresponds to a path switch mode operation. FFS_FIXME: Clarify what happens upon “release”. And discuss need for value infinity. E.g. “If ul-DataSplitThreshold is set to release, the UL PDCP entity does not deliver data to RLC entities other than the “prioritizedRlc”

pdcpDuplication Indicates whether or not uplink duplication is activated. Set to FALSE in this version of the specification.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present for DRBs, not present for SRBs. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of an additional logical channel to the PDCP entity. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent and all its included parameters are released. |
| Rlc-AM | The field is mandatory present upon setup of a PDCP entity for a radio bearer configured with RLC AM. Otherwise, the field is optionally present, need M. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. Otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::= 						SEQUENCE {

-- Enables and configures code-block-group (CBG) based transmission (see 38.213, section 9.1.1)

codeBlockGroupTransmission				SetupRelease { SEQUENCE {

-- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1)

maxCodeBlockGroupsPerTransportBlock		ENUMERATED {n2, n4, n6, n8},

-- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see 38.212, section 7.3.1.2.2)

codeBlockGroupFlushIndicator			BOOLEAN

} }																														OPTIONAL,	-- Need M

-- Identifer used to initalite data scrambling (c_init) for both PDSCH.

-- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section)

-- FFS:_Replace by tye ScramblingId used in other places?

dataScramblingIdentityPDSCH			INTEGER (0..1007)															OPTIONAL,

dmrs-Downlink 						SetupRelease { DMRS-DownlinkConfig }

OPTIONAL,	-- Need M

-- Configures downlink PTRS .

-- If absent of released, the UE assumes that downlink PTRS are not present. See 38.214 section 5.1.6.3

phaseTrackingRS						SetupRelease { PTRS-DownlinkConfig	}										OPTIONAL,	-- Need M

-- A list of Transmission Configuration Indicator (TCI) states for dynamically indicating (over DCI) a transmission configuration

-- which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports

-- (see 38.214, section 5.1.4)

tci-StatesToAddModList					SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State					OPTIONAL,	-- Need N

tci-StatesToReleaseList					SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId						OPTIONAL,	-- Need N

-- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description.

-- Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2)

xOverhead								ENUMERATED { xOh0, xOh6, xOh12, xOh18 }									,

-- Interleaving unit configurable between 2 and 4 PRBs

-- Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section 6.3.1.6)

vrb-ToPRB-Interleaver					ENUMERATED {n2, n4}															,

-- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI

-- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 5.1.2)

resourceAllocation						ENUMERATED {

resourceAllocationType0					,

resourceAllocationType1					,

dynamicSwitch

}																													,

-- Resources that the UE should rate match PDSCH around.

rateMatchResourcesPDSCH					SEQUENCE {

-- Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources

-- indicated in the nexted bitmaps. Corresponds to L1 parameter 'Resource-set-BWP' (see 38.214, section 5.1.2.2.3)

-- FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary.

rateMatchPatternToAddModList

SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern

OPTIONAL, -- Need N

rateMatchPatternToReleaseList			SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, -- Need N

-- The IDs of a first group of RateMatchPatterns defined in the rateMatchPatternToAddModList.

-- Corresponds to L1 parameter 'Resource-set-group-1'. (see 38.214, section FFS_Section)

rateMatchPatternGroup1					SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, -- Need R

-- The IDs of a second group of RateMatchPatterns defined in the rateMatchPatternToAddModList

-- Corresponds to L1 parameter 'Resource-set-group-2'. (see 38.214, section FFS_Section)

rateMatchPatternGroup2					SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId		OPTIONAL, -- Need R

lte-CRS-ToMatchAround					SetupRelease {

SEQUENCE {

-- Center of the LTE carrier. Corresponds to L1 parameter 'center-subcarrier-location' (see 38.214, section 5.1.4)

carrierFreqDL							INTEGER (0..maxEARFCN),

-- BW of the LTE carrier in numbewr of PRBs. Corresponds to L1 parameter 'BW' (see 38.214, section 5.1.4)

carrierBandwidthDL						ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

-- LTE MBSFN subframe configuration. Corresponds to L1 parameter 'MBSFN-subframconfig' (see 38.214, section 5.1.4)

-- FFS_ASN1: Import the LTE MBSFN-SubframeConfigList

mbsfn-SubframeConfigList				OCTET STRING (CONTAINING MBSFN-SubframeConfigList)				OPTIONAL,	-- Need M

-- Number of LTE CRS antenna port to rate-match around.

-- Corresponds to L1 parameter 'rate-match-resources-numb-LTE-CRS-antenna-port' (see 38.214, section 5.1.4)

nrofCRS-Ports							ENUMERATED {n1, n2, n4},

-- Shifting value v-shift in LTE to rate match around LTE CRS

-- Corresponds to L1 parameter 'rate-match-resources-LTE-CRS-v-shift' (see 38.214, section 5.1.4)

v-Shift									ENUMERATED {n0, n1, n2, n3, n4, n5}			}

}																												OPTIONAL,	-- Need M

...

},

-- Selection between config 1 and config 2 for RBG size for PDSCH. Corresponds to L1 parameter 'RBG-size-PDSCH' (see 38.214, section 5.1.2.2.1)

rbg-Size								ENUMERATED {config1, config2},

-- Indicates which MCS table the UE shall use for PDSCH. Corresponds to L1 parameter 'MCS-Table-PDSCH' (see 38.214, section 5.1.3.1).

mcs-Table								ENUMERATED {qam64, qam256},

-- Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

maxNrofCodeWordsScheduledByDCI			ENUMERATED {n1, n2}															OPTIONAL, 	-- Need R

-- The number of HARQ processes to be used on the PDSCH of a serving cell.

-- Corresponds to L1 parameter 'number-HARQ-process-PDSCH' (see 38.214, section REF)

nrofHARQ-ProcessesForPDSCH				ENUMERATED {n2, n4, n6, n8, n10, n12, n16},

-- HARQ-ACK codebook is configured to be either semi-static of dynamic. This is applicable to both CA and none CA operation

-- Corresponds to L1 parameter 'HARQ-ACK-codebook' (see 38.213, section FFS_Section)

harq-ACK-Codebook						ENUMERATED {semiStatic, dynamic},

-- Bundle size the UE may assume. Corresponds to L1 paramter 'PDSCH-bundle-size' (see 38.211, section 7.3.1.5).

-- If the field is absent, the UE assumes value n2.

pdsch-BundleSize						ENUMERATED {n4, wideband}													OPTIONAL, 	-- Need S

-- Bundle size the UE may assume. Corresponds to L1 paramter 'PDSCH-bundle-size2' (see 38.211, section 7.3.1.5).

-- If the field is absent, the UE assumes value n2.

pdsch-BundleSize2						ENUMERATED {n4, wideband}													OPTIONAL, 	-- Need S

-- If set to true, the network indicates the PRB bundle size dynamically via DCI. Corresponds to L1 parameter 'PRB_bundling'

-- (see 38.214, section 5.1.2.3)

prb-BundlingEnabled					BOOLEAN,

-- A list of Zero-Power (ZP) CSI-RS resources.

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigList' (see 38.214, section FFS_Section)

aperiodic-ZP-CSI-RS-ResourceList						SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource		OPTIONAL,

...

}

-- Corresponds to L1 IE 'rate-match-PDSCH-resource-set'

RateMatchPattern ::= 						SEQUENCE {

rateMatchPatternId							RateMatchPatternId,

patternType									CHOICE {

bitmaps										SEQUENCE {

-- A resource block level bitmap in the frequency domain. It indicates the PRBs to which the symbolsInResourceBlock bitmap applies.

-- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap1' (see 38.214, section FFS_Section)

-- FFS_ASN1: Consider multiple options with different number of bits (for narrower carriers)

resourceBlocks								BIT STRING (SIZE (275)),

-- A symbol level bitmap in time domain. It indicates (FFS: with a bit set to true) the symbols which the UE shall rate match around.

-- This pattern recurs (in time domain) with the configured periodicityAndOffset.

-- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap2' (see 38.214, section FFS_Section)

-- FFS: Why not split it into two BIT STRINGs of 14 bit each?

symbolsInResourceBlock						CHOICE {

oneSlot										BIT STRING (SIZE (14)),

twoSlots									BIT STRING (SIZE (28))

},

-- A time domain repetition pattern. It determines the periodicity (FFS: And offset???) at which the symbolsInResourceBlock

-- pattern recurs. Absence of this field indicates the value n1, i.e., the symbolsInResourceBlock recurs every 14 symbols.

-- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap3' (see 38.214, section FFS_Section)

-- FFS: Doesn’t one require also an offset to configure from where the repetitions start?

periodicityAndOffset						CHOICE {

n1											NULL,

n2											INTEGER (0..1),

n4											INTEGER (0..3),

n5											INTEGER (0..4),

n8											INTEGER (0..7),

n10											INTEGER (0..9),

n20											INTEGER (0..19),

n40											INTEGER (0..39)

}																										OPTIONAL -- Need S

},

-- This ControlResourceSet us used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it.

controlResourceSet							ControlResourceSetId

},

-- The SubcarrierSpacing for this resource pattern. Corresponds to L1 parameter 'resource-pattern-scs' (see 38.214, section FFS_Section)

subcarrierSpacing							ENUMERATED {n0, n1, n2, n3, n4, n5},

-- FFS_Description, FFS_Section

mode										ENUMERATED { dynamic, semiStatic }

}

-- Corresponds to L1 parameter 'resource-set-index' (see 38.214, section 5.1.2.2.3)

RateMatchPatternId ::=						INTEGER (0..maxNrofRateMatchPatterns-1)

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

#### –	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=						SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

#### –	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=				SEQUENCE {

start							PhysCellId,

range							ENUMERATED {

n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,

spare1} 					OPTIONAL	-- Need OP

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

#### –	PCI-RangeIndex

The IE PCI-RangeIndex identifies of physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-START

PCI-RangeIndex ::=						INTEGER (0..maxNrofPCI-Ranges)

-- TAG-PCI-RANGE-INDEX-STOP

-- ASN1STOP

#### –	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indices of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-LIST-START

PCI-RangeIndexList ::=						SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-Range-INDEX-LIST-STOP

-- ASN1STOP

#### –	PhysCellId

The PhysCellId identifies the physical cell identity (PCI).

PhysCellId information element

-- ASN1START

-- TAG-PHYS-CELL-ID-START

PhysCellId ::=						INTEGER (0..1007)

-- TAG-PHYS-CELL-ID-STOP

-- ASN1STOP

#### –	PRB-Id

The PRB-Id indentifies a Physical Resource Block (PRB) position within a carrier.

PRB-Id information element

-- ASN1START

-- TAG-PRB-ID-START

PRB-Id ::=						INTEGER (0..maxNrofPhysicalResourceBlocks-1)

-- TAG-PRB-ID-STOP

-- ASN1STOP

#### –	PTRS-DownlinkConfig

The IE PTRS-DownlinkConfig is used to configure downlink phase tracking reference signals (PTRS) (see 38.214 section5.1.6.3)

PTRS-DownlinkConfig information element

-- ASN1START

-- TAG-PTRS-DOWNLINKCONFIG-START

PTRS-DownlinkConfig ::= 					SEQUENCE {

-- Presence and  frequency density of DL PT-RS as a function of Scheduled BW

-- Corresponds to L1 parameter 'DL-PTRS-frequency-density-table' (see 38.214, section 5.1)

-- FFS: To be Configured  per BWP according to RAN1

frequencyDensity							SEQUENCE (SIZE (2)) OF INTEGER (1..276),

-- Presence and time density of DL PT-RS  as a function of MCS

-- Corresponds to L1 parameter 'DL-PTRS-time-density-table' (see 38.214, section 5.1)

timeDensity									SEQUENCE (SIZE (4)) OF INTEGER (0..28),

-- Indicates the number of DL PTRS ports. This is equal or smaller than the number of DMRS groups (related to PDSCH parameters

-- dmrs-group1 and dmrs-group2). Corresponds to L1 parameter 'DL-PTRS-ports' (see 38.214, section 5.1)

nrofPorts									ENUMERATED {n1, n2},

-- EPRE ratio between PTRS and PDSCH. Value 0 correspond to the codepoint ”00” in table 4.1-2. Value 1 corresponds to codepoint ”01”

-- Corresponds to L1 parameter 'DL-PTRS-EPRE-ratio' (see 38.214, section 4.1)

epre-Ratio									INTEGER (0..3)																OPTIONAL,

-- Indicates the subcarrier offset for DL PTRS. Corresponds to L1 parameter 'DL-PTRS-RE-offset' (see 38.214, section 5.1.6.3)

resourceElementOffset						ENUMERATED { offset00, offset01, offset10, offset11 }							,

...

}

-- TAG-PTRS-DOWNLINKCONFIG-STOP

-- ASN1STOP

#### –	PTRS-UplinkConfig

The IE PTRS-UplinkConfig is used to configure uplink Phase-Tracking-Reference-Signals (PTRS).

PTRS-UplinkConfig information element

-- ASN1START

-- TAG-PTRS-UPLINKCONFIG-START

PTRS-UplinkConfig ::= 					SEQUENCE {

-- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices

-- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1)

-- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM

-- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts?

-- And if so, should this structure be conditional to the nrofPorts being set to n2?

-- FFS_CHECK: Is this supposed to be a list with the length of the configured SRS resources? If so, why don't we put this field into the

-- SRS-Resource?

srs-MappingListNonCodebook				SEQUENCE (SIZE (1..maxNrofSRS-Resources)) OF SEQUENCE {

srs

SRS-ResourceId,

ptrs-PortIndex							ENUMERATED {n0, n1}

}																														OPTIONAL,	-- Need M

modeSpecificParameters					CHOICE {

-- Configuration of UL PTRS for CP-OFDM

cp-OFDM									SEQUENCE {

-- Presence and  frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW

-- Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1)

-- FFS: Configuration is supposed to be per BWP according to RAN1

frequencyDensity						SEQUENCE (SIZE (2)) OF INTEGER (1..276),

-- Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS

-- Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1)

-- FFS: Configuration is supposed to be per BWP according to RAN1

timeDensity								SEQUENCE (SIZE (4)) OF INTEGER (0..29),

-- Indicator related to the number of UL PTRS ports for CP-OFDM. Details to be further decided.

-- Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.1)

nrofPorts								ENUMERATED {n1, n2},

-- Indicates the subcarrier offset for UL PTRS for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1)

resourceElementOffset					ENUMERATED { offset00, offset01, offset10, offset11 }			,

-- UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1)

ptrs-Power								ENUMERATED {f1, f2, f3, f4}

},

-- Configuration of UL PTRS for DFT-S-OFDM.

dft-S-OFDM								SEQUENCE {

-- Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4},

-- that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should

-- use depending on the scheduled BW according to the table in 38.214 FFS_Section.

-- FFS: Configuration is supposed to be per BWP according to RAN1

-- Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1)

sampleDensity						SEQUENCE (SIZE (5)) OF INTEGER (0..28)									OPTIONAL,

-- Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM

-- Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1)

timeDensity							ENUMERATED {d1, d2}														OPTIONAL

}

}																													OPTIONAL, -- Cond M

...

}

-- TAG-PTRS-UPLINKCONFIG-STOP

-- ASN1STOP

#### –	PUCCH-Config

The PUCCH-Config IE is used to configure the UE specific PUCCH parameters.

PUCCH-Config information element

-- ASN1START

-- TAG-PUCCH-CONFIG-START

PUCCH-ConfigCommon ::=					SEQUENCE {

-- An entry into a 16-row table where each row configures a set of cell-specific PUCCH resources/parameters

-- Corresponds to L1 parameter 'PUCCH-resource-common' (see 38.213, section 9.2)

-- FFS_CHECK: Is this configuration REPLACED by the PUCCH-Config?

-- FFS_CHECK: Can one say that this is applied on the initial Search Space (ID=0) and initial CORESET (ID=0)

pucch-ResourceCommon					BIT STRING (SIZE (4))															OPTIONAL,

-- Configuration of group- and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. “neither” implies neither group

-- or sequence hopping is enabled. “enable” enables group hopping and disables sequence hopping. “disable” disables group

-- hopping and enables sequence hopping. Corresponds to L1 parameter 'PUCCH-GroupHopping' (see 38.211, section 6.4.1.3)

pucch-GroupHopping						ENUMERATED { neither, enable, disable },

-- Cell-Specific scrambling ID for group hoppping and sequence hopping if enabled.

-- Corresponds to L1 parameter 'ScramblingID' (see 38.211, section 6.4.1.3)

hoppingId								BIT STRING (SIZE (10))															OPTIONAL,

-- Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed.

-- Corresponds to L1 parameter 'p0-nominal-pucch' (see 38.213, section 7.2)

p0-nominal								INTEGER (-202..24)																OPTIONAL,

-- deltaF for PUCCH format 0 (see 38.213, section 7.2)

deltaF-PUCCH-f0							FFS_Value																		OPTIONAL,

-- deltaF for PUCCH format 1 (see 38.213, section 7.2)

deltaF-PUCCH-f1							FFS_Value																		OPTIONAL,

-- deltaF for PUCCH format 2 (see 38.213, section 7.2)

deltaF-PUCCH-f2							FFS_Value																		OPTIONAL,

-- deltaF for PUCCH format 3 (see 38.213, section 7.2)

deltaF-PUCCH-f3							FFS_Value																		OPTIONAL,

-- deltaF for PUCCH format 4 (see 38.213, section 7.2)

deltaF-PUCCH-f4							FFS_Value																		OPTIONAL,

...

}

PUCCH-Config ::= 						SEQUENCE {

-- Lists for adding and releasing PUCCH resource sets (see 38.213	, section 9.2)

resourceSetToAddModList					SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet				OPTIONAL,	-- Need N

resourceSetToReleaseList				SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId			OPTIONAL,	-- Need N

-- Parameters that are common for all PUCCH resources of format 1

format1									SetupRelease { SEQUENCE {

-- Enabling inter-slot frequency hopping when PUCCH Format 1 is repetead over multiple slots.

interslotFrequencyHopping				ENUMERATED {enabled}														OPTIONAL,	-- Need R

-- Number of slots with the same PUCCH F1. When the field is absent the UE applies the value n1.

-- Corresponds to L1 parameter 'PUCCH-F1-number-of-slots' (see 38.213, section 9.2)

-- FFS_Value: Undefined values y1-y3 in range!

nrofSlots								ENUMERATED {n1,ny1,y2,y3}

} }																														OPTIONAL,	-- Need M

-- Parameters that are common for all PUCCH resources of format 2

format2									SetupRelease { SEQUENCE {

-- Maximum coding rate to determine how to feedback UCI on PUCCH Format 2.

-- Corresponds to L1 parameter 'PUCCH-F2-maximum-coderate' (see 38.213, section 9.2)

maxCodeRate											PUCCH-MaxCodeRate,

-- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format2

-- Corresponds to L1 parameter 'PUCCH-F2-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2)

-- When the field is absent the UE applies the value OFF

simultaneousHARQ-ACK-CSI				ENUMERATED {true}															OPTIONAL	-- Need R

} }																														OPTIONAL,	-- Need M

-- Parameters that are common for all PUCCH resources of format 3

format3									SetupRelease { SEQUENCE {

-- Enabling inter-slot frequency hopping when PUCCH Format 3 is repetead over multiple slots.

interslotFrequencyHopping				ENUMERATED {enabled}														OPTIONAL,	-- Need R

-- Enabling 2 DMRS symbols per hop of a PUCCH Format 3 if both hops are more than X symbols when FH is enabled (X=4).

-- Enabling 4 DMRS sybmols for a PUCCH Format 3 with more than 2X+1 symbols when FH is disabled (X=4).

-- Corresponds to L1 parameter 'PUCCH-F3-F4-additional-DMRS' (see 38.21X, section FFS_Section)

additionalDMRS							ENUMERATED {true}															OPTIONAL,	-- Need R

-- Max coding rate to determine how to feedback UCI on PUCCH Format 3

-- Corresponds to L1 parameter 'PUCCH-F3-maximum-coderate' (see 38.213, section 9.2)

maxCodeRate												PUCCH-MaxCodeRate,

-- Number of slots with the same PUCCH F3. When the field is absent the UE applies the value n1.

-- Corresponds to L1 parameter 'PUCCH-F3-number-of-slots' (see 38.213, section 9.2)

-- FFS_Value: Undefined values y1-y3 in range!

nrofSlots								ENUMERATED {n1,y1,y2,y3}													,

-- Enabling pi/2 BPSK for UCI symbols instead of QPSK for PUCCH.

-- Corresponds to L1 parameter 'PUCCH-PF3-PF4-pi/2PBSK' (see 38.21X, section FFS_Section)

pi2PBSK									ENUMERATED {enabled}														OPTIONAL,	-- Need R

-- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format3

-- Corresponds to L1 parameter 'PUCCH-F3-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2)

-- When the field is absent the UE applies the value OFF

simultaneousHARQ-ACK-CSI				ENUMERATED {true}															OPTIONAL	-- Need R

} }																														OPTIONAL,	-- Need M

-- Parameters that are common for all PUCCH resources of format 4

format4									SetupRelease { SEQUENCE {

-- Enabling inter-slot frequency hopping when PUCCH Format 4 is repetead over multiple slots.

interslotFrequencyHopping				ENUMERATED {enabled}														OPTIONAL, 	-- Need R

-- Enabling 2 DMRS symbols per hop of a PUCCH Format 4 if both hops are more than X symbols when FH is enabled (X=4).

-- Enabling 4 DMRS sybmols for a PUCCH Format 4 with more than 2X+1 symbols when FH is disabled (X=4).

-- Corresponds to L1 parameter 'PUCCH-F3-F4-additional-DMRS' (see 38.21X, section FFS_Section)

additionalDMRS							ENUMERATED {true}															OPTIONAL,	-- Need R

-- Max coding rate to determine how to feedback UCI on PUCCH Format 4

-- Corresponds to L1 parameter 'PUCCH-F4-maximum-coderate' (see 38.213, section 9.2)

maxCodeRate												PUCCH-MaxCodeRate,

-- Number of slots with the same PUCCH F4. When the field is absent the UE applies the value n1.

-- Corresponds to L1 parameter 'PUCCH-F4-number-of-slots' (see 38.213, section 9.2)

-- FFS_Value: Undefined values y1-y3 in range!

nrofSlots								ENUMERATED {n1,y1,y2,y3}													,

-- Enabling pi/2 BPSK for UCI symbols instead of QPSK for PUCCH.

-- Corresponds to L1 parameter 'PUCCH-PF3-PF4-pi/2PBSK' (see 38.21X, section FFS_Section)

pi2PBSK									ENUMERATED {enabled}														OPTIONAL, 	-- Need R

-- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format4

-- Corresponds to L1 parameter 'PUCCH-F4-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2)

-- When the field is absent the UE applies the value OFF

simultaneousHARQ-ACK-CSI				ENUMERATED {true}															OPTIONAL	-- Need R

} }																														OPTIONAL,	-- Need M

schedulingRequestResourceToAddModList

SEQUENCE (SIZE (1..maxNrofSR-Resoruces)) OF SchedulingRequestResourceConfig

OPTIONAL, -- Need M

schedulingRequestResourceToReleaseList		SEQUENCE (SIZE (1..maxNrofSR-Resoruces)) OF SchedulingRequestResourceId		OPTIONAL, -- Need M

-- Scambling ID for PUCCH. Corresponds to L1 parameter 'ScramblingID' (see 38.213, section FFS_Section)

-- FFS_DefaultValue: At other occasions the default value is supposed to be the UE ID. Not for SRS?

scramblingID							ScramblingId																	OPTIONAL,

-- Configuration of the spatial relation between a reference RS and PUCCH. Reference RS can be SSB/CSI-RS/SRS.

-- If the list has more than one element, MAC-CE selects a single element.

-- FFS: How does the MAC CE refer to these spatialRelationInfo entries... any why?

-- Corresponds to L1 parameter 'PUCCH-SpatialRelationInfo' (see 38.213, section FFS_Section)

spatialRelationInfoToAddModList			SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF

PUCCH-SpatialRelationInfo	OPTIONAL,	-- Need N

spatialRelationInfoToReleaseList		SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId	OPTIONAL,	-- Need N

pucch-PowerControl						PUCCH-PowerControl																OPTIONAL,	-- Need M

...

}

PUCCH-MaxCodeRate ::= 						ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}

PUCCH-SpatialRelationInfo ::=				SEQUENCE {

pucch-SpatialRelationInfoId					PUCCH-SpatialRelationInfoId,

referenceSignal 							CHOICE {

ssb-Index									SSB-Index,

csi-RS-Index								NZP-CSI-RS-ResourceId,

srs											SRS-ResourceId

}

}

PUCCH-SpatialRelationInfoId ::= 			INTEGER (1..maxNrofSpatialRelationInfos)

-- A set with one or more PUCCH resources

PUCCH-ResourceSet ::=							SEQUENCE {

pucch-ResourceSetId								PUCCH-ResourceSetId,

-- Between 4 and 8 PUCCH resources. PUCCH resources of format0 and format1 are only allowed in the first PUCCH reosurce set,

-- i.e., in a PUCCH-ResourceSet with pucch-ResourceSetId = 0. PUCCH resources of format2, format3 and format4 are only allowed

-- in a PUCCH-ReosurceSet with pucch-ResourceSetId > 0.

-- The UE chooses a PUCCH-Resource from this list based on the 2-bit ARI field in DCI as speciied in 38.213, FFS_section.

-- FFS_ASN1: Consider converting to a AddMod/Release List

resources										SEQUENCE (SIZE (4..maxNrofPUCCH-ResourcesPerSet)) OF PUCCH-Resource,

-- Maximum number of payload bits minus 1 that the UE may transmit using this PUCCH resource set. In a PUCCH occurrence, the UE

-- chooses the first of its PUCCH-ResourceSet which supports the number of bits that the UE wants to transmit.

-- The field is not present in the first set (Set0) since the maximum Size of Set0 is specified to be 3 bit.

-- The field is not present in the last configured set since the UE derives its maximum payload size as specified in 38.213.

-- This field can take integer values that are multiples of 4. Corresponds to L1 parameter 'N_2' or 'N_3' (see 38.213, section 9.2)

maxPayloadMinus1								INTEGER (4..256)

}

PUCCH-ResourceSetId ::=							INTEGER (0..maxNrofPUCCH-ResourceSets-1)

PUCCH-Resource ::= 								SEQUENCE {

pucch-ResourceId								PUCCH-ResourceId,

startingPRB										PRB-Id,

-- Corresponds to the L1 parameter 'PUCCH-frequency-hopping' (see 38.213, section 9.2)

intraSlotFrequencyHopping						ENUMERATED { enabled }													OPTIONAL,	-- Need R

-- Index of starting PRB for second hop of PUCCH in case of FH. This value is appliable for intra-slot frequency hopping.

-- Corresponds to L1 parameter 'PUCCH-2nd-hop-PRB' (see 38.213, section 9.2)

secondHopPRB									PRB-Id																	OPTIONAL,

-- Selection of the PUCCH format and format-specific parameters

format											CHOICE {

format0											PUCCH-format0,														-- Cond InFirstSetOnly

format1											PUCCH-format1,														-- Cond InFirstSetOnly

format2											PUCCH-format2,														-- Cond NotInFirstSet

format3											PUCCH-format3,														-- Cond NotInFirstSet

format4											PUCCH-format4														-- Cond NotInFirstSet

}

}

PUCCH-ResourceId ::=							INTEGER (0..maxNrofPUCCH-ResourcesPerSet-1)

-- A PUCCH Format 0 resource configuration (see 38.213, section 9.2)

-- Corresponds to L1 parameter 'PUCCH-F0-resource-config' (see 38.213, section 9.2)

PUCCH-format0 ::=								SEQUENCE {

initialCyclicShift								INTEGER(0..11),

nrofSymbols										INTEGER (1..2),

startingSymbolIndex								INTEGER(0..13)

}

-- A PUCCH Format 1 resource configuration (see 38.213, section 9.2)

-- Corresponds to L1 parameter 'PUCCH-F1-resource-config' (see 38.213, section 9.2)

PUCCH-format1 ::= 								SEQUENCE {

initialCyclicShift								INTEGER(0..11),

nrofSymbols										INTEGER (4..14),

startingSymbolIndex								INTEGER(0..10),

timeDomainOCC									INTEGER(0..6)

}

-- A PUCCH Format 2 resource configuration (see 38.213, section 9.2)

-- Corresponds to L1 parameter 'PUCCH-F2-resource-config' (see 38.213, section 9.2)

PUCCH-format2 ::= 								SEQUENCE {

nrofPRBs										INTEGER (1..16),

nrofSymbols										INTEGER (1..2),

startingSymbolIndex								INTEGER(0..13)

}

-- A PUCCH Format 3 resource configuration(see 38.213, section 9.2)

-- Corresponds to L1 parameter 'PUCCH-F3-resource-config' (see 38.213, section 9.2)

PUCCH-format3 ::= 								SEQUENCE {

-- The supported values are 1,2,3,4,5,6,8,9,10,12,15 and 16

nrofPRBs										INTEGER (1..16),

nrofSymbols										INTEGER (4..14),

startingSymbolIndex								INTEGER(0..10)

}

-- A PUCCH Format 4 resource configuration (see 38.213, section 9.2)

-- Corresponds to L1 parameter 'PUCCH-F4-resource-config' (see 38.213, section 9.2)

PUCCH-format4 ::= 								SEQUENCE {

nrofSymbols										INTEGER (4..14),

occ-Length										ENUMERATED {n2,n4},

occ-Index										ENUMERATED {n0,n1,n2,n3},

startingSymbolIndex								INTEGER(0..10)

}

PUCCH-PowerControl ::= 				SEQUENCE {

-- RNTI used for PUCCH TPC.	Corresponds to L1 parameter 'TPC-PUCCH-RNTI' (see 38.213, section 10).

-- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity

-- and other PDCCH parameters (if any)

tpc-PUCCH-RNTI						RNTI-Value																				OPTIONAL, -- Need M

-- A set with dedicated P0 values for PUCCH, i.e.,  {P01, P02,... }. Corresponds to L1 parameter 'p0-pucch-set' (see 38.213, section 7.2)

p0-Set								SEQUENCE (SIZE (1..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH									OPTIONAL, -- Need M

-- A set of Reference Signals (e.g. a CSI-RS config or a SSblock) to be used for PUCCH pathloss estimation.

-- Up to maxNrofPUCCH-PathlossReference-RSs may be configured

-- FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB?

-- Corresponds to L1 parameter 'pucch-pathlossReference-rs-config' (see 38.213, section 7.2)

pathlossReferenceRSs				SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReferenceRSs)) OF PUCCH-PathlossReferenceRS	OPTIONAL, -- Need M

-- Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains

-- two power control states (i.e., g(i,0) and g(i,1)). Otherwise, it applies one (i.e., g(i,0)).

-- Corresponds to L1 parameter 'num-pucch-pcadjustment-states' (see 38.213, section 7.2)

twoPUCCH-PC-AdjustmentStates		ENUMERATED {twoStates}																	OPTIONAL, -- Need R

...

}

-- P0 value for PUCCH. Corresponds to L1 parameter 'p0-pucch' (see 3,213, section 7.2)

P0-PUCCH ::=							FFS_Value

-- A reference signal (RS) configured as pathloss reference signal for PUCCH power control

-- Corresponds to L1 parameter 'pucch-pathlossReference-rs' (see 38.213, section 7.2)

PUCCH-PathlossReferenceRS ::=					SEQUENCE {

pucch-PathlossReferenceRS-Id 				PUCCH-PathlossReferenceRS-Id,

referenceSignal								CHOICE {

ssb-Index									SSB-Index,

csi-RS-Index								NZP-CSI-RS-ResourceId

}

}

-- ID for a referemce signal (RS) configured as PUCCH pathloss reference

-- Corresponds to L1 parameter 'pucch-pathlossreference-index' (see 38.213, section 7.2)

-- FFS_CHECK: Is this ID used anywhere except inside the PUCCH-PathlossReference-RS	itself? If not, remove.

PUCCH-PathlossReferenceRS-Id ::=			INTEGER (0..maxNrofPUCCH-PathlossReferenceRSs-1)

-- TAG-PUCCH-CONFIG-STOP

-- ASN1STOP

#### –	PUSCH-Config

The IE PUSCH-Config IE is used to configure the UE specific PUSCH parameters. The IE PUSCH-ConfigCommon IE is used to configure the cell specific PUSCH parameters.

PUSCH-Config information element

-- ASN1START

-- TAG-PUSCH-CONFIG-START

PUSCH-ConfigCommon ::= 						SEQUENCE {

-- Sequence-group hopping can be enabled or disabled by means of this cell-specific parameter.

-- Corresponds to L1 parameter 'Group-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section)

-- This field is Cell specific

groupHoppingEnabledTransformPrecoding		ENUMERATED {enabled}													OPTIONAL,	-- Need R

-- ------------------------

-- Power control parameters

-- Power offset between msg3 and RACH preamble transmission. Corresponds to L1 parameter 'Delta-preamble-msg3' (see 38.213, section 7.1)

msg3-DeltaPreamble							FFS_Value																OPTIONAL,	-- Need R

-- P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed.

-- Corresponds to L1 parameter 'p0-nominal-pusch-withgrant' (see 38.213, section 7.1)

-- This field is cell specific

p0-NominalWithGrant							INTEGER (-202..24)														OPTIONAL,	-- Need R

...

}

PUSCH-Config ::= 						SEQUENCE {

-- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref

codeBlockGroupTransmission				SetupRelease { SEQUENCE {

-- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref)

-- For 2 codewords, only the values { n2, n4 } are valid.

maxCodeBlockGroupsPerTransportBlock		ENUMERATED {n2, n4, n6, n8},

...

}	}																													OPTIONAL,	-- Need M

-- Identifer used to initalite data scrambling (c_init) for both PUSCH.

-- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section)

-- FFS: Replace by tye ScramblingId used in other places?

dataScramblingIdentityPUSCH			INTEGER (0..1007)															OPTIONAL,

-- Whether UE uses codebook based or non-codebook based transmission. Corresponds to L1 parameter 'ulTxConfig' (see 38.214, section 6.1.1)

txConfig								ENUMERATED {codebook, nonCodebook},

dmrs-Uplink 							DMRS-UplinkConfig

OPTIONAL,	-- Need M

pusch-PowerControl						PUSCH-PowerControl															OPTIONAL, -- Need M

-- Configured one of two supported frequency hopping mode. If not configured frequency hopping is not configured

-- Corresponds to L1 parameter 'Frequency-hopping-PUSCH' (see 38.214, section 6)

-- When the field is absent the UE applies the value Not configured

frequencyHopping						ENUMERATED {mode1, mode2}													OPTIONAL,	-- Need S

-- Enables LBRM (Limited buffer rate-matching).

-- When the field is absent the UE applies FBRM (Full buffer rate-matchingLBRM)

-- Corresponds to L1 parameter 'LBRM-FBRM-selection' (see 38.212, section 5.4.2)

rateMatching							ENUMERATED {limitedBufferRM}									OPTIONAL,	-- Need S

-- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI

-- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 6.1.2)

resourceAllocation						ENUMERATED {

resourceAllocationType0					,

resourceAllocationType1					,

dynamicSwitch

}																													,

-- Indicates which MCS table the UE shall use for PUSCH without transform precoder

-- Corresponds to L1 parameter 'MCS-Table-PUSCH' (see 38.214, section 6.1.4)

-- When the field is absent the UE applies the value 64QAM

mcs-Table								ENUMERATED {qam64, qam256}													OPTIONAL,	-- Need S

-- Indicates which MCS table the UE shall use for PUSCH with transform precoding

-- Corresponds to L1 parameter 'MCS-Table-PUSCH-transform-precoding' (see 38.214, section 6.1.4)

-- When the field is absent the UE applies the value 64QAM

mcs-TableTransformPrecoder				ENUMERATED { qam256}													OPTIONAL,	-- Need S

-- The UE specific selection of transformer precoder for PUSCH. When the field is absent the UE applies the value msg3-tp.

-- Corresponds to L1 parameter 'PUSCH-tp' (see 38.211, section 6.3.1.4)

transformPrecoder						ENUMERATED {enabled, disabled}												OPTIONAL,	-- Need S

-- Selection between config 1 and config 2 for RBG size for PUSCH. When the field is absent the UE applies the value config1.

-- Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1)

rbg-Size								ENUMERATED { config2}												OPTIONAL,	-- Need S

-- Selection between and configuration of dynamic and semi-static beta-offset.

-- If the field is absent or released, the UE applies the value 'semiStatic' and the BetaOffsets according to

-- FFS [BetaOffsets and/or section 9.x.x).

-- Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.213, section 9.3)

uci-OnPUSCH							SetupRelease { CHOICE {

dynamic									SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic								BetaOffsets

}	}																												OPTIONAL, -- Need M

-- Indicates a scaling factor to limit the number of resource elements assigned to UCI on PUSCH.

-- Value f0p5 corresponds to 0.5, value f0p65 corresponds to 0.65, and so on.

-- Corresponds to L1 parameter 'uci-on-pusch-scaling' (see 38.212, section 6.3)

uci-OnPUSCH-Scaling						ENUMERATED { f0p5, f0p65, f0p8, f1 },

-- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description.

-- Corresponds to L1 parameter 'Xoh-PUSCH' (see 38.214, section 5.1.3.2)

xOverhead								ENUMERATED {xoh0, xoh6, xoh12, xoh18}										OPTIONAL,

-- Interleaving unit configurable between 2 and 4 PRBs

-- Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section 6.3.1.6)

vrb-ToPRB-Interleaver					ENUMERATED {n2, n4},

-- Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2

-- Corresponds to L1 parameter 'Frequency-hopping-offsets-set' (see 38.214, section 6.3)

frequencyHoppingOffsets					SEQUENCE (SIZE (1..4)) OF INTEGER (1.. maxNrofPhysicalResourceBlocks-1)				OPTIONAL,

...

}

-- TAG-PUSCH-CONFIG-STOP

-- ASN1STOP

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::= 						SEQUENCE {

-- RNTI used for PUSCH TPC. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10)

-- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity

-- and other PDCCH parameters (if any)

tpc-PUSCH-RNTI								RNTI-Value																OPTIONAL,

-- If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation.

-- If absent, TPC accumulation is enabled. Corresponds to L1 parameter 'Accumulation-enabled' (see 38.213, section 7.1)

tpc-Accumulation							ENUMERATED { disabled }													OPTIONAL,	-- Need R

-- Dedicated alpha value for msg3 PUSCH. Corresponds to L1 parameter 'alpha-ue-pusch-msg3' (see 38.213, section 7.1)

-- When the field is absent the UE applies the value 1.

msg3-Alpha									Alpha																	OPTIONAL, 	-- Need S

-- P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed.

-- Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1)

p0-NominalWithoutGrant						INTEGER (-202..24)														OPTIONAL,	-- Need M,

-- configuration {p0-pusch,alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},…}.

-- Corresponds to L1 parameter 'p0-push-alpha-setconfig' (see 38,213, section 7.1)

p0-AlphaSets								SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet		OPTIONAL,	-- Need M,

-- A set of Reference Signals (e.g. a CSI-RS config or a SSblock) to be used for PUSCH path loss estimation.

-- Up to maxNrofPUSCH-PathlossReferenceRSs may be configured when 'PUSCH beam indication' is present (FFS: in DCI???).

-- Otherwise, there may be only one entry.

-- Corresponds to L1 parameter 'pusch-pathlossReference-rs-config' (see 38.213, section 7.1)

pathlossReferenceRSToAddModList				SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL,	-- Need N

pathlossReferenceRSToReleaseList			SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,	-- Need N

-- Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains

-- two power control states (i.e., fc(i,1) and fc(i,2)). Otherwise, it applies one (i.e., fc(i,1)).

-- Corresponds to L1 parameter 'num-pusch-pcadjustment-states' (see 38.213, section 7.1)

twoPUSCH-PC-AdjustmentStates				ENUMERATED {twoStates}													OPTIONAL, -- Need S

-- Indicates whether to apply dela MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH.

-- Corresponds to L1 parameter 'deltaMCS-Enabled' (see 38.213, section 7.1)

deltaMCS									ENUMERATED {enabled}													OPTIONAL -- Need S

}

-- A set of p0-pusch and alpha used for PUSCH with grant. 'PUSCH beam indication' (if present) gives the index of the set to

-- be used for a particular PUSCH transmission.

-- FFS_CHECK: Is the ”PUSCH beam indication” in DCI which schedules the PUSCH? If so, clarify in field description

-- Corresponds to L1 parameter 'p0-pusch-alpha-set' (see 38.213, section 7.1)

P0-PUSCH-AlphaSet ::= 						SEQUENCE {

p0-PUSCH-AlphaSetId 						P0-PUSCH-AlphaSetId,

-- P0 value for PUSCH with grant (except msg3). Corresponds to L1 parameter 'p0-pusch' (see 38,213, section 7.1)

p0											FFS_Value																OPTIONAL,

-- alpha value for PUSCH with grant (except msg3) (see 38.213, section 7.1)

-- When the field is absent the UE applies the value 1

alpha 										Alpha																	OPTIONAL	-- Need S

}

-- ID for a P0-PUSCH-AlphaSet. Corresponds to L1 parameter 'p0alphasetindex' (see 38.213, section 7.1)

P0-PUSCH-AlphaSetId ::= 					INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

-- A reference signal (RS) configured as pathloss reference signal for PUSCH power control

-- Corresponds to L1 parameter 'pusch-pathlossReference-rs' (see 38.213, section 7.1)

PUSCH-PathlossReferenceRS ::=				SEQUENCE {

pusch-PathlossReferenceRS-Id 				PUSCH-PathlossReferenceRS-Id,

referenceSignal								CHOICE {

ssb-Index									SSB-Index,

csi-RS-Index									NZP-CSI-RS-ResourceId

}

}

-- ID for a referemce signal (RS) configured as PUSCH pathloss reference

-- Corresponds to L1 parameter 'pathlossreference-index' (see 38.213, section 7.1)

-- FFS_CHECK: Is this ID used anywhere except inside the PUSCH-PathlossReference-RS	itself?

PUSCH-PathlossReferenceRS-Id ::=			INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

-- A set of beta-offset values

BetaOffsets ::= 							SEQUENCE {

-- Up to 2 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-1' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 11

betaOffsetACK-Index1						INTEGER(0..31)															OPTIONAL, -- Need S

-- Up to 11 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-2' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 11

betaOffsetACK-Index2						INTEGER(0..31)															OPTIONAL, -- Need S

-- Above 11 bits HARQ-ACK. Corresponds to L1 parameter 'betaOffset-ACK-Index-3' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 11

betaOffsetACK-Index3						INTEGER(0..31)															OPTIONAL, -- Need S

-- Up to 11 bits of CSI part 1 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-1-Index-1' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 13

betaOffsetCSI-Part1-Index1						INTEGER(0..31)															OPTIONAL, -- Need S

-- Above 11 bits of CSI part 1 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-1-Index-2' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 13

betaOffsetCSI-Part1-Index2						INTEGER(0..31)															OPTIONAL, -- Need S

-- Up to 11 bits of CSI part 2 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-2-Index-1' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 13

betaOffsetCSI-Part2-Index1						INTEGER(0..31)															OPTIONAL, -- Need S

-- Above 11 bits of CSI part 2 bits. Corresponds to L1 parameter 'betaOffset-CSI-part-2-Index-2' (see 38.213, section 9.3)

-- When the field is absent the UE applies the value 13

betaOffsetCSI-Part2-Index2						INTEGER(0..31)															OPTIONAL -- Need S

}

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

#### –	Q-OffsetRange

The IE Q-OffsetRange is used to indicate a cell, beam or frequency specific offset to be applied when evaluating candidates for cell re-selection or when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, dB-22 corresponds to -22 dB and so on.

Q-OffsetRange information element

-- ASN1START

Q-OffsetRange ::=						ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- ASN1STOP

Editor’s Note: FFS Confirm the exact values that are supported.

#### –	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITY-CONFIG-START

QuantityConfig ::=							SEQUENCE {

quantityConfigNR-List						QuantityConfigNR-List												OPTIONAL, 	-- Need M

...

}

QuantityConfigNR-List::=						SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR

QuantityConfigNR::=							SEQUENCE {

quantityConfigCell							QuantityConfigRS,

quantityConfigRS-Index						QuantityConfigRS													OPTIONAL 	-- Need M

}

QuantityConfigRS ::=						SEQUENCE {

-- SS Block based

ssb-FilterCoefficientRSRP					FilterCoefficient													DEFAULT ffsTypeAndValue,

ssb-FilterCoefficientRSRQ					FilterCoefficient													DEFAULT ffsTypeAndValue,

ssb-FilterCoefficientRS-SINR					FilterCoefficient												    DEFAULT ffsTypeAndValue,

-- CSI-RS based

csi-	rs-FilterCoefficientRSRP				FilterCoefficient													DEFAULT ffsTypeAndValue,

csi-	rs-FilterCoefficientRSRQ				FilterCoefficient													DEFAULT ffsTypeAndValue,

csi-rs-FilterCoefficientRS-SINR				FilterCoefficient												    DEFAULT ffsTypeAndValue

}

-- TAG-QUANTITY-CONFIG-STOP

-- ASN1STOP

QuantityConfig field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigNR Specifies filter configurations for NR measurements.

quantityConfigRSindex Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

ssb-FilterCoefficientRSRP Specifies L3 filter configurations for SS-RSRP measurement results from the L1 filter(s), as defined in 38.215 [9].

ssb-FilterCoefficientRSRQ Specifies L3 filter configurations for SS-RSRQ measurement results from the L1 filter(s), as defined in 38.215 [9].

ssb-FilterCoefficientSINR Specifies L3 filter configurations for SS-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

csi-rs-FilterCoefficientRSRP Specifies L3 filter configurations for CSI-RSRP measurement results from the L1 filter(s), as defined in 38.215 [9].

csi-rs-FilterCoefficientRSRQ Specifies L3 filter configurations for CSI-RSRQ measurement results from the L1 filter(s), as defined in 38.215 [9].

csi-rs-FilterCoefficientRSRP Specifies L3 filter configurations for CSI-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

#### –	RACH-ConfigCommon

The RACH-ConfigCommon IE is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-START

RACH-ConfigCommon ::= 				SEQUENCE {

-- Generic RACH parameters

rach-ConfigCommonGeneric			RACH-ConfigCommonGeneric,

groupBconfigured 					SEQUENCE {

-- FFS: ra-Msg3SizeGroupA values

ra-Msg3SizeGroupA					ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5,

spare4, spare3, spare2, spare1},

-- Threshold for preamble selection.  Value in dB.  Value minusinfinity corresponds to –infinity.

-- Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see FFS_Spec, section FFS_Section)

messagePowerOffsetGroupB			ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA			FFS_Value

}																															OPTIONAL,

-- The initial value for the contention resolution timer (see 38.321, section 5.1.5)

ra-ContentionResolutionTimer			ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

-- Msg1 (RA preamble):

-- UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission

-- based on SS blocks that satisfy the threshold (see 38.213, section REF)

rsrp-ThresholdSSB						RSRP-Range																			OPTIONAL,

-- FFS: Provide proper description

-- Corresponds to L1 parameter 'SUL-RSRP-Threshold' (see FFS_Spec, section FFS_Section)

sul-RSRP-Threshold						RSRP-Range																			OPTIONAL,

-- PRACH configuration index. Corresponds to L1 parameter 'PRACHConfigurationIndex' (see 38.211, section 6.3.3.2)

prach-ConfigurationIndex				INTEGER (0..255)																	,

-- PRACH root sequence index. Corresponds to L1 parameter 'PRACHRootSequenceIndex' (see 38.211, section 6.3.3.1).

-- The value range depends on whether L=839 or L=139

prach-RootSequenceIndex					CHOICE {

l839									INTEGER (0..837),

l139									INTEGER (0..137)

}																															,

-- Subcarrier spacing of PRACH. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section)

msg1-SubcarrierSpacing					SubcarrierSpacing																	,

-- The number of PRACH transmission occasions FDMed in one time instance.

-- Corresponds to L1 parameter 'prach-FDM' (see 38.211, section FFS_Section)

msg1-FDM								ENUMERATED {one, two, four, eight},

-- Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of initial active UL BWP(s).

-- The value is configured so that the corresponding RACH resource is entirely within the bandwidth of initial active UL BWP.

-- Corresponds to L1 parameter 'prach-frequency-start' (see 38,211, section FFS_Section)

-- FFS_FIXME: Clarify whether it is ”initial” or ”firstActive” UL BWP, i.e., whether this is meant for SpCell and/or SCell

-- FFS_FIXME: What is PRB 0 or a BWP? PRB 0 defines the lower edge of the carrier.

msg1-FrequencyStart						INTEGER (0..maxNrofPhysicalResourceBlocks-1)										,

-- Configuration of an unrestricted set or one of two types of restricted sets, see 38.211	6.3.3.1

restrictedSetConfig						ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

-- Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion') and the number of Contention Based preambles per SSB

-- (L1 parameter 'CB-preambles-per-SSB'). By multiplying the two values, the UE determines the total number of CB preambles.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB	CHOICE {

oneEighth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth								ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf									ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two										ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four									INTEGER (1..16),

eight									INTEGER (1..8),

sixteen									INTEGER (1..4)

}																															OPTIONAL,	-- Need M

-- Subcarrier spacing for Msg3. Corresponds to L1 parameter 'msg3-scs' (see 38.213, section 8.1)

msg3-SubcarrierSpacing					SubcarrierSpacing,

-- Indicates to a UE whether transform precoding is enabled for Msg3 transmission.

-- Corresponds to L1 parameter 'msg3-tp' (see 38.213, section 8.1)

msg3-transformPrecoding					ENUMERATED {enabled}																OPTIONAL -- Need R

}

-- TAG-RACH-CONFIG-COMMON-STOP

-- ASN1STOP

#### –	RACH-ConfigCommonGeneric

The RACH-ConfigCommonGeneric IE is used to specify the cell specific random-access parameters both for regular random access as well as for beam failure recovery.

RACH-ConfigCommonGeneric information element

-- ASN1START

-- TAG-RACH-CONFIG-COMMON-GENERIC-START

RACH-ConfigCommonGeneric ::= 			SEQUENCE {

-- N-CS configuration, see Table 6.3.3.1-3 in 38.211

zeroCorrelationZoneConfig				INTEGER(0..15),

-- The target power level at the network receiver side (see 38.213, section 7.4, 38.321, section 5.1.2, 5.1.3)

-- FFS_Value: Actual values to be updated based on input from RAN4 (see LS in R2-1800004.

preambleReceivedTargetPower				ENUMERATED {

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106,

dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88,

dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76, dBm-74, dBm-72, dBm-70,

dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52,

dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34,

dBm-32, dBm-30,	dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16,

dBm-14, dBm-12, dBm-10, dBm-8, dBm-6, dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6

}																					,

-- Max number of RA preamble transmission perfomed before declaring a failure (see 38.321, section 5.1.4, 5.1.5)

preambleTransMax 						ENUMERATED {n3, n4, n5, n6, n7,	n8, n10, n20, n50, n100, n200},

-- Power ramping steps for PRACH (see 38.321,5.1.3)

powerRampingStep						ENUMERATED {dB0, dB2, dB4, dB6}														,

-- Msg2 (RAR) window length in number of slots. The network configures a value lower than or euqal to 10 ms (see 38.321, section 5.1.4)

ra-ResponseWindow						ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80}

}

-- TAG-RACH-CONFIG-COMMON-GENERIC-STOP

-- ASN1STOP

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIG-DEDICATED-START

-- FFS_Standlone: resources for msg1-based on-demand SI request

RACH-ConfigDedicated ::=		SEQUENCE {

-- Resources for handover to the cell

cfra-Resources					CFRA-Resources,

-- Subcarrier spacing for msg1 for contention-free RA procedure for handover

-- FFS_CHECK: How does it then work for PDCCH ordered CFRA? In that case the UE does not have RACH-ConfigDedicated!

cfra-msg1-SubcarrierSpacing					SubcarrierSpacing,

-- Subcarrier spacing for msg2 for contention-free RA procedure for handover

-- FFS_CHECK: How does it then work for PDCCH ordered CFRA? In that case the UE does not have RACH-ConfigDedicated!

cfra-msg2-SubcarrierSpacing			SubcarrierSpacing

}

CFRA-Resources ::= 					CHOICE {

cfra-ssb-ResourceList				SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

cfra-csirs							SEQUENCE {

cfra-csirs-ResourceList				SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

cfra-csirs-DedicatedRACH-Threshold	RSRP-Range

}

}

CFRA-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-Id,

ra-PreambleIndex				INTEGER (0..63),

-- 	PRACH configuration for SSB configuration (i.e. time and frequency location)

ra-Resources					RA-Resources -- Definition FFS

}

CFRA-CSIRS-Resource ::= 		SEQUENCE {

csi-RS							NZP-CSI-RS-ResourceId, -- FFS where the CSI-RS are defined (e.g. MO)

ra-PreambleIndex				INTEGER (0..63),

-- 	PRACH configuration for CSIRS configuration (i.e. time and frequency location)

ra-Resources					RA-Resources -- Definition FFS

}

-- TAG-RACH-CONFIG-DEDICATED-STOP

-- ASN1STOP

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList						SRB-ToAddModList										OPTIONAL, -- Need M

srb3-ToRelease							ENUMERATED{true}												OPTIONAL, -- Need N

drb-ToAddModList						DRB-ToAddModList										OPTIONAL, -- Need M

drb-ToReleaseList						DRB-ToReleaseList										OPTIONAL, -- Need N

securityConfig 							SecurityConfig											OPTIONAL, -- Cond RBTermChange

...

}

SRB-ToAddModList ::=					SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=						SEQUENCE {

srb-Identity							SRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 		-- Need N

discardOnPDCP                           ENUMERATED{true}										OPTIONAL,		-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,		-- Cond PDCP

...

}

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation							CHOICE {

-- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

eps-BearerIdentity						INTEGER (0..15),												-- EPS-DRB-Setup

--	The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC

sdap-Config								SDAP-Config													-- 5GC

} 										OPTIONAL, -- Cond DRBSetup

drb-Identity							DRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 		-- Need N

recoverPDCP								ENUMERATED{true}										OPTIONAL, 		-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,		-- Cond PDCP

...

}

DRB-ToReleaseList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=						SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Cond RBTermChange

keyToUse								ENUMERATED{keNB, s-KgNB}								OPTIONAL,	-- Cond RBTermChange

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

RadioBearerConfig field descriptions

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts of the configuration.

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

keyToUse Indicates if the bearers configured with the list in this radioBearerConfig is using KeNB or S-KgNB for deriving ciphering and/or integrity protection keys. Network should not configure SRB1 and SRB2 with S-KgNB and SRB3 with KeNB. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this radioBearerConfig.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to TRUE whenever the security key used for this radio bearer changes.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only.

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this radioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig.

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this radioBearerConfig When the field is not included, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig.

srb3-toRelease Release SRB3. SRB3 release can only be done at SCG release and reconfiguration with sync

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of set up of signalling and data radio bearer and change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding RB is being setup or reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |

#### –	ReportConfigId

The IE ReportConfigId is used to identify a measurement reporting configuration.

ReportConfigId information element

-- ASN1START

-- TAG-REPORT-CONFIG-ID-START

ReportConfigId ::=							INTEGER (1..maxReportConfigId)

-- TAG-REPORT-CONFIG-ID-STOP

-- ASN1STOP

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event. Measurement reporting events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS. These events are labelled AN with N equal to 1, 2 and so on.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

Event A6:	Neighbour becomes amount of offset better than SCell.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORT-CONFIG-START

ReportConfigNR ::=							SEQUENCE {

reportType									CHOICE {

periodical									PeriodicalReportConfig,

eventTriggered								EventTriggerConfig,

-- reportCGI is to be completed before the end of Rel-15.

reportCGI									ENUMERATED {ffsTypeAndValue},

...

}

}

-- FFS / TODO: Consider separating trgger configuration (trigger, periodic, …) from report configuration.

-- Current structure allows easier definiton of new events and new report types e.g. CGI, etc.

EventTriggerConfig::=						SEQUENCE {

eventId										CHOICE {

eventA1										SEQUENCE {

a1-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA2										SEQUENCE {

a2-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA3										SEQUENCE {

a3-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA4										SEQUENCE {

a4-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA5										SEQUENCE {

a5-Threshold1								MeasTriggerQuantity,

a5-Threshold2								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA6										SEQUENCE {

a6-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

...

},

rsType										ENUMERATED {ssb, csi-rs},

-- Common reporting config (at least to periodical and eventTriggered)

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

-- Cell reporting configuration

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

-- RS index reporting configuration

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL, 	-- Need M

maxNrofRSIndexesToReport					INTEGER (1..maxNrofIndexesToReport) 								OPTIONAL, 	-- Need M

includeBeamMeasurements						BOOLEAN,

-- If configured the UE includes the best neighbour cells per serving frequency

reportAddNeighMeas							ENUMERATED {setup}									OPTIONAL,		-- Need R

...

}

PeriodicalReportConfig ::=					SEQUENCE {

rsType										ENUMERATED {ss, csi-rs},

-- Common reporting config (at least to periodical and eventTriggered)

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

-- Cell reporting configuration

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

-- RS index reporting configuration

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL, 	-- Need R

maxNrofRsIndexesToReport						INTEGER (1..maxNrofIndexesToReport) 									OPTIONAL, 	-- Need R

includeBeamMeasurements						BOOLEAN,

...

}

MeasTriggerQuantity ::=						CHOICE {

rsrp										RSRP-Range,

rsrq										RSRQ-Range,

sinr										SINR-Range

}

MeasTriggerQuantityOffset ::=				CHOICE {

rsrp										INTEGER (ffsValue),

rsrq										INTEGER (ffsValue),

sinr										INTEGER (ffsValue)

}

MeasReportQuantity ::=						SEQUENCE {

rsrp										BOOLEAN,

rsrq										BOOLEAN,

sinr										BOOLEAN

}

-- TAG-REPORT-CONFIG-START

-- ASN1STOP

ReportConfigNR field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5.

eventId Choice of NR event triggered reporting criteria.

maxReportCells Max number of non-serving cells to include in the measurement report.

maxNrofRsIndexesToReport Max number of measurement information per RS index to include in the measurement report for A1-A6 events.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRsIndexes Indicates which measurement information per RS index the UE shall include in the measurement report.

reportAddNeighMeas Indicates that the UE shall includes the best neighbour cells per serving frequency.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

Editor’s Note: FFS ASN.1 details of reportCGI.

#### –	ReportConfigToAddModList

The IE ReportConfigToAddModList concerns a list of reporting configurations to add or modify.

ReportConfigToAddModList information element

-- ASN1START

-- TAG-REPORT-CONFIG-TO-ADD-MOD-LIST-START

ReportConfigToAddModList ::=		SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod

ReportConfigToAddMod ::=	SEQUENCE {

reportConfigId						ReportConfigId,

reportConfig						CHOICE {

reportConfigNR						ReportConfigNR,

reportConfigEUTRA					ReportConfigEUTRA

}

}

-- TAG- REPORT-CONFIG-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

Editor’s Note: FFS Definition of reportConfigEUTRA in 38.331.

#### –	ReportInterval

The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerType event as well as for triggerType periodical. Value ms120 corresponds to 120 ms, ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

ReportInterval ::=					ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,

min6, min12, min30, spare2, spare1}

-- ASN1STOP

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs and DRBs.

RLC-Config information element

-- ASN1START

-- TAG-RLC-CONFIG-START

RLC-Config ::=						CHOICE {

am									SEQUENCE {

ul-AM-RLC							UL-AM-RLC,

dl-AM-RLC							DL-AM-RLC

},

um-Bi-Directional					SEQUENCE {

ul-UM-RLC							UL-UM-RLC,

dl-UM-RLC							DL-UM-RLC

},

um-Uni-Directional-UL				SEQUENCE {

ul-UM-RLC							UL-UM-RLC

},

um-Uni-Directional-DL				SEQUENCE {

dl-UM-RLC							DL-UM-RLC

},

...

}

UL-AM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthAM,

t-PollRetransmit					T-PollRetransmit,

pollPDU								PollPDU,

pollByte							PollByte,

maxRetxThreshold					ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }

}

DL-AM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthAM,

t-Reassembly						T-Reassembly,

t-StatusProhibit					T-StatusProhibit

}

UL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthUM

}

DL-UM-RLC ::=						SEQUENCE {

sn-FieldLength						SN-FieldLengthUM,

t-Reassembly						T-Reassembly

}

T-PollRetransmit ::=				ENUMERATED {

ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms2000, ms4000, spare5, spare4, spare3,

spare2, spare1}

PollPDU ::=							ENUMERATED {

p4, p8, p16, p32, p64, p128, p256, p512, p1024, p2048, p4096, p6144, p8192, p12288, p16384, p20480,

p24576, p28672, p32768, p40960, p49152, p57344, p65536, infinity, spare8, spare7, spare6, spare5, spare4,

spare3, spare2, spare1}

PollByte ::=						ENUMERATED {

kB1, kB2, kB5, kB8, kB10, kB15, kB25, kB50, kB75,

kB100, kB125, kB250, kB375, kB500, kB750, kB1000,

kB1250, kB1500, kB2000, kB3000, kB4000, kB4500,

kB5000, kB5500, kB6000, kB6500, kB7000, kB7500,

mB8, mB9, mB10, mB11, mB12, mB13, mB14, mB15,

mB16, mB17, mB18, mB20, mB25, mB30, mB40, infinity,

spare20, spare19, spare18, spare17, spare16,

spare15, spare14, spare13, spare12, spare11,

spare10, spare9, spare8, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

T-Reassembly ::=					ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms110,

ms120, ms130, ms140, ms150, ms160, ms170,

ms180, ms190, ms200, spare1}

T-StatusProhibit ::=				ENUMERATED {

ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35,

ms40, ms45, ms50, ms55, ms60, ms65, ms70,

ms75, ms80, ms85, ms90, ms95, ms100, ms105,

ms110, ms115, ms120, ms125, ms130, ms135,

ms140, ms145, ms150, ms155, ms160, ms165,

ms170, ms175, ms180, ms185, ms190, ms195,

ms200, ms205, ms210, ms215, ms220, ms225,

ms230, ms235, ms240, ms245, ms250, ms300,

ms350, ms400, ms450, ms500, ms800, ms1000,

ms1200, ms1600, ms2000, ms2400, spare2, spare1}

SN-FieldLengthUM ::=				ENUMERATED {size6, size12}

SN-FieldLengthAM ::=				ENUMERATED {size12, size18}

-- TAG-RLC-CONFIG-STOP

-- ASN1STOP

RLC-Configfield descriptions

maxRetxThreshold Parameter for RLC AM in TS 38.322 [4]. Value t1 corresponds to 1 retransmission, t2 to 2 retransmissions and so on.

pollByte Parameter for RLC AM in TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, kB50 to 50 kBytes and so on. kBInfinity corresponds to an infinite amount of kBytes.

pollPDU Parameter for RLC AM in TS 38.322 [4]. Value p4 corresponds to 4 PDUs, p8 to 8 PDUs and so on. pInfinity corresponds to an infinite number of PDUs.

sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, size12 means 12 bits, size18 means 18 bits.

t-PollRetransmit Timer for RLC AM inTS 38.322 [4], in milliseconds. Value ms5 means 5ms, ms10 means 10ms and so on.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0ms, ms5 means 5ms and so on.

t-StatusProhibit Timer for status reporting in TS 38.322 [4], in milliseconds. Value ms0 means 0ms, ms5 means 5ms and so on.

#### –	RLF-TimersAndConstants

Editor’s Note: FFS / TODO: Insert the RLF timers and related functionality. Check what is needed for EN-DC.

The RLF-TimersAndConstants IE is used to configure UE specific timers and constants.

RLF-TimersAndConstants information element

-- ASN1START

-- TAG-RLF-TIMERS-AND-CONSTANTS-START

RLF-TimersAndConstants ::= 		SetupRelease {

SEQUENCE {

t310								ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000, ms4000, ms6000},

n310								ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

n311								ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

...

}

}

-- TAG-RLF-TIMERS-AND-CONSTANTS-STOP

-- ASN1STOP

RLF-TimersAndConstants field descriptions

n3xy Constants are described in section 7.4. n1 corresponds with 1, n2 corresponds to 2 and so on.

t3xy Timers are described in section 7.3. Value ms0 corresponds with 0 ms, ms50 corresponds to 50 ms and so on.

#### –	RNTI-Value

The RNTI-Value IE represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=						BIT STRING (SIZE (16))

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

#### –	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. Integer value for RSRP measurements according to mapping table in TS 38.133 [14].

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=						INTEGER(0..124)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

#### –	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 38.133 [14].

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=						INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

#### –	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. Integer value for SINR measurements is according to mapping table in TS 38.133 [14].

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=						INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

#### –	SCellIndex

The IE SCellIndex concerns a short identity, used to identify an SCell.

SCellIndex information element

-- ASN1START

-- TAG-SCELL-INDEX-START

-- The value range is shared across the Cell Groups

SCellIndex ::=						INTEGER (1..31)

-- TAG-SCELL-INDEX-STOP

-- ASN1STOP

#### –	SchedulingRequest-Config

The IE SchedulingRequest-Config is used to configure the parameters, for the dedicated scheduling request (SR) resources.

SchedulingRequest-Config information element

-- ASN1START

-- TAG-SCHEDULING-REQUEST-CONFIG-START

SchedulingRequestConfig ::= 		SEQUENCE {

schedulingRequestToAddModList		SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod			OPTIONAL, -- Need N

schedulingRequestToReleaseList		SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId				OPTIONAL  -- Need N

}

SchedulingRequestToAddMod ::=		SEQUENCE {

schedulingRequestID					SchedulingRequestId,

sr-prohibitTimer					ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms32, ms64, ms128},

sr-TransMax							ENUMERATED { n4, n8, n16, n32, n64, spare3, spare2, spare1}

}

-- FFS_TODO: provide resources for each SchedulingRequestID in ServingCellConfig (TBD whether directly, in PUCCH-Config, in each BWP)

-- TAG-SCHEDULING-REQUEST-CONFIG-STOP

-- ASN1STOP

SchedulingRequest-Configfield descriptions

schedRequestToAddModList List of Scheduling Request configurations to add or modify.

SchedulingRequestId Used to modify a SR configuration and to indicate, in LogicalChannelConfig, the SR configuration to which a logical channel is mapped.

sr-prohibitTimer Timer for SR transmission on PUCCH in TS 38.321 [3]. Value in ms. ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on.

sr-TransMax Maximum number of SR transmissions as described in 38.321 [3]. n4 corresponds to 4, n8 corresponds to 8, and so on.

#### –	SchedulingRequestResourceConfig

The IE SchedulingRequestResourceConfig determines physical layer resources on PUCCH where the UE may send the dedicated scheduling request (D-SR) (see 38.213, section 9.2.2).

SchedulingRequestResourceConfig information element

-- ASN1START

-- TAG-SCHEDULING-REQUEST-RESOURCE-CONFIG-START

SchedulingRequestResourceConfig ::=			SEQUENCE {

schedulingRequestResourceId						SchedulingRequestResourceId,

-- The ID of the SchedulingRequestConfig that uses this scheduling request resource.

schedulingRequestID								SchedulingRequestId,

-- SR periodicity and offset in number of slots. Corresponds to L1 parameter 'SR-periodicity' and 'SR-offset' (see 38.213, section 9.2.2)

-- The following periodicities may be configured depending on the chosen subcarrier spacing:

-- SCS =  15 kHz: 2sym, 7sym, 1sl, 2sl, 5sl, 10sl, 20sl, 40sl, 80sl

-- SCS =  30 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 10sl, 20sl, 40sl, 80sl, 160sl

-- SCS =  60 kHz: 2sym, 7sym/6sym, 1sl, 2sl, 4sl, 8sl, 20sl, 40sl, 80sl, 160sl, 320sl

-- SCS = 120 kHz: 2sym, 7sym, 1sl, 2sl, 4sl, 8sl, 16sl, 40sl, 80sl, 160sl, 320sl, sl640

-- sym6or7 corresponds to 6 symbols if extended cyclic prefix and a SCS of 60 kHz are configured, otherwise it corresponds to 7 symbols.

-- For periodicities sym2, sym7 and sl1 the UE assumes an offset of 0 slots.

periodicityAndOffset							CHOICE {

sym2											NULL,

sym6or7											NULL,

sl1												NULL,						-- Recurs in every slot

sl2												INTEGER (0..1),

sl4												INTEGER (0..3),

sl5												INTEGER (0..4),

sl8												INTEGER (0..7),

sl10											INTEGER (0..9),

sl16											INTEGER (0..15),

sl20											INTEGER (0..19),

sl40											INTEGER (0..39),

sl80											INTEGER (0..79),

sl160											INTEGER (0..159),

sl320											INTEGER (0..319),

sl640											INTEGER (0..639)

}																														OPTIONAL,	-- Need M

-- Format, length, ... of this SR reosurce. The network configures a PUCCH-Resource of PUCCH-format0 or PUCCH-format1

-- (other formats not supported). Corresponds to L1 parameter 'SR-resource' (see 38.213, section 9.2.2)

resource

PUCCH-Resource															OPTIONAL	-- Need M

}

-- TAG-SCHEDULING-REQUEST-RESOURCE-CONFIG-STOP

-- ASN1STOP

#### –	SchedulingRequestResourceId

The IE SchedulingRequestResourceId is used to identify scheduling request resources on PUCCH.

SchedulingRequestResourceId information element

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCEID-START

SchedulingRequestResourceId ::=					INTEGER (1..maxNrofSR-Resoruces)

-- TAG-SCHEDULINGREQUESTRESOURCEID-STOP

-- ASN1STOP

#### –	ScramblingId

The IE ScramblingID is used for scrambling channels and reference signals.

-- ASN1START

-- TAG-SCRAMBLING-ID-START

ScramblingId ::=					BIT STRING (SIZE (10))

-- TAG-SCRAMBLING-ID-STOP

-- ASN1STOP

#### –	SDAP-Config

The IE SDAP-Config is used to set the configurable SDAP parameters for a data radio bearer. All configured instances of SDAP-Config with the same value of pdu-Session correspond to the same SDAP entity as specified in TS 37.324 [FFS_Ref].

SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=						SEQUENCE {

-- FFS / TODO: Definition of PDU-SessionID to be added

pdu-Session							PDU-SessionID,

-- FFS: separate configuration for UL and DL

sdap-HeaderDL 						ENUMERATED {present, absent},

sdap-HeaderUL 						ENUMERATED {present, absent},

defaultDRB							BOOLEAN,

reflectiveQoS						BOOLEAN,	-- FFS_Standalone: It is FFS whether this field is needed

-- A list of QoS-Flow-IDs that the UE shall map to the DRB of this SDAP-Config.

mappedQoS-FlowsToAdd				SEQUENCE (SIZE (0..maxNrofQFIs)) OF QFI 									OPTIONAL, -- Need N

-- A list of QoS-Flow-IDs that the UE shall no longer map to the DRB of this SDAP-Config.

mappedQoS-FlowsToRelease			SEQUENCE (SIZE (0..maxNrofQFIs)) OF QFI 									OPTIONAL, -- Need N

...

}

QFI ::= 							INTEGER (0..maxQFI)

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

SDAP-Configfield descriptions

defaultDRB Indicates whether or not this is the default DRB for this PDU session. Among all configured instances of SDAP-Config with the same value of pdu-Session, this field shall be set to TRUE in one instance of SDAP-Config and to FALSE in all other instances.

mappedQoS-FFlows List of QFIs of QoS flows of the PDU session indicated by pdu-Session which are configured to be mapped to this DRB. A QFI value can be included at most once in all configured instances of SDAP-Config with the same value of pdu-Session.

pdu-Session Identity of the PDU session whose QoS flows are mapped to the DRB

reflectiveQoS Indicates whether or not reflective QoS is active for QoS flows transmitted via this DRB.

sdap-HeaderUL Indicates whether or not a SDAP header is present for UL data on this DRB.

sdap-HeaderDL Indicates whether or not a SDAP header is present for DL data on this DRB.

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::= 						SEQUENCE {

-- Identity of the search space. SearchSpaceId = 0 identifies the SearchSpace configured via PBCH (MIB) or ServingCellConfigCommon.

searchSpaceId							SearchSpaceId,

-- The CORESET applicable for this SearchSpace.

-- Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon?

-- Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling?

controlResourceSetId					ControlResourceSetId,

-- Slots for PDCCH Monitoring configured as periodicity and offset. Corresponds to L1 parameters 'Montoring-periodicity-PDCCH-slot' and

-- 'Montoring-offset-PDCCH-slot' (see 38.213, section 10)

monitoringSlotPeriodicityAndOffset		CHOICE {

sl1										NULL,

sl2										INTEGER (0..1),

sl4										INTEGER (0..3),

sl5 									INTEGER (0..4),

sl8										INTEGER (0..7),

sl10 									INTEGER (0..9),

sl16 									INTEGER (0..15),

sl20 									INTEGER (0..19)

}																													OPTIONAL,

-- Symbols for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset).

-- The most significant (left) bit represents the first OFDM in a slot. The least significant (right) bit represents the last symbol.

-- Corresponds to L1 parameter 'Montoring-symbols-PDCCH-within-slot' (see 38.213, section 10)

monitoringSymbolsWithinSlot					BIT STRING (SIZE (14))													OPTIONAL,

-- Number of candidates per aggregation level. Corresponds to L1 parameter 'Aggregation-level-1' to 'Aggregation-level-8'

-- (see 38.213, section 10)

nrofCandidates							SEQUENCE {

aggregationLevel1						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16						ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

},

-- Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

searchSpaceType							CHOICE {

-- Configures this search space as common search space (CSS) and DCI formats to monitor.

common									SEQUENCE {

-- If configured, the UE monitors the DCI formats 0_0 and 1_0 with CRC scrambled by C-RNTI, CS-RNTI (if configured),

-- SP-CSI-RNTI (if configured), RA-RNTI, TC-RNTI, P-RNTI, SI-RNTI

format0-0-AndFormat1-0					SEQUENCE {

...

}																												OPTIONAL,	-- Need R

-- If configured, UE monitors the DCI format format 2_0 with CRC scrambled by SFI-RNTI

format2-0								SEQUENCE {

-- Configuration of SFI-related parameters to be applied in this search space

slotFormatIndicatorSFI					SlotFormatIndicatorSFI												OPTIONAL,	--	Need M

...

}																												OPTIONAL,	-- Need R

-- If configured, UE monitors the DCI format format 2_1 with CRC scrambled by INT-RNTI

format2-1								SEQUENCE {

-- Configuration of downlink preemtption indications to be monitored in this cell.

-- Corresponds to L1 parameter 'Preemp-DL' (see 38.214, section 11.2)

downlinkPreemption						DownlinkPreemption													OPTIONAL,	-- Need M

...

}																												OPTIONAL,	-- Need R

-- If configured, UE monitors the DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI

format2-2								SEQUENCE {

...

}																												OPTIONAL,	-- Need R

-- If configured, UE monitors the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI

format2-3								SEQUENCE {

...

}																												OPTIONAL	-- Need R

},

-- Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled

-- by C-RNTI, CS-RNTI (if configured), TC-RNTI (if a certain condition is met), and SP-CSI-RNTI (if configured)

ue-Specific								SEQUENCE {

-- Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

formats									ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...

}

}																														OPTIONAL -- Need M

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

#### –	SlotFormatIndicatorSFI

The IE SlotFormatIndicatorSFI is used to configure monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI).

SlotFormatIndicatorSFI information element

-- ASN1START

-- TAG-SLOTFORMATINDICATORSFI-START

SlotFormatIndicatorSFI ::= 				SEQUENCE {

-- RNTI used for SFI on the given cell

-- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section 11.1.1)

sfi-RNTI									RNTI-Value													OPTIONAL,

-- The number of PDCCH candidates for the configured aggregation level.

-- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section 11.1.1)

nrofCandidates								ENUMERATED {n1, n2},

-- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section 11.1.1)

aggregationLevel							ENUMERATED {n1, n2, n4, n8, n16},

-- Total length of the DCI payload scrambled with SFI-RNTI.

-- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section 11.1.1)

dci-PayloadSize								INTEGER (1..maxSFI-DCI-PayloadSize),

-- A list of SlotFormatCombinations for the UE's serving cells.

-- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section 11.1.1)

slotFormatConfigurations					SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell		OPTIONAL,

...

}

-- TAG-SLOTFORMATINDICATORSFI-STOP

-- ASN1STOP

#### –	DownlinkPreemption

The IE DownlinkPreemption is used to configure the UE to monitor PDCCH for the INT-RNTI (interruption).

DownlinkPreemption information element

-- ASN1START

-- TAG-DOWNLINKPREEMPTION-START

-- Configuration of downlink preemption indication on PDCCH.

DownlinkPreemption ::=						SEQUENCE {

-- RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space.

-- Corresponds to L1 parameter 'INT-RNTI', where ”INT” stands for ”interruption” (see 38.213, section 10)

int-RNTI								RNTI-Value,

-- Slots for PDCCH Monitoring of INT_RNTI configured as periodicity and offset.

monitoringSlotPeriodicityAndOffset		CHOICE {

sl1										NULL,

sl2										INTEGER (0..1),

sl4										INTEGER (0..3)

},

-- Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1)

-- The set determines how the UE interprets the DL preemption DCI payload.

timeFrequencySet						ENUMERATED {set0, set1},

-- Total length of the DCI payload scrambled with INT-RNTI. The value must be an integer multiple of 14 bit.

-- Corresponds to L1 parameter 'INT-DCI-payload-length' (see 38.213, section 11.2)

dci-PayloadSize							INTEGER (0..maxINT-DCI-PayloadSize)												OPTIONAL,

-- Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload.

-- Corresponds to L1 parameter 'INT-cell-to-INT' and 'cell-to-INT' (see 38.213, section 11.2)

int-ConfigurationPerServingCell			SEQUENCE (SIZE (1..maxNrofServingCells)) OF INT-ConfigurationPerServingCell		OPTIONAL	-- Need M

}

INT-ConfigurationPerServingCell ::= 		SEQUENCE {

servingCellId							ServCellIndex,

-- Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI

-- payload. Must be multiples of 14 (bit). Corresponds to L1 parameter 'INT-values' (see 38.213, section 11.2)

positionInDCI							INTEGER (0..maxINT-DCI-PayloadSize-1)

OPTIONAL

}

-- TAG-DOWNLINKPREEMPTION-STOP

-- ASN1STOP

#### –	SearchSpaceId

The IE SearchSpaceId is used to identify Search Spaces. The search space with the SearchSpaceId = 0 identifies the search space configured via PBCH (MIB) and in ServingCellConfigCommon. The number of Search Spaces per BWP is limited to 10 including the initial Search Space.

SearchSpaceId information element

-- ASN1START

-- TAG-SEARCHSPACEID-START

SearchSpaceId ::= 							INTEGER (1..maxNrofSearchSpaces)

-- TAG-SEARCHSPACEID-STOP

-- ASN1STOP

#### –	SecurityAlgorithmConfig

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm (SRBs) and AS ciphering algorithm (SRBs and DRBs).

SecurityAlgorithmConfig information element

-- ASN1START

-- TAG-SECURITY-ALGORITHM-CONFIG-START

SecurityAlgorithmConfig ::=			SEQUENCE {

cipheringAlgorithm						CipheringAlgorithm,

integrityProtAlgorithm				IntegrityProtAlgorithm			OPTIONAL	-- Need R

}

IntegrityProtAlgorithm ::=			ENUMERATED {

nia0, nia1, nia2, nia3, spare4, spare3,

spare2, spare1, ...}

CipheringAlgorithm ::=				ENUMERATED {

nea0, nea1, nea2, nea3, spare4, spare3,

spare2, spare1, ...}

-- TAG-SECURITY-ALGORITHM-CONFIG-STOP

-- ASN1STOP

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. For EN-DC, the algorithms configured for bearers using KeNB shall be the same as for all bearers using KeNB and the algorithms configured for bearers using KgNB shall be the same as for all bearers using KgNB.

integrityProtAlgorithm For EN-DC, this IE indicates the integrity protection algorithm to be used for SRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 is identical to the LTE algorithms eia0-3. For EN-DC, the algorithms configured for SRBs using KeNB shall be the same as for all SRBs using KeNB and the algorithms configured for bearers using KgNB shall be the same as for all bearers using KgNB.

#### –	ServCellIndex

The IE ServCellIndex concerns a short identity, used to identify a serving cell (i.e. the PCell or an SCell). Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

ServCellIndex information element

-- ASN1START

-- TAG-SERV-CELL-INDEX-START

ServCellIndex ::=					INTEGER (0..maxNrofServingCells-1)

-- TAG-SERV-CELL-INDEX-STOP

-- ASN1STOP

#### –	ServingCellConfigCommon

The ServingCellConfigCommon IE is used to configure cell specific parameters of a UE’s serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-COMMON-START

ServingCellConfigCommon ::=		SEQUENCE {

physCellId							PhysCellId													OPTIONAL, -- Cond HOAndServCellAdd,

frequencyInfoDL						FrequencyInfoDL												OPTIONAL, -- Cond InterFreqHOAndServCellAdd

-- The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

-- FFS: Discuss and then clarify in condition which serving cells have an initial BWP

initialDownlinkBWP		DownlinkBWP-Common								OPTIONAL,	-- Cond FFS

-- FFS: Possibly remove the condition on uplinkConfigCommon or replace by ”UL”. Note that the entire ServingCellConfigCommon can

-- only be sent when upon reconfiguration with sync and upon PSCell/SCell addition.

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL, -- Cond ReconfWithSyncAndSCellAdd

supplementaryUplinkConfig

UplinkConfigCommon

OPTIONAL, -- Cond SUL

-- Subcarrier spacing for SIB1, Msg.2/4 for initial access and SI-messages.

-- Values 15, and 30 kHz are applicable for carrier frequencies <6GHz; Values 60 and 120 kHz are applicable for carrier frequencies >6GHz

-- FFS: This must be one of the SCSs defined already inside FrequencyInfoDL. Consider flagging one of those as ”common” instead of this field.

subcarrierSpacingCommon				SubcarrierSpacing,

-- Indicates the time domain positions of the transmitted SS-blocks in an SS-burst.

-- Corresponds to L1 parameter 'SSB-Transmitted' (see 38.213, section 4.1)

-- FFS_CECHK: Is the NW required to provide always a valid bitmap? If not, we cannot use “need M”

ssb-PositionsInBurst				CHOICE {

-- bitmap for sub 3 GHz

shortBitmap							BIT STRING (SIZE (4)),

-- bitmap for 3-6 GHz

mediumBitmap						BIT STRING (SIZE (8)),

-- bitmap for above 6 GHz

longBitmap							BIT STRING (SIZE (64))

}																									OPTIONAL, -- Need M,

-- The SSB periodicity in msec for the rate matching purpose (see 38.211, section [7.4.3.1])

ssb-periodicityServingCell			ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }				OPTIONAL,

-- Position of (first) DL DM-RS (see 38.211, section 7.4.1.1.1)

dmrs-TypeA-Position					ENUMERATED {pos2, pos3},

-- Subcarrier spacing of SSB. Used only for non-initial access (e.g. SCells, PCell of SCG).

-- If the field is absent the UE shall assume the default value of the band.

-- FFS in RAN1: Possibility to have several default values? May the field be absent in that case?

subcarrierSpacingSSB				SubcarrierSpacingSSB											OPTIONAL, -- Need S

-- A cell-specific TDD UL/DL configuration.

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

-- A second cell-specific TDD UL/DL configuration.

-- FFS_CHECK: What does the UE do with two? Which one applies? A union of both? If so, how?

tdd-UL-DL-ConfigurationCommon2		TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

-- TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power.

-- (see 38.213, section 7.4)

ss-PBCH-BlockPower					INTEGER (-60..50)

-- =====================================

-- FFS which of the following are needed

--

--	bcch-Config 						BCCH-Config,

--	pcch-Config 						PCCH-Config,

}

UplinkConfigCommon ::=				SEQUENCE {

-- Absolute uplink frequency configuration and subcarrier specific virtual carriers.

frequencyInfoUL						FrequencyInfoUL											OPTIONAL, -- Cond InterFreqHOAndUplinkSCellAdd

-- The initial uplink BWP configuration for a SpCell (PCell of MCG or SCG). Corresponds to L1 parameter 'initial-UL-BWP'.

-- (see 38.331, section FFS_Section).

-- FFS: Discuss and then clarify in condition which serving cells have an initial BWP

initialUplinkBWP			UplinkBWP-Common											OPTIONAL	-- Cond FFS

}

-- TAG-SERVING-CELL-CONFIG-COMMON-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| HOAndServCellAdd | This field is mandatory present for inter-cell handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent. |
| InterFreqHOAndServCellAdd | This field is mandatory present for inter-frequency inter-cell handover and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent. |

#### –	ServingCellConfig

The ServingCellConfig IE is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts).

ServingCellConfig information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-START

ServingCellConfig ::=		SEQUENCE {

-- L1 parameters:

tdd-UL-DL-ConfigurationDedicated	TDD-UL-DL-ConfigDedicated														OPTIONAL, -- Cond TDD

-- The dedicated (UE-specific) configuration for the initial downlink bandwidth-part.

-- FFS: Discuss and then clarify in condition which serving cells have an initial BWP

initialDownlinkBWP					DownlinkBWP-Dedicated														OPTIONAL,	-- Need M

-- List of additional downlink bandwidth parts to be released. (see 38.211, 38.213, section 12).

downlinkBWP-ToReleaseList			SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id									OPTIONAL,	-- Need N

-- List of additional downlink bandwidth parts to be added or modified. (see 38.211, 38.213, section 12).

downlinkBWP-ToAddModList			SEQUENCE (SIZE (1..maxNrofBWPs)) OF DownlinkBWP								OPTIONAL, 	-- Need N

-- ID of the downlink bandwidth part to be used upon MAC-activation of an  SCell. If not provided, the UE uses the default BWP.

-- The initial bandwidth part is referred to by BWP-Id = 0.

firstActiveDownlinkBWP-Id			BWP-Id																		OPTIONAL,	-- Cond SCellOnly

-- The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.321, section 5.15)

-- The value 0.5 ms is only applicable for carriers >6 GHz.

-- FFS: RAN2 to discuss/confirm value range. RAN1 just suggested values from 1ms/0.5ms and up to about 50 ms.

-- When the network releases the timer configuration, the UE stops the timer without swithching to the default BWP.

bwp-InactivityTimer					SetupRelease { ENUMERATED {

ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10,

ms20, ms30, ms40, ms50, ms60, ms80, spare} }					OPTIONAL,	--	Need M

-- Corresponds to L1 parameter 'default-DL-BWP'. The initial bandwidth part is referred to by BWP-Id = 0.

-- ID of the downlink bandwidth part to be used upon expiry of txxx.

-- This field is UE specific. When the field is absent the UE uses the the initial BWP as default BWP.

-- (see 38.211, 38.213, section 12 and 38.321, section 5.15)

-- FFS: Whether to add a default uplink BWP

defaultDownlinkBWP-Id				BWP-Id																		OPTIONAL, 	-- Need M

uplinkConfig						UplinkConfig																OPTIONAL,	-- Need M

supplementaryUplink					UplinkConfig 																OPTIONAL, 	-- Need M

-- FFS in RAN1: Tracking Reference Signals configuration: TRS-Config?

csi-MeasConfig						CSI-MeasConfig																OPTIONAL,

-- MAC parameters:

sCellDeactivationTimer				ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240, ms320, ms400, ms480, ms520, ms640,

ms720, ms840, ms1280, spare2,spare1}				OPTIONAL,	-- Cond ServingCellWithoutPUCCH

-- Indicates whether this SCell is cross-carrier scheduled by another serving cell.

-- FFS: How to indicate whether CIF is present in the DCIs of the PCell? Should the CrossCarrierSchedulingConfig be included

-- and set to own so that the field “cif-Presence” can be set?

crossCarrierSchedulingConfig		CrossCarrierSchedulingConfig												OPTIONAL,	-- Cond SCell

-- Timing Advance Group ID, as specified in TS 38.321 [3],  which this cell belongs to.

tag-Id								TAG-Id,

-- Enables the "UE beam lock function (UBF)", which disable changes to the UE beamforming configuration when in NR_RRC_CONNECTED.

-- FFS: Parameter added preliminary based on RAN4 LS in R4-1711823. Decide where to place it (maybe ServingCellConfigCommon or

-- in a BeamManagement IE??)

ue-BeamLockFunction					ENUMERATED {enabled}														OPTIONAL,

-- Indicates whether UE shall apply as pathloss reference either the downlink of PCell or of SCell that corresponds with this uplink

-- (see 38.213, section 7)

pathlossReferenceLinking			ENUMERATED {pCell, sCell}													OPTIONAL	-- Cond SCell

}

UplinkConfig ::=					SEQUENCE {

-- The dedicated (UE-specific) configuration for the initial uplink bandwidth-part.

-- FFS: Discuss and then clarify in condition which serving cells have an initial BWP

initialUplinkBWP					UplinkBWP-Dedicated													OPTIONAL, 	-- Need M

-- The additional bandwidth parts for uplink. In case of TDD uplink- and downlink BWP with the same bandwidthPartId are considered

-- as a BWP pair and must have the same center frequency.

uplinkBWP-ToReleaseList				SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id							OPTIONAL,	-- Need N

uplinkBWP-ToAddModList				SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkBWP						OPTIONAL, 	-- Need N

-- ID of the uplink bandwidth part to be used upon MAC-activation of an  SCell. If not provided, the UE uses the FFS: default BWP.

-- The initial bandwidth part is referred to by BandiwdthPartId = 0.

firstActiveUplinkBWP-Id				BWP-Id																OPTIONAL -- Cond SCellOnly

}

-- TAG-SERVING-CELL-CONFIG-STOP

-- ASN1STOP

#### –	SlotFormatCombinationsPerCell

The IE SlotFormatCombinationsPerCell is used to configure FFS

SlotFormatCombinationsPerCell information element

-- ASN1START

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-START

-- The SlotFormatCombinations applicable for one serving cell. Corresponds to L1 parameter 'cell-to-SFI' (see 38.213, section 11.1.1)

SlotFormatCombinationsPerCell ::=								SEQUENCE {

-- The ID of the serving cell for which the slotFormatCombinations are applicable

servingCellId								ServCellIndex,

-- A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see 38.211, section 4.3.2)

-- FFS_CHECK: RAN1 indicates that the combinations could be of two different types... but they don't specify the second

slotFormatCombinations						SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination	OPTIONAL,

-- The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload.

-- Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section)

positionInDCI							INTEGER(0..maxSFI-DCI-PayloadSize-1)													OPTIONAL

}

SlotFormatCombination ::= 					SEQUENCE {

-- This ID is used in the DCI payload to dynamically select this SlotFormatCombination.

-- Corresponds to L1 parameter 'SFI-index' (see 38.213, section FFS_Section)

slotFormatCombinationId						SlotFormatCombinationId,

-- Slot formats that occur in consecutive slots in time domain order as listed here. The the slot formats are

-- defined in 38.211, table 4.3.2-3 and numbered with 0..255.

slotFormats									SEQUENCE (SIZE (1..maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255),

-- Reference subcarrier spacing for this Slot Format Combination. Corresponds to L1 parameter 'SFI-scs' (see 38.213, section FFS_Section)

subcarrierSpacing							SubcarrierSpacing,

-- Reference subcarrier spacing for a Slot Format Combination on an FDD or SUL cell.

-- Corresponds to L1 parameter 'SFI-scs2' (see 38.213, section FFS_Section).

-- For FDD, subcarrierSpacing (SFI-scs) is the reference SCS for DL BWP and subcarrierSpacing2 (SFI-scs2) is the reference SCS for UL BWP.

-- For SUL, subcarrierSpacing (SFI-scs) is the reference SCS for non-SUL carrier and subcarrierSpacing2 (SFI-scs2) is the reference

-- SCS for SUL carrier.

subcarrierSpacing2							SubcarrierSpacing															OPTIONAL	-- Need R

}

-- SFI index that is assoicated with a certian slot-format-combination

-- Corresponds to L1 parameter 'SFI-index' (see 38.213, section FFS_Section)

SlotFormatCombinationId	::=					INTEGER (0..maxNrofSlotFormatCombinationsPerSet-1)

-- TAG-SLOTFORMATCOMBINATIONSPERCELL-STOP

-- ASN1STOP

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=					INTEGER (1..3)

-- TAG-SRB-IDENTITY-STOP

-- ASN1STOP

#### –	SPS-Config

Editor’s Note: FFS: RAN1 indicated in the L1 table: “Note: Multiple configurations is possible, how many needs to be determined". RAN2 agreed that SPS can be used on Pcell and SCell... But each UE can use it on at most one serving cell of a cell group at a time. Are the ”multiple configuration” meant for one carrier? Does the UE then use several SPS-RNTIs?

The SPS-Config IE is used to configure downlink semi-persistent transmission according to two possible schemes.

SPS-Config information element

-- ASN1START

-- TAG-SPS-CONFIG-START

-- Downlink SPS may be configured on the PCell as well as on SCells. But it shall not be configured for more than

-- one serving cell of a cell group at once.

SPS-Config ::= 							SEQUENCE {

-- Periodicity for DL SPS

-- Corresponds to L1 parameter 'semiPersistSchedIntervalDL' (see 38.214 and 38.321, section FFS_Section)

-- FFS_Value: Support also shorter periodicities for DL?

periodicity								ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640,

spare6, spare5, spare4, spare3, spare2, spare1}						,

-- Number of configured HARQ processes for SPS DL. Corresponds to L1 parameter 'numberOfConfSPS-Processes' (see 38.214, section FFS_Section)

nrofHARQ-Processes						INTEGER (1..8)																	,

-- HARQ resource for PUCCH for DL SPS. The network configures the resource either as format0 or format1. (see 38.214, section FFS_Section)

n1PUCCH-AN									PUCCH-Resource

OPTIONAL	-- Need M

}

-- TAG-SPS-CONFIG-STOP

-- ASN1STOP

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2).

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

-- FFS CHECK: Add possibility to release UL SPS

ConfiguredGrantConfig ::=				SEQUENCE {

-- Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see 38.213, section FFS_Section)

powerControlLoopToUse					ENUMERATED {n0, n1},

-- Index of the P0-PUSCH-AlphaSet to be used for this configuration

p0-PUSCH-Alpha							P0-PUSCH-AlphaSetId,

-- Enable transformer precoder for type1 and type2. Absence indicates that it is disabled.

-- Corresponds to L1 parameter 'UL-TWG-tp' (see 38.214, section 6.1.3)

transformPrecoder						ENUMERATED {enabled}															OPTIONAL,	-- Need R

-- The number of HARQ processes configured. It applies for both Type 1 and Type 2

-- Corresponds to L1 parameter 'UL-TWG-numbHARQproc' (see 38.321, section 5.8.2)

nrofHARQ-processes						INTEGER(1..16)															,

-- 	The number or repetitions of K:

repK									ENUMERATED {n1, n2, n4, n8},

-- If repetitions is used, this field indicates the redundancy version (RV) sequence to use.

-- Corresponds to L1 parameter 'UL-TWG-RV-rep' (see 38.321, section 5.8.2)

repK-RV									ENUMERATED {s1-0231, s2-0303, s3-0000}											OPTIONAL,

-- Periodicity for UL transmission without UL grant for type 1 and type 2

-- Corresponds to L1 parameter 'UL-TWG-periodicity' (see 38.321, section 5.8.2)

-- The following periodicities are supported depending on the configured subcarrier spacing [ms]:

--	15kHz: 2 symbols, 7 symbols, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640

--	30kHz: 2 symbols, 7 symbols, 0.5, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640

--	60kHz: 2 symbols, 7 symbols (6 symbols for ECP), 0.25,0.5,1,2,5,10,20,32, 40, 64, 80, 128, 160, 320, 640

--	120kHz: 2 symbols, 7 symbols, 0.125,0.25,0.5,1,2,5,10,20, 32, 40, 64, 80, 128, 160, 320, 640	OPTIONAL,

periodicity								ENUMERATED {sym2, sym7, ms0dot125, ms0dot25, ms0dot5, ms1, ms2, ms5, ms10, ms20,

ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640}				,

-- If configured, the UE uses the configured grant timer (see 38.321, section 5.8.2) with this initial timer value.

configuredGrantTimer					FFS_Value																		OPTIONAL,	-- Need R

-- Selection between "configured grant" transmission with fully RRC-configured UL grant (Type1)

-- or with UL grant configured by DCI addressed to CS-RNTI (Type2).

rrc-ConfiguredUplinkGrant		CHOICE {

type1									SEQUENCE {

-- Offset related to SFN=0

timeDomainOffset						INTEGER  (0..ffsValue),

-- Corresponding to the DCI field of time domain resource assignment, and the maximum bit width is 4.

--(see 38.214, section 6.1.2 and 38.212, section 7.3.1)

timeDomainAllocation					 INTEGER  (0..15), -- RAN1 indicated just "Mapping-type,Index-start-len"

-- Corresponding to the DCI field of freq domain resource assignment, and FFS the range.

-- (see 38.214, section 6.1.2, and 38.212, section 7.3.1)

frequencyDomainAllocation				INTEGER  (0..ffsValue),

-- UE-specific DMRS configuration: corresponding to the DCI field of antenna ports, and the maximum bitwidth is 5.

-- (see 38.214, section 6.1.2, and 38.212, section 7.3.1)

dmrs									INTEGER (0..31),

-- The modulation order, target code rate and TB size (see 38.214, section 6.1.2)

mcsAndTBS								INTEGER (0..31),

-- Enables intra-slot frequency hopping with the given frequency hopping offset

-- Corresponds to L1 parameter 'UL-TWG-hopping' (see 38.214, section FFS_Section)

frequencyHopping						SetupRelease {FFS_Value  }								OPTIONAL, -- Need M

...

},

type2							SEQUENCE {

...

}

}																			OPTIONAL  -- Need M

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

#### –	SRS-Config

The SRS-Config IE is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources. The network triggers the transmission of the set of SRS-Resources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

-- SRS configuration allowing to add and remove sets of SRS resources

SRS-Config ::= 							SEQUENCE {

srs-ResourceSetToReleaseList 			SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId			OPTIONAL, 	-- Need M

srs-ResourceSetToAddModList 			SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet				OPTIONAL, 	-- Need M

srs-ResourceToReleaseList 	 			SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId					OPTIONAL,	-- Need M

srs-ResourceToAddModList 	 			SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource					OPTIONAL, 	-- Need M

-- RNTI used for SRS TPC. Corresponds to L1 parameter 'TPC-SRS-RNTI' (see 38.213, section 10)

-- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity

-- and other PDCCH parameters (if any)

tpc-SRS-RNTI							RNTI-Value																	OPTIONAL,

-- If absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation

-- (this applies to SRS when a separate closed loop is configured for SRS)

-- Corresponds to L1 parameter 'Accumulation-enabled-srs' (see 38,213, section 7.3)

tpc-Accumulation							ENUMERATED {disabled}													OPTIONAL,	-- Need S

-- Includes parameters for configuration of carrier based SRS  switching

-- Corresponds to L1 parameter 'SRS-CarrierSwitching' (see 38,214, section FFS_Section)

-- FFS_CHECK: Check with RAN1 whether this was correctly moved by RAN2 to the top-level of SRS-Config (rather than at resource level)

carrierSwitching						SRS-CarrierSwitching														OPTIONAL,

...

}

-- A set of SRS resources

SRS-ResourceSet ::= 					SEQUENCE {

srs-ResourceSetId						SRS-ResourceSetId,

srs-ResourcesIds						SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId,

-- The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration.

-- Corresponds to L1 parameter 'AperiodicSRS-ResourceTrigger' (see 38.214, section FFS_Section)

aperiodicSRS-ResourceTrigger			INTEGER (0..maxNrofSRS-TriggerStates-1)					OPTIONAL,

-- ID of CSI-RS resource associated with SRS resource set in non-codebook based operation

-- Corresponds to L1 parameter 'SRS-AssocCSIRS' (see 38.214, section 6.2.1)

associatedCSI-RS-Index						NZP-CSI-RS-ResourceId														OPTIONAL, -- Cond nonCodebook

-- Indicates if the SRS resource set is used for beam management vs. used for either codebook based or non-codebook based transmission.

-- Corresponds to L1 parameter 'SRS-SetUse' (see 38.214, section 6.2.1)

-- FFS_CHECK: Isn't codebook/noncodebook already known from the ulTxConfig in the SRS-Config? If so, isn't the only distinction

-- in the set between BeamManagement, AtennaSwitching and "Other”? Or what happens if SRS-Config=Codebook but a Set=NonCodebook?

usage									ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching}		,

-- alpha value for SRS power control. Corresponds to L1 parameter 'alpha-srs' (see 38.213, section 7.3)

-- When the field is absent the UE applies the value 1

alpha									Alpha																		OPTIONAL, -- Need S

-- P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed.

-- Corresponds to L1 parameter 'p0-srs' (see 38.213, section 7.3)

p0										INTEGER (-202..24)															OPTIONAL, -- Need M

-- A reference signal (e.g. a CSI-RS config or a SSblock) to be used for SRS path loss estimation.

-- Corresponds to L1 parameter 'srs-pathlossReference-rs-config' (see 38.213, section 7.3)

pathlossReferenceRS						CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index							NZP-CSI-RS-ResourceId

}																													OPTIONAL, -- Need m

-- Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured)

-- or serarate close loop is configured for SRS. This parameter is applicable only for Uls on which UE also transmits PUSCH.

-- If absent or release, the UE applies the value sameAs-Fci1

-- Corresponds to L1 parameter ‘srs-pcadjustment-state-config’ (see 38.213, section 7.3)

srs-PowerControlAdjustmentStates		ENUMERATED { sameAsFci2, separateClosedLoop}					OPTIONAL, -- Need R

…

}

SRS-ResourceSetId ::= 					INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-Resource ::= 						SEQUENCE {

srs-ResourceId							SRS-ResourceId,

nrofSRS-Ports							ENUMERATED {port1, ports2, ports4},

-- Comb value (2 or 4) and comb offset (0..combValue-1). Corresponds to L1 paramet‘r 'SRS-TransmissionC’mb' (see 38.214, section 6.2.1)

transmissionComb						CHOICE {

n2										SEQUENCE {

combOffset-n2							INTEGER (0..1),

-- Cyclic shift configuration. Corresponds to L1 paramet‘r 'SRS-CyclicShiftCon’ig' (see 38.214, section 6.2.1)

cyclicShift-n2							INTEGER (0..7)

},

n4										SEQUENCE {

combOffset-n4							INTEGER (0..3),

-- Cyclic shift configuration. Corresponds to L1 paramet‘r 'SRS-CyclicShiftCon’ig' (see 38.214, section 6.2.1)

cyclicShift-n4							INTEGER (0..11)

}

}																													OPTIONAL,	-- Cond Setup

-- OFDM symbol location of the SRS resource within a slot including number of OFDM symbols (N = 1, 2 or 4 per SRS resource),

-- startPosition (SRSSymbolStartPosition = 0..“;”"0" refers to the last symbo“,”"1" refers to the second last symbol) and

-- RepetitionFactor (r = 1, 2 or 4).

-- Corresponds to L1 paramet‘r 'SRS-ResourceMapp’ng' (see 38.214, section 6.2.1 and 38.211, section 6.4.1.4).

-- FFS: Apparently, RAN1 considers replacing these three fields by a table in RAN1 specs and a corresponding index in ASN.1?!

resourceMapping							SEQUENCE {

startPosition							INTEGER (0..5),

nrofSymbols								ENUMERATED {n1, n2, n4},

repetitionFactor						ENUMERATED {n1, n2, n4}

},

-- Parameter(s) defining frequency domain position and configurable shift to align SRS allocation to 4 PRB grid.

-- Corresponds to L1 paramet‘r 'SRS-FreqDomainPosit’on' (see 38.214, section 6.2.1)

freqDomainPosition						INTEGER (0..67)													OPTIONAL,

freqDomainShift							INTEGER (0..268)															OPTIONAL,

-- Includes parameters capturing SRS frequency hopping

-- Corresponds to L1 paramet‘r 'SRS-FreqHopp’ng' (see 38.214, section 6.2.1)

freqHopping								SetupRelease { SEQUENCE {

c-SRS									INTEGER (0..63),

b-SRS									INTEGER (0..3),

b-hop									INTEGER (0..3)

} }																													OPTIONAL, -- Need M

-- Parameter(s) for configuring group or sequence hopping

-- Corresponds to L1 paramet‘r 'SRS-GroupSequenceHopp’ng' (see 38.211, section FFS_Section)

groupOrSequenceHopping					ENUMERATED { neither, groupHopping, sequenceHopping }						OPTIONAL,

-- Time domain behavior of SRS resource configuration.

-- Corresponds to L1 paramet‘r 'SRS-ResourceConfigT’pe' (see 38.214, section 6.2.1).

-- For codebook based uplink transmission, the network configures SRS resources in the same resource set with the same

-- time domain behavior on periodic, aperiodic and semi-persistent SRS.

-- FFS: Add configuration parameters for the different SRS resource types?

resourceType							CHOICE {

aperiodic								NULL,

semi-persistent							SEQUENCE {

-- Periodicity and slot offset for for this SRS resource. All values “n "number of sl”ts".

-- sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on.

-- For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots.

-- Corresponds to L1 paramet‘r 'SRS-SlotCon’ig' (see 38.214, section 6.2.1)

periodicityAndOffset-sp						SRS-PeriodicityAndOffset

},

periodic								SEQUENCE {

-- Periodicity and slot offset for for this SRS resource. All values “n "number of sl”ts"

-- sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on.

-- For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots.

-- Corresponds to L1 paramet‘r 'SRS-SlotCon’ig' (see 38.214, section 6.2.1)

periodicityAndOffset-sp						SRS-PeriodicityAndOffset

}

}																													OPTIONAL, -- Need M

“”

‘’

-- Sequence ID used to initialize psedo random group and sequence hopping.

-- Corresponds to L1 paramet‘r 'SRS-Sequenc’Id' (see 38.214, section 6.2.1)

sequenceId								BIT STRING (SIZE (10)),

‘’

‘’

-- Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS

-- Corresponds to L1 paramet‘r 'SRS-SpatialRelationI’fo' (see 38.214, section 6.2.1)

spatialRelationInfo						CHOICE {

ssb-Index								SSB-Index,

csi-RS-Index									NZP-CSI-RS-ResourceId,

srs										SRS-ResourceId

}																													OPTIONAL,

-- Subset of PMIs addressed by TPMI, where PMIs are those supported by UEs with maximum coherence capabilities of "fully coherent",

-- "partially coherent", or "non-coherent" transmission. Corresponds to L1 parameter 'ULCodebookSubset' (see 38.211, section 6.3.1.5)

codebookSubset			ENUMERATED {fullAndPartialAndNonCoherent, partialCoherent, nonCoherent}					OPTIONAL,

-- Subset of PMIs addressed by TRIs from 1 to ULmaxRank. Corresponds to L1 parameter 'ULmaxRank' (see 38.211, section 6.3.1.5)

maxRank									INTEGER (1..4)											OPTIONAL

}

SRS-ResourceId ::= 						INTEGER (0..maxNrofSRS-Resources-1)

SRS-PeriodicityAndOffset ::=			CHOICE {

sl1										NULL,

sl2										INTEGER(0..1),

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl32									INTEGER(0..31),

sl40									INTEGER(0..39),

sl64									INTEGER(0..63),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319),

sl640									INTEGER(0..639),

sl1280									INTEGER(0..1279),

sl2560									INTEGER(0..2559)

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-ResourceSet or SRS-Resource and optional (Need M) otherwise |

#### –	SRS-CarrierSwitching

The IE SRS-CarrierSwitching is used to configure FFS

SRS-CarrierSwitching information element

-- ASN1START

-- TAG-SRS-CARRIERSWITCHING-START

SRS-CarrierSwitching ::=				SEQUENCE {

-- Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell.

-- During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH

-- in the same CG to allow the PUSCH-less cell to transmit SRS. (see 38.214, section 6.2.1.3)

srs-SwitchFromServCellIndex				INTEGER (0..31)																OPTIONAL,	-- Cond Setup

-- Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any.

srs-TPC-PDCCH-Group					CHOICE {

-- Type A trigger configuration for SRS transmission on a PUSCH-less SCell.

-- Corresponds to L1 parameter 'typeA-SRS-TPC-PDCCH-Group' (see 38.212, 38.213, section 7.3.1, 11.3)

typeA								SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

-- Type B trigger configuration for SRS transmission on a PUSCH-less SCell.

-- Corresponds to L1 parameter 'typeB-SRS-TPC-PDCCH-Config' (see 38.212, 38.213, section 7.3.1, 11.3)

typeB								SRS-TPC-PDCCH-Config

}																													OPTIONAL, 	-- Cond Setup

-- Maps a specific cell to a given SFI value within the DCI message

-- Corresponds to L1 parameter 'SRS-cell-to-SFI' (see 38.212, 38.213, section 7.3.1, 11.3)

srs-CellToSFI						SEQUENCE (SIZE (1..maxNrofServingCells)) OF SlotFormatCombinationsPerCell			OPTIONAL, 	-- Cond Setup

-- Monitoring periodicity of SRS PDCCH in number of slots.

-- Corresponds to L1 parameter 'SRS-monitoring-periodicity' (see 38.212, 38.213, section 7.3.1, 11.3)

monitoringPeriodicity				ENUMERATED {n1, n2, n5, n10, n20, spare3, spare2, spare1}						OPTIONAL, 	-- Cond Setup

-- The number of PDCCH candidates for the configured aggregation level.

-- Corresponds to L1 parameter 'SRS-Num-PDCCH-cand' (see 38.212, 38.213, section 7.3.1, 11.3)

nrofPDCCH-Candidates				ENUMERATED {n1, n2}																,

-- A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI

-- Corresponds to L1 parameter 'SRS-monitoring-cells' (see 38.212, 38.213, section 7.3.1, 11.3)

monitoringCells		SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex						OPTIONAL	-- Cond Setup

}

-- One trigger configuration for SRS-Carrier Switching. (see 38.212, 38.213, section 7.3.1, 11.3)

SRS-TPC-PDCCH-Config ::=				SEQUENCE {

-- The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands

-- for a PUSCH-less SCell. (see 38.212, 38.213, section 7.3.1, 11.3)

startingBitOfFormat2-3					INTEGER (1..31)																OPTIONAL, 	-- Cond Setup

-- The type of a field within the group DCI with SRS request fields (optional) for a PUSCH-less SCell,

-- which indicates how many bits in the field are for SRS request (0 or 1/2).

-- Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its

-- own TPC command bits. See TS 38.212. Network configures this field with the same value for all PUSCH-less SCells.

-- (see 38.212, 38.213, section 7.3.1, 11.3)

fieldTypeFormat2-3						INTEGER (0..1)																OPTIONAL, 	-- Cond Setup

-- A list of paris of [cc-SetIndex; cc-IndexInOneCC-Set] (see 38.212, 38.213, section 7.3.1, 11.3)

-- FFS: Improve description. What is a “CC”? Where is a CC-Set defined? ...

srs-CC-SetIndexlist						SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex 									OPTIONAL	-- Cond Setup

}

SRS-CC-SetIndex ::=						SEQUENCE {

-- Indicates the CC set index for Type A associated (see 38.212, 38.213, section 7.3.1, 11.3)

cc-SetIndex								INTEGER (0..3)															OPTIONAL, 	-- Cond Setup

-- Indicates the CC index in one CC set for Type A (see 38.212, 38.213, section 7.3.1, 11.3)

cc-IndexInOneCC-Set						INTEGER (0..7)															OPTIONAL		-- Cond Setup

}

-- TAG-SRS-CARRIERSWITCHING-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon configuration of SRS-CarrierSwitching or SRS-TPC-PDCCH-Config and optional (Need M) otherwise |

#### –	SSB-Index

The IE SSB-Index identifies an SS-Block within an SS-Burst (FFS_CHECK: Is this correct?)

SSB-Index information element

-- ASN1START

-- TAG-SSB-INDEX-START

SSB-Index ::=								INTEGER (0..63)

-- TAG-SSB-INDEX-STOP

-- ASN1STOP

#### –	SubcarrierSpacing

The SubcarrierSpacing IE determines the subcarrier spacing.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIER-SPACING-START

-- Check value range! Currently used for subcarrierSpacingCommon (SIB1, Msg2, Msg4

SubcarrierSpacing ::= 					ENUMERATED {kHz15, kHz30, kHz60, kHz120}

-- 15 or 30 kHz (<6GHz), 120 and 240 kHz (>6GHz).

SubcarrierSpacingSSB ::= 				ENUMERATED {kHz15, kHz30, kHz120, kHz240}

-- FFS_CHECK: Can probably be removed since PRACH Msg1 uses the SubcarrierSpacing values above.

SubcarrierSpacingRACH ::= 				ENUMERATED {ffsTypeAndValue}

-- 15, 30 or 60 kHz  (<6GHz), 60 or 120 kHz (>6GHz)

SubcarrierSpacingCSI-RS ::=				ENUMERATED {kHz15, kHz30, khz60, kHz120}

-- TAG-SUBCARRIER-SPACING-STOP

-- ASN1STOP

#### –	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 				SEQUENCE {

tci-StateId					TCI-StateId,

qcl-Type1					QCL-Info,

qcl-Type2					QCL-Info																					OPTIONAL

}

TCI-StateId ::=				INTEGER (0..ffsValue)

QCL-Info ::=				SEQUENCE {

referenceSignal				CHOICE {

csi-rs						NZP-CSI-RS-ResourceConfigId,

ssb							SSB-Id,

-- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a NZP-CSI-ResourceSetId

trs							NZP-CSI-ResourceSetId

},

qcl-Type					ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

#### –	TDD-UL-DL-Config

The TDD-UL-DL-Config IEs determines the Uplink/Downlink TDD configuration. There are both, UE- and cell specific IEs.

TDD-UL-DL-Config information element

-- ASN1START

-- TAG-TDD-UL-DL-CONFIG-START

TDD-UL-DL-ConfigCommon ::=			SEQUENCE {

-- Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific

-- virtual carriers, i.e., independent of the actual subcarrier spacing using for data transmission.

-- Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section)

referenceSubcarrierSpacing			SubcarrierSpacing																	OPTIONAL,

-- Periodicity of the DL-UL pattern. Corresponds to L1 parameter 'DL-UL-transmission-periodicity' (see 38.211, section FFS_Section)

dl-UL-TransmissionPeriodicity		ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}						OPTIONAL,

-- Number of consecutive full DL slots at the beginning of each DL-UL pattern.

-- Corresponds to L1 parameter 'number-of-DL-slots' (see 38.211, Table 4.3.2-1)

nrofDownlinkSlots					INTEGER (0..maxNrofSlots)																	OPTIONAL,

-- Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots).

-- If the field is absent or released, there is no partial-downlink slot.

-- Corresponds to L1 parameter 'number-of-DL-symbols-common' (see 38.211, section FFS_Section).

nrofDownlinkSymbols					INTEGER (0..maxNrofSymbols-1)															OPTIONAL,	-- Need R

-- Number of consecutive full UL slots at the end of each DL-UL pattern.

-- Corresponds to L1 parameter 'number-of-UL-slots' (see 38.211, Table 4.3.2-1)

nrofUplinkSlots						INTEGER (0..maxNrofSlots)																	OPTIONAL,

-- Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots).

-- If the field is absent or released, there is no partial-uplink slot.

-- Corresponds to L1 parameter 'number-of-UL-symbols-common' (see 38.211, section FFS_Section)

nrofUplinkSymbols					INTEGER (0..maxNrofSymbols-1)															OPTIONAL	-- Need R

}

TDD-UL-DL-ConfigDedicated ::=		SEQUENCE {

-- The slotSpecificConfiguration allows overriding UL/DL allocations provided in tdd-UL-DL-configurationCommon.

-- FFS_ASN1: Consider making this an AddMod/Release list

-- FFS_ASN1: Replace absolute numbers by variables... once RAN1 confirms.

-- FFS_CHECK: This list will grow very large if used for many slots.

slotSpecificConfigurationsToAddModList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotConfig

OPTIONAL, -- Need N

slotSpecificConfigurationsToreleaseList		SEQUENCE (SIZE (1..maxNrofSlots)) OF TDD-UL-DL-SlotIndex						OPTIONAL -- Need N

}

TDD-UL-DL-SlotConfig ::=			SEQUENCE {

-- Identifies a slot within a dl-UL-TransmissionPeriodicity (given in tdd-UL-DL-configurationCommon)

slotIndex							TDD-UL-DL-SlotIndex,

-- The direction (downlink or uplink) for the symbols in this slot. "allDownlink" indicates that all symbols in this slot are used

-- for downlink; "allUplink" indicates that all symbols in this slot are used for uplink; "explicit" indicates explicitly how many symbols

-- in the beginning and end of this slot are allocated to downlink and uplink, respectively.

symbols								CHOICE {

allDownlink							NULL,

allUplink							NULL,

explicit							SEQUENCE {

-- Number of consecutive DL symbols in the beginning of the slot identified by slotIndex.

-- If the field is absent the UE assumes that there are no leading DL symbols.

-- Corresponds to L1 parameter 'number-of-DL-symbols-dedicated' (see 38.211, section FFS_Section)

nrofDownlinkSymbols					INTEGER (1..maxNrofSymbols-1)												OPTIONAL,	-- Need R

-- Number of consecutive UL symbols in the end of the slot identified by slotIndex.

-- If the field is absent the UE assumes that there are no trailing UL symbols.

-- Corresponds to L1 parameter 'number-of-UL-symbols-dedicated' (see 38.211, section FFS_Section)

nrofUplinkSymbols					INTEGER (1..maxNrofSymbols-1)												OPTIONAL	-- Need R

}

}

}

TDD-UL-DL-SlotIndex ::=				INTEGER (0..maxNrofSlots-1)

-- TAG-TDD-UL-DL-CONFIG-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a A Zero-Power (ZP) CSI-RS resource. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfig' (see 38.214, section 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=						SEQUENCE {

-- ZP CSI-RS resource configuration ID

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigId' (see 38.214, section FFS_Section)

zp-CSI-RS-ResourceId						ZP-CSI-RS-ResourceId,

-- OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceMapping' (see 38.214, section FFS_Section)

resourceMapping								SEQUENCE {

-- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1. FFS: Table correct?

-- The number of bits that may be set to one depend on the chosen row in that table.

frequencyDomainAllocation					CHOICE {

row1										BIT STRING (SIZE (4)),

row2										BIT STRING (SIZE (12)),

row4										BIT STRING (SIZE (3)),

other										BIT STRING (SIZE (6))

},

-- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS.

firstOFDM-SymbolInTimeDomain				INTEGER (0..13)

}																												OPTIONAL,

-- Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS

-- Corresponds to L1 parameter 'ZP-CSI-RS-timeConfig' (see 38.214, section FFS_Section)

periodicityAndOffset						CHOICE {

sl4											INTEGER (0..3),

sl5											INTEGER (0..4),

sl8											INTEGER (0..7),

sl10										INTEGER (0..9),

sl16										INTEGER (0..15),

sl20										INTEGER (0..19),

sl32										INTEGER (0..31),

sl40										INTEGER (0..39),

sl64										INTEGER (0..63),

sl80										INTEGER (0..79),

sl160										INTEGER (0..159),

sl320										INTEGER (0..319),

sl640										INTEGER (0..639)

}																															OPTIONAL,

-- Includes parameters to enbale configuration of frequency-occupancy of ZP-CSI)RS

-- Corresponds to L1 parameter 'ZP-CSI-RS-FreqBand' (see 38.214, section FFS_Section)

freqBand									SEQUENCE {

-- PRB where this NZP-CSI-RS-Resource starts in relation to PRB 0 of the associated BWP. Only multiples of 4 are allowed (0, 4, ...)

startingRB									INTEGER (0..maxNrofPhysicalResourceBlocks-1),

-- Number of PRBs across which this NZP-CSI-RS-Resource spans. Only multiples of 4 are allowed. The smallest configurable

-- number is the minimum of 24 and the width of the associated BWP.

nrofRBs										INTEGER (24..maxNrofPhysicalResourceBlocks)

}																															,

-- Density of ZP-CSI-RS resource measured in RE/port/PRB.

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

-- Corresponds to L1 parameter 'ZP-CSI-RS-Density' (see 38.214, section FFS_Section)

density											CHOICE {

dot5											ENUMERATED {evenPRBs, oddPRBs},

one												NULL,

three											NULL,

spare											NULL

}																															,

-- Time domain behavior of ZP-CSI-RS resource configuration.

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigType' (see 38.214, section FFS_Section)

resourceType									ENUMERATED {aperiodic, periodic}											,

-- QCL type for source RS ==> target RS association. Corresponds to L1 parameter 'QCL-Type' (see 38.214, section FFS_Section)

qcl-Type										ENUMERATED {typeA, typeB, typeC, typeD}

}

ZP-CSI-RS-ResourceId ::=							INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

### 6.3.3	UE capability information elements

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations.

BandCombinationList information element

-- ASN1START

-- TAG-BAND-COMBINATION-LIST-START

BandCombinationList ::=	SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombination ::= SEQUENCE {

bandAndParametersDLList			BandAndDL-ParametersList,

bandCombinationsUL				BIT STRING (SIZE (1.. maxBandCombUL))   OPTIONAL

}

-- Bands and DL band parameters

BandAndDL-ParametersList ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandAndDL-Parameters

BandAndDL-Parameters ::= SEQUENCE {

frequencyBand					FreqBandInformation,

bandParametersDL				BandParametersDL			OPTIONAL  -- Not included in case of SUL

}

-- UL band combinations (without signalling of frequency bands)

BandParameterCombinationListUL ::= SEQUENCE (SIZE (1..maxBandCombUL)) OF BandParameterCombinationUL

BandParameterCombinationUL ::= SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF BandParametersUL

BandParametersUL ::= SEQUENCE {

bandParametersUL					BandParametersUL			OPTIONAL  -- Not included in case of DL-only band

}

-- Others

FreqBandInformation::= CHOICE {

bandEUTRA             				FreqBandIndicatorEUTRA,

bandNR                				FreqBandIndicatorNR

}

BandParametersDL ::= SEQUENCE {

bandwidthClassInfoDL			CHOICE {

ca-BandwidthClassDL-EUTRA		CA-BandwidthClassDL-EUTRA,

ca-BandwidthClassDL-NR			CA-BandwidthClassDL-NR

},

...

}

BandParametersUL ::= SEQUENCE {

bandwidthClassInfoUL			CHOICE {

ca-BandwidthClassUL-EUTRA		CA-BandwidthClassUL-EUTRA,

ca-BandwidthClassUL-NR			CA-BandwidthClassUL-NR

},

...

-- FFS How to address NC CA in relation to carrier separation

-- intraBandSimultaneousTxRx will be added with FFS (per UE or per band combination)

-- multipleTimingAdvance will be added with FFS (per UE or per band combination)

-- singleTx will be included per band combination

-- scalingFactor will be included per band per band combination

}

-- TAG-BAND-COMBINATION-LIST-STOP

-- ASN1STOP

BandCombinationList field descriptions

bandCombinationsUL Bit string with pointers to entries in BandCombinationListUL. Only the UL combinations of the same number of entries as in bandAndParametersDLList can be pointed to.

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, mrdc, spare1, ...}

-- FFS utra, geran-cs, geran-ps and cdma2000-1XRTT

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ue-CapabilityRAT-Container			OCTET STRING

}

-- TAG-UE-CAPABILITY-RAT-CONTAINER-LIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For NR: the encoding of UE capabilities is defined in UE-NR-Capability. For MRDC: the encoding of UE capabilities is defined in UE-MRDC-Capability For E UTRA: the octet string contains the UE-EUTRA-Capability as defined in TS 36.331 [xx].

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=	SEQUENCE {

measParameters-MRDC			MeasParameters-MRDC,

rf-Parameters-MRDC			RF-Parameters-MRDC,

phyLayerParameters-MRDC		PhyLayerParameters-MRDC

-- FFS on other parameters

}

RF-Parameters-MRDC ::= SEQUENCE {

supportedBandCombination	BandCombinationList

-- FFS on other parameters

}

PhyLayerParameters-MRDC ::= SEQUENCE {

supportedBasebandProcessingCombination-MRDC		BasebandProcessingCombination-MRDC

-- FFS if supportedBasebandProcessingCombination-MRDC is included here or BandCombinationList

-- FFS on other parameters

}

BasebandProcessingCombination-MRDC ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF LinkedBasebandProcessingCombination

LinkedBasebandProcessingCombination ::= SEQUENCE {

basebandProcessingCombinationIndex-EUTRAN		BasebandProcessingCombinationIndex,

basebandProcessingCombinationLinkedIndex-NR		SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

}

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

MeasParameters-MRDC ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-MeasType1				ENUMERATED {supported}	OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PhyLayerParameters ::=	SEQUENCE {

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination

-- FFS on other parameters

}

RF-Parameters ::= SEQUENCE {

supportedBandListNR					SupportedBandListNR,

supportedBandCombination			BandCombinationList,

intraBandAsyncFDD					ENUMERATED {supported}	OPTIONAL

-- FFS Whether intraBandAsyncFDD is included per UE or per band combination

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF BandNR

SupportedBandCombination ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

SupportedBasebandProcessingCombination ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombination

BasebandProcessingCombination ::= SEQUENCE {

basebandParametersPerBand		SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBand

-- FFS on other parameters

}

BasebandParametersPerBand ::= SEQUENCE {

ca-BandwidthClassDL			CA-BandwidthClass,

ca-BandwidthClassUL			CA-BandwidthClass,

basebandParametersPerCC		SEQUENCE (SIZE (1..maxNrofCC)) OF BasebandParametersPerCC,

supportedBW-PerCC			BW-PerCC

-- FFS on the need (e.g. if ca-BandwidthClass is sufficient to cover BW-PerCC)

-- FFS on other parameters

}

BasebandParametersPerCC ::= SEQUENCE {

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL,

modulationOrder					ModulationOrder,

subCarrierSpacing				SubCarrierSpacing

-- FFS if modulationOrder and subCarrierSpacing are included per Band or per CC

-- FFS on other parameters

}

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicatorNR,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL

-- FFS on other parameters

}

CA-BandwidthClass ::= ENUMERATED {a, b, c, d, e, f, ...}

MIMO-Capability ::= SEQUENCE {

-- FFS on the parameters

}

ModulationOrder ::= SEQUENCE {

-- FFS on the parameters

}

SubCarrierSpacing ::= SEQUENCE {

-- FFS on the parameters

}

PDCP-Parameters ::= SEQUENCE {

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL,

volteOverNR-PDCP				ENUMERATED {supported}	OPTIONAL

}

RLC-Parameters ::= SEQUENCE {

am-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WithShortSN					ENUMERATED {supported}	OPTIONAL,

um-WIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4, ...}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4, ...}	OPTIONAL -- FFS value range

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

### 6.3.4	Other information elements

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb								INTEGER ::=	ffsValue		-- Maximum number of DL band combinations

maxBasebandProcComb						INTEGER ::=	ffsValue		-- Maximum number of base band processing combinations

maxNrofServingCells						INTEGER ::=	16		-- Max number of serving cells (SpCell + SCells) per cell group

maxNrofServingCells-1					INTEGER ::= 15		-- Max number of serving cells (SpCell + SCells) per cell group minus 1

maxNrofSCells							INTEGER ::=	15		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	ffsValue		-- Maximum number of entries in each of the cell lists in a measurement object

maxNrofSS-BlocksToAverage				INTEGER ::= ffsValue		-- Max number for the (max) number of SS blocks to average to determine cell

-- measurement

maxNrofCSI-RS-ResourcesToAverage		INTEGER ::= ffsValue		-- Max number for the (max) number of CSI-RS to average to determine cell

-- measurement

maxNrofDL-Allocations 					INTEGER ::= ffsValue

maxNrofSR-ConfigPerCellGroup			INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

maxLC-ID								INTEGER ::= ffsValue		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs					INTEGER ::= 4		-- Maximum number of BWPs per serving cell

maxNrofSymbols-1							INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots							INTEGER ::= 320		-- Maximum number of slots in a 10 ms period

maxNrofSlots-1							INTEGER ::= 319		-- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs

maxNrofControlResourceSets 				INTEGER ::= 12 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= 11  	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces						INTEGER ::= 40		-- Max number of Search Spaces

maxNrofSearchSpaces-1					INTEGER ::= 339		-- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxINT-DCI-PayloadSize					INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1				INTEGER ::= ffsValue		-- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns				INTEGER ::= 4		-- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1				INTEGER ::= 3		-- Max number of rate matching patterns that may be configured minus 1

maxNrofCSI-Reports						INTEGER ::= ffsValue 	-- Maximum number of report configurations

maxNrofCSI-RS-CellsRRM 					INTEGER ::= ffsValue 	-- Maximum number of FFS

maxNrofReportConfigIdsPerTrigger		INTEGER ::= 16	 	-- Maximum number of report configurations per reportTrigger

maxNrofCSI-ResourceConfigurations		INTEGER ::= ffsValue		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= ffsValue		-- Maximum number of resource configurations minus 1

maxNrofCSI-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration

maxNrofCSI-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of resource sets per resource configuration minus 1

maxNrofFailureDetectionResources		INTEGER ::= ffsValue		-- Maximum number of failure detection resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= ffsValue		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= 3		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= 2		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= ffsValue		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofCSI-RS-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-RS resources per resource set

maxNrofCSI-MeasId						INTEGER ::= ffsValue		-- Maximum number of link configurations

maxNrofCSI-MeasId-1						INTEGER ::= ffsValue		-- Maximum number of link configurations minus 1

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= ffsValue		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofObjectId							INTEGER ::= ffsValue		-- Maximum number of configured measurement objects

maxNrofPCI-Ranges						INTEGER ::= ffsValue		-- Maximum number of PCI ranges

maxReportConfigId					INTEGER ::= ffsValue		-- Maximum number of reporting configurations

maxNrofMeasId							INTEGER ::= ffsValue		-- Maximum number of configured measurements

maxNrofQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofSRS-ResourceSets					INTEGER ::= ffsValue		-- Maximum number of SRS resource sets.

maxNrofSRS-ResourceSets-1				INTEGER ::= ffsValue		-- Maximum number of SRS resource sets minus 1.

maxNrofSRS-Resources					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= ffsValue		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxNrofSRS-TriggerStates-1 				INTEGER ::= 3		-- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxRAT-CapabilityContainers				INTEGER ::= ffsValue		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands					INTEGER ::= ffsValue		-- Maximum number of simultaneously aggregated bands

maxNrofSlotFormatCombinationsPerSet		INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1	INTEGER ::= ffsValue		-- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofPUCCH-ResourceSets				INTEGER ::= 4		-- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1				INTEGER ::= 3		-- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-ResourcesPerSet-1			INTEGER ::= 7		-- Maximum number of PUCCH Resources per PUCCH-ResourceSet minus 1.

maxNrofPUCCH-P0-PerSet					INTEGER ::= 8		-- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofP0-PUSCH-AlphaSets				INTEGER ::= 30		-- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1)

maxNrofP0-PUSCH-AlphaSets-1				INTEGER ::= 29		-- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1)

maxNrofPUSCH-PathlossReferenceRSs		INTEGER ::= 4		-- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1	INTEGER ::= 3		-- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxEARFCN								INTEGER ::= 262143	-- Highest value of extended E-UTRA EARFCN range

maxBands 								INTEGER ::= ffsValue

maxCellPrep		 						INTEGER ::= ffsValue

maxCellReport		 					INTEGER ::= ffsValue

maxCellSCG		 						INTEGER ::= ffsValue

maxDRB		 							INTEGER ::= ffsValue

maxFreq		 							INTEGER ::= ffsValue

maxLCH									INTEGER ::= ffsValue

maxNrofCSI-RS	 						INTEGER ::= ffsValue

maxNrofAggregatedCellsPerCellGroup 		INTEGER ::= ffsValue

maxNrofCandidateBeams 					INTEGER ::= ffsValue

maxNrofCSI-ReportConfig-1 				INTEGER ::= ffsValue

maxNrofCSI-ResrouceConfigurations 		INTEGER ::= ffsValue

maxNrofPCIsPerSMTC 						INTEGER ::= ffsValue

maxNrofQFIs 							INTEGER ::= ffsValue

maxNrofSR-Resoruces		 		INTEGER ::= ffsValue

maxNrofSlotFormatsPerCombination 		INTEGER ::= ffsValue

maxNrofSpatialRelationInfos 			INTEGER ::= ffsValue

maxNrofSRS-ResourcesPerSet 				INTEGER ::= ffsValue

maxNrofIndexesToReport 					INTEGER ::= ffsValue

maxNrofSSBs 							INTEGER ::= ffsValue

maxNrofTCI-StatesPDCCH 					INTEGER ::= ffsValue

maxNrofTCI-States						INTEGER ::= 64

maxNrofTCI-States-1						INTEGER ::= 63

maxNrofUL-Allocations 					INTEGER ::= ffsValue

maxQFI 									INTEGER ::= ffsValue

maxRA-CSIRS-Resources 					INTEGER ::= ffsValue

maxRA-SSB-Resources 						INTEGER ::= ffsValue

maxSCSs									INTEGER ::= ffsValue

maxSecondaryCellGroups 							INTEGER ::= ffsValue

ffsValue								INTEGER ::= 64

-- IE definitions introduced to not get warning at ASN.1 syntax check

CandidateRS-IndexInfoList ::=	ENUMERATED {ffsTypeAndValue}

CellIdentity ::=				ENUMERATED {ffsTypeAndValue}

CSI-RS-Index ::=					ENUMERATED {ffsTypeAndValue}

FilterCoefficient ::=			ENUMERATED {ffsTypeAndValue}

Hysteresis ::=					ENUMERATED {ffsTypeAndValue}

MeasObjectEUTRA ::=				ENUMERATED {ffsTypeAndValue}

MeasResultListEUTRA ::=			ENUMERATED {ffsTypeAndValue}

MeasResultSSTD ::=				ENUMERATED {ffsTypeAndValue}

PDU-SessionID ::=				ENUMERATED {ffsTypeAndValue}

PhyCellNR ::=					ENUMERATED {ffsTypeAndValue}

PhysCellIdEUTRA ::=				ENUMERATED {ffsTypeAndValue}

PhysCellIdRange ::=				ENUMERATED {ffsTypeAndValue}

P-Max ::=						ENUMERATED {ffsTypeAndValue}

RA-Resources ::=				ENUMERATED {ffsTypeAndValue}

ReportConfigEUTRA ::=			ENUMERATED {ffsTypeAndValue}

RRC-TransactionIdentifier ::=	ENUMERATED {ffsTypeAndValue}

SchedulingRequestId ::=			ENUMERATED {ffsTypeAndValue}

ShortMAC-I ::=					ENUMERATED {ffsTypeAndValue}

SSB-Id ::=						ENUMERATED {ffsTypeAndValue}

TimeToTrigger ::=				ENUMERATED {ffsTypeAndValue}

UECapabilityInformation ::=	ENUMERATED {ffsTypeAndValue}

BW-PerCC ::=		ENUMERATED {ffsTypeAndValue}

FFS_Value ::=		ENUMERATED {ffsTypeAndValue}

FreqBandIndicatorNR ::=		ENUMERATED {ffsTypeAndValue}

MBSFN-SubframeConfigList ::=		ENUMERATED {ffsTypeAndValue}

NZP-CSI-RS-ResourceConfigId ::=		ENUMERATED {ffsTypeAndValue}

SlotFormatIndicator ::=		ENUMERATED {ffsTypeAndValue}

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

# 7	Variables and constants

## 7.1	Timers

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T304 | Reception of RRCConnectionReconfiguration message including MobilityControlInfoSCG | Successful completion of random access on the PSCell, upon initiating re-establishment and upon SCG release | Inform E-UTRAN/NR about the SCG change failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, and upon initiating the connection re-establishment procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Selection of a suitable NR cell or a cell using another RAT. | Enter RRC_IDLE |

### 7.1.2	Timer handling

When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

## 7.2	Counters

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |

## 7.3	Constants

| Constant | Usage |
| --- | --- |
| N310 | Maximum number of consecutive "out-of-sync" indications for the PCell received from lower layers |
| N311 | Maximum number of consecutive "in-sync" indications for the PCell received from lower layers |

## 7.4	UE variables

NOTE: 	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

MeasId,

MeasIdToAddModList,

MeasObjectToAddModList,

PhysCellIdEUTRA,

PhyCellNR,

ReportConfigToAddModList,

RSRP-Range,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId

FROM NR-RRC-Definitions;

-- ASN1STOP

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

VarMeasConfig UE variable

-- ASN1START

-- TAG-VAR-MEAS-CONFIG-START

VarMeasConfig ::=					SEQUENCE {

-- Measurement identities

measIdList							MeasIdToAddModList					OPTIONAL,

-- Measurement objects

measObjectList						MeasObjectToAddModList				OPTIONAL,

-- Reporting configurations

reportConfigList					ReportConfigToAddModList			OPTIONAL,

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,

s-MeasureConfig							CHOICE {

ssb-RSRP								RSRP-Range,

csi-RSRP								RSRP-Range

} 																		OPTIONAL

}

-- TAG-VAR-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Revisit whether we really need VarMeasConfig.

#### –	VarMeasReportList

The UE variable VarMeasReportList includes information about the measurements for which the triggering conditions have been met.

VarMeasReportList UE variable

-- ASN1START

-- TAG-VAR-MEAS-REPORT-START

VarMeasReportList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF VarMeasReport

VarMeasReport ::=					SEQUENCE {

-- List of measurement that have been triggered

measId								MeasId,

cellsTriggeredList					CellsTriggeredList				OPTIONAL,

numberOfReportsSent					INTEGER

}

CellsTriggeredList ::=				SEQUENCE (SIZE (1.. maxNrofCellMeas)) OF CHOICE {

physCellIdEUTRA						PhysCellIdEUTRA,

phyCellNR							PhyCellNR

}

-- TAG-VAR-MEAS-REPORT-STOP

-- ASN1STOP

#### –	End of NR-UE-Variables

-- ASN1START

END

-- ASN1STOP

# 8	Protocol data unit abstract syntax

## 8.1	General

The RRC PDU contents in clause 6 and clause 10 are described using abstract syntax notation one (ASN.1) as specified in ITU-T Rec. X.680 [6] and X.681 [7]. Transfer syntax for RRC PDUs is derived from their ASN.1 definitions by use of Packed Encoding Rules, unaligned as specified in ITU-T Rec. X.691 [8].

The following encoding rules apply in addition to what has been specified in X.691:

-	When a bit string value is placed in a bit-field as specified in 15.6 to 15.11 in X.691, the leading bit of the bit string value shall be placed in the leading bit of the bit-field, and the trailing bit of the bit string value shall be placed in the trailing bit of the bit-field.

NOTE:	The terms 'leading bit' and 'trailing bit' are defined in ITU-T Rec. X.680. When using the 'bstring' notation, the leading bit of the bit string value is on the left, and the trailing bit of the bit string value is on the right.

-	When decoding types constrained with the ASN.1 Contents Constraint ("CONTAINING"), automatic decoding of the contained type should not be performed because errors in the decoding of the contained type should not cause the decoding of the entire RRC message PDU to fail. It is recommended that the decoder first decodes the outer PDU type that contains the OCTET STRING or BIT STRING with the Contents Constraint, and then decodes the contained type that is nested within the OCTET STRING or BIT STRING as a separate step.

-	When decoding a) RRC message PDUs, b) BIT STRING constrained with a Contents Constraint, or c) OCTET STRING constrained with a Contents Constraint, PER decoders are required to never report an error if there are extraneous zero or non-zero bits at the end of the encoded RRC message PDU, BIT STRING or OCTET STRING.

## 8.2	Structure of encoded RRC messages

An RRC PDU, which is the bit string that is exchanged between peer entities/across the radio interface contains the basic production as defined in X.691.

RRC PDUs shall be mapped to and from PDCP SDUs (in case of DCCH) or RLC SDUs (in case of PCCH, BCCH or CCCH) upon transmission and reception as follows:

-	when delivering an RRC PDU as an PDCP SDU to the PDCP layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the PDCP SDU and onwards; and

-	when delivering an RRC PDU as an RLC SDU to the RLC layer for transmission, the first bit of the RRC PDU shall be represented as the first bit in the RLC SDU and onwards; and

-	upon reception of an PDCP SDU from the PDCP layer, the first bit of the PDCP SDU shall represent the first bit of the RRC PDU and onwards; and

-	upon reception of an RLC SDU from the RLC layer, the first bit of the RLC SDU shall represent the first bit of the RRC PDU and onwards.

## 8.3	Basic production

The 'basic production' is obtained by applying UNALIGNED PER to the abstract syntax value (the ASN.1 description) as specified in X.691. It always contains a multiple of 8 bits.

## 8.4	Extension

The following rules apply with respect to the use of protocol extensions:

-	A transmitter compliant with this version of the specification shall, unless explicitly indicated otherwise on a PDU type basis, set the extension part empty. Transmitters compliant with a later version may send non-empty extensions;

-	A transmitter compliant with this version of the specification shall set spare bits to zero;

## 8.5	Padding

If the encoded RRC message does not fill a transport block, the RRC layer shall add padding bits. This applies to PCCH and BCCH.

Padding bits shall be set to 0 and the number of padding bits is a multiple of 8.

Figure 8.5-1: RRC level padding

# 9	Specified and default radio configurations

Specified and default configurations are configurations of which the details are specified in the standard. Specified configurations are fixed while default configurations can be modified using dedicated signalling.

Editor’s Note: FFS / FIXME: Default configurations

## 9.1	Specified configurations

Editor’s Note: FFS

### 9.1.1	Logical channel configurations

### 9.1.2	SRB configurations

#### 9.1.2.1	SRB1/SRB1S

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 1 |  |  |

#### 9.1..2.2	SRB2/SRB2S

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 2 |  |  |

#### 9.1.2.3	SRB3

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  |  |  |
| logicalChannelIdentity | 3 |  |  |

## 9.2	Default radio configurations

### 9.2.1	SRB configurations

#### 9.2.1.1	SRB1/SRB1S

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms25 FFS ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 1 | Highest priority |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | FFS |  |  |

#### 9.2.1.2	SRB2/SRB2S

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms25 FFS ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 3 |  |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | FFS |  |  |

#### 9.2.1.3	SRB3

Parameters (FFS)

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC-Config CHOICE | am |  |  |
| ul-RLC-Config >sn-FieldLength >t-PollRetransmit >pollPDU >pollByte >maxRetxThreshold | size12 ms45 infinity infinity t4 |  |  |
| dl-RLC-Config >sn-FieldLength >t-Reassembly >t-StatusProhibit | size12 ms25 FFS ms0 |  |  |
| LogicalChannelConfig |  |  |  |
| >priority | 1 | Highest priority |  |
| >prioritisedBitRate | infinity |  |  |
| >bucketSizeDuration | N/A |  |  |
| >allowedSubCarrierSpacing | FFS |  |  |
| >allowedTiming | FFS |  |  |
| >logicalChannelGroup | 0 |  |  |
| >logicalChannelSR-DelayTimerApplied | FFS |  |  |

# 10	Generic error handling

## 10.1	General

The generic error handling defined in the subsequent sub-clauses applies unless explicitly specified otherwise e.g. within the procedure specific error handling.

The UE shall consider a value as not comprehended when it is set:

-	to an extended value that is not defined in the version of the transfer syntax supported by the UE.

-	to a spare or reserved value unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved value.

The UE shall consider a field as not comprehended when it is defined:

-	as spare or reserved unless the specification defines specific behaviour that the UE shall apply upon receiving the concerned spare/reserved field.

## 10.2	ASN.1 violation or encoding error

The UE shall:

1>	when receiving an RRC message on the [FFS] for which the abstract syntax is invalid [6]:

2>	ignore the message;

NOTE:	This section applies in case one or more fields is set to a value, other than a spare, reserved or extended value, not defined in this version of the transfer syntax. E.g. in the case the UE receives value 12 for a field defined as INTEGER (1..11). In cases like this, it may not be possible to reliably detect which field is in the error hence the error handling is at the message level.

## 10.3	Field set to a not comprehended value

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that has a value that the UE does not comprehend:

2>	if a default value is defined for this field:

3>	treat the message while using the default value defined for this field;

2>	else if the concerned field is optional:

3>	treat the message as if the field were absent and in accordance with the need code for absence of the concerned field;

2>	else:

3>	treat the message as if the field were absent and in accordance with sub-clause 10.4;

## 10.4	Mandatory field missing

The UE shall:

1>	if the message includes a field that is mandatory to include in the message (e.g. because conditions for mandatory presence are fulfilled) and that field is absent or treated as absent:

2>	if the RRC message was received on DCCH or CCCH:

3>	ignore the message;

2>	else:

3>	if the field concerns a (sub-field of) an entry of a list (i.e. a SEQUENCE OF):

4>	treat the list as if the entry including the missing or not comprehended field was not present;

3>	else if the field concerns a sub-field of another field, referred to as the 'parent' field i.e. the field that is one nesting level up compared to the erroneous field:

4>	consider the 'parent' field to be set to a not comprehended value;

4>	apply the generic error handling to the subsequent 'parent' field(s), until reaching the top nesting level i.e. the message level;

3>	else (field at message level):

4>	ignore the message;

NOTE 1:	The error handling defined in these sub-clauses implies that the UE ignores a message with the message type or version set to a not comprehended value.

NOTE 2:	The nested error handling for messages received on logical channels other than DCCH and CCCH applies for errors in extensions also, even for errors that can be regarded as invalid NR operation e.g. NR not observing conditional presence.

The following ASN.1 further clarifies the levels applicable in case of nested error handling for errors in extension fields.

-- /example/ ASN1START

-- Example with extension addition group

ItemInfoList ::=					SEQUENCE (SIZE (1..max)) OF ItemInfo

ItemInfo ::=						SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need N

...

[[	field3-r9						Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9						Field4-r9				OPTIONAL			-- Need N

]]

}

-- Example with traditional non-critical extension (empty sequence)

BroadcastInfoBlock1 ::=				SEQUENCE {

itemIdentity						INTEGER (1..max),

field1								Field1,

field2								Field2					OPTIONAL,			-- Need N

nonCriticalExtension				BroadcastInfoBlock1-v940-IEs	OPTIONAL

}

BroadcastInfoBlock1-v940-IEs::=	SEQUENCE {

field3-r9							Field3-r9				OPTIONAL,			-- Cond Cond1

field4-r9							Field4-r9				OPTIONAL,			-- Need N

nonCriticalExtension				SEQUENCE {}				OPTIONAL			-- Need S

}

-- ASN1STOP

The UE shall, apply the following principles regarding the levels applicable in case of nested error handling:

-	an extension additon group is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire itemInfo entry to be ignored (rather than just the extension addition group containing field3 and field4)

-	 a traditional nonCriticalExtension is not regarded as a level on its own. E.g. in the ASN.1 extract in the previous, a error regarding the conditionality of field3 would result in the entire BroadcastInfoBlock1 to be ignored (rather than just the non critical extension containing field3 and field4).

## 10.5	Not comprehended field

The UE shall, when receiving an RRC message on any logical channel:

1>	if the message includes a field that the UE does not comprehend:

2>	treat the rest of the message as if the field was absent;

NOTE:	This section does not apply to the case of an extension to the value range of a field. Such cases are addressed instead by the requirements in section 10.3.

# 11	Radio information related interactions between network nodes

## 11.1	General

This section specifies RRC messages that are transferred between network nodes. These RRC messages may be transferred to or from the UE via another Radio Access Technology. Consequently, these messages have similar characteristics as the RRC messages that are transferred across the NR radio interface, i.e. the same transfer syntax and protocol extension mechanisms apply.

## 11.2	Inter-node RRC messages

### 11.2.1	General

This section specifies RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

-- ASN1START

NR-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

CandidateRS-IndexInfoList,

CellIdentity,

maxCellPrep,

maxCellSCG,

maxRS-IndexReport,

MeasResultSCG-Failure,

MeasResultSSTD,

P-Max,

PhysCellId,

RadioBearerConfig,

RRCReconfiguration,

RSRP-Range,

RSRQ-Range,

SSB-Index,

ShortMAC-I,

UECapabilityInformation,

UE-CapabilityRAT-ContainerList

FROM NR-RRC-Definitions;

-- ASN1STOP

### 11.2.2	Message definitions

#### –	HandoverCommand

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverCommand-r15					HandoverCommand-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverCommand-r15-IEs ::=				SEQUENCE {

handoverCommandMessage				OCTET STRING (CONTAINING RRCReconfiguration),

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB.

#### –	HandoverPreparationInformation

This message is used to transfer the NR RRC information used by the target gNB during handover preparation, including UE capability information.

Direction: source gNB/source RAN to target gNB

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation-r15		HandoverPreparationInformation-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-r15-IEs ::= SEQUENCE {

ue-CapabilityRAT-List				UE-CapabilityRAT-ContainerList,

sourceConfig						OCTET STRING (CONTAINING RRCReconfiguration),

rrm-Config							RRM-Config				OPTIONAL,

as-Context							AS-Context				OPTIONAL,

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

AS-Context ::=							SEQUENCE {

reestablishmentInfo						SEQUENCE {

sourcePhysCellId					PhysCellId,

targetCellShortMAC-I				ShortMAC-I,

additionalReestabInfoList			ReestabNCellInfoList				OPTIONAL

}																				OPTIONAL,

-- FFS Whether to change e.g. move all re-establishment info to Xx

configRestrictInfo					ConfigRestrictInfoSCG						OPTIONAL,

...

}

ReestabNCellInfoList ::=		SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::=	SEQUENCE{

cellIdentity						CellIdentity,

key-gNodeB-Star					BIT STRING (SIZE (256)),

shortMAC-I							ShortMAC-I

}

RRM-Config ::=				SEQUENCE {

ue-InactiveTime				INTEGER,

candidateCellInfoList		CandidateCellInfoList		OPTIONAL,

...

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB.

sourceConfig The radio resource configuration as used in the source cell.

rrm-Config Local RAN context used mainly for RRM purposes.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE. FFS whether certain capabilities are mandatory to provide by source e.g. of target and/or source RAT.

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB.

Direction: Secondary gNB to master gNB or eNB

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=						SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-Config-r15					CG-Config-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-Config-r15-IEs ::=				SEQUENCE {

scg-CellGroupConfig				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

fullConfigSN						ENUMERATED {true}									OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfig)	OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG								OPTIONAL,

drx-InfoSCG							DRX-Info											OPTIONAL,

candidateCellInfoList				CandidateCellInfoList								OPTIONAL,

nonCriticalExtension				SEQUENCE {}											OPTIONAL

}

ConfigRestrictModReqSCG ::=			SEQUENCE {

requestedBandCombinationMRDC					BandCombinationIndex					OPTIONAL,

requestedBasebandCombinationListMRDC				SEQUENCE OF INTEGER					OPTIONAL,

-- FFS Signalling details of BPC restrictions requested by SgNB to be alleviated

requestedP-MaxFR1						P-Max												OPTIONAL,

...

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

fullConfigSN Set to true in case scg-CellGroupdConfig and scg-RB-Config concern the full configuration rather than the changes (i.e. delta) compared to the current configuration.

requestedP-MaxFR1 Requested value for the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

configRestrictModReq Used by SN to re-negotiate SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. used to request configure an NR band combination which use MN has previously forbidden.

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-ConfigInfo-r15					CG-ConfigInfo-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-ConfigInfo-r15-IEs ::=			SEQUENCE {

eutra-CapabilityInfo			OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,  -- Cond SN-Addition

candidateCellInfoListMN			CandidateCellInfoList								OPTIONAL,

candidateCellInfoListSN			CandidateCellInfoList								OPTIONAL,

measResultSSTD					MeasResultSSTD										OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType					ENUMERATED { t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, maxUL-TimingDiff,

scg-ChangeFailure, scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG				OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																					OPTIONAL,

configRestrictInfo				ConfigRestrictInfoSCG								OPTIONAL,

drx-InfoMCG						DRX-Info											OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        OPTIONAL,

mcg-RB-Config					OCTET STRING (CONTAINING RadioBearerConfig)	OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

ConfigRestrictInfoSCG ::=			SEQUENCE {

allowedBandCombinationListMRDC				BandCombinationIndexList							OPTIONAL,

allowedBasebandCombinationListMRDC		SEQUENCE OF INTEGER				OPTIONAL,

-- FFS Signalling details of BC and BPC restrictions to be observed by SgNB

-- FFS Signalling details regarding power coordination

p-maxFR1						P-Max												OPTIONAL,

servCellIndexRangeSCG				SEQUENCE {

lowBound							ServCellIndex,

upBound								ServCellIndex

},

maxMeasFreqsSCG-NR					INTEGER							OPTIONAL,

...

}

BandCombinationIndexList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationIndex

DRX-Info ::=					SEQUENCE {

cycle							INTEGER,

offset							INTEGER

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBandCombinationListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select an NR band combination.. All MR-DC band combinations indicated by this field comprise the same LTE band combination.

allowedBasebandCombinationListMRDC Indicates the list of NR BPCs the SN is allowed to configure.

candidateCellInfoList Contains information regarding cells that the master or source node suggests the target gNB to consider configuring.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change.

p-maxFR1 Indicates the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

scg-RB-Config Contains the IE RadioBearerConfig of the SN, used to support delta configuration e.g. during SN change. When master eNB or gNB decides to configure UE with full configuration, this field is absent.

sourceConfigSCG Includes the current dedicated SCG configuration in the same format as CG-Config, i.e. not only CellGroupConfig but also e.g. measConfig. When master eNB or gNB decides to configure UE with full configuration, this field is absent.

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

servCellIndexRangeSCG Range of indices that SN is allowed to use for SCG serving cells.

| Conditional Presence | Explanation |
| --- | --- |
| SN-Addition | The field is mandatory present upon SN addition. |

## 11.3	Inter-node RRC information element definitions

#### –	CandidateCellInfoList

The CandidateCellInfoList IE contains information regarding cells that the source suggests the target gNB to consider configuring.

CandidateCellInfoList information element

-- ASN1START

-- TAG-CANDIDATE-CELL-INFO-LIST-START

CandidateCellInfoList ::=	SEQUENCE (SIZE (1..maxCellSCG)) OF CandidateCellInfo

CandidateCellInfo ::=				SEQUENCE {

cellIdentification				SEQUENCE {

physCellId						PhysCellId,

dl-CarrierFreq					ARFCN-ValueNR

},

measResultCell					ResultsThreeQuantities

OPTIONAL,

candidateRS-IndexListSSB				CandidateRS-IndexInfoListSSB	OPTIONAL,

candidateRS-IndexListCSI-RS				CandidateRS-IndexInfoListCSI-RS	OPTIONAL,

...

}

CandidateRS-IndexInfoListSSB ::=	SEQUENCE (SIZE (1..maxRS-IndexReport)) OF CandidateRS-IndexInfoSSB

CandidateRS-IndexInfoSSB ::=				SEQUENCE {

-- FFS whether to support CSI RS based beam results also

ssb-Index							SSB-Index,

measResultSSB						ResultsThreeQuantities

OPTIONAL,

...

}

CandidateRS-IndexInfoListCSI-RS ::=	SEQUENCE (SIZE (1..maxRS-IndexReport)) OF CandidateRS-IndexInfoCSI-RS

CandidateRS-IndexInfoCSI-RS ::=			SEQUENCE {

csi-RS-Index							CSI-RS-Index,

measResultCSI-RS					ResultsThreeQuantities						OPTIONAL,

...

}

ResultsThreeQuantities ::= 				SEQUENCE {

rsrp								RSRP-Range									OPTIONAL,

rsrq								RSRQ-Range									OPTIONAL,

sinr								SINR-Range									OPTIONAL

}

-- TAG-CANDIDATE-CELL-INFO-LIST-STOP

-- ASN1STOP

## 11.4	Inter-node RRC multiplicity and type constraint values

### –	End of NR-InterNodeDefinitions

-- ASN1START

END

-- ASN1STOP

# 12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 11.2-1: Illustration of RRC procedure delay

Table 11.2-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | X |  |

Annex A (informative):	Guidelines, mainly on use of ASN.1

## A.1	Introduction

The following clauses contain guidelines for the specification of RRC protocol data units (PDUs) with ASN.1.

## A.2	Procedural specification

### A.2.1	General principles

The procedural specification provides an overall high level description regarding the UE behaviour in a particular scenario.

It should be noted that most of the UE behaviour associated with the reception of a particular field is covered by the applicable parts of the PDU specification. The procedural specification may also include specific details of the UE behaviour upon reception of a field, but typically this should be done only for cases that are not easy to capture in the PDU section e.g. general actions, more complicated actions depending on the value of multiple fields.

Likewise, the procedural specification need not specify the UE requirements regarding the setting of fields within the messages that are sent to the network i.e. this may also be covered by the PDU specification.

### A.2.2	More detailed aspects

The following more detailed conventions should be used:

-	Bullets:

-	Capitals should be used in the same manner as in other parts of the procedural text i.e. in most cases no capital applies since the bullets are part of the sentence starting with 'The UE shall:'

-	All bullets, including the last one in a sub-clause, should end with a semi-colon i.e. an ';'

-	Conditions

-	Whenever multiple conditions apply, a semi-colon should be used at the end of each conditions with the exception of the last one, i.e. as in 'if cond1, or cond2:

## A.3	PDU specification

### A.3.1	General principles

#### A.3.1.1	ASN.1 sections

The RRC PDU contents are formally and completely described using abstract syntax notation (ASN.1), see X.680 [13], X.681 (02/2002) [14].

The complete ASN.1 code is divided into a number of ASN.1 sections in the specifications. In order to facilitate the extraction of the complete ASN.1 code from the specification, each ASN.1 section begins with the following:

- 	a first text paragraph consisting entirely of an ASN.1 start tag, which consists of a double hyphen followed by a single space and the text string "ASN1START" (in all upper case letters).

-	a second text paragraph consisting entirely of a block start tag is included, which consists of a double hyphen followed by a single space and the text string "TAG_NAME_START" (in all upper case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters).

Similarly, each ASN.1 section ends with the following:

-	a first text paragraph consisting entirely of a block stop tag, which consists of a double hyphen followed by a single space and the text string "TAG_NAME_STOP" (in all upper-case letters), where the "NAME" refers to the main name of the paragraph (in all upper-case letters).

-	a second text paragraph consisting entirely of an ASN.1 stop tag, which consists of a double hyphen followed by a singlespace and the text "ASN1STOP" (in all upper case letters):

This results in the following tags:

-- ASN1START

-- TAG_NAME_START

-- TAG_NAME_STOP

-- ASN1STOP

The text paragraphs containing either of thestart and stop tags should not contain any ASN.1 code significant for the complete description of the RRC PDU contents. The complete ASN.1 code may be extracted by copying all the text paragraphs between an ASN.1 start tag and the following ASN.1 stop tag in the order they appear, throughout the specification.

NOTE:	A typical procedure for extraction of the complete ASN.1 code consists of a first step where the entire RRC PDU contents description (ultimately the entire specification) is saved into a plain text (ASCII) file format, followed by a second step where the actual extraction takes place, based on the occurrence of the ASN.1 start and stop tags.

#### A.3.1.2	ASN.1 identifier naming conventions

The naming of identifiers (i.e., the ASN.1 field and type identifiers) should be based on the following guidelines:

-	Message (PDU) identifiers should be ordinary mixed case without hyphenation. These identifiers, e.g., the RRCConnectionModificationCommand, should be used for reference in the procedure text. Abbreviations should be avoided in these identifiers and abbreviated forms of these identifiers should not be used.

-	Type identifiers other than PDU identifiers should be ordinary mixed case, with hyphenation used to set off acronyms only where an adjacent letter is a capital, e.g., EstablishmentCause, SelectedPLMN (not Selected-PLMN, since the "d" in "Selected" is lowercase), InitialUE-Identity and MeasSFN-SFN-TimeDifference.

-	Field identifiers shall start with a lowercase letter and use mixed case thereafter, e.g., establishmentCause. If a field identifier begins with an acronym (which would normally be in upper case), the entire acronym is lowercase (plmn-Identity, not pLMN-Identity). The acronym is set off with a hyphen (ue-Identity, not ueIdentity), in order to facilitate a consistent search pattern with corresponding type identifiers.

-	Identifiers should convey the meaning of the identifier and should avoid adding unnecessary postfixes (e.g. abstractions like 'Info') for the name.

-	Identifiers that are likely to be keywords of some language, especially widely used languages, such as C++ or Java, should be avoided to the extent possible.

-	Identifiers, other than PDU identifiers, longer than 25 characters should be avoided where possible. It is recommended to use abbreviations, which should be done in a consistent manner i.e. use 'Meas' instead of 'Measurement' for all occurrences. Examples of typical abbreviations are given in table A.3.1.2.1-1 below.

-	For future extension: When an extension is introduced a suffix is added to the identifier of the concerned ASN.1 field and/or type. A suffix of the form "-rX" is used, with X indicating the release, for ASN.1 fields or types introduced in a later release (i.e. a release later than the original/first release of the protocol) as well as for ASN.1 fields or types for which a revision is introduced in a later release replacing a previous version, e.g., Foo-r9 for the Rel-9 version of the ASN.1 type Foo. A suffix of the form "-rXb" is used for the first revision of a field that it appears in the same release (X) as the original version of the field, "-rXc" for a second intra-release revision and so on. A suffix of the form "-vXYZ" is used for ASN.1 fields or types that only are an extension of a corresponding earlier field or type (see sub-clause A.4), e.g., AnElement-v10b0 for the extension of the ASN.1 type AnElement introduced in version 10.11.0 of the specification. A number 0...9, 10, 11, etc. is used to represent the first part of the version number, indicating the release of the protocol. Lower case letters a, b, c, etc. are used to represent the second (and third) part of the version number if they are greater than 9. In the procedural specification, in field descriptions as well as in headings suffices are not used, unless there is a clear need to distinguish the extension from the original field.

-	More generally, in case there is a need to distinguish different variants of an ASN.1 field or IE, a suffix should be added at the end of the identifiers e.g. MeasObjectUTRA, ConfigCommon. When there is no particular need to distinguish the fields (e.g. because the field is included in different IEs), a common field identifier name may be used. This may be attractive e.g. in case the procedural specification is the same for the different variants.

-	It should be avoided to use field identifiers with the same name within the elements of a CHOICE, including using a CHOICE inside a SEQUENCE (to avoid certain compiler errors).

Table A.3.1.2-1: Examples of typical abbreviations used in ASN.1 identifiers

| Abbreviation | Abbreviated word |
| --- | --- |
| Config | Configuration |
| DL | Downlink |
| Ext | Extension |
| Freq | Frequency |
| Id | Identity |
| Ind | Indication |
| Meas | Measurement |
| MIB | MasterInformationBlock |
| Neigh | Neighbour(ing) |
| Param(s) | Parameter(s) |
| Phys | Physical |
| PCI | Physical Cell Id |
| Proc | Process |
| Reconfig | Reconfiguration |
| Reest | Re-establishment |
| Req | Request |
| Rx | Reception |
| Sched | Scheduling |
| SIB | SystemInformationBlock |
| Sync | Synchronisation |
| Thr | Threshold |
| Tx | Transmission |
| UL | Uplink |

NOTE:		The table A.3.1.2.1-1 is not exhaustive. Additional abbreviations may be used in ASN.1 identifiers when needed.

#### A.3.1.3	Text references using ASN.1 identifiers

A text reference into the RRC PDU contents description from other parts of the specification is made using the ASN.1 field identifier of the referenced type. The ASN.1 field and type identifiers used in text references should be in the italic font style. The "do not check spelling and grammar" attribute in Word should be set. Quotation marks (i.e., " ") should not be used around the ASN.1 field or type identifier.

A reference to an RRC PDU should be made using the corresponding ASN.1 field identifier followed by the word "message", e.g., a reference to the rrcRelease message.

A reference to a specific part of an RRC PDU, or to a specific part of any other ASN.1 type, should be made using the corresponding ASN.1 field identifier followed by the word "field", e.g., a reference to the prioritisedBitRate field in the example below.

-- /example/ ASN1START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							Priority,

prioritisedBitRate					PrioritisedBitRate,

bucketSizeDuration					BucketSizeDuration,

logicalChannelGroup					INTEGER (0..3)

}		OPTIONAL

}

-- ASN1STOP

NOTE:	All the ASN.1 start tags in the ASN.1 sections, used as examples in this annex to the specification, are deliberately distorted, in order not to include them when the ASN.1 description of the RRC PDU contents is extracted from the specification.

A reference to a specific type of information element should be made using the corresponding ASN.1 type identifier preceded by the acronym "IE", e.g., a reference to the IE LogicalChannelConfig in the example above.

References to a specific type of information element should only be used when those are generic, i.e., without regard to the particular context wherein the specific type of information element is used. If the reference is related to a particular context, e.g., an RRC PDU type (message) wherein the information element is used, the corresponding field identifier in that context should be used in the text reference.

A reference to a specific value of an ASN.1 field should be made using the corresponding ASN.1 value without using quotation marks around the ASN.1 value, e.g., 'if the status field is set to value true'.

### A.3.2	High-level message structure

Within each logical channel type, the associated RRC PDU (message) types are alternatives within a CHOICE, as shown in the example below.

-- /example/ ASN1START

DL-DCCH-Message ::= SEQUENCE {

message					DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

dlInformationTransfer					DLInformationTransfer,

handoverFromEUTRAPreparationRequest		HandoverFromEUTRAPreparationRequest,

mobilityFromEUTRACommand				MobilityFromEUTRACommand,

rrcConnectionReconfiguration			RRCConnectionReconfiguration,

rrcConnectionRelease					RRCConnectionRelease,

securityModeCommand						SecurityModeCommand,

ueCapabilityEnquiry						UECapabilityEnquiry,

spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

A nested two-level CHOICE structure is used, where the alternative PDU types are alternatives within the inner level c1 CHOICE.

Spare alternatives (i.e., spare1 in this case) may be included within the c1 CHOICE to facilitate future extension. The number of such spare alternatives should not extend the total number of alternatives beyond an integer-power-of-two number of alternatives (i.e., eight in this case).

Further extension of the number of alternative PDU types is facilitated using the messageClassExtension alternative in the outer level CHOICE.

### A.3.3	Message definition

Each PDU (message) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfiguration ::=	SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcConnectionReconfiguration-r8		RRCConnectionReconfiguration-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-r8-IEs ::= SEQUENCE {

-- Enter the IEs here.

...

}

-- ASN1STOP

Hooks for critical and non-critical extension should normally be included in the PDU type specification. How these hooks are used is further described in sub-clause A.4.

Critical extensions are characterised by a redefinition of the PDU contents and need to be governed by a mechanism for protocol version agreement between the encoder and the decoder of the PDU, such that the encoder is prevented from sending a critically extended version of the PDU type, which is not comprehended by the decoder.

Critical extension of a PDU type is facilitated by a two-level CHOICE structure, where the alternative PDU contents are alternatives within the inner level c1 CHOICE. Spare alternatives (i.e., spare3 down to spare1 in this case) may be included within the c1 CHOICE. The number of spare alternatives to be included in the original PDU specification should be decided case by case, based on the expected rate of critical extension in the future releases of the protocol.

Further critical extension, when the spare alternatives from the original specifications are used up, is facilitated using the criticalExtensionsFuture in the outer level CHOICE.

In PDU types where critical extension is not expected in the future releases of the protocol, the inner level c1 CHOICE and the spare alternatives may be excluded, as shown in the example below.

-- /example/ ASN1START

RRCConnectionReconfigurationComplete ::= SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcConnectionReconfigurationComplete-r8

RRCConnectionReconfigurationComplete-r8-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfigurationComplete-r8-IEs ::= SEQUENCE {

-- Enter the fields here.

...

}

-- ASN1STOP

Non-critical extensions are characterised by the addition of new information to the original specification of the PDU type. If not comprehended, a non-critical extension may be skipped by the decoder, whilst the decoder is still able to complete the decoding of the comprehended parts of the PDU contents.

Non-critical extensions at locations other than the end of the message or other than at the end of a field contained in a BIT or OCTET STRING are facilitated by use of the ASN.1 extension marker "...". The original specification of a PDU type should normally include the extension marker at the end of the sequence of information elements contained.

Non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING may be facilitated by use of an empty sequence that is marked OPTIONAL e.g. as shown in the following example:

-- /example/ ASN1START

RRCMessage-r8-IEs ::= 					SEQUENCE {

field1									InformationElement1,

field2									InformationElement2,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

The ASN.1 section specifying the contents of a PDU type may be followed by a field description table where a further description of, e.g., the semantic properties of the fields may be included. The general format of this table is shown in the example below. The field description table is absent in case there are no fields for which further description needs to be provided e.g. because the PDU does not include any fields, or because an IE is defined for each field while there is nothing specific regarding the use of this IE that needs to be specified.

%PDU-TypeIdentifier% field descriptions

%field identifier% Field description.

%field identifier% Field description.

The field description table has one column. The header row shall contain the ASN.1 type identifier of the PDU type.

The following rows are used to provide field descriptions. Each row shall include a first paragraph with a field identifier (in bold and italic font style) referring to the part of the PDU to which it applies. The following paragraphs at the same row may include (in regular font style), e.g., semantic description, references to other specifications and/or specification of value units, which are relevant for the particular part of the PDU.

The parts of the PDU contents that do not require a field description shall be omitted from the field description table.

### A.3.4	Information elements

Each IE (information element) type is specified in an ASN.1 section similar to the one shown in the example below.

-- /example/ ASN1START

PRACH-ConfigSIB ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo

}

PRACH-Config ::=					SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need N

}

PRACH-ConfigInfo ::=				SEQUENCE {

prach-ConfigIndex					ENUMERATED {ffs},

highSpeedFlag						ENUMERATED {ffs},

zeroCorrelationZoneConfig			ENUMERATED {ffs}

}

-- ASN1STOP

IEs should be introduced whenever there are multiple fields for which the same set of values apply. IEs may also be defined for other reasons e.g. to break down a ASN.1 definition in to smaller pieces.

A group of closely related IE type definitions, like the IEs PRACH-ConfigSIB and PRACH-Config in this example, are preferably placed together in a common ASN.1 section. The IE type identifiers should in this case have a common base, defined as the generic type identifier. It may be complemented by a suffix to distinguish the different variants. The "PRACH-Config" is the generic type identifier in this example, and the "SIB" suffix is added to distinguish the variant. The sub-clause heading and generic references to a group of closely related IEs defined in this way should use the generic type identifier.

The same principle should apply if a new version, or an extension version, of an existing IE is created for critical or non-critical extension of the protocol (see sub-clause A.4). The new version, or the extension version, of the IE is included in the same ASN.1 section defining the original. A suffix is added to the type identifier, using the naming conventions defined in sub-clause A.3.1.2, indicating the release or version of the where the new version, or extension version, was introduced.

Local IE type definitions, like the IE PRACH-ConfigInfo in the example above, may be included in the ASN.1 section and be referenced in the other IE types defined in the same ASN.1 section. The use of locally defined IE types should be encouraged, as a tool to break up large and complex IE type definitions. It can improve the readability of the code. There may also be a benefit for the software implementation of the protocol end-points, as these IE types are typically provided by the ASN.1 compiler as independent data elements, to be used in the software implementation.

An IE type defined in a local context, like the IE PRACH-ConfigInfo, should not be referenced directly from other ASN.1 sections in the RRC specification. An IE type which is referenced in more than one ASN.1 section should be defined in a separate sub-clause, with a separate heading and a separate ASN.1 section (possibly as one in a set of closely related IE types, like the IEs PRACH-ConfigSIB and PRACH-Config in the example above). Such IE types are also referred to as 'global IEs'.

NOTE:	Referring to an IE type, that is defined as a local IE type in the context of another ASN.1 section, does not generate an ASN.1 compilation error. Nevertheless, using a locally defined IE type in that way makes the IE type definition difficult to find, as it would not be visible at an outline level of the specification. It should be avoided.

The ASN.1 section specifying the contents of one or more IE types, like in the example above, may be followed by a field description table, where a further description of, e.g., the semantic properties of the fields of the information elements may be included. This table may be absent, similar as indicated in sub-clause A.3.3 for the specification of the PDU type. The general format of the field description table is the same as shown in sub-clause A.3.3 for the specification of the PDU type.

### A.3.5	Fields with optional presence

A field with optional presence may be declared with the keyword DEFAULT. It identifies a default value to be assumed, if the sender does not include a value for that field in the encoding:

-- /example/ ASN1START

PreambleInfo ::=					SEQUENCE {

numberOfRA-Preambles				INTEGER (1..64)						DEFAULT 1,

...

}

-- ASN1STOP

Alternatively, a field with optional presence may be declared with the keyword OPTIONAL. It identifies a field for which a value can be omitted. The omission carries semantics, which is different from any normal value of the field:

-- /example/ ASN1START

PRACH-Config ::=				SEQUENCE {

rootSequenceIndex					INTEGER (0..1023),

prach-ConfigInfo					PRACH-ConfigInfo					OPTIONAL	-- Need N

}

-- ASN1STOP

The semantics of an optionally present field, in the case it is omitted, should be indicated at the end of the paragraph including the keyword OPTIONAL, using a short comment text with a need code. The need code includes the keyword "Need", followed by one of the predefined semantics tags (S, M, N or R) defined in sub-clause 6.1. If the semantics tag S is used, the semantics of the absent field are further specified either in the field description table following the ASN.1 section, or in procedure text.

The addition of OPTIONAL keywords for capability groups is based on the following guideline. If there is more than one field in the lower level IE, then OPTIONAL keyword is added at the group level. If there is only one field in the lower level IE, OPTIONAL keyword is not added at the group level.

### A.3.6	Fields with conditional presence

A field with conditional presence is declared with the keyword OPTIONAL. In addition, a short comment text shall be included at the end of the paragraph including the keyword OPTIONAL. The comment text includes the keyword "Cond", followed by a condition tag associated with the field ("UL" in this example):

-- /example/ ASN1START

LogicalChannelConfig ::=			SEQUENCE {

ul-SpecificParameters				SEQUENCE {

priority							INTEGER (0),

...

}		OPTIONAL																	-- Cond UL

}

-- ASN1STOP

When conditionally present fields are included in an ASN.1 section, the field description table after the ASN.1 section shall be followed by a conditional presence table. The conditional presence table specifies the conditions for including the fields with conditional presence in the particular ASN.1 section.

| Conditional presence | Explanation |
| --- | --- |
| UL | Specification of the conditions for including the field associated with the condition tag = "UL". Semantics in case of optional presence under certain conditions may also be specified. |

The conditional presence table has two columns. The first column (heading: "Conditional presence") contains the condition tag (in italic font style), which links the fields with a condition tag in the ASN.1 section to an entry in the table. The second column (heading: "Explanation") contains a text specification of the conditions and requirements for the presence of the field. The second column may also include semantics, in case of an optional presence of the field, under certain conditions i.e. using the same predefined tags as defined for optional fields in A.3.5.

Conditional presence should primarily be used when presence of a field depends on the presence and/or value of other fields within the same message. If the presence of a field depends on whether another feature/function has been configured, while this function can be configured independently e.g. by another message and/or at another point in time, the relation is best reflected by means of a statement in the field description table.

If the ASN.1 section does not include any fields with conditional presence, the conditional presence table shall not be included.

Whenever a field is only applicable in specific cases e.g. TDD, use of conditional presence should be considered.

### A.3.7	Guidelines on use of lists with elements of SEQUENCE type

Where an information element has the form of a list (the SEQUENCE OF construct in ASN.1) with the type of the list elements being a SEQUENCE data type, an information element shall be defined for the list elements even if it would not otherwise be needed.

For example, a list of PLMN identities with reservation flags is defined as in the following example:

-- /example/ ASN1START

PLMN-IdentityInfoList ::=				SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=				SEQUENCE {

plmn-Identity						PLMN-Identity,

cellReservedForOperatorUse			ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

rather than as in the following (bad) example, which may cause generated code to contain types with unpredictable names:

-- /bad example/ ASN1START

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..6)) OF SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

-- ASN1STOP

### A.3.8	Guidelines on use of parameterised SetupRelease type

The usage of the parameterised SetupRelease type is like a function call in programming languages where the element type parameter is passed as a parameter. The parameterised type only implies a textual change in abstract syntax where all references to the parameterised type are replaced by the compiler with the release/setup choice. Two examples of the usage are shown below;

-- /example/ ASN1START

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15				SetupRelease { IE-r15 }					OPTIONAL,	--	Need M

...

}

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15		SetupRelease { Element-r15 }

}																	OPTIONAL,	-- Need M

Element-r15 ::= SEQUENCE {

field1-r15					IE1-r15,

field2-r15					IE2-r15								OPTIONAL	-- Need N

}																	OPTIONAL,	-- Need M

-- /example/ ASN1STOP

The SetupRelease is always be used with only named IEs, i.e. the example below is not allowed:

-- /example/ ASN1START

RRCMessage-r15-IEs ::= SEQUENCE {

field-r15		SetupRelease { SEQUENCE { 	-- Unnamed SEQUENCEs are not allowed!

field1-r15					IE1-r15,

field2-r15					IE2-r15							OPTIONAL	-- Need N

}

}																	OPTIONAL,	-- Need M

}

-- /example/ ASN1STOP

If a field defined using the parameterized SetupRelease type requires procedural text, the field is referred to using the values defined for the type itself, namely, "setup" and "release". For example, procedural text for field-r15 above could be as follows:

1> if field-r15 is set to "setup":

2> do something;

1> else (field-r15 is set to "release"):

2> release field-r15 (if appropriate);

### A.3.9	Guidelines on use of ToAddModList and ToReleaseList

In order to benefit from delta signalling when modifying lists with many and/or large elements, so-called add/mod- and release- lists should be used. Instead of a single list containing all elements of the list, the ASN.1 provides two lists. One list is used to convey the actual elements that are to be added to the list or modified in the list. The second list conveys only the identities (IDs) of the list elements that are to be released from the list. In other words, the ASN.1 defines only means to signal modifications to a list maintained in the receiver (typically the UE). An example is provided below:

-- /example/ ASN1START

AnExampleIE ::= 		SEQUENCE {

elementsToAddModList	SEQUENCE (SIZE (1..maxNrofElements)) OF Element										OPTIONAL,	--	Need N

elementsToReleaseList	SEQUENCE (SIZE (1..maxNrofElements)) OF ElementId									OPTIONAL,	--	Need N

...

}

Element ::=			SEQUENCE {

elementId				ElementId,

aField					INTEGER (0..16777215),

anotherField			OCTET STRING,

...

}

ElementId ::=			INTEGER (0..maxNrofElements-1)

maxNrofElements 		INTEGER ::= 50

maxNrofElements-1 		INTEGER ::= 49

-- /example/ ASN1STOP

As can be seen, the elements of the list must contain an identity (INTEGER) that identifies the elements unambiguously upon addition, modification and removal. It is recommended to define an IE for that identifier (here ElementId) so that it can be used both for a field inside the element as well as in the elementsToReleaseList.

Both lists should be made OPTIONAL and flagged as ”Need N”. The need code reflects that the UE does not main the received lists as such but rather updates its configuration using the information therein. In other words, it is not possible to provide via delta signalling an update to a previously signalled elementsToAddModList or elementsToReleaseList (which Need M would imply). The update is always in relation to the UE's internal configuration.

If no procedural text is provided for a set of ToAddModList and ToReleaseList, the following generic procedure applies:

The UE shall:

1>	for each ElementId in the elementsToReleaseList,:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	release the Element from the current UE configuration;

1>	for each Element in the elementsToAddModList:

2>	if the current UE configuration includes an Element with the given ElementId:

3>	modify the configured Element in accordance with the received Element;

2>	else:

3>	add received Element to the UE configuration;

## A.4	Extension of the PDU specifications

### A.4.1	General principles to ensure compatibility

It is essential that extension of the protocol does not affect interoperability i.e. it is essential that implementations based on different versions of the RRC protocol are able to interoperate. In particular, this requirement applies for the following kind of protocol extensions:

-	Introduction of new PDU types (i.e. these should not cause unexpected behaviour or damage).

-	Introduction of additional fields in an extensible PDUs (i.e. it should be possible to ignore uncomprehended extensions without affecting the handling of the other parts of the message).

-	Introduction of additional values of an extensible field of PDUs. If used, the behaviour upon reception of an uncomprehended value should be defined.

It should be noted that the PDU extension mechanism may depend on the logical channel used to transfer the message e.g. for some PDUs an implementation may be aware of the protocol version of the peer in which case selective ignoring of extensions may not be required.

The non-critical extension mechanism is the primary mechanism for introducing protocol extensions i.e. the critical extension mechanism is used merely when there is a need to introduce a 'clean' message version. Such a need appears when the last message version includes a large number of non-critical extensions, which results in issues like readability, overhead associated with the extension markers. The critical extension mechanism may also be considered when it is complicated to accommodate the extensions by means of non-critical extension mechanisms.

### A.4.2	Critical extension of messages and fields

The mechanisms to critically extend a message are defined in A.3.3. There are both "outer branch" and "inner branch" mechanisms available. The "outer branch" consists of a CHOICE having the name criticalExtensions, with two values, c1 and criticalExtensionsFuture. The criticalExtensionsFuture branch consists of an empty SEQUENCE, while the c1 branch contains the "inner branch" mechanism.

The "inner branch" structure is a CHOICE with values of the form "MessageName-rX-IEs" (e.g., "RRCConnectionReconfiguration-r8-IEs") or "spareX", with the spare values having type NULL. The "-rX-IEs" structures contain the complete structure of the message IEs for the appropriate release; i.e., the critical extension branch for the Rel-10 version of a message includes all Rel-8 and Rel-9 fields (that are not obviated in the later version), rather than containing only the additional Rel-10 fields.

The following guidelines may be used when deciding which mechanism to introduce for a particular message, i.e. only an 'outer branch', or an 'outer branch' in combination with an 'inner branch' including a certain number of spares:

-	For certain messages, e.g. initial uplink messages, messages transmitted on a broadcast channel, critical extension may not be applicable.

-	An outer branch may be sufficient for messages not including any fields.

-	The number of spares within inner branch should reflect the likelihood that the message will be critically extended in future releases (since each release with a critical extension for the message consumes one of the spare values). The estimation of the critical extension likelyhood may be based on the number, size and changeability of the fields included in the message.

-	In messages where an inner branch extension mechanism is available, all spare values of the inner branch should be used before any critical extensions are added using the outer branch.

The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release

-- /example/ ASN1START					-- Original release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

-- ASN1STOP

-- /example/ ASN1START					-- Later release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

rrcMessage-r10						RRCMessage-r10-IEs,

rrcMessage-r11						RRCMessage-r11-IEs,

rrcMessage-r14						RRCMessage-r14-IEs

},

later							CHOICE {

c2									CHOICE{

rrcMessage-r16						RRCMessage-r16-IEs,

spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

}

-- ASN1STOP

It is important to note that critical extensions may also be used at the level of individual fields i.e. a field may be replaced by a critically extended version. When sending the extended version, the original version may also be included (e.g. original field is mandatory, EUTRAN is unaware if UE supports the extended version). In such cases, a UE supporting both versions may be required to ignore the original field. The following example illustrates the use of the critical extension mechanism by showing the ASN.1 of the original and of a later release

-- /example/ ASN1START					-- Original release

RRCMessage ::=							SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE{

rrcMessage-r8						RRCMessage-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCMessage-rN-IEs ::= SEQUENCE {

field1-rN							ENUMERATED {

value1, value2, value3, value4}	OPTIONAL,	-- Need N

field2-rN							InformationElement2-rN				OPTIONAL,	-- Need N

nonCriticalExtension				RRCConnectionReconfiguration-vMxy-IEs	OPTIONAL

}

RRCConnectionReconfiguration-vMxy-IEs ::= SEQUENCE {

field2-rM							InformationElement2-rM			OPTIONAL, -- Cond NoField2rN

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NoField2rN | The field is optionally present, need N, if field2-rN is absent. Otherwise the field is not present |

Finally, it is noted that a critical extension may be introduced in the same release as the one in which the original field was introduced e.g. to correct an essential ASN.1 error. In such cases a UE capability may be introduced, to assist the network in deciding whether or not to use the critically extension.

### A.4.3	Non-critical extension of messages

#### A.4.3.1	General principles

The mechanisms to extend a message in a non-critical manner are defined in A.3.3. W.r.t. the use of extension markers, the following additional guidelines apply:

-	When further non-critical extensions are added to a message that has been critically extended, the inclusion of these non-critical extensions in earlier critical branches of the message should be avoided when possible.

-	The extension marker ("…") is the primary non-critical extension mechanism that is used but empty sequences may be used if length determinant is not required. Examples of cases where a length determinant is not required:

-	at the end of a message,

-	at the end of a structure contained in a BIT STRING or OCTET STRING

-	When an extension marker is available, non-critical extensions are preferably placed at the location (e.g. the IE) where the concerned parameter belongs from a logical/ functional perspective (referred to as the 'default extension location')

-	It is desirable to aggregate extensions of the same release or version of the specification into a group, which should be placed at the lowest possible level.

-	In specific cases it may be preferrable to place extensions elsewhere (referred to as the 'actual extension location') e.g. when it is possible to aggregate several extensions in a group. In such a case, the group should be placed at the lowest suitable level in the message. <TBD: ref to seperate example>

-	In case placement at the default extension location affects earlier critical branches of the message, locating the extension at a following higher level in the message should be considered.

-	In case an extension is not placed at the default extension location, an IE should be defined. The IE's ASN.1 definition should be placed in the same ASN.1 section as the default extension location. In case there are intermediate levels in-between the actual and the default extension location, an IE may be defined for each level. Intermediate levels are primarily introduced for readability and overview. Hence intermediate levels need not allways be introduced e.g. they may not be needed when the default and the actual extension location are within the same ASN.1 section. <TBD: ref to seperate example>

#### A.4.3.2	Further guidelines

Further to the general principles defined in the previous section, the following additional guidelines apply regarding the use of extension markers:

-	Extension markers within SEQUENCE

-	Extension markers are primarily, but not exclusively, introduced at the higher nesting levels

-	Extension markers are introduced for a SEQUENCE comprising several fields as well as for information elements whose extension would result in complex structures without it (e.g. re-introducing another list)

-	Extension markers are introduced to make it possible to maintain important information structures e.g. parameters relevant for one particular RAT

-	Extension markers are also used for size critical messages (i.e. messages on BCCH, BR-BCCH, PCCH and CCCH), although introduced somewhat more carefully

-	The extension fields introduced (or frozen) in a specific version of the specification are grouped together using double brackets.

-	Extension markers within ENUMERATED

-	Spare values may be used until the number of values reaches the next power of 2, while the extension marker caters for extension beyond that limit, given that the use of spare values in a later Release is possible without any error cases

-	A suffix of the form "vXYZ" is used for the identifier of each new value, e.g. "value-vXYZ".

-	Extension markers within CHOICE:

-	Extension markers are introduced when extension is foreseen and when comprehension is not required by the receiver i.e. behaviour is defined for the case where the receiver cannot comprehend the extended value (e.g. ignoring an optional CHOICE field). It should be noted that defining the behaviour of a receiver upon receiving a not comprehended choice value is not required if the sender is aware whether or not the receiver supports the extended value.

-	A suffix of the form "vXYZ" is used for the identifier of each new choice value, e.g. "choice-vXYZ".

Non-critical extensions at the end of a message/ of a field contained in an OCTET or BIT STRING:

-	When a nonCriticalExtension is actually used, a "Need" code should not be provided for the field, which always is a group including at least one extension and a field facilitating further possible extensions. For simplicity, it is recommended not to provide a "Need" code when the field is not actually used either.

Further, more general, guidelines:

-	In case a need code is not provided for a group, a "Need" code is provided for all individual extension fields within the group i.e. including for fields that are not marked as OPTIONAL. The latter is to clarify the action upon absence of the whole group.

#### A.4.3.3	Typical example of evolution of IE with local extensions

The following example illustrates the use of the extension marker for a number of elementary cases (sequence, enumerated, choice). The example also illustrates how the IE may be revised in case the critical extension mechanism is used.

NOTE	In case there is a need to support further extensions of release n while the ASN.1 of release (n+1) has been frozen, without requiring the release n receiver to support decoding of release (n+1) extensions, more advanced mechanisms are needed e.g. including multiple extension markers.

-- /example/ ASN1START

InformationElement1 ::= 			SEQUENCE {

field1								ENUMERATED {

value1, value2, value3, value4-v880,

..., value5-v960 },

field2								CHOICE {

field2a								BOOLEAN,

field2b								InformationElement2b,

...,

field2c-v960						InformationElement2c-r9

},

...,

[[	field3-r9							InformationElement3-r9		OPTIONAL		-- Need R

]],

[[	field3-v9a0							InformationElement3-v9a0	OPTIONAL,		-- Need R

field4-r9							InformationElement4			OPTIONAL		-- Need R

]]

}

InformationElement1-r10 ::=			SEQUENCE {

field1								ENUMERATED {

value1, value2, value3, value4-v880,

value5-v960, value6-v1170, spare2, spare1, ... },

field2								CHOICE {

field2a								BOOLEAN,

field2b								InformationElement2b,

field2c-v960						InformationElement2c-r9,

...,

field2d-v12b0						INTEGER (0..63)

},

field3-r9							InformationElement3-r10			OPTIONAL,	-- Need R

field4-r9							InformationElement4				OPTIONAL,	-- Need R

field5-r10							BOOLEAN,

field6-r10							InformationElement6-r10			OPTIONAL,	-- Need R

...,

[[	field3-v1170						InformationElement3-v1170		OPTIONAL	-- Need R

]]

}

-- ASN1STOP

Some remarks regarding the extensions of InformationElement1 as shown in the above example:

–	The InformationElement1 is initially extended with a number of non-critical extensions. In release 10 however, a critical extension is introduced for the message using this IE. Consequently, a new version of the IE InformationElement1 (i.e. InformationElement1-r10) is defined in which the earlier non-critical extensions are incorporated by means of a revision of the original field.

–	The value4-v880 is replacing a spare value defined in the original protocol version for field1. Likewise value6-v1170 replaces spare3 that was originally defined in the r10 version of field1

–	Within the critically extended release 10 version of InformationElement1, the names of the original fields/IEs are not changed, unless there is a real need to distinguish them from other fields/IEs. E.g. the field1 and InformationElement4 were defined in the original protocol version (release 8) and hence not tagged. Moreover, the field3-r9 is introduced in release 9 and not re-tagged; although, the InformationElement3 is also critically extended and therefore tagged InformationElement3-r10 in the release 10 version of InformationElement1.

#### A.4.3.4	Typical examples of non critical extension at the end of a message

The following example illustrates the use of non-critical extensions at the end of the message or at the end of a field that is contained in a BIT or OCTET STRING i.e. when an empty sequence is used.

-- /example/ ASN1START

RRCMessage-r8-IEs ::=			SEQUENCE {

field1							InformationElement1,

field2							InformationElement2,

field3							InformationElement3					OPTIONAL,	-- Need N

nonCriticalExtension			RRCMessage-v860-IEs					OPTIONAL

}

RRCMessage-v860-IEs ::=			SEQUENCE {

field4-v860						InformationElement4					OPTIONAL,	-- Need S

field5-v860						BOOLEAN								OPTIONAL,	-- Cond C54

nonCriticalExtension			RRCMessage-v940-IEs					OPTIONAL

}

RRCMessage-v940-IEs ::=			SEQUENCE {

field6-v940						InformationElement6-r9					OPTIONAL,	-- Need R

nonCriticalExtensions			SEQUENCE {}								OPTIONAL

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The InformationElement4 is introduced in the original version of the protocol (release 8) and hence no suffix is used.

#### A.4.3.5	Examples of non-critical extensions not placed at the default extension location

The following example illustrates the use of non-critical extensions in case an extension is not placed at the default extension location.

#### –	ParentIE-WithEM

The IE ParentIE-WithEM is an example of a high level IE including the extension marker (EM). The root encoding of this IE includes two lower level IEs ChildIE1-WithoutEM and ChildIE2-WithoutEM which not include the extension marker. Consequently, non-critical extensions of the Child-IEs have to be included at the level of the Parent-IE.

The example illustrates how the two extension IEs ChildIE1-WithoutEM-vNx0 and ChildIE2-WithoutEM-vNx0 (both in release N) are used to connect non-critical extensions with a default extension location in the lower level IEs to the actual extension location in this IE.

ParentIE-WithEM information element

-- /example/ ASN1START

ParentIE-WithEM ::=					SEQUENCE {

-- Root encoding, including:

childIE1-WithoutEM					ChildIE1-WithoutEM				OPTIONAL,		-- Need N

childIE2-WithoutEM					ChildIE2-WithoutEM				OPTIONAL,		-- Need N

...,

[[	childIE1-WithoutEM-vNx0				ChildIE1-WithoutEM-vNx0		OPTIONAL,		-- Need N

childIE2-WithoutEM-vNx0				ChildIE2-WithoutEM-vNx0		OPTIONAL		-- Need N

]]

}

-- ASN1STOP

Some remarks regarding the extensions shown in the above example:

–	The fields childIEx-WithoutEM-vNx0 may not really need to be optional (depends on what is defined at the next lower level).

–	In general, especially when there are several nesting levels, fields should be marked as optional only when there is a clear reason.

#### –	ChildIE1-WithoutEM

The IE ChildIE1-WithoutEM is an example of a lower level IE, used to control certain radio configurations including a configurable feature which can be setup or released using the local IE ChIE1-ConfigurableFeature. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature. The example is based on the following assumptions:

–	when initially configuring as well as when modifying the new field, the original fields of the configurable feature have to be provided also i.e. as if the extended ones were present within the setup branch of this feature.

–	when the configurable feature is released, the new field should be released also.

–	when omitting the original fields of the configurable feature the UE continues using the existing values (which is used to optimise the signalling for features that typically continue unchanged upon handover).

–	when omitting the new field of the configurable feature the UE releases the existing values and discontinues the associated functionality (which may be used to support release of unsupported functionality upon handover to an eNB supporting an earlier protocol version).

The above assumptions, which affect the use of conditions and need codes, may not always apply. Hence, the example should not be re-used blindly.

ChildIE1-WithoutEM information elements

-- /example/ ASN1START

ChildIE1-WithoutEM ::=				SEQUENCE {

-- Root encoding, including:

chIE1-ConfigurableFeature			ChIE1-ConfigurableFeature		OPTIONAL		 -- Need N

}

ChildIE1-WithoutEM-vNx0 ::=		SEQUENCE {

chIE1-ConfigurableFeature-vNx0		ChIE1-ConfigurableFeature-vNx0	OPTIONAL	-- Cond ConfigF

}

ChIE1-ConfigurableFeature ::=		CHOICE {

release								NULL,

setup								SEQUENCE {

-- Root encoding

}

}

ChIE1-ConfigurableFeature-vNx0 ::=	SEQUENCE {

chIE1-NewField-rN					INTEGER (0..31)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE1-ConfigurableFeature is included and set to "setup"; otherwise the field is not present and the UE shall delete any existing value for this field. |

#### –	ChildIE2-WithoutEM

The IE ChildIE2-WithoutEM is an example of a lower level IE, typically used to control certain radio configurations. The example illustrates how the new field chIE1-NewField is added in release N to the configuration of the configurable feature.

ChildIE2-WithoutEM information element

-- /example/ ASN1START

ChildIE2-WithoutEM ::=				CHOICE {

release								NULL,

setup								SEQUENCE {

-- Root encoding

}

}

ChildIE2-WithoutEM-vNx0 ::=			SEQUENCE {

chIE2-NewField-rN					INTEGER (0..31)					OPTIONAL	-- Cond ConfigF

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ConfigF | The field is optional present, need R, in case of chIE2-ConfigurableFeature is included and set to "setup"; otherwise the field is not present and the UE shall delete any existing value for this field. |

## A.5	Guidelines regarding inclusion of transaction identifiers in RRC messages

The following rules provide guidance on which messages should include a Transaction identifier

1:	DL messages on CCCH that move UE to RRC-Idle should not include the RRC transaction identifier.

2:	All network initiated DL messages by default should include the RRC transaction identifier.

3:	All UL messages that are direct response to a DL message with an RRC Transaction identifier should include the RRC Transaction identifier.

4:	All UL messages that require a direct DL response message should include an RRC transaction identifier.

5:	All UL messages that are not in response to a DL message nor require a corresponding response from the network should not include the RRC Transaction identifier.

## A.6	Guidelines regarding use of need codes

The following rule provides guidance for determining need codes for optional downlink fields:

- if the field needs to be stored by the UE (i.e. maintained) when absent:

- use Need M (=Maintain)

- else, if the field needs to be released by the UE when absent:

- use Need R (=Release)

- else, if UE shall take no action when the field is absent (i.e. UE does not even need to maintain any existing value of the field):

- use Need N (=None)

- else (UE behaviour upon absence doesn’t fit any of the above conditions):

- use Need S (=Specified)

- specify the UE behaviour upon absence of the field in the procedural text or in the field description table.

## A.7	Guidelines regarding use of conditions

Conditions are primarily used to specify network restrictions, for which the following types can be distinguished:

-	CondC: Message Contents related constraints e.g. that a field B is mandatory present if the same message includes field A and when it is set value X

-	CondC: Configuration Constraints e.g. that a field D can only be signalled if field C is configured and set to value Y. (i.e. regardless of whether field C is present in the same message or previously configured)

The use of these conditions is illustrated by an example.

-- /example/ ASN1START

RRCMessage-IEs ::= SEQUENCE {

fieldA							FieldA					OPTIONAL,	-- Need M

fieldB							FieldB					OPTIONAL,	-- CondM-FieldAsetToX

fieldC							FieldC					OPTIONAL,	-- Need M

fieldD							FieldD					OPTIONAL,	-- CondC-FieldCsetToY

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

-- /example/ ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Message (content) constraints |  |
| CondM-FieldAsetToX | The field is mandatory present if fieldA is included and set to valueX. Otherwise the field is optional present, need R. |
| Configuration constraints |  |
| CondC- FieldCsetToY | The field is optional present, need M, if fieldC is configured and set to valueY. Otherwise the field is not present and the UE does not maintain the value |

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |