3GPP TSG-RAN WG2 Meeting #101	R2-180xxxx

# Athens, Greece, 26th February - 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on EN-DC - CSI L1 parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections identified during ASN.1 review (RAN2 NR AH 1801), and email discussions after the AH. This CR is based on R2-1801218 Baseline TS 38331 v1.0.1 for ASN.1 review |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be completed. Guidance for CR editors: - To avoid change marks for language formatting (typically happens when many users edit the same doc), please do the following word setting: Review panel => Language => Set proofing languge => Detect automatically => OFF - Set the “User name” to indicate the company name. - When storing the CR in 3GPP folder, companies should add their Company ID (one letter) to the file name (see RIL). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

[AdditionalSpectrumEmission, Alpha, ARFCN-ValueNR, BandwidthPart-Config, BeamFailureDetectionConfig, BeamFailureRecoveryConfig, CellGroupConfig, ControlResourceSet, CrossCarrierSchedulingConfig]

#### –	CSI-MeasConfig

The CSI-MeasConfig IE is used to configure the UE for measuring CSI-RS (reference signals) and for reporting those measurements on L1 (PUCCH, PUSCH) as channel state information. See also 38.214, section 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

CSI-MeasConfig ::=					SEQUENCE {

csi-ResourceConfigs 				SEQUENCE (SIZE (1..maxNrofCSI-ResrouceConfigurations)) OF CSI-ResourceConfig 			OPTIONAL,

csi-ReportConfigs					SEQUENCE (SIZE (1..maxNrofCSI-Reports)) OF CSI-ReportConfig								OPTIONAL,

csi-MeasIdToAddModList				SEQUENCE (SIZE (1..maxNrofCSI-MeasId)) OF CSI-MeasIdToAddMod								OPTIONAL,

csi-MeasIdToReleaseList				SEQUENCE (SIZE (1..maxNrofCSI-MeasID)) OF CSI-MeasId									OPTIONAL

-- Size of CSI request field in DCI (bits). Corresponds to L1 parameter 'ReportTriggerSize' (see 38.214, section 5.2)

reportTriggerSize				INTEGER (0..6)																				OPTIONAL,

-- Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations

-- and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement.

-- New description (still not clear and not fitting to other fields): Each trigger state is associated with one or multiple ReportConfigs where each ReportConfig is linked to one or two or three P/SP/AP CSI-RS resource setting(s). If a resource setting linked to a ReportConfig has multiple aperiodic resource sets and only a subset of the aperiodic resource sets is associated with the trigger state, a bitmap (with the bitwidth Nbit =number of resource sets in a resource setting.  Number of one(s) in the bitmap None = 1 (FFS on 2) for CSI acquisition) is RRC configured per trigger state per resource setting to select CSI-IM/NZP CSI-RS resource set(s) from the resource setting.

-- FFS_CHECK: Is this the appropriate place for the IE or should it be inside the resource configuration or in a set?

-- FFS_FIXME: This is just one report trigger. But of course it should be a list. Maximum number of configured triggers depends

-- on the trigger size

-- FFS: How to address the MAC-CE configuration

reportTrigger					SEQUENCE {

aperiodic						SEQUENCE {

-- The CSI-ReportConfig (their IDs) associated with this reportTrigger

associatedReportConfigs			SEQUENCE (SIZE (1..maxNrofReportConfigIdsPerTrigger)) OF CSI-ReportConfigId,

-- bitmap with the bitwidth Nbit =number of resource sets (max number Nbit = 64) in a linked resource setting per report trigger tate.

-- Number of one(s) in the bitmap None = 1 for CSI acquisition (FFS 1<= None <= 64 for beam management).

-- FFS: To enforce the number of linked resources, the linking information should instead be in the report that uses the resource

-- Corresponds to L1 parameter 'ResourceSetBitmap' (see 38.214, section FFS_Section)

-- FFS_FIXME: The following list assumes that all NZP- and IM resource sets use a common ID space. But that is not ensured

-- due to having separate lists of sets.

associatedResourceSets			SEQUENCE (SIZE (1..64)) OF NZP-CSI-ResourceSetId												OPTIONAL,

-- For a trigger state within aperiodicReportTrigger that triggers a ap-CSI-RS resource set, contains a list of

-- references to TCI-State elements configured in PDSCH-Config for providing the QCL source and QCL type for each ap-CSI-RS

-- resource within the triggered set of ap-CSI-RS resources. The length of the list is equal to the number of

-- aperiodic CSI-RS resources in the set (CSI-RS-ResourceSet). For a target aperiodic CSI-RS assoicated with each

-- triggering state, contains a reference to one TCI-RS-Set in TCI-States for providing the QCL source and QCL type.

-- Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section 5.2.1.5.1)

qcl-Info-aPeriodicReportingTrigger			SEQUENCE (SIZE(1..ffsValue)) OF TCI-StateId									OPTIONAL

},

semiPersistentOnPUSCH				SEQUENCE {

associatedReportConfig				CSI-ReportConfigId

}

}

}

-- TAG-CSI-MEAS-CONFIG-STOP

-- ASN1STOP

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig comprises of one or more NZP-CSI-RS-ResourceSets, CSI-IM-ResourceSet and/or CSI-SSB-Resource

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

-- One CSI resource configuration comprising of one or more resource sets

CSI-ResourceConfig ::= 				SEQUENCE {

csi-ResourceConfigId				CSI-ResourceConfigId,

-- Contains up to maxNrofCSI-ResourceSets resource CSI-ReosurceSets if ResourceConfigType is 'aperiodic' and 1 otherwise.

-- Corresponds to L1 parameter 'ResourceSetConfigList' (see 38.214, section 5.2.1.3.1)

csi-RS-ResourceSets 	 			CHOICE {

nzp-CSI-RS-ResourceSets				SEQUENCE (SIZE (1..maxNrofCSI-ResourceSets)) OF NZP-CSI-RS-ResourceSet,

csi-IM-ResourceSets					SEQUENCE (SIZE (1..maxNrofCSI-ResourceSets)) OF CSI-IM-ResourceSet

},

-- List of SSB resources used for beam measurement and reporting in a resource set

-- Corresponds to L1 parameter 'resource-config-SS-list' (see 38,214, section FFS_Section)

ssb-Resources						SEQUENCE (SIZE (1..maxNrofSSB-Resources)) OF	CSI-SSB-Resource		OPTIONAL, --Cond OnlyWithNZPResourceSets

-- The DL BWP which the CSI-RS assocaited with this CSI-ResourceConfig are located in.

-- Corresponds to L1 parameter 'BWP-Info' (see 38.214, section FFS_Section)

bwp-Id								BWP-Id,

-- Time domain behavior of resource configuration. Corresponds to L1 parameter 'ResourceConfigType' (see 38.214, section 5.2.2.3.5)

resourceType							CHOICE {

aperiodic								NULL,

semiPersistent 						NULL,

periodic								SEQUENCE {

-- For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and

-- QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS.

-- Corresponds to L1 parameter 'QCL-Info-PeriodicCSI-RS' (see 38.214, section FFS_Section)

qcl-InfoPeriodicCSI-RS					TCI-StateId																	OPTIONAL

}

},

-- Indication of which Serving Cell the configured CSI-RS is located in.

-- FFS_CHECK: RAN1 intended to enable cross-carrier scheduling of aperiodoic CSI-RS. This field would indicate on which ServingCell

-- the UE finds these resources. Discuss whether and how this works considering that currently a CSI-MeasConfig exists per ServingCell

-- Corresponds to L1 parameter 'CC_Info' (see 38.214, section 5.2.2.3.1)

crossCarrierInfo						FFS_Value																				OPTIONAL,

...

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::= INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSet

The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::= 					SEQUENCE {

nzp-CSI-ResourceSetId					NZP-CSI-ResourceSetId,

-- NZP-CSI-RS-Resources assocaited with this NZP-CSI-RS resource set.

-- Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2)

-- FFS: Better make the csi-rs-Resources a common pool on CSI-MeasConfig level?

nzp-CSI-RS-Resources						SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-Resource,

-- Indicates whether repetition is on/off.

If set to set to 'OFF', the UE may not assume that the

-- NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter

-- and with same NrofPorts in every symbol.

-- Corresponds to L1 parameter 'CSI-RS-ResourceRep' (see 38.214, sections 5.2.2.3.1 and 5.1.6.1.2)

repetition									ENUMERATED { on, off },

-- Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the

-- CSI-RS resource set is transmitted. When the field is absent the UE applies the value 0.

-- Corresponds to L1 parameter 'Aperiodic-NZP-CSI-RS-TriggeringOffset' (see 38,214, section FFS_Section)

-- FFS_CHECK: Is this field at the correct place? Or should it be in the trigger configuration instead?

aperiodicTriggeringOffset			 		FFS_Value																		OPTIONAL	-- Need S

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

#### –	NZP-CSI-ResourceSetId

The IE NZP-CSI-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RESOURCESETID-START

NZP-CSI-ResourceSetId ::= INTEGER (0..maxNrofCSI-ResourceSets-1)

-- TAG-NZP-CSI-RESOURCESETID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS-Resource, which the UE may be configured to measure on (see 38.214, section 5.2.2.3.1).

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=					SEQUENCE {

nzp-CSI-RS-ResourceId					NZP-CSI-RS-ResourceId,

-- Frequency domain allocation within a physical resource block in accordance with 38.211, section 7.4.1.5.3 including table 7.4.1.5.2-1.

-- The number of bits that may be set to one depend on the chosen row in that table.

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12)),

row3								BIT STRING (SIZE (6)),

row4								BIT STRING (SIZE (3)),

row5								BIT STRING (SIZE (6)),

row7								BIT STRING (SIZE (6)),

row8								BIT STRING (SIZE (6)),

row9								BIT STRING (SIZE (6)),

row10								BIT STRING (SIZE (6)),

row11								BIT STRING (SIZE (6)),

row12								BIT STRING (SIZE (6)),

row13								BIT STRING (SIZE (6)),

row14								BIT STRING (SIZE (6)),

row15								BIT STRING (SIZE (6)),

row16								BIT STRING (SIZE (6)),

row17								BIT STRING (SIZE (6)),

row18								BIT STRING (SIZE (6)),

row19								BIT STRING (SIZE (6))

},

-- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS.

-- Parameter l0 in 38.211, section 7.4.1.5.3. Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

firstOFDMSymbolInTimeDomain			INTEGER (0..13),

-- Time domain allocation within a physical resource block. Parameter l1 in 38.211, section 7.4.1.5.3.

firstOFDMSymbolInTimeDomain2		INTEGER (0..13)

,

-- CDM type (see 38.214, section 5.2.2.3.1)

cdm-Type								ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

-- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density'  (see 38.211, section 7.4.1.5.3)

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

density									CHOICE {

dot5									ENUMERATED {evenPRBs, oddPRBs},

one										NULL,

three									NULL,

spare									NULL

},

-- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1)

freqBand								CSI-FrequencyOccupation

,

-- Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, sections 5.2.2.3.1 and 4.1)

powerControlOffset						INTEGER(-8..15),

-- Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section FFS_Section)

powerControlOffsetSS					ENUMERATED{db-3, db0, db3, db6}													OPTIONAL,

-- Scrambling ID (see 38.214, section 5.2.2.3.1)

scramblingID							ScramblingId,

-- Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on.

-- The corresponding offset is also given in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1)

periodicityAndOffset					CSI-ResourcePeriodicityAndOffset

,

-- Indicates whether or not the antenna ports of NZP CSI-RS resources in the CSI-RS resource set is same

-- Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1)

trs-Info								ENUMERATED {true}																OPTIONAL

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=				SEQUENCE {

-- PRB where this CSI resource starts in relation to PRB 0 of the associated BWP.

-- Only multiples of 4 are allowed (0, 4, ...)

startingRB								INTEGER (0..maxNrofPhysicalResourceBlocks-1),

-- Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable

-- number is the minimum of 24 and the width of the associated BWP.

nrofRBs									INTEGER (24..maxNrofPhysicalResourceBlocks)

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::= 					INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSet

The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSet information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESET-START

CSI-IM-ResourceSet ::=					SEQUENCE {

-- FFS: Where is the csi-im-ResourceSetId used?

csi-IM-ResourceSetId					CSI-IM-ResourceSetId,

-- CSI-IM-Resources associated with this CSI-IM-ResourceSet

-- Corresponds to L1 parameter 'CSI-IM-ResourceConfigList' (see 38.214, section 5.2)

csi-IM-Resources						SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-Resource

}

-- TAG-CSI-IM-RESOURCESET-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=				INTEGER (0..maxNrof CSI-IM-ResourceSets)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-IM-Resource

The IE CSI-IM-Resource is used to configure one CSI Interference Management (IM) resource.

CSI-IM-Resource information element

-- ASN1START

-- TAG-CSI-IM-RESOURCE-START

CSI-IM-Resource ::= 					SEQUENCE {

csi-IM-ResourceId						CSI-IM-ResourceId,

-- The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters.

-- Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4)

csi-IM-ResourceElementPattern				CHOICE {

pattern0									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p0						ENUMERATED { s0, s2, s4, s6, s8, s10 },

-- OFDM symbol location of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p0							INTEGER (0..12)

},

pattern1									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p1						ENUMERATED { s0, s4, s8 },

-- OFDM symbol location of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p1							INTEGER (0..13)

}

}																														OPTIONAL,	--Need M

-- Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2)

freqBand									CSI-FrequencyOccupation											OPTIONAL,	-- Need M

-- Periodicity and slot offset for periodic/semi-persistent CSI-IM. Corresponds to L1 parameter 'CSI-IM-timeConfig'

periodicityAndOffset					CSI-ResourcePeriodicityAndOffset

}

-- TAG-CSI-IM-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::= 			INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-SSB-Resource

The IE CSI-SSB-Resource is used to configure one SSB resource.

CSI-SSB-Resource information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCE-START

CSI-SSB-Resource ::=						SEQUENCE {

-- FFS: Undefined what the IE CSI-SSB-Resource contains.

}

-- TAG-CSI-SSB-RESOURCE-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources. both, the periodicity and the offset are given in number of slots. The periodicity value sl4 corresponds to 4 slots, sl5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=	CHOICE {

sl4											INTEGER (0..3),

sl5											INTEGER (0..4),

sl8											INTEGER (0..7),

sl10										INTEGER (0..9),

sl16										INTEGER (0..15),

sl20										INTEGER (0..19),

sl32										INTEGER (0..31),

sl40										INTEGER (0..39),

sl64										INTEGER (0..63),

sl80										INTEGER (0..79),

sl160										INTEGER (0..159),

sl320										INTEGER (0..319),

sl640										INTEGER (0..639)

}

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CSI-ReportConfig

The IE CSI-ReportConfig is used to configure FFS

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

-- Configuration of a CSI-Report sent on L1 (e.g. PUCCH) (see 38.214, section 5.2.1)

CSI-ReportConfig ::=					SEQUENCE {

reportConfigId							CSI-ReportConfigId,

-- Time domain behavior of reporting configuration

reportConfigType						CHOICE {

periodic								SEQUENCE {

-- Periodicity and slot offset . Corresponds to L1 parameter 'ReportPeriodicity'and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentPUCCH						SEQUENCE {

-- Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity' and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl4										INTEGER(0..3),

sl5										INTEGER(0..4),

sl8										INTEGER(0..7),

sl10									INTEGER(0..9),

sl16									INTEGER(0..15),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentPUSCH						SEQUENCE {

-- Periodicity. Corresponds to L1 parameter 'Reportperiodicity-spCSI'. (see 38.214, section 5.2.1.1?FFS_Section)

reportSlotConfig						ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

-- RNTI for SP CSI-RNTI, Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section 5.2.1.5.2)

-- FFS: RAN1 models different RNTIs as different Search Spaces with independent configurations. Align the configuration

-- of this one (e.g. group with monitoring periodicity, PDCCH candidate configuration, DCI-Payload size...)?

csi-RNTI								RNTI-Value,

-- Index of the p0-alpha set determining the power control for this CSI report transmission.

-- Corresponds to L1 parameter 'SPCSI-p0alpha' (see 38.214, section FFS_Section)

p0alpha									P0-PUSCH-AlphaSetId

},

aperiodic								SEQUENCE {

-- Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. A particular value is indicated in DCI.

-- (see 38.214, section 5.2.3)

-- FFS_Value: Range wasn’t final in RAN1 table.

-- FFS_FIXME: How are the DCI codepoints mapped to the allowed offsets?

reportSlotOffset					SEQUENCE (SIZE (1..4)) OF INTEGER (0..8)

}

},

-- The CSI related quanities to report. Corresponds to L1 parameter 'ReportQuantity' (see 38.214, section REF)

reportQuantity							CHOICE {

none									NULL,

cri-RI-PMI-CQI							NULL,

cri-RI-i1								NULL,

cri-RI-i1-CQI							SEQUENCE {

-- PRB bundling size to assume for CQI calcuation when reportQuantity is CRI/RI/i1/CQI

-- Corresponds to L1 parameter 'PDSCH-bundle-size-for-CSI' (see 38.214, section 5.2.1.4)

pdsch-BundleSizeForCSI					ENUMERATED {n2, n4}		OPTIONAL

},

cri-RI-CQI								NULL,

cri-RSRP								NULL,

ssb-Index-RSRP							NULL,

cri-RI-LI-PMI-CQI						NULL

},

-- Reporting configuration in the frequency domain. (see 38.214, section 5.2.1.4)

reportFreqConfiguration					 SEQUENCE {

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see 38.214, section 5.2.1.4)

cqi-FormatIndicator						ENUMERATED { widebandCQI, subbandCQI },

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see 38.214, section 5.2.1.4)

pmi-FormatIndicator						ENUMERATED { widebandPMI, subbandPMI },

-- Indicates a contiguous or non-contigous subset of subbands in the bandwidth part which CSI shall be reported

-- for. FFS: Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the

-- lowest subband in the BWP. (see 38.214, section 5.2.1.4)

-- FFS: Size of the bitmap. Introduce a CHOICE with different bitmap lengths depening on number of subbands in carrier/BWP?

csi-ReportingBand						BIT STRING (SIZE (ffsValue))

},

-- Time domain measurement restriction for the channel (signal) measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-channel' (see 38.214, section 5.2.1.1)

timeRestrictionForChannelMeasurements				ENUMERATED {configured, notConfigured},

-- Time domain measurement restriction for interference measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-interference' (see 38.214, section 5.2.1.1)

timeRestrictionForInterferenceMeasurements			ENUMERATED {configured, notConfigured},

-- Codebook configuration for Type-1 or Type-II including codebook subset restriction

codebookConfig								CodebookConfig,

-- Maximum number of CQIs per CSI report (cf. 1 for 1-CW, 2 for 2-CW)

nrofCQIsPerReport							ENUMERATED {n1, n2},

-- Turning on/off group beam based reporting (see 38.214, section 5.2.1.4)

groupBasedBeamReporting						CHOICE {

enabled										SEQUENCE {

-- Number of beams to report for group based beam reporting (see 38.214, section REF)

nrofBeamsToReport						ENUMERATED {ffsTypeAndValue}

},

disabled 								SEQUENCE {

-- The number (N) of measured RS resources to be reported per report setting in a non-group-based report.

-- N <= N_max, where N_max is either 2 or 4 depending on UE capability.

-- FFS: The signaling mechanism for the gNB to select a subset of N beams for the UE to measure and report.

-- FFS: Note: this parameter may not be needed for certain resource and/or report settings

-- FFS_ASN1: Change groupBasedBeamReporting into a CHOICE and include this field into the “no” option?

-- (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

nrofReportedRS							ENUMERATED {n1, n2, n3, n4}												OPTIONAL	-- Need S

}

},

-- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-table' (see 38.214, section 5.2.2.1)

cqi-Table								ENUMERATED {table1, table2, spare2, spare1}										OPTIONAL,

-- Indicates one out of two possible BWP-dependent values for the subband size as indicated in 38.214 table 5.2.1.4-2

-- Corresponds to L1 parameter 'SubbandSize' (see 38.214, section 5.2.1.4)

subbandSize								ENUMERATED {value1, value2},

-- BLER target that the UE shall be assume in its CQI calculation.

-- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section 5.2.2.1)

-- FFS_Values (now filled with spares)

bler-Target								ENUMERATED {zerodot1, spare3, space2, spare1}										OPTIONAL,

-- Port indication for RI/CQI calculation. For each  CSI-RS resource in the linked ResourceConfig for channel measurement,

-- a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback.

-- Corresponds to L1 parameter 'Non-PMI-PortIndication' (see 38.214, section FFS_Section)

non-PMI-PortIndication					FFS_Value																		OPTIONAL,

-- Which DL BWP the CSI-ReportConfig is associated with. (see 38.214, section FFS_Section)

-- FFS_CHECK: Should it be possible to link a report to several BWPs? If not, shouldn’t the report configuration be in the BWP?

-- FFS_CHECK: Should it be possible to link a report to the initial BWP? If so, which ID does that have?

bwp-Id								BWP-Id,																	OPTIONAL

}

PUCCH-CSI-Resource ::= 				CHOICE {

uplinkBandwidthPartId				BWP-Id,

-- PUCCH resource for the assocaited uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported.

pucch-Resource						PUCCH-Resource

}

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::= 					INTEGER (0..maxNrofCSI-ReportConfig-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see 38.214, section 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::= 						SEQUENCE {

-- Number of antenna ports in first dimension

codebookConfigN1						ENUMERATED {n1,n2,n3,n4,n6,n8,n12,n16},

-- Number of antenna ports in second dimension

codebookConfigN2						ENUMERATED {n1,n2,n3,n4},

-- Codebook subset restriction for the different codebooks

-- CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType'

-- (see 38.214, section 5.2.2.2)

codebookType 							CHOICE {

type1 									SEQUENCE {

subType									ENUMERATED {typeI-SinglePanel, typeI-MultiPanel},

-- Switch between Config 1 and Config 2

codebookMode							ENUMERATED {config1, config2},

-- Number of panels, Ng, used in multi-panel codebook

numberOfPanels							ENUMERATED {twopanels, fourpanels}					OPTIONAL,		-- Cond TypeI-MultiPanel

codebookSubsetRestrictionType1			CHOICE {

-- Codebook subset restriction for Type I Single-panel codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

-- FFS_Value: RAN1 indicated Bitmap of size N1*N2*O1*O2

singlePanel								BIT STRING (SIZE (ffsValue)),

-- Codebook subset restriction for 2TX codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

singlePanel2TX							BIT STRING (SIZE (6)),

-- Codebook subset restriction for Type I Multi-panel codebook

-- Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

multiPanel								BIT STRING (SIZE (ffsValue)),

-- i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214, section FFS_Section)

singlePanelCodebookSubsetRestriction-i2	BIT STRING (SIZE (16))

},

ri-Restriction							CHOICE {

-- Restriction for RI for TypeI-SinglePanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-SinglePanelRI-Restriction			BIT STRING (SIZE (8)),

-- Restriction for RI for TypeI-MultiPanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-MultiPanelRI-Restriction			BIT STRING (SIZE (4))

}

},

type2									SEQUENCE {

subType									ENUMERATED {typeII, typeII-PortSelection},

-- The size of the PSK alphabet, QPSK or 8-PSK

phaseAlphabetSize						ENUMERATED {n4, n8},

-- If subband amplitude reporting is activated (true)

subbandAmplitude						BOOLEAN,

-- Number of beams, L,  used for linear combination

numberOfBeams							ENUMERATED {two, three, four},

-- The size of the port selection codebook (parameter d)

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Cond TypeII-PortSelection

-- Codebook subset restriction for Type II codebook.

-- FFS: Clarify the meaning of the bitmap

-- FFS: The size of the bitmap is ceil(log2(nchoosek(O1*O2,4)))+8*N1*N2 ==> Clarify size. Present different bitmap sizes by CHOICE?

codebookSubsetRestrictionType2			BIT STRING (SIZE (ffsValue)),

ri-Restriction							CHOICE {

-- Restriction for RI for TypeII-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section 5.2.2.3)

typeII-RI-Restriction					BIT STRING (SIZE (2)),

-- Restriction for RI for TypeII-PortSelection-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section 5.2.2.4)

typeII-PortSelectionRI-Restriction		BIT STRING (SIZE (2))

}

}

}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

#### –	CSI-MeasIdToAddMod

The IE CSI-MeasIdToAddMod is used to link a CSI-RS-ResourceConfig to a CSI-ReportConfig (see 38.214, section 5.2)

CSI-MeasIdToAddMod information element

-- ASN1START

-- TAG-CSI-MEASIDTOADDMOD-START

CSI-MeasIdToAddMod ::=					SEQUENCE {

csi-measId								CSI-MeasId,

csi-RS-resourceConfigId					CSI-ResourceConfigId,

csi-ReportConfigId						CSI-ReportConfigId,

-- For CQI-Emulation, i.e., how to measure and compute the CQI.

-- CHECK: Clarify further what the values mean.

-- CHECK: Is there a need to inform the UE which resource to use for which measurement (signal, interference, ...)?

measQuantity							ENUMERATED {channel, interference}

}

-- TAG-CSI-MEASIDTOADDMOD-STOP

-- ASN1STOP

#### –	CSI-MeasId

The IE CSI-MeasId is used to identify one CSI-MeasIdToAddMod entry

CSI-MeasId information element

-- ASN1START

-- TAG-CSI-MEASID-START

CSI-MeasId ::= 							INTEGER (0..maxNrofCSI-MeasId-1)

-- TAG-CSI-MEASID-STOP

-- ASN1STOP

[DMRS-DownlinkConfig, DMRS-UplinkConfig, DRB-Identity, MeasResultSCG-Failure, FrequencyInfoDL, SCS-SpecificVirtualCarrier, FrequencyInfoUL, GSCN-ValueNR, LogicalChannelConfig, MAC-CellGroupConfig, MeasConfig, MeasGapConfig, MeasId, MeasIdToAddModList, MeasObjectEUTRA, MeasObjectId, MeasObjectNR, MeasObjectToAddModList, MeasResults, PDCCH-ConfigCommon, PDCCH-Config, PDCP-Config, PDSCH-Config, PCI-List, PCI-Range, PCI-RangeIndex, PCI-RangeIndexList, PhysCellId, PRB-Id, PTRS-DownlinkConfig, PTRS-UplinkConfig, PUCCH-Config, PUSCH-Config, PUSCH-PowerControl, Q-OffsetRange, QuantityConfig, RACH-ConfigCommon, RACH-ConfigCommonGeneric, RACH-ConfigDedicated, RadioBearerConfig, ReportConfigId, ReportConfigNR, ReportConfigToAddModList, ReportInterval, RLC-Config, RLF-TimersAndConstants, RNTI-Value, RSRP-Range, RSRQ-Range, SINR-Range, SCellIndex, SchedulingRequest-Config, SchedulingRequestResourceConfig, SchedulingRequestResourceId, ScramblingId, SDAP-Config, SearchSpace, SlotFormatIndicatorSFI, DownlinkPreemption, SearchSpaceId, SecurityAlgorithmConfig, ServCellIndex, ServingCellConfigCommon, ServingCellConfig, SlotFormatCombinationsPerCell, SRB-Identity, SPS-Config, ConfiguredGrantConfig, SRS-Config, SRS-CarrierSwitching, SSB-Index, SubcarrierSpacing]

#### –	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 				SEQUENCE {

tci-StateId					TCI-StateId,

qcl-Type1					QCL-Info,

qcl-Type2					QCL-Info																					OPTIONAL

}

TCI-StateId ::=				INTEGER (0..ffsValue)

QCL-Info ::=				SEQUENCE {

referenceSignal				CHOICE {

csi-rs						NZP-CSI-RS-ResourceConfigId,

ssb							SSB-Id,

-- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a NZP-CSI-ResourceSetId

trs							NZP-CSI-ResourceSetId

},

qcl-Type					ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

[TDD-UL-DL-Config]

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a A Zero-Power (ZP) CSI-RS resource. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfig' (see 38.214, section 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=						SEQUENCE {

-- ZP CSI-RS resource configuration ID

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigId' (see 38.214, section FFS_Section)

zp-CSI-RS-ResourceId						ZP-CSI-RS-ResourceId,

-- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1. FFS: Table correct?

-- The number of bits that may be set to one depend on the chosen row in that table.

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceMapping' (see 38.214, section FFS_Section)

frequencyDomainAllocation					CHOICE {

row1										BIT STRING (SIZE (4)),

row2										BIT STRING (SIZE (12)),

row4										BIT STRING (SIZE (3)),

row5										BIT STRING (SIZE (6)),

row6										BIT STRING (SIZE (6)),

row7										BIT STRING (SIZE (6)),

row8										BIT STRING (SIZE (6)),

row9										BIT STRING (SIZE (6)),

row10										BIT STRING (SIZE (6)),

row11										BIT STRING (SIZE (6)),

row12										BIT STRING (SIZE (6)),

row13										BIT STRING (SIZE (6)),

row14										BIT STRING (SIZE (6)),

row15										BIT STRING (SIZE (6)),

row16										BIT STRING (SIZE (6)),

row17										BIT STRING (SIZE (6)),

row18										BIT STRING (SIZE (6))

},

-- Time domain location for the first OFDM symbol of the CSI-RS resource relative to the start of the slot.

firstOFDM-SymbolInTimeDomain				INTEGER (0..13),

-- Time domain location for the first OFDM symbol of the second pair of OFDM symbols in a 4-symbol resource if the two

-- symbol pairs are non-contiguous.

-- FFS_RAN1: Check that this parameter is intended to be present also for ZP-CSI-RS-Resource.

firstOFDM-SymbolInTimeDomainSecondPair		INTEGER (0..13)													OPTIONAL,	-- Need R

-- Density of ZP-CSI-RS resource measured in RE/port/PRB.

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

-- Corresponds to L1 parameter 'ZP-CSI-RS-Density' (see 38.214, section FFS_Section)

density											CHOICE {

dot5											ENUMERATED {evenPRBs, oddPRBs},

one												NULL,

three											NULL,

spare											NULL

},

-- Frequency-occupancy across PRBs of this ZP-CSI-RS-Resource.

-- Corresponds to L1 parameter 'ZP-CSI-RS-FreqBand' (see 38.214, section FFS_Section)

freqBand									CSI-FrequencyOccupation

,

-- Time domain behavior of ZP-CSI-RS resource configuration.

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigType' (see 38.214, section FFS_Section)

resourceType									CHOICE {

aperiodic										SEQUENCE {

...

},

periodic										SEQUENCE {

-- Periodicity and slot offset for periodic ZP-CSI-RS resources.

-- Corresponds to L1 parameter 'ZP-CSI-RS-timeConfig' (see 38.214, section FFS_Section)

periodicityAndOffset							CSI-ResourcePeriodicityAndOffset,

...

}

},

...

}

ZP-CSI-RS-ResourceId ::=							INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds. It corresponds to the L1 parameter 'ZP-CSI-RS-ResourceSetConfigList'.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::= 						SEQUENCE {

zp-CSI-RS-ResourceSetId							ZP-CSI-RS-ResourceSetId,

-- The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

zp-CSI-RS-ResourceIdList							SEQUENCE {1..maxNrofZP-CSI-RS-ResourcesPerSet) OF ZP-CSI-RS-ResourceId

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::= 					INTEGER (0..maxNrofZP-CSI-RS-ResourceSetIds-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

[UE capability information elements]