3GPP TSG-RAN WG2 Meeting #101	R2-180xxxx

# Athens, Greece, 26th February - 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections identified during ASN.1 review (RAN2 NR AH 1801), and email discussions after the AH. This CR is based on R2-1801218 Baseline TS 38331 v1.0.1 for ASN.1 review |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be completed. Guidance for CR editors: - To avoid change marks for language formatting (typically happens when many users edit the same doc), please do the following word setting: Review panel => Language => Set proofing languge => Detect automatically => OFF - Set the “User name” to indicate the company name. - When storing the CR in 3GPP folder, companies should add their Company ID (one letter) to the file name (see RIL). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5	Procedures

5.5	Measurements

5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration.

The network may configure the UE to perform the following types of measurements:

-	NR measurements.

-	Inter-RAT measurements of E-UTRA frequencies.

The network may configure the UE to perform the following NR measurements for mobility, based on different RS types SS/PBCH Block or CSI-RS:

-	SS/PBCH Block based intra-frequency measurements: measurements at SSB(s) of neighbour cell(s) where both the center frequency(ies) and subcarrier spacing are the same as the cell-defining SSB of each serving cell.

-	SS/PBCH Block based inter-frequency measurements: measurements at SSB(s) of neighbour cell(s) that have different center frequency(ies) or different subcarrier spacing compared to the cell-defining SSB of each serving cell.

-	CSI-RS based intra-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are within the bandwidth(s) of the CSI-RS resource(s) on the serving cell(s) configured for measurements and having the same subcarrier spacing of the CSI-RS resource(s) on the serving cell(s) configured for measurements.

-	CSI-RS based inter-frequency measurements: measurements at CSI-RS(s) resource(s) of configured neighbour cell(s) whose bandwidth(s) are not within the bandwidth(s) or having different subcarrier spacing compared to the CSI-RS resource(s) on the serving cell(s) configured for measurements.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block.

-	Measurement results per cell based on SS/PBCH block(s).

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource.

-	Measurement results per cell based on CSI-RS resource(s).

-	CSI-RS resource measurement identifiers.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object is associated to an NR carrier frequency. Associated with this NR carrier frequency, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

- 	UE determines which MO corresponds to the serving cell frequency from the frequency location of the cell-defining SSB that is contained within the serving cell configuration and the reference frequency for CSI-RS resources (in case multiple MOs have the same SSB).

-	For inter-RAT E-UTRA measurements a measurement object is a single EUTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and/or cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and/or per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting of that measurement type. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements, i.e. no (UL, DL) transmissions are scheduled.

An RRC_CONNECTED UE maintains a single measurement object list, a single reporting configuration list, and a single measurement identities list. The measurement object list possibly includes NR intra-frequency object(s), NR inter-frequency object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration. A UE operating in EN-DC may have equivalent lists provided by EUTRAN measurement configuration.

The measurement procedures distinguish the following types of cells:

1.	The serving cell(s) - these are the SpCell and one or more SCells, if configured for a UE supporting CA.

2.	Listed cells - these are cells listed within the measurement object(s).

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the carrier frequency(ies) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells.

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

5.5.2	Measurement configuration

5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for each serving frequency;

Editor’s Note: FFS How the procedure is used for CGI reporting.

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

Editor’s Note: FFS Whether we can simplify the procedural text and avoid using VarMeasConfig.

5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer if running and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured;

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, cellsToRemoveList, blackCellsToRemoveList, whiteCellsToRemoveList, absThreshSS-BlocksConsolidation, absThreshCSI-RS-Consolidation, nrofSS-BlocksToAverage, nroCSI-RS-ResourcesToAverage;

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId  included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId  from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId  value included in the cellsToAddModList:

5>	if an entry with the matching physCellId  exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId  to the cellsToAddModList;

3>	if the received measObject includes the blackCellsToRemoveList:

4>	for each pci-RangeIndex included in the blackCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the blackCellsToAddModList;

NOTE 1:	For each pci-RangeIndex included in the blackCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the black list of cells only if all cell indexes containing it are removed.

3>	if the received measObject includes the blackCellsToAddModList:

4>	for each pci-RangeIndex included in the blackCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the blackCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the blackCellsToAddModList;

3>	if the received measObject includes the whiteCellsToRemoveList:

4>	for each pci-RangeIndex included in the whiteCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the whiteCellsToAddModList;

3>	if the received measObject includes the whiteCellsToAddModList:

4>	for each pci-RangeIndex included in the whiteCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the whiteCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the whiteCellsToAddModList;

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig;

5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer and reset the associated information (e.g. timeToTrigger) for this measId;

5.5.2.9	Measurement gap configuration

Editor’s Note: FFS How measurement gaps are configured.

Editor’s Note: FFS how to capture the e.g. following agreement: For the independent gap case where UE is able to apply a different gap pattern for LTE/FR1 and FR2:	a	NR RRC configures a measurement gap configuration for FR2.

5.5.2.10	Reference signal measurement timing configuration

Editor’s Note: FFS How SS/PBCH block measurement timing is configured.

5.5.3	Performing measurements

5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results in RRC_CONNECTED the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria and measurement reporting. For cell measurements, the network can configure RSRP, RSRQ or SINR as trigger quantity. Reporting quantities can be the same as trigger quantity or combinations of quantities (i.e. RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR).

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact layer 1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs:

3>	if at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	if at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the associated reportConfig contains rsType set to ssb:

3>	if the measId contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the associated reportConfig contains rsType set to csi-rs:

3>	if the measId contains a reportQuantityRsIndexes:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is not set to reportCGI:

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRsIndexes for the associated reportConfig is configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRsIndexes for the associated reportConfig is configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRsIndexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for each trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject;

2>	perform the evaluation of reporting criteria as specified in 5.5.4.

5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity and for each beam measurement quantity that the UE performs measurements according to 5.5.3.1:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and

a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms;

Editor’s Note: FFS Exact value of the sampling rate (i.e. X) for layer 3 filtering.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

5.5.3.3	Derivation of cell measurement results

The network may configure the UE to derive RSRP, RSRQ and SINR measurement results per cell associated to NR carrier frequencies based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block 	or CSI-RS).

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage in the associated measObject is not configured; or

2>	if absThreshSS-BlocksConsolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear average of the power values of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource on the associated frequency to be applicable for deriving RSRP when the concerned CSI-RS resource is included in the csi-rs-ResourceConfigMobility with the corresponding cellId and CSI-RS-ResourceId-RRM within the VarMeasConfig for this measId;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on CSI-RS as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear average of the power values of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nroCSI-RS-ResourcesToAverage;

5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2;

5.5.4	Measurement report triggering

5.5.4.1	General

If security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical;

3>	if the corresponding measObject concerns NR;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	for events involving a serving cell on one frequency and neighbours on another frequency, consider any serving cell on the other frequency to be a neighbouring cell;

5>	if useWhiteCellList is set to TRUE:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is included in the whiteCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	if the reportAmount exceeds 1:

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the SpCell;

4>	else (i.e. the reportAmount is equal to 1):

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the SpCell and for the strongest cell among the applicable cells;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell to be the NR SpCell or NR SCell that are configured on the frequency indicated in the associated measObjectNR;

Inequality A1-1 (Entering condition)

Inequality A1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell to be the NR SpCell or NR SCell that is configured on the frequency indicated in the associated measObjectNR;

Inequality A2-1 (Entering condition)

Inequality A2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	in EN-DC, use the PSCell for Mp, Ofp and Ocp;

NOTE	The cell(s) that triggers the event is on the frequency indicated in the associated measObjectNR which may be different from the frequency used by the SpCell.

Inequality A3-1 (Entering condition)

Inequality A3-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the PCell/PSCell, not taking into account any offsets.

Ofp is the frequency specific offset of the frequency of the PCell/PSCell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the PCell/PSCell).

Ocp is the cell specific offset of the PCell/PSCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the PCell/PSCell), and is set to zero if not configured for the PCell/PSCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled;

Inequality A4-1 (Entering condition)

Inequality A4-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	in EN-DC, use the PSCell for Mp;

NOTE:	The cell(s) that triggers the event is on the frequency indicated in the associated measObjectNR which may be different from the frequency used by the SpCell.

Inequality A5-1 (Entering condition 1)

Inequality A5-2 (Entering condition 2)

Inequality A5-3 (Leaving condition 1)

Inequality A5-4 (Leaving condition 2)

The variables in the formula are defined as follows:

Mp is the measurement result of the SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell that is configured on the frequency indicated in the associated measObjectNR to be the serving cell;

NOTE 1:	The neighbour(s) is on the same frequency as the SCell i.e. both are on the frequency indicated in the associated measObjectNR.

NOTE 2:	In EN-DC, The cell(s) that triggers the event is on the frequency indicated in the associated measObject shall be different from the frequency used by the PSCell.

Inequality A6-1 (Entering condition)

Inequality A6-2 (Leaving condition)

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the serving frequency), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

5.5.5	Measurement reporting

5.5.5.1	General

Figure 5.5.5-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServingCell within measResultServingFreqList to include RSRP, RSRQ and the available SINR for each configured serving cell derived based on the rsType indicated in the associated reportConfig;

1>	in EN-DC, set the measResultServingCell within measResultServingFreqList to include for each NR SCell that is configured, if any, the servFreqId;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes:

2>	for each configured serving cell, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting infcludes reportAddNeighMeas:

2> for each serving frequency for which measObjectId is referenced in the measIdList, other than the frequency corresponding with the measId that triggered the measurement reporting:

3>	set the measResultBestNeighCell within measResultServingFreqList to include the physCellId and the available measurement quantities and rsType indicated in reportConfig of the best non-serving cell on the concerned serving frequency;

3>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRsIndexes:

4>	for each best non-serving cell included in the measurement report

5> include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the reportType is set to eventTriggered:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns NR:

6>	if rsType in the associated reportConfig is set to ssb:

7> set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8>	if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

6>	else if rsType in the associated reportConfig is set to csi-rs:

7> set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in order of decreasing trigger quantity, i.e. the best cell is included first:

8> if reportQuantityRsIndexes is configured, include beam measurement information as described in 5.5.5.2;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1> if the UE is configured with EN-DC:

2> if SRB3 is configured:

3> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2> else:

3> submit the MeasurementReport message via the EUTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10];

1> else:

2>	 submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity;

1>	if reportType is set to periodical:

2> if a single reporting quantity is set to TRUE in reportQuantityRsIndexes;

3> consider the configured single quantity as the sorting quantity;

2> else:

3> if rsrp is set to TRUE;

4> consider RSRP as the sorting quantity;

3> else:

4> consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRsIndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting 	quantity and the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if  includeBeamMeasurements is configured, include the SS/PBCH based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject;

3>	if includeBeamMeasurements is configured, include the CSI-RS based measurement results for the quantities in reportQuantityRsIndexes set to TRUE for each CSI-RS index;

5.7	Other

5.7.3	SCG failure information

5.7.3.4	Setting the contents of MeasResultSCG-Failure

The UE shall set the contents of the MeasResultSCG-Failure as follows:

1>	set the measResultServFreqList to include for each SCG cell that is configured by the SN to be measured, if any, within measResultServingCell the quantities of the concerned SCell, if available, according to performance requirements in [FFS_Ref];

1>	for each SCG serving frequency included in measResultServFreqList include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements by the SN for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

2>	for each neighbour cell included:

3>	include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

6.2	RRC messages

6.2.2	Message definitions

–	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=				SEQUENCE {

criticalExtensions					CHOICE {

measurementReport							MeasurementReport-IEs,

criticalExtensionsFuture					SEQUENCE {}

}

}

MeasurementReport-IEs ::=		SEQUENCE {

measResults						MeasResults,

-- FFS

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

6	Protocol data units, formats and parameters (ASN.1)

6.3	RRC information elements

6.3.2	Radio resource control information elements

–	MeasResultSCG-Failure

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of EN-DC.

MeasResultSCG-Failure information element

-- ASN1START

-- TAG-MEAS-RESULT -SCG-FAILURE-START

-- FFS if failureType is needed

MeasResultSCG-Failure ::= 			SEQUENCE {

measResultServFreqList					MeasResultServFreqList2NR,

measResultNeighCells						MeasResultList2NR,

...

}

MeasResultServFreqList2NR ::=				SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServFreq2NR

MeasResultServFreq2NR ::=				SEQUENCE {

carrierFreq								ARFCN-ValueNR,

measResultServingCell						MeasResultNR,

measResultBestNeighCell				MeasResultNR		OPTIONAL

}

MeasResultList2NR ::=					SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

MeasResult2NR ::=						SEQUENCE {

carrierFreq								ARFCN-ValueNR,

measResultListNR								MeasResultListNR

}

-- TAG-MEAS-RESULT -SCG-FAILURE-STOP

-- ASN1STOP

–	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL, 	-- Need N

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL, 	-- Need N

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL, 	-- Need N

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL, 	-- Need N

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL, 	-- Need N

measIdToAddModList						MeasIdToAddModList												OPTIONAL, 	-- Need N

-- Other parameters

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-RSRP								RSRP-Range,

csi-RSRP								RSRP-Range

} 																										OPTIONAL, 	-- Need M

quantityConfig						QuantityConfig														OPTIONAL, 	-- Need M

-- Placehold for measGapConfig

measGapConfig							SetupRelease{MeasGapConfig}													OPTIONAL 	-- Need M

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Whether UE speed based TTT scaling (e.g. speedStatePars) is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS Whether measScaleFactor (or equivalent) is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS How to support allowInterruptions in NR (RAN4 input needed) in Rel-15.

MeasConfig field descriptions

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for SpCell RSRP measurement controlling when the UE is required to perform measurements associated to neighbouring cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

–	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

-- ASN1START

-- TAG-MEAS-GAP-CONFIG-START

MeasGapConfig ::=				SEQUENCE {

gapFR2 						GapConfig				OPTIONAL,

...

}

GapConfig ::=					SEQUENCE {

gapOffset 					INTEGER (0..159),

mgl 						ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp 						ENUMERATED {ms20, ms40, ms80, ms160},

...

}

-- TAG-MEAS-GAP-CONFIG-STOP

-- ASN1STOP

MeasGapConfig field descriptions

gapFR2 Indicates measurement gap configuration applies to FR2 only. The applicability of the measurement gap is according to Table 9.1.2-2 in TS 38.133 [14].

gapOffset Value gapOffset is the gap offset of the gap pattern with MGRP indicated in the field mgrp. The value range should be from 0 to mgrp-1.

Mgl Value mgl is the measurement gap length in (ms) of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14].

Mgrp Value mgrp is measurement gap repetition period in (ms) of the measurement gap. The applicability of the measurement gap is according to in Table 9.1.2-1 and Table 9.1.2-2 in TS 38.133 [14].

–	MeasId

The IE MeasId is used to identify a measurement configuration, i.e., linking of a measurement object and a reporting configuration.

MeasId information element

-- ASN1START

-- TAG-MEAS-ID-START

MeasId ::=							INTEGER (1..maxNrofMeasId)

-- TAG-MEAS-ID-STOP

-- ASN1STOP

–	MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-START

MeasIdToAddModList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=					SEQUENCE {

measId								MeasId,

measObjectId						MeasObjectId																	OPTIONAL,

reportConfigId						ReportConfigId

}

-- TAG-MEAS-ID-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

–	MeasObjectEUTRA

The IE MeasObjectEUTRA specifies information applicable for intra-frequency or inter-frequency E-UTRA cells.

Editor’s Note: FFS Details of measObjectEUTRA that can be configured via NR (not applicable for EN-DC).

–	MeasObjectId

The IE MeasObjectId used to identify a measurement object configuration.

MeasObjectId information element

-- ASN1START

-- TAG-MEAS-OBJECT-ID-START

MeasObjectId ::=							INTEGER (1..maxNrofObjectId)

-- TAG-MEAS-OBJECT-ID-STOP

-- ASN1STOP

–	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) intra/inter-frequency measurements or CSI-RS intra/inter-frequency measurements.

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=							SEQUENCE {

ssbAbsoluteFreq								GSCN-ValueNR,

--FFS whether reference frequency represents pointA

refFreqCSI-RS								ARFCN-ValueNR															OPTIONAL,

--RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

referenceSignalConfig						ReferenceSignalConfig,

--Consolidation of L1 measurements per RS index

absThreshSS-BlocksConsolidation			ThresholdNR																	OPTIONAL, 	-- Need R

absThreshCSI-RS-Consolidation			ThresholdNR																	OPTIONAL, 	-- Need R

--Config for cell measurement derivation

nrofSS-BlocksToAverage					INTEGER (2..maxNrofSS-BlocksToAverage)										OPTIONAL, 	-- Need R

nrofCSI-RS-ResourcesToAverage			INTEGER (2..maxNrofCSI-RS-ResourcesToAverage)								OPTIONAL, 	-- Need R

-- Filter coefficients applicable to this measurement object

quantityConfigIndex							INTEGER (1.. maxNrofQuantityConfig),

--Frequency-specific offsets

offsetFreq									Q-OffsetRangeList,

-- Cell list

cellsToRemoveList							PCI-List																OPTIONAL, 	-- Need N

cellsToAddModList							CellsToAddModList															OPTIONAL, 	-- Need N

-- Black list

blackCellsToRemoveList						PCI-RangeIndexList																OPTIONAL, 	-- Need N

blackCellsToAddModList						BlackCellsToAddModList														OPTIONAL, 	-- Need N

-- White list

whiteCellsToRemoveList						PCI-RangeIndexList																OPTIONAL, 	-- Need N

whiteCellsToAddModList						WhiteCellsToAddModList														OPTIONAL 	-- Need N

-- FFS: Where to include L1 parameters for RSSI measurements (SS-RSSI-MeasurementConfig in L1 table)

}

ReferenceSignalConfig ::=     			 SEQUENCE {

subcarrierSpacingSSB                     	SubcarrierSpacingSSB,

-- SSB configuration for mobility (nominal SSBs, timing configuration)

ssb-ConfigMobility						SSB-ConfigMobility				OPTIONAL, 	-- Need M

-- CSI-RS resources to be used for CSI-RS based RRM measurements

csi-rs-ResourceConfigMobility			CSI-RS-ResourceConfigMobility	OPTIONAL -- Need R

}

-- A measurement timing configuration

SSB-ConfigMobility ::= 	SEQUENCE {

subcarrierSpacingSSB                    SubcarrierSpacingSSB,

-- The set of SS blocks to be measured within the SMTC measurement duration.

-- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section)

-- When the field is absent the UE measures on all SS-blocks

-- FFS_CHECK: Is this IE placed correctly.

ssb-ToMeasure							SetupRelease {

CHOICE {

-- bitmap for sub 3 GHz

shortBitmap							BIT STRING (SIZE (4)),

-- bitmap for 3-6 GHz

mediumBitmap						BIT STRING (SIZE (8)),

-- bitmap for above 6 GHz

longBitmap							BIT STRING (SIZE (64))

}

} 																	OPTIONAL,	-- Need M

-- Indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell:

useServingCellTimingForSync				BOOLEAN,

-- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements.

smtc1									SEQUENCE {

-- Periodicity and offset of the measurement window in which to receive SS/PBCH blocks.

-- Periodicity and offset are given in number of subframes.

-- FFS_FIXME: This does not match the L1 parameter table! They seem to intend an index to a hidden table in L1 specs.

-- (see 38.213, section REF):

periodicityAndOffset					CHOICE {

sf5										INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160									INTEGER (0..159)

},

-- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes

-- (see 38.213, section 4.1)

duration								ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

-- The set of SS blocks to be measured within the SMTC measurement duration.

-- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section)

-- When the field is absent the UE measures on all SS-blocks

-- FFS_CHECK: Is this IE placed correctly.

ssb-ToMeasure							SetupRelease {

CHOICE {

-- bitmap for sub 3 GHz

shortBitmap							BIT STRING (SIZE (4)),

-- bitmap for 3-6 GHz

mediumBitmap						BIT STRING (SIZE (8)),

-- bitmap for above 6 GHz

longBitmap							BIT STRING (SIZE (64))

}

}

},

-- Secondary measurement timing confguration for explicitly signalled PCIs. It uses the offset and duration from smtc1.

-- It is supported only for intra-frequency measurements in RRC CONNECTED.

smtc2 									SEQUENCE {

-- PCIs that are known to follow this SMTC.

pci-List								SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId		OPTIONAL, 	-- Need M

-- Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1.

periodicity								ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, spare2, spare1}

}																										OPTIONAL -- Cond IntraFreqConnected

}

CSI-RS-ResourceConfigMobility ::= 		SEQUENCE {

-- MO specific values

isServingCellMO						BOOLEAN

,

-- Subcarrier spacing of CSI-RS.

-- Supported values are 15, 30 or 60 kHz  (<6GHz), 60 or 120 kHz (>6GHz).

-- Corresponds to L1 parameter 'Numerology' (see 38.211, section FFS_Section)

subcarrierSpacingCSI-RS						SubcarrierSpacingCSI-RS,

-- List of cells

csi-RS-CellList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM))	OF CSI-RS-CellMobility

}

CSI-RS-CellMobility ::=				SEQUENCE {

cellId									PhysCellId,

csi-rs-MeasurementBW					SEQUENCE {

-- Allowed size of the measurement BW in PRBs

-- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section)

nrofPRBs			ENUMERATED { size24, size48, size96, size192, size264},

-- Starting PRB index of the measurement bandwidth

-- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section)

-- FFS_Value: Upper edge of value range unclear in RAN1

startPRB			INTEGER(0..251)

},

-- Frequency domain density for the 1-port CSI-RS for L3 mobility

-- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section)

density									ENUMERATED {d1,d3}		OPTIONAL,

-- List of resources

csi-rs-ResourceList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=				SEQUENCE {

csi-rs-ResourceId-RRM					CSI-RS-ResourceId-RRM,

-- FFS_CHECK whether the following fields are supposed to be per resource (here) or in the resource config (above)

-- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)FFS_Ref

slotConfig								CHOICE {

ms5										INTEGER (0..79),

ms10									INTEGER (0..159),

ms20									INTEGER (0..319),

ms40									INTEGER (0..639)

},

-- Each CSI-RS resource may be associated with one SSB. If such SSB is indicated, the NW also indicates whether the UE may assume

-- quasi-colocation of this SSB with this CSI-RS reosurce.

-- Corresponds to L1 parameter 'Associated-SSB' (see FFS_Spec, section FFS_Section)

-- FFS: What does the UE do if it there is no such SSB-Index?

associatedSSB							SEQUENCE {

-- FFS_Value: Check the value range

ssb-Index								SSB-Index	,

-- The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters

-- Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section)

isQuasiColocated						BOOLEAN

}					OPTIONAL,

-- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref

resourceElementMappingPattern			ENUMERATED {ffsTypeAndValue},

-- Sequence generation parameter for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref

sequenceGenerationConfig				INTEGER (0..1023),

...

}

CSI-RS-ResourceId-RRM ::= 				INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

Q-OffsetRangeList ::=					SEQUENCE {

rsrpOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrqOffsetSSB							Q-OffsetRange				DEFAULT dB0,

sinrOffsetSSB							Q-OffsetRange				DEFAULT dB0,

rsrpOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

rsrqOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0,

sinrOffsetCSI-RS						Q-OffsetRange				DEFAULT dB0

}

ThresholdNR ::=							SEQUENCE{

thresholdRSRP							RSRP-Range					OPTIONAL,

thresholdRSRQ							RSRQ-Range					OPTIONAL,

thresholdSINR							SINR-Range					OPTIONAL

}

CellsToAddModList ::=					SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellsToAddMod

CellsToAddMod ::=						SEQUENCE {

physCellId								PhysCellId,

cellIndividualOffset					Q-OffsetRangeList

}

BlackCellsToAddModList ::=				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF BlackCellsToAddMod

BlackCellsToAddMod ::=					SEQUENCE {

pci-RangeIndex							PCI-RangeIndex,

pci-Range								PCI-Range

}

WhiteCellsToAddModList ::=				SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF WhiteCellsToAddMod

WhiteCellsToAddMod ::=					SEQUENCE {

pci-RangeIndex							PCI-RangeIndex,

physCellIdRange							PhysCellIdRange

}

-- TAG-MEAS-OBJECT-NR-STOP

-- ASN1STOP

Editor’s Note: FFS How to support CGI reporting and whether changes are required in MeasObjectNR (e.g. introduction of cellForWhichToReportCGI). Not applicable for EN-DC.

Editor’s Note: FFS Whether alternative TTT is supported in Rel-15 (not applicable for EN-DC).

Editor’s Note: FFS measCycleSCell. (not applicable for EN-DC)

Editor’s Note: FFS reducedMeasPerformance (not applicable for EN-DC).

MeasObjectNR field descriptions

absThreshCSI-RS-Consolidation Absolute threshold for the consolidation of measurement results per CSI-RS resource(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per CSI-RS resource as described in 5.5.3.2.

absThreshSS-BlocksConsolidation Absolute threshold for the consolidation of measurement results per SS/PBCH block(s) from L1 filter(s). The values above the threshold are used as input to the derivation of cell measurement results as described in 5.5.3.3 and the L3 filter(s) per SS/PBCH block index as described in 5.5.3.2.

blackCellsToAddModList List of cells to add/modify in the black list of cells.

blackCellsToRemoveList List of cells to remove from the black list of cells.

carrierFreq Identifies NR carrier frequency for which this configuration is valid.

cellIndividualOffset Cell individual offsets applicable to a specific cell.

cellsToAddModList List of cells to add/modify in the cell list.

cellsToRemoveList List of cells to remove from the cell list.

nrofCSInrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell in that carrierFreq.

nrofSS-BlocksToAverage Indicates the maximum number of measurement results per beam based on SS/PBCH blocks to be averaged. The same value applies for each detected cell in that carrierFreq.

offsetFreq Offset values applicable to the carrier frequency.

physCellId Physical cell identity of a cell in the cell list.

quantityConfigIndex Indicates the n-th element of quantityConfigNR-List provided in MeasConfig.

pci-Range Physical cell identity or a range of physical cell identities.

slotConfig Indicates the CSI-RS periodicity (in milliseconds) and for each periodicity the offset (in number of slots). When subcarrierSpacingCSI-RS is set to 15kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 4/9/19/39 slots. When subcarrierSpacingCSI-RS is set to 30kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 9/19/39/79 slots. When subcarrierSpacingCSI-RS is set to 60kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 19/39/79/159 slots. When subcarrierSpacingCSI-RS is set 120kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 39/79/159/319 slots. When subcarrierSpacingCSI-RS is set 240kHZ, the maximum offset values for periodicities ms5/ms10/ms20/ms40 are 79/159/319/639 slots.

whiteCellsToAddModList List of cells to add/modify in the white list of cells.

whiteCellsToRemoveList List of cells to remove from the white list of cells.

–	MeasObjectToAddModList

The IE MeasObjectToAddModList concerns a list of measurement objects to add or modify.

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-START

MeasObjectToAddModList ::=					SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=						SEQUENCE {

measObjectId								MeasObjectId,

measObject									CHOICE {

measObjectNR								MeasObjectNR

}

}

-- TAG-MEAS-OBJECT-TO-ADD-MOD-LIST-STOP

-- ASN1STOP

–	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, and inter-RAT mobility.

MeasResults information element

-- ASN1START

-- TAG-MEAS-RESULTS-START

MeasResults ::=								SEQUENCE {

measId										MeasId,

measResultServingFreqList					MeasResultServFreqList,

measResultNeighCells						CHOICE {

measResultListNR							MeasResultListNR,

...

}																														OPTIONAL,

...

}

MeasResultServFreqList ::=					SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServFreq

MeasResultServFreq ::=						SEQUENCE {

servFreqId									ServCellIndex,

measResultServingCell						MeasResultNR,

measResultBestNeighCell						MeasResultNR,

...

}

MeasResultListNR ::=						SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=							SEQUENCE {

physCellId									PhysCellId																	OPTIONAL,

--FFS: Details of cgi info

cgi-Info									ENUMERATED {ffsTypeAndValue}												OPTIONAL,

measResult									SEQUENCE {

cellResults									SEQUENCE{

resultsSSB-Cell								MeasQuantityResults														OPTIONAL,

resultsCSI-RS-Cell 							MeasQuantityResults													OPTIONAL

},

rsIndexResults								SEQUENCE{

resultsSSB-Indexes							ResultsPerSSB-IndexList												OPTIONAL,

resultsCSI-RS-Indexes						ResultsPerCSI-RS-IndexList 											OPTIONAL

}																													OPTIONAL

},

...

}

MeasQuantityResults ::= 		SEQUENCE {

rsrp							RSRP-Range													OPTIONAL,

rsrq							RSRQ-Range													OPTIONAL,

sinr							SINR-Range													OPTIONAL

}

ResultsPerSSB-IndexList ::= 					SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::= 						SEQUENCE {

ssb-Index									SSB-Index,

ssb-Results									MeasQuantityResults							OPTIONAL

}

ResultsPerCSI-RS-IndexList ::= 				SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::= 					SEQUENCE {

csi-		RS-Index									CSI-RS-Index,

csi-RS-Results								MeasQuantityResults							OPTIONAL

}

-- TAG-MEAS-RESULTS-STOP

-- ASN1STOP

Editor’s Note: FFS locationInfo.

MeasResults field descriptions

csi-rs-CellRSRP Measured RSRP result per NR cell based on CSI-RSRP value(s) from the L1 filter(s).

csi-rs-CellRSRQ Measured RSRQ result per NR cell based on CSI-RSRQ value(s) from the L1 filter(s).

csi-rs-CellSINR Measured SINR result per NR cell based on CSI-SINR value(s) from the L1 filter(s).

csi-rs-Index CSI-RS resource index associated to the measurement information to be reported.

csi-RSRP L3 filtered CSI-RSRP measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRP is defined in TS 38.215 [9].

csi-RSRQ L3 filtered CSI-RSRQ measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-RSRQ is defined in TS 38.215 [9].

csi-SINR L3 filtered CSI-SINR measurement per CSI-RS resource index, as defined in 5.5.4.x. CSI-SINR is defined in TS 38.215 [9].

measId Identifies the measurement identity for which the reporting is being performed.

measResult Measured results of an NR cell.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultServingFreqList Measured results of the serving frequencies including measurement results of SpCell, configured SCell(s) and best neighbouring cell on each serving frequency.

resultsCSI-RS-Indexes List of measurement information per CSI-RS resource index of an NR cell.

resultsSSB-Indexes List of measurement information per SS/PBCH index of an NR cell.

resultsCSI-RS-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived from CSI-RS measurements.

resultSSB-Cell Cell level measurement results (e.g. RSRP, RSRQ, SINR) to be reported derived on SS/PBCH block measurements.

smtc2 Secondary measurement timing configuration for explicitly signalled PCIs. The timing offset is equal to SMTC1 offset mod SMTC2 periodicity.

ssb-CellRSRP Measured RSRP result per NR cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-CellRSRQ Measured RSRQ result of an NR Cell based on SS-RSRP value(s) from the L1 filter(s).

ssb-CellSINR Measured SS-SINR result of an NR Cell based on SS-SINR value(s) from the L1 filter(s)..

ssb-Index SS/PBCH block index associated to the measurement information to be reported.

ss-rsrp L3 filtered SS-RSRP measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRP is defined in TS 38.215 [9].

ss-rsrq L3 filtered SS-RSRQ measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-RSRQ is defined in TS 38.215 [9].

ss-sinr L3 filtered SS-SINR measurement per SS/PBCH block index, as defined in 5.5.4.x. SS-SINR is defined in TS 38.215 [9].

–	PCI-List

The IE PCI-List concerns a list of physical cell identities, which may be used for different purposes.

PCI-List information element

-- ASN1START

-- TAG-PCI-LIST-START

PCI-List ::=						SEQUENCE (SIZE (1..maxNrofCellMeas)) OF PhysCellId

-- TAG-PCI-LIST-STOP

-- ASN1STOP

–	PCI-Range

The IE PCI-Range is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of PCI-Range, the Network may configure overlapping ranges of physical cell identities.

PCI-Range information element

-- ASN1START

-- TAG-PCI-RANGE-START

PCI-Range ::=				SEQUENCE {

start							PhysCellId,

range							ENUMERATED {

n4, n8, n12, n16, n24, n32, n48, n64, n84,

n96, n128, n168, n252, n504, n1008,

spare1} 					OPTIONAL	-- Need OP

}

-- TAG-PCI-RANGE-STOP

-- ASN1STOP

PCI-Range field descriptions

range Indicates the number of physical cell identities in the range (including start). Value n4 corresponds with 4, n8 corresponds with 8 and so on. The UE shall apply value 1 in case the field is absent, in which case only the physical cell identity value indicated by start applies.

start Indicates the lowest physical cell identity in the range.

–	PCI-RangeIndex

The IE PCI-RangeIndex identifies of physical cell id range, which may be used for different purposes.

PCI-RangeIndex information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-START

PCI-RangeIndex ::=						INTEGER (0..maxNrofPCI-Ranges)

-- TAG-PCI-RANGE-INDEX-STOP

-- ASN1STOP

–	PCI-RangeIndexList

The IE PCI-RangeIndexList concerns a list of indices of physical cell id ranges, which may be used for different purposes.

PCI-RangeIndexList information element

-- ASN1START

-- TAG-PCI-RANGE-INDEX-LIST-START

PCI-RangeIndexList ::=						SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeIndex

-- TAG-PCI-Range-INDEX-LIST-STOP

-- ASN1STOP

ReportConfigNR information element

-- ASN1START

-- TAG-REPORT-CONFIG-START

ReportConfigNR ::=							SEQUENCE {

reportType									CHOICE {

periodical									PeriodicalReportConfig,

eventTriggered								EventTriggerConfig,

-- reportCGI is to be completed before the end of Rel-15.

reportCGI									ENUMERATED {ffsTypeAndValue},

...

}

}

-- FFS / TODO: Consider separating trgger configuration (trigger, periodic, …) from report configuration.

-- Current structure allows easier definiton of new events and new report types e.g. CGI, etc.

EventTriggerConfig::=						SEQUENCE {

eventId										CHOICE {

eventA1										SEQUENCE {

a1-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA2										SEQUENCE {

a2-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger

},

eventA3										SEQUENCE {

a3-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA4										SEQUENCE {

a4-Threshold								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA5										SEQUENCE {

a5-Threshold1								MeasTriggerQuantity,

a5-Threshold2								MeasTriggerQuantity,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

eventA6										SEQUENCE {

a6-Offset									MeasTriggerQuantityOffset,

reportOnLeave								BOOLEAN,

hysteresis									Hysteresis,

timeToTrigger								TimeToTrigger,

useWhiteCellList							BOOLEAN

},

...

},

rsType										NR-RS-Type,

-- Common reporting config (at least to periodical and eventTriggered)

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

-- Cell reporting configuration

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

-- RS index reporting configuration

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL, 	-- Need M

maxNrofRSIndexesToReport					INTEGER (1..maxNrofIndexesToReport) 								OPTIONAL, 	-- Need M

includeBeamMeasurements						BOOLEAN,

-- If configured the UE includes the best neighbour cells per serving frequency

reportAddNeighMeas							ENUMERATED {setup}									OPTIONAL,		-- Need R

...

}

PeriodicalReportConfig ::=					SEQUENCE {

rsType										NR-RS-Type,

-- Common reporting config (at least to periodical and eventTriggered)

reportInterval								ReportInterval,

reportAmount								ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

-- Cell reporting configuration

reportQuantityCell							MeasReportQuantity,

maxReportCells								INTEGER (1..maxCellReport),

-- RS index reporting configuration

reportQuantityRsIndexes						MeasReportQuantity													OPTIONAL, 	-- Need R

maxNrofRsIndexesToReport						INTEGER (1..maxNrofIndexesToReport) 									OPTIONAL, 	-- Need R

includeBeamMeasurements						BOOLEAN,

...

}

NR-RS-Type ::= ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=						CHOICE {

rsrp										RSRP-Range,

rsrq										RSRQ-Range,

sinr										SINR-Range

}

MeasTriggerQuantityOffset ::=				CHOICE {

rsrp										INTEGER (1..ffsValue),

rsrq										INTEGER (1..ffsValue),

sinr										INTEGER (1..ffsValue)

}

MeasReportQuantity ::=						SEQUENCE {

rsrp										BOOLEAN,

rsrq										BOOLEAN,

sinr										BOOLEAN

}

-- TAG-REPORT-CONFIG-START

-- ASN1STOP

–	QuantityConfig

The IE QuantityConfig specifies the measurement quantities and layer 3 filtering coefficients for NR and inter-RAT measurements.

QuantityConfig information element

-- ASN1START

-- TAG-QUANTITY-CONFIG-START

QuantityConfig ::=							SEQUENCE {

quantityConfigNR-List						QuantityConfigNR-List												OPTIONAL, 	-- Need M

...

}

QuantityConfigNR-List::=						SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR

QuantityConfigNR::=							SEQUENCE {

quantityConfigCell							QuantityConfigRS,

quantityConfigRS-Index						QuantityConfigRS													OPTIONAL 	-- Need M

}

QuantityConfigRS ::=						SEQUENCE {

-- SS Block based L3 filter configurations:

ssb-FilterConfig					FilterConfig,

-- CSI-RS based L3 filter configurations:

cs-RS-FilterConfig				FilterConfig

}

FilterConfig ::=				SEQUENCE {

filterCoefficientRSRP			FilterCoefficient										DEFAULT ffsTypeAndValue,

filterCoefficientRSRQ			FilterCoefficient										DEFAULT ffsTypeAndValue,

filterCoefficientRS-SINR			FilterCoefficient										DEFAULT ffsTypeAndValue

}

-- TAG-QUANTITY-CONFIG-STOP

-- ASN1STOP

QuantityConfig field descriptions

quantityConfigCell Specifies L3 filter configurations for cell measurement results for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

quantityConfigNR Specifies filter configurations for NR measurements.

quantityConfigRSindex Specifies L3 filter configurations for measurement results per RS index for the configurable RS Types (e.g. SS/PBCH block and CSI-RS) and the configurable measurement quantities (e.g. RSRP, RSRQ and SINR).

ssb-FilterConfig Specifies L3 filter configurations for SS-RSRP, SS-RSRQ and SS-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

csi-rs-FilterConfig Specifies L3 filter configurations for CSI-RSRP, CSI-RSRQ and CSI-SINR measurement results from the L1 filter(s), as defined in 38.215 [9].

–	ReportInterval

The ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerType event as well as for triggerType periodical. Value ms120 corresponds to 120 ms, ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

ReportInterval ::=					ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,

min6, min12, min30, spare2, spare1}

-- ASN1STOP

–	RSRP-Range

The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. Integer value for RSRP measurements according to mapping table in TS 38.133 [14].

RSRP-Range information element

-- ASN1START

-- TAG-RSRP-RANGE-START

RSRP-Range ::=						INTEGER(0..124)

-- TAG-RSRP-RANGE-STOP

-- ASN1STOP

–	RSRQ-Range

The IE RSRQ-Range specifies the value range used in RSRQ measurements and thresholds. Integer value for RSRQ measurements is according to mapping table in TS 38.133 [14].

RSRQ-Range information element

-- ASN1START

-- TAG-RSRQ-RANGE-START

RSRQ-Range ::=						INTEGER(0..127)

-- TAG-RSRQ-RANGE-STOP

-- ASN1STOP

–	SINR-Range

The IE SINR-Range specifies the value range used in SINR measurements and thresholds. Integer value for SINR measurements is according to mapping table in TS 38.133 [14].

SINR-Range information element

-- ASN1START

-- TAG-SINR-RANGE-START

SINR-Range ::=						INTEGER(0..127)

-- TAG-SINR-RANGE-STOP

-- ASN1STOP

7	Variables and constants

7.4	UE variables

NOTE: 	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

–	NR-UE-Variables

This ASN.1 segment is the start of the NR UE variable definitions.

-- ASN1START

NR-UE-Variables DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

MeasId,

MeasIdToAddModList,

MeasObjectToAddModList,

PhysCellIdEUTRA,

PhyCellNR,

ReportConfigToAddModList,

RSRP-Range,

QuantityConfig,

maxNrofCellMeas,

maxNrofMeasId

FROM NR-RRC-Definitions;

-- ASN1STOP

–	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

VarMeasConfig UE variable

-- ASN1START

-- TAG-VAR-MEAS-CONFIG-START

VarMeasConfig ::=					SEQUENCE {

-- Measurement identities

measIdList							MeasIdToAddModList					OPTIONAL,

-- Measurement objects

measObjectList						MeasObjectToAddModList				OPTIONAL,

-- Reporting configurations

reportConfigList					ReportConfigToAddModList			OPTIONAL,

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,

s-MeasureConfig							CHOICE {

ssb-RSRP								RSRP-Range,

csi-RSRP								RSRP-Range

} 																		OPTIONAL

}

-- TAG-VAR-MEAS-CONFIG-STOP

-- ASN1STOP

Editor’s Note: FFS Revisit whether we really need VarMeasConfig.

–	VarMeasReportList

The UE variable VarMeasReportList includes information about the measurements for which the triggering conditions have been met.

VarMeasReportList UE variable

-- ASN1START

-- TAG-VAR-MEAS-REPORT-START

VarMeasReportList ::=				SEQUENCE (SIZE (1..maxNrofMeasId)) OF VarMeasReport

VarMeasReport ::=					SEQUENCE {

-- List of measurement that have been triggered

measId								MeasId,

cellsTriggeredList					CellsTriggeredList				OPTIONAL,

numberOfReportsSent					INTEGER

}

CellsTriggeredList ::=				SEQUENCE (SIZE (1.. maxNrofCellMeas)) OF CHOICE {

physCellIdEUTRA						PhysCellIdEUTRA,

phyCellNR							PhyCellNR

}

-- TAG-VAR-MEAS-REPORT-STOP

-- ASN1STOP

–	End of NR-UE-Variables

-- ASN1START

END

-- ASN1STOP