3GPP TSG-RAN WG2 NR Ad-Hoc meeting	R2-18xxxxx

# Vancouver, Canada, 22nd - 26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update UE capabilities according to the following agreements: - Linking MR-DC BCs to BPCs (R2-1800909) - BC structure with UL and DL decoupling (R2-181620) - BPC capability coordination in MR-DC (R2-1800740) - Clarifications on BPC capabilities (R2-1801532) - UE capabilities on dynamic power sharing (R2-1801520) - L2/3 capabilities (R2-1801608) - I.044 (R2-1800955) - N.037, N.038, N.040, N.045, N.046, N.221, N.222 (R2-1800831) - 5.6, 5.7 to 5.7.1 (E.027), 6.3.3 (C.033, M.052, M.054, Z.078, I.078, I.083. H.277) in RIL 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “FFS if supportedBasebandProcessingCombination-MRDC is included here or BandCombinationList” in the ASN.1. - Fill in the blanks of BandCombinationList with UL and DL decoupling. - Add BPC coordination information in SCG-ConfigInfo and SCG-Config. - Updates supportedBW-PerCC and modulationOrder as per CC capability. - Add dynamicPowerSharing and eutra-BasedTDM in MR-DC container. - Update L2/3 capabilities: - Relocate intraBandAsyncFDD into RF-Parameters in MR-DC container. - Add intraAndInterF-MeasAndReport and eventA-MeasAndReport into MeasParameters in UE-NR-Capability. - Add splitSRB-WithOneUL-Path and directSN-Addition into generalParameters in MR-DC container. - Add fdd-UE-MRDC-Capability, tdd-UE-MRDC-Capability, fdd-UE-NR-Capability, and tdd-UE-NR-Capability. - Remove “FFS utra, geran-cs, geran-ps and cdma2000-1XRTT” - Remove volteOverNR-PDCP from NR ASN.1 - Change MR-DC to eutra-nr in RAT-Type, add FreqBandList IE and replace requestedFreqBandList by FreqBandList in 5.6.1.4. - Update the following changes: - Change sentence to “if FreqBandList is received:” in 5.6.1.4. - Change maxSimultaneousBands to maxRequestedBands. - Add “MN” and “SN” into each basebandProcessingCombinationIndex in LinkedBasebandProcessingCombination. - Change maxServCell to maxNrofCC. - Change subCarrierSpacing to supportedSubCarrierSpacingList. - Clarify maxRateDRB-IP is not supported for EN-DC. - Remove SupportedBandCombination ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination - Update field description to remove E-UTRA in RAT-Type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

The First Change

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

#### Editor’s Note: Targeted for completion in June 2018.5.6.1.2	Initiation

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

Editor’s Note: Targeted for completion in June 2018.

#### 5.6.1.4	Compilation of band combinations supported by the UE

The UE shall:

1>	if FreqBandList is received:

2>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message,  only consisting of bands included in FreqBandList, and prioritized in the order of FreqBandList, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

2>	for each band combination included in the candidate list:

3>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of candidates as specified in TS 38.306 [xx]:

4>	remove the band combination from the list of candidates.

2>	include all band combinations in the candidate list into supportedBandCombination.

1>	else:

2>	include all band combinations supported by the UE into supportedBandCombination, excluding fallback band combinations with the same capabilities of another band combination included in the list of band combinations supported by the UE.

#### 5.6.1.5	Compilation of baseband processing combinations supported by the UE

The UE shall:

1>	for each band combination included in supportedBandCombination:

2>	include the baseband processing combination supported for the band combination into supportedBasebandProcessingCombination, unless it is already included;

2>	if there are the fallback baseband processing combinations of this baseband processing combination as specified in TS 38.306 [xx] for which supported baseband capabilities are different from this baseband processing combination:

3>	include the fallback baseband processing combinations into supportedBasebandProcessingCombination.

The Next Change

### 6.3.3	UE capability information elements

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA and/or MR-DC band combinations.

BandCombinationList information element

-- ASN1START

BandCombinationList ::=	SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombination ::= SEQUENCE {

bandCombinationDL		BandCombinationDL,

bandCombinationsUL		BIT STRING (SIZE (1..maxBandCombUL))	OPTIONAL

}

BandCombinationDL ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandDL-Info

BandDL-Info ::= SEQUENCE {

frequencyBand		FreqBandInformation,

bandParametersDL	BandParametersDL					OPTIONAL -- Not included in case of SUL

}

BandCombinationUL-PerBC-DL ::= SEQUENCE (SIZE (1..maxBandCombUL)) OF BandCombinationUL-Parameters

BandCombinationUL-Parameters ::= SEQUENCE {

bandCombinationUL			BandCombinationUL,

multipleTimingAdvances		ENUMERATED {supported}		OPTIONAL,

singleTransmission			ENUMERATED {supported}		OPTIONAL,

intraBandSimultaneousTxRx	ENUMERATED {supported}		OPTIONAL	-- FFS per UE or per band Combination

}

BandCombinationUL ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandUL-Info

BandUL-Info ::= SEQUENCE {

bandParametersUL		BandParametersUL				OPTIONAL -- Not included in case of DL-only band

}

FreqBandInformation ::= CHOICE {

bandEUTRA		FreqBandIndicatorEUTRA,

bandNR			FreqBandIndicator

}

BandParametersDL ::= SEQUENCE {

bandwidthClassDL		CHOICE {

ca-BandwidthClassDL-EUTRA		CA-BandwidthClassEUTRA,

ca-BandwidthClassDL				CA-BandwidthClass

},

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL

}

BandParametersUL ::= SEQUENCE {

bandwidthClassUL		CHOICE {

ca-BandwidthClassUL-EUTRA		CA-BandwidthClassEUTRA,

ca-BandwidthClassUL				CA-BandwidthClass

},

scalingFactor0dot75			ENUMERATED {supported}		OPTIONAL

}

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used to contain list of NR and/or E-UTRA frequency bands for which the UE is requested to provide its supported NR CA and/or MR-DC band combinations (i.e. within the UE capability containers for NR and MR-DC, as requested by E-UTRA).

FreqBandList information element

-- ASN1START

FreqBandList ::=	SEQUENCE (SIZE (1..maxRequestedBands)) OF FreqBandInformation

FreqBandInformation ::= CHOICE {

bandEUTRA				FreqBandIndicatorEUTRA,

bandNR					FreqBandIndicator

}

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/ transferred UE capabilities.

RAT-Type information element

-- ASN1START

RAT-Type ::= ENUMERATED {nr, eutra-nr, spare2,spare1, ...}

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of containers, one for each RAT for which UE capabilities are transferred, if any.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

UE-CapabilityRAT-ContainerList ::=SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::= SEQUENCE {

rat-Type							RAT-Type,

ueCapabilityRAT-Container			OCTET STRING

}

-- ASN1STOP

UECapabilityRAT-ContainerList field descriptions

ueCapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For NR: the encoding of UE capabilities is defined in UE-NR-Capability. For MRDC: the encoding of UE capabilities is defined in UE-MRDC-Capability.

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [yy].

UE-MRDC-Capability information element

-- ASN1START

UE-MRDC-Capability ::=	SEQUENCE {

measParameters-MRDC			MeasParameters-MRDC,

rf-Parameters-MRDC			RF-Parameters-MRDC,

phyLayerParameters-MRDC		PhyLayerParameters-MRDC,

generalParameters-MRDC		GeneralParameters-MRDC		OPTIONAL,

fdd-UE-MRDC-Capability		XDD-UE-MRDC-Capability		OPTIONAL,

tdd-UE-MRDC-Capability		XDD-UE-MRDC-Capability		OPTIONAL

-- FFS on other parameters

}

RF-Parameters-MRDC ::= SEQUENCE {

supportedBandCombination	BandCombinationList,

dynamicPowerSharing			ENUMERATED {supported}		OPTIONAL,

eutra-BasedTDM				ENUMERATED {supported}		OPTIONAL,

intraBandAsyncFDD			ENUMERATED {supported}		OPTIONAL

-- FFS whether intraBandAsyncFDD is included per UE or per band combination

-- FFS on other parameters

}

PhyLayerParameters-MRDC ::= SEQUENCE {

supportedBasebandProcessingCombination-MRDC		BasebandProcessingCombination-MRDC

-- FFS on other parameters

}

BasebandProcessingCombination-MRDC ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF LinkedBasebandProcessingCombination

}

LinkedBasebandProcessingCombination ::= SEQUENCE {

basebandProcessingCombinationIndexMN			BasebandProcessingCombinationIndex,

BasebandProcessingCombinationLinkedIndexSN		SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

}

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

MeasParameters-MRDC ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-MeasType1				ENUMERATED {supported}	OPTIONAL

}

GeneralParameters-MRDC ::= SEQUENCE {

splitSRB-WithOneUL-Path		ENUMERATED {supported}	OPTIONAL,

directSN-Addition			ENUMERATED {supported}	OPTIONAL

}

XDD-UE-MRDC-Capability ::= SEQUENCE {

intraCarrierConcurrentMeas	ENUMERATED {supported}	OPTIONAL,

independentGapConfig		ENUMERATED {supported}	OPTIONAL,

sstd-MeasType1				ENUMERATED {supported}	OPTIONAL,

splitSRB-WithOneUL-Path		ENUMERATED {supported}	OPTIONAL,

directSN-Addition			ENUMERATED {supported}	OPTIONAL

}

--ASN1STOP

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [yy].

UE-NR-Capability information element

-- ASN1START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-UE-NR-Capability			XDD-UE-NR-Capability				OPTIONAL,

tdd-UE-NR-Capability			XDD-UE-NR-Capability				OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PhyLayerParameters ::=	SEQUENCE {

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination,

-- FFS on other parameters

}

RF-Parameters ::= SEQUENCE {

supportedBandListNR					SupportedBandListNR,

supportedBandCombination			BandCombinationList,

}

SupportedBandListNR ::=	SEQUENCE (SIZE (1..maxBands)) OF BandNR

SupportedBasebandProcessingCombination ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombination

BasebandProcessingCombination ::= SEQUENCE {

basebandParametersPerBand		SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BasebandParametersPerBand

-- FFS on other parameters

}

BasebandParametersPerBand ::= SEQUENCE {

ca-BandwidthClassDL			CA-BandwidthClass						OPTIONAL,

ca-BandwidthClassUL			CA-BandwidthClass						OPTIONAL,

basebandParametersPerCC		SEQUENCE (SIZE (1..maxNrofCC)) OF BasebandParametersPerCC,

-- FFS on other parameters

}

BasebandParametersPerCC ::= SEQUENCE {

bandwidthPerCC-DL				BandwidthPerCC						OPTIONAL,

bandwidthPerCC-UL				BandwidthPerCC						OPTIONAL,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL,

supportedModulationOrderDL					ModulationOrder			OPTIONAL,

supportedModulationOrderUL		ModulationOrder						OPTIONAL,

supportedSubCarrierSpacingList				SupportedSubCarrierSpacingList,

-- FFS if supportedSubCarrierSpacing is included per Band or per CC and whether to separate one for DL and UL.

-- FFS on other parameters

}

BandNR ::=	SEQUENCE {

bandNR							FreqBandIndicator,

supportedMIMO-CapabilityDL		MIMO-Capability						OPTIONAL,

supportedMIMO-CapabilityUL		MIMO-Capability						OPTIONAL,

-- FFS on other parameters

}

CA-BandwidthClass ::= ENUMERATED {a, b, c, d, e, f, ...}

-- FFS value ranges

MIMO-Capability ::= SEQUENCE {

-- FFS on the parameters

}

BandwidthPerCC ::= ENUMERATED {TBD}

-- FFS value ranges

ModulationOrder ::= SEQUENCE {

-- FFS on the parameters

}

SupportedSubCarrierSpacingList ::= SEQUENCE {

-- FFS on the parameters

}

PDCP-Parameters ::= SEQUENCE {

dataRateDRB-IP		ENUMERATED {64kbps, spare7, spare6, spare5, spare4, spare3, spare2, spare1}		OPTIONAL, -- Not supported for EN-DC.

supportedROHC-Profiles	SEQUENCE {

profile0x0000		BOOLEAN,

profile0x0001		BOOLEAN,

profile0x0002		BOOLEAN,

profile0x0003		BOOLEAN,

profile0x0004		BOOLEAN,

profile0x0006		BOOLEAN,

profile0x0101		BOOLEAN,

profile0x0102		BOOLEAN,

profile0x0103		BOOLEAN,

profile0x0104		BOOLEAN

},

maxNumberROHC-ContextSessions	ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles			ENUMERATED {supported}	OPTIONAL,

continueROHC-Context			ENUMERATED {supported}	OPTIONAL,

outOfOrderDelivery				ENUMERATED {supported}	OPTIONAL,

shortSN							ENUMERATED {supported} 	OPTIONAL,

}

RLC-Parameters ::= SEQUENCE {

amWithShortSN					ENUMERATED {supported}	OPTIONAL,

umWithShortSN					ENUMERATED {supported}	OPTIONAL,

umWIthLongSN					ENUMERATED {supported}	OPTIONAL

}

MAC-Parameters ::= SEQUENCE {

lcp-Restriction					ENUMERATED {supported}	OPTIONAL,

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4,…}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4,…}	OPTIONAL -- FFS value range

}

MeasParameters ::= SEQUENCE {

intraAndInterF-MeasAndReport		ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

}

XDD-UE-NR-Capability ::= SEQUENCE {

skipUplinkTxDynamic				ENUMERATED {supported}	OPTIONAL,

logicalChannelSR-DelayTimer		ENUMERATED {supported}	OPTIONAL,

longDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

shortDRX-Cycle					ENUMERATED {supported}	OPTIONAL,

numberOfSR-Configurations		ENUMERATED {n2, n3, n4,…}	OPTIONAL, -- FFS value range

numberOfConfiguredGrantConfigurations	ENUMERATED {n2, n3, n4,…}	OPTIONAL, -- FFS value range

intraAndInterF-MeasAndReport		ENUMERATED {supported}	OPTIONAL,

eventA-MeasAndReport			ENUMERATED {supported}	OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

### 6.3.4	Other information elements

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxBandComb								INTEGER ::=	FFS		-- Maximum number of DL band combinations

maxBandCombUL							INTEGER ::= FFS		-- Maximum number of UL band combinations

maxBasebandProcComb						INTEGER ::=	FFS		-- Maximum number of base band processing combinations

maxNrofSCells							INTEGER ::=	15		-- Max number of secondary serving cells per cell group

maxNrofCellMeas							INTEGER ::=	FFS		-- Maximum number of entries in each of the cell lists in a measurement object

maxNroSS-BlocksToAverage				INTEGER ::= FFS		-- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNroCSI-RS-ResourcesToAverage			INTEGER ::= FFS		-- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofSR-CongigPerCellGroup			INTEGER ::= 8		-- Maximum number of SR configurations per cell group

maxLCG-ID								INTEGER ::= 7		-- Maximum value of LCG ID

macLC-ID								INTEGER ::= FFS		-- Maximum value of Logical Channel ID

maxNrofTAGs								INTEGER ::=	4		-- Maximum number of Timing Advance Groups

maxNrofTAGs-1							INTEGER ::=	3		-- Maximum number of Timing Advance Groups minus 1

maxNrofBandwidthParts					INTEGER ::= 4		-- Maximum number of BWPs per serving cell

maxNrofBandwidthParts-1					INTEGER ::= 3		-- Maximum number of BWPs per serving cell minus 1

maxSymbolIndex							INTEGER ::= 13		-- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofPhysicalResourceBlocks			INTEGER ::= 275		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1			INTEGER ::= 274		-- Maximum number of PRBs

maxNrofPhysicalResourceBlocksTimes4		INTEGER ::= FFS:1100	-- Maximum number of PRBs (used to reference PRBs in another subcarrier spacing)

maxNrofControlResourceSets 				INTEGER ::= FFS 	-- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1			INTEGER ::= FFS 	-- Max number of CoReSets configurable on a serving cell minus 1

maxCoReSetStartSymbol					INTEGER ::= FFS		-- Highest possible start symbol for a control resource set

maxCoReSetDuration						INTEGER ::= 3		-- Max number of OFDM symbols in a control resource set

maxNrofSearchSpacesPerCoReSet			INTEGER ::= FFS		-- Max number of search spaces configurable per Control Resource Set

maxNrofRateMatchPatterns				INTEGER ::= FFS		-- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1				INTEGER ::= FFS		-- Max number of rate matching patterns that may be configured minus 1

maxNrofCSI-Reports						INTEGER ::= FFS 	-- Maximum number of report configurations

maxNrofCSI-Reports-1	 				INTEGER ::= FFS 	-- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations		INTEGER ::= FFS		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= FFS		-- Maximum number of resource configurations minus 1

maxNrofCSI-ResourceSets					INTEGER ::= FFS		-- Maximum number of resource sets per resource configuration

maxNrofCSI-ResourceSets-1				INTEGER ::= FFS		-- Maximum number of resource sets per resource configuration minus 1

maxNrofNZP-CSI-RS-Resources				INTEGER ::= FFS		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= FFS		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= FFS		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= FFS		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= FFS		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= FFS		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofSSB-Resources					INTEGER ::= 64		-- Maximum number of SSB resources in a resource set

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofCSI-RS-ResourcesPerSet			INTEGER ::= 8		-- Maximum number of CSI-RS resources per resource set

maxNrofCSI-MeasId						INTEGER ::= FFS		-- Maximum number of link configurations

maxNrofCSI-MeasId-1						INTEGER ::= FFS		-- Maximum number of link configurations minus 1

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= FFS		-- Maximum number of CSI-RS resources for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1			INTEGER ::= FFS		-- Maximum number of CSI-RS resources for an RRM measurement object minus 1

maxNrofObjectId							INTEGER ::= FFS		-- Maximum number of configured measurement objects

maxNrOfRA-PreamblesPerSSB				INTEGER ::=	FFS		-- Maximum number of Random Access Preamble value per SSB

maxNrofReportConfigId					INTEGER ::= FFS		-- Maximum number of reporting configurations

maxNrofMeasId							INTEGER ::= FFS		-- Maximum number of configured measurements

maxNroQuantityConfig					INTEGER	::= 2		-- Maximum number of quantity configurations

maxNrofSRS-ResourceSets					INTEGER ::= FFS		-- Maximum number of SRS resource sets.

maxNrofSRS-ResourceSets-1				INTEGER ::= FFS		-- Maximum number of SRS resource sets minus 1.

maxNrofSRS-Resources					INTEGER ::= FFS		-- Maximum number of SRS resources in an SRS resource set.

maxNrofSRS-Resources-1					INTEGER ::= FFS		-- Maximum number of SRS resources in an SRS resource set minus 1.

maxRA-PreambleIndex						INTEGER ::= FFS		-- Maxximum value of Random Access Preamble Index

maxRAT-CapabilityContainers				INTEGER ::= FFS		-- Maximum number of interworking RAT containers (incl NR and MRDC)

maxRequestedBands						INTEGER ::= FFS		-- Maximum number of simultaneously requested bands

maxNrofCC								INTEGER ::=	FFS		-- Maximum number of carriers

maxSimultaneousBands					INTEGER ::= FFS		-- Maximum number of simultaneously aggregated bands

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

The Next Change

#### –	SCG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB.

Direction: Secondary gNB to master gNB or eNB

SCG-Config message

-- ASN1START

-- TAG-SCG-CONFIG-START

SCG-Config ::=						SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

scg-Config-r15					SCG-Config-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCG-Config-r15-IEs ::=				SEQUENCE {

scg-CellGroupdConfig				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

p-maxFR1							P-Max												OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfiguration)	OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG								OPTIONAL,

candidateCellInfoList				CandidateCellInfoList								OPTIONAL,

nonCriticalExtension				SEQUENCE {}											OPTIONAL

}

ConfigRestrictModReqSCG ::=			SEQUENCE {

requestedBC-List-NR					SEQUENCE OF INTEGER					OPTIONAL,

requestedBPC-List-NR				BasebandProcessingCombinationList					OPTIONAL,

-- FFS Signalling details of BC requested by SgNB to be alleviated

...

}

BasebandProcessingCombinationList ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

-- TAG-SCG-CONFIG-STOP

-- ASN1STOP

SCG-Config field descriptions

p-maxFR1 Indicates the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the target SgNB

configRestrictModReq Used by SN to re-negotiate SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. used to request configure an NR band combination which use MN has previously forbidden.

#### –	SCG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration.

Direction: Master eNB or gNB to secondary gNB

SCG-ConfigInfo message

-- ASN1START

-- TAG-SCG-CONFIG-INFO-START

SCG-ConfigInfo ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

scg-ConfigInfo-r15					SCG-ConfigInfo-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCG-ConfigInfo-r15-IEs ::=			SEQUENCE {

eutra-CapabilityInfo			OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,

candidateCellInfoList			CandidateCellInfoList								OPTIONAL,

measResultSSTD					MeasResultSSTD										OPTIONAL,

configRestrictInfo				ConfigRestrictInfoSCG								OPTIONAL,

drx-InfoMCG						DRX-Info											OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

p-maxFR1						P-Max												OPTIONAL,

mcg-RB-Config					OCTET STRING (CONTAINING RadioBearerConfiguration)	OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

ConfigRestrictInfoSCG ::=			SEQUENCE {

restrictedBandCombinationNR				INTEGER							OPTIONAL,

restrictedBasebandCombinationNR		BasebandProcessingCombinationList				OPTIONAL,

-- FFS Signalling details of BC restrictions to be observed by SgNB

-- FFS Signalling details regarding power coordination

maxMeasFreqsSCG-NR					INTEGER							OPTIONAL,

...

}

DRX-Info ::=					SEQUENCE {

cycle							INTEGER,

offset							INTEGER

}

BasebandProcessingCombinationList ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BasebandProcessingCombinationIndex

BasebandProcessingCombinationIndex ::= INTEGER (1..maxBasebandProcComb)

-- TAG-SCG-CONFIG-INFO-STOP

-- ASN1STOP

SCG-ConfigInfo field descriptions

candidateCellInfoList Contains information regarding cells that the source suggests the target gNB to consider configuring.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change.

p-maxFR1 Indicates the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

sourceConfigSCGIncludes the current dedicated SCG configuration in the same format as SCG-Config, i.e. not only CellGroupConfig but also e.g. rb-Config, measConfig.

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

restrictedBandCombinationNR Indicates restrictions regarding the NR BCs the SN can configure by signalling the LTE BC selected by MN. The SN may configure any EN-BC including the indicated LTE BC selected by MN.

restrictedBasebandCombinationNR Indicates restrictions regarding the NR BPCs the SN can/ cannot configure i.e. by signalling the list of NR BPC the SN may configure.