ö3GPP TSG-RAN2#AH-1801	Tdoc R2-18xxxxx

Vancouver, Canada, 22-26 Jan 2018

Agenda Item:		x.y

Source:				Ericsson

Title:					Review issue list for TS 38.331 EN-DC ASN.1 freeze

Version:				0.0

Document for:		Discussion and decision

# Introduction

This document provides an overview of list of issues resulting from the review of the PDU specification and related procedure text and field descriptions.

For some issues the proposed solution is indicated as well as the company & Tdoc introducing this in the standard. For some of the issues this document includes further considerations. The following companies volunteered for the review.

“ID” identifies the company, and consists of “X” (<letter>), as “E” for Ericsson.

| ID | Companies | Contact persons | Email | Last iss |
| --- | --- | --- | --- | --- |
| E | Ericsson | Håkan Palm Riikka Susitaival Henning Wiemann | hakan.l.palm@ericsson.com riikka.susitaival@ericsson.com henning.wiemann@ericsson.com |  |
| S | Samsung | Himke van der Velde | himke.vandervelde@samsung.com |  |
| N | Nokia | Tero Henttonen Amaanat Ali | tero.henttonen@nokia.com amaanat.ali@nokia.com |  |
| Z | ZTE | Eswar LiuJing HuangHe Sergio | eswar.vutukuri@zte.com.cn liu.jing30@zte.com.cn huang.he4@zte.com.cn Sergio.Parolari@zte.com.cn |  |
| H | Huawei | Nathan Tenny David Lecompte Brian Martin | nathan.tenny@huawei.com david.lecompte@huawei.com brian.alexander.martin@huawei.com |  |
| I | Intel | Guo, Yi Jeong, Kyeongin Palat, Sudeep | yi.guo@intel.com kyeongin.jeong@intel.com K sudeep.k.palat@intel.com |  |
| V | Vivo | Kimba Dit AdamouChen, Li Wu, Yumin | boubacar kimba@vivo.com chenli5g@vivo.com wuyumin@vivo.com |  |
| D | DOCOMO | Hideaki Takahashi Wuri Hapsari Toru Uchino Kenji Kai | hideaki.takahashi.vx@nttdocomo.com wuri@nttdocomo.com tooru.uchino.fv@nttdocomo.com kenji.kai.gx@nttdocomo.com |  |
| C | CATT | Chandrika Worrall Jing Liang | chandrika@catt.cn liangjing@catt.cn |  |
| O | OPPO | Lei Yixue | leiyixue@oppo.com |  |
| Q | QUALCOMM | Keiichi Kubota Umesh Phuyal Masato Kitazoe | keiichik@qti.qualcomm.com uphuyal@qti.qualcomm.com mkitazoe@qti.qualcomm.com |  |
| L | LGE | Bokyung Jaewook Oanyong | bokyung.byun@lge.com jaewook.lee@lge.com aidoy.lee@lge.com |  |
| M | Mediatek | Alex Hsu YuanYuan Zhang Li-Chuan Tseng Johan Johansson | alex.hsu@mediatek.com YuanY.Zhang@mediatek.com Li-Chuan.Tseng@mediatek.com Chun-Fan.Tsai@mediatek.com johan.johansson@mediatek.com |  |

# Instructions for RIL and CR storage

RIL and Editorial CR is stored in ftp://ftp.3gpp.org/Email_Discussions/RAN2/.

Companies are requested to provide their review comments and change proposals directly in the stored documents (see Classification below). Companies are encouraged to continuously introduce their comments/changes in the RILs/CR, e.g. when one area have been reviewed (and not provide all comments together on the last day).

When storing the documents after providing updates, companies should add their Company ID (one letter, see section 1) to the file name.

Companies are encouraged to try to resolve collisions. Also the Rapporteur will take an active role in this (and storage/merging problems can be discussed via email). In future reviews, we can potentially use more sophisticated tools.

# Instructions for RIL

Issue Number (I-No)

All issues should be numbered in a format Xyyy where

- X is the unique ID (<letter>) assigned to each company, see the table in clause 1.

- yyy is a running number starting from 001, i.e. 001, 002, …. 999.

- Ex: “E103”.

To avoid duplicated I-No numbers, companies may use the table very last in this document.

Description

Describe the issue in a few words.

Classification (Class):

| Class 1 | Straigthforward clarification/correction will not be included in RIL but company will directly include in “Minor corrections CR”. This can include small things like addition of need codes (as long as relatively straightforward) |
| --- | --- |
| Class 2 | Small issue i.e. solution requires some discussion but possible to concluded as part of ASN.1 review. Within column Details in the RIL, the company raising the issue is invited to suggest a proposed way forward, that other companies review and if not agreeable may suggest alternatives for. |
| Class 3 | More significant issue, i.e. requiring more extensive analysis by a contribution. Class 3 issues are within the scope of the ASN.1 review (i.e. does not concern more functional aspects). Companies are requested to volunteer for drafting a contribution (CR). A contribution may address multiple issues, but these should be clearly marked. |
| Class 4 | Issue of type 4 are like type 3, with the exception that the issue is not only adressing ASN.1 aspects but also more functional aspects. Companies are still invited to draft a contribution, but this would be treated in the agenda item covering the concerned related functionality. |

Companies are requested to provide contribution details, to have an overview of the status (in particular regarding which issues are not covered).

Details (proposed solution/ discussion)

Mainly relevant for issues of class 2, the cell is intended to discuss/ agree the proposed way forward. The company raising the issue is invited to suggest a proposed way forward, that other companies review and if not agreeable may suggest alternatives for.

Companies are encouraged to descripe solutions in the same manner as they correct issues in CRs, i.e. propose changes that are shown in the same manner using change marks.

It is therefore suggested to use “simulated change marks” for the issue reporting, i.e.

- Added parts are marked with underlined red coloured text, e.g. new text .

- Deleted parts are marked with strikethough red coloured text, e.g. delated text.

- If there is a need to high-light something by marking text with a colour, e.g. to high-light small changes, it is recommended that yellow colour is used, e.g. spelling error.

- Reason for these “simulated change marks” is to alow for more easy moving/copy/paste without loosing the changes.

Companies are encouraged to comment issues introduced by other companies, both on agreeing or objecting. These comments shall be tagged with the company name for easy search. E.g. “Ericsson: We agree”.

Status/Ref (to be filled in by the Rapporteur)

Status of the issue, in particular:

Class 2: indicate FFS if no (confirmed) way forward yet

Class 3: indicate company planning to bring a contribution

Class 4: same as 3, but also indicate agenda item

(coding/coloring TBD)

# Conclusion & recommendation

This paper includes a of list of issues resulting from the review of [1]. RAN2 is requested to endorse the status including the solutions proposed.

# References

[1] 	TS 38.331 RRC specification

# Review issue list (Annex)

#### Foreword

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 1	Scope

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 2	References

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 3	Definitions, symbols and abbreviations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 3.1	Definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 3.2	Abbreviations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4	General

#### 4.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.2	Architecture

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.2.1	UE states and state transitions including inter RAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.2.2	Signalling radio bearers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3	Services

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3.1	Services provided to upper layers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.3.2	Services expected from lower layers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 4.4	Functions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5	Procedures

#### 5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.1.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.1.2	General requirements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.2	System information

Only MIB acquisition applicable for EN-DC.

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3	Connection control

#### 5.3.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.2	Paging

Targeted for completion in June 2018.

#### 5.3.3	RRC connection establihshment

Targeted for completion in June 2018.

#### 5.3.4	Initial security activation

Targeted for completion in June 2018.

#### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.2	Initiation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.4	Secondary cell group release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5	Cell Group configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H001 | The reconfigurationWithSync is included in the spCellConfig. The description on reconfigurationWithSync can be merged with spCellConfig. | 2 | 1> if the received CellGroupConfig contains the spCellConfig with reconfigurationWithSync: 2> if the spCellConfig contains the reconfigurationWithSync: 23> perform Reconfiguration with sync according to 5.3.5.5.2; 23> resume all suspended radio bearers and resume SCG transmission for all radio bearers, if suspended; 2> configure the SpCell as specified in 5.3.5.5.7; 1> if the CellGroupConfig contains the rlc-BearerToReleaseList: 2> perform RLC bearer release as specified in 5.3.5.5.3; 1> for each element in RLC-BeaererToAddModList: 2> configure the RLC bearer as specified in 5.3.5.5.4; 1> if the CellGroupConfig contains the mac-CellGroupConfig: 2> configure the MAC entity of this cell group as specified in 5.3.5.5.5; 1> if the CellGroupConfig contains the rlf-TimersAndConstants 2> configure the RLF timers for this cell group as specified in 5.3.5.5.6; 1> if the CellGroupConfig contains the sCellToReleaseList: 2> for each entry in the sCellToReleaseList: 3> release the SCell as specified in 5.3.5.5.8; 1> if the CellGroupConfig contains the spCellConfig: 2> configure the SpCell as specified in 5.3.5.5.7; |  |

#### 5.3.5.5.2	Reconfiguration with synch

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H002 | Cannot find carrierFreq and targetPhysCellId in ASN.1. The frequencyInfoDL and physCellId in ServingCellConfigCommon seems can be used here. | 2 | 1> if the carrierFreqfrequencyInfoDL is included: 2> consider the target SpCell to be one on the frequency indicated by the carrierFreqfrequencyInfoDL with a physical cell identity indicated by the targetPphysCellId; 1> else: 2> consider the target SpCell to be one on the frequency of the source SpCell with a physical cell identity indicated by the targetPphysCellId; |  |
| H003 | 1> perform the measurement related actions as specified in 5.5.6.1; This behavior is not appliable to EN-DC, and there is no 5.5.6.1 subsection. | 2 | Add Editor’s Note: In EN-DC, the following behavior is not needed. |  |

#### 5.3.5.5.3	Logical Channel release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H004 | One logial channel ID corresponds to one RLC entity. | 2 | 1> for each LogicalChannelIdentity value that is to be released as the result of full configuration option according to 5.3.5.7 or as the result of an SCG release according to 5.3.5.4: 2> release the RLC entity or entities (includes discarding all pending RLC PDUs and RLC SDUs); 2> release the DTCH logical channel. |  |

#### 5.3.5.5.4	Logical Channel addition/modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H005 | One logial channel ID corresponds to one RLC entity. | 2 | 1> for each LogicalChannelIdentity value that is to be released as the result of full configuration option according to 5.3.5.7 or as the result of an SCG release according to 5.3.5.4: 2> release the RLC entity or entities (includes discarding all pending RLC PDUs and RLC SDUs); 2> release the DTCH logical channel. |  |
| H006 | The IE name used in the procedure is not consistent | 1 | 1> else (a logical channel with the given IDlogicalChannelIdentity was not configured before): 2> if the logical channel IDlogicalChannelIdentity corresponds to an SRB (i.e. IDlogicalChannelIdentity is less than or equal to 3) and rlc-Config is not included: 3> establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB; |  |
| H007 | The MAC entity configuration according to mac-LogicalChannelConfig is redundant | 2 | 2> if the logical channel IDlogicalChannelIdentity corresponds to an SRB (i.e. IDlogicalChannelIdentity less than or equal to 3) and if mac-LogicalChannelConfig is not included: 3> configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB; 2> else: 3> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig; 2> configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig; |  |

#### 5.3.5.5.5	MAC entity configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.6	RLF Timers & Constants configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.7	SpCell Configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.8	SCell Release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.5.9	SCell Addition/Modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6	Radio Bearer configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.2	SRB release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.3	SRB addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H008 | According to the following RAN2 agreement: “For handling 2 (i.e. no PDCP re-establishment) of SRBs, PDCP should discard all stored SDUs and PDUs”, RRC should indicate PDCP to discard all stored SDUs and PDUs when there is no PDCP re-establishment. | 2 | 1> for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration: 2> if reestablishPDCP is set: 3> configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the KeNB/S-KgNB as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure; 3> configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the KeNB/S-KgNB as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure; 3> re-establish the PDCP entity of this SRB as specified in 38.323; 3> resume the SRB, if suspended; 2> else: 3> configure the PDCP entity to discard all the stored PDCP SDUs and PDUs. 2> if the pdcp-Config is included: 3> reconfigure the PDCP entity in accordance with the received pdcp-Config. |  |

#### 5.3.5.6.4	DRB release

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.6.5	DRB addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.7	Full configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.8	Security key update

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9	Reconfiguration failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.1	Integrity check failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.2	Inability to comply with RRCReconfiguration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.5.9.3	T304 expiry (Reconfiguration with synch Failure)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.6	Counter check

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.7	RRC connection re-establishment

Targeted for completion in June 2018.

#### 5.3.8	RRC connection release

Targeted for completion in June 2018.

#### 5.3.9	RRC connection release requested by upper layers

Targeted for completion in June 2018.

#### 5.3.10	Radio resource configuration

Targeted for completion in June 2018.

#### 5.3.11	Radio link failure related actions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### i5.3.11.1	Detection of physical layer problems in RRC_CONNECTED

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.11.2	Recovery of physical layer problems

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.11.3	Detection of radio link failure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.3.12	UE actions upon leaving RRC_CONNECTED

Targeted for completion in June 2018.

#### 5.3.13	UE actions upon PUCCH/SRS release request

Targeted for completion in June 2018.

#### 5.4	Inter-RAT mobility

Targeted for completion in June 2018.

#### 5.5	Measurements

#### 5.5.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2	Measurement configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.2	Measurement identity removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.3	Measurement identity addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.4	Measurement object removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.5	Measurement object addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.6	Reporting configuration removal

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.7	Reporting configuration addition/ modification

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.8	Quantity configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.9	Measurement gap configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.2.10	Reference signal measurement timing configuration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3	Performing measurements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.2	Layer 3 filtering

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.3.3	Derivation of measurement results

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4	Measurement report triggering

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than PCell/ PSCell)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.6	Event A5 (PCell/ PSCell becomes worse than threshold1 and neighbour becomes better than threshold2)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.5	Measurement reporting

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.5.5.2	Reporting of beam measurement information

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H009 | Since RAN1 does not use the term beam, we prefer to use the SS/PBCH block indexes or CSI-RS index, instead of beam index. |  | For beam measurement information to be included in a measurement report the UE shall: 1> set rsIndexResults to include up to maxNrofRsIndexesToReport beam indexesSS/PBCH block indexes or CSI-RS indexes in order of decreasing quantity as follows: 2> if the measurement information to be included is based on SS/PBCH block: 3> include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block quantity and the remaining beams whose quantity is above absThreshSS-BlocksConsolidation defined in the VarMeasConfig for the corresponding measObject; 3> if onlyReportBeamIds is not configured, include the SS/PBCH based measurement results associated to each beam index SS/PBCH block index; 2> else if the beam measurement information to be included is based on CSI-RS: 3> include within resultsCSI-RSIndexes the index associated to the best beam for that CSI-RS quantity and the remaining beams whose quantity is above absThreshCSI-RS-Consolidation defined in the VarMeasConfig for the corresponding measObject; 3> if onlyReportBeamIds is not configured, include the CSI-RS based measurement results associated to each beam indexCSI-RS index; |  |

#### 5.6	UE capabilities

#### 5.6.1	UE capability transfer

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7	Other

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.1	DL information transfer

Targeted for completion in June 2018.

#### 5.7.2	UL information transfer

Targeted for completion in June 2018.

#### 5.7.3	SCG failure information

#### 5.7.3.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.3.2	Initiation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.3.3	Failure type determination

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 5.7.3.4	Setting the contents of FailureReportSCGtoOtherRAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6	Protocol data units, formats and parameters (ASN.1)

#### 6.1	General

#### 6.1.1	Introduction

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.1.2	Need codes for optional downlink fields

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.2	RRC messages

#### 6.2.1	General message structure

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	NR-RRC-Definitions

#### –	BCCH-BCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	DL-DCCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	UL-DCCH-Message

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.2.2	Message definitions

#### –	MIB

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H010 | halfFrameIndex is missing | 2 | pdcchConfigSIB1 INTEGER(0..255), halfFrameIndex INTEGER (0..1), |  |
| H011 | ssb-IndexExplicit is missing | 2 | pdcchConfigSIB1 INTEGER(0..255), halfFrameIndex INTEGER (0..1), ssb-IndexExplicit INTEGER (0..7), |  |

#### –	MeasurementReport

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RRCReconfiguration

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RRCReconfigurationComplete

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H012 | Based on current discussion, we don’t see any other parameters are needed in this message for now. Thus this message can be completed. | 2 | RRCReconfigurationComplete-IEs ::= SEQUENCE { -- FFS lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE{} OPTIONAL } |  |

#### –	SIB1

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H013 | Keep SIB1? | 2 | Need to decide if we keep SIB1 (and add BCCH-DL-SCH-Message class) or delete it. Our preference would be to delete. |  |
| H014 | ssb-PeriodicityServingCell duplication | 2 | Introduce a common type for the two instances of ssb-PeriodicityServingCell (here and in ServingCellConfigCommon). ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2 }SSB-PeriodicityServingCell, […] SSB-PeriodicityServingCell ::= ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } […] [in ServingCellConfigCommon] ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2 }SSB-PeriodicityServingCell, |  |
| H015 | pucch-ConfigCommon is not needed | 2 | PUCCH-ConfigCommon is already included in UplinkConfigCommon, it doesn’t need to appear separately here. pdcch-ConfigCommon PDCCH-ConfigCommon OPTIONAL, pucch-ConfigCommon PUCCH-ConfigCommon OPTIONAL |  |
| H016 | tdd-UL-DL-ConfigurationSet2 is needed | 2 | Per RAN1 agreements, our understanding is tdd-UL-DL-ConfigurationSet2 should be signalled along with tdd-UL-DL-ConfigurationSet (same as in ServingCellConfigCommon). tdd-UL-DL-configuration TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD tdd-UL-DL-ConfigurationSet2 TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD |  |

#### 6.3	RRC information elements

#### –	SetupRelease Information Element

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.3.1	System information blocks

#### 6.3.2	Radio resource control information elements

#### –	Alpha

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	DRB-Identity

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	BandwidthPart-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H017 | Configuration of SUL BWPs is missing | 2 | Add: supplementary-UL-BWP-ToReleaseList SEQUENCE (SIZE (1..maxNrofBandwidthParts)) OF BandwidthPartId OPTIONAL, -- CondSULconfigured supplementary-UL-BWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBandwidthParts)) OF UplinkBandwidthPart OPTIONAL, -- CondSULconfigured |  |
| H018 | Indication of first active UL should indicate whether it is UL BWP or SUL BWP | 2 | Add: firstActiveUplinkCarrier ENUMERATED { uplink, supplementaryUplink }, |  |
| H019 | Up to 4 bandwidth part pairs (DL + UL ID) can be configured for TDD. | 2 | Define: BandwidthPartPair ::= SEQUENCE { bandwidthPartPairId BandwidthPartPairId, downlinkBandwidthPart BandwidthPartId, uplinkBandwidthPart BandwidthPartId } Add: bandwidthPartPairsToReleaseList SEQUENCE (SIZE (1..maxNrofBandwidthPartPairs)) OF BandwidthPartPairID OPTIONAL, -- Cond TDD bandwidthPartPairsToAddModList SEQUENCE (SIZE (1..maxNrofBandwidthPartParis)) OF BandwidthPartPair OPTIONAL, -- Cond TDD |  |
| H020 | In BandwidthPart, subcarrierSpacing can take values n0 to n4 but in RAN1 sheet it is up to n5 | 2 | Add n5: subcarrierSpacing ENUMERATED {n0, n1, n2, n3, n4, n5} OPTIONAL, |  |
| H021 | For UplinkBandwidthPart , "common" and "dedicated" resources may not include PUCCH or PUSCH depending on the configuration for SUL case. | 2 | Use Setup Release and OPTIONAL: pusch-ConfigCommon SetupRelease {PUSCH-ConfigCommon} OPTIONAL, pusch-Config SetupRelease {PUSCH-Config} OPTIONAL, -- FFS: Is the PUSCH also BWP-specific?? pucch-ConfigCommon SetupRelease {PUCCH-ConfigCommon}, pucch-Config SetupRelease {PUCCH-Config} OPTIONAL, |  |
| H022 | For UplinkBandwidthPart, there may not always be RACH resources for all UL BWPs | 2 | Add Setup Release and OPTIONAL: rach-ConfigCommon SetupRelease {RACH-ConfigCommon} OPTIONAL, |  |
| H023 | According to RAN2 agreements: “For reconfiguration with synchronisation or for SCell addition, one DL/UL BWP indicated in the RRC message is the active BWP until it is changed by DCI”, The first active DL/UL BWPs configured in firstActiveDownlinkBwp-Id and firstActiveUplinkBwp-Id should not only be applicable to SCell but also to SpCell. On the other hand, the first active DL/UL BWP shall be configured and shall only be configured when downlinkBandwidthPartsToAddModList/ uplinkBandwidthPartsToAddModList is configured for the first time for the serving cell. | 3 | -- ID of the downlink bandwidth part to be used upon MAC-activation of an SCell. If not provided, the UE uses the default BWP firstActiveDownlinkBwp-Id BandwidthPartId OPTIONAL, -- Cond SCellOnlyFirstDLBWPConfig -- ID of the uplink bandwidth part to be used upon MAC-activation of an SCell. If not provided, the UE uses the FFS: default BWP firstActiveUplinkBwp-Id BandwidthPartId OPTIONAL, -- Cond SCellOnlyFirstULBWPConfig The corresponding explaination of those two conditions should be added. |  |
| H024 | For “ -- FFS: Conditions! What to do when certain fields or the entire bandwidth part is omitted? Assume parameters of the carrier instead? -- Or use the initialBWP derived from SIB1 or ServingCellConfigCommon? Or make it mandatory to provide at least one BWP”, before the initial BWP configuration after initial access, the UE continues to use the initial DL/UL BWPs. | 2 | -- The UE uses the initial DL/UL BWPs derived from MIB/SIB1 or ServingCellConfigCommon before dedicated BWPs have not been configured in BandwidthPart-Config. FFS: Conditions! What to do when certain fields or the entire bandwidth part is omitted? Assume parameters of the carrier instead? -- Or use the initialBWP derived from SIB1 or ServingCellConfigCommon? Or make it mandatory to provide at least one BWP. |  |
| H025 | For “-- FFS: May the NW change the default BWP with a regular RRC reconfiguration or only with Reconfiguration with sync?”, the default DL BWP can be changed with either a regular RRC reconfiguration or with Reconfiguration with sync. | 2 | -- FFS: May the NW change the default BWP with a regular RRC reconfiguration or only with Reconfiguration with sync? |  |
| H026 | For “-- FFS: Whether to add a default uplink BWP”, the default uplink BWP is not needed. | 2 | -- FFS: Whether to add a default uplink BWP |  |
| H027 | For “-- The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.321, section FFS_Section)”, the section is 5.15. | 2 | -- The duration in ms after which the UE falls back to the default Bandwidth Part. (see 38.321, section FFS_Section5.15) |  |
| H028 | For “ -- FFS: For TDD the UE switches also the paired uplink BWP to the one with the defaultDownlinkBwp-Id”, for TDD the UE should switch to the DL/UL BWP pair. The details have already been agreed by RAN1 and the procedure is specified in TS38.321. | 2 | -- FFS: For TDD the UE switches also the paired uplink BWP to the one with the defaultDownlinkBwp-Id. |  |
| H029 | For “ -- FFS: For FDD the UE switches the uplink BWP????? If only DL is affected, consider moving it into the DownlinkBandwidthPart”, for FDD, the uplink BWP is not switched based on this timer. A relevant issue is whether the configuration of this timer needs to be BWP specfic. Although the timer can be started per BWP, it may not be necessary to configure different values for different BWPs. Therefore, we think it is fine to leave this configuration as it is. | 2 | -- FFS: For FDD the UE switches the uplink BWP????? If only DL is affected, consider moving it into the DownlinkBandwidthPart |  |
| H030 | For “ -- FFS: RAN2 to discuss/confirm value range. RAN1 just suggested values from 1ms/0.5ms and up to about 50 ms. -- FFS: Rapporteur adopted DRX inactivity timers as baseline”, from the instruction in R1-1721581, seems RAN1 suggesting “Granularity of the timer: 1 ms (subframe) for sub6, 0.5 ms (half-subframe) for mmWave”, so it is not the minimum value but the granularity being 0.5/1ms. But that granularity would make the configuration more bit-consuming. We are fine to follow RAN1 or discuss further. | 3 | Change the values to what RAN1 suggested, i.e. with the granularity of 0.5/1ms, or further discuss. Options could include INTEGER (1..50), INTEGER (1..100) with the real value being half the signalled value, or a more complex structure allowing different granularity in low and high ranges e.g. a CHOICE between low and high with different ranges for each. Could also consider a CHOICE between 1 ms granularity (sub6) and 0.5 ms granularity (>6). -- FFS: RAN2 to discuss/confirm value range. RAN1 just suggested values from 1ms/0.5ms and up to about 50 ms. -- FFS: Rapporteur adopted DRX inactivity timers as baseline. -- When the network releases the timer configuration, the UE stops the timer without swithching to the default (FFS: and uplink?) BWP. bandwidthPartInactivityTimer SetupRelease { ENUMERATED { ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80, spare}} OPTIONAL -- Need M |  |
| H031 | For the bandwidthPartId in IE BandwidthPart, as this IE is also used for initial DL/UL BWP, it should be clear whether the ID is needed for the initial BWPs. From the RAN1’s LS (and also based on inputs from our RAN1 colleagues), the ID is not needed for initial BWPs. | 2 | The ID does not need to be configured for the initial BWPs. -- Parameters used in UplinkBandwidthPart and DownlinkBandwidthPart BandwidthPart ::= SEQUENCE { -- An identifier for this bandwidth part. -- Corresponds to L1 parameter 'UL-BWP-index'. (see 38.211, 38.213, section 12) -- The ID is absent for initial BWP configurations. bandwidthPartId BandwidthPartId OPTIONAL, -- Need S |  |
| H032 | The fields locationAndBandwidth and subcarrierSpacing should be mandatorily configured. | 2 | Remove “Optional” for fields locationAndBandwidth and subcarrierSpacing. -- Parameters used in UplinkBandwidthPart and DownlinkBandwidthPart BandwidthPart ::= SEQUENCE { -- An identifier for this bandwidth part. -- Corresponds to L1 parameter 'UL-BWP-index'. (see 38.211, 38.213, section 12) bandwidthPartId BandwidthPartId, -- Frequency domain location and bandwidth of this bandwidth part defined commonly in a table (FFS_Section). It is represents the -- distance in number of PRBs in relation to the lowest usable subcarrier defined by the scsSpecificCarrier -- with the same subcarrier spacing as this BWP. -- Corresponds to L1 parameter 'DL-BWP-loc'. (see 38.211, section FFS_Section). -- In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bandwidthPartId) must have the same location (see 38.211, section REF) -- FFS_Value: RAN1 seems to discuss the final range. locationAndBandwidth INTEGER (1..65536) OPTIONAL, -- Subcarrier spacing to be used in this BWP. It is applied to at least PDCCH, PDSCH and corresponding DMRS. -- The values provided here are converted into a subcarrier spacing as indicated in 38.211, Table 4.1-2. -- FFS: Isn't the SCS known from the SCS configured in the scsSpecificCarrier? subcarrierSpacing ENUMERATED {n0, n1, n2, n3, n4} OPTIONAL, -- Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. -- Normal CP is supported for all numerologies and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. -- (see 38.211, section 4.2.2) cyclicPrefix ENUMERATED { extended } OPTIONAL } |  |
| H033 | SRS configuration is missing, propose to add it in UplinkBandwidthPart | 2 | Add UplinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, -- Frequency location of the uplink "direct current" frequency. OPTIONAL, -- Corresponds to L1 parameter 'UL-BWP-DC'. (see 38.211, section FFS_Section) directCurrentLocation INTEGER (0..3299) OPTIONAL, -- Cond UplinkOnly -- FFS_CHECK: Several (UE specific) BWPs may be configured with RACH resources. Hence, they must be provided with -- the information in RACH-ConfigCommon... even though it is in this case strictly speaking not a cell-specific parameter. -- OK to keep or re-structure the RACH config? rach-ConfigCommon SetupRelease {RACH-ConfigCommon} OPTIONAL, pusch-ConfigCommon SetupRelease {PUSCH-ConfigCommon} OPTIONAL, pusch-Config SetupRelease {PUSCH-Config} OPTIONAL, -- FFS: Is the PUSCH also BWP-specific?? pucch-ConfigCommon SetupRelease {PUCCH-ConfigCommon}, pucch-Config SetupRelease {PUCCH-Config} OPTIONAL, srs-Config SetupRelease {SRS-Config} OPTIONAL } |  |
| H034 | suo-duo-mode is missing | 3 | RAN1 parameter list shows this as CHOICE{dual,case1,case2}. A clear indication from the network is needed on the operation modes, and the UE capabilities need to be correctly captured. A separate contribution will be provided |  |

#### –	CellGroupConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H035 | The value range of CellGroupId should be (0..maxSCellGroups), since cellGroupId 0 is used for the MCG. | 2 | CellGroupId ::= INTEGER (10.. maxSCellGroups) |  |
| H036 | LCH-Config inlcudes rlc configuration and mac configuration. This IE actually is used for RLC bearer configuraion. So the IE name can be changed accordingly. | 2 | rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF LCH-ConfigRLC-BearerConfig LCH-ConfigRLC-BearerConfig ::= SEQUENCE { |  |
| H037 | The condition for sCellConfigCommon and sCellConfigDedicated looks confusion. ‘SCellAddMod’ looks like the IE is mandatory when scell is added or modified. | 2 | SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAddOnly sCellConfigDedicated ServingCellConfigDedicated OPTIONAL, -- Cond SCellAddMod } And the conditional presence also needs to be updated accordingly. |  |
| H038 | Allocation of CFRA resources on SUL for reconfiguration with sync is missing | 2 | Add within a CHOICE SpCellConfig ::= SEQUENCE { -- Parameters for the synchronous reconfiguration to the target SpCell: reconfigurationWithSync SEQUENCE { spCellConfigCommon ServingCellConfigCommon, newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000-v1310}, rach-ConfigDedicated CHOICE { rach-ConfigDedicatedUL RACH-ConfigDedicated, rach-ConfigDedicatedSUL RACH-ConfigDedicated } OPTIONAL -- Need M } OPTIONAL, -- Cond SpCellChange spCellConfigDedicated ServingCellConfigDedicated OPTIONAL -- Need M } |  |

#### –	CellIndexList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ControlResourceIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CrossCarrierSchedulingConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CSI-MeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H039 | Split into BWPs? | 3 | Each CSI-ReportConfig and each CSI-ResourceConfig is associated with a single BWP. Could a measID link two such itesm associated with different BWP? Probably no, it is associated with a single BWP. What remains is the list of trigger states. Can a trigger state be associated with CSI-ReportConfig associated to different BWPs? Check with RAN1. |  |
| H040 | In CSI-MeasConfig, reportTrigger contains a list of parameters for one trigger state but the description copied from RAN1 specifications says there is a list of trigger states | 2 | According to RAN1 conculsion, there can be up to 128 report trigger states (which can also be seen in LS R1-1721661). Proposal is to include a list in CSI-MeasConfig: reportTriggerList SEQUENCE (SIZE (1.. maxNrofTriggerStates)) OF ReportTrigger OPTIONAL With ReportTrigger ::= SEQUENCE { aperiodic SEQUENCE { -- The CSI-ReportCongig (their IDs) associated with this reportTrigger associatedReportConfigs SEQUENCE (SIZE (1..maxNrofReportConfigIdsPerTrigger)) OF CSI-ReportConfigId, -- bitmap with the bitwidth Nbit =number of resource sets (max number Nbit = 64) in a linked resource setting per report trigger tate. -- Number of one(s) in the bitmap None = 1 for CSI acquisition (FFS 1<= None <= 64 for beam management). -- FFS: To enforce the number of linked resources, the linking information should instead be in the report that uses the resource -- Corresponds to L1 parameter 'ResourceSetBitmap' (see 38.214, section FFS_Section) -- FFS_FIXME: The following list assumes that all NZP- and IM resource sets use a common ID space. But that is not ensured -- due to having separate lists of sets. associatedResourceSets SEQUENCE (SIZE (1..64)) OF CSI-ResourceSetId OPTIONAL, -- For a trigger state within aperiodicReportTrigger that triggers a ap-CSI-RS resource set, contains a list of -- references to TCI-RS-SetConfig's in TCI-States for providing the QCL source and QCL type for each ap-CSI-RS -- resource within the triggered set of ap-CSI-RS resources. The length of the list is equal to the number of -- aperiodic CSI-RS resources in the set (CSI-RS-ResourceSet). For a target aperiodic CSI-RS assoicated with each -- triggering state, contains a reference to one TCI-RS-Set in TCI-States for providing the QCL source and QCL type. -- Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section 5.2.1.5.1) qcl-Info-aPeriodicReportingTrigger SEQUENCE (SIZE(1..ffsValue)) OF TCI-RS-SetId OPTIONAL }, semiPersistentOnPUSCH SEQUENCE { associatedReportConfig CSI-ReportConfigId } } (Note that there are corrections on the contents discussed below) And define; maxNrofTriggerStates INTEGER ::= 128 -- Maximum number of report trigger states |  |
| H041 | In CSI-MeasConfig, in reportTrigger, there is associatedResourceSets which is list of CSI resource set IDs does not match with RAN1 sheet/specifications. | 2 | The definition of the parameter is is: For a UE configured with the higher layer parameter AperiodicReportTrigger, if a resource setting linked to a ReportConfig has multiple aperiodic resource sets and only a subset of the aperiodic resource sets is associated with the trigger state, a higher layer configured bitmap ResourceSetBitmap is configured per trigger state per resource setting to select the CSI-IM/NZP CSI-RS resource set(s) from the resource setting. So the parameter is a bitmap and, for each trigger state, there is one bitmap per resource config in a link (measID) to an associated ReportConfig. According to 5.2.1.4.1: Each trigger state configured using the higher layer parameter ReportTrigger is associated one or multiple ReportConfig where each ReportConfig is linked to periodic, or semi-persistent, or aperiodic resource setting(s): - When one resource setting is configured, the resource setting is for channel measurement for L1-RSRP computation. - When two resource settings are configured, the first one resource setting is for channel measurement and the second one is for interference measurement performed on CSI-IM or on non-zero power CSI-RS. - When three resource settings are configured, the first one resource setting is for channel measurement, the second one is for CSI-IM based interference measurement and the third one is for non-zero power CSI-RS based interference measurement. So there can be 1, 2 or 3 link to a CSI-ReportConfig. We can reflect this with the following changes: ReportTrigger ::= SEQUENCE { aperiodic SEQUENCE { associatedReportConfigs SEQUENCE (SIZE (1..maxNrofReportConfigIdsPerTrigger)) OF SEQUENCE { reportConfigId CSI-ReportConfigId, resourceSetBitmaps SEQUENCE (SIZE (1..3)) OF SEQUENCE { resourceConfigId CSI-ResourceConfigId, resourceSetBitmap BIT STRING (SIZE (ffs_value)) } associatedResourceSets SEQUENCE (SIZE (1..64)) OF CSI-ResourceSetId OPTIONAL, |  |
| H042 | In CSI-ResourceConfig, it is not clear what "CC-info" is useful for. | 3 | Confirm with RAN1. |  |
| H043 | Before NZP-CSI-RS-ResourceSet, there is a comment "-- Corresponds to L1 parameter 'ResourceSetConfig''NZP-CSI-RS-ResourceSetConfigList' (see 38.214, section 5.2) " but there is no such parameter in 38.214 | 2 | Remove the comment -- A set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters. -- Corresponds to L1 parameter 'NZP-CSI-RS-ResourceSetConfigList' (see 38.214, section 5.2) NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |
| H044 | In NZP-CSI-RS-ResourceSet, parameter repetition is BOOLEAN but in 38.214 sections 5.2.2.3.1 and 5.1.6.1.2 it is ON or OFF | 2 | Change description in comment to align with RAN1: -- Indicates whether repetition is on/off. Change definition to ENUMERATED {on, off}, -- A set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters. -- Corresponds to L1 parameter 'NZP-CSI-RS-ResourceSetConfigList' (see 38.214, section 5.2) NZP-CSI-RS-ResourceSet ::= SEQUENCE { -- FFS: Where is the CSI-ResourceSetId used? csi-ResourceSetId CSI-ResourceSetId, -- NZP-CSI-RS-Resources assocaited with this NZP-CSI-RS resource set. -- Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2) -- FFS: Better make the csi-rs-Resources a common pool on CSI-MeasConfig level? nzp-csi-rs-Resources SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-Resource, -- Indicates whether repetition is on/off. Repetition on (off), means that The UE can (cannot) assume that -- the network maintains a fixed TX beam over the resources in the set. off means that the UE may not assume that the CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter and with same NrofPorts in every symbol. -- Corresponds to L1 parameter 'ResourceRep' (see 38.214, section FFS_Section) repetition BOOLEANENUMERATED { on, off }, |  |
| H045 | In NZP-CSI-RS-Resource, it is not clear whether aperiodicTriggeringOffset should be there. | 3 | Confirm with RAN1 whether it should be per trigger state, per ReportConfig, per resource set or per NZP CSI resource. Then move it to right place (either in the trigger state list, or in the ReportConfig list within trigger state list, or in resource set or in NZP CSI resource). |  |
| H046 | In NZP-CSI-RS-Resource, parameter CSI-RS-Resource map from 38.211 section 7.4.1.5.3 is incorrectly implemented. | 2 | To determine allocation in a slot, it is necessary to know the row number of table 7.4.1.5.2-1 which is missing. Also, the parameter "starting position of a CSI-RS in a slot" proposed in ASN.1 does not exist, there are one or multiple positions determined according to 5th column of the table (parameter k with a bar) which are calculated based on the k0, k1, etc, derived from the contents of the bitmap. Besides, there is no need to signal separately parameters such as nrOfPorts and cdmType which are fully determined by the row. With respect to density, it has either 1 value only per row, or 2 values only. Proposal: - have the row number as parameter, make it a CHOICE including the bitmap with corresponding length - for each row number, include nrofPorts, cdmType and density with the only allowed values See changes below -- A CSI-RS (reference signal) resource which the UE may be configured to measure on (see 38.214, section 5.2.21.3.1) NZP-CSI-RS-Resource ::= SEQUENCE { nzp-csi-rs-ResourceId NZP-CSI-RS-ResourceId, -- Number of ports (see 38.214, section 5.2.2.3.1) nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32}, -- OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource -- Corresponds to L1 parameter 'CSI-RS-ResourceMapping' (see 38.214, section 5.2.2.3.1) resourceMapping SEQUENCE { -- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1. -- The number of bits that may be set to one depend on the chosen row in that table. frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, -- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. -- Value 2 is supported only when DL-DMRS-typeA-pos equals 3. firstOFDMSymbolInTimeDomain INTEGER (0..13) }, -- CDM type (see 38.214, section 5.2.2.3.1) cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4}, -- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density' (see 38.214, section 5.2.2.3.1) -- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, -- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, -- value 1 (one) is allowed for X=4, 8, 12. -- For density = 1/2, includes 1 bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL }, -- Frequency domain allocation within a physical resource block in accordance with 38.211 section 7.4.1.5.3, table 7.4.1.5.2-1. -- The number of bits that may be set to one depend on the chosen row in that table. csi-RS-LocationWithinSlot CHOICE { row1 SEQUENCE { -- Number of ports (see 38.214, section 5.2.2.3.1) nrofPorts ENUMERATED {p1}, -- CDM type (see 38.211 section 7.4.1.5.3) cdm-Type ENUMERATED {noCDM}, -- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density' (see 38.214, section 5.2.2.3.1) -- For density = 1/2, includes 1 bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS density ENUMERATED {three}, csi-RS-ResourceMapping BIT STRING (SIZE (4)) }, row2 SEQUENCE { nrofPorts ENUMERATED {p1}, cdm-Type ENUMERATED {noCDM}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (12)) }, row3 SEQUENCE { nrofPorts ENUMERATED {p2}, cdm-Type ENUMERATED {fd-CDM2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row4 SEQUENCE { nrofPorts ENUMERATED {p4}, cdm-Type ENUMERATED {fd-CDM2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (3)) } row5 SEQUENCE { nrofPorts ENUMERATED {p8}, cdm-Type ENUMERATED {fd-CDM2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row7 SEQUENCE { nrofPorts ENUMERATED {p8}, cdm-Type ENUMERATED {fd-CDM2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row8 SEQUENCE { nrofPorts ENUMERATED {p8}, cdm-Type ENUMERATED {fd-CDM2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row9 SEQUENCE { nrofPorts ENUMERATED {p8}, cdm-Type ENUMERATED {cdm4-FD2-TD2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row10 SEQUENCE { nrofPorts ENUMERATED {p12}, cdm-Type ENUMERATED {fd-CDM2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row11 SEQUENCE { nrofPorts ENUMERATED {p12}, cdm-Type ENUMERATED {cdm4-FD2-TD2}, density ENUMERATED {one}, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row12 SEQUENCE { nrofPorts ENUMERATED {p16}, cdm-Type ENUMERATED {fd-CDM2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row13 SEQUENCE { nrofPorts ENUMERATED {p16}, cdm-Type ENUMERATED {cdm4-FD2-TD2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row14 SEQUENCE { nrofPorts ENUMERATED {p24}, cdm-Type ENUMERATED {fd-CDM2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row15 SEQUENCE { nrofPorts ENUMERATED {p24}, cdm-Type ENUMERATED {cdm4-FD2-TD2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row16 SEQUENCE { nrofPorts ENUMERATED {p24}, cdm-Type ENUMERATED {cdm8-FD2-TD4}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row17 SEQUENCE { nrofPorts ENUMERATED {p32}, cdm-Type ENUMERATED {fd-CDM2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row18 SEQUENCE { nrofPorts ENUMERATED {p32}, cdm-Type ENUMERATED {cdm4-FD2-TD2}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) }, row19 SEQUENCE { nrofPorts ENUMERATED {p32}, cdm-Type ENUMERATED {cdm8-FD2-TD4}, density CHOICE { one NULL, dot5 ENUMERATED {evenPRBs, oddPRBs} }, csi-RS-ResourceMapping BIT STRING (SIZE (6)) } } -- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1) freqBand SEQUENCE { -- PRB where this NZP-CSI-RS-Resource starts in relation to PRB 0 of the associated BWP. Only multiples of 4 are allowed (0, 4, ...) startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), -- Number of PRBs across which this NZP-CSI-RS-Resource spans. Only multiples of 4 are allowed. The smallest configurable -- number is the minimum of 24 and the width of the associated BWP. nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocks) }, -- Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, section 5.2.2.3.1) powerControlOffset INTEGER(-8..15), -- Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section FFS_Section) powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Scrambling ID (see 38.214, section 5.2.2.3.1) scramblingID ScramblingId, -- Periodicity and slot offset in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1) periodicityAndOffset CHOICE { sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) }, -- Indicates whether or not the antenna ports of NZP CSI-RS resources in the CSI-RS resource set is same -- Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1) trs-Info ENUMERATED {true} OPTIONAL } |  |
| H047 | In NZP-CSI-RS-Resource, startingRB and nrofRBs are with granularity one RB while RAN1 excel sheet says it should be 4. | 2 | Change the value from 0/24 to 68 (68 * 4 = 272) and indicate that the number of RBs is 4 times the signalled value. -- A CSI-RS (reference signal) resource which the UE may be configured to measure on (see 38.214, section 5.2.1.3.1) NZP-CSI-RS-Resource ::= SEQUENCE { nzp-csi-rs-ResourceId NZP-CSI-RS-ResourceId, -- Number of ports (see 38.214, section 5.2.2.3.1) nrofPorts ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32}, -- OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource -- Corresponds to L1 parameter 'CSI-RS-ResourceMapping' (see 38.214, section 5.2.2.3.1) resourceMapping SEQUENCE { -- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1. -- The number of bits that may be set to one depend on the chosen row in that table. frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, -- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. -- Value 2 is supported only when DL-DMRS-typeA-pos equals 3. firstOFDMSymbolInTimeDomain INTEGER (0..13) }, -- CDM type (see 38.214, section 5.2.2.3.1) cdm-Type ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4}, -- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density' (see 38.214, section 5.2.2.3.1) -- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, -- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, -- value 1 (one) is allowed for X=4, 8, 12. -- For density = 1/2, includes 1 bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL }, -- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1) freqBand SEQUENCE { -- PRB where this NZP-CSI-RS-Resource starts in relation to PRB 0 of the associated BWP. Only multiples of 4 are allowed (0, 4, ...)The actual value is 4*signalled value. startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-168), -- Number of PRBs across which this NZP-CSI-RS-Resource spans. Only multiples of 4 are allowed. The smallest configurable -- number is the minimum of 24 and the width of the associated BWP. -- The actual value is 4*signalled value. nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocks68) }, |  |
| H048 | In NZP-CSI-RS-Resource, periodicityAndOffset contain a period and an offset but the corresponding parameter in RAN1 specification already indicates the encoding from a single number. |  | In 38.211, there is Table 7.4.1.5.2-6: Slots containing CSI-RS. CSI-RS-timeConfig; Slot offset; Periodicity. 0 – 4; 5. 5 – 14; 10. 15 – 34; 20. 35 – 74; 40. 75 – 154; 80. 155 – 314; 160. 315 – 634; 320. 635 – 1274; 640. Inform RAN1 that the table should be removed. |  |
| H049 | In CSI-IM-Resource, should resourceMapping and freqBand really be OPTIONAL? | 2 | Remove OPTIONAL for both CSI-IM-Resource ::= SEQUENCE { csi-IM-ResourceId CSI-IM-ResourceId, -- The resource element pattern for the CSI-IM resource -- Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4) csi-IM-ResourceElementPattern ENUMERATED {pattern2-2, pattern4-1}, -- OFDM symbol and subcarrier occupancy of the CSI-IM resource within a slot -- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4) -- FFS_Values: RAN1 indicated “symbol locations: [0..13] and subcarrier locations: [0..9]” Should this be a bitmap of 9x13? Or two separate? resourceMapping ENUMERATED {ffsTypeAndValue} OPTIONAL, -- Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2) freqBand FFS_Value OPTIONAL} |  |
| H050 | In CSI-IM-Resource, periodicityAndOffset is in excel sheet but not in ASN.1 | 2 | Add periodicityAndOffset CHOICE { sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) } Inform RAN1 because it is in their excel sheet but not in their specifications |  |
| H051 | In CSI-ReportConfig, there is p0alpha (index) which is in RAN1 excel sheet but not in RAN1 specifications. | 2 | This is in the RAN1 spreadsheet but UL power control is specified in 38.213 and there is no indication, neither in 38.213 nor in 38.214 that there is a particular set configured for CSI transmission on PUSCH. Contact RAN1 to check. |  |
| H052 | In CSI-ReportConfig, according to RAN1 specifications, groupBasedBeamReporting is only for the case where the report quantity is CSI/RSRP | 2 | In reportQuantity, change all CHOICE values as SEQUENCE and move nrofBeamsToReport to CSI/RSRP and nrofReportedRS to other CHOICE values. Also, nrofReportedRS need not be OPTIONAL. |  |
| H053 | In CSI-ReportConfig, values of parameter cqi-Table don't match with RAN1 specification | 2 | Use table number (tableX, tableY, etc) corresponding to table numbers in 38.214 section 5.2.2.1 -- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-Table' (see 38.214, section FFS_Section) -- FFS: Whether URLLC2 should be added as one option cqi-Table ENUMERATED {qam64, qam256, urllc1, urllc2table1, table2, spare2, spare1} OPTIONAL, |  |
| H054 | In CSI-ReportConfig, values of parameter cqi-Table is OPTIONAL but isn't the presence fully determined by reportQuantity? | 2 | Confirm with RAN1 and move the parameter to the relevant CHOICEs of report quantity |  |
| H055 | In CSI-ReportConfig, bler-Target is OPTIONAL but it seems the presence is only for 1 of the cqi table option. | 2 | Create a CHOICE for cqi-Table and move bler-Target to the relevant CHOICE (and no need for OPTIONAL). -- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-Table' (see 38.214, section FFS_Section) -- FFS: Whether URLLC2 should be added as one option cqi-Table ENUMERATEDCHOICE { table1 NULL, -- BLER target that the UE shall be assume in its CQI calculation. -- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section FFS_Section) -- FFS_Values (now filled with spares) table2 ENUMERATED {zerodot1, spare3, spare2, spare1}, spare2 NULL, spare1 NULL qam64, qam256, urllc1, urllc2} OPTIONAL, -- Indicates one out of two possible BWP-dependent values for the subband size -- Corresponds to L1 parameter 'SubbandSize' (see 38.214, section FFS_Section) -- FFS_Value: Clarify what value1 and value2 mean. subbandSize ENUMERATED {value1, value2}, -- BLER target that the UE shall be assume in its CQI calculation. -- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section FFS_Section) -- FFS_Values (now filled with spares) bler-Target ENUMERATED {zerodot1, spare3, space2, spare1} OPTIONAL, |  |
| H056 | In CSI-ReportConfig, non-PMI-PortIndication seems to apply to resources, not to ReportConfig. | 3 | Confirm with RAN1 whether it belongs there or should be move to resource configuration. |  |
| H057 | In CodeBookConfig, many parameters are signalled fully separately although they are largely dependent on each other | 2 | This seems to allow 32 combinations of (N1,N2) but according to 36.214 section 5.2.2.2 and subsections: - for typeI-SinglePanel/typeII, there are 13 combinations possible - for typeI-MultiPanel, there are 8 combinations including numberOfPanels - For typeII-PortSelection, there doesn't seem to be such parameters but instead the number of CSI-RS ports (which needs not be indicated for other types as it is 2xN1xN2xNg where Ng is the number of panels for typeI-MultiPanel) which is missing here. - for the parameter singlePanel, the size is uniquely determined by (N1,N2) Besides, parameter singlePanel2Tx, which is put as mandatory for Type 1, is only for the case when N1=N2=1. Proposal: - change subTypes to CHOICE, move all parameters there (especially the subType-specific parameters, see conditions) - change N1 and N2 to a single parameter with allowed combinations, place them in the CHOICE of subType, and make them CHOICE as well - move singlePanel, singlePanel2Tx and codebookSubsetRestrictionType2 to the CHOICE of N1,N2 - inform RAN1 Propose to replace CodebookConfig as follows: -- Codebook configuration for Type-I and Type-II (see 38.214, section 5.2.2.2) CodebookConfig ::= SEQUENCE { -- Number of antenna ports in first dimension -- CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType' -- (see 38.214, section 5.2.2.2) codebookType CHOICE { type1 SEQUENCE { subType CHOICE { typeI-SinglePanel SEQUENCE { nrOfAntennaPorts CHOICE { two SEQUENCE { -- Codebook subset restriction for 2TX codebook -- Corresponds to L1 parameter ' TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.1) twoTX-CodebookSubsetRestriction BIT STRING (SIZE (6)) }, moreThanTwo SEQUENCE { -- Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction -- Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2' n1-n2 CHOICE { two-one SEQUENCE { -- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction ' (see 38.214 section 5.2.2.2.1) codebookSubsetRestriction BIT STRING (SIZE (8)) }, two-two SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (64)), -- i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI -- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214 section 5.2.2.2.1) codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, four-one SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (16)) }, three-two SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (96)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, six-one SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (24)) }, four-two SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (128)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, eight-one SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (32)) }, four-three SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (192)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, six-two SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (192)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, twelve-one SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (48)) }, four-four SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (256)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, eight-two SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (256)), codebookSubsetRestriction-i2 BIT STRING (SIZE (16)) OPTIONAL }, sixteen-one SEQUENCE { codebookSubsetRestriction BIT STRING (SIZE (64)) }, }, -- Restriction for RI for TypeI-SinglePanel-RI-Restriction -- Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section 5.2.2.2.1) ri-Restriction BIT STRING (SIZE (8)) } }, -- CodebookMode as specified in 38.214 section 5.2.2.2.1 codebookMode INTEGER (1..2) }, typeI-MultiPanel SEQUENCE { ng-n1-n2 CHOICE { two-two-one SEQUENCE { -- CodebookMode as specified in 38.214 section 5.2.2.2.2 codebookMode INTEGER (1..2), -- Codebook subset restriction for Type I Multi-panel codebook -- Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section 5.2.2.2.2) codebookSubsetRestriction BIT STRING (SIZE (8)) }, two-four-one SEQUENCE { codebookMode INTEGER (1..2), codebookSubsetRestriction BIT STRING (SIZE (32)) }, four-two-one SEQUENCE { codebookMode INTEGER (1), codebookSubsetRestriction BIT STRING (SIZE (32)) }, two-two-two SEQUENCE { codebookMode INTEGER (1..2), codebookSubsetRestriction BIT STRING (SIZE (128)) }, two-eight-one SEQUENCE { codebookMode INTEGER (1..2), codebookSubsetRestriction BIT STRING (SIZE (64)) }, four-four-one SEQUENCE { codebookMode INTEGER (1), codebookSubsetRestriction BIT STRING (SIZE (64)) }, two-four-two SEQUENCE { codebookMode INTEGER (1..2), codebookSubsetRestriction BIT STRING (SIZE (256)) }, four-two-two SEQUENCE { codebookMode INTEGER (1), codebookSubsetRestriction BIT STRING (SIZE (256)) } }, -- Restriction for RI for TypeI-MultiPanel-RI-Restriction -- Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section 5.2.2.2.2) ri-Restriction BIT STRING (SIZE (4)) } } }, type2 SEQUENCE { subType CHOICE { typeII SEQUENCE { -- Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction -- Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2' -- The CHOICE name indicates the value of n1 and n2, the CHOICE contents is the codebook subset restriction bitmap -- Corresponds to L1 parameter ' TypeII-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.3) -- Number of bits for codebook subset restriction is ceil(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!) n1-n2-codebookSubsetRestriction CHOICE { two-one BIT STRING (SIZE (17)), two-two BIT STRING (SIZE (139)), four-one BIT STRING (SIZE (33)) three-two BIT STRING (SIZE (59)), six-one BIT STRING (SIZE (49)), four-two BIT STRING (SIZE (75)), eight-one BIT STRING (SIZE (65)) four-three BIT STRING (SIZE (107)), six-two BIT STRING (SIZE (107)), twelve-one BIT STRING (SIZE (129)), four-four BIT STRING (SIZE (139)), eight-two BIT STRING (SIZE (139)), sixteen-one BIT STRING (SIZE (129)) }, -- Restriction for RI for TypeII-RI-Restriction -- Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section 5.2.2.2.3) typeII-RI-Restriction BIT STRING (SIZE (2)), }, typeII-PortSelection SEQUENCE { -- The size of the port selection codebook (parameter d) portSelectionSamplingSize ENUMERATED {n1, n2, n3, n4}, -- Restriction for RI for TypeII-PortSelection-RI-Restriction -- Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section 5.2.2.4) typeII-PortSelectionRI-Restriction BIT STRING (SIZE (2)) } }, -- The size of the PSK alphabet, QPSK or 8-PSK phaseAlphabetSize ENUMERATED {n4, n8}, -- If subband amplitude reporting is activated (true) subbandAmplitude BOOLEAN, -- Number of beams, L, used for linear combination numberOfBeams ENUMERATED {two, three, four} } } } } |  |
| H058 | In CodeBookConfig, where in 38.214 is singlePanelCodebookSubsetRestriction-i2 described? | 2 | Check with RAN1 which CHOICE this would belong to with the above change. |  |
| H059 | Where should BeamManagement be? | 3 | In BWP? In CSI? In RLF/RLM? |  |
| H060 | In BeamManagement, should failureDetectionResources be a list of NZP-CSI-RS-Resource? | 2 | Check with RAN1 |  |
| H061 | In BeamManagement, all parameters are OPTIONAL while there seem no option in RAN1 specifications/excel sheet to actually omit them. | 2 | Check with RAN1 |  |
| H062 | In BeamManagmenent, parameter ra-PreambleIndexConfig-BFR is missing | 2 | Add the parameter (0 to 255). BeamManagement ::= SEQUENCE { beamFailureDetection SEQUENCE { -- List of CSI-RS resouces used for beam failure detection -- FFS: How does this relate to the beam indicates in MAC CE? -- Corresponds to L1 parameter 'Beam-Failure-Detection-RS-ResourceConfig' (see 38.213, section 6) failureDetectionResources FFS_Value OPTIONAL, -- Number of beam failure instances before the UE declares beam failure -- Corresponds to L1 parameter 'Beam-Failure-Instance-MaxCount' (see 38.321, section FFS_Section) beamFailureInstanceMaxCount FFS_Value OPTIONAL, -- Details on UE behaviour related to the timer is FFS. (Is this like T310, i.e., the timer to monitor whether the actual -- beams come back? Or is it like T304, i.e., to monitor whether the recovery towards candidate beams succeeds?) -- FFS: Rename to beamFailureDetectionTimer? -- Corresponds to L1 parameter 'Beam-failure-recovery-Timer' (see 38.321?, section FFS_Section) beamFailurerRecoveryTimer FFS_Value OPTIONAL }, beamFailureRecovery SEQUENCE { -- Use of PRACH or/and PUSCH for beam in some combination, details FFS. -- FFS_CHECK: Can be removed since beam recovery is only supported with RA?! linkReconfigurationRequest FFS_Value OPTIONAL, -- A RACH configuration which the UE may uses for beam recovery upon beam failure detection -- FFS: If this field is absent, the UE uses the RACH-ConfigCommon configuration appliable for this serving cell?? -- FFS: Compare with the parameters in RACH-ConfigCommon and try align/re-use. rach-ConfigCommon SEQUENCE { -- PRACH root sequence index for beam failure recovery -- Corresponds to L1 parameter 'RootSequenceIndex-BFR' (see 38.211, section 6.3.3.1) rootSequenceIndex INTEGER (0..137) OPTIONAL, -- RA preamble index for beam failure recovery -- Corresponds to L1 parameter 'ra-PreambleIndexConfig-BFR' (see 38.211, section 6.3.3.2) ra-PreambleIndexConfigBFR INTEGER (0..255), -- N-CS configuration for beam falure recovery, see Table 6.3.3.1-3 in 38.211 -- Corresponds to L1 parameter 'ZeroCorrelationZoneConfig-BFR' (see 38.211, section 6.3.3.1) zeroCorrelationZoneConfig INTEGER (0..15) OPTIONAL, -- Received target power for beam failure request for PRACH -- Corresponds to L1 parameter 'PreambleInitialReceivedTargetPower-BFR' (see 38.213, section 7.4) preambleReceivedTargetPower FFS_Value OPTIONAL, -- Maximum number of beam failure request transmissions -- Corresponds to L1 parameter 'PreambleTransMax-BFR' (see 38.321?, section FFS_Section) preambleTransMax FFS_Value OPTIONAL, -- Power ramping steps for beam failure request via PRACH -- Corresponds to L1 parameter 'powerRampingStep-BFR' (see 38.321?, section FFS_Section) powerRampingStep ENUMERATED {dB0, dB2,dB4, dB6} OPTIONAL, -- RAR-Response Window for beamfailure recovery -- FFS_Value: Use same value range as for normal RAR window?! -- Corresponds to L1 parameter 'Beam-failure-recovery-request-window' (see 38.213, section 6) beamFailureRecoveryRequestWindow ENUMERATED {ffsTypeAndValue} OPTIONAL }, -- FFS: A set of specific candidate beams of this cell and associated dedicated RA preambles which the UE may use to recover -- FFS: If this field is absent or if the UE does not detect any of these candidate beams, it may recover towards any other -- suitable beam of its serving cell using CB-RA. rach-ConfigDedicated SEQUENCE { -- The candidate beam can be considered identified when metric X (FFS) of candidate beam is higher than a threshold" -- Corresponds to L1 parameter 'Beam-failure-candidate-beam-threshold' (see 38.213, section 6) -- FFS: Can this ever be different than the cell suitability criteria? If it is higher, the UE may declare cell-RLF even though -- there was actually a suitable beam. And if it is lower, the UE cannot camp/stay on this cell anyway. beamFailureCandidateBeamThreshold ENUMERATED {ffsTypeAndValue} OPTIONAL, -- List of candidate beam identification RSs and corresponding RA resources -- FFS: Compare to and align with rach-ConfigDedicated. Re-use the association of CSI/SSB resources to RA preambles defined there. -- FFS: Make this a AddMod/Release list? -- Corresponds to L1 parameter 'Candidate-Beam-RS-List' (see 38.213?, section 6) candidateBeams SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF SEQUENCE { -- Reference signal used to identify candidate beam -- Corresponds to L1 parameter 'Candidate-Beam-RS-Identification-Resource' (see 38.213, section 6) -- FFS: Confirm that this is meant to be a choice of SSB or CSI-RS beamFailureCandidateBeamResource CHOICE { ssbId SSB-Index, csi-RS-Id NZP-CSI-RS-ResourceId }, -- Preamble index used to select one from a sequence pool -- Corresponds to L1 parameter 'ra-PreambleIndex-BFR' (see 38.211?, section FFS_Section) ra-PreambleIndex FFS_Value OPTIONAL, -- Same meaning as in initial access -- Corresponds to L1 parameter 'prach-FreqOffset-BFR' (see 38.211?, section FFS_Section) -- Is this really meant to be configured per CF preamble? Or does it belong in COMMON? prach-FreqOffset FFS_Value OPTIONAL, -- Time domain mask. -- Corresponds to L1 parameter 'RACH-resource-mask-BFR' (see 38.211?, section FFS_Section) -- Is this really meant to be configured per CF preamble? Or does it belong in COMMON? rach-resourceMask FFS_Value OPTIONAL } OPTIONAL, -- ID of the CORESET in which the UE receives the Beam Failure Recovery Response. -- Corresponds to L1 parameter 'Beam-Failure-Recovery-Response-CORESET' (see 38.213, section 6) -- When the field is absent the UE applies the value FFS_DefaultValue recoveryControlResourceSetId ControlResourceSetId OPTIONAL } OPTIONAL } } |  |
| H063 | The value of scramblingID can be confirmed as per RAN1 agreement | 2 | As per RAN1 agreements below, the scramblingID included in CSI-MeasConfig has a 10-bit length and should be with the value range of 0 to 2^10-1; also, it has no default value. Agreement: For NR CSI-RS sequence, CSI-RS scrambling ID has a length of 10 bits There is no default value for the scrambling ID Agreement For NR CSI-RS sequence initialization,  Similar to LTE, the sequence initialization values should be different for all OFDM symbols within a frame (10ms) , where ns denotes slot index within a radio frame, and l is OFDM symbol index within a slot and NID∈{0,1,…,210-1} is UE specifically configured scrambling ID The scramblingID actually refers to the IE ScramblingId below. The current ScramblingId has already captured the above RAN1 agreement and thus shouldn’t be replaced by ‘PhysCellId’ (since the value range is differnt), so the FFS can be removed. #### – ScramblingId The IE ScramblingID is used for scrambling channels and reference signals. -- ASN1START -- TAG-SCRAMBLING-ID-START -- FFS: Replace by type PhysCellId? ScramblingId ::= BIT STRING (SIZE (10)) -- TAG-SCRAMBLING-ID-STOP -- ASN1STOP |  |
| H064 | Regarding csi-ResourceConfigs, a field description may be needed to limit the total ammount of CSI-RS resources in all the resource sets as per RAN1 agreement. | 2 | There seems to be related RAN1 agreement saying that “For the beam management use case, support configuration of up to S=16 CSI-RS resources sets per resource setting, and Ks=1~64 CSI-RS resources per resource set. The total number of CSI-RS resources in all sets cannot be more than 128”. So perhaps a field description may be needed to capture such RAN1 restriction, perhaps as follows csi-ResourceConfigs The resource setting(s) for CSI-RS measurement. For each resource setting configured, there can be up to S=16 CSI-RS resource sets, and Ks=1~64 CSI-RS resources per resource set. The totaol number of CSI-RS resources in all sets cannot be more than 128. |  |
| H065 | The value of maxNrofCSI-Reports should be changed from ‘ffs_value’ to ‘16’. | 2 | There is an agreement from RAN1 saying that “A list of ReportConfigIDs per report trigger state (max number of ReportConfigIDs = 16)”. From this point, it can be seen that at most maxNrofCSI-Reports=16 CSI-ReportConfig can be configured, and thus the following change is proposed. maxNrofCSI-Reports INTEGER ::= ffsValue16 -- Maximum number of report configurations |  |
| H066 | Not clear how qcl-Info-aPeriodicReportingTrigger in reportTrigger works. | 3 | It seems that the current specification place a list of TCI-RS-SetId in order to hold a one-to-one mapping with the list of CSI-ResourceSetId. Unfortunately, there is no such agreement so far in RAN1. Also, it is not clear whether this TCI-RS-SetID needs to be further linked to the resource settings corresponding to the same CSI-ReportConfig, and if yes, how. So there might be potential problem for this current field, and it is suggested to wait for further RAN1 confirmation on how to deal with the TCI states included in each report trigger state. Or perhaps the rapporteur can clarify how this field works? |  |
| H067 | The filed semiPersistentOnPUSCH is not well defined. | 2 | As now there is only one filed included in the SEQUENCE, maybe the following structure is better? Also, do we need a list of SP PUSCH reporting configurations, or do we need just one? semiPersistentOnPUSCH SEQUENCE { associatedReportConfig CSI-ReportConfigId } semiPersistentOnPUSCH SEQUENCE (SIZE (1..ffs_value)) OF CSI-ReportConfigId OPTIONAL, |  |
| H068 | The naming of csi-ResourceSetId is not proper. | 2 | From wording perspective, it is better to change the name of csi-ResourceSetId into nzp-csi-rs-ResourceSetId for more accuracy. Note that there are also zp CSI-RS resources, so a common name as the original one may lead to ambiguity. NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-csiCSI-ResourceSetId CSI-ResourceSetId, nzp-csi-rs-Resources SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-Resource, repetition BOOLEAN, aperiodicTriggeringOffset FFS_Value OPTIONAL } |  |
| H069 | One parameter, CSI-IM-timeConfig, in TS 38.214is now missing, and should be added. | 2 | In the CSI-IM resource configuration (i.e. CSI-IM-Resource), the paremeter CSI-IM-timeConfig, which is defined as follows in TS 38.214, 5.2.4.4, is now missing. So RAN2 needs to discuss how to add this CSI-IM-timeConfig as per the definition in TS 38.214 or related RAN1 agreement. ‘CSI-IM-timeConfig defines the CSI-IM periodicity and slot offset for periodic/semi-persistent CSI-IM according to the Subclause TBD of [4, TS 38.211].’ |  |
| H070 | Not Clear why the periodicity and offset of semiPersistentPUCCH and those of semiPersistentPUSCH have different IE format. | 2 | Compared between semiPersistentPUCCH and semiPersistentPUSCH, why does the former have a joint periodicity and slot offset configurations, but the later has only the periodicity configuration, as in corresponding reportSlotConfig? Perhaps, the the reportSlotConfig of semiPersistentPUSCH can be changed to the same as that in semiPersistentPUCCH, if regarded as needed. |  |
| H071 | In reportQuantity, a parameter, which is needed for the value of SSBRI/RSRP as per TS 38.214, is now missing. Also, it is unclear what the existing field cri means, and it should be removed. | 2 | A parameter, ssbri-RSRP, is needed to configure the value of ‘SSBRI/RSRP’ according to TS 38.214, subclause 5.2.3: ‘When the higher layer parameter ReportQuantity is configured with one of the values ‘CRI/RSRP’ or ‘SSBRI/RSRP’, the CSI feedback consists of a single part.’ Also, it seems to make no sense to have a ‘cri’ field, with other fields already being there. reportQuantity CHOICE { none NULL, cri-RI-PMI-CQI NULL, cri-RI-i1 NULL, cri-RI-i1-CQI SEQUENCE { pdsch-BundleSizeForCSI ENUMERATED {n2, n4} OPTIONAL }, cri-RI-CQI NULL, cri NULL, cri-RSRP NULL, ssbri-RSRP NULL, cri-RI-LI-PMI-CQI NULL }, Confirm with RAN1. |  |
| H072 | The measRestrictionTimeForChannel and measRestrictionTimeForInterference should be removed, since there is no agreement for them and they are not in TS 38.214 (as in the comment) either. | 2 | We are not sure what measRestrictionTimeForChannel and measRestrictionTimeForInterference actually refer to. As far as we know, there has not been any agreement supporting them; neither do they appear in any subclause in TS 38.214. if this is the case, then they should be removed form the Spec. reportFreqConfiguration SEQUENCE { cqi-FormatIndicator ENUMERATED { widebandCQI, subbandCQI }, pmi-FormatIndicator ENUMERATED { widebandPMI, subbandPMI }, csi-ReportingBand BIT STRING (SIZE (ffsValue)) }, measRestrictionTimeForChannel ENUMERATED {ffsTypeAndValue}, measRestrictionTimeForInterference ENUMERATED {ffsTypeAndValue}, codebookConfig CodebookConfig, nrofCQIsPerReport ENUMERATED {n1, n2}, |  |
| H073 | The naming of failureDetectionResources is not proper. | 2 | To be more specific and more accurate, it is better to change the name of this field into beamFailureDetectionResources. |  |
| H074 | The field beamFailurerRecoveryTimer is not in a right place. The field beamFailureCandidateBeamThreshold may not be in a right place either. | 2 | We think that this timer should be moved into the below beamFailureRecovery field, because it is more related to beam failure recovery, than beam failure detection. Also, it seems that the beamFailureCandidateBeamThreshold which is a threhold for the beam selection in case of BFR ought to be a common one, instead of a beam specific one. If this is the case, then it should be moved to rach-ConfigCommon. Perhaps the following example helps: beamFailureDetection SEQUENCE { failureDetectionResources FFS_Value OPTIONAL, beamFailureInstanceMaxCount FFS_Value OPTIONAL, beamFailurerRecoveryTimer FFS_Value OPTIONAL }, beamFailureRecovery SEQUENCE { linkReconfigurationRequest FFS_Value OPTIONAL, beamFailureRecoveryTimer FFS_Value OPTIONAL, beamFailureCandidateBeamThreshold ENUMERATED {ffsTypeAndValue} OPTIONAL, rach-ConfigCommon SEQUENCE { rootSequenceIndex INTEGER (0..137) OPTIONAL, zeroCorrelationZoneConfig INTEGER (0..15) OPTIONAL, preambleReceivedTargetPower FFS_Value OPTIONAL, preambleTransMax FFS_Value OPTIONAL, powerRampingStep ENUMERATED {dB0, dB2,dB4, dB6} OPTIONAL, beamFailureRecoveryRequestWindow ENUMERATED {ffsTypeAndValue} OPTIONAL }, rach-ConfigDedicated SEQUENCE { beamFailureCandidateBeamThreshold ENUMERATED {ffsTypeAndValue} OPTIONAL, candidateBeams SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF SEQUENCE { beamFailureCandidateBeamResource CHOICE { ssbId SSB-Index, csi-RS-Id NZP-CSI-RS-ResourceId }, ra-PreambleIndex FFS_Value OPTIONAL, prach-FreqOffset FFS_Value OPTIONAL, rach-resourceMask FFS_Value OPTIONAL } OPTIONAL, recoveryControlResourceSetId ControlResourceSetId OPTIONAL } OPTIONAL } |  |
| H075 | The naming of beamFailureRecoveryRequestWindow is not appropriate, and should be changed to a more proper one. | 2 | Similar to the ra-ResponseWindow, we suggest to change the current name beamFailureRecoveryRequestWindow to beamFailureRecoveryResponseWindow”. Note that this is also in line with the naming in 38.321 (i.e. bfr-ResponseWindow) beamFailureRecoveryRequestResponseWindow ENUMERATED {ffsTypeAndValue} OPTIONAL |  |
| H076 | The value of scramblingID can be confirmed as per RAN1 agreement | 2 | As per RAN1 agreements below, the scramblingID included in CSI-MeasConfig has a 10-bit length and should be with the value range of 0 to 2^10-1; also, it has no default value. Agreement: For NR CSI-RS sequence, CSI-RS scrambling ID has a length of 10 bits There is no default value for the scrambling ID Agreement For NR CSI-RS sequence initialization,  Similar to LTE, the sequence initialization values should be different for all OFDM symbols within a frame (10ms) , where ns denotes slot index within a radio frame, and l is OFDM symbol index within a slot and NID∈{0,1,…,210-1} is UE specifically configured scrambling ID The scramblingID actually refers to the IE ScramblingId below. The current ScramblingId has already captured the above RAN1 agreement and thus shouldn’t be replaced by ‘PhysCellId’ (since the value range is differnt), so the FFS can be removed. #### – ScramblingId The IE ScramblingID is used for scrambling channels and reference signals. -- ASN1START -- TAG-SCRAMBLING-ID-START -- FFS: Replace by type PhysCellId? ScramblingId ::= BIT STRING (SIZE (10)) -- TAG-SCRAMBLING-ID-STOP -- ASN1STOP |  |

#### –	FailureReportSCGtoOtherRAT

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	FrequencyInfoDL

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H077 | ssb-SubcarrierOffset wrong range | 2 | The range of the subcarrier offset here doesn’t match the one in the MIB. In our reading of the RAN1 list the ranges should be the same. Propose to change the range to 0..15 -- The frequency domain offset between SSB and the overall resource block grid in number of subcarriers. -- Absence of the field indicates that no offset if applied (offset = 0). See 38.211, section 7.4.3.1) ssb-subcarrier-offset INTEGER (01..115) OPTIONAL, |  |

#### –	FrequencyInfoUL

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	LogicalChannelConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H078 | Names of some IEs are not aligned with TS 38.321. In addition, Some parameters also needed to be considered when performing LCH selection are not included in the LogicalChannelConfig. Scheduling request ID is missed in the structure. | 2 | -- ASN1START -- TAG-LOGICAL-CHANNEL-CONFIG-START LogicalChannelConfig ::= SEQUENCE { ul-SpecificParameters SEQUENCE { priority INTEGER (1..16), prioritisedBitRate ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512, kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity}, bucketSizeDuration ENUMERATED {ms50, ms100, ms150, ms300, ms500, ms1000, spare2, spare1}, -- FFS: Detailed handling of restrictions (UP email discussion) -- Defined in L1 parameters but the value range must be checked. allowedSubCarrierSpacing lcp-allowedSCS SubcarrierSpacing OPTIONAL, allowedTiming lcp-MaxPUSCH-Duration ENUMERATED {ffsTypeAndValue} OPTIONAL, lcp-ConfiguredGrantType1Allowed BOOLEAN OPTIONAL, lcp-AllowedServingCells BIT STRING (SIZE(0.. maxNrofSCells+1)) OPTIONAL, -- Cond CA-duplication logicalChannelGroup INTEGER (0..maxLCid) OPTIONAL, logicalChannelSR-Mask BOOLEAN, schedulingRequestIDList SEQUENCE (SIZE (1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestId OPTIONAL, logicalChannelSR-DelayTimerApplied BOOLEAN } OPTIONAL -- Cond UL -- other parameters } -- TAG-LOGICAL-CHANNEL-CONFIG-STOP -- ASN1STOP |  |
| H079 | The field description of this IE is not clear and the name is not aligned with TS 38.321 as mentioned above. | 2 | allowedTiming lcp-maxPUSCH-Duration If present, UL MAC PDUs from this logical channel can only be transmitted in the indicated timing indicates the maximum PUSCH transmission duration allowed for transmission for UL MAC PDUs from this logical as specified in TS 38.321 [3]. |  |
| H080 | Add the field description of the missing IE in LogicalChannelConfig | 2 | lcp-AllowedServingCells If present, UL MAC PDUs from this logical channel can only be mapped to the indicated cells as specified in TS 38.321 [3]. |  |
| H081 | Add the field description of the missing IE lcp-ConfiguredGrantType1Allowed in LogicalChannelConfig | 2 | lcp-ConfiguredGrantType1Allowed If present, UL MAC PDUs from this logical channel is allowed to be transmitted on a configured grant type 1 as specified in TS 38.321 [3]. schedulingRequestIDList If present, indicates the list of schedulingRequestId of the associated scheduling request configuration. |  |
| H082 | Add the explanation of condition “CA duplication” | 2 | CA duplication This field is mandatory present upon RRC reconfiguration with setup of a radio bearer associated with this logical channel and with another logical channel of the same cell group and upon RRC reconfiguration with addition of a logical channel of the same cell group like this logical channel and associated to the same radio bearer. Upon RRC reconfiguration when a logical channel of the same cell group like this logical channel is associated to the same radio bearer, this field is optionally present need M. Otherwise, tis field is absent and previously stored value is released. |  |

#### –	MAC-CellGroupConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H083 | The description of drx-HARQ-RTT-TimerDL is wrong according to endorsed TP of L2 parameters | 2 | drx-HARQ-RTT-TimerDL Value in multiple integers of 1ms. ms0 corresponds to 0, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on. Value in number of symbols. |  |
| H084 | The description of drx-Inactivity Timer is wrong according to endorsed TP of L2 parameters | 2 | drx-InactivityTimer Value in number of symbols. Value in multiple integers of 1ms. ms0 corresponds to 0, ms1 corresponds to 1ms, ms2 corresponds to 2ms, and so on. |  |

#### –	MeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H085 | RLM parameters missing | 2 | There is an FFS on where to capture the RLM parameters. This affects L1 parameters RLM-IS-OOS-thresholdConfig, RLM-RS, RLM-RS-List, RLM-SSB, RLM-CSIRS. All these parameters are indicated as “UE specific” in the spreadsheet and could be included in MeasConfig. MeasConfig ::= SEQUENCE { -- Measurement objects measObjectToRemoveList MeasObjectToRemoveList OPTIONAL, measObjectToAddModList MeasObjectToAddModList OPTIONAL, -- Reporting configurations reportConfigToRemoveList ReportConfigToRemoveList OPTIONAL, reportConfigToAddModList ReportConfigToAddModList OPTIONAL, -- Measurement identities measIdToRemoveList MeasIdToRemoveList OPTIONAL, measIdToAddModList MeasIdToAddModList OPTIONAL, -- Other parameters -- s-Measure config s-MeasureConfig CHOICE { ssb-rsrp RSRP-Range, csi-rsrp RSRP-Range } OPTIONAL, quantityConfig QuantityConfig OPTIONAL, -- Placehold for measGapConfig measGapConfig MeasGapConfig OPTIONAL, rlm-IS-OOS-ThresholdConfig ENUMERATED { t0, t1 } OPTIONAL, rlm-RS-List SEQUENCE (SIZE (1..8)) OF RLM-RS, } RLM-RS ::= CHOICE { rlm-SSB INTEGER (0..63), rlm-CSI-RS INTEGER (0..NZP-CSI-RS-ResourceMax-1) } |  |

#### –	MeasId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasIdToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasObjectEUTRA

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasObjectId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasObjectNR

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H086 | subcarrierSpacing wrong type (in CSI-RS-ResourceConfig-Mobility) | 2 | Subcarrier spacing range should be { 15, 30, 60 } for sub6, { 60, 120, 240 } for >6. The type SubcarrierSpacing doesn’t have 240, which is correct for its other uses e.g. for the L1 parameter ref-SCS. Unfortunately it seems we need to add a new type for SubcarrierSpacingCSI-RS. -- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB subcarrierSpacing SubcarrierSpacingCSI-RS, […] SubcarrierSpacingCSI-RS ::= ENUMERATED { kHz15, kHz30, kHz60, kHz120, kHz240 } |  |
| H087 | Duration field wrong range (in SSB-MeasurementTimingConfiguration) | 2 | The duration field has range { sf1, sf5 } when it should be { 1, 2, 3, 4, 5 } according to the spreadsheet. Could change to { sf1, sf2, sf3, sf4, sf5 } or INTEGER (1..5). -- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes -- (see 38.213, section 4.1) -- FFS: RAN1 discusses additional allowed durations: duration ENUMERATED { sf1, sf5 }INTEGER (1..5), |  |
| H088 | associatedSSB should be OPTIONAL (in CSI-RS-ResourceConfig-Mobility) | 2 | Currently ssb-Index is OPTIONAL inside associatedSSB. It seems that the correct semantics are for associatedSSB to be OPTIONAL and ssb-Index to be mandatory within the structure. CSI-RS-ResourceConfig-Mobility ::= SEQUENCE { -- MO specific values csi-rs-MeasurementBW SEQUENCE { -- Size of the measurement BW in PRBs -- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section) nrofPRBs ENUMERATED { size24, size48, size96, size192, size268}, -- Starting PRB index of the measurement bandwidth -- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section) -- FFS_Value: Upper edge of value range unclear in RAN1 startPRB INTEGER(0..251), -- Each CSI-RS resource may be associated with one SSB. If such SSB is indicated, the NW also indicates whether the UE may assume -- quasi-colocation of this SSB with this CSI-RS reosurce. -- Corresponds to L1 parameter 'Associated-SSB' (see FFS_Spec, section FFS_Section) -- FFS: What does the UE do if it there is no such SSB-Index? associatedSSB SEQUENCE { -- FFS_Value: Check the value range ssb-Index SSB-Index OPTIONAL, -- The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters -- Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section) isQuasiColocated BOOLEAN } OPTIONAL, isServingCellMO BOOLEAN }, -- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB subcarrierSpacing SubcarrierSpacing, -- FFS_Description. -- FFS_CHECK: Should this be in the resource-config (here) or in the resource (below)? -- Corresponds to L1 parameter 'Common-PRB-Grid-offset' (see FFS_Spec, section FFS_Section) prb-GridOffset INTEGER (maxNrofPhysicalResourceBlocksTimes4) OPTIONAL, -- List of resources csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility } |  |
| H089 | SS-RSSI-MeasurementConfig is missing | 2 | Missing L1 parameter ‘SS-RSSI-MeasurementConfig’. Propose to add it inside SSB-MeasurementTimingConfiguration as follows: -- A measurement timing configuration SSB-MeasurementTimingConfiguration ::= SEQUENCE { -- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements. smtc1 SEQUENCE { -- Periodicity and offset of the measurement window in which to receive SS/PBCH blocks. -- Periodicity and offset are given in number of subframes. -- FFS_FIXME: This does not match the L1 parameter table! They seem to intend an index to a hidden table in L1 specs. -- (see 38.213, section REF): periodicityAndOffset CHOICE { sf5 INTEGER (0..4), sf10 INTEGER (0..9), sf20 INTEGER (0..19), sf40 INTEGER (0..39), sf80 INTEGER (0..79), sf160 INTEGER (0..159) }, -- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes -- (see 38.213, section 4.1) -- FFS: RAN1 discusses additional allowed durations: duration ENUMERATED { sf1, sf5 }, -- L1 parameter 'SS-RSSI-MeasurementConfig' ss-RSSI-MeasurementConfig SEQUENCE { ss-RSSI-MeasurementSlotConfig BIT STRING (SIZE (160)), -- Number of slots in the SMTC window: maximum duration 5 ms, -- maximum 32 slots per ms ss-RSSI-MeasurementSymbolConfig INTEGER (0..3) } |  |
| H090 | frequencyOffset needs a type (inside ReferenceSignalConfig) | 3 | Contribution to be provided |  |
| H091 | ssb-ToMeasure should be outside SMTC structure | 2 | The ssb-ToMeasure field should be outside the SMTC1 and SMTC2 structures since it applies to both of them. Move to the top level of SSB-MeasurementTimingConfiguration. -- A measurement timing configuration SSB-MeasurementTimingConfiguration ::= SEQUENCE { -- The set of SS blocks to be measured within the SMTC measurement duration. -- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section) -- When the field is absent the UE measures on all SS-blocks ssb-ToMeasure SetupRelease { CHOICE { -- bitmap for sub 3 GHz shortBitmap BIT STRING (SIZE (4)), -- bitmap for 3-6 GHz mediumBitmap BIT STRING (SIZE (8)), -- bitmap for above 6 GHz longBitmap BIT STRING (SIZE (64)) } } OPTIONAL -- Need M -- Primary measurement timing configuration. Applicable for intra- and inter-frequency measurements. smtc1 SEQUENCE { -- Periodicity and offset of the measurement window in which to receive SS/PBCH blocks. -- Periodicity and offset are given in number of subframes. -- FFS_FIXME: This does not match the L1 parameter table! They seem to intend an index to a hidden table in L1 specs. -- (see 38.213, section REF): periodicityAndOffset CHOICE { sf5 INTEGER (0..4), sf10 INTEGER (0..9), sf20 INTEGER (0..19), sf40 INTEGER (0..39), sf80 INTEGER (0..79), sf160 INTEGER (0..159) }, -- Duration of the measurement window in which to receive SS/PBCH blocks. It is given in number of subframes -- (see 38.213, section 4.1) -- FFS: RAN1 discusses additional allowed durations: duration ENUMERATED { sf1, sf5 }, -- The set of SS blocks to be measured within the SMTC measurement duration. -- Corresponds to L1 parameter 'SSB-measured' (see FFS_Spec, section FFS_Section) -- When the field is absent the UE measures on all SS-blocks -- FFS_CHECK: Is this IE placed correctly. ssb-ToMeasure SetupRelease { CHOICE { -- bitmap for sub 3 GHz shortBitmap BIT STRING (SIZE (4)), -- bitmap for 3-6 GHz mediumBitmap BIT STRING (SIZE (8)), -- bitmap for above 6 GHz longBitmap BIT STRING (SIZE (64)) } } OPTIONAL -- Need M }, -- Secondary measurement timing confguration for explicitly signalled PCIs. It uses the offset and duration from smtc1. -- It is supported only for intra-frequency measurements in RRC CONNECTED. smtc2 SEQUENCE { -- PCIs that are known to follow this SMTC. pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysicalCellId OPTIONAL, -- Periodicity for the given PCIs. Timing offset and Duration as provided in smtc1. periodicty ENUMERATED {ffsTypeAndValue} } OPTIONAL -- Cond IntraFreqConnected } |  |
| H092 | resourceElementMappingPattern needs a type (in CSI-RS-Resource-Mobility | 2 | RAN1 have agreed the RE mapping pattern for RRM reuses the parameters from beam management. However, it isn’t clear from the spreadsheet what the equivalent parameters from beam management are; RAN1 input is needed. CSI-RS-Resource-Mobility ::= SEQUENCE { csi-rs-ResourceId-RRM CSI-RS-ResourceId-RRM, cellId PhysicalCellId, -- FFS_CHECK whether the following fields are supposed to be per resource (here) or in the resource config (above) -- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)FFS_Ref slotConfig CHOICE { ms5 INTEGER (0..4), ms10 INTEGER (0..9), ms20 INTEGER (0..19), ms40 INTEGER (0..39) }, -- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref resourceElementMappingPattern ENUMERATED {ffsTypeAndValue}, -- Sequence generation parameter for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref sequenceGenerationConfig INTEGER (0..1023), -- Frequency domain density for the 1-port CSI-RS for L3 mobility -- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section) density ENUMERATED {d1,d3} OPTIONAL, ... } |  |
| H093 | Inconsistant field names between procedure description and ASN.1: nroSS-BlocksToAverage vs nrofSS-BlocksToAverage, nroCSI-RS-ResourcesToAverage vs nrofCSI-RS-ResourcesToAverage | 2 | MeasObjectNR ::= SEQUENCE { carrierFreq ARFCN-ValueNR, --RS configuration (e.g. SMTC window, CSI-RS resource, etc.) referenceSignalConfig ReferenceSignalConfig OPTIONAL, --Consolidation of L1 measurements per RS index absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, --Config for cell measurement derivation nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, nrofCSI-RS-ResourcesToAverage INTEGER (2..maxNrofCSI-RS-ResourcesToAverage) OPTIONAL, -- Filter coefficients applicable to this measurement object quantityConfigIndex INTEGER (1..maxQuantityConfigId), --Frequency-specific offsets (only for events A3, A6) offsetFreq Q-OffsetRangeList, |  |

#### –	MeasObjectToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	MeasResults

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PDCCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H094 | OSI search space should have type SearchSpace [Note: we propose elsewhere to change this name to SearchSpaceSet] | 2 | The spreadsheet indicates the value range as search-space-config which corresponds to the ASN.1 type SearchSpace. PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOtherSystemInformation FFS_ValueSearchSpace OPTIONAL, -- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10) -- FFS: Which BWP and CORESET to assume? -- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space? pagingSearchSpace FFS_Value OPTIONAL } |  |
| H095 | Paging search space should have type SearchSpace [Note: we propose elsewhere to change this name to SearchSpaceSet] | 2 | The spreadsheet indicates the value range as search-space-config which corresponds to the ASN.1 type SearchSpace. PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOtherSystemInformation FFS_Value OPTIONAL, -- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10) -- FFS: Which BWP and CORESET to assume? -- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space? pagingSearchSpace FFS_ValueSearchSpace OPTIONAL } |  |
| H096 | L1 parameter list indicates searchSpace is a CHOICE between CORESET and CSS. It isn’t clear, how this should work - the other parameters according to L1 parameters are child IEs of SlotFormatIndicatorSFI (SFI-PDCCH) and not part of this SearchSpace field, are they only used when search space is CSS, or are parameters are always there outside of this choice and SearchSpace is just an enumerated selection between CORESET and CSS as L1 parameters list suggests? RAN1 should clarify. | 3 | -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace CHOICESEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId , css SEQUENCE { OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} } }, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H097 | maxNrofAggregatedCellsPerCellGroup is FFS – needs RAN1 feedback | 2 | -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H098 | No default value for dci-PayloadSize. Value range is FFS in RAN1 | 2 | -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H099 | No default value for monitoringPeriodicity | 2 | SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H100 | SlotFormatIndicatorSFI is defined but not referenced anywhere – should be in PDCCH-Config? L1 parameters list states “common (search space)”, however it is indicated as UE specific and different mapping may exist for different UE? SRS-Config (SRS-CarrierSwitching) references FFS type of CellToSFI so should the whole thing be moved to SRS-Config? | 2 | FFS |  |
| H101 | maxNrofAggregatedCellsPerCellGroup is FFS | 2 | -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H102 | maxNrofSlotFormatCombinationsPerSet is FFS | 2 | -- Mapping for a given cell to SFI value within DCI message. Corresponds to L1 parameter 'cell-to-SFI' (see 38.213, section FFS_Section) SlotFormatCombinationsPerCell ::= SEQUENCE { -- The ID of the serving cell for which the slotFormatCombinations are applicable servingCellId ServCellIndex, -- A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see 38.211, section 4.3.2) -- FFS_CHECK: RAN1 indicates that the combinations could be of two different types... but they don't specify the second slotFormatCombinations SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination OPTIONAL, -- The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload. -- Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section) positionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL } |  |
| H103 | Also the L1 parameter list specifies 2 types of slotformatcombination, but the second is undefined. | 2 | -- Mapping for a given cell to SFI value within DCI message. Corresponds to L1 parameter 'cell-to-SFI' (see 38.213, section FFS_Section) SlotFormatCombinationsPerCell ::= SEQUENCE { -- The ID of the serving cell for which the slotFormatCombinations are applicable servingCellId ServCellIndex, -- A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see 38.211, section 4.3.2) -- FFS_CHECK: RAN1 indicates that the combinations could be of two different types... but they don't specify the second slotFormatCombinations SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination OPTIONAL, -- The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload. -- Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section) positionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL } |  |
| H104 | maxNrofSlotFormatsPerCombination is FFS | 2 | SlotFormatCombination ::= SEQUENCE { -- This ID is used in the payload to dynamically select this SlotFormatCombination. -- Corresponds to L1 parameter 'SFI-index' (see 38.213, section FFS_Section) slotFormatCombinationId SlotFormatCombinationId, -- Slot formats that occur in consecutive slots in time domain order as listed here. The the slot formats are -- defined in 38.211, table 4.3.2-3 and numbered with 0..255. slotFormats SEQUENCE (SIZE (1.. maxNrofSlotFormatsPerCombination)) OF INTEGER (0..255) } |  |
| H105 | mappingType, index have no default so should be mandatory TO discuss: is index referring to 38.214? the Time-domain PDSCH resources field of the DCI provides a row index of an RRC configured table [pdsch-symbolAllocation], In this case it is not an RRC parameter. RAN1 may need to clarify. | 3 | -- FFS: Is there a default timing (to be used at least until first reconfiguration). Are the fields optionally present? timing SEQUENCE { -- Configuration value of DL assignment to DL data timing dl-assignment-to-DL-data SEQUENCE { -- FFS_Description. Corresponds to L1 parameter 'K0' (see 38.214, section FFS_Section) -- When the field is absent the UE applies the value 0 k0 INTEGER (0..3) OPTIONAL, -- FFS_Description. Isn't this the delay from UL-Grant to UL-Data? If so, why is it in this group? -- Corresponds to L1 parameter 'K2' (see 38.214, section FFS_Section) -- When the field is absent the UE applies the value 0 k2 INTEGER (0..7) OPTIONAL, -- PDSCH mapping type. Corresponds to L1 parameter 'Mapping-type' (see 38.214, section FFS_Section) mappingType ENUMERATED {typeA, typeB} OPTIONAL, -- An index into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded) -- Corresponds to L1 parameter 'Index-start-len' (see 38.214, section FFS_Section) startSymbolAndLength BIT STRING (SIZE (6)), -- FFS_Description. (see 38.214, section FFS_Section) index INTEGER (0..15) OPTIONAL }, |  |
| H106 | startSymbolAndLength should be defined properly by RAN1 | 2 | -- FFS: Is there a default timing (to be used at least until first reconfiguration). Are the fields optionally present? timing SEQUENCE { -- Configuration value of DL assignment to DL data timing dl-assignment-to-DL-data SEQUENCE { -- FFS_Description. Corresponds to L1 parameter 'K0' (see 38.214, section FFS_Section) -- When the field is absent the UE applies the value 0 k0 INTEGER (0..3) OPTIONAL, -- FFS_Description. Isn't this the delay from UL-Grant to UL-Data? If so, why is it in this group? -- Corresponds to L1 parameter 'K2' (see 38.214, section FFS_Section) -- When the field is absent the UE applies the value 0 k2 INTEGER (0..7) OPTIONAL, -- PDSCH mapping type. Corresponds to L1 parameter 'Mapping-type' (see 38.214, section FFS_Section) mappingType ENUMERATED {typeA, typeB} OPTIONAL, -- An index into a table/equation in RAN1 specs capturing valid combinations of start symbol and length (jointly encoded) -- Corresponds to L1 parameter 'Index-start-len' (see 38.214, section FFS_Section) startSymbolAndLength BIT STRING (SIZE (6)), -- FFS_Description. (see 38.214, section FFS_Section) index INTEGER (0..15) OPTIONAL }, |  |
| H107 | In ControlResourceSet, reg-BundleSize, precoderGranularity, interleaverSize (this is the name used in specifications, not interleaverRows) and shiftIndex are always provided while, according to 38.211 section 7.3.2.2, they are only for the case when cce-reg-MappingType is "interleaved" | 2 | Make cc-reg-MappingType a CHOICE with the listed parameter within a SEQUENCE for the value interleaved. -- Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles. -- (see 38.211, section 7.3.2.2) reg-BundleSize ENUMERATED {n2, n3, n6}, -- Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG). (see 38.211, 38.213, FFS_REF) cce-reg-MappingType ENUMERATEDCHOICE { interleaved , nonInterleaved },SEQUENCE { -- Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles. -- (see 38.211, section 7.3.2.2) reg-BundleSize ENUMERATED {n2, n3, n6}, -- Precoder granularity in frequency domain (see 38.2131, section FFS_REF7.3.2.2 and 7.4.1.3.2) precoderGranularity ENUMERATED {ffsTypeAndValue sameAsREG-Bundle, allContiguousRBs }, -- Corresponds to L1 parameter 'CORESET-interleaver-rowssize' (see 38.211, 38.213, section FFS_Section) interleaverRowsSize ENUMERATED {n2, n3, n6} OPTIONAL, -- Corresponds to L1 parameter 'CORESET-shift-index' (see 38.211, 38.213, section FFS_Section7.3.2.2) shiftIndex INTEGER(0..maxNrofPhysicalResourceBlocks-1) OPTIONAL, }, nonInterleaved NULL } |  |
| H108 | ControlResourceSet=>tci-PresentInDCI | 2 | The name should be changed to tci-PresenceDCI to align with sfi and pi -- A time/frequency control resource set (CORESET) in which to search for downlink control information (see 38.213, section x.x.x.x)FFS_Ref ControlResourceSet ::= SEQUENCE { controlResourceSetId ControlResourceSetId, -- Frequency domain resources for the CORESET. The network ensures that the CORESET is within the BWP configured for a UE. (see 38.213, REF) frequencyDomainResources ENUMERATED {ffsTypeAndValue}, -- Contiguouse time duration of the CORESET in number of symbols see 38.213, section x.x.x.x)FFS_Ref duration INTEGER (1..maxCoReSetDuration), -- Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles. -- (see 38.211, section 7.3.2.2) reg-BundleSize ENUMERATED {n2, n3, n6}, -- Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG). (see 38.211, 38.213, FFS_REF) cce-reg-MappingType ENUMERATED { interleaved, nonInterleaved }, -- Precoder granularity in frequency domain (see 38.213, section FFS_REF) precoderGranularity ENUMERATED {ffsTypeAndValue}, -- Corresponds to L1 parameter 'CORESET-interleaver-rows' (see 38.211, 38.213, section FFS_Section) interleaverRows ENUMERATED {n2, n3, n6} OPTIONAL, -- Corresponds to L1 parameter 'CORESET-shift-index' (see 38.211, 38.213, section FFS_Section) shiftIndex INTEGER(0..maxNrofPhysicalResourceBlocks-1) OPTIONAL, -- A subset of the TCI states defined in TCI-States used for providing QCL relationships between the DL RS(s) in one RS Set -- (TCI-RS-Set) and the PDCCH DMRS ports. Corresponds to L1 parameter 'TCI-StatesPDCCH' (see 38.214, section FFS_Section) -- FFS_Description: Explains what the UE does with each TCI-RS-SetConfig. -- FFS_Value: Shouldn’t this be just a list of indexes? I.e., aren’t the set-configs provided elsewhere? tci-StatesPDCCH SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-RS-SetId OPTIONAL, -- If at least spatial QCL is configured/indicated, this field indicates if TCI field is present or not present in DL-related DCI. -- When the field is absent the UE considers the TCI to be absent/disabled. -- Corresponds to L1 parameter 'TCI-PresentInDCI' (see 38,213, section 5.1.5) tci-PresencetInDCI ENUMERATED {enabled} OPTIONAL, -- PDCCH DMRS scrambling initalization. Corresponds to L1 parameter 'PDCCH-DMRS-Scrambling-ID' (see 38.214, section 5.1) -- When the field is absent the UE applies the value '0'. pdcch-DMRS-ScramblingID BIT STRING (SIZE (16)) OPTIONAL } |  |
| H109 | ControlResourceSet=>tci-StatesPDCCH | 2 | tci-StateRefId -- FFS_Description: Explains what the UE does with each TCI-RS-SetConfig. Not clear if PDSCH-Config contains one list of TCI states (with some structure) and PDCCH-Config only has a list of IDs, or if PDCCH-Config also has definitions of the TCI states. RAN1 to confirm. |  |
| H110 | DownlinkPreemption | 3 | The configuraiton of downlink preemption should be per BWP, as indicated in the following RAN 1 agreement: Configuration of UE monitoring of preemption indication is per DL BWP Needs to be discussed in general how per BWP parameters will be structured. |  |
| H111 | DownlinkPreemption | 2 | The parameter Preemp-DL, as given in R1-1721581, is missing. The function of this parameter is to turn on or off the monitoring of DL preemption DCI. This can be inferred from the presence/absence of monitoringPeriodicity so it may only be necessary to document the parameter in a comment. -- Configuration of downlink preemption indication on PDCCH -- FFS: How does it relate to a BWP? How is it linked to one (or several) CORESETs? DownlinkPreemption ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: Need to indicate the CORESET(s) on which to apply the INT-RNTI SearchSpace!? controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space. -- (see 38.213, section 10) -- FFS: What does the abbreviation stand for? Add a better description int-RNTI RNTI-Value, -- Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1) -- FFS: Clarify description. Clarify what TF means. Clarify field name. timeFrequencySet ENUMERATED {set0, set1}, -- Monitoring periodicity of DCI with INT-RNTI in number of slots. sl1 corresponds to ”every slot”, s2 corresponds to ”every second slot”. -- Corresponds to L1 parameter 'INT-monitoring-periodicity' (see 38.213, section 11.2) -- Presence flag corresponds to L1 parameter 'Preemp-DL' (see 38.214, section 11.2) monitoringPeriodicity ENUMERATED {sl1, sl2, spare2, spare1} OPTIONAL }, -- Total length of the DCI payload scrambled with INT-RNTI. The value must be an integer multiple of 14 bit. -- Corresponds to L1 parameter 'INT-DCI-payload-length' (see 38.213, section 11.2) dci-PayloadSize INTEGER (0..maxINT-DCI-PayloadSize) OPTIONAL, -- Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload. -- Corresponds to L1 parameter 'INT-cell-to-INT' and 'cell-to-INT' (see 38.213, section 11.2) int-ConfigurationPerServingCell SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { servingCellId ServCellIndex, -- Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI -- payload. Must be multiples of 14 (bit). Corresponds to L1 parameter 'INT-values' (see 38.213, section 11.2) positionInDCI INTEGER (0..maxINT-DCI-PayloadSize-1) } OPTIONAL } |  |
| H112 | DownlinkPreemption=> DLpreemption=> timeFrequencySet | 2 | FFS: Clarify description. Clarify what TF means. Clarify field name. Also, this parameter should not be within the search space configuration of DownlinkPreemption A description is given in R1-1721581: “Set selection for DL-preemption indication, the set indication two different manners the DL preemption DCI is interpreteded by the UE.” -- Configuration of downlink preemption indication on PDCCH -- FFS: How does it relate to a BWP? How is it linked to one (or several) CORESETs? DownlinkPreemption ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: Need to indicate the CORESET(s) on which to apply the INT-RNTI SearchSpace!? controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space. -- (see 38.213, section 10) -- FFS: What does the abbreviation stand for? Add a better description int-RNTI RNTI-Value, -- Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1) -- FFS: Clarify description. Clarify what TF means. Clarify field name. timeFrequencySet ENUMERATED {set0, set1}, -- Monitoring periodicity of DCI with INT-RNTI in number of slots. sl1 corresponds to ”every slot”, s2 corresponds to ”every second slot”. -- Corresponds to L1 parameter 'INT-monitoring-periodicity' (see 38.213, section 11.2) monitoringPeriodicity ENUMERATED {sl1, sl2, spare2, spare1} OPTIONAL }, -- Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1) -- The set indicates two different manners the DL preemption DCI is interpreteded by the UE. timeFrequencySet ENUMERATED {set0, set1}, -- Total length of the DCI payload scrambled with INT-RNTI. The value must be an integer multiple of 14 bit. -- Corresponds to L1 parameter 'INT-DCI-payload-length' (see 38.213, section 11.2) dci-PayloadSize INTEGER (0..maxINT-DCI-PayloadSize) OPTIONAL, -- Indicates (per serving cell) the position of the 14 bit INT values inside the DCI payload. -- Corresponds to L1 parameter 'INT-cell-to-INT' and 'cell-to-INT' (see 38.213, section 11.2) int-ConfigurationPerServingCell SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { servingCellId ServCellIndex, -- Starting position (in number of bit) of the 14 bit INT value applicable for this serving cell (servingCellId) within the DCI -- payload. Must be multiples of 14 (bit). Corresponds to L1 parameter 'INT-values' (see 38.213, section 11.2) positionInDCI INTEGER (0..maxINT-DCI-PayloadSize-1) } OPTIONAL } |  |
| H113 | DownlinkPreemption=> DLpreemption=>int-RNTI | 2 | The abbreviation stands for interruption-RNTI, which is a possible description for this parameter. -- Configuration of downlink preemption indication on PDCCH -- FFS: How does it relate to a BWP? How is it linked to one (or several) CORESETs? DownlinkPreemption ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: Need to indicate the CORESET(s) on which to apply the INT-RNTI SearchSpace!? controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space. -- (see 38.213, section 10) -- FFS: What does the abbreviation stand for? Add a better descriptionThe abbreviation stands for interruption-RNTI. int-RNTI RNTI-Value, |  |
| H114 | DownlinkPreemption=>dci-PayloadSize | 2 | Suggest to change the name to int-DCI-PayloadSize as there is also a dci-payloadsize for sfi -- Configuration of downlink preemption indication on PDCCH -- FFS: How does it relate to a BWP? How is it linked to one (or several) CORESETs? DownlinkPreemption ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: Need to indicate the CORESET(s) on which to apply the INT-RNTI SearchSpace!? controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space. -- (see 38.213, section 10) -- FFS: What does the abbreviation stand for? Add a better description int-RNTI RNTI-Value, -- Set selection for DL-preemption indication. Corresponds to L1 parameter 'int-TF-unit' (see 38.213, section 10.1) -- FFS: Clarify description. Clarify what TF means. Clarify field name. timeFrequencySet ENUMERATED {set0, set1}, -- Monitoring periodicity of DCI with INT-RNTI in number of slots. sl1 corresponds to ”every slot”, s2 corresponds to ”every second slot”. -- Corresponds to L1 parameter 'INT-monitoring-periodicity' (see 38.213, section 11.2) monitoringPeriodicity ENUMERATED {sl1, sl2, spare2, spare1} OPTIONAL }, -- Total length of the DCI payload scrambled with INT-RNTI. The value must be an integer multiple of 14 bit. -- Corresponds to L1 parameter 'INT-DCI-payload-length' (see 38.213, section 11.2) int-DCIdci-PayloadSize INTEGER (0..maxINT-DCI-PayloadSize) OPTIONAL, |  |
| H115 | DownlinkPreemption=>searchSpace | 2 | Similar as above. The name should be changed to downlinkPreemptionSearchSpaceSet -- Configuration of downlink preemption indication on PDCCH -- FFS: How does it relate to a BWP? How is it linked to one (or several) CORESETs? DownlinkPreemption ::= SEQUENCE { downlinkPreemptionSsearchSpaceSet SEQUENCE { -- FFS: Need to indicate the CORESET(s) on which to apply the INT-RNTI SearchSpace!? |  |
| H116 | pdcch-ConfigCommon | 2 | A common search space for RAR needs to be added PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOtherSystemInformation FFS_Value OPTIONAL, -- Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) ra-SearchSpace SearchSpace OPTIONAL, |  |
| H117 | pdcch-ConfigCommon =>pagingSearchSpace | 2 | FFS: which BWP and CORESET to assume?Can one just instantiate a common search space? In clause 10.1 of 38.213, the following specification was made: For Type0A-PDCCH common search space or for Type-2 PDCCH common search space, the control resource set is same as the control resource set for Type0-PDCCH common search space. A UE is provided a configuration for Type0A-PDCCH common search space by higher layer parameter osi-SearchSpace. A UE is provided a configuration for Type2-PDCCHcommon search space by higher layer parameter paging-SearchSpace. …… If a UE is configured for downlink bandwidth part (BWP) operation, as described in Subclause 12, the above configurations for the common search spaces apply for the initial active DL BWP. The UE can be additionally configured a control resource set for Type0-PDCCH common search space, Type0A-PDCCH common search space, Type1-PDCCH common search space, or Type2-PDCCH common search space for each configured DL BWP on the primary cell, other than the initial active DL BWP, as described in Subclause 12. Based on the above specifications, we think that the BWP for paging can be the initial BWP part. In addition, the BWP can also be additionaly configured. The CORESET for paging is the same as the CORESET for type0-PDCCH common search space. Furthermore, based on the configuration given in R1-1721581 for paging-SearchSpace, the value range is searchSpace-config. Hence, here the value and type can be SearchSpaceSet (currently SearchSpace; we propose to change the name, see below). PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOtherSystemInformation FFS_Value OPTIONAL, -- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10) -- FFS: Which BWP and CORESET to assume? -- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space? pagingSearchSpace FFS_ValueSearchSpaceSet OPTIONAL } |  |
| H118 | pdcch-ConfigCommon=> searchSpaceOtherSystemInformation pdcch-ConfigCommon=>pagingSearchSpace | 2 | We suggest to chanage the name to otherSystemInformationSearchSpaceSet to be aligned with the name for paging pagingSearchSpace. We also suggest to change pagingSearchSpace to pagingSearchSpaceSet PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOotherSystemInformationSearchSpaceSet FFS_Value OPTIONAL, -- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10) -- FFS: Which BWP and CORESET to assume? -- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space? pagingSearchSpaceSet FFS_Value OPTIONAL } |  |
| H119 | searchSpace | 2 | Based on agreement in RAN1 #91 meeting, it will be better to change “Search space” to “Search space set”, “search space ” is usually defined as set of PDCCH candidate per AL, “Search space set” should be set of search space over all ALs within one CORESET, corresponding to one-or more RNTI, to be confirmed or clarified in next meeting RAN 1 agreement in #91 - Introduce a linkage between search space set and CORESET via an index to the CORESET configuration - CORESET is removed from the search space configuration - In Rel-15, the max no. of CORESETs configurable for a BWP in a cell for a UE is [3] In Rel-15, the max no. of search space sets configurable for a BWP in a cell for a UE is [10] -- A search space defines how/where to search for PDCCH candidates. A search space is associated with one Control Resource Set SearchSpaceSet ::= SEQUENCE { searchSpaceId SearchSpaceId, [Note: if agreed, this would require also to change the places the type is used, but we haven’t included all of them in the table.] |  |
| H120 | searchSpace=>searchSpaceId | 2 | -- FFS: Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon? -- FFS: Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling? The indexing should be discussed in next RAN1# meeting. |  |
| H121 | searchSpace=>searchSpaceType | 2 | -- FFS: How many CSSs can the NW configure? And can a CSS only be in the common CORESET? Working assumption in RAN 1 is CCS+USS=10 per BWP Needs final agreement from RAN1 |  |
| H122 | searchSpaceId | 2 | maxNrofSearchSpaces to be confirmed in next meeting but the working assumption is 10, as can be seen in the following agreement in RAN 1 Agrrement in RAN1#91 meeting: In Rel-15, the max no. of search space sets configurable for a BWP in a cell for a UE is [10] Needs final agreement in RAN1 |  |
| H123 | SlotFormatCombinationsPerCell=> positionInDCI | 2 | This parameter should be in SlotFormatIndicatorSFI, not inside SlotFormatCombinationsPerCell. This is specified in R1-1721581 in the parent field of the parameter sfi-PositionInDCI. We suggest to change the name of the field to sfi-PositionInDCI, as we also have the positionInDCI parameter for DL preemption -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, -- The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload. -- Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section) sfi-PositionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL ... } -- Mapping for a given cell to SFI value within DCI message. Corresponds to L1 parameter 'cell-to-SFI' (see 38.213, section FFS_Section) SlotFormatCombinationsPerCell ::= SEQUENCE { -- The ID of the serving cell for which the slotFormatCombinations are applicable servingCellId ServCellIndex, -- A list with SlotFormatCombinations. Each SlotFormatCombination comprises of one or more SlotFormats (see 38.211, section 4.3.2) -- FFS_CHECK: RAN1 indicates that the combinations could be of two different types... but they don't specify the second slotFormatCombinations SEQUENCE (SIZE (1..maxNrofSlotFormatCombinationsPerSet)) OF SlotFormatCombination OPTIONAL, -- The (starting) position (bit) of the slotFormatCombinationId (SFI-Index) for this serving cell (servingCellId) within the DCI payload. -- Corresponds to L1 parameter 'SFI-values' (see 38.213, section FFS_Section) positionInDCI INTEGER(0..maxSFI-DCI-PayloadSize-1) OPTIONAL } |  |
| H124 | SlotFormatIndicatorSFI | 3 | SlotFormatIndicatorSFI should be configured per cell, as given in the following RAN1 agreement: UE can be configured to monitor SFI in group common PDCCH for a Scell on a different cell. For cross cell GC-PDCCH monitoring, support by RRC configuration for a UE the following: * The same SFI can be applicable to more than one cell * Different SFI fields in one GC-PDCCH can be applied to different cells * FFS interaction with multiple BWP configuration per cell PDCCH-ConfigCommon ::= SEQUENCE { -- Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) -- FFS: Must indicate the CORESET(s) that it is associated with. Must indicate the RNTI(s) to use (note that RAN2 intends to allow -- several in order to be able to send several SI messages in a the same slot. Is it limited to certain CORESETs and or BWPs? -- (e.g. on the initial CSS or on a CSS configured in the dedicated BWP?). Is the field optional? What does the UE do if it is not present? searchSpaceOtherSystemInformation FFS_Value OPTIONAL, -- Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10) -- FFS: Which BWP and CORESET to assume? -- FFS: Need to configure P-RNTI? Or is it specified? Can one just instantiate a common search space? pagingSearchSpace FFS_Value OPTIONAL, slotFormatIndicatorSFI SlotFormatIndicatorSFI } |  |
| H126 | SlotFormatIndicatorSFI=>dci-PayloadSize | 2 | Suggest to change the name to sfi-DCI-PayloadSize, to avoid the duplication with the dci-payloadsize defined in DL pre-emption -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { searchSpace SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) sfi-DCIdci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H127 | SlotFormatIndicatorSFI=>searchSpace | 2 | The name should be modified as the name searchSpace is already defined. We suggest that the name to be changed to sfi-SearchSpaceSet -- Configuration of monitoring a Group-Common-PDCCH for Slot-Format-Indicators (SFI) SlotFormatIndicatorSFI ::= SEQUENCE { sfi-SsearchSpaceSet SEQUENCE { -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! controlResourceSetId ControlResourceSetId OPTIONAL, -- RNTI used for SFI on the given cell -- Corresponds to L1 parameter 'SFI-RNTI' (see 38.213, section FFS_Section) sfi-RNTI RNTI-Value OPTIONAL, -- Monitoring periodicity of SFI PDCCH in slots. -- o For 15KHz SCS (slots based on 15kHz): 1, 2, 5, 10, 20 -- o For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20 -- o For 60KHz SCS (slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20 -- o For 120KHz SCS (slots based on 120kHz): 1, 2, 4, 5, 10, 20 -- Corresponds to L1 parameter 'SFI-monitoring-periodicity' (see 38.213, section FFS_Section) monitoringPeriodicity ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl20, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SFI-Num-PDCCH-cand' (see 38.213, section FFS_Section) nrofCandidates ENUMERATED {n1, n2}, -- The aggregation level for the SFI-PDCCH. Corresponds to L1 parameter 'SFI-Aggregation-Level' (see 38.213, section FFS_Section) aggregationLevel ENUMERATED {n1, n2, n4, n8, n16} }, -- Total length of the DCI payload scrambled with SFI-RNTI. -- Corresponds to L1 parameter 'SFI-DCI-payload-length' (see 38.213, section FFS_Section) dci-PayloadSize INTEGER (1..maxSFI-DCI-PayloadSize) OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SFI-cell-to-SFI' (see 38.213, section FFS_Section) slotFormatConfigurations SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, ... } |  |
| H128 | SlotFormatIndicatorSFI=>searchSpace=>controlResourceId | 3 | -- FFS: RAN1 indicates that the UE uses the ”CSS” if no CORESET is provided. But a CSS is not the same as a CORESET?!?!! It will not be possible that there is no associated CORESET. Needs RAN1 clarification |  |
| H129 | timing | 2 | A more suitable name may be timeDomainResourceAllocation, more aligned with 214 clause 5.1.2.1 titled with “resource allocation in time domain”. PDCCH-Config ::= SEQUENCE { -- List of Control Resource Sets (CORESETs) to be used by the UE controlResourceSetToAddModList SEQUENCE(SIZE (1..maxNrofControlResourceSets)) OF ControlResourceSet OPTIONAL, controlResourceSetToReleaseList SEQUENCE(SIZE (1..maxNrofControlResourceSets)) OF ControlResourceSetId OPTIONAL, -- FFS: RAN1 decided to model each RNTI which the UE monitors on PDCCH as a separate Search Space. Many of those are configured -- in the respective feature/channel configurations (e.g. CSI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, SPS-RNTI). Others are configured -- below (SFI-RNTI, INT-RNTI). ==> Aim to find a common generic structure. searchSpacesToAddModList SEQUENCE(SIZE (1..maxNrofSearchSpaces)) OF SearchSpace OPTIONAL, searchSpacesToReleaseList SEQUENCE(SIZE (1..maxNrofSearchSpaces)) OF SearchSpaceId OPTIONAL, -- Configuration of downlink preemtption indications to be monitored in this cell -- FFS: Can there be just one or multiple such configurations within a PDCCH-Config? How does it relate to BWP, CORESET(s)? downlinkPreemption DownlinkPreemption OPTIONAL, -- Configuration of Slot-Format-Indicators to be monitored in this cell -- FFS: Can there be just one or multiple such configurations within a PDCCH-Config? How does it relate to BWP, CORESET(s)? slotFormatIndicator SlotFormatIndicator OPTIONAL, -- FFS: Is this timing information applicable to the entire PDCCH or could it be different per CORESET? -- FFS: Is there a default timing (to be used at least until first reconfiguration). Are the fields optionally present? timingtimeDomainResourceAllocation SEQUENCE { -- Configuration value of DL assignment to DL data timing dl-assignment-to-DL-data SEQUENCE { |  |
| H130 | timing | 3 | The current structure of configuration under timing is a bit confusing. We suggest to organize the UL, DL time resource allocation and K1 inside timing under “PUSCH-TimeDomain-Resource-Allocation”, “PDSCH-TimeDomain-Resource-Allocation” and “dl-data-to-UL-ACK”, respectively: |  |
| H131 | timing | 3 | The following agreement was made in the previous meeting in RAN 1: - One table for UL, one table for DL configured by RRC in R-15 - Each table is up to 16 rows - In each table, each row is configured by RRC with - K0 with 2 bits( for DL table), K2 with 3 bits ( for UL table) - An index (6 bits) into a table/equation in RAN 1 specs capturing valid combinations of start symbol and length (jointly decoded) - PDSCH mapping type A or B - The reference point for starting OFDM symbol - No RRC impact (e.g., slot boundary, start of CORESET where the PDCCH was found, or part of the table/equation in RAN 1 specs. FFS details) - Aggregation factor (1,2,4,8 for DL or UL) is semi-stattically configured separtely (i.e., not part of table) - No addtional RRC impact how to use the aggregation factor along with tables In clause 5.1.2.1, the time domian resource allocation for the downlink if specified as follows: When the UE is scheduled to receive PDSCH by a DCI, the Time-domain PDSCH resources field of the DCI provides a row index of an RRC configured table [pdsch-symbolAllocation], where the indexed row defines the slot offset K0, the start and length indicator SLIV, and the PDSCH mapping type to be assumed in the PDSCH reception. Similarly, in clause 6.1.2.1, time domian resource allocation for the uplink: When the UE is scheduled to transmit PUSCH by a DCI, the Time-domain PUSCH resources field of the DCI provides a row index of an RRC configured table [pusch-symbolAllocation], where the indexed row defines the slot offset K2, the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH reception. The current configuration is unable to reflect such agreements, both in the DL and UL. |  |

#### –	PDCP-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H132 | PDCP-Config=>t-reordering | 2 | FFS: whether ms0 is the same like outOfOrderDelivery FFS: new values for t-Reordering Ms0 in the configuration of t-Reordering is not the same as out-of-order delivery. As we have discussed in the meeting, by setting t-reordering to zero, the packets could have been received will be lost, as in R2-1707252 There is no current discussion on the new values for t-reordering The first FFS can be removed. |  |
| H133 | PDCP-Config=>moreThanOneRLC=> ul-DataSplitThreshold | 2 | This IE should only be present in case of split bearer. Hence, Cond should be added. moreThanOneRLC SEQUENCE { primaryPath SEQUENCE { cellGroup CellGroupId, logicalChannel LogicalChannelIdentity }, ul-DataSplitThreshold CHOICE { release NULL, setup ENUMERATED { b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} } OPTIONAL, -- Cond SplitBearer ul-Duplication BOOLEAN } OPTIONAL, -- Cond MoreThanOneRLC |  |
| H134 | PDCP-Config=>moreThanOneRLC=> ul-DataSplitThreshold | 2 | Release is not necessary here. For the link switching, we have made the agreement that the mode of link switching for UL split bearer shall be enabled by setting the threshold to infinity. moreThanOneRLC SEQUENCE { primaryPath SEQUENCE { cellGroup CellGroupId, logicalChannel LogicalChannelIdentity }, ul-DataSplitThreshold CHOICE { release NULL, setup ENUMERATED { b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1} }, ul-Duplication BOOLEAN } OPTIONAL, -- Cond MoreThanOneRLC |  |
| H135 | PDCP-Config=>drb=>headerCompression | 2 | For the field description, the two FFSs are: FFS: restrictions for split bearers FFS: restrictions on reconfigurations (e.g. only at reconfiguration involving PDCP re-establishment) RoHC should be configured for UM split bearer. ROHC should be configured at reconfiguration involving PDCP re-establsihment if the RB is previously configured with ROHC headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC should be configured for UM split bearer. ROHC should be configured at reconfiguration involving PDCP re-establishment if the RB is previously configured with ROHC. FFS: restrictions for split bearers FFS: restrictions on reconfigurations (e.g. only at reconfiguration involving PDCP re-establishment) |  |
| H136 | outOfOrderDelivery This parameter is only applicable for DRB | 1 | Conditional presence should be added for this parameter outOfOrderDelivery BOOLEAN OPTIONAL, -- Cond DRB |  |

#### –	PDSCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H137 | TCI-RS-SetId needs a value range | 2 | Per spreadsheet, it appears that the range should be (1..2) TCI-RS-SetId ::= INTEGER (0..ffsValue1..2) |  |
| H138 | rateMatchPatterns needs dynamic/semi-static selection | 2 | Per spreadsheet under ‘Resource-set-cell’, the rateMatchPattern structure should always be accompanied by a selection between dynamic and semi-static (the spreadsheet indicates it as a CHOICE but it appears sufficient to have an enumeration). The most expedient approach seems to be to include this field in RateMatchPattern. Alternative would be to define a new type for { RateMatchPattern, dynamic/semistatic }. RateMatchPattern ::= SEQUENCE { rateMatchPatternId RateMatchPatternId, patternType CHOICE { bitmaps SEQUENCE { -- A resource block level bitmap in the frequency domain. It indicates the PRBs to which the symbolsInResourceBlock bitmap applies. -- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap1' (see 38.214, section FFS_Section) -- FFS_ASN1: Consider multiple options with different number of bits (for narrower carriers) resourceBlocks BIT STRING (SIZE (275)), -- A symbol level bitmap in time domain. It indicates (FFS: with a bit set to true) the symbols which the UE shall rate match around. -- This pattern recurs (in time domain) with the configured periodicityAndOffset. -- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap2' (see 38.214, section FFS_Section) -- FFS: Why not split it into two BIT STRINGs of 14 bit each? symbolsInResourceBlock CHOICE { oneSlot BIT STRING (SIZE (14)), twoSlots BIT STRING (SIZE (28)) }, -- A time domain repetition pattern. It determines the periodicity (FFS: And offset???) at which the symbolsInResourceBlock -- pattern recurs. Absence of this field indicates the value n1, i.e., the symbolsInResourceBlock recurs every 14 symbols. -- Corresponds to L1 parameter 'rate-match-PDSCH-bitmap3' (see 38.214, section FFS_Section) -- FFS: Doesn’t one require also an offset to configure from where the repetitions start? periodicityAndOffset CHOICE { n5 INTEGER (0..4), n10 INTEGER (0..9), n20 INTEGER (0..19), n40 INTEGER (0..39) } OPTIONAL -- Need R }, -- This ControlResourceSet us used as a PDSCH rate matching pattern, i.e., PDSCH reception rate matches around it. controlResourceSet ControlResourceSetId }, mode ENUMERATED { dynamic, semiStatic } } |  |
| H139 | Type of xOverhead can be populated | 2 | RAN1 agreement is for { 0, 0.5, 1, 1.5 } * 12 -- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description. -- Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2) xOverhead FFS_ValueENUMERATED { xOh0, xOh6, xOh12, xOh18 } OPTIONAL, |  |
| H140 | nrofHARQ-ProcessesForPDSCH should have range 1..16 | 2 | RAN1 agreement that the maximum value is 16 -- The number of HARQ processes to be used on the PDSCH of a serving cell. -- Corresponds to L1 parameter 'number-HARQ-process-PDSCH' (see 38.214, section REF) nrofHARQ-processesForPDSCH ENUMERATED {ffsTypeAndValue}INTEGER (1..16), |  |
| H141 | epre-Ratio in Downlink-PTRS-Config needs its type | 2 | RAN1 agreement is 2 bits -- Parameters for configuration of downlink PTRS (see 38.211 section 7.4.1.2.2) Downlink-PTRS-Config ::= SEQUENCE { -- Presence and frequency density of DL PT-RS as a function of Scheduled BW -- Corresponds to L1 parameter 'DL-PTRS-frequency-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of DL PT-RS as a function of MCS -- Corresponds to L1 parameter 'DL-PTRS-time-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1. timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicates the number of DL PTRS ports. This is equal or smaller than the number of DMRS groups (related to PDSCH parameters -- dmrs-group1 and dmrs-group2). Corresponds to L1 parameter 'DL-PTRS-ports' (see 38.214, section 5.1) nrofPorts ENUMERATED {n1, n2}, -- EPRE ratio between PTRS and PDSCH. Corresponds to L1 parameter 'DL-PTRS-EPRE-ratio' (see 38.214, section 5.1) -- FFS: Whether there is one EPRE value per port (a comment in the L1 parameters hints that) epre-Ratio FFS_ValueINTEGER (0..3) OPTIONAL, -- Indicates the subcarrier offset for DL PTRS. Corresponds to L1 parameter 'DL-PTRS-RE-offset' (see 38.214, section 5.1) resourceElementOffset FFS_Value OPTIONAL } |  |
| H142 | resourceElementOffset in Downlink-PTRS-Config needs its type | 2 | RAN1 agreement is 2 bits -- Parameters for configuration of downlink PTRS (see 38.211 section 7.4.1.2.2) Downlink-PTRS-Config ::= SEQUENCE { -- Presence and frequency density of DL PT-RS as a function of Scheduled BW -- Corresponds to L1 parameter 'DL-PTRS-frequency-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of DL PT-RS as a function of MCS -- Corresponds to L1 parameter 'DL-PTRS-time-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1. timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicates the number of DL PTRS ports. This is equal or smaller than the number of DMRS groups (related to PDSCH parameters -- dmrs-group1 and dmrs-group2). Corresponds to L1 parameter 'DL-PTRS-ports' (see 38.214, section 5.1) nrofPorts ENUMERATED {n1, n2}, -- EPRE ratio between PTRS and PDSCH. Corresponds to L1 parameter 'DL-PTRS-EPRE-ratio' (see 38.214, section 5.1) -- FFS: Whether there is one EPRE value per port (a comment in the L1 parameters hints that) epre-Ratio FFS_Value OPTIONAL, -- Indicates the subcarrier offset for DL PTRS. Corresponds to L1 parameter 'DL-PTRS-RE-offset' (see 38.214, section 5.1) resourceElementOffset FFS_Value ENUMERATED { offset00, offset01, offset10, offset11 } OPTIONAL } |  |
| H143 | codeBlockGroupTransmission is not needed, and maxCodeBlockGroupsPerTransportBlock should be OPTIONAL | 2 | codeBlockGroupTransmission can be replaced by implicit indication from the presence of maxCodeBlockGroupsPerTransportBlock as indicated in the comments. The latter should be OPTIONAL. PDSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.213, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission BOOLEAN, -- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8} OPTIONAL, -- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see FFS_Specification, section FFS_Section) codeBlockGroupFlushIndicator BOOLEAN, |  |
| H144 | dmrs-Group2 has wrong range | 2 | Range should be (1000..1011) instead of (1000..1012) -- DM-RS groups that are QCL:ed, i.e. group 1 (see 38.214, section 5.1) -- FFS CHECK: Clarify how to configure the DMRS groups and the relation to TCI. -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group1 INTEGER (1000..1011), -- DM-RS groups that are QCL:ed, i.e. group 2 (see 38.214, section 5.1) -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group2 INTEGER (1000..10121), |  |
| H145 | 16 bits is correct for scramblingId (and it needs a need code) | 2 | Per RAN1 agreements, 16 bits is correct for this scramblingId and the FFS can be removed. -- DL DMRS scrambling initalization -- Corresponds to L1 parameter 'DL-DMRS-Scrambling-ID' (see 38.214, section 5.1) -- When the field is absent the UE applies the value "Physical cell ID + 6 fixed bits (e.g. 000000)" -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS_CHECK: Is it really 16 bit whereas all other scrambling IDs are just 10 bit? If this is also 10, replace by type ScramblingId scramblingID BIT STRING (SIZE (16)) OPTIONAL -- Need S |  |
| H146 | resourceAllocation can be replaced by an enum type | 2 | Per RAN1 agreements, we understand the current structure is correct i.e. the CHOICE can be replaced by an enum. -- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI -- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 5.1.2) -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? resourceAllocation CHOICE { resourceAllocationType0 NULL, resourceAllocationType1 NULL, dynamicSwitch NULL }ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch } OPTIONAL, |  |
| H147 | rbg-Size | 2 | This parameter is per BWP and can be moved into DownlinkBandwidthPart. The excerpt below shows how this could be done for this field only, but it may be preferable to have the whole PDSCH-Config provided per BWP. Needs to be discussed as a general issue. PDSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.213, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission BOOLEAN, -- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, -- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see FFS_Specification, section FFS_Section) codeBlockGroupFlushIndicator BOOLEAN, dmrs-Downlink SEQUENCE { -- Selection of the DMRS type to be used for DL (see 38.211, section 7.4.1.1.1) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. -- CHECK: Listed in RAN1 table. But should this really be in dedicated signalling? dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- DM-RS groups that are QCL:ed, i.e. group 1 (see 38.214, section 5.1) -- FFS CHECK: Clarify how to configure the DMRS groups and the relation to TCI. -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group1 INTEGER (1000..1011), -- DM-RS groups that are QCL:ed, i.e. group 2 (see 38.214, section 5.1) -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group2 INTEGER (1000..1012), -- The maximum number of OFDM symbols for DL front loaded DMRS -- Corresponds to L1 parameter 'DL-DMRS-max-len' (see 38.214, section 5.1) maxLength ENUMERATED {len1, len2}, -- DL DMRS scrambling initalization -- Corresponds to L1 parameter 'DL-DMRS-Scrambling-ID' (see 38.214, section 5.1) -- When the field is absent the UE applies the value "Physical cell ID + 6 fixed bits (e.g. 000000)" -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS_CHECK: Is it really 16 bit whereas all other scrambling IDs are just 10 bit? If this is also 10, replace by type ScramblingId scramblingID BIT STRING (SIZE (16)) OPTIONAL }, -- Configures downlink PTRS . -- If absent of released, the UE assumes that downlink PTRS are not present. See 38.211 section 7.4.1.2.2 phaseTracking-RS SetupRelease { Downlink-PTRS-Config } OPTIONAL, -- Need M -- Contains Transmission Configuration Indicator (TCI) states for dynamically indicating (over DCI) a transmission configuration -- which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports -- (see 38.214, section 5.1.4) tci-States SEQUENCE (SIZE(1..maxNrof-TCI-RS-Sets)) OF TCI-RS-Set, -- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description. -- Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2) xOverhead FFS_Value OPTIONAL, -- Interleaving unit configurable between 2 and 4 PRBs -- Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section FFS_Section) vrb-to-PRB-Interleaver ENUMERATED {n2, n4} OPTIONAL, -- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI -- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 5.1.2) -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? resourceAllocation CHOICE { resourceAllocationType0 NULL, resourceAllocationType1 NULL, dynamicSwitch NULL } OPTIONAL, -- Indexes to possible symbol allcoations (the indexes point to entries in a UE specific table) -- Corresponds to L1 parameter 'PDSCH-symbol-allocation' (see 38.214, section 5.1.2.1) pdsch-symbolAllocation FFS_Value OPTIONAL, -- Resources that the UE should rate match PDSCH around. rateMatchResourcesPDSCH SEQUENCE { -- Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources -- indicated in the nexted bitmaps. Corresponds to L1 parameter 'rate-match-PDSCH-resource-set' (see 38.214, section 5.1.2.2.3) -- FFS_ASN1: Consider replacing by AddMod/Release lists -- FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary. -- So far it is unclear whether or not the entire PDSCH-Config moves into the BWP configuration. rateMatchPatterns SetupRelease { SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern } OPTIONAL, -- Need M lte-CRS-ToMatchAround SetupRelease { SEQUENCE { -- Center of the LTE carrier. Corresponds to L1 parameter 'center-subcarrier-location' (see 38.214, section 5.1.4) carrierFreqDL INTEGER (0..maxEARFCN), -- BW of the LTE carrier in numbewr of PRBs. Corresponds to L1 parameter 'BW' (see 38.214, section 5.1.4) carrierBandwidthDL ENUMERATED {n6, n15, n25, n50, n75, n100, spare1}, -- LTE MBSFN subframe configuration. Corresponds to L1 parameter 'MBSFN-subframconfig' (see 38.214, section 5.1.4) -- FFS_ASN1: Import the LTE MBSFN-SubframeConfigList mbsfn-SubframeConfigList OCTET STRING (CONTAINING MBSFN-SubframeConfigList) OPTIONAL, -- Number of LTE CRS antenna port to rate-match around. -- Corresponds to L1 parameter 'rate-match-resources-numb-LTE-CRS-antenna-port' (see 38.214, section 5.1.4) nrofCRS-Ports ENUMERATED {n1, n2, n4}, -- Shifting value v-shift in LTE to rate match around LTE CRS -- Corresponds to L1 parameter 'rate-match-resources-LTE-CRS-v-shift' (see 38.214, section 5.1.4) v-Shift ENUMERATED {n0, n1, n2, n3, n4, n5} } } OPTIONAL -- Need M }, -- Selection between config 1 and config 2 for RBG size for PDSCH. Corresponds to L1 parameter 'RBG-size-PDSCH' (see 38.214, section 5.1.2.2.1) rbg-Size ENUMERATED {config1, config2}, -- Indicates which MCS table the UE shall use for PDSCH. Corresponds to L1 parameter 'MCS-Table-PDSCH' (see 38.214, section 5.1.3.1). mcs-Table ENUMERATED {qam64, qam256}, -- Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2. maxNrofCodeWordsScheduledByDCI ENUMERATED {n1, n2} OPTIONAL, -- Need R -- The number of HARQ processes to be used on the PDSCH of a serving cell. -- Corresponds to L1 parameter 'number-HARQ-process-PDSCH' (see 38.214, section REF) nrofHARQ-processesForPDSCH ENUMERATED {ffsTypeAndValue}, -- HARQ-ACK codebook is configured to be either semi-static of dynamic. This is applicable to both CA and none CA operation -- Corresponds to L1 parameter 'HARQ-ACK-codebook' (see 38.213, section FFS_Section) harq-ACK-Codebook ENUMERATED {semiStatic, dynamic}, -- Bundle size the UE may assume. Corresponds to L1 paramter 'PDSCH-bundle-size' (see 38.211, section 7.3.1.5). -- If the field is absent, the UE assumes value n2. -- FFS: Better description pdsch-BundleSize ENUMERATED {n2, n4, wideband} OPTIONAL, -- If set to true, the network indicates the PRB bundle size dynamically via DCI. Corresponds to L1 parameter 'PRB_bundling' -- (see 38.214, section 5.1.2.3) prbBundlingEnabled BOOLEAN, -- A list of Zero-Power (ZP) CSI-RS resources. -- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigList' (see 38.214, section FFS_Section) zp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource OPTIONAL, ... } [...] DownlinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, pdcch-ConfigCommon PDCCH-ConfigCommon OPTIONAL, -- Selection between config 1 and config 2 for RBG size for PDSCH. Corresponds to L1 parameter 'RBG-size-PDSCH' (see 38.214, section 5.1.2.2.1) rbg-Size ENUMERATED {config1, config2} } |  |
| H148 | ZP-CSI-RS parameters should not be OPTIONAL | 2 | The ZP-CSI-RS-Resources list itself is OPTIONAL inside PDSCH-Config, but if it is present, the parameters freqBand, density, resourceType, and qcl-Type should always be present. -- A Zero-Power (ZP) CSI-RS resource configuration. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfig' (see 38.214, section FFS_Section) ZP-CSI-RS-Resource ::= SEQUENCE { -- ZP CSI-RS resource configuration ID -- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigId' (see 38.214, section FFS_Section) zp-CSI-RS-ResourceId ZP-CSI-RS-ResourceId, -- OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot -- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceMapping' (see 38.214, section FFS_Section) resourceMapping SEQUENCE { -- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1. FFS: Table correct? -- The number of bits that may be set to one depend on the chosen row in that table. frequencyDomainAllocation CHOICE { row1 BIT STRING (SIZE (4)), row2 BIT STRING (SIZE (12)), row4 BIT STRING (SIZE (3)), other BIT STRING (SIZE (6)) }, -- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS. firstOFDMSymbolInTimeDomain INTEGER (0..13) } OPTIONAL, -- Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS -- Corresponds to L1 parameter 'ZP-CSI-RS-timeConfig' (see 38.214, section FFS_Section) periodicityAndOffset CHOICE { sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) } OPTIONAL, -- Includes parameters to enbale configuration of frequency-occupancy of ZP-CSI)RS -- Corresponds to L1 parameter 'ZP-CSI-RS-FreqBand' (see 38.214, section FFS_Section) freqBand SEQUENCE { -- PRB where this NZP-CSI-RS-Resource starts in relation to PRB 0 of the associated BWP. Only multiples of 4 are allowed (0, 4, ...) startingRB INTEGER (0..maxNrofPhysicalResourceBlocks-1), -- Number of PRBs across which this NZP-CSI-RS-Resource spans. Only multiples of 4 are allowed. The smallest configurable -- number is the minimum of 24 and the width of the associated BWP. nrofRBs INTEGER (24..maxNrofPhysicalResourceBlocks) } OPTIONAL, -- Density of ZP-CSI-RS resource measured in RE/port/PRB. -- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1, -- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32, -- value 1 (one) is allowed for X=4, 8, 12. -- For density = 1/2, includes 1 bit indication for RB level comb offset indicating whether odd or even RBs are occupied by CSI-RS -- Corresponds to L1 parameter 'ZP-CSI-RS-Density' (see 38.214, section FFS_Section) density CHOICE { dot5 ENUMERATED {evenPRBs, oddPRBs}, one NULL, three NULL, spare NULL } OPTIONAL, -- Time domain behavior of ZP-CSI-RS resource configuration. -- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigType' (see 38.214, section FFS_Section) resourceType ENUMERATED {aperiodic, periodic} OPTIONAL, -- QCL type for source RS ==> target RS association. Corresponds to L1 parameter 'QCL-Type' (see 38.214, section FFS_Section) qcl-Type ENUMERATED {typeA, typeB, typeC, typeD} OPTIONAL } |  |
| H149 | The checkpoint “CHECK: Listed in RAN1 table. But should this really be in dedicated signalling?” is confirmed. The configuration should be dedicated signaling. | 2 | Remove this checkpoint. PDSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.213, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission BOOLEAN, -- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, -- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see FFS_Specification, section FFS_Section) codeBlockGroupFlushIndicator BOOLEAN, dmrs-Downlink SEQUENCE { -- Selection of the DMRS type to be used for DL (see 38.211, section 7.4.1.1.1) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. -- CHECK: Listed in RAN1 table. But should this really be in dedicated signalling? dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R |  |
| H150 | For the check point “FFS CHECK: Clarify how to configure the DMRS groups and the relation to TCI”, DMRS can be included in TCI-RS-Set similar to CSI-RS and SSB. | 2 | Remove the check note, and update TCI-RS-Set as below: RAN1 input needed. It’s not clear from the description what fields need to be included in TCI-RS-Set (I can’t find where DMRS is included for CSI-RS and SSB as the description indicates). TCI-RS-Set ::= SEQUENCE { tci-RS-SetId TCI-RS-SetId, qcl-Type1 SEQUENCE { referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceConfigId, ssb SSB-Id, -- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a CSI-ResourceSetId trs CSI-ResourceSetId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD} }, qcl-Type2 SEQUENCE { referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceConfigId, ssb SSB-Id, -- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a CSI-ResourceSetId trs CSI-ResourceSetId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD} } OPTIONAL } |  |
| H151 | The check point “FFS_Value: Check whether these are really meant to be these few intergers” is confirmed. They are really meant to be integers with these ranges as indicated by the RAN1 spreadsheet. | 2 | Remove this checkpoint. PDSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.213, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission BOOLEAN, -- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, -- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see FFS_Specification, section FFS_Section) codeBlockGroupFlushIndicator BOOLEAN, dmrs-Downlink SEQUENCE { -- Selection of the DMRS type to be used for DL (see 38.211, section 7.4.1.1.1) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. -- CHECK: Listed in RAN1 table. But should this really be in dedicated signalling? dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- DM-RS groups that are QCL:ed, i.e. group 1 (see 38.214, section 5.1) -- FFS CHECK: Clarify how to configure the DMRS groups and the relation to TCI. -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group1 INTEGER (1000..1011), -- DM-RS groups that are QCL:ed, i.e. group 2 (see 38.214, section 5.1) -- FFS_Value: Check whether these are really meant to be these few intergers dmrs-group2 INTEGER (1000..1012), |  |
| H152 | For DL DMRS scrambling, two scrambing IDs should be configured. (Based on our consultation with RAN1 colleagues, this was the intention of the spreadsheet description “Two ids are possible to configure”, which was mistakenly attached to the PDSCH and PUSCH scrambling ID parameters instead.) | 2 | -- DL DMRS scrambling initalization -- Corresponds to L1 parameter 'DL-DMRS-Scrambling-ID' (see 38.214, section 5.1) -- When the field is absent the UE applies the value "Physical cell ID + 6 fixed bits (e.g. 000000)" -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS_CHECK: Is it really 16 bit whereas all other scrambling IDs are just 10 bit? If this is also 10, replace by type ScramblingId scramblingID1 BIT STRING (SIZE (16)) OPTIONAL, scramblingID2 BIT STRING (SIZE (16)) OPTIONAL -- Need S |  |
| H153 | For “-- Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section FFS_Section)”, the FFS_Section should be Section 6.3.1.6. | 2 | -- Interleaving unit configurable between 2 and 4 PRBs -- Corresponds to L1 parameter 'VRB-to-PRB-interleaver' (see 38.211, section FFS_Section6.3.1.6) vrb-to-PRB-Interleaver ENUMERATED {n2, n4} OPTIONAL, |  |
| H154 | For “FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary”, as per RAN1 agreements, there should be up to 4 patterns per BWP plus 4 patterns per cell. | 3 | Discuss together with the issue which parameters in PDSCH-config should be per BWP, and how to split the configuration. |  |
| H155 | Not clear why the field zp-CSI-RS-Resources is configured under PDSCH-Config. | 2 | Remove the field zp-CSI-RS-Resources. RAN1 input needed. If this is removed, the ZP-CSI-RS-Resources will not be used anywhere; if we propose to remove it, we need to propose where to put it instead! |  |
| H156 | The configuration in TCI-RS-Set should include a ID, two DL RSs and a QCL type. Furthermore, DMRS should be included as a DL RS. | 2 | RAN1 input needed to confirm that this is correct Change the definition of TCI-RS-Set from TCI-RS-Set ::= SEQUENCE { tci-RS-SetId TCI-RS-SetId, qcl-Type1 SEQUENCE { referenceSignal1 CHOICE { csi-rs NZP-CSI-RS-ResourceConfigId, ssb SSB-Id, -- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a CSI-ResourceSetId trs CSI-ResourceSetId, dmrs FFS-Type }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD} }, qcl-Type2 SEQUENCE { referenceSignal2 CHOICE { csi-rs NZP-CSI-RS-ResourceConfigId, ssb SSB-Id, -- A TRS (Tracking Reference Signal) configuration represented as a set of CSI-RS-Resources in a CSI-ResourceSetId trs CSI-ResourceSetId, dmrs FFS-Type }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD} } OPTIONAL } |  |
| H157 | In the description “Parameters for configuration of downlink PTRS (see 38.211 section 7.4.1.2.2)”, it is better to refer 38.214: section 5.1.6.3 | 2 | Change to (see 38.214 section 5.1.6.3). -- Parameters for configuration of downlink PTRS (see 38.2114 section 7.4.1.2.25.1.6.3) Downlink-PTRS-Config ::= SEQUENCE { |  |
| H158 | For the following two FFS -- FFS: To be Configured per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue} -- FFS: To be Configured per BWP according to RAN1. timeDensity ENUMERATED {ffsTypeAndValue}, it is confirmed that they should be configured per BWP. | 3 | Discuss together with the issue which parameters in PDSCH-config should be per BWP, and how to split the configuration. |  |
| H159 | Regarding “FFS: Whether there is one EPRE value per port (a comment in the L1 parameters hints that)”, one EPRE value per port is confirmed. | 2 | Confirm that it is one EPRE value per port. Downlink-PTRS-Config ::= SEQUENCE { -- Presence and frequency density of DL PT-RS as a function of Scheduled BW -- Corresponds to L1 parameter 'DL-PTRS-frequency-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of DL PT-RS as a function of MCS -- Corresponds to L1 parameter 'DL-PTRS-time-density-table' (see 38.214, section 5.1) -- FFS: To be Configured per BWP according to RAN1. timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicates the number of DL PTRS ports. This is equal or smaller than the number of DMRS groups (related to PDSCH parameters -- dmrs-group1 and dmrs-group2). Corresponds to L1 parameter 'DL-PTRS-ports' (see 38.214, section 5.1) nrofPorts ENUMERATED {n1, n2}, -- EPRE ratio between PTRS and PDSCH. Corresponds to L1 parameter 'DL-PTRS-EPRE-ratio' (see 38.214, section 5.1) -- FFS: Whether there is one EPRE value per port (a comment in the L1 parameters hints that) epre-Ratio FFS_Value OPTIONAL, -- Indicates the subcarrier offset for DL PTRS. Corresponds to L1 parameter 'DL-PTRS-RE-offset' (see 38.214, section 5.1) resourceElementOffset FFS_Value OPTIONAL } |  |

#### –	PhysCellId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	PUCCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H160 | nrOfSymbols should be in PUCCH format specific structures | 2 | Currently nrOfSymbols appears in PUCCH-Resource with a range of 1..2. This is correct for formats 0 and 2, but formats 1, 3, and 4 have a different range 4..14. Need to break this field out into PUCCH-format0/1/2/3/4 with the appropriate ranges in each place. Changes combined with the issue below |  |
| H161 | startingSymbol should be in PUCCH format specific structures | 2 | Currently startingSymbol appears in PUCCH-Resource with a range of 0..13. This is correct for formats 0 and 2, but formats 1, 3, and 4 have a different range 0..10. Need to break this field out into PUCCH-format0/1/2/3/4 with the appropriate ranges in each place. PUCCH-Resource ::= SEQUENCE { pucch-ResourceId PUCCH-ResourceId, startingPRB PRB-Id, startingSymbolIndex INTEGER(0..13), nrofSymbols ENUMERATED {n1, n2}, -- Index of starting PRB for second hop of PUCCH in case of FH. This value is appliable for intra-slot frequency hopping. -- Corresponds to L1 parameter 'PUCCH-2nd-hop-PRB' (see 38.213, section 9.2) secondHopPRB PRB-Id OPTIONAL, -- Selection of the PUCCH format and format-specific parameters format CHOICE { format0 PUCCH-format0, -- Cond InFirstSetOnly format1 PUCCH-format1, -- Cond InFirstSetOnly format2 PUCCH-format2, -- Cond NotInFirstSet format3 PUCCH-format3, -- Cond NotInFirstSet format4 PUCCH-format4 -- Cond NotInFirstSet } } PUCCH-ResourceId ::= INTEGER (0..maxNrofPUCCH-ResourcesPerSet-1) -- A PUCCH Format 0 resource configuration (see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F0-resource-config' (see 38.213, section 9.2) PUCCH-format0 ::= SEQUENCE { intraSlotfrequencyHopping BOOLEAN, initialCyclicShift INTEGER(0..11), startingSymbolIndex INTEGER (0..13), nrOfSymbols INTEGER (1..2) } -- A PUCCH Format 1 resource configuration (see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F1-resource-config' (see 38.213, section 9.2) PUCCH-format1 ::= SEQUENCE { intraSlotfrequencyHopping BOOLEAN, initialCyclicShift INTEGER(0..11), timeDomainOCC INTEGER(0..6) , startingSymbolIndex INTEGER (0..10), nrOfSymbols INTEGER (4..14) } -- A PUCCH Format 2 resource configuration (see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F2-resource-config' (see 38.213, section 9.2) PUCCH-format2 ::= SEQUENCE { nrofPRBs INTEGER (1..16), intraSlotfrequencyHopping BOOLEAN, startingSymbolIndex INTEGER(0..13), nrOfSymbols INTEGER (1..2) } -- A PUCCH Format 3 resource configuration(see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F3-resource-config' (see 38.213, section 9.2) PUCCH-format3 ::= SEQUENCE { nrofPRBs INTEGER (1..16), intraSlotfrequencyHopping BOOLEAN, startingSymbolIndex INTEGER(0..10) , nrOfSymbols INTEGER (4..14) } -- A PUCCH Format 4 resource configuration (see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F4-resource-config' (see 38.213, section 9.2) PUCCH-format4 ::= SEQUENCE { frequencyHopping BOOLEAN, occ-Length ENUMERATED {n2,n4}, occ-Index ENUMERATED {n0,n1,n2,n3}, startingSymbolIndex INTEGER(0..10) , nrOfSymbols INTEGER (4..14) } |  |
| H162 | Repeated enumeration for max code rates | 2 | In format2, format3, and format4, maxCodeRate uses the same enumeration. Suggest breaking it out into a separate type. PUCCH-Config ::= SEQUENCE { -- PUCCH resource sets (see 38.213 9.2) resourceSets SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet OPTIONAL, format1 SetupRelease { SEQUENCE { -- Enabling inter-slot frequency hopping when PUCCH Format 1 is repetead over multiple slots. interslotFrequencyHopping ENUMERATED {enabled} OPTIONAL, -- Number of slots with the same PUCCH F1. When the field is absent the UE applies the value n1. -- Corresponds to L1 parameter 'PUCCH-F1-number-of-slots' (see 38.213, section 9.2) -- FFS_Value: Undefined values y1-y3 in range! nrofSlots ENUMERATED {n1,ny1,y2,y3} OPTIONAL } } OPTIONAL, -- Need M format2 SetupRelease { SEQUENCE { -- Maximum coding rate to determine how to feedback UCI on PUCCH Format 2. -- Corresponds to L1 parameter 'PUCCH-F2-maximum-coderate' (see 38.213, section 9.2) maxCodeRate ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80}MaxCodeRatePUCCH OPTIONAL, -- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format2 -- Corresponds to L1 parameter 'PUCCH-F2-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2) -- When the field is absent the UE applies the value OFF simultaneousHARQ-ACK-CSI ENUMERATED {true} OPTIONAL -- Need R } } OPTIONAL, -- Need M format3 SetupRelease { SEQUENCE { -- Enabling inter-slot frequency hopping when PUCCH Format 3 is repetead over multiple slots. interslotFrequencyHopping ENUMERATED {enabled} OPTIONAL, -- Enabling 2 DMRS symbols per hop of a PUCCH Format 3 if both hops are more than X symbols when FH is enabled (X=4). -- Enabling 4 DMRS sybmols for a PUCCH Format 3 with more than 2X+1 symbols when FH is disabled (X=4). -- Corresponds to L1 parameter 'PUCCH-F3-F4-additional-DMRS' (see 38.21X, section FFS_Section) additionalDMRS ENUMERATED {true} OPTIONAL, -- Need R -- Max coding rate to determine how to feedback UCI on PUCCH Format 3 -- Corresponds to L1 parameter 'PUCCH-F3-maximum-coderate' (see 38.213, section 9.2) maxCodeRate ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80} MaxCodeRatePUCCH OPTIONAL, -- Number of slots with the same PUCCH F3. When the field is absent the UE applies the value n1. -- Corresponds to L1 parameter 'PUCCH-F3-number-of-slots' (see 38.213, section 9.2) -- FFS_Value: Undefined values y1-y3 in range! nrofSlots ENUMERATED {n1,y1,y2,y3} OPTIONAL, -- Enabling pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. -- Corresponds to L1 parameter 'PUCCH-PF3-PF4-pi/2PBSK' (see 38.21X, section FFS_Section) pi2PBSK ENUMERATED {enabled} OPTIONAL, -- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format3 -- Corresponds to L1 parameter 'PUCCH-F3-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2) -- When the field is absent the UE applies the value OFF simultaneousHARQ-ACK-CSI ENUMERATED {true} OPTIONAL -- Need R } } OPTIONAL, -- Need M format4 SetupRelease { SEQUENCE { -- Enabling inter-slot frequency hopping when PUCCH Format 4 is repetead over multiple slots. interslotFrequencyHopping ENUMERATED {enabled} OPTIONAL, -- Enabling 2 DMRS symbols per hop of a PUCCH Format 4 if both hops are more than X symbols when FH is enabled (X=4). -- Enabling 4 DMRS sybmols for a PUCCH Format 4 with more than 2X+1 symbols when FH is disabled (X=4). -- Corresponds to L1 parameter 'PUCCH-F3-F4-additional-DMRS' (see 38.21X, section FFS_Section) additionalDMRS ENUMERATED {true} OPTIONAL, -- Need R -- Max coding rate to determine how to feedback UCI on PUCCH Format 4 -- Corresponds to L1 parameter 'PUCCH-F4-maximum-coderate' (see 38.213, section 9.2) maxCodeRate ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80} MaxCodeRatePUCCH OPTIONAL, -- Number of slots with the same PUCCH F4. When the field is absent the UE applies the value n1. -- Corresponds to L1 parameter 'PUCCH-F4-number-of-slots' (see 38.213, section 9.2) -- FFS_Value: Undefined values y1-y3 in range! nrofSlots ENUMERATED {n1,y1,y2,y3} OPTIONAL, -- Enabling pi/2 BPSK for UCI symbols instead of QPSK for PUCCH. -- Corresponds to L1 parameter 'PUCCH-PF3-PF4-pi/2PBSK' (see 38.21X, section FFS_Section) pi2PBSK ENUMERATED {enabled} OPTIONAL, -- Enabling simultaneous transmission of CSI and HARQ-ACK feedback with or without SR with PUCCH Format4 -- Corresponds to L1 parameter 'PUCCH-F4-Simultaneous-HARQ-ACK-CSI' (see 38.213, section 9.2) -- When the field is absent the UE applies the value OFF simultaneousHARQ-ACK-CSI ENUMERATED {true} OPTIONAL -- Need R } } OPTIONAL, -- Need M [...] MaxCodeRatePUCCH ::= ENUMERATED {zeroDot08, zeroDot15, zeroDot25, zeroDot35, zeroDot45, zeroDot60, zeroDot80} |  |
| H163 | In PUCCH-format3, nrofPRBs has a non-ideal range | 2 | The type is INTEGER (1..16) when it could be ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n9, n10, n12, n15, n16 }. -- A PUCCH Format 3 resource configuration(see 38.213, section 9.2) -- Corresponds to L1 parameter 'PUCCH-F3-resource-config' (see 38.213, section 9.2) PUCCH-format3 ::= SEQUENCE { nrofPRBs INTEGER (1..16)ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n9, n10, n12, n15, n16 }, intraSlotfrequencyHopping BOOLEAN } |  |
| H164 | PUCCH formats 3 and 4 are missing their sequence hopping indication | 2 | RAN1 agreements are that slot-level base sequence hopping is supported for PUCCH formats 0, 1, 3, and 4. The corresponding flag is present for formats 0 and 1 but not for 3 and 4. PUCCH-ConfigCommon ::= SEQUENCE { -- PUCCH resource configuration for HARQ-ACK before RRC connection setup -- Corresponds to L1 parameter 'PUCCH-resource-common' (see 38.213, section 9.2) -- FFS_Value: RAN1 to provide more details on the value range pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, -- Enables hopping of base sequence of PUCCH Format 0 when transmitted in different slots. -- Corresponds to L1 parameter 'PUCCH-F0-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format0-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Enabling hopping of base sequence of PUCCH Format 1 when transmitted in different slots -- Corresponds to L1 parameter 'PUCCH-F1-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format1-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Enabling hopping of base sequence of PUCCH Format 1 when transmitted in different slots -- Corresponds to L1 parameter 'PUCCH-F3-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format3-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Enabling hopping of base sequence of PUCCH Format 1 when transmitted in different slots -- Corresponds to L1 parameter 'PUCCH-F4-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format4-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Cell-Specific scrambling ID for Group hoppping and sequence hopping if enabled. -- Corresponds to L1 parameter 'ScramblingID' (see 38.211, section 6.4.1.3) sequenceHoppingId BIT STRING (SIZE (10)) OPTIONAL, -- Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pucch' (see 38.213, section 7.2) p0-nominal INTEGER (-202..24) OPTIONAL, -- deltaF for PUCCH format 0 (see 38.213, section 7.2) deltaF-pucch-f0 FFS_Value OPTIONAL, -- deltaF for PUCCH format 1 (see 38.213, section 7.2) deltaF-pucch-f1 FFS_Value OPTIONAL, -- deltaF for PUCCH format 2 (see 38.213, section 7.2) deltaF-pucch-f2 FFS_Value OPTIONAL, -- deltaF for PUCCH format 3 (see 38.213, section 7.2) deltaF-pucch-f3 FFS_Value OPTIONAL, -- deltaF for PUCCH format 4 (see 38.213, section 7.2) deltaF-pucch-f4 FFS_Value OPTIONAL } |  |
| H165 | The Configurable ID for cyclic shift hopping is now missing. | 2 | Now there is only sequenceHoppingId in the current PUCCH-Config. In addition to that, however, it was also agreed in RAN1 that there should also be the Cyclic Shift hopping ID, for all of the PUCCH formats 0, 1, 3 and 4, as follows. So perhaps a cyclicShiftHoppingId should be added as per the following RAN1 agreement: - For PUCCH format 0 & PUCCH format 1, symbol-level cyclic shift hopping as in LTE is reused - FFS details especially regarding symbol indexing and slot indexing considering the difference between NR and LTE - Symbol-level cyclic shift hopping is a function at least based on a configurable ID - Note that: the configurable ID is already agreed for PUCCH base sequence hopping - For PUCCH format 3 & PUCCH format 4, symbol-level cyclic shift hopping as in LTE is reused - FFS details especially regarding symbol indexing and slot indexing considering the difference between NR and LTE - Symbol-level cyclic shift hopping is a function at least based on a configurable ID - Note that the configurable ID that is already agreed for PUCCH base sequence hopping is reused. PUCCH-ConfigCommon ::= SEQUENCE { -- PUCCH resource configuration for HARQ-ACK before RRC connection setup -- Corresponds to L1 parameter 'PUCCH-resource-common' (see 38.213, section 9.2) -- FFS_Value: RAN1 to provide more details on the value range pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, -- Enables hopping of base sequence of PUCCH Format 0 when transmitted in different slots. -- Corresponds to L1 parameter 'PUCCH-F0-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format0-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Enabling hopping of base sequence of PUCCH Format 1 when transmitted in different slots -- Corresponds to L1 parameter 'PUCCH-F1-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format1-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Corresponds to L1 parameter 'ScramblingID' (see 38.211, section 6.4.1.3) sequenceHoppingId BIT STRING (SIZE (10)) OPTIONAL, cyclicShiftHoppingId BIT STRING (SIZE (10)) OPTIONAL, -- Power control parameter P0 for PUCCH transmissions. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pucch' (see 38.213, section 7.2) p0-nominal INTEGER (-202..24) OPTIONAL, -- deltaF for PUCCH format 0 (see 38.213, section 7.2) deltaF-pucch-f0 FFS_Value OPTIONAL, -- deltaF for PUCCH format 1 (see 38.213, section 7.2) deltaF-pucch-f1 FFS_Value OPTIONAL, -- deltaF for PUCCH format 2 (see 38.213, section 7.2) deltaF-pucch-f2 FFS_Value OPTIONAL, -- deltaF for PUCCH format 3 (see 38.213, section 7.2) deltaF-pucch-f3 FFS_Value OPTIONAL, -- deltaF for PUCCH format 4 (see 38.213, section 7.2) deltaF-pucch-f4 FFS_Value OPTIONAL } |  |
| H166 | The enable/disable of base sequence hopping for PUCCH format 3/4 are missing. | 2 | As per RAN1 agreement below, the enable/disable of base sequence hopping should not only be applied to PUCCH formats 0 and 1, but also be applied to PUCCH formats 3 and 4. However, those for formats 3 and 4 are now missing, and should be added similar to formats 0 and 1. - For PUCCH formats 0, 1, 3 & 4, slot-level base sequence hopping as in LTE is reused - FFS details especially regarding slot indexing considering the difference between NR and LTE - Slot-level base sequence hopping for PUCCH format 3 and 4 is a function at least based on a configurable ID, similarly as agreed for PUCCH format 0 and 1. - Note that: the configurable ID is already agreed for PUCCH base sequence hopping - FFS: whether and how to support symbol-level base sequence hopping An exemplary change is as follows: PUCCH-ConfigCommon ::= SEQUENCE { -- PUCCH resource configuration for HARQ-ACK before RRC connection setup -- Corresponds to L1 parameter 'PUCCH-resource-common' (see 38.213, section 9.2) -- FFS_Value: RAN1 to provide more details on the value range pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, -- Enables hopping of base sequence of PUCCH Format 0 when transmitted in different slots. -- Corresponds to L1 parameter 'PUCCH-F0-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format0-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Enabling hopping of base sequence of PUCCH Format 1 when transmitted in different slots -- Corresponds to L1 parameter 'PUCCH-F1-Base-sequence-hopping' (see 38.211, section 6.4.1.3) pucch-Format1-BaseSequenceHopping ENUMERATED {true} OPTIONAL, pucch-Format3-BaseSequenceHopping ENUMERATED {true} OPTIONAL, pucch-Format4-BaseSequenceHopping ENUMERATED {true} OPTIONAL, -- Cell-Specific scrambling ID for Group hoppping and sequence hopping if enabled. -- Corresponds to L1 parameter 'ScramblingID' (see 38.211, section 6.4.1.3) sequenceHoppingId BIT STRING (SIZE (10)) OPTIONAL, […] } |  |

#### –	PUSCH-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H167 | srs-MappingNonCodebook has a misplaced close brace | 2 | The brace mistakenly appears further down, after dft-S-OFDM. Delete one of the three close braces after dft-S-OFDM and use it to close srs-MappingNonCodebook. Uplink-PTRS-Config ::= SEQUENCE { -- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices -- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1) -- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM -- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts? -- And if so, should this structure be conditional to the nrofPorts being set to n2? srs-MappingNonCodebook SEQUENCE { srs CHOICE { resource SRS-ResourceId, resourceSet SRS-ResourceSetId }, ptrs-PortIndex ENUMERATED {n1, n2} OPTIONAL, } -- Configuration of UL PTRS for CP-OFDM cp-OFDM SetupRelease { SEQUENCE { -- Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW -- Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS -- Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicator related to the number of UL PTRS ports for CP-OFDM. Details to be further decided. -- Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.1) nrofPorts ENUMERATED {n1, n2}, -- Indicates the subcarrier offset for UL PTRS for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1) resourceElementOffset FFS_Value OPTIONAL, -- UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1) ptrs-Power ENUMERATED {f1, f2, f3, f4} } } OPTIONAL, -- Cond M -- Configuration of UL PTRS for DFT-S-OFDM. dft-S-OFDM SetupRelease { SEQUENCE { -- Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4}, -- that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should -- use depending on the scheduled BW according to the table in 38.214 FFS_Section. -- FFS: Configuration is supposed to be per BWP according to RAN1 -- Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1) sampleDensity FFS_Value OPTIONAL, -- Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM -- Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1) timeDensity ENUMERATED {d1, d2} OPTIONAL } } } OPTIONAL -- Cond M } |  |
| H168 | p0-NominalWithoutGrant | 2 | In the RRC parameters list in R1-1721581, p0-NominalWithoutGrant is UE-specific. However according to RAN1 agreement in RAN1#90BIS:” • Value of P_0 is composed by cell specific component and UE specific component; • At least three cell specific component values of P_0 can be configured”. The p0-NominalWithoutGrant should be cell specific parameter (i.e. the description on p0-NominalWithoutGrant in RRC parameter list is incorrect.), and needs to be moved from PUSCH-PowerControl to PUSCH-ConfigCommon. PUSCH-ConfigCommon ::= SEQUENCE { -- Sequence-group hopping can be enabled or disabled by means of this cell-specific parameter. -- Corresponds to L1 parameter 'Group-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) -- This field is Cell specific groupHoppingEnabledTransformPrecoding ENUMERATED {enabled} OPTIONAL, -- ------------------------ -- Power control parameters -- Power offset between msg3 and RACH preamble transmission. Corresponds to L1 parameter 'Delta-preamble-msg3' (see 38.213, section 7.1) msg3-DeltaPreamble FFS_Value OPTIONAL, -- P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withgrant' (see 38.213, section 7.1) -- This field is cell specific p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, -- P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1) p0-NominalWithoutGrant INTEGER (-202..24) OPTIONAL, ... } [...] PUSCH-PowerControl ::= SEQUENCE { -- RNTI used for PUSCH TPC. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-PUSCH-RNTI RNTI-Value OPTIONAL, -- If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation -- Corresponds to L1 parameter 'Accumulation-enabled' (see 38.213, section 7.1) tpcAccumulation ENUMERATED { enabled } OPTIONAL, -- Need R -- Dedicated alpha value for msg3 PUSCH. Corresponds to L1 parameter 'alpha-ue-pusch-msg3' (see 38.213, section 7.1) -- When the field is absent the UE applies the value 1. msg3-Alpha Alpha OPTIONAL, -- P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1) p0-NominalWithoutGrant INTEGER (-202..24) OPTIONAL, -- configuration {p0-pusch,alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},…}. -- Corresponds to L1 parameter 'p0-push-alpha-setconfig' (see 38,213, section 7.1) p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet OPTIONAL, -- A set of Refernce Signals (e.g. a CSI-RS config or a SSblock) to be used for PUSCH path loss estimation. -- Up to maxNrofPUSCH-PathlossReference-RSs may be configured when 'PUSCH beam indication' is present (FFS: in DCI???). -- Otherwise, there may be only one entry. FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB? -- Corresponds to L1 parameter 'pusch-pathlossReference-rs-config' (see 38.213, section 7.1) pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReference-RSs)) OF PUSCH-PathlossReference-RS OPTIONAL, -- Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains -- two power control states (i.e., fc(i,1) and fc(i,2)). Otherwise, it applies one (i.e., fc(i,1)). -- Corresponds to L1 parameter 'num-pusch-pcadjustment-states' (see 38.213, section 7.1) twoPUSCH-PC-AdjustmentStates ENUMERATED {twoStates} OPTIONAL, -- Need R -- Indicates whether to apply dela MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH. -- Corresponds to L1 parameter 'deltaMCS-Enabled' (see 38.213, section 7.1) deltaMCS ENUMERATED {enabled} OPTIONAL -- Need R } |  |
| H169 | maxCodeBlockGroupsPerTransportBlock | 2 | In RAN1, there is one limitation i.e. for 2 code words, the value can only be {2, 4}, which needs to be added in the field description when the field description is added. Suggest capturing in the comments now. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) -- For 2 codewords, only the values { n2, n4 } are valid maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, |  |
| H170 | dmrs-Type | 2 | It should not be optional since there is no default values in RAN1. There is only a fixed pattern “Type-1 with 2 additional DMRS” before RRC connection setup, but it is not a default values after RRC connection setup. In RAN1 RRC parameter list, the default value for this IE is “TBD”, so the DMRS type should be configured. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2}, OPTIONAL, -- Need R |  |
| H171 | dmrs-AdditionalPosition | 2 | The note above this IE is incorrect. This is for PUSCH configuration, so this should be UL and corresponds to L1 paramenter ‘UL-DMRS-add-pos’. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DLUL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DLUL. Corresponds to L1 parameter 'UL-DMRS-add-pos'.. dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R |  |
| H172 | cp-OFDM and dft-S-OFDM in dmrs-Uplink | 2 | The cp-OFDM and dft-S-OFDM cannot be configured simultaneously, so we think a choice structure can be considered to cover both of these two cases. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref phaseTracking-RS SetupRelease { Uplink-PTRS-Config } OPTIONAL, -- Need M -- The maximum number of OFDM symbols for UL front loaded DMRS. -- Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2) maxLength ENUMERATED {len1, len2} OPTIONAL, -- FFS: If CP-OFDM and DFT-S-OFDM cannot be configured simultaneously, make the two blocks below a CHOICE cp-OrDFT-S-OFDM CHOICE { -- DMRS related parameters for Cyclic Prefix OFDM cp-OFDM SEQUENCE { -- UL DMRS scrambling initalization for CP-OFDM -- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID' (see 38.214, section 6.4.1.1.2) -- When the field is absent the UE applies the value Physical cell ID + 6 fixed bits (e.g. 000000) -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS: Is this parameter also needed for cell specific signallign scramblingID BIT STRING (SIZE (16)) OPTIONAL }, -- DMRS related parameters for DFT-s-OFDM (Transform Precoding) dft-S-OFDM SEQUENCE { -- Parameter: N_ID^(csh_DMRS) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nDMRS-CSH-Identity-Transform-precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nDMRS-CSH-Identity INTEGER(0..1007) OPTIONAL, -- Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nPUSCH-Identity INTEGER(0..1007) OPTIONAL, -- Sequence-group hopping for PUSCH can be disabled for a certain UE despite being enabled on a cell basis. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Disable-sequence-group-hopping-Transform-precoding' (see 38.211, section FFS_Section) disableSequenceGroupHopping ENUMERATED {disabled} OPTIONAL, -- Determines if sequence hopping is enabled or not. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) sequenceHoppingEnabled ENUMERATED {enabled} OPTIONAL, -- Orthogonal Cover Code (OCC) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Activate-DMRS-with OCC-Transform-precoding' (see 38.211, section FFS_Section) activateDMRS-WithOCC ENUMERATED {enabled} OPTIONAL, -- CS for the ZC sequence. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'CyclicShift-Transform-precoding' (see 38.211, section FFS_Section) cyclicShift INTEGER (0..7) OPTIONAL, -- Parameter: Delta_ss for sequence shift pattern. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'groupAssignmentPUSCH-Transform-precoding' (see 38.211, section FFS_Section) -- When the field is absent the UE applies the value 'CellID mod 30' -- FFS: Is the CellID meant to be the PCI? Or the entire CellID? groupAssignmentPUSCH INTEGER (0..29) OPTIONAL } } }, |  |
| H173 | disableSequenceGroupHopping, sequenceHoppingEnabled, activateDMRS-WithOCC, cyclicShift, groupAssignmentPUSCH | 2 | There is no any agreement in PUSCH for sequence hopping and group hopping. The sequece hoppiong and group hopping are used for PUCCH. The description in RRC parameter list is incorrect. So, these parameters needs to be removed from PUSCH part. The corresponding PUCCH parameters are already there. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref phaseTracking-RS SetupRelease { Uplink-PTRS-Config } OPTIONAL, -- Need M -- The maximum number of OFDM symbols for UL front loaded DMRS. -- Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2) maxLength ENUMERATED {len1, len2} OPTIONAL, -- FFS: If CP-OFDM and DFT-S-OFDM cannot be configured simultaneously, make the two blocks below a CHOICE -- DMRS related parameters for Cyclic Prefix OFDM cp-OFDM SEQUENCE { -- UL DMRS scrambling initalization for CP-OFDM -- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID' (see 38.214, section 6.4.1.1.2) -- When the field is absent the UE applies the value Physical cell ID + 6 fixed bits (e.g. 000000) -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS: Is this parameter also needed for cell specific signallign scramblingID BIT STRING (SIZE (16)) OPTIONAL }, -- DMRS related parameters for DFT-s-OFDM (Transform Precoding) dft-S-OFDM SEQUENCE { -- Parameter: N_ID^(csh_DMRS) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nDMRS-CSH-Identity-Transform-precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nDMRS-CSH-Identity INTEGER(0..1007) OPTIONAL, -- Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nPUSCH-Identity INTEGER(0..1007) OPTIONAL, -- Sequence-group hopping for PUSCH can be disabled for a certain UE despite being enabled on a cell basis. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Disable-sequence-group-hopping-Transform-precoding' (see 38.211, section FFS_Section) disableSequenceGroupHopping ENUMERATED {disabled} OPTIONAL, -- Determines if sequence hopping is enabled or not. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) sequenceHoppingEnabled ENUMERATED {enabled} OPTIONAL, -- Orthogonal Cover Code (OCC) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Activate-DMRS-with OCC-Transform-precoding' (see 38.211, section FFS_Section) activateDMRS-WithOCC ENUMERATED {enabled} OPTIONAL, -- CS for the ZC sequence. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'CyclicShift-Transform-precoding' (see 38.211, section FFS_Section) cyclicShift INTEGER (0..7) OPTIONAL, -- Parameter: Delta_ss for sequence shift pattern. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'groupAssignmentPUSCH-Transform-precoding' (see 38.211, section FFS_Section) -- When the field is absent the UE applies the value 'CellID mod 30' -- FFS: Is the CellID meant to be the PCI? Or the entire CellID? groupAssignmentPUSCH INTEGER (0..29) OPTIONAL } }, |  |
| H174 | cp-OFDM and dft-S-OFDM in Uplink-PTRS-Config | 2 | The cp-OFDM and dft-S-OFDM cannot be configured simultaneously, so we think a choice structure can be considered to cover both of these two cases. -- Configuration of Uplink Phase-Tracking-Reference-Signals (PTRS) -- FFS: Is it possible to configure PTRS for CP-OFDM and DFT-S OFDM simultaneously or just one. If the latter, replace below by a CHOICE. -- FFS: Some but not all of these parameters are supposed to be per BWP. Can we anyway just move all of them into BWP? Uplink-PTRS-Config ::= SEQUENCE { -- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices -- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1) -- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM -- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts? -- And if so, should this structure be conditional to the nrofPorts being set to n2? srs-MappingNonCodebook SEQUENCE { srs CHOICE { resource SRS-ResourceId, resourceSet SRS-ResourceSetId }, ptrs-PortIndex ENUMERATED {n1, n2} OPTIONAL, cpOrDFT-S-OFDM CHOICE { -- Configuration of UL PTRS for CP-OFDM cp-OFDM SetupRelease { SEQUENCE { -- Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW -- Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS -- Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicator related to the number of UL PTRS ports for CP-OFDM. Details to be further decided. -- Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.1) nrofPorts ENUMERATED {n1, n2}, -- Indicates the subcarrier offset for UL PTRS for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1) resourceElementOffset FFS_Value OPTIONAL, -- UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1) ptrs-Power ENUMERATED {f1, f2, f3, f4} } }, OPTIONAL, -- Cond M -- Configuration of UL PTRS for DFT-S-OFDM. dft-S-OFDM SetupRelease { SEQUENCE { -- Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4}, -- that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should -- use depending on the scheduled BW according to the table in 38.214 FFS_Section. -- FFS: Configuration is supposed to be per BWP according to RAN1 -- Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1) sampleDensity FFS_Value OPTIONAL, -- Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM -- Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1) timeDensity ENUMERATED {d1, d2} OPTIONAL } } } } OPTIONAL -- Cond M } |  |
| H175 | srs-MappingNonCodebook | 2 | The description in RRC parameter list is incorrect. Based on the RAN1 agreements, the PTRS port index is only for each SRS resource as below: Agreement: • For non-codebook based UL transmission, the UL PTRS port index is associated to each SRS resource. So the ‘resourceSet’ in srs-MappingNonCodebook is not needed, and corresponding descripion in the note about this IE needs to be updated. In addition, the port index can be port 0 and port 1 as agreed in RAN1. Uplink-PTRS-Config ::= SEQUENCE { -- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices -- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1) -- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM -- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts? -- And if so, should this structure be conditional to the nrofPorts being set to n2? srs-MappingNonCodebook SEQUENCE { srs CHOICE { resource SRS-ResourceId, resourceSet SRS-ResourceSetId }, ptrs-PortIndex ENUMERATED {n1, n2n0, n1} OPTIONAL, |  |
| H176 | frequencyDensity timeDensity rbg-Size | 2 | This parameter is per BWP, so the FFS above the IE can be removed and this IE needs to be put under BWP. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref phaseTracking-RS SetupRelease { Uplink-PTRS-Config } OPTIONAL, -- Need M -- The maximum number of OFDM symbols for UL front loaded DMRS. -- Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2) maxLength ENUMERATED {len1, len2} OPTIONAL, -- FFS: If CP-OFDM and DFT-S-OFDM cannot be configured simultaneously, make the two blocks below a CHOICE -- DMRS related parameters for Cyclic Prefix OFDM cp-OFDM SEQUENCE { -- UL DMRS scrambling initalization for CP-OFDM -- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID' (see 38.214, section 6.4.1.1.2) -- When the field is absent the UE applies the value Physical cell ID + 6 fixed bits (e.g. 000000) -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS: Is this parameter also needed for cell specific signallign scramblingID BIT STRING (SIZE (16)) OPTIONAL }, -- DMRS related parameters for DFT-s-OFDM (Transform Precoding) dft-S-OFDM SEQUENCE { -- Parameter: N_ID^(csh_DMRS) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nDMRS-CSH-Identity-Transform-precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nDMRS-CSH-Identity INTEGER(0..1007) OPTIONAL, -- Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nPUSCH-Identity INTEGER(0..1007) OPTIONAL, -- Sequence-group hopping for PUSCH can be disabled for a certain UE despite being enabled on a cell basis. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Disable-sequence-group-hopping-Transform-precoding' (see 38.211, section FFS_Section) disableSequenceGroupHopping ENUMERATED {disabled} OPTIONAL, -- Determines if sequence hopping is enabled or not. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) sequenceHoppingEnabled ENUMERATED {enabled} OPTIONAL, -- Orthogonal Cover Code (OCC) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Activate-DMRS-with OCC-Transform-precoding' (see 38.211, section FFS_Section) activateDMRS-WithOCC ENUMERATED {enabled} OPTIONAL, -- CS for the ZC sequence. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'CyclicShift-Transform-precoding' (see 38.211, section FFS_Section) cyclicShift INTEGER (0..7) OPTIONAL, -- Parameter: Delta_ss for sequence shift pattern. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'groupAssignmentPUSCH-Transform-precoding' (see 38.211, section FFS_Section) -- When the field is absent the UE applies the value 'CellID mod 30' -- FFS: Is the CellID meant to be the PCI? Or the entire CellID? groupAssignmentPUSCH INTEGER (0..29) OPTIONAL } }, pusch-PowerControl PUSCH-PowerControl OPTIONAL, -- Need M -- Configured one of two supported frequency hopping mode. If not configured frequency hopping is not configured -- Corresponds to L1 parameter 'Frequency-hopping-PUSCH' (see 38.214, section 6) -- When the field is absent the UE applies the value Not configured frequencyHopping ENUMERATED {mode1, mode2}, -- Configure either LBRM or FBRM for PUSCH. FBRM = Full buffer rate-matchingLBRM = Limited buffer rate-matching -- Corresponds to L1 parameter 'LBRM-FBRM-selection' (see 38.212, section 5.4.2) rateMatching ENUMERATED {fullBufferRM, limitedBufferRM}, -- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI -- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 6.1.2) -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? resourceAllocation CHOICE { resourceAllocationType0 NULL, resourceAllocationType1 NULL, dynamicSwitch NULL } OPTIONAL, -- Indicates which MCS table the UE shall use for PUSCH without transform precoder -- Corresponds to L1 parameter 'MCS-Table-PUSCH' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-Table ENUMERATED {qam64, qam256}, -- Indicates which MCS table the UE shall use for PUSCH with transform precoding -- Corresponds to L1 parameter 'MCS-Table-PUSCH-transform-precoding' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-TableTransformPrecoder ENUMERATED {qam64, qam256}, -- The UE specific selection of transformer precoder for PUSCH. When the field is absent the UE applies the value msg3-tp. -- Corresponds to L1 parameter 'PUSCH-tp' (see 38.211, section 6.3.1.4) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- Selection between config 1 and config 2 for RBG size for PUSCH. Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1) rbg-Size ENUMERATED {config1, config2}, -- Selection between and configuration of dynamic and semi-static beta-offset -- Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.214, section 9.3) uci-on-PUSCH SetupRelease { CHOICE { dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets, semiStatic BetaOffsets } } OPTIONAL, -- Need M -- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description. -- Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2) xOverhead FFS_Value OPTIONAL, -- Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 -- Corresponds to L1 parameter 'Frequency-hopping-offsets-set' (see 38.214, section 6.1.4) frequencyHoppingOffsets SEQUENCE (SIZE (1..4)) OF FFS_Value OPTIONAL, ... } [...] -- Configuration of Uplink Phase-Tracking-Reference-Signals (PTRS) -- FFS: Is it possible to configure PTRS for CP-OFDM and DFT-S OFDM simultaneously or just one. If the latter, replace below by a CHOICE. -- FFS: Some but not all of these parameters are supposed to be per BWP. Can we anyway just move all of them into BWP? Uplink-PTRS-Config ::= SEQUENCE { -- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices -- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1) -- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM -- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts? -- And if so, should this structure be conditional to the nrofPorts being set to n2? srs-MappingNonCodebook SEQUENCE { srs CHOICE { resource SRS-ResourceId, resourceSet SRS-ResourceSetId }, ptrs-PortIndex ENUMERATED {n1, n2} OPTIONAL, -- Configuration of UL PTRS for CP-OFDM cp-OFDM SetupRelease { SEQUENCE { -- Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW -- Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS -- Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicator related to the number of UL PTRS ports for CP-OFDM. Details to be further decided. -- Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.1) nrofPorts ENUMERATED {n1, n2}, -- Indicates the subcarrier offset for UL PTRS for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1) resourceElementOffset FFS_Value OPTIONAL, -- UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1) ptrs-Power ENUMERATED {f1, f2, f3, f4} } } OPTIONAL, -- Cond M -- Configuration of UL PTRS for DFT-S-OFDM. dft-S-OFDM SetupRelease { SEQUENCE { -- Sample density of PT-RS for DFT-s-OFDM, pre-DFT, indicating a set of thresholds T={NRBn,n=0,1,2,3,4}, -- that indicates dependency between presence of PT-RS and scheduled BW and the values of X and K the UE should -- use depending on the scheduled BW according to the table in 38.214 FFS_Section. -- FFS: Configuration is supposed to be per BWP according to RAN1 -- Corresponds to L1 parameter 'UL-PTRS-pre-DFT-density' (see 38.214, section 6.1) sampleDensity FFS_Value OPTIONAL, -- Time density (OFDM symbol level) of PT-RS for DFT-s-OFDM -- Corresponds to L1 parameter 'UL-PTRS-time-density-transform-precoding' (see 38.214, section 6.1) timeDensity ENUMERATED {d1, d2} OPTIONAL } } } OPTIONAL -- Cond M } [...] UplinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, -- Frequency location of the uplink "direct current" frequency. -- Corresponds to L1 parameter 'UL-BWP-DC'. (see 38.211, section FFS_Section) directCurrentLocation INTEGER (0..3299) OPTIONAL, -- FFS_CHECK: Several (UE specific) BWPs may be configured with RACH resources. Hence, they must be provided with -- the information in RACH-ConfigCommon... even though it is in this case strictly speaking not a cell-specific parameter. -- OK to keep or re-structure the RACH config? rach-ConfigCommon RACH-ConfigCommon, pusch-ConfigCommon PUSCH-ConfigCommon, pusch-ConfigPerBWP PUSCH-Config, -- FFS: Is the PUSCH also BWP-specific??SEQUENCE { frequencyDensityUL-PTRS ENUMERATED {ffsTypeAndValue}, timeDensityUL_PTRS ENUMERATED {ffsTypeAndValue}, -- Selection between config 1 and config 2 for RBG size for PUSCH. Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1) rbg-Size ENUMERATED {config1, config2}, } pucch-ConfigCommon PUCCH-ConfigCommon, pucch-Config PUCCH-Config } |  |
| H177 | resourceElementOffset | 2 | RAN1 has agreed “PTRS-RE-Offset” consists 2bits with values 00, 01, 10, and 11. Therefore the ‘FFS value’ can be updated accordingly, e.g. ENUMERATED {value00, value01, value10, value11}. Uplink-PTRS-Config ::= SEQUENCE { -- The PTRS port index for each configured SRS resource/resource set for non-codebook based UL MIMO, with at most UL-PTRS-ports port indices -- Corresponds to L1 parameter 'UL-PTRS-SRS-mapping-non-CB' (see 38.214, section 6.1) -- FFS_CHECK: Is this only for CP-OFDM or also for DFT-S-OFDM -- FFS_CHECK: Is it correct that the port index can only be 1 or 2? And if so, is the value further restricted by the parameter nrofPorts? -- And if so, should this structure be conditional to the nrofPorts being set to n2? srs-MappingNonCodebook SEQUENCE { srs CHOICE { resource SRS-ResourceId, resourceSet SRS-ResourceSetId }, ptrs-PortIndex ENUMERATED {n1, n2} OPTIONAL, -- Configuration of UL PTRS for CP-OFDM cp-OFDM SetupRelease { SEQUENCE { -- Presence and frequency density of UL PT-RS for CP-OFDM waveform as a function of scheduled BW -- Corresponds to L1 parameter 'UL-PTRS-frequency-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 frequencyDensity ENUMERATED {ffsTypeAndValue}, -- Presence and time density of UL PT-RS for CP-OFDM waveform as a function of MCS -- Corresponds to L1 parameter 'UL-PTRS-time-density-table' (see 38.214, section 6.1) -- FFS: Configuration is supposed to be per BWP according to RAN1 timeDensity ENUMERATED {ffsTypeAndValue}, -- Indicator related to the number of UL PTRS ports for CP-OFDM. Details to be further decided. -- Corresponds to L1 parameter 'UL-PTRS-ports' (see 38.214, section 6.1) nrofPorts ENUMERATED {n1, n2}, -- Indicates the subcarrier offset for UL PTRS for CP-OFDM. Corresponds to L1 parameter 'UL-PTRS-RE-offset' (see 38.214, section 6.1) resourceElementOffset FFS_ValueENUMERATED { offset00, offset01, offset10, offset11 } OPTIONAL, -- UL PTRS power boosting factor per PTRS port. Corresponds to L1 parameter 'UL-PTRS-power' (see 38.214, section 6.1) ptrs-Power ENUMERATED {f1, f2, f3, f4} } } OPTIONAL, -- Cond M |  |
| H178 | tpc-PUSCH-RNTI | 2 | The RNTIs in PDCCH are not needed here. For now there is no detailed agreement, it is most likely to be included in the DL configuration. Therefore the FFS in the note above this IE can be removed. PUSCH-PowerControl ::= SEQUENCE { -- RNTI used for PUSCH TPC. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-PUSCH-RNTI RNTI-Value OPTIONAL, |  |
| H179 | resourceAllocation can be changed to enum, assuming FFS is that these have no type. -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? | 2 | -- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI -- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 6.1.2) -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? resourceAllocation ENUMERATEDCHOICE {resourceAllocationType0 NULL, resourceAllocationType1 NULL, dynamicSwitch NULL} |  |
| H180 | frequencyHopping should be optional because default is “not configured” | 2 | -- Configured one of two supported frequency hopping mode. If not configured frequency hopping is not configured -- Corresponds to L1 parameter 'Frequency-hopping-PUSCH' (see 38.214, section 6) -- When the field is absent the UE applies the value Not configured frequencyHopping ENUMERATED {mode1, mode2} OPTIONAL, |  |
| H181 | rateMatching should be optional because default is full buffer rate matching | 2 | -- Configure either LBRM or FBRM for PUSCH. FBRM = Full buffer rate-matchingLBRM = Limited buffer rate-matching -- Corresponds to L1 parameter 'LBRM-FBRM-selection' (see 38.212, section 5.4.2) -- When the field is absent the UE applies the value fullBufferRM rateMatching ENUMERATED {fullBufferRM, limitedBufferRM} OPTIONAL, |  |
| H182 | mcs-Table should be optional because default is 64QAM | 2 | -- Indicates which MCS table the UE shall use for PUSCH without transform precoder -- Corresponds to L1 parameter 'MCS-Table-PUSCH' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-Table ENUMERATED {qam64, qam256} OPTIONAL, |  |
| H183 | mcs-TableTransformPrecoder should be optional because default is 64QAM | 2 | -- Indicates which MCS table the UE shall use for PUSCH with transform precoding -- Corresponds to L1 parameter 'MCS-Table-PUSCH-transform-precoding' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-TableTransformPrecoder ENUMERATED {qam64, qam256} OPTIONAL, |  |
| H184 | transformPrecoder should have both enable and disable options. The default is the same as Msg3 | 2 | -- The UE specific selection of transformer precoder for PUSCH. When the field is absent the UE applies the value msg3-tp. -- Corresponds to L1 parameter 'PUSCH-tp' (see 38.211, section 6.3.1.4) transformPrecoder ENUMERATED {enabled, disabled} OPTIONAL, |  |
| H185 | rbg-Size default value is config1 | 2 | -- Selection between config 1 and config 2 for RBG size for PUSCH. Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1) -- When the field is absent the UE applies the value config1 rbg-Size ENUMERATED {config1, config2} OPTIONAL, |  |
| H186 | Dynamic choice in uci-on_PUSCH is a set of 4 according to L1 parameter list. The description in RRC parameter list is not exact. The reference should be 38.213 instead of 38.214, thus the corresponding note above this IE needs to be updated. | 2 | -- Selection between and configuration of dynamic and semi-static beta-offset -- Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.2143, section 9.3) uci-on-PUSCH SetupRelease { CHOICE { dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets, semiStatic BetaOffsets } } OPTIONAL, -- Need M |  |
| H187 | xOverhead no value range specified in L1 parameter list RAN1 Agreements: - The set of possible Xoh values are [0 0.5 1 1.5]*12 Default value is 0 (for both UL and DL) In PUSCH, this parameter should corresponds to L1 parameter ‘Xoh-PUSCH’ instead of ‘Xoh-PDSCH’, and the note above this IE needs to be updated accordingly | 2 | -- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description. -- Corresponds to L1 parameter 'Xoh-PDSCH' 'Xoh-PUSCH' (see 38.214, section 5.1.3.2) -- When the field is absent the UE applies the value 0 xOverhead ENUMERATED {xoh0, xoh6, xoh12, xoh18} FFS_Value OPTIONAL, |  |
| H188 | frequencyHoppingOffsets is a set of 4 according to L1 parameter list. The description in RRC parameter list is not exact. The reference should be section 6.3 instead of 6.1.4, thus the corresponding note above this IE needs to be updated. No RAN1 agreement on the FFS, RAN1 update needed. | 2 | -- Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 -- Corresponds to L1 parameter 'Frequency-hopping-offsets-set' (see 38.214, section 6.3 6.1.4) frequencyHoppingOffsets SEQUENCE (SIZE (1..4)) OF FFS_Value OPTIONAL, |  |
| H189 | tpcAccumulation default value is enabled so enumerated value should be “disable” | 2 | PUSCH-PowerControl ::= SEQUENCE { -- RNTI used for PUSCH TPC. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-PUSCH-RNTI RNTI-Value OPTIONAL, -- If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation -- Corresponds to L1 parameter 'Accumulation-enabled' (see 38.213, section 7.1) -- If absent TPC accumulation is enabled tpcAccumulation ENUMERATED { disabledenabled } OPTIONAL, -- Need R -- Dedicated alpha value for msg3 PUSCH. Corresponds to L1 parameter 'alpha-ue-pusch-msg3' (see 38.213, section 7.1) -- When the field is absent the UE applies the value 1. msg3-Alpha Alpha OPTIONAL, -- P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1) p0-NominalWithoutGrant INTEGER (-202..24) OPTIONAL, -- configuration {p0-pusch,alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},…}. -- Corresponds to L1 parameter 'p0-push-alpha-setconfig' (see 38,213, section 7.1) p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet OPTIONAL, -- A set of Refernce Signals (e.g. a CSI-RS config or a SSblock) to be used for PUSCH path loss estimation. -- Up to maxNrofPUSCH-PathlossReference-RSs may be configured when 'PUSCH beam indication' is present (FFS: in DCI???). -- Otherwise, there may be only one entry. FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB? -- Corresponds to L1 parameter 'pusch-pathlossReference-rs-config' (see 38.213, section 7.1) pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReference-RSs)) OF PUSCH-PathlossReference-RS OPTIONAL, -- Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains -- two power control states (i.e., fc(i,1) and fc(i,2)). Otherwise, it applies one (i.e., fc(i,1)). -- Corresponds to L1 parameter 'num-pusch-pcadjustment-states' (see 38.213, section 7.1) twoPUSCH-PC-AdjustmentStates ENUMERATED {twoStates} OPTIONAL, -- Need R -- Indicates whether to apply dela MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH. -- Corresponds to L1 parameter 'deltaMCS-Enabled' (see 38.213, section 7.1) deltaMCS ENUMERATED {enabled} OPTIONAL -- Need R } |  |
| H190 | Msg3-DeltaPreamble needs to be specified by RAN1 | 2 | PUSCH-ConfigCommon ::= SEQUENCE { -- Sequence-group hopping can be enabled or disabled by means of this cell-specific parameter. -- Corresponds to L1 parameter 'Group-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) -- This field is Cell specific groupHoppingEnabledTransformPrecoding ENUMERATED {enabled} OPTIONAL, -- ------------------------ -- Power control parameters -- Power offset between msg3 and RACH preamble transmission. Corresponds to L1 parameter 'Delta-preamble-msg3' (see 38.213, section 7.1) msg3-DeltaPreamble FFS_Value OPTIONAL, -- P0 value for PUSCH with grant (except msg3). Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withgrant' (see 38.213, section 7.1) -- This field is cell specific p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, ... } |  |
| H191 | p0-nominal-pusch-withgrant, p0-nominal-pusch-withoutgrant, p0-nominal-pucch, p0-srs – step size 2 not enforced could use smaller range and convert, similar to e.g. RSRP-RangeSL4 in 36.331 to discuss whether such optimisation is necessary (maybe it is less clear) | 2 | -- P0 value for PUSCH with grant (except msg3). Range is -202 to +24dBM with step size 2. Value 0 means -202dBm, value 1 means -200dBM, value 2 mens -198dBm and so on up to value 113 means +24dBM. -- Corresponds to L1 parameter 'p0-nominal-pusch-withgrant' (see 38.213, section 7.1) -- This field is cell specific p0-NominalWithGrant INTEGER (0..113-202..24) OPTIONAL, |  |
| H192 | RAN1 need to define the values | 2 | -- A set of p0-pusch and alpha used for PUSCH with grant. 'PUSCH beam indication' (if present) gives the index of the set to -- be used for a particular PUSCH transmission. -- FFS_CHECK: Is the ”PUSCH beam indication” in DCI which schedules the PUSCH? If so, clarify in field description -- Corresponds to L1 parameter 'p0-pusch-alpha-set' (see 38.213, section 7.1) P0-PUSCH-AlphaSet ::= SEQUENCE { p0-PUSCH-AlphaSetId P0-PUSCH-AlphaSetId, -- P0 value for PUSCH with grant (except msg3). Corresponds to L1 parameter 'p0-pusch' (see 38,213, section 7.1) p0 FFS_Value OPTIONAL, -- alpha value for PUSCH with grant (except msg3) (see 38.213, section 7.1) -- When the field is absent the UE applies the value 1 alpha Alpha OPTIONAL } |  |
| H193 | PUSCH-PathlossReference-RS default is 1, and it is unclear how pathloss can be estimated without these. | 2 | We understand that pathlossReferenceRSs should be mandatory, and the “default” value refers to the case that ‘PUSCH beam indication’ is absent and the sequence contains only one entry. PUSCH-PowerControl ::= SEQUENCE { -- RNTI used for PUSCH TPC. Corresponds to L1 parameter 'TPC-PUSCH-RNTI' (see 38.213, section 10) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-PUSCH-RNTI RNTI-Value OPTIONAL, -- If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation -- Corresponds to L1 parameter 'Accumulation-enabled' (see 38.213, section 7.1) tpcAccumulation ENUMERATED { enabled } OPTIONAL, -- Need R -- Dedicated alpha value for msg3 PUSCH. Corresponds to L1 parameter 'alpha-ue-pusch-msg3' (see 38.213, section 7.1) -- When the field is absent the UE applies the value 1. msg3-Alpha Alpha OPTIONAL, -- P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed. -- Corresponds to L1 parameter 'p0-nominal-pusch-withoutgrant' (see 38.213, section 7.1) p0-NominalWithoutGrant INTEGER (-202..24) OPTIONAL, -- configuration {p0-pusch,alpha} sets for PUSCH (except msg3), i.e., { {p0,alpha,index1}, {p0,alpha,index2},…}. -- Corresponds to L1 parameter 'p0-push-alpha-setconfig' (see 38,213, section 7.1) p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet OPTIONAL, -- A set of Refernce Signals (e.g. a CSI-RS config or a SSblock) to be used for PUSCH path loss estimation. -- Up to maxNrofPUSCH-PathlossReference-RSs may be configured when 'PUSCH beam indication' is present (FFS: in DCI???). -- Otherwise, there may be only one entry. FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB? -- Corresponds to L1 parameter 'pusch-pathlossReference-rs-config' (see 38.213, section 7.1) pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReference-RSs)) OF PUSCH-PathlossReference-RS OPTIONAL, -- Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains -- two power control states (i.e., fc(i,1) and fc(i,2)). Otherwise, it applies one (i.e., fc(i,1)). -- Corresponds to L1 parameter 'num-pusch-pcadjustment-states' (see 38.213, section 7.1) twoPUSCH-PC-AdjustmentStates ENUMERATED {twoStates} OPTIONAL, -- Need R -- Indicates whether to apply dela MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH. -- Corresponds to L1 parameter 'deltaMCS-Enabled' (see 38.213, section 7.1) deltaMCS ENUMERATED {enabled} OPTIONAL -- Need R } |  |
| H194 | maxNrofPUCCH-P0-PerSet has value 8 should it be 7? RAN1 range is 0-7 in “num-p0-pucch” so 0 would be absent, and 7 the max? Or is RAN1 list wrong? | 2 | PUCCH-PowerControl ::= SEQUENCE { -- RNTI used for PUCCH TPC. Corresponds to L1 parameter 'TPC-PUCCH-RNTI' (see 38.213, section 10). -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-PUCCH-RNTI RNTI-Value OPTIONAL, -- A set with dedicated P0 values for PUCCH, i.e., {P01, P02,... }. Corresponds to L1 parameter 'p0-pucch-set' (see 38.213, section 7.2) p0-Set SEQUENCE (SIZE (10..maxNrofPUCCH-P0-PerSet)) OF P0-PUCCH OPTIONAL, -- A set of Refernce Signals (e.g. a CSI-RS config or a SSblock) to be used for PUCCH pathloss estimation. -- Up to maxNrofPUCCH-PathlossReference-RSs may be configured -- FFS_CHECK: Is it possible not to configure it at all? What does the UE use then? Any SSB? -- Corresponds to L1 parameter 'pucch-pathlossReference-rs-config' (see 38.213, section 7.2) pathlossReferenceRSs SEQUENCE (SIZE (1..maxNrofPUCCH-PathlossReference-RSs)) OF PUCCH-PathlossReference-RS OPTIONAL, -- Number of PUCCH power control adjustment states maintained by the UE (i.e., g(i)). If the field is present (n2) the UE maintains -- two power control states (i.e., g(i,0) and g(i,1)). Otherwise, it applies one (i.e., g(i,0)). -- Corresponds to L1 parameter 'num-pucch-pcadjustment-states' (see 38.213, section 7.2) twoPUSCH-PC-AdjustmentStates ENUMERATED {twoStates} OPTIONAL, -- Need R ... } maxNrofPUCCH-P0-PerSet INTEGER ::= 78 -- Maximum number of P0-pucch present in a p0-pucch set |  |
| H195 | RAN1 to define | 2 | -- P0 value for PUCCH. Corresponds to L1 parameter 'p0-pucch' (see 3,213, section 7.2) P0-PUCCH ::= FFS_Value |  |
| H196 | deltaF-pucch-f0, deltaF-pucch-f1, deltaF-pucch-f2, deltaF-pucch-f3, deltaF-pucch-f4 value needs to be defined by RAN1 | 2 | RAN1 input needed |  |
| H197 | Group-hopping-enabled-Transform-precoding is missing (this is not the same like Sequence-hopping-enabled-Transform-precoding) | 2 | Add the parameter BOOLEAN. PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dmrs-Uplink SEQUENCE { -- Selection of the DMRS type to be used for UL (see section 38.211, section 6.4.1.1.2) dmrs-Type ENUMERATED {type1, type2} OPTIONAL, -- Need R -- Position for additional DM-RS in DL, see Table 7.4.1.1.2-4 in 38.211. -- The four values represent the cases of 1+0, 1+1, 1+1+1. 1+1+1+1 non-adjacent OFDM symbols for DL. dmrs-AdditionalPosition ENUMERATED {pos0, pos1, pos2, pos3} OPTIONAL, -- Need R -- Configures uplink PTRS (see 38.211, section x.x.x.x) FFS_Ref phaseTracking-RS SetupRelease { Uplink-PTRS-Config } OPTIONAL, -- Need M -- The maximum number of OFDM symbols for UL front loaded DMRS. -- Corresponds to L1 parameter 'UL-DMRS-max-len' (see 38.214, section 6.4.1.1.2) maxLength ENUMERATED {len1, len2} OPTIONAL, -- FFS: If CP-OFDM and DFT-S-OFDM cannot be configured simultaneously, make the two blocks below a CHOICE -- DMRS related parameters for Cyclic Prefix OFDM cp-OFDM SEQUENCE { -- UL DMRS scrambling initalization for CP-OFDM -- Corresponds to L1 parameter 'UL-DMRS-Scrambling-ID' (see 38.214, section 6.4.1.1.2) -- When the field is absent the UE applies the value Physical cell ID + 6 fixed bits (e.g. 000000) -- FFS: Clarify default value: Are the 6 bits zeros (says e.g.). Are they the MSBs or LSBs? -- FFS: Is this parameter also needed for cell specific signallign scramblingID BIT STRING (SIZE (16)) OPTIONAL }, -- DMRS related parameters for DFT-s-OFDM (Transform Precoding) dft-S-OFDM SEQUENCE { -- Parameter: N_ID^(csh_DMRS) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nDMRS-CSH-Identity-Transform-precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nDMRS-CSH-Identity INTEGER(0..1007) OPTIONAL, -- Parameter: N_ID^(PUSCH) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'nPUSCH-Identity-Transform precoding' (see 38.211, section FFS_Section) -- FFS: Should we reaplace this explicit type by the type ScramblingId? nPUSCH-Identity INTEGER(0..1007) OPTIONAL, -- Per-cell enabling/disabling of sequence group hopping. -- Corresponds to L1 parameter 'Group-hopping-enabled-transform-precoding’ (see 38.211, section FFS_Section) groupHoppingEnabledTransformPrecoding ENUMERATED {enabled) OPTIONAL, -- Sequence-group hopping for PUSCH can be disabled for a certain UE despite being enabled on a cell basis. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Disable-sequence-group-hopping-Transform-precoding' (see 38.211, section FFS_Section) disableSequenceGroupHopping ENUMERATED {disabled} OPTIONAL, -- Determines if sequence hopping is enabled or not. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Sequence-hopping-enabled-Transform-precoding' (see 38.211, section FFS_Section) sequenceHoppingEnabled ENUMERATED {enabled} OPTIONAL, -- Orthogonal Cover Code (OCC) for DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'Activate-DMRS-with OCC-Transform-precoding' (see 38.211, section FFS_Section) activateDMRS-WithOCC ENUMERATED {enabled} OPTIONAL, -- CS for the ZC sequence. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'CyclicShift-Transform-precoding' (see 38.211, section FFS_Section) cyclicShift INTEGER (0..7) OPTIONAL, -- Parameter: Delta_ss for sequence shift pattern. For DFT-s-OFDM DMRS -- Corresponds to L1 parameter 'groupAssignmentPUSCH-Transform-precoding' (see 38.211, section FFS_Section) -- When the field is absent the UE applies the value 'CellID mod 30' -- FFS: Is the CellID meant to be the PCI? Or the entire CellID? groupAssignmentPUSCH INTEGER (0..29) OPTIONAL } }, pusch-PowerControl PUSCH-PowerControl OPTIONAL, -- Need M -- Configured one of two supported frequency hopping mode. If not configured frequency hopping is not configured -- Corresponds to L1 parameter 'Frequency-hopping-PUSCH' (see 38.214, section 6) -- When the field is absent the UE applies the value Not configured frequencyHopping ENUMERATED {mode1, mode2}, -- Configure either LBRM or FBRM for PUSCH. FBRM = Full buffer rate-matchingLBRM = Limited buffer rate-matching -- Corresponds to L1 parameter 'LBRM-FBRM-selection' (see 38.212, section 5.4.2) rateMatching ENUMERATED {fullBufferRM, limitedBufferRM}, -- Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI -- Corresponds to L1 parameter 'Resouce-allocation-config' (see 38.214, section 6.1.2) -- FFS_Value: Are these values just 3 flags (ENUMERATED) or the actual configurations? If the latter, where are they defined? resourceAllocation CHOICE { resourceAllocationType0 NULL, resourceAllocationType1 NULL, dynamicSwitch NULL } OPTIONAL, -- Indicates which MCS table the UE shall use for PUSCH without transform precoder -- Corresponds to L1 parameter 'MCS-Table-PUSCH' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-Table ENUMERATED {qam64, qam256}, -- Indicates which MCS table the UE shall use for PUSCH with transform precoding -- Corresponds to L1 parameter 'MCS-Table-PUSCH-transform-precoding' (see 38.214, section 6.1.4) -- When the field is absent the UE applies the value 64QAM mcs-TableTransformPrecoder ENUMERATED {qam64, qam256}, -- The UE specific selection of transformer precoder for PUSCH. When the field is absent the UE applies the value msg3-tp. -- Corresponds to L1 parameter 'PUSCH-tp' (see 38.211, section 6.3.1.4) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- Selection between config 1 and config 2 for RBG size for PUSCH. Corresponds to L1 parameter 'RBG-size-PUSCH' (see 38.214, section 6.1.2.2.1) rbg-Size ENUMERATED {config1, config2}, -- Selection between and configuration of dynamic and semi-static beta-offset -- Corresponds to L1 parameter 'UCI-on-PUSCH' (see 38.214, section 9.3) uci-on-PUSCH SetupRelease { CHOICE { dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets, semiStatic BetaOffsets } } OPTIONAL, -- Need M -- Accounts for overhead from CSI-RS, CORESET, etc. FFS: Clarify value range and description. -- Corresponds to L1 parameter 'Xoh-PDSCH' (see 38.214, section 5.1.3.2) xOverhead FFS_Value OPTIONAL, -- Set of frequency hopping offsets used when frequency hopping is enabled for granted transmission (not msg3) and type 2 -- Corresponds to L1 parameter 'Frequency-hopping-offsets-set' (see 38.214, section 6.1.4) frequencyHoppingOffsets SEQUENCE (SIZE (1..4)) OF FFS_Value OPTIONAL, ... } |  |

#### –	Q-OffsetRange

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	QuantityConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H198 | Editorial error? |  | QuantityConfig ::= SEQUENCE { quantityConfigNREUTRA QuantityConfigNREUTRA OPTIONAL, quantityConfigNR-list QuantityConfigNR-List OPTIONAL } |  |

#### –	RACH-ConfigCommon

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H199 | UL and DL are in a single IE, while L1 parameters lists separately. Also RACH parameters are (re)defined in the IE BeamManagement – should refer to this IE? | 3 | Consider to break the UL and DL fields out into separate structures. -- ASN1START -- TAG-RACH-CONFIG-COMMON-START RACH-ConfigCommon ::= SEQUENCE { -- FFS: whether any of the parameter(s) in the L1 TP should be within CBRA-SSB-ResourceList groupBconfigured SEQUENCE { -- FFS: ra-Msg3SizeGroupA values ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, -- FFS: Need and definition of messagePowerOffsetGroupB messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18} } OPTIONAL, cbra-SSB-ResourceList CBRA-SSB-ResourceList, ra-ContentionResolutionTimer ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}, -- Msg1 (RA preamble): -- UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission -- based on SS blocks that satisfy the threshold (see 38.213, section REF) ssb-Threshold RSRP-Range OPTIONAL, -- FFS: Provide proper description -- Corresponds to L1 parameter 'SUL-RSRP-Threshold' (see FFS_Spec, section FFS_Section) sul-RSRP-Threshold RSRP-Range OPTIONAL, -- PRACH configuration index. Corresponds to L1 parameter 'PRACHConfigurationIndex' (see 38.211, section 6.3.3.2) prach-ConfigurationIndex INTEGER (0..255) OPTIONAL, -- PRACH root sequence index. Corresponds to L1 parameter 'PRACHRootSequenceIndex' (see 38.211, section 6.3.3.1). -- The value range depends on whether L=839 or L=139 prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } OPTIONAL, -- N-CS configuration, see Table 6.3.3.1-3 in 38.211 zeroCorrelationZoneConfig INTEGER(0..15), -- Subcarrier spacing of PRACH. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section) -- FFS_DefaultValue: Same as DL SCS? msg1-SubcarrierSpacing SubcarrierSpacing OPTIONAL, -- The number of PRACH transmission occasions FDMed in one time instance. -- Corresponds to L1 parameter 'prach-FDM' (see 38,211, section FFS_Section) -- FFS_DefaultValue? msg1-FDM BIT STRING (SIZE (2)), -- Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of initial active UL BWP(s) -- Corresponds to L1 parameter 'prach-frequency-start' (see 38,211, section FFS_Section) -- FFS_FIXME: Clarify whether it is ”initial” or ”firstActive” UL BWP, i.e., whether this is meant for SpCell and/or SCell -- FFS_FIXME: What is PRB 0 or a BWP? PRB 0 defines the lower edge of the carrier. msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1) OPTIONAL, -- Configuration of restricted sets, see 38.211 6.3.3.1 -- FFS_CHECK: RAN1 value said "restrictedTypeA". Does it mean "restrictedToTypeA"? If not, what else? restrictedSetConfig ENUMERATED {unrestricted, restrictedToTypeA, restrictedToTypeB}, -- (see 38.213, section 7.4) preambleReceivedTargetPower ENUMERATED { dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76, dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52, dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30, dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6, dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 } OPTIONAL, -- Power ramping steps for PRACH (see 38.321, FFS_section) powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6} OPTIONAL, -- Need R -- FFS_CHECK: PreambleTransMax parameter usage (parameter was not provided by RAN1 and not yet discussed in RAN2) preambleTransMax ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}, -- Corresponds to L1 parameter 'CB-preambles-per-SSB' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”4 bit” but there should be actual values here... and not hidden in a table. cb-preamblesPerSSB FFS_Value OPTIONAL, -- Number of SSBs per RACH occasion. By multiplying with cb-preamblesPerSSB, the UE determines the total number of CB preambles. -- Corresponds to L1 parameter 'SSB-per-rach-occasion' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”3 bit” but there should be actual values here... and not hidden in a table. ssb-perRACH-Occasion FFS_Value OPTIONAL, -- Msg2 (RAR) window length. Corresponds to L1 parameter 'msg2-scs' (see 38.213, section 8.1) -- FFS_Value: To be decided by RAN2 ra-ResponseWindow ENUMERATED {ffsTypeAndValue}, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover. -- Corresponds to L1 parameter 'msg2-scs' (see 38.321?, section FFS_Section) msg2-SubcarrierSpacing SubcarrierSpacing, -- CORESET configured for random access. When the field is absent the UE uses the CORESET according to pdcchConfigSIB1 -- Corresponds to L1 parameter 'rach-coreset-configuration' (see 38.211?, section FFS_Section) ra-ControlResourceSet ControlResourceSetId OPTIONAL, -- Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) -- FFS: If the field is absent the UE uses the SearchSpace according to pdcchConfigSIB1 ra-SearchSpace SearchSpace OPTIONAL, -- Subcarrier spacing for Msg3. Corresponds to L1 parameter 'msg3-scs' (see 38.213, section 8.1) msg3-SubcarrierSpacing SubcarrierSpacing, -- Indicates to a UE whether transform precoding is enabled for Msg3 transmission. -- Corresponds to L1 parameter 'msg3-tp' (see 38.213, section 8.1) msg3-transformPrecoding ENUMERATED {true} OPTIONAL -- Need R } |  |
| H200 | In L1 parameters list there’s no default for msg3-transformPrecoding so should be mandatory with enable/disable | 2 | -- Indicates to a UE whether transform precoding is enabled for Msg3 transmission. -- Corresponds to L1 parameter 'msg3-tp' (see 38.213, section 8.1) msg3-transformPrecoding ENUMERATED {trueenabled, disabled} OPTIONAL -- Need R |  |
| H201 | powerRampingStep has no default and it is not clear what would happen if this was absent with need R. | 2 | -- Power ramping steps for PRACH (see 38.321, FFS_section) powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6} OPTIONAL, -- Need R |  |
| H202 | No default specified for prach-ConfigurationIndex and not clear what would happen is not present – PRACH configuration needs an index. | 2 | -- PRACH configuration index. Corresponds to L1 parameter 'PRACHConfigurationIndex' (see 38.211, section 6.3.3.2) prach-ConfigurationIndex INTEGER (0..255) OPTIONAL, |  |
| H203 | No default specified for prach-RootSequenceIndex What is L? Do both values need to be provided – is it possible for both L to be supported for one PRACH? | 2 | -- PRACH root sequence index. Corresponds to L1 parameter 'PRACHRootSequenceIndex' (see 38.211, section 6.3.3.1). -- The value range depends on whether L=839 or L=139 prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } OPTIONAL, |  |
| H204 | preambleReceivedTargetPower range needs to be checked by RAN1. L1 parameters list just states 6 bits and it is not clear from RAN1 spec (to me) No default specified | 2 | -- (see 38.213, section 7.4) preambleReceivedTargetPower ENUMERATED { dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76, dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52, dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30, dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6, dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 } OPTIONAL, |  |
| H205 | FFS – needs to be decided in RAN2 preambleTransMax | 2 | -- FFS_CHECK: PreambleTransMax parameter usage (parameter was not provided by RAN1 and not yet discussed in RAN2) preambleTransMax ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}, |  |
| H206 | ra-ResponseWindow to be decided in RAN2. | 2 | -- Msg2 (RAR) window length. Corresponds to L1 parameter 'msg2-scs' (see 38.213, section 8.1) -- FFS_Value: To be decided by RAN2 ra-ResponseWindow ENUMERATED {ffsTypeAndValue}, |  |
| H207 | Should just use RAN1 suggested values for restrictedSetConfig, we can check offline. -- FFS_CHECK: RAN1 value said "restrictedTypeA". Does it mean "restrictedToTypeA"? If not, what else? | 2 | -- Configuration of restricted sets, see 38.211 6.3.3.1 -- FFS_CHECK: RAN1 value said "restrictedTypeA". Does it mean "restrictedToTypeA"? If not, what else? restrictedSetConfig ENUMERATED {unrestricted, restrictedToTypeA, restrictedToTypeB}, |  |
| H208 | For SUL, according to the RAN2 agreement: “For contention based RA, if the network does not explicitly tell the UE which carrier to use, the UE shall perform UL selection based on the RSRP threshold as initial access”, there should be an option to explicitly indicate the uplink carrier. The MAC spec has already captured this condition as below. 1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; | 2 | # -- FFS: Provide proper description -- Corresponds to L1 parameter 'SUL-RSRP-Threshold' (see FFS_Spec, section FFS_Section) cbra-ul-carrier CHOICE { explicit ENUMERATED {normal, sul}, sul-RSRP-Threshold RSRP-Range } OPTIONAL, OPTIONAL, |  |
| H209 | No default specified for prach-Msg1SubcarrierSpacing but the suggestion is to have default “same as DL SCS”. RAN1 to check – if no agreement then IE should be mandatory. Section number to be updated | 2 | -- Subcarrier spacing of PRACH. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section) -- FFS_DefaultValue: Same as DL SCS? msg1-SubcarrierSpacing SubcarrierSpacing OPTIONAL, |  |
| H210 | msg1-FDM should probably have values specified instead of just “2 bits”. RAN1 FFS. | 2 | -- The number of PRACH transmission occasions FDMed in one time instance. -- Corresponds to L1 parameter 'prach-FDM' (see 38,211, section FFS_Section) -- FFS_DefaultValue? msg1-FDM BIT STRING (SIZE (2)), |  |
| H211 | -- section to be updated -- FFS_FIXME: Clarify whether it is ”initial” or ”firstActive” UL BWP, i.e., whether this is meant for SpCell and/or SCell -- FFS_FIXME: What is PRB 0 or a BWP? PRB 0 defines the lower edge of the carrier. -- no default specified | 2 | -- Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of initial active UL BWP(s) -- Corresponds to L1 parameter 'prach-frequency-start' (see 38,211, section FFS_Section) -- FFS_FIXME: Clarify whether it is ”initial” or ”firstActive” UL BWP, i.e., whether this is meant for SpCell and/or SCell -- FFS_FIXME: What is PRB 0 or a BWP? PRB 0 defines the lower edge of the carrier. msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1) OPTIONAL, |  |
| H212 | searchSpace for RAR probably should be defined in PDCCH-ConfigCommon. RACH-config is for the configuration of msg1 transmission. SO ra-SearchSpace and Ra-CORESET should not be defined here. | 2 | -- Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) -- FFS: If the field is absent the UE uses the SearchSpace according to pdcchConfigSIB1 ra-SearchSpace SearchSpace OPTIONAL, |  |
| H213 | CSI-RS-threshold is defined as a cell specific parameter (with ?) in the L1 list. This is missing from the common config asn1. There is a parameter in dedicated config for CFRA. If the threshold is used only for CFRA and not CBRA then it may be OK as it is. | 2 | TBD if needed in common config |  |
| H214 | cb-preamblesPerSSB | 2 | A better name should be cb-preamblePerSSBPerRO -- Corresponds to L1 parameter 'CB-preambles-per-SSB' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”4 bit” but there should be actual values here... and not hidden in a table. cb-preamblesPerSSBPerRO FFS_Value OPTIONAL, |  |
| H215 | ssb-perRACH-Occasion | 2 | This parameter is contradictory to CBRA-SSB-resource or RA resource -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”3 bit” but there should be actual values here... and not hidden in a table. - Number of CBRA preambles per SSB per RACH transmission occasion - Maximum size for the range of values: 4 bits - Number of SSBs per RACH occasion - Maximum size for the range of values: 3 bits R1-1709612 -- Number of SSBs per RACH occasion. By multiplying with cb-preamblesPerSSB, the UE determines the total number of CB preambles. -- Corresponds to L1 parameter 'SSB-per-rach-occasion' (see 38.211?, section FFS_Section) -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. -- FFS_Value: RAN1 indicated ”3 bit” but there should be actual values here... and not hidden in a table. ssb-perRACH-Occasion FFS_Value OPTIONAL, |  |
| H216 | RACH-ConfigCommon =>msg2-subcarrierSpacing | 2 | This parameter is not needed in CFRA -- Msg2 (RAR) window length. Corresponds to L1 parameter 'msg2-scs' (see 38.213, section 8.1) -- FFS_Value: To be decided by RAN2 ra-ResponseWindow ENUMERATED {ffsTypeAndValue}, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover. -- Corresponds to L1 parameter 'msg2-scs' (see 38.321?, section FFS_Section) msg2-SubcarrierSpacing SubcarrierSpacing, -- CORESET configured for random access. When the field is absent the UE uses the CORESET according to pdcchConfigSIB1 -- Corresponds to L1 parameter 'rach-coreset-configuration' (see 38.211?, section FFS_Section) ra-ControlResourceSet ControlResourceSetId OPTIONAL, -- Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) -- FFS: If the field is absent the UE uses the SearchSpace according to pdcchConfigSIB1 ra-SearchSpace SearchSpace |  |
| H217 | RACH-ConfigCommon =>msg1-subcarrierSpacing | 2 | Possible to be configured in the BandwidthPart-Config The following agreement was made in RAN1 90b  The numerology of the initial active UL BWP is the same as the numerology of Msg3 PUSCH configured in RMSI. -- N-CS configuration, see Table 6.3.3.1-3 in 38.211 zeroCorrelationZoneConfig INTEGER(0..15), -- Subcarrier spacing of PRACH. Corresponds to L1 parameter 'prach-Msg1SubcarrierSpacing' (see 38.211, section FFS_Section) -- FFS_DefaultValue: Same as DL SCS? msg1-SubcarrierSpacing SubcarrierSpacing OPTIONAL, -- The number of PRACH transmission occasions FDMed in one time instance. -- Corresponds to L1 parameter 'prach-FDM' (see 38,211, section FFS_Section) -- FFS_DefaultValue? msg1-FDM BIT STRING (SIZE (2)), -- Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of initial active UL BWP(s) -- Corresponds to L1 parameter 'prach-frequency-start' (see 38,211, section FFS_Section) -- FFS_FIXME: Clarify whether it is ”initial” or ”firstActive” UL BWP, i.e., whether this is meant for SpCell and/or SCell -- FFS_FIXME: What is PRB 0 or a BWP? PRB 0 defines the lower edge of the carrier. |  |
| H218 | RACH-ConfigCommon =>msg1-FrequencyStart | 3 | The range of the value is wrong; RAN 1 has made the following agreement: • Relative frequency offset of Msg1 o Note: This defines the offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0 of initial active UL BWP(s) o Value: {0,1,…,Bandwidth of initial active UL BWP in terms of PRBs – Bandwidth of the RACH occasion in terms of PRBs} The range of the values is INTEGER (0..maxNrofPhysicalResourceBlocks-1). However, the agreement in RAN 1 says the range should be {0,1,…,Bandwidth of initial active UL BWP in terms of PRBs – Bandwidth of the RACH occasion in terms of PRBs}. The difference is that the # of BWP RBs should be smaller than that of the whole bandwidth. msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks- |  |
| H219 | CBRA-SSB-ResourceList RACH-ConfigCommon =>cb-preamblesPerSSB | 3 | -- FFS_CHECK: Relation to (old) RAN2 CBRA-SSB-ResourceList handling the CB-RA preambles/resources per beam. This configuration has not been agreed in RAN 1 and should be removed The RAN 1 agreements are given as follows and they are the reason why the two parameters cb-preamblesPerSSB(PerRO) and ssb-perRACH-Occasion are added 90b - For contention based RACH, support the same number of PRACH preambles for all actually transmitted SS blocks. • FFS: Same/different number of PRACH preambles for SS blocks in CFRA. 91 - gNB configures in RMSI the following: - Number of CBRA preambles per SSB per RACH transmission occasion - Number of SSBs per RACH occasion However, one of the main issues with the current configuration is that there is redundancy in groupBconfigured and CBRA-SSB-ResourceList. Rapporteur also showed his concern by a FFS -- FFS: whether any of the parameter(s) in the L1 TP should be within CBRA-SSB-ResourceList groupBconfigured SEQUENCE { -- FFS: ra-Msg3SizeGroupA values ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, -- FFS: Need and definition of messagePowerOffsetGroupB messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18} } OPTIONAL, Hence, we may consider to have a further discussion on this in the future meetings |  |
| H220 | RACH-ConfigCommon =>groupBConfigured=>messagePowerOffsetGroupB |  | -- FFS: Need and definition of messagePowerOffsetGroupB Definition: already in LTE. As in 36.331 and can be reused: Threshold for preamble selection in TS 36.321 [6]. Value in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on RACH-ConfigCommon ::= SEQUENCE { -- FFS: whether any of the parameter(s) in the L1 TP should be within CBRA-SSB-ResourceList groupBconfigured SEQUENCE { -- FFS: ra-Msg3SizeGroupA values ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, -- FFS: Need and definition of messagePowerOffsetGroupBThreshold for preamble selection. Value in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18} } OPTIONAL, |  |
| H221 | RACH-ConfigCommon =>cb-preamblesPerSSB |  | -- FFS_Value: RAN1 indicated ”4 bit” but there should be actual values here... and not hidden in a table The following agreement was made in the previous RAN 1 meeting: Support following values for the configuration of the number of PRACH preambles for CBRA per SSB: - 4 * N (where 1 <= N <= 16) - FFS: 0, 1, 2, 3, 5, 6, 7, 10, 14, 18, 22, 26, 30, values greater than 64 FFS: different granularity for different number of PRACH preambles for CBRA and CFRA per SSB RAN1 decision needed |  |
| H222 | Consider using the same SSB threshold name as RAN1 and the latest MAC spec 38.321. Or make a note to explain the link of this threshold with the rsrp-ThresholdSSB. | 2 | … ra-ContentionResolutionTimer ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}, -- Msg1 (RA preamble): -- UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission -- based on SS blocks that satisfy the threshold (see 38.213, section REF) ssb-Thresholdrsrp-ThresholdSSB TYPE_FFS! OPTIONAL, -- FFS: Provide proper description -- Corresponds to L1 parameter 'SUL-RSRP-Threshold' (see FFS_Spec, section FFS_Section) sul-RSRP Threshold FFS_Value OPTIONAL, … |  |

#### –	RACH-ConfigDedicated

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H223 | Cfra-csirs-Threshold should be outside of the CHOICE | 2 | -- FFS: resources for msg1-based on-demand SI request -- FFS: resources for beam failure recovery request RACH-ConfigDedicated ::= SEQUENCE { -- Resources for handover to the cell cfra-Resources CFRA-Resources, cfra-csirs-Threshold RSRP-Range, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover rar-SubcarrierSpacing SubcarrierSpacing } -- FFS_CHECK: Isn’t it sufficient to have just one list and the CHOICE inside the list element (around the ssb/csirs)? CFRA-Resources ::= CHOICE { cfra-ssb-ResourceList SEQUENCE (SIZE(1..maxRAssbResources)) OF CFRA-SSB-Resource, cfra-csirs-ResourceList SEQUENCE (SIZE(1..maxRAcsirsResources)) OF CFRA-CSIRS-Resource, cfra-csirs-Threshold RSRP-Range } |  |
| H224 | The length of each list maay be different, depending on the FFS range. In any case it may be better to keep as it is in case one list changes/extends and the other not..propose just to remove FFS | 2 | -- FFS_CHECK: Isn’t it sufficient to have just one list and the CHOICE inside the list element (around the ssb/csirs)? CFRA-Resources ::= CHOICE { cfra-ssb-ResourceList SEQUENCE (SIZE(1..maxRAssbResources)) OF CFRA-SSB-Resource, cfra-csirs-ResourceList SEQUENCE (SIZE(1..maxRAcsirsResources)) OF CFRA-CSIRS-Resource, cfra-csirs-Threshold RSRP-Range } |  |
| H225 | FFS | 2 | -- FFS: resources for msg1-based on-demand SI request |  |
| H226 | FFS | 2 | -- FFS: resources for beam failure recovery request |  |
| H227 | FFS | 2 | CFRA-SSB-Resource ::= SEQUENCE { ssb SSB-Id, ra-PreambleIndex INTEGER (0..63), -- PRACH configuration for SSB configuration (i.e. time and frequency location) ra-Resources RA-Resources -- Definition FFS } CFRA-CSIRS-Resource ::= SEQUENCE { csirs NZP-CSI-RS-ResourceId, -- FFS where the CSI-RS are defined (e.g. MO) ra-PreambleIndex INTEGER (0..63), -- PRACH configuration for CSIRS configuration (i.e. time and frequency location) ra-Resources RA-Resources -- Definition FFS } |  |
| H228 | FFS | 2 | CFRA-SSB-Resource ::= SEQUENCE { ssb SSB-Id, ra-PreambleIndex INTEGER (0..63), -- PRACH configuration for SSB configuration (i.e. time and frequency location) ra-Resources RA-Resources -- Definition FFS } CFRA-CSIRS-Resource ::= SEQUENCE { csirs NZP-CSI-RS-ResourceId, -- FFS where the CSI-RS are defined (e.g. MO) ra-PreambleIndex INTEGER (0..63), -- PRACH configuration for CSIRS configuration (i.e. time and frequency location) ra-Resources RA-Resources -- Definition FFS } |  |
| H229 | RACH-ConfigDedicated =>rar-SubcarrierSpacing |  | A better name would be be cfra-msg2-SubcarrierSpacing RACH-ConfigDedicated ::= SEQUENCE { -- Resources for handover to the cell cfra-Resources CFRA-Resources, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover cfra-msg2-rar-SubcarrierSpacing SubcarrierSpacing } |  |
| H230 | RACH-ConfigDedicated In the previous meeting, the following agreement was made on the scs for RA procedure for CBRA and CFRA, respectively. For contention-based NR 4-step RA procedure • SCS for Msg 1 • configured in the RACH configuration • SCS for Msg 2 • the same as the numerology of RMSI • SCS for Msg 3 • configured in the RACH configuration separately from SCS for Msg1 • SCS for Msg 4 • the same as in Msg2 For contention-free RA procedure for handover, the SCS for Msg1 and the SCS for Msg2 are provided in the handover command While the configuration for msg2 is given by rar-SubcarrierSpacing, there is no SCS configuration for msg 1 |  | RACH-ConfigDedicated ::= SEQUENCE { -- Resources for handover to the cell cfra-Resources CFRA-Resources, -- Subcarrier spacing for msg1 for contention-free RA procedure for handover cfra-msg1-SubcarrierSpacing SubcarrierSpacing, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover cfra-msg2-rar-SubcarrierSpacing SubcarrierSpacing } |  |
| H231 | In Reno meeting RAN2 reached following agreements on HO access: 1. ssb-Threshold signalled in handover command (for both common and dedicated RACH). 2. csirs-dedicatedRACH-Threshold signalled in handover command. The ssb-Threshold is already specified in RACH-ConfigCommon. Only the threshold for CSI-RS should be captured here. | 2 | … CFRA-CSIRS-Resource ::= SEQUENCE { csirs CSIRS-ID, -- FFS where the CSI-RS are defined (e.g. MO) ra-PreambleIndex INTEGER (0..FFS_XX), -- PRACH configuration for CSIRS configuration (i.e. time and frequency location) ra-Resources RA-Resources, -- Definition FFS -- CSI-RS quality threshold for selection of beams with dedicated RACH resources for HO access. Subject to RAN1 agreement. csi-rs-DedicatedRACH-Threshold FFS_Value OPTIONAL } -- TAG-RACH-CONFIG-DEDICATED-STOP -- ASN1STOP |  |
| H232 | In Reno meeting RAN2 reached following agreement on beam recovery: 1: For beam recovery purposes RRC signalling allows the case of configuring both SSB + CSI-RS (i.e. simultaneously) for new candidate beam identification. The case where only one of SSB or CSI-RS resource is configured is also covered – i.e. this is network configuration. Currently CHOICE is used in RACH-configDedicated information element. It is customized for HO access requirement. But it does not support the latest agreement for beam recovery which allows configuring ssb and csirs with dedicated RACH at the same time. To support both HO and beam recovery cases, a simple solution is changing “CHOICE” to “SEQUENCE”. | 2 | … RACH-ConfigDedicated ::= SEQUENCE { -- Resources for handover to the cell cfra-Resources CFRA-Resources, -- Subcarrier spacing for msg2 for contention-free RA procedure for handover rar-SubcarrierSpacing SubcarrierSpacing } -- CHECK: Isn’t it sufficient to have just one list and the CHOICE inside the list element (around the ssb/csirs)? CFRA-Resources ::= SEQUENCECHOICE { cfra-ssb-ResourceList SEQUENCE (SIZE(1..maxRAssbResources) OF CFRA-SSB-Resource OPTIONAL, cfra-csirs-ResourceList SEQUENCE (SIZE(1..maxRAcsirsResources)OF CFRA-CSIRS-Resource OPTIONAL } … |  |

#### –	RadioBearerConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ReportConfigId

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ReportConfigNR

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H233 | In 5.5.5.2, for beam reporting, there is a FFS for the ordering of reported beam: Editor’s Note: FFS which quantity to use for ordering beam measurement results. We suggest to indicate the quantity for beams sort by the network for both event and periodical MR, |  | To clarify the beam ordering criteria for beams report, one possible change is to sort the beams by the quantity triggered by the event, but it is still unclear on how to sort beams for periodical MR. another options is to indicate the quantity for beams sort explicitly by the network for event and periodical MR, in that case an additional configuration is needed to indicate the quantity for beam sort in MR. EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventA1 SEQUENCE { a1-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA2 SEQUENCE { a2-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA3 SEQUENCE { a3-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN OPTIONAL }, eventA4 SEQUENCE { a4-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN OPTIONAL }, eventA5 SEQUENCE { a5-Threshold1 MeasTriggerQuantity, a5-Threshold2 MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN OPTIONAL }, eventA6 SEQUENCE { a6-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN OPTIONAL } }, rsType ENUMERATED {ss, csi-rs}, -- Common reporting config (at least to periodical and eventTriggered) reportInterval ReportInterval, reportAmount ENUMERATED {ffsTypeAndValue}, rankQuantity RankQuantity OPTIONAL, -- Cell reporting configuration reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), -- RS index reporting configuration reportQuantityRsIndexes MeasReportQuantity OPTIONAL, maxNrofIndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, onlyReportBeamIds BOOLEAN OPTIONAL, -- If configured the UE includes the best neighbour cells per serving frequency reportAddNeighMeas ENUMERATED {ffsTypeAndValue} } PeriodicalReportConfig ::= SEQUENCE { rsType ENUMERATED {ssb, csi-rs}, -- Common reporting config (at least to periodical and eventTriggered) reportInterval ReportInterval, reportAmount ENUMERATED {ffsTypeAndValue}, -- Cell reporting configuration reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), -- RS index reporting configuration reportQuantityRsIndexes MeasReportQuantity OPTIONAL, maxNrofRsIndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, onlyReportBeamIds BOOLEAN, rankQuantity RankQuantity OPTIONAL OPTIONAL } RankQuantity::= ENUMERATED {rsrp, rsrq, sinr} |  |

#### –	ReportConfigToAddModList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	RLC-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H234 | For the FFS in the field description of t-Reassembly “If is FFS whether ms1600 is supported in this version of the specification”, it is fine to us to add this value, given that there is still a spare value left and the length of t-Reassembly needs to be extended in NR compared to LTE. |  | T-Reassembly ::= ENUMERATED { ms0, ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms110, ms120, ms130, ms140, ms150, ms160, ms170, ms180, ms190, ms200, spare1, ms1600} Remove “If is FFS whether ms1600 is supported in this version of the specification.” From the field description for T-Reassembly. |  |

#### –	RLF-TimersAndConstants

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SCellIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SchedulingRequest-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SchedulingRequestResource-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H235 | Wrong values in periodicity range | 2 | The spreadsheet shows the periodicity parameter with range { sym2, sym7, ms0p125, ms0p25, ms0p5, ms1, ms2, ms5, ms10, ms20, ms40, ms80 } only, but the CHOICE includes 160, 320, and 640. Also the units are described as slots although the spreadsheet says ms. SchedulingRequestResource-Config ::= SEQUENCE { -- SR periodicity. Corresponds to L1 parameter 'SR-periodicity' and 'SR-offset' (see 38.213, section 9.2.2) -- FFS_Value: Check whether value ranges are implemented correctly for higher SCSs. periodicityAndOffset CHOICE { -- FFS_RAN1: Need to signal an offset or is it known from PUCCH format configuration? sym2 NULL, -- FFS_RAN1: Need to signal an offset or is it known from PUCCH format configuration? sym7 NULL, ms0p125 NULL, ms0p25 NULL, ms0p5 NULL, msl1 NULL, -- Recurs in every slot msl2 INTEGER (0..1), msl5 INTEGER (0..4), msl10 INTEGER (0..9), msl20 INTEGER (0..19), msl40 INTEGER (0..39), msl80 INTEGER (0..79), sl160 INTEGER (0..159), -- Only for 30, 60 and 120 Khz Subcarrier Spacing sl320 INTEGER (0..319), -- Only for 60 and 120 Khz Subcarrier Spacing sl640 INTEGER (0..639) -- Only for 120 Khz Subcarrier Spacing } OPTIONAL, -- Format, length, ... of this SR reosurce. Corresponds to L1 parameter 'SR-resource' (see 38.213, section 9.2.2) -- FFS_CHECK: Is the implementation as intended by RAN1? Or were these supposed to be just IDs pointing to a resource configured elsewhere? resource CHOICE { format0 PUCCH-format0, format1 PUCCH-format1 } OPTIONAL } |  |
| H236 | Add schedulingRequestID into each SchedulingReqeustResource-Config, as per previous agreement. | 2 | There was a UP agreement that ‘An SR configuration consists of a collection of sets of PUCCH resources across different BWPs and cells’. To achieve this, it was further agreed in the L2 parameter email discussion that a scheduling request ID is used to associate the PUCCH resources for SR with corresonding SR configurations, and this is already captured in a comment in the current TS 38.331, as below (in SchedulingRequest-Config): -- FFS_TODO: provide resources for each SchedulingRequestID in ServingCellConfigDedicated (TBD whether directly, in PUCCH-Config, in each BWP) Now the SchedulingReqeustResource-Config is included in PUCCH-Config, which is configured per BWP in corresponding serving cell.; this means the PCCUH resource for SR defined in each ScehdulingReqeustResource-Config has already been configured per BWP per celd. So, to acehive the above FFS, what we need to do is just to add schedulingRequestID into each SchedulingRequestResource-Config, as follows: SchedulingRequestResource-Config ::= SEQUENCE { schedulingRequestID SchedulingRequestId, periodicityAndOffset CHOICE { sym2 NULL, sym7 NULL, sl1 NULL, sl2 INTEGER (0..1), sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) } OPTIONAL, resource CHOICE { format0 PUCCH-format0, format1 PUCCH-format1 } OPTIONAL } In this way, it can be realized that each SR configuration includes a collection of PUCCH SR resources across different BWPs and Cells. |  |

#### –	SDAP-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SecurityAlgorithmConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ServCellIndex

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	ServingCellConfigCommon

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H237 | Definition of the IE ServingCellConfigCommon |  | 1) For a cell with mutliple SSB, SSB parameters are not the same for all UEs in the cell so this is not a "cell specific parameter". Besides, this information is not really useful -> remove first sentence 2) Which parameter derived from SSB or MIB would be included in dedicated signalling? If none, remove "SSB, MIB or" 3) Remove "typically" |  |
| H238 | UplinkConfigCommon uses UplinkBandwidthPart where some parameters may be absent (e.g. PUCCH-ConfigCommon) for dedicated case. Since this parameter is used for SIB1, it is better to optimize its size, using another type. |  | Change type used, define: InitialUplinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, -- Frequency location of the uplink "direct current" frequency. directCurrentLocation INTEGER (0..3299), rach-ConfigCommon RACH-ConfigCommon, pucch-ConfigCommon PUCCH-ConfigCommon, pusch-ConfigCommon PUSCH-ConfigCommon } (type change) UplinkConfigCommon ::= SEQUENCE { -- Absolute uplink frequency configuration and subcarrier specific virtual carriers. frequencyInfoUL FrequencyInfoUL OPTIONAL, -- Cond InterFreqHOAndUplinkSCellAdd -- The initial uplink BWP configuration for a SpCell (PCell of MCG or SCG). Corresponds to L1 parameter 'initial-UL-BWP'. -- (see 38.331, section FFS_Section). initialUplinkBandwidthPart InitialUplinkBandwidthPart OPTIONAL -- Cond FDD-SpCell } |  |

#### –	ServingCellConfigDedicated

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H239 | dataScramblingIdentity fields should have a common type | 2 | dataScramblingIdentityPDSCH and dataScramblingIdentityPUSCH use 1007 as a “magic number” in the range. Propose to introduce a type DataScramblingIdentityPDSCH-PUSCH ::= INTEGER (0..1007). Note this is different from the existing ScramblingIdentity which has the full 10-bit range. ServingCellConfigDedicated ::= SEQUENCE { -- L1 parameters: tdd-UL-DL-configurationDedicated TDD-UL-DL-Config OPTIONAL, -- Cond TDD bandwidthParts BandwidthPart-Config OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PDSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS:_Replace by tye ScramblingId used in other places? dataScramblingIdentityPDSCH INTEGER (0..1007) DataScramblingIdentityPDSCH-PUSCH OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PUSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS: Replace by tye ScramblingId used in other places? dataScramblingIdentityPUSCH INTEGER (0..1007)DataScramblingIdentityPDSCH-PUSCH OPTIONAL, […] DataScramblingIdentityPDSCH-PUSCH ::= INTEGER (0..1007) |  |
| H240 | dataScramblingIdentityPDSCH and dataScramblingIdentityPUSCH could be moved to PDSCH-Config and PUSCH-Config respectively | 2 | These two fields are specific to the respective channels and could be moved to the corresponding “-Config” IEs. ServingCellConfigDedicated ::= SEQUENCE { -- L1 parameters: tdd-UL-DL-configurationDedicated TDD-UL-DL-Config OPTIONAL, -- Cond TDD bandwidthParts BandwidthPart-Config OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PDSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS:_Replace by tye ScramblingId used in other places? dataScramblingIdentityPDSCH INTEGER (0..1007) OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PUSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS: Replace by tye ScramblingId used in other places? dataScramblingIdentityPUSCH INTEGER (0..1007) OPTIONAL, [...] PDSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.213, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission BOOLEAN, -- Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW the maximum CBG is 4 (see 38.213, section 9.1.1) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, -- Indicates whether CBGFI for CBG based (re)transmission in DL is enabled (true). (see FFS_Specification, section FFS_Section) codeBlockGroupFlushIndicator BOOLEAN, dataScramblingIdentityPDSCH INTEGER (0..1007), [...] PUSCH-Config ::= SEQUENCE { -- Indicates whether to use code-block-group (CBG) based transmission (see 38.214, section x.x.x.x) FFS_Ref -- FFS: Is this BOOLEAN parameter needed or can it be derived from the presence of the codeBlockGroupsPerTransportBlock? codeBlockGroupTransmission ENUMERATED {true} OPTIONAL, -- Need R -- Maximum number of code-block-groups (CBGs) per TB (see 38.xxx, section x.x.x, FFS_Ref) maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, dataScramblingIdentityPUSCH INTEGER (0..1007), |  |
| H241 | RAN2#100 agreed that SPS can be configured per BWP per serving cell. | 2 | 1. “SPS may be configured on the PCell as well as SCells,” shall be revised to “SPS can be configured per DL BWP on the PCell as well as on the SCells”. 2. The SPS-config IE can be configured per BWP per serving cell and therefore it shall be moved to the field of DownlinkBandwidthPart IE from ServingCellConfigDedicated IE. DownlinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, pdcch-ConfigCommon PDCCH-ConfigCommon OPTIONAL, sps-Config SPS-Config OPTIONAL } <skip> ServingCellConfigDedicated information element -- ASN1START -- TAG-SERVING-CELL-CONFIG-DEDICATED-START ServingCellConfigDedicated ::= SEQUENCE { -- L1 parameters: tdd-UL-DL-configurationDedicated TDD-UL-DL-Config OPTIONAL, -- Cond TDD bandwidthParts BandwidthPart-Config OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PDSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS:_Replace by tye ScramblingId used in other places? dataScramblingIdentityPDSCH INTEGER (0..1007) OPTIONAL, -- Identifer used to initalite data scrambling (c_init) for both PUSCH. -- Corresponds to L1 parameter 'Data-scrambling-Identity' (see 38,214, section FFS_Section) -- FFS: Replace by tye ScramblingId used in other places? dataScramblingIdentityPUSCH INTEGER (0..1007) OPTIONAL, -- FFS: Is the PDSCH-Config BWP-specific? If so, move into DownlinkBandwidthPart pdsch-Config PDSCH-Config OPTIONAL, -- FFS in RAN1: Tracking Reference Signals configuration: TRS-Config? csi-MeasConfig CSI-MeasConfig OPTIONAL, -- FFS: Is the PUSCH-Config BWP-specific? If so, move into UplinkBandwidthPart pusch-Config PUSCH-Config OPTIONAL, srs-Config SRS-Config OPTIONAL, -- MAC parameters: -- FFS: Is SPS-Config per BWP? If so, split it in UL and DL and move it to the respective UL- and DL BWP) sps-Config SPS-Config OPTIONAL, |  |
| H242 | RAN2#100 agreed to use a common cs-RNTI for both SPS (DL) and Configured grant type 1 and 2 (UL) for the UE. | 2 | ServingCellConfigDedicated information element -- ASN1START -- TAG-SERVING-CELL-CONFIG-DEDICATED-START ServingCellConfigDedicated ::= SEQUENCE { -- L1 parameters: tdd-UL-DL-configurationDedicated TDD-UL-DL-Config OPTIONAL, -- Cond TDD bandwidthParts BandwidthPart-Config OPTIONAL, csi-MeasConfig CSI-MeasConfig OPTIONAL, -- FFS: Is the PUSCH-Config BWP-specific? If so, move into UplinkBandwidthPart pusch-Config PUSCH-Config OPTIONAL, srs-Config SRS-Config OPTIONAL, -- MAC parameters: -- FFS: Is SPS-Config per BWP? If so, split it in UL and DL and move it to the respective UL- and DL BWP) sps-Config SPS-Config OPTIONAL, cs-RNTI RNTI-Value OPTIONAL } -- TAG-SERVING-CELL-CONFIG-DEDICATED-STOP -- ASN1STOP |  |

#### –	SRB-Identity

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SPS-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H243 | RAN2#100 agreed to use a common cs-RNTI for both SPS (DL) and Configured grant type 1 and 2 (UL) for the UE. + terminology of sps- RNTI is not aligned with 38.321. | 2 | - sps-RNTI should be revised to cs-RNTI. - cs-RNTI should be moved to the field of ServingCellConfigDedicated from SPS-config IE -- SPS may be configured on the PCell as well as on SCells. But it shall not be configured for more than -- one serving cell of a cell group at once. SPS-Config ::= SEQUENCE { downlink SEQUENCE { -- FFS: Discuss in RAN2 whether for UL and DL use same SPS-RNTI (like in LTE)? See also naming FFS above. -- RNTI for DL SPS. Corresponds to L1 parameter 'SPS C-RNTI' (see 38.214, section FFS_Section) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) sps-RNTI RNTI-Value OPTIONAL, -- Periodicity for DL SPS -- Corresponds to L1 parameter 'semiPersistSchedIntervalDL' (see 38.214 and 38.321, section FFS_Section) -- FFS_Value: Support also shorter periodicities for DL? periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Number of configured HARQ processes for SPS DL. Corresponds to L1 parameter 'numberOfConfSPS-Processes' (see 38.214, section FFS_Section) nrofHARQ-Processes INTEGER (1..8) OPTIONAL, -- HARQ resource for PUCCH for DL SPS. (see 38.214, section FFS_Section) -- FFS_Value: Is this supposed to be the actual configuration or just an ENUMERATED (configuration eslewhere)? n1PUCCH-AN CHOICE { format0 PUCCH-resource-config-PF0, -- FFS: Is this supposed to be PUCCH-format0? format1 PUCCH-resource-config-PF1 -- FFS: Is this supposed to be PUCCH-format1? } OPTIONAL }, |  |
| H244 | RAN2#100 made agreement on separate naming of DL and UL, and SPS is for downlink transmission only. The description and parameters of SPS-config IE are not correct, which should only include DL configurations. | 2 | 1. The SPS-Config IE is used to configure transmission without dynamic grant for the downlink. 2. The SPS-Config IE should only include parameters for the downlink and therefore parameters for the uplink shall be removed. (Note that cs-RNTI shall be removed as well because it should be added in the field of ServingCellConfigDedicated IE) 3. The parameters of “periodicity” and “nrofHARQ-Processes” in SPS-Config IE should not be optional as in LTE. #### – SPS-Config The SPS-Config IE is used to configure semi-persistent transmission for the downlink according to two possible schemes . SPS-Config information element -- ASN1START -- TAG-SPS-CONFIG-START -- SPS may be configured on the PCell as well as on SCells. But it shall not be configured for more than SPS can be configured per DL BWP on the PCell as well as on SCells -- one serving cell of a cell group at once. SPS-Config ::= SEQUENCE { downlink SEQUENCE { -- FFS: Discuss in RAN2 whether for UL and DL use same SPS-RNTI (like in LTE)? See also naming FFS above. -- RNTI for DL SPS. Corresponds to L1 parameter 'SPS C-RNTI' (see 38.214, section FFS_Section) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) sps-RNTI RNTI-Value OPTIONAL, -- Periodicity for DL SPS -- Corresponds to L1 parameter 'semiPersistSchedIntervalDL' (see 38.214 and 38.321, section FFS_Section) -- FFS_Value: Support also shorter periodicities for DL? periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Number of configured HARQ processes for SPS DL. Corresponds to L1 parameter 'numberOfConfSPS-Processes' (see 38.214, section FFS_Section) nrofHARQ-Processes INTEGER (1..8) OPTIONAL, -- HARQ resource for PUCCH for DL SPS. (see 38.214, section FFS_Section) -- FFS_Value: Is this supposed to be the actual configuration or just an ENUMERATED (configuration eslewhere)? n1PUCCH-AN CHOICE { format0 PUCCH-resource-config-PF0, -- FFS: Is this supposed to be PUCCH-format0? format1 PUCCH-resource-config-PF1 -- FFS: Is this supposed to be PUCCH-format1? } OPTIONAL }, } |  |
| H245 | The descriptions of parameters in SPS-Config IE are not aligned with RAN1 agreements and common understanding. | 2 | 1. The parameters of “periodicity”, “nroHARQ-processes” in proposed ConfiguredGrant-Config IE should not be optional as in LTE. -- FFS_Value: Support also shorter periodicities for DL? periodicity ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL, -- Number of configured HARQ processes for SPS DL. Corresponds to L1 parameter 'numberOfConfSPS-Processes' (see 38.214, section FFS_Section) nrofHARQ-Processes INTEGER (1..8) OPTIONAL, -- HARQ resource for PUCCH for DL SPS. (see 38.214, section FFS_Section) |  |
| H246 | Regarding the EN: FFS, Relation between (UL-)SPS and “ULtransmissionWithoutGrant”, RAN2#100 agreed SPS is for downlink only and for the uplink, Configured grant Type 1 and Type 2 are used. As there are no detailed agreements about the type and values of many parameters from RAN1, many of them are still ffs. Here we provide some suggested values based our own understanding. No agreements on the exact range of these parameters. For GB or Type-2 GF transmission, these parameters are indicated with DCI, thus we can just follow the way of DCI indication here. For some parameters, the range is suggested based on the maximum bitwidth from RAN1 spec. For other parameters, the range is still uncertain from current spec or agreements. | 2 | 1. Add a separate ConfiguredGrant-Config IE for the uplink transmission according to RAN2#100 agreements. 2. The ConfigureGrant-Config IE is used to configure transmission without dynamic grant for the uplink according to Configured grant Type 1 and Type 2. 3. “UL-SPS” shall be revised to “Configured grant” in corresponding descriptions and “sps-RNTI” shall be revised to “cs-RNTI”. 4. RAN1 agreed to introduce both timeDomainOffset and timeDomainAllocation to determine the occasions of configured grant so “FFS: Merge the following two into one. Possibly don’t use periodicity for rrcConfiguredUplinkGrant ” shall be removed. Add “Offset related to SFN=0” and “Allocate Time-domain PUSCH resources (see 38.214, Section 6.1.2)” to above two parameters respectively. #### – ConfiguredGrant-Config The Configuredgrant-Config IE is used to configure uplink transmission without dynamic grant according to Configured Grant Type1 and Configured Grant Type 2. ConfiguredGrant-Config information element ConfiguredGrant-Config ::= SEQUENCE { uplink SEQUENCE { -- Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see 38.213, section FFS_Section) powerControlLoopToUse ENUMERATED {n0, n1}, -- Index of the P0-PUSCH-AlphaSet to be used for this configuration p0-PUSCH-Alpha P0-PUSCH-AlphaSetId, -- Enable transformer precoder for type1 and type2. Absence indicates that it is disabled. -- Corresponds to L1 parameter 'UL-TWG-tp' (see 38.214, section FFS_Section) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- The number of HARQ processes configured. It applies for both Type 1 and Type 2 -- Corresponds to L1 parameter 'UL-TWG-numbHARQproc' (see 38.214, section FFS_Section) nrofHARQ-processes INTEGER(1..ffsValue) OPTIONAL, -- The number or repetitions of K: -- Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see 38.213, section FFS_Section) powerControlLoopToUse ENUMERATED {n0, n1}, -- Index of the P0-PUSCH-AlphaSet to be used for this configuration p0-PUSCH-Alpha P0-PUSCH-AlphaSetId, -- Enable transformer precoder for type1 and type2. Absence indicates that it is disabled. -- Corresponds to L1 parameter 'UL-TWG-tp' (see 38.214, section 6.1.3FFS_Section) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- The number of HARQ processes configured. It applies for both Type 1 and Type 2 -- Corresponds to L1 parameter 'UL-TWG-numbHARQproc' (see 38.321214, section 5.8.2FFS_Section) nrofHARQ-processes INTEGER(1..16ffsValue) OPTIONAL, -- The number or repetitions of K: repK ENUMERATED {n1, n2, n4, n8}, -- If repetitions is used, this field indicates the redundancy version (RV) sequence to use. -- Corresponds to L1 parameter 'UL-TWG-RV-rep' (see 38.214, section 6.1.2FFS_Section) repK-RV ENUMERATED {s1-0231, s2-0303, s3-0000} OPTIONAL, -- Periodicity for UL transmission without UL grant for type 1 and type 2 -- Corresponds to L1 parameter 'UL-TWG-periodicity' (see 38.321, section 5.8.2FFS_Section) -- The following periodicities are supported depending on the configured subcarrier spacing [ms]: -- 15kHz: 2 symbols, 7 symbols, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 -- 30kHz: 2 symbols, 7 symbols, 0.5, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 -- 60kHz: 2 symbols, 7 symbols (6 symbols for ECP), 0.25,0.5,1,2,5,10,20,32, 40, 64, 80, 128, 160, 320, 640 -- 120kHz: 2 symbols, 7 symbols, 0.125,0.25,0.5,1,2,5,10,20, 32, 40, 64, 80, 128, 160, 320, 640 OPTIONAL, periodicity ENUMERATED {sym2, sym7, ms0dot125, ms0dot25, ms0dot5, ms1, ms2, ms5, ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640} OPTIONAL, ConfiguredGrantTimer FFS_Value -- UL-SPS Configured grant transmission with fully RRC-configured UL grant (Type1) (see 38.214, section 6.1.2x.x.x.x). FFS_Ref -- If not provided or set to release, use UL-SPS Configured grant transmission with UL grant configured by DCI addressed to SPScs-RNTI (Type2). rrcConfiguredUplinkGrant CHOICE { setup SEQUENCE { -- Offset related to SFN=0FFS: Merge the following two into one. Possibly don’t use “periodicity” for rrcConfiguredUplinkGrant timeDomainOffset INTEGER(0..ffsValue) ENUMERATED {ffsTypeAndValue}, -- Allocate Time-domain PUSCH resources (see 38.214, Section 6.1.2) timeDomainAllocation INTEGER(0..ffsValue) ENUMERATED {ffsTypeAndValue}, -- RAN1 indicated just "Mapping-type,Index-start-len" -- Determine the resource block assignment in frequency domain (see 38.214, Section 6.1.2) frequencyDomainAllocation INTEGER(0..ffsValue) ENUMERATED {ffsTypeAndValue}, -- UE-specific DMRS configuration: (see 38.214, Section 6.1.2) dmrs INTEGER(0..ffsValue) ENUMERATED {ffsTypeAndValue}, -- Determine the modulation order, target code rate and TB size (see 38.214, Section 6.1.2) mcsAndTBS INTEGER (0..31), -- Enables intra-slot frequency hopping with the given frequency hopping offset -- Corresponds to L1 parameter 'UL-TWG-hopping' (see 38.214, section FFS_Section) frequencyHopping SetupRelease { SEQUENCE { ffs FFS_Value } } OPTIONAL -- Need M }, release NULL } OPTIONAL -- Need M } OPTIONAL -- Need M } |  |
| H247 | RAN2#100 agreed to introduce ConfiguredGrantTimer for both Type 1 and 2, which is also captured into 38.321. However, it was missing in ASN.1. | 2 | 1.Add ConfiguredGrantTimer to ConfiguredGrant-Config IE and values can be FFS pending to RAN2 discussions. #### – ConfiguredGrant-Config The Configuredgrant-Config IE is used to configure uplink transmission without dynamic grant according to Configured Grant Type1 and Configured Grant Type 2. ConfiguredGrant-Config information element ConfiguredGrant-Config ::= SEQUENCE { uplink SEQUENCE { -- Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see 38.213, section FFS_Section) powerControlLoopToUse ENUMERATED {n0, n1}, -- Index of the P0-PUSCH-AlphaSet to be used for this configuration p0-PUSCH-Alpha P0-PUSCH-AlphaSetId, -- Enable transformer precoder for type1 and type2. Absence indicates that it is disabled. -- Corresponds to L1 parameter 'UL-TWG-tp' (see 38.214, section FFS_Section) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- The number of HARQ processes configured. It applies for both Type 1 and Type 2 -- Corresponds to L1 parameter 'UL-TWG-numbHARQproc' (see 38.214, section FFS_Section) nrofHARQ-processes INTEGER(1..ffsValue), -- The number or repetitions of K: -- Closed control loop to apply. Corresponds to L1 parameter 'PUSCH-closed-loop-index' (see 38.213, section FFS_Section) powerControlLoopToUse ENUMERATED {n0, n1}, -- Index of the P0-PUSCH-AlphaSet to be used for this configuration p0-PUSCH-Alpha P0-PUSCH-AlphaSetId, -- Enable transformer precoder for type1 and type2. Absence indicates that it is disabled. -- Corresponds to L1 parameter 'UL-TWG-tp' (see 38.214, section 6.1.2) transformPrecoder ENUMERATED {enabled} OPTIONAL, -- The number of HARQ processes configured. It applies for both Type 1 and Type 2 -- Corresponds to L1 parameter 'UL-TWG-numbHARQproc' (see 38.321, section 5.8.2) nrofHARQ-processes INTEGER(1..16) , -- The number or repetitions of K: repK ENUMERATED {n1, n2, n4, n8}, -- If repetitions is used, this field indicates the redundancy version (RV) sequence to use. -- Corresponds to L1 parameter 'UL-TWG-RV-rep' (see 38.214, section 6.1.2) repK-RV ENUMERATED {s1-0231, s2-0303, s3-0000}, -- Periodicity for UL transmission without UL grant for type 1 and type 2 -- Corresponds to L1 parameter 'UL-TWG-periodicity' (see 38.321, section 5.8.2) -- The following periodicities are supported depending on the configured subcarrier spacing [ms]: -- 15kHz: 2 symbols, 7 symbols, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 -- 30kHz: 2 symbols, 7 symbols, 0.5, 1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640 -- 60kHz: 2 symbols, 7 symbols (6 symbols for ECP), 0.25,0.5,1,2,5,10,20,32, 40, 64, 80, 128, 160, 320, 640 -- 120kHz: 2 symbols, 7 symbols, 0.125,0.25,0.5,1,2,5,10,20, 32, 40, 64, 80, 128, 160, 320, 640 OPTIONAL, periodicity ENUMERATED {sym2, sym7, ms0dot125, ms0dot25, ms0dot5, ms1, ms2, ms5, ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640}, ConfiguredGrantTimer FFS_Value |  |
| H248 | RAN2#100 agreed that Configred grant Type 1 and 2 can be configured per BWP per serving cell. | 2 | 1. The proposed ConfiguredGrant-Config onfig can be configured per BWP per serving cell and therefore it shall be moved to the field of UplinkBandwidthPart IE from ServingCellConfigDedicated IE. UplinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, -- Frequency location of the uplink "direct current" frequency. -- Corresponds to L1 parameter 'UL-BWP-DC'. (see 38.211, section FFS_Section) directCurrentLocation INTEGER (0..3299) OPTIONAL, -- FFS_CHECK: Several (UE specific) BWPs may be configured with RACH resources. Hence, they must be provided with -- the information in RACH-ConfigCommon... even though it is in this case strictly speaking not a cell-specific parameter. -- OK to keep or re-structure the RACH config? rach-ConfigCommon RACH-ConfigCommon, pusch-ConfigCommon PUSCH-ConfigCommon, pusch-Config PUSCH-Config, -- FFS: Is the PUSCH also BWP-specific?? pucch-ConfigCommon PUCCH-ConfigCommon, pucch-Config PUCCH-Config, configuredgrant-Config ConfiguredGrant-Config OPTIONAL – Need R } |  |
| H249 | RAN2 agreed Configured grant and SPS can be released by RRC, which has been reflected in 38.321 1. SPS and Configured Grant (both Type 1 and 2) can be released by RRC | 2 | -- FFS CHECK: Add possibility to release UL SPS Already rrcConfiguredUplinkGrant can be set to “release”. For the change to UplinkBandwidthPart the IE ConfiguredGrant-Config is OPTIONAL need R. |  |
| H250 | frequencyHopping : The exact meaning of this parameter is to be confirmed. In corresponding DCI, it can be 0 or 1 bit depending on the resource allocation type. Frequency-hopping-offset is FFS in RAN1. The freq hopping part is still missing in RAN1 spec, and it is supposed in Section 6.3 or 6.1.2.3 | 2 | -- Enables intra-slot frequency hopping with the given frequency hopping offset -- Corresponds to L1 parameter 'UL-TWG-hopping' (see 38.214, section FFS_Section) frequencyHopping SetupRelease { SEQUENCE { ffs FFS_Value } } OPTIONAL -- Need M |  |

#### –	SRS-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H251 | removal of tpc-SRS-RNTI The tpc-SRS-RNTI is included in the specific IE and therefore is not necessary to be duplicated here. | 2 | -- SRS configuration allowing to add and remove sets of SRS resources SRS-Config ::= SEQUENCE { srs-ResourceSetToReleaseList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need M srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need M srs-ResourceToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need M srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need M -- RNTI used for SRS TPC. Corresponds to L1 parameter 'TPC-SRS-RNTI' (see 38.213, section 10) -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) tpc-SRS-RNTI RNTI-Value OPTIONAL, -- If enabled or absent, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation -- (this applies to SRS when a separate closed loop is configured for SRS) -- Corresponds to L1 parameter 'Accumulation-enabled-srs' (see 38,213, section 7.3) tpcAccumulation ENUMERATED {disabled} OPTIONAL, -- Need R -- Whether UE uses codebook based or non-codebook based transmission. Corresponds to L1 parameter 'ulTxConfig' (see 38.214, section 6.1.1) txConfig ENUMERATED {codebook, nonCodebook} } |  |
| H252 | None of the fields in SRS-CarrierSwitching have a default value specified in the L1 parameters list | 2 | SRS-CarrierSwitching ::= SEQUENCE { -- Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell. -- During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH -- in the same CG to allow the PUSCH-less cell to transmit SRS. (see 38.214, section 6.2.1.3) srs-SwitchFromServCellIndex INTEGER (0..31) OPTIONAL, -- Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any. srs-TPC-PDCCH-Group CHOICE { -- Type A trigger configuration for SRS transmission on a PUSCH-less SCell. -- Corresponds to L1 parameter 'typeA-SRS-TPC-PDCCH-Group' (see 38.212, 38.213, section 7.3.1, 11.3) typeA SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config, -- Type B trigger configuration for SRS transmission on a PUSCH-less SCell. -- Corresponds to L1 parameter 'typeB-SRS-TPC-PDCCH-Config' (see 38.212, 38.213, section 7.3.1, 11.3) typeB SRS-TPC-PDCCH-Config } OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SRS-cell-to-SFI' (see 38.212, 38.213, section 7.3.1, 11.3) srs-CellToSFI SEQUENCE (SIZE (1..maxNrofServingCells)) OF CellToSFI OPTIONAL, -- Monitoring periodicity of SRS PDCCH in number of slots. -- Corresponds to L1 parameter 'SRS-monitoring-periodicity' (see 38.212, 38.213, section 7.3.1, 11.3) monitoringPeriodicity ENUMERATED {n1, n2, n5, n10, n20, spare3, spare2, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SRS-Num-PDCCH-cand' (see 38.212, 38.213, section 7.3.1, 11.3) nrofPDCCH-Candidates ENUMERATED {n1, n2} OPTIONAL, -- A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI -- Corresponds to L1 parameter 'SRS-monitoring-cells' (see 38.212, 38.213, section 7.3.1, 11.3) -- FFS_CHECK: "Could this be on several serving cells? If so, it should be a list, right? -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) moitoringCells INTEGER (0.. 31) OPTIONAL } |  |
| H253 | CellToSFI is not defined, however in PDCCH-Config there is SlotFormatCombinationsPerCell so should update reference and move the definition of SlotFormatCombinationsPerCell | 2 | SRS-CarrierSwitching ::= SEQUENCE { -- Indicates the serving cell whose UL transmission may be interrupted during SRS transmission on a PUSCH-less cell. -- During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH -- in the same CG to allow the PUSCH-less cell to transmit SRS. (see 38.214, section 6.2.1.3) srs-SwitchFromServCellIndex INTEGER (0..31) OPTIONAL, -- Network configures the UE with either typeA-SRS-TPC-PDCCH-Group or typeB-SRS-TPC-PDCCH-Group, if any. srs-TPC-PDCCH-Group CHOICE { -- Type A trigger configuration for SRS transmission on a PUSCH-less SCell. -- Corresponds to L1 parameter 'typeA-SRS-TPC-PDCCH-Group' (see 38.212, 38.213, section 7.3.1, 11.3) typeA SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config, -- Type B trigger configuration for SRS transmission on a PUSCH-less SCell. -- Corresponds to L1 parameter 'typeB-SRS-TPC-PDCCH-Config' (see 38.212, 38.213, section 7.3.1, 11.3) typeB SRS-TPC-PDCCH-Config } OPTIONAL, -- Maps a specific cell to a given SFI value within the DCI message -- Corresponds to L1 parameter 'SRS-cell-to-SFI' (see 38.212, 38.213, section 7.3.1, 11.3) srs-CellToSFI SEQUENCE (SIZE (1..maxNrofServingCells)) OF CellToSFI SlotFormatCombinationsPerCell OPTIONAL, -- Monitoring periodicity of SRS PDCCH in number of slots. -- Corresponds to L1 parameter 'SRS-monitoring-periodicity' (see 38.212, 38.213, section 7.3.1, 11.3) monitoringPeriodicity ENUMERATED {n1, n2, n5, n10, n20, spare3, spare2, spare1} OPTIONAL, -- The number of PDCCH candidates for the configured aggregation level. -- Corresponds to L1 parameter 'SRS-Num-PDCCH-cand' (see 38.212, 38.213, section 7.3.1, 11.3) nrofPDCCH-Candidates ENUMERATED {n1, n2} OPTIONAL, -- A set of serving cells for monitoring PDCCH conveying SRS DCI format with CRC scrambled by TPC-SRS-RNTI -- Corresponds to L1 parameter 'SRS-monitoring-cells' (see 38.212, 38.213, section 7.3.1, 11.3) -- FFS_CHECK: "Could this be on several serving cells? If so, it should be a list, right? -- FFS: RAN1 models different RNTIs (on PDCCH) as different Search Spaces. Do the same here? Group e.g. with monitoring periodicity -- and other PDCCH parameters (if any) moitoringCells INTEGER (0.. 31) OPTIONAL } |  |
| H254 | No default values specified for fields in SRS-TPC-PDCCH-Config | 2 | SRS-TPC-PDCCH-Config ::= SEQUENCE { -- RNTI for SRS trigger and power control using DCI format X (see 38.212, 38.213, section 7.3.1, 11.3) srs-TPC-RNTI RNTI-Value OPTIONAL, -- The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands -- for a PUSCH-less SCell. (see 38.212, 38.213, section 7.3.1, 11.3) startingBitOfFormatX INTEGER (1..31) OPTIONAL, -- The type of a field within the group DCI with SRS request fields (optional) and TPC commands for a PUSCH-less SCell, -- which indicates how many bits in the field are for SRS request (0 or 1/2) and how many bits in the field are for TPC -- (1 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its -- own TPC command bits. See TS 38.212. Network configures this field with the same value for all PUSCH-less SCells. -- (see 38.212, 38.213, section 7.3.1, 11.3) fieldTypeFormatX INTEGER (1..4) OPTIONAL, -- A list of paris of [cc-SetIndex; cc-IndexInOneCC-Set] (see 38.212, 38.213, section 7.3.1, 11.3) -- FFS: Improve description. What is a “CC”? Where is a CC-Set defined? ... srs-CC-SetIndexlist SEQUENCE (SIZE(1..4)) OF SEQUENCE { -- Indicates the CC set index for Type A associated (see 38.212, 38.213, section 7.3.1, 11.3) cc-SetIndex INTEGER (0..3) OPTIONAL, -- Indicates the CC index in one CC set for Type A (see 38.212, 38.213, section 7.3.1, 11.3) cc-IndexInOneCC-Set INTEGER (0..7) OPTIONAL } } |  |
| H255 | srs-CC-SetIndexlist SEQUENCE OF SEQUENCE asn1 crime. should use a similar example to that in 36.331 A.3.7 and split this up (define separate IE for SRS-CC-SetIndex) | 2 | SRS-TPC-PDCCH-Config ::= SEQUENCE { -- RNTI for SRS trigger and power control using DCI format X (see 38.212, 38.213, section 7.3.1, 11.3) srs-TPC-RNTI RNTI-Value OPTIONAL, -- The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands -- for a PUSCH-less SCell. (see 38.212, 38.213, section 7.3.1, 11.3) startingBitOfFormatX INTEGER (1..31) OPTIONAL, -- The type of a field within the group DCI with SRS request fields (optional) and TPC commands for a PUSCH-less SCell, -- which indicates how many bits in the field are for SRS request (0 or 1/2) and how many bits in the field are for TPC -- (1 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its -- own TPC command bits. See TS 38.212. Network configures this field with the same value for all PUSCH-less SCells. -- (see 38.212, 38.213, section 7.3.1, 11.3) fieldTypeFormatX INTEGER (1..4) OPTIONAL, -- A list of paris of [cc-SetIndex; cc-IndexInOneCC-Set] (see 38.212, 38.213, section 7.3.1, 11.3) -- FFS: Improve description. What is a “CC”? Where is a CC-Set defined? ... srs-CC-SetIndexlist SEQUENCE (SIZE(1..4)) OF SRS-CC-SetIndex SEQUENCE { -- Indicates the CC set index for Type A associated (see 38.212, 38.213, section 7.3.1, 11.3) cc-SetIndex INTEGER (0..3) OPTIONAL, -- Indicates the CC index in one CC set for Type A (see 38.212, 38.213, section 7.3.1, 11.3) cc-IndexInOneCC-Set INTEGER (0..7) OPTIONAL } } SRS-CC-SetIndex ::= SEQUENCE { -- Indicates the CC set index for Type A associated (see 38.212, 38.213, section 7.3.1, 11.3) cc-SetIndex INTEGER (0..3), -- Indicates the CC index in one CC set for Type A (see 38.212, 38.213, section 7.3.1, 11.3) cc-IndexInOneCC-Set INTEGER (0..7) } |  |
| H256 | SRS-Resource should refer to SRS-CarrierSwitching of course | 2 | SRS-Resource ::= SEQUENCE { <skip> -- Includes parameters for configuration of carrier based SRS switching -- Corresponds to L1 parameter 'SRS-CarrierSwitching' (see 38,214, section FFS_Section) carrierSwitching SRS-CarrierSwitchingFFS_Value OPTIONAL, <skip> OPTIONAL } |  |
| H257 | aperiodicSRS-ResourceTriggers should be inside SRS-Resource according to L1 parameters list, however is it just missing from SRS-Resource, but needed here too? Or an error in L1 parameter list? | 2 | May need to confirm with RAN1 what is the intended location of this parameter. It seems wrong to have it included inside SRS-ResourceSet. -- A set of SRS resources SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourcesIds SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId, -- The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration. -- FFS: Verify definition and usage. -- Corresponds to L1 parameter 'AperiodicSRS-ResourceTrigger' (see 38.214, section FFS_Section) aperiodicSRS-ResourceTriggers SEQUENCE (SIZE (1..maxNrofSRSTriggerStates)) OF FFS_Value OPTIONAL, -- ID of CSI-RS resource associated with SRS resource set in non-codebook based operation -- Corresponds to L1 parameter 'SRS-AssocCSIRS' (see 38.214, section 6.2.1) associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Indicates if the SRS resource set is used for beam management vs. used for either codebook based or non-codebook based transmission. -- Corresponds to L1 parameter 'SRS-SetUse' (see 38.214, section 6.2.1) -- FFS_CHECK: Isn't codebook/noncodebook already known from the ulTxConfig in the SRS-Config? If so, isn't the only distinction -- in the set between BeamManagement, AtennaSwitching and "Other”? Or what happens if SRS-Config=Codebook but a Set=NonCodebook? usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} OPTIONAL, -- alpha value for SRS power control. Corresponds to L1 parameter 'alpha-srs' (see 38.213, section 7.3) -- When the field is absent the UE applies the value 1 alpha Alpha OPTIONAL, -- Need M -- P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed. -- Corresponds to L1 parameter 'p0-srs' (see 38.213, section 7.3) p0 INTEGER (-202..24) OPTIONAL, -- Need M -- A reference signal (e.g. a CSI-RS config or a SSblock) to be used for SRS path loss estimation. -- Corresponds to L1 parameter 'srs-pathlossReference-rs-config' (see 38.213, section 7.3) pathlossReferenceRS CHOICE { ssb-Index SSB-Index, csirsIndex NZP-CSI-RS-ResourceId } OPTIONAL, -- Need m -- Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) -- or serarate close loop is configured for SRS. This parameter is applicable only for ULs on which UE also transmits PUSCH. -- Corresponds to L1 parameter 'srs-pcadjustment-state-config' (see 38.213, section 7.3) srs-PowerControlAdjustmentStates ENUMERATED {sameAs-Fci1, sameAs-Fci2, separateClosedLoop} OPTIONAL, -- Need M ... } |  |
| H258 | No defaults defined for cyclicShift value, needs to be mandaory. | 2 | SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, -- Comb value (2 or 4) and comb offset. Corresponds to L1 parameter 'SRS-TransmissionComb' (see 38.214, section 6.2.1) -- FFS: What is the “offset”? transmissionComb CHOICE { n2 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..7) OPTIONAL }, n4 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..11) OPTIONAL } |  |
| H259 | resourceType can be replaced by enum | 2 | -- FFS: Add configuration parameters for the different SRS resource types? resourceType ENUMERATED { aperiodic, semi-persistent, periodic} CHOICE { aperiodic SEQUENCE { }, semi-persistent SEQUENCE { }, periodic SEQUENCE { } } |  |
| H260 | The resourceMapping is not correctly reflecting the joint coding from RAN1. RAN1’s conclusion is that the value of each paramater has associations with each other, an example is shown as below: For nrofSymbols=1, the number of possible values for {startPosition , repetitionFactor}={6,1}. For nrofSymbols=2, the number of possible values for {startPosition , repetitionFactor}={5,2}. For nrofSymbols=4, the number of possible values for {startPosition , repetitionFactor}={3,3}. Therefore, the value range of resourceMapping should be a range to index RAN1’s different combinations and we propose as below: resourceMapping INTEGER (0…24) | 2 | resourceMapping SEQUENCE { startPosition INTEGER (0..5), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, resourceMapping INTEGER (0…24) -- each value indicates a specific combination of the above three parameters defined in RAN1 |  |
| H261 | spatialRelationInfo structure can be confirmed | 2 | In our understanding the structure matches RAN1 intention and the FFS can be removed. -- FFS_Value: Check whether the CHOICE below is correct (L1 table was pretty vague). Can an SRS really be an RS for an SRS? spatialRelationInfo CHOICE { ssb-Index SSB-Index, csi-RS NZP-CSI-RS-ResourceId, srs SRS-ResourceId } |  |
| H262 | Max rank Specified as a range 1 … NrofSRS-Ports in L1 table. | 2 | Note that the value 3 is meaningful, because two parameters “maxRank” and “codebooksubset” together as whole will determine the size of TPMI in DCI, as a condidate set of UL precoder. 3 means all precoders from rank 1 to rank 3. maxRank INTEGER (1..4) ENUMERATED {port1, ports2, ports4} OPTIONAL |  |
| H263 | Default is 1, however it may make more sense to uuse need M if value does not change often. | 2 | Consider change as follows if it is preferred to follow the default value from the L1 spreadsheet: -- Corresponds to L1 parameter 'srs-pcadjustment-state-config' (see 38.213, section 7.3) -- If absent, the UE applies the value sameAs-Fci1 srs-PowerControlAdjustmentStates ENUMERATED {sameAs-Fci1, sameAs-Fci2, separateClosedLoop} OPTIONAL, -- Need MS |  |
| H264 | SRS configuration should be per BWP | 3 | The SRS-Config is BWP dependent and therefore should discuss further whether SRS configuration should be included under BWP configuration, or to be a separate IE with some paraters referenced to different BWPs. As far as we understand, SRS numerology, SRS-FreqDomainPosition periodicityAndOffset may be configured per UL BWP, since different numerology has different slot length. |  |
| H265 | Position of txConfig The UL configuration for codebook and non-codebook is a generic transmission mode for PUSCH and therefore seems not appropriate to be included here.this seems better to be included in PUSCH configuration. | 2 | SRS-Config ::= SEQUENCE { srs-ResourceSetToReleaseList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need M srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need M srs-ResourceToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need M srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need M tpc-SRS-RNTI RNTI-Value OPTIONAL, -- If enabled or absent, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation -- (this applies to SRS when a separate closed loop is configured for SRS) -- Corresponds to L1 parameter 'Accumulation-enabled-srs' (see 38,213, section 7.3) tpcAccumulation ENUMERATED {disabled} OPTIONAL, -- Need R -- Whether UE uses codebook based or non-codebook based transmission. Corresponds to L1 parameter 'ulTxConfig' (see 38.214, section 6.1.1) txConfig ENUMERATED {codebook, nonCodebook} } |  |
| H266 | The pathlossReferenceRS might be different for different UL BWP and therefore this part needs to consider per UL BWP configuration. | 3 | SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, BWP ID srs-ResourcesIds SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId, -- The DCI "code point" upon which the UE shall transmit SRS according to this SRS resource set configuration. -- FFS: Verify definition and usage. -- Corresponds to L1 parameter 'AperiodicSRS-ResourceTrigger' (see 38.214, section FFS_Section) aperiodicSRS-ResourceTriggers SEQUENCE (SIZE (1..maxNrofSRSTriggerStates)) OF FFS_Value OPTIONAL, -- ID of CSI-RS resource associated with SRS resource set in non-codebook based operation -- Corresponds to L1 parameter 'SRS-AssocCSIRS' (see 38.214, section 6.2.1) associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Indicates if the SRS resource set is used for beam management vs. used for either codebook based or non-codebook based transmission. -- Corresponds to L1 parameter 'SRS-SetUse' (see 38.214, section 6.2.1) -- FFS_CHECK: Isn't codebook/noncodebook already known from the ulTxConfig in the SRS-Config? If so, isn't the only distinction -- in the set between BeamManagement, AtennaSwitching and "Other”? Or what happens if SRS-Config=Codebook but a Set=NonCodebook? usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching} OPTIONAL, -- alpha value for SRS power control. Corresponds to L1 parameter 'alpha-srs' (see 38.213, section 7.3) -- When the field is absent the UE applies the value 1 alpha Alpha OPTIONAL, -- Need M -- P0 value for SRS power control. The value is in dBm. Only even values (step size 2) are allowed. -- Corresponds to L1 parameter 'p0-srs' (see 38.213, section 7.3) p0 INTEGER (-202..24) OPTIONAL, -- Need M -- A reference signal (e.g. a CSI-RS config or a SSblock) to be used for SRS path loss estimation. -- Corresponds to L1 parameter 'srs-pathlossReference-rs-config' (see 38.213, section 7.3) pathlossReferenceRS CHOICE { ssb-Index SSB-Index, csirsIndex NZP-CSI-RS-ResourceId } OPTIONAL, -- Need m -- Indicates whether hsrs,c(i) = fc(i,1) or hsrs,c(i) = fc(i,2) (if twoPUSCH-PC-AdjustmentStates are configured) -- or serarate close loop is configured for SRS. This parameter is applicable only for ULs on which UE also transmits PUSCH. -- Corresponds to L1 parameter 'srs-pcadjustment-state-config' (see 38.213, section 7.3) srs-PowerControlAdjustmentStates ENUMERATED {sameAs-Fci1, sameAs-Fci2, separateClosedLoop} OPTIONAL, -- Need M ... |  |
| H267 | The offset definition is clear in RAN1 and the FFS can be removed. They are used to determine the starting position of SRS resource at frequency domain with Comb 2/4. In 38.211, “The transmission comb offset is contained in the higher layer parameter SRS-TransmissionComb and is a frequency position index.” | 2 | SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, -- Comb value (2 or 4) and comb offset. Corresponds to L1 parameter 'SRS-TransmissionComb' (see 38.214, section 6.2.1) -- FFS: What is the “offset”? transmissionComb CHOICE { n2 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..7) OPTIONAL }, n4 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..11) OPTIONAL } }, |  |
| H268 | periodicityAndOffset Similar comment as the resourceMapping, 38.211 has already defined Table 6.4.1.4.4-1 where periodicity and slot offset are jointly coded, with a range from 0~5077. Therefore in our understanding this should be a value range to reference to the RAN1 table. | 2 | periodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER(0..1), sl5 INTEGER(0..4), sl10 INTEGER(0..9), sl20 INTEGER(0..19), sl40 INTEGER(0..39), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639), sl1280 INTEGER(0..1279), sl2560 INTEGER(0..2559) }, periodicityAndOffset INTEGER (0…5077) |  |
| H269 | downlinkRefernceSignal | 2 | This IE seems redundant, as we already have associatedCSI-RS which includes the same thing. So this IE can be removed. SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, -- Comb value (2 or 4) and comb offset. Corresponds to L1 parameter 'SRS-TransmissionComb' (see 38.214, section 6.2.1) -- FFS: What is the “offset”? transmissionComb CHOICE { n2 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..7) OPTIONAL }, n4 SEQUENCE { -- Cyclic shift configuration. Corresponds to L1 parameter 'SRS-CyclicShiftConfig' (see 38.214, section 6.2.1) cyclicShift INTEGER (0..11) OPTIONAL } }, -- OFDM symbol location of the SRS resource within a slot including number of OFDM symbols (N = 1, 2 or 4 per SRS resource), -- startPosition (SRSSymbolStartPosition = 0..5; "0" refers to the last symbol, "1" refers to the second last symbol) and -- RepetitionFactor (r = 1, 2 or 4). -- Corresponds to L1 parameter 'SRS-ResourceMapping' (see 38.214, section 6.2.1). -- FFS: Apparently, RAN1 considers replacing these three fields by a table in RAN1 specs and a corresponding index in ASN.1?! resourceMapping SEQUENCE { startPosition INTEGER (0..5), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, -- Parameter(s) defining frequency domain position and configurable shift to align SRS allocation to 4 PRB grid. -- Corresponds to L1 parameter 'SRS-FreqDomainPosition' (see 38.214, section 6.2.1) freqDomainPosition INTEGER (0..67) OPTIONAL, freqDomainShift INTEGER (0..268) OPTIONAL, -- Includes parameters capturing SRS frequency hopping -- Corresponds to L1 parameter 'SRS-FreqHopping' (see 38.214, section 6.2.1) freqHopping SetupRelease { SEQUENCE { c_SRS INTEGER (0..63), b_SRS INTEGER (0..3), b_hop INTEGER (0..3) } } OPTIONAL, -- Need M -- Parameter(s) for configuring group or sequence hopping -- Corresponds to L1 parameter 'SRS-GroupSequenceHopping' (see 38.211, section FFS_Section) groupOrSequenceHopping ENUMERATED { neither, groupHopping, sequenceHopping } OPTIONAL, -- Time domain behavior of SRS resource configuration -- Corresponds to L1 parameter 'SRS-ResourceConfigType' (see 38.214, section 6.2.1) -- FFS: Add configuration parameters for the different SRS resource types? resourceType CHOICE { aperiodic SEQUENCE { }, semi-persistent SEQUENCE { }, periodic SEQUENCE { } } OPTIONAL, -- Need M -- Periodicity and slot offset for periodic/semi-persistent SRS. All values in "number of slots" -- Corresponds to L1 parameter 'SRS-SlotConfig' (see 38.214, section 6.2.1) periodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER(0..1), sl5 INTEGER(0..4), sl10 INTEGER(0..9), sl20 INTEGER(0..19), sl40 INTEGER(0..39), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), sl640 INTEGER(0..639), sl1280 INTEGER(0..1279), sl2560 INTEGER(0..2559) }, -- Sequence ID used to initialize psedo random group and sequence hopping. -- Corresponds to L1 parameter 'SRS-SequenceId' (see 38.214, section 6.2.1) sequenceId BIT STRING (SIZE (10)), -- Includes parameters for configuration of carrier based SRS switching -- Corresponds to L1 parameter 'SRS-CarrierSwitching' (see 38,214, section FFS_Section) carrierSwitching FFS_Value OPTIONAL, -- Reference to a CSI-RS for UE to to calculate UL candidate precoders for precoded periodic/sem-persistent SRS -- FFS_CHECK: Is this parameter meant to be here? It was listed under CSI/BeamManagement. -- FFS_Value: Is it correct that this can only be a CSI-RS resource? The spatialRelationInfo below says that it could be SSB, too?! -- Corresponds to L1 parameter 'DlMeasRSRef' (see 38.213, section FFS_Section) downlinkRefernceSignal NZP-CSI-RS-ResourceId OPTIONAL, |  |
| H270 | Csi-RS It is a bit unclear whether the SRS ID and NZP ID is a global ID, if not we may need set ID+NZP ID+SRS ID to uniquely identify it. Confirmation needed with RAN1. | 3 | csi-RS NZP-CSI-RS-ResourceId, |  |
| H272 | Range of startingBitOfFormat2_3 to be confirmed with RAN1 | 2 | We are unsure what is the source of this range. RAN1 input needed -- The starting bit position of a block within the group DCI with SRS request fields (optional) and TPC commands -- for a PUSCH-less SCell. (see 38.212, 38.213, section 7.3.1, 11.3) startingBitOfFormat2_3 INTEGER (1.. 31) |  |

#### –	SubcarrierSpacing

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	TDD-UL-DL-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H273 | Wrong range for nrofDownlinkSymbols | 2 | Range goes to maxSymbolIndex=13, should go to maxSymbolIndex+1=14. TDD-UL-DL-ConfigCommon ::= SEQUENCE { -- Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific -- virutal carriers, i.e., independent of the actual subcarrier spacing using for data transmission. -- Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section) referenceSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Periodicity of the DL-UL pattern. Corresponds to L1 parameter 'DL-UL-transmission-periodicity' (see 38.211, section FFS_Section) dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, -- Number of consecutive full DL slots at the beginning of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-DL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofDownlinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). -- Corresponds to L1 parameter 'number-of-DL-symbols-common' (see 38.211, section FFS_Section). nrofDownlinkSymbols INTEGER (0..maxSymbolIndexPlus1) OPTIONAL, -- Number of consecutive full UL slots at the end of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-UL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofUplinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). -- Corresponds to L1 parameter 'number-of-UL-symbols-common' (see 38.211, section FFS_Section) nrofUplinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL } [...] ### – Multiplicity and type constraint definitions -- ASN1START -- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START maxBandComb INTEGER ::= ffsValue -- Maximum number of DL band combinations maxBasebandProcComb INTEGER ::= ffsValue -- Maximum number of base band processing combinations maxNrofServingCells INTEGER ::= 16 -- Max number of serving serving cells (SpCell + SCells) per cell group maxNrofSCells INTEGER ::= 15 -- Max number of secondary serving cells per cell group maxNrofCellMeas INTEGER ::= ffsValue -- Maximum number of entries in each of the cell lists in a measurement object maxNrofSS-BlocksToAverage INTEGER ::= ffsValue -- Max number for the (max) number of SS blocks to average to determine cell measurement maxNrofCSI-RS-ResourcesToAverage INTEGER ::= ffsValue -- Max number for the (max) number of CSI-RS to average to determine cell measurement maxNrofSR-ConfigPerCellGroup INTEGER ::= 8 -- Maximum number of SR configurations per cell group maxLCG-ID INTEGER ::= 7 -- Maximum value of LCG ID macLC-ID INTEGER ::= ffsValue -- Maximum value of Logical Channel ID maxNrofTAGs INTEGER ::= 4 -- Maximum number of Timing Advance Groups maxNrofTAGs-1 INTEGER ::= 3 -- Maximum number of Timing Advance Groups minus 1 maxNrofBandwidthParts INTEGER ::= 4 -- Maximum number of BWPs per serving cell maxNrofBandwidthParts-1 INTEGER ::= 3 -- Maximum number of BWPs per serving cell minus 1 maxSymbolIndex INTEGER ::= 13 -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13) maxSymbolIndexPlus1 INTEGER ::= 14 -- Maximum index identifying a symbol within a slot plus 1 |  |
| H274 | Wrong range for nrofUplinkSymbols | 2 | Range goes to maxSymbolIndex=13, should go to maxSymbolIndex+1=14. TDD-UL-DL-ConfigCommon ::= SEQUENCE { -- Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific -- virutal carriers, i.e., independent of the actual subcarrier spacing using for data transmission. -- Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section) referenceSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Periodicity of the DL-UL pattern. Corresponds to L1 parameter 'DL-UL-transmission-periodicity' (see 38.211, section FFS_Section) dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, -- Number of consecutive full DL slots at the beginning of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-DL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofDownlinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). -- Corresponds to L1 parameter 'number-of-DL-symbols-common' (see 38.211, section FFS_Section). nrofDownlinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL, -- Number of consecutive full UL slots at the end of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-UL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofUplinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). -- Corresponds to L1 parameter 'number-of-UL-symbols-common' (see 38.211, section FFS_Section) nrofUplinkSymbols INTEGER (0..maxSymbolIndexPlus1) OPTIONAL } |  |
| H275 | Wrong range for nrofDownlinkSlots | 2 | Range goes to 160, should go to maximum number of slots per frame=320 (per Table 4.3.2-1 of 38.211) TDD-UL-DL-ConfigCommon ::= SEQUENCE { -- Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific -- virutal carriers, i.e., independent of the actual subcarrier spacing using for data transmission. -- Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section) refereceSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Periodicity of the DL-UL pattern. Corresponds to L1 parameter 'DL-UL-transmission-periodicity' (see 38.211, section FFS_Section) dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, -- Number of consecutive full DL slots at the beginning of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-DL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofDownlinkSlots INTEGER (0..160320) OPTIONAL, -- Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). -- Corresponds to L1 parameter 'number-of-DL-symbols-common' (see 38.211, section FFS_Section). nrofDownlinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL, -- Number of consecutive full UL slots at the end of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-UL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofUplinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). -- Corresponds to L1 parameter 'number-of-UL-symbols-common' (see 38.211, section FFS_Section) nrofUplinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL } |  |
| H276 | Wrong range for nrofUplinkSlots | 2 | Range goes to 160, should go to maximum number of slots per frame=320 (per Table 4.3.2-1 of 38.211) TDD-UL-DL-ConfigCommon ::= SEQUENCE { -- Reference SCS used to determine the time domain boundaries in the UL-DL pattern which must be common across all subcarrier specific -- virutal carriers, i.e., independent of the actual subcarrier spacing using for data transmission. -- Corresponds to L1 parameter 'reference-SCS' (see 38.211, section FFS_Section) refereceSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Periodicity of the DL-UL pattern. Corresponds to L1 parameter 'DL-UL-transmission-periodicity' (see 38.211, section FFS_Section) dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, -- Number of consecutive full DL slots at the beginning of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-DL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofDownlinkSlots INTEGER (0..160) OPTIONAL, -- Number of consecutive DL symbols in the beginning of the slot following the last full DL slot (as derived from nrofDownlinkSlots). -- Corresponds to L1 parameter 'number-of-DL-symbols-common' (see 38.211, section FFS_Section). nrofDownlinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL, -- Number of consecutive full UL slots at the end of each DL-UL pattern. -- Corresponds to L1 parameter 'number-of-UL-slots' (see 38.211, section FFS_Section) -- FFS_Value: Verify that 160 is correct (maximum number of slots within the longest period (10 ms) and the largest subcarrier spacing. nrofUplinkSlots INTEGER (0..160320) OPTIONAL, -- Number of consecutive UL symbols in the end of the slot preceding the first full UL slot (as derived from nrofUplinkSlots). -- Corresponds to L1 parameter 'number-of-UL-symbols-common' (see 38.211, section FFS_Section) nrofUplinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL } |  |

#### 6.3.3	UE capability information elements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H277 | RAT-type doesn’t need both a spare value and an extension marker. | 2 | RAT-Type ::= ENUMERATED {nr, mrdc, spare1, ...} |  |
| H278 | RetuningTimeInfo is missing from asn1. | 2 | The UE capability needs to be added (similar to RetuningTimeInfo in LTE) but since it is a per-band-pair capability it is not yet obvious how to add it. One possibility would be to capture it as a per-band capability with a list of entries, so that the structure in LTE signalling can be re-used. BandNR ::= SEQUENCE { bandNR FreqBandIndicatorNR, supportedMIMO-CapabilityDL MIMO-Capability OPTIONAL, supportedMIMO-CapabilityUL MIMO-Capability OPTIONAL, retuningTimeInfoBandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF RetuningTimeInfo OPTIONAL -- FFS on other parameters } RetuningTimeInfo ::= SEQUENCE { retuningInfo SEQUENCE { rf-RetuningTimeDL ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7, spare1} OPTIONAL, rf-RetuningTimeUL ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7, spare1} OPTIONAL } } |  |

#### 6.3.4	Other information elements

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 6.4	RRC multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H279 | ARFCN-ValueNR can be populated | 2 | RAN4 have now agreed the ARFCN has a range of 0..3279167. The IE can be populated and should be moved to a permanent location. ARFCN-ValueNR ::= ENUMERATED {ffsTypeAndValue}INTEGER (0..3279167) |  |

#### 7	Variables and constants

#### 7.1	Timers

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.1.1	Timers (Informative)

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.1.2	Timer handling

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.2	Counters

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.3	Constants

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 7.4	UE variables

#### –	VarMeasConfig

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	VarMeasReportList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8	Protocol data unit abstract syntax

#### 8.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.2	Structure of encoded RRC messages

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.3	Basic production

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.4	Extension

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 8.5	Padding

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9	Specified and default radio configurations

#### 9.1	Specified configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H280 | The logicalChannelIdentity in SRB configurations is fixed and should be included in specified configurations. | 2 | Add SRB configurations subsection: 9.1.1 SRB configurations #### 9.1.1.1 SRB1/SRB1S Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 1. #### 9.1.1.2 SRB2/SRB2S Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 2. #### 9.1.1.3 SRB3 Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 3. |  |

#### 9.2	Default radio configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H281 | The logicalChannelIdentity of SRBs are captured as default configurations, which means the logical channel IDs can be changed. However, the logical channel IDs of the SRBs are fixed and should not be changed. Related to the above comment. | 2 | Remove the logical channel ID configurations of SRBs in 9.2.2: ### 9.2.2 SRB configurations #### 9.2.2.1 SRB1/SRB1S Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 1. #### 9.2.2.2 SRB2/SRB2S Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 2. #### 9.2.2.3 SRB3 Parameters Name; Value; Semantics description; Ver. RLC configuration. logicalChannelIdentity; 3. |  |

#### 9.2.1	SRB configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.1	SRB1/SRB1S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.2	SRB2/SRB2S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.1.3	SRB3

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2	SRB configurations

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.1	SRB1/SRB1S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.2	SRB2/SRB2S

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 9.2.2.3	SRB3

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10	Generic error handling

#### 10.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.2	ASN.1 violation or encoding error

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.3	Field set to a not comprehended value

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.4	Mandatory field missing

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 10.5	Not comprehended field

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11	Radio information related interactions between network nodes

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2	Inter-node RRC messages

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2.1	General

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.2.2	Message definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	HandoverCommand

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	HandoverPreparationInformation

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |
| H282 | Inconsistent IE name. |  | AS-Context ::= SEQUENCE { reestablishmentInfo SEQUENCE { sourcePhysCellId PhysCellId, targetCellShortMAC-I ShortMAC-I, additionalReestabInfoList AdditionalReestabInfoList OPTIONAL } OPTIONAL, -- FFS Whether to change e.g. move all re-establishment info to Xx configRestrictInfo ConfigRestrictInfoSCG OPTIONAL, ... } ReestabNCellInfoListAdditionalReestabInfoList ::= SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo ReestabNCellInfo::= SEQUENCE{ cellIdentity CellIdentity, key-gNodeB-Star BIT STRING (SIZE (256)), shortMAC-I ShortMAC-I } |  |

#### –	SCG-Config

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	SCG-ConfigInfo

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.3	Inter-node RRC information element definitions

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### –	CandidateCellInfoList

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 11.4	Inter-node RRC multiplicity and type constraint values

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### 12	Processing delay requirements for RRC procedures

| I-No | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- |

#### Annex A (informative):	Guidelines, mainly on use of ASN.1

#### A.3.8	Guidelines on use of parameterised SetupRelease type

#### –	ParentIE-WithEM

#### –	ChildIE1-WithoutEM

#### –	ChildIE2-WithoutEM

#### A.6	Guidelines regarding use of need codes

#### Annex <X> (informative): Change history	180

# Sections not part of the review (for information)

-

# List of last I-No (Issue Number)

Companies indicate their last used I-No, to avoid duplication.

| Company | Last used I-No |
| --- | --- |