3GPP TSG-RAN WG2 Meeting #101	R2-180xxxx

# Athens, Greece, 26th February - 2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on EN-DC - CSI L1 parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections identified during ASN.1 review (RAN2 NR AH 1801), and email discussions after the AH. This CR is based on R2-1801218 Baseline TS 38331 v1.0.1 for ASN.1 review |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be completed. Guidance for CR editors: - To avoid change marks for language formatting (typically happens when many users edit the same doc), please do the following word setting: Review panel => Language => Set proofing languge => Detect automatically => OFF - Set the “User name” to indicate the company name. - When storing the CR in 3GPP folder, companies should add their Company ID (one letter) to the file name (see RIL). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

[AdditionalSpectrumEmission, Alpha, ARFCN-ValueNR, BandwidthPart-Config, BeamFailureDetectionConfig, BeamFailureRecoveryConfig, CellGroupConfig, ControlResourceSet, CrossCarrierSchedulingConfig]

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with list of trigger states for measuring aperiodic CSI-RS (reference signals) and for reporting those measurements on L1 (PUCCH) as channel state information. See also 38.214, section 5.2.

CSI-AperiodicTriggerStateList information element

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

AperiodicTriggerStateList ::=	SEQUENCE (SIZE (1.. maxNrOfCSI-AperiodicTriggers)) OF AperiodicTriggerState

AperiodicTriggerState ::=		SEQUENCE {

associatedReportConfigInfo		SEQUENCE (SIZE(1.. maxNrofReportConfigPerAperiodicTrigger)) OF AssociatedReportConfigInfo,

...

}

AssociatedReportConfigInfo ::=				SEQUENCE {

-- The reportConfigId of one of the CSI-ReportConfigToAddMod configured in CSI-MeasConfig

reportConfigId								CSI-ReportConfigId,

-- Indicates a NZP-CSI-RS-ResourceSet for channel measurement and provides QCL information per resource in this NZP-CSI-RS-ResourceSet

nzp-CSI-RS-ResourcesforChannel				CSI-ResourceSet-and-QCL-Info,

-- The ID of a CSI-IM-ResourceSet for interference measurement as defined in the csi-IM-ResourceSetToAddModList. This ID must

-- also be present in the csi-IM-ResourceSetsList of the csi-IM-ResourcesForInterference that is included in the CSI-ReportConfig

Value 1 refers to the first entry in

-- in that csi-IM-ResourceSetList, 2 to the second entry, and so on.

csi-IM-ResourcesforInteference				INTEGER(1.. maxNrofCSI-IM-ResourceSetsPerConfig)		OPTIONAL, --Cond CSI-IM-forInterference

-- Indicates a NZP-CSI-RS-ResourceSet for interference measurement. The indicated NZP-CSI-RS-ResourceSet should have exactly the same number of

-- resources like the NZP-CSI-RS-ResourceSet indicated in nzp-CSI-RS-ResourcesforChannel.

-- This field can only be present if the CSI-ReportConfig identified by reportConfigId includes nzp-CSI-RS-ResourcesForInterference. If present,

-- its value is the index of an entry in nzp-CSI-RS-ResourceSetList included in the CSI-ResourceConfig identified by

-- nzp-CSI-RS-ResourcesForInterference included in the CSI-ReportConfig identified by reportConfigId. Value 1 refers to the first entry in

-- in that nzp-CSI-RS-ResourceSetList, 2 to the second entry, and so on.

nzp-CSI-RS-ResourcesforInterference			NZP-CSI-RS-ResourceSetId							OPTIONAL, --Cond LinkedNZP-CSI-RS-forInterference

...

}

CSI-ResourceSet-and-QCL-Info ::=		SEQUENCE {

-- Indication of a NZP-CSI-RS-ResourceSet within a csi-ResourceConfig (1 corresponds to the first entry in nzp-CSI-RS-ResourceSetList,

--	2 corresponds to the second entry, and so on).

resourceSetIndex							INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

-- List of references to TCI-States for providing the QCL source and QCL type for for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources

-- of the NZP-CSI-and-SS-PBCH-ResourceSet identified by resourceSetId. qcl-info has the same number of items like nzp-CSI-RS-Resources in

-- the the NZP-CSI-and-SS-PBCH-ResourceSet identified by resourceSetId and first entry in qcl-info corresponds to first entry in

-- nzp-CSI-RS-Resources, second entry in qcl-info corresponds to second entry in nzp-CSI-RS-Resources, and so on.

-- Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section 5.2.1.5.1)

qcl-info								SEQUENCE (SIZE(1.. maxNrofNZP-CSI-RS-ResourcesPerSet)) OF TCI-StatedId

}

maxNrOfCSI-AperiodicTriggers				INTEGER ::= 127		-- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger		INTEGER ::= 16		-- Maximum number of report configurations associated to a trigger state for aperiodic CSI reporting

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

#### –	CSI-MeasConfig

The CSI-MeasConfig IE is used to configure the UE for measuring CSI-RS (reference signals) and for reporting those measurements on L1 (PUCCH, PUSCH) as channel state information. See also 38.214, section 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

CSI-MeasConfig ::=					SEQUENCE {

-- Pool of NZP CSI-RS Resources which can be referred to from NZP-CSI-RS-ResourceSet

nzp-CSI-RS-ResourceToAddModList		SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceToAddMod			OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId				OPTIONAL, -- Need N

-- Pool of NZP CSI-RS Resource sets which can be referred to from CSI-ResourceConfig or from MAC CEs

nzp-CSI-RS-ResourceSetToAddModList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetToAddMod	OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId			OPTIONAL, -- Need N

-- Pool of CSI-IM Resources which can be referred to from CSI-IM-ResourceSet

csi-IM-ResourceToAddModList			SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceToAddMod					OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId						OPTIONAL, -- Need N

-- Pool of CSI-IM Resource sets which can be referred to from CSI-ResourceConfig or from MAC CEs

csi-IM-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetToAddMod			OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId					OPTIONAL, -- Need N

-- Pool of SS/PBCH block resource sets which can be referred to from CSI-ResourceConfig

csi-SSB-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetToAddMod			OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddReleaseList	SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId				OPTIONAL, -- Need N

-- Configured CSI resource settings as specified in TS 38.214 section 5.2.1.2

csi-ResourceConfigToAddModList 				SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigToAddMod			OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList 	SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId			OPTIONAL, -- Need N

-- Configured CSI report settings as specified in TS 38.214 section 5.2.1.1

csi-ReportConfigToAddModList					SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigToAddMod								OPTIONAL, -- Need N

csi-ReportConfigToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId				OPTIONAL, -- Need N

-- Size of CSI request field in DCI (bits). Corresponds to L1 parameter 'ReportTriggerSize' (see 38.214, section 5.2)

reportTriggerSize				INTEGER (0..6)																				OPTIONAL,

-- Contains trigger states for dynamically selecting one or more aperiodic and semi-persistent reporting configurations

-- and/or triggering one or more aperiodic CSI-RS resource sets for channel and/or interference measurement.

-- FFS: How to address the MAC-CE configuration

reportTrigger					SEQUENCE {

aperiodic						SetupRelease { CSI-aperiodicTriggerStateList }											OPTIONAL, -- Need M

semiPersistentOnPUSCH			SetupRelease { CSI-semiPersistentOnPUSCH-TriggerStateList }								OPTIONAL  -- Need M

},

...

}

maxNrofNZP-CSI-RS-ResourceSets		INTEGER ::= 		64

maxNrofNZP-CSI-RS-ResourceSets-1	INTEGER ::= 		63

maxNrofCSI-SSB-ResourceSets 		INTEGER ::=			64

maxNrofCSI-SSB-ResourceSets-1 		INTEGER ::=			63

maxNrofCSI-IM-ResourceSets 			INTEGER ::=			64

maxNrofCSI-IM-ResourceSets-1		INTEGER ::=			63

-- TAG-CSI-MEAS-CONFIG-STOP

-- ASN1STOP

#### –	CSI-ResourceConfigToAddMod

The IE CSI-ResourceConfigToAddMod refers to one or more NZP-CSI-RS-ResourceSets, CSI-IM-ResourceSet and/or SS/PBCH blocks resources.

CSI-ResourceConfigToAddMod information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGTOADDMOD-START

-- One CSI resource configuration comprising of one or more resource sets

CSI-ResourceConfigToAddMod ::= 				SEQUENCE {

-- Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig

csi-ResourceConfigId				CSI-ResourceConfigId,

-- Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if ResourceConfigType is 'aperiodic' and 1 otherwise.

-- Corresponds to L1 parameter 'ResourceSetConfigList' (see 38.214, section 5.2.1.3.1)

csi-RS-ResourceSetList 	 			CHOICE {

nzp-CSI-RS-ResourceSetList				SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId,

csi-IM-ResourceSetList					SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

-- List of SSB resources used for beam measurement and reporting in a resource set

-- Corresponds to L1 parameter 'resource-config-SS-list' (see 38,214, section FFS_Section)

csi-SSB-ResourceSetList						SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-	SSB-ResourceSetId		OPTIONAL, --Cond OnlyWithNZPResourceSets

-- The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in.

-- Corresponds to L1 parameter 'BWP-Info' (see 38.214, section 5.2.1.2

bwp-Id								BWP-Id,

-- Time domain behavior of resource configuration. Corresponds to L1 parameter 'ResourceConfigType' (see 38.214, section 5.2.2.3.5)

resourceType							ENUMERATED { aperiodic, semiPersistent, periodic },

...

}

-- TAG-CSI-RESOURCECONFIGTOADDMOD-STOP

-- ASN1STOP

#### –	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::= INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

#### –	CSI-semiPersistentOnPUSCH-TriggerStateList

The CSI-semiPersistentOnPUSCH-TriggerStateList IE is used to configure the UE with list of trigger states for measuring semi-persistent CSI-RS (reference signals) and for reporting those measurements on L1 (PUSCH) as channel state information. See also 38.214, section 5.2.

CSI-semiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-semiPersistentOnPUSCH-TriggerStateList ::=	SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-semiPersistentOnPUSCH-TriggerState

CSI-semiPersistentOnPUSCH-TriggerState ::=		SEQUENCE {

associatedReportConfigInfo						CSI-ReportConfigId,

...

}

maxNrOfSemiPersistentPUSCH-Triggers					INTEGER ::= 64		-- Maximum number of triggers for semi persistent reporting on PUSCH

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSetToAddMod

The IE NZP-CSI-RS-ResourceSetToAddMod is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters.

NZP-CSI-RS-ResourceSetToAddMod information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETTOADDMOD-START

NZP-CSI-RS-ResourceSetToAddMod ::= 					SEQUENCE {

nzp-CSI-ResourceSetId					NZP-CSI-RS-ResourceSetId,

-- NZP-CSI-RS-Resources assocaited with this NZP-CSI-RS resource set.

-- Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2)

-- For CSI, there are at most 8 NZP CSI RS resources per resource set

nzp-CSI-RS-Resources						SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

-- Indicates whether repetition is on/off.

If set to set to 'OFF', the UE may not assume that the

-- NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter

-- and with same NrofPorts in every symbol.

-- Corresponds to L1 parameter 'CSI-RS-ResourceRep' (see 38.214, sections 5.2.2.3.1 and 5.1.6.1.2)

repetition									ENUMERATED { on, off },

-- Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the

-- CSI-RS resource set is transmitted. When the field is absent the UE applies the value 0.

-- Corresponds to L1 parameter 'Aperiodic-NZP-CSI-RS-TriggeringOffset' (see 38,214, section FFS_Section)

aperiodicTriggeringOffset			 		INTEGER(0..4)																		OPTIONAL,	-- Need S

-- Indicates that the antenna port for all NZP-CSI-RS resources in the CSI-RS resource set is same.

-- Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1)

trs-Info								ENUMERATED {true}																OPTIONAL,

...

}

-- TAG-NZP-CSI-RS-RESOURCESETTOADDMOD-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceSetId

The IE NZP-CSI-RS-ResourceSetId is used to identify one NZP-CSI-RS-ResourceSet.

NZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESETID-START

NZP-CSI-RS-ResourceSetId ::= INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- TAG-NZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceToAddMod

The IE NZP-CSI-RS-ResourceToAddMod is used to configure Non-Zero-Power (NZP) CSI-RS-Resource, which the UE may be configured to measure on (see 38.214, section 5.2.2.3.1).

NZP-CSI-RS-ResourceToAddMod information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCETOADDMOD-START

NZP-CSI-RS-ResourceToAddMod ::=					SEQUENCE {

nzp-CSI-RS-ResourceId				NZP-CSI-RS-ResourceId,

-- Number of ports (see 38.214, section 5.2.2.3.1)

nrofPorts								ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

-- OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource

resourceMapping							CSI-RS-ResourceMapping

,

-- Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, sections 5.2.2.3.1 and 4.1)

powerControlOffset						INTEGER(-8..15),

-- Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section 5.2.2.3.1)

powerControlOffsetSS					ENUMERATED{db-3, db0, db3, db6}													OPTIONAL,

-- Scrambling ID (see 38.214, section 5.2.2.3.1)

scramblingID							ScramblingId,

-- Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on.

-- The corresponding offset is also given in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1)

periodicityAndOffset					CSI-ResourcePeriodicityAndOffset

OPTIONAL, --Cond PeriodicOrSemiPersistent

-- For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and

-- QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS.

-- Corresponds to L1 parameter 'QCL-Info-PeriodicCSI-RS' (see 38.214, section 5.2.2.3.1)

qcl-InfoPeriodicCSI-RS						TCI-StateId												OPTIONAL, --Cond Periodic

...

}

-- TAG-NZP-CSI-RS-RESOURCETOADDMOD-STOP

-- ASN1STOP

#### –	CSI-FrequencyOccupation

The IE CSI-FrequencyOccupation is used to configure the frequency domain occupation of a channel state information measurement resource (e.g. NZP-CSI-RS-Resource, CSI-IM-Resource).

CSI-FrequencyOccupation information element

-- ASN1START

-- TAG-CSI-FREQUENCYOCCUPATION-START

CSI-FrequencyOccupation ::=				SEQUENCE {

-- PRB where this CSI resource starts in relation to PRB 0 of the associated BWP.

-- Only multiples of 4 are allowed (0, 4, ...)

startingRB								INTEGER (0..maxNrofPhysicalResourceBlocks-1),

-- Number of PRBs across which this CSI resource spans. Only multiples of 4 are allowed. The smallest configurable

-- number is the minimum of 24 and the width of the associated BWP.

nrofRBs									INTEGER (24..maxNrofPhysicalResourceBlocks)

}

-- TAG-CSI-FREQUENCYOCCUPATION-STOP

-- ASN1STOP

#### –	CSI-RS-ResourceMapping

The IE CSI-RS-ResourceMapping is used to configure the resource element mapping of a CSI-RS resource in time- and frequency domain.

CSI-RS-ResourceMapping information element

-- ASN1START

-- TAG-CSI-RS-RESOURCEMAPPING-START

CSI-RS-ResourceMapping ::= 				SEQUENCE {

-- Frequency domain allocation within a physical resource block in accordance with 38.211, section 7.4.1.5.3 including table 7.4.1.5.2-1.

-- The number of bits that may be set to one depend on the chosen row in that table. For the choice "other", the row can be determined from

-- the parmeters below and from the number of bits set to 1 in frequencyDomainAllocation.

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12)),

row4								BIT STRING (SIZE (3)),

other								BIT STRING (SIZE (6))

},

-- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS.

-- Parameter l0 in 38.211, section 7.4.1.5.3. Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

firstOFDMSymbolInTimeDomain			INTEGER (0..13),

-- Time domain allocation within a physical resource block. Parameter l1 in 38.211, section 7.4.1.5.3.

firstOFDMSymbolInTimeDomain2		INTEGER (0..13)																OPTIONAL,	-- Need R

-- CDM type (see 38.214, section 5.2.2.3.1)

cdm-Type								ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

-- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density'  (see 38.211, section 7.4.1.5.3)

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

density									CHOICE {

dot5									ENUMERATED {evenPRBs, oddPRBs},

one										NULL,

three									NULL,

spare									NULL

},

-- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1)

freqBand								CSI-FrequencyOccupation,

...

}

-- TAG-CSI-RS-RESOURCEMAPPING-STOP

-- ASN1STOP

#### –	NZP-CSI-RS-ResourceId

The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource.

NZP-CSI-RS-ResourceId information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCEID-START

NZP-CSI-RS-ResourceId ::= 					INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

-- TAG-NZP-CSI-RS-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSetToAddMod

The IE CSI-IM-ResourceSetToAddMod is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters.

CSI-IM-ResourceSetToAddMod information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETTOADDMOD-START

CSI-IM-ResourceSetToAddMod ::=					SEQUENCE {

csi-IM-ResourceSetId					CSI-IM-ResourceSetId,

-- CSI-IM-Resources associated with this CSI-IM-ResourceSet

-- Corresponds to L1 parameter 'CSI-IM-ResourceConfigList' (see 38.214, section 5.2)

csi-IM-Resources						SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId,

...

}

-- TAG-CSI-IM-RESOURCESETTOADDMD-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceSetId

The IE CSI-IM-ResourceSetId is used to identify CSI-IM-ResourceSets.

CSI-IM-ResourceSetId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCESETID-START

CSI-IM-ResourceSetId ::=				INTEGER (0..maxNrofCSI-IM-ResourceSets)

-- TAG-CSI-IM-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceToAddMod

The IE CSI-IM-ResourceToAddMod is used to configure one CSI Interference Management (IM) resource.

CSI-IM-ResourceToAddMod information element

-- ASN1START

-- TAG-CSI-IM-RESOURCETOADDMOD-START

CSI-IM-ResourceToAddMod ::= 					SEQUENCE {

csi-IM-ResourceId						CSI-IM-ResourceId,

-- The resource element pattern (Pattern0 (2,2) or Pattern1 (4,1)) with corresponding parameters.

-- Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4)

csi-IM-ResourceElementPattern				CHOICE {

pattern0									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p0						ENUMERATED { s0, s2, s4, s6, s8, s10 },

-- OFDM symbol location of the CSI-IM resource for Pattern0

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p0							INTEGER (0..12)

},

pattern1									SEQUENCE {

-- OFDM subcarrier occupancy of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

subcarrierLocation-p1						ENUMERATED { s0, s4, s8 },

-- OFDM symbol location of the CSI-IM resource for Pattern1

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

symbolLocation-p1							INTEGER (0..13)

}

}																														OPTIONAL,	--Need M

-- Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2)

freqBand									CSI-FrequencyOccupation											OPTIONAL,	-- Need M

-- Periodicity and slot offset for periodic/semi-persistent CSI-IM. Corresponds to L1 parameter 'CSI-IM-timeConfig'

periodicityAndOffset					CSI-ResourcePeriodicityAndOffset

OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

-- TAG-CSI-IM-RESOURCETOADDMOD-STOP

-- ASN1STOP

#### –	CSI-IM-ResourceId

The IE CSI-IM-ResourceId is used to identify one CSI-IM-Resource.

CSI-IM-ResourceId information element

-- ASN1START

-- TAG-CSI-IM-RESOURCEID-START

CSI-IM-ResourceId ::= 			INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-IM-RESOURCEID-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSetId

The IE CSI-SSB-ResourceSetId is used to identify one SS/PBCH block resource set.

CSI-SSB-ResourceId information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETID-START

CSI-SSB-ResourceSetId ::=	INTEGER (0.. maxNrofCSI-SSB-ResourceSets-1)

-- TAG-CSI-SSB-RESOURCESETID-STOP

-- ASN1STOP

#### –	CSI-SSB-ResourceSetToAddMod

The IE CSI-SSB-ResourceSetToAddMod is used to configure one SS/PBCH block resource set.

CSI-SSB-ResourceSetToAddMod information element

-- ASN1START

-- TAG-CSI-SSB-RESOURCESETTOADDMOD-START

CSI-SSB-ResourceSetToAddMod ::=						SEQUENCE {

csi-SSB-ResourceSetId						CSI-SSB-ResourceSetId,

csi-SSB-ResourceList						SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...

}

-- TAG-CSI-SSB-RESOURCESETTOADDMOD-STOP

-- ASN1STOP

#### –	CSI-ReportConfigToAddMod

The IE CSI-ReportConfigToAddMod is used to configure reports sent on L1 (e.g. PUCCH).

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGTOADDMOD-START

-- Configuration of a CSI-Report sent on L1 (e.g. PUCCH) (see 38.214, section 5.2.1)

CSI-ReportConfigToAddMod ::=					SEQUENCE {

reportConfigId							CSI-ReportConfigId,

-- Indicates in which serving cell the CSI-ResourceConfigToAddMod(s) below are to be found.

-- If the field is absent, the resources are on the same serving cell as this report configuration.

carrier									ServCellIndex			OPTIONAL,	-- Need S

-- Linked resources for channel measurement.

-- It is the csi-ResourceConfigId of a CSI-ResourceConfigToAddMod included in the configuration of the serving cell

-- indicated with the field "carrier" above.

-- This CSI-ReportConfigToAddMod is associated with the DL BWP indicated by bwp-Id in that CSI-ResourceConfigToAddMod.

resourcesForChannelMeasurement			CSI-ResourceConfigId,

-- Linked CSI IM resources for interference measurement.

-- It is the csi-ResourceConfigId of a CSI-ResourceConfigToAddMod included in the configuration of the serving cell

-- indicated with the field "carrier" above. The bwp-Id in that CSI-ResourceConfigToAddMod is the same value like the bwp-Id in the

-- CSI-ResourceConfigToAddMod indicated by resourcesForChannelMeasurement.

csi-IM-ResourcesForInterference			CSI-ResourceConfigId	OPTIONAL,	-- Need R

-- Linked NZP CSI RS resources for interference measurement.

-- It is the csi-ResourceConfigId of a CSI-ResourceConfigToAddMod included in the configuration of the

-- serving cell indicated with the field "carrier" above. The bwp-Id in that CSI-ResourceConfigToAddMod is the same value like the bwp-Id in the

-- CSI-ResourceConfigToAddMod indicated by resourcesForChannelMeasurement.

nzp-CSI-RS-ResourcesForInterference		CSI-ResourceConfigId	OPTIONAL, 	-- Need R

-- Time domain behavior of reporting configuration

reportConfigType						CHOICE {

periodic								SEQUENCE {

-- Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity'and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CSI-ReportPeriodicityAndOffset

,

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH						SEQUENCE {

-- Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity' and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CSI-ReportPeriodicityAndOffset

,

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-ResourceList					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH						SEQUENCE {

-- Periodicity. Corresponds to L1 parameter 'Reportperiodicity-spCSI'. (see 38.214, section 5.2.1.1?FFS_Section)

reportSlotConfig						ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

-- RNTI for SP CSI-RNTI, Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section 5.2.1.5.2)

-- FFS: RAN1 models different RNTIs as different Search Spaces with independent configurations. Align the configuration

-- of this one (e.g. group with monitoring periodicity, PDCCH candidate configuration, DCI-Payload size...)?

csi-RNTI								RNTI-Value,

-- Index of the p0-alpha set determining the power control for this CSI report transmission.

-- Corresponds to L1 parameter 'SPCSI-p0alpha' (see 38.214, section FFS_Section)

p0alpha									P0-PUSCH-AlphaSetId

},

aperiodic								SEQUENCE {

-- Timing offset Y for aperiodic reporting using PUSCH. This field lists the allowed offset values. A particular value is indicated in DCI.

-- (see 38.214, section 5.2.3)

-- FFS_Value: Range wasn’t final in RAN1 table.

-- FFS_FIXME: How are the DCI codepoints mapped to the allowed offsets?

reportSlotOffsetList					SEQUENCE (SIZE (1..4)) OF INTEGER (0..7)

}

},

-- The CSI related quanities to report. Corresponds to L1 parameter 'ReportQuantity' (see 38.214, section REF)

reportQuantity							CHOICE {

none									NULL,

cri-RI-PMI-CQI							NULL,

cri-RI-i1								NULL,

cri-RI-i1-CQI							SEQUENCE {

-- PRB bundling size to assume for CQI calcuation when reportQuantity is CRI/RI/i1/CQI

-- Corresponds to L1 parameter 'PDSCH-bundle-size-for-CSI' (see 38.214, section 5.2.1.4)

pdsch-BundleSizeForCSI					ENUMERATED {n2, n4}		OPTIONAL

},

cri-RI-CQI								NULL,

cri-RSRP								NULL,

ssb-Index-RSRP							NULL,

cri-RI-LI-PMI-CQI						NULL

},

-- Reporting configuration in the frequency domain. (see 38.214, section 5.2.1.4)

reportFreqConfiguration					 SEQUENCE {

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see 38.214, section 5.2.1.4)

cqi-FormatIndicator						ENUMERATED { widebandCQI, subbandCQI },

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see 38.214, section 5.2.1.4)

pmi-FormatIndicator						ENUMERATED { widebandPMI, subbandPMI },

-- Indicates a contiguous or non-contigous subset of subbands in the bandwidth part which CSI shall be reported

-- for. Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the

-- lowest subband in the BWP. (see 38.214, section 5.2.1.4)

-- The number of subbands is determined according to 38.214 section 5.2.1.4. It is absent if there are less than 24 PRBs (no sub band)

-- and present otherwise, the number of sub bands can be from 3 (24 PRBs, sub band size 8) to 18 (72 PRBs, sub band size 4).

csi-ReportingBand						CHOICE {

subbands3								BIT STRING(SIZE(3)),

subbands4								BIT STRING(SIZE(4)),

subbands5								BIT STRING(SIZE(5)),

subbands6								BIT STRING(SIZE(6)),

subbands7								BIT STRING(SIZE(7)),

subbands8								BIT STRING(SIZE(8)),

subbands9								BIT STRING(SIZE(9)),

subbands10								BIT STRING(SIZE(10)),

subbands11								BIT STRING(SIZE(11)),

subbands12								BIT STRING(SIZE(12)),

subbands13								BIT STRING(SIZE(13)),

subbands14								BIT STRING(SIZE(14)),

subbands15								BIT STRING(SIZE(15)),

subbands16								BIT STRING(SIZE(16)),

subbands17								BIT STRING(SIZE(17)),

subbands18								BIT STRING(SIZE(18))

}	OPTIONAL	-- Need S

},

-- Time domain measurement restriction for the channel (signal) measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-channel' (see 38.214, section 5.2.1.1)

timeRestrictionForChannelMeasurements				ENUMERATED {configured, notConfigured},

-- Time domain measurement restriction for interference measurements.

-- Corresponds to L1 parameter 'MeasRestrictionConfig-time-interference' (see 38.214, section 5.2.1.1)

timeRestrictionForInterferenceMeasurements			ENUMERATED {configured, notConfigured},

-- Codebook configuration for Type-1 or Type-II including codebook subset restriction

codebookConfig								CodebookConfig,

-- Maximum number of CQIs per CSI report (cf. 1 for 1-CW, 2 for 2-CW)

nrofCQIsPerReport							ENUMERATED {n1, n2},

-- Turning on/off group beam based reporting (see 38.214, section 5.2.1.4)

groupBasedBeamReporting						CHOICE {

enabled										NULL

,

disabled 									SEQUENCE {

-- The number (N) of measured RS resources to be reported per report setting in a non-group-based report.

-- N <= N_max, where N_max is either 2 or 4 depending on UE capability.

-- FFS: The signaling mechanism for the gNB to select a subset of N beams for the UE to measure and report.

-- FFS: Note: this parameter may not be needed for certain resource and/or report settings

-- FFS_ASN1: Change groupBasedBeamReporting into a CHOICE and include this field into the “no” option?

-- (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

nrofReportedRS							ENUMERATED {n1, n2, n3, n4}												OPTIONAL	-- Need S

}

},

-- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-table' (see 38.214, section 5.2.2.1)

cqi-Table								ENUMERATED {table1, table2, spare2, spare1}										OPTIONAL,

-- Indicates one out of two possible BWP-dependent values for the subband size as indicated in 38.214 table 5.2.1.4-2

-- Corresponds to L1 parameter 'SubbandSize' (see 38.214, section 5.2.1.4)

subbandSize								ENUMERATED {value1, value2},

-- BLER target that the UE shall be assume in its CQI calculation.

-- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section 5.2.2.1)

-- FFS_Values (now filled with spares)

bler-Target								ENUMERATED {zerodot1, spare3, space2, spare1}										OPTIONAL,

-- Port indication for RI/CQI calculation. For each  CSI-RS resource in the linked ResourceConfig for channel measurement,

-- a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback.

-- Corresponds to L1 parameter 'Non-PMI-PortIndication' (see 38.214, section FFS_Section)

-- The first entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the first entry in

-- nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the

-- CSI-ResourceConfig whose CSI-ResourceConfigId is indicated in a CSI-MeasId together with the above CSI-ReportConfigId,

-- the second entry in non-PMI-PortIndication corresponds to the NZP-CSI-RS-Resource indicated by the second entry in

-- nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the

-- same CSI-ResourceConfig, and so on until the NZP-CSI-RS-Resource indicated by the last entry in nzp-CSI-RS-Resources

-- in the in the NZP-CSI-RS-ResourceSet indicated in the first entry of nzp-CSI-RS-ResourceSetList of the

-- same CSI-ResourceConfig, then the next entry corresponds to the NZP-CSI-RS-Resource indicated by the first entry

-- in nzp-CSI-RS-Resources in the NZP-CSI-RS-ResourceSet indicated in the second entry of nzp-CSI-RS-ResourceSetList of the

-- same CSI-ResourceConfig and so on.

non-PMI-PortIndication					SEQUENCE (SIZE (1.. maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF PortIndexFor8Ranks																		OPTIONAL,

...

}

CSI-ReportPeriodicityAndOffset ::= CHOICE {

slots4								INTEGER(0..3),

slots5								INTEGER(0..4),

slots8								INTEGER(0..7),

slots10								INTEGER(0..9),

slots16								INTEGER(0..15),

slots20								INTEGER(0..19),

slots40								INTEGER(0..39),

slots80								INTEGER(0..79),

slots160							INTEGER(0..159),

slots320							INTEGER(0..319)

}

PUCCH-CSI-Resource ::= 				CHOICE {

uplinkBandwidthPartId				BWP-Id,

-- PUCCH resource for the associated uplink BWP. Only PUCCH-Resource of format 2, 3 and 4 is supported.

pucch-Resource						PUCCH-Resource

}

-- The PortIndexFor8Ranks allows to indicate port indexes for 1 to 8 ranks using a port index ranges from 0 to 31, or from 0 to 15, or from 0

-- to 7, or from 0 to 3, or from 0 to 1, or with 0 only.

PortIndexFor8Ranks ::=	CHOICE {

portIndex32				SEQUENCE{

rank1-32				PortIndex32,

rank2-32				SEQUENCE(SIZE(1..2)) OF PortIndex32,

rank3-32				SEQUENCE(SIZE(1..3)) OF PortIndex32,

rank4-32				SEQUENCE(SIZE(1..4)) OF PortIndex32,

rank5-32				SEQUENCE(SIZE(1..5)) OF PortIndex32,

rank6-32				SEQUENCE(SIZE(1..6)) OF PortIndex32,

rank7-32				SEQUENCE(SIZE(1..7)) OF PortIndex32,

rank8-32				SEQUENCE(SIZE(1..8)) OF PortIndex32

},

portIndex16				SEQUENCE{

rank1-16				PortIndex16,

rank2-16				SEQUENCE(SIZE(1..2)) OF PortIndex16,

rank3-16				SEQUENCE(SIZE(1..3)) OF PortIndex16,

rank4-16				SEQUENCE(SIZE(1..4)) OF PortIndex16,

rank5-16				SEQUENCE(SIZE(1..5)) OF PortIndex16,

rank6-16				SEQUENCE(SIZE(1..6)) OF PortIndex16,

rank7-16				SEQUENCE(SIZE(1..7)) OF PortIndex16,

rank8-16				SEQUENCE(SIZE(1..8)) OF PortIndex16

},

portIndex8				SEQUENCE{

rank1-8					PortIndex8,

rank2-8					SEQUENCE(SIZE(1..2)) OF PortIndex8,

rank3-8					SEQUENCE(SIZE(1..3)) OF PortIndex8,

rank4-8					SEQUENCE(SIZE(1..4)) OF PortIndex8,

rank5-8					SEQUENCE(SIZE(1..5)) OF PortIndex8,

rank6-8					SEQUENCE(SIZE(1..6)) OF PortIndex8,

rank7-8					SEQUENCE(SIZE(1..7)) OF PortIndex8,

rank8-8					SEQUENCE(SIZE(1..8)) OF PortIndex8

},

portIndex4				SEQUENCE{

rank1-4					PortIndex4,

rank2-4					SEQUENCE(SIZE(1..2)) OF PortIndex4,

rank3-4					SEQUENCE(SIZE(1..3)) OF PortIndex4,

rank4-4					SEQUENCE(SIZE(1..4)) OF PortIndex4

},

portIndex2				SEQUENCE{

rank1-2					PortIndex2,

rank2-2					SEQUENCE(SIZE(1..2)) OF PortIndex2

},

portIndex1				NULL

}

PortIndex32::=	INTEGER (0..31)

PortIndex16::=	INTEGER (0..15)

PortIndex8::=	INTEGER (0..7)

PortIndex4::=	INTEGER (0..3)

PortIndex2::=	INTEGER (0..1)

-- TAG-CSI-REPORTCONFIGTOADDMOD-STOP

-- ASN1STOP

#### –	CSI-ReportConfigId

The IE CSI-ReportConfigId is used to identify one CSI-ReportConfig.

CSI-ReportConfigId information element

-- ASN1START

-- TAG-CSI-REPORTCONFIGID-START

CSI-ReportConfigId ::= 					INTEGER (0..maxNrofCSI-ReportConfigurations-1)

-- TAG-CSI-REPORTCONFIGID-STOP

-- ASN1STOP

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=	CHOICE {

slots4							INTEGER (0..3),

slots5							INTEGER (0..4),

slots8							INTEGER (0..7),

slots10							INTEGER (0..9),

slots16							INTEGER (0..15),

slots20							INTEGER (0..19),

slots32							INTEGER (0..31),

slots40							INTEGER (0..39),

slots64							INTEGER (0..63),

slots80							INTEGER (0..79),

slots160						INTEGER (0..159),

slots320						INTEGER (0..319),

slots640						INTEGER (0..639)

}

-- TAG-CSI-RESIYRCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

#### –	CodebookConfig

The IE CodebookConfig is used to configure codebooks of Type-I and Type-II (see 38.214, section 5.2.2.2)

CodebookConfig information element

-- ASN1START

-- TAG-CODEBOOKCONFIG-START

CodebookConfig ::= 									SEQUENCE {

-- CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType'

-- (see 38.214, section 5.2.2.2)

codebookType 										CHOICE {

type1 												SEQUENCE {

subType										CHOICE {

typeI-SinglePanel									SEQUENCE {

nrOfAntennaPorts									CHOICE {

two													SEQUENCE {

-- Codebook subset restriction for 2TX codebook

-- Corresponds to L1 parameter ' TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.1)

twoTX-CodebookSubsetRestriction						BIT STRING (SIZE (6))

},

moreThanTwo											SEQUENCE {

-- Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction

-- Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2'

-- 'TypeI-SinglePanel-CodebookSubsetRestriction ' (see 38.214 section 5.2.2.2.1)

n1-n2												CHOICE {

two-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (8)),

two-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (64)),

four-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (16)),

three-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (96)),

six-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (24)),

four-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (128)),

eight-one-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (32)),

four-three-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (192)),

six-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (192)),

twelve-one-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (48)),

four-four-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (256)),

eight-two-TypeI-SinglePanel-Restriction				BIT STRING (SIZE (256)),

sixteen-one-TypeI-SinglePanel-Restriction			BIT STRING (SIZE (64))

},

-- i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214 section 5.2.2.2.1)

typeI-SinglePanel-codebookSubsetRestriction-i2		BIT STRING (SIZE (16))	OPTIONAL

}

},

-- Restriction for RI for TypeI-SinglePanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section 5.2.2.2.1)

typeI-SinglePanel-ri-Restriction					BIT STRING (SIZE (8))

},

typeI-MultiPanel									SEQUENCE {

-- Codebook subset restriction for Type I Multi-panel codebook

-- Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section 5.2.2.2.2)

ng-n1-n2												CHOICE {

two-two-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (8)),

two-four-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (32)),

four-two-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (32)),

two-two-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (128)),

two-eight-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (64)),

four-four-one-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (64)),

two-four-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (256)),

four-two-two-TypeI-MultiPanel-Restriction				BIT STRING (SIZE (256))

},

-- Restriction for RI for TypeI-MultiPanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section 5.2.2.2.2)

ri-Restriction							BIT STRING (SIZE (4))

}

},

-- CodebookMode as specified in 38.214 section 5.2.2.2.2

codebookMode										INTEGER (1..2)

},

type2									SEQUENCE {

subType									CHOICE {

typeII									SEQUENCE {

-- Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction

-- Corresponds to L1 parameters 'CodebookConfig-N1', 'CodebookConfig-N2'

-- The CHOICE name indicates the value of n1 and n2, the CHOICE contents is the codebook subset restriction bitmap

-- Corresponds to L1 parameter ' TypeII-CodebookSubsetRestriction' (see 38.214 section 5.2.2.2.3)

-- Number of bits for codebook subset restriction is ceil(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!)

n1-n2-codebookSubsetRestriction			CHOICE {

two-one									BIT STRING (SIZE (17)),

two-two									BIT STRING (SIZE (139)),

four-one								BIT STRING (SIZE (33)),

three-two								BIT STRING (SIZE (59)),

six-one									BIT STRING (SIZE (49)),

four-two								BIT STRING (SIZE (75)),

eight-one								BIT STRING (SIZE (65)),

four-three								BIT STRING (SIZE (107)),

six-two									BIT STRING (SIZE (107)),

twelve-one								BIT STRING (SIZE (129)),

four-four								BIT STRING (SIZE (139)),

eight-two								BIT STRING (SIZE (139)),

sixteen-one								BIT STRING (SIZE (129))

},

-- Restriction for RI for TypeII-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section 5.2.2.2.3)

typeII-RI-Restriction					BIT STRING (SIZE (2))

},

typeII-PortSelection					SEQUENCE {

-- The size of the port selection codebook (parameter d)

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Cond TypeII-PortSelection

-- Restriction for RI for TypeII-PortSelection-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section 5.2.2.4)

typeII-PortSelectionRI-Restriction		BIT STRING (SIZE (2))

}

},

-- The size of the PSK alphabet, QPSK or 8-PSK

phaseAlphabetSize						ENUMERATED {n4, n8},

-- If subband amplitude reporting is activated (true)

subbandAmplitude						BOOLEAN,

-- Number of beams, L,  used for linear combination

numberOfBeams							ENUMERATED {two, three, four}

-- The size of the port selection codebook (parameter d)

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Cond TypeII-PortSelection

}

}

}

-- TAG-CODEBOOKCONFIG-STOP

-- ASN1STOP

[DMRS-DownlinkConfig, DMRS-UplinkConfig, DRB-Identity, MeasResultSCG-Failure, FrequencyInfoDL, SCS-SpecificVirtualCarrier, FrequencyInfoUL, GSCN-ValueNR, LogicalChannelConfig, MAC-CellGroupConfig, MeasConfig, MeasGapConfig, MeasId, MeasIdToAddModList, MeasObjectEUTRA, MeasObjectId, MeasObjectNR, MeasObjectToAddModList, MeasResults, PDCCH-ConfigCommon, PDCCH-Config, PDCP-Config, PDSCH-Config, PCI-List, PCI-Range, PCI-RangeIndex, PCI-RangeIndexList, PhysCellId, PRB-Id, PTRS-DownlinkConfig, PTRS-UplinkConfig, PUCCH-Config, PUSCH-Config, PUSCH-PowerControl, Q-OffsetRange, QuantityConfig, RACH-ConfigCommon, RACH-ConfigCommonGeneric, RACH-ConfigDedicated, RadioBearerConfig, ReportConfigId, ReportConfigNR, ReportConfigToAddModList, ReportInterval, RLC-Config, RLF-TimersAndConstants, RNTI-Value, RSRP-Range, RSRQ-Range, SINR-Range, SCellIndex, SchedulingRequest-Config, SchedulingRequestResourceConfig, SchedulingRequestResourceId, ScramblingId, SDAP-Config, SearchSpace, SlotFormatIndicatorSFI, DownlinkPreemption, SearchSpaceId, SecurityAlgorithmConfig, ServCellIndex, ServingCellConfigCommon, ServingCellConfig, SlotFormatCombinationsPerCell, SRB-Identity, SPS-Config, ConfiguredGrantConfig, SRS-Config, SRS-CarrierSwitching, SSB-Index, SubcarrierSpacing]

#### –	TCI-State

The TCI-State IE associates one or two DL reference signals with a corresponding quasi-colocation (QCL) type.

TCI-State information element

-- ASN1START

-- TAG-TCI-STATE-START

TCI-State ::= 				SEQUENCE {

tci-StateId					TCI-StateId,

qcl-Type1					QCL-Info,

qcl-Type2					QCL-Info																					OPTIONAL,	-- Need R

nrofPTRS-Ports				ENUMERATED {n1, n2}																			OPTIONAL,	-- Need R

...

}

TCI-StateId ::=				INTEGER (0.. maxNrofTCI-States-1)

QCL-Info ::=				SEQUENCE {

-- The carrier which the RS is located in. If the field is absent, it applies to the serving cell in which the TCI-State is configured

cell						ServCellIndex																				OPTIONAL,	-- Need R

-- The DL BWP which the RS is located in.

bwp-Id						BWP-Id,

referenceSignal				CHOICE {

csi-rs						NZP-CSI-RS-ResourceId,

ssb							SSB-Index,

-- A set of CSI-RS resources for tracking

csi-RS-for-tracking							NZP-CSI-RS-ResourceSetId

},

qcl-Type					ENUMERATED {typeA, typeB, typeC, typeD},

...

}

-- TAG-TCI-STATE-STOP

-- ASN1STOP

[TDD-UL-DL-Config]

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource. Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfig' (see 38.214, section 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=						SEQUENCE {

-- ZP CSI-RS resource configuration ID

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigId' (see 38.214, section 5.1.4.2)

zp-CSI-RS-ResourceId						ZP-CSI-RS-ResourceId,

-- OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot

resourceMapping								CSI-RS-ResourceMapping

,

-- Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS

-- Corresponds to L1 parameter 'ZP-CSI-RS-timeConfig' (see 38.214, section 5.1.4.2)

periodicityAndOffset						CSI-ResourcePeriodicityAndOffset

OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=							INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-ResourceSet

The IE ZP-CSI-RS-ResourceSet refers to a set of ZP-CSI-RS-Resources using their ZP-CSI-RS-ResourceIds. It corresponds to the L1 parameter 'ZP-CSI-RS-ResourceSetConfigList'.

ZP-CSI-RS-ResourceSet information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESET-START

ZP-CSI-RS-ResourceSet ::= 						SEQUENCE {

zp-CSI-RS-ResourceSetId							ZP-CSI-RS-ResourceSetId,

-- The list of ZP-CSI-RS-ResourceId identifying the ZP-CSI-RS-Resource elements belonging to this set.

zp-CSI-RS-ResourceIdList							SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId,

-- Time domain behavior of ZP-CSI-RS resource configuration.

-- Corresponds to L1 parameter 'ZP-CSI-RS-ResourceConfigType' (see 38.214, section 5.1.4.2)

resourceType									ENUMERATED {aperiodic, semiPersistent, periodic},

...

}

-- TAG-ZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

#### –	ZP-CSI-RS-ResourceSetId

The IE ZP-CSI-RS-ResourceSetId identifies a ZP-CSI-RS-ResourceSet.

ZP-CSI-RS-ResourceSetId information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCESETID-START

ZP-CSI-RS-ResourceSetId ::= 					INTEGER (0..maxNrofZP-CSI-RS-ResourceSets-1)

-- TAG-ZP-CSI-RS-RESOURCESETID-STOP

-- ASN1STOP

[UE capability information elements]

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxNrofCSI-ReportConfigurations						INTEGER ::= 256 	-- Maximum number of CSI report configurations

maxNrofCSI-ReportConfigurations-1		INTEGER ::= 255 	-- Maximum number of CSI report configurations minus 1

maxNrofCSI-ResourceConfigurations		INTEGER ::= 128		-- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1		INTEGER ::= 127		-- Maximum number of resource configurations minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig					INTEGER ::= 16		-- Maximum number of NZP CSI RS resource sets per resource configuration

maxNrofCSI-IM-ResourceSetsPerConfig 	INTEGER ::= 16		-- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourceSetsPerConfig 	INTEGER ::= 1		-- Maximum number of CSI SSB resource sets per resource configuration

maxNrofNZP-CSI-RS-Resources				INTEGER ::= 256		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1			INTEGER ::= 255		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-Resources				INTEGER ::= 3		-- Maximum number of Zero-Power (NZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1			INTEGER ::= 2		-- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1

maxNrofCSI-IM-Resources					INTEGER ::= 256		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-Resources-1				INTEGER ::= 255		-- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourcesPerSet			INTEGER ::= 64		-- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214

maxNrofSSB-Resources-1					INTEGER ::= 63		-- Maximum number of SSB resources in a resource set minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet			INTEGER ::= 64		-- Maximum number of NZP CSI-RS resources per resource set

maxNrofCSI-SSB-ResourcePerSet 			INTEGER ::= 64		-- Maximum number of SSB-

maxNrofTCI-States						INTEGER ::= 64

maxNrofTCI-States-1						INTEGER ::= 63

maxNrofZP-CSI-RS-ResourceSets-1			INTEGER ::= 16

maxNrofZP-CSI-RS-ResourcesPerSet		INTEGER ::= 16

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP