For arch 1a, TR section 8 defines four bullets for functional support of Adapt:

- Identification of the UE-bearer for the PDU,

- Routing across the wireless backhaul topology,

- QoS-enforcement by the scheduler on DL and UL on the wireless backhaul link,

- Mapping of UE user-plane PDUs to backhaul RLC channels,

These are the identifier options for each of these bullets:

- Identification of UE-bearer for the PDU, based on (options):

- UE-bearer Id

- UE-id + UE-specific bearer Id

- Routing across the wireless backhaul topology, based on (options):

- UE-bearer Id

- UE-id

- IAB-node Id/IAB-donor Id

- QoS enforcement by scheduler, based on (options):

- UE-bearer Id

- UE-id + UE-specific bearer-Id

- Aggregate QoS-Id

- Mapping of UE UP PDUs to backhaul RLC channels (options):

- Adapt above MAC: They could use all the same RLC channel

- Adapt above RLC: Mapping uses the same Id as QoS enforcement

This leads to the following set of identifiers to be considered for Adapt:

- UE-bearer Id

- UE-Id + UE-specific bearer Id

- IAB-node Id/IAB-donor Id

- Aggregate QoS-Id

Further assumptions:

- Adapt is generated on access IAB-node for northbound PDUs and on IAB-donor DU for southbound PDUs.

- Adapt is not modified along the path across wireless backhaul.

We need to consider:

- Generation of Adapt Id at initial node where Adapt is generated

- Processing of Adapt Id at final node where Adapt is terminated

- Processing of Adapt Id at intermediate node

Generation of Adapt Id at initial node where Adapt is generated:

- UE-bearer Id:

  - At access-IAB-node, deterministically mapped from F1-U GTP-U TEID, which is configured on UE-bearer’s DU based on native F1-AP procedures.

  - At IAB-donor DU, deterministically mapped from F1-U GTP-U TEID of arriving fronthaul PDU.

- UE-Id:

  - At access-IAB-node, mapped from C-RNTI; mapping needs to be configured when UE-bearer is established.

  - At IAB-donor DU, mapped from F1-U GTP-U TEID; mapping needs to be configured when UE-bearer is established.

- UE-specific bearer-Id:

  - At access-IAB-node, deterministically mapped from LCID of arriving access PDU.

  - At IAB-donor DU, mapped from F1-U GTP-U TEID; mapping needs to be configured when UE-bearer is established.

- IAB-node/donor-DU Id:

  - At access-IAB-node, based on value configured on node; could be deterministically mapped from existing Id, e.g. CGI or PCI, or needs to be configurated when IAB-node attaches to topology.

  - At IAB-donor DU, based on value configured on node; could be deterministically mapped from existing Id, e.g. CGI or PCI, or needs to be configured when IAB-donor is integrated.

- Aggregate QoS-Id:

  - At access-IAB-node, mapped from QoS class identifier configured for access bearer. Mapping may be semi-static, e.g. configured when IAB-node attaches to topology, or bearer-specific, e.g. configured when UE-bearer is established.

  - At IAB-donor DU, mapped from DSCP value or F1-U GTP-U TEID of arriving fronthaul PDU. Mapping may be semi-static, e.g. configured when IAB-node attaches to topology, or bearer-specific, e.g. configured when UE-bearer is established.

Processing of Adapt Id at final node where Adapt is terminated

- UE-bearer Id:

  - At access-IAB-node, deterministically mapped to F1-U GTP-U TEID, which is configured on UE-bearer’s DU based on native F1-AP procedures.

  - At IAB-donor DU, deterministically mapped to F1-U GTP-U TEID of PDU forwarded on fronthaul.

- UE-Id:

  - At access-IAB-node, mapped to C-RNTI; mapping needs to be configured when UE-bearer is established.

  - At IAB-donor DU, mapped from F1-U GTP-U TEID; mapping need to be configured when UE-bearer is established.

- UE-specific bearer-Id:

  - At access-IAB-node, deterministically mapped from LCID of arriving access PDU.

  - At IAB-donor DU, mapped to F1-U GTP-U TEID; mapping needs to be configured when UE-bearer is established.

- IAB-node/donor-DU Id:

  - At access-IAB-node, PDU is terminated when IAB-node-Id value on Adapt matches that configured for access-IAB-node.

  - Ad IAB-donor DU, PDU is forwarded to fronthaul when IAB-donor Id value on Adapt matches that configured for IAB-donor DU.

- Aggregate QoS-Id:

  - At access-IAB-node, not used.

  - At IAB-donor DU, it may be mapped to DSCP value of PDU forwarded on fronthaul.

Processing of Adapt Id at intermediate nodes

- Identification of UE-bearer for the PDU: None

- Routing across the wireless backhaul topology: IAB-node matches routing Id on Adapt to routing table entry and derives backhaul link where PDU has to be forwarded

- If done via UE-bearer Id, routing table needs to be reconfigured when UE-bearer is established or released at access IAB-node.

- If done via UE-Id, routing table needs to be reconfigured when UE connects to or leaves access IAB-node.

- If done via IAB-node/IAB-donor-Id, routing table needs to be reconfigured when topology changes.

- QoS enforcement by scheduler: Scheduler matches QoS-Id to entry in scheduling policy table, selects RLC bearer and applies corresponding scheduling policy when forwarding PDU.

- If done via UE-bearer Id, policy table needs to be reconfigured when UE-bearer is established or released at access IAB-node.

- If done via UE-Id + UE-specific bearer Id, policy table needs to be reconfigured when UE-bearer is established or released at access IAB-node.

- If done via IAB-node/IAB-donor-Id, policy table needs to be updated when topology changes.