3GPP TSG RAN WG2 Meeting #104                                                       R2-181xxxx

Spokane, USA, 12th - 16th November 2018

Agenda item:	9.7.x

Source:		Intel Corporation (rapporteur)

Title:  		Email discussion report on [103bis#43][ LTE/eLTE]] Capture NR agreements (Intel)

Document for:	Discussion and decision

# Introduction

This is the email discussion report on [103bis#43][LTE/5GC] Capture NR agreements (Intel).

[103bis#43][LTE/eLTE] Capture NR agreements (Intel)

Capture the agreements made in NR session which are also applied for eLTE

Intended outcome: 36.331 CR submitted to next meeting

Deadline:  Thursday 2018-11-01

In this email discussion, Rapporteur lists corresponding in principle agreed CRs for NR, and would like to  collect views from companies on whether all related NR agreements are applicable for LTE/5GC , and then provide TS36.331 CR for eLTE (maybe also TS36.304 CR). Rapporteur would suggest to have two phases discussion:

Phase 1: Companies are invited to provide your view on whether each NR agreements are applicable for LTE/5GC or not; Deadline for phase 1: Friday 2018-10-25 Rapporteur may miss some agreements, companies are invited to add if any in section 3.

Phase 2: Companies are invited to provide comments on stage 3 TS36.331 CR; Deadline for Phase 2:  Thursday 2018-11-01

# NR agreements which are related to LTE/5GC

Note: NR agreements which have been captured in LTE/5GC in the meeting, are listed in section 5 Annex.

| # | Agreements/Tdoc number | Companies are invited to provide view on whether agreements are applicable for LTE/5GC or not |
| --- | --- | --- |
| 1 | R2-1816056 CR on fallback to the setup procedure R2-1816052 RIL I556, I557, I558 on RB handling when resuming - discard the suspendConfig if stored - discard the AS security context - discard radio resources and RRC configuration for all RBs except SRB0 TS36.331 5.3.3.4, the handling on INACTIVE shall be updated “1> if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE: ” |  |
| 2 | R2-1815834 CR to 38.331 on the ambiguity of targetCellIdentity in Resume/Reestablishment MAC-I calculation TS36.331 7.1 Variable for resume/reestablishment in case of E-UTRA/5GC should be updated, e.g. VarShortINACTIVE-MAC-Input. |  |
| 3 | R2-1815835 Handling on security keys for resume procedure (I774) TS36.331 Handling of keys/NCC shall be updated; |  |
| 4 | R2-1814961 Clarifications on RNA update and CN registration (N023) TS36.331 Handling on SIB1 and RNAU shall be updated; |  |
| 5 | R2-1815836 CR to 38.331 on pendingRNAUpdate setting TS36.331 Handling on variable pendingRNAUpdate |  |
| 6 | R2-1815838 Handling of Resume TS36.331, In 5.3.3, add For INACTIVE UE, the UE shall enter IDLE if the UE is unable to comply with (part of) the configuration included in the RRCResume message |  |
| 7 | R2-1815952 E573 Configuration of SRB1 during Resume TS36.331, 5.3.3.3a PDCP configuration shall not be restored before Reception of the RRCResume. restore the RRC configuration and security context from the stored UE AS context except physical layer and MAC configuration; |  |
| 8 | R2-1815958 Security for RRC connection release TS36.331, For E-UTRA/5GC, UE shall release the RRC connection but ignore the Re-direction, Dedicated frequency reselection priorities, De-prioritisation information, and SuspendConfig fields if included in an RRC release message received before security activation. |  |
| 9 | R2-1815959 RRC connection release triggered by upper layers [H323] TS36.331, For E-UTRA/5GC, reference should be 24.501, in addition, the changes in 5.3.9 is applicable for E-UTRA/5GC |  |
| 10 | R2-1815961 CR to 38.304 on Release and Redirect in 2-step procedure TS36.304 shall be updated. 5.2.7 Cell Selection when leaving RRC_CONNECTED state |  |
| 11 | R2-1815992 Remain issue for T302 and T390 TS36.331 5.3.3.8, handling on reject is missing, i.e. stop the timer first, and start it if configured; |  |
| 12 | R2-1815993 CR to 38.331 on stopping T390 and UE related actions TS36.331 Handling on T390 can be aligned. |  |
| 13 | R2-1815837 CR for 38.331 for UE AS context Al2 has been postponed to next meeting. Rapporteur assume similar changes are needed for eLTE; |  |
| 14 | R2-1813624 On UE behaviour upon going out of service, CRs on NR will be provided by Samsung in next meeting. We could align with them if any. |  |

# Any other issues?

Companies are invited to provide if anything is missing.

Anything related to LTE/5GC?

| Company's name | NR agreements/CRs | Remark |
| --- | --- | --- |

# Email discussion report

To be added.

# Annex

## NR agreements which have been captured in LTE/5GC

| # | NR Agreements/Tdoc number | eLTE Agreements/Tdoc number |
| --- | --- | --- |
| 1 | R2-1815822 clarification of guami-Type | R2-1815859 [E201] CR to 36.331 on handling of mapped GUMMEI at idle mode mobility from 5GS to EPS |
| 2 | R2-1815962 Correction on CN type indication for Redirection from NR to E-UTRA | R2-1815787 Correction of CN type indication for RRC Redirection from E-UTRA/5GC to E-UTRA/5GC or E-UTRAN |
| 3 | R2-1816047 Correction to full configuration | R2-1816008 Correction to DRB release |
| 4 | R2-1813968 Handling on simultaneously triggered NAS/AS events (I770) | R2-1813976 Open issues on E-UTRA connected to 5GC for UAC |
| 5 | R2-1813823 [C204] Handling of timer T380 | R2-1815789 TS36.331 CR on Open issues on E-UTRA connected to 5GC for INACTIVE |
| 6 | R2-1816029 Addition of RAN specific Access Category | R2-1813643 Addition of RAN specific Access Category |