3GPP TSG-RAN WG2 Meeting #105bis                                                          R2-19xxxxx

Xi’an, China, April 8th – 12th, 2019

Agenda item:	11.1.1

Source: 		Ericsson (Rapporteur)

Title: 	Email discussion [105#47] [NR_IAB-Core] Bearer Mapping

Document for:	Discussion

# Introduction

This document contains email discussion:

R2-1902645	Email Discussions 		Qualcomm Inc

discussion

- 	Huawei think that Adapt entities should be discussed separately.

- 	Chair proposes to remove the part on entities

- [105#47] [NR_IAB-Core] Bearer mapping (Ericsson)

Intended outcome: a report to identify options and possible “easy” agreements.

Deadline:  Thursday 2019-03-28

Discussion:

The TR defines the term “bearer mapping” as “UE-bearer to RLC-channel mapping” but this is too restrictive since it does not apply to F1-C. Further, F1-C messages of different type might also have different priorities and therefore use different RLC channels. This raises the following questions:

- What are the criteria we want to allow for the mapping of an Adapt PDU to a BH RLC channel? (e.g. QoS, UP vs CP, PDU session, F1 connection, others).

- What information must be available at the access-IAB-node to enable this mapping in upstream direction? (e.g. mapping table with (F1-connection Id, BH LCID)-pairs).

- Which of these criteria require a re-mapping on intermediate IAB-nodes?

- What information must be available at the intermediate-IAB-node to enable this re-mapping? (e.g. none if remapping is not required).

- What information needs to be carried in the adapt header to enable the remapping on the intermediate-IAB-node?

# 2. Discussion

The purpose of this email discussion is to apprehend different options for bearer mapping in an IAB network and identify common themes on this topic to be presented in RAN2#105bis. To make this discussion more productive, we have rephrased the above questions by bringing up explicitly the underlying issues of mapping in Donor DU, intermediate-IAB-node and Access-IAB-node. In addition, to provide companies the opportunity to express their point of view on how these issues can be tackled for N:1 and 1:1 mapping, as well as the mapping aspect of F1-C and OAM traffic, we have included three sets of questions.

It should be noted that we have not included the case of the DL mapping at the Access-IAB-node and UL mapping at the Donor DU because there is no BH RLC channel at the next hop and we can rely on F1/IP mechanisms.

Companies are invited to share their point of view, specifically about what information is used/needed for the mapping in each case, how the node(s) obtain the information, whether the information is provided via configuration or inband (e.g. Adaptation/IP header), and motivation behind remapping at the intermediate-IAB-node(s).

In the following, two sets of questions one for each type of mapping are provided, covering the bearer mapping issues raised in R2-1902645:

Question 1: N:1 mapping UP

1a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |

Summary:

1b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |

Summary:

1c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |

Summary:

1d: Additional comments/aspects related to N:1 mapping?

| ## Company | ## Comments |
| --- | --- |

Summary:

Question 2: 1:1 mapping UP

2a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |

Summary:

2b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |

Summary:

2c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |

Summary:

2d: Additional comments/aspects regarding 1:1 mapping?

| ## Company | ## Comments |
| --- | --- |

Summary:

Question 3: Mapping of F1-C and OAM

3a: How is the mapping performed for the UL at the Access-IAB-node?

| ## Company | ## Comments |
| --- | --- |

Summary:

3b: How is the mapping performed for the DL at the Donor DU?

| ## Company | ## Comments |
| --- | --- |

Summary:

3c: How is the mapping performed for the UL/DL at the intermediate-IAB-node(s)?

| ## Company | ## Comments |
| --- | --- |

Summary:

3d: Additional comments/aspects regarding F1-C/OAM mapping?

| ## Company | ## Comments |
| --- | --- |

Summary:

# 3. Summary