3GPP TSG-RAN2 Meeting #106	R2-190xxxx

- Reno, US, 13 – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | - | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on filters used to generate FeatureSets (36.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Procedures for SRS carrier switching request for EN-DC is currently missing. A generic way to include it, as performed for NR SA case in R2-1902628, is to add the handling of requestedCapabilityNR field to the procedures related to EN-DC capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE - Included handling of requestedCapabilityNR for the case EN-DC capabilities were requested Impact analysis Impacted 5G architecture options: EN-DC Impacted functionality: UE Capability transfer Inter-operability: If the network implements the CR and the UE does not, the UE will not report capabilities related to EN-DC SRS carrier switching. If the UE implements the CR and the network does not, the network may expect consistent feature set IDs and feature set combination IDs in UE capability transfer regardless of SRS carrier switching request (i.e. whether srs-SwitchingTimeRequest field is present or not) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When SRS carrier switching request is used for EN-DC, the report of corresponding capabilities will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.6.3.3	Reception of the UECapabilityEnquiry by the UE

The UE shall:

>>>>>>>>>>>>>>>>>>>>>>>> omitted unchanged parts <<<<<<<<<<<<<<<<<<<<<<<<<<

2>	if the ue-CapabilityRequest includes nr and if the UE supports NR:

3>	include the UE radio access capabilities for NR within a ue-CapabilityRAT-Container, with the rat-Type set to nr;

3>	include band combinations and feature sets as specified in TS 38.331 [82], clause 5.6.1.4, considering the included requestedFreqBandsNR-MRDC, requestedCapabilityNR and the eutra-nr-only flag (if present);

2>	if the ue-CapabilityRequest includes eutra-nr and if the UE supports EN-DC:

3>	include the UE radio access capabilities for EUTRA-NR within a ue-CapabilityRAT-Container, with the rat-Type set to eutra-nr;

3>	include band combinations and feature sets as specified in TS 38.331 [82], clause 5.6.1.4, considering the included requestedFreqBandsNR-MRDC and requestedCapabilityNR (if present);

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

END OF FIRST CHANGE