3GPP TSG-RAN WG2 Meeting #110-e		R2-200xxxx

Electronic, June 1 – 12, 2020

Source: 			Qualcomm Incorporated

Title:

Document for: 	Decision

Agenda Item: 	5.4.3

# Introduction

This document summarizes the following email discussion.

# Discussion

## UE setting of xDD FRx split capabilities

The current procedural text in TS38.306 is as follows.

The UE may support different functionalities between FDD and TDD, and/or between FR1 and FR2. The UE shall indicate the UE capabilities as follows. […] 1> set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC-Capabilities, tdd-Add-UE-NR/MRDC-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports; 1> if UE supports both FDD and TDD and if (some of) the UE capability fields have a different value for FDD and TDD 2> if for FDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field fdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD; 2> if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field tdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD; 1> if UE supports both FR1 and FR2 and if (some of) the UE capability fields have a different value for FR1 and FR2: 2> if for FR1, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field fr1-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR1; 2> if for FR2, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability: 3> include field fr2-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR2;

The following sections summarize the setting of UE capability signalling.

Interpretation 1-a (e.g. R2-2002573, Qualcomm)

Interpretation 1-b (e.g. R2-2003454, Huawei)

Interpretation 2 (e.g. R2-2003269, Ericsson)

NOTE:

- “Unable to signal” indicates that the set of UE capabilities is allowed according to the UE capability differentiation rules in the standard, i.e. ‘yes’ or ‘no’ indications in the “FDD-TDD DIFF” and “FR1-FR2 DIFF” in TS38.306. But the UE capability signalling is not capable of expressing the set of UE capabilities

- “Not allowed” indicates the set of UE capabilities is NOT allowed according to the UE capability differentiation rules in the standard, i.e. ‘yes’ or ‘no’ indications in the “FDD-TDD DIFF” and “FR1-FR2 DIFF” in TS38.306.

In both cases, the UE can fallback to an allowed UE capability setting.

## UE capabilities with xDD and FRx differentiations

Interpretation 1-a

| Support for the feature |  | UE capability containers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | xDD-Diff in common | FRX-diff in common | fdd-Add | tdd-Add | fr1-Add | fr2-Add |
| Case 1 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Supported | Supported | Not included | Not included | Not included | Not included |
| Case 2 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Not included | Not included | Not included | Not included |
| Case 3 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Not supported | Supported | Not included | Supported | Not included | Not included |
| Case 4 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Not supported | Not supported | Not included | Supported | Not included | Supported |
| Case 5 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Not included | Supported | Supported | Not included |
| Case 6 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Unable to signal |  |  |  |  |  |
| Case 7 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Supported | Not included | Supported | Not included |
| Case 8 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Supported | Not supported | Not included | Not included | Supported | Not included |

Interpretation 1-b

| Support for the feature |  | UE capability containers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | xDD-Diff in common | FRX-diff in common | fdd-Add | tdd-Add | fr1-Add | fr2-Add |
| Case 1 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Supported | Supported | Not included | Not included | Not included | Not included |
| Case 2 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Not included | Not included | Not included | Not included |
| Case 3 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Not supported | Not supported | Not included | Supported | Not included | Not included |
| Case 4 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Not supported | Not supported | Not included | Supported | Not included | Supported |
| Case 5 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Not included | Supported | Supported | Not included |
| Case 6 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Unable to signal |  |  |  |  |  |
| Case 7 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Supported | Not included | Supported | Not included |
| Case 8 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not supported | Not supported | Not included | Not included | Supported | Not included |

Interpretation 2

Label

1		UE supports the feature for the given FRX/XDD mode

0		UE does not support the feature for the given FRX/XDD

x		UE does not support the given FRX/XDD mode

fdd		UE includes the feature in fdd-Add-UE-NR/MRDC-Capabilities

tdd		UE includes the feature in tdd-Add-UE-NR/MRDC-Capabilities

fr1		UE includes the feature in fr1-Add-UE-NR/MRDC-Capabilities

fr2		UE includes the feature in fr2-Add-UE-NR/MRDC-Capabilities

common	UE includes the feature in the common branch (xDD-diff and FRx-diff) for features that do not require FR1/FR2 or FDD/TDD differentiation

|  | FR1 only |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDD | TDD |  |  | FDD | TDD |  |  | FDD | TDD |
| FR1 | 1 | 0 |  | FR1 | 0 | 1 |  | FR1 | 1 | 1 |
| FR2 | x | x |  | FR2 | x | x |  | FR2 | x | x |
| fdd |  |  |  | tdd |  |  |  | common |  |  |
|  | FDD | TDD |  |  | FDD | TDD |  |  |  |  |
| FR1 | 1 | x |  | FR1 | x | 1 |  |  |  |  |
| FR2 | x | x |  | FR2 | x | x |  |  |  |  |
| common |  |  |  | common |  |  |  |  |  |  |
|  | TDD only |  |  |  |  |  |  |  |  |  |
|  | FDD | TDD |  |  | FDD | TDD |  |  | FDD | TDD |
| FR1 | x | 1 |  | FR1 | x | x |  | FR1 | x | 1 |
| FR2 | x | 1 |  | FR2 | x | 1 |  | FR2 | x | 0 |
| common |  |  |  | common |  |  |  | fr1 |  |  |
|  | FDD | TDD |  |  |  |  |  |  |  |  |
| FR1 | x | 0 |  |  |  |  |  |  |  |  |
| FR2 | x | 1 |  |  |  |  |  |  |  |  |
| fr2 |  |  |  |  |  |  |  |  |  |  |
|  | FR1+FR2 |  |  |  |  |  |  |  |  |  |
|  | FDD | TDD |  |  | FDD | TDD |  |  | FDD | TDD |
| FR1 | 1 | 0 |  | FR1 | 0 | 1 |  | FR1 | 0 | 0 |
| FR2 | x | 0 |  | FR2 | x | 0 |  | FR2 | x | 1 |
| fdd |  |  |  | Unable to signal |  |  |  | fr2 |  |  |
|  | FDD | TDD |  |  | FDD | TDD |  |  | FDD | TDD |
| FR1 | 1 | 1 |  | FR1 | 0 | 1 |  | FR1 | 1 | 0 |
| FR2 | x | 0 |  | FR2 | x | 1 |  | FR2 | x | 1 |
| fr1+fdd |  |  |  | fr2+tdd |  |  |  | fr2+fdd |  |  |
|  | FDD | TDD |  |  | FDD | TDD |  |  | FDD | TDD |
| FR1 | 1 | 1 |  | FR1 | 1 | x |  | FR1 | 0 | x |
| FR2 | x | 1 |  | FR2 | x | 1 |  | FR2 | x | 1 |
| common |  |  |  | common |  |  |  | fr2+tdd |  |  |
|  | FDD | TDD |  |  |  |  |  |  |  |  |
| FR1 | 1 | x |  |  |  |  |  |  |  |  |
| FR2 | x | 0 |  |  |  |  |  |  |  |  |
| fr1+fdd |  |  |  |  |  |  |  |  |  |  |

## UE capabilities with xDD differentiation only

Rapproteur’s note: It is rapporteur’s understanding that there is no difference in UE capability bit setting among different interpretations.

Interpretation 1-a, 1-b and 2

| Support for the feature |  | UE capability containers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | xDD-Diff in common | FRX-diff in common | fdd-Add | tdd-Add | fr1-Add | fr2-Add |
| Case 1 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Supported | N/A | Not included | Not included | N/A | N/A |
| Case 2 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | N/A | Not included | Not included | N/A | N/A |
| Case 3 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Not supported | N/A | Not included | Supported | N/A | N/A |
| Case 4 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Not allowed |  |  |  |  |  |
| Case 5 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not allowed |  |  |  |  |  |
| Case 6 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Not allowed |  |  |  |  |  |
| Case 7 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not supported | N/A | Supported | Not included | N/A | N/A |
| Case 8 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not allowed |  |  |  |  |  |

## UE capabilities with FRx differentiation only

Rapproteur’s note: It is rapporteur’s understanding that there is no difference in UE capability bit setting among different interpretations.

Interpretation 1-a, 1-b and 2

| Support for the feature |  | UE capability containers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | xDD-Diff in common | FRX-diff in common | fdd-Add | tdd-Add | fr1-Add | fr2-Add |
| Case 1 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | N/A | Supported | N/A | N/A | Not included | Not included |
| Case 2 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | N/A | Not supported | N/A | N/A | Not included | Not included |
| Case 3 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘supported | Not allowed |  |  |  |  |  |
| Case 4 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | N/A | Not supported | N/A | N/A | Not included | Supported |
| Case 5 | - FR1 FDD: ‘not supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | Not allowed |  |  |  |  |  |
| Case 6 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘supported | Not allowed |  |  |  |  |  |
| Case 7 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘not supported’ - FR2 TDD: ‘not supported | Not allowed |  |  |  |  |  |
| Case 8 | - FR1 FDD: ‘supported’ - FR1 TDD: ‘supported’ - FR2 TDD: ‘not supported | N/A | Not supported | N/A | N/A | Supported | Not included |

Proposal 1:	xxxx

# Summary

xxxxxxxxxx

# Conclusion

xxxxxxxxxx