3GPP TSG RAN WG2 Meeting #110-e                                                         R2-200xxxx

# Electronic meeting, 1st – 12th June 2020

Agenda item:		6.11.1

Source:			Intel Corporation

Title:		Report of email discussion [Post109bis-e][941][PowSav] UE capabilities

Document for:	 	Discussion and decision

# Introduction

This contribution discusses the new RAN2 UE capabilities required to specify the features defined for Rel-16 UE power saving WI considering the related RAN2#109bis-e agreements and the related drafted CRs 38.306 [1] and 38.331 [2] under this email discussion until next RAN2#110-e meeting.

[Post109bis-e][941][PowSav] UE capabilities (Intel)

Intended outcome: draft CR for 38.306 and 38.331 to reflect the latest RAN2 capabilities agreements (on R2 capabilities and the description of related field on 38.331 email discussion) and to discuss if any new R2 capability is required

Deadline: Next Meeting

The deadline of this email discussion #941 is 2020-05-20 23:59 PST, however we suggest having 2 phases:

- Phase 1 until 2020-05-14 23:59 PST for companies to provide their views on the discussion points listed that are related to the RAN2#109bis-e agreements and the drafted CRs to 38.306 and 38.331.

- Phase 2 until 2020-05-20 23:59 PST for companies to provide their views on new discussion points and/or updated CRs, if this is identified necessary by rapporteur based on companies’ inputs during phase 1.

# Discussion

## Review of the drafted CRs on the new RAN2 UE capabilities for Rel-16 POW-SAV WI

This section discusses the drafted CRs to 38.306 and 38.331 that capture the following agreements on RAN2 UE capabilities taken in RAN2#109bis-e meeting:

- A new UE capability (referred e.g. as drx-Preference) is defined to indicate its preference on DRX parameters for power saving in RRC_CONNECTED. It is included in section 4.2.2 “General parameters” of TS 38.306.

- A new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED. It is included in section 4.2.2 “General parameters” of TS 38.306.

- A new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED. It is included in section 4.2.2 “General parameters” of TS 38.306.

- A new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED. It is included in section 4.2.2 “General parameters” of TS 38.306.

- It is not agreed to bundle into a separate new UE capability the support for UE’s assistance of the maxCC, maxBW and maxMIMO.

- A new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving. It is included in section 4.2.2 “General parameters” of TS 38.306.

- New UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature. It is included in section 5 “Optional features without UE radio access capability parameters” of TS 38.306.

- For drx-Preference, maxCC-Preference, release-Preference, maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

- For drx-Preference, maxCC-Preference, and release-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

- For maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as “FR1-FR2 DIFF: Yes” (to the characteristic associated to FR1/FR2 are not equally applicable for FR1/FR2).

- Rel-16 capability maxMIMO-LayerPreference is not linked to Rel-15 capability maxLayersMIMO-Indication

Note that these drafted CRs may have further updates based on companies’ views provided in the section 2.2 below.

### Discussion point 1 in 38.306 drafted CR on RAN2 UE capabilities

- Companies are invited to provide their views on the 38.306 drafted CR [1] that aims to capture the RAN2 UE capability related agreements shown above.

| Company’s name | Company’s comments, if any |
| --- | --- |
| OPPO | We think some description of relaxed measurement should be given in the table of section 5, e.g. relax RRM measurement for neighbouring cells when UE is in RRC IDLE/INACTIVE mode. |
| Ericsson | We share the view from OPPO. Adding on to the suggested wording: Indicates whether the UE supports relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [xx]. [xx] 3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state". |

Summary of inputs provided on Discussion point 1): 2 companies (OPPO and Ericsson) suggest adding further description for the relaxed measurement capability in section 5.

- The description of the relaxed measurement capability in section 5 is updated to include “Indicates whether the UE supports relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [xx]” (as shown in TP of version _v3 of 38.306 CR).

- Companies are invited to provide their views if they do not agree or want to suggest an update on Proposal 1.

| Company’s name | Company’s comments, if any |
| --- | --- |

### Discussion point 2 - 4 in 38.331 drafted CR on RAN2 UE capabilities

- How to do you prefer capturing the POW-SAV feature within the UE-NR-Capability IE? Note that exemplary ASN.1 of a similar legacy structure is also added as reference in Annex A.

- All power saving features are bundled together in ASN.1 within a POW-SAV group, for example, as shown in version #2 of drafted CR to 38.331 (reference of related TP also copied below).

UE-NR-Capability-v16xy ::=               SEQUENCE {

inDeviceCoexInd-r16	E		NUMERATED {supported}			OPTIONAL,

dl-DedicatedMessageSegmentation-r16	ENUMERATED 				OPTIONAL,

nru-Parameters-r16				NRU-Parameters-r16			OPTIONAL,

pow-sav-Preference-r16			POW-SAV-Preference-r16			OPTIONAL,

fr1-Add-UE-NR-Capabilities-v16xy		UE-NR-CapabilityAddFRX-Mode-v16xy	OPTIONAL,

fr2-Add-UE-NR-Capabilities-v16xy		UE-NR-CapabilityAddFRX-Mode-v16xy	OPTIONAL,

nonCriticalExtension			SEQUENCE {}				OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v16xy ::=    SEQUENCE {

pow-sav-PreferencesFRX-Diff     POW-SAV-PreferencesFRX-Diff                OPTIONAL

}

-- TAG-POW-SAV-PREFERENCES-START

POW-SAV-Preference-r16 ::=         SEQUENCE {

pow-sav-PreferencesCommon			POW-SAV-PreferencesCommon					OPTIONAL,

pow-sav-PreferencesFRX-Diff			POW-SAV-PreferencesFRX-Diff					OPTIONAL,

...

}

POW-SAV-PreferencesCommon ::=   SEQUENCE {

drx-Preference-r16				ENUMERATED {supported}					OPTIONAL,

maxCC-Preference-r16			ENUMERATED {supported}					OPTIONAL,

release-Preference-r16			ENUMERATED {supported}					OPTIONAL,

...

}

POW-SAV-PreferencesFRX-Diff ::= SEQUENCE {

maxBW-Preference-r16			ENUMERATED {supported}					OPTIONAL,

maxMIMO-LayerPreference-r16		ENUMERATED {supported}					OPTIONAL,

...

}

-- TAG-POW-SAV-PREFERENCES-STOP

- Each power saving features is captured in ASN.1 separately, for example, as shown in version #0 of drafted CR to 38.331 (reference of related TP also copied below).

UE-NR-Capability-v16xy ::=               SEQUENCE {

inDeviceCoexInd-r16						ENUMERATED {supported}					OPTIONAL,

dl-DedicatedMessageSegmentation-r16		ENUMERATED {supported}					OPTIONAL,

nru-Parameters-r16						NRU-Parameters-r16						OPTIONAL,

drx-Preference-r16						ENUMERATED {supported}					OPTIONAL,

maxBW-Preference-r16					ENUMERATED {supported}					OPTIONAL,

maxCC-Preference-r16					ENUMERATED {supported}					OPTIONAL,

maxMIMO-LayerPreference-r16				ENUMERATED {supported}					OPTIONAL,

release-Preference-r16					ENUMERATED {supported}					OPTIONAL,

fr1-Add-UE-NR-Capabilities-v16xy		UE-NR-CapabilityAddFRX-Mode-v16xy		OPTIONAL,

fr2-Add-UE-NR-Capabilities-v16xy		UE-NR-CapabilityAddFRX-Mode-v16xy		OPTIONAL,

nonCriticalExtension                    SEQUENCE {}								OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v16xy ::=    SEQUENCE {

maxBW-Preference-r16					ENUMERATED {supported}					OPTIONAL,

maxMIMO-LayerPreference-r16				ENUMERATED {supported}					OPTIONAL

}

- If other approach is preferable, please explain your motivation and your suggested TP.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Intel | Option a) |  |
| OPPO | Option a) or Option b) | Both options are ok for us. |
| Huawei | Option a) or b) | Both are ok. |
| Sony | Option a) or b) | No strong view. Option b) is more straight forward, while option a) is more clear for a power save point of view . Though there may be a risk in future when adding new capabilities, how to group those. |
| ZTE | Option a) |  |
| CATT | Option a) | We think “pow-sav-Parameters-r16” would be more appropriate than “pow-sav-Preference-r16” though. |
| Ericsson | Option a) | Also a preference to use “pow-sav-Parameters-r16”, “pow-sav-ParametersCommon” and “pow-savParametersFRX-Diff”. |
| MediaTek | Option a) | I assume UE-NR-CapabilityAddFRX-Mode-v16xy is not needed in option a, as pow-sav-PreferencesFRX-Diff is present in POW-SAV-Preference-r16? Also agree with CATT that it should refer to ‘parameters’ and not ‘preferences’ |
| vivo | Option a) or b) | Both are OK. |

Summary of inputs provided on Discussion point 2): option (a) was supported by 8 companies (OPPO, Huawei, Sony, ZTE, CATT, Ericsson, MediaTek and Intel) while option (b) was also supported by  3 companies (OPPO, Huawei and Sony). One company (Ericsson) also suggest changing the word “preferences” for “parameters” on the group name.

- [ASN.1] All power saving capabilities are bundled together in ASN.1 within a group (as shown in version _v3 of 38.331 CR).

- [ASN.1] For the abbreviation used for the group of all the UE’s power saving features, the word “preferences” is changed for “parameters” (as shown in version _v3 of 38.331 CR).

- Companies are invited to provide their views if they do not agree or want to suggest an update on Proposal 2 or Proposal 3.

| Company’s name | Company’s comments, if any |
| --- | --- |

- If companies supported option a) in previous discussion point 2) or other approach that also requires the usage of the “UE power saving” terminology. Companies are invited to provide their views on the preferable term to be used when abbreviating “UE power saving” reference in 38.331 drafted CR [1] considering e.g. POW-SAV (currently used on version _v2), PS, UPS or UPW.

| Company’s name | Preferred terminology | Company’s comments, if any |
| --- | --- | --- |
| Intel | POW-SAV | We are OK going with majority view on the term to use |
| OPPO | PS or UPS | It is brief and clear to abbreviating with the capitals. |
| Huawei | PS or UPS |  |
| SONY | PS or POW-SAV | PS is shorter. UPS may relate to packet delivery outside 3GPP. |
| ZTE | PS |  |
| CATT | POW-SAV | No strong view though. |
| Ericsson | PS or UPS | In case of pow-sav this should not be in capitals, or? |
| MediaTek | powSav | PS typically refers to the protocol stack, while UPS is a common reference to United Parcel Service. Following the identifier naming rules in RRC (A.3.1.2) the field identifier should be powSavParameters while the type identifier should be PowSavParameters. Only the first letter of an abbreviated word is in upper case and no hyphens are needed. Similarly the hyphen in release-Preference should be removed. |
| vivo | PowSav or PS | We can follow the majority. |

Summary of inputs provided on Discussion point 3): the suggested terminology for power saving features were POW-SAV by 3 (Intel, Sony, CATT), powSav by 1 (MediaTek), PS by 5 (OPPO, Huawei, Sony, ZTE, Ericsson) and  UPS by (OPPO, Huawei and Ericsson). There were concerns raised that UPS may relate to packet delivery outside 3GPP, that PS is also used for protocol stack, and that if POW-SAV were used, it should not be in capital. The TP of version _v3 is updated based on majority view (i.e. PS), although we suggest further discuss in phase 2 whether companies prefer either of the two options that had larger support powSav and PS (note that the hyphen is removed from the names to follow the identifier naming rules in RRC (A.3.1.2)).

- To discuss if companies prefer option (a) powSav vs option (b) PS as an abbreviation used for the group of all the UE’s power saving features.

- [ASN.1] To remove the hyphens from the release-Preference and drx-Preference to align with the identifier naming rules in RRC section A.3.1.2 (as shown in TP of version _v3 of 38.306 and 38.331 CR).

- Companies are invited to provide their views on Proposal 4 (i.e. whether for the abbreviation of the UE’s power saving features: option (a) powSav vs option (b) PS.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Intel | (a) | We have slightly preference to use powSav but are ok going with majority view on this. |
| vivo | a | We think PowSav is more clear than PS. |
| MediaTek | a | powSav is unambiguous |

- Companies are invited to provide their views if they do not agree or want to suggest an update on Proposal 5.

| Company’s name | Company’s comments, if any |
| --- | --- |
| MediaTek | Agree for release-Preference. However, the hyphen in drx-Preference should not be removed. From the naming rules: ‘Field identifiers shall start with a lowercase letter and use mixed case thereafter, e.g., establishmentCause. If a field identifier begins with an acronym (which would normally be in upper case), the entire acronym is lowercase (plmn-Identity, not pLMN-Identity). The acronym is set off with a hyphen (ue-Identity, not ueIdentity), in order to facilitate a consistent search pattern with corresponding type identifiers.’ |

- Companies are invited to provide their views on other aspects of the 38.306 drafted CR [2] that aims to capture the RAN2 UE capability related agreements shown above.

| Company’s name | Company’s comments, if any |
| --- | --- |

## Additional RAN2 UE capabilities for Rel-16 POW-SAV WI or other miscellaneous topics

This section discusses additional RAN2 UE capabilities that may need to be considered based on the following agreements taken in RAN2#109bis-e meeting:

- In NR-DC, SCG specific UAI for power saving can be configured by the network

- The reported UAI for power saving is specific to a cell group

- In (NG)EN-DC, SCG specific UAI for power saving can be configured by the network via SRB1 (using nr-SecondaryCellGroupConfig) or SRB3 (using RRCReconfiguration).

- In (NG)EN-DC, SCG specific UAI for power saving is transmitted in ULInformationTransferMRDC on the LTE leg.

- In (NG)EN-DC, SCG specific UAI for power saving is transmitted on the NR leg via SRB3, if SRB3 is configured.

- In NR-DC, SCG specific UAI for power saving can be configured by the network via SRB1 (using mrdc-SecondaryCellGroup) or SRB3 (using RRCReconfiguration).

- In NR-DC, SCG specific UAI for power saving is transmitted in ULInformationTransferMRDC on SRB1.

- In NR-DC, SCG specific UAI for power saving is transmitted on the SCG via SRB3, if SRB3 is configured.

The latest POW-SAV CR to 38.331 describe how the new UAI features for drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference are associated to an specific cell group. Therefore these UAI related features (with their specific prohibit timers) can be configured specifically for (NG)EN-DC and NR-DC, as shown below:

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, for UE assistance (re-)configuration and reporting for power savings, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, and to reconfigure SDAP for DRBs associated with S-KgNB in NGEN-DC and NR-DC, and to add/modify/release conditional PSCell change configuration, provided that the (re-)configuration does not require any MN involvement. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig, conditionalReconfiguration, otherConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3.

The corresponding UEAssistanceInformation message is sent differently depending on the intended cell group, as shown in reference below from TS 38.331:

The UE shall:

1>	if the UE is in (NG)EN-DC:

2>	if SRB3 is configured:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the UE assistance configuration that triggered this UE assistance information is associated with the SCG:

3>	if SRB3 is configured:

4>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

3>	else:

4>	submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else:

2> submit the UEAssistanceInformation message to lower layers for transmission.

### Discussion point 5

- Should different NR UE capabilities be defined to indicate the UE support of new UE assistance feature for power saving purposes (i.e. drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference) to an specific cell group for NR vs (NG)EN-DC vs NR-DC?

- No.

- Yes. If so, please explained your motivation and your suggested change.

| Company’s name | Option | Company’s comments, if any |
| --- | --- | --- |
| Intel | No | We do not see essential to define different UE capabilities for each specific cell group as the minimum that the UE needs to support is the setup (or release) of the related prohibit timer when the feature is configured (or release). Understanding that later it is up to UE whether to report/trigger UAI or not. |
| OPPO | No | Since UE assistance feature for power saving purposes is introduced for NR but not supported for LTE, a common NR UE capability could be used for SA and the NR CG in (NG)EN-DC or NR-DC. |
| Huawei | No | We don’t see the strong motivation of introducing a new capability. |
| Sony | No | No strong motivation |
| ZTE | No | No clear motivation, |
| CATT | No | We don’t need to define additional NR UE capabilities for MR-DC. But the definitions of these new capabilities in TS 38.306 need to be updated to reflect these capabilities indicate whether UE supports providing the corresponding preference of a cell group. For example: drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. |
| Ericsson | No | Share the view that it should be clarified that the DRX, BW, CC, and MIMO preference capability is per cell group. |
| MediaTek | No | Agree with Oppo |
| vivo | No | We agree that there is no need to define different NR capabilities for NR vs. EN-DC vs. NR-DC. We also share the same view with CATT that the capability should be updated to indicate for a cell group. |

Summary of inputs provided on Discussion point 5): all companies support not defining different NR UE capabilities to indicate the UE support of new UE assistance feature for power saving purposes (i.e. drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference) to an specific cell group for NR vs (NG)EN-DC vs NR-DC.

One company (CATT) indicates that the definition of each capability (in 38.306 CR) should be updated to indicate that UE supports providing the corresponding preference of a cell group.

- Different NR UE capabilities are not defined to indicate the UE support of new UE assistance feature for power saving purposes (i.e. drxPreference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference) per specific cell group.

- The definitions of the new capabilities on drxPreference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference is updated to indicate that the preference is corresponding to a cell group (as shown in TP of version _v3 of 38.306 CR).

- Companies are invited to provide their views if they do not agree or want to suggest an update on Proposal 6 and Proposal 7.

| Company’s name | Company’s comments, if any |
| --- | --- |

### Discussion point 6

- Companies are invited to provide their views on other RAN2 UE capabilities related aspects for Rel-16 UE power saving WI that was not addressed in previous discussion points.

| Company’s name | Company’s comments, if any |
| --- | --- |
| Sony | We would like to highlight as already mentioned at latest RAN2 meeting and in R2-2002670, that based on RAN1 input, a minimum time gap preference should be included. |
| MediaTek | Shouldn’t a capability for ‘MinSchedulingOffsetPreference’ also be included in the list of UAI capabilities? |

Summary of inputs provided on Discussion point 6): two companies suggest defining capabilities for two new UE assistance information: MinTimeGapPreference (by Sony) and MinSchedulingOffsetPreference (by MediaTek). As rapporteur we understand this features are RAN1-driven and should be discussed, if any, in the other email discussion addressing power saving features lead by RAN1/4 and if applicable, they could be combined in the same power saving structure when merging the 38.331 CR to capture all the UE capabilities for RAN1/2/4 driven features. For further reference, note that RAN1 capability table includes “Support of reporting preferred minimum K0/K2 via UE assistance information” as part of UAI (in element 19-4a) but there is no reference on any minimum time gap preference that we are aware of.

- Companies are invited to provide their views if they do not agree or want to suggest an update on the summary of Discussion point 6).

| Company’s name | Company’s comments, if any |
| --- | --- |
| vivo | We should leave these issues to RAN1. If RAN1 did not reach any consensus that UE assistance information on MinTimeGapPreference and MinSchedulingOffsetPreference have benefit, there is no motivation for RAN2 to define such UE assistance information. |
| MediaTek | We are fine with the rapporteur’s suggestion to leave this to R1. |

# Conclusion

In phase-1, 8 companies participated on this email discussion (OPPO, Huawei, Sony, ZTE, CATT, Ericsson, MediaTek, and Intel) and section 2 includes the corresponding conclusions and new discussion points for phase-2. In addition, the CRs to 38.306 and 38.331 are updated accordingly in version _v3.

The proposals captured are the following:

Proposal 1.	The description of the relaxed measurement capability in section 5 is updated to include “Indicates whether the UE supports relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [xx]” (as shown in TP of version _v3 of 38.306 CR).

Proposal 2.	[ASN.1] All power saving capabilities are bundled together in ASN.1 within a group (as shown in version _v3 of 38.331 CR).

Proposal 3.	[ASN.1] For the abbreviation used for the group of all the UE’s power saving features, the word “preferences” is changed for “parameters” (as shown in version _v3 of 38.331 CR).

Proposal 4.	To discuss if companies prefer option (a) powSav vs option (b) PS as an abbreviation used for the group of all the UE’s power saving features.

Proposal 5.	[ASN.1] To remove the hyphens from the release-Preference and drx-Preference to align with the identifier naming rules in RRC section A.3.1.2 (as shown in TP of version _v3 of 38.306 and 38.331 CR).

Proposal 6.	Different NR UE capabilities are not defined to indicate the UE support of new UE assistance feature for power saving purposes (i.e. drxPreference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference) per specific cell group.

Proposal 7.	The definitions of the new capabilities on drxPreference, maxBW-Preference, maxCC-Preference, maxMIMO-Preference, and releasePreference is updated to indicate that the preference is corresponding to a cell group (as shown in TP of version _v3 of 38.306 CR).

# References

- R2-200xxxx	UE capabilities for Rel-16 Power Saving WI, Intel Corporation, Rel-16, TS 38.306, v16.0.0, draftCR, category B, NR_UE_pow_sav.

- R2-200xxxx	UE capabilities for Rel-16 Power Saving WI, Intel Corporation, Rel-16, TS 38.331, v16.0.0, draftCR, category B, NR_UE_pow_sav.

# Annex A

The features that can be configured different for FR1/FR2 or TDD/FDD are defined in a way that a UE can indicate via the general section whether it support it for both (e.g. FR1/FR2), or if it only supports it for one (e.g. FR1 or FR2). For better understanding, an example on how this is captured in TS 38.306 is shown below. It is highlighted in yellow the common structure that includes amongst others, the features that are supported for both FR1/FR2 and, in blue the section that is used by UE to indicate when the feature is only supported for FR1 or FR2. For further reference, it is highlighted in green the common field that is used for those features that could be supported differently for FR1 or FR2.

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                     SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC         ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC       ENUMERATED {supported}        OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

For further clarification, the UE’s expected handling of the capability signaling just explained is also described within the following procedural text:

1>  set all fields of UE-NR/MRDC-Capability except fdd-Add-UE-NR/MRDC-Capabilities, tdd-Add-UE-NR/MRDC-Capabilities, fr1-Add-UE-NR/MRDC-Capabilities and fr2-Add-UE-NR/MRDC-Capabilities, to include the values applicable for all duplex mode(s) and frequency range(s) that the UE supports;

1>  if UE supports both FDD and TDD and if (some of) the UE capability fields have a different value for FDD and TDD

2>  if for FDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>  include field fdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD;

2>  if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>  include field tdd-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD;

1>  if UE supports both FR1 and FR2 and if (some of) the UE capability fields have a different value for FR1 and FR2:

2>  if for FR1, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>  include field fr1-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR1;

2>  if for FR2, the UE supports additional functionality compared to what is indicated by the previous fields of UE-NR/MRDC-Capability:

3>  include field fr2-Add-UE-NR/MRDC-Capabilities and set it to include fields reflecting the additional functionality applicable for FR2;