3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

# Online, June 01 – 12, 2020

Agenda item:	x.y.z

Source: 	Ericsson

Title: 	Email discussion report: [Post109bis-e][947][POS] TRP-ID structure (Ericsson)

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

- [Post109bis-e][947][POS] TRP-ID structure (Ericsson)

Scope: Discuss the proposals for restructuring the TRP-ID and determine which fields are needed for which case.  The table in R2-2003997 can be used as starting point.

Intended outcome: Summary for next meeting

Deadline:  Long

To allow a discussion of the summary as well, companies are asked to provide comments no later than Tuesday May 19th, 10.00 UTC.

Section 2 provides discussion templates per IE (in some cases pairs of IEs), where companies are asked to provide technical motivations if additional TRP identifiers are considered needed. The baseline is the analysis in [3] and the company comments in [1].

## References

[1]	R2-2003997, "Email discussion report: [AT109bis-e][610][POS] LPP proposals (Ericsson).

[2]	R2-20xxxxx, "RAN2 Chairman’s Notes", RAN2#109bis-e.

[3] R2-2003318, "Handling on TRP-ID", Intel Corporation

# 2. 	Discussion

It is argued in [2] and [3] that the IE TRP-ID in RAN2 need to be better defined to avoid confusion with RAN3, its use needs to be clarified and in what IEs it is needed	and how the identifiers associated to a TRP shall be represented.

According to [2], it is enough with the PRS ID to uniquely identify a TRP within an LPP session between LMF and a UE. Furthermore, [3] provides a summary of TRP-ID issues. Ultimately, the following Table of required TRP-ID elements for various IEs is derived in [4]:

| IE name | Required fields |
| --- | --- |
| NR-Multi-RTT-MeasElement | dl-PRS-ID |
| NR-DL-AoD-MeasElement | dl-PRS-ID |
| NR-DL-TDOA-MeasElement | dl-PRS-ID |
| NR-MeasuredResultsElement | pci, CGI and ARFCN |
| NR-TimeStamp | None |
| DL-PRS-IdInfo | dl-PRS-ID |
| NR-DL-PRS-AssistanceDataPerTRP | dl-PRS-ID and ARFCN |
| NR-SSB-Config | PCI and ARFCN |
| ReferenceTRP-RTD-Info | dl-PRS-ID |
| RTD-InfoElement | dl-PRS-ID |
| NR-DL-PRS-BeamInfo | dl-PRS-ID |
| TRP-LocationInfoElement | dl-PRS-ID |

The above Table summarizes the IEs which currently make use of the IE TRP-ID, and which fields of the IE TRP-ID is/are required for the functionality in the corresponding parent IE.

In addition, the summary Rapporteur’s comment [1] is that a Cell-ID may be required in NR-TimeStamp to indicate the cell/TRP from which the SFN has been derived.

In order to avoid unnecessary identifiers, we start this email discussion from a baseline of a minimalistic set of identifiers based on [3] (Table above), as suggested during the online discussion at RAN2#109bis-e. The simplest approach for a minimalistic set if to redefine TRP-ID to represent the INTEGER (0..255) identifying a TRP among the TRPs a target device can handle as per RAN1 agreement:

#### – TRP-ID The IE TRP-ID provides the ID to identify the TRP among the TRPs the target device can handle. -- ASN1START TRP-ID-r16 ::= INTEGER (0..255) -- ASN1STOP TRP-ID field descriptions. TRP-ID This field is used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource. This ID can be associated with multiple DL PRS Resource Sets associated with a single TRP. Each TRP can only be associated with one such ID..

This means that identifiers used in LTE might be omitted, but can be added based on a technical motivation. In the following subsections, the individual IEs and the necessary additional identifers are discussed

## 2.1	 NR-Multi-RTT-MeasElement

The NR-Multi-RTT-MeasElement IE is part of the IE NR-Multi-RTT-SignalMeasurementInformation and is defined as below:

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range to be decided in RAN4

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16	OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-TimingMeasQuality-r16			NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16		OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in FFS.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value or the nr-UE-RxTxTimeDiffAdditional value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.1 Need for additional TRP identifiers in NR-Multi-RTT-MeasElement-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | dl-PRS-ID, nr-PhysCellId, nr-CellGlobalId Considering broadcast and positioning SIB may be different for different cell, Cell ID should be included, and optional. No need for ARFCN, as dl-PRS-PointA is already provided in NR-DL–PRS-PositioningFrequencyLayer |
| Qulcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId I understand the nr-ARFCN can disambiguate the nr-PhysCellId in some cases. I.e., this is the same as in Rel-15 LPP where PCI/ARFCN can be provided as pair. If this is not applicable to NR (and we made a mistake in Rel-15), then I agree with Huawei and nr-ARFCN is not needed (note, I understand this nr-ARFCN is not supposed to be the dl-PRS-PointA). As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurements. E.g., when the assistance data are not provided from the same source or the same LPP session. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in this IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in UL report here for multi-RTT. |
| Ericsson | We read the RAN1 agreement differently. The TRP ID is like the country code of a telephone number that together with an area code and a local number identifies the number identity. Same here, where the DL PRS resource is identified by a TRP-ID, a DL-PRS resource set ID and a DL PRS resource ID. Therefore, to name the “country code” a “local number identifier” would be strange, and to name the identify of the TRP a DL-PRS ID would also be confusing. It is important to consider the context here. A UE requesting DL-PRS assistance data is including the nr-CellGlobalId to the LMF and in return obtains a DL-PRS resources in a hierarchy based on TRPs per frequency layers. A UE retrieving assistance data via system information broadcast from a cell also obtains the nr-CellGlobalId of that cell. Therefore, there is already nr-CellGlobalId + TRP ID provided to the UE to ensure that the UE can handle information from different sources for UEB. Therefore, it is enough to provide a TRP ID 0..255 to the UE. When the UE provides measurements to the LMF, the corresponding measurement is tied to a TRP with a TRP ID, and since the UE can be configured with up to 4*64=256 TRPs, the TRP ID 0..255 is enough to identify the measurement as part of UEA. With a globally unique cell identifier in the unicast AD request and in the broadcast SIB1, and a list of TRPs, each with a TRP ID, how can there be a need for something in addition to that? We do not see any technical motivation for additional identifiers. |

## 2.2	NR-DL-AoD-MeasElement

The NR-DL-AoD-MeasElement IE is part of the IE NR-DL-AoD-SignalMeasurementInformation and is defined as below:

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16		NR-DL-AoD-AdditionalMeasurements-r16,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-MeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.2 Need for additional TRP identifiers in NR-DL-AoD-MeasElement-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | dl-PRS-ID, nr-PhysCellId, nr-CellGlobalId Considering broadcast and positioning SIB may be different for different cell, Cell ID should be included, and optional. No need for ARFCN, as dl-PRS-PointA is already provided in NR-DL–PRS-PositioningFrequencyLayer |
| Qualcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId I understand the nr-ARFCN can disambiguate the nr-PhysCellId in some cases. I.e., this is the same as in Rel-15 LPP where PCI/ARFCN can be provided as pair. If this is not applicable to NR (and we made a mistake in Rel-15), then I agree with Huawei and nr-ARFCN is not needed (note, I understand this nr-ARFCN is not supposed to be the dl-PRS-PointA). As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurements. E.g., when the assistance data are not provided from the same source or the same LPP session. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in this IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in UL report here for DL-AoD. |
| Ericsson | We read the RAN1 agreement differently. The TRP ID is like the country code of a telephone number that together with an area code and a local number identifies the number identity. Same here, where the DL PRS resource is identified by a TRP-ID, a DL-PRS resource set ID and a DL PRS resource ID. Therefore, to name the “country code” a “local number identifier” would be strange, and to name the identify of the TRP a DL-PRS ID would also be confusing. It is important to consider the context here. A UE requesting DL-PRS assistance data is including the nr-CellGlobalId to the LMF and in return obtains a DL-PRS resources in a hierarchy based on TRPs per frequency layers. A UE retrieving assistance data via system information broadcast from a cell also obtains the nr-CellGlobalId of that cell. Therefore, there is already nr-CellGlobalId + TRP ID provided to the UE to ensure that the UE can handle information from different sources for UEB. Therefore, it is enough to provide a TRP ID 0..255 to the UE. When the UE provides measurements to the LMF, the corresponding measurement is tied to a TRP with a TRP ID, and since the UE can be configured with up to 4*64=256 TRPs, the TRP ID 0..255 is enough to identify the measurement as part of UEA. With a globally unique cell identifier in the unicast AD request and in the broadcast SIB1, and a list of TRPs, each with a TRP ID, how can there be a need for something in addition to that? We do not see any technical motivation for additional identifiers. |

## 2.3	NR-DL-TDOA-MeasElement

The NR-DL-TDOA-MeasElement IE is part of the IE NR-DL-TDOA-SignalMeasurementInformation and is defined as below:

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16	NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16				NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),	-- FFS on the value range to be decided in RAN4

dl-PRS-RSRP-ResultDiff-r16	INTEGER (FFS)		OPTIONAL, -- FFS on the value range	to be decided in RAN4

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs per UE

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.3 Need for additional TRP identifiers in NR-DL-TDOA-MeasElement-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | dl-PRS-ID, nr-PhysCellId, nr-CellGlobalId Considering broadcast and positioning SIB may be different for different cell, Cell ID should be included, and optional. No need for ARFCN, as dl-PRS-PointA is already provided in NR-DL–PRS-PositioningFrequencyLayer |
| Qualcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId I understand the nr-ARFCN can disambiguate the nr-PhysCellId in some cases. I.e., this is the same as in Rel-15 LPP where PCI/ARFCN can be provided as pair. If this is not applicable to NR (and we made a mistake in Rel-15), then I agree with Huawei and nr-ARFCN is not needed (note, I understand this nr-ARFCN is not supposed to be the dl-PRS-PointA). As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurements. E.g., when the assistance data are not provided from the same source or the same LPP session. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in this IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in UL report here for DL-TDOA. |
| Ericsson | We read the RAN1 agreement differently. The TRP ID is like the country code of a telephone number that together with an area code and a local number identifies the number identity. Same here, where the DL PRS resource is identified by a TRP-ID, a DL-PRS resource set ID and a DL PRS resource ID. Therefore, to name the “country code” a “local number identifier” would be strange, and to name the identify of the TRP a DL-PRS ID would also be confusing. It is important to consider the context here. A UE requesting DL-PRS assistance data is including the nr-CellGlobalId to the LMF and in return obtains a DL-PRS resources in a hierarchy based on TRPs per frequency layers. A UE retrieving assistance data via system information broadcast from a cell also obtains the nr-CellGlobalId of that cell. Therefore, there is already nr-CellGlobalId + TRP ID provided to the UE to ensure that the UE can handle information from different sources for UEB. Therefore, it is enough to provide a TRP ID 0..255 to the UE. When the UE provides measurements to the LMF, the corresponding measurement is tied to a TRP with a TRP ID, and since the UE can be configured with up to 4*64=256 TRPs, the TRP ID 0..255 is enough to identify the measurement as part of UEA. With a globally unique cell identifier in the unicast AD request and in the broadcast SIB1, and a list of TRPs, each with a TRP ID, how can there be a need for something in addition to that? We do not see any technical motivation for additional identifiers. |

## 2.4	NR-MeasuredResultsElement

The NR-MeasuredResultsElement IE is part of the IE NR-ECID-SignalMeasurementInformation and is defined as below:

-- ASN1START

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16	OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber				BIT STRING (SIZE (10)),

trp-ID-r16						TRP-ID-r16					OPTIONAL,

nr-PhysCellId-r16				NR-PhysCellId-r16,

nr-CellGlobalId-r16				NCGI-r15					OPTIONAL,	-- Need ON

nrARFCNRef-r16					ARFCN-ValueNR-r15,

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

...

}

MeasQuantityResults-r16 ::= SEQUENCE {

nr-RSRP-r16	INTEGER (0..127)		OPTIONAL,

nr-RSRQ-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

-- ASN1STOP

NR-ECID-SignalMeasurementInformation field descriptions

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

nr-PhysCellId This field specifies the physical cell identity of the measured cell.

nr-CellGlobalId This field specifies the NCGI of the measured cell. The target device shall provide this field if it was able to determine the NCGI of the measured cell at the time of measurement .

nrARFCNRef This field specifies the NRARFCN of the measured NR carrier frequency.

resultsSSB-Cell This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsCSI-RS-Cell This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

ssb-Results This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement per SSB resource, as defined in TS 38.331 [35].

csi-RS-Results This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) per CSI-RS resource, as defined in TS 38.331 [35].

primaryCellMeasuredResults This field contains measurements for the primary cell when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements for the primary cell is reported in the measuredResultsList.

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.4 Need for additional TRP identifiers in NR-MeasuredResultsElement-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | For ARFCN, first we do not think that we need “Ref”. Second the ARFCN should either be SSB frequency if RRM is based on SSB or pointA of CSI-RS if RRM is based on CSI-RS. UE may not be able to identify the carrier ARFCN of a neighbouring cell through RRM. |
| Qualcomm | nr-PhysCellId/nr-ARFCN, nr-CellGlobalId are needed. If they are provided individually, the trp-ID-r16 is not needed. I understand the nr-ARFCN can disambiguate the nr-PhysCellId in some cases. I.e., this is the same as in Rel-15 LPP where PCI/ARFCN can be provided as pair. If this is not applicable to NR (and we made a mistake in Rel-15), then nr-ARFCN can be removed. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in the IE and can be provided when needed/appropriate. However, if desired, the DL-PRS ID could be moved to the NR-DL-PRS-Config IE. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in UL report here for NR ECID. |
| Ericsson | If the timestamp does not include any reference to which cell it has been adopted from, then it should be in the reference TRP information, and not in every element in NR-MeasuredResultsList-r16. |

## 2.5	NR-TimeStamp

The NR-TimeStamp IE is part of several IEs and is defined as below:

-- ASN1START

NR-TimeStamp-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16			OPTIONAL,-- Cond NotSameAsRefServ0

nr-PhysCellId-r16			NR-PhysCellId-r16 	OPTIONAL,-- Cond NotSameAsRefServ0

nr-SFN-r16			INTEGER (0..1023),

nr-Slot-r16 		CHOICE {

scs15			INTEGER (0..9),

scs30			INTEGER (0..19),

scs60			INTEGER (0..39),

scs120			INTEGER (0..79)

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsRefServ0 | The field is mandatory present if the SFN is not from the reference TRP; otherwise it is not present. |

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.5 Need for additional TRP identifiers in NR-TimeStamp-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | No need to include TRP ID or PCI, as it was agreed in RAN1 and captured in RAN1 specification, that the assistance data reference is used to identify the time stamp timing. Agreement (RAN1#99): Modify the previous agreement on the definition of the time stamp as follows: A UE measurement can be associated with a time stamp. For UE RSTD, DL PRS RSRP and UE Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing. These values correspond to the reference provided by the DL-PRS-RstdReferenceInfo. TS 38.214 For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo. |
| Qualcomm | The proposed NR-PhysCellId-r16 in the ASN.1 above is included in IE TRP-ID-r16, so no change is needed. The NR-TimeStamp-r16 can also provide the time stamp for the location estimate (UE-based); e.g., IE NR-DL-TDOA-LocationInformation, for which the RAN1 agreement cited by Huawei above seems not applicable (i.e., the TRP-ID is optional present). |
| OPPO | We are not sure about the necessity of PCI/Arfcn/CGI information here in timestamp. If take DL TDOA as an example: - For the time stamp included in NR-DL-AoD-MeasElement-r16, we assume the agreement cited by Huawei is applicable, so no need for additional information at all (not even PCI); - For the time stamp included in NR-DL-TDOA-LocationInformation, if Qualcomm comment is correct, and thus cell information is needed, we wonder if PCI is enough, considering the possible PCI confusion issue. As commented above, So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. |
| Ericsson | In response to the QC comment about this already being present in a complex IE: The clear majority of companies from the RAN2#109bis email discussion were in favor of splitting the TRP ID of the baseline into separate fields, so TRP ID (or another name) in this context is 0..255 and not including PCI. We agree with QC on the necessity to ensure that SFN is well-defined in all cases. |

## 2.6	DL-PRS-IdInfo

The IE DL-PRS-IdInfo provides the IDs of the reference and neighbour TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-IdInfo-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16		,

nr-DL-PRS-ResourceID-List-r16	(SIZE (1..nrMaxResourceIDs)) OF NR-DL-PRS-ResourceId-r16 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL

}

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.6 Need for additional TRP identifiers in DL-PRS-IdInfo-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | dl-PRS-ID, nr-PhysCellId, nr-CellGlobalId Considering broadcast and positioning SIB may be different for different cell, Cell ID should be included, and optional. No need for ARFCN, as dl-PRS-PointA is already provided in NR-DL–PRS-PositioningFrequencyLayer |
| Qualcomm | dl-PRS-ID , nr-PhysCellId/nr-ARFCN, nr-CellGlobalId are needed. As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurement. E.g., when the assistance data are not provided from the same source or the same LPP session. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in the IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in DL PRS info here. |
| Ericsson | As explained above, there is a nr-CellGlobalId presented together with the DL-PRS in the broadcast, and a nr-CellGlobalId in the AD request in case of unicast, so an additional cell ID is not needed. |

## 2.7	NR-DL-PRS-AssistanceDataPerTRP

The NR-DL-PRS-AssistanceDataPerTRP IE is part of the IE NR-DL-PRS-AssistanceData and is defined as below:

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 DL-PRS-IdInfo-r16	OPTIONAL,		-- Need ON

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16			SEQUENCE (SIZE (0..255)) OF NR-SSB-Config-r16,

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceDataPerFreq (SIZE (1..nrMaxTRPsPerFreq)) OF NR-DL-PRS-AssistanceDataPerTRP-r16,

nr-DL–PRS-PositioningFrequencyLayer-r16	NR-DL–PRS-PositioningFrequencyLayer-r16	OPTIONAL,	--Need ON

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

nr-DL-PRS-expectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-expectedRSTD-uncerainty-r16	INTEGER (-246..246),

trp-ID-r16					TRP-ID-r16		,

nr-DL-PRS-Config-r16			NR-DL-PRS-Config-r16,

...

}

NR-DL–PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16		INTEGER (1..63),

dl-PRS-StartPRB-r16					INTEGER(0..2176),

dl-PRS-PointA-r16					ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16				ENUMERATED {normal, extended, ...},

...

}

nrMaxFreqLayers	INTEGER ::= 4	-- Max freq layers

nrMaxTRPsPerFreq		INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxResourceIDs INTEGER ::= 64		-- Max ResourceIDs

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.7 Need for additional TRP identifiers in NR-DL-PRS-AssistanceDataPerTRP-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | dl-PRS-ID, nr-PhysCellId, nr-CellGlobalId Considering broadcast and positioning SIB may be different for different cell, Cell ID should be included, and optional. No need for ARFCN, as dl-PRS-PointA is already provided in NR-DL–PRS-PositioningFrequencyLayer |
| Qualcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId I understand the nr-ARFCN can disambiguate the nr-PhysCellId in some cases. I.e., this is the same as in Rel-15 LPP where PCI/ARFCN can be provided as pair. If this is not applicable to NR (and we made a mistake in Rel-15), then I agree with Huawei and nr-ARFCN is not needed (note, I understand this nr-ARFCN is not supposed to be the dl-PRS-PointA). As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurement. E.g., when the assistance data are not provided from the same source or the same LPP session. In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in the IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in DL PRS info here. |
| Ericsson | As explained above, there is a nr-CellGlobalId presented together with the DL-PRS in the broadcast, and a nr-CellGlobalId in the AD request in case of unicast, so an additional cell ID is not needed. |

## 2.8	NR-SSB-Config

The IE NR-SSB-Config is part of the IE NR-DL-PRS-AssistanceData and is defined as:

-- ASN1START

NR-SSB-Config-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16,

nr-PhysCellId-r16			NR-PhysCellId-r16,

nrARFCNRef-r16				ARFCN-ValueNR-r15,

ss-PBCH-BlockPower-r16		INTEGER (-60..50),

halfFrameIndex-r16					INTEGER (0..1),

SSB-periodicity-r16					ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, ...},

ssb-PositionsInBurst-r16			CHOICE {

shortBitmap-r16						BIT STRING (SIZE (4)),

mediumBitmap-r16					BIT STRING (SIZE (8)),

longBitmap-r16						BIT STRING (SIZE (64))

}	OPTIONAL, --Need OR

ssbSubcarrierSpacing-r16			ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ...},

sfn-SSB-Offset-r16					INTEGER (0..15),

smtc-r16							SEQUENCE {

periodicityAndOffset-r16				CHOICE {

sf5										INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160									INTEGER (0..159)

},

duration-r16						ENUMERATED { sf1, sf2, sf3, sf4, sf5, ... }

}

}

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.8 Need for additional TRP identifiers in NR-SSB-Config-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | No need to include TRP-ID or PRS-ID for SSB configuration. OK have PCI and ARFCN, but nrARFCNRef-r16 should be nrARFCN-r16. |
| Qaulcomm | No need for DL-PRS-ID. But since all fields in IE TRP-ID-r16 are optional present, I assume a NW is smart enough to provide the required fields in each case. Alternativlely, as mentioned above, the DL-PRS ID could be moved to the NR-DL-PRS-Config IE. |
| OPPO | dl-PRS-ID + nr-PhysCellId/nr-ARFCN + nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. However, on top of that, for SSB configuration, the information of PCI and Arfcn helps the UE to locate the SSB, i.e., not from the uniquely identifying the SSB perspective, but from helping the UE to locate the SSB perspective, the additional information is helpful. So for this reason, the additional information of PCI and Arfcn can be used, even in addition to CGI information. |
| Ericsson | We agree, nrARFCNRef-r16 should be nrARFCN-r16. The TRP-ID shall be removed, it is not needed. |

## 2.9	ReferenceTRP-RTD-Info and RTD-InfoElement

The IEs ReferenceTRP-RTD-Info and RTD-InfoElement is part of the IE NR-RTD-Info and is defined by

-- ASN1START

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingMeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..4)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..63)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

trp-id-r16					TRP-ID-r16,

subframeOffset-r16			INTEGER (0..1966079),

rtd-Quality-r16				NR-TimingMeasQuality-r16,

...

}

-- ASN1STOP

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.9 Need for additional TRP identifiers in ReferenceTRP-RTD-Info-r16 and RTD-InfoElement-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | We think only dl-PRS-ID is needed. |
| Qualcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurement. E.g., when the assistance data are not provided from the same source or the same LPP session. Also, e.g. RTD and TRP location info can be provided in different posSIBs, and a UE may get the posSIBs from different cells. A UE need to be able to uniquely associate the assistance data to the correct TRP, even when provided from different sources (e.g., different cells/posSIBs, different LPP messages of the same or different LPP session, MO-LR, etc.). In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in the TRP-ID IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in RTD info here. |
| Ericsson | It is important to consider the context here. A UE requesting DL-PRS assistance data is including the nr-CellGlobalId to the LMF and in return obtains a DL-PRS resources in a hierarchy based on TRPs per frequency layers. A UE retrieving assistance data via system information broadcast from a cell also obtains the nr-CellGlobalId of that cell. Therefore, there is already nr-CellGlobalId + TRP ID provided to the UE to ensure that the UE can handle information from different sources for UEB. Therefore, it is enough to provide a TRP ID 0..255 to the UE. |

## 2.10	NR-TRP-LocationInfo and NR-DL-PRS-BeamInfo

The IEs NR-TRP-LocationInfo and, NR-DL-PRS-BeamInfo are defined as below:

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - trp-id: This field provides an identity of the TRP. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-DL-PRS-BeamInfoPerFreqLayer-r16

NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..64)) OF NR-DL-PRS-BeamInfo-r16

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

trp-id-r16							TRP-ID-r16,

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16,

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..2)) OF DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..64)) OF DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

trp-id This field provides an identity of the TRP.

lcs-gcs-translation-parameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field is absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-prs-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 0.1 degrees; range 0 to 180 degrees.

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

beta This field specifies the downtilts angle β for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

Companies are asked to identify additional TRP identifiers that are considered needed as to provide a solid technical motivation.

| Table 2.10 Need for additional TRP identifiers in NR-TRP-LocationInfo-r16 and NR-DL-PRS-BeamInfo-r16 |  |
| --- | --- |
| Company | Comments |
| Huawei/HiSilicon | We think only dl-PRS-ID is needed. |
| Qualcomm | dl-PRS-ID, nr-PhysCellId/nr-ARFCN, nr-CellGlobalId As mentioned in the comment above (RAN1 agreement), the dl-PRS-ID identifies a DL-PRS Resource of a TRP, but not necessarily the TRP. Therefore, the possible identifiers of a TRP (nr-PhysCellId/nr-ARFCN, nr-CellGlobalId) need to be provided in some cases to uniquely identify a TRP and associated measurement. E.g., when the assistance data are not provided from the same source or the same LPP session. Also, e.g. RTD and TRP location info can be provided in different posSIBs, and a UE may get the posSIBs from different cells. A UE need to be able to uniquely associate the assistance data to the correct TRP, even when provided from different sources (e.g., different cells/posSIBs, different LPP messages of the same or different LPP session, MO-LR, etc.). In general, we cannot see anything wrong with the current LPP (apart from the somewhat misleading name of the TRP-ID IE). All fields are optional present in the TRP-ID IE and can be provided when needed/appropriate. |
| OPPO | dl-PRS-ID + Either nr-PhysCellId/nr-ARFCN Or nr-CellGlobalId PRS ID only is not enough since it is unique within a TRP but not across TRPs. So to uniquely identify a TRP, either the combination of nr-PhysCellId/nr-ARFCN or nr-CellGlobalId can work, by assuming no local PCI confusion at a same local area for a same frequency. May be the latter one, i.e., nr-CellGlobalId, is safer. This applies to both UL and DL. So we fail to see the need to include both the combination of nr-PhysCellId/nr-ARFCN and nr-CellGlobalId in TRP location and beam info here. |
| Ericsson | It is important to consider the context here. A UE requesting DL-PRS assistance data is including the nr-CellGlobalId to the LMF and in return obtains a DL-PRS resources in a hierarchy based on TRPs per frequency layers. A UE retrieving assistance data via system information broadcast from a cell also obtains the nr-CellGlobalId of that cell. Therefore, there is already nr-CellGlobalId + TRP ID provided to the UE to ensure that the UE can handle information from different sources for UEB. Therefore, it is enough to provide a TRP ID 0..255 to the UE. |

# 3. 	Summary

The ambition of the email discussion is to agree on a text proposal based on the technical discussion.

# 4. 	Conclusion

# Annex 1, Text proposal to 3GPP TS 37.355 for TRP-ID

[…]

#### 6.4.3.1	Common NR assistance data Information Elements

[…]

#### –	TRP-ID

The IE TRP-ID provides the ID to identify the TRP among the TRPs the target device can handle. This field is used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource. This ID can be associated with multiple DL PRS Resource Sets associated with a single TRP.

Each TRP can only be associated with one such ID.

-- ASN1START

TRP-ID-r16 ::=

INTEGER (0..255)

-- ASN1STOP

#### –	NR-SSB-Config

The IE NR-SSB-Config defines SSB configuration.

-- ASN1START

NR-SSB-Config-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16,

nr-PhysCellId-r16			NR-PhysCellId-r16,

nrARFCNRef-r16				ARFCN-ValueNR-r15,

ss-PBCH-BlockPower-r16		INTEGER (-60..50),

halfFrameIndex-r16					INTEGER (0..1),

SSB-periodicity-r16					ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, ...},

ssb-PositionsInBurst-r16			CHOICE {

shortBitmap-r16						BIT STRING (SIZE (4)),

mediumBitmap-r16					BIT STRING (SIZE (8)),

longBitmap-r16						BIT STRING (SIZE (64))

}	OPTIONAL, --Need OR

ssbSubcarrierSpacing-r16			ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ...},

sfn-SSB-Offset-r16					INTEGER (0..15),

smtc-r16							SEQUENCE {

periodicityAndOffset-r16				CHOICE {

sf5										INTEGER (0..4),

sf10									INTEGER (0..9),

sf20									INTEGER (0..19),

sf40									INTEGER (0..39),

sf80									INTEGER (0..79),

sf160									INTEGER (0..159)

},

duration-r16						ENUMERATED { sf1, sf2, sf3, sf4, sf5, ... }

}

}

-- ASN1STOP

NR-SSB-Config field descriptions

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [39], clause 4.1. The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-periodicityServingCell The SSB periodicity in ms for the rate matching purpose. If the field is absent, the UE applies the value ms5. (see TS 38.213 [39], clause 4.1).

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

smtc The SSB periodicity/offset/duration configuration.

ssb-Index For a DL PRS resource, SSB index indicated for QCL Type D and QCL Type C is same.

[…]

#### –	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..64)) OF TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

trp-id-r16						TRP-ID-r16,

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..2)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..64)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - trp-id: This field provides an identity of the TRP. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this resource set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	ReferencePoint

The IE ReferencePoint provides a well defined location relative to which other locations may be defined.

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

RelativeLocation-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255),

verticalConfidence-r15					INTEGER (0..100)

}

-- ASN1STOP

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 meter, and 1 meters, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [meters]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..4)) OF NR-DL-PRS-BeamInfoPerFreqLayer-r16

NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..64)) OF NR-DL-PRS-BeamInfo-r16

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE {

trp-id-r16							TRP-ID-r16,

lcs-gcs-translation-parameter-r16	LCS-GCS-Translation-Parameter-r16	OPTIONAL,	-- Need OP

dl-prs-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16,

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..2)) OF DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..64)) OF DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

trp-id This field provides an identity of the TRP.

lcs-gcs-translation-parameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field is absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-prs-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 0.1 degrees; range 0 to 180 degrees.

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

beta This field specifies the downtilts angle β for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 0.1 degrees; range 0 to 359.9 degrees.

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs.

-- ASN1START

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

ref-trp-id-r16					TRP-ID-r16,

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingMeasQuality-r16				OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..4)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..63)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

trp-id-r16					TRP-ID-r16,

subframeOffset-r16			INTEGER (0..1966079),

rtd-Quality-r16				NR-TimingMeasQuality-r16,

...

}

-- ASN1STOP

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - ref-trp-id: This field specifies the identity of the reference TRP. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

trp-id-r16 This fields provides the identity of the TRP for which the RTD-InfoElement is applicable.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [41]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 DL-PRS-IdInfo-r16	OPTIONAL,		-- Need ON

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers)) OF NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16			SEQUENCE (SIZE (0..255)) OF NR-SSB-Config-r16,

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceDataPerFreq (SIZE (1..nrMaxTRPsPerFreq)) OF NR-DL-PRS-AssistanceDataPerTRP-r16,

nr-DL–PRS-PositioningFrequencyLayer-r16	NR-DL–PRS-PositioningFrequencyLayer-r16	OPTIONAL,	--Need ON

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

nr-DL-PRS-expectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-expectedRSTD-uncerainty-r16	INTEGER (-246..246),

trp-ID-r16					TRP-ID-r16		,

nr-DL-PRS-Config-r16			NR-DL-PRS-Config-r16,

...

}

NR-DL–PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16		INTEGER (1..63),

dl-PRS-StartPRB-r16					INTEGER(0..2176),

dl-PRS-PointA-r16					ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16				ENUMERATED {normal, extended, ...},

...

}

nrMaxFreqLayers	INTEGER ::= 4	-- Max freq layers

nrMaxTRPsPerFreq		INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxResourceIDs INTEGER ::= 64		-- Max ResourceIDs

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-Config This field specifies the PRS configuration of the TRP.

nr-DL-PRS-ReferenceInfo This field indicates the IDs of the reference TRP.

nr-DL-PRS-ResourceID-List The list of nr-DL PRS resource ID. Only a single nr-DL-PRS-ResourceId is included if the field is used in measurement reporting.

#### –	DL-PRS-IdInfo

The IE DL-PRS-IdInfo provides the IDs of the reference and neighbour TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-IdInfo-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16		,

nr-DL-PRS-ResourceID-List-r16	(SIZE (1..nrMaxResourceIDs)) OF NR-DL-PRS-ResourceId-r16 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL

}

-- ASN1STOP

[…]

[…]

#### 6.4.3.2	Common NR report Information Elements

#### –	NR-TimingMeasQuality

The IE NR-TimingMeasQuality defines the target device′s best estimate of the quality of measurements.

-- ASN1START

NR-TimingMeasQuality-r16 ::= SEQUENCE {

timingMeasQualityValue-r16			INTEGER (0..31),

timingMeasQualityResolution-r16 	ENUMERATED {mdot1, m1, m10, m30, ...},

...

}

-- ASN1STOP

NR-TimingMeasQuality field descriptions

timingMeasQualityValue This parameter provides the best estimate of the uncertainty of the measurement.

timingMeasQualityResolution This parameter provides the resolution levels used in the Value field.

#### –	NR-TimeStamp

The IE NR-TimeStamp defines the UE measurement associated  time stamp.

-- ASN1START

NR-TimeStamp-r16 ::= SEQUENCE {

trp-ID-r16					TRP-ID-r16			OPTIONAL,-- Cond NotSameAsRefServ0

nr-PhysCellId-r16			NR-PhysCellId-r16 	OPTIONAL,-- Cond NotSameAsRefServ0

nr-SFN-r16			INTEGER (0..1023),

nr-Slot-r16 		CHOICE {

scs15			INTEGER (0..9),

scs30			INTEGER (0..19),

scs60			INTEGER (0..39),

scs120			INTEGER (0..79)

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsRefServ0 | The field is mandatory present if the SFN is not from the reference TRP; otherwise it is not present. |

[…]

#### 6.5.9.2	NR-ECID Location Information Elements

#### –	NR-ECID-SignalMeasurementInformation

The IE NR-ECID-SignalMeasurementInformation is used by the target device to provide NR ECID measurements to the location server.

-- ASN1START

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16	OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber				BIT STRING (SIZE (10)),

nr-PhysCellId-r16				NR-PhysCellId-r16,

nr-CellGlobalId-r16				NCGI-r15					OPTIONAL,	-- Need ON

nrARFCNRef-r16					ARFCN-ValueNR-r15,

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

...

}

MeasQuantityResults-r16 ::= SEQUENCE {

nr-RSRP-r16	INTEGER (0..127)		OPTIONAL,

nr-RSRQ-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

-- ASN1STOP

NR-ECID-SignalMeasurementInformation field descriptions

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

nr-PhysCellId This field specifies the physical cell identity of the measured cell.

nr-CellGlobalId This field specifies the NCGI of the measured cell. The target device shall provide this field if it was able to determine the NCGI of the measured cell at the time of measurement .

nrARFCNRef This field specifies the NRARFCN of the measured NR carrier frequency.

resultsSSB-Cell This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsCSI-RS-Cell This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

ssb-Results This attribute specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement per SSB resource, as defined in TS 38.331 [35].

csi-RS-Results This attribute specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) per CSI-RS resource, as defined in TS 38.331 [35].

primaryCellMeasuredResults This field contains measurements for the primary cell when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements for the primary cell is reported in the measuredResultsList.

[…]

#### 6.5.10.4	NR-DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR-DL TDOA measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP in case RSTD measurements are reported. The first TRP in the list may or may not be the reference TRP indicated in the NR-DL-PRS-AssistanceData. Furthermore, the target device selects a reference resource per TRP, and compiles the measurements per TRP based on the selected reference resource.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16	NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16				NR-DL-TDOA-AdditionalMeasurements-r16,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),	-- FFS on the value range to be decided in RAN4

dl-PRS-RSRP-ResultDiff-r16	INTEGER (FFS)		OPTIONAL, -- FFS on the value range	to be decided in RAN4

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16		OPTIONAL,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs per UE

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in FFS. Mapping of the measured quantity is defined as in FSS.

nr-TimingMeasQuality This field specifies the target device′s best estimate of the quality of the measurement.

#### –	NR-DL-TDOA-LocationInformation

The IE NR-DL-TDOA-LocationInformation is included by the target device when location information derived using NR-DL-TDOA is provided to the location server.

-- ASN1START

NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

systemFrameNumber-r16			NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

[…]

#### 6.5.11.4	NR-DL-AoD Location Information Elements

#### –	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL AoD measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16		NR-DL-AoD-AdditionalMeasurements-r16,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-MeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-DL-AoD-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

#### –	NR-DL-AoD-LocationInformation

The IE NR-DL-AoD-LocationInformation is included by the target device when location information derived using NR-DL-AoD is provided to the location server.

-- ASN1START

NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

sfn-time-r16					NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-AoD-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

[…]

#### 6.5.12.4	NR-Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server. The measurements are provided as a list of TRPs, where the first TRP in the list is used as reference TRP.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16	NR-Multi-RTT-MeasList-r16,

...

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1.. nrMaxTRPs)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-UE-RxTxTimeDiff-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range to be decided in RAN4

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16	OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-TimingMeasQuality-r16			NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-Multi-RTT-AdditionalMeasurements-r16		NR-Multi-RTT-AdditionalMeasurements-r16,

...

}

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-PRS-RSRP-ResultDiff-r16			INTEGER (FFS)			OPTIONAL, -- FFS, value range to be decided in RAN4.

nr-UE-RxTxTimeDiffAdditional-r16				INTEGER (0..ffs)	OPTIONAL,	-- FFS on the value range

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16		OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...

}

nrMaxTRPs		INTEGER ::= 256		-- Max TRPs

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-PRS-RSRP-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 38.331 [35].

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in FFS.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value or the nr-UE-RxTxTimeDiffAdditional value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]

[…]