3GPP TSG-RAN WG2 Meeting #110-e	R2-200xxxx

# Online, June 01 – 12, 2020

Agenda item:	x.y.z

Source: 	Qualcomm Incorporated

Title: 	Email discussion report: [Post109bis-e][948][POS] LPP ASN.1 review (Qualcomm)

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion:

[Post109bis-e][948][POS] LPP ASN.1 review (Qualcomm)

Scope: Gather and discuss issues and develop a running CR for ASN.1 corrections, with R2-2003981 as a baseline.

Intended outcome: Open issues list and CR to next meeting

Deadline:  Long

Section 2 lists the open issues identified during RAN2#109bis-e [1], [2], [3].

Section 3 collects companies’ views on the open issues. If there were possible options for a solution already identified at RAN2#109bis-e, the options are listed.

Section 4 is for collection of any additional ASN.1 issues. Note, R2-2003981 [4] should be used as baseline for any new issue.

## References

[1]	R2-2003982, "Email discussion report: [AT109bis-e][601][POS] LPP ASN.1 issue gathering and easy agreements", Qualcomm.

[2]	R2-2003983, "Email discussion report: [AT109bis-e][602][POS] LPP ASN.1 structural issues", Ericsson.

[3]	R2-2003805, "Report of session on Rel-15 and -16 LTE and NR positioning", Session Chair (MediaTek).

[4]	R2-2003981, "LPP Clean-Up", Qualcomm Incorporated.

[5]	R1-20xxxxx, "RAN1 Chairman’s Notes", RAN1#100bis-e.

# 2. 	Open Issues List

During RAN2 #109bis-e, the following open issues have been identified [1], [2], [3]:

|  | Reference | Issue # (digits before -x refer to LPP section) | Brief Description / Headline |
| --- | --- | --- | --- |
| 1 | Sec. 2.2 in [1] | 6.4.1-2 | The definition of NR-PhysCellId-r16 may fit better in the new collapsed 6.4.3 The new IE RelativeLocation-r16 may fit better in the common section 6.4.1 |
| 2 | Sec. 3.1 in [1] | 6.4.3-1 | Consider renaming the IE NR-TimingMeasQuality. |
| 3 | Sec. 3.2.1 in [1] | 6.4.3-2 | Reference TRP Information. The current LPP is unclear about the definition/signalling of "assistance data reference TRP" and "RSTD reference TRP". Includes also potential issues on nr-DL-PRS-ReferenceInfo and nr-DL-PRS-SFN0-Offset fields, as described. |
| 4 | Sec. 3.2.2a in [1] | 6.4.3-4 | Currently, the field dl-PRS-ResourceRepetitionFactor is mandatory within DL-PRS-ResourceSet. While should be possible that the field is not configured and there is no repetition. The same rationale also goes for dl-PRS-ResourceTimeGap. Also, check if nr-DL-PRS-expectedRSTD-r16 and nr-DL-PRS-expectedRSTD-uncerainty-r16 need to be mandatory. |
| 5 | Sec. 3.2.2b in [1] | 6.4.3-5 | For the indication of SSB as PRS QCL, currently the field PCI is mandatory, however, it is unnecessary when the SSB and PRS locate on the same frequency layer. |
| 6 | Sec. 3.2.5 in [1] | 6.4.3-8 | Need codes are currently missing in IE TRP-ID and the existing condition is confusing/wrong. Issue depends on the conclusion related to TRP-ID. |
| 7 | Sec. 3.2.6 in [1] | 6.4.3-9 | Conditional presence of trp-id field in IE NR-TimeStamp is confusing/wrong. |
| 8 | Sec. 3.3 in [1] Sec. 2.4 in [2] | 6.4.3-10 | The IE NR-PositionCalculationAssistance may not be needed. It may be better moved to the IE definitions of NR-UEB-TRP-LocationData and NR-UEB-TRP-RTD-Info from 7.4.2 to 6.4.3. |
| 9 | Sec. 3.3 in [1] | 6.4.3-11 | The description of ‘nr-DL-PRS-SFN0-Offset’ should be modified for UE-assisted positioning. |
| 10 | Sec. 2.2 in [2] | 6.4.3-12 | Representation of beam directions: (a) 0.1 degrees resolution (current spec.) (b) 1 degree resolution with an optional refinement to 0.1 degrees. |
| 11 | Sec. 4.1.2 in [1] | 6.5.9-2 | The TRP-ID in the IE NR-ECID-SignalMeasurementInformation is currently optional present. However, an identifier of the TRP/cell for which the measurements are applicable is always needed. The systemFrameNumber can usually only be included if the NR-MeasuredResultsElement is provided for a serving cell. |
| 12 | Sec. 5.1 in [1] Sec. 2.5 in [2] | 6.5.10-1 | There is currently no complete description/explanation for the sharing of the assistance data provided in IE NR DL PRS AssistanceData and NR-SelectedDL-PRS-IndexList. DL-PRS AssistanceData placement in the LPP message structure. |
| 13 | Sec. 5.3.1 in [1] | 6.5.10-3 | DL-PRS RSRP measurements can optionally be provided for DL-TDOA positioning. However, there is currently confusion, since RSTD is a measurement for a pair of TRPs, but the RSRP is a single TRP measurement only. |
| 14 | Sec. 5.3.2 in [1] | 6.5.10-4 | The IE NR-TimingMeasQuality is used to provide the quality of the RSTD measurement. However, the quality of the reference TRP TOA used for RSTD cannot be provided. Further, the quality of the additional RSTD measurements per TRP pair (up to 3) can also not be provided. |
| 15 | Sec. 5.3.5 in [1] | 6.5.10-7 | The IE NR-DL-TDOA-MeasElement provides the RSTD measurements for up to 256 TRPs. However, since the RSTD measurement is between a pair of TRPs, only up to 255 report elements for IE NR-DL-TDOA-MeasElement are possible. |
| 16 | Sec. 5.4.1 in [1] | 6.5.10-8 | The IE NR-DL-TDOA-RequestLocationInformation reserves a BIT STRING Size 1..8 for the requested RSRP measurement. However, a single bit would be sufficient. |
| 17 | Sec. 5.5.1 in [1] | 6.5.10-10 | The capability for periodic reporting cannot be indicated separately for the positioning mode. |
| 18 | Sec. 5.5.2 in [1] | 6.5.10-11 | A BIT STRING Size 1..8 is used for indicating support for DL-PRS RSRP measurements for DL-TDOA positioning. (related to Issue 6.5.10-8) |
| 19 | Sec. 6.1 in [1] | 6.5.11-1 | Same as 6.5.10-1, but for DL-AoD |
| 20 | Sec. 6.4.1 in [1] | 6.5.11-6 | Same as 6.5.10-10, but for DL-AoD |
| 21 | Sec. 6.6 in [1] | 6.5.11-8 | nr-DL-PRS-RxBeamIndex-r16: a single bit of information, viz. Boolean may be enough. May be no need to report the ID of the RX beam used. |
| 22 | Sec. 7.1 in [1] | 6.5.12-1 | Same as 6.5.10-1, but for Multi-RTT |
| 23 | Sec. 7.3.1 in [1] | 6.5.12-4 | Same as 6.5.10-8, but for Multi-RTT |
| 24 | Sec. 7.4.1 in [1] | 6.5.12-6 | Same as 6.5.10-11, but for Multi-RTT |

# 3. 	Discussion

The NOTE’s in this section are Rapporteur’s comments/understanding/questions.

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 1 | Sec. 2.2 in [1] | 6.4.1-2 | The definition of NR-PhysCellId-r16 may fit better in the new collapsed 6.4.3 The new IE RelativeLocation-r16 may fit better in the common section 6.4.1 |

Description:

(a) The IE NR-PhysCellId is currently defined in section 6.4.1 (Common Lower-Level IEs), but it is currently used for NR Positioning only. It was proposed to move this IE to section 6.4.3 (Common NR Positioning Information Elements).

NOTE 1: 	A NR Physical Cell ID is already used in Rel-15 (e.g., for GNSS Fine Time Assistance), but not defined as a separate IE. Note also, that section 6.4.1 already defines ARFCN-ValueNR and NCGI (Common Lower-Level IEs).

(b) The IE RelativeLocation is currently defined in section 6.4.3 (Common NR Positioning Information Elements), since only used for NR Positioning. It was proposed to move this IE to section 6.4.1 (Common Lower-Level IEs).

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 2 | Sec. 3.1 in [1] | 6.4.3-1 | Consider renaming the IE NR-TimingMeasQuality. |

Description:

The IE NR-TimingMeasQuality is currently used in

-	NR-AdditionalPathList

-	NR-RTD-Info

-	NR-DL-TDOA-SignalMeasurementInformation

-	NR-Multi-RTT-SignalMeasurementInformation

Given that not all usages may be considered as "measurements", it was proposed to consider changing the name of the IE; for example, NR-TimingQuality.

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 3 | Sec. 3.2.1 in [1] | 6.4.3-2 | Reference TRP Information. The current LPP is unclear about the definition/signalling of "assistance data reference TRP" and "RSTD reference TRP". Includes also potential issues on nr-DL-PRS-ReferenceInfo and nr-DL-PRS-SFN0-Offset fields, as described. |

Description:

(a) Reference TRP Information

Currently, the "Reference Info" is provided by DL-PRS-IdInfo in IE NR-DL-PRS-AssistanceData:

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 		DL-PRS-IdInfo-r16					OPTIONAL,	-- Need ON

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (0..255)) OF NR-SSB-Config-r16,

...

}

The field description for nr-DL-PRS-ReferenceInfo defines: "This field indicates the IDs of the reference TRP.". I.e., the field provides the IDs (i.e., plural) which is needed for the "RSTD Reference" candidates. However, for the "Assistance Data Reference", the DL-PRS ID should be sufficient, since (a) expected RSTD is defined for a pair of TRPs, and (b) RTDs can be provided only on DL-PRS ID level (not on Resource Set ID and Resource ID level).

From the discussion/comments in section 3.2 in [1], there appears to be the following options:

Option 1:	Distinguish between "assistance data reference TRP" and "RSTD reference TRP". The currently used DL-PRS-IdInfo in IE NR-DL-PRS-AssistanceData is the (requested/recommended) "RSTD reference TRP" (and can be moved to NR-DL-TDOA-RequestLocationInformation), and the "assistance data reference TRP" is indicated separately using a DL-PRS-ID/TRP-ID only (i.e., no Resource Set ID, Resource ID).

Option 2:	The assistance data (e.g., nr-DL-PRS-AssistanceDataList, NR-PositionCalculationAssistance) are provided as a list of TRPs (as currently defined) and the first entry of the list defines the (assistance data) Reference TRP.

Option 3:	"Assistance Data Reference TRP" is the same as "RSTD reference TRP" (i.e., DL-PRS-IdInfo), and no change to the specification is needed.

NOTE 3a:	An "Assistance Data Reference TRP" seems always needed (to indicate SFN-offset and expected RSTD), whereas a "RSTD reference TRP" would only be needed for UE-assisted DL-TDOA.

NOTE 3b:	If Option 3 is desired, it seems the DL-PRS-IdInfo-r16 should be mandatory present (given NOTE 3a).

NOTE 3c:	The "assistance data reference TRP" should be a TRP/cell from which the UE can obtain the SFN. In case of broadcast assistance data, this cell/TRP is the cell from which the UE obtains the posSIB (i.e., each cell would broadcast a different order of the list with Option 2).

| Company | Comments |
| --- | --- |

(b) nr-DL-PRS-SFN0-Offset

The definition of the SFN0-offset (nr-DL-PRS-SFN0-Offset-r16) also requires definition of a "assistance data reference TRP". The field is currently misplaced in IE NR-DL-PRS-Config-r16 (which has no notion of a "assistance data reference TRP" whatsoever). It should appear at the same level as the expected RSTD:

-- ASN1START

NR-DL-PRS-Config-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetList-r16	SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF

NR-DL-PRS-ResourceSet-r16,

...

}

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n6, n8, n16, n32, ...},

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...},

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 		DL-PRS-IdInfo-r16					OPTIONAL,	-- Need ON

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (0..255)) OF NR-SSB-Config-r16,

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

trp-ID-r16								TRP-ID-r16,

# nr-DL-PRS-SFN0-Offset-r16				NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-expectedRSTD-r16				INTEGER (-3841..3841),

nr-DL-PRS-expectedRSTD-uncerainty-r16	INTEGER (-246..246),

nr-DL-PRS-Config-r16					NR-DL-PRS-Config-r16,

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9)					OPTIONAL,	-- Need OP

...

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 4 | Sec. 3.2.2a in [1] | 6.4.3-4 | Currently, the field dl-PRS-ResourceRepetitionFactor is mandatory within DL-PRS-ResourceSet. While should be possible that the field is not configured and there is no repetition. The same rationale also goes for dl-PRS-ResourceTimeGap. Also, check if nr-DL-PRS-expectedRSTD-r16 and nr-DL-PRS-expectedRSTD-uncerainty-r16 need to be mandatory. |

Description:

dl-PRS-ResourceRepetitionFactor and dl-PRS-ResourceTimeGap are mandatory present. If there is typically no repetition, the fields could be optional present (with default interpretation when absent):

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL,	-- Need OP

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 5 | Sec. 3.2.2b in [1] | 6.4.3-5 | For the indication of SSB as PRS QCL, currently the field PCI is mandatory, however, it is unnecessary when the SSB and PRS locate on the same frequency layer. |

Description:

pci should be OPTIONAL in IE DL-PRS-QCL-Info, with conditional present tag that if the SSB is on the same frequency layer as the PRS, the field is absent.

DL-PRS-QCL-Info-r16 ::= CHOICE {

ssb-r16						SEQUENCE {

nr-ARFCNRSource-r16				ARFCN-ValueNR-r15

OPTIONAL,	-- Cond NotSameAsPRS-FreqLayer

pci-r16							NR-PhysCellId-r16

OPTIONAL,	-- Cond NotSameAsPRS-FreqLayer

ssb-Index-r16					INTEGER (0..63),

rs-Type-r16						ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

dl-PRS-r16					SEQUENCE {

qcl-dl-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16,

qcl-dl-PRS-ResourceSetId-r16	NR-DL-PRS-ResourceSetId-r16

}

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 6 | Sec. 3.2.5 in [1] | 6.4.3-8 | Need codes are currently missing in IE TRP-ID and the existing condition is confusing/wrong. Issue depends on the conclusion related to TRP-ID. |

Issue depends on the conclusion related to TRP-ID. See "[Post109bis-e][947][POS] TRP-ID structure (Ericsson)".

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 7 | Sec. 3.2.6 in [1] | 6.4.3-9 | Conditional presence of trp-id field in IE NR-TimeStamp is confusing/wrong. |

Description:

Conditional presence of trp-id field in IE NR-TimeStamp is confusing/wrong.

NR-TimeStamp-r16 ::= SEQUENCE {

trp-ID-r16			TRP-ID-r16					OPTIONAL,	-- Cond NotSameAsRefServ0

nr-SFN-r16			INTEGER (0..1023),

nr-Slot-r16 		CHOICE {

scs15-r16			INTEGER (0..9),

scs30-r16			INTEGER (0..19),

scs60-r16			INTEGER (0..39),

scs120-r16			INTEGER (0..79)

},

...

}

From the discussion/comments in section 3.2.6 in [1], there appears to be the following options:

Option 1:	Remove the conditional presence of trp-ID in IE NR-TimeStamp.

Option 2:	Remove the trp-ID in IE NR-TimeStamp.

NOTE 7a:	TRP-ID here means some ID of the TRP for which the SFN is valid (i.e., final name depends on solution of TRP issue)

NOTE 7b:	Option 2 assumes the trp-ID is the same as the (assistance data) reference TRP, and therefore, it is not needed.

NOTE 7c:	If Option 2 is desired, what should happen if the UE can not obtain the SFN from the (assistance data) reference TRP? E.g., reference TRP is not the same as serving cell anymore (e.g., after cell change)? Not report any measurements, or not report any time stamp for the measurements (which may effectively be the same)?

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 8 | Sec. 3.3 in [1] Sec. 2.4 in [2] | 6.4.3-10 | The IE NR-PositionCalculationAssistance may not be needed. It may be better moved to the IE definitions of NR-UEB-TRP-LocationData and NR-UEB-TRP-RTD-Info from 7.4.2 to 6.4.3. |

Description:

Currently, all IEs needed for UE-based only are collected in a single IE:

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-trp-LocationInfo-r16 		NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-dl-prs-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-rtd-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

This IE is then included in the XXX-ProvideAssistanceData messages; e.g.:

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

It was proposed to split the NR-PositionCalculationAssistance into two IEs (analogous to posSIBs):

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL,	-- Need ON

nr-UEB-TRP-LocationData-r16			NR-UEB-TRP-LocationData-r16 		OPTIONAL, 	-- Cond UEB

nr-UEB-TRP-RTD-Info-r16				NR-UEB-TRP-RTD-Info-r16 			OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

The NR-UEB-TRP-LocationData and NR-UEB-TRP-RTD-Info are currently defined outside the LPP-PDU-Definitions (as basic production for posSIBs) and would have to be moved to section 6.4.3 (Common NR Positioning Information Elements).

NOTE 8:	If the above split is desired, should the same apply to both, DL-TDOA and DL-AoD? I.e., should the NR-UEB-TRP-RTD-Info also be present for DL-AoD? With the current structure, the option exists at least.

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 9 | Sec. 3.3 in [1] | 6.4.3-11 | The description of ‘nr-DL-PRS-SFN0-Offset’ should be modified for UE-assisted positioning. |

Description:

The description of nr-DL-PRS-SFN0-Offset should be modified for UE-assisted positioning. This parameter is used for the UE to obtain the timing of TRPs. If the UE doesn’t known the timing of reference TRP, this parameter makes no sense.

Option 1: 	Change "reference TRP" to "serving cell".

Option 2:	Add a description like "The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. a TRP from the serving cell", so that the UE may also obtain the timing of each TRP.

Option 3:	Providing absolute SFN0 timing of reference TRP for the UE.

NOTE 9a:	There may not always be a "serving cell" (e.g., broadcast), and a serving cell may change during a positioning session.

NOTE 9b: The nr-DL-PRS-SFN0-Offset is always needed for PRS processing (i.e., not only for UE-assisted mode).

NOTE 9c: Option 2 was sufficient for LTE, but is it also sufficient for NR? I.e., in LTE, there is no SFN offset.

NOTE 9d:	The issue seems related to 3 (#6.4.3-2; Reference TRP Information) and 7 (#6.4.3-9; trp-id field in IE NR-TimeStamp).

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 10 | Sec. 2.2 in [2] | 6.4.3-12 | Representation of beam directions: (a) 0.1 degrees resolution (current spec.) (b) 1 degree resolution with an optional refinement to 0.1 degrees. |

Description:

Direction/angle information is currently provided as a single field with 0.1 degrees resolution:

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..3599),

dl-PRS-Elevation-r16			INTEGER (0..1800)				OPTIONAL,	-- Need ON

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..3599),

beta-r16						INTEGER (0..3599),

gamma-r16						INTEGER (0..3599),

...

}

It was proposed to use 1-degree resolution together with a 0.1-degree delta-field:

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..359),

dl-PRS-Azimuth-fine-r16			INTEGER (0..9)					OPTIONAL,	-- Need OP

dl-PRS-Elevation-r16			INTEGER (0..180)				OPTIONAL,	-- Need ON

dl-PRS-Elevation-fine-r16		INTEGER (0..9)					OPTIONAL,	-- Need OP

...

}

LCS-GCS-Translation-Parameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..359),

alpha-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

beta-r16						INTEGER (0..359),

beta-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

gamma-r16						INTEGER (0..359),

gamma-fine-r16					INTEGER (0..9) 					OPTIONAL,	-- Cond AzElFine

...

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 11 | Sec. 4.1.2 in [1] | 6.5.9-2 | The TRP-ID in the IE NR-ECID-SignalMeasurementInformation is currently optional present. However, an identifier of the TRP/cell for which the measurements are applicable is always needed. The systemFrameNumber can usually only be included if the NR-MeasuredResultsElement is provided for a serving cell. |

Description:

The system frame number in IE NR-ECID-SignalMeasurementInformation is currently mandatory present. However, since the measurement element is used for all measured cells/TRPs (i.e., also for neighbour TRPs), a SFN of the measured cell may not always be available at the target device (as also clarified by the field description):

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16				OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF NR-MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber-r16			BIT STRING (SIZE (10)),

trp-ID-r16						TRP-ID-r16									OPTIONAL,

resultsSSB-Cell-r16				MeasQuantityResults-r16						OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16						OPTIONAL,

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16					OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16				OPTIONAL,

...

}

NR-ECID-SignalMeasurementInformation field descriptions

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

The above ASN.1 looks like a typo and should be:

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16				OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF NR-MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

systemFrameNumber-r16			BIT STRING (SIZE (10))						OPTIONAL,

trp-ID-r16						TRP-ID-r16									,

resultsSSB-Cell-r16				MeasQuantityResults-r16						OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16						OPTIONAL,

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16					OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16				OPTIONAL,

...

}

NOTE 11:	TRP-ID here means some ID of the TRP for which the measurements are provided (i.e., final name depends on solution of TRP issue).

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 12 | Sec. 5.1 in [1] Sec. 2.5 in [2] | 6.5.10-1 | There is currently no complete description/explanation for the sharing of the assistance data provided in IE NR DL PRS AssistanceData and NR-SelectedDL-PRS-IndexList. DL-PRS AssistanceData placement in the LPP message structure. |

Description:

In case of multiple Provide Assistance Data IEs for multiple NR positioning methods, the IE NR-DL-PRS-AssistanceData for the TRPs would need to be provided only once. There were essentially two remaining options identified in [1],[2]:

Option 1:	Keep the current LPP structure and add proper field description.

Option 2: Move the NR-DL-PRS-AssistanceData to the Message Body IEs.

Option 1: (DL-TDOA as example; the same applies to DL-AoD and Multi-RTT)

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL, -- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL, -- Cond Shared

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, -- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL, -- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present for UE based NR DL-TDOA; otherwise it is not present. |
| Shared | The field is optionally present if not all DL-PRS Resources provided in nr-DL-PRS-AssistanceData are applicable for this NR-DL-TDOA-ProvideAssistanceData message, or if the IE NR-DL-PRS-AssistanceData is provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData. |

NR-DL-TDOA-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. If this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData is provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

nr-PositionCalculationAssistanceData This field provides TRP location and timing information for the TRPs provided in nr-DL-PRS-AssistanceData or nr-SelectedDL-PRS-IndexList to enable UE-based DL-TDOA.

nr-DL-TDOA-Error This field provides DL-TDOA error reasons.

Option 2: (DL-TDOA as example; the same applies to DL-AoD and Multi-RTT)

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

]],

[[

nr-DL-PRS-AssistanceData-r16	NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-Multi-RTT-ProvideAssistanceData-r16

NR-Multi-RTT-ProvideAssistanceData-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-ProvideAssistanceData-r16

NR-DL-AoD-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16

NR-DL-TDOA-ProvideAssistanceData-r16

OPTIONAL	-- Need ON

]]

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 13 | Sec. 5.3.1 in [1] | 6.5.10-3 | DL-PRS RSRP measurements can optionally be provided for DL-TDOA positioning. However, there is currently confusion, since RSTD is a measurement for a pair of TRPs, but the RSRP is a single TRP measurement only. |

Description:

DL-PRS RSRP can provide an auxiliary measurement for DL-TDOA, e.g., to indicate an additional quality for the DL-PRS measurement, etc.. However, with the current measurement results structure for DL-TDOA, the RSRP for the reference TRP cannot be provided.

It was proposed adding the RSRP measurements for the RSTD Reference TRP to the IE NR-DL-TDOA-SignalMeasurementInformation:

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-PRS-RSRP-ResultRef-r16		INTEGER (FFS)									OPTIONAL,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingMeasQuality-r16		NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)									OPTIONAL,

-- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 14 | Sec. 5.3.2 in [1] | 6.5.10-4 | The IE NR-TimingMeasQuality is used to provide the quality of the RSTD measurement. However, the quality of the reference TRP TOA used for RSTD cannot be provided. Further, the quality of the additional RSTD measurements per TRP pair (up to 3) can also not be provided. |

Description:

The RSTD measurement is a TDOA measurement, and the quality of the RSTD can be indicated by the IE NR-TimingMeasQuality. The RSTD quality would only be the main diagonal element of a weighting matrix for TDOA; the off-diagonal elements of the weighting matrix are determined by the quality of the reference TRP TOA measurement used for the TDOA (see also LTE OTDOA in LPP). E.g., the selection of the RSTD reference TRP affects all the RSTD (TDOA) measurements.

The issue was also discussed at RAN1#100bis-e, with the following conclusion [5]:

Conclusion:

- It is RAN1 understanding that the NR-TimingMeasQuality is the quality for time of arrival measurements

- NR-TimingMeasQuality is left up to UE implementation

- Notes:

  - No RAN1 specification changes are required.

  - NR-TimingMeasQuality measurement is also applicable for the reference timing used in RSTD measurements

Therefore, the NR-TimingMeasQuality is not the quality of the RSTD, but the quality of the TOA which is used to calculate the RSTD. I.e., there are two qualities needed for a single RSTD measurement (reference quality and neighbour quality):

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

# nr-TOA-Ref-Quality-r16			NR-TimingMeasQuality-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TOA-Quality-r16				NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)									OPTIONAL,

-- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 						OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			INTEGER (0..ffs),

-- FFS on the value range to be decided in RAN4

nr-TOA-Quality-r16				NR-TimingMeasQuality-r16,

dl-PRS-RSRP-ResultDiff-r16		INTEGER (FFS)										OPTIONAL,

-- FFS on the value range	to be decided in RAN4

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16							OPTIONAL,

...

}

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 15 | Sec. 5.3.5 in [1] | 6.5.10-7 | The IE NR-DL-TDOA-MeasElement provides the RSTD measurements for up to 256 TRPs. However, since the RSTD measurement is between a pair of TRPs, only up to 255 report elements for IE NR-DL-TDOA-MeasElement are possible. |

Description:

The IE NR-DL-TDOA-MeasElement provides the DL-TDOA measurements for one TRP. Assistance data can be provided for up to 256 TRPs. This implies that there can be up to 255 TRPs for RSTD measurements.

NOTE 15:		Note, in [1] some companies think the measurements for the RSTD reference TRP are included in one NR-DL-TDOA-MeasElement. However, it is unclear what an RSTD (TDOA) of a single (reference) TRP is.

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-IdInfo-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-1-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						INTEGER (0..ffs),	-- FFS on the value range

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingMeasQuality-r16		NR-TimingMeasQuality-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)									OPTIONAL,

-- FFS, value range to be decided in RAN4.

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...

}

nrMaxTRPs-1-r16		INTEGER ::= 255

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 16 | Sec. 5.4.1 in [1] | 6.5.10-8 | The IE NR-DL-TDOA-RequestLocationInformation reserves a BIT STRING Size 1..8 for the requested RSRP measurement. However, a single bit would be sufficient. |

Description:

The IE NR-DL-TDOA-RequestLocationInformation reserves a BIT STRING Size 1..8 for the requested RSRP measurement. However, a single bit would be sufficient.

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL, -- Need ON

nr-DL-PRS-RSRP-Requested					ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...

}

NOTE16a:	In [1], some companies think that reserving 8-bits is more future proof. However, how likely is it that 7 additional DL-TDOA measurements will be defined? And even if so, they can be added in the same way as a single bit.

NOTE 16b:	This item seems also contingent on RAN1 capabilities discussions. If the capability for the DL-PRS RSRP measurement will be removed, the nr-RequestedMeasurements or nr-DL-PRS-RSRP-Requested will not be needed.

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 17 | Sec. 5.5.1 in [1] | 6.5.10-10 | The capability for periodic reporting cannot be indicated separately for the positioning mode. |

Description:

The capability for periodic reporting can be different for UE-based and UE-assisted mode; e.g., may be supported by a UE for UE-based but not for UE-assisted or vice versa. Currently, there is no differentiation in the DL-TDOA capabilities:

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16					PositioningModes,

nr-DL-TDOA-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16					OPTIONAL,

nr-DL-TDOA-MeasSupported-r16		BIT STRING { prsrsrpSup (0)} (SIZE(1..8)),

additionalPathsReport-r16			ENUMERATED { supported }						OPTIONAL,

periodicalReporting-r16				ENUMERATED { supported }						OPTIONAL,

...

}

It was proposed replacing the "ENUMERATED { supported }" for the field periodicalReporting in IE NR-DL-TDOA-ProvideCapabilities with field "PositioningModes".

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16					PositioningModes,

nr-DL-TDOA-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16					OPTIONAL,

nr-DL-TDOA-MeasSupported-r16		BIT STRING { prsrsrpSup (0)} (SIZE(1..8)),

additionalPathsReport-r16			ENUMERATED { supported }						OPTIONAL,

periodicalReporting-r16				PositioningModes								OPTIONAL,

...

}

NOTE 17:		This is the same as for all other LPP methods which support multiple positioning modes.

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 18 | Sec. 5.5.2 in [1] | 6.5.10-11 | A BIT STRING Size 1..8 is used for indicating support for DL-PRS RSRP measurements for DL-TDOA positioning. (related to Issue 6.5.10-8) |

Description:

For the indication of DL-PRS RSRP support, a single-bit ENUMERATED { supported } would be sufficient.

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16					PositioningModes,

nr-DL-TDOA-MeasCapability-r16 		NR-DL-PRS-MeasCapability-r16					OPTIONAL,

nr-DL-PRS-RSRP-MeasSupported-r16	ENUMERATED { supported }						OPTIONAL,

additionalPathsReport-r16			ENUMERATED { supported }						OPTIONAL,

periodicalReporting-r16				ENUMERATED { supported }						OPTIONAL,

...

}

NOTE 18:			NOTEs 16a/b apply here as well

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 19 | Sec. 6.1 in [1] | 6.5.11-1 | Same as 6.5.10-1, but for DL-AoD |

It is assumed the same solution for 12 (#6.5.10-1) will apply for DL-AoD as well.

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 20 | Sec. 6.4.1 in [1] | 6.5.11-6 | Same as 6.5.10-10, but for DL-AoD |

It is assumed the same solution for 17 (#6.5.10-10) will apply for DL-AoD as well.

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 21 | Sec. 6.6 in [1] | 6.5.11-8 | nr-DL-PRS-RxBeamIndex-r16: a single bit of information, viz. Boolean may be enough. May be no need to report the ID of the RX beam used. |

Description:

The field nr-DL-PRS-RxBeamIndex in NR-DL-AoD-MeasElement is currently used to indicate which (of the up to 8) DL-PRS RSRP measurements have been made with the same RX beam by the UE. I.e., the RSRP measurements in NR-DL-AoD-MeasList which have been made with the same RX beam will get the same value of nr-DL-PRS-RxBeamIndex:

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)									OPTIONAL,

-- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8),

nr-DL-Aod-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16			OPTIONAL,

...

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-PRS-RSRP-ResultDiff-r16		INTEGER (FFS)									OPTIONAL,

-- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8),

...

}

-- ASN1STOP

Some other understanding of the RAN1 agreement is just to indicate whether the same RX beam has been used for all measurements for AoD or not. So, a single bit of information, viz. Boolean is enough. No need to report the ID of the RX beam used. [1]

RAN 1 Agreement:

- When the UE reports DL-PRS RSRP measurements on DL-PRS resources from one DL-PRS Resource Set, the UE may indicate in the measurement report for each TRP which DL-PRS RSRP measurements, if any, have been measured using the same Rx beam.

NOTE 21:	For the Rapporteur, it is unclear how the measurement report should be structured with a single-bit  indicator for one resource set, so an ASN.1 example would be needed. It seems the NR-DL-PRS-ResourceSetId in NR-DL-AoD-AdditionalMeasurementElement is not needed, and another SEQUENCE for 1:2 NR-DL-PRS-ResourceSetId’s may be required.

| Company | Comments |
| --- | --- |

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 22 | Sec. 7.1 in [1] | 6.5.12-1 | Same as 6.5.10-1, but for Multi-RTT |

It is assumed the same solution for 12 (#6.5.10-1) will apply for Multi-RTT as well.

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 23 | Sec. 7.3.1 in [1] | 6.5.12-4 | Same as 6.5.10-8, but for Multi-RTT |

It is assumed the same solution for 16 (#6.5.10-8) will apply for Multi-RTT as well.

|  | Reference | Issue # | Brief Description / Headline |
| --- | --- | --- | --- |
| 24 | Sec. 7.4.1 in [1] | 6.5.12-6 | Same as 6.5.10-11, but for Multi-RTT |

It is assumed the same solution for 18 (#6.5.10-11) will apply for Multi-RTT as well.

# 4. 	Additional Issues

Any additional ASN.1 issues please (not new features).

| Company | Description/Problem | Proposed Solution |
| --- | --- | --- |