3GPP TSG-RAN2 Meeting #110-e	R2-2006333

# eMeeting, 1st – 12th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1632 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of secondary DRX group CR 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-19 |
| Category: | C |  |  |  |  |  | Release: |  |  | REL-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When both FR1 and FR2 cells are configured via Carrier Aggregation the UE power consumption is high, because the same cDRX parameters are used for both FR1 and FR2 cells. The power consumption is reduced when a separate drx-InactivityTimer and drx-onDurationTimer can be configured for the FR2 cells enabling FR2 to go to sleep more quickly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A secondary DRX group is introduced to enable a different configuration of the drx-InactivityTimer and drx-onDurationTimer for the second DRX group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High UE power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.306 CR 0321 TS/TR 38.321 CR 0746 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 6.3.2	Radio resource control information elements

<TEXT OMITTED>

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                    OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity              OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                 OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                             OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                        OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                   OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                    OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                              OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-RLC-ChannelConfig-r16    OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxLC-ID-Iab-r16)) OF BH-LogicalChannelIdentity-r16 OPTIONAL, -- Need N

dormancySCellGroups                        DormancySCellGroups                                                 OPTIONAL,   -- Need N

simultaneousTCI-UpdateList-r16             SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousTCI-UpdateListSecond-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList-r16        SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedListSecond-r16  SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex    OPTIONAL    -- Need R

]]

}

DormancySCellGroups::=               SEQUENCE {

withinActiveTimeToAddModList         SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Need N

withinActiveTimeToReleaseList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL,   -- Need N

outsideActiveTimeToAddModList        SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroup-r16    OPTIONAL,   -- Cond DormancyWUS

outsideActiveTimeToReleaseList       SEQUENCE (SIZE (1..maxNrofDormancyGroups)) OF DormancyGroupID-r16  OPTIONAL    -- Need N

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                         OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]]

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Need SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]]

}

DormancyGroup-r16 ::=               SEQUENCE {

dormancyGroupID-r16                 DormancyGroupID-r16,

dormancySCellList-r16               SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex

}

DormancyGroupID-r16 ::=             INTEGER (0..4)

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

CellGroupConfig field descriptions

bap-Address BAP address of node that is hosting this cell group.

bh-RLC-ChannelToAddModList Configuration of the MAC Logical Channel, the corresponding backhaul RLC enitities to be added and modified.

bh-RLC-ChannelToReleaseList List of MAC Logical Channel, the corresponding backhaul RLC enitities to be released.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the legacy DRX group shall belong to another Frequency Range.

simultaneousTCI-UpdateList, simultaneousTCI-UpdateListSecond List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList and simultaneousTCI-UpdateListSecond shall not contain same serving cells.

simultaneousSpatial-UpdatedList, simultaneousSpatial-UpdatedListSecond List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList and simultaneousSpatial-UpdatedList shall not contain same serving cells.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

DormancyGroup field descriptions

dormancySCellList List of SCells within the same SCell dormancy group.

dormancyGroupID The field indicates an SCell group corresponding to the explicit information field in DCI, i.e., bitmap with 1 bit per DormancyGroup for indicating dormancy/non-dormancy of SCells, as specified in TS 38.213.

DormancySCellGroups field descriptions

outsideActiveTimeToAddModList List of Dormancy outside active time SCell groups to be added or modified. The use of the Dormancy outside active time SCell groups is specified in TS 38.213 [13].

withinActiveTimeToAddModList List of Dormancy within active time SCell groups SCell groups to be added or modified. The use of the Dormancy within active time SCell groups is specified in TS 38.213 [13].

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SCellConfig field descriptions

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DRX-Config2 | The field is optionally present, Need N, if drx-ConfigSecondaryGroup is configured. It is absent otherwise. |
| DormancyWUS | The field is optionally present, Need N, if WUS is configured; otherwise it is absent. |
| ReconfWithSync | The field is mandatory present in case of SpCell change, PSCell addition, SCG resume with NR-DC or (NG)EN-DC, update of required SI for PSCell, and AS security key change; otherwise it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optional present in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

<TEXT OMITTED>

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CrossCarrierSchedulingConfig-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...,

[[

carrierIndicatorSize                SEQUENCE {

carrierIndicatorSizeForDCI-Format1-2-r16        INTEGER (0..3),

carrierIndicatorSizeForDCI-Format0-2-r16        INTEGER (0..3)

}                                                   OPTIONAL  -- Cond CIF-PRESENCE

]]

}

-- TAG-CrossCarrierSchedulingConfig-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

carrierIndicatorSizeForDCI-Format0-2, carrierIndicatorSizeForDCI-Format1-2 Configures the number of bits for the field of carrier indicator in PDCCH DCI format 0_2/1_2. The field carrierIndicatorSizeForDCI-Format0-2 refers to DCI format 0_2 and the field carrierIndicatorSizeForDCI-Format1-2 refers to DCI format 1_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13].

other Parameters for cross-carrier scheduling, i.e., a serving cell is scheduled by a PDCCH on another (scheduling) cell. The network configures this field only for SCells. When SCS of scheduling PDCCH is different from SCS of scheduled PDSCH, the time gap delta-values between the end of the PDCCH and start of the PDSCH is required to be not smaller than the minimal values specified in TS 38.214 [19].

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell. If drx-ConfigSecondaryGroup is configured in the MAC-CellGroupConfig associated with this serving cell, the scheduling cell and the scheduled cell belong to the same Frequency Range.

| Conditional Presence | Explanation |
| --- | --- |
| CIF-PRESENCE | The field is mandatory present if the cif-Presence is set to true. The field is absent otherwise. |

<TEXT OMITTED>

#### –	DRX-Config

The IE DRX-Config is used to configure DRX related parameters.

DRX-Config information element

-- ASN1START

-- TAG-DRX-CONFIG-START

DRX-Config ::=                      SEQUENCE {

drx-onDurationTimer                 CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                 ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),

drx-HARQ-RTT-TimerUL                INTEGER (0..56),

drx-RetransmissionTimerDL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-RetransmissionTimerUL           ENUMERATED {

sl0, sl1, sl2, sl4, sl6, sl8, sl16, sl24, sl33, sl40, sl64, sl80, sl96, sl112, sl128,

sl160, sl320, spare15, spare14, spare13, spare12, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-LongCycleStartOffset            CHOICE {

ms10                                INTEGER(0..9),

ms20                                INTEGER(0..19),

ms32                                INTEGER(0..31),

ms40                                INTEGER(0..39),

ms60                                INTEGER(0..59),

ms64                                INTEGER(0..63),

ms70                                INTEGER(0..69),

ms80                                INTEGER(0..79),

ms128                               INTEGER(0..127),

ms160                               INTEGER(0..159),

ms256                               INTEGER(0..255),

ms320                               INTEGER(0..319),

ms512                               INTEGER(0..511),

ms640                               INTEGER(0..639),

ms1024                              INTEGER(0..1023),

ms1280                              INTEGER(0..1279),

ms2048                              INTEGER(0..2047),

ms2560                              INTEGER(0..2559),

ms5120                              INTEGER(0..5119),

ms10240                             INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                                           OPTIONAL,   -- Need R

drx-SlotOffset                      INTEGER (0..31)

}

-- TAG-DRX-CONFIG-STOP

-- ASN1STOP

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle. A value of 1 corresponds to drx-ShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

#### –	DRX-ConfigSecondaryGroup

The IE DRX-ConfigSecondaryGroup is used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3].

DRX-ConfigSecondaryGroup information element

-- ASN1START

-- TAG-DRX-CONFIGSECONDARYGROUP-START

DRX-ConfigSecondaryGroup ::=       SEQUENCE {

drx-onDurationTimer                CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

drx-InactivityTimer                ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-DRX-CONFIGSECONDARYGROUP-STOP

-- ASN1STOP

DRX-ConfigSecondaryGroup field descriptions

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-InactivityTimer value for the second DRX group that is smaller than the drx-InactivityTimer configured for the default DRX group in IE DRX-Config.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on, as specified in TS 38.321 [3]. The network configures a drx-onDurationTimer value for the second DRX group that is smaller than the drx-onDurationTimer configured for the default DRX group in IE DRX-Config.

<TEXT OMITTED>

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                                BOOLEAN                                                     OPTIONAL,   -- Need M

dataInactivityTimer                     SetupRelease { DataInactivityTimer }                        OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,    -- Need M

lbt-FailureRecoveryConfig-r16       LBT-FailureRecoveryConfig-r16                                   OPTIONAL,    -- Need M

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,    -- Need M

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,    -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,    -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup }                       OPTIONAL     -- Need M

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

usePreBSR If set to true, the MAC entity of the IAB-MT will activate the pre-BSR.

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

lch-BasedPrioritization If this field is present, the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see see TS 38.321 [3]. Editor's Note: It is FFS whether SR/data prioritization can be a separate configurable parameter from data/data prioritization.

schedulingRequestID-BFR-SCell If present, it indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

skipUplinkTxDynamic If set to true, the UE skips UL transmissions as described in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need M, for the MAC-CellGroupConfig of the MCG. It is absent otherwise. |

<TEXT OMITTED>

<End of modified section>

<Start of modified section>

### 6.3.3	UE capability information elements

<TEXT OMITTED>

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                 ENUMERATED {supported}      OPTIONAL,

dummy                           ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction          ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate              ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery         ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16 ENUMERATED {supported}     OPTIONAL,

secondaryDRX-Group              ENUMERATED {supported}      OPTIONAL

]]

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic             ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer     ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                   ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                  ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations       ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants    ENUMERATED {supported}         OPTIONAL,

...

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

<TEXT OMITTED>

<End of modified section>