‎3GPP TSG-RAN WG2 Meeting #111 electronic	R2-20xxxxx

Online, August 17th - 28th, 2020‎

# Agenda item:	6.4.3

# Source:	CATT

# Title:	‎Discussion paper on [Post111-e][703][V2X] 37.324 corrections

# Document for:	Discussion and Agreement

1	Introduction

This is to report the result of the following email discussion after RAN2#111-e Meeting [1].

[Post111-e][703][V2X] 37.324 corrections (CATT)

Discuss the need of changes proposed from R2-2006616 and R2-2007734. If the need is agreed, prepare agreeable 37.324 CR in R2-2008359. Note cover-page should be written well (with reasonable impact analysis). CR will be approved by email.

Deadline Short

The remainder of this document is organized to discuss the 1st correction from R2-2007734 ‎as the following. The discussions are in Section 2 and the conclusions are summarized in Section 3.

2	Discussion

2.1	SDAP Rx behaviour

The related proposal is available in the below table:

| Company | Tdoc | Proposals |
| --- | --- | --- |
| ASUSTeK | R2-2007734‎ | - Change for PC5 QoS flow remapping in subclause 5.2.4: Option 1: A NOTE is added in subclause 5.2.4 to indicate the agreement, i.e. “NOTE: SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery.” Option 2: After a PC5 QoS flow is remapped from the previously-mapped SL-DRB to the current SL-DRB, the UE shall not deliver the retrieved SDAP SDU to the upper layer until an End-Marker control PDU for the PC5 QoS flow is received. |

For the 1st change made by R2-2007734, it’s proposed to specify the SDAP Rx behaviour upon remapping for ‎insequence delivery‎ as following options:

Option 1: A NOTE is added in subclause 5.2.4 to indicate the agreement, i.e. “NOTE: SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery.”

Option 2: After a PC5 QoS flow is remapped from the previously-mapped SL-DRB to the current SL-DRB, the UE shall not deliver the retrieved SDAP SDU to the upper layer until an End-Marker control PDU for the PC5 QoS flow is received.

This issue has been discussed during RAN2#108 meeting. The related agreement was made as: SDAP Rx behaviour upon remapping is left to UE implementation for insequence delivery. And it will not be captured in the specification. Rapporteur ‎thinks it would be better to respect the previous agreement and not capture anything for the SDAP Rx behaviour upon remapping for ‎insequence delivery‎ in the spec. Companies views are appreciated to be collected in the following table.

Question: Does company agree to specify the SDAP Rx behaviour upon remapping for ‎insequence delivery in 37.324, as proposed two options as above from R2-2007734?

- Agree as above Option1;

- Agree as above Option2;

- Disagree, i.e., respect the previous agreement and keep the current spec as it is.

| Company Name | Views: Agree/Disagree | Comments |
| --- | --- | --- |

Proposed conclusion:

TBD

3	Conclusion

TBD

4	References

[1] RAN2-111-e_V2X_Kyeongin_2020-08-28_EOM