3GPP TSG-RAN WG2 Meeting #112e	R2-20xxxxx

Online, 2-13 November 2020

Agenda item:	X.X.X

Source:	Nokia, Nokia Shanghai Bell

Title:	[DRAFT] E-mail discussion: [Post111-e][924][R17 URLLC/IIoT] Propagation delay for TSN (Nokia)

WID/SID:	NR_IIOT_URLLC_enh

Document for:	Discussion and Decision

# 1	Introduction

This document aims to collect views from companies for the following email discussion agreed during RAN2 #111e:

- [Post111-e][924][R17 URLLC/IIoT] Propagation delay for TSN (Nokia)

1st phase: Agree on baseline scenarios and then for each scenario the high-level breakdown on the delay components and agree on assumptions.  Identify the aspects that RAN1 should investigate

2nd phase: Identify the set of possible options to continue investigating and how they address each component

Intended outcome: report to next meeting

Deadline: Long

For the first phase, we aim to agree on baseline scenarios that will be examined in this WI. Furthermore, for each scenario, we should reach a consensus on the assumption of the Uu interface time synchronization error budget from a high-level E2E time synchronization error budget breakdown. This can form the basis for identifying any aspect that would require further investigation by RAN1. These would serve as a foundation for work relating to potential propagation delay compensation enhancement.  Remarkably, the scope of the first phase coincides with a latest LS from RAN1 (R1-2007446), which indicates that RAN1 already identified two use cases from TS 22.104:

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Besides, the LS R1-2007446 also request RAN2 to provide feedback on Uu interface error budget for each of the two use cases, this fits the objectives of this email discussion. Hence, the first phase of this email discussion will use RAN1 agreements as the basis.

For the second phase, we should identify the options of propagation delay compensation that we will further investigate.

# 2	Phase-1 Discussion: Use Cases, Scenarios, and Assumptions on Synchronization Error Budget

## 2.1 Use Cases and Scenarios

The scenarios for accurate time synchronizations are provided in TS 22.104 Table 5.6.2-1, and are copied below. As aforementioned, RAN1 has already agreed two of the use cases for further study, which are highlighted in the table below:

Table 1 - Clock synchronization service performance requirements for the 5G System

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 1 | Up to 300 UEs | ≤900 ns | ≤ 100 m x 100 m | - Motion control - Control-to-control communication for industrial controller |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 3 | Up to 10 UEs | < 10 µs | ≤ 2500 m2 | - High data rate video streaming |
| 3a | Up to 100 UEs | <1 µs | ≤10 km2 | - AVProd synchronisation and packet timing |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |
| 5 | Up to 10 UEs | < 50 µs | 400 km | - Telesurgery and telediagnosis |
| NOTE: The clock synchronicity requirement refers to the clock synchronicity budget for the 5G system, as described in Clause 5.6.1. |  |  |  |  |

Question 1: In addition to use cases identified by RAN1 (Highlighted in Table 1), do you think RAN2 should further study any other use cases listed in TS 22.104? If so, please provide the additional use cases you consider should be studied and arguments why.

| Company | Comments |
| --- | --- |
| Nokia | No We think the use cases identified by RAN1 are sufficient for Rel-17. |
| Ericsson | No |

Aligned with the agreed assumptions on one Uu (smart grid) and two Uu interfaces (control-to-control) as listed in LS R1-2007446, we can consider two placements of the 5G GM for the control-to-control use case. These are illustrated in Figure 1. In one case, the TSC GM is connected to a device behind the 5GS CN and the E2E accuracy budget would apply from the time-stamping entity at the NW-TT to the time-stamping entity at the DS-TT. The other placement is that the TSC GM is connected to a device behind the UE, where the 5GS E2E budget applies from DS-TT timestamping entity at the source UE to the DS-TT timestamping entity at the target UE. The smart grid use-cases is different as the TSC GM is the 5G GM (or similar TD), so in this case the 5GS E2E accuracy is from the 5G GM to the DS-TT at a UE, as illustrated in Figure 2. These 5G GM placements for different use cases could be considered as the baseline scenarios.

Figure 1. Illustration of the control-to-control use case with two possible scenarios (Scenario 1 and 2) to consider.

Figure 2. Illustration of the smart grid use case and a corresponding scenario (Scenario 3).

Question 2: Based on the two use cases identified by RAN1, which of the following scenarios should be considered as the baseline?

- Scenario 1: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the CN. The 5GS introduced error is caused by the relative time-stamping inaccuracy at the NW-TT and the DS-TTs.

- Scenario 2: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs.

- Scenario 3: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT.

- Scenario 4: Other

| Company | Scenario |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |  |
| Nokia | V | V | V |  | We think Scenario 1, 2 and 3 should be considered as baseline. |
| Ericsson |  | Y | Y |  | Scenario 1 and scenario 2 belong to the same use case, but scenario 1 has a looser Uu requirement than scenario 2. Thus, it is not necessary to separately consider scenario 1. Note that RAN1 has agreed (see LS R1-2007446): - Two Uu interfaces are assumed for control-to-control. |

## 2.2 High-level 5GS E2E Synchronization Budget Breakdown

In the LS R1-2007446. RAN1 has requested RAN2 to provide some information on the error budget in the Uu interface. The budget for the Uu interface depends on how much error could be consumed in other segments of the E2E path, as also mentioned in e.g. [7], [8], [16], [5], [16] and [3]. In particular, on top of the network-side error, it has also been mentioned that the UE-side would introduce an error and hence needs to be taken into account as a part of the E2E path error budget. Taking [3] as a starting point, the E2E path could be broken into three components, as illustrated in Figure 1, which could include:

- RAN / Uu interface – Account for the time synchronization errors introduced by the Uu interface i.e. between the UE and the gNB. This includes the aspects of antenna alignment errors, ReferenceTimeInfo delivery, SFN estimation including the impact on propagation delay (PD) compensation. Here we also  include errors introduced by the gNB architecture splits (e.g. use of gNB-CU and gNB-DU).

- Network – Accounts for the time synchronization errors caused between the GM and the gNB. When the 5G GM source is shared between the UPF and the gNB, the synchronization error involved in this, should also be accounted for here. In case of split architecture, the gNB is a gNB-CU.

- Device – Accounts for the time synchronization errors introduced by the device implementation for maintaining the 5G clock at the DS-TT and potentially also the device output interface to the TSC devices connected to the device.

Figure 3. Possible breakdown of the 5GS E2E path

Question 3: Do you agree that the 5GS E2E synchronization budget could be split into three parts namely Device, Uu interface and Network? If the answer is No, please provide proposals for alternative approach.

| Company | Comments |
| --- | --- |
| Nokia | Yes We think this is a sensible way to evaluate the budget in different segments of the E2E path, and hence determine the propagation delay compensation scheme to be introduced. |
| Ericsson | Partially On a high-level, Ericsson is fine to have such a split, but some further clarifications are needed: - Network component includes the delivery of the 5G reference time from the 5G GM to one gNB radio transmission unit. RAN2 should simplify the analysis with the assumption that one gNB radio transmission unit covers one cell, the same as in RAN1. In our view, RAN2 should not mandate one particular network architecture as it is the choice of the network vendor and the deployment. In other words, RAN2 has to consider a network architecture without CU/DU split. - If CU/DU split deployment is preferred by some companies, its budget should be considered here rather than in RAN/Uu interface. As a matter of fact, there will be inaccuracies for reference time delivery between CU and DU and one may approximate the accuracy deterioration as similar to one gPTP hop. - RAN/Uu interface component includes the delivery of the 5G reference time on the Uu interface from one gNB radio transmission unit to one UE. The transmission timing error at gNB in included, as it has been considered in the RAN1 analysis. Since in the worst case more than one gNB might be involved in time sync, it is reasonable to assume that 5G GM may be placed at the UPF. Additionally, RAN2 needs to further make clear that - If synchronization GM is at the network side (e.g., smart grid, scenario 3), the E2E path includes one device component, one RAN/Uu interface component and one network component. - If synchronization GM is at the device side (e.g., control-to-control with TSN uplink time synchronization, scenario 2), the E2E path includes two device components, two RAN/Uu interface components and two network components. The 5GM GM is assumed to be placed at the UPF. One goal of this email discussion should be to provide feedback on the synchronicity budget for Uu interface, as requested in the RAN1 LS. As a summary of the below answers from Ericsson (100 ns for device component, 160 ns for the network component, 5 ns for the granularity of the reference time), the Uu interface synchronicity budget is: - If synchronization GM is at the network side (e.g., smart grid, scenario 3), then it is 1000 ns – 100 ns – 160 ns – 5 ns = 735 ns - If synchronization GM is at the device side (e.g., control-to-control with TSN uplink time synchronization, scenario 2), then it is (900 ns – 100 ns * 2– 160 ns * 2 – 5 ns * 2) / 2 = 185 ns Note that implementation-related inaccuracies (see answers to question 13) are not considered yet, such as the channel variations between when the reference time delivery is provided and when the propagation delay is compensated, etc. |

All subsequent questions of Phase-1 are based on the above assumption where the 5GS E2E synchronization budget is split into Device, Uu interface and Network parts.

## 2.3 Assumption on Error Budget for Network Part

Several papers have proposed to capture a budget for a network related part of the 5GS E2E budget. For example, [8] and [16] have proposed that at least 100ns inaccuracy should be assumed for the network part, whereas some other papers (e.g. [7] and [5]) have mentioned the importance of network part budget. In [3], for example, two separate network part inaccuracies are proposed, one for each use case.  There could be at least two options for the network part budget, as described below.

In the first option, particularly for the control-to-control use case, a single 5G GM clock source (e.g. from a GNSS receiver or a TSC GM) is distributed to the gNB and UPF (NW-TT) with a (g)PTP framework. It is assumed that the 5G GM clock source, UPF and gNB are located within the same facility and potentially within the same rack. The connection between UPF (NW-TT) and gNB is assumed to span over maximum four (g)PTP capable hops relative to the 5G GM. According to The RAN3 LS in R3-187252 this can introduce a maximum error of TE<|4 ∙40ns|, corresponding to an error within ±80ns.

In the second option, specifically for the smart grid use case, multiple 5G GM clock instances (of the same time-domain, e.g. from multiple GNSS receivers) are distributed in the service area (e.g. one at each gNB and one at the UPF). With the multiple 5G GM clock sources (of the same reference), provided throughout the scenario, the NW accuracy does not depend on the path between the 5GS components, but on the synchronization error between two 5G GM clock instances (e.g. GNSS receivers). Considering the 5G GM instance is provided by GNSS receivers, according to R3-187252 the maximum error between the GNSS receivers is 200ns, which translates to a time synchronization error range of maximum ±100ns.

Question 4: Do you agree to use the above two options as the network part budget, i.e. ±100ns for the smart grid and ±80ns for the control-to-control use case related scenarios respectively? If not, please provide your views on the network part budget for the considered use cases and scenarios. Please also indicate if any additional error components to be considered.

| Company | Comments |
| --- | --- |
| Nokia | Yes We are fine with having two network budgets for the two use cases. For simplicity we can also assume ±100ns for both use cases. Per scenario this means that: Scenario 1 is the control-to-control use case, where the time synchronization error is between DS-TT and the NW-TT. Here the network budget of e.g. ±80ns applied. Scenario 2 is the control-to-control use case, where the time synchronization error is between a DS-TT and another DS-TT. We assume only a single gNB is involved, which means that the network budget could be kept at ±80ns. Scenario 3 is the smart grid use case where the time synchronization error is between a gNB and the DS-TT. Here the network budget of ±100ns applied. |
| Ericsson | No It is okay to use RAN3 LS R3-187252 as reference and to have separate network budgets for the two use cases. On the other hand, Ericsson has different understandings on the exact budget for the control-to-control use case. Smart grid: - It is written in R3-187252 that “¦TE¦ = 100 ns absolute, 200ns relative between nodes.”. This means a budget of ±100ns. Control-to-Control: - It is written in R3-187252 that “¦TE¦ ~N*40ns, where N is number of PTP hops.” Note that the absolute symbol is around TE and one PTP hop introduces ±40ns inaccuracy. Ericsson is okay to assume that a maximum four (g)PTP capable hops or equivalent is needed to deliver the 5G GM to the gNB. For the delivery from the 5G GM to one gNB, the maximum inaccuracy is then ¦TE¦ ~N*40ns where N = 4 and so the budget is ±160ns. However, the service area is indoor with size of 1000 meters by 100 meters. Multiple gNBs are needed to cover this service area and so the inaccuracy between two gNBs should be considered, and so the total budget on the network component is ±320ns. In summary, the synchronization budget of the network interface component is ±320ns for control-to-control use case and ±160ns for smart grid use case. |

As per Section 2.1, scenario 2 states time synchronization error between two DS-TT in control-to-control use case, where UEs can belong to same or different cell, and might have a different realization of the 5G clock, e.g. because the number of hops to synchronize the gNB with the 5G GM can be different.

Question 5: Can it be assumed that the involved UEs in scenario 2, will be connected to different gNBs and if so, what can we expect of relative 5G GM synchronization error between two gNBs?

| Company | Comments |
| --- | --- |
| Nokia | No We consider it likely that a single gNB with multiple gNB-DU is deployed if synchronization accuracy is the priority. As the termination point of the network budget would then be the gNB-CU, the network part is irrelevant for this scenario. However, the relative inaccuracy between the involved gNB-DU should be accounted for in the Uu interface assumptions (see Question 13). |
| Ericsson | Yes As answered in Question 3, RAN2 should not mandate a network deployment with CU/DU split. The service area is 1000 meters by 100 meters and hence very unlikely covered by one gNB. Even with CU/DU split architecture, it is much easier to analyze this in the network interface component, as all these falls into the expertise of RAN3. This approach also clarifies that the RAN/Uu interface component is related with one gNB to one UE which falls exactly in the scope of RAN1. The relative 5G GM synchronization error between two gNBs is the sum of the synchronization error between each gNB and the common reference point (i.e., 5G GM clock), and thus in this case doubled (i.e., ±320ns). |

## 2.4 Assumption on Error Budget for Device Part

The device part is intended to capture the device introduced time synchronization errors. Such errors could be the DS-TT clock instance maintenance or at the DS-TT output interfaces.

Question 6: What error component do you see should be accounted for in the device part and what assumptions can be made on the required device budget?

| Company | Comments |
| --- | --- |
| Nokia | Input from device vendors would be appreciated. |
| Ericsson | Inputs/confirmations from the UE/device vendors are appreciated. A value of 0 ns, of course, would be strange. See Ericsson’s estimate in the answer to the Question 7 below. |

Question 7: What can we assume of device part budget for each scenario described in Question 2?

| Company | Comments |
| --- | --- |
| Nokia | Scenario 1 and 3, only one device is involved in the E2E path. One option is to consider the device as a boundary clock with e.g. an uncertainty of ±40ns. Scenario 2 two devices are involved in the E2E path and the device budget should be accounted twice. That is assuming that both the source and target device introduce a similar time synchronization magnitude. One option is to consider the device as a boundary clock with e.g. an uncertainty of ±40ns, so a total device budget of ±80ns. |
| Ericsson | It is not clear the difference between this question and the above Question 6. Ericsson tends to agree with Nokia that a value of ±40 ns would be a reasonable starting point, but to leave place for UE/device implementation, a value of ±100ns would be acceptable, which should be more than enough. In summary, the synchronization budget of the device interface component is ±200ns for control-to-control use case and ±100ns for smart gird use case. |

## 2.5 Assumption on Error Budget for Uu Interface

According to TS 22.104 Table 5.6.2-1, the service area defined for control to control and smart grid use case is ≤ 1000 m x 100 m and < 20 km2 respectively. However, the assumption on the deployment of BS in the service area will impact the maximum propagation delay to be expected and hence agreeing on this will be helpful in the continued discussions. Companies are requested to provide their views on the number of BS, the maximum cell size and eventually the maximum expected propagation delay to be expected in each use case.

Question 8: For each use case, how many BS can be expected to be needed, what is the maximum cell size to be expected and eventually what is the maximum propagation delay to be expected?

| Company | Comments |
| --- | --- |
| Nokia | For the control-to-control, it is reasonable to use the deployment layout described in TS 38.901 Table 7.8-7 for indoor factory (copied in for convenience). In fact, two big halls each with a hall size of L=300m x W=150m could be used as a starting point. In this deployment the distance between BS are D=50m would give an expected maximum direct path between a UE and the nearest BS of 35m and hence the maximum expected propagation delay would be ~120ns. The corresponding calibration exercises in R1-1909704 reveals that even with this number of BS, the experienced coupling gain can in some cases give challenging signal conditions, so we would consider this number of BS to be a minimum. For the smart grid scenarios, we propose that any cell sizes supported by NR should also be supported for the smart grid use case. Hence we consider it reasonable to assume a standard hexagonal cellular grid with 1732m ISD. This would give an expected cell radius of about 1200m and hence a corresponding direct propagation path delay of about 4000ns. |
| Ericsson | These are indeed RAN1 questions and should be studied in RAN1. For example, in the TR 38.825, RAN1 has provided simulation results for different ISDs. What RAN2 can provide is the synchronization accuracy budget of the Uu interface under different (reasonable) scenarios. The bottom line is whether the end-to-end synchronization path involves multiple gNBs, and consequently the network interface accuracy budget and whether the propagation delay compensation should be applied or not. For smart grid, - One gNB with a large cell size is assumed. Thus, the delay compensation is needed. For control-to-control, RAN1 has assumed in Rel-16 a small cell size (e.g., in TR 38.824, system simulation assumption for factory automation assume ISD of 20 m for factory floor size of 120 m x 50 m) and no delay compensation is needed. In Rel-17, the service area is 1000 meters by 100 meters and the exact deployment needs to be discussed. - With a large (1000 m x 100 m) service area, to re-use the same cell size of 20m would result in a very dense gNB deployments. Thus, the cell size would need to be sufficiently large and so a propagation delay compensation is needed. Note that in RAN1, 15 kHz SCS is agreed to be considered (see question 9 below) since it is assumed that the cell size can be sufficiently large. - Multiple gNBs are needed since it is difficult to cover 1000 meters in indoor environment by one gNB, and then the network interface synchronization budget should consider between two gNBs (i.e., ±320ns in Ericsson’s analysis). |

Another important assumption when evaluating the Uu interface time synchronization accuracy is the assumption on the sub-carrier spacing. In the LS R1-2007446 it has been agreed to evaluate both 15 and 30 kHz for both use cases.

Question 9: Do you agree to use 15 kHz and 30 kHz sub-carrier spacing in the evaluations of the Uu interface time synchronization accuracy for all scenarios?

| Company | Comments |
| --- | --- |
| Nokia | Yes We are fine by using the RAN1 agreement as a baseline for the evaluations for time synchronization budgets. |
| Ericsson | Yes. No reason to discuss again RAN1 agreement |

Moreover, according to RAN1 agreements, one and two Uu interfaces are considered for the control-to-control and smart grid use cases respectively.

Agreements: - For 5GS synchronicity budget requirement, - One Uu interface is assumed for smart grid. - Two Uu interfaces are assumed for control-to-control.

Question 10: Do you agree on the assumptions on the involved number of Uu interfaces in the considered scenarios?

| Company | Comments |
| --- | --- |
| Nokia | Yes |
| Ericsson | Yes. No reason to discuss again RAN1 agreement |

While discussing the scenario where two Uu interfaces are involved, it should be considered if the involved Uu interfaces require a similar time synchronization accuracy budget.  For instance, the UE deployment can cause different propagation delays, or even that propagation delay compensation is only applies for one or none of the Uu interfaces.

Question 11: When two Uu interfaces are involved, do you agree that the two Uu interfaces should be assumed to use the same time synchronization accuracy budget?

| Company | Comments |
| --- | --- |
| Nokia | Yes, In this case, we would have to assume the worst case assumption. One worst case assumption is if propagation delay compensation is needed for both Uu interfaces. Another one, if propagation delay compensation is not needed for both Uu interfaces, the worst case is that one UE is located very close to a TRP, and the other is located furthest away possible from a TRP. |
| Ericsson | Yes. The same budget is the easiest approach and indeed it is the optimal allocation of the budget. Assuming there is a fixed total budget for the two Uu interfaces, an uneven split would result in one of them with a tighter requirement. |

Related to Question 5 and 6 on the network part budget, the Uu interface budget part could account for a relative error between TRPs (assuming that between gNBs are accounted for in the network part).

Question 12: Should the Uu interface part account for an error between TRPs? And if so, what can be assumed on the maximum time synchronization error between TRPs?

| Company | Comments |
| --- | --- |
| Nokia | Yes Here we would assume that the TAE requirement given in 38.104 bounds the maximum synchronization error between any two any two antenna-connectors. |
| Ericsson | No Multiple TRPs are part of the (e)MIMO scheme which is not considered in the IIoT/eURLLC WI. If the intention of the question is about the Base Station transmit timing error (inferred from the above Nokia answer), RAN1 has considered this, see below agreements Agreements: For BS transmit timing error, further study the following three options: - Option 1: 65 ns - Option 2: ±130ns for the indoor scenario and ±200ns for the smart grid scenario - Option 3: 82.5 ns |

Finally, please indicate if anything is missing or should be considered in this Phase-1 of the email discussion and if RAN2 thinks there are any additional aspects which RAN1 should consider.

Question 13: Please indicate if anything additional should be considered in Phase-1 discussion.

| Company | Comments |
| --- | --- |
| Ericsson | The granularity of the reference time delivery from gNB to UE should be considered, which is ±5ns and included in the TR 38.825. Note that this has not been considered by RAN1 in Rel-16 and the assumption is that this will not be considered in Rel-17 by RAN1 either. In addition, some accuracy budget buffer should be allocated to account for implementation related aspects. One example is related with the 5G reference time delivery and the information for propagation delay compensation. It is important to signal the information needed by a UE to determine a downlink propagation delay value and a 5G system clock value in close time proximity. The closer these two events are in time the more accurate the UE acquires the 5G reference time. However, network cannot guarantee it is fulfilled all the time, as some messages on the Uu interface might have HARQ retransmissions. Lastly, RAN2 agreed in RAN2#110-e that - UE can calculate/predict the reference timing based on DL timing information after receiving the referenceTimeInfo from gNB once. (No spec impact) In other words, there is no UE clock drift issue in relation to the 5G reference time and RAN2 can consider a sufficiently stable UE clock so that it is not of a concern in propagation delay compensation context. It is good to confirm this baseline conclusion in the Rel-17 work. |

Question 14: Any aspects that RAN1 has not yet considered that RAN2 thinks that RAN1 should consider?

| Company | Comments |
| --- | --- |
| Nokia | No |

# 3	Phase-2 Discussion: Options for Propagation Delay Compensation

TBD

# 4	Conclusions

TBD

# References

[1] R2-2007999		Consideration on Time Synchronization for TSN in R17	CMCC

[2] R2-2006719	 IIoT Enhancements for Support of Time Synchronization	Intel Corporation

[3] R2-2006922		Discussion on enhancements for support of propagation delay compensation for accurate time synchronization	Nokia, Nokia Shanghai Bell

[4] R2-2006906		Propagation Delay Compensation for Reference Timing Delivery	Qualcomm

[5] R2-2006701		Enhancements for support of time synchronization		Ericsson

[6] R2-2006635		Discussion on Time Synchronization in Rel-17	CATT

[7] R2-2006697		Discussion on enhancements for support of time synchronization	Huawei, HiSilicon

[8] R2-2006831		Enhancements for time synchronization in TSN	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

[9] R2-2006864		Topics for time synchronization in IIoT		Fujitsu

[10] R2-2007141		Consideration of TSN time synchronization enhancements	OPPO

[11] R2-2007145		Discussion on the TSN enhancements		vivo

[12] R2-2007294		Discussion on uplink time synchronization for TSN		NTT DOCOMO INC.

[13] R2-2007475		Considerations on time synchronization enhancement	Lenovo, Motorola Mobility

[14] R2-2007611		On propagation delay compensation		MediaTek Inc.

[15] R2-2007627		Enhancements for support of time synchronization		Sequans Communications

[16] R2-2008033		Discussion on support of time synchronization	LG Electronics Inc.

[17] R2-2008059		Enhancements for Timing Synchronization	Samsung