3GPP TSG-RAN WG2 #113-e	R2-21xxxxx

E-meeting, 25 January – 05 February 2021

Agenda item:	8.4.2

Source: 	Samsung

Title: 	Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung)

Document for:	Discussion and decision

# Introduction

This submission contains the report from the following post-RAN2#112-e email discussion:

- [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung)

Scope: Starting from previous outcomes, determine which issues to address. If time permits can have a round of discussion on candidate solutions, e.g. which solution to address identified issue. Details also as proposed in [Post112-e][030], Intermediate deadlines by Rapporteur.

Intended outcome: Report, including at least an agreeable list of issues to focus on going forward, in the order of priority/support, pave the way for meeting discussions and agreements.

Deadline: Long

As a reminder, the following has been agreed as one of the objectives of the Rel-17 IAB WI:

Topology, routing and transport enhancements [RAN2-led, RAN3]:

- Specifications of enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation

Starting from guidelines provided by the Chair in the post-RAN2#111-e email discussion [Post111-e][902][eIAB], and the submissions on the relevant issues to RAN2#111-e (none of them were treated as there were no TUs allocated to Rel-17 IAB WI at RAN2#111-e, but were a very useful and valid starting point nonetheless), a discussion aimed at agreeing a baseline and discussing main issues was held and is captured in R2-2011061. A follow-up, more detailed discussion was then held during RAN2#112-e ([AT112-e][030][eIAB], captured in R2-2011142), aiming at discussing initial solutions.

This document aims to revisit the fundamental issues in more detail, capture and quantify support for different identified issues, and map the solutions already identified in R2-2011142 to a comprehensive list of issues.

# Phase-1: collecting issues

In Tables 1/2/3 below, rapporteur has captured Fairness/Latency/Congestion issues based on R2-2011142 and R2-2011061, and various offline discussions already held. Companies are invited to add further issues. At this stage there is no need to dispute an issue, or indicate support for an issue (although naturally adding your own issue which is not initially in the rapporteur’s list will indicate some level of support for that specific issue).

You are however welcome to share clarifications on certain issues in the final column (this is especially important if you were one of the original proposers of an issue/related solutions, and what is captured is not complete or correct in your view).

Please note there is currently no mapping of solutions to these issues (this will happen in Phase-3) – at this stage suffice it to say that there are fewer issues than solutions (as is expected – this does not mean that down-selection of solutions has taken place). Also, several of Topology and Latency issues overlap, so they are only captured in one place.

## Topology-wide fairness

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IF-1 | The scheduler of an IAB node may not have all the information needed (e.g. link quality across multiple hops) to make appropriate scheduling decisions (e.g. proportional fair) for different bearers/RLC channels across multiple child-IAB nodes | The scheduler of an IAB node does not have the information on links further downstream | Improves scheduling efficiency and increases fairness | [Intel]: It suits for both upstream and downstream [QC] The issue description needs to capture the scheduler’s expected behavior in case there is different downstream link quality for two BH RLC Channels. Without such description, it is not clear if any information is missing or what information would have to added. It is also not clear why adding information would make things any better. [Apple] One point to note here is that the bar of how fair the IAB/eIAB system should be can be set to that of a single hop system. The issue as mentioned gets trickier esp. with N:1 mappings in two ways -The RLC latency is normalized if aggregation is done across link qualities and - loss of aggregated BH RLC SDUs can be felt across multiple UEs which does not happen in the 1:1 mapping cases. In the name of fairness, we should tip the scales by providing undue advantages to UEs that might for example, be further away from the IAB Nodes in terms of RF (note: the point here is different from those UEs that are further away from the donor due to multiple hops). [CATT] We need to discuss this issue based on unified understanding of “fairness”. [Sony]: We think the scheduler issue does exist and may impact the topology-wide fairness. We share the opinion that scheduler’s behavior should be elaborated and what’s the potential missing information. [AT&T]: One expected behavior of the scheduler would be to use the downstream link quality measurements/metrics to adjust the scheduling (e.g. PF weights) to take into account the overall route link quality and not just the immediate next hop. For example the scheduler may prioritize resources for a BH RLC channel which has lower quality on a downstream hop over one with higher quality, even if the quality of the two channels on the immediate next hop is the same or similar in order to improve fairness. The information related to the downstream quality could be existing measurements (e.g. CSI feedback) or some form of a ‘benefit metric’ which aggregates/abstracts the multi-hop link quality in a uniform manner. |
| IF-2 | Workload on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced | IAB nodes do not have the QoS info of the bearers themselves, while CU has no info of the BH links load | Helps achieve similar latency for UEs with same QoS requirement | [Ericsson]: We are not sure that “while CU has no info of the BH links load” is an entirely correct statement. The CU is in control of the BH RLC traffic traversing a certain IAB node and hence the load. [Intel]: The key/final issue caused by unbalanced between BH RLC channels is that it increases user latency experience due to (unbalanced) heavy load or even congestion at IAB nodes. Hence, following changes are suggested: Adding “, leading to some users experiencing longer latency” to reflect the real issue [ZTE] This issue is not quite clear to me. The load in BH RLC channels are unbalanced. However, IAB node can get the QoS info of BH RLC channels and then schedule appropriately. It is suggested to provide more detailed description. [QC] What is the assumption on the reason for the unbalance of the workload: Different load among bearers or different number of bearers per BH RLC CH? What is the expected behavior of the scheduler to fix the issue if it had all information under the sun [CATT] This issue is difficult to understand. Please give an example to illustrate the potential enhancement. We think CU has the info of the BH link load. |
| IF-3 | IAB node cannot prioritize bearers with e.g. higher number of hops | The scheduler of an IAB node does not have a latency reference (e.g. total number of hops, remaining number of hops) for the packets being scheduled | Helps provide same QoS experience regardless of hop number | [CATT] IAB network is not same as non-IAB network, because packets need to go through multiple hops to reach the destination. If IAB-node scheduling ignores multiple hops, packets with the same QoS requirement would have different latency. That is unfair to the packets with the same QoS requirement. |
| IF-4 | IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers | The scheduler has no knowledge of number of aggregated bearers on a BH channel | Helps provide same QoS experience regardless of bearer mapping | [HW]: Want to clarify if “BH RLC CHs that aggregate more bearers” is equal to “BH RLC CHs that aggregate large data rate”. It seems the actual data load/data rate on a certain BH RLC is not exactly same as the number of aggregated bearers. [QC]: On HW’s comment: The observation is correct that the actual load on BH RLC channels depends not only of the number of bearers aggregated but also on the load per bearer. The following should be achievable: If two BH RLC channels have same buffer load but carry different number of DRBs with same QoS, the scheduler should be able to give the BH RLC channel with more DRBs higher weight. For that purpose, it needs to know the number of DRBs in the BH RLC channel. [Apple] As in IF-1 this is a tricky situation and depends on how the aggregation itself is being done. In providing more resources the overall fairness shouldn’t be moved from one extreme to the other. Issue as listed by rapporteur however remains. |
| IF-5 | IAB node cannot offer per-bearer fairness | The scheduler does not have QoS information of individual bearers, and feedback from CU to access node (via NR UP protocol) is not visible to intermediate nodes | Helps provide same QoS experience regardless of bearer mapping | [Ericsson]: What is the difference between this issue and IF-2? Both IF-5 and IF-2 seems to address the same problem. It is also not clear what is the “feedback from CU” and why such feedback should be visible to intermediate nodes. It would be good to clarify what would be the purpose of such feedback. In general, that seems to hint to a possible solution, so it should not be included here in the problem description. [HW]: Is it the E2E bearer when we say “per-bearer”? Does IF-5 covers IF-4? [ZTE] I guess the per-beare means the UE bearer aggregated to the BH RLC channel. However, we are confused with the feedback from CU to access node. What kind of feedback is it? [QC] As we discussed online, the term “fairness” is not about meeting QoS requirements but it relates to the resource distribution among bearers for BE services. The goal of 1:1 mapping was to enable per-bearer QoS support. It has always been understood that there is only limited per-bearer QoS support possible when bearers are aggregated. Also, we cannot assume that there is any sub-BH-RLC-CH QoS differentiation since this would imply prioritization of some packets over others within the same ARQ window which disturbs the ARQ packet order and will be canceled out via reordering at the receiver. [CATT] We understand IF-5 is same as IF-2. Does it mean the fairness for the bearers in the same BH RLC channel? We can use the 1:1 mapping for per-bearer fairness. |
| IF-6 | How the success of fairness mechanisms is evaluated | No such mechanism defined in Rel-16 | Provide a criterion for solution evaluation | [Intel]: Capturing a FFS issue from R2-112e. [LG]: We understand that “FFS how the success of such mechanisms is evaluated” is important. But, we think that it seems a different kind of issue unlike other issues. Actually this is needed when we evaluate and determine a solution for an identified issue. Given that this email discussion focuses on identifying a real issue, not an issue on how to evaluate a solution, it would be good to clarify whether this issue is relevant with this email discussion. [QC] We should not try to define fairness. The present approach taken here is much better where we look at individual problem scenarios, see how they should be solved and what is missing. [Apple] This can be part of the solutions phase as others have suggested and is not necessary in the issue description phase. |
| F-1 to 5 |  |  |  | We appreciate the classification among the various issues provided by the rapporteur in this table. Moving forward, we propose to define in a little more detail the underlying assumptions (col 2) and the (tentatively) expected behavior of the scheduler. Otherwise, it is hard to see why Rel-16 wouldn’t be able to address the issue and to converge on what additional information is needed at the scheduler. |

## Multi-hop latency

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IL-1 | IAB node cannot help ensure that overall or remaining PDB is met for a packet | IAB node does not have information on the remaining PDB or overall PDB | Helps to meet required PDB |  |
| IL-2 | IAB node may need to report joint buffer status for LCHs with rather differing QoS requirements | IAB node’s scheduling granularity is limited by the Rel-16 number of LCGs | Helps to meet required PDB | [CATT] Does this issue is to increase the number of LCGs? |
| IL-3 | Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors | Buffer size calculation for pre-emptive BSR is left to implementation | Helps ensure more consistent scheduling across the network(s) |  |
| IL-4 | IAB node cannot determine the one-hop latency for the access link | Latency between donor CU and access IAB-node-DU, including possible multiple hops in between, is unknown to the access IAB node | Helps to meet required PDB | [Ericsson]: We added “access” since the issue description is about “the one-hop latency for the access link”. Is that correct understanding? [ZTE] why emphasize access link? IAB node may not determine the one hop latency for backhaul link. Is there any special for access link? [QC] Agree with Ericsson. The access link PDB configuration includes the backhaul-related delay. For a wireline system, this can be hard configured on the DU via OAM. For IAB, it includes the wireless backhaul delay as well which is subject to change. [Sony]: We agree with Ericsson and think the backhaul delay between CU and access link should be considered. |
| IL-5 | The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free | The CU does not have information on the buffer status of IAB nodes or status of individual links | Helps ensure more efficient scheduling and meet required PDB |  |
| IL-6 | The CU is unable to configure routing based on actual latency per BH RLC channel | The CU does not have information (e.g. measurement reports) on one-hop latency per BH RLC channel | Helps to meet required PDB | [HW]: We need to clarify if this “actual latency” is the real-time latency status or some long term average latency status. [ZTE] I think it may be a filtered latency status. |
| IL-7 | IAB node cannot reliably/efficiently discard packets | The only PDB information known to the IAB node is one-hop PDB (used for scheduling) – the CU cannot currently provide e.g. a recommended discard PDB | Helps ensure more efficient scheduling and meet required PDB | [ZTE] Just to clarify that the one hop PDB provided from CU to IAB DU is for the BH RLC channel between IAB DU and child IAB MT. For the collocated IAB MT, it does not know the PDB for the BH RLC channel between itself and parent IAB DU. [Apple] …… while still operating within the packet error rate boundaries of the 5QI since PDB is just one of the 5QI. For BE flows, the additional packet dropping mechanisms shouldn’t cause any additional drops than those experienced by a single hop system. |
| … |  |  |  |  |

## Congestion mitigation

| Issue no. | Issue description | Why cannot the issue be solved with Rel-16 baseline and/or implementation? | Why is the issue worth pursuing? | Any comments? |
| --- | --- | --- | --- | --- |
| IC-1 | Long-term congestion on a single link cannot be alleviated using existing mechanisms | Rel-16 HbH flow control cannot handle long-term congestion | Helps alleviate and prevent congestion | [HW]: We’d better clarify this is for UL or DL. [Intel]: We wonder IC-2/3/5 cover IC-1? [QC] We disagree. AQM on the intermediate node will drop packets in response to congestion. This packet loss will be reported back by the access IAB-node via DDDS to the CU-CP. This report can be used by the CU-UP to throttle throughput. However, this mechanism relies on packet drop. It would be better to improve this mechanism to not require packets to be dropped [CATT] We agree. R16 HbH flow control can not alleviate the long term congestion. R17 need some enhancement mechanisms. to solve this issue, e.g UL HbH flow control, DL HbH flow control enhancement, local re-routing and so on. |
| IC-2 | Congestion in a chain (local part of the network) cannot be alleviated using existing mechanisms | Rel-16 HbH flow control only handles localized congestion | Helps alleviate and prevent congestion | [QC] See comment in IC-1 [CATT] We think this issue is same as IC-1. |
| IC-3 | Child node keeps requesting UL resources and/or allocating UL resources to its descendant nodes, even if the parent node is experiencing upstream congestion | There is no UL HbH flow control in Rel-16 | It can bring enhancements on top of existing UL scheduling procedures and potentially alleviate upstream congestion | [Nokia] The issue is rather as expressed as Observation 1 in our R2-2009798: “Based purely on uplink-scheduling mechanisms, an IAB node in DC with only one of its cell groups (MCG or SCG) congested has no means to selectively limit receiving uplink data to be routed over the congested cell group from child nodes.” [QC] The issue description does not refer to DC. As we interpret the issue description, it is not clear why UL HbH flow control would fix anything. The congestion point can only be pushed down to the access IAB-node. It cannot be pushed down to the UE, i.e. the traffic source. [CATT] We think this is a UL type of long-term congestion as IC-1. UL flow control is needed to solve this issue. |
| IC-4 | IAB node does not know if the parent node has recovered (or is likely to recover) from BH RLF | IAB node has no knowledge of the progress of recovery from BH RLF – only that initial recovery has failed | Minimizes unnecessary topology changes, helps prevent congestion | [HW]: Is this for RLF handling or congestion mitigation? [QC]: In Rel-16 the IAB-node does not even know that the parent node has BH RLF until the parent node’s recovery attempts have failed. This topic is handled by the email discussion on topology adaptation. |
| IC-5 | IAB node may send flow control feedback multiple times | Triggering of flow control feedback is done based on a simple threshold rule | Keeps overhead in check |  |
| IC-6 | Child node stays with the same parent node, even if the parent node is experiencing downstream congestion, which could cause congestion to worsen further downstream | Child node is unaware of congestion at parent node | Helps alleviate and prevent congestion | [Intel]: Avoid resource waste at child IAB node and reduce packet drop is another reason why this issue should be further pursued [QC] This issue is completely incomprehensible. What is proposed her: The child node performs reestablishment because the parent was congested? [Apple] Should this issue be dealt in the topology adaptation discussion? Or, proactive messaging to avoid congestion? |
| IC-7 | CU cannot update the routing path that is experiencing congestion | CU has no knowledge of local congestion conditions | Minimizes unnecessary topology changes, helps prevent congestion | vivo: Another reason why we think this issue should be pursued is that it also helps to maintain the centralized management role of CU, so that the routing behaviors under within the same topology can be well-controlled. [Sony] It’s indeed that CU may not get up-to-date local congestion information. And to keep the routing table in real-time manner would be costly in terms of signaling overhead. |
| IC-8 | Parent node does not know the desired data rate from the per hop HbH flow control feedback | The desired data rate information is not supported in R16 per hop flow control | Help parent node to throttle the transmission rate, in addition to the data volume. | [QC] BAP flow-control feedback achieves the same goal. Since the desired data rate varies on short time scale due to changes on downstream link condition, absolute data-rate values need to be frequently updated which creates high CP overhead |
| … |  |  |  |  |

# Phase-2: consolidating the list of issues, collecting support for individual issues

# Phase-3: mapping of previously proposed solutions to the consolidated list of issues

# Conclusions