3GPP TSG RAN WG2 #112-e			R2-20xxxxx

e-Meeting,  2nd – 13th November, 2020

Title:	[draft] LS on Conditional PSCell Addition/Change agreements

Response to:

Release:	Release 17

Work Item:	LTE_NR_DC_enh-Core

Source:	CATT [RAN2]

To:	RAN3

Cc:

Contact Person:

Name:	Chandrika Worrall

E-mail Address:	chandrika@catt.cn

Attachments: -

1. Overall Description:

RAN2 has discussed support of conditional PSCell addition(CPA) and conditional PSCell Change (CPC) under Rel-17 WI. The following agreements have been made.

- Maintain Rel-15 principle that only one PScell is active at a time even with conditional PScell addition/change.

- Usage of CPAC is decided by the network. The UE evaluates when the condition is valid.

- The baseline operation for CPAC procedure assumes the RRC Reconfiguration message contains SCG addition/change triggering condition(s) and the RRC configuration(s) for candidate target PSCells. The UE accesses the prepared PSCell when the relevant condition is met.

- CPAC execution condition and/or candidate PSCell configuration can be updated by modifying the existing CPAC configuration.

- Support configuration of one or more candidate cells for CPAC.

- For CPA and MN initiated Inter-SN CPC, the MN generates and transmits the conditional configuration message (i.e. RRCReconfiguration/RRCConnectionReconfiguration message) to the UE.  The RRCReconfiguration provided by the candidate PSCell(s) is encapsulated in the final conditional reconfiguration message to the UE. The MN is not allowed to alter the RRCReconfiguration provided by the candidate PSCell(s).

- In MN initiated inter-SN CPC and CPA, the MN is not required to indicate the execution condition(s) to other involved entities (e.g. target SN, source SN).

- For SN initiated inter-SN CPC, the MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

Furthermore, in order to progress the signaling design RAN2 would like to know whether RAN3 inter-node messages (i.e. XnAP fields) will only support addition/ modification of a single candidate PSCells (alike for PSCell in REL-16)?

2. Actions:

To RAN3

ACTION: 	RAN2 respectfully asks RAN3 to take the above information into account in future work. RAN3 is kindly asked to provide feedback on RAN2’s doubt regarding multiple PSCell candidates in XnAP messages.

3. Date of Next TSG-RAN2 Meetings:

3GPP RAN2#113-e		25 January – 5 February 2021		Electronic Meeting

3GPP RAN2#113-bis-e 		12 April – 20 April 2021			Electronic Meeting