3GPP RAN WG2 Meeting #113e	R2-210xxxx
E-meeting, 25 Jan - 05 Feb, 2021

Agenda Item:	8.6.1

Source:	Email discussion rapporteur (Lenovo)

Title:		Report from email discussion POST112-e][550][SDT] Further details of CG
aspects

Document for:	Discussion, Decision

# Introduction

This document contains the summary of email discussion to agree further details of the CG-based solution for small data transmission in RRC_INACTIVE:

LONG EMAIL DISCUSSION

- [POST112-e][550][SDT]  Further details of CG aspects (Lenovo)

Scope: Get input on:

•     Resource configuration, reconfiguration and release of CG resources (i.e. validity during cell reselection other validity criteria such as RSRP etc).

•     TA procedure

•     Single vs multiple CG configurations

•     Which BWP to use (initial or dedicated)

Output: set of agreeable proposals

Deadline for company comments:

To allow sufficient time to summarise and submit the summary and proposals to the upcoming meeting, the following deadline for company comments is proposed:

Wed Jan 13th 0700 UTC

# CG resource configuration, reconfiguration and release

## Configuration of CG-SDT resource (RRC_CONNECTED)

It has been agreed in RAN2#112e that the configuration of the configured grant resources for UE uplink small data transfer is contained in the RRCRelease message. It is currently FFS whether other dedicated messages can also configure CG-SDT resources for RRC_INACTIVE. Some contributions mentioned that RRCReconfiguration message may also be used for providing the CG-SDT resources while UE is still in RRC_CONNECTED. However no clear benefit or specific use case was basically brought forward. On the other hand in [7,18] it is argued that RRC Release seems better suited than the RRC Reconfiguration, because the gNB will decide if the SDT is allowed or not at late stage, e.g. when deciding the RRC Release procedure.

Question 1: Do companies agree, that CG-SDT resource configuration is provided to UEs in RRC_Connected only within the RRCRelease message, i.e. no need to also include it in RRCReconfiguration message. (If no, please justify)

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | N | it should be up to the network’s implementation to decide whether the UE continues to utilize the CG resource that it has been configured in the CONNNECTED or overthrow this configuration and replace it with a new one. We would prefer to allow for continuing to use the connected mode configuration. If for a UE in connected mode transmitting SDT data with CG type1, the UE should still be able to use the same configuration in INACTIVE. Another aspect is that, by performing delta configuration to the CG config in CONNECTED mode, the signaling overhead can be reduced |
| ZTE | Y | When the UE is in RRC_CONNECTED state, it maybe possible to use also RRCReconfiguration message to configure with a set of CG resources applicable for SDT (i.e. the CG resources to be used in INACTIVE state). However, we think RRCRelease is enough for this purpose (since anyway these resources are not used during the RRC_CONNECTED state) and including these resources in RRCReconfiguration message seems like an optimization that is not really necessary. Using RRCReconfiguration message also creates additional complexity since the CG-SDT resources are only valid in the cell in which the UE enters RRC_INACTIVE state. However, if RRCReconfiguration is used during the RRC_CONNECTED sate and if there is a handover, then the network has to clean-up the SDT resources configured by the previous cell before the UE is moved to INACTIVE. We think this just adds unnecessary complexity. So, RRCRelease itself is enough in our view (i.e. same as LTE PUR). Note that if the UE already has CG configuration in connected mode, the RRCRelease message can include some delta configuration for this connected mode CG configuration in the RRCRelease message and use this as the CG-SDT configuration to be applicable to RRC_INACTIVE state. So, configuring CG-SDT configuration specifically for RRC_INACTIVE state just to enable delta signalling seems not really necessary in our view. When the UE is INACTIVE state (i.e. during the SDT procedure), then we certainly think that RRCReconfiguration message shall not be allowed for configuring or reconfiguring the CG resources as this will result in much more complexity and the SDT phase should be used for exchange of data rather than radio level signalling messages. So, at least in this scenario we certainly want to preclude use of RRCReconfiguration message. |

## SUL for SDT

There can be two UL carriers (NUL and SUL) configured for a serving cell. The two carriers have different characteristics (e.g. different frequency bands FR1/FR2). Since payload size of a CG-SDT PUSCH resource might be larger, it may be beneficial to also support the configuration of CG-SDT resources on the SUL carrier. Basically, as argued in [4,6] it should be possible to configure CG resources for SDT separately for NUL and SUL, i.e. RRCRelease message provides CG-SDT resource for NUL and SUL.

Question 2: Do companies agree, that CG-PUSCH resources are separately configured for NUL and SUL? (If no, please justify)

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | In R15, CG type1 can only be configured on one UL carrier because R15 stipulates that there should only be only CG type1 configuration within one serving cell. In R16, this restriction was removed because multiple CG configurations are supported and CG type 1 can be separately configured for NUL and SUL. For R17 small data, we think it should follow R16 as the baseline and CG-PUSCH should be separately configured for NUL and SUL |
| ZTE | Y | Indeed it makes sense for the network to be able to configure CG-SDT resources on SUL or NUL or both. |

## Reconfiguration/release of CG-SDT while in INACTIVE

In general, gNB may want to reconfigure or release the CG-SDT resources while the UE is performing SDT in RRC_INACTIVE state. Since an RRC-based solution is considered as a baseline for SDT, the gNB would include a DL RRC message, e.g. RRCRelease message, to respond to an UL RRC message or to end the SDT procedure, without moving the UE to RRC_CONNECTED state. A solution to reconfigure or release CG-SDT resources while UE is in RRC_INACTIVE state could be therefore achieved with this RRC message, e.g. RRCRelease message. It should be noted that in LTE, PUR is terminated by RRC signalling, i.e. RRCConnectionRelease together with other information such as e.g. release cause, I-RNTI, NCC, etc.. For simplicity and less specification impacts, a similar message can be reused, e.g. RRCRelease message is used to release the CG-SDT resources in NR.

One remark is that the DL RRC message (in response to UL RRC message) may not be the same as legacy RRCRelease messages but rather a new RRC message similar to the RRCRelease message.

Question 3: Do companies agree, that the RRCRelease message (or similar) is used to reconfigure or release the CG-SDT resources while UE is in RRC_INACTIVE. (If no, please justify).

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | This was legacy PUR solution and can be reused for SDT |
| ZTE | Y | Yes, we think we can agree to use RRCRelease message itself. |

## CG-SDT resource request by UE

For LTE PUR, configuring a UE with PUR can be triggered either by the UE sending a PUR Configuration Request message to an eNB when in RRC connected state or by the network (e.g., based on subscription information and/or identification of a periodic traffic pattern). Based on the UE request the Network may decide to configure PUR to the UE.

Several companies suggest that similar mechanism should be also used for NR CG-SDT, i.e. the UE requesting the CG-SDT configuration. It should be noted that the detailed content of such UE request message - if agreed to be introduced - needs to be further discussed.

Question 4: Do companies agree, that UE should be allowed to request to the Network for CG-SDT resource while being in RRC_Connected?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | This was legacy PUR solution and can be reused for SDT |
| ZTE | N | We don’t think this is essential. Our concern main is that the discussions on the UE assistance framework, reliability of UE assistance and the necessary prohibit mechanisms to prevent repeated requests from UEs might end up taking fairly long time to discuss and specify this. CG-SDT is useful in specific cases where the UE is known to generate a well-known traffic pattern and the UE is stationary. Such a UE should be known to the network at subscription level (e.g. the UE is known to be a smart meter etc) and any additional UE assistance is not really as reliable as the subscription information for this purpose. However, if majority view is to have this, we would like to stress the importance of reusing some existing frame work (e.g. the UE assistance information) to minimize the extra work needed to specify this. |

## Implicit release of CG-SDT resource (skipped CG resources)

In LTE PUR, the resources are periodically assigned. If PUR resources are assigned to a UE which does not have data to send for a long time, it will inevitably cause a waste of resources. Therefore, in LTE PUE a UE can be configured in such a way that it is allowed to skip transmitting a certain number of PUR occasions in a row (up to eight can be configured), after which the PUR configuration is implicitly released.

Several companies proposed to introduce a similar mechanism for NR CG-SDT, i.e. implicit release of CG-SDT resources. The released CG-SDT resources can be further allocated to other UEs. A simple way would be to reuse the skipping mechanism, in that a UE skipping SDT CG resources for a configured number of times also releases the CG-SDT configuration, i.e. the configured uplink grant is released by upper layers and corresponding uplink grants are cleared if UE skips consecutively a configured number of CG grants. This would also reduce the signalling overhead, e.g. RRCRelease message.

Question 5: Do companies support to introduce an implicit CG-SDT resource release mechanism, i.e. upon having skipped a consecutive number of CG-SDT occasions.

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | This was legacy PUR solution and can be reused for SDT. But, we need to further discuss the impact from the CG to SSB mapping, which is new in NR and the possible multiple CG configuration on the same BWP, if it is supported |
| ZTE | Y | We think this would be useful. We agree that this will need some discussion on what “skipping” means, considering the multi-beam system and also considering the potential that multiple CG resources may be configured for the UE (based on the outcome of Q12). |

## CG resource handling at Cell reselection

It has been already agreed that the configuration of CG-SDT resources for UE small data transmission is valid only in the same serving cell. Therefore, UE shall implicitly release the CG resource configuration when it moves to another cell.

Question 6: Do companies agree that UE releases the CG-SDT resources when it moves to a different cell? (If no, please justify)

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | N | UE releases the CG configuration only when it initiates RRC Resume Request on another cell. This is the legacy PUR behavior and should not be changed here. The reason is that the network is not aware of the cell reselection from the UE and there is no need for the UE to release the resource by itself without letting the network know about this. Also, the UE may actually go back to its previous cell where it has a CG configured before having to send data and in this case such CG configuration could still be used. |
| ZTE | N (but can be left to stage 3) | The existing agreement clearly requires the UE to not use the CG resources configured by one cell (i.e. the cell in which RRCRelease is received) in a different cell. There are two ways to specify this: - Specify that UE releases this whenever there is a cell reselection during INACTIVE state - Specify the UE release procedure only upon initiating the RESUME procedure (i.e. at that point UE will check whether CG-SDT resources are valid based on a number of conditions and if it is in a different cell it can release the resources at this point) We think both 1) and 2 can work. Perhaps option 2) is simpler to capture in RRC (and is probably what Huawei are hinting towards too). With this in mind, we think our answer to this question is No. |

## Subsequent data transmission and indication

Although there was some discussion in email discussion R2-2009189 regarding the subsequent data transmission phase for CG, we were not able to agree the details of this. The following observations were noted in that email discussion outcome:

Comments Summary: - 18/26 companies think that UE shall monitor a dedicated RNTI - Of these 11/25 think it can be same as the previous C-RNTI - 6/25 think a new SDT-RNTI can be configured by the network - 8/26 companies think further discussion is needed - 4/22 companies said that this depends on whether the subsequent data uses DG or CG Open issues: - For CG, should the subsequent data be sent only over CG or is DG also allowed? Seems natural to allow DG as is the case for CG today (note that anyway for DL transmission during subsequent data phase, it seems we need DG?) – we can check this understanding via an explicit proposal - If DG is also allowed should we use C-RNTI or a new SDT-RNTI?

Based on the above, it would be good if we could agree whether subsequent transmissions after initial UL message in CG resource for CG-SDT can use both CG and DG.

Based on the above, the following question is raised:

Question 7: For CG-SDT do companies agree that the subsequent data transmission can use the CG resource or DG (i.e dynamic grant addressed to UE’s C-RNTI)? Note: The C-RNTI can be the same as the previous C-RNTI or may be configured explicitly by the network (we can discuss this later – FFS)

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | It depends | We think the answer to this question depends on whether we support multiple HARQ processes for CG-based scheme. If there is only one HARQ process, DG will have to share the HARQ process with CG. The motivation for using DG with PDCCH addressed to C-RNTI is not clear. In R15, the motivation that the UL grant for a HARQ process can be provided by either CG or DG was that there might be data with higher priority arriving for the CG resource. While for Small data, we do not see the same motivation for data transmission during INACTIVE. Note that in LTE PUR, only one HARQ process was supported. While if there are multiple HARQ processes and DG does not need to share HARQ process with CG, there seem to be no issue to support CG and DG at the same time from RAN2 perspective. However, there might be several issues need to be addressed for dynamic scheduling from RAN1 perspective. There is that there is no beam management for INACTIVE. It is unclear how the gNB should determine the beam for PDCCH and which beam the UE should monitor. Also, PDCCH should include SRI, indication for SRS, for the uplink transmission of the UE. How this can be done in INACTIVE is not clear either. From this perspective, we prefer this also to be discussed in RAN1 and see if it is feasible to have DG in INACTIVE. |
| ZTE | Y | In general we think we can reuse the Rel-16 framework for this. It should be noted that the CG resource will be associated with an SSB and hence the network can derive the beam information basedo n the CG resource on which the initial transmission is received. So, the network has enough beam related information to schedule the PDCCH (similar to the case of RA-SDT). For the HARQ process, although NB-IOT UE may have single HARQ process, we think multiple HARQ process will be supported for NR UE (even for REDCAP UE, multiple HARQ process will be supported it seems). So, we think it is up to NW to determine whether to use DG or not, but it seems there are no show-stoppers to reuse the Rel-16 framework for DG support. |

# Timing Alignment procedure

## Starting of TAT-SDT

It has been agreed in RAN2#112e that a new TA timer for TA maintenance specified for CG-SDT in RRC_INACTIVE is introduced. This new TA timer, referred to as TAT-SDT in the following, is configured together with the CG-SDT configuration in the RRCRelease message. UE is only allowed to perform CG-SDT transmissions in case it has a valid TA, e.g. TAT-SDT is running in RRC_INACTIVE.

Similar to the TA maintenance procedure in RRC_Connected, where UE starts the TAT upon reception of a TA command from gNB, e.g. in RAR, rapporteur assumes that UE should also start TAT-SDT upon receiving the TAT configuration from network, i.e. in RRCRelease message.

In the same way UE should also (re)start the TAT-SDT upon reception of TA command from gNB, e.g. received within RACH procedure performed after expiry of TAT-SDT.

Question 8: Do companies agree that the TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and (re)started upon reception of TA command?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| ZTE | Y |  |

## Timing alignment validation

In LTE PUR, a TA timer and associated procedures are defined to maintain the validity of the uplink time alignment for PUR transmissions. Before performing a PUR transmission, the UE must evaluate the validity of the TA based on certain defined conditions. The associated conditions in TS 36.331 are given below:

5.3.3.19 Timing alignment validation for transmission using PUR A UE shall consider the timing alignment value for transmission using PUR to be valid when all of the following conditions are fulfilled: 1> if pur-TimeAlignmentTimer is configured: 2> pur-TimeAlignmentTimer is running as confirmed by lower layers; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured: 2> since the last TA validation, the serving cell RSRP has not increased by more than increaseThresh; and 2> since the last TA validation, the serving cell RSRP has not decreased by more than decreaseThresh;

Whenever the TA validity conditions are not met (e.g. the cell RSRP level has changed more than a configured threshold) then the PUR configured resources are released.

Several companies propose to introduce a similar TA validation procedure also for NR CG-SDT, i.e. using a RSRP based criteria for the TA validation. Even if the TAT-SDT timer is running, significant path loss changes due to UE's mobility may result in a change of the timing advance. Basically UE would need to check in order to validate the TA whether the serving cell RSRP has changed more than a configured threshold since the last last time TAT-SDT has been (re)started. Exemplary UE behaviour for TA validation could be like the following:

- If TAT-SDT is running; and

- If the serving cell RSRP has not increased by more than SDT-RSRP-IncreaseThresh since the last time TAT-SDT was started; and

- If the SS-RSRP of the pathloss reference has not decreased by more than SDT-RSRP-DecreaseThresh since the last time TAT-SDT was (re)started:

  - TA is considered valid

Question 9: Do companies agree to configure SDT-RSRP-ChangeThresholds for the validation of the TA in RRC_INACTIVE?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes, but | Agree that similar solution as PUR can be adopted for SDT, but for the threshold of increase and decrease, the values should be separated configured, similar to PUR. Also, we are not clear why it is serving cell RSRP for RSRP increase and SS-RSRP for RSRP decrease in the description. Maybe this is not intentional. |
| ZTE | Y | We agree it should be SS-RSRP of the pathloss reference in both cases. Note that the beam could have changed. i.e. the SSB used for pathloss reference could be different in each case. It is worth clarifying that this is the common understanding. |

## UE behaviour at invalid TA

It has been already agreed that UE can only use configured CG-SDT if it has a valid TA. The behaviour when UE’s TA is invalid, i.e. expiry of TAT-SDT or RSRP has changed more than a configured threshold (if agreed to be introduced) needs to be further specified. It seems to be the common understanding that in these cases the UE will need to revert to either a RACH based SDT access, assuming conditions for this are fulfilled, or otherwise to legacy connection resume procedure.

One point to clarify is whether UE shall store the CG-SDT configuration or whether UE shall discard the CG-SDT configuration when TA is invalid, e.g. upon TAT-SDT expiry. It can be argued that it is beneficial if UE stores the CG-SDT configuration. Since the UE can only use RACH-based SDT when SDT is triggered and TA is invalid, a TA Command will be received in the RAR and time alignment will be regained, i.e. TAT-SDT timer is restarted upon reception of TA command. Hence the SDT-TAT could be restarted then, and the stored CG-SDT can be used again for the following SDT phase.

Question 10: Do companies agree that for cases when UE has no valid TA in RRC_Inactive state, e.g. expiry of TAT-SDT, UE stores the CG-SDT configuration?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | In addition to the motivation mentioned by the rapporteur, another motivation to keep CG configuration upon invalid TA is that, when the RSRP changes beyond the threshold, the TA will become invalid. But if the UE moves to the region within which the RSRP is within the thresholds, the UE will be able to use the CG resource again. But this is, of course, under the condition that the TAT is still running. TA validity should only be checked when the SDT transmission is triggered for CG-based scheme |
| ZTE | No strong view | We are okay for the UE to store the CG configuration in this case if this is the majority view. |

In RRC_CONNECTED, a UE maintains NTA (defined in TS 38.211) of all TAGs or the corresponding STAG in case of TAT expiry. The reason for such behaviour is, that gNB may send a TAC within the MAC CE in order to restart TAT, e.g. UE is not performing random access procedure. However, for RRC_INACTIVE rapporteur thinks that there is no need to maintain the NTA value since TAT-SDT can be only restarted by a random access procedure when UE has no valid TA, where an absolute TA command is signalled to the UE within the RAR. Therefore, UE should just discard NTA when TA is invalid, e.g. upon TAT-SDT expiry.

Question 11: Do companies agree that UE shall discard NTA when TA is invalid, e.g. upon expiry of TAT-SDT, while being in INACTIVE state?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | It depends | This depends on whether the UE continues to perform PDCCH monitoring after the TA expiry. If the UE does not, there is indeed no need to keep the NTA anymore. However, if it does, the UE still can obtain the TA command, either in DCI or in MAC CE like in legacy R15. |
| ZTE | Yes but… | We think the UE can discard NTA. We just note that in Rel-16 we have introduced an absolute TAC MAC CE too. Do companies think this could be used to provide the absolute TA to the UE? In any case we don’t have a strong view and we think we can use the simple approach of relying on RACH procedure. In our view, the CG-SDT should only be used where there is a very reliable knowledge that the TA won’t change (as we explained, based on subscription level information). So, we are okay with not optimizing this too much. |

# Single vs multiple CG configurations

## Single vs multiple CG configurations

For RRC_CONNECTED mode, NW can configure multiple CGs per UE for different service transmissions. In [13, it is argued that multiple types of traffic are supported for small data transmission, such as traffic from Instant Messaging services, Heart-beat/keep-alive traffic, traffic from wearables, sensors and so on. Since a UE may possibly support multiple services, the configuration of multiple CG configurations for an RRC_INACTIVE UE should be hence supported. 
On the other hand, it should be also considered that allowing to configure multiple CG-SDT configuration for one UE will require a large amount of PUSCH resources in particular in the context of multi-beam operation. It should be noted that the CG-SDT resources may be configured on the initial UL BWP.

Question 12: Do companies see a need to support configuring multiple CG-SDT configurations per carrier in RRC_INACTIVE?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y | It should be up to the judgement of the network whether to configure multiple CG resources or not on one UL carrier. On the other hand, the network should have such flexibility. Multiple CG configurations have been supported In R16. We think the multiple CG configurations in R17 small data should follow R16 as the baseline. FFS the maximum number of CG configurations. |
| ZTE | N | We think a single CG-SDT resource can be configured and this is enough (note that the intention is already to support the CG-SDT resources to be associated with multiple SSBs or multiple carriers – UL and SUL – i.e. in the end more than one CG-SDT resource can be configured to the UE, but having multiple CG-SDT per the same carrier and same SSB seems not necessary in our view). The CG-SDT resource can be used to aggregate the data generated by multiple applications with different periodicities etc (since the use case should be for low priority and infrequent data anyway, using single CG-SDT resource as an aggregator for all the data generated within this period seems to be sufficient). If there is a strong majority to configure multiple CG-SDT resources, we have to consider how to count the invalidation of these when these are not used (i.e. whether the invalidation count runs across all the CG-SDT resources or whether it is per CG-SDT resource etc). This creates a bit more complexity in our view and needs further discussion. |

# Bandwidth Part

According to the Rel16, CG resources are configured per BWP, i.e. the CG resources are valid in the BWP in which the corresponding CG configuration is received. For UEs in RRC_INACTIVE, the UE only has a valid RACH configuration on initial BWP to perform RACH procedure. Since the CG-SDT configuration for UE in INACTIVE is provided by the RRCRelease message, UE needs to know the BWP associated with the CG-SDT resources.

Two options have been considered by companies:

- Option 1: The BWP associated with CG-SDT resources is initial UL BWP

- Option 2: The BWP associated with CG-SDT resources is configurable, 
       e.g. UE specific dedicated UL BWP.

It is argued by companies in favour of option 2 that option2 is more flexible in terms of allocating CG resources as different UE(s) can be allocated resources in different BWPs of cell. However, it should be noted that option 2 would require that UE supports UL transmissions in RRC_INACTIVE also in non-initial UL BWP, which would increase the UE complexity.

Question 13: Which of the above options do you prefer for the UL BWP used for CG-SDT resources?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option2 | For Option1, even if we may transmit CG on initial UL BWP, it is still configured under the dedicated configuration of the initial BWP since CG configuration should be under BWP-UplinkDedicated. For Option2, different from RACH-based scheme, for CG there is no issue of context relocation since the CG resource is only valid on the cell where the UE receives RRCRelease with suspendConfig. While for the argument from the rapporteur, we are not sure why UE complexity increases when it transmits on non-initial BWP. UE’s support for UE-specific dedicated BWP is mandatory without capability signalling as FG 6-1. Configuration of BWP in addition to initial UL BWP increase the flexibility of CG configuration, e.g., when the time and frequency resource of the initial UL BWP is insufficient, which we think is beneficial. |
| ZTE | Option 2 | We think configurable BWP in UL and DL is good and works for this case since the UE resources are dedicated and are cell specific. Having this possibility also alleviates the congestion on initial BWP. |

For Option 2, i.e. CG-SDT resources can be configured on a UE specific dedicated UL BWP, the UE needs to be explicitly informed about the BWP, e.g. BWP ID, associated with the CG-SDT. It would be straightforward to include such information within the RRCRelease message. It needs to be also further discussed, whether only the UL BWP configuration for the CG-SDT resources needs to be explicitly configured to a UE in RRC _INACTIVE, i.e. DL BWP for the response messages is the initial DL BWP. It may be simpler if the DL BWP used for SDT is the initial DL BWP as UE may need to monitor initial DL BWP anyway for e.g. paging. Companies are invited to express their opinion the DL BWP.

Question 14: In case of Option 2, do companies agree that UL BWP associated with the CG-SDT resources is signalled within in the RRCRelease message?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Y, but it depends. | It depends on the above question no whether we allow to perform delta configuration for ConfiguredGrantConfig in RRC_CONNECTED for CG-based scheme in RRC_INACTIVE. If it is supported, the UL BWP associated with the CG-SDT resource in RRC_INACTIVE can also be signaled by RRCReconfiguration. |
| ZTE | Y | We think RRCRelease is enough per our explanation above. We think just the BWP ID will be signaled in case the BWP for CONNECTED mode can be reused for SDT and we also think the configuration can also include the BWP ID for both UL and DL – since DL is also applicable for subsequent data transmission phase (i.e. this is not just for UL BWP). |

# Conclusion

RAN2 should discuss the above and agree to the following:

[TBA]

# References

[1]   R2-2010007	Discussion on CG based NR small data transmission	Qualcomm Incorporated

[2]   R2-2009057	CG-based SDT  	MediaTek Inc.

[3]   R2-2008995	Handling of Configured grant for SDT	Intel Corporation

[4]   R2-2008961	Supporting Small Data Transmission via CG configuration	vivo

[5]   R2-2009015	Discussion on CG based small data transmission	OPPO

[6]   R2-2009094	Configured Grant based Small Data Transmission	Samsung Electronics Co., Ltd

[7]   R2-2009192	Configured grant based small data transmission	ZTE Corporation, Sanechips

[8]   R2-2009345	SDT handling in RRC_INACTIVE state	ETRI	discussion

[9]   R2-2009350	Discussion on aspects specific to CG based SDT	Nokia, Nokia Shanghai Bell

[10]  R2-2009369	Analysis on SDT Procedures using CG	CATT

[11]  R2-2009458	Coexistence of CG and RACH configuraiton for SDT	LG Electronics Inc.

[12]  R2-2009459	CG resources for Small Data Transmission	LG Electronics Inc.

[13]  R2-2009493	CG based SDT procedure	Apple	discussion

[14]  R2-2009649	TAT maintenance for CG based SDT	ITL

[15]  R2-2009874	Consideration on CG based small data transmission	Lenovo, Motorola Mobilit

[16]  R2-2009890	Details of CG-based schemes for SDT in NR	Sony

[17]  R2-2009964	Details of CG based SDT	Ericsson

[18]  R2-2009973	Discussion on CG-based Small Data Transmissions	NEC Telecom MODUS Lt

[19]  R2-2010107	CG-based SDT selection and configuration	InterDigital

[20]  R2-2010108	Beam selection and maintenance for CG-based SDT	InterDigital

[21]  R2-2010391	Consideration on CG based SDT	CMCC

[22]  R2-2010432	Association between Pre-configured PUSCH resources and beam	ASUSTeK

# Annex (contact details for email discussions)

| Company | Contact name | Contact email |
| --- | --- | --- |
| Huawei, HiSilicon | Yinghao Guo | yinghaoguo@huawei.com |