3GPP TSG RAN WG2 Meeting #113bis-e		R2-210xxxx

Electronic, 12th Apr – 20th Apr 2021

Title:	[Draft] LS on multiple TACs per PLMN

Response to:

Release:	Release 17

Work Item:	NR_NTN_solutions-Core

Source:	Huawei [to be RAN2]

To:	CT1, SA2

Cc:	RAN3

Contact person:	Xun TANG

tangxun (at) huawei (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 has discussed the tracking area update procedure and has made the following agreements:

- The network may broadcast more than one TACs per PLMN in a cell

- When the network stops broadcasting a TAC, the UE needs to know it

- RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area

- RAN2 confirm that in NTN when TAC change in SI happens is up to network implementation, i.e. it may not exactly sync up with real-time illumination on ground

To follow the principle “the AS shall report tracking area information to the NAS” specified in TS 38.304, RAN2 has discussed the following two options:

- Option 1: AS still reports only one TAC for one PLMN even if more than one TACs per PLMN are broadcasted in an NTN cell.

- Option 2: AS indicates all received TAC(s) for one PLMN to NAS layer.

RAN2 would like to inform CT1 that some companies think option 1 has a larger NAS impact in which case RAN2’s preference would be option 2.

RAN2 would also like to ask the following question to CT1:

- Is option 2, i.e. “AS indicates all received TAC(s) for one PLMN to NAS layer”, feasible from a NAS layer perspective?

# 2	Actions

To CT1:

ACTION: CT1 is kindly requested to take the above information into account and provide feedback.

To SA2:

ACTION: SA2 is kindly requested to take the above information into account.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #114-e	19 May – 27 May 2021, Electronic