3GPP TSG-RAN WG1 Meeting #115	R2-21xxxxx

Electronic, August 9th – 27th, 2021

Agenda:	8.2.3

Source:	Ericsson

Title:	Report for Inter-node message design

Document for:	Discussion, Decision

# 1	Introduction

This document contains the report for discussion on details of inter-node messages for CPAC, related to the following e-mail discussion:

[Post115-e][216][R17 DCCA] Inter-node message design (Ericsson)

Scope: Discuss details of inter-node messages for CPAC and provide draft CR of the resulting option(s).

Intended outcome: Draft CR

Deadline:  Long

# 2	Discussion

## 2.1	Inter-node messages for CPAC

The impact to the inter-node signalling for the Conditional PSCell Addition and Change (CPAC) procedures, such as how to transfer information related to more than one PSCell candidate in a single procedure, has been discussed at the latest RAN2 meetings. The following agreements have been reached:

RAN2#114-e

1: In order to exchange per-PSCell parameter by reusing existing inter-node RRC message for CPAC, a list of CG-Config associated to each candidate PSCell should be sent from candidate SN to MN.

FFS if a list of CG-ConfigInfo from MN to candidate SN is needed. FFS if a list of CG-Config from source SN to MN is needed.

Discuss in Stage-3 whether new message is useful or not (based on signalling details)

RAN2#115-e

6	The inter-node signalling from (at least) target SN to MN for CPAC procedures only includes a single container (FFS which IE), even if several PSCell candidates are provided.

This e-mail discussion assesses the details of the inter-node message for CPAC on the different impacted interfaces, i.e.:

- from target SN to MN;

- from MN to target SN;

- from source SN to MN; and

- from MN to source SN.

### 2.1.1	Target SN to MN inter-node signalling for CPAC

On the interface from the target SN to the MN, the inter-node message should include the per candidate PSCell configuration in the XnAP S-NODE ADDITION REQUEST ACKNOWLEDGE message. Today the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains a single RRC container that includes the CG-Config:

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| [...] |  |  |  |  |  |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| [...] |  |  |  |  |  |  |

It has been agreed in RAN2 that “…a list of CG-Config associated to each candidate PSCell should be sent from candidate SN to MN” and that “The inter-node signalling from (at least) target SN to MN for CPAC procedures only includes a single container (FFS which IE), even if several PSCell candidates are provided”.

The S-NODE ADDITION REQUEST ACKNOWLEDGE message should thus include a list of CG-Config within a single S-NG-RAN node to M-NG-RAN node container. The following solutions have then been proposed in contributions R2-2108112 and R2-2105988 for this:

- a) To extend the existing CG-Config message to include the list of additional CG-Config(s)

- b) To introduce a new RRC message that includes the full list CG-Config(s)

In solution a) an optional list of additional CG-Config(s) is added to the CG-Config message, i.e. so that the existing message includes a first PSCell candidate and the list includes CG-Config(s) for the other PSCell candidate(s), if any. At the RAN2#115-e meeting it was commented that there should be no empty SEQUENCE in the middle of the message, which would be the case if there is a CG-Config (which has an empty CG-Config at the end) included directly within another message. This issue exists for both solution a) and b) and can be handled by including the additional CG-Config(s) as an OCTET STRING, similar to how e.g. the condRRCReconfig includes an additional RRCReconfiguration in legacy. The implementation of solution a) in 38.331 could look like this:

***************************************************************************************************************************

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                           CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

[…skipped parts…]

CG-Config-v1640-IEs ::=             SEQUENCE {

servCellInfoListSCG-NR-r16          ServCellInfoListSCG-NR-r16                      OPTIONAL,

servCellInfoListSCG-EUTRA-r16       ServCellInfoListSCG-EUTRA-r16                   OPTIONAL,

nonCriticalExtension            SEQUENCE {}CG-Config-r17-IEs                                         OPTIONAL

}

CG-Config-r17-IEs ::=             SEQUENCE {

additionalCG-ConfigList-r17         SEQUENCE (SIZE (1..FFS)) OF AdditionalCG-Config-r17 OPTIONAL,

nonCriticalExtension                SEQUENCE {}                     OPTIONAL

}

AdditionalCG-Config-r17 ::= OCTET STRING (CONTAINING CG-Config)

***************************************************************************************************************************

In solution b) a new inter-node RRC message, e.g. called “CG-CandidateList” is introduced to include the full list of CG-Config(s) for all the candidate PSCells. There is then however a need to update TS 38.423 to include that the RRC container in the S-NODE ADDITION REQUEST ACKNOWLEDGE message may include two different RRC messages (CG-Config or “CG-CandidateList”).

The implementation of solution b) in 38.331 and 38.423, respectively, could look like this:

***************************************************************************************************************************

38.331:

CG-CandidateList message

-- ASN1START

-- TAG-CG-CANDIDATELIST-START

CG-CandidateList ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-CandidateList                       CG-CandidateList-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-CandidateList-IEs ::=     SEQUENCE {

cg-CandidateList-r17         SEQUENCE (SIZE (1..FFS)) OF OCTET STRING (CONTAINING CG-Config)  OPTIONAL,

nonCriticalExtension            SEQUENCE {}                     OPTIONAL

}

-- TAG-CG-CANDIDATELIST-STOP

-- ASN1STOP

38.423:

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| [...] |  |  |  |  |  |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| [...] |  |  |  |  |  |  |

***************************************************************************************************************************

Question 1: Which solution do you think is most suitable for inclusion of multiple PSCell candidates in a single container from target SN to MN?

| Company | Solution (a or b) | Comments |
| --- | --- | --- |
| CATT | Solution c is prefered, solution b with modification is acceptable | Solution c is prefered. RAN3 has already agreed to include a list of accepted PSCells and the corresponding CG-Config within the SN/SgNB Addition Request ACK, please refer RAN3 agreed CR R3-212968 and R3-212969 during RAN3#112 meeting. Take SN Addition Request ACK message as an example, the S-NODE ADDITION REQUEST ACKNOWLEDGE message >Candidate PSCell ID List 1 >>Candidate PSCell ID Item 1 .. <maxnoofPSCellCandidate> >>>PSCell ID M Target Cell Global ID >>>RRC Container M OCTET STRING We think it is better to provide the candidate PSCells accepted by T-SN in parallel to the MN, which is clearer. The nested structure in solution a is not preferred. Solution b can be accepted with modification. Considering the MN should make the association of the execution condition and the candidate PSCell configuration, we think it is better to indicate the accepted PSCell explicitly which should be added in parallel with the container CG-ConfigInfo. This is to avoid the MN to decode the CG-ConfigInfo to obtain the PSCell ID information. e.g. CG-CandidateList-IEs ::= SEQUENCE { cg-CandidateList-r17 SEQUENCE (SIZE (1..FFS)) OF CG-CandidateInfo-r17 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } CG-CandidateInfo-r17 ::= SEQUENCE { ssbFrequency ARFCN-ValueNR, physCellId PhysCellId, candidateCG-Config OCTET STRING (CONTAINING CG-Config) } |
| Qualcomm | Solution b) | Solution b) seems to be cleaner and simpler. |
| ZTE | Both solution a) and b) with modification | We also think it’s better to explicitly indicate the accepted PSCell to make the MN associate the execution condition and candidate PSCell configuration easier. And considering the current CG-Config-IEs have indicate pSCellFrequency, we think it’s simpler to just introduce an new IE for physCellId in CG-Config-IEs to avoid defining the redundant PSCell frequency information. e.g. CG-Config-v17xy-IEs ::= SEQUENCE { physCellId PhysCellId, OPTIONAL nonCriticalExtension SEQUENCE {} OPTIONAL } With the modification above, we are fine to both solution a) and b). |
| NEC | Solution b with modification | Solution b with modification by adding the accepted candidate cell explicitly linked to corresponding PSCell config, as commented by CATT. |
| Lenovo, Motorola Mobility | Solution b | Solution b seems cleaner compared to solution a. What CATT proposed to include PSCell ID explicitly in parallel with the container CG-ConfigInfo is also fine to us. |
| Nokia | a) | We prefer a new CG-Config for Rel-17 which shall contain a list of CG-Configs. The exact IE names are to be discussed (not sure if the ‘‘Additional‘‘ in the name is required). It has to be also noted that MN may not be able to match each CG-Config with each target PSCell without decoding the RRC configuration provided for each cell. To avoid such complexity, an ordered list of cell identifiers should be sent outside of CG-Config. Alternatively, RAN2 can change its decision to use a single container for all configurations and support having separate, per each candidate target PSCell. |

### 2.1.2	Source SN to MN inter-node signalling for CPAC

At an SN initiated inter-SN CPC the S-SN sends information about the proposed PSCell candidate(s) and about execution conditions to the MN. The information can be sent in the S-NODE CHANGE REQUIRED message and/or the S-NODE MODIFICATION REQUIRED message, depending on solution chosen.

Both these messages include a container that consists of the CG-Config message. The CG-Config already includes the candidateCellInfoListSN, which contains information (such as measurement results) about cells that the S-SN suggests the target SN to consider. In the SN initiated inter-SN CPC case, the source SN needs to explicitly indicate what candidate PSCell(s) that the target SN is allowed to configure, but it should also be possible for the source SN to provide measurement information for other cells. One proposal in R2-2108112 has been to extend the candidateCellInfoListSN to include also execution conditions (for the candidate PSCells). However, since the candidateCellInfoListSN consists of MeasResultList2NR, which is used also in signalling by the UE, one should avoid introducing changes there due to inter-node signalling.

The list of proposed candidate PSCells as well as execution conditions could instead be introduced as a new list in CG-Config.

A text proposal for implementation of such a new list in 38.331 could look like this:

Option a:

***************************************************************************************************************************

CG-Config-IEs ::=                   SEQUENCE {

[…]

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

[…]

}

CG-Config-v1640-IEs ::=             SEQUENCE {

servCellInfoListSCG-NR-r16          ServCellInfoListSCG-NR-r16                      OPTIONAL,

servCellInfoListSCG-EUTRA-r16       ServCellInfoListSCG-EUTRA-r16                   OPTIONAL,

nonCriticalExtension            SEQUENCE {}CG-Config-v17xy-IEs                                         OPTIONAL

}

CG-Config-v17xy-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17        CandidateCellInfoListCPC-r17						OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

CandidateCellInfoListCPC-r17 ::=    SEQUENCE (SIZE (1..FFS)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                   OPTIONAL,

candidateList-r17	                SEQUENCE (SIZE (1..FFS)) OF CandidateList-r17 	OPTIONAL

}

CandidateList-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                         OPTIONAL,

condExecutionCond-SCG-r17              SEQUENCE (SIZE (1..2)) OF MeasId 				  OPTIONAL

}

***************************************************************************************************************************

Option b: Considering the execution condition doesn’t need to be transferred to the T-SN, add the explicit information “candidateCellList” to indicate the recommended cells by the S-SN in parallel with the candidateCellInfoListCPC-r17.  This way the MN can directly transfer the list of the recommended cells by the S-SN to the T-SN, without decoding candidateCellInfoListCPC-r17 to generate a new list of candidate cells recommended by the S-SN. And the candidateCellInfoListCPC-r17 can be kept by the MN.

***************************************************************************************************************************

CG-Config-v17xy-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17        CandidateCellInfoListCPC-r17						OPTIONAL,

candidateCellList-r17                 CandidateCellList-r17						OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

CandidateCellInfoListCPC-r17 ::=    SEQUENCE (SIZE (1..FFS)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                   OPTIONAL,

candidateList-r17	                SEQUENCE (SIZE (1..FFS)) OF Candidate-r17 	OPTIONAL

}

Candidate-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                         OPTIONAL,

condExecutionCond-r17            OCTET STRING (CONTAINING CondReconfigExecCond-r17) OPTIONAL

}

CandidateCellList-r17::=           	    SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                   OPTIONAL,

physCellId-r17                      PhysCellId                                         OPTIONAL

}

CondReconfigExecCond-r17   ::=     SEQUENCE (SIZE (1..2)) OF MeasId

***************************************************************************************************************************

Option c: Similar with option b, but the candidate cells recommended by the S-SN (the candidateCellList in option b) and transferred to the T-SN is indicated by bit mapping like method, where each bit of the bit string represents the corresponding cell within the candidateCellInfoListSN.  Value 1 means the corresponding cell is a candidate cell, otherwise it is not. Same as option b, another parallel field (candidateCellInfoListCPC) will be kept by the MN carrying the execution condition and the associated candidate PScell ID.

CG-Config-v17xy-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17          CandidateCellInfoListCPC-r17						OPTIONAL,

candidateCellList-r17                   OCTET STRING (CONTAINING CandidateCellList-r17)              OPTIONAL,

nonCriticalExtension                         SEQUENCY{}            OPTIONAL

}

CandidateCellInfoListCPC-r17 ::=    SEQUENCE (SIZE (1..FFS)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                   OPTIONAL,

candidateList-r17	                SEQUENCE (SIZE (1..FFS)) OF Candidate-r17 	OPTIONAL

}

Candidate-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                         OPTIONAL,

condExecutionCond-r17            OCTET STRING (CONTAINING CondReconfigExecCond-r17) OPTIONAL

}

CandidateCellList-r17  ::=  BIT STRING (SIZE (1..64))

CondReconfigExecCond-r17   ::=     SEQUENCE (SIZE (1..2)) OF MeasId

Option d:based on option a, additionally define separate candidateCPCCellInfoListSN to indicate the candidate cells recommended by the S-SN. The candidateCPCCellInfoListSN reuses the IE MeasResultList2NR, only the measurement of the candidate PSCell recommended by the S-SN are included. The original candidateCellInfoListSN doesn’t include the measurement of the candidate PSCell recommended by the S-SN.

CG-Config-IEs ::=                   SEQUENCE {

[……]

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

[……]

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1640-IEs ::=             SEQUENCE {

servCellInfoListSCG-NR-r16          ServCellInfoListSCG-NR-r16                      OPTIONAL,

servCellInfoListSCG-EUTRA-r16       ServCellInfoListSCG-EUTRA-r16                   OPTIONAL,

nonCriticalExtension            CG-Config-v17xy-IEs                                         OPTIONAL

}

CG-Config-v17xy-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17        CandidateCellInfoListCPC-r17						OPTIONAL,

candidateCPCCellInfoListSN-r17     OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

nonCriticalExtension                         SEQUENCY{}            OPTIONAL

}

CandidateCellInfoListCPC-r17 ::=    SEQUENCE (SIZE (1..FFS)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                   OPTIONAL,

candidateList-r17	                SEQUENCE (SIZE (1..FFS)) OF Candidate-r17 	OPTIONAL

}

Candidate-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                         OPTIONAL,

condExecutionCond-r17            OCTET STRING (CONTAINING CondReconfigExecCond-r17) OPTIONAL

}

CondReconfigExecCond-r17   ::=     SEQUENCE (SIZE (1..2)) OF MeasId

Option e:based on option a, additionally add an indicator under the candidateCellInfoListSN (i.e. in MeasResultNR) to indicate whether the related cell is a CPC candidate cell or not.

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

[…]

[[

cpcCandidate-r17              ENUMERATED {true}              OPTIONAL,

]],

}

Question 2: Do you agree that a separate list of proposed PSCell candidates, including optional execution conditions, should be introduced in CG-Config according to the text proposal above?

| Company | Comments |
| --- | --- |
| CATT | Agree with a seperate list of proposed PSCell candidates in addition to the candidateCellInfoListSN, as well as the execution condition. As for “optional execution conditions”, if it means the execution condition should be optional present even if the list of proposed PSCell candidates is present? If our understanding is right, then we think it depends on whether the solution 2 is agreed (now is a work assumption), and when to provide the execution condition even if solution 2 is agreed, i.e., step 1 or step 3. All of these are FFS at the moment. As for the detailed signalling design, please see the added solution b/c/d. In general, we think that the information transefered from thew S-SN to thye MN can be divided into 2 parts: - information required by the T-SN: One part that need to be transfered to the T-SN, including the candidate cells recommended by the S-SN - information required by the MN: information used by the MN, including the execution condition and the associated candidated PSCell ID It is better to define the 2 fields seperately, so when the MN receives the information from the S-SN, the MN can keep the “information required by the MN” itself and transfer the “information required by the T-SN” to the T-SN without additional decoding and re-generation for signalling. Like the option b,c, d the field of candidateCellInfoListCPC-r17 is the “information required by the MN“ which should be kept by the MN. The field of candidateCellList in option b,c and the field of “candidateCPCCellInfoListSN” in solution d are the “information required by the T-SN” which can be transfered to the T-SN by the MN directly. |
| Qualcomm | Agree. |
| ZTE | - We agree with that a separate list for the candidate PSCell and the corresponding execution condition (e.g. CandidateCellInfoListCPC-r17) is needed regardless of which solution is adopted. The main difference is in which step the list can be provided? E.g. the list must be provided in step 1 for solution 1, while it may be not provided in step 1 (i.e. only in step 3) for solution 2. - For the list of suggested candidate PSCell (no execution conditions) provided from the source SN to the MN and from the MN to the target SN, we think whether it’s needed depend on which solution is agreed. For solution 1, since the execution conditions should be provided in step 1, the suggested candidate PSCells are always involved. And the MN should forward the suggested candidate PSCells to the target SN for the PSCell selection. For solution 2, if the execution conditions are only provided in step 3, it seems no much gain to just provide a suggested candidate PSCell list to the target SN through the MN. Anyway, the target SN can select the candidate PSCell from the candidateCellInfoListSN as the legacy PSCell change procedure. And the source SN can provide the corresponding execution conditions after knowing the accepted candidate PSCells. - We share the same view with CATT that the detailed signalling design can be divided into two parts: information required by the MN and information required by the target SN. And considering it’s agreed that the MN performs the association between the execution conditions and candidate PSCell configuration, it seems no much need to transfer the execution conditions from the MN to the target SN, i.e. the suggested candidate PSCell list is enough in same cases. If the suggested candidate PSCell list to the target SN is required (e.g. in solution 1), we prefer to combine this with the candidateCellInfoListSN (anyway it shall be transferred to the target SN), to avoid providing redundant cell information. So the option c or option e like solutions can be considered. |
| NEC | Agree that a separate list of proposed PSCell candidates should be introduced. Regarding the example structure, we think the „condExecutionCond“ should be added in OCTET STRING which can be transparent to the MN. Among the options so far, we prefer to take the option b as baseline. |
| Lenovo, Motorola Mobility | Agree a separate list of proposed PSCell candidateas should be introduced. We also agree with analysis by CATT and ZTE, whether the execution conditions are optional presented and in which step depends on whether option 1 or option 2 will be taken. |
| Nokia | Yes. We are fine with the optional list of execution conditions. The structure shown above looks correct to us. However, if signalling the conditions is optional in SN change required, then it shall be provided to the MN in the message sent from S-SN when it is informed which PSCells have been accepted. |

RAN2 made the agreement:

The MN does not need to comprehend the execution condition set by the source SN. The MN can associate the execution condition configuration to an RRCReconfiguration message provided by the target –SN without comprehending the execution condition set by the source SN.

The agreement says that the MN does not need to comprehend the execution conditions, but it does not forbid the MN to comprehend the execution conditions either. It can be seen that it causes extra overhead to make the execution conditions non-visible to the MN. Such an implementation could look like:

CandidateList-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                         OPTIONAL,

condExecutionCond-r17              OCTET STRING (CONTAINING CondReconfigExecCond-r17) OPTIONAL

}

CondReconfigExecCond-r17 ::= SEQUENCE (SIZE (1..2)) OF MeasId

The RRC rapporteur questions the use of OCTET STRING in this case, as the conditions are just a list of integers. Normally OCTET STRING is used for messages or configuration of subsections/groups in order to get future extensions included, but here it is just a matter of integers. The reason for using OCTET STRING seems unclear here.

Question 3: Do you think the execution conditions should be added within an OCTET STRING in the inter-node message?

| Company | Comments |
| --- | --- |
| CATT | Yes, since for SN initiated inter-SN CPC, the execution condition is generated by the S-SN, the MN just needs to include the received execution condition generated by the S-SN directly within the CG-Config, The MN does not need to comprehend the execution conditions. |
| Qualcomm | Agree. the OCTET STRING does not seem to be needed as there is no need to explicitly try to make the execution conditions not visible to the MN. |
| ZTE | Yes, since the MN is not required to comprehend the execution conditions and measurements set by the source SN, we prefer to add the execution conditions as an OCTET STRING. |
| NEC | Yes. we can simply go with the legacy way, although we can see the point raised by Rapporteur (but no need to take different way). |
| Lenovo, Motorola Mobility | Yes |
| Nokia | No need to define it using OCTET STRING, i.e. we agree with the rapporteur it can be done simpler, with measurement identifiers. |

### 2.1.3	MN to target SN inter-node signalling for CPAC

The list of target candidate PSCell(s) that the MN received from S-SN should be sent from MN to T-SN. The easiest option is to reuse the same list in the CG-ConfigInfo container to T-SN as was used from S-SN to MN in CG-Config as discussed in 2.1.2. This is similar to how CG-ConfigInfo and CG-Config both include the candidateCellInfoListSN, which contains information (such as measurement results) about cells that the S-SN suggests the target SN to consider. A corresponding TP for including a list of proposed target candidate PSCells in CG-ConfigInfo is shown below.

A question is whether the MN is allowed to include the execution conditions also in CG-ConfigInfo. It may be easier for the MN to include the same list as was received from S-SN, but it also means some extra information in the signalling. The execution conditions could be optionally included by the MN.

CG-ConfigInfo ::=               SEQUENCE {

[…]

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

[…]

CG-ConfigInfo-v1640-IEs ::=             SEQUENCE {

servCellInfoListMCG-NR-r16              ServCellInfoListMCG-NR-r16                   OPTIONAL,

servCellInfoListMCG-EUTRA-r16           ServCellInfoListMCG-EUTRA-r16                OPTIONAL,

nonCriticalExtension                    SEQUENCE {}CG-ConfigInfo-r17-IEs                                         OPTIONAL

}

CG-ConfigInfo-v17xy-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17            CandidateCellInfoListCPC-r17			  	 OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                  OPTIONAL

}

CandidateCellInfoListCPC-r17 ::=   SEQUENCE (SIZE (1..FFS)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR                                    OPTIONAL,

candidateList-r17                   SEQUENCE (SIZE (1..FFS)) OF CandidateList-r17 	 OPTIONAL

}

CandidateList-r17::=           	    SEQUENCE {

physCellId-r17                     PhysCellId                                        OPTIONAL,

condExecutionCond-r17              SEQUENCE (SIZE (1..2)) OF MeasId                  OPTIONAL

}

Option b: similar with the option b/c in 2.1.2, define separate field by OCTET STRING to carry the information received from the S-SN, to indicate the candidate cells recommended by the S-SN. And define a separate field to indicate the candidate cells recommend by the MN

- CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

criticalExtensions              CHOICE {

c1                              CHOICE{

cg-ConfigInfo               CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=           SEQUENCE {

[……]

candidateCellInfoListMN         MeasResultList2NR                                                 OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

[……]

}

[……]

CG-ConfigInfo-v1640-IEs ::=             SEQUENCE {

servCellInfoListMCG-NR-r16              ServCellInfoListMCG-NR-r16                   OPTIONAL,

servCellInfoListMCG-EUTRA-r16           ServCellInfoListMCG-EUTRA-r16                OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v17xy-IEs                                  OPTIONAL

}

Option b:

CG-ConfigInfo-v17xy-IEs ::=             SEQUENCE {

candidateCellListSN-r17                   OCTET STRING (CONTAINING CandidateCellList-r17)      OPTIONAL,

candidateCellListMN-r17                   CandidateCellList-r17              OPTIONAL,

nonCriticalExtension                         SEQUENCY{}

}

[……]

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

Option c: similar with the option d in 2.1.2, define separate field reuse the structure of candidateCellInfoListSN/candidateCellInfoListMN, to indicate the candidate cells recommended by S-SN/MN..

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

criticalExtensions              CHOICE {

c1                              CHOICE{

cg-ConfigInfo               CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=           SEQUENCE {

[……]

candidateCellInfoListMN         MeasResultList2NR                                                 OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

[……]

}

[……]

CG-ConfigInfo-v1640-IEs ::=             SEQUENCE {

servCellInfoListMCG-NR-r16              ServCellInfoListMCG-NR-r16                   OPTIONAL,

servCellInfoListMCG-EUTRA-r16           ServCellInfoListMCG-EUTRA-r16                OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v17xy-IEs                                  OPTIONAL

}

CG-ConfigInfo-v17xy-IEs ::=             SEQUENCE {

candidateCPACCellInfoListMN-r17        MeasResultList2NR                OPTIONAL,

candidateCPCCellInfoListSN-r17         OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

nonCriticalExtension                      SEQUENCY{}            OPTIONAL

}

Question 4: Do you agree that the same list of proposed PSCell candidates should be introduced in CG-ConfigInfo as in CG-Config? Do you think the execution conditions could be included in CG-ConfigInfo?

| Company | Comments |
| --- | --- |
| CATT | As for the issue on whethe to indicate the execution condition to T-SN: - For MN initiated inter-SN CPC, it is already an agreement that there is no need for the MN to indicate the execution condition to the T-SN. - For SN initiated inter-SN CPC, since we already agreed that it is the MN to make the association of the execution condition and the RRC Reconfigutation received from the T-SN, we think there is no need for the MN to indicate the execution condition to the T-SN. As for the ASN.1 signalling of the list of the proposed PSCell candidates, candidate options b/c is added. Considering for the CPC initiated by the SN, the candidate PSCell recommended by the S-SN is generated by the S-SN. Therefore, it is better to be OCTET STRING which means that the MN doesn’t need to decode and re-generate the candidate PSCell information. As for the CPAC initated by the MN, the candidate PSCell is recommended by the MN, the infomration of the candidate PSCell is generated by the MN. Therefore, the information of the candidate PSCell should be IE. Due to the 2 cases, we prefer to define 2 separte fields for SN initiated CPC and MN initated CPAC to carry the candidate PSCell information. |
| Qualcomm | Yes, the same list of proposed PSCell candidates should be in CG-ConfigInfo as in CG-Config. The execution conditions should not be included in CG-ConfigInfo because it seems that the target SN does not need them for any purpose. |
| ZTE | - For the execution conditions, we think it’s not required to be transferred to the target SN in both MN initiated CPA/CPC and SN initiated CPC considering the MN performs the association between the execution conditions and the candidate PSCell configuration. - For the list of proposed PSCell candidates, as our comment in Q2, it can be depend on which solution is adopted. If solution 1 is agreed, this is always needed. And we think the MN can simply transfer this information provided by the source SN to the the target SN, e.g. reusing option c or option e in 2.1.2. - For MN initiated CPA/CPC, we have not directly discussed whether a separate list of proposed PSCell candidates (different from candidateCellInfoListMN) is required, or we can simply reuse candidateCellInfoListMN as the legacy PSCell selection. We are open for this discussion. |
| NEC | Regarding the list of PSCell candidates, we understand the MN needs to understand the candidate cell so that the MN can send a CPC request for a proper target SN, unlike execution condition. So, the MN can use the similar list as the one receiveyd from the S-MN but no need to include the same list (i.e. as it is). Regarding the execution condtion, No, we do not think it is necessary. The MN associates the execution conditon to the configuration of the candidate cell. |
| Lenovo, Motorola Mobility | Agree that proposed PSCell candidates should be introduced in CG-ConfigInfo as in CG-Config, for both CPA, and MN/SN initiated CPC. Since RAN2 didnt agree that target SN shall understand the execution condition, we dont think execution conditions could be included in CG-ConfigInfo. |
| Nokia | Yes, it is easiest to use the same list as sent from S-SN to MN. We agree to specify the optional possibility to include measurement execution conditions in the message sent from MN to T-SN. |

### 2.1.4	MN to source SN inter-node signalling for CPAC

At an SN initiated inter-SN CPC the MN needs to send information to the S-SN about the accepted PSCell candidate(s), independent on if solution 1 and solution 2 is implemented. In both cases the S-SN needs the information in order to decide if a reconfiguration of e.g. the SCG measConfig is needed. The message that is sent to S-SN in legacy is S-NODE CHANGE CONFIRM. In this message there is currently no RRC container included and no information about selected target PSCell either. For SN initiated inter-SN CPC, either an RRC container could be added or a list of the selected target candidate PSCell(s) could be added directly in the XnAP message. Both options have RAN3 impact and should be consulted with RAN3.

Question 5: Do you have any comments on whether to add information about accepted target candidate PSCell(s) in an RRC container or directly in the XnAP message? Or whether just to consult RAN3 on this question?

| Company | Comments |
| --- | --- |
| CATT | The information can be included in the SN CHANGE CONFIRM message is based on which solutin is used (sloution 1 or solution 2). If solution 1 is adopted, we thnk RAN3’s agreements made in#113e meeting has already covered the question, i.e the information of the accpeted target candidate PSCells is added directly in the XnAP message,. RAN3#113e Signalling design of SN initiated inter-SN CPC - FFS: Introduce the RRC complete message for source SN in the SN Change Confirm message, in order to provide from MN to the source SN about the embedded RRC complete message for source SN, after confirmation of receiving CPAC configuration from the UE. - Introduce “List of Prepared PSCell IDs” in SN Change Confirm. if solution 2 is adopted, another new message from the MN to the S-SN is still needed, beside the SN CHANGE CONFIRM message. Then we should first decide which step to use the SN CHANGE CONFIRM message, before we discuss the content of the SN CHANGE CONFIRM message. |
| Qualcomm | It seems in the last RAN3 meeting (RAN3 #113-e) this has already been agreed: Introduce “List of Prepared PSCell IDs” in SN Change Confirm. It is likely that RAN3 has agreed to have this information directly in the XnAP message, but this can be confirmed. |
| ZTE | Based on RAN3#113e agreements, it seems RAN3 has decided to add the information of the accepted target candidate PSCells directly in the XnAP message. Depend on the selected solution (i.e. 1 or 2), it can be up to RAN3 further discussion and decision which message can be used to contain this information. |
| NEC | The list of accepted target candidate cells has been discussed in RAN3 and their assumption is to include such list directly in XnAP message. We think this can be the same for the MN to the S-SN. If RAN3 sees some issues, then they will inform us. |
| Lenovo, Motorola Mobility | Similar view as CATT |
| Nokia | How to include those cells within inter-node messages can be up to RAN3 to decide. |

# Conclusion

Based on the above, the following is proposed:

# 4	References