3GPP TSG-RAN2 Meeting #116e								R2-210

# Electronic, 1st– 12th December, 2021

Source: 		Email discussion Rapporteur (Huawei, HiSilicon)

Title: 	Summary of [Post115-e][507][SDT] MAC running CR update (Huawei) review issue list

Agenda item:	8.6.1

Document for: 	Discussion and Decision

General

This document contains the list of comments made during the review of the MAC CR for SDT in the email discussion [Post115-e][507][SDT] MAC running CR update (Huawei).

For the issue found in the draft CR under Please fill in the form according to the following:

- On the column of index, fill in an index with the company initial letter + discussion number + issue number by increasing order.

  - For example, for the discussion in Post114ePhaseI, for an issue from Huawei, HiSilicon, one can fill in “H (company initial letter) + 0 (discussion number for Post114e)+ 00 (Issue number)”=> H000

  - Please use 1 for Post115e

- On the column of brief description of the issue, as the name suggests, please give a description on the issue

- On the column of suggested change/company comment, please give the proposed change on the draft spec based on the description on the issue. Companies can also give comments on the proposed change in this column by adding a marking of [Company] in this column

- On the column of proposed way forward by rapporteur, please leave it empty at the time of email discussion. At the conclusion of the discussion, email discussion rapporteur would give a way forward according to the inputs from different companies on the issue.

On the section of “Any Other Clause”, if a certain issue is found under a Clause in the spec that has not been listed, please fill the issue in the form under this section.

Please edit the document in draft view (View -> Draft) to view the entire table.

Contacts

| Name | Company | Email address |
| --- | --- | --- |

Post115e

3.2	Definitions

| # | Brief description of the issue | Suggested change/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.2	Random Access Resource selection

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.2a	Random Access Resource selection for 2-step RA type

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.3	Random Access Preamble transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.3a	MSGA transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.4a	MSGB reception and contention resolution for 2-step random access

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.5	Contention Resolution

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.2	Maintenance of Uplink Time Alignment

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.3.1	DL Assignment reception

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.3.2.1	HARQ Entity

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

#### 5.3.2.2	HARQ process

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.1	UL Grant reception

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.2.1	HARQ Entity

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.2.2	HARQ process

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.4	Scheduling Request

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.6	Power Headroom Reporting

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.8.2	Uplink

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.8.2.x	Validation for CG-SDT

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.15	Bandwidth Part (BWP) operation

5.15.1	Downlink and Uplink

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.16	SUL operation

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.x	Small Data Transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

6.1.5a	MAC PDU (MSGB)

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

7	Variables and constants

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

Any Other Clause

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

Post114e

3.2	Definitions

| # | Brief description of the issue | Suggested change/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z000 | CG-SDT Configured Grant type 1-based Small Data Transmission Since SDT is also defined separately, we could avoid using the full expansion and use the SDT abbreviation here already. | CG-SDT Configured Grant type 1-based Small Data Transmission SDT | [Rapp] Corrected |
| Z001 | Same as Z000 for RA-SDT | RA-SDT Random Access-based Small Data Transmission SDT | [Rapp] Corrected |
| N000 | CG-SDT Configured Grant type 1-based Small Data Transmission Enough to say “Configured Grant-based SDT” without “type 1” since what CG type is supported is clear from the procedure and configuration and stage 2. Agree with ZTE001. | CG-SDT Configured Grant type 1-based Small Data Transmission SDT | [Rapp] Corrected |

5.1.1	Random Access procedure initialization

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z002 | prach-ConfigurationIndex These are also applicable to Msg1 in 4-step RA-SDT type if the PRACH occasions are shared between 4-step RA type and 4-step RA-SDT type. These are also applicable to the Random Access Preamble for MSGA in 2-step RA-SDT type if the PRACH occasions are shared between 4-step RA type and 2-step RA-SDT type General Comment: Do we really need to define new 4-step-RA-SDT type? With the above sentence, it seems we need to define “4-step RA-SDT type” and “2-step RA-SDT type”. However, since the RA type itself is not changed due to introduction of SDT. We could refer to existing RA types with and without SDT. Please see the suggested rewording. On the other hand if we do define a new RA type, perhaps this needs to be defined (e.g. in stage-2) etc. Also there will be other changes needed in MAC spec in other sections too in this case since we use checks such as “if RA_TYPE is set to 2-stepRA” etc elsewhere and we need to now redefine all these with new RA types etc. It would be preferable to avoid a new RA type if possible to avoid such changes. | - prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to Msg1 for RA-SDT if the PRACH occasions are shared between Random Access procedures with and without SDT for 4-step RA type. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types. These are also applicable to MSGA PRACH for RA-SDT if the PRACH occasions are shared between 4-step RA type and 2-step RA type with SDT. | [Rapp] Thanks for the comments @ ZTE. On the new RACH type, the main reasons that why it is introduced are that - In section 5.1.1a for initialization of parameters, I suspect certain parameters would be different from the legacy types of RACHs, e.g., preambleTransMax, etc. (but of course this is subject to further discussion) If such differences do exist, introducing a new RACH type to the UE variable RA_TYPE fits better with the current framework - For RACH resource selection, the procedure will for sure be different between SDT and nonSDT. For example, preamble group selection, RACH occasion selection (as the current running CR puts it), etc. The solution in R16 2-stepRACH was to introduce a new chapter, i.e., Clause 5.1.2a. But I think for SDT, we can use the existing chapters and then, use the new RACH type to differentiate the procedures for the SDT RACH and non-SDT RACH for 2-step RACH and 4-step RACH - In the previous meeting, we have agreed to allow for fallback from SDT to non-SDT. Introducing a new RACH type is compatible with the procedures in the above two sections I think we can keep the RACH type as it is for now and we can come back to this later to further examine its necessity. I have put an editor note here to mark it as FFS. |
| Z003 | msgA-PRACH-ConfigurationIndex Similar comment as Z002 (please see the corresponding suggestion). Further, it is not clear why these occasions should be shared with MSG1 in 4-step RA type with SDT as defined in the new definition. In case of shared occasions between 2-step and 4-step, these should be signalled via prach-CongurationIndex-SDT. | - msgA-PRACH-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type. These are also applicable to MSGA PRACH for RA-SDT if the PRACH occasions are shared between Random Access procedures with and without SDT for 2-step RA type. | [Rapp] For the previous agreement, I think it does not really forbid RACH occasion sharing between 2-step RACH and 4-step RACH with SDT. RAN2#112e 10: As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT - If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed. - If ROs for SDT and non SDT are same, preamble partitioning is needed FFS if common configuration should be allowed. I have put an editor note here to mark it as FFS. |
| Z004 | prach-ConfigurationIndex-SDT and msgA-PRACH-ConfigurationIndex-SDT Similar comment as Z002 | - prach-ConfigurationIndex-SDT: the available set of PRACH occasions for the transmission of the Random Aceess Preamble for Msg1 in 4-step RA type with SDT; - msgA-PRACH-ConfigurationIndex-SDT: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type with SDT; - sdt-MSGA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type with SDT and 4-step RA type with SDT when both 2-step and 4-step RA type Random Access Resources for SDT are configured in the UL BWP; | [Rapp] Ref to the previous comments |
| Z005 | Similar comments as Z002 apply also to the definitions of groupB-Configured-SDT and groupB-ConfiguredTwoStepRA-SDT |  | [Rapp] Ref to the previous comments |
| Z006 | 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]: 2> if the Random Access procedure was initiated for Small Data Transmission as specified in clause 5.x: 3> set the PCMAX to PCMAX,f,c of the selected UL carrier. 2> else if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 3> select the SUL carrier for performing Random Access procedure; 3> set the PCMAX to PCMAX,f,c of the SUL carrier. 2> else: 3> select the NUL carrier for performing Random Access procedure; 3> set the PCMAX to PCMAX,f,c of the NUL carrier. Comment: It seems we could simplify the changes a bit by existing condition about signalled carrier… Please see the proposed alternative. Both can work though, so no strong view. | 1> if the carrier to use for the Random Access procedure is explicitly signalled or determined as specified in subclause 5.x for SDT: 2> select the signalled or determined carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the selected carrier. 1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier. | [Rapp] I have adopted the proposed solution, which is quite concise, but may lack some readability on the other side. One issue that remains to be resolved is that for subsequent CG-SDT transmission, whether UL carrier selection needs to be performed again. The way the current spec is specified is to assume that (a) the UL carrier selection is only performed for initial CG transmission; (b) the RSRP threshold is the same between RA_SDT and CG_SDT. These issues need to be further addressed. However, if we finally agree that for subsequent CG-transmission, UL carrier selection needs to be done again and the threshold can be different between CG and RACH, it is better to move the carrier selection for SDT from subclause 5.x to RA and CG. From this aspect, it is better to keep the previous chunk of procedure as it is as suggested by ZTE I have also added the following Editor’s Note per discussion above. Editor’s Note: FFS whether UL carrier selection is performed for both initial and subsequent UL for CG-SDT and whether the RSRP threshold is common for both CG and RA-SDT. |
| Z100 | General comment to section 5.1.1: A number of changes to this section will likely overlap with similar changes coming from other WIs that require RACH partitioning. We need to understand how we could integrate these changes. For instance, the statements such as “These are also applicable to Msg1 for RA-SDT if the PRACH occasions are shared between Random Access procedures with and without SDT for 4-step RA type” etc which exist in this section may not be exclusive to this WI. i.e. these preambles or ROs may also be shared by other features requiring the RACH partitioning and such statement above may need to be updated to cover all such cases. We hence need a general discussion on how to combine these features. Perhaps we could even have to think about a common MAC CR for overlapping WIs in this case. Something we need to discuss further at the next meeting. |  | [Rapp] Agree with the observation from ZTE that this needs to be considered in conjunction with the other WIs that may proposed to introduce RACH changes in this release. And we also need to consider for forward compatibility in the future releases which may further increase the cases for RACH. The current way to capture the procedure does not quite seem to be forward-compatible. |
| N001 | The additions to prach-ConfigurationIndex and msgA-PRACH-ConfigurationIndex do not seem to be needed. | Remove the addition to prach-ConfigurationIndex and msgA-PRACH-ConfigurationIndex description. It should be made clear in RRC field description if anything needed. | [Rapp] This is to align with the similar description with the RACH resource sharing in 2-step RACH. If this is not technically wrong, I think we can keep it as it is. |
| N002 | In the Editor’s NOTE, we do not see why this is up to RAN1 to decide: “Editor’s NOTE: FFS whether there can be separate configurations for related to the configuration of Random Access Preambles group B for RA-SDT , which is up to RAN1 to decide.” | Remove “which is up to RAN1 to decide” or remove the Editor’s NOTE. | [Rapp] I can remove the note but keep in mind during R16 for the discussion in 2-step RACH, it is RAN1 that makes the decision to use mask index for RACH occasion sharing. It still might not be RAN2’s call to do that. Remove up to RAN1 to decide |
| N003 | We should not define terms 2/4-step SDT-RA type which makes the spec very confusing and is essentially not true since still we perform the same actions as with 2-step and 4-step RA. It should be noted that Stage-2 would not define such types and to keep the specs consistent, such terms shall not be defined. This is also inconsistent to what is said in 5.x: ”Small Data Transmission can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT)” Can just use, e.g., “2/4-step RA type for SDT” | Use ” 4-step RA type for SDT” and “2-step RA type for SDT” instead of defining new RA types which is not true. | [Rapp] Please see the replies to Z002. IMHO, without the RA type, the spec will look more confusing. Not clear to me what will be the difference between another RA type and RA type for SDT |

5.1.1a	Initialization of variables specific to Random Access type

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.2	Random Access Resource selection

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z007 | 1> else if an SSB is selected above: 2> if the selected RA type is set to 4-stepRA-SDT: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB). Comment: It is unclear why the highlighted part is needed. Isn’t the existing text sufficient? | Delete the newly added text | [Rapp] The reason is that - For RA for SDT, we don’t need to consider the measurement gap, since it is in RRC_INACTIVE - We don’t need to consider ra-ssb-OccasionMaskIndex either, since it cannot be CFRA We can add some conditions to the previous text to rule out the above cases for SDT, but I think a cleaner solution would be add a new sentence to dedicatedly address the case of RA-SDT |

5.1.2a	Random Access Resource selection for 2-step RA type

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z008 | 1> if the selected RA type is set to 2-stepRA-SDT: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB). 1> else: Same comment as Z007 |  | [Rapp] Ref to the above comment |
| Z101 | NOTE1: Based on the agreement in RAN2#113bis-e: “Switching from SDT to non-SDT is supported”. The agreement “switching from SDT to non-SDT is supported” doesn’t mean we will support fallback from SDT RACH resource to non-SDT RACH resource within one RACH procedure or PRACH retransmission attempt. The switching can be triggered e.g. by either a DCCH message or new CCCH procedure (FFS) and may also be triggered by network (e.g. by sending RRCResume etc). So, we are not sure if we need changes in this section and this note can be deleted. |  | [Rapp] I can remove this editor note and put it under issue list We have agreed on the following for the fallback 11 UE switches from SDT to non-SDT in following cases: - Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. - Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. - FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times In section 5.1.4, we have Editor’s Note: FFS RACH procedure trigger for SR for small data In sectin 5.1.4a, we have Editor’s Note: FFS fallback from 2-stepRA-SDT to 4-stepRA-SDT and non-SDT In section 5.1.5, we have Editor’s Note: FFS fallback from 2-stepRA-SDT to 4-stepRA-SDT So, these editor notes correspond to the case when the fallback happen for RACH re-transmission as you have mentioned. If fallback within one RACH procedure is not supported, these section will not be affected. |
| N004 | Agree with ZTE101. Not sure what is the intention of the Editors note on the supported fallbacks. We do not see this impact MAC as it should be rather a RRC procedure upon reception of the resume RRC msg from the NW: “NOTE1: Based on the agreement in RAN2#113bis-e: “Switching from SDT to non-SDT is supported”.” | Remove the NOTE. | [Rapp] Please see the reply above |

5.1.3	Random Access Preamble transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.3a	MSGA transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| L000 | We don't understand why "or for Scheduling Request in Small Data Transmission in clause 5.x" is included. | [LG] Remove the sentence | [Rapp] Thanks for the comments @LGE In the previous RAN2 meeting, we have agreed on the following for the SR for subsequent UL based on DG 6 SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA because SR resource is not available, same as legacy While different from the legacy RACH procedure in RRC_IDLE/INACTIVE, for SR in RRC_INACTIVE, the UE may not need to carry CCCH message and should include a RNTI, similar to the connected mode scenario. The reason why I made this change previously was I thought it is quite straightforward. I can remove this and put it under editor note |
| Z009 | We agree with L000 comment |  | [Rapp] See comments above. |
| N005 | Agree with L000 |  |  |

5.1.4a	MSGB reception and contention resolution for 2-step random access

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.5	Contention Resolution

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.1.6	Completion of the Random Access procedure

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.2	Maintenance of Uplink Time Alignment

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z010 | 1> when the cg-SDT-TimeAlignmentTimer expires: 2> notify RRC to release configured grant type 1 configuration(s) for Small Data Transmission. The notification should only be that the CG-TAT has expired or not running etc. In RRC the actions can be taken based on this indication (e.g. release the CG resources at the next RRC Resume or release it if there is an ongoing SDT etc)… | 1> when the cg-SDT-TimeAlignmentTimer expires: 2> notify RRC that the cg-SDT-TimeAlignmentTimer has expired. | [Rapp] Thanks for the comments. I am not quite sure why the RRC layer should release the source at the next RRC resume, since both the network and the UE are maintaining this timer and the network does not need another RRC resume to notify the network (Different from cell reselection in RRC_INACTIVE and RSRP change beyond a certain threshold?) Also, the legacy spec for PUCCH and SRS has directly indicated to the RRC layer to release the resource instead of indicating the expiry of the TAT. 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with the PTAG: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. |
| X001 | When the UE initiate the RACH procedure, the UE would receive the TAC from the Msg2. It is not clear how/whether the TAC from the Msg2 impacts the validation of the CG resource for SDT. | RAN2 should discuss whether the cg-SDT-TimeAlignmentTimer can be affected by any TAC. | [Rapp] Thanks for your comments @Xiaomi Agree with Xiaomi’s comment. I have added the following editor’s note Editor’s Note: FFS the interplay between the legacy TAT and cg-SDT-TAT when legacy RACH is initiated |

5.3.1	DL Assignment reception

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.3.2.1	HARQ Entity

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

#### 5.3.2.2	HARQ process

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z102 | 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and; 1> if the transmission for the HARQ process is initiated for CG-SDTand cg-SDT-TimeAlignmentTimer is stopped or expired: Comment: The interaction between the regular TAT and the cg-SDT-TimeAlignmentTimer is a bit unclear from the above. i.e.: - Is the UE considered to be time aligned only if both TAT and the cg-SDT-TimeAlignmentTimer are both running? The “and” in the above seems to suggest this but this is probably not the common understanding. - Also, if the above is true then we also need to understand the interaction between TAC and the cg-SDT-TimeAlignmentTimer. Further, the following agreement is not yet implemented: 5. TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command. Assuming that the CG-SDT-TAT can be restarted upon TA command, there seems to be no need for checking both regular TAT and CG-SDT-TAT for CG-SDT transmissions?? |  | [Rapp] This is also related to the comment above in X001, which has been addressed by the editor’s note. From my side, it seems that the legacy TAT is only applicable when legacy RACH is initiated during the RACH procedure and where the UE variable NTA should be kept independently. TAT can control whether PUSCH and PUCCH can be sent during RACH. When contention resolution is successful, the UE should stop the TAT, similar to the way we treat RACH for on-demand SI request. At this time, the NTA obtained by RACH can be applied to the NTA for CG-SDT. On the previous agreement on TAC, my previous thinking was that it should be further determined how this is conveyed to the UE, e.g., whether by DCI or MAC CE. I have put the following FFS for the TA command: Editor’s Note: FFS how the TA command is conveyed to the UE for cg-SDT-TAT |

5.4.1	UL Grant reception

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.2.1	HARQ Entity

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.2.2	HARQ process

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.4.4	Scheduling Request

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z011 | For a logical channel serving a radio bearer configured with SDT, no PUCCH resource for SR is configured. Comment: The above sentence is not needed and seems not correct in any case. Note that the RB will be the same in connected mode too (and in connected mode, the RB may be configured with SR resources). | Delete the sentence “For a logical channel serving a radio bearer configured with SDT, no PUCCH resource for SR is configured.” | [Rapp] This was based on the previous agreement, that no SR-PUCCH resource is configured for SR 6 SR resource is not configured for SDT. When the BSR is triggered by SDT data, the UE will trigger RA because SR resource is not available, same as legacy On how to treat the relationship between the connected mode configuration and the UE configuration in INACTIVE for SDT, the following note has been captured. Editor’s Note: How to handle the connected mode configuration in the RRC_INACTIVE UE context e.g., logical channel configuration. I think we need to further clarify that for SDT, the connected mode configuration is only kept in the UE context but not applied to the UE. INACTIVE mode UE for SDT can have a separate set of configurations. |
| N006 | Agree with Z011 |  | [Rapp] See the reply above |

5.8.2	Uplink

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z012 | When CG-SDT is triggered, the MAC entity shall: 1> if at least one of the SSBs with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available: 2> select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB; 2> select the configured grant type 1 configuration on BWP of the selected UL carrier associated with the selected SSB; 2> select the CG occasion corresponding to the selected SSB and the selected configured grant type 1 configuration. 1> else if RA-SDT is configured: 2> initiate Random Access procedure on the selected UL carrier for Small Data Transmission according to clause 5.1; 1> else: 2> initiate Random Access procedure in clause 5.1 for CCCH logical channel (i.e., not for Small Data Transmission). Comment: Firstly, switching between CG-SDT and RA-SDT has not yet been agreed. We only agreed that if none of the SSBs are above the threshold for initial CG transmission, then UE is not allowed to select any SSB. Instead, UE will select RA-SDT directly before transmitting the first initial message. However, since the initial UL message has not yet been sent, this doesn’t constitute a switching from CG-SDT to RA-SDT. For the subsequent CG transmissions, we need further discussion on how to handle the transmissions/retransmissions. So, for this change, we will likely need separate description for the initial CG-SDT transmission and the subsequent data transmission with CG resource during CG SDT. For the initial SDT type selection, I guess we can have a separate section (e.g. 5.x) instead of the section for CG transmission. For the subsequent data transmission with CG, I guess the SSB quality check can be captured in section 5.4.1 UL Grant reception (e.g. only deliver the UL grant to HARQ process in case the RSRP of the SSB associated to the UL grant is qualified). The understanding is that if there is no UL grant then RACH will be triggered (but this is normal RACH, not RA-SDT). |  | [Rapp] In the last meeting RAN2#114e, we have agreed on the following: - For initial CG transmission, UE does not select any SSB if none of the SSBs’ RSRP is above the RSRP threshold. FFS if re-evaluation for every CG transmission is necessary If you remember during the online discussion, there was an FFS for when none of the SSBs’ RSRP is above the threshold, what shall the UE do. Then, a comment from Huawei and Nokia proposed that this FFS should be removed because it is clear that if none of the SSB’s RSRP is above the threshold, the UE has no option but to do RACH. The only remaining issue is: when the UE is configured with RA-SDT, whether the UE is allowed to do RA-SDT Note that the MAC PDU still has not been built yet, hence there is no issue of MAC PDU rebuilding here. I can put the following Editor’s Note here, but the current text can be kept as it is, unless people disagree to fallback to RA-SDT after discussion. Editor’s Note: FFS whether CG-SDT can fallback to RA-SDT if none of the SSB’s RSRP is above the threshold for initial CG transmission. Please note that this is only for initial transmission, for subsequent uplink this is still FFS per the agreement above. On the organization of clauses, this is a bit complex but my understanding is that it is not quite proper to put the things related to SSB selection for CG and CG resource selection to section 5.4.1. Section 5.4.1 is used by multiple procedures as a common procedure and it is only related to processing UL grant, i.e., process the UL grant and deliver HARQ information and UL grant to HARQ entities. For the subsequent UL transmission, as long as the CG configuration for SDT is initialized, by initial CG transmission, for subsequent transmission, when CG occasions come, the UE can process the CG occasion and deliver the UL grant to UL grant reception section 5.4.1 (if the SSB selection is not performed for subsequent uplink). From my perspective, the current formulation is fine. |
| N007 | Why put the CG-SDT/RA-SDT selection in this section? Should be in section 5.X as part of CG validation. | Move the RSRP validation for CG-SDT and the CG/RA-SDT selection to section 5.X | [Rapp] Please see my comments above |

5.14	Handling of measurement gaps

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.15	Bandwidth Part (BWP) operation

5.15.1	Downlink and Uplink

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.16	SUL operation

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |

5.x	Small Data Transmission

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| L001 | The selection of BWP configured for SDT should be considered on SDT procedure. This is because a separate BWP for SDT can be configured, and we think it is also possible to configure multiple separate BWPs for SDT. | [LG] BWP switching from initial BWP to separate BWP for SDT should be considered when SDT procedure is initiated. BWP switching amongst separate BWPs configured for SDT is also considered. | [Rapp] Thanks for your comments @LGE I have put the following editor note for the issues raised by LGE. We can discuss on this in the future meetings, since the agreements we have now may not be enough for the correction here. Editor’s Note: FFS BWP switching when multiple BWPs are configured for CG-SDT |
| Z014 | General comment: Replace all occurrences of Small Data Transmission with SDT (except in the subclause heading). | Replace all occurrences of Small Data Transmission with SDT. | [Rapp] Corrected |
| N010 | 4> initiate Random Access procedure in clause 5.1 for CCCH logical channel (i.e., not for Small Data Transmission); This cannot be done without RRC intervention as the RRC procedure shall also change, we need only an indication to RRC that SDT cannot be initiated. | Should indicate to RRC layer other than just initiating non-SDT procedure by MAC when SDT verification fails which is not possible given the RRC procedure needs to change as well. | [Rapp] Thanks for the comment. I agree that based on the result of the email discussion, only an indication to the RRC of the non-SDT selection, is needed. After that the RRC will trigger legacy RACH like normally. Corrected the sentence to “indicate to the upper layer that the conditions to initiate SDT cannot be fulfilled” |
| N011 | For Small Data Transmission procedure, the MAC entity considers the suspended radios bearers configured with Small Data Transmission for data volume calculation. If RRC resumes the SDT bearers already, they are not suspended anymore. Furthermore, rather RRC shall do the data volume calculation before requesting MAC anything. | Remove the sentence. | [Rapp] I understand there is no pervious explicit agreement on this, but I think this is already implied the current MAC/RRC modeling, otherwise how can the UE do the SDT/non-SDT selection. If people still want to spend time to confirm on this, I am fine with it. Removed |

5.x.1	Validation for Small Data Transmission using CG

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| L002 | The expression "the time alignment value for SDT using CG type 1 to be valid " is not familiar. | [LG] The Text could be changed to " The MAC entity shall consider CG-SDT resource is valid when the following conditions are fulfilled:" | [Rapp] Corrected |
| L003 | TA timer should also be considered for validation for CG-SDT. | [LG] Add "1> cg-SDT-TimeAlignmentTimer is configured and running;" | [Rapp] Thanks for the comment. The condition cg-SDT-TAT is running is already implicit included here, since in Section 5.x.1, we have the condition “ if CG-SDT is configured on the selected UL carrier”. If cg-SDT-TAT is not running, there would be no CG-SDT resource configured |
| Z016 | For L003, please also see our comment above in Z102. To us it seems more discussion is needed to understand how the normal TA and the CG-SDT-TAT interact. |  | [Rapp] We can have more discussion on the interplay of CG-TAT and legacy TAT as discussed. |
| N012 | Why do we need a separate sub-section for this? | Could just be listed as conditions in section 5.x | [Rapp] Do we need to perform validation for every CG transmission? if yes, we need to keep this section separate; if no, as you said, we can put it under section 5.x. This is already reflected by the following editor’s Note, in case you have not read it Editor’s NOTE: FFS whether RSRP change would affect the TA valididation for DG. FFS whether the TA validation is only for initial CG-SDT transmission Keep it as it is |

6.1.5a	MAC PDU (MSGB)

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z017 | - a MAC subheader and MAC SDU for CCCH or DCCH or DTCH; Comment: Why was the DTCH added here. i.e. which agreement is this based on? Our understanding is that we did not agree any changes to MSGB format. | Remove the DTCH | [Rapp] This is from the WID that subsequent DL transmission is supported for all types of SDT. No new RRC state should be introduced in this WID. Transmission of smalldata in UL, subsequent transmission of smalldata in DL and the state transition decisions should be under network control. This would also include msgB for SDT based on 2-step RACH and implies a change to the msgB format. |
| N013 | Agree with Z017 |  | [Rapp] Please see the comments above |

Any Other Clause

| # | Brief description of the issue | Suggested resolution/company comments | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| N014 | Lots of Editor’s notes not based on any meeting FFS is added | Remove all the Editor’s notes that are not based on any discussions so far. | [Rapp] One important thing of doing this practice of running CR is that it can help us discover the remaining issues in time. This helps promote the progress of the topic. If there is any editor’s note people do not find suitable, you can point it out directly, just like thing s have been done above. |