3GPP TSG RAN WG2 Meeting #116-e                                                         R2-210xxxx

# Electronic meeting, Online, November 2021

Agenda item:		8.x

Source:			Intel Corporation

Title:		Summary of [Post115-e][605][POS] Pre-configured assistance data

Document for:	 	Discussion, Agreement

- Introduction

This document aims at collecting company views on pre-configured assistance data as per the following email discussion:

- [Post115-e][605][POS] Pre-configured assistance data (Intel)

Scope: Discuss signalling and validity criteria for pre-configured assistance data:

- Options for validity conditions:

    - Option A: Based on a validity area (e.g. a list of cells)

    - Option B: Based on a (configured) validity timer or a numerical limit on number of times it is utilized

    - Option C: Based on explicit modification or release from the LMF/NG-RAN

    - Option D: Based on the UE’s current location and/or the time

- Validity in relation to the duration of the positioning session

- Need for enhancements for signalling and use of pre-configured assistance data:

  - Add/mod/release mechanism for PRS configurations

  - Dynamic triggering of a preconfigured PRS at UE by LMF or gNB for making measurements on DL-PRS

  - Dynamic triggering of a preconfigured SRS at UE by gNB for transmitting SRS based on measurement report provided by UE

  - Priority indications for multiple (pre-)configured assistance data sets corresponding to multiple position fixes

- Stage 2 impact of pre-configured assistance data

Intended outcome: Report to next meeting

Deadline:  Long

- Contact Information

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | YinghaoGuo | yinghaoguo@huawei.com |
| ZTE | Yu Pan | pan.yu24@zte.com.cn |

Phase 1 discussion

Validity in relation to the duration of the positioning session

Based on the FFS from RAN2#114 meeting, there were proposals from several companies discussing the need for defining some validity criteria associated with the pre-configured assistance data in order to enable usage of the positioning assistance data for more than one consecutive positioning sessions. [6] proposes to optionally configure validity conditions for enabling usage of the positioning assistance data for more than one consecutive positioning sessions and lists several possible options. [8] and [13] proposes that the pre-configuration of assistance data is valid within a specific area and period. It is proposed in [10] that the pre-configured positioning assistance data is considered valid unless explicitly modified or released by the LMF/NG-RAN. [27] also proposes that the pre-configured assistance data can be activated based on the condition which can be specified by the UE’s current location and/or the time.

Based on the above, it would be good to get a baseline understanding of company views on when it would be useful to consider and associate validity condition for usage of pre-configured assistance data to the UE for a single positioning session. In other words, if pre-configured assistance data is configured during a positioning session and is only considered valid during that positioning session, whether we still need to consider additional validity conditions. So, companies are invited to comment on whether the validity condition(s) should be considered for usage of pre-configured assistance data across a single positioning session.

Question1-1: Do companies think validity condition(s) need to be defined for pre-configured assistance data configured during a single positioning session?

| Company | Yes/No | Comments |  |
| --- | --- | --- | --- |
| Huawei, HiSilicon | No | If assistance data is considered as valid only during a single positioning session, there is no gain for the assistance data pre-onfiguration for latency reduction compared with the current solution. |  |
| ZTE | No |  | Firstly to our understanding, the single positioning session in the question means a single LPP session. A single LPP session corresponds to a single location request (e.g., for a single MT-LR, MO-LR or NI-LR). Commonly there is only one of a LPP transaction for assistance data transfer in a single LPP session, so bring forward this assistance data transfer procedure will not reduce the latency. since pre-configured assistance data is not needed in a single LPP session, the validity conditions are not needed, too. |

Summary:

Question 1-2: If the answer to the above question is Yes, what should be UE behavior when the validity condition(s) for usage of pre-configured assistance data are no longer met?

| Company | Comments |
| --- | --- |

Summary:

Before discussing the applicability of the pre-configured assistance data to multiple positioning sessions, it should be discussed what the relationship between pre-configured assistance data and a given positioning session is, i.e. whether the former can be configured independently of a positioning session. Companies are invited to provide input to the following questions:

Question 2-1: Do companies think pre-configured assistance data can be configured independent of any positioning session, i.e. not necessarily configured for a specific positioning session?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | If pre-configuration is found useful, it should be kept even after the positioning sessions end such that it can be reused in the next positioning session. |
| ZTE | Yes | The question is opposite to Q1-1. Please refer to our comments on Q1-1 and Q2-2 |

Summary:

Depending on whether pre-configured assistance data is configured independent of any positioning session, the next question is whether pre-configured assistance data should be considered valid for more than one positioning session. In other words, whether the pre-configured assistance data can be utilized for positioning procedures across multiple positioning sessions should be discussed.

Question 2-2: Do companies think whether pre-configured assistance data should be considered valid for usage across multiple (consecutive) positioning sessions?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No, but see comments | For assistance data delivered by dedicated signaling, they should not be considered as valid across sessions as legacy. For assistance data delivered by broadcast, they should be considered as valid across positioning sessions, since they are not associated with positioning sessions. |
| ZTE | Yes | If the assistance data is pre-configured, that means multiple positioning sessions can use the pre-configured assistance data, and it saves the time of assistance data exchange/transfer procedure in these multiple positioning sessions. That is what pre-configured assistance data is aimed for. |

Summary:

If the answer to the above question is no, then we may not need to consider definition of validity conditions for pre-configured assistance data across multiple sessions since it can be implicitly assumed that previously pre-configured assistance data is no longer valid for a new positioning session. However, if the answer to the above question is yes, the same principle as in Question 1-1 needs to be established for multiple positioning sessions as well, so company views are invited for that case.

Question 2-3: If the answer to question 2-2 is yes, do companies agree that validity condition(s) need to be defined for usage of pre-configured assistance data across multiple (consecutive) positioning sessions?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | If pre-configured assistance data can be used across multiple LPP sessions, the validity conditions of pre-configured assistance data should also be the same, i.e., available across multiple LPP sessions. |

Summary:

Question 2-4: If the answer to the above question is Yes, what should be UE behavior when the validity condition(s) for usage of pre-configured assistance data are no longer met?

| Company | Comments |
| --- | --- |
| ZTE | We don’t think there should be an issue on the dissatisfaction about validity conditions needed to be solved. Explicitly activate or release the pre-configured assistance data from the LMF/NG-RAN is enough. |

Summary:

Validity conditions for pre-configured assistance data

Regarding the specific validity conditions to be defined, it was captured in the last meeting minutes to consider at least the following options:

	Option A: Based on a validity area (e.g. a list of cells)

	Option B: Based on a (configured) validity timer or a numerical limit on number of times it is utilized

	Option C: Based on explicit modification or release from the LMF/NG-RAN

	Option D: Based on the UE’s current location and/or the time

Companies are invited to comment on whether they support one or more of the validity conditions individually captured above.

Question 3-1: Regarding the validity conditions/criteria associated with pre-configured assistance data, do companies think validity condition based on a specified area (e.g. a list of cells where the pre-configured assistance data is considered valid) should be supported?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We think it is important that we first establish common understanding that the network and the UE should be synchronized on their understanding of the UE’s usage of assistance data. However, the current spec seems to lack such clarifications, e.g., the UE can keep the configuration that is receives long time ago, and what’s the relationship between the assistance data delivered by dedicated signaling and those delivered by broadcast. |
| ZTE | No | - The validity condition is quite difficult for a LMF to configure accurately. For example, what criteria should a LMF to take for determining a list of cells or a validity timer? How can LMF ensure the configured list of cells or a timer is actually valid for using assistance data? - The validity condition lacks the scheduling flexibility. - If a list of cells or validity timer is configured, an issue about dissatisfaction of validity conditions should also be solved, which leads to large spec impact. |

Summary:

Question 3-2: Regarding the validity conditions/criteria associated with pre-configured assistance data, do companies think validity condition based on a configured validity timer should be supported?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| ZTE | No | Similar comments as Q3-1 |

Summary:

Question 3-3: Regarding the validity conditions/criteria associated with pre-configured assistance data, do companies think validity condition based on an upper limit on the number of times the UE utilizes the assistance data for positioning should be supported?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| ZTE | No | Similar comments as Q3-1 |

Summary:

Question 3-4: Regarding the validity conditions/criteria associated with pre-configured assistance data, do companies think whether the UE continuing to use assistance data for positioning until explicit modification/release by LMF/NG-RAN should be supported?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | We think this can be enabled by release/add/modification mechanism |
| ZTE | Yes | This method brings the largest flexibility to the network |

Summary:

Question 3-5: Regarding the validity conditions/criteria associated with pre-configured assistance data, do companies think validity condition based on UE’s current position/location should be supported?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| ZTE |  | Firstly, this option is not a parallel option compared to option A, B and C. It should be assumed as an additional conditions of option A and B. Although we are not supportive of option A and B, it still make sense that the option A and B should be based on the UE’s current location. |

Summary:

In addition to the above options captured in the meeting minutes, companies are invited to propose if they have some other validity condition in mind to consider. Companies are suggested to add helpful details for the proposed options to make sure the proposed solution is clearly understood.

Question 3-5: Do companies think some other validity condition(s) than those discussed in Questions 3-1 to 3-5 should be supported? If so, please provide details of how it would work.

| Company | Comments |
| --- | --- |

Summary:

Need for enhancements for signalling and use of pre-configured assistance data

Several enhancements related to signaling and use of pre-configured assistance data were proposed by companies during the last meeting. As per the meeting minutes, the following enhancements were identified for further discussion:

- Add/mod/release mechanism for PRS configurations and complete definition of priority of PRS configuration for measurement, including the PRS configuration received by dedicated LPP signalling and posSIB [7] - Dynamic triggering of a preconfigured PRS at UE by LMF or gNB for making measurements on DL-PRS [13] - Dynamic triggering of a preconfigured SRS at UE by gNB for transmitting SRS based on measurement report provided by UE [13] - Priority indications for multiple (pre-)configured assistance data sets corresponding to multiple position fixes [15]

Given the fact that the above options are based on individual company proposals and there has not been an opportunity to collect views form other companies on the need and feasibility of supporting these enhancements,  rapporteur thinks it would be good to collect company views and comments in Phase 1 discussion. Based on the outcome, we can then discuss further details as needed in Phase 2.

Question 4-1: Which of the following proposed enhancements need to be pursued in Rel-17 NR positioning discussion? (Select all that apply)

- The introduction of an Add/mod/release mechanism for PRS configurations  and a complete definition of priority of PRS configuration for measurement

- Dynamic triggering of a preconfigured PRS at UE by LMF or gNB for making measurements on DL-PRS

- Dynamic triggering of a preconfigured SRS at UE by gNB for transmitting SRS based on measurement report provided by UE

- Priority indications for multiple (pre-)configured assistance data sets corresponding to multiple position fixes

| Company | Options | Comments/Reason |
| --- | --- | --- |
| Huawei, HiSilicon | 1 | We think that 1 can be the baseline solution for R17 latency reduction. With this mechanism, the network can synchronize its record of the UE’s AD with the UE. The current issue with priority of PRS configuration is that (a) priority is not defined between different frequency layers (b) priorities handling between AD received by dedicated signaling and broadcast are not defined. After these problems are resolved, we may think about other optimizations, such as solution 4 for dynamically varying the priorities between different configurations. |
| ZTE | Option 2 | Option 2 is almost the same as option C in section 3-2, which brings the largest flexibility to the network |

Summary:

Other issues

Companies are invited to comment whether there are any other open issues with respect to pre-configuration of assistance data that need to be discussed.

Question 5-1: Do companies think there are any other critical issues to be addressed regarding pre-configured assistance data?

| Company | Comments |
| --- | --- |

Summary:

- Phase 2 discussion

- References

- R2-2107090	Discussion on positioning latency reduction	ZTE	discussion

- R2-2107091	Discussion on scheduled location time	ZTE	discussion

- R2-2107132	Discussion on Response LS on Scheduling Location in Advance to reduce Latency from SA2	CATT	discussion	Rel-17	NR_pos_enh-Core

- R2-2107134	Discussion on Enhancements for Latency Reduction	CATT	discussion	Rel-17	NR_pos_enh-Core

- R2-2107135	Discussion on storage of UE Positioning Capabilities LS from SA2 and the granularity of response time LS from RAN1	CATT	discussion	Rel-17	NR_pos_enh-Core

- R2-2107399	Further consideration of positioning latency enhancements	OPPO	discussion	Rel-17	NR_pos_enh-Core

- R2-2107500	Discussion on positioning latency	Huawei, HiSilicon	discussion	Rel-17	NR_pos_enh-Core

- R2-2107641	Discussion on latency enhancement	vivo	discussion	Rel-17	NR_pos_enh-Core

- R2-2107642	Discussion on Scheduling Location in Advance to reduce Latency	vivo	discussion	Rel-17	NR_pos_enh-Core

- R2-2107670	Scheduled location time based latency reduction	Intel Corporation	discussion	Rel-17	NR_pos_enh

- R2-2107673	Storing UE positioning capability in AMF	Intel Corporation	discussion	Rel-17	NR_pos_enh

- R2-2107680	Summary of agenda 8.11.2	Latency enhancements	Intel Corporation	discussion	Rel-17	NR_pos_enh	Late

- R2-2107681	Discussion on Enhancements for Latency Reduction	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh

- R2-2107962	Discussion on the response time	Samsung	discussion	Rel-17

- R2-2108127	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2108175	Positioning enhancements on latency reduction	Xiaomi	discussion

- R2-2108367	Scheduling Location in Advance to Reduce Latency 	Qualcomm Incorporated	discussion

- R2-2108376	[draft] Response LS on Scheduling Location in Advance to reduce Latency	Qualcomm Incorporated	LS out	Rel-17	FS_NR_pos_enh	To:SA2	Cc:RAN1, RAN3

- R2-2108377	LPP impacts for UE positioning capability storage	Qualcomm Incorporated	discussion

- R2-2108378	[draft] Response LS on storage of UE Positioning Capabilities	Qualcomm Incorporated	LS out	Rel-17	To:SA2	Cc:RAN3

- R2-2108393	Utilizing Time T and other associated parameters	Ericsson	discussion

- R2-2108397	On UE Positioning Capabilities	Ericsson	discussion

- R2-2108536	Discussion on latency reduction for positioning	CMCC	discussion	Rel-17	NR_pos_enh-Core

- R2-2108704	Enhancement to reduce latency for high volume positioning	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

- R2-2108769	Handling of multiple QoS for latency reduction	Samsung Electronics	discussion	NR_pos_enh-Core

- R2-2108771	Latency reduction via configured grant for positioning	Samsung Electronics	discussion	NR_pos_enh-Core

- R2-2108773	Discussion on the scheduled location time	Samsung Electronics	discussion	NR_pos_enh-Core

- R2-2106918	Reply LS to SA2 on Scheduling Location in Advance (R1-2106312; contact: Qualcomm)	RAN1	LS in	Rel-17	NR_pos_enh	To:SA2	Cc:RAN2, RAN3

- R2-2107133	Draft Response LS to SA2 on the scheduled location time	CATT	LS out	Rel-17	NR_pos_enh-Core	To:SA2	Cc:RAN1, RAN3

- R2-2106919	LS on granularity of response time (R1-2106316; contact: Huawei)	RAN1	LS in	Rel-17	NR_pos_enh	To:RAN2

- R2-2106971	LS on storage of UE Positioning Capabilities (S2-2105153; contact: Qualcomm)	SA2	LS in	Rel-17	5G_eLCS_ph2	To:RAN2	Cc:RAN3

- R2-2107680	Summary of agenda 8.11.2	Latency enhancements	Intel Corporation	discussion	Rel-17	NR_pos_enh